A TECHNICAL REPORT AND PRESENTATION

ON

STUDENT INDUSTRIAL WORK EXPERIENCE SCHEME (SIWES)

HELD AT

SWIFT WIRE TECH SOLUTION

AYEDUN COMMUNITY, OKINNI, OSOGBO, OSUN STATE.

NAME: OBISESAN BASIT OLAMIDE

MATRIC NUMBER: ND/23/EEE/PT/0098

DEPARTMENT OF ELECTRICAL ELECTRONIC ENGINEERING

INSTITUTE OF INFORMATION TECHNOLOGY

KWARA STATE POLYTECHNIC.

P.M.B 1375, ILORIN

AUGUST-DECEMBER 2025

DEDICATION

I dedicate this technical report to Almighty God and the giver of knowledge, wisdom and understanding. The giver of all in all and sustainer of my life.

I also dedicate this report to my lovely, dearly and caring parents elders, siblings, dearly and friends.

ACKNOWLEDGMENTS

I would like to express my sincere gratitude to the management and staff of [Swift Wire Tech Solution] for providing me with the opportunity to undergo my SIWES program at their esteemed organization. Special thanks to my supervisor for their guidance, support, and mentorship throughout my internship.

TABLE OF CONTENTS

Title Page											i

Dedication											ii

Acknowlegment										iii

Table of contents										iv

CHAPTER ONE

1.1 Background

1.2 Importance and Objectives of Siwes

1.3 Objectives of Siwes

1.4 Importance of Siwes

CHAPTER TWO

2.0 Description of Workdone

2.1 Installation of an Inverter

2.2 Laying of Pipes for a Four-bedroom House

2.3 Coupling and Installation of a New Ceiling Fan

2.4 Installation of a Wall Stabilizer

2.5 Repair an Extension Socket

2.6 Installation of 3-Phase Meter

2.7 Installation of Plumbing Machine Control Box

2.8 Wiring a Room

2.9 Connecting Aluminum Wiring to a Wooden Pole

3.0 Installing a Lightning Arrester

CHAPTER THREE

3.0 Description of Workdone

3.1 Wiring Fence Lights

3.2 Gear Selector Switch

3.3 How to Lay Pipes for Camera Installation?

3.4 Clearing an Electrical Fault

3.5 Coupling Distribution (D.W) Three-Phased Board

3.6 Room Self-Con (Self-Contained) Unit

3.7 Installation of One-Way Gang Switch with a Point of Light

3.8 Installation of Generator Wiring

3.9 Installation of GOTV

4.0 How to Change a Black Cutout to a Ceramic Cutout?

CHAPTER FOUR

4.1 Challenges Faced

4.2 Conclusion

CHAPTER ONE

1.1	BACKGROUND

SIWES stands for acronym for Student Industrial Work Experience Scheme.

The Student Industrial Work-Experience Scheme (SIWES) is a planned and supervised training intervention based on stated and specific learning and career objectives, geared towards developing the occupational competencies of the participants.

It is a programme required to be undertaken by all students of tertiary institutions in Nigeria pursuing courses in “specialized engineering, technical, business, applied sciences and applied arts and Mass Communication”.

The Federal Government on 8th October 1971 established the industrial training (ITF) to acquaint the students on the industrial work method. The SIWES which is a subsidiary formed in 1973 was initiated to improve the students’ technical abilities to expose them to industrial culture thereby getting the acquainted with the role to play towards the technological advancement of the nation.

It creates an avenue on environment in which the students are exposed to areas of their various disciplines to enhance their mental and creative minds in the aspects of Practical.

It is therefore a practical aspect of the academic work; which students may not have opportunity to carry out throughout their stay in the higher institutions.

1.2 	IMPORTANCE AND OBJECTIVES OF SIWES

The Students Industrial Work Experience Scheme (SIWES) is the accepted skills training programme, which forms part of the approved Minimum Academic Standards in the various degree programmes for all the Nigerian universities.

It is an effort to bridge the gap existing between theory and practice of engineering and technology, science, agriculture, medial, management and other professional educational and Mass Communications programmes in the Nigerian tertiary institutions.

It is aimed at exposing students to machines and equipment, professional work methods and ways of safe-guarding the work area s and workers in industries and other organization.

1.3	OBJECTIVES OF SIWES

Specifically, the objectives of the student’s industrial work experience scheme are to

i.)	Prepare students for the work situation they are likely to meet after graduation.

ii.)    Provide an avenue for students in the Nigerian Universities to acquire industrial skills and experience in their course of study.

iii.)    Make the transition from the university to the world of work easier, and thus enhance students’ contacts for later job placements.

iv.)   	Enlist and strengthen employers’ involvement in the entire educational process of preparing university graduates for employment   in industry.

v)	Provide students with an opportunity to apply their theoretical knowledge in real work situation, thereby bridging the gap between university work and actual practices; and Expose students to work methods and techniques in handling equipment and machinery that may not be available in the universities.

1.4	IMPORTANCE OF SIWES

i)	It provides students with an opportunity to apply their theoretical knowledge in real life situations.

ii.)	It exposes students to more practical work methods and techniques.

iii.)	It strengthens links between the employers, universities and industrial training fund (ITF).

iv). 	It also prepares the students for the labour   market after graduation.

CHAPTER TWO

2.0 DESCRIPTION OF WORK DONE

2.1 INSTALLATION OF AN INVERTER

Key points to consider when installing an inverter:

- Location: Choose a well-ventilated area away from direct sunlight and moisture. Ensure there's enough space around the inverter for cooling.

- Mounting: Mount the inverter securely on a sturdy surface using appropriate hardware to prevent vibrations and ensure safety.

- Wiring: Follow the manufacturer's instructions for proper wiring, ensuring correct polarity and gauge size for both DC and AC connections.

- Grounding: Properly ground the inverter according to local electrical codes to prevent electrical shock and ensure safety.

- Protection: Install appropriate overcurrent protection devices, such as fuses or circuit breakers, on both the DC and AC sides of the inverter.

- Clearance: Leave adequate space around the inverter for maintenance and airflow. Avoid obstructing ventilation ports.

- Cooling: Ensure sufficient airflow around the inverter to prevent overheating, especially during heavy usage.

- Monitoring: Consider installing monitoring equipment to track the performance and status of the inverter remotely, if available.

- Safety: Always prioritize safety during installation. If you're not confident in your abilities, consider hiring a qualified electrician to install the inverter.

- Testing: After installation, test the inverter to ensure proper operation and performance before putting it into regular use.

2.2 LAYING OF PIPES FOR A FOUR-BEDROOM HOUSE

Laying Pipes for Four-Bedroom House

Objective: To outline the procedure for the installation of plumbing pipes for a four-bedroom residential building.

PROCEDURE:

1. Planning: Begin by creating a detailed plumbing plan, marking the location of fixtures, water outlets, and the main supply line. Ensure compliance with local building codes and regulations.

2. Supply Line Installation:

- Install the main water supply line from the source to the house, typically underground. Use PVC pipes for cold water and copper pipes for hot water.

- Connect the supply line to the house plumbing system, ensuring proper fittings and connections.

3. Distribution System:

- Lay out the pipes from the main supply line to each fixture location, including sinks, toilets, showers, and appliances.

- Use appropriate fittings and supports to secure the pipes along the walls or ceiling, ensuring proper slope for drainage.

4. Hot Water Lines:

- Install copper pipes for hot water distribution, using soldered fittings to connect pipes and fixtures.

- Insulate hot water pipes to prevent heat loss and ensure energy efficiency.

5. Cold Water Lines:

- Use PVC pipes for cold water lines, securing them with pipe straps and fittings as per the plumbing plan.

- Consider installing shut-off valves at strategic points for maintenance and emergencies.

6. Fixture Connections:

- Connect each fixture to the respective water supply and drainage lines, ensuring watertight seals with plumber's putty or Teflon tape.

- Test each fixture for leaks and proper water flow.

7. Final Inspection:

- Conduct a thorough inspection of the entire plumbing system, checking for leaks, proper fittings, and compliance with regulations.

- Make any necessary adjustments or repairs before completing the installation.

SAFETY PRECAUTIONS:

- Use appropriate personal protective equipment (PPE), such as gloves and safety goggles, when working with pipes and fittings.

- Exercise caution when using soldering torches to avoid fire hazards and burns.

- Follow manufacturer's instructions and best practices for handling and installing plumbing materials.

CONCLUSION:

Proper installation of plumbing pipes is crucial for the efficient and reliable operation of a residential water supply system. By following the outlined procedure and adhering to safety protocols, you can ensure a successful plumbing installation for a four-bedroom house.

Feel free to adjust the details as needed based on specific requirements or preferences.

2.3 COUPLING AND INSTALLATION OF A NEW CEILING FAN

Objective: To provide guidance on safely coupling and installing a new ceiling fan.

PROCEDURE:

1. Preparation:

- Turn off the power to the existing ceiling fixture at the circuit breaker to prevent electrical accidents.

- Gather all necessary materials and tools.

2. Remove Old Fixture (if applicable):

- Unscrew and remove the existing ceiling fixture, exposing the electrical box and wiring.

3. Mounting Bracket Installation:

- Install the mounting bracket onto the electrical box securely using screws and anchors provided with the ceiling fan.

4. Ceiling Fan Assembly:

- Follow the manufacturer's instructions to assemble the ceiling fan components, including attaching the blades to the motor housing.

5. Electrical Connections:

- Identify the wires in the electrical box: typically, there will be a black (hot), white (neutral), and green or bare copper (ground) wire.

- Connect the corresponding wires from the ceiling fan (black to black, white to white, green to ground), securing them with wire nuts and wrapping with electrical tape for added safety.

- If installing a ceiling fan with a light fixture, connect the additional wires following the manufacturer's instructions.

6. Mounting the Fan:

- Lift the assembled ceiling fan and align it with the mounting bracket on the ceiling.

- Secure the fan to the bracket using the provided screws, ensuring it is stable and level.

7. Finishing Touches:

- Install any included light kit onto the ceiling fan if applicable.

- Attach any pull chains or remote control receiver (if included) according to the manufacturer's instructions.

8. Testing:

- Turn the power back on at the circuit breaker.

- Test the fan and light (if applicable) to ensure proper functionality.

- Check for any wobbling or unusual noises, indicating improper installation.

SAFETY PRECAUTIONS:

- Always turn off the power at the circuit breaker before beginning any electrical work.

- Use a stable ladder or platform when working at heights.

- Follow all manufacturer instructions and local electrical codes.

CONCLUSION:

By following these steps carefully and ensuring proper electrical connections and mounting, you can safely couple and install a new ceiling fan, providing comfort and functionality to the room.

Feel free to reach out if you need further assistance or clarification on any step.

2.4 INSTALLATION OF A WALL STABILIZER

To install a wall stabilizer (assuming you're referring to a voltage stabilizer for electrical appliances), follow these steps:

PROCEDURE:

1. Choose Location:

- Select a suitable location near the main power source where the stabilizer can be easily accessed and where it won't obstruct movement or other appliances.

2. Mounting:

- Use mounting brackets to secure the stabilizer to the wall. Ensure the brackets are securely attached and can support the weight of the unit.

3. Electrical Connection:

- Turn off the main power supply to the area where you'll be installing the stabilizer.

- Use a voltage tester to confirm that the power is off.

- Remove the cover of the stabilizer to access the internal wiring.

- Connect the incoming power supply wires (usually live, neutral, and ground) to the corresponding terminals on the stabilizer, following the manufacturer's instructions.

- Use wire strippers to strip the insulation off the wires, then securely fasten them to the terminals using screws or other provided connectors.

- Insulate any exposed wires with electrical tape.

- If the stabilizer has output terminals, connect the output wires to the distribution board or to the appliances you want to protect.

4. Grounding:

- Ensure that the stabilizer is properly grounded according to local electrical codes. Connect the grounding wire to the designated terminal on the stabilizer.

5. Testing:

- Once the connections are made, turn the main power supply back on.

- Test the stabilizer to ensure it is functioning correctly by connecting appliances and monitoring the voltage output.

- If your stabilizer has a digital display, verify that it shows the correct voltage level.

6. Final Checks:

- Double-check all connections to make sure they are tight and secure.

- Confirm that there are no exposed wires or potential safety hazards.

- Test the stabilizer under load conditions to ensure it can handle the required power demands.

7. Documentation:

- Keep the manufacturer's manual and any warranty information in a safe place for future reference.

SAFETY PRECAUTIONS:

- Always turn off the power before working with electrical connections.

- Use appropriate safety gear, such as gloves and safety glasses, when handling electrical components.

- If you're unsure about any step of the installation process, consult a qualified electrician.

By following these steps, you can install a wall stabilizer to protect your electrical appliances from voltage fluctuations and ensure safe and reliable operation.

2.5 REPAIR AN EXTENSION SOCKET

To repair an extension socket, follow these steps:

PROCEDURE:

1. Safety First:

- Ensure the power to the extension socket is turned off at the circuit breaker to prevent electrical shock.

2. Inspect the Damage:

- Examine the socket for any visible damage, such as cracks, burns, or loose connections. Determine if the socket can be repaired or if it needs to be replaced entirely.

3. Disassemble the Socket:

- If the socket is repairable, use a screwdriver to remove the cover plate and expose the internal wiring.

4. Check Wiring Connections:

- Inspect the wiring connections inside the socket for any signs of damage, loose wires, or corrosion.

- Use a voltage tester to ensure there is no power flowing to the socket before proceeding.

5. Repair or Replace Wires:

- If any wires are damaged or loose, use a wire stripper to trim the ends and expose fresh wire.

- Reconnect the wires securely to the appropriate terminals on the socket. Match the colors of the wires (usually black for live, white for neutral, and green or bare copper for ground) to the corresponding terminals.

6. Secure Connections:

- Twist the wires together and use wire nuts or terminal connectors to secure the connections.

- Wrap electrical tape around the wire nuts or connectors to provide insulation and prevent them from coming

2.6 INSTALLATION OF 3-PHASE METER

To install a 3-phase meter, which is commonly used in industrial or commercial settings, follow these steps:

PROCEDURE:

- Preparation:

  - Ensure you have the necessary permits and approvals from the local utility company before proceeding with the installation.

  - Determine the appropriate location for installing the meter, following the utility company's guidelines and regulations.

- Mounting the Meter Socket:

  - Install the meter socket or enclosure securely on a wall or mounting surface, ensuring it is level and properly grounded according to local codes.

- Connect Incoming Power:

  - Turn off the power at the main breaker panel before working on electrical connections.

  - Connect the incoming 3-phase power cables to the appropriate terminals in the meter socket. Typically, there will be three hot wires (L1, L2, L3), a neutral wire, and a ground wire.

  - Use wire connectors to secure the connections, and insulate them with electrical tape.

- Install Meter:

  - Mount the 3-phase meter securely onto the meter socket or enclosure, following the manufacturer's instructions.

  - Ensure the meter is properly aligned and seated in the socket.

- Connect Load Wiring:

  - Connect the load wires from the electrical distribution panel to the corresponding terminals on the meter. These wires will supply power to the building's electrical circuits.

  - Again, use wire connectors to secure the connections and insulate them with electrical tape.

- Testing:

  - Turn on the power at the main breaker panel.

  - Use a voltage tester to verify that there is power at the meter and that it is functioning correctly.

  - Check the display on the meter to ensure it is accurately measuring the incoming power.

- Final Checks:

  - Double-check all connections to ensure they are tight and secure.

  - Confirm that the meter is properly sealed and that there are no exposed wires or hazards.

  - Record the meter reading and document the installation for future reference.

- Utility Verification:

  - Contact the utility company to schedule an inspection and verification of the meter installation.

- Ensure that the installation meets all regulatory requirements and specifications provided by the utility company.

SAFETY PRECAUTIONS:

- Always turn off the power before working on electrical installations.

- Use proper safety gear and tools when handling electrical components.

- If you're unsure about any step of the installation process, consult a qualified electrician or contact the utility company for assistance.

By following these steps carefully and adhering to safety protocols, you can successfully install a 3-phase meter for your industrial or commercial facility.

2.7 INSTALLATION OF PLUMBING MACHINE CONTROL BOX

To install a plumbing machine control box, such as those used in water pump systems, follow these general steps:

PROCEDURE:

1. Preparation:

- Ensure you have the necessary tools and materials for the installation.

- Identify a suitable location for mounting the control box, preferably near the pump and within reach of the electrical supply.

2. Mounting:

- If the control box requires mounting, securely attach it to a wall or mounting surface using the provided hardware. Ensure the mounting surface is sturdy and can support the weight of the control box.

3. Electrical Connections:

- Turn off the power supply to the area where you'll be installing the control box.

- Use a voltage tester to confirm that the power is off before proceeding.

- Remove the cover of the control box to access the internal wiring.

- Connect the incoming power supply wires to the terminals inside the control box according to the manufacturer's instructions. Typically, there will be terminals for live (L), neutral (N), and ground (G) wires.

- Use wire connectors to secure the connections, and insulate them with electrical tape.

- If the control box has additional terminals for connecting to the pump or sensors, follow the wiring diagram provided in the user manual to make the appropriate connections.

4. Testing:

- Turn the power back on at the circuit breaker.

- Test the control box to ensure it is functioning correctly. Check for any indicators or display lights that confirm power supply and proper operation.

- If the control box is connected to a pump, test the pump to ensure it starts and stops as expected when the control box is activated.

5. Final Checks:

- Double-check all electrical connections to ensure they are tight and secure.

- Confirm that the control box is properly sealed and that there are no exposed wires or hazards.

- Refer to the user manual for any specific setup or calibration procedures required for your control box model.

6. Documentation:

- Keep the user manual and any warranty information in a safe place for future reference.

- Document the installation details for reference and troubleshooting purposes.

SAFETY PRECAUTIONS:

- Always turn off

2.8 WIRING A ROOM

Wiring a room involves several steps to ensure safety and functionality. Here's a basic guide:

PROCEDURE:

1. Design and Plan:

- Determine the layout of the room and decide where you want electrical outlets, switches, and light fixtures to be located.

- Plan the wiring route, considering the shortest and most efficient paths from the circuit breaker panel to each outlet and fixture.

2. Turn Off Power:

- Before beginning any electrical work, turn off the power to the room at the circuit breaker panel. Use a voltage tester to confirm that the power is off.

3. Install Electrical Boxes:

- Install electrical boxes for outlets, switches, and light fixtures according to your planned layout. Secure them to the wall studs using screws.

4. Run Wiring:

- Run electrical cables from the circuit breaker panel to each electrical box, following your planned wiring route. Leave extra cable length at each box to make connections easier.

- Use appropriate cable staples or straps to secure the cables to the wall studs and prevent them from sagging or coming loose.

5. Connect Outlets and Switches:

- Strip the ends of the electrical wires inside each electrical box using a wire stripper.

2.9 CONNECTING ALUMINIUM WIRING TO A WOODEN POLE

Connecting aluminum wiring to a wooden pole requires a secure and safe installation method to ensure longevity and reliability. Here's a general guide to accomplish this task:

Procedure:

1. PREPARATION:

- Ensure the wooden pole is clean and free from any obstructions.

- Cut the aluminum wiring to the desired length, leaving some extra for allowances.

2. Mounting the Aluminum Wiring:

- Decide on the position along the wooden pole where you want to attach the aluminum wiring.

- Use metal clamps or brackets designed for outdoor use to securely attach the aluminum wiring to the wooden pole. Position the clamps evenly along the length of the pole to provide adequate support.

- Secure the clamps or brackets to the wooden pole using screws or bolts, ensuring they are tight and stable.

3. Connecting the Aluminum Wiring:

- Strip the insulation from the ends of the aluminum wiring using wire strippers, exposing the bare metal.

- If necessary, apply an antioxidant compound to the exposed aluminum wire to prevent oxidation and ensure a good electrical connection.

- Use wire connectors suitable for aluminum wiring to connect the aluminum wiring to the electrical system or other components as needed. Follow the manufacturer's instructions for proper installation of the connectors.

4. Testing:

- Once the aluminum wiring is securely mounted and connected, test the electrical system to ensure proper functionality

3.0 INSTALLING A LIGHTNING ARRESTER

Installing a lightning arrester, also known as a lightning rod or thunder arrester, requires careful planning and execution to ensure effective protection against lightning strikes. Here's a general guide to install a lightning arrester:

PROCEDURE:

1. Location Selection:

- Choose a suitable location for installing the lightning arrester. It should be the highest point on the building or structure to attract lightning away from critical areas.

2. Mounting the Lightning Arrester:

- Securely mount the lightning arrester to the highest point of the building or structure using appropriate mounting hardware. Follow the manufacturer's instructions for proper installation.

3. Grounding:

- Drive a grounding rod into the ground near the lightning arrester. The grounding rod should be made of copper and should extend deep into the earth for effective grounding.

- Connect the copper grounding wire to the lightning arrester using suitable connectors and clamps. Ensure a secure and tight connection to provide a low-resistance path to ground.

- Connect the other end of the grounding wire to the grounding rod using similar connectors and clamps. Drive the grounding rod deep into the earth until only a few inches are visible above the ground surface.

4. Testing:

- Use a voltage tester to verify that the lightning arrester

CHAPTER THREE

3.0 DESCRIPTION OF WORK DONE

3.1 WIRING FENCE LIGHTS

Wiring fence lights involves connecting them to a power source and possibly installing switches for control. Here's a general guide to help you with the process:

PROCEDURE:

1. Plan the Layout:

- Decide where you want to install the fence lights along the fence line. Consider spacing, aesthetics, and practicality.

2. Prepare the Power Source:

- Determine the nearest power source for the lights, which could be an outdoor electrical outlet or an existing electrical circuit.

- Turn off the power supply to the area where you'll be working, either at the circuit breaker or by unplugging the outdoor outlet.

3. Run Electrical Cables:

- Run electrical cables from the power source to each fence light location. If needed, use a drill to create holes for the cables to pass through any fences or walls.

- Ensure the cables are buried underground or secured along the fence line to protect them from damage.

4. Connect the Lights:

- Strip the insulation from the ends of the electrical cables using wire strippers.

- Connect the wires from each fence light to the corresponding wires in the electrical cable using wire connectors. Match the wires according to their colors (typically black for hot, white for neutral, and green or bare copper for ground).

- Insulate the wire connectors with electrical tape to prevent exposure to moisture

3.2 GEAR SELECTOR SWITCH

To connect a gear switch (also known as a gear selector switch) for a grinding machine, you'll need to wire it properly to control the machine's operation. Here's a general guide to help you with the process:

PROCEDURE:

1. Preparation:

- Ensure the power to the grinding machine is turned off at the main switch or circuit breaker panel.

- Gather all the necessary materials and tools for the installation.

2. Identify Wiring Points:

- Determine the wiring points on both the gear switch and the grinding machine. The gear switch typically has terminals labeled for connection.

3. Wire Stripping:

- Strip the insulation from the ends of the electrical cables using a wire stripper. Ensure you strip enough insulation to make proper connections.

4. Connect the Power Supply:

- Identify the power supply wires coming into the gear switch. These are usually live (hot), neutral, and ground wires.

- Connect the live wire from the power supply to the appropriate terminal on the gear switch. This is typically labeled as "L" or "LIVE."

- Connect the neutral wire from the power supply to the corresponding terminal on the gear switch. This is usually labeled as "N" or "NEUTRAL."

- Connect the ground wire from the power supply to the ground terminal on the gear switch.

5. Connect the Grinding Machine:

- Identify the wires from the grinding machine that need to be connected to the gear switch. These wires typically control the operation of the machine.

- Connect the wires from the grinding machine to the appropriate terminals on the gear switch, following the manufacturer's instructions or wiring diagram.

- Ensure the connections are secure and properly insulated using wire connectors.

6. Testing:

- Turn on the power supply to the grinding machine at the main switch or circuit breaker panel.

- Test the gear switch to ensure it properly controls the operation of the grinding machine. Verify that each gear position corresponds to the correct machine operation.

7. Final Checks:

- Double-check all connections to ensure they are tight and secure.

- Use a voltage tester to verify that there is power at the gear switch terminals and that the switch is functioning correctly.

SAFETY PRECAUTIONS:

- Always turn off the power before working on electrical installations.

- Use proper safety gear and tools when handling electrical components.

- If you're unsure about any step of the installation process, consult a qualified electrician.

By following these steps carefully, you can effectively connect a gear switch to control a

3.3 HOW TO LAY PIPES FOR CAMERA INSTALLATION?

To lay pipes for camera installation, particularly for surveillance or security cameras, you'll need to plan the layout carefully to ensure proper coverage and functionality. Here's a general guide to help you with the process:

PROCEDURE:

1. Plan the Layout:

- Determine the locations where you want to install the cameras and plan the pipe layout accordingly.

- Consider factors such as the area to be covered, camera angles, distances between cameras, and accessibility for maintenance.

2. Dig Trenches:

- Use digging tools to excavate trenches where the pipes will be laid. The depth of the trenches will depend on local building codes and the type of pipes being used.

3. Lay the Pipes:

- Lay the PVC pipes (or conduit pipes) in the trenches, following the planned layout. Use pipe fittings as needed to navigate corners or changes in direction.

- Ensure the pipes are laid securely and at the proper depth to protect them from damage and to provide adequate support.

4. Secure the Pipes:

- Use pipe straps or clamps to secure the pipes to the ground or to other structures, such as walls or fences. This helps prevent movement and protects the pipes from accidental damage.

5. Install Camera Wiring (if applicable):

- If the cameras require wiring, such as power cables or data cables, feed the wires through the pipes before sealing them.

- Use cable ties to secure the wires inside the pipes and prevent them from tangling or becoming damaged.

6. Seal the Pipes (if applicable):

- If you're using PVC pipes, use cement and pipe glue to seal the pipe joints and connections. This helps prevent water ingress and ensures the pipes remain watertight.

7. Test the Installation:

- Once the pipes are laid and sealed, test the camera installation to ensure proper functionality.

- Connect the cameras to the wiring (if applicable) and verify that they are receiving power and transmitting video signals correctly.

8. Final Checks

3.4 CLEARING AN ELECTRICAL FAULT

Clearing an electrical fault involves identifying and rectifying any issues within an electrical system that are causing malfunctions or safety hazards. Here's a general guide on how to clear electrical faults:

1. Identify the Fault:

- Use visual inspection and testing equipment such as a multimeter to identify the source of the electrical fault. Look for signs of damage, overheating, loose connections, or other abnormalities in the electrical system.

2. Isolate the Power:

- Before attempting any repairs, turn off the power to the affected circuit or area at the circuit breaker panel. Ensure that the power is completely disconnected to prevent electrical shock or further damage.

3. Inspect Wiring and Connections:

- Examine the wiring, outlets, switches, and other electrical components for any signs of damage, wear, or loose connections. Tighten any loose connections and replace any damaged components as necessary.

4. Test Circuits and Components:

- Use a multimeter or other testing equipment to check the continuity, voltage, and resistance of circuits, wires, and components. Test each component systematically to identify any abnormalities.

5. Replace Faulty Components:

- If any components are found to be faulty during testing, such as a damaged wire, outlet, switch, or circuit breaker, replace them with new ones of the same type and rating. Ensure that replacements are installed correctly and securely.

6. Check Grounding and Earthing:

- Verify that the electrical system is properly grounded and earthed to prevent electrical shocks and fire hazards. Inspect grounding connections and electrodes and repair or replace them if necessary.

7. Reset Circuit Breakers:

- After addressing the fault and making necessary repairs, reset the circuit breakers or replace any blown fuses. Ensure that the circuit breaker panel is functioning correctly and that all circuits are properly labeled.

8. Test the Electrical System:

- Once repairs are completed and power is restored, test the electrical system to ensure that the fault has been cleared and that all components are functioning properly. Check for proper voltage levels, continuity, and operation of switches, outlets, and appliances.

9. Perform Preventive Maintenance:

- Regularly inspect and maintain the electrical system to prevent future faults and ensure safety and reliability.

3.5 COUPLING DISTRIBUTION (D.W) THREE-PHASED BOARD

Coupling a distribution (D.W) three-phase board involves connecting it to the main power supply and distributing power to various circuits or loads. Here's a general guide on how to couple a three-phase distribution board:

PROCEDURE:

1. Turn Off Power:

- Turn off the main power supply to the area where you'll be installing the distribution board. This ensures safety during installation.

2. Mount the Distribution Board:

- Choose a suitable location for mounting the distribution board. It should be easily accessible and comply with any local regulations or codes regarding clearances and accessibility.

- Securely mount the distribution board to the wall using appropriate screws and anchors.

3. Connect Power Supply:

- Identify the incoming three-phase power supply cables. These usually consist of three live (phase) wires (L1, L2, L3), one neutral wire, and one ground wire.

- Connect the incoming power supply wires to the corresponding terminals on the distribution board. Follow the manufacturer's instructions for proper connection.

- Use wire connectors to secure the connections and ensure they are tight.

4. Install Circuit Breakers:

- Install circuit breakers (MCBs or MCCBs) into the distribution board for each circuit or load that requires protection. Choose breakers with appropriate current ratings for the circuits they will protect.

- Connect the outgoing cables from each circuit or load to the terminals of the corresponding circuit breaker.

- Secure the cables and tighten the connections to ensure they are secure.

5. Label Circuits:

- Label each circuit or breaker to identify its corresponding load or area of the facility. This helps with troubleshooting and maintenance in the future.

6. Test the Installation:

- Once all connections are made, turn on the main power supply.

- Use a voltage tester to verify that there is power at the distribution board and that each circuit breaker is functioning correctly.

- Test each circuit to ensure that power is being distributed properly to the intended loads.

7. Final Checks:

- Double-check all connections and ensure they are tight and secure.

- Verify that there are no exposed wires or potential safety hazards.

- Document the installation details for future reference and maintenance.

SAFETY PRECAUTIONS:

- Always turn off the power before working on electrical installations.

- Use proper safety gear and tools when handling electrical components.

- If you're unsure about any step of the installation process, consult a qualified electrician.

By following these steps carefully, you can effectively couple a three-phase distribution board and ensure proper distribution of power to various circuits or loads.

3.6 ROOM SELF-CON (SELF-CONTAINED) UNIT

To fully fix a room's self-con (self-contained) unit, which typically includes various electrical, plumbing, and finishing tasks, follow these steps:

PROCEDURE:

1. Electrical Installation:

- Install electrical outlets, switches, and lighting fixtures according to the room's layout and design plan. Ensure proper wiring and grounding for safety.

2. Plumbing Installation:

- Install plumbing fixtures such as sinks, toilets, showers, or bathtubs. Connect them to the main water supply and drainage system. Test for leaks and proper functionality.

3. Wall Construction:

- Construct or install wall partitions using drywall or other suitable materials. Ensure proper insulation and soundproofing if needed. Cut holes for electrical outlets and switches.

4. Finishing Walls:

- Apply paint or other wall finishes to the walls. Allow proper drying time between coats. Install any wall trims or moldings as desired.

5. Flooring Installation:

- Install flooring materials such as tiles, hardwood, laminate, or carpeting. Ensure proper leveling and sealing if necessary.

6. Cabinetry and Countertops:

- Install cabinets, shelves, and countertops in the room as needed. Ensure proper alignment and secure attachment to the walls or floor.

7. Door and Window Fixtures:

- Install door frames, doors, and window fixtures. Ensure proper alignment, smooth operation, and weatherproofing.

8. Final Touches:

- Install any remaining fixtures or accessories, such as towel bars, mirrors, or decorative lighting. Ensure everything is securely attached and properly functioning.

9. Cleaning and Inspection:

- Clean the room thoroughly, removing any construction debris or dust. Inspect all installations for any defects or issues that need to be addressed.

10. Final Testing:

- Test all electrical and plumbing fixtures to ensure they are working correctly. Check for any leaks, malfunctions, or safety hazards.

11. Final Inspection:

- Conduct a final inspection of the room to ensure it meets all building codes and regulations. Address any deficiencies or issues found during the inspection.

12. Handover:

- Once the room is fully fixed and inspected, hand it over to the owner or occupant. Provide any necessary instructions or maintenance tips for keeping the room in good condition.

SAFETY PRECAUTIONS:

- Follow all safety guidelines and regulations when working with electrical and plumbing systems.

- Use proper safety gear, such as gloves and safety glasses, when handling tools and materials.

- Ensure proper ventilation when using paints, adhesives, or other chemicals.

- If you're unsure about any aspect of the installation, consult a professional contractor or tradesperson

3.7 INSTALLATION OF ONE-WAY GANG SWITCH WITH A POINT OF LIGHT

To install a one-way gang switch with a point of light, which typically involves installing a single light switch to control a light fixture, follow these steps:

PROCEDURE:

1. Turn Off Power:

- Turn off the power supply to the area where you'll be working at the main circuit breaker or fuse box. Use a voltage tester to ensure the power is off before proceeding.

2. Prepare the Wiring:

- Plan the wiring route from the electrical panel to the location of the light fixture and switch.

- If necessary, run electrical cables from the electrical panel to the location of the light fixture and switch. Use appropriate cable clips to secure the cables along the route.

3. Install the Light Fixture:

- Mount the light fixture in the desired location according to the manufacturer's instructions. Ensure it is securely attached to the ceiling or wall.

4. Wire the Light Fixture:

- Connect the wires from the light fixture to the electrical cables using wire connectors. Typically, you'll connect the live wire (usually brown or red) to the live (L) terminal, the neutral wire (usually blue or black) to the neutral (N) terminal, and the earth wire (usually green and yellow) to the earth (E) terminal.

5. Install the Gang Switch:

- Mount the gang switch in the desired location on the wall using appropriate screws. Ensure it is level and securely attached.

6. Wire the Gang Switch:

- Connect the electrical cables to the gang switch. Typically, the

3.8 INSTALLATION OF GENERATOR WIRING

To install generator wiring, follow these general steps:

PROCEDURE:

1. Safety Precautions:

- Turn off the main power supply to your home or building.

- Make sure the generator is turned off and cool before starting any installation work.

2. Determine the Generator Location:

- Choose a suitable location for the generator, considering factors such as ventilation, accessibility for maintenance, and noise levels.

3. Install Transfer Switch:

- Mount the transfer switch near the main electrical panel. Follow the manufacturer's instructions for proper installation.

- Connect the transfer switch to the main electrical panel using appropriate electrical cables and wire connectors.

4. Connect Generator to Transfer Switch:

- Run electrical cables from the generator to the transfer switch. Use appropriate gauges based on the generator's output capacity and distance.

- Connect the generator cables to the designated terminals on the transfer switch, following the manufacturer's instructions.

5. Grounding:

- Ensure proper grounding for the generator and transfer switch according to local electrical codes. Connect the grounding wires to grounding rods or the building's grounding system.

6. Install Circuit Breakers (if needed):

- If the transfer switch does not have built-in circuit breakers, install them as needed to protect individual circuits. Connect them to the appropriate terminals on the transfer switch.

7. Testing:

- Test the wiring connections using a voltage tester to ensure they are properly connected and secure.

- Test the transfer switch operation by simulating a power outage and verifying that it switches power sources correctly.

8. Final Checks:

- Double-check all wiring connections, ensuring they are tight and properly insulated.

- Secure any loose cables and ensure they are routed away from moving parts or potential hazards.

- Label the transfer switch and circuit breakers to indicate their function and corresponding circuits.

9. Start Generator and Test:

- Start the generator and test each circuit to ensure

3.9 INSTALLATION OF GOTV

To install GOtv, a digital terrestrial television service, follow these steps:

PROCEDURE:

1. Choose Installation Location:

- Decide where to place your GOtv decoder. Ideally, it should be close to your TV and have access to a power source.

2. Connect the Antenna:

- If you're using an indoor antenna, simply connect it to the back of the decoder using a coaxial cable. For an outdoor antenna, you may need to install it outside and run the cable indoors.

3. Connect to the TV:

- Connect the decoder to your TV using either an HDMI cable (for HD picture quality) or AV cables (for standard definition). Plug one end of the cable into the corresponding ports on the decoder and the other end into the TV's input ports.

4. Power Up:

- Connect the power adapter to the decoder and plug it into a power outlet. Turn on both the decoder and the TV.

5. Tune Channels:

- Follow the on-screen prompts to complete the initial setup. This usually involves scanning for channels. GOtv will automatically detect available channels in your area.

6. Activate Subscription:

- If you haven't already done so, activate your GOtv subscription by paying

4.0 HOW TO CHANGE A BLACK CUTOUT TO A CERAMIC CUTOUT?

To change a black cutout (presumably a fuse) to a ceramic cutout, you'll need to follow these steps:

PROCEDURE:

1. Turn Off Power:

- Before beginning any electrical work, ensure the power to the circuit is turned off at the main circuit breaker or fuse box.

2. Locate the Black Cutout:

- Identify the location of the black cutout in the electrical circuit. This may be in a fuse box, distribution board, or electrical panel.

3. Remove the Old Black Cutout:

- Use a screwdriver to carefully remove the screws holding the black cutout in place. Take note of the wires connected to it.

4. Disconnect Wires:

- Carefully disconnect the wires from the terminals of the black cutout. Use wire strippers to strip the ends of the wires if necessary.

5. Install the Ceramic Cutout:

- Attach the wires to the terminals of the ceramic cutout. Make sure they are securely connected and tightened in place.

6. Secure the Ceramic Cutout:

- Position the ceramic cutout in place of the old black cutout and secure it using the screws. Ensure it is properly seated and aligned.

7. Test the Circuit:

- Before restoring power, use a voltage tester to double-check that there is no voltage present at the connections.

- Once confirmed, turn the power back on and test the circuit to ensure the ceramic cutout is functioning correctly.

8. Final Checks:

- Inspect the installation to ensure all connections are secure and there are no loose wires or exposed conductors.

- Use electrical tape to cover any exposed metal parts or connections for added safety.

9. Labeling (Optional):

- Consider labeling the ceramic cutout or the circuit it protects for easier identification in the future.

SAFETY PRECAUTIONS:

- Always turn off the power before working on electrical circuits to prevent electric shock or injury.

- Use insulated tools and wear appropriate safety gear, such as gloves and safety glasses.

- If you are unsure or uncomfortable working with electricity, it's best to consult a licensed electrician for assistance.

By following these steps carefully, you can safely replace a black cutout with a ceramic cutout in an electrical circuit.

CHAPTER FOUR

4.1 CHALLENGES FACED

- One of the challenges I encountered during my SIWES program was adapting to tight deadlines and managing multiple tasks simultaneously.

- Additionally, mastering advanced features of the electrical electronic engineering required extensive practice and guidance from experienced professionals.

4.2 CONCLUSION

The SIWES experience provided invaluable hands-on training in various aspects of electrical installation, repair, and maintenance. Through these tasks, I acquired practical skills, enhanced my understanding of electrical systems, and developed a strong commitment to safety and quality standards in the field of electrical engineering. I am grateful for the opportunity and look forward to applying the knowledge gained in my future endeavors.