A TECHNICAL REPORT

ON

STUDENT INDUSTRIAL WORK EXPERIENCE SCHEME (SIWES)

HELD AT

FEDERAL MINISTRY OF HOUSING AND URBAN DEVELOPMENT

FEDERAL SCREATARIAT FATE ILORIN

WRITTEN BY

ABDULRASAQ BASHIR

ND/23/CEC/PT/0139

SUBMITTED TO:

DEPARTMENT OF CIVIL ENGINEERING

INSTITUTE OF TECHNOLOGY (I.O.T) KWARA STATE POLYTECHNIC, ILORIN

INPARTIAL FUFILMENT OF THE REQUIREMENT FOR THE AWARD OF NATIONAL DIPLOMA (ND) CERTIFICATION IN CIVIL ENGINEERING TECHNOLOGY

AUGUST TO NOVEMBER, 2024

DEDICATION

This report is dedicated to Almighty God, the king of kings for his supreme and absolute control over the entire universe, for his protection, love and kindness. I also give my profound gratitude to Almighty God for sparing my life through this program.

ACKNOWLEDGMENT

I hereby express my profound gratitude to Almighty God for his absolute control over my life during this program. I also express my gratitude to my parents Mr and Mrs Abdulrasaq who have done all within their capacity to ensure me of a better and brighter future. Despite all challenge of life, they still strive to see me through this stage of my life. I pray Almighty God in his mercy will grant you good health and make sure they reap the fruit of their labour.

My profound appreciation to all those who have contributed to my success during this training program, my amiable friends won’t be left out. I hereby say a very big thank you to you all. May God reward you back abundantly.

PREFACE

The main body of this report contains details on how my industrial training attached was spent at Federal Ministry of Housing & Urban Development

TABLE OF CONTENT

Title page											i

Dedication											ii

Acknowledgement										iii

Preface											iv

Table of content										v

CHAPTER ONE

1.0	Introduction										1

1.1	Aims and objectives of SIWES							1

1.3	The organization chart								4

CHAPTER TWO

2.0	Section / units of the organization							5

2.1	specific function of the organization						5

2.2	Construction										6

2.3	Types of construction in tear of BUILDING TECHNOLOGY			6

2.4	Major building components							6

2.5	Site processes and operation							8

2.6	Site investigation									8

2.7	Site preparation									8

2.8	Preliminary work									9

2.9	Setting out										9

3.9.1  Setting out equipment                           					 	10

3.9.2   Method of setting out								11

3.9.3   Excavation										11

3.9.4    Method of excavation						  		11

CHAPTER THREE

3.0	Student specific involvement at various sections					13

3.1	Foundation 										15

CHAPTER FOUR

4.0	Relevance of experience gain at the field of study				14

4.1 	Concrete work									14

4.2     Drainage										15

4.3	 Important of drainage								15

CHAPTER FIVE

5.0	Conclusion										17

5.1	Suggestion and recommendation to the organization and polytechnic concerning the SIWES program							17

CHAPTER ONE

- INTRODUCTION

Student industrial work experience scheme (SIWES) programmed, was establish in year 1973 by federal Government of Nigeria through the industrial Training Fund (ITF) under the NBTE (National Board for Technical Examination). It has it’s headquarter in jos, plateau state. The major reason behind the establishment of SIWES programmed and the importance of the student to display their talents and also learn the practical aspects of their fields of study. After their graduation, they will have something valuable to contribute to the society. They will also be able to face future challenges in their respective field of study.

In accomplishment of my compulsory four months training program, I did my SIWES at Ace bricks Construction

I was involved in several operation on site and experience in relation to my career of study (Civil Engineering).

  - AIM AND OBJECTIVES OF SIWES

- To enhance the knowledge of students (theory) of what they have been

taught in school by backing it with enough practicals.

- To provide students the opportunity to apply their theoretical knowledge

in real work situation, bridging the gap between university work and actual site work.

- To provide an avenue for students to acquire industrial skill and

experience in their course study.

- To enable students to develop more affection for their chosen profession.

- To expose students to working method and technique in handling

equipment and machineries that may not be available to them in school.

1.2	ORGANIZATION CHART

CONTROLLER

Engr.

Admin           Acct.                 Land         Physical         Public building

.

Civil Engineering

Archi

Q/S

BUILDING

BUILDING

STRUCTURERE

CHAPTER TWO

2.0	We were taught about Scaffolding

It’s a temporary structure used to aid in the construction maintenance and repair of blinding bridges and other human made structure

Two Major of scaffolding

We were taught about dependent scaffolding is a type of scaffolding that relies on the building or structure it is against for support

Independent Scaffolding is a type of scaffolding that can freely stands on its own without relying on the building

Types of Scaffolding

Suspended Scaffold it is used for task like window clearing, painting and repairs

Supported Scaffolding used for extension works such as building maintenance repairs and construction

2.1	SECTION / UNIT OF THE ORGANIZATION

FEDERAL MINISTRY OF HOUSING was divided into four section / unit they are as follows.

- Planning Approval Department

- Construction Management Department

- Building  / Structural Department

- Administration Section

2.2	SPECIFIC FUNCTION OF THE ORGANIZATION

- PLANNING   APPROVAL   DEPARTMENT

This unit deals with planning approval of a proposal to use or develop land against the rules in planning scheme. The planning process is mainly concerned with the impacts of a proposed use or development upon neighboring land.

- CONSTRUCTION    DEPARTMENT

This department provides construction management services on a project such as building, highway and other BUILDING TECHNOLOGY projects.

- BUILDING TECHNOLOGY   DEPARTMENT

The department is made up of Civil & Structural engineering with wide range of experience in design detailing and supervision of structure.

- ADMINISTRATION SECTION

This section ensure the smooth running of the organization .The section make sure that the administration setup by the organization is in good order.

2.3	CONSTRUCTION

Construction is defined as the act of process of building something, but in BUILDING TECHNOLOGY construction is the process of designing of structure (ie construction of building) for dam, railway, dam water way and bridge.

2.4	TYPE OF CONSTRUCTION IN TEAR OF BUILDING TECHNOLOGY

- Building construction / Structural

- High way /road construction

- Construction of dam

Building construction: this aspect of construction is involved In building and erecting of structure examples are  office, house, churches, mosques etc

High /road construction: this is the process of constructing bridges, road, highways or idealization of road.

Dam construction: this kind of construction is based on the construction of dam.

2.5	MAJOR BUILDING COMPONENTS

A building has two basics parts

- Substructure or foundations, and

- Super structure

SUB- STRUCTURE OR FOUNDATION : is the lower potion of the building, usually located below the ground laurel, which transmits the pads of the superstructure to the supporting soils. A foundation is the therefore that of the structure which is in direct contact with the ground to which the loads are transmitted .

SUPER – STRUCTURE : Is  the part of the structure which is above the ground level and which serves the purpose of its intended use. A part of the super-structure, located between the purpose of its intended use. A part of the super-structure, located between the ground level and the floor hovel is known as plinth. plinth is therefore defined as the portion of the structure between the surface of the floor, immediately above the ground. The level of the floor is usually known as the plinth level. The component of a building can be broadly summarized as follows

- Foundation

- Columns

- Floors

- Walls

- Doors

- Windows and ventilation

- Roots

- Stairs

SITE INVESTIGATION

This is a process of ascending the suitability of a particular chosen site for building construction purpose.

The type and nature of sub soil, geological condition and behavior, the depth of the water table the load bearing capacity, the difficulties that may arise construction and the change or occurrence that may arise after construction.

SITE PREPARATION

Site preparation includes everything you need to do to get the ready to build on. that include clearing of tree, rock , underbrush, anything that may be in the way of your foundation .this must be removed from site clearance in an initial major work to be carried out. this extend of work require labour plant and equipment depend on factor line size of site nature constituent to be removed from site, ground condition of the site distance debris to be remove from site  in some cases or instant site clearance could involves large or small scale demonation of existing structures.

2.8	PRELIMINARY WORK

This involve all the activities before the building can be setting out for construction his include site investigation and consideration of site suitability.

2.9	SETTING OUT

The method of setting out is the reverse of surveying process. the process includes the position and level of the building lines and road alignment shown on the construction plans to be established on the grounds by various techniques and instrument. it is necessary to have a good knowledge of geometry in order to ensure accurate work. the first task in setting out in building is to established base line from which the whole of the building can be set out. The  position of this line must be mark on site so that it can be re-established at anytime. the building line is frequently deterring by the highway authority and from the back of the public foot path. If other building have been erected at the area the building line can be determine from this existing building.

    - EXCAVATION

Excavation is done to received the foundation that will be constructed for a building. This is done after the setting out and marking out.

    - METHOD OF EXCAVATION

- Manual method

- Mechanical method

The choice of method of excavation is depend on the size work in small construction, it is more economic to used the manual method, whole large works will be economic to be executed using the mechanical method of excavation but the one that was fully involve is manual method, which labourer were digging the ground to specific depth and the width which was specified by engineer on site.

CONSTRUCTION

Construction is defined as the act of process of building something, but in BUILDING TECHNOLOGY construction is the process of making or creating or creation a structure (ie construction of building) for movement purpose.

  - TYPE OF CONSTRUCTION IN TEAR OF BUILDING TECHNOLOGY

- Building construction

- High way /road construction

- Construction of dam

Building construction: this aspect of construction is involved In building and erecting of structure examples are  office, house, churches, mosques etc

High /road construction: this is the process of constructing bridges, road, highways or idealization of road.

Dam construction: this kind of construction is based on the construction of dam.

What is PPE

PPE which is personal protective equipment: it refers to the equipment that protect workers from workplace hazards. The occupational health and safety administration applies of special standard pipe used in construction

Importance of PPE

Eye safety e.g Glass

Boot protection from sharp object

Helmet in protection the skull

Type of PPE protection

Head protection

Hearing protection

Foot Protection

CHAPTER THREE

STUDENT SPECIFIC INVOLVEMENT AT VARIOUS SECTIONS

1.	Civil Engineering Design Unit

•	I took part in drawing of General Arrangement of a building such as

columns, Beams and Slab.

•	I also took part in Auto card.

2.	Civil Engineering Construction Unit

•	I took part in the supervision of construction

•	I also took part in the supervision of mass concrete

I was introduce to supervisor and staff of Kwara State Ministry of Works and Transport and I was told the aims and the objective of the organization including capital project the one in progress and other pending were made know to us the engineering in charge of SIWES in the organization in person of Engr. SABA PHILIPS and tell us about the consultancy services to the construction company.

In the design unit I was introduced to draw a plan and produce the general arrangement, and I was told to design the slab to the general arrangement.

In the construction unit I was attached with supervisory unit to supervise the construction at OSI CAMPUS were we supervise the construction of conferences hall and also at along SANGO road which we supervise the construction of Road.

FOUNDATION

Foundation is the lowest part of a structure which provides a base for super structure proper.

The foundation is the part of the construction where the base of the building meets the ground. Foundation is usually place below ground level because it provides:

- Stability

- Protection against impact

- Carries the load of the building and safely transmit it to soils.

- Protection from the extreme’s of weather such as excessive rain or drought. Although , the depth will vary according to the condition on site. The best load bearing ground is normally 900mm below the surface.

4.1  TYPES OF FOUNDATION

- Strip foundation

- Raft foundation

- Stepped foundation

- Piled foundation

STRIP FOUNDATION

This type of foundation is use most frequently. They consist of continuous mass concrete strips poured in the bottom of trenches. This foundation will support load bearing walls which are centered on the concrete strips to spread pressure from the walls, roof and the floor loads evenly. This strip is usually a uniform width and depth. The foundation must be wide and deep enough to avoid soil movement that could cause instability. Depending on soil condition, the maximum depth may be 900mm. building regulation may suggest the suitable minimum width for strip foundation. The concrete must be at least as thick as its projection from the base of the wall. This ensures that the pressure of the building loads is distributed in the concrete at angle of 45 degree.

RAFT FOUNDATION

This covers the whole area of the building and usually extended beyond it.

Its consist primarily of a reinforced concrete slab up to 300mm thick which is often

thickened under load bearing walls. The level of the base of the raft is usually

within 300mm of two layers of fabric reinforcement one being near the top and the other near bottom of the slab. Raft foundation is best suited for use on soft natural ground.

STEPPED FOUNDATION

They are normally used on sloppy sites so as to reduce the expenses of cores

Strutting a foundation that is the same level around the perimeter of the building. A

Foundation at the higher end of the slope would be extremely deep. So to overcome

This, we can step the foundation at different levels. The higher  level  foundation

should extend over and meet with lower one for a distance not less than its

thinckness. These should never be less than 300mm.

PILED FOUNDATION

The word pile is used to describe columns usually of reinforced concrete,

driven  or cast into the ground in order to carry foundation loads to some deep

underlying firm stratum or to transmit load to the soil by friction of their surfaces in contact with the soil.

CASTING OF FLOOR

Casting of floor is divided into two namely

•	Mass concrete

Mass concrete is the process of using 1:3:6 (one head pan of cement, three head pan of sharp sand and six head pan of granite).By putting the expansion joint for the concrete to expand when load come on it

•	Mass reinforcement concrete.

Mass reinforcement concrete is the process by using 1:2:4 (one head pan of cement ,two head pan of  sharp sand and four head pan of granite) with reinforcement ,by applying expansion joint.

2.8.2	PROCESS INVOLVED IN FLOOR CASTING

•	CLEARING: clearing is the removal of unwanted material from the site e.g. planks, cement bags etc.

•	EXCAVATION: Is a process in which soil, rock, and other materials are removed from the site. OR is the process of cutting or loosing and removing earn including rock from its original position transporting and dumping it as a fill or spoil bank.

•	SPREADING OF LATERITE: This is the process by which a soil called LATERITE is spread to the excavated area in order to increase soil strength and to get the uniform level of the land.

•	WETTING: Is the spraying of water on the base of the floor .wetting is done after the laterite is been spread, the reason for the wetting is to prevent dust and to make the area firm and durable.

•	COMPACTING: Compacting is done immediately after wetting of laterite so as to increase the soil strength for adequate smoothening.

•	LAYING OF CONCRETE: Concrete is the mixture of granite, sharp sand and cement with a certain volume of water, the mixing ratio used is 1:2:4.i.e  One head pan of cement

Two head pan of sharp sand

Four head pan of granite.

The machine used for mixing is Concrete mixer.

INSTRUMENT USED IN LAYING OF CONCRETE

	SHOVEL

HAND TROWEL

	HEAD PAN

								SPIRIT LEVEL

•	CURING: Curing is done after the laying the concrete by applying water to increase the strength. Before curing we spread sharp sand on the concrete to reduce the evaporation and to allow the water to penetrate.

CHAPTER FOUR

4.0	RELEVANCE OF EXPERIENCE GAIN AT THE FIELD OF STUDY

In Civil Engineering , casting means the mixture of granite / gravel, sharp sand and cement with a certain volume of water.

4.1  CONCRETE WORK

Concrete works can be defined as the mixture of fine aggregate, course aggregate, cement and water and when mixed at its fluid state it forms paste and consistency that can be mounded to any shapes and sizes and on drying it forms a very hard and soil materials inform of rocks. There are two major classification of  concrete (a)pre-cast concrete (b)cast-in-situ.

CONCRETE WORK

Concrete works can be defined as the mixture of fine aggregate, course aggregate, cement and water and when mixed at its fluid state it forms paste and consistency that can

be mounded to any shapes and sizes and on drying it forms a very hard and soil materials inform of rocks. There are two major classification of  concrete (a)pre-cast concrete (b)cast-in-situ.

2.7 THE DESIGN OF CONCRETE

The objective of mix design is to procedure as economically as possible.

- Systematic analysis of materials properties and knowledge of how they affect concrete properties.

- Batch quantities for a given volume of concrete such that the properties of both fresh and hardened material are required for the specific a purpose.

Water/cement ratio=           	Mass of water in a concrete sample

Mass of the concrete

Aggregate/cement ratio=   	Mass of water in a concrete sample

Mass of the concrete sample

2.7.1	PROPERTIES OF FRESH CONCRETE

Workability: This is the most importance property of fresh concrete which is define as the property of concrete which determines its ability be placed compacted and finished of these three operations, the greatest to be emphasized in compaction (elimination of air voids) workability of concrete can be measured by slump test compacting factor test and vibe.

2.7.2	 CONCRETE MIXING

The proportion in which the concrete, time aggregate and coarse aggregate are based on unit volume of cement ratio in civil engineering instruction mixing ratio is 1:2:4 which is one head pan cement, two head pan of coarse aggregate sine it is important that the quality of cement should be batched by weight.

2.8	RELEVANCE OF EXPERIENCE GAIN AT THE FIELD OF STUDY

In civil engineering, casting means the mixture of granite / gravel, sharp sand and cement with a certain volume of water.

Concrete works done on site are follows:-

- BEAM: these  are horizontal members which support and distribute the

weight  of upper  floor and roofs (usually of timber, steel, or reinforced concrete).

- LINTELS:- These are small beams placed over door and window openings and  Support the load of the wall over these opening.

- COLUMNS:-They are bull with stool or reinforced concrete. They are

usually  Isolated vertical members which carry the weight of beams, floors and roofs in framed buildings. The walls of framed building are non-load bearing.

Concrete works done on site are follows:-

- BEAM: these  are horizontal members which support and distribute the

weight  of upper  floor and roofs (usually of timber, steel, or reinforced concrete).

- LINTELS:- These are small beams placed over door and window openings

and  Support the load of the wall over these opening.

- COLUMNS:-They are bull with stool or reinforced concrete. They are

usually  Isolated vertical members which carry the weight of beams, floors and roofs in framed buildings. The walls of framed building are non-load bearing.

4.2	DRAINAGE

Drainage system usually provided in the design of high way for the protection at the investment make in high way structure and for safety of the users in rural areas. Some portion of high way that provide for the surface drainage.

4.3	 IMPORTANT OF DRAINAGE

Drainage and culvert are important part of highway and the reason why it cannot just be left out of the designing stage of highway are follow:

- Water retained or logged on highway surface could cause fatal accident for moving vehicle especially those on high speed.

- Where drainage facilities are not provided, the sides of the roadway are normally subjected to erosion.

- See page of water it pavement and sub-grade will cause a structure failure of the road.

- The absence of the destruction of pipe culvert course hindrance to traffic flows.

In view of the above reasons, various types of drainage and culverts facilities   are used to protect the highway against surface and sub-grade water. This is designed to covey water under, across along or away from the highway in the most economical, efficient and practical manner without damaging the highway or adjacent property.

INTERLOCKING

Is a type of clay bricks designed to constructively lock together without the need for the bricks fit together like pazzle process to form a structure

Importance of Interlocking

Support

Safer zone

Maintainance

Less labour

Types of Interlocking

Brick Interlocking

Rubber Interlocking

Concretal Interlocking

CHAPTER FIVE

5.0    CONCLUSION

I was highly impressed on the ways the company carried out their activities.

They work with me time to time and skill acquisition is of great import. I learnt a lot from the company.

5.1    SUGGESTION        AND      RECOMMEDATION     TO   THE ORGANIZATION  AND     POLYTECHNIC     CONCERNING      THE     SIWES     PROGRAM

SIWES coordinator and the polytechnic authority should try to stop the habit of rejecting student for SIWES program by the industry. The institution supervisor should make it a priority to visit their designated student in the various organizations to update the student’s log book.

Finally, they should resist the habit of staying in school and wait for their  wards to finish the SIWES program only for them to sign their training log book  and award them grade of the choices without checking the performance of the

student in the field.