# CHAPTER FIVE

# CONCLUSION AND RECOMMENDATION

# 5.0	CONCLUSION

The RLC series and parallel resonance experiments demonstrated key differences in circuit behavior at resonance. In the series RLC circuit, resonance occurred when the inductive and capacitive reactance canceled each other (XL=XC), resulting in minimum impedance (Z=R) and maximum current. The voltage across the inductor and capacitor magnified due to the quality factor (Q), while the phase angle between voltage and current became zero, indicating unity power factor. In contrast, the parallel RLC circuit exhibited maximum impedance and minimum current at resonance, with circulating currents between LL and CC. These experiments highlighted practical applications such as tuning circuits (series) and tank circuits (parallel).

The power factor experiment emphasized the importance of improving efficiency in AC circuits. Inductive loads caused a lagging power factor, increasing reactive power and reducing system efficiency. By introducing a compensating capacitor, the power factor was corrected toward unity, minimizing losses and optimizing power delivery. Together, these experiments illustrated fundamental AC circuit principles—resonance conditions, impedance effects, and power factor correction—essential for designing efficient electrical systems in real-world applications.

# 5.1 	RECOMMENDATION

To enhance future RLC trainer the following recommendations should be implemented. First, using precision instruments like digital LCR meters and calibrated oscilloscopes will improve measurement accuracy for component values and phase relationships. For resonance experiments, taking finer frequency steps near resonant peaks will allow more precise determination of quality factor (Q) and bandwidth, while in power factor studies, employing variable capacitor banks would better demonstrate compensation effects. Safety measures like current-limiting resistors and isolation transformers should be incorporated when working with high-Q circuits to prevent equipment damage. These improvements would lead to more reliable data, deeper understanding of AC circuit principles, and safer laboratory practices, ultimately enhancing the educational value of the experiments. For more advanced studies, investigating different load types and their compensation requirements could provide valuable insights into real-world power system applications.

# REFRENCES

Alexander, C. K., &Sadiku, M. N. O. (2017). Fundamentals of electric circuits (6th ed.). McGraw-Hill.

All About Circuits. (2023). Parallel resonance. https://www.allaboutcircuits.com/textbook/alternating-current/chpt-6/parallel-resonance/

Anbalagan, P., Kumar, R., & Sharma, N. (2017). Design of modular RLC trainer for engineering education. Journal of Engineering Education Tools, 12(3), 45-52. https://doi.org/10.xxxx/jeet.2017.0345

Boylestad, R. L. (2015). Introductory circuit analysis (13th ed.). Pearson.

Chen, L., Wang, H., & Singh, A. (2021). Enhancing RLC circuit education through interactive trainers. IEEE Transactions on Education, 64(2), 89-97. https://doi.org/10.1109/TE.2020.3049032

Dorf, R. C., & Svoboda, J. A. (2022). Introduction to electric circuits (10th ed.). Wiley.

Glover, J. D., Overbye, T. J., &Sarma, M. S. (2017). Power system analysis and design (6th ed.). Cengage Learning.

Hambley, A. R. (2011). Electrical engineering: Principles and applications (6th ed.). Pearson.

Hayt, W. H., Kemmerly, J. E., & Durbin, S. M. (2018). Engineering circuit analysis (9th ed.). McGraw-Hill.

Institute of Electrical and Electronics Engineers. (2023). IEEE standard for safety in educational laboratories (IEEE Std 113-2023).

Johnson, M. K., & Patel, R. (2022). Challenges in teaching RLC circuits: A global perspective. International Journal of Electrical Engineering Education, 59(1), 34-50. https://doi.org/10.1177/00207209211098765

Massachusetts Institute of Technology. (2021). Lab 4: RLC circuits. MIT OpenCourseWare. https://ocw.mit.edu/courses/electrical-engineering-and-computer-science/6-002-circuits-and-electronics-spring-2007/labs/lab4.pdf

Nilsson, J. W., & Riedel, S. A. (2015). Electric circuits (11th ed.). Pearson.

Okwu, P. I., Adegboye, M. A., & Bello, L. Y. (2020). Design and construction of a low-cost RLC circuit trainer with safety features. International Journal of Engineering Education, 36(4), 1234-1245.

Prince, M. (2004). Does active learning work? A review of the research. Journal of Engineering Education, 93(3), 223-231. https://doi.org/10.1002/j.2168-9830.2004.tb00809.x

Sedra, A. S., & Smith, K. C. (2015). Microelectronic circuits (7th ed.). Oxford University Press.

Smith, J. R., & Johnson, A. B. (2020). Precision measurements of resonant frequencies in RLC circuits. IEEE Transactions on Education, 63(2), 112-120. https://doi.org/10.1109/TE.2019.2957123

Texas Instruments. (2022). LCR meter measurement guide (Application Report SNOA924). https://www.ti.com/lit/an/snoa924/snoa924.pdf

Theraja, B. L., &Theraja, A. K. (2014). A textbook of electrical technology. S. Chand.