# CHAPTER FOUR

### 4.1 RESULTS AND DISCUSSIONS

This chapter presents the results and discussions of the traffic congestion analysis at Oja-Oba Roundabout intersection in Ilorin, Kwara State. Geometry measurements for Oja-Oba Roundabout intersection was obtained on the site using a measuring tape. The measured geometric elements of the intersection is shown on Table 4.1.

### Table 4.1: Geometry Features of Oja-Oba Roundabout intersection

| Approach | Approach Width(m) |
| --- | --- |
| Approach 1 (Abdul-Azeez Attah Road) | 11.2 each |
| Approach 2 (Old Yidi Road) | 11.2 |
| Approach 3 (Emir Palace Road) | 11.2 |
| Approach 4 (Pakata Road) | 8.2 |

# 4.3 Traffic Data Analysis

The traffic data collected from the intersection was analyzed in this section in order to streamline data related to traffic flow, traffic volume and traffic patterns. The goal was to understand traffic behaviour and identify trends and patterns which would help me to draw impactful decisions for my research.

## 4.3.1 Traffic Volume Data

The traffic volume data collected for each approach to the intersection includes peak hour traffic conditions for Oja-Oba Roundabout. The peak hours observed during seven days occurred between 8:00-9:00 am and 5:00-6:00 pm respectively. Video coverage method was used.

### Table 4.1: Geometric Features of the Road Intersection; Approach 1

| Road Name | Road Features | Units | Measurement (m) |
| --- | --- | --- | --- |
| AbdulAzeez Attah Road | Lane | 4 | 3.5 |
|  | Shoulder | 2 | 2.5 |
|  | Median | 1 | 2 |
|  | Carriage width | 2 | 11.2 |
| Total width of the road |  |  | 21 |

### Table 4.2: Geometric Features of the Road Intersection; Approach 2

| Road Name | Road Features | Units | Measurement (m) |
| --- | --- | --- | --- |
| Old Yidi Road | Lane | 2 | 3.5 |
|  | Shoulder | 2 | 2.5 |
|  | Median | 0 | 0 |
|  | Carriage width | 1 | 11.2 |
| Total width of the road |  |  | 12 |

### Table 4.3: Geometric Features of the Road Intersection; Approach 3

| Road Name | Road Features | Units | Measurement (m) |
| --- | --- | --- | --- |
| Emir Palace Road | Lane | 2 | 3.5 |
|  | Shoulder | 2 | 2.5 |
|  | Median | 0 | 0 |
|  | Carriage width | 1 | 11.2 |
| Total width of the road |  |  | 12 |

### Table 4.4: Geometric Features of the Road Intersection; Approach 4

| Road Name | Road Features | Units | Measurement (m) |
| --- | --- | --- | --- |
| Pakata Road | Lane | 2 | 3.5 |
|  | Shoulder | 2 | 2.5 |
|  | Median | 0 | 0 |
|  | Carriage width | 1 | 8.2 |
| Total width of the road |  |  | 12 |

4.2 ACCUMULATED DATA FOR ANALYSIS

Check Appendix II for all traffic flow data.

### 4.3  	VOLUME OF THE ROAD TRAFFIC INTERSECTION

Tables below show the average traffic volumes in vehicles per hour at the studied intersection for both morning and evening peaks.

Table 4.3: Total volume vehicles at Approach 1

| Days | Morning veh/hr | Evening veh/hr | Total |
| --- | --- | --- | --- |
| Day 1 (Tuesday) | 610 | 728 | 1338 |
| Day 2 (Wednesday) | 790 | 682 | 1472 |
| Day 3 (Thursday) | 400 | 682 | 1082 |
| Day 4 (Friday) | 766 | 575 | 1341 |
| Day 5 (Saturday) | 510 | 644 | 1154 |
| Day 6 (Sunday) | 328 | 304 | 632 |
| Day 7 (Monday) | 952 | 800 | 1752 |
| Total | 4356 | 4415 |  |
| Average total volume | 622.3 | 630.7 |  |

Average total volume per hour = (622.3 + 630.7)/2

= 626.5 vph

0

100

200

300

400

500

600

700

800

900

1000

Day 1

Day 2

Day 3

Day 4

Day 5

Day 6

Day 7

TRAFFIC VOLUME FOR APPROACH 1

Morning

Evening

Monday has the highest total traffic volume with 1752 veh/hr, indicating it’s the busiest day. Sunday has the lowest total traffic volume with 632 veh/hr, indicating it's the least busy day. The total evening volume (4415 veh/hr) slightly exceeds the total morning volume (4356 veh/hr), indicating a higher traffic volume in the evening on average.

Table 4.4: Total volume vehicles at Approach 2

| Days | Morning veh/hr | Evening veh/hr | Total |
| --- | --- | --- | --- |
| Day 1 (Tuesday) | 92 | 448 | 540 |
| Day 2 (Wednesday) | 131 | 210 | 341 |
| Day 3 (Thursday) | 91 | 610 | 701 |
| Day 4 (Friday) | 478 | 245 | 723 |
| Day 5 (Saturday) | 240 | 407 | 647 |
| Day 6 (Sunday) | 128 | 114 | 242 |
| Day 7 (Monday) | 876 | 529 | 1405 |
| Total | 2036 | 2563 |  |
| Average total volume | 290.9 | 366.1 |  |

Average total volume per hour = (290.9 + 366.1)/2

= 328.5 vph

0

100

200

300

400

500

600

700

800

900

1000

Day 1

Day 2

Day 3

Day 4

Day 5

Day 6

Day 7

TRAFFIC VOLUME FOR APPROACH 2

Morning

Evening

Monday has the highest total traffic volume with 1405 veh/hr, indicating it’s the busiest day. Sunday has the lowest total traffic volume with 242 veh/hr, indicating it's the least busy day. Morning peak: Monday with 876 veh/hr. Evening peak: Thursday with 610 veh/hr. The total evening volume (2563 veh/hr) exceeds the total morning volume (2036 veh/hr), indicating a higher traffic volume in the evening on average.

Table 4.5: Total volume vehicles at Approach 3

| Days | Morning veh/hr | Evening veh/hr | Total |
| --- | --- | --- | --- |
| Day 1 (Tuesday) | 506 | 371 | 877 |
| Day 2 (Wednesday) | 632 | 496 | 1128 |
| Day 3 (Thursday) | 432 | 556 | 988 |
| Day 4 (Friday) | 642 | 604 | 1246 |
| Day 5 (Saturday) | 357 | 555 | 912 |
| Day 6 (Sunday) | 382 | 153 | 535 |
| Day 7 (Monday) | 771 | 659 | 1430 |
| Total | 3722 | 3394 |  |
| Average total volume | 531.7 | 484.9 |  |

Average total volume per hour = (531.7 + 484.9)/2

= 508.3 vph

0

100

200

300

400

500

600

700

800

900

Day 1

Day 2

Day 3

Day 4

Day 5

Day 6

Day 7

TRAFFIC VOLUME FOR APPROACH 3

Morning

Evening

Monday has the highest total traffic volume with 1430 veh/hr, indicating it’s the busiest day. Sunday has the lowest total traffic volume with 535 veh/hr, indicating it's the least busy day. Morning peak: Monday with 771 veh/hr. Evening peak: Friday with 604 veh/hr. The total morning volume (3722 veh/hr) exceeds the total evening volume (3394 veh/hr), indicating a higher traffic volume in the morning on average.

Table 4.6: Total volume vehicles at Approach 4

| Days | Morning veh/hr | Evening veh/hr | Total |
| --- | --- | --- | --- |
| Day 1 (Tuesday) | 601 | 357 | 958 |
| Day 2 (Wednesday) | 669 | 537 | 1206 |
| Day 3 (Thursday) | 163 | 569 | 732 |
| Day 4 (Friday) | 466 | 619 | 1085 |
| Day 5 (Saturday) | 385 | 583 | 968 |
| Day 6 (Sunday) | 329 | 97 | 426 |
| Day 7 (Monday) | 832 | 600 | 1432 |
| Total | 3445 | 3362 |  |
| Average total volume | 492.1 | 480.3 |  |

Average total volume per hour = (492.1 + 480.3)/2

= 486.2 vph

0

100

200

300

400

500

600

700

800

900

Day 1

Day 2

Day 3

Day 4

Day 5

Day 6

Day 7

TRAFFIC VOLUME FOR APPROACH 4

Morning

Evening

Monday has the highest total traffic volume with 1432 veh/hr, indicating it’s the busiest day. Sunday has the lowest total traffic volume with 426 veh/hr, indicating it's the least busy day. Morning peak: Monday with 832 veh/hr. Evening peak: Friday with 619 veh/hr. The total morning volume (3445 veh/hr) is slightly higher than the total evening volume (3362 veh/hr), indicating a higher traffic volume in the morning on average.

Table 4.7: Total volume of the road intersection.

| Days | Morning veh/hr | Evening veh/hr | Total |
| --- | --- | --- | --- |
| Day 1 (Tuesday) | 1809 | 1904 | 3713 |
| Day 2 (Wednesday) | 2222 | 1925 | 4147 |
| Day 3 (Thursday) | 1086 | 2417 | 3503 |
| Day 4 (Friday) | 2350 | 2042 | 4392 |
| Day 5 (Saturday) | 1462 | 2189 | 3651 |
| Day 6 (Sunday) | 1167 | 668 | 1835 |
| Day 7 (Monday) | 3431 | 2423 | 5854 |
| Total | 13527 | 13568 |  |
| Average total volume | 1932.4 | 1938.3 |  |

Average total volume per hour = ( 1932.4+1938.3 )/2

= 1935.4 vph

### Chart 4.1: Distribution of Traffic Volume

0

1000

2000

3000

4000

5000

6000

7000

Tuesday

Wednesday

Thursday

Friday

Saturday

Sunday

Monday

TOTAL VPH

DAYS

Total

Table 4.3 and chart 4.1 above shows that for both morning and evening peaks,

Monday recorded the highest volume of traffic among the studied roads. These are 3431 vph and 2423 vph of total volume respectively. This is followed by Friday with 2350 vph and 2042 vph for morning and evening peaks respectively. Wednesday ranks third recording 2222 vph and 1925 vph for morning and evening peaks respectively. Saturday ranks fourth recording 1462 vph and 2189 vph for morning and evening respectively. Tuesday ranks fifth with 1809 vph and 1904 vph for morning and evening peaks respectively. Thursday ranks sixth with 1086 vph and 2417 vph for morning and evening peaks respectively. While Sunday ranks seventh with the least volume of 1167 vph and 668 vph for morning and evening peaks respectively.

0

500

1000

1500

2000

2500

3000

3500

4000

Tuesday

Wednesday

Thursday

Friday

Saturday

Sunday

Monday

ROAD VOLUME

Morning

Evening

The chart above shows the traffic pattern for seven days and it can be deduced that the traffic volumes are relatively high on weekdays, especially Monday mornings and Thursday evenings, reflecting typical workday patterns. While Friday shows a high volume as people likely head out for Jum’at services and weekend activities.

### 4.4 DELAYS AT THE SELECTED INTERSECTION

Using the peak volume per hour data which is on day 7, Morning,  with the value of 3431 vehicles per hour,

| The volume of the intersection at 30 minutes interval | Time of delay (sec) |
| --- | --- |
| 8:00 - 8:35 | 1200 |
| 8:30 - 9:00 | 720 |

Delay for peak volume per hour = average of the time of delay

= (1200+720)/2 = 960 seconds

### Table 4.5: LOS criteria for Unsignalized Intersection

| LOS | AVERAGE CONTROL DELAY SEC/VEH |
| --- | --- |
| A | 0-10 |
| B | >10-15 |
| C | >15-25 |
| D | >25-35 |
| E | >35-50 |
| F | >50 |

Based on delay the Highway Capacity Manual (HCM) provides thresholds for LOS based on the average control delay per vehicle therefore, the delay for the Oja-Oba Roundabout falls under LOS F: Greater than 50 seconds which implies that there is severe congestion at the intersection which results in these long delays.