CHAPTER TWO: LITERATURE REVIEW

2.1.0 Introduction to Cocoa Drying

Cocoa drying is a critical step in the post-harvest processing of cocoa beans, aimed at reducing moisture content to a level suitable for safe storage and further processing. The primary goal of drying is to prevent the growth of mold and yeast, which can spoil the beans and lead to the production of mycotoxins. Traditionally, cocoa beans are dried using sun drying or open-air drying methods, which are highly dependent on weather conditions and often result in inconsistent drying, contamination, and quality issues (Afoakwa, 2014).Cocoa beans are critical agricultural products in many tropical countries, contributing significantly to the economy. Drying is a vital step in cocoa processing, which reduces the moisture content of the beans to about 7.5%, preventing the growth of microorganisms and ensuring the beans are safe for storage and processing (Schwan & Wheals, 2004). Proper drying improves the flavor profile and quality of cocoa beans, making them suitable for chocolate production and other uses (Afoakwa, 2014).

2.1.1 Challenges of Traditional Cocoa Drying Methods

Traditional sun drying has several drawbacks, including prolonged drying time, exposure to rain and pests, and the potential for uneven drying, which can lead to fungal contamination and inferior bean quality. Additionally, these methods are labor-intensive and require large drying areas, making them less efficient (Fagunwa et al., 2020).

Traditional sun drying methods involve spreading cocoa beans on large surfaces exposed to the sun. These methods are highly weather-dependent and expose beans to contamination from dust, insects, and animals. Furthermore, traditional methods are labor-intensive and time-consuming, requiring constant monitoring to ensure uniform drying (Fagunwa et al., 2020). Variability in drying conditions can lead to inconsistent bean quality and increase the risk of fungal growth, which can result in mycotoxin contamination (Barimah et al., 2021).

2.2.0 Overview of Solar Drying Technology

Solar drying offers an environmentally friendly and cost-effective alternative to traditional drying methods. Solar dryers utilize solar energy to heat air and facilitate the drying process. There are three main types of solar dryers: direct solar dryers, where the product is directly exposed to solar radiation; indirect solar dryers, where solar energy is used to heat the air that then passes over the product; and mixed-mode solar dryers, which combine both direct and indirect methods (S. Janjai, N. Lamlert, and B.K. Bala, 2021).

Solar drying is an effective alternative to traditional drying methods. Solar dryers use solar energy, which is abundant and renewable, to heat air and facilitate the drying process. The primary types of solar dryers are:

- Direct Solar Dryers: Beans are exposed directly to solar radiation within a covered environment, reducing the risk of contamination and speeding up the drying process (Sharma et al., 2009).

- Indirect Solar Dryers: Solar energy heats air that is then circulated over the beans in a separate drying chamber. This method protects beans from direct sunlight, which can lead to overheating (Koyuncu, 2006).

- Mixed-Mode Solar Dryers: This type combines both direct and indirect methods, utilizing solar energy to heat both the air and the drying chamber, optimizing drying efficiency (Afzal et al. (2023)

- 2.2.1 Advantages and Disadvantages of Solar Drying

- Advantages:

Solar drying systems present numerous advantages over traditional methods. These include faster drying rates, reduced risk of contamination, better quality control, and independence from weather conditions. Solar dryers can also be designed to operate at low temperatures, which is crucial for preserving the quality of heat-sensitive agricultural products like cocoa beans (Belessiotis & Delyannis, 2011).

-Quality Control : Solar dryers provide better control over drying conditions, leading to improved product quality (Kumar et al., 2016).

- Reduced Contamination: The enclosed design minimizes contamination risks from external elements.

- Energy Efficiency: Solar dryers use renewable energy, reducing dependency on fossil fuels and lowering operational costs (Belessiotis & Delyannis, 2011).

Disadvantages:

- Initial Cost: The setup of solar dryers can be costly due to the need for materials such as solar panels, collectors, and insulated chambers (Hii et al., 2012).

- Weather Dependency: Solar drying efficiency can still be affected by weather conditions, though to a lesser extent than traditional methods.

2.3 Design Principles for Solar-Powered Cocoa Dryers

The design of a solar-powered cocoa dryer must consider several factors to ensure efficient and effective drying. Key design parameters include the size and orientation of the solar collector, air flow rate, drying chamber design, temperature control and material and insulation . The drying temperature should be carefully regulated to prevent overheating, which can degrade the quality of cocoa beans (Afoakwa et al., 2020)

- Collector Design: The efficiency of the solar dryer depends significantly on the design of the solar collector. Flat-plate collectors are commonly used for their simplicity and cost-effectiveness. The material of the collector should have high absorptivity and low reflectivity to maximize solar energy absorption (El Rhafiki et al 2023) . A well-designed collector maximizes the absorption of solar energy. Materials like black-pai nted metals or selective coatings can enhance energy absorption (El Rhafiki et al 2023).

- Temperature Control: Automated systems to monitor and control temperature can prevent overheating, preserving the quality of the beans (Akinola et al., 2006).

- Air Flow and Ventilation: Proper ventilation is crucial for removing moisture-laden air from the drying chamber. Forced convection systems, which use fans to circulate air, are often preferred over natural convection systems for more consistent drying results (Hossain & Bala, 2007). Forced convection using fans powered by solar panels can enhance air circulation, providing uniform drying and reducing drying times (Hossain & Bala, 2007).

- Drying Chamber Design: The chamber should allow for uniform air distribution and should be made of materials that prevent heat loss, such as insulated panels or reflective materials (El-Sebaii & Shalaby, 2012).

- Material and Insulation: The choice of materials for the construction of the dryer affects its efficiency and durability. Materials with good thermal insulation properties help retain heat and improve drying efficiency (Belessiotis & Delyannis, 2011).

2.4 Case Studies and Applications of Solar Dryers in Cocoa Processing

Several studies have demonstrated the effectiveness of solar dryers in cocoa processing. A study by Fagunwa et al. (2020) evaluated a hybrid solar-biomass dryer for cocoa beans and reported significantly reduced drying times and improved bean quality compared to traditional sun drying.

- Hybrid Solar-Biomass Dryers: A study by Fagunwa et al. (2020) demonstrated the effectiveness of a hybrid solar-biomass dryer, which combines solar energy with biomass for heating. This system provided continuous drying even in low sunlight conditions, improving drying efficiency and bean quality.

- Community-Based Solar Drying Projects: In Ghana, community-based solar drying projects have been implemented to support smallholder farmers. These projects have demonstrated improved bean quality, reduced drying times, and increased incomes for farmers (Ortis et al., 2018).

- 2.5 Materials for Construction of Solar Dryers

- The choice of materials significantly affects the efficiency and durability of solar dryers:

- - Frame and Structure: Lightweight, corrosion-resistant materials such as aluminum or galvanized steel are commonly used for the frame (Smith et al, 2024).

- - Insulation: Materials like foam or fiberglass are used to insulate the drying chamber, reducing heat loss and improving efficiency (Belessiotis & Delyannis, 2011).

- - Glazing: Transparent materials like glass or UV-stabilized plastic allow solar radiation to enter the dryer while trapping heat inside (El-Sebaii et al., 2002).

- 2.6 Energy Efficiency and Performance Optimization

- Solar dryers can be optimized for energy efficiency by incorporating:

- - Solar Photovoltaic Panels: These panels can power fans and other electrical components, reducing reliance on external power sources (Hossain & Bala, 2007).

- - Thermal Storage: Incorporating thermal storage systems, such as water-filled containers, can help maintain drying temperatures during cloudy periods or at night (Hii et al., 2012).

- - Reflective Surfaces: Using reflective materials to direct more sunlight into the dryer can enhance the drying rate and reduce energy losses (Belessiotis & Delyannis, 2011).

2.7 Economic and Environmental Impact

Economic Benefits:

- Solar dryers can reduce drying times and improve product quality, leading to higher market prices for cocoa beans (Afoakwa, 2014).

- Lower operational costs compared to traditional drying methods due to the use of free solar energy (Hii et al., 2012).

Environmental Impact:

- Solar drying reduces the carbon footprint associated with cocoa processing by minimizing the use of fossil fuels (Belessiotis & Delyannis, 2011).

- The reduced need for large drying areas helps preserve land and prevents deforestation (Barimah et al., 2021).

2.8  Solar Energy for Agricultural Drying

Solar energy is one of the most abundant and renewable sources of energy, particularly in tropical regions where cocoa is predominantly grown (Chandel et al., 2014). Solar dryers harness solar radiation and convert it into heat energy, which is used to reduce the moisture content of agricultural products. Cocoa beans, being highly sensitive to moisture, require controlled drying conditions to ensure product quality (Osei et al., 2016).

Solar drying has been widely studied for various agricultural products such as fruits, vegetables, grains, and spices. According to Sharma et al. (2009), the application of solar energy in post-harvest operations has not only reduced post-harvest losses but has also improved the shelf-life and quality of agricultural products. Given the geographical locations where cocoa is grown, solar-powered drying is a highly suitable and sustainable technology for cocoa farmers.

.2.8.1 Heat Transfer Mechanisms in Solar Dryers

Efficient drying processes depend on the heat transfer mechanisms employed within the dryer. Solar dryers use the combination of three heat transfer modes: conduction, convection, and radiation. In the case of cocoa drying, indirect solar dryers and mixed-mode dryers are more effective in protecting beans from direct exposure to UV radiation, which can deteriorate their chemical composition (Ekechukwu & Norton, 1999).

- Conduction: Heat conduction occurs within the solar collector, where the solar radiation heats the surface. The collector is typically made of materials with high thermal conductivity, such as aluminum or copper, to maximize heat transfer to the circulating air (Kavak Akpinar, 2010).

- Convection: Convective heat transfer is the most critical mode for moving heat from the solar collector into the drying chamber. Efficient airflow management and the use of fans in forced convection dryers significantly enhance the drying rate (Garg & Prakash, 2000).

-Radiation: Solar radiation is absorbed by the dryer’s glazing material, which is then converted into heat energy. Materials such as glass and transparent plastics are typically used for glazing to allow maximum penetration of solar radiation while retaining heat inside the dryer (Kumar et al., 2016).

.2.8.2 Optimization of Solar Dryer Performance

Optimization of solar dryers aims to improve drying efficiency, reduce energy consumption, and enhance product quality. Various strategies are employed to achieve these objectives, such as the use of thermal energy storage, design modifications, and computational modeling.

- Thermal Energy Storage: Solar dryers can be equipped with thermal energy storage systems, which store excess heat during peak solar hours and release it when solar radiation is insufficient, such as during cloudy weather or at night (Belessiotis & Delyannis, 2011). Thermal storage materials, such as water, rocks, or phase change materials (PCMs), have been explored for this purpose (El-Sebaii et al., 2010).

- Design Modifications: Design optimization often involves improving the solar collector’s efficiency, enhancing insulation, and refining the airflow patterns in the drying chamber. Incorporating reflective surfaces and positioning solar panels at optimal angles can increase the amount of solar radiation absorbed (Akbulut & Durmuş, 2010). Additionally, multi-tray dryers with staggered arrangements have been found to promote uniform drying of cocoa beans (Akinola et al., 2006).

- Computational Modeling: Numerical simulations and computational fluid dynamics (CFD) modeling have been used to predict the performance of solar dryers and optimize their design. These models help determine the best configurations for airflow, drying rates, and temperature distribution within the dryer (Tzempelikos et al., 2013).

4. Material Selection and Durability in Solar Dryer Fabrication

The choice of materials for the construction of solar dryers is critical for ensuring durability, thermal efficiency, and cost-effectiveness. Materials should be selected based on their ability to withstand high temperatures, resist corrosion, and maintain thermal efficiency over time (Mumba, 1995).

- Absorbing Surfaces: The absorbing surfaces of the solar collector are typically coated with black paint or a selective coating that maximizes absorption and minimizes reflection of solar radiation. Studies by Kumar et al. (2012) have shown that selective coatings can enhance the thermal efficiency of collectors by up to 15%.

- Glazing: Glazing materials, such as glass or transparent plastic sheets, are used to cover the solar collector and drying chamber. Glass is preferred for its high transmissivity and durability, though it is more expensive and heavier than plastic alternatives like polycarbonate or acrylic (Duffie & Beckman, 2013).

- Insulation: The insulation of the drying chamber and collector is vital for retaining heat within the system. Materials such as fiberglass, foam, and sawdust are commonly used to reduce heat losses (Bala, 1998). Proper insulation ensures the dryer operates efficiently even during periods of lower solar radiation.

.2.8.3 Environmental and Socio-Economic Impact of Solar Dryers

Environmental Impact: The adoption of solar drying technologies has several environmental benefits, especially in reducing greenhouse gas emissions and minimizing deforestation. Traditional drying methods often involve cutting down trees to create large drying spaces or burning fossil fuels to accelerate the drying process (Hii et al., 2012). By utilizing solar energy, solar dryers eliminate the need for deforestation and fossil fuel consumption, contributing to environmental sustainability (Belessiotis & Delyannis, 2011).

Socio-Economic Impact: Solar dryers have significant socio-economic benefits, particularly for smallholder cocoa farmers in rural areas. The reduction in drying time and improvement in bean quality lead to better prices for dried cocoa beans (Afoakwa, 2014). Moreover, solar dryers provide an alternative to labor-intensive traditional methods, freeing up time for farmers to engage in other productive activities (Ortis et al., 2018).

In Ghana, for instance, solar dryer projects have contributed to increased productivity, higher incomes, and improved living standards for cocoa farmers (Barimah et al., 2021). These technologies are also recognized as crucial for reducing post-harvest losses, which can account for up to 30% of cocoa production (Schwan & Wheals, 2004).

.2.8.4 Case Studies on Solar Dryers for Cocoa

- Hybrid Solar Dryers in Nigeria: A hybrid solar-biomass dryer designed by Fagunwa et al. (2020) for cocoa beans demonstrated a 30% reduction in drying time compared to traditional sun drying methods. The hybrid system provided consistent drying rates even during periods of low sunlight, leading to improved cocoa quality and reduced labor requirements.

- Solar Tunnel Dryer in Ghana: A study by Hii et al. (2012) evaluated the performance of a solar tunnel dryer for drying cocoa beans in Ghana. The dryer, equipped with photovoltaic panels for powering fans, achieved uniform drying and reduced drying times by 40%. The beans retained their natural flavor, and contamination risks were minimized compared to open-air drying.

7. Future Trends in Solar Drying Technology

Hybrid Systems: Future developments in solar drying will likely focus on hybrid systems that combine solar energy with other renewable energy sources such as biomass, wind, or geothermal energy. These systems offer continuous drying capabilities regardless of weather conditions, further improving efficiency (Kemp, 2011).

Automation and Smart Dryers: The integration of sensors, automated temperature control, and IoT (Internet of Things) technologies into solar dryers is an emerging trend that can significantly improve drying precision and reduce the need for human intervention (Mekhilef et al., 2011). Automated systems can monitor moisture content, temperature, and drying progress, ensuring optimal conditions for cocoa drying.