CHAPTER FIVE
CONCLUSIONS AND RECOMMENDATIONS.

5.1 Conclusions

- The incorporation of sawdust and 1.5 % PKO yields a workable foam concrete mix with good consistency for applications such as partition blocks and insulation panels.

- The hybrid foam concrete exhibits reduced density compared to conventional concrete, confirming its lightweight character and potential for energy‐efficient construction.

- Strength development over 28 days demonstrates that, although lower than traditional concrete, the compressive strength of the hybrid mix (3.77 N/mm²) meets requirements for non‐load-bearing components.

- Palm kernel oil is an effective natural surfactant in foam concrete production, providing stable air voids without reliance on synthetic chemicals.

5.2 Recommendations

- Use the developed hybrid foam concrete in wall panels, ceiling infill, and insulation applications where reduced weight and thermal performance are prioritized over load-bearing capacity.

- For projects requiring slightly higher strength, explore combination with supplementary cementitious materials (e.g., fly ash or silica fume) to improve mechanical properties without significantly increasing density.

- Implement pre-soaking of sawdust and strict control of PKO purity to ensure consistent mix behavior and performance across different batches.