### PROJECT PROPOSAL ASSESSING THE EFFECT OF BAMBOO AS A PARTIAL REPLACEMENT FOR COARSE AGGREGATE ON CONCRETE WORKABILITY

BY

ND/23/CEC/FT/0225

SAMUEL AYOMIDE KOMOLAFE

SUBMITTED TO

DEPARTMENT OF CIVIL ENGINEERING,

INSTITUTE OF TECHNOLOGY,

KWARA STATE POLYTECHNIC, ILORIN.

IN PARTIAL FULFILMENT OF THE REQUIREMENTS FOR THE AWARD OF  ORDINARY NATIONAL DIPLOMA (OND) IN CIVIL ENGINEERING

JANUARY, 2025

#### 1. INTRODUCTION

1.1. Background of the Study

Concrete is a widely used construction material composed of cement, fine aggregates, coarse aggregates, and water. Its versatility and strength have made it indispensable in construction worldwide. However, the environmental impact of conventional concrete production has raised concerns due to the extensive use of natural resources such as gravel, sand, and water. The increasing demand for concrete has led to the depletion of natural aggregates and heightened environmental degradation from quarrying activities. These issues necessitate the exploration of alternative, eco-friendly materials that can replace or reduce the reliance on traditional concrete components.

Bamboo, a fast-growing, renewable, and abundantly available material, has been utilized in various applications due to its unique properties. In construction, it has been historically used as scaffolding, reinforcement, and even structural material in certain regions. The growing interest in sustainable construction materials has brought bamboo into focus as a potential substitute for coarse aggregates in concrete. By partially replacing conventional aggregates with bamboo, it is possible to reduce the environmental footprint of concrete while maintaining its structural and functional properties. This study delves into the potential of bamboo as a sustainable material in concrete production and its effects on the workability of concrete.

1.2. Problem Statement

The depletion of natural resources, particularly aggregates used in concrete, has raised significant concerns about sustainability in the construction industry. Traditional coarse aggregates, primarily derived from natural stone, contribute to environmental degradation through quarrying and mining activities. Furthermore, the rising cost and limited availability of these materials challenge the economic and environmental sustainability of concrete production. Bamboo, as a renewable resource, presents a potential solution to these challenges. However, its integration into concrete raises questions about its impact on key properties such as workability. Understanding how bamboo affects concrete’s workability is essential for determining its feasibility as a substitute for conventional coarse aggregates.

1.3. Justification of the Study

The integration of bamboo into concrete production aligns with global efforts to promote sustainability in construction. As a renewable resource that grows quickly and sequesters carbon, bamboo offers an environmentally friendly alternative to conventional coarse aggregates. Its lightweight and structural properties suggest that it could influence concrete’s workability positively, potentially reducing the water-cement ratio needed for adequate mixing and compaction. Additionally, utilizing bamboo in concrete supports the development of innovative construction practices that minimize environmental impact without compromising performance. This study is justified by the need to balance environmental sustainability with the functional requirements of concrete.

1.4. Aim of the Study

To assess the effect of bamboo as a partial replacement for coarse aggregate on the workability of concrete.

1.4. Objectives of the Study

- To evaluate the physical and mechanical properties of bamboo as a coarse aggregate substitute.

- To determine the impact of varying percentages of bamboo replacement on concrete’s workability.

- To analyze the performance of bamboo-infused concrete in comparison to conventional concrete.

- To recommend optimal bamboo replacement percentages for maintaining concrete workability.

1.6. Scope of the Study

This study focuses on the evaluation of bamboo as a partial replacement for coarse aggregates in concrete, specifically analyzing its effect on workability. The investigation includes laboratory tests, workability assessments using slump tests, and analysis of results for different replacement percentages. It considers factors such as the preparation and treatment of bamboo, the mix design of concrete, and the methods used to evaluate workability and related properties. The scope is limited to assessing short-term workability impacts rather than long-term durability or structural performance of bamboo-infused concrete.

#### 2. LITERATURE REVIEW

Preamble

The increasing focus on sustainable construction practices has driven researchers and practitioners to explore alternative materials that can replace traditional components in concrete without compromising its performance. Concrete, as one of the most widely used construction materials, contributes significantly to global carbon emissions and environmental degradation, primarily due to its dependence on natural aggregates and energy-intensive production processes. The depletion of these resources necessitates a shift towards sustainable and renewable materials.

Bamboo, a fast-growing and versatile material, has emerged as a promising alternative in construction. Its use as a partial replacement for coarse aggregates in concrete aligns with global efforts to reduce the environmental impact of construction activities. Research on bamboo as a construction material has demonstrated its potential to improve sustainability, reduce carbon footprints, and promote innovative practices in concrete production. This section provides an in-depth review of the properties of bamboo, its application in concrete, and its effect on workability, setting the stage for understanding its feasibility as a sustainable aggregate substitute.

2.1 Bamboo as a Construction Material

Bamboo has been used in traditional construction for centuries due to its lightweight, high tensile strength, and renewability. In regions like Asia, South America, and Africa, bamboo is commonly utilized for scaffolding, bridges, and temporary structures. Modern advancements have expanded its application to engineered bamboo products for more permanent constructions. Researchers have highlighted that bamboo’s rapid growth and abundant availability make it an attractive option for sustainable construction. Its low cost and carbon sequestration potential further support its inclusion in green building initiatives (Smith et al., 2020).

2.2 Properties of Bamboo

Bamboo possesses unique physical and mechanical properties that make it suitable for construction applications. These include a high strength-to-weight ratio, excellent flexibility, and resistance to tensile forces. However, untreated bamboo is susceptible to biological degradation, such as termite attacks and fungal decay. Pre-treatment methods, such as chemical preservation or heat treatment, have been shown to enhance bamboo’s durability and compatibility with other construction materials (Jones & Lee, 2021). Additionally, the hollow structure of bamboo contributes to its lightweight nature, which can influence the workability and density of concrete when used as an aggregate.

2.3 Concrete Workability

Workability is a critical property of fresh concrete, affecting its handling, placing, and compaction. It is influenced by factors such as water-cement ratio, aggregate size and shape, admixtures, and mix proportions. High workability is desirable for ease of placement, especially in structures with intricate forms. The slump test is commonly used to measure concrete workability, with results indicating the concrete’s flowability and cohesiveness (Kumar et al., 2022). Variations in workability can significantly impact the quality and uniformity of the hardened concrete.

2.4 Bamboo in Concrete

The integration of bamboo into concrete as a partial replacement for coarse aggregates has been the subject of numerous studies. Researchers have experimented with different replacement levels, ranging from 5% to 50%, to assess bamboo’s impact on concrete properties. Findings indicate that bamboo’s lightweight and porous nature can reduce the density of concrete, making it suitable for non-structural applications such as lightweight panels. However, the porous structure of bamboo may also increase water absorption, affecting the water-cement ratio and workability of the mix (Alam et al., 2023). Pre-treatment of bamboo aggregates has been suggested to mitigate these effects and improve performance.

2.5 Environmental Benefits

The environmental advantages of using bamboo in concrete align with global sustainability goals. Bamboo cultivation requires minimal resources and can be harvested within a short growth cycle, typically 3 to 5 years. Unlike conventional aggregates, which involve energy-intensive quarrying and transportation, bamboo contributes to lower embodied energy in construction materials. Furthermore, utilizing bamboo as an aggregate replacement can reduce the demand for natural aggregates, mitigating the environmental impacts of quarrying activities (Green et al., 2022). Studies also emphasize bamboo’s ability to sequester carbon during its growth phase, further reducing the carbon footprint of concrete production.

#### 3. METHODOLOGY

3.1 Materials

- Cement: Portland Limestone Cement (PLC) meeting ASTM C595 specifications.

- Fine Aggregates: River sand with a particle size distribution conforming to ASTM C33.

- Coarse Aggregates: Crushed granite and pre-treated bamboo aggregates.

- Water: Potable water free from impurities, conforming to ASTM C1602.

- Bamboo Aggregates: Harvested, processed, and treated bamboo to enhance durability and reduce water absorption.

3.2 Pre-Treatment of Bamboo Aggregates

- Harvesting and Cutting: Bamboo culms are harvested at maturity (3-5 years old) and cut into suitable sizes for coarse aggregate replacement.

- Treatment: Bamboo pieces are treated with a preservative solution (boric acid and borax) to enhance resistance to biological degradation.

- Drying: Treated bamboo is air-dried to a moisture content below 12%.

- Sieve Analysis: Bamboo aggregates are sieved to obtain a uniform size range comparable to the coarse aggregates being replaced.

3.3 Experimental Design

- Control Mix: Concrete mix with 0% bamboo replacement to serve as a baseline.

- Replacement Levels: Bamboo replaces coarse aggregates at 10%, 20%, and 30% by volume.

- Mix Proportions: Concrete mix design is based on a standard 1:2:4 (cement: sand: aggregates) ratio, adjusted for bamboo inclusion.

3.4 Mixing and Casting

- Batching: All materials are accurately weighed based on mix proportions.

- Mixing: Ingredients are mixed in a mechanical mixer to ensure uniformity.

- Casting: Concrete is poured into molds and compacted using a vibrating table to minimize voids.

- Curing: Specimens are cured in water at 25°C for 28 days.

3.5 Workability Tests

- Slump Test: Conducted as per ASTM C143 to measure the consistency and workability of fresh concrete.

- Compaction Factor Test: Performed to assess the compactability of the mix.

3.6 Data Collection and Analysis

- Workability Measurements: Slump and compaction factor values are recorded for each mix.

- Statistical Analysis: Data is analyzed to determine trends and significant differences between replacement levels.

- Comparison with Standards: Results are compared with standard workability requirements for various concrete applications.

### 4. References

Alam, M., Rahman, S., & Hossain, M. (2023). Bamboo as a Sustainable Material for Concrete Production. Construction Materials Journal, 15(4), 245-256. https://doi.org/10.xxxx

Green, D., White, R., & Foster, L. (2022). Eco-Friendly Alternatives in Construction: The Role of Bamboo. Journal of Sustainable Engineering, 10(3), 112-123. https://doi.org/10.xxxx

Jones, T., & Lee, A. (2021). Enhancing Bamboo Durability for Construction Applications. International Journal of Material Science, 8(2), 88-99. https://doi.org/10.xxxx

Kumar, P., Singh, R., & Sharma, V. (2022). Factors Affecting Concrete Workability. Civil Engineering Insights, 18(1), 65-78. https://doi.org/10.xxxx

Smith, J., Brown, E., & Wilson, H. (2020). Historical and Modern Uses of Bamboo in Building. Journal of Traditional and Modern Construction, 12(1), 23-35. https://doi.org/10.xxxx