CHAPTER FOUR

DATA PROCESSING AND ANALYSIS OF RESULT

Data processing is also referred to as the computation stage. It is the intermediary between the field observation and data presentation stage. At this stage, all the data acquired from the field were processed and analyzed in order to proceed to the final stage.

Data processing and analysis comprises of the following

* Traverse field book reduction

* Traverse computation

* Computation of leveling

* Detailing computation

* Traverse field book reduction - Angular

4.1 DATA DOWNLOAD AND EDITING

This is the transfer of data from the memory unit of digital instrument into the computer system for the processing and storage stage for easy retrieval. The total station was connected to the computer through a data transfer cable using a data processing software for the downloading.

Data Processing in AUTOCAD

To open AUTOCAD software and following the underlisted procedure:

* Switch on the computer and allowed it to boot

* Start menu was clicked

* Select program was clicked

* From the Notepad, a script files for the coordinates, line, text and other was structured.

* Files was saved with the extension Script.

* AUTOCAD was launched

* File menu was clicked

* Sub menu (New) was clicked and the name was saved.

* Format was clicked and all necessary settings were carried out (i.e units, dimension etc.)

* Then 'OK' was clicked to the aspect of parameter setting.

* Tools were selected

* Run script was clicked on.

* Escape key was clicked, Z enter and E enter were pressed three after the other.

* Text was clicked

* Escape key was pressed, Z then E enter.

* Text writing and other necessary editing were done.

* Coordinates of the details were all typed.

* Coordinates were pasted and then the point were all displayed.

* With polyline the point were joined as they were sketched.

| Stn. | Bearing | Distance | Δ East | Δ North | Easting | Northing | Stn. |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | 675605.928 | 938052.240 | A |
| A | 073˚ 10’ 02’’ | 110.05 | -57.891 | -254.551 | 675548.031 | 937793.689 | B |
| B | 185˚ 19’ 46’’ | 120.54 | -150.574 | 34.798 | 675397.457 | 937832.487 | C |
| C | 254˚ 09’ 58’’ | 110.78 | 103.191 | -23.065 | 675500.648 | 937809.422 | D |
| D | 345˚ 16’ 25’’ | 121.66 | 105.280 | 242.819 | 675605.928 | 938052.240 | A |

BACK COMPUTATION