CHAPTER THREE

3.0 System Design and Methodology

3.1.0 System Requirements

3.1.1 Safety Components

- DC circuit breakers/fuses between charge controller and batteries

- AC circuit breakers/fuses for inverter output

- Grounding for all metallic enclosures

3.1.2 Monitoring

- Basic LCD/LED display on charge controller for real-time voltage, current, and SOC (state of charge)

- Option for future upgrade to remote monitoring

3.1.3 Environmental Condition

- Operating temperature range: 0°C – 50°C

- Indoor installation, protected from rain and direct sunlight

3.2 Block Diagram

Figure 3.2

3.3.0 Component Selection

3.3.1 System Capacity

- Rated Power Output: 2kVA (2000VA)

- AC Output Voltage: 220V ± 10%, 50Hz (standard for residential and office appliances)

- DC Input Voltage: 24V (battery bank voltage)

### 3.3.2  Solar Panels

- Type: Monocrystalline solar panels

- Number of Panels: 4

- Individual Power Rating: 450W per panel

- Total Solar Array Capacity: 1800W (4 × 450W)

- Operating Voltage: ~37-40V per panel

- Series/Parallel Configuration: Configured to charge a 24V battery bank efficiently via MPPT

### 3.3.3 Batery Bank

- Type: Lithium-ion batteries

- Number of Batteries: 4

- Individual Voltage: 12V

- Total Configuration: 24V (two 12V batteries in series, paralleled with another series pair)

- Total Capacity: ~200Ah (depending on the specific battery amp-hour rating)

- Depth of Discharge (DoD): Up to 80-90% (typical for lithium batteries)

### 3.3.4 Charge Controller

- Type: MPPT (Maximum Power Point Tracking)

- Current Rating: 60A

- Input Voltage Range: Compatible with solar panel array voltage (typically 30-100V)

- Output Voltage: 24V for charging the battery bank

- Features: Overcharge, over-discharge, and short circuit protection

### 3.3.5 Inverter Specification

- Type: Pure Sine Wave Inverter

- Output Power: 2kVA continuous, with short-term surge capability

- Efficiency: 85-90% (typical for modern inverters)

- Cooling: Forced-air cooling (internal fan)

- Protections: Short-circuit, over-temperature, overload, and low/high voltage shutdown

### 3.4.0 Circuit Diagram

Figure 3.4

3.4.1 Explanation of the Circuit Diagram

3.4.1.2 Solar Panels

- 4 pieces of 450W panels convert solar radiation into DC electrical energy.

3.4.1.3 MPPT Charge Controller (60A)

- It tracks the maximum power point to regulate the charging voltage and current to the battery bank.

- It also prevents overcharging and prolongs battery life.

3.4.1.4 Lithium Battery Bank (24V)

- 4 lithium batteries configured to produce a nominal 24V storage system.

- Provides backup power when solar generation is low.

3.4.1.5 2kVA Pure Sine Wave Inverter

- Converts DC from the batteries into 220V AC (50Hz) for household appliances.

- Pure sine wave output ensures safe and reliable power for sensitive loads.

3.4.1.6 AC Loads

- Lights, fans, TVs, and other typical household devices powered from the inverter AC output.

3.5.0  Software Tools

Here’s a short overview of the software that was used in the completed solar-powered inverter system project for simulation and design purposes:

- Proteus Design Suite

Used for simulating and testing DC and AC electrical circuits, including inverter control circuits.

- MATLAB/Simulink

Useful for modeling and simulating the overall behavior of renewable energy systems, MPPT algorithms, and battery management.

- PVsyst

A powerful tool for simulating solar energy systems to estimate solar energy yield and performance.

- AutoCAD Electrical

Ideal for drawing detailed electrical schematics, wiring diagrams, and panel layouts.

- ETAP

Often used for advanced load flow analysis, short circuit calculations, and system protection.

3.6.0 Safety Considerations

Ensuring the safety and reliability of the solar-powered inverter system is paramount. The following measures were implemented during design and construction:

- Proper Component Selection

- Certified components were chosen (solar panels, charge controller, lithium batteries, inverter) to ensure high-quality and reliability.

- Overcurrent ratings and compatibility checks were conducted to prevent system overloading.

- Circuit Protection Devices

- DC and AC Fuses/Circuit Breakers were installed:

  - Between the solar panels and charge controller

  - Between the charge controller and battery bank

  - Between the inverter and AC loads

- These protect against short circuits, overcurrent, and potential electrical fires.

- Battery Protection

The lithium batteries include a Battery Management System (BMS), which:

- Prevents overcharge and deep discharge

- Monitors cell temperature and voltage

- Balances cells during charging

- Grounding and Earthing

- All metal parts of the system (e.g., inverter casing, control panel enclosures) were properly grounded to prevent electric shocks.

- Proper Cable Sizing and Termination

- Cables were selected based on load current with proper insulation to reduce overheating and energy losses.

- Crimped terminals and lugs were used for secure connections, reducing the risk of loose connections.

- Environmental Protection

- Indoor installation in a well-ventilated area to protect against rain and direct sunlight.

- The inverter and batteries were placed in enclosures to prevent accidental contact.

- Load Management

- The system was designed with load estimation to avoid overloading the inverter and batteries.

- A load schedule can be implemented to prioritize critical loads during low solar input.

- Periodic Maintenance

Recommendations for regular inspection of:

- Battery voltage and condition

- Cable connections and fuse conditions

- Inverter operation (fan, temperature, alarms)