# CHAPTER THREE: METHODOLOGY

### 3.1 Design Considerations

The door was designed to be:

- 1800 mm height × 1200 mm width (600 mm per panel)

- Reinforced with internal stiffeners

- Hinged on a mild steel frame with bolt locks

### 3.2 Material Selection

- Mild steel sheet (1.5 mm thick)

- Hollow square pipe (25 mm × 25 mm for frame)

- Hinges, lockset, angle bars

- Welding rods (E6013)

- The materials selected for the fabrication of the door were based on availability, cost, strength, and durability:

- Mild Steel Sheets (16-gauge): Used for the door panels due to their ease of welding and sufficient strength.

- Mild Steel Angle Bars (30mm x 30mm): Used for the door frame and panel reinforcement.

- Steel Hinges (Heavy-duty, 3”): For door mounting and operation.

- Metal Handles and Lockset: For functional entry and security.

- Electrodes (E6013): Used for Shielded Metal Arc Welding (SMAW).

Red Oxide Primer and Enamel Paint: For corrosion resistance and final finishing.

3.2 Tools and Equipment Used

- Measuring Tape and Try Square – For accurate marking and squaring edges.

- Hack Saw/Angle Grinder – For cutting steel to size.

- Welding Machine (AC Arc Welder) – For joining metal parts using SMAW.

- Clamps – To hold workpieces in position during welding.

- Bench Vise – For holding components during grinding or shaping.

- Grinding Machine – For surface smoothing and weld clean-up.

- Paint Brush/Spray Gun – For applying finishing coats.

  - Design and Construction Process

  - Step 1: Design Drafting

- A manual sketch and basic CAD drawing were prepared to outline the dimensions and structural layout of the door (1980 mm x 900 mm).

- Step 2: Cutting of Materials

- Angle bars and mild steel sheets were marked and cut using an angle grinder according to the specified dimensions for the frame and panels

- Step 3: Frame Assembly

- The cut angle bars were tack-welded to form the outer frame. Diagonal measurements were checked to ensure squareness before final welding.

Step 4: Panel Fabrication

- Two mild steel sheets were cut and fitted within the frame to create twin panels. Vertical and horizontal stiffeners were added using flat bars for reinforcement.

Step 5: Welding

- All components were permanently welded, ensuring continuous welds for strength and minimizing weak joints.

- Step 6: Grinding and Finishing

- Welds were ground smooth, and all surfaces cleaned to remove slag, rust, or dirt before painting.

- Step 7: Priming and Painting

- A coat of red oxide primer was applied, followed by two coats of oil-based enamel paint for aesthetics and corrosion protection.

  - Assembly Techniques Hinges were attached using arc welding and aligned to ensure smooth swing action.

  - Finishing Processes

4.0 Personal Protective Equipment (PPE) like gloves, welding helmets, and boots were worn throughout.

5.0Fire extinguishers were kept nearby due to flammable materials.

6.0Welding was done in a well-ventilated area to prevent gas inhalation.