A PROJECT REPORT

ON

ROUTE SURVEY OF LAJOLO TO ELEKO ANFEYIN ROAD ILORIN, MORO LOCAL GOVERNMENT AREA, KWARA STATE.

BY

DAHUNSI OLAWUNMI COMFORT  HND/23/SGI/FT/0069

SUBMMITED TO:

THE DEPARTMENT OF SURVEYING AND GEO-INFORMATICS, INSTITUTE OF ENVIRONMENTAL STUDIES, KWARA STATE POLYTECHNIC ILORIN.

IN PARTIAL FULFILLMENT OF THE REQUIREMENT FOR THE

AWARD OF HIGHER NATIONAL DIPLOMA IN SURVEYING AND GEO-INFORMATICS.

JUNE, 2025

## CERTIFICATE

I hereby certified that the information given in this project was obtained as a result of the observation and measurement made by me and that the survey was carried out in accordance with survey laws, regulations and departmental instructions

_________________________________

## DAHUNSI OLAWUNMI COMFORT

Matric. No: HND/23/SGI/FT/0069

# CERTIFICATION

This is to certify that this project was carried out by DAHUNSI OLAWUNMI COMFORT with Matric No.: HND/23/SGI/FT/0069 under my instruction and supervision for the award of Higher National Diploma in Surveying and Geo-informatics, Kwara State Polytechnic, Ilorin, Kwara State Nigeria. I hereby declared that she has conducted herself with due diligence and honesty on the said duties.

………………………………… 	 	 	 	………………………………

## SURV. A. AYUBA 	 	 	 	 	 	 	DATE

(Project Supervisor)

………………………………. 	 	 	 	…………………………

## SURV. R.S. AWOLEYE 	 	 	 	 	 	DATE

(Project Coordinator)

……………………………… 	 	 	 	………………………………

## SURV. ABIMBOLA I ISAU 	 	 	 	 	 	DATE

Head of Department (H.O.D)

Surveying & Geo-informatics

……………………………….. 	 	 	 	…………………………….. EXTERNAL EXAMINER 	 	 	 	 	 	DATE

# DEDICATION

This project is dedicated to Almighty GOD, the author and the finisher of everything who gave me the grace and privilege to attain this stage in my education pursuit.

## ACKNOWLEDGEMENT

I give thanks and glory to my Creator for the privilege bestowed on me that resulted in the accomplishment of this project work and also a big thanks to the management of Kwara State Polytechnic for the privilege and opportunity given to me to study in their great institution.

My sincere gratitude goes to my Head of Department (HOD); A.I Isau and my Supervisor; Surv. A. Ayuba for being my backbone right from the beginning of this project and all his support academically, morally, I really appreciate you sir.

To all the lecturers in the department of Surveying and Geo-informatics; Surv. A.I. Isau, Surv. A. Ayuba, Surv. Banji, Surv. Awoleye, Surv. A.G. Aremu, Surv. Kabir, Surv. Kazeem, Surv. Asonibare and Surv. Diran who has imparted their knowledge in me in one area or the other during my year of study, I am grateful for all your effort and support. May Almighty GOD bless you all (Amen).

I'm deeply grateful to my parents; Mr. & Mrs. Dahunsi for their unconditional love, support, and wisdom. Their encouragement and sacrifices have been the driving force behind my endeavors. Thank you, Mom and Dad, for being my pillars of strength and inspiration. I acknowledge this project to you, with love and appreciation.

My appreciation also goes to Bamidele, you are such a darling. Thanks for your support over this journey of education, thanks for your love and care. I really appreciate you dear may all your wishes and dreams come to reality.

Special thanks to my siblings for their unwavering love, care, and dedication that have shaped me into the person I am today. Their selfless love and their influence and values have guided me throughout this project, and I'm forever grateful.

I would like to extend my special and heartfelt thanks to Surv. A. Ayuba who stood by me in all circumstances and supported me throughout the journey of this Higher National Diploma, I pray, may the Lord continue to bless and keep you in Jesus name.

My thanks goes to my school father, my role model, my big head, my sunshine, my sugarplum; Surv. A.G. Aremu. Thanks so much for your love and caring towards this journey, I am very grateful Sir, no one like you dear. It is my prayer that sudden death will not be your portion, you will never know sorrow in your life.

My thanks also goes to my friends; Uyo Ojo, Damilola, Nafisat, Kaffy, Bamidele Abiodun Temitope for their support, care and all. Thanks you all, May Almighty GOD bless you all (Amen).

# TABLE OF CONTENTS

Pages

Title Page 	 	 	 	 	 	 	 	 	 	i

Certification  	 	 	 	 	 	 	 	 	ii

Dedication  	 	 	 	 	 	 	 	 	 	iii

Acknowledgment  	 	 	 	 	 	 	 	 	iv

Table of contents 	 	 	 	 	 	 	 	 	v

List of Tables  	 	 	 	 	 	 	 	 	ix

List of Figures  	 	 	 	 	 	 	 	 	x

Abstract  	 	 	 	 	 	 	 	 	 	xi

## CHAPTER ONE

General Introduction

1.1  	Background to the Study  	 	 	 	 	 	 	1

1.2  	Statement of the Problem  	 	 	 	 	 	 	6

1.3 	Significance of the Project 	 	 	 	 	 	 	6

1.4 	Aim 	 	 	 	 	 	 	 	 	 	6

1.5 	Objectives 	 	 	 	 	 	 	 	 	6

1.6 	Scopes of the Project  	 	 	 	 	 	 	7

1.7 	Project Specification   	 	 	 	 	 	 	7

1.8 	Personnel 	 	 	 	 	 	 	 	 	8

1.9 	Study Area 	 	 	 	 	 	 	 	 	9

## CHAPTER TWO

Literature review

2.1 	Research Analogy  	 	 	 	 	 	 	 	10

## CHAPTER THREE

Research Methodology

3.1 	Research Design  	 	 	 	 	 	 	 	21

3.2 	Planning  	 	 	 	 	 	 	 	 	21

3.2.1 Office Planning  	 	 	 	 	 	 	 	22

3.2.2 Reconnaissance 	 	 	 	 	 	 	 	23

3.3 	Equipment Used  	 	 	 	 	 	 	 	26

3.3.1 Hardware Requirements  	 	 	 	 	 	 	26

3.3.2 Software Used 	 	 	 	 	 	 	 	26

3.4 	Test of Instrument 	 	 	 	 	 	 	 	27

3.4.1 Test of Sokkia Total Station  	 	 	 	 	 	27

3.5 	Data Acquisition 	 	 	 	 	 	 	 	28

3.5.1 Control Check  	 	 	 	 	 	 	 	29

3.5.1.1 Initial Control Check  	 	 	 	 	 	 	29

3.5.2 Traversing (Profiling)  	 	 	 	 	 	 	30

3.5.3 Detailing  	 	 	 	 	 	 	 	 	31

3.6 	Total Station Data Downloading  	 	 	 	 	 	31

## CHAPTER FOUR

Data Processing and Result Analysis

4.1 	Data Processing Procedure 	 	 	 	 	 	 	32

4.2 	Data Editing  	 	 	 	 	 	 	 	32

4.3 	Plan Production 	 	 	 	 	 	 	 	32

4.4 	Result Analysis  	 	 	 	 	 	 	 	38

## CHAPTER FIVE

Costing, Summary, Problem Encountered, Conclusion and Recommendations

5.1  	Cost Estimation  	 	 	 	 	 	 	 	39

5.2 	Summary 	 	 	 	 	 	 	 	 	41

5.3  	Problem Encountered  	 	 	 	 	 	 	42

5.4 	Recommendations 	 	 	 	 	 	 	 	42

5.5 	Conclusion 	 	 	 	 	 	 	 	 	42

References

Appendix

# LIST OF TABLES

Pages

Table 3.1: Table showing the list of coordinate of the existing ground controls used

26

Table 3.2: Test of Sokkia Total Station 	 	 	 	 	 	27

Table 3.3: Table showing the coordinates of the starting control pillars 	 	29

Table 3.4: Table showing the back computation from the existing coordinates of the starting

control pillars   	 	 	 	 	 	 	 	 	30

Table 4.1: Table showing the abstract of TBM coordinates   	 	 	38

Table 5.1: Table showing the cost estimation  	 	 	 	 	39

# LIST OF FIGURES

Pages

Fig. 1.1: Maps Showing the Study Area 	 	 	 	 	 	9

Fig. 3.0: Flowchart Showing the Planning Procedure 	 	 	 	22

Fig. 4.1: Diagram showing how to create points 	 	 	 	 	34

Fig. 4.2: Picture showing how to create surface 	 	 	 	 	35

Fig. 4.3: Picture showing how to create alignment 	 	 	 	 	37

# ABSTRACT

The project is focused on the execution of route surveying from the Lajolo junction to

Maya/Anfeyin Road, Ilorin Kwara State in Moro Local Government Area. Stations were selected along the existing route at regular intervals as in the specifications. Total Station traverse was run over stations. All data were gathered with effective orientation from the

controls, these were used to produce the plans (with appropriate scale) showing horizontal alignment, profile and cross sections. The methodology involved reconnaissance, data

acquisition using advanced surveying instruments such as total stations and GNSS receivers, and data processing with industry-standard software like AutoCAD Civil 3D. Key aspects of the survey included horizontal and vertical alignment studies, cross-sectional profiling, and identification of existing utilities and environmental constraints.

CHAPTER ONE

INTRODUCTION

## 1.1  	BACKGROUND OF THE STUDY

Engineering Surveying is one of the branches of surveying which form the general express for any survey work carried out in connection with construction engineering and building projects, it is a large scale topographical map survey which form the basis for design of engineering works such as highways, railways, canals, tunnels, dams, pipelines and transmission lines.

In the World of engineering and environmental studies, surveying has been recognized as the inevitable operation been described as be the bedrock of every meaningful development. The greater part of its responsibility is seen from the various types of surveying which are all geared toward promoting route ways, convenience, accessibility and spatial declination of both natural and social environments. In addition, surveying means the activities of planning, designing, constructions, development and rehabilitation of roads, depending on the survey data required by the surveyor.

Route survey which is an aspect of engineering survey may be defined as the survey operation that is done for the establishment of the horizontal and vertical alignment of transportation facilities. It involved: planning, design and setting out of any route such as railways, highways, pipelines and canals etc. as obtained by a surveyor and it also involved the proper assessment of natural and man-made features.

In addition, this type of survey should be applied when there is need for alignment, expansion, or rehabilitation at any existing route (road) e.g. for traffic purposes. The reason is that route survey provides a plan/map that shows the alignment, details, profile and cross sectional leveling which depicts the nature of the terrain of a given strip of land which serves the purpose of location, design and construction of route networks.

Good road network is one of the basic amenities needed by human being as movement is one of the characteristics of living things. There are needs for people to move from one place to the other and transportation of goods and services. Therefore, for any meaningful development in a country, good road is a must.

Route surveying is such a survey exercise that requires all field works and calculations made for the purpose of locating and constructing a cross-country social utilities such as highways, railways, canals, transmission lines and pipeline. This involve the determination of the ground configuration and location of physical features (naturals and manmade) along the route, establishing the line on the ground and computing the volume of earthwork.

The main purpose of any route survey is to: Select one or more tentative general route for the road way or utility gather enough information about the general route to make it possible for the route designers.

With these purposes, route survey usually entails reconnaissance, preliminary and final location survey phases that satisfy respectively, each of the purposes given above. Sometimes the prevailing circumstances may prelude the requirement to perform all three phases, for example if a new road or utility is to be constructed on a military installation, having a marked vertical and horizontal control networks and up to date topographic maps and utility maps, then the reconnaissance and preliminary survey phases may not necessarily be required .

Roads in Nigeria can be classified in Nigeria can be classified in to three major groups which are:

Federal roads (interstate roads)

State roads (intra state roads)

Local roads (rural roads)

Each class of roads differ in the sense that it control access to different degree, also the amount of traffic that can be safely supported and the speed at which traffic can safely travel. The federal government is responsible for the construction and maintenance of federal roads through its agencies like federal ministry of works; federal ministry of Niger delta affairs, or federal roads maintenance agency (FERMA) State Governments are responsible for the construction and maintenance of intra state roads e.g roads that falls within a state capital or roads that traverse between towns in a state. This is done through their different states ministry of works and transportation or any other agencies established by the state government for this purpose. While Local Government Councils are responsible for the construction and maintenance of rural and street roads. The road under the subject matter here falls within the third category mentioned above i.e.

local government road.

Various types of roads are in use around the world. Roads ranges in size from private driveways, to the stereotypical two – lane highway, to high capacity dual carriageway routes, such as freeways, motorways, and high-quality dual carriageways. They are classified into three: highways, urban streets and rural roads and these differ in location and amount of traffic it can safely support as well as the speed at which the traffic can safely travel.

All over the world, Surveying has been recognized as the ground breaking operation for every environmental, infrastructural and meaningful development. It has developed over years to pave ways for maintenance and sustenance of geophysical landmarks by ways of detecting and forestalling disasters and asserting the tiny line (boundary) whose inconspicuousness can beget feud and animosity between individuals, communities, towns, nations and continents, organization and engineering is unarguably great.

Route surveying is an important aspect of Engineering Survey. It can be defined as a large scale engineering survey for the purpose of obtaining necessary data required by engineers for the planning, designing and setting out of any route such as roads, rail lines, highways, oil pipelines, tunnels, canals, communication and transmissions lines. Route Surveying deals with all the field works and office studies performed in connection with investigation of any route of transportation and the detailed layout of it. Route project, such as those for highways, are so designed as to satisfy geometry criteria with respect to horizontal and vertical alignment. The horizontal alignment of such projects consists of straight lines which are tangential to the curves. Route survey are used by various professionals among whom are geologists, civil engineers, surveyors, town planners, the military and architects to mention but few. It facilitates our movement from one location to another, also, to deliver manufactured and farm products from the production point to the end users. It is the most common mode of transportation unlike other means of transport, air, rail or water due to the possibility of constructing a road where it may be difficult or impossible for an aircraft, train or ship to operate. Other government agencies such as, fire departments, medical services, postal agency, and so on, all depends on good road network for effective service delivery.

The Nigerian road network from the colonial days to the present day have been classify into four categories namely: A, B, C and F. The estimated total road length in 1946 was 11,427km of Trunk A, Trunk B roads and feeder road.  By 1985, road length had increased to 114,800km and as at January, 1996 the road length equalled 193,200km (FMW, 2013), presently the road length in kilometre is in excess of 200,000km out of which 34,000km (17.3%) can be classified under federal or Trunk A roads.  The rest were share amongst the Trunk B, C, and F category respectively.

TRUNK A ROADS: It forms the skeleton of the national road grid that cut across regional boundaries in the country and even extended to the international borders of neighbouring West African countries. These categories of roads are under federal government ownership. They are designed, constructed, maintained and financed by the federal government through the federal ministry of works.  The Federal Roads Maintenance Agency (FERMA) is in charge of carrying out maintenance of this class of roads.  Its total length is about 34,000 (17.3%) of the total road network in Nigeria (FMW, 2013).

TRUNK B ROADS: They are the second category of main road in Nigeria.  They link the major cities within state with the state capitals.  These roads are designed, developed, financed and maintained by various state governments through their ministry of works, transport and infrastructure.  The primary objectives of Trunk B roads are to enhance the socio-economic development of the various states in the country.  It has total length of about 30,000 (15.7%) of the total road network in Nigeria.

TRUNK C ROADS: They are local feeders ‘roads constructed and maintained by the works department of local government authorities in Nigeria.  This class of road are primarily not concrete, asphalted and are affected by seasonal weather changes.  The road link villages and communities in the remote part of each local government region.  It’s about 136,000km and constitute about 67% of the total road network in Nigeria. Also, TRUNK F roads are roads that formerly belong to Trunk B and C categories but later taken over by the Federal Government for direct finance, redesigned, construction and maintenance.

As regards this project, the classes which this study belongs to is TRUNK C. This route is a vital rural road that serves as a links to some villages from Lajolo to Maya Anfeyin Road which traverse the town where about 20 percent of food stuff coming to the city are being cultivated. However, this project was carried out in order to provide baseline survey data for future engineering design, re-alignment and construction of this particular rural road. The longitudinal and cross sections data were acquired at every 25m intervals along the center line of the road and 15m intervals away from center line of the road.

## 1.2  	STATEMENT OF THE PROBLEM

It was discovered that the Route from Lajolo junction to Maya Anfeyin ,Ilorin, Kwara State in Moro Local Government Area, Kwara State, had been badly eroded and full of potholes which is responsible for jams along the road. Rehabilitation of this road should be carried out and certain information about the road must be acquired which calls for route survey so as to get the data necessary for the road.

## 1.3  	SIGNIFICANCE OF THE PROJECT

The project was carried out to provide information (geometric data) for future usage for reconstruction or perhaps the extension of the route which when carried out is expected to decongest traffics and enhance social and economic interaction between the areas it linked. It also availed the opportunity of putting into practice the theoretical knowledge of engineering survey obtained from the classroom

## 1.4  	AIM

The main aim of this project is to carry-out Route from Lajolo junction to Maya Anfeyin ,Ilorin, Kwara State in Moro Local Government

## 1.5  	OBJECTIVES

In actualization of the said aim of the project, the following objectives were followed sequentially:

- Reconnaissance which include office planning and field reconnaissance.

- Identification of existing features and adjoining roads

- Stability of the ground controls to be use for orientation.

- Determination of center line and marking of chainage along the center line at an interval of 25m.

## 1.6  	SCOPES OF THE PROJECT

The scope of the project covered the following operation:

i. 	Reconnaissance    ii. 	Test of instrument iii. 	Control check iv. 	Marking of chainage points at every 25m along the route

- Longitudinal section at 25m interval.

- Detailing of all features (both natural and artificial features) within the right of way.

- Data downloading, Data processing/computation (volume, total length and linear accuracy) and analysis.

- Alignment designs  ix. AutoCAD drafting (Plotting of the vertical coordinates against the horizontal coordinates to obtain the longitudinal profile of the Centre line points) and presentation.

x. 	Report writing.

## 1.7  	PROJECT SPECIFICATION

- The following are the specification to be ascertained in the project:

- Traverse must commence on three coordinated (known) controls and closed on another set of three coordinated controls which must be confirmed undisturbed by necessary measurement (control checks).

- Third order traverse must be run along the route on all turning points at one zero observation and the angular difference from both faces should not be more than thirty seconds (30"), the angular misclosure is determined by 30"√n. Where ‘n’ is the total number of station observed. iv. Establishing traverse points by using pegs together with nails and bottle corks.

v. Leveling must be observed at every 25m intervals on the center line and at 5m intervals on both sides of the center line for the cross sectioning. Edges of drainage at both sides should be heightened. vi. Fixing of relevant features to enhance assessment and necessary composition for good interpretation of plan.

vii. The accuracy of the project must fall within the order of the project.

## 1.8  	PERSONNEL

The following names are the members of HND23 (Group 6B) that was assigned to carry out

this project:

NAMES  	  	  	  	  	  	MATRIC NO.

- DAHUNSI OLAWUNMI COMFORT   	 HND /23/SGI/FT/0069

- AROWOLO NURUDEEN AYINDE   	 HND/23/SGI/FT/0067

- ISAAC JOY SALOMI  	  	  	 HND/23/SGI/FT/0068

- ADELOLU DANIEL ADEWALE  	  	 HND/23/SGI/FT/0070

- OJERINDE ROMOKE HABEEBAT   	 HND/23/SGI/FT/0112

- ABASS KAFAYAT OPEYEMI  	  	 HND /23/SGI/FT/0060

- ABDULLAHI YUSUF SOFIYULLAHI   	 HND/23/SGI/FT/0117

- BAMIGBOSE IDOWU MAYOWA    	 HND/23/SGI/FT/0066

## 1.9 	STUDY AREA

The Road which connect Lajolo to Maya Anfeyin Road in Moro Local government was an ancient road which connect the two towns. The study area is dominated with residential development, other land uses that exist includes agricultural, commercial, industrial among others.

### Geographical Extent and Location of the Study Area

The Study Area is Route from Lajolo junction to Maya/Anfeyin Road, Ilorin Kwara State in Moro Local Government Area. It covers a distance of about 4.5km. The project site lies between latitude (ɸ) range (8˚ 32’ 35.44” to 8˚ 31’ 33.44”N) and longitude (λ) range (4˚ 38’ 06.61”E to 4˚ 38’ 47.19”E) respectively.

The Project Route is verged red (Source: Google Earth Imagery, February, 2024)

FIG 1.1 THE STUDY AREA (NOT TO SCALE)

CHAPTER TWO

LITERATURE REVIEW

## 2.1  	RESEARCH ANALOGY

Surveying has to do with the determination of the relative spatial location of points on or near the surface of the earth. It is the art of measuring horizontal and vertical distances between objects, measuring angles between lines, determining the direction of lines and establishing points by predetermined angular and linear measurements. Distances, angles, directions, locations, elevations, areas and volumes are thus determined from the data of survey. Survey data is portrayed graphically by the construction of maps, profiles, cross sections and diagrams (Oregon Department of Transportation, 2000).

Schofield (2001) defined surveying as the science of determining the position, in three dimension of natural and man-made features on or beneath the surface of the earth. These features may then be represented in analog form as a contoured map, plan or chart, or in digital form as a three-dimensional mathematical model stored in the computer.

There are different operations in surveying namely; Control survey, Boundary survey, Topographical survey, Hydrographic survey and Route survey.

Route survey is therefore defined as the survey done along a comparatively narrow strip of territory for the location, design and construction of any route of transportation. Oregon

Department of Transportation (2000), while presenting a seminar also stated that it refers to those controls, topographical and construction surveys necessary for the location and construction of highways, railroads, canals, transmission lines and pipelines. Route survey is very useful in the determination and establishment of difference in elevation between two or more points. It includes the field and office work required to plan, design and lay out any “long and narrow" transportation facilities. Also, it deals with earthwork which covers the movement of soil or rock from one location to the other for construction purposes. (Harry.L. Field and Michael T. 2006).

The need for surveying as a base for planning and the process of acquiring data of route alignment with special attention on road network, construction and rehabilitation cannot be overemphasized. In view of this, the principle of surveying in general and route surveying in particular is the comprehensive aim of the project.

Transportation being a great function and purpose of route survey is regarded as the fulcrum upon which every other sector of the economy revolves. It is as well the movement of people, goods and services from one place to another, be it on land, water or by air. (Microsoft Encarta Encyclopedia, 2009).

In this wise, transportation has contributed immensely to the economic development of nations in which our dear country (Nigeria) is involved. Here are some benefits derived as a result of good transportation network.

It aids movement of goods and services

It assists in the dissemination of ideas as well as technology

Opening up of new land and abandoned areas

It assists in national integration

Besides, route survey has contributed immensely as touching construction sectors of the engineering surveying.

Engineering surveying which breeds both route and construction survey involves the application of knowledge to the analysis, design and execution of surveying and mapping projects, and the design of land mapping and information system. Mikhail. E (1977) opined that surveyors rely on an understanding of the science of surveying measurements and the surveying principles.

Moreover, it is vividly seen that surveying plays extremely important role in any construction project. Surveying can take many forms, it aids in establishing the location and alignment of highways, buildings, pipes and other man made or cultural projects. (James C. 1985).

A route survey as the name implies is a survey that deals with the route or course that imaginary road or utility line will follow while the end product of a route survey for a highway certainly differs from that of a utility line. The major reason for carrying out route surveying is to facilitate movement of people, yielding to socioeconomic benefits (i.e by determining the best general route between terminals).

Reginal (1968) defined route survey as the topographical and construction survey necessary for the location and construction of lines of transportation or communication such as highways, canals, transmission lines and pipelines. He stated that the location and construction survey may consist of:

Establishing the center line by setting at intervals and running level to determine profile of the ground along the center line.

Taking cross section.

Plotting such profile and fixing grades.

Calculation volume of earthwork.

Measurement of drainage areas.

Laying out structures such as bridges and culvert.

It is a reality that surveyors are the major professionals needed when it comes to working on any engineering project, for example, Building project. They provide special information such as; site location, size of the parcel, the dimension and total area. The finished product (plan) forms the basis upon which further development depends.

The line projected by preliminary surveys, after it has been carefully studied and compared with regards to the cost and operating expenses would be made of the scheme to be adopted. Final location may be performed entirely on the field whereby the surveyors use the gradients shown in the profile as a guide seeking for improvement in the alignment. The detailed work would be performed by traversing (David, 1983).

The execution of route surveying entails some basic processes as far as data acquisition is concerned. These are outlined as follows:

Traversing

Leveling

Detailing

Curve designation

Traverse is a sequence of connected straight lines whose direction and distances have been measured. Traverse is also the process of connecting series of lines with known bearings and distances. It is subdivided into first order for precise measurement and second order for the establishment of secondary controls while the third order is meant for survey of tertiary and topographical features.

Furthermore, route surveying consists of the following sequence of survey:

Reconnaissance of the terrain between the terminals

Preliminary survey over one more location along the general route recommended in the reconnaissance report

Location survey  Construction survey.

Reconnaissance survey: it is a rapid but thorough examination of an area or a stripe of territory between the terminal of the project to determine which of the several possible routes may be worthy of a detailed survey. Reconnaissance survey is the most important of the series of surveys mentioned above. A very thorough and exhaustive examination of the whole area should be made to ensure that no possible route has been overlooked.

Preliminary survey: it is the detailed survey of a strip of territory through which the proposed line is expected to run. The preliminary survey is made of the best several lines of directions investigated previously on the reconnaissance survey. The purpose is to prepare an accurate topographic map of the belt of country along the selected route, and thus arrive at fairly close estimate of the cost of the line/direction surveyed.

Location survey: The location survey is the ground location of the proposed. Line marked on the map. The main purpose of location survey is to make minor improvements on the line as may appear desirable on the ground, and to fix up the final grades. Profile levels are run over the centerline, benchmark is established, and profile which shows the existing ground level and the grade line is attained. Cross section notes are taken in order that the quantity of earth work for filling or cutting may be computed.

Construction surveys:  The purpose of construction survey is to re-establish points, lines   and grades on the ground during construction. It also consists of staking out various details culverts and bridges and in carrying out such other surveying as may be needed for the purpose of construction.

(Schofield 2001).

At the concept and design stage, large scale topographical surveys are produced and other measurements upon which projects are designed. Since this data forms basis to a great extent on the precision and thoroughness with which the survey is carried out.

Profile leveling (longitudinal leveling) is an operation performed to determine the elevation points spaced apart at known distances along given line in order to obtain the accurate outline of the surface of the ground along the line. It is very useful for projects like construction and design of sewer, pipelines etc., and to determine the cut and fill. It is frequently essential to run a longitudinal section along various proposed centerlines and to compare their costs to select a suitable one (Duggal S.k, 2006).

Cross sectioning, according to Duggal (2006), is a leveling operation performed to determine the elevation of the points at right angles on either side of the centerline of the proposed road and radially on the curves. This is done to find out the vertical sections of the surface of the earth on the ground. The detailed information regarding the levels of the ground on either side of the longitudinal section helps in computing the quantity of the earth work. The cross sections are plotted in the same manner as longitudinal sections.

Flying leveling (checking leveling) should be done to connect the bench mark (BM) to the starting point of the work. The records pertaining to profile leveling are entered accordingly along with the cross sectional leveling while the leveling work is in progress. The cross sections are taken perpendicular to the center line of the alignment at some regular intervals. The purpose of cross section is to know the undulation of the ground surface.

According to Anderson and Edward (1985), route survey refers to those controls and construction surveys necessary for the location and construction of line transportation and communication which may include highways, railways, canals, transmission lines, pipelines.

Finally, Route survey involves the determination of ground configuration and location of physical features both natural and artificial along the proposed route, establishing the line on the ground and computing volumes of earthwork involved where applicable (Schofield, 2001).

This kind of survey operation is very important on all road networks and in construction of new road which will increase the durability and carrying capacity of road networks.

According to Surveyors Council of Nigeria (CAP 425) (SURCON) ‘Surveying’ means the art or science of measuring, determining, depicting or representing the dimensions, extent, features or relative position of the earth’s surface.

According to Abubakar (2009), “land is the solid part of the earth’s surface. It is the foundation of all human activities; sources of all materials wealth, from which the earth’s inhabitants get everything needed”. In order words, land is a major source of wealth in all aspects of human endeavor.

The science of surveying is very wide and it cuts across many disciplines. It basically involves precise measurements and computations using sophisticated tools and equipment. This field of science plays an important role in the planning, design, layout and construction of our physical environment. Several works in engineering industry has been carried out by individuals, cooperate bodies, and government agencies for environmental developments. Few of them are mentioned below:

According to the work done by Olamide et al (2019) in their work; route survey of Ilobu to

Erin Ile, a route survey was carried out in the preliminary survey for the design of the Ilobu, Osun

State to Erin-Ile, Kwara State road and the following equipment were employed, Sanding System RTK T9 Dual Frequency GNSS (DGPS), Garmin Handheld GPS78SC, The centre lines of the route were marked at 25m chainages and the route plan was surveyed along the centre lines; the longitudinal section profiles were carried out in 25m chainages while the cross sections were carried out in 50m intervals. RTK (real time kinematics) observation mode was used which provides immediate result. .  According to the work done by Victor et al (2018) in their work; the development of Divine Hectares Estate, Enugu state in Nigeria, a route survey was carried out in the preliminary survey for the design of the estate roads and the following equipment were employed, Hi – Target Dual Frequency GNSS System (DGPS) V30, Garmin Handheld GPS78SC, Hi – Target Total Station ZTS–320R, Hi-Target Automatic Level Instrument. The centre lines of the routes were marked at 10m chainages and the route plan was surveyed along the centre lines; the longitudinal section profiles were carried out in 10m chainages while the cross sections were carried out in 2.5m intervals. The results of the route survey were submitted to a consultant Civil Engineer in AutoCAD format for route design. The cut and fill volume was easily determined based on the difference between the natural route surface and the designed route surface.

Earnest (2015)  used modern method of land surveying techniques for route definition of a road from Eleke Junction to Olorunla Village in Atiba local government area, Oyo. In the execution of the work, he acquired data for both horizontal and vertical alignments with the aid of Total Station.  He processed the data using AutoCad Civil 3d.  The work produced longitudinal and cross-sectional views of the proposed road. All errors obtained are within allowable limit.

Site Surveying Services (2014) carried out a route survey of Accrington area. A full, closed traverse was adopted with laser scanner around the site with many cross checks and then calculated as a networking using Least Squares. Once the topographical survey was completed, a full Laser Scan survey was performed in order to draw the elevations.

Władysław Koc and Cezary Specht (2011), used Sokkia Smart Station system 1200 Smart Rover (with ATX1230 GG antennas) using a GNSS method and a conventional traverse method for route with canopy  to carried out a route survey along a rail route in Poland which has curves and straight edges. The accuracy achieved was better than a convention total station and the application of the results were applied in the design and earth volume computation of the rail.

Sidereal Geoservices limited  (2008)  carried out route survey for the  proposed Lagos Metroline  project  by  the  use  of  Promark  Differential  Global  Positioning  System  (DGPS) receiver, using  stop-and-go  technique  of  data  acquisition.  The project was divided into segments. The first set was for the centerline, the second for cross-sections, and detailing as the third segment.  Each segment was treated as a different layer.  The DGPS data were downloaded, processed, plotted and analyzed in a computer based system. In summary, surveyors who are called to carry out engineering survey must therefore  have  a  thorough  understanding  of  the  accuracy  of  the  job  which  determines  the  method, approach and the instrument to be used.

From the above, it was deduced that most route survey carried out are either by the use of total stations or GNSS methods but the end products is the route survey plan and design, the plan are either given to the cost estimator i.e. quantity surveyor for the road costing or to the civil engineer for design and volume computation. The earthwork volume is one of the most important objectives in horizontal and vertical alignment optimization, so most researches firstly focused on the cut-fill balancing to minimize the cost. There are several methods of calculating earth volume by surveyors and other allied professionals, method such as average-end-method, prismoidal method, grid and triangular prism methods etc. Each method differs in procedures and accuracy but since the advent of software like AutoCAD Civil 3D, AutoCAD Land Development, LSS,  BricsCAD, Site CEM etc earth volume computations are done using the software.  Therefore, some of the volume computation projects were reviewed below.

Aruga et al (2005), developed a forest road design program based on a high-resolution Digital Elevation Model (DEM) from a light detection and ranging (LIDAR) system. After a designer had located the intersection points on a horizontal plane, the model firstly generated the horizontal alignment and the ground prism, and then it could precisely generate cross-sections and accurately calculate earthwork volumes using a high-resolution DEM. A shortage of this model was that it couldn’t properly optimize horizontal and vertical alignments simultaneously.

Akiije (2013) innovate mass haul diagram development for highway earthwork in an electronics office via a general purpose application program approach as compared to tedious conventional hand methodology or using costly specific purpose application programs. The methodology makes use of Microsoft Excel spreadsheet as a tool to develop a graphical representation of mass haul diagram of a portion of road accurately. It showcased an efficient and effective interactive user friendly methodology via Information and Communication Technology. There by providing the most needed convenient means of studying haul, overhaul and for computing payment of a highway earthwork project. Iterations involving the computation of earthwork volume by average end-area method were done speedily and at easy in spite of a large number of design alternatives carried out at regular interval along the alignment.

It is superior for the route cross-section survey to use the radiation opposite side surveying with the function of the total station to obtain the horizontal distance and difference of elevation between two target points, combining with CASS to realize the function of automatically plotting section graph.

Jian-chuan et al (2013); used laser scanning and GPS technology to compute volume of landslide earthwork. They employed these two advanced technology to create the contour after landslide. The original contour of this area could be obtained from the government of the Forestry Bureau. The volume of collapse was estimated by the difference between before and after landslide of terrain contours by the contour method with the Simpson's rule.

Bao (2011) applied DTM on the earthwork calculation of land consolidation where the

MapGIS DTM analysis function was utilized to transform the elevation points into Triangulated Irregular Network (TIN) and fixed the layout elevation by iterative calculation aimed at the balance between cut and fill. This method could also be used in roadway design and volume calculation.

Kerry et al (2012), employed three-dimensional laser scanning to create a surface of the original terrain based on the finite element method. Then this surface was converted to TIN file and earthwork quantities were computed by comparing the TIN of original terrain to that of the finished project. This was only applicable for the computation of the final quantities of roadway.

The method was constrained by the problems such as data size, scanning accuracy and data processing.

Studies show that, it is the duty of surveyors to compute the quantities of earthwork required to construct roadways. In this regards, when the grade line lies above the existing ground surface, embankment (fill) is required; when the grade is lies below the existing ground, excavation (cut) is required. Collecting the topographic data needed for design of the alignment and earthwork volume computations and doing the field work required to stake out the alignment make up the activity called route surveying.

As reviewed above, route surveying design have several methods such as tacheometry,

traversing, aerial methods etc. and  volume computations  such as average-end-method, prismoidal formula and other models improved based on them, are not accurate in theory but practically used in engineering. The real and accurate volume could be got by DTM method, especially with the help of updated CAD software, GIS, GPS and laser scanning technology.

Furthermore, the knowledge of computer and Geographical Information System (GIS) is

widely used in planning, investigation, computation and plotting network analysis. As a result, GIS and advanced survey techniques, air borne and radar of remote sensing and mobile mapping system are used in data acquisition. In other words, route surveying has advanced and is more interesting with the application of computer, GIS, and other advanced survey techniques.

CHAPTER THREE

RESEARCH METHODOLOGY

## 3.1  	RESEARCH DESIGN

This is a vital phase of the project that entails executing the objectives and scope of the project. A well-organized program was designed to speedily and professionally accomplish the aims and objectives of this project. The planning of this project started with meeting with the group members where all the necessary requirements were fully discussed

This is an important aspect of surveying also involves preliminary inspection of the area before the commencement of the actual data acquisition on the project site, for the purpose of planning on how to execute the project as well as fixing stations, etc. Its value prior to the real survey work cannot be over emphasized and it enables the surveyor in obtaining the true picture of the whole area in his mind to work economically in terms of time, energy, labor and of course cost.

## 3.2  	PLANNING

The essence of a good planning cannot be overemphasized. The resources and time spent on planning are never futile and it is the foundation of the project planning. This involved having a general view of the project site in order to have an overall picture of the area and also for locating controls for orientation for execution of the project. It is also said to be rapid approximate survey conducted to examine the area in which the project site occupied.

It involved the use of available records such as plan, topographical map sheet, charts, and other relevant information about the project area. Meanwhile the benefit of the reconnaissance survey can never be overlooked since it enables the surveyor to foreseen the favour, problems, challenges or obstacle that are likely to be encountered during the real survey work.  The two phases of reconnaissance are:

i. Office planning  ii. Field reconnaissance (Data search).

Figure 3.0: Flowchart Showing the Planning Procedure

RECONNAISSANCE

OFFICE / FIELD

DATA ACQUISITION

DATA PROCESSING

DOWNLOADING/

COMPUTATION

PLOTTING

PLAN PRODUCTION

A series of planning, pre-observation and analysis as shown in the flow chart were carried out during the project implementation.

### 3.2.1  OFFICE PLANNING

This involved the collection of relevant materials and information needed for the successful execution of the project at hand. This was done to help in determine the expected accuracy, necessary instrument to be use, choice of scale and personnel. Based on the observations made during field reconnaissance many decisions and conclusion were made. Since no nearby control point for control extension, we decided to set up the instrument on an arbitrary point, due to unfriendly nature of the environment in terms of human and environmental factors; The choice of instrumentation and method of data acquisition was decided and adopted for the project.

The main information that was sourced for in this project were the collection of existing control stations, coordinates and instruments from Department, Surveying and Geoinformatics, Ilorin and test of instruments to ascertain its condition before use. The table 3.1 below shows the coordinate of control stations used for the project.

Table 3.1: Coordinate of the Existing Ground Controls Used

| POINT ID | EASTINGS(m) | NORTHINGS(m) | HEIGHTS (m) |
| --- | --- | --- | --- |
| Point 01 | 679748.116 | 944715.534 | 350.785 |
| Point 02 | 679981.321 | 944665.616 944716.757 | 349.954 348.872 |
| Point 03 | 680215.990 |  |  |

### 3.2.2  RECONNAISSANCE

A reacquaint visit was made to the project site in order to get familiar with the site condition, to access the volume of work to be carried out and to be carried out and to have a general overview of the topography of the route. A recci diagram which indicated the course of the road and points of interest on the road was drawn while carrying out the reconnaissance in order to: (i) 	Ascertain the natural terrain configuration of the route.

- Search for possible controls along or near the route.

- Select the starting point for the project

- Select the traverse of stations and pegging of those stations

- Cutting of lines to make the traverse stations inter-visible.

- Assess of the volume of vehicle traffic on the route.

- Interact with the people in the neighborhood to know the level of security in the project area as well as the names of the adjoining roads.

- Ascertain the possibility of picking details according to the specification from the field reconnaissance

## 3.3  	EQUIPMENT USED

These consist of instruments used in the execution of this project. It equally included both hardware and software used. However, they were categorized into two and given as follows:

3.3.1 	HARDWARE REQUIREMENTS

The hardware components used for this project includes:

- A set of Total Station Sokkia its accessories

- Tripod Stand

- 50m Steel tape   	 	 	 	 	 	 iv. 	3m Pocket tape  	 	 	 	 	 	 	 v. 	Cutlass  	 	 	 	 	 	 	 	 vi. 	Hammer

- Nails with bottle corks

- Writing Materials ix. 	Hp EliteBook laptop computer for data processing

- (A4 Printer) for printing reports.

- An HP Color LaserJet 5550dtn Printer (A3 printer) for printing hard copy plans.

### 3.3.2 SOFTWARE USED

The software components used for this project includes:

i. 	AutoCAD Civil 3D 2021 for plotting of acquired data ii. 	Microsoft office 2016 (Word and Excel) for grouping and formatting downloaded data  iii. 	Notepad for grouping and formatting downloaded data

## 3.4  	TEST OF INSTRUMENT

This is an important exercise in any survey operation, it has to be carried out before embarking on any survey project, and this was done in order to confirm the good working condition or otherwise of the equipment used.

### 3.4.1  TEST OF SOKKIA TOTAL STATION

The total station used for this project was tested for both horizontal collimation and vertical index error.  The following procedures were carried out in testing Sokkia Total Station:

Two points A and B were selected on a relatively level ground and were used as instrument and target station respectively.

- The instrument was set over the first point ‘A’, centered, leveled and focussed, while the target was set over the second point ‘B’.

- The instrument was powered ON and its menu option was navigated through. From the configuration mode screen “Collimation” was selected and the instrument prompted for sighting on face one (F1).

- At this jucnture, the target was bisected on face left of the instrument and enter button was pressed. The instrument again, prompted for sighting on second face (F2).

- The instrument was transited and the target bisected on face right of the instrument and enter button was pressed again. The results of the test were displayed and shown in the table 3.2 below.

Table 3.2: Test of Sokkia Total Station

| Collimation | Horizontal reading | Vertical Reading |
| --- | --- | --- |
| New | +000 00’ 08” | +000 00’ 07” |
| Old | +000 00’ 10’’ | +000 00’ 12’’ |

Source: Field Observation (January, 2025) Expected angular accuracy is:

30”  n     where n is the number of station occupied n=1

30”   1  =  30”

Obtained accuracy is: = +000 00’ 20”

Horizontal Collimation Error = 0o 00’ 08”

Vertical Index Error = 0o 00’ 07”

Allowable Angular accuracy = 30”√n, Where n is the number of station occupied

Therefore, n=1

Allowable Angular accuracy = 30”√1 = 30”

Each of obtained Horizontal Collimation and Vertical Index errors was lesser than the allowable

Angular accuracy of 30”. Therefore, the instrument was suitable for data acquisition

## 3.5 	 DATA ACQUISITION

This has to do with the different field operations carried out and the technology adopted to acquire necessary data (observed facts) about the project site which. Schedules of operations are as follows:

- Third order traversing using total station.

- Detailing using total station the center line at 25m interval and cross section at 5m and 10m on either sides of the center line were determined.

Furthermore, both man-made and natural features were fixed. All the data acquired were fully automated

### 3.5.1 CONTROL CHECK

#### 3.5.1.1   INITIAL CONTROL CHECK

The control check was carried out to ascertain that the controls used for orientation still maintained their position for correct orientation of the project work. The instrument was set up on

Point A and all necessary temporary adjustments were carried out.  The reflector at back station on Point B was bisected, read and recorded. The instrument was turn to fore station Point C and reflector was bisected and also read and recorded.

The difference between computed and observed was deduced and it was within the allowable misclosure because the result obtained is less than expected misclosure as shown below;   Table 3.3: Coordinates of the Starting Control Pillars

| STATIONS | EASTING(m) | NORTHING(m) | HEIGHT(m) |
| --- | --- | --- | --- |
| Point 01 | 679748.116 | 944715.534 | 350.785 |
| Point 02 | 679981.321 | 944665.616 | 349.954 |
| Point 03 | 680215.990 | 944716.757 | 348.872 |

Table 3.4: Back Computation from the Existing Coordinates of the Starting Control Pillars

| From Stn | Bearing   ˮ | DDist(m) | ∆E(m) | ∆N(m) | Eastings (m) | Northings (m) | Height (m) | To Stn |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | 679748.116 | 944715.534 | 3350.785 | Point 01 |
| Point 02 | 282 25 39 | 1238.49m | -233.205 | -- 49.918 | 679981.321 | 944665.616 | 3349.954 | Point 02 |
| Point 03 | 287 42 21 | 2240.18m | -- 234.669 | - 51.141 | 680215.990 | 944716.757 | 3348.872 | Point 03 |

### 3.5.2 TRAVERSING (PROFILING)

The instrument was set up on point B while a target was held vertically on Point A as the back station. Temporary adjustments (Centering, Levelling and Focusing) were carried out on the instrument and the instrument was then powered on. A job was created on the memory of the instrument and ‘coordinate’ option was selected from the menu list. Coordinate of the instrument station was keyed in to the instrument and the height of the instrument and that of reflector were measured and keyed in as well. The coordinate of the back station was also keyed in. having supplied the coordinate of the instrument station, the back station was bisected and the bearing and distance between the instrument station and the back sight were displayed on the display unit of the total station.

The option ‘yes’ was clicked to accept the orientation direction as bisected and the parameters displayed. At this juncture, the instrument had been oriented and observation started. The target was taken to the first chainage of the center line of the route.

Horizontal coordinates of the centerline points at 25m interval were determined along the route. Every point visible form the control points were coordinated before the instrument was shifted to the first temporary point. On the temporary point, the same procedures were repeated until no other point is visible and the instrument station thereby changed. However, the 25m interval was not maintained whenever there is noticeable change in the course of the route and therefore data were acquired along such curve.

### 3.5.3  DETAILING

This was carried out in order to fix and to determine the true position of the natural and artificial features existing along the route. The details were taken along the route by holding the reflector at the edge of detail and turn the telescope of the instrument to bisected the prism of the reflector. After observation has been taken, the coordinate display on the instrument screen is stored in the instrument memory. And adequate Recce diagram was drawn to aid the proper plotting of the detailing. At least three points were picked in detail like buildings. The procedures were repeated at every detail around the route. The features fixed include the uncompleted buildings, the adjoining roads, stream, well and completed building electric poles, trees, fences, etc.

Total station was used to acquire the location data (x,y,z) of the features

## 3.6  	TOTAL STATION DATA DOWNLOADING

The data acquired through Total Station was downloaded using Bluetooth connecting from the total station storage to the laptop.

CHAPTER FOUR

DATA PROCESSING AND RESULT ANALYSIS

## 4.1 	DATA PROCESSING PROCEDURE

This is the process that follows downloading of data into the computer system. It is the manipulation of data into a more used form. Data processing includes numerical calculation, classification of data and the transmission of data from one place to another. Below is the procedure used for data processing:

The file was opened and point data were displayed. This was then copied to a notepad and Microsoft excel environment for further processing.

On notepad, data were well spaced to differentiate one column from the other and unwanted information deleted. On Microsoft excel, data were prepared for scripting purpose in AutoCAD.

## 4.2  	DATA EDITING

Data editing is done using the microsoft excel. The following steps are followed to edit our data in the project:

- The microsoft excel was lunched.

- Click on file, the click on “all file” and select the group data. iii. 	On open “test import wizard”, select “delimitated” and click on next.

- Select comma, tap and space then click on next. All the co-ordinates will be arranged  then click on finish.

- Cut and copy and arrange in its appropriate positions if there is any misclosure

4.3  	PLAN PRODUCTION

Plan and profile production using AutoCAD Civil 3D provides a comprehensive overview of the tools and techniques necessary for creating detailed plans and profiles for civil engineering projects. This typically covers the step-by-step process of generating plan and profile, starting from importing survey data to creating alignments, profiles, and sheets.  The steps are as follows:  1.Creating points

The X, Y, and Z data are used to create point. The data could be in any of the following format:  .pm.

.csv  .xyz

.auf

.nez

.pnt

To import the point, open the tools pace, click on prospector at the right of the toolspace, then right click on points to create points. The create points toolbar will then appear, click on import points to import the data. The import point tool will then appear, click on add files then browse to the folder which Contains that data. Select the data, click ok, then select point format, and click ok. Close all the dialogue and zoom to extent to display the points.

Fig. 4.1:

Picture

showing how to create points

2.

Creating Surface

After the points have been created, the next is to create surface. On the tool

-

space, right click on

surface and select create surface, then ”create surface” dialogue will appear, type the n

ame of the

surface e.g. New Surface and click ok.

Then expand the New Surface, expand “Definition”, then right click on “Point Groups” and click

“Add”, the “Point Group” dialogue will then appear and “All points” and then ok. Now the surface

has been created.

Fig. 4.2: Picture showing how to create surface

### 3. Creating alignment

The next thing after creating the surface is to create alignment. To create alignment, click on

“Home” menu, click on alignment, then “Alignment Creation Tools”, the “Create Alignment

layout” will appear, then name the alignment e.g. Align 1, then click o

k. The "Alignment Layout Tools

-

Align 1" will then appear, select "Tangent

-

Tangent (With Curves)" then zoom to the starting point

and click on the  center points till the last , then press escape and close the "Alignment Layout Tools

-

Align 1" window . Wit

h that, the alignment has been created.

Profile

Once the alignment has been drawn, the profile can be plotted. To do this, click on the  Home ribbon click on “profile” then “create profile from surface”, the create profile from surface window will then appear. Click on “add” to add the surface then click “Draw in profile view”, then ‘Create Profile View – General” will also appear, you can type the name of the profile or accept default, then click next, next, next, next, next, next and then click

“Create Profile View”, you can then select profile view origin which can be any desired space on the drawing page. Now the profile has been plotted.

Fig. 4.3: Picture showing how to create alignment

### Sample line

To create sample lines for the cross section, click on the “sample line” on the home ribbon the select the alignment created earlier, the “create sample line group” window will appear, give it a name of accept default and click ok. On the “sample line tools” window, click on

“sample line creation method” and select “by range of stations”, the " "Create Sample Lines - By Station Range" window will appear, enter the left swath width and right swath width, then press ok and the escape. Now, the sample line has been created.

Once the sample line has been drawn, the cross section can be plotted. To plot the cross section, click on “Section View’ on the home ribbon and then, click “Create Multiple Views” then, the "Create Multiple Section Views - General" window will then appear, you can name it or accept default, then click next, next, next, next, next then click “Create Section Views”, then select any desired empty area on the drawing page for the section view origin. The cross section has been plotted.

## 4.4  	RESULT ANALYSIS

This was done to compare the results obtained with the minimum allowable accepted for the order of survey job according to survey rules and departmental instructions. The analysis of the result table is shown in the appendices.

The route survey traverse started from TBM 1 and closed back on TBM 4

Table 4.1: Abstract of TBM coordinates

| TBM NO. | EASTINGS (m) | NORTHINGS(m) | HEIGHTS(m) |
| --- | --- | --- | --- |
| TBM 1 | 680079.673 | 944607.829 | 350.254 |
| TBM 2 | 680498.890 | 943188.23 | 350.119 |
| TBM 3 | 681274.349 | 942704.360 | 352.702 |
| TBM 4 | 678881.884 | 943974.523 | 324.328 |

CHAPTER FIVE

## COST ESTIMATION, SUMMARY, CONCLUSION AND RECOMMENDATIONS

5.1 	COST ESTIMATION

This stage the total cost spent on the project from day one to the final stage.

Table 5.1.1: Direct cost parameters

| S/N | DIRECT COST |  |  |
| --- | --- | --- | --- |
| 1 | Reconnaissance (1 day) |  |  |
|  | PERSONNEL | Rate | Total Amount (N:K) |
|  | 1 x surveyor | 10500 | 10500 |
|  | 1 x assistant surveyor | 7500 | 7500 |
|  | 1 x technical officer | 6500 | 6500 |
|  | 3 x skilled labour | 4000 | 12000 |
|  | Basic equipment | 5000 | 5000:00 |
|  | Transportation (vehicle, Driver and fuel) | 10000 | 10000:00 |
|  |  | Sub-total | 51,500 |
| 2 | Monumentation (1 day) |  |  |
|  | PERSONNEL |  |  |
|  | 1 x assistant surveyor | 7500 | 7500 |
|  | 1 skilled labour | 4000 | 4000 |
|  | Transportation (vehicle, Driver and fuel) | 10000 | 10000 |
|  | Basic equipment | 5000 | 5000:00 |
|  | Beacon (6) | 1000 | 6000 |
|  |  | Sub-total | 32,500 |
| 3 | Data acquisition (2 days) |  |  |
|  | PERSONNEL |  |  |

|  | 1 x assistant surveyor | 7500 | 7500 |
| --- | --- | --- | --- |
|  | 1 x technical officer | 6500 | 6500 |
|  | 3 x skilled labour | 4000 | 12000 |
|  | Total Station and accessories | 15000 | 15000 |
|  | Transportation (vehicle, Driver and fuel) | 10000 | 10000 |
|  |  | Sub-total | 51,000 |
| 5 | Data Processing (2 days) |  |  |
|  | PERSONNEL |  |  |
|  | 1 x Surveyor | 10500 | 10500 |
|  | 1 x Assistant surveyor | 7500 | 7500 |
|  | 1 x Technical officer | 6500 | 6500 |
|  | Computer / Accessories | 15000 | 15000 |
|  |  | Sub-total | 39,500 |
| 7 | Technical Report (1 day) |  |  |
|  | PERSONNEL |  |  |
|  | 1 x surveyor | 10500 | 10500 |
|  | 1 x assistant surveyor | 7500 | 7500 |

|  | 1 x secretary | 5000 | 5000:00 |
| --- | --- | --- | --- |
|  | Computer / accessories | 15000 | 15000:00 |
|  |  | Sub-total | 38,000 |

TOTAL (DIRECT COST) 212,500 MOBILIZATION/DEMOBILIZATION = 5% of Direct Cost

= 0.05 × 212,500

= N10,625

CONTINGENCY ALLOWANCE

= 5% of Direct Cost

= 0.05 × 212,500

= N10,225

GRAND TOTAL     = N212,500 + N10,625 + N10,625

= N233,750

VAT (@5% GRAND TOTAL)   = 0.05 × 233750

= N11,687.5

TOTAL COST PAYABLE                GRAND TOTAL + VAT

= N233,750+ N11,687.5   	       = N245,437.5

## 5.2  	SUMMARY

The project covered a total length of 4.325km. The field work however involved the following processes; Reconnaissance, Monumentation, Traversing, and Detailing. Sokkia Total Station was used for the data acquisition. The adjusted coordinates were used for the production of the final plans.

## 5.3  	PROBLEM ENCOUNTER

During the cause of the project, the following problems were encountered

- Movement of vehicles along the project site.

- Obstruction of signal by residence of project area

## 5.4  	RECOMMENDATION

Having successfully carried out the project exercise, I hereby recommend the following,

i. The data and results obtained from this project could be used for further construction analysis ii. The results obtained can further be used for more research work along/within the road limit iii. Practical of such should be given to students not for final year project alone as it broaden student knowledge of engineering aspect of surveying

- Application of computer programming should be fully implemented so as to make the student carry out the data processing exercise faster and efficiently

- The school authority should provide adequate equipment especially, digital instruments to the students so as to improve the accuracy and the speed of the project execution

## 5.5  	CONCLUSION

The aim of the project was successfully. The field work, processing and presentation was a success and this was due to proper planning and precaution taken at every stage of the project, and the entire information for the design and construction of the road was obtained within a short period.

At the end of the exercise a vertical and horizontal alignment plan was produced. REFERENCES

Anderson, J. M., & Mikhail, E. M. (2018). Surveying: Theory and Practice. New York: McGraw-Hill.

Baltsavias, E. P. (1999). Airborne laser scanning: Existing systems and firms and other resources.

ISPRS Journal of Photogrammetry and Remote Sensing, 54(2-3), 164-198.

El-Rabbany, A. (2006). Introduction to GPS: The Global Positioning System. Boston: Artech House. Ghilani, C. D., & Wolf, P. R. (2012). Elementary Surveying: An Introduction to Geomatics. Pearson Education.

Ghosh, S. (2013). Introduction to Transport Engineering. Boca Raton: CRC Press.

Kavanagh, B. F., & Bird, S. J. (2014). Surveying: Principles and Applications. Upper Saddle River, NJ: Pearson Prentice Hall.

Longley, P. A., Goodchild, M. F., Maguire, D. J., & Rhind, D. W. (2015). Geographic Information Systems and Science. John Wiley & Sons.

Paul, W. (2011). Transportation Infrastructure Engineering. Springer.

Schofield, W., & Breach, M. (2007). Engineering Surveying. Routledge.

Smith, R., Johnson, D., & Lee, H. (2019). Cost-benefit analysis of precision surveying technologies. Journal of Surveying Engineering, 145(4), 1-15.

Uren, J., & Price, W. F. (2010). Surveying for Engineers. Palgrave Macmillan.

## APPENDICES

NORTHING EASTING    HEIGHT  	 	       NORTHING EASTING     HEIGHT

680057.200 	944604.780 	347.497	680095.520 	943435.790	341.017

680060.610 	944605.900 	347.566	680117.910 	943425.260	342.678

680064.130 	944607.790 	347.687	680116.600 	943423.600	342.592

680070.800 	944590.570 	347.207	680115.170 	943421.820	342.497

680068.170 	944589.670 	347.215	680135.480 	943408.970	343.819

680064.980 	944588.220 	347.224 	680136.910 	943410.520 	343.917

680071.650 	944572.880 	347.781 	680138.880 	943412.520 	344.035

680075.060 	944574.000 	347.925 	680168.600 	943387.870 	345.979

680077.700 	944574.780 	348.045 	680167.730 	943385.330 	345.921

680082.470 	944564.630 	348.800 	680166.200 	943383.220 	345.823

680078.630 	944562.840 	348.623 	680196.350 	943360.230 	345.719

680074.450 	944561.610 	348.342 	680197.670 	943361.670 	345.766

680080.470 	944543.830 	349.877 	680199.310 	943363.780 	345.833

680084.210 	944544.170 	350.369 	680227.140 	943344.770 	345.729

680088.500 	944544.630 	350.909 	680225.600 	943343.210 	345.736

680094.640 	944525.860 	353.391 	680223.190 	943341.320 	345.715

680091.230 	944525.620 	352.883 	680240.090 	943327.460 	346.915

680087.700 	944525.280 	352.370 	680241.410 	943328.570 	346.882

680088.940 	944518.760 	353.083 	680242.940 	943330.230 	346.834

680070.800 	944514.590 	350.893 	680272.120 	943303.920 	347.967

680071.910 	944510.280 	351.441 	680270.480 	943301.820 	347.928

680088.850 	944514.880 	353.378 	680268.280 	943299.590 	347.860

680091.660 	944501.180 	354.617 	680288.400 	943281.980 	348.663

680096.390 	944502.090 	355.175 	680275.680 	943269.540 	347.942

680100.790 	944503.430 	355.724 	680277.230 	943267.120 	348.071

680105.120 	944495.490 	356.046 	680291.160 	943279.780 	348.882

680135.700 	944501.260 	356.748 	680294.580 	943277.700 	349.070

680135.930 	944498.380 	356.654 	680295.560 	943279.360 	349.046

680106.780 	944492.510 	356.071 	680296.880 	943281.800 	348.994

680106.510 	944479.120 	356.017 	680319.170 	943267.960 	350.200

680103.100 	944478.000 	356.000 	680317.420 	943265.740 	350.212

680099.030 	944476.880 	355.709 	680315.670 	943262.970 	350.244

680109.360 	944455.360 	354.881 	680335.650 	943251.550 	350.405

680113.650 	944455.710 	354.984 	680336.960 	943253.540 	350.299

680117.390 	944456.170 	355.082 	680338.050 	943256.090 	350.177

680124.980 	944432.090 	354.328 	680366.200 	943239.510 	350.133

680122.240 	944430.750 	354.194 	680364.990 	943237.400 	350.183

680118.940	944429.080	354.031	680363.240	943234.520	350.263

680131.160	944402.920	353.546	680394.580	943218.840	350.032

680134.570	944404.370	353.738	680395.670	943220.500	350.057

680138.090	944405.380	353.916	680396.870	943223.050	350.097

680148.130	944374.560	353.516	680408.490	943210.710	349.502 680144.390	944372.890	353.151	680408.170	943206.620	349.131

680141.090	944371.550	352.827	680398.170	943202.040	348.874

680148.330	944352.660	352.428	680399.060	943200.170	348.702

680151.300	944353.340	352.752	680414.110	943207.530	349.163

680155.370	944354.460	353.187	680415.210	943209.080	349.305

680162.060	944332.700	354.345	680416.410	943211.190	349.509

680159.650	944331.920	354.139	680429.760	943203.950	348.806

680156.680	944331.350	353.866	680428.670	943202.720	348.680

680162.370 	944311.910 	355.042 	680427.350 	943201.170 	348.530

680165.010 	944312.590 	355.283 	680437.730 	943194.360 	347.865

680167.210 	944313.260 	355.458 	680439.050 	943196.020 	348.019

680176.460 	944288.300 	356.606 	680440.030 	943198.130 	348.231

680173.490 	944287.520 	356.357 	680457.790 	943188.580 	347.669

680170.190 	944286.730 	356.089 	680456.700 	943186.590 	347.462

680187.420 	944245.990 	354.680 	680455.170 	943184.590 	347.319

680190.060 	944246.890 	354.865 	680480.430 	943171.760 	347.632

680193.140 	944247.900 	355.087 	680481.640 	943172.870 	347.694

680208.150 	944213.340 	354.193 	680482.520 	943174.420 	347.775

680204.850 	944212.440 	354.132 	680505.690 	943160.580 	347.355

680201.770 	944211.430 	354.081 	680504.270 	943158.920 	347.241

680204.880 	944180.810 	354.408 	680502.410 	943157.030 	347.115

680207.960 	944180.600 	354.355 	680529.120 	943139.890 	347.873

680210.930 	944180.390 	354.303 	680531.420 	943143.330 	347.891

680201.290 	944169.070 	354.791 	680532.400 	943145.760 	347.898

680197.880 	944169.500 	354.885 	680565.290 	943128.100 	347.896

680194.570 	944170.370 	354.959 	680563.650 	943124.550 	347.993

680179.200 	944159.800 	355.810 	680561.800 	943120.340 	347.691

680181.850 	944157.820 	355.787 	680588.610 	943104.970 	346.714

680184.830 	944156.840 	355.720 	680590.030 	943107.850 	346.899

680185.960 	944125.100 	356.975 	680591.560 	943111.950 	347.168

680183.420 	944125.200 	357.020 	680624.240 	943091.070 	346.846

680181.110 	944125.410 	357.060 	680622.710 	943087.860 	346.796

680177.880 	944083.150 	358.291 	680620.520 	943084.980 	346.726

680181.290 	944082.830 	358.379 	680656.600 	943065.550 	346.156

680185.700 	944082.520 	358.495 	680658.240 	943068.100 	346.186

680192.220 	944050.360 	358.040 	680659.440 	943071.320 	346.255

680189.690 	944050.350 	357.954 	680687.250 	943056.170 	346.440

680185.610 	944050.330 	357.821 	680686.160 	943053.510 	346.576

680194.020 	944014.200 	355.825 	680684.520 	943050.850 	346.681

680197.430 	944014.880 	356.098 	680711.010 	943034.260 	348.573

680201.610	944015.780	356.429	680713.200	943037.480	348.355

680211.890	943979.880	354.320	680715.060	943040.810	348.106

680209.590	943978.870	354.184	680755.020	943015.210	347.394

680206.400	943978.090	354.016	680753.050	943013.320	347.336

680207.210	943966.920	353.442	680750.640	943010.980	347.303 680183.580	943958.410	351.421	680774.370	942996.040	345.985

680184.590	943953.660	351.240	680776.340	942998.480	346.030

680210.100	943961.840	353.325	680777.660	943000.590	346.074

680217.150	943935.220	353.679	680799.300	942985.200	345.112

680219.890	943936.450	353.883	680797.990	942983.420	345.045

680224.180	943937.570	354.204	680796.120	942981.540	345.000

680240.580	943888.090	356.371	680817.890	942962.490	343.314

680237.060	943887.190	356.259	680819.430	942964.600	343.274

680234.310 	943885.850 	356.172 	680820.630 	942966.930 	343.282

680247.270 	943841.560 	355.148 	680841.190 	942946.220 	340.531

680250.680 	943842.900 	355.272 	680842.940 	942949.000 	340.541

680254.310 	943843.580 	355.364 	680843.810 	942951.430 	340.648

680264.900 	943813.210 	355.743 	680871.820 	942940.820 	339.949

680261.270 	943811.870 	355.668 	680870.520 	942937.720 	339.751

680258.520 	943810.530 	355.613 	680869.650 	942934.290 	339.527

680269.250 	943773.410 	356.614 	680896.810 	942916.600 	339.236

680273.210 	943773.650 	356.523 	680898.890 	942919.820 	339.607

680276.400 	943774.330 	356.456 	680901.520 	942922.810 	340.001

680282.850 	943732.770 	354.815 	680913.490 	942904.170 	338.329

680279.330 	943731.760 	354.810 	680929.890 	942878.030 	335.986

680274.600 	943731.410 	354.872 	680934.450 	942867.540 	335.983

680287.080 	943696.180 	352.907 	680939.070 	942868.340 	336.066

680290.050 	943696.520 	353.013 	680931.280 	942889.210 	336.862

680293.020 	943696.750 	353.270 	680934.120 	942892.760 	337.206

680302.630 	943663.390 	354.506 	680937.080 	942896.200 	337.563

680300.100 	943662.610 	354.415 	680964.000 	942884.150 	337.170

680298.020 	943661.270 	354.386 	680963.130 	942881.710 	336.976

680311.800 	943630.140 	356.517 	680962.590 	942879.500 	336.855

680315.100 	943630.600 	356.495 	680990.050 	942867.890 	337.360

680317.190 	943631.270 	356.468 	680990.810 	942869.660 	337.446

680326.800 	943597.800 	356.436 	680991.800 	942871.320 	337.532

680325.150 	943597.460 	356.424 	681021.450 	942862.930 	339.024

680323.060 	943597.120 	356.414 	681020.910 	942860.940 	338.927

680333.780 	943562.220 	354.855 	681020.250 	942859.940 	338.847

680335.980 	943563.110 	354.946 	681053.690 	942840.510 	338.274

680338.180 	943563.010 	354.992 	681055.010 	942842.060 	338.411

680355.820 	943530.350 	355.667 	681057.420 	942843.840 	338.578

680352.860 	943528.340 	355.803 	681065.750 	942828.620 	337.591

680349.790 	943526.340 	355.716 	681064.430 	942827.280 	337.505

680360.920 	943522.290 	355.698 	681062.230 	942826.060 	337.427

680376.220	943524.350	355.195	681072.730	942817.030	336.837

680376.230	943520.920	355.306	681074.160	942817.920	336.895

680363.360	943518.650	355.643	681074.920	942819.140	336.971

680373.330	943504.100	355.986	681088.050	942814.110	336.642

680370.140	943502.650	355.933	681086.950	942812.230	336.521 680367.180	943500.860	355.863	681085.860	942810.340	336.396

680389.230	943466.670	356.517	681104.150	942807.540	335.511

680391.210	943467.680	356.641	681104.030	942808.980	335.634

680393.410	943468.680	356.773	681104.250	942810.860	335.776

680409.470	943445.410	356.675	681129.570	942810.640	334.676

680406.510	943443.630	356.364	681129.140	942808.540	334.461

680404.860	943441.740	356.134	681128.820	942805.880	334.197

680410.630	943432.370	355.897	681153.940	942801.670	332.702

680410.590 	943416.330 	354.891 	681154.270 	942802.780 	332.853

680403.810 	943404.470 	353.904 	681155.140 	942804.440 	333.063

680405.360 	943401.710 	353.803 	681183.240 	942797.930 	332.880

680412.470 	943413.020 	354.773 	681182.590 	942796.600 	332.740

680419.390 	943418.140 	355.260 	681182.150 	942795.380 	332.616

680422.020 	943419.690 	355.419 	681188.670 	942789.100 	332.229

680425.100 	943421.480 	355.623 	681190.650 	942789.660 	332.355

680437.880 	943392.220 	353.514 	681192.630 	942790.560 	332.479

680435.020 	943390.990 	353.467 	681198.080 	942776.980 	331.185

680432.170 	943389.650 	353.429 	681196.660 	942775.860 	330.989

680451.020 	943357.330 	351.681 	681194.240 	942775.520 	330.837

680453.880 	943359.110 	351.759 	681201.930 	942754.100 	328.977

680456.180 	943360.670 	351.816 	681203.460 	942755.100 	329.159

680470.040 	943338.940 	351.850 	681205.440 	942755.550 	329.324

680481.350 	943345.950 	352.633 	681210.320 	942747.060 	328.804

680483.010 	943343.750 	352.795 	681209.230 	942745.500 	328.598

680473.360 	943334.860 	352.093 	681207.910 	942743.730 	328.365

680465.880 	943331.730 	351.949 	681214.320 	942738.450 	327.985

680457.480 	943314.660 	352.223 	681221.300 	942729.520 	327.126

680455.370 	943318.080 	352.048 	681222.280 	942730.630 	327.220

680463.770 	943336.040 	351.741 	681223.710 	942732.180 	327.353

680481.010 	943296.400 	353.116 	681234.410 	942726.360 	326.957

680484.530 	943297.410 	353.195 	681233.650 	942724.480 	326.916

680487.830 	943297.870 	353.270 	681232.780 	942722.380 	326.870

680491.520 	943258.620 	352.706 	681247.980 	942720.340 	326.507

680493.710 	943259.410 	352.760 	681248.300 	942721.780 	326.548

680495.690 	943260.080 	352.807 	681248.300 	942723.550 	326.616

680506.070 	943227.830 	352.367 	681251.940 	942721.350 	326.389

680503.980 	943227.710 	352.254 	681257.740 	942702.910 	325.246

680501.010 	943227.030 	352.074 	681261.150 	942703.470 	325.072

680508.510 	943223.750 	352.277 	681256.450 	942722.260 	326.249

680509.080 	943218.770 	352.159 	681267.010 	942724.510 	325.967

680525.720	943216.410	352.622	681267.110	942726.060	326.056

680525.390	943214.530	352.429	681267.330	942727.830	326.150

680511.890	943206.060	351.163	681286.380	942725.700	326.200

680509.360	943205.280	351.010	681286.060	942724.260	326.063

680505.840	943204.600	350.816	681285.630	942721.940	325.832 680512.900	943175.320	348.532	681311.670	942709.330	324.348

680514.880	943176.210	348.656	681311.880	942711.540	324.650

680517.300	943176.890	348.781	681311.980	942713.980	324.984

680521.140	943153.020	347.522	680061.040	944608.670	350.745

680516.140	943162.730	347.814	680077.180	944567.920	351.276

680521.420	943163.860	348.063	680088.210	944535.790	354.613

678832.000	944021.720	324.000	680096.920	944507.400	358.048

678835.410	944023.720	324.000	680102.180	944486.630	358.920

678839.800 	944025.510 	324.000 	680111.880 	944459.020 	358.135

678850.970 	944013.720 	324.000 	680125.470 	944421.800 	357.007

678846.910 	944011.270 	324.000 	680139.490 	944386.800 	356.470

678844.280 	944008.280 	324.000 	680151.850 	944353.790 	355.824

678869.000 	943993.890 	322.570 	680162.220 	944322.750 	357.683

678871.190 	943997.110 	322.460 	680173.590 	944289.400 	359.490

678873.270 	943999.330 	322.350 	680184.060 	944259.910 	358.249

678908.810 	943978.020 	320.126 	680192.750 	944236.280 	357.346

678906.740 	943973.150 	320.313 	680201.650 	944214.090 	357.095

678904.340 	943968.930 	320.480 	680209.260 	944185.250 	357.244

678945.340 	943956.830 	320.283 	680202.270 	944171.290 	357.704

678946.320 	943959.490 	320.374 	680185.240 	944163.360 	358.497

678947.740 	943962.700 	320.501 	680181.960 	944132.930 	359.662

678982.030 	943949.800 	319.503 	680183.460 	944089.810 	361.305

678981.160 	943947.580 	319.255 	680184.470 	944060.500 	361.119

678979.860 	943944.260 	318.992 	680250.120 	943845.330 	358.362

679009.190 	943932.550 	319.259 	680272.440 	943772.650 	359.545

679010.390 	943934.430 	319.454 	680278.550 	943734.410 	358.053

679010.820 	943937.090 	319.661 	680303.000 	943654.660 	357.968

679038.850 	943921.060 	319.506 	680324.640 	943588.610 	359.134

679037.330 	943918.070 	319.182 	680354.520 	943526.030 	358.826

679035.570 	943915.510 	318.970 	680421.680 	943422.460 	358.622

679040.610 	943922.060 	319.646 	680467.300 	943336.160 	354.850

679047.840 	943931.160 	320.610 	680480.640 	943304.700 	356.159

679049.940 	943928.850 	320.457 	680486.230 	943284.700 	356.080

679042.490 	943919.970 	319.514 	680491.940 	943261.720 	355.761

679086.440 	943889.630 	318.663 	680496.190 	943246.040 	355.630

679084.580 	943885.970 	318.168 	680501.440 	943229.470 	355.189

679081.070 	943883.410 	317.608 	680506.360 	943212.230 	354.588

679102.690 	943872.330 	317.194 	680511.960 	943187.820 	352.384

679104.890 	943874.330 	317.686 	680517.890 	943165.610 	351.062

679107.080 	943877.000 	318.248 	680520.810 	943152.020 	350.527

679117.510	943883.790	319.985	678835.840	944025.600	324.000

679120.490	943881.590	319.882	678853.200	944007.540	323.872

679110.390	943874.910	318.195	678874.510	943994.030	322.126

679133.270	943854.100	317.021	678896.140	943981.180	320.646

679130.410	943851.880	316.720	678918.640	943971.540	320.325 679126.900	943848.880	316.295	678942.570	943961.460	320.463

679163.460	943822.600	314.825	678969.370	943950.960	319.463

679165.760	943825.370	315.003	678997.710	943940.790	319.341

679167.730	943828.260	315.259	679016.800	943930.480	319.465

679184.180	943815.720	314.366	679032.030	943920.140	319.201

679195.140	943828.920	314.757	679053.670	943906.630	318.854

679199.000	943826.290	314.351	679073.320	943893.770	318.216

679187.830	943813.740	314.236	679094.740	943879.160	317.730

679201.430 	943799.530 	313.891 	679114.180 	943864.530 	316.989

679200.440 	943797.430 	313.819 	679134.720 	943848.580 	316.533

679198.580 	943795.320 	313.671 	679153.060 	943834.940 	315.783

679259.070 	943757.970 	319.000 	679176.690 	943816.900 	314.447

679263.570 	943762.850 	319.000 	679199.560 	943797.640 	313.772

679266.740 	943766.850 	318.797 	679228.150 	943779.400 	316.087

679334.370 	943734.280 	319.750 	679261.260 	943761.630 	319.000

679332.080 	943729.300 	319.731 	679296.000 	943747.950 	319.000

679330.340 	943722.540 	319.764 	679326.230 	943731.930 	319.527

679385.840 	943695.680 	319.409 	679359.660 	943715.810 	319.417

679389.230 	943700.120 	319.562 	679391.990 	943698.030 	319.688

679392.730 	943704.560 	319.664 	679424.210 	943680.690 	321.947

679396.120 	943710.770 	319.775 	679439.000 	943672.680 	323.009

679404.380 	943735.690 	319.999 	679465.940 	943653.330 	324.268

679400.970 	943735.670 	319.989 	679493.440 	943633.200 	324.631

679394.020 	943713.410 	319.709 	679516.850 	943617.160 	325.775

679388.100 	943706.750 	319.514 	679547.870 	943598.480 	327.038

679435.670 	943679.410 	322.585 	679594.630 	943578.880 	331.000

679434.360 	943675.540 	322.827 	679618.700 	943562.510 	330.967

679432.290 	943671.430 	322.953 	679643.750 	943549.450 	330.190

679462.640 	943653.760 	324.217 	679664.500 	943536.600 	330.467

679463.850 	943654.870 	324.171 	679691.520 	943526.100 	330.481

679465.270 	943656.090 	324.095 	679722.620 	943513.730 	330.863

679491.330 	943637.180 	324.440 	679753.490 	943503.680 	332.532

679489.800 	943635.400 	324.582 	679787.980 	943495.540 	334.201

679488.050 	943633.510 	324.736 	679814.430 	943489.120 	335.491

679518.850 	943613.850 	325.918 	679850.580 	943481.090 	336.783

679520.710 	943615.620 	325.852 	679883.180 	943478.020 	338.155

679522.020 	943617.510 	325.790 	679918.640 	943476.290 	339.749

679556.900 	943597.530 	327.514 	679943.300 	943477.060 	339.677

679555.040 	943594.200 	327.508 	679968.840 	943478.050 	339.859

679553.080 	943590.100 	327.522 	680004.080 	943474.000 	340.521

679598.620	943572.150	331.000	680034.320	943458.530	339.905

679600.600	943574.260	331.000	680077.330	943441.350	339.418

679602.240	943576.930	331.000	680132.260	943415.260	343.611

679630.490	943560.680	330.580	680175.910	943379.170	346.077

679629.400	943558.240	330.618	680216.670	943347.150	345.524 679627.980	943555.800	330.662	680232.670	943337.050	346.153

679653.580	943540.310	330.344	680272.360	943299.390	347.930

679655.120	943542.090	330.286	680292.470	943281.010	348.893

679656.760	943543.870	330.230	680329.900	943258.050	350.380

679686.100	943531.490	330.488	680374.380	943231.800	350.147

679684.790	943528.830	330.584	680411.570	943211.390	349.553

679683.260	943526.170	330.697	680458.910	943185.160	347.447

679721.520	943513.170	330.860	680496.980	943164.300	347.509

679722.610 	943514.720 	330.797 	680520.920 	943152.020 	347.532

679723.600 	943516.610 	330.708 	680541.250 	943137.170 	347.942

679730.760 	943514.210 	331.100 	680568.720 	943122.140 	347.835

679736.650 	943526.950 	330.887 	680591.360 	943107.520 	346.878

679739.520 	943526.520 	330.979 	680613.880 	943092.800 	346.511

679734.950 	943513.450 	331.289 	680636.490 	943083.270 	346.912

679750.390 	943506.550 	332.249 	680658.350 	943068.990 	346.208

679749.410 	943504.660 	332.334 	680680.540 	943055.700 	346.354

679748.540 	943502.340 	332.456 	680703.390 	943041.420 	347.766

679783.030 	943493.970 	334.139 	680719.590 	943036.510 	348.632

679784.240 	943496.070 	334.045 	680740.790 	943023.000 	348.016

679785.430 	943499.170 	333.880 	680765.510 	943009.280 	346.961

679828.750 	943488.630 	335.992 	680778.440 	942998.050 	345.957

679828.320 	943486.530 	336.111 	680799.760 	942981.000 	344.885

679827.770 	943484.310 	336.240 	680813.580 	942968.010 	343.822

679836.680 	943487.780 	336.168 	680831.230 	942958.900 	342.116

679843.120 	943500.630 	335.662 	680849.680 	942943.830 	340.153

679845.100 	943500.310 	335.674 	680875.700 	942934.760 	339.645

679838.660 	943487.120 	336.222 	680902.200 	942916.730 	339.455

679855.300 	943483.210 	336.634 	680915.010 	942907.610 	338.657

679854.320 	943480.770 	336.845 	680922.200 	942899.670 	337.881

679853.560 	943478.670 	337.023 	680929.870 	942883.560 	336.310

679882.200 	943475.580 	338.390 	680942.980 	942854.750 	336.039

679882.630 	943477.580 	338.177 	680947.540 	942844.370 	335.648

679883.500 	943480.790 	337.834 	680949.890 	942888.620 	337.142

679913.230 	943479.810 	339.243 	680975.590 	942875.900 	337.153

679912.910 	943476.710 	339.506 	680998.420 	942867.700 	337.677

679912.480 	943473.940 	339.742 	681022.450 	942860.400 	339.007

679955.190 	943475.560 	339.520 	681041.770 	942848.750 	338.854

679955.190 	943477.330 	339.578 	681054.010 	942844.160 	338.554

679955.170 	943479.980 	339.668 	681063.650 	942828.390 	337.574

679990.850 	943478.260 	340.371 	681068.640 	942822.320 	337.183

679990.530	943475.710	340.192	681076.250	942817.380	336.859

679990.220	943472.720	339.990	681086.180	942813.220	336.586

680002.120	943470.230	340.214	681101.940	942809.970	335.802

679995.740	943441.670	339.137	681117.030	942808.480	335.008

679998.940	943440.570	339.163	681133.880	942807.670	334.155 680007.520	943468.700	340.286	681159.440	942802.140	332.863

679999.330	943478.840	340.682	681178.730	942798.240	332.825

680000.670	943499.200	342.053	681186.560	942794.840	332.662

680006.630	943496.570	342.078	681191.320	942789.330	332.346

680004.400	943477.430	340.754	681197.890	942771.670	330.572

680030.440	943463.600	340.164	681203.250	942753.770	329.031

680029.130	943461.160	340.043	681208.680	942745.610	328.577

680027.710	943457.950	339.893	681218.750	942733.600	327.499

680047.780 	943451.070 	339.405 	681227.580 	942727.110 	326.983

680048.320 	943452.840 	339.523 	681239.050 	942722.620 	326.838

680048.860 	943454.720 	339.647 	681248.850 	942722.220 	326.544

680073.010 	943446.310 	339.478 	681260.400 	942725.150 	326.243

680072.360 	943444.980 	339.421 	681271.630 	942726.410 	325.916

680071.260 	943443.420 	339.342 	681283.960 	942725.470 	326.120

680093.990 	943432.680 	340.756 	681295.000 	942719.320 	325.713

680094.750 	943434.230 	340.891 	681311.100 	942712.200 	324.758