CHAPTER THREE

3.0				 PROJECT METHODOLOGY

It details the selection of materials, machine components, design principles, and engineering calculations employed to ensure functionality, efficiency, and safety. The focus is on a shredding-type recycling system that transforms fabric waste into shredded material suitable for reuse.

3.1	Materials Selection

The selection of materials for fabrication of a Portable Fabric Recycling Machine is crucial to ensure the durability, performance, safety, and cost-effectiveness of the portable fabric recycling machine. The materials were chosen based on mechanical properties such as tensile strength, toughness, wear resistance, corrosion resistance, ease of fabrication, and availability in the local market. The materials selected are as follows for each component’s parts of the machine.

- Frame: Mild steel (angle iron)

- Blades: Hardened carbon steel

- Motor: 1 HP electric motor

- Bearings: Pillow block bearings

- Belt and pulley system for power transmission

Table 3.1 Detailed Material Selection of the Machine Components

| S/N | Component | Material Selected | Reasons for Selection |
| --- | --- | --- | --- |
| 1 | Machine Frame | Mild Steel (Angle Iron) | High structural strength, weldable, cost-effective, widely available |
| 2 | Shredding Blades | High Carbon Tool Steel | High hardness, excellent wear resistance, retains sharp edges |
| 3 | Rotating Shaft | Medium Carbon Steel | Good torsional strength, machinability, affordable |
| 4 | Hopper and covers | Galvanized Mild Steel Sheet | Corrosion-resistant, lightweight, easy to fabricate |
| 5 | Motor Mount Base | Mild Steel Plate (5 mm) | Strong and rigid to support motor vibration and weight |
| 6 | Bearings | Chrome Steel (Standard Ball Bearings) | Low friction, supports rotational loads efficiently |
| 7 | Pulley System | Cast Aluminum Alloy | Lightweight, corrosion-resistant, good mechanical properties |
| 8 | Belt | Neoprene Rubber (V-Belt) | High flexibility, good grip, long life span |
| 9 | Output Tray | Plastic/Aluminum Sheet | Lightweight and corrosion-resistant for easy handling of shredded fabric |
| 10 | Fasteners (bolts, nuts) | Stainless Steel | Corrosion-resistant and strong for structural integrity |
| 11 | Electrical Casing | PVC or Bakelite | Electrical insulation, impact resistance |

| 3.2 Machine Description The machine consists of a rectangular frame, a feeding hopper, rotating blade assembly mounted on a shaft, and a 1HP motor that drives the shaft via a pulley-belt system. The shredded fabric exits into a collection bin. Safety covers and emergency stop features are included for operator protection. 3.3 Materials and Component Used The components parts of the machine, material used and reason for selection of each material were listed in Table 3.2 below: Table 3.2 Materials and Components Used S/N; Component; Material; Reason for Selection. 1; Frame; Mild Steel (Angle Iron); Good strength, weldability, cost-effective. 2; Shaft; Medium Carbon Steel; High torsional strength, machinable. 3; Shredding Blades; Hardened Tool Steel; Wear resistance, edge retention. 4; Hopper; Galvanized Sheet Metal; Corrosion resistance, lightweight. 5; Motor (1 HP, 1440 RPM); Electric AC Motor; Sufficient torque for fabric shredding. 6; Bearings; Deep Groove Ball Bearing; For smooth shaft rotation. 7; Pulley and Belt System; Cast Aluminum + V-Belt; Power transmission, speed reduction. 8; Output Bin; Plastic/Metal Tray; Lightweight, easy removal of shredded material. 3.4 Design Considerations The following parameters are considered during the design stage of the machine components. It includes: Blade type and geometry for clean cutting; Torque and power required for cutting fabrics; Rotational speed for efficient shredding; Structural integrity of the frame; Safety and operator ergonomics and Portability and ease of maintenance 3.5 Design Calculations 3.5.1 Torque Required (T) Estimated force to shred medium-weight fabric = 80 N based on empirical cutting tests) Effective radius of blade = 0.075 m T=F × r (Kuznetsov and Severina, 2012) = 80 N×0.075 m=6 Nm According to (Bhandari, 2010) ,adding safety factor of 2: Tdesign= 6×2 = 12 Nm 3.5.2 Power Requirement (P) Given: - Torque - Angular velocity - Torque T=12 Nm - Angular velocity ω=2πN /60 where N=1440 RPMN ω=2π×1440/60=150.8 rad/s P= T × ω= 12×150.8 =1809.6 W ≈1.81 kW Since this exceeds 1 HP, we use belt speed reduction to lower blade speed and torque requirement. Final selected motor: 1 HP (0.75 kW) with a pulley ratio of 2:1, which reduces shaft RPM to 720 RPM, making it sufficient for shredding after mechanical advantage (Khurmi & Gupta, 2005). 3.5.3 Blade Speed Blade Revolution per minute RPM =1440/2=720 RPM This speed provides enough shear action for effective fabric shredding without overheating or jamming (Kuznetsov and Severina, 2012). 3.5.4 Shaft Diameter Design Using maximum torque T=12 Nm, allowable shear stress for mild steel (Khurmi and Gupta, 2005). T=12Nm, Allowable shear stress for mild steel τ=40 MPa Torsion formula: T = π/16 x τd3 (Bhandari, 2010). d3=16T/πτ d3=16×12/π×40×106 = 1.53×10−6 d= 0.0117 m≈12 mm Selected shaft diameter = 20 mm (for added safety and load-bearing) (Bhandari, 2010). 3.5.5 Frame Strength Check Maximum load = 100 kg = 981 N Cross-section of frame = 40 mm × 40 mm mild steel angle (Bhandari, 2010). Yield strength of mild steel = 250 MPa (Khurmi and Gupta, 2005) Allowable stress = 140 MPa (with safety factor) Stress: σ = F/A= 981/(40×2×3) = 4.09 MPa 3.6 Blade Design - Material: Tool steel (HRC ~60) - Blade thickness: 8 mm - Blade length: 100 mm - Cutting force (empirical): 80 N per fabric strip - Number of blades per shaft: 4 3.6.1 Shear Force on Blade: Let’s assume the blade shears fabric across its width. Τ = F/A = 80/(100×8)=0.1 MPa Tool steel shear strength ≈ 600 MPa, Safe according to Khurmi and Gupta (2005) 3.6.2. Shaft Design (Torsion) Torque T=12 Nm Allowable shear stress, τ = 40 MPa (for mild steel) T= πτd3/16 ( Bhandari , 2010) d3=16T/πτ d3=16×12/π×40×106 = 1.53×10−6 d≈0.0117 m≈12 mm Selected shaft diameter = 20 mm (with SF = 2) 3.6.3. Pulley and Belt System - Motor RPM = 1440 - Desired shaft RPM = 720 - Pulley ratio = 14407/20=2:1 If motor pulley diameter = 100 mm, then driven pulley = 200 mm. 3.6.3.1 Belt Length: L=2C+π/2(D + d) + (D−d)2//4C (Khurmi and Gupta ,2005); (Shigley ,2004) Where: - C = 300mm (centre distance) - D = 200mm, d=100 mm, d =100mm L= 600+3.142/(300)+1002/1200 ≈600+471+8.3=1079.3 mm3 Belt Length = ~1080 mm 3.6.4 Bearing Selection Load on shaft = ~100 N Required life: 3 years at 4 hrs/day = ~4000 hrs Shaft RPM = 720 Dynamic Load Rating: L10=(C/P)3×106 Shigley and Mischke (2004) C = P × (L10/106)1/3 P = 100 N, L10= 720×4000×60 =1.73×108 rev C = 100 × (1.73×108/106)1/3 ≈100 × (173)1/3≈100×5.55=555 N Select ball bearing with dynamic rating ≥ 600 N 3.6.5 Hopper Design - Hopper height = 300 mm - Top opening: 250 mm × 250 mm - Bottom discharge: 100 mm × 100 mm Volume: V=1/3×h×(A1+A1⋅A2 +A2) (Bhandari 2010) A1 = 250×250=62500 mm2, A2=10000 mm2 V=1/3×300× (62500+25000+10000) = 1/3×300×97500=9,750,000 mm3=9.75 litres 3.6.6. Frame Design The total weight on frame = 60 kg → Load = 600 N Material: Mild steel angle iron (40×40×4 mm) Allowable stress = 140 MPa Stress check: σ = FA = 60040 × 4 =3.75 MPa≪140 MPa Frame is structurally safe. |  |  |  |
| --- | --- | --- | --- |
| - Material Specifications (Sample) 3.7.1 Mild Steel (Frame Material): - Yield Strength: 250 MPa - Density: 7850 kg/m³ - Welding and cutting friendly - Tool Steel (Blade Material): - Hardness: ~60 HRC - Tensile Strength: ~800–1000 MPa - Heat-treatable for edge retention - Medium Carbon Steel (Shaft): Tensile Strength: ~600 MPa Torsional Strength: Medium to High Machinability is Good 3.7.4 Aluminum Alloy (Pulley): Density: 2700 kg/m³ Non-rusting, good for moving parts |  |  |  |