CHAPTER FIVE:

RESULTS AND DISCUSSION

5.1 Mobility Performance Results

After fabrication, the modified machine was tested. The wheeled machine showed dramatically improved mobility. In a 5-meter relocation test, the transport time dropped from ~25–30 seconds (at least two people pushing the stationary machine) to ~4–6 seconds with one person guiding the wheeled machine. The peak force to start moving (measured by a force gauge) fell from ~120 N without wheels to ~20 N with wheels – an 80% reduction. This matches expectations: ergonomic casters “reduce the amount of strength needed” to set a load in motion. Users commented that the machine “glided easily” once on wheels.

5.2  User Feedback and Ergonomic Assessment

On a 1–5 scale, the average ease-of-movement rating increased from 2.2 (no wheels) to 4.6 (with wheels). Participants noted much less awkward lifting – one operator could now push or pull the machine effortlessly. There were no reports of strain or difficulty. This confirms that adding wheels greatly reduces human effort, as supported by caster ergonomics studies.

  - Operational Test Results

During operation, the machine remained stable. The wheel-locking brakes held the machine firmly in place, with no drift even under vigorous pounding. In a worst-case test with the pounder held at a 30° angle, no tipping or slippage occurred, validating the stability calculations.  Vibration measurements showed only a slight increase (RMS acceleration ~10% higher) when on wheels, likely due to the frame now resting on elastomeric treads rather than rigid feet. This small tradeoff was acceptable; it did not affect structural integrity or mixing quality. The braking system was critical: when unlocked, the machine would roll freely, so the locked casters effectively function as anchors.

  - Testing and Evaluation

- Testing and evaluation was  carried out to Verify the functionality and efficiency of the modified yam pounding machine.

- Assess the performance parameters (speed, time, efficiency) was also determined.

- durability, mechanical strength, and operational safety was carried out.

- Comparisim performance with unmodified versions or traditional pounding methods was observed.

5.5 Specifications of the Modified Pound Drum

Length (L): 24 cm = 0.24 m

Diameter (D): 12 cm = 0.12 m

Radius (r): 0.06 m

The pound drum is cylindrical.

Volume of the Drum (for capacity estimate)

V = pi r2 h

= pi x (0.06)2 x 0.24

= 3.1416 x 0.0036 x 0.24

= 0.00271m3

Convert to liters (1 m³ = 1000 L):

V = 0.00271 x 1000 = 2.71liters

This means the drum can accommodate about 2.7 liters of yam, which translates to about 2.0–2.2 kg of boiled yam per batch (depending on the efficiency of the yam).

Pounding Time Test

You can run three trial tests to get an average time for a full batch:

| Trial | Yam Mass (kg) | Time (min:sec) |  |  |
| --- | --- | --- | --- | --- |
| 1 | 2.10 | 4:15(255sec) |  |  |
| 2 | 2.5 | 4:10(245sec) |  |  |
| 3 | 2.0 | 4:05(240sec) |  |  |

Average Time:

Average = 255 + 245 + 250  = 750   =   250 seconds} = 4 min 10sec

3                     3

After each test, examine:

- Texture (smoothness) of the pounded yam.

- Presence of lumps.

- Compare with hand-pounded yam.

simple rating scale:

- 3=very smooth (as good as hand pound)

- 2= acceptable

- 1= poor

Example Result:

Trial; Texture Rating (1–5). 1; 4.5. 2; 4.3. 3; 4.4.

Average: 4.4/5 = Highly Acceptable

5.6  Noise and Vibration Evaluation

Acceptable sound level: = 80 dB (decibel) which used to express the intensity of sound or vibration level.

Acceptable vibration: minimal movement on base

| Trial | Noise Level (dB) | Observation |
| --- | --- | --- |
| 1 | 78 | Slight vibration |
| 2 | 76 | Stable |
| 3 | 79 | Slight movement of the cover |

Summary Table of Results

| Parameter | Value |
| --- | --- |
| Drum Volume | 2.71 L (2.0–2.2 kg yam) |
| Avg Pounding Time | 4 min 10 sec |
| Engine Input Power | 2.5hp |
| Estimated Output | 480 W |
| Efficiency | 80.2% |
| Drum Speed | 720 rpm |
| Texture Rating | 4.4 / 5 |
| Noise Level | 76–79 dB |

5.7  Evaluation Conclusion

- The modified yam pounding machine operates efficiently and reliably.

- It reduces time and labor compared to traditional pounding.

- The texture is acceptable and consistent.

- Power consumption is reasonable, and noise is within limits.

It can handle 2 kg of yam per batch, making it suitable for small to medium households or food vendors.

5.8. Transport time and effort.

We quantified the gains: the wheeled machine moved on average 4–5 times faster. Operators also reported that setting up in a new location took less than half the time. For example, moving the machine from the corner to the workbench (about 3 meters) went from ~20 seconds pre-modification to ~3 seconds with wheels. These practical improvements align with known effects of wheels reducing friction.

Discussion of Findings

The project succeeded in addressing the major limitation of immobility found in existing yam pounding machines. The frame modification provided a practical and efficient solution. The redesign did not interfere with the machine’s primary function and improved user experience, safety, and flexibility.

5.9. Comparison with Unmodified Machine

| Feature | Original machine | Modified machine |
| --- | --- | --- |
| Mobility | Stationary | Mobile with lockable wheel |
| Cleaning accessibility | Poor | Improved |
| Structural rigidity | Rigid | Rigid with reinforced joints |
| Ergonomics | Less flexible | More user friendly |

In summary, the results demonstrate that wheel integration meets the project goals. Performance in pounding (efficiency and quality) was essentially unchanged, while handling and portability were significantly enhanced. The user experience shifted from “heavy and awkward” to “easy to move and position.” Any drawbacks were minor (need to remember to lock brakes, slightly higher vibration), and can be mitigated by user training.

This study confirms that adding castor wheels to agricultural processing equipment can greatly improve ergonomics without compromising functionality. The design follows sound mechanical principles and standard caster selection criteria. These findings could inform future improvements to other bulky kitchen machines, making them more adaptable to real-world usage.