CHAPTER THREE

MATERIALS AND METHODOLOGY

3.1 Preamble

This research project was designed and implemented in two phases: hardware development and system simulation. The methodology includes sourcing components locally in Ilorin, Kwara State, Nigeria, assembling the circuit, programming the Arduino, and testing the system in a simulated environment.

3.2 Materials and Their Sources in Ilorin

Table3.1ListofMaterialsandtheirsources

| Component | Specification | SourceinIlorin |
| --- | --- | --- |
| Arduino Uno | AT mega 328 P micro controller board | Kwara tech Electronics,Tanke Junction |
| Ultrasonic Sensors (2x) | HC-SR04, range: 2cm-400cm | JKK Electronics, Challenge Bookshop Complex |
| Servo Motors (2x) | SG90 Micro Servo | Tech city Hub, Unity Road |
| LEDs (Red and Green) | 5mm, 2V | Ilorin Electronics Market, Taiwo Isale |
| Buzzer | 5V Piezo electric Buzzer | Just Electronics, GRA Junction |
| Jumper Wires | Male-to-Male, Male-to-Female | Unity Road Market |
| Bread board | 830-point | Kwara tech Electronics Tanke Junction |
| Resistors | 220Ω,330Ω | JKK Electronics |
| Power Supply | 9V Battery or 5V USB Adapter | Tech City Hubor Spar Mall |
| Laptop with Arduino IDE | For Programming and Simulation | University of Ilorin ICT Lab/Personal Laptop |

3.3	RESEARCH PROCEDURE

Step1: Component Acquisition

Electronic components were identified based on compatibility with Arduino. Vendors across Ilorin (Tanke, Taiwo, Unity Road) provided reliable access to affordable and tested hardware.

Step2: Circuit Design and Assembly

- A schematic was drawn with Arduino Uno as the main controller.

- The entry ultrasonic sensor was placed before the crossing, and the exit sensor after it, to detect train approach and departure.

- Two servo motors controlled mock barrier gates.

- LEDs represented traffic lights (Red and Green).

- A buzzer provided audible warnings.

All connections were made on a bread board using jumper wires, with current-limiting resistors for the LEDs.

Step3: Arduino Programming (C/C++)

- Programming was done using Arduino IDE in C/C++ language. The logic flow followed:

- When the entry sensor detects a train (distance < 30cm):

- Buzzer is activated.

- Red light turns ON.

- Servo motor lowers the gate,

- When the exit sensor detects the train has left:

- Buzzer turns OFF.

- Green light turns ON.

- Servo motor raises the gate.

Sample Code Snippet:

#include < Servo.h>

Servo gate;

const int triggerPin = 9;

const in techoPin = 10;

const int redLed = 6;

const int greenLed = 7;

const int buzzer = 5;

void setup(){

pinMode(triggerPin, OUTPUT);

pinMode(echoPin, INPUT);

pinMode(redLed, OUTPUT);

pinMode(greenLed, OUTPUT);

pinMode(buzzer, OUTPUT);

gate.attach(3);// Servo connected to pin 3

Serial.begin(9600);

}

Void loop(){

long duration;

int distance;

digitalWrite(triggerPin, LOW);

delayMicroseconds(2);

digitalWrite(triggerPin, HIGH);

delayMicroseconds(10);

digitalWrite(triggerPin, LOW);

duration = pulseIn(echoPin, HIGH);

distance = duration *0.034/2;

if (distance <30){

digitalWrite(redLed, HIGH);

digitalWrite(greenLed, LOW);

digitalWrite(buzzer, HIGH);

gate.write(0); // Close gate

}else {

digitalWrite(redLed, LOW);

digitalWrite(greenLed, HIGH);

digitalWrite(buzzer, LOW);

gate.write(90); // Open gate

}

delay(100);

}

Step4: System Testing and Calibration

- Sensors were tested for accurate readings under different object distances.

- Trigger distances were calibrated to detect model trains at 20-30cm.

- The servo motor’s closed and open angles were set to 0o and 90o, respectively.

- The buzzer’s timing was adjusted to avoid unnecessary noise post-departure.

Step5: Simulation on Tinker cad

- Auto desk Tinker cad was used to simulate the entire circuit virtually.

- Components were placed in a simulated environment.

- The same Arduino code was uploaded to test barrier operations and sensor responses.

- Real-time debugging was done using the Serial Monitor and visual observations.

Step6: Evaluation and Troubleshooting

- The system was tested repeatedly for consistency.

- Any false triggers from sensors were addressed by: Adjusting sensor alignment, adding debounce delays in the code, using software averaging for distance measurements.

Step7: Documentation and Analysis

- All findings and behaviors were documented.

- Observations confirmed that the system met its intended objectives.

- Identified are as for improvement (e.g., sensor sensitivity to light and sound interference).

| Component | Specification | Source in Ilorin |
| --- | --- | --- |
| Arduino Uno | ATmega328P microcontroller board | Kwaratech Electronics, Tanke Junction |
| Ultrasonic Sensors (2x) | HC-SR04, range:2cm–400cm | JKK Electronics, Challenge Bookshop Complex |
| Servo Motors (2x) | SG90 Micro Servo | Tech City Hub, Unity Road |
| LEDs (Red & Green) | 5mm, 2V | Ilorin Electronics Market, Taiwo Isale |
| Buzzer | 5V Piezo electric Buzzer | Just Electronic, GRA Junction |
| Jumper Wires | Male-to-Male, Male-to-Female | Unity Road Market |
| Bread board | 830-point | Kwaratech Electronics, Tanke Junction |
| Resistors | 220, 330 | JKK Electronics |
| Power Supply | 9V Battery or 5VUSB Adapter | Tech City Hubor Spar Mall |
| Laptop with ArduinoIDE | For programming and simulation | University of Ilorin ICT Lab/Personal Laptop |