CHAPTERFOUR

DATA PRESENTATION, ANALYSIS AND DISCUSSION OF FINDINGS

This section presents the experimental results and analysis from both the physical proto type and the simulation of the Automated Railway Level Crossing System. The results are organized around the major functions of the system: train detection, barrier gate operation, traffic signal control, and system responsiveness.

4.1 SUMMARY OF KEY FUNCTIONAL RESULTS

| Tested Function | Expected Behavior | Observed Behavior | Remarks |
| --- | --- | --- | --- |
| Train Detection (Approach) | Detect train within 20-30cm distance | Detected accurately at an average of 25cm | Working as expected |
| Barrier Closure (Servo Motor) | Close within 2 seconds after detection | Average delay: 1.85 seconds | Prompt and smooth transition |
| Traffic Signal (LEDs) | Switch from green to red upon train detection | Instant transition (<0.5sec) | Properly synchronized |
| Audible Alert (Buzzer) | Buzzer should sound when train is detected | Loud, consistent buzz until train exits | Alert functionality confirmed |
| Train DepartureDetection | Detect exiting train within 20-30cm | Detected at average of 24.5cm | Departure successfully tracked |
| Barrier Re-opening | Reopen barrier within 2 seconds after train exits | Average re-opening delay:1.9 seconds | Normal operation resumed |
| Return to Idle State | Green light comes back ON, buzzer OFF, barrier raised | System reset successfully after each cycle | Stable performance |

4.2	SYSTEM OPERATION BREAKDOWN

A. Train Detection Performance

The ultrasonic sensors (HC-SR04) provided accurate and consistent measurements, both in real-world testing and during simulation. The following table summarizes the sensor detection accuracy:

| Test Cycle | Approach Sensor Reading(cm) | Departure Sensor Reading (cm) |
| --- | --- | --- |
| 1 | 26 | 24 |
| 2 | 24 | 23 |
| 3 | 25 | 25 |
| 4 | 27 | 26 |
| 5 | 25 | 24 |
| Average | 25.4 | 24.4 |

Sensor readings fluctuated slightly within the 2-3cm range, which is acceptable for proximity detection. The system reliably identified train presence and departure with a response time of less than 300 milliseconds after detection.

B. Barrier Gate and Servo Motor Response

The servo motors operated using PWM signals to change positions between 0° (closed) and 90° (open). The average time from detection to complete barrier motion is shown below:

| Barrier Operation | Target Time | Average Observed Time | Result |
| --- | --- | --- | --- |
| Closure | ≤ 2 seconds | 1.85 seconds | ✔ Within standard |
| Opening | ≤ 2 seconds | 1.9 seconds | ✔ Within standard |

C. Traffic Signal and Buzzer Synchronization

Traffic lights and the buzzer were activated simultaneously with the servo during train detection. The control logic implemented ensured proper synchronization.

| System State | LED Status | Buzzer | Barrier Gate |
| --- | --- | --- | --- |
| No Train | Green ON | OFF | Open (90°) |
| Train Approaching | Red ON | ON | Closing (to0°) |
| Train Passing | Red ON | ON | Closed (0°) |
| Train Departed | Green ON | OFF | Opening (to90°) |

4.3 ACHIEVEMENT OF OBJECTIVES

| Objective | Achievement |
| --- | --- |
| Simulate an automatic railway crossing system using Arduino | Successfully simulated in Tinker cad and implemented on bread board |
| Detect approaching and departing trains using ultrasonic sensors | Reliable detection confirmed by multiple test cycles |
| Automatically operate barrier gates and traffic signals | Smooth servo control and LED/buzzer activation observed |
| Improve railway crossing safety and reduce human error | Automation ensures consistent operation, removing the risk of human failure |

4.4	OBSERVATIONS AND LIMITATIONS

Despite the system's reliability in a controlled environment, the following limitations were observed:

- Short Sensor Range: Detection is effective only within limited range (2-4 min real implementation), which may not be sufficient for high-speed trains.

- Environmental Interference: Ultrasonic sensors may be affected by temperature or surface reflectivity.

- Power Dependence: The prototype requires an uninterrupted 5V-9V power supply; fluctuations can cause erratic behavior.

- Scale Limitation: Model-scale simulation differs from full-scale real-world dynamics (e.g., train speed, gate size).

4.5	RECOMMENDATIONS FOR ENHANCEMENT

- Use long-range LIDAR or IR sensors for better outdoor performance.

- Integrate solar power and backup batteries for rural installations.

- Introduce wireless/GSM modules to alert central control units of faults or unusual activity.

- Add camera-based object detection using Raspberry Pior AI modules for future scalability.