CHAPTER FIVE

5.0	CONCLUSION AND RECOMMENDATIONS

5.1	Conclusion

The study evaluate the concentration of heavy metals cadmium (Cd), chromium (Cr), Nickel  (Ni), lead ( pb2+) and manganese (Mn2+) in the water samples collected from the gold mining area in Alagbede daba community. The results revealed that;

Cadmium concentrations exceeded the World Health organization (WHO) permissible limit (0.003mg/l) in 8 out of 10 samples

- Nickel levels were also above the recommended limit (0.02mg/l) in the majority of samples.

- Lead exceeded the World Health Organization (WHO 2020) threshold (0.01mg/l) in four samples posing a serious health risks

- Chromium and manganese remained within acceptable limits in all samples, although chromium values in some samples approached the World Health Organization limit.

These findings indicates significant contamination of local water sources, likely resulting from gold mining activities and related environmental degradation. The presence of toxic Metals like cadmium and lead in drinking water sources poses a serious public health risk especially to vulnerable populations such as children, pregnant women and the elderly.

The reason why the LW1 table under the measured Physico- chemical Parameter in table 2 might be low and not up to the standard of the World Health Organization (WHO, 2019) is because the stream is not a stagnant water but instead a flowing stream so they might be a possibility of the chemicals settling down along the pathway before getting to LW3.

5.2 Recommendations

Based on the findings, the following recommendations are made;

- Water Treatment: immediate implementation of appropriate water treatment technologies such as reverse osmosis or activated carbon filtration in affected areas.

- Monitoring and Regulations: regular environmental monitoring by local authorities (e.g Ministry of Environment, Water Resources Agencies) to track heavy metals level in both surface and groundwater.

- Public Health Awareness:  launching community awareness programs to educate residents on the dangers of using untreated water especially near mining sites.

- Mining Regulations Enforcement: stricter enforcement of environmental regulations on mining operations to minimize waste discharge and groundwater contamination

- Alternative Water Sources: Development and provision of safe, alternative Water Sources ( e.g boreholes, rainwater, harvesting) in highly contaminated communities.