## CHAPTER FIVE

## 5.0 SUMMARY, CONCLUSION AND RECOMMENDATIONS

### 5.1 	Conclusion

The solar-powered, mini submersible DC pumping machine successfully met its objectives of providing an efficient, cost-effective, and sustainable water pumping solution for small-scale applications. The prototype achieved a flow rate of 8 m³/h at a 10-meter head, with a high efficiency of 0.11 m³/h per watt, comparable to larger systems in the literature. The PWM-based variable speed control ensured adaptability to varying solar conditions, and the batteryless design reduced costs to approximately $150, making it viable for rural deployment. Despite limitations in low-light performance (2.2 m³/h at 400 W/m²), the system’s reliability, durability, and simplicity validate its potential for irrigation and domestic water supply in off-grid areas. This project demonstrates the feasibility of compact, low-cost solar pumping systems, contributing to sustainable water access in resource-constrained settings.

### 5.4 	Recommendations

To enhance the prototype’s performance and scalability, the following recommendations are proposed:

- Low-Light Optimization: Optimize the PWM algorithm to improve flow stability at irradiance levels below 400 W/m², potentially increasing efficiency by 10–15%. Alternatively, explore low-cost MPPT integration for better power extraction.

- Battery Integration: Consider adding a small, low-cost battery (e.g., 12 V, 20 Ah) for applications requiring continuous operation, balancing cost and functionality.

- Extended Testing: Conduct long-term testing (6–12 months) to assess durability under varying water quality and environmental conditions.

- Scalability: Develop modular designs to scale the system for larger flow rates or heads, targeting diverse applications like drip irrigation.

- Local Manufacturing: Partner with local manufacturers to produce components, further reducing costs and improving accessibility. These recommendations aim to enhance the system’s efficiency, reliability, and adoption in off-grid communities.

REFERENCES

[1] 	REN21, Renewables 2020 Global Status Report, Paris, France: REN21 Secretariat, 2020.

[2] 	S. A. Kalogirou, Solar Energy Engineering: Processes and Systems, 2nd ed. Oxford, UK: Academic Press, 2014.

[3] 	S. S. Chandel, M. N. Naik, and R. Chandel, “Review of solar photovoltaic water pumping system technology for irrigation and community drinking water supplies,” Renew. Sustain. Energy Rev., vol. 49, pp. 1084–1099, Sep. 2015.

[4] 	T. D. Short and M. A. Mueller, “Solar powered water pumps: Problems, pitfalls and potential,” in Proc. IEE Int. Conf. Power Electron. Mach. Drives, Bath, UK, Apr. 2002, pp. 280–285.

[5] 	A. A. Ghoneim, “Design optimization of photovoltaic powered water pumping systems,” Energy Convers. Manag., vol. 47, no. 11–12, pp. 1449–1463, Jul. 2006.

[6] 	M. A. Vitorino, M. B. R. Correa, and C. B. Jacobina, “Single-phase photovoltaic pumping system with power management and MPPT capabilities,” in Proc. IEEE Energy Convers. Congr. Expo., Pittsburgh, PA, USA, Sep. 2014, pp. 3740–3747.

[7] 	E. Muljadi and R. Taylor, “Variable speed control of a photovoltaic pumping system,” in Proc. IEEE Photovoltaic Spec. Conf., Las Vegas, NV, USA, Oct. 1992, pp. 1234–1239.

[8] 	R. Krishnan, Electric Motor Drives: Modeling, Analysis, and Control, Upper Saddle River, NJ, USA: Prentice Hall, 2001.

[9] 	A. K. Mishra and R. Singh, “Performance evaluation of solar photovoltaic water pumping systems for irrigation in India,” Sol. Energy, vol. 159, pp. 847–856, Jan. 2018.

[10] 	J. Roy and S. K. Saha, “Design and performance analysis of a solar-powered submersible DC pump for rural applications,” Renew. Energy, vol. 130, pp. 434–445, Jan. 2019.

[11]	M. Margolis, Arduino Cookbook, 2nd ed. Sebastopol, CA, USA: O’Reilly Media, 2011.

[12] 	C. Gopal, M. Mohanraj, P. Chandramohan, and P. Chandrasekar, “Renewable energy source water pumping systems—A literature review,” Renew. Sustain. Energy Rev., vol. 25, pp. 351–370, Sep. 2013.

[13] 	K. J. Åström and T. Hägglund, PID Controllers: Theory, Design, and Tuning, 2nd ed. Research Triangle Park, NC, USA: ISA, 1995.

[14] 	C. L. Phillips and J. M. Parr, Feedback Control Systems, 5th ed. Upper Saddle River, NJ, USA: Prentice Hall, 2011.

[15] 	B. K. Bose, Power Electronics and Motor Drives: Advances and Trends, Burlington, MA, USA: Academic Press, 2006.

[16] 	M. A. Eltawil and Z. Zhengming, “Solar photovoltaic water pumping systems: A comprehensive review,” Renew. Sustain. Energy Rev., vol. 59, pp. 1034–1067, Jun. 2016.