CHAPTER TWO

LITERATURE REVIEW

2.1 Overview of Solar Energy

Introduction to Solar Energy

Solar energy refers to the energy emitted by the sun that reaches the Earth as sunlight. It stands as one of the most plentiful, renewable, and sustainable energy sources available. Each hour, the Earth receives more energy from the sun than the total energy consumed by the entire human population in a year. Solar energy can be captured and transformed into usable energy forms such as heat and electricity through various technologies.

Forms of Solar Energy Utilization

Solar energy can be harnessed in two main forms:

Solar Thermal Energy

This method involves utilizing the sun’s heat for purposes such as heating water, warming spaces, and supporting industrial processes. It generally employs collectors to absorb and store thermal energy.

Solar Photovoltaic (PV) Energy

This is the most prevalent technique for generating electricity from sunlight. It utilizes solar panels, which are constructed from semiconductor materials like silicon, to convert solar radiation directly into direct current (DC) electricity via the photovoltaic effect.

Photovoltaic Effect

The photovoltaic (PV) effect refers to the mechanism through which solar cells transform sunlight into electrical energy. When light photons strike a solar cell, they energize electrons within the semiconductor material, resulting in the formation of electron-hole pairs. These liberated electrons are subsequently collected and directed through an external circuit, generating direct current (DC) electricity.

Components of a Solar Power System

A fundamental standalone solar power system comprises:

Solar Panels: Transform sunlight into DC electricity.

Charge Controller: Manages the voltage and current from the panels to avert battery overcharging.

Battery Bank: Accumulates surplus energy for utilization during periods without sunlight (e.g., nighttime).

Inverter: Converts DC electricity from the batteries into AC electricity suitable for powering residential or commercial devices.

Advantages of Solar Energy

Renewable and Abundant: Accessible worldwide and inexhaustible.

Environmentally Friendly: Produces no greenhouse gas emissions during its operation.

Reduces Electricity Bills: After installation, solar systems diminish dependence on grid electricity.

Low Operating Costs: Requires minimal maintenance and has no fuel needs.

Scalable: Applicable in both small residential systems and extensive solar farms.

Challenges of  Solar Energy

Intermittency: Solar energy is unavailable at night and may be less efficient on cloudy or rainy days.

High Initial Cost: Although prices are declining, the initial investment for solar installations can pose a challenge.

Energy Storage Requirement: Batteries are necessary to guarantee power availability during periods without sunlight, which increases both cost and complexity.

Space Requirement: Large-scale solar installations necessitate considerable roof or land space.

2.2 Inverter Technology

Introduction to Inverter Technology

An inverter is an electronic device that transforms Direct Current (DC) into Alternating Current (AC). This transformation is crucial in renewable energy systems, particularly in solar setups, as the majority of household appliances and industrial machinery function on AC, whereas solar panels and batteries generate DC. Inverter technology is fundamental to any solar power system designed to operate standard AC-powered devices. In the absence of an inverter, solar energy would not be suitable for most electrical loads.

Basic  Working Principle of an Inverter

The primary role of an inverter is to invert the current flow from DC to AC. This process is accomplished through several stages:

Oscillation Stage: Converts DC into a square wave AC utilizing an oscillator circuit (transistors or microcontrollers).

Amplification Stage: Increases the low-voltage AC signal to a higher voltage, generally employing a transformer.

Filtering Stage: Refines the waveform, particularly in pure sine wave inverters, to eliminate noise and harmonics.

Control and Regulation: Guarantees voltage and frequency stability, incorporating protective features such as overload and short circuit protection.

Types of Inverters

Based on Output Waveform:

Square Wave Inverter: Generates a square-shaped AC waveform. It is simple and cost-effective but can produce noise and is unsuitable for sensitive electronic devices.

Modified Sine Wave Inverter: Creates a stepped waveform that resembles a sine wave. It is more efficient and compatible with most devices, although it may still cause problems with certain appliances (such as motors and audio equipment).

Pure Sine Wave Inverter: Produces a smooth, clean AC waveform akin to that from the national grid. It is ideal for all appliances, particularly sensitive and inductive loads. However, it is more complex and costly.

Based on Phase:

Single-phase Inverter: Utilized in residential settings and small businesses (as seen in this 2kVA project).

Three-phase Inverter: Employed in industrial applications that demand higher power levels.

Based on Source Integration:

Stand-alone (Off-grid) Inverter: Functions independently of the national grid. It operates with solar panels and batteries.

Grid-tied Inverter: Synchronizes with the national grid and channels excess power back into it.

Hybrid Inverter: Merges solar, grid, and battery systems into a single unit.

Key Components of a Solar Inverter System

Oscillator Circuit: Produces the initial alternating pulses from the DC source.

Switching Devices: Usually MOSFETs, BJTs, or IGBTs, employed to rapidly switch current flow to generate an AC waveform.

Driver Circuit: Regulates the switching devices using Pulse Width Modulation (PWM) for shaping the waveform.

Transformer: Increases the low-voltage AC to 230V (the standard grid voltage).

Filter Circuit: Smooths the output waveform to minimize distortion and harmonics.

Protection Circuitry: Comprises fuses, overvoltage/undervoltage detection, overload shutdown, and temperature protection.

Applications of Inverter Technology

- Solar Power Systems

- Uninterruptible Power Supplies (UPS)

- Electric Vehicles (EVs)

- Telecommunication Systems

- Industrial Motor Drives

The Significance of Inverters in Solar Energy Systems

The inverter functions as the core component of any solar power system. It enables users to:

- Operate AC appliances directly using solar energy.

- Track system performance and identify fault conditions.

- Guarantee compatibility with residential or commercial loads.

- Enhance the efficiency of energy utilization from solar panels and batteries.

Recent Developments in Inverter Technology

- Utilization of microcontrollers and digital signal processors (DSPs) for enhanced control.

- Adoption of Maximum Power Point Tracking (MPPT) technology within inverters.

- Incorporation of Internet of Things (IoT) for remote system monitoring.

- Application of GaN (Gallium Nitride) and SiC (Silicon Carbide) semiconductors for improved efficiency and thermal performance.

2.3 Principles of Solar Inverter Design

A solar inverter plays a vital role in solar energy systems, tasked with converting the DC (Direct Current) electricity produced by solar panels into AC (Alternating Current) electricity that is suitable for use in household or industrial devices. Grasping the principles of solar inverter design is crucial for developing an efficient and dependable power solution. The fundamental design principles are detailed below.

DC to AC Conversion

The main purpose of a solar inverter is to convert DC from the solar panel or battery bank into a usable 220V–240V, 50Hz AC output. This process is accomplished through:

Oscillation: Producing a high-frequency alternating signal.

Switching: Employing MOSFETs or IGBTs to control current flow.

Transformation: A step-up transformer elevates low-voltage AC to the standard grid voltage.

Filtering: Ensures that the output waveform is stable and clean.

Consideration of Waveform

There are three types of output waveforms:

Square wave: Inexpensive and simple to produce, but incompatible with most appliances.

Modified sine wave: Approximates a sine wave; suitable for most non-sensitive devices.

Pure sine wave: Closely mimics grid power; ideal for all loads but is more costly and complex.

This project emphasizes modified sine wave output for cost-effectiveness.

Power Rating

The inverter is rated at 2kVA, which means it can manage loads up to 2000 volt-amperes. The actual usable power is influenced by the power factor (typically 0.8), allowing for approximately 1600W of real load support.

Integration of Solar Panels

Solar panels act as the primary energy source. Their sizing must correspond to the anticipated load of the system. The inverter design must:

- Accept varying DC voltages from the panels.

- Incorporate a charge controller to oversee battery charging.

- Facilitate battery storage for energy supply when solar input is not available.

Battery Support

The inverter must draw power from a 24V battery bank (typically consisting of two 12V batteries in series). The batteries should possess sufficient capacity (e.g., 200A) to ensure reliable operation.

Control and Safeguarding

To guarantee durability and security:

- Overload protection circuits sever excessive loads.

- Low-voltage cut-off safeguards against battery damage.

- Fuses and circuit breakers ensure fault isolation.

- Cooling systems (fans, heat sinks) regulate thermal output.

Effectiveness and Functionality

Design choices should aim to reduce power losses caused by:

- Heat in switching devices

- Core and copper losses in transformers

- Inaccuracies in waveform generation

- An efficiency range of 80–90% is preferred for residential solar inverters.

2.4 Challenges in Solar Inverter Development

Although solar inverters are essential elements in photovoltaic (PV) systems, their design and deployment encounter numerous technical, economic, and environmental obstacles. Addressing these issues is crucial to guarantee the reliability, efficiency, and sustainability of solar power systems, particularly in resource-constrained or off-grid settings such as many rural areas of Nigeria and other developing nations.

- Efficiency Losses

A significant challenge in the development of solar inverters is ensuring high efficiency during the conversion from DC (Direct Current) to AC (Alternating Current). Efficiency losses can arise from:

- Switching losses in transistors (MOSFETs/IGBTs)

- Transformer losses (core and copper)

- Heat dissipation, necessitating effective thermal management. An inverter with low efficiency leads to energy wastage, overheating, and diminished performance.

- Cost of Components

The expense associated with manufacturing a high-quality solar inverter continues to be a significant hurdle, especially in developing nations. Major contributors to these costs include:

- High-quality semiconductor switches (e.g., MOSFETs, IGBTs)

- Pure sine wave generation circuitry

- MPPT (Maximum Power Point Tracking) charge controllers

- High-efficiency transformers. The balance between cost-effectiveness and performance can influence the long-term feasibility of the inverter.

- Complexity of Control Systems

The creation of control circuits for functionalities such as waveform shaping, battery protection, overload cut-offs, and voltage regulation introduces additional complexity. This is particularly true for pure sine wave inverters, which utilize:

- Microcontrollers

- PWM (Pulse Width Modulation)

- Feedback control systems. This complexity demands both design proficiency and thorough testing, potentially leading to increased development time and costs.

- Heat Management

Solar inverters produce considerable heat during their operation, particularly when subjected to heavy loads or extended usage. In the absence of adequate cooling systems, such as heat sinks and fans, critical components may overheat and fail. Nevertheless, the integration of effective cooling systems contributes to increased design bulk and expense.

- Environmental Factors

In numerous installations, particularly those that are outdoor or off-grid:

- Dust, moisture, temperature fluctuations, and humidity can compromise the integrity of electronic components.

- Achieving weatherproof enclosures and ensuring robust system protection can prove to be both challenging and expensive.

- Battery Dependency and Sizing

The performance of a solar inverter is significantly influenced by the availability and capacity of batteries. Inadequate battery sizing or subpar battery maintenance practices can result in:

- Short runtime

- Battery degradation

- Frequent low-voltage cutoffs

This underscores the importance of proper battery-bank design as a critical challenge.

- Synchronization with Load and Grid

For hybrid or grid-tied inverters, achieving synchronization with grid voltage and frequency can be technically complex. Any discrepancies may lead to damage to appliances or grid faults. Even standalone inverters face the challenge of managing variable loads (inductive, capacitive, resistive) without causing waveform distortion.

- Limited Local Manufacturing and Technical Skills

In areas such as sub-Saharan Africa, access to high-quality electronic components is restricted. Furthermore:There is a lack of skilled engineers or technicians experienced in embedded systems and inverter circuitry.

This results in a dependence on imported inverters, which tend to be expensive and may not be suitable for local conditions.

- Reliability and Maintenance Issues

Frequent component failures, particularly in low-cost inverters, lead to elevated maintenance requirements. The scarcity of spare parts and inadequate after-sales support further exacerbates the sustainability of the system.