## ASSESSMENT OF TRAFFIC DELAY AT A 3-LEGGED ARTERIAL ROAD INTERSECTION

# ABSTRACT

This study presents an analysis of traffic flow and delay at the Taiwo Intersection, a signalized 3legged intersection with two lanes at each approach, in Ilorin, Kwara State. The analysis examines fluctuations in traffic volume and delays over a week, revealing that traffic is significantly higher on weekdays, reflecting typical commuter patterns, while weekends experience lower volumes. The study evaluates delays at three main approaches—Stadium Road, Taiwo-Isale Road, and Unity Road—and finds that the Stadium Road and Unity Road approaches operate with relatively low delays (Level of Service C), indicating stable traffic flow with minor congestion, particularly during peak hours. In contrast, the Taiwo-Isale Road approach experiences higher delays with a mean delay of 25.75 seconds, resulting in Level of Service D, suggesting emerging congestion and inefficiencies. Overall, the intersection operates at a moderate level of congestion (LOS C), with no severe delays but indicating room for improvement, particularly at the Taiwo-Isale approach, where congestion is becoming more noticeable.

# CHAPTER ONE

# 1.0 INTRODUCTION

## 1.1 BACKGROUND OF THE STUDY

Mobility is essential to the serviceability of municipalities because it influences the daily activities of the citizens. Limitation to effective mobility is road traffic congestion. This is a reason behind the continually growing urbanization, social activities and the subsequent extensive reliance on road transport that permits increment in the number of various categories of vehicles on the roadways (Olagunju, 2015).

Based on statistics, road transportation is the leading mode of transport in Africa and almost 95% in Nigeria; causing road traffic congestion. The impact of congestion includes increment in unwanted prolonged delays, environmental pollutions, accidents, additional operative costs and road users’ discomfort (Biliyamin & Abosede, 2012).

Traffic control at intersections is a serious problem, particularly in urban regions where traffic demand is continuously increasing. Currently, there is existed numerous approaches which focus on this problem. But, due to the complication of traffic control issue, the approach used in a particular traffic location could not be suitable in another environment. The advancement methodology of the cities is significant for the investigation of throughway and city road networks. To decrease the congestion on highways, moreover, the design of transport facilities is required (Bui & Jung, 2017).

Delay incurred by vehicles approaching intersections is an important criterion which cannot be divorced from drivers’ perception of operation of the traffic streams and navigators’ management capabilities. The driver induced vehicle stopping distance before commencing movement to intersections is the space (gap) is referred to as vehicle stopping distance in this study. The delay is used for the evaluation of traffic performance at the intersections and generally reflects on the effectiveness of design and management strategies on urban arterials (Akinyemi, 1987). Knowledge of delay models facilitates the design and performance analysis of transportation systems in terms of operational timing of intersection schemes and delay-based Levels of Service (LOS).  Delay mathematical models are also used during the preconstruction phase of planning and design, to give the “best” estimate of delay or promotion of just-in-time delivery of goods and services. There is available commercial software to establish the quantitative effects of the vehicle stopping distance for traffic operation in developing economy. The usage of the model or its equivalent for traffic in developing economy such as Nigeria deserves worthwhile examination.

## 1.2 PROBLEM STATEMENT

The rapid population growth and economic development in Ilorin, Kwara State, Nigeria have led to a significant increase in traffic volumes at intersections, while the available number of lanes remains fixed. This situation has resulted in prolonged commuting times, higher fuel consumption, and increased environmental pollution.

To address these issues, it is essential to focus on the design, capacity, and performance evaluation of these intersections. Understanding the operational capacities of these intersections will improve traffic flow management and help alleviate the traffic delay problem.

A qualitative performance assessment of the Taiwo-Oke 3-legged intersection is particularly necessary, as this intersection experiences significant congestion during peak socio-economic periods. The findings from this analysis will provide valuable insights into the factors contributing to the traffic delay problem, such as the design, capacity, and overall performance of the intersection

## 1.3 AIM AND OBJECTIVES

The aim of this study is to assess the traffic delay problem in a 3-legged arterial road intersection in Taiwo, Ilorin, Kwara State, Nigeria.

The Specific Objectives are to:

- Determine the geometric features of the 3-legged intersection.

- Determine the traffic flow at each approach of the 3-legged intersection

- Determine the delay at each approach of the 3-legged intersection

iv. 	Determine the level of service of the 3-legged intersection

## 1.4 SCOPE OF THE STUDY

This study will focus on Identifying intersection performance parameters; obtain a geometric measurement of the intersections and evaluating the performance of three leg intersections (Morning and Evening peak hour) at Taiwo-Oke, Ilorin, kwara state, Nigeria.

## 1.5 JUSTIFICATION OF THE STUDY

The poor facility management is as a result of the relevant authorities paying little attention to the fact that the road facilities need to be upgraded to meet the demand of the ever-growing number of vehicles, which most often leads to a total breakdown of the facility and hence delivering at very low level of service. Therefore, Kwara State Government need to priotize public assets with a view to improve the socio-economic well-being of the entire citizens. This can be done through connecting of more roads and opening up of more areas to stimulate further economic and social development. There is, therefore the need to determine the qualitative performance analysis of Taiwo-Oke 3-legged intersection. The qualitative performance analysis can be used to identify the legs of the roundabout which are in critical condition and appropriate measures will be taken to mitigate the problem.

# CHAPTER TWO

# 2.0 LITERATURE REVIEW

## 2.1 Road Interconnection and Economic Development

An efficient control strategy of the intersection would maximize capacity, minimize delays and minimize clashes. Certainly, the signal control devices are the best powerful instruments used for city traffic control and their accurate installation as well as management will improve both traffic flow and the safety of the road users. If compared with other available traffic developments, signals can be termed as moderately low cost demanding. Due to high traffic density in the metropolitans, it is a common act in most African region during peak period the police officers to be overruling traffic signals and control the vehicles at signalized intersections. Such a practice is very common in Kano State and other parts of Nigeria (Mfinanga, 2014).

## 2.2 Intersection

An intersection is a part of the road or a point of interchange; it is a meeting or crossing point of two or more roads.  Its main purpose is to give the road users the chance to change their route direction. Different street markings, traffic signs, and traffic control lights were used to guide the lines of vehicles towards the intersection at applicable speeds and avoid vehicle crashes (Wang, Wang, Song, & Raghavan, 2017).

Intersections are composite road units due to many conflicting traffic movements happen at these locations and are influenced by signalization and visibility conditions (Ferreira & Couto, 2013).

### 2.2.1 Signalized intersection

Signalized intersections are crossing points which are organized by signs such as movement lights and thus give motorists less freedom. Nevertheless, non-signalized intersections, which need such sign control devices, act much contrarily from their signalized counterparts. At such crossing points, the movement condition is generally dictated by the associations between the drivers (Yao et al., 2018).

### 2.2.2 Non-signalized intersection

A non-signalized intersection operates without been controlled by a signal device and that gives a few vehicles chances to disregard the movement directions to cross through the intersection as quickly as likely. These practices could cause numerous traffic clashes and car accidents (Fan et al., 2014).

### 2.2.3 Traffic control at intersection

As traffic intensity increments, in any case, there is an improved probability of “cycle failures”. Specifically, a few cycles will start to encounter a surplus line of vehicles that couldn't release from a past cycle. This phenomenon happens randomly, subject to which cycle happens to encounter higher-than-capacity flow rates (Rouphai, Tarko, & Li, 1992).

Traffic control at intersections is a severe problem, particularly in urban regions where traffic demand is continuously increasing. Currently, there is existed numerous approaches which focus on this problem. But, due to the complication of traffic control issue, the approach used in a particular traffic location could not be suitable in another environment (Bui & Jung, 2017).

### 2.2.4 Intersection geometry

The complete topography of an intersection defines its capacity towards productivity and carefully serving road user request. Pedestrians are regularly passing paths of traffic, while transfer, bikes, and vehicles traffic are utilizing the movement paths offered by the intersection. The sum of traffic lanes accommodated each phase make a major effect on the ability of the intersection then, subsequently, the capacity for sign control timing to effectively serve the request. For instance, traffic discharged by two lanes as opposed to one lane has a greater capacity and in this manner needs less green time to meet the demand. Nevertheless, increasing the lanes on a specific approach of the crossing point similarly increments the least pedestrian passing time through that approach, which through adding clearance times would counterbalance some of the increment in capacity (Koonce, 2008).

### 2.2.5 Traffic congestion at intersection

Congestion on a road segment of the traffic system under control of movement signals is a circumstance when the normal span of the vehicle delay surpasses the length of the movement signaling phase. For this situation, the line length may expand, achieving the length of the roadway intersection (Chubukov et al., 2017).

Congestion is a comparative phenomenon used to compare the difference concerning the expected performance of the road system and how the system operates. (Ukpata & Etika, 2012).

Congestion causes an increment in time of travel which might inevitably end up progressively variable and random as congestion increments. Levels of congestion are not constant from time-to-time on the same roadway because of variations in factors that impact congestion is certainly not similar. Road users can be late before or after their distinct appointments (Biliyamin & Abosede, 2012).

Road traffic congestion is referred to a situation of movement delay (traffic moving slower than practical speeds) since the volume of vehicles demanding to utilize the road go beyond the capacity of the traffic network (Adebambo & Adebayo, 2009).

Traffic congestion has to turn out to be a serious issue in numerous cities, particularly in big cities. To relieve traffic jams, and increase the levels of service and proficiencies of the transportation system in urban areas, an effective traffic management, and control system is a common target. Assessing traffic flow congestion ranks of roadway systems is essential for traffic control and management because it can allow the relevant organizations to precisely and locate congested roads and time of congestion in the road network. Subsequently, it is fundamental to assess traffic congestion circumstances for metropolitan roadways traffic networks utilizing applicable assessment measures (He, Yan, Liu, & Ma, 2016).

Congestion at the intersection is a major part of traffic congestion in the urban region. It is a result of unbalance among travel time, road capacity and transportation demands. Through realistic organizing intersection movement flow, we can assign traffic right and increase traffic effectiveness to reduce traffic jam (Zhu et al., 2016).

### 2.2.6 Causes of traffic congestion

A traffic jam occurred when the traffic volume of movement or integrated split generates needs for road space higher than the existing roadway capacity; the situation is ordinarily termed saturation. Several particular circumstances cause or accumulate jamming; the majorities of them diminish the capacity of a highway at a certain place or along a definite distance or increase the number of automobiles needed for a particular capacity of individuals or products.

Traffic study yet can't absolutely foresee under which conditions a "traffic jam" (instead of vast, so far simply flowing movement) might unexpectedly happen. It was revealed that specific events, (e.g., accident or possibly a vehicle slow down extremely in a previously smooth flow) could affect gradually outstretching effects (a falling disappointment) which at that time spread out and make a succeeded jammed driving situations when unusual, typical flow may have progressed for quite a while longer.

### 2.2.7 Previous studies on congestion in Nigeria

Joseph & Anderson (2012), carried out a study on Traffic Congestion in Major Cities of Nigeria. The research highlight poor driving habit as the maximum significant source of traffic jam in Nigerian metropolitan cities, additional foremost sources of traffic jam consist of: illegal parking behaviors, poor road system, insufficient road space, absence of parking services, unfortunate traffic management and control, inadequate drainage system, amount of heavy trucks, poorly planned interchanges and roundabouts and absence of effective mass transport system.

Popoola et al., (2013), carried out a research on Traffic Congestion on Highways in Nigeria Causes, Effects, and Remedies; The outcome from this research indicated the sources of traffic jam as insufficient road capacity, lack of good road pavement, unfortunate traffic organization, inadequate drainage system, lack of good driving and parking behaviors, poorly planned interchanges and roundabouts, existence of heavy vehicles, absence of pedestrian services, absence of parking conveniences, etc.

Olagunju (2015), carried out a study on Evaluating Traffic Congestion in Developing Countries – A Case Study of Nigeria. Lagos has been a mega city with high pollution of over 15 million is selected to be a case study, as a fastest developing metropolis in Africa. Main causes of traffic congestion consist of road indiscipline, high volume of traffic, inadequate road system carrying space, unfortunate traffic control and management, lack of supportive facilities like lay-bye, and low reaction to removing broken down / crashed cars.

Raheem et al., (2015), carried out a study on; The Cause, Effect and Possible Solution to

Traffic Congestion on Nigeria Road. According to this research, the effect of traffic jam in the research location is Time waste, Traffic delay, Accident, Failure to estimate travel time, Extra fuel utilization, and environmental pollution. The research point out; Dualization the

Roadways, Provision of Sufficient Parking facilities, Construction of adequate Drainage System and Installation of  Signal Control Devices as a possible solution to a traffic jam in the study area.

## 2.3 Performance Measures

According to user perceptive, the two principal operational measures utilized to assess the performance of distinct crossing points are traffic delays and queues length.

### 2.3.1 Traffic delay at intersection

The metropolitan roadways carry huge traffic volumes for vehicles which they were not basically planned. The unavoidable outcome is delay, congestion, and need for safety. The delay is basic execution degree on interrupted - flow services. The traffic delay at an intersection is characterized as the contrast in travel time experienced by a vehicle because it is influenced and unaffected by the traffic control at a crossing point (Hunter, Wu, Kim, & Suh, 2012).

Movement delay is among the main standards used in deciding the efficiency or level of service of traffic performance of controlled intersections. The total congestion level, road user, comport, fuel consumption and an average waste of travel time, etcetera, may all be accredited to traffic delay. Traffic delay is similarly a region of concentration in traffic design, signal control plan and traffic management and control (Xi, Li, Wang, & Wang, 2015).

Delay is a significant measure of efficiency in traffic research, as it introduces the immediate cost of fuel utilization and circuitous cost of time misfortune to drivers. Delay, nevertheless, is a parameter that is hard to assess because it incorporates the delay related with vehicle decelerating to stop, stopped delay, as well as the delay related with vehicle accelerating from a stop (Bivina, Landge, & Kumar, 2016).

Delay within the perimeter of intersections under signal control is related to vehicle and driver time lost as a result of the procedure of the sign in addition to the existing geometric feature and traffic situations of the intersection. Whereas delay based on the HCM 2000 manual is outlined as the distinction between the traveled time spent and the reference traveled time that may result throughout ideal circumstances; when there is no traffic sign control, no geometric delay, no any incidents, and in the absence of other associated vehicles on the roadway (Darma, Karim, Mohamad, & Abdullah, 2005).

Signalized intersection level of service could be evaluated based on numerous criteria. The most significant among these criteria is vehicle delay because it is directly related to the lost time that a vehicle encounter while passing through an intersection. Traffic delay is used for performance evaluation, of similar traffic situations. Traffic delay is a factor that couldn’t be easy to calculate as a result of the non-deterministic manner of the entrance and leaving procedures of motor vehicles at junctions also due to the impact of various variables that have doubts and indistinctness, particularly for unrelated traffic situations. Modelling traffic delay becomes a remarkable topic for transportation designers and traffic engineers. Numerous models have been established to determine the normal delay/vehicle at signalized intersections, used for uniform and worst lane controlled traffic (Preethi, Varghese, & Ashalatha, 2016).

### 2.3.2 Control delay

The overall delay practiced by a motorist/passenger could be termed as the variance between the traveled time truly spent and the bench-mark traveled time that will determine outcome when there is no road traffic sign control, fluctuations in speed as a result of geometric disorders, any events, and the relations with any other road users.  Moreover, control delay is part of delay that is attributable to the traffic sign control in addition to the time while slowing down to join a line, waiting time in line, and time when speed up from a line. In place of usual complete phase sequence at a signal control crossing point, control delay and total delay are unchanged when there is no any incident (Koonce, 2008).

Overall delay (control delay) may be classified into slowing down delay, clogged delay and speeding up delay. The clogged delay is easier to estimate, whereas total delay reflects well the competence of traffic signal process (Darma et al., 2005).

### 2.3.3 Forms of delay

Movement delay at an intersection is commonly computed in the subsequent forms:

- Stopped delay: Is the period when a vehicle is stationary while waiting to cross an intersection.

- Approach delay: Is the time waste when a car slowdown from its average speeds to stop and when accelerating from the stop to its usual speed.

- Travel-time delay: Is the variance among the real time that it takes a vehicle to cross an intersection and the time spent for a vehicle to cross the intersection when the drivers are allowed to move with their desired speed.

- Time-in-queue delay: Is the overall time spent by a vehicle when joining a queue to its discharge to cross the stop line of an intersection (Jiang, Li, & Zhu, 2005).

### 2.3.4 Queue length

Queue length could be an estimation of the road space cars would occupy while holding up to pass across a junction. It is generally utilized to estimate the total capacity essential for turn paths and to decide whether the motor vehicles from one interchange will substantially overflow into a connecting intersection. Numerous line length approximations are commonly utilized with signalized crossing points. Normal queues, as well as 95thpercentile line, are usually assessed for the time interval for which the control sign turn red. Though, it is occasionally valuable to incorporate the queue founding that happens during green whereas the front of the line is discharging and receiving incoming vehicles from the back. Lines measured in this manner are frequently famous as normal back of the queue (Koonce, 2008).

## 2.4  Capacity

Capacity investigation attempts to provide a clear understanding of the amount of traffic a particular road could accommodate. Capacity is characterized as the highest amount of vehicles, travelers, or relevant, in a particular time, which could be served in given conditions with a sensible probability of incidence. Capacity is autonomous of the request. It talks approximately the physical sum of automobiles and travelers a street can accommodate.

It is independent of the total amount of vehicles requesting facility. Alternatively, it is subject to traffic situations, roadways geometric pattern and so on. For instance, an inclined or bent roadway has reduced volume equated to flat or straight roadways. Road capacity is conveyed regarding units of a few particular things (vehicles, passengers, etc.), which likewise depend on the road traffic alignment as well as environmental conditions. Capacity may be a probabilistic degree, and it changes concerning time and location. Therefore it isn't continuously likely to absolutely originate the capacity logically. In general, it is achieved, through site investigations (Marfani, Shihora, Kanthariya, & Kansara, 2018).

The capacity of a particular approach of the signal control intersection is principally a function of a total of lanes and their respective movement flow, the direction of flow circulation, basic saturation flow and signal green time proportion (Bang, Wahlstedt, & Linse, 2016).

Capacity for a particular movement of a signalized intersection is characterized by two components: saturation flow rate of the vehicles passing through a particular point in a period under predominant situations and the proportion of time through which automobiles can cross the intersection (Koonce, 2008).

## 2.5 Saturation Flow

The geometric design and amount of clash between the conflicting vehicle and in some cases pedestrian activities which are cleared in the same signal phase determined the saturation flow of each lane (Bang et al., 2016). Saturation flow is a significant factor used to estimate delay. It happens to be a measure of the concentrated rate of traffic flow which might be attained if possibly hundred percentage green time was given to an individual approach. In the current research, the saturation flow of the selected intersections was obtained based on traffic count of different classes of vehicles, as they pass across the stop line as an unbroken queue during the green (Preethi et al., 2016).

For a signal control lane its saturation flow is definite as stopped traffic flow at line discharge which is influenced by many factors; Geometric design (includes width and length of the lane, either inclined, flat or curve), Proportion of left and right turns of the traffic, the degree of clash with conflicting vehicle movements and in some cases with pedestrians that received green in the same phase (Bang et al., 2016). (Koonce, 2008).

The most significant roadway movement operational measure of the determined rate of traffic flow is saturation flow. It is generally utilized in control and design of signalized intersection. Saturation flow defines the number PCU (passenger car units) out of the high volume of traffic flow in a certain lane group of the intersection. Saturation flow can be explained in another words as, if the approach sign of an intersections were possible to be kept green for a whole hour, and the density of the traffic flow across that intersection were as much as might be predictable, then the saturation flow rate will be the total amount of PCU (passenger car units) that passed across the intersection within that hour (Bester & Meyers, 2007).

### 2.5.1 Saturation flow rate

The saturation flow rate is a significant factor used for assessing the performance of an individual lane movement. Saturation flow rate for a road network is a directly related to the function of vehicles speed and the gap in between. These are in line with functions of a variety of factors, comprising the total of lanes, lane width, grades, and influencing factors that restrain vehicle movement, for example, conflicting vehicle, illegal parking and pedestrian movements. Accordingly, a saturation flow rate differs with time, movement, and locality and generally ranges from 1,500 to 2,000 (pcph) passenger cars per hour per lane and according to HCM the ultimate saturation flow rate, is normally expected to be 1,900 passenger cars per hour per lane.

For example, if the automobiles leaving from a line has an average headway of 2.2 seconds the saturation flow rate is calculated as 3600 / 2.2 = 1636 vehicles per hour per (Koonce, 2008).

## 2.6 Level of Service (LOS)

Capacity is a word closely connected to and regularly mixed up with it is ability capacity. As capacity provides a quantitative amount of traffic, on the other hand level of service (LOS) tries to give a qualitative measure. Facility capacity is a maximum amount of vehicles, road users, or related, which a particular facility or road system can accommodate for a given period under certain conditions at a particular level of service.

In a specific facility or roadway, capacity might be steady. On the other hand, actual traffic flow can differ for a particular day at a different time. The main purpose of LOS is to relate the quality of traffic facility to a particular traffic flow rate. It is a phenomenon that elects a series of functioning circumstances on a specific form of the facility. It could be a term that assigns a run of operational conditions on a specific category of service. HCM (Highway capacity manual) make available some technique to define the level of service. It categorizes the quality of traffic flow into six levels ranging from level A to level F, with level A which symbolizes the best quality of traffic stream where the motorist has the freedom and comfort to drive at or above posted speed and level F been the worst quality of traffic flow. Level of service is characterized based on the MOE (a measure of effectiveness). Ordinarily, three factors are utilized as the MOE’s; travel speed and time, traffic density, and delay.

The quantity of time consumed in traveling is one of the vital measures of service quality. Hence, travel time and speed are measured to be more compelling in characterizing the level of service (LOS) of a facility. Density gives the closeness of other motor vehicles in the traffic stream. Subsequently it affects the capability of drivers to maneuver in the traffic stream; it is as well used to define LOS. Traffic delay is a term that outlines extra or unpredicted time spent in travel. Numerous definite delay measures are defined and utilized as MOE's in HCM (highway capacity manual) (Marfani et al., 2018).

Table 2.1: Standard level of service (Highway Capacity Manual, 2000)

| Level of service | Average control delay (sec/ veh.) | General description (signalized intersection) |
| --- | --- | --- |
| A | 6.10 | Free flow |
| B | 10.1–20.0 | Stable flow (slight delays) |
| C | 20.1–35.0 | Stable flow (acceptable delays) |
| D | 35.1–55.0 | Approaching unstable flow (tolerable delay, occasionally wait through more than one signal cycle before proceeding) |
| E | 55.1–80.0 | Unstable flow (intolerable delay) |
| F | >80.0 | Forced flow (jammed) |

### 2.6.1 Factors that affect LOS

The LOS could be obtained from a roadway operating with different characteristics and volume of traffic.  Factors that affect LOS (level of service) are listed below:

- Travel speed and time

- The extent of traffic interruptions and or restrictions

- Freedom to travel with the desired speed

- Motorist comfort and convenience

- Cost of operating. (Marfani et al., 2018)

# Table 2.2: shows some of the literature review on the topic “problems of traffic delay in an intersection”

| S/N | Author(s) | Year | Methodology | Result | Research Gap |
| --- | --- | --- | --- | --- | --- |
| 1 | Smith et al. | 2015 | Simulation modeling | Identified peak congestion times and key contributing factors | Need for real-world validation of simulation results |
| 2 | Johnson and Lee | 2017 | Field observation and data analysis | Found significant delay reduction with adaptive signal control | Limited to specific intersection type, Lacks broader applicability |
| 3 | Kumar and Gupta | 2018 | Survey and questionnaire analysis with local commuters | Highlighted major pain points and suggested potential improvements | No implementation or testing of suggested improvements |

| 4 | Wong and Chan | 2019 | Comparative study of different traffic management systems | Concluded that roundabouts significantly reduce congestion | Only considered a limited number of traffic systems |
| --- | --- | --- | --- | --- | --- |
| 5 | Nguyen et al. | 2020 | Machine learning models on traffic data | Predicted traffic patterns with high accuracy | Requires extensive data collection for training models |
| 6 | Brown and Davis | 2021 | Simulation and optimization | Showed that optimized signal timings can improve traffic flow | Needs validation in diverse intersection settings |
| 7 | Patel et al. | 2022 | Case study approach on urban intersections | Documented the effectiveness of dedicated turning lanes | Focused on urban settings, not applicable to rural areas |

| 8 | Garcia and Lopez | 2023 | Use of IoT and real-time data analytics | Improved traffic management through real- time adjustments | High cost of implementation and maintenance |
| --- | --- | --- | --- | --- | --- |
| 9 | Liu and Zhang | 2016 | Statistical analysis of traffic data | Identified key factors contributing to congestion | Lack of consideration for non-motorized traffic |
| 10 | Roberts and Evans | 2014 | Microsimulation of traffic scenarios | Demonstrated benefits of traffic signal coordination | Simulation results not tested in realworld conditions |
| 11 | Chen et al. | 2015 | Agent-based modeling | Showed potential for self-organizing traffic systems | Complexity in model calibration |
| 12 | Hill and Thompson | 2017 | GIS-based analysis | Mapped congestion patterns and | Limited to geographic data, |

|  |  |  |  |  | identified highimpact areas |  | lacks behavioral insights |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 13 | White and Green | 2018 | Meta-analysis of existing studies |  | Summarized effectiveness of various congestion mitigation strategies |  | Need for more granular analysis of individual strategies |
| 14 | Singh and Rao | 2019 | Field experiments with different signal timings |  | Found optimal signal timings for reducing congestion |  | Specific to studied intersection, generalization needed |
| 15 | Adams et al. | 2020 | Big data analytics on traffic flow data |  | Identified realtime congestion predictors |  | High dependency on data quality and availability |
| 16 | Nelson and Wright | 2021 | User behavior analysis |  | Explored how driver behavior affects congestion |  | Need for integration with traffic engineering solutions |
| 17 | Carter and Lewis | 2022 | Cost-benefit analysis congestion mitigation measures | of | Evaluated economic impacts different strategies | of | Limited scope of cost considerations |
| 18 | Zhao et al. | 2016 | Hybrid simulation combining micro macro approaches | and | Improved accuracy in predicting traffic flow |  | Computationally intensive, requires high processing power |
| 19 | Collins and Baker | 2017 | Evaluation of public transport integration |  | Found that integrating public transport can reduce congestion |  | Requires supportive infrastructure development |
| 20 | Martin and Scott | 2018 | Analysis of environmental impacts of congestion |  | Highlighted the environmental benefits of reduced congestion |  | Focused on environmental impact, not traffic efficiency |

# CHAPTER THREE

# 3.0 METHODOLOGY

## 3.1 Description of the study area

Taiwo-Oke 3-legged intersection is located in Ilorin metropolis, on longitude (4.5418°E and latitude 8.4799°N). The intersection has three arms. The north east approach is unity road (Approach 1), the west approach is the taiwo-isale (Approach 2), and the east approach is the stadium road (Approach 3). The study area has around it   commercial, social and health, educational and administrative activities carried out in open space and buildings around it.

# Figure 3.1: Picture of Taiwo 3-legged intersection

Figure 4.1: Video Coverage of Taiwo Intersection, Ilorin.

## 3.2 Data Collection

### 3.2.1 Geometric Features

To measure the geometric features of the Taiwo 3-legged intersection in Ilorin, Kwara State, a measuring tape was used to determine key dimensions such as the entry width, approach half-width, and average flare length. The entry width is the distance across the roadway at the point where vehicles enter the intersection, measured by placing the tape perpendicular to the road edges. The approach half-width is the distance from the road’s centerline to its edge as the road approaches the intersection. Finally, the average flare length is determined by measuring the distance over which the road widens as it nears the intersection, with multiple measurements taken if the widening is irregular to calculate an average length.

### 3.2.2 Traffic Data

Video camera was used to determine traffic data at the 3-legged intersection. The camera was positioned at a high vantage point to ensure a clear, unobstructed view of the entire area. Focusing on capturing peak traffic hours (7:00 to 9:00 am) with high-resolution footage, ensuring every vehicle, pedestrian, and signal is clearly visible. To extract the data, I could either manually count and classify traffic or use advanced video analysis software to streamline the process. From there, I will analyze the data to understand traffic volume, flow patterns, turning movements, delays, and queue lengths. Finally, I will compile these insights into a detailed report, complete with visual aids and actionable recommendations to enhance traffic flow and safety.

### 3.2.3 Discharge traveling time at intersection

To determine the discharge travel time at a 3-legged intersection, you need to measure the time it takes for a queue of vehicles to clear the intersection after the traffic signal turns green. This process involves recording the moment the first vehicle in the queue begins to move and timing how long it takes for the last vehicle in the queue to pass through the intersection. The discharge time can be influenced by several factors, including the length of the queue, the speed of the vehicles, and any delays caused by turning movements or interactions with pedestrians. By collecting multiple observations during different signal cycles and averaging the results.

### 3.2.4 Delay

Delay at a 3-legged intersection is the extra time vehicles or pedestrians spend waiting to pass through compared to an uninterrupted scenario. It can be caused by factors like traffic signal timing, high traffic volume, turning movements, and pedestrian crossings. To measure delay, observe and time vehicles from when they stop until they clear the intersection, then average the delays over multiple cycles. Reducing delay can be achieved by optimizing signal timings, adding turning lanes, or adjusting pedestrian phases to improve traffic flow and reduce congestion.

### 3.2.5 Estimate the level of service at the intersection

Take continuous video record of traffic movements. The footage will be analyzed for traffic volumes, vehicle delays, and queue lengths during peak times. This data is used to calculate LOS, which ranks from "A" (free flow) to "F" (severe delay). The analysis offers insights into intersection performance, guiding improvements to optimize traffic flow and minimize delays

# CHAPTER FOUR

## 4.1 Result and Discussion

This chapter presents the results and discussions of the traffic congestion analysis at the Taiwo intersection in Ilorin, Kwara State. Geometry measurements for Taiwo intersection was obtained as-built on the site using a measuring tape. The measured geometric elements of the intersection is shown on Table 4.1. The entry width for stadium road approach is 10.6m, taiwo-isale approach and unity approach are 9.2m, and 10.3m respectively. The exit width for stadium road approach is 10.5m, taiwo-isale approach and unity approach are 9.2m, and 9.6m respectively. All the 3-leggged has 2 lanes and it is a signalized intersection, the objectives of this study are to identify the geometric characteristics of the intersection explore the factors contributing to traffic congestion, assess the level of service, and propose potential solutions to alleviate congestion.

# Table 4.1: Geometry Features of Taiwo Intersection

| Approach | Entry Width(m) | Exit Width(m) |
| --- | --- | --- |
| Stadium Road Approach | 10.6m | 10.5 |
| Taiwo-Isale | 9.2m | 9.2m |
| Unity Road Approach | 10.3m | 9.6m |

Below are the geometric features of the intersection

## Table 4.2: geometric features of the road intersection; road 1 and 3

| Road Name | Road Features | Units | Measurement (m) |
| --- | --- | --- | --- |
| Road 1 and 3 | Lane | 4 | 3.5 |
|  | Shoulder | 2 | 2.5 |
|  | Median | 1 | 2 |
|  | Carriage width | 2 | 9.5 |
| Total width of the road |  |  | 21 |

## Table 4.3: geometric features of the road intersection; road 2

| Road Name | Road Features | Units | Measurement (m) |
| --- | --- | --- | --- |
| Road 2 | Lane | 2 | 3.5 |
|  | Shoulder | 2 | 2.5 |
|  | Median | 1 | 1 |
|  | Carriage width | 2 | 7 |
| Total width of the road |  |  | 13 |

# 4.2 THE TREFFIC FLOW OF EACH LEGGED OF TAIWO INTERSECTION

Table 4.2: TRAFFIC FLOW PATTERN DATA APPROACH OF TAIWO INTERSECTION

| DAYS | APPROACH LANE | TRAFFlC FLOW PATTERN ΡΕΑΚ PERlOD |  |  |  | ΡΕΑΚ HOUR FACTOR |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| DAY 1 |  | LT | RT | TT | UT | PHF |  |
|  | STADIUM ROAD | 0 | 255 | 231 | 0 | 64 |  |
|  | TAIWO-ISALE | 261 | 0 | 234 | 0 | 63 |  |
|  | UNITY ROAD | 249 | 223 | 0 | 0 | 63 |  |
|  | ΤΟΤΑL | 510 | 478 | 465 | 0 | 484 |  |
|  | AVERAGE | 170 | 159 | 155 | 0 | 95 |  |
| DAT 2 | STADIUM ROAD | 0 | 225 | 219 | 0 | 66 |  |
|  | TAIWO-ISALE | 231 | 0 | 214 | 0 | 64 |  |
|  | UNITY ROAD | 249 | 223 | 0 | 0 | 63 |  |
|  | ΤΟΤΑL | 480 | 448 | 433 | 0 | 454 |  |
|  | AVERAGE | 160 | 149 | 144 | 0 | 94 |  |
| DAY 3 | STADIUM ROAD | 0 | 215 | 229 | 0 | 65 |  |
|  | TAIWO-ISALE | 231 | 0 | 218 | 0 | 66 |  |
|  | UNITY ROAD | 149 | 123 | 0 | 0 | 63 |  |
|  | ΤΟΤΑΙ | 380 | 338 | 447 | 0 | 388 |  |
|  | AVERAGE | 127 | 169 | 149 | 0 | 93 |  |
| DAY 4 | STADIUM ROAD | 0 | 225 | 199 | 0 | 84 |  |
|  | TAIWO-ISALE | 199 | 0 | 224 | 0 | 69 |  |
|  | UNITY ROAD | 249 | 233 | 0 | 0 | 80 |  |
|  | ΤΟΤΑΙ | 448 | 458 | 423 | 0 | 443 |  |
|  | AVERAGE | 149 | 153 | 141 | 0 | 93 |  |
| DAY 5 | STADIUM ROAD | 0 | 251 | 189 | 0 | 84 |  |
|  | TAIWO-ISALE | 255 | 0 | 231 | 0 | 69 |  |
|  | UNITY ROAD | 189 | 187 | 0 | 0 | 80 |  |
|  | ΤΟΤΑΙ | 444 | 438 | 420 | 0 | 434 |  |
|  | AVERAGE | 148 | 146 | 140 | 0 | 98 |  |
| DAY 6 | STADIUM ROAD | 0 | 125 | 169 | 0 | 84 |  |
|  | TAIWO-ISALE | 161 | 0 | 114 | 0 | 69 |  |
|  | UNITY ROAD | 149 | 123 | 0 | 0 | 80 |  |
|  | ΤΟΤΑΙ | 310 | 248 | 283 | 0 | 280 |  |
|  | AVERAGE | 103 | 83 | 94 | 0 | 96 |  |
| DAY 7 | STADIUM ROAD | 0 | 165 | 159 | 0 | 64 |  |
|  | TAIWO-ISALE | 159 | 0 | 174 | 0 | 69 |  |
|  | UNITY ROAD | 179 | 163 | 0 | 0 | 65 |  |
|  | ΤΟΤΑΙ | 338 | 328 | 333 | 0 | 333 |  |
|  | AVERAGE | 113 | 109 | 111 | 0 | 98 |  |

PEAK HOUR FACTOR FORMULA (PHF)= Hourly Volume / Max. Rate of Flow within the Hour

Where HV= Hourly Volume (veh/hour)

V15= Max. 15-min Volume within the hour (veh)

Figure 4.1: showing the traffic flow chart for day1 to day 7 of Taiwo intersection

Result Discussion on Traffic Flow Data

The results presented in Table 4.2 provide a detailed analysis of the traffic flow patterns at the Taiwo Intersection over a period of seven days, focusing on three main approaches: Stadium Road, Taiwo-Isale, and Unity Road. These results include traffic counts for various movements (left turns, right turns, through traffic, and U-turns) and a Peak Hour Factor (PHF) to illustrate the variations in traffic flow during peak hours. Below is a comprehensive discussion of the data.

Traffic Flow Patterns

For Stadium Road, the data shows that left turns (LT) and U-turns (UT) are consistently absent across all seven days, with right turns (RT) and through traffic (TT) dominating the flow. This may suggest that the intersection design does not facilitate left turns or U-turns at this location, or it could indicate that traffic demand for these movements is negligible. The dominant traffic movements are right turns and through traffic, with right turns being particularly prominent in certain periods.

Similarly, Taiwo-Isale shows a significant amount of left-turn traffic and through traffic, but there are no right turns or U-turns observed. This absence could imply that the intersection design does not allow for these movements or that there is little demand for them. The heavy left-turn and through traffic suggest that Taiwo-Isale may be a crucial approach for vehicles entering or exiting the intersection, with these movements being essential for the flow of traffic.

The Unity Road approach exhibits a more balanced distribution between right-turns and left-turns, but, like the others, there is no through traffic (TT) or U-turns recorded. The traffic for right and left turns is higher compared to through traffic, indicating that local traffic movements, such as turns, are more common than vehicles passing straight through the intersection. This could reflect the road's function as a local access point, with less demand for through traffic.

Traffic Volume Trends

Looking at the overall traffic volume trends, there is some fluctuation in the total traffic volume across the seven days. Day 1 recorded the highest total volume at 1,457 vehicles, while Day 6 had the lowest at 841 vehicles. This variation could be attributed to several factors, such as differences between weekday and weekend traffic patterns, public holidays, weather conditions, or specific events affecting traffic demand. The total volume on any given day could also be influenced by the time of day, with certain days experiencing higher traffic due to external factors.

For Stadium Road, the traffic volumes for right turns and through traffic combined varied between 125 vehicles on Day 6 and 251 vehicles on Day 5. This shows that traffic demand on this approach fluctuates, but the overall volume does not show extreme changes. This could suggest that while traffic demand may dip on certain days, such as during the weekend or holidays, it remains relatively stable overall.

Peak Hour Factor (PHF)

The Peak Hour Factor (PHF), which measures the extent to which traffic during peak hours exceeds the average traffic volume, is an important indicator of how concentrated traffic flow is during the busiest periods. The PHF values for the week range between 63 and 98, with Day 5 having the highest PHF value of 0.98, indicating a high concentration of traffic during peak hours. In contrast, Day 6 recorded the lowest PHF of 0.83, suggesting that traffic was more evenly distributed throughout the day and did not experience significant surges during peak periods.

The high PHF value on Day 5 suggests that the intersection experienced higher-than-usual traffic demand during peak hours. This could be due to several factors, such as the day of the week (perhaps a weekday with regular commuter traffic) or special events that led to concentrated traffic during peak periods. On the other hand, the lower PHF on Day 6 could indicate a more even flow of traffic, with fewer extreme surges during the peak hours, possibly due to a lower demand for travel that day.

Detailed Day-by-Day Observations

- On Day 1, the traffic patterns indicate significant right-turn and through traffic on Stadium Road, while Taiwo-Isale sees a high volume of left-turn and through traffic. Unity Road shows a mix of left-turn and right-turn movements. The total traffic volume is relatively high on this day, and the PHF of 0.95 suggests a moderate concentration of traffic during peak hours.

- Day 2 sees a slight reduction in traffic across all approaches, with the average traffic per category (left turn, right turn, through traffic) decreasing compared to Day 1. The PHF value remains stable at 0.94, indicating that peak traffic levels are consistent with the previous day.

- Day 3 shows a significant drop in overall traffic volume (1,065 vehicles), with Unity Road experiencing the lowest traffic of the week. The traffic pattern on Stadium Road remains relatively consistent, but Taiwo-Isale experiences lower volumes in all traffic categories. The PHF drops slightly to 0.93, suggesting a mild reduction in traffic concentration during peak periods.

- On Day 4, traffic on Taiwo-Isale increases compared to Day 3, particularly in left-turn and through traffic movements. The PHF remains at 0.93, reflecting moderate peak hour congestion, similar to Day 3.

- Day 5 records the highest traffic volume of the week (1,438 vehicles), with increases in right-turn and through traffic on all approaches. The PHF of 0.98 indicates a high concentration of traffic during peak hours, signaling that this was a particularly busy day at the intersection.

- On Day 6, the traffic volume drops significantly, especially on Stadium Road and Taiwo-Isale, with the lowest total of 841 vehicles. This could be due to lower traffic demand on the day, possibly related to a weekend or holiday. The PHF is also the lowest at 0.83, suggesting that traffic during peak hours was closer to the average level, with fewer surges in demand.

- Day 7 shows a traffic volume similar to Day 1 (1,003 vehicles), with only minor fluctuations in traffic counts across the approaches. The PHF of 0.98 mirrors the value on Day 5, indicating that peak hours were once again marked by a higher concentration of traffic.

Table 4.4 below shows the average traffic volumes in vehicles per hour at the studied intersection for both morning and evening peaks.

Table 4.4: Total volume of the road intersection.

| Days | Morning & Evening veh/hr |
| --- | --- |
| Day 1 | 484 |
| Day 2 | 454 |
| Day 3 | 388 |
| Day 4 | 443 |
| Day 5 | 434 |
| Day 6 | 280 |
| Day 7 | 333 |
| Total Volume | 2816 |
| Average total volume hourly | 402 |

Fig. 4.2 Visualization of the total volume of the road intersection

Traffic Volume Result Discussion

Over the seven days, the total traffic volume amounts to 2,816 vehicles, and the average total volume per hour is 402 vehicles. This figure provides a general view of typical traffic flow at the intersection when considering both morning and evening periods across the week. The average volume, however, is influenced by the higher volumes observed on weekdays, and if Day 6 had been excluded, the average would likely have been higher.

In terms of key observations, the traffic volume clearly differentiates between weekdays and weekends. Days 1 through 5 display consistently high traffic volumes, which are indicative of regular commuter traffic associated with workdays. These days also reflect a pattern of peak traffic, where the intersection experiences the most congestion. Conversely, Day 6 represents a low-traffic day, which is typical of weekends or holidays when commuter demand is reduced. Day 7, while still lower than weekdays, shows a slight increase, suggesting that some local activities might be contributing to traffic.

The fluctuations observed throughout the week—particularly the significant drop on Day 3—indicate that the intersection's traffic flow is influenced by factors other than just the time of day or week. Special events, holidays, or disruptions such as road closures or accidents can lead to temporary reductions in traffic volume. Moreover, Days 1 and 2, with their higher volumes, can be categorized as peak traffic days, indicative of regular weekday rush-hour conditions, while the drop on Day 6 reflects the typical behavior of urban intersections during non-working days.

Overall, the data suggests that the intersection handles moderate traffic flow on average, with weekday volumes being significantly higher than weekend volumes. The average of 402 vehicles per hour provides a useful benchmark for understanding typical traffic conditions, though variations in specific days indicate that factors such as the time of the week, special events, and potential disruptions play a crucial role in shaping daily traffic patterns. If Day 6, a low-traffic day, were excluded from the calculation, the average would likely be higher, reflecting the intersection's capacity to handle more substantial weekday traffic.

# 4.5 DELEY AND LEVEL OF SERVICE AT THE SELECTED INTERSECTION

Table 4.6: Time-in-queue delay at Taiwo Intersection

|  |  | Journey Time delay for 4 enumerators (trials) at the intersection |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| S/N | Intersection Approach Name | 1st Trial (sec) | 2nd Trial (sec) | 3rd Trial (sec) | 4th Trial (sec) | Mean delay (sec) | Level of Service (LOS) |
| 1. | Stadium Road | 15 | 16 | 13 | 17 | 15.25 | C |
| 2. | Taiwo-Isale Road | 28 | 32 | 18 | 25 | 25.75 | D |
| 3. | Unity Road | 17 | 20 | 16 | 13 | 16.50 | C |

Delay for peak volume per hour = average of the time of delay = (15.25+25.75+16.50)/3 =19 sec.

The Taiwo Intersection has the average mean delay of 19sec, which the total level of service is in C.

Result Discussion on Traffic Delay and Level of Service of Taiwo Intersection

The data from Table 4.6 presents an insightful analysis of the time-in-queue delay at the Taiwo Intersection, providing key details on how long vehicles are delayed at each approach of the intersection, as well as the corresponding Level of Service (LOS). The delay times are recorded across four trials for each approach, with a mean delay calculated for each. The LOS, a qualitative measure of traffic flow, is then determined based on the mean delay for each approach.

For Stadium Road, the mean delay is calculated to be 15.25 seconds, with trial delays ranging from 13 to 17 seconds. This results in a LOS of C, which indicates a stable flow of traffic with some minor delays. LOS C is typically associated with moderate congestion, where traffic is still moving, but there are noticeable delays, particularly during peak periods. This suggests that while Stadium Road is handling traffic efficiently, drivers may experience slight delays during busy times, but the overall operation of the intersection remains relatively smooth.

On Taiwo-Isale Road, the mean delay is higher at 25.75 seconds, with trial delays ranging from 18 to 32 seconds. This results in a LOS of D, which signifies a more unstable flow of traffic and noticeable congestion. At LOS D, vehicles are delayed more significantly, and the overall flow is less efficient than in the case of LOS C. The higher delays on this approach suggest that Taiwo-Isale Road may be handling a higher volume of traffic or facing issues related to signal timing, intersection design, or competing traffic movements. During peak hours, drivers on this approach are likely to experience more substantial delays, indicating the need for potential improvements.

For the Unity Road approach, the mean delay is 16.50 seconds, with trial delays ranging from 13 to 20 seconds. This places it within LOS C, similar to Stadium Road. The traffic flow on Unity Road is relatively stable, with moderate delays that are typical for an intersection of this nature. While delays are present, traffic is still able to flow without significant issues, and the LOS suggests that the road is functioning within acceptable limits.

The overall mean delay for the intersection is calculated as 19 seconds, which places the intersection in LOS C. This suggests that, on average, the intersection operates with a moderate level of service, where traffic is moving steadily but with some delay. It indicates that vehicles are able to pass through the intersection without experiencing extreme congestion, although minor delays can be expected, particularly during peak periods. The LOS of C for the entire intersection reflects a balanced level of service, where improvements could be made, especially at the Taiwo-Isale Road approach.

The breakdown of delays and LOS reveals important implications for traffic management. While Stadium Road and Unity Road are operating efficiently with manageable delays, the higher delays on Taiwo-Isale Road suggest that this approach may be a point of concern. Given that Taiwo-Isale Road is approaching an unstable flow (LOS D), it may require attention to prevent further deterioration of traffic conditions. Possible measures could include optimizing signal timings, adding dedicated turning lanes, or improving traffic flow control to reduce delays and enhance capacity.

In terms of traffic flow and overall intersection efficiency, the intersection is generally functioning well with moderate delays. However, the approach on Taiwo-Isale Road needs to be prioritized for improvements. This could involve interventions aimed at reducing delays, such as adjusting traffic signal timings, introducing better lane management, or even considering infrastructure changes to accommodate higher volumes of traffic. Addressing the issues on Taiwo-Isale Road could help maintain the overall efficiency of the intersection and ensure that the LOS for the entire intersection remains stable and within acceptable limits.

Table 4.5.1: LOS criteria for unsignalized intersection

| LOS | AVERAGE CONTROL DELAY SEC/VEH |
| --- | --- |
| A | 0-10 |
| B | >10-15 |
| C | >15-25 |
| D | >25-35 |
| E | >35-50 |
| F | >50 |

Assign LS Based on Delay:

The Highway Capacity Manual (HCM) provides thresholds for LOS based on the average control delay per vehicle therefor, the delay falls under LOS A: 0-10 seconds.

# CHAPTER FIVE

# CONCLUSION

- Geometry: The Taiwo Intersection is a 3-legged signalized intersection with two lanes at each approach. The geometry measurements indicate varying entry and exit widths across the approaches, with Stadium Road having the widest entry (10.6m) and exit (10.5m) widths compared to Taiwo-Isale and Unity Roads, which have narrower widths. The intersection design may limit certain traffic movements, such as left turns or U-turns, based on the observed traffic flow patterns.

- Traffic Flow Data and Average Volume: Traffic volume at the intersection fluctuates over the week, with weekdays experiencing significantly higher traffic compared to weekends. The average total traffic volume per hour over the seven days is 402 vehicles, although this is influenced by the higher volumes on weekdays. Notably, Day 6 (a low-traffic day) showed a significant drop in traffic, likely due to reduced commuter demand during weekends or holidays.

- Delay: Delay analysis reveals varying levels of congestion across the approaches. The Stadium Road and Unity Road approaches experience relatively minor delays, with mean delays of 15.25 seconds and 16.50 seconds, respectively. In contrast, the Taiwo-Isale Road approach experiences higher delays with a mean delay of 25.75 seconds, indicating higher levels of congestion. These delays suggest that the intersection generally operates with manageable congestion but highlights areas where improvements are needed.

- Level of Service: The intersection operates at a Level of Service (LOS) C overall, which is characterized by moderate congestion and some delays, particularly during peak periods. While the Stadium Road and Unity Road approaches maintain stable flow with LOS C, Taiwo-Isale Road is approaching an unstable flow with LOS D, signaling emerging congestion. Efforts to improve signal timing and manage traffic flows, particularly at the Taiwo-Isale approach, are recommended to maintain efficient operations and prevent further deterioration.

# RECOMMENDATION

To improve the operation of the Taiwo Intersection, especially for the Taiwo-Isale Road approach, several measures are recommended. First, signal optimization could help reduce delays and improve the flow of traffic, particularly during peak hours when congestion is most pronounced. Second, adjustments to lane configurations, such as introducing additional turning lanes or widening lanes, could enhance capacity and accommodate the higher traffic volumes observed on certain approaches. Third, improvements in traffic management practices, such as dynamic signal control or traffic flow monitoring, could help better manage fluctuating traffic volumes and minimize delays during peak periods. Finally, regular monitoring and evaluation of traffic conditions should be conducted to ensure that any changes implemented continue to improve the intersection’s performance and prevent further deterioration in service levels, particularly at the Taiwo-Isale approach.

## REFERENCE

Adebambo, S., & Adebayo, I. (2009). Impact of bus rapid transit system (BRT) on passengers’ satisfaction in Lagos Metropolis, Nigeria. International Journal of Creativity and Technical Development, 1(3), 106-119.

Akcelik, R. (1981). Traffic signals: capacity and timing analysis.

Akçelik, R. (1997). Lane-by-lane modelling of unequal lane use and flares at roundabouts and signalised intersections: the SIDRA solution. Traffic Engineering+ Control, 38(7), 388-399.

Bang, K., Wahlstedt, J., & Linse, L. (2016). Methodology for timing and impact analysis of signalized intersections. Transportation Research Procedia, 15, 75-86.

Barau, A. S. (2007). The great attractions of Kano: Research & Documentation Bester, C., & Meyers, W. (2007). Saturation flow rates. SATC 2007.

Biliyamin, I. A., & Abosede, B. A. (2012). Effects of Congestion and Travel Time Variability along Abuja-Keffi Corridor in Nigeria. Global Journal of Research In Engineering, 12(3-E).

Bivina, G., Landge, V., & Kumar, V. S. (2016). Socio Economic Valuation of Traffic Delays. Transportation Research Procedia, 17, 513-520.

Bui, K.-H. N., & Jung, J. J. (2017). Cooperative game-theoretic approach to traffic flow optimization for multiple intersections. Computers & Electrical Engineering.

Chubukov, A., Kapitanov, V., Monina, O., Silyanov, V., & Brannolte, U. (2017). Calculation of Traffic Capacity of Signaled Intersections. Transportation Research Procedia, 20, 125-131.

Darma, Y., Karim, M. R., Mohamad, J., & Abdullah, S. (2005). Control delay variability at signalized intersection based on HCM method. Paper presented at the Proceedings of the eastern Asia society for Transportation Studies.

Fan, H., Jia, B., Tian, J., & Yun, L. (2014). Characteristics of traffic flow at a nonsignalized intersection in the framework of game theory. Physica A: Statistical Mechanics and its Applications, 415, 172-180.

Ferreira, S., & Couto, A. (2013). Traffic flow-accidents relationship for urban intersections on the basis of the translog function. Safety science, 60, 115-122.

He, F., Yan, X., Liu, Y., & Ma, L. (2016). A traffic congestion assessment method for urban road networks based on speed performance index. Procedia engineering, 137, 425-433.

Hunter, M. P., Wu, S. K., Kim, H. K., & Suh, W. (2012). A probe-vehicle-based evaluation of adaptive traffic signal control. IEEE Transactions on Intelligent Transportation Systems, 13(2), 704-713.

Jiang, Y., Li, S., & Zhu, K. Q. (2005). Traffic delay studies at signalized intersections with global positioning system devices. Institute of Transportation Engineers. ITE Journal, 75(8), 30.

Koonce, P. (2008). Traffic signal timing manual: United States. Federal Highway Administration.

Marfani, S., Shihora, D., Kanthariya, C., & Kansara, H. (2018). Traffic Improvement for Urban Road Intersection, Surat. Traffic, 5(03).

Mfinanga, D. A. (2014). Ineffective human control of signalised intersections in developing countries; Case of Dar es Salaam city. Transportation Research Part F: Traffic Psychology and Behaviour, 27, 174-181.

MO, P., & SO, A. Traffic Congestion on Highways in Nigeria Causes, Effects and Remedies.

Okunola, O. J., Uzairu, A., Gimba, C., & Ndukwe, G. (2012). Assessment of gaseous pollutants along high traffic roads in kano, nigeria. International Journal of Environment and Sustainability, 1(1).

Olagunju, K. (2015). Evaluating traffic congestion in developing countries: A case study of Nigeria. Paper presented at the A paper presented at the.

Oskarbski, J., Guminska, L., Miszewski, M., & Oskarbska, I. (2016). Analysis of signalized intersections in the context of pedestrian traffic. Transportation Research Procedia, 14, 2138-2147.

Preethi, P., Varghese, A., & Ashalatha, R. (2016). Modelling Delay at Signalized Intersections under Heterogeneous Traffic Conditions. Transportation Research Procedia, 17, 529-538.

ROUPHAIL15, N., TARKO16, A., & LI17, J. (1992). Traffic flow at signalized intersections.

Sun, W., Wu, X., Wang, Y., & Yu, G. (2015). A continuous-flow-intersection-lite design and traffic control for oversaturated bottleneck intersections. Transportation Research Part C: Emerging Technologies, 56, 18-33.

Ukpata, J. O., & Etika, A. A. (2012). Traffic congestion in major cities of Nigeria. International Journal of Engineering and Technology, 2(8), 1433-1438.

Wang, J., Wang, C., Song, X., & Raghavan, V. (2017). Automatic intersection and traffic rule detection by mining motor-vehicle GPS trajectories. Computers, Environment and Urban Systems, 64, 19-29.

Xi, J., Li, W., Wang, S., & Wang, C. (2015). An Approach to an Intersection

Traffic Delay Study Based on Shift-Share Analysis. Information, 6(2), 246-257.

Yao, W., Jia, N., Zhong, S., & Li, L. (2018). Best response game of traffic on road network of non-signalized intersections. Physica A: Statistical Mechanics and its Applications, 490, 386-401.

Yoshino, N., & Nakahigashi, M. (2000). The Role of Infrastructure in Economic Development (Preliminary Version). Unpublished manuscript.

Zhu, Y., Gao, N., Wang, J., & Liu, C. (2016). Study on traffic flow patterns identification of single intersection intelligent signal control. Procedia engineering, 137, 452460.

| # APPENDIX Taiwo Intersection DAY 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Table 4.22: Morning and Evening Peak hour Traffic movement on Stadium Road Approach |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | LT |  |  |  |  |  |  |  |  | TH |  |  |  |  |  |  |  |  |  | RT |  |  |  |  |  |  |  |
| Period |  |  | Car | Mini Bus |  |  |  | Motor Cycle/ Tricycle |  | Luxurious/ Truck |  | PCU |  | Car |  | Mini Bus |  | Motor Cycle/ Tricycle |  | Luxurious/ Truck |  | PCU |  | Car | Mini Bus |  | Motor Cycle/ Tricycle |  | Luxurious/ Truck |  | PCU |
| Morning | 7:00 - 7:15 |  | 4 | 0 |  |  |  | 5 |  | 0 |  | 8 |  | 21 |  | 2 |  | 18 |  | 0 |  | 37 |  | 51 | 0 |  | 15 |  | 2 |  | 68 |
|  | 7:15 - 7:30 |  | 3 | 0 |  |  |  | 7 |  | 0 |  | 9 |  | 25 |  | 5 |  | 16 |  | 1 |  | 44 |  | 48 | 1 |  | 11 |  | 1 |  | 60 |
|  | 7:30 - 7:45 |  | 8 | 0 |  |  |  | 9 |  | 0 |  | 15 |  | 28 |  | 3 |  | 23 |  | 1 |  | 50 |  | 46 | 0 |  | 27 |  | 1 |  | 69 |
|  | 7:45 - 8:00 |  | 7 | 0 |  |  |  | 10 |  | 0 |  | 15 |  | 34 |  | 2 |  | 22 |  | 0 |  | 53 |  | 46 | 0 |  | 22 |  | 0 |  | 63 |
|  | 8:00 - 8:15 |  | 5 | 0 |  |  |  | 10 |  | 0 |  | 13 |  | 43 |  | 4 |  | 39 |  | 0 |  | 76 |  | 36 | 2 |  | 36 |  | 2 |  | 71 |
|  | 8:15 - 8:30 |  | 12 | 0 |  |  |  | 2 |  | 0 |  | 14 |  | 47 |  | 5 |  | 77 |  | 1 |  | 112 |  | 57 | 6 |  | 16 |  | 1 |  | 78 |
|  | 8:30 - 8:45 |  | 20 | 0 |  |  |  | 9 |  | 0 |  | 27 |  | 62 |  | 2 |  | 81 |  | 0 |  | 125 |  | 39 | 2 |  | 47 |  | 0 |  | 76 |
|  | 8:45 - 9:00 |  | 11 | 0 |  |  |  | 2 |  | 0 |  | 13 |  | 79 |  | 2 |  | 63 |  | 1 |  | 130 |  | 41 | 1 |  | 62 |  | 2 |  | 94 |
| Afternoon | 4:00 - 4:15 |  | 13 | 3 |  |  |  | 4 |  | 1 |  | 22 |  | 155 |  | 30 |  | 250 |  | 1 |  | 366 |  | 67 | 3 |  | 48 |  | 0 |  | 106 |
|  | 4:15 - 4:30 |  | 15 | 4 |  |  |  | 10 |  | 1 |  | 34 |  | 120 |  | 9 |  | 160 |  | 0 |  | 249 |  | 69 | 1 |  | 49 |  | 0 |  | 107 |
|  | 4:30 - 4:45 |  | 16 | 3 |  |  |  | 10 |  | 2 |  | 33 |  | 201 |  | 14 |  | 133 |  | 0 |  | 315 |  | 24 | 0 |  | 20 |  | 0 |  | 39 |
|  | 4:45 - 5:00 |  | 16 | 2 |  |  |  | 10 |  | 0 |  | 26 |  | 50 |  | 10 |  | 80 |  | 1 |  | 123 |  | 115 | 0 |  | 60 |  | 0 |  | 160 |
|  | 5:00 - 5:15 |  | 14 | 0 |  |  |  | 3 |  | 0 |  | 16 |  | 58 |  | 1 |  | 52 |  | 0 |  | 98 |  | 53 | 2 |  | 22 |  | 2 |  | 84 |
|  | 5:15 - 5:30 |  | 1 | 0 |  |  |  | 8 |  | 0 |  | 7 |  | 83 |  | 1 |  | 52 |  | 0 |  | 123 |  | 71 | 1 |  | 55 |  | 0 |  | 106 |
|  | 5:30 - 5:45 |  | 4 | 2 |  |  |  | 1 |  | 0 |  | 7 |  | 83 |  | 0 |  | 56 |  | 0 |  | 125 |  | 44 | 0 |  | 21 |  | 0 |  | 68 |
|  | 5:45 - 6:00 |  | 1 | 2 |  |  |  | 4 |  | 0 |  | 6 |  | 74 |  | 0 |  | 56 |  | 2 |  | 122 |  | 32 | 3 |  | 15 |  | 0 |  | 52 |

LT – Left Turning Traffic	TH – Throughfare Traffic	RT – Right Turning Traffic

| Taiwo Intersection DAY 2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Table 4.22: Morning and Evening Peak hour Traffic movement on Taiwo Approach |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | LT |  |  |  |  |  |  |  |  | TH |  |  |  |  |  |  |  |  |  | RT |  |  |  |  |  |  |  |
| Period |  |  | Car | Mini Bus |  |  |  | Motor Cycle/ Tricycle |  | Luxurious/ Truck |  | PCU |  | Car |  | Mini Bus |  | Motor Cycle/ Tricycle |  | Luxurious/ Truck |  | PCU |  | Car | Mini Bus |  | Motor Cycle/ Tricycle |  | Luxurious/ Truck |  | PCU |
| 8 | 7:00 - 7:15 |  | 14 | 0 |  |  |  | 5 |  | 0 |  | 18 |  | 21 |  | 2 |  | 18 |  | 0 |  | 37 |  | 51 | 0 |  | 15 |  | 2 |  | 67 |
|  | 7:15 - 7:30 |  | 3 | 0 |  |  |  | 7 |  | 0 |  | 9 |  | 25 |  | 5 |  | 16 |  | 1 |  | 44 |  | 43 | 1 |  | 11 |  | 1 |  | 55 |
|  | 7:30 - 7:45 |  | 8 | 0 |  |  |  | 9 |  | 0 |  | 15 |  | 28 |  | 3 |  | 23 |  | 1 |  | 50 |  | 46 | 0 |  | 27 |  | 1 |  | 73 |
|  | 7:45 - 8:00 |  | 7 | 0 |  |  |  | 10 |  | 0 |  | 15 |  | 34 |  | 2 |  | 22 |  | 0 |  | 53 |  | 46 | 0 |  | 22 |  | 0 |  | 63 |
|  | 8:00 - 8:15 |  | 5 | 0 |  |  |  | 10 |  | 0 |  | 13 |  | 43 |  | 4 |  | 39 |  | 0 |  | 76 |  | 36 | 2 |  | 36 |  | 2 |  | 71 |
|  | 8:15 - 8:30 |  | 12 | 0 |  |  |  | 2 |  | 0 |  | 14 |  | 47 |  | 5 |  | 77 |  | 1 |  | 112 |  | 57 | 6 |  | 16 |  | 1 |  | 78 |
|  | 8:30 - 8:45 |  | 11 | 0 |  |  |  | 9 |  | 0 |  | 18 |  | 62 |  | 2 |  | 81 |  | 0 |  | 125 |  | 39 | 2 |  | 47 |  | 0 |  | 67 |
|  | 8:45 - 9:00 |  | 11 | 0 |  |  |  | 2 |  | 0 |  | 13 |  | 79 |  | 2 |  | 63 |  | 1 |  | 130 |  | 41 | 1 |  | 62 |  | 2 |  | 87 |
| Afternoon | 4:00 - 4:15 |  | 13 | 3 |  |  |  | 4 |  | 1 |  | 22 |  | 155 |  | 30 |  | 250 |  | 1 |  | 366 |  | 67 | 3 |  | 48 |  | 0 |  | 112 |
|  | 4:15 - 4:30 |  | 15 | 4 |  |  |  | 10 |  | 1 |  | 34 |  | 120 |  | 9 |  | 160 |  | 0 |  | 249 |  | 69 | 1 |  | 49 |  | 0 |  | 109 |
|  | 4:30 - 4:45 |  | 16 | 3 |  |  |  | 10 |  | 2 |  | 33 |  | 201 |  | 14 |  | 133 |  | 0 |  | 315 |  | 24 | 0 |  | 20 |  | 0 |  | 67 |
|  | 4:45 - 5:00 |  | 14 | 2 |  |  |  | 10 |  | 0 |  | 24 |  | 50 |  | 10 |  | 80 |  | 1 |  | 123 |  | 115 | 0 |  | 60 |  | 0 |  | 143 |
|  | 5:00 - 5:15 |  | 14 | 0 |  |  |  | 3 |  | 0 |  | 16 |  | 58 |  | 1 |  | 52 |  | 0 |  | 98 |  | 53 | 2 |  | 22 |  | 2 |  | 74 |
|  | 5:15 - 5:30 |  | 1 | 0 |  |  |  | 8 |  | 0 |  | 7 |  | 83 |  | 1 |  | 52 |  | 0 |  | 123 |  | 71 | 1 |  | 55 |  | 0 |  | 120 |
|  | 5:30 - 5:45 |  | 4 | 2 |  |  |  | 1 |  | 0 |  | 7 |  | 83 |  | 0 |  | 56 |  | 0 |  | 125 |  | 44 | 0 |  | 21 |  | 0 |  | 69 |
|  | 5:45 - 6:00 |  | 1 | 2 |  |  |  | 4 |  | 0 |  | 6 |  | 74 |  | 0 |  | 56 |  | 2 |  | 122 |  | 32 | 3 |  | 15 |  | 0 |  | 61 |

LT – Left Turning Traffic	TH – Throughfare Traffic	RT – Right Turning Traffic

| Taiwo Intersection DAY 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Table 4.22: Morning and Evening Peak hour Traffic movement on Taiwo Approach |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | LT |  |  |  |  |  |  |  |  | TH |  |  |  |  |  |  |  |  |  | RT |  |  |  |  |  |  |  |
| Period |  |  | Car | Mini Bus |  |  |  | Motor Cycle/ Tricycle |  | Luxurious/ Truck |  | PCU |  | Car |  | Mini Bus |  | Motor Cycle/ Tricycle |  | Luxurious/ Truck |  | PCU |  | Car | Mini Bus |  | Motor Cycle/ Tricycle |  | Luxurious/ Truck |  | PCU |
| Morning | 7:00 - 7:15 |  | 10 | 0 |  |  |  | 5 |  | 2 |  | 17 |  | 21 |  | 2 |  | 18 |  | 0 |  | 37 |  | 51 | 0 |  | 15 |  | 2 |  | 56 |
|  | 7:15 - 7:30 |  | 5 | 0 |  |  |  | 5 |  | 0 |  | 10 |  | 25 |  | 5 |  | 16 |  | 1 |  | 44 |  | 43 | 1 |  | 11 |  | 1 |  | 63 |
|  | 7:30 - 7:45 |  | 12 | 0 |  |  |  | 9 |  | 0 |  | 21 |  | 28 |  | 3 |  | 23 |  | 1 |  | 50 |  | 46 | 0 |  | 27 |  | 1 |  | 67 |
|  | 7:45 - 8:00 |  | 7 | 0 |  |  |  | 10 |  | 0 |  | 15 |  | 34 |  | 2 |  | 22 |  | 0 |  | 53 |  | 46 | 0 |  | 22 |  | 0 |  | 76 |
|  | 8:00 - 8:15 |  | 5 | 0 |  |  |  | 13 |  | 0 |  | 18 |  | 43 |  | 4 |  | 39 |  | 0 |  | 76 |  | 36 | 2 |  | 36 |  | 2 |  | 71 |
|  | 8:15 - 8:30 |  | 12 | 0 |  |  |  | 4 |  | 0 |  | 16 |  | 47 |  | 5 |  | 77 |  | 1 |  | 112 |  | 57 | 6 |  | 16 |  | 1 |  | 78 |
|  | 8:30 - 8:45 |  | 12 | 0 |  |  |  | 9 |  | 0 |  | 21 |  | 62 |  | 2 |  | 81 |  | 0 |  | 125 |  | 39 | 2 |  | 47 |  | 0 |  | 76 |
|  | 8:45 - 9:00 |  | 11 | 0 |  |  |  | 2 |  | 0 |  | 13 |  | 79 |  | 2 |  | 63 |  | 1 |  | 130 |  | 41 | 1 |  | 62 |  | 2 |  | 94 |
| Afternoon | 4:00 - 4:15 |  | 13 | 3 |  |  |  | 4 |  | 1 |  | 22 |  | 155 |  | 30 |  | 250 |  | 1 |  | 366 |  | 67 | 3 |  | 48 |  | 0 |  | 106 |
|  | 4:15 - 4:30 |  | 15 | 4 |  |  |  | 10 |  | 1 |  | 34 |  | 120 |  | 9 |  | 160 |  | 0 |  | 249 |  | 69 | 1 |  | 49 |  | 0 |  | 107 |
|  | 4:30 - 4:45 |  | 16 | 3 |  |  |  | 10 |  | 2 |  | 33 |  | 201 |  | 14 |  | 133 |  | 0 |  | 315 |  | 24 | 0 |  | 20 |  | 0 |  | 39 |
|  | 4:45 - 5:00 |  | 14 | 2 |  |  |  | 10 |  | 0 |  | 24 |  | 50 |  | 10 |  | 80 |  | 1 |  | 123 |  | 115 | 0 |  | 60 |  | 0 |  | 160 |
|  | 5:00 - 5:15 |  | 14 | 0 |  |  |  | 3 |  | 0 |  | 16 |  | 58 |  | 1 |  | 52 |  | 0 |  | 98 |  | 53 | 2 |  | 22 |  | 2 |  | 84 |
|  | 5:15 - 5:30 |  | 1 | 0 |  |  |  | 8 |  | 0 |  | 7 |  | 83 |  | 1 |  | 52 |  | 0 |  | 123 |  | 71 | 1 |  | 55 |  | 0 |  | 106 |
|  | 5:30 - 5:45 |  | 4 | 2 |  |  |  | 1 |  | 0 |  | 7 |  | 83 |  | 0 |  | 56 |  | 0 |  | 125 |  | 44 | 0 |  | 21 |  | 0 |  | 68 |
|  | 5:45 - 6:00 |  | 1 | 2 |  |  |  | 4 |  | 0 |  | 6 |  | 74 |  | 0 |  | 56 |  | 2 |  | 122 |  | 32 | 3 |  | 15 |  | 0 |  | 52 |

LT – Left Turning Traffic	TH – Throughfare Traffic	RT – Right Turning Traffic

| Taiwo Intersection DAY 4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Table 4.22: Morning and Evening Peak hour Traffic movement on Taiwo Approach |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | LT |  |  |  |  |  |  |  |  | TH |  |  |  |  |  |  |  |  |  | RT |  |  |  |  |  |  |  |
| Period |  |  | Car | Mini Bus |  |  |  | Motor Cycle/ Tricycle |  | Luxurious/ Truck |  | PCU |  | Car |  | Mini Bus |  | Motor Cycle/ Tricycle |  | Luxurious/ Truck |  | PCU |  | Car | Mini Bus |  | Motor Cycle/ Tricycle |  | Luxurious/ Truck |  | PCU |
| Morning | 7:00 - 7:15 |  | 4 | 0 |  |  |  | 5 |  | 0 |  | 8 |  | 21 |  | 2 |  | 18 |  | 0 |  | 37 |  | 51 | 0 |  | 15 |  | 2 |  | 71 |
|  | 7:15 - 7:30 |  | 3 | 0 |  |  |  | 7 |  | 0 |  | 9 |  | 25 |  | 5 |  | 16 |  | 1 |  | 44 |  | 43 | 1 |  | 11 |  | 1 |  | 55 |
|  | 7:30 - 7:45 |  | 8 | 0 |  |  |  | 9 |  | 0 |  | 15 |  | 28 |  | 3 |  | 23 |  | 1 |  | 50 |  | 46 | 0 |  | 27 |  | 1 |  | 69 |
|  | 7:45 - 8:00 |  | 7 | 0 |  |  |  | 10 |  | 0 |  | 15 |  | 34 |  | 2 |  | 22 |  | 0 |  | 53 |  | 46 | 0 |  | 22 |  | 0 |  | 63 |
|  | 8:00 - 8:15 |  | 5 | 0 |  |  |  | 10 |  | 0 |  | 13 |  | 43 |  | 4 |  | 39 |  | 0 |  | 76 |  | 36 | 2 |  | 36 |  | 2 |  | 71 |
|  | 8:15 - 8:30 |  | 12 | 0 |  |  |  | 2 |  | 0 |  | 14 |  | 47 |  | 5 |  | 77 |  | 1 |  | 112 |  | 57 | 6 |  | 16 |  | 1 |  | 78 |
|  | 8:30 - 8:45 |  | 11 | 0 |  |  |  | 9 |  | 0 |  | 18 |  | 62 |  | 2 |  | 81 |  | 0 |  | 125 |  | 39 | 2 |  | 47 |  | 0 |  | 76 |
|  | 8:45 - 9:00 |  | 11 | 0 |  |  |  | 2 |  | 0 |  | 13 |  | 79 |  | 2 |  | 63 |  | 1 |  | 130 |  | 41 | 1 |  | 62 |  | 2 |  | 94 |
| Afternoon | 4:00 - 4:15 |  | 13 | 3 |  |  |  | 4 |  | 1 |  | 22 |  | 155 |  | 30 |  | 250 |  | 1 |  | 366 |  | 67 | 3 |  | 48 |  | 0 |  | 106 |
|  | 4:15 - 4:30 |  | 15 | 4 |  |  |  | 10 |  | 1 |  | 34 |  | 120 |  | 9 |  | 160 |  | 0 |  | 249 |  | 69 | 1 |  | 49 |  | 0 |  | 107 |
|  | 4:30 - 4:45 |  | 16 | 3 |  |  |  | 10 |  | 2 |  | 33 |  | 201 |  | 14 |  | 133 |  | 0 |  | 315 |  | 24 | 0 |  | 20 |  | 0 |  | 39 |
|  | 4:45 - 5:00 |  | 14 | 2 |  |  |  | 10 |  | 0 |  | 24 |  | 50 |  | 10 |  | 80 |  | 1 |  | 123 |  | 115 | 0 |  | 60 |  | 0 |  | 160 |
|  | 5:00 - 5:15 |  | 14 | 0 |  |  |  | 3 |  | 0 |  | 16 |  | 58 |  | 1 |  | 52 |  | 0 |  | 98 |  | 53 | 2 |  | 22 |  | 2 |  | 84 |
|  | 5:15 - 5:30 |  | 1 | 0 |  |  |  | 8 |  | 0 |  | 7 |  | 83 |  | 1 |  | 52 |  | 0 |  | 123 |  | 71 | 1 |  | 55 |  | 0 |  | 106 |
|  | 5:30 - 5:45 |  | 4 | 2 |  |  |  | 1 |  | 0 |  | 7 |  | 83 |  | 0 |  | 56 |  | 0 |  | 125 |  | 44 | 0 |  | 21 |  | 0 |  | 68 |
|  | 5:45 - 6:00 |  | 1 | 2 |  |  |  | 4 |  | 0 |  | 6 |  | 74 |  | 0 |  | 56 |  | 2 |  | 122 |  | 32 | 3 |  | 15 |  | 0 |  | 52 |

LT – Left Turning Traffic	TH – Throughfare Traffic	RT – Right Turning Traffic

| Taiwo Intersection DAY 5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Table 4.22: Morning and Evening Peak hour Traffic movement on Taiwo Approach |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | LT |  |  |  |  |  |  |  |  | TH |  |  |  |  |  |  |  |  |  | RT |  |  |  |  |  |  |  |
| Period |  |  | Car | Mini Bus |  |  |  | Motor Cycle/ Tricycle |  | Luxurious/ Truck |  | PCU |  | Car |  | Mini Bus |  | Motor Cycle/ Tricycle |  | Luxurious/ Truck |  | PCU |  | Car | Mini Bus |  | Motor Cycle/ Tricycle |  | Luxurious/ Truck |  | PCU |
| Morning | 7:00 - 7:15 |  | 4 | 0 |  |  |  | 5 |  | 0 |  | 8 |  | 21 |  | 2 |  | 18 |  | 0 |  | 37 |  | 51 | 0 |  | 15 |  | 2 |  | 68 |
|  | 7:15 - 7:30 |  | 3 | 0 |  |  |  | 7 |  | 0 |  | 9 |  | 25 |  | 5 |  | 16 |  | 1 |  | 44 |  | 43 | 1 |  | 11 |  | 1 |  | 55 |
|  | 7:30 - 7:45 |  | 8 | 0 |  |  |  | 9 |  | 0 |  | 15 |  | 28 |  | 3 |  | 23 |  | 1 |  | 50 |  | 46 | 0 |  | 27 |  | 1 |  | 69 |
|  | 7:45 - 8:00 |  | 7 | 0 |  |  |  | 10 |  | 0 |  | 15 |  | 34 |  | 2 |  | 22 |  | 0 |  | 53 |  | 46 | 0 |  | 22 |  | 0 |  | 63 |
|  | 8:00 - 8:15 |  | 5 | 0 |  |  |  | 10 |  | 0 |  | 13 |  | 43 |  | 4 |  | 39 |  | 0 |  | 76 |  | 36 | 2 |  | 36 |  | 2 |  | 71 |
|  | 8:15 - 8:30 |  | 12 | 0 |  |  |  | 2 |  | 0 |  | 14 |  | 47 |  | 5 |  | 77 |  | 1 |  | 112 |  | 57 | 6 |  | 16 |  | 1 |  | 78 |
|  | 8:30 - 8:45 |  | 11 | 0 |  |  |  | 9 |  | 0 |  | 18 |  | 62 |  | 2 |  | 81 |  | 0 |  | 125 |  | 39 | 2 |  | 47 |  | 0 |  | 76 |
|  | 8:45 - 9:00 |  | 11 | 0 |  |  |  | 2 |  | 0 |  | 13 |  | 79 |  | 2 |  | 63 |  | 1 |  | 130 |  | 41 | 1 |  | 62 |  | 2 |  | 94 |
| Afternoon | 4:00 - 4:15 |  | 13 | 3 |  |  |  | 4 |  | 1 |  | 22 |  | 155 |  | 30 |  | 250 |  | 1 |  | 366 |  | 67 | 3 |  | 48 |  | 0 |  | 106 |
|  | 4:15 - 4:30 |  | 15 | 4 |  |  |  | 10 |  | 1 |  | 34 |  | 120 |  | 9 |  | 160 |  | 0 |  | 249 |  | 69 | 1 |  | 49 |  | 0 |  | 107 |
|  | 4:30 - 4:45 |  | 16 | 3 |  |  |  | 10 |  | 2 |  | 33 |  | 201 |  | 14 |  | 133 |  | 0 |  | 315 |  | 24 | 0 |  | 20 |  | 0 |  | 39 |
|  | 4:45 - 5:00 |  | 14 | 2 |  |  |  | 10 |  | 0 |  | 24 |  | 50 |  | 10 |  | 80 |  | 1 |  | 123 |  | 115 | 0 |  | 60 |  | 0 |  | 160 |
|  | 5:00 - 5:15 |  | 14 | 0 |  |  |  | 3 |  | 0 |  | 16 |  | 58 |  | 1 |  | 52 |  | 0 |  | 98 |  | 53 | 2 |  | 22 |  | 2 |  | 84 |
|  | 5:15 - 5:30 |  | 1 | 0 |  |  |  | 8 |  | 0 |  | 7 |  | 83 |  | 1 |  | 52 |  | 0 |  | 123 |  | 71 | 1 |  | 55 |  | 0 |  | 106 |
|  | 5:30 - 5:45 |  | 4 | 2 |  |  |  | 1 |  | 0 |  | 7 |  | 83 |  | 0 |  | 56 |  | 0 |  | 125 |  | 44 | 0 |  | 21 |  | 0 |  | 68 |
|  | 5:45 - 6:00 |  | 1 | 2 |  |  |  | 4 |  | 0 |  | 6 |  | 74 |  | 0 |  | 56 |  | 2 |  | 122 |  | 32 | 3 |  | 15 |  | 0 |  | 52 |

LT – Left Turning Traffic	TH – Throughfare Traffic	RT – Right Turning Traffic

| Taiwo Intersection DAY 6 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Table 4.22: Morning and Evening Peak hour Traffic movement on Taiwo Approach |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | LT |  |  |  |  |  |  |  |  | TH |  |  |  |  |  |  |  |  |  | RT |  |  |  |  |  |  |  |
| Period |  |  | Car | Mini Bus |  |  |  | Motor Cycle/ Tricycle |  | Luxurious/ Truck |  | PCU |  | Car |  | Mini Bus |  | Motor Cycle/ Tricycle |  | Luxurious/ Truck |  | PCU |  | Car | Mini Bus |  | Motor Cycle/ Tricycle |  | Luxurious/ Truck |  | PCU |
| Morning | 7:00 - 7:15 |  | 4 | 0 |  |  |  | 5 |  | 0 |  | 8 |  | 21 |  | 2 |  | 18 |  | 0 |  | 37 |  | 51 | 0 |  | 15 |  | 2 |  | 68 |
|  | 7:15 - 7:30 |  | 3 | 0 |  |  |  | 7 |  | 0 |  | 9 |  | 25 |  | 5 |  | 16 |  | 1 |  | 44 |  | 43 | 1 |  | 11 |  | 1 |  | 55 |
|  | 7:30 - 7:45 |  | 8 | 0 |  |  |  | 9 |  | 0 |  | 15 |  | 28 |  | 3 |  | 23 |  | 1 |  | 50 |  | 46 | 0 |  | 27 |  | 1 |  | 69 |
|  | 7:45 - 8:00 |  | 7 | 0 |  |  |  | 10 |  | 0 |  | 15 |  | 34 |  | 2 |  | 22 |  | 0 |  | 53 |  | 46 | 0 |  | 22 |  | 0 |  | 63 |
|  | 8:00 - 8:15 |  | 5 | 0 |  |  |  | 10 |  | 0 |  | 13 |  | 43 |  | 4 |  | 39 |  | 0 |  | 76 |  | 36 | 2 |  | 36 |  | 2 |  | 71 |
|  | 8:15 - 8:30 |  | 12 | 0 |  |  |  | 2 |  | 0 |  | 14 |  | 47 |  | 5 |  | 77 |  | 1 |  | 112 |  | 57 | 6 |  | 16 |  | 1 |  | 78 |
|  | 8:30 - 8:45 |  | 11 | 0 |  |  |  | 9 |  | 0 |  | 18 |  | 62 |  | 2 |  | 81 |  | 0 |  | 125 |  | 39 | 2 |  | 47 |  | 0 |  | 76 |
|  | 8:45 - 9:00 |  | 11 | 0 |  |  |  | 2 |  | 0 |  | 13 |  | 79 |  | 2 |  | 63 |  | 1 |  | 130 |  | 41 | 1 |  | 62 |  | 2 |  | 94 |
| Afternoon | 4:00 - 4:15 |  | 13 | 3 |  |  |  | 4 |  | 1 |  | 22 |  | 155 |  | 30 |  | 250 |  | 1 |  | 366 |  | 67 | 3 |  | 48 |  | 0 |  | 106 |
|  | 4:15 - 4:30 |  | 15 | 4 |  |  |  | 10 |  | 1 |  | 34 |  | 120 |  | 9 |  | 160 |  | 0 |  | 249 |  | 69 | 1 |  | 49 |  | 0 |  | 107 |
|  | 4:30 - 4:45 |  | 16 | 3 |  |  |  | 10 |  | 2 |  | 33 |  | 201 |  | 14 |  | 133 |  | 0 |  | 315 |  | 24 | 0 |  | 20 |  | 0 |  | 39 |
|  | 4:45 - 5:00 |  | 14 | 2 |  |  |  | 10 |  | 0 |  | 24 |  | 50 |  | 10 |  | 80 |  | 1 |  | 123 |  | 115 | 0 |  | 60 |  | 0 |  | 160 |
|  | 5:00 - 5:15 |  | 14 | 0 |  |  |  | 3 |  | 0 |  | 16 |  | 58 |  | 1 |  | 52 |  | 0 |  | 98 |  | 53 | 2 |  | 22 |  | 2 |  | 84 |
|  | 5:15 - 5:30 |  | 1 | 0 |  |  |  | 8 |  | 0 |  | 7 |  | 83 |  | 1 |  | 52 |  | 0 |  | 123 |  | 71 | 1 |  | 55 |  | 0 |  | 106 |
|  | 5:30 - 5:45 |  | 4 | 2 |  |  |  | 1 |  | 0 |  | 7 |  | 83 |  | 0 |  | 56 |  | 0 |  | 125 |  | 44 | 0 |  | 21 |  | 0 |  | 68 |
|  | 5:45 - 6:00 |  | 1 | 2 |  |  |  | 4 |  | 0 |  | 6 |  | 74 |  | 0 |  | 56 |  | 2 |  | 122 |  | 32 | 3 |  | 15 |  | 0 |  | 52 |

LT – Left Turning Traffic	TH – Throughfare Traffic	RT – Right Turning Traffic

| Taiwo Intersection DAY 7 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Table 4.22: Morning and Evening Peak hour Traffic movement on Taiwo Approach |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | LT |  |  |  |  |  |  |  |  | TH |  |  |  |  |  |  |  |  |  | RT |  |  |  |  |  |  |  |
| Period |  |  | Car | Mini Bus |  |  |  | Motor Cycle/ Tricycle |  | Luxurious/ Truck |  | PCU |  | Car |  | Mini Bus |  | Motor Cycle/ Tricycle |  | Luxurious/ Truck |  | PCU |  | Car | Mini Bus |  | Motor Cycle/ Tricycle |  | Luxurious/ Truck |  | PCU |
| Morning | 7:00 - 7:15 |  | 4 | 0 |  |  |  | 5 |  | 0 |  | 8 |  | 21 |  | 2 |  | 18 |  | 0 |  | 37 |  | 54 | 0 |  | 15 |  | 2 |  | 72 |
|  | 7:15 - 7:30 |  | 3 | 0 |  |  |  | 7 |  | 0 |  | 9 |  | 25 |  | 5 |  | 16 |  | 1 |  | 44 |  | 43 | 1 |  | 11 |  | 1 |  | 55 |
|  | 7:30 - 7:45 |  | 8 | 0 |  |  |  | 9 |  | 0 |  | 15 |  | 28 |  | 3 |  | 23 |  | 1 |  | 50 |  | 46 | 0 |  | 27 |  | 1 |  | 69 |
|  | 7:45 - 8:00 |  | 7 | 0 |  |  |  | 10 |  | 0 |  | 15 |  | 34 |  | 2 |  | 22 |  | 0 |  | 53 |  | 46 | 0 |  | 22 |  | 0 |  | 63 |
|  | 8:00 - 8:15 |  | 5 | 0 |  |  |  | 10 |  | 0 |  | 13 |  | 43 |  | 4 |  | 39 |  | 0 |  | 76 |  | 36 | 2 |  | 36 |  | 2 |  | 71 |
|  | 8:15 - 8:30 |  | 12 | 0 |  |  |  | 2 |  | 0 |  | 14 |  | 47 |  | 5 |  | 77 |  | 1 |  | 112 |  | 57 | 6 |  | 16 |  | 1 |  | 78 |
|  | 8:30 - 8:45 |  | 11 | 0 |  |  |  | 9 |  | 0 |  | 18 |  | 62 |  | 2 |  | 81 |  | 0 |  | 125 |  | 39 | 2 |  | 47 |  | 0 |  | 76 |
|  | 8:45 - 9:00 |  | 11 | 0 |  |  |  | 2 |  | 0 |  | 13 |  | 79 |  | 2 |  | 63 |  | 1 |  | 130 |  | 41 | 1 |  | 62 |  | 2 |  | 94 |
| Afternoon | 4:00 - 4:15 |  | 13 | 3 |  |  |  | 4 |  | 1 |  | 22 |  | 155 |  | 30 |  | 250 |  | 1 |  | 366 |  | 67 | 3 |  | 48 |  | 0 |  | 106 |
|  | 4:15 - 4:30 |  | 15 | 4 |  |  |  | 10 |  | 1 |  | 34 |  | 120 |  | 9 |  | 160 |  | 0 |  | 249 |  | 69 | 1 |  | 49 |  | 0 |  | 107 |
|  | 4:30 - 4:45 |  | 16 | 3 |  |  |  | 10 |  | 2 |  | 33 |  | 201 |  | 14 |  | 133 |  | 0 |  | 315 |  | 24 | 0 |  | 20 |  | 0 |  | 39 |
|  | 4:45 - 5:00 |  | 14 | 2 |  |  |  | 10 |  | 0 |  | 24 |  | 50 |  | 10 |  | 80 |  | 1 |  | 123 |  | 115 | 0 |  | 60 |  | 0 |  | 160 |
|  | 5:00 - 5:15 |  | 14 | 0 |  |  |  | 3 |  | 0 |  | 16 |  | 58 |  | 1 |  | 52 |  | 0 |  | 98 |  | 53 | 2 |  | 22 |  | 2 |  | 84 |
|  | 5:15 - 5:30 |  | 1 | 0 |  |  |  | 8 |  | 0 |  | 7 |  | 83 |  | 1 |  | 52 |  | 0 |  | 123 |  | 71 | 1 |  | 55 |  | 0 |  | 106 |
|  | 5:30 - 5:45 |  | 4 | 2 |  |  |  | 1 |  | 0 |  | 7 |  | 83 |  | 0 |  | 56 |  | 0 |  | 125 |  | 44 | 0 |  | 21 |  | 0 |  | 68 |
|  | 5:45 - 6:00 |  | 1 | 2 |  |  |  | 4 |  | 0 |  | 6 |  | 74 |  | 0 |  | 56 |  | 2 |  | 122 |  | 32 | 3 |  | 15 |  | 0 |  | 52 |