CHAPTER ONE

1.0	INTRODUCTION

An As-Built Survey is an essential geospatial activity that records the precise locations and dimensions of completed infrastructure projects. Unlike preliminary or design surveys, the As-Built Survey provides a real-time record of the actual positions of built features. It serves multiple purposes such as project documentation, quality assurance, legal referencing, and future maintenance planning. With the introduction of Global Positioning System (GPS) technologies, As-Built Surveys have gained improved accuracy, speed, and efficiency. GPS enables real-time data collection with minimal manual intervention, reducing errors and significantly enhancing the quality of survey outputs.

As-built surveys serve multiple purposes across various industries, including architecture, civil engineering, construction management, and facility maintenance. They are typically carried out after construction or installation is completed, but may also be performed periodically during different stages of a project to verify progress and ensure consistency with design specifications. The surveys record precise locations of walls, doors, windows, columns, utilities, and other structural or infrastructural elements. The importance of as-built surveys has grown significantly with the advancement of technologies such as total stations, 3D laser scanning, drones, and building information modeling (BIM). These tools enhance the accuracy and efficiency of data collection and allow for the creation of detailed digital models of physical structures. The output from as-built surveys is critical for project close-out, renovation planning, asset management, compliance verification, and dispute resolution.

In many construction projects, discrepancies often arise between the original design and the actual construction. This can lead to future design challenges, legal issues, or costly alterations. Lack of accurate and up-to-date records of existing conditions can also hinder maintenance, expansion, or demolition works. Therefore, there is a need for a reliable method to capture and document existing structures precisely as they are built, especially for academic institutions like the project area.

Overall, as-built surveys bridge the gap between design and reality, providing essential documentation that reflects the true built environment. They play a vital role in quality assurance, ensuring that construction work aligns with intended plans and meets regulatory and client requirements.

1.1	STATEMENT OF THE PROBLEM

In many construction projects, discrepancies often arise between the original design and the actual construction. This can lead to future design challenges, legal issues, or costly alterations. Lack of accurate and up-to-date records of existing conditions can also hinder maintenance, expansion, or demolition works. Therefore, there is a need for a reliable method to capture and document existing structures precisely as they are built, especially for academic institutions like the project area.

1.1 	STATEMENT OF PROBLEM

In many construction projects, discrepancies often arise between the original design and the actual construction. This can lead to future design challenges, legal issues, or costly alterations. Lack of accurate and up-to-date records of existing conditions can also hinder maintenance, expansion, or demolition works. Therefore, there is a need for a reliable method to capture and document existing structures precisely as they are built, especially for academic institutions like the project area.

1.2 	AIM OF THE PROJECT

The aim of this project is to conduct an accurate as-built survey of an existing structure or infrastructure to provide detailed, verifiable documentation of its actual conditions upon completion, specifically for the project area.

1.3 	OBJECTIVE OF THE PROJECT

- To identify and record deviations from original design plans.

- To produce precise drawings that represents the existing physical state of the project.

- To produce a record surveying plan of the under-construction structure using the total station.

- To enhance understanding and application of geospatial techniques in post-construction documentation.

1.4 	SCOPES OF THE PROJECT

This project focuses on conducting an as-built survey for a selected site, including the collection of field data, data processing, and generation of final as-built drawings. It covers planimetric and elevation features but excludes sub-surface or utility surveys Successful execution of this project involved the following scopes:

- Project planning.

- Field reconnaissance.

- Data acquisition.

- Data processing.

- Information presentation.

- Technical report writing

1.5 	PROJECT SPECIFICATIONS

The specifications on this project involve the rules and regulations guiding the practice and processes of surveying, which are:

- Connection of the survey to national framework (control pillars).

- A close traverse must be laid out for the perimeter survey to ensure checking on the final survey work.

- Provision of the back computation sheet to show the list of the coordinates used for the perimeter survey.

- Linear accuracy of the survey work must be done accurately.

- The plan to be produced must be a signed using standard as-built plan scales.

### 1.6 	PROJECT LOCATION DESCRIPTION

The project site is located along Kwara state polytechnic Mini-Campus, inside university of Ilorin Mini-Campus, Off General Gari Alimi road, Ilorin-west local Government Area of Kwara State. Approximately the site is located on latitude  8°28'53.20"N ; longitude  4°31'37.75"E and 8°28'55.41"N; 4°31'30.28"E and the imagery below show the project area.

1.7 	PERSONNEL INVOLVED

The following are the members of the group that was involved in the as built survey project work, they are;

| S/N | Names | Matric No | Role |
| --- | --- | --- | --- |
| 1. | Wasiu Abdulganiyu Bayonle | ND/23/SGI/PT/0001 | Author |
| 2. | Ilori Waris Opeyemi | ND/22/SGI/PT/012 | Member |
| 3. | Sakariyahu Abdulganiyu | ND/23/SGI/PT/0003 | Member |
| 4. | Bello Muhammad Abdullateef | ND/23/SGI/PT/0002 | Member |
| 5. | Ibrahim Aishat Khadijat | ND/23/SGI/PT/0007 | Member |
| 6. | Yahya Ridwan | ND/23/SGI/PT/0006 | Member |
| 7. | Betiku Funmilola Oyinkannsola | ND/23/SGI/PT/0005 | Member |
| 8. | Solayinka Mujeeb Olanrewaju | ND/23/SGI/PT/0004 | Member |

CHAPTER TWO

2.0	LITERATURE REVIEW

An as-built survey is a critical part of the construction and civil engineering process, aimed at capturing the actual state of a structure or site post-construction. Recent technological advances have shifted traditional survey methods toward high-precision tools such as 3D laser scanning. These tools not only increase accuracy but also efficiency in data collection, especially for large or complex projects (Tang et al., 2010).

Laser scanning, also known as LiDAR (Light Detection and Ranging), involves using laser beams to measure distances to surfaces and generate high-density 3D point clouds. These point clouds represent the exact dimensions and spatial relationships of the scanned object or environment. In the context of as-built surveys, laser scanning enables the creation of precise 3D models for analysis, documentation, or comparison with design plans (Barazzetti et al., 2011).

Laser scanners, such as terrestrial laser scanners (TLS), are typically mounted on tripods and used to scan the site from multiple locations. The resulting data is then registered and processed using software like Autodesk ReCap or FARO Scene. This method allows for: Rapid data acquisition, Non-contact measurement, detailed visualization of complex geometries (Boehler&Marbs, 2002).

Results obtained the studies show that laser scanning provides superior detail and accuracy compared to traditional surveying techniques. The collected point cloud data can be transformed into 3D CAD models or Building Information Models (BIM), enabling engineers to detect deviations from design and prevent costly errors during construction (Volk et al., 2014). Laser scanning also enhances safety by reducing the need for physical access to hazardous or hard-to-reach areas.

In conclusion, the adoption of laser scanning in as-built surveying significantly improves documentation precision and project outcomes. Its ability to quickly and accurately capture spatial data makes it an essential tool in modern construction and infrastructure projects. As technology continues to evolve, integration with AI and automated processing is expected to further enhance its applications and efficiency (Tang et al., 2010).

An as-built survey is a critical part of construction and engineering projects, especially in validating the actual construction against the original design plans. It ensures that deviations are documented and adjustments can be made during or after the construction process. As defined in the reviewed work, an as-built survey involves capturing the physical state of a completed or ongoing project using precise survey methods. It records real-world coordinates, spot heights, and other data to reflect the actual structure against the proposed design. This process is fundamental for updating design drawings, monitoring construction progress, and ensuring structural integrity.

The method employed in this study includes the use of a theodolite, a traditional yet reliable instrument for angular measurement. The survey process involved:

- 	Traverse survey and detail pick-up, ensuring linear accuracy.

- 	Control pillar connection, aligning with national coordinate frameworks.

- 	Back computation and area computation sheets to verify data accuracy.

- 	Close traverses for perimeter checks and adjustment of errors.

Data acquisition was done manually using the theodolite, followed by data processing with CAD and GIS software for generating as-built plans. The team included student surveyors, field engineers, and CAD technicians who ensured accuracy and quality control.

The survey successfully documented the existing state of the project site. The data obtained were reliable and consistent with the required project specifications. The plans produced reflected the true dimensions and layout of the site, allowing for accurate representation and analysis. The survey outputs included:

- 	As-built drawings

- 	Spot heights

- 	Coordinates

- 	Area computation

In conclusion, the use of a theodolite in this as-built survey proved effective for capturing precise data. Despite being less automated than modern GNSS equipment, the theodolite offered a dependable alternative for achieving project goals. The project emphasized the importance of adherence to national standards, thorough documentation, and quality control in surveying practices ([Author Surname], 2025).

As-built surveys are critical in documenting the accurate physical layout of a construction site after project completion. They serve as verification tools to compare the built structure with original design plans. Adebayo (2023) emphasizes the importance of post-construction surveys to detect deviations, guide future modifications, and provide essential documentation for legal, planning, and maintenance purposes. According to Adebayo (2023), an as-built survey is defined as a comprehensive survey conducted to measure, document, and record the actual state of a completed structure or site. The results are typically used to produce detailed drawings reflecting the real-world implementation, which are vital for technical reports, future development, and regulatory compliance.

The methodology employed in Adebayo's study centers on theodolite traversing and detail pick-up techniques. A closed travers was established around the site perimeter to ensure accuracy and consistency. The survey included linear measurements, angular readings, and area computation. Instruments used included a theodolite for angle measurement, measuring tapes, and prisms. The data were processed with AutoCAD and GIS software to produce coordinate-based as-built drawings.

The as-built survey revealed minor deviations from the original design, particularly in perimeter alignment and elevation differences. The processed data yielded accurate coordinate listings, spot heights, and area computations. The findings were presented in both graphical form (as-built plan) and tabulated computations, aligning closely with project specifications.

Adebayo (2023) concludes that the as-built survey using theodolite traversing provides reliable and accurate documentation necessary for final project evaluation. The study confirms that adherence to survey specifications and the use of proper control frameworks are vital for successful as-built documentation.

The growing need for precise, efficient, and time-saving methods in construction and infrastructure development has increased the use of drone technology in surveying practices. One of the most significant applications is in conducting As-Built Surveys, which ensure that construction outcomes align with original design specifications. Traditionally, these surveys were carried out manually using ground-based equipment, often requiring extensive labor and time. However, the integration of unmanned aerial vehicles (UAVs) equipped with high-resolution cameras and GNSS receivers has revolutionized this field. This literature review explores the methodology, advantages, and outcomes of using drone technology in As-Built Surveys, emphasizing its impact on data accuracy, project efficiency, and decision-making in construction projects.

An As-Built Survey is conducted after construction to verify that structures were built according to approved plans. It records the exact location of constructed elements and deviations from original designs. It serves in engineering and construction as a tool for field condition validation, legal documentation, and planning for renovations or maintenance.

Method used are; Data Acquisition, Aerial data captured using drones with GNSS/RTK. Ground control points (GCPs) were established. Data Processing, Drone imagery processed using CAD and GIS software to produce DEMs, orthophotos, and 3D models. Field Validation: Drone data verified using GNSS survey equipment. Results obtained are, High positional accuracy, Faster and safer data acquisition. Reduced manpower and time requirements. Improved visualization of structures. Conclusion: Drone technology is reliable and efficient for As-Built surveys. Enhances accuracy, reduces time, and provides current spatial data. Integration with CAD/GIS improves spatial analysis and documentation.

CHAPTER THREE

3.0 	METHODOLOGY

This aspect refers to a system of methods and principles, rules used in a field of study to achieve the aim and objectives of this project. The modality to successful execute this project was carried out based on the survey rules and regulation, supervisor advice and departmental instructions.

3.1 	RECONNAISSANCE

Reconnaissance survey is a preliminary stage for exploratory investigations conducted to gather initial data concern the choice of Site area of Subject before more detailed research or filed work commenced. Reasons for reconnaissance survey of the project area are the following;

- To know the location of the site.

- To determine the preliminary area/perimeter of the land.

- To know the survey method suitable for the field data acquisition and the instrument need for the it.

Reconnaissance in this project involved preliminary investigations to gather information about the survey area before conducting a detailed survey, reconnaissance of this project was carried out in two way namely as follows.

- Office planning

- Field Reconnaissance

3.1.1 	OFFICE PLANNING

It involved the process adopted in the collection of relevant data and specification about the as-built Surveying with the used of traversing survey method. During the office planning the given coordinate for control point HND2001, TBM0110, and HND2002, was obtained and computed as shown in the table 3.1 below. The choice of equipment to be use for project was obtained from the survey store in the department of surveying and Geoinformatic, Kwara State Polytechnic, Ilorin, and it was handled accordingly to the survey rules and regulations and departmental instruction.

TABLE 3.1 Showing the list of the coordinate of the control pillars used

| Stations | Northing (m) | Easting (m) | Remark |
| --- | --- | --- | --- |
| HND2001 | 937892.5580 | 668076.4680 | Given |
| TBM0110 | 937900.4120 | 668069.5750 | Given |
| HND2002 | 937909.7109 | 668061.2683 | Given |

3.1.2 	FIELD RECONNAISSANCE

Field recce and site visitation was conducted as preliminary survey for investigation of the basic information about a specified project area, location, or subject in the field. It allows the members of the groups to be familiar with the terrain, boundary lines, selection number of station required, Equipment that will be used for the job, pillars number HND2001, TBM0110, and HND2002 used as a control for the project and so on. The stations points selected was Inter-visible to one another and finally a reconnaissance diagram was produced which was attached to this project

RECCE DIAGRAM

3.2	INSTRUMENT SELECTION

The equipment used for this project was grouped into two categories;

- Hardware

- Software

3.2.1	Hardware used

- Total Station with its Accessories (KTS-442L)

- Reflector poles

- Field book

- Writing material (pen)

- Survey beacons

- Steel Tape (5m)

- Tripod Stand

3.2.2	Software used

Below are the list of the software used during the project processing;

- AutoCAD 2007

- Notepad

- Microsoft word

- Microsoft Excel

3.3 	INSTRUMENT TEST

The instrument test was carried out before it used for angular observation. The purpose of testing the instrument before used is to know the assurance and maybe other instrument is in good Condition the instrument was set up over station A and it was pointed a Sighted to station B whore a pole Is hold, and the angle was recorded for face left the instrument was turn to farce right on the same point and it was sighted to station B, where a pole is held at the same position and the angle was recorded. The distance between station A and B 1s 10m. The steel tape was hold at 60m an point A and it was extend to point B. the results for the instrument test was shown i to figure 3.3 below and table 3:2 below.

Total station

LINE OF SIGHT

RANGING POLE

20M

STATION  X

TERRAIN

STATION  Y

Figure 3.1 showing the instrument set up for collimation test

Table 3.2 showing the test of instrument result

| STATION | SIGHT | FACE | HORIZONTAL READING | DEDUCED ANGLE |
| --- | --- | --- | --- | --- |
| A | B | L | 180° 00’ 03” |  |
|  | B | R | 00°00’ 00” | 180° 00’ 03” |

Different in horizontal circle reading: 180° 00’ 03’’

Horizontal collimation error

= 180° 00’ 03’’

= 000° 00 03’’

Collimation error = 00° 00'03"2

Horizontal collimation error = 00° 00’ 01”

Because the collimation  error falls within the allowable 01”. The instrument was in good condition.

3.4 	MONUMENTATION

This is the process in which the beacon is being buried in to ground. On the project site there are same existing beacons on the boundary lines m, nail with cap (bottle cork) was used as the 	station pegs.

18cm

15cm above the ground

60cm below the ground

18cm

30cm

Nail

Bottle cork

PILLAR							   PEG

FIGURE 3.4	Diagram and Dimension of Pillars buried

3.5 	CONTROL CHECK

The Control check was carried out to know the control point HND2001, TBM0110, and HND2002 are still in their positions. The instrument was set on Control FP01107 and the temporary adjustment was carried out i.e centring, levelling, and focusing. Back site was taken to control HND2001 and it was recorded.

The fore sight was served to PEG1 and it was recorded Steel tape was used to measure the distance between TBM0110 and PEGI and it was recorded. The data observed was computed for given coordinate and Observed Coordinate, as shown in the table 3.2 below.

TABLE 3.2	THE GIVEN COORDINATE OF THE CONTROL POINTS

| Stations | Northing (m) | Easting (m) | Remark |
| --- | --- | --- | --- |
| HND2001 | 937892.558 | 668076.468 | Given |
| TBM0110 | 937900.412 | 668069.575 | Given |
| HND2002 | 937909.711 | 668061.268 | Given |

TABLE 3.3	The Backward Computation for the Given Controls pillars

| From station | Bearing | Distance | ∆N | ∆E | Northing (m) | Easting (m) | To station |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | 937892.558 | 668076.468 | HND2001 |
| HND2001 | 318° 43' 42’’ | 10.472 | 7.854 | -6.893 | 937900.412 | 668069.575 | TBM0110 |
| TBM0110 | 318° 13' 33’’ | 12.469 | 9.299 | -8.307 | 937909.711 | 668061.268 | HND2002 |

TBM0110  to HND2002 	BEARING = 318° 13' 33’’  	DISTANCE = 12.469m

HND2002 to TBM0110 	BEARING = 138° 43' 42’’	DISTANCE = 10.472m

TABLE 3.4	Show the Observed Coordinate of the Controls Pillars

| Stations | Northing (m) | Easting (m) | Remark |
| --- | --- | --- | --- |
| HND2001 | 937892.550 | 668076.460 | Observed |
| TBM0110 | 937900.412 | 668069.575 | Observed |
| HND2002 | 937909.715 | 668061.271 | Observed |

TABLE 3.5	Show the Backward Computation for the Observed Control Pillars

| From station | Bearing | Distance | ∆N | ∆E | Northing (m) | Easting (m) | To station |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | 937892.550 | 668076.460 | HND2001 |
| HND2001 | 318° 43' 25’’ | 10.451 | 7.862 | -6.885 | 937900.412 | 668069.575 | TBM0110 |
| TBM0110 | 318° 14' 51’’ | 12.470 | 9.303 | -8.304 | 937909.715 | 668061.271 | HND2002 |

TBM0110  to HND2002 	BEARING = 318° 13' 51’  	DISTANCE = 12.470m

HND2002 to TBM0110 	BEARING = 138° 43' 25’’	DISTANCE = 10.451m

Therefore the difference between the given and observer bearing was 00° 00’ 19” and 00° 00’ 17” and the difference between the given and the observer linear distance was 0.003m and 0.021m the results obtained fall within the allowable accuracy for both angular and linear observation.

It was concluded the instrument was in good condition and fit to be used for the project.

3.6 	DATA AQUISITION

This was carried out on the field using Total station with its accessories (KTS-442L) was used for coordinating the entire site for both the perimeter and details of the site. The Total station was set on the 	control Pillar TBM0110, and the temporary adjustment was carried out centering, leveling and 	focusing. The coordinates of the occupied station was input after that the coordinates of the backsight HND2001 was record as well and the observation was done to backsight pillar and the coordinates was take and check against the given. Having check and found accurate then the perimeter survey commenced for the entire project site.

3.6.1 	DETAILING SURVEY

The detailing survey was done immediately after completely carried out the perimeter traverse to determine the position of the features on the Site was fixed using radiation survey method. During this several stations was established to pick the coordinates of the details and the coordinates was stored on the total station as its observed. This was later downloaded. The details that was picked during the detailing survey was coded in different ways like Building edges as ‘BLD’, Electric Poles ‘EP’, Drainage ‘DR’, Roads Line ‘RD’, Spot Height ‘SP’ and so on.

CHAPTER FOUR

4.0   	DATA PROCESSING

This involved several step to ensure accurate and reliable result. It explains the method used in capturing; processing, downloading, and analyzing the data acquired Data reduction in surveying involves various methods to simplify and summarize the collected data for analysis. These are some common methods of data reduction that involves

- Combining multiple values into a single value. The average value is calculated by summing up all the values and dividing by the number of values.

- Least Square Adjustment (LSA) is a method of data reduction that involves minimizing the sum of the squared errors between observed and calculated values.

- Interpolation is a method of data reduction that involves estimating values between observed data points.

- Spatial data reduction refers to the process of simplifying and reducing the complexity of spatial data, such as geographic information system (GIS) data, while preserving its essential characteristics.

4.1	DATA EDITING

- Open excel

- Click on the desire file

- Set to delimited

- Click on next

- Mack space and comma

- Click

- On next

- Then click on finish

4.3	DATA PROCESSING USING AUTOCAD

The plan produced digitally with AutoCAD software. The following are the procedure used:

STEP 1: Open Notepad

Open Notepad on your computer. You can find it in the start menu (Window)

STEP 2: Create a New File

Click on "file" "New" to create a new file

STEP3: Enter Coordinates

Enter your coordinates in the following format:

X. Y. Data

STEP 4: Save the Fil

Click on "File" > "Save as" and choose a location to save your file. "ALADA Coordinate" txt, 	and select "All files" as the file type.

Importing the Coordinate File Into 2007 AutoCAD

STEP 1: Double click AutoCAD

Go under format

Click on format after that pick your unit

Drawing unit

Type.... Decimal

Precision... 0.000

Angle type Degree/Min/See

Precision Od00

Clockwise.... 노

Unit to scale Meters

Direction control North 270d0

Press "OK"

STEP 2: Create a new drawing

Click on "File" > "Name" to create a new drawing

STEP 3: Import the coordinate file

Click on "Tools" >"Reference" > "Import" and select "ALADA coordinate (.txt)" as the file 	type. Navigate to the location where you saved your coordinate file ("ALADA coordinate .txt") 	and select it.

STEP 4: Configure Import Setting. In the import select "Point" as the object type and choose 	the correct coordinate format (eg. X and Y) click "OK" to import the coordinates.

4.4	DATA EXTRACTION

The data recorded in the field book was picked to compute for the bearing and distance. Forward computation and backward computation after all necessary correction have been applied. Correction like slope correction, tension correction and standardization correction.

4.3  	BACK COMPUTATION

The result get for forward computation was used for Back computation.

The following program was required for back computation

i. Input N2-N1 to get ∆N

ii. Input E2-E1 to get ∆E

iii.Input (∆N2+∆E2 to get distance

iv.Input shift tan ∆E∆Nto get the corrected bearing when there us more station repeat step ¡¡¡ and ii.

Table 4.1 Showing Boundary Coordinates

| Station | Northing (m) | Easting(m) | Remark |
| --- | --- | --- | --- |
| PIL1 | 937720.924 | 668157.605 | Observed |
| PIL2 | 937720.560 | 668042.167 | Observed |
| PIL3 | 937748.847 | 667988.759 | Observed |
| PIL4 | 937788.194 | 667965.709 | Observed |
| PIL5 | 937930.248 | 668056.923 | Observed |
| PIL6 | 937924.001 | 668159.813 | Observed |

Table 4.1.1 Showing back computation for the boundary coordinate

| Station form | Bearing | Distance | ∆N | ∆E | Northing | Easting | Station to |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | 937720.924 | 668157.605 | PIL 1 |
| PIL1 | 89° 49’ 09’’ | 115.484 | 0.364 | 115.438 | 937720.560 | 668042.167 | PIL 2 |
| PIL 2 | 117° 54’26’’ | 45.302 | -28.287 | 53.408 | 937748.847 | 667988.759 | PIL 3 |
| PIL 3 | 149° 38’ 15” | 31.887 | -39.347 | 23.05 | 937788.194 | 667965.709 | PIL 4 |
| PIL 4 | 180° 35’ 31” | 108.901 | -142.054 | -91.214 | 937930.248 | 668056.923 | PIL 5 |
| PIL 5 | 253° 31’ 46” | 102.700 | 6.247 | -102.89 | 937924.001 | 668159.813 | PIL 6 |
| PIL 6 | 360° 00’ 39” | 203.082 | 203.077 | 2.208 | 937720.924 | 668157.605 | PIL 1 |

4.4	AREA COMPUTATION

TABLE 4.1.2	showing area computation for the boundary coordination using double latitude

| ∆E | ∆N | PRODUCT |
| --- | --- | --- |
| 115.438 | X 0.364 | = 42.019432 |
| 115.438 |  |  |
| 230.876 |  |  |
| 53.408 |  |  |
| 284.284 | X -28.287 | =-8041.541508 |
| 53.408 |  |  |
| 337.692 |  |  |
| 23.05 |  |  |
| 360.742 | X -39.347 | =-14194.11547 |
| 23.05 |  |  |
| 283.792 |  |  |
| -91.214 |  |  |
| 292.578 | X -142.054 | = -41561.87521 |
| -91.214 |  |  |
| 201.364 |  |  |
| -102.89 |  |  |
| 98.474 | X 6.247 | = 615.167078 |
| -102.89 |  |  |
| -4.416 |  |  |
| 2.208 |  |  |
| -2.208 | X 2203.077 | = -448.394016 |
| 2.208 |  |  |
| 0.000 |  |  |

= 42.01943 – 8041.541508 – 14194.11547- 41561.87521+ 615.167078 – 448.394016		= -63588.73969\2											= 31794.36985m²\10,000.										= 3.1794 HECTRES

4.5	PLAN PRODUCTION

All the plan were produces in both soft copies and hard copies, the hard copies were shown on the appendix page attached with the project.

CHAPTER FIVE

5.0 	SUMMARY, PROBLEMS ENCOUNTERED, CONCLUSION, RECOMMENDATION

5.1 	SUMMARY

In order to determine the shape, size and location of the Kwara state Polytehnic Mini Campus, reconnaissance was carried out which helps in taking further decisions. Beacons were coordinated using total station with the help of existing control points very close to the project site. Data were downloaded and processed using appropriate AutoCAD 2007 software and the final adjusted coordinates of the perimeter were used to plot the boundary and other details . The resulting plan was printed in scale 1: 2,500. A comprehensive Technical report was prepared and printed.

5.2 	PROBLEM ENCOUNTERED

- Too much of rainfall which does not allowed site

- Unavailable of the group members due to dean's cup football tournament

- While taking observing there are some object obstructing the view when taking the observation e.g electric pole, building, tree etc.

5.3 	RECOMMENDATION

The department should considered the season and time that the students should be given the terminal project.

5.4 	CONCLUSION

At the end the aim and objective of this project was achieved. It was carried out using third order surveying method this project was carried out in accordance with survey rules and regulation and departmental instruction.

REFERENCES

Alsharif, F., Gerges, M., &Mcheick, H. (2017). "As-built BIM integration using Total Station." Journal of Construction Engineering and Management, 143(3).

Aryal, A., & Pandey, M. (2013). "As-built survey of utilities using Total Station and GPS." Nepalese Journal of Civil Engineering, 2(1), 34-39.

Awange, J. L., &Kiema, J. B. (2013). Environmental Geoinformatics: Monitoring and Management Springer.

Ogundare, J.O. (2016). Precision Surveying: The Principles and Geomatics Practice. Wiley.

Okeke, F.I., & Igbokwe, J.O. (2011). "Use of Robotic Total Station for High-Risk Surveying Tasks." Journal of Applied Science and Environmental Management, 15(4), 77-83.

Olatunji, S. A. (2017). Practical Surveying and Geoinformatics. Ibadan University Press.

Pather, S., & Campbell, G. (2015). "The Role of GNSS in Construction Documentation." South African Journal of Geomatics, 4(1), 33-45.

Uren, J., & Price, B. (2010). Surveying for Engineers (5th ed.). Palgrave Macmillan.

Uren, J., & Price, W. F. (2010). Surveying for Engineers (5th ed.). Palgrave Macmillan.

RAW DATA

| Name | Code | E | N | h |  | PT260 | SH | 668115.4 | 937791.5 | 317.946 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PT1 | ST17 | 668069.6 | 937900.4 | 324.137 |  | PT261 | SH | 668118 | 937790.7 | 318.041 |
| PT2 | CV | 668071.1 | 937900.4 | 324.553 |  | PT262 | SH | 668117.7 | 937795.6 | 317.967 |
| PT3 | CV | 668072 | 937900.3 | 324.541 |  | PT263 | SH | 668113.5 | 937797.2 | 317.656 |
| PT4 | BLD | 668072.6 | 937900.6 | 324.383 |  | PT264 | SH | 668110.1 | 937800.8 | 317.647 |
| PT5 | BLD | 668066.5 | 937905.8 | 323.724 |  | PT265 | SH | 668110.4 | 937806.9 | 317.571 |
| PT6 | SH | 668069.7 | 937897.9 | 324 |  | PT266 | SH | 668114.1 | 937808.4 | 317.326 |
| PT7 | SH | 668072.6 | 937896.1 | 324.671 |  | PT267 | SH | 668117.8 | 937810 | 317.601 |
| PT8 | SH | 668072.2 | 937891.5 | 324.604 |  | PT268 | SH | 668117.6 | 937814.7 | 317.532 |
| PT9 | SH | 668076.5 | 937892.6 | 324.821 |  | PT269 | SH | 668113.8 | 937816 | 317.212 |
| PT10 | SH | 668083.2 | 937892.3 | 324.831 |  | PT270 | SH | 668109 | 937815.1 | 317.485 |
| PT11 | SH | 668085.6 | 937889.4 | 324.87 |  | PT271 | SH | 668104.5 | 937814.4 | 317.27 |
| PT12 | BLD | 668085.4 | 937886.8 | 324.848 |  | PT272 | SH | 668104.8 | 937819.5 | 317.197 |
| PT13 | SH | 668088.7 | 937888.5 | 324.887 |  | PT273 | SH | 668109.7 | 937818.5 | 317.278 |
| PT14 | SH | 668090.4 | 937893.3 | 324.882 |  | PT274 | SH | 668113.6 | 937819 | 317.192 |
| PT15 | SH | 668095.3 | 937895.1 | 324.957 |  | PT275 | SH | 668117.7 | 937821.2 | 317.334 |
| PT16 | SH | 668095.6 | 937891.4 | 324.958 |  | PT276 | SH | 668114.1 | 937823.2 | 317.152 |
| PT17 | BLD | 668139.4 | 937901.3 | 324.586 |  | PT277 | SH | 668109 | 937824.1 | 317.181 |
| PT18 | CV | 668139.1 | 937901.2 | 316.047 |  | PT278 | SH | 668104.9 | 937822.2 | 317.161 |
| PT19 | CV | 668139.5 | 937900.4 | 316.104 |  | PT279 | SH | 668105 | 937825.4 | 317.107 |
| PT20 | CV | 668137.3 | 937899 | 317.048 |  | PT280 | SH | 668103.5 | 937827.1 | 317.276 |
| PT21 | SH | 668133.7 | 937894.7 | 316.106 |  | PT281 | SH | 668108.1 | 937827.2 | 317.179 |
| PT22 | SH | 668136.7 | 937894.8 | 316.021 |  | PT282 | SH | 668112 | 937827.1 | 317.097 |
| PT23 | BLD | 668137 | 937893.5 | 316.015 |  | PT283 | SH | 668116.6 | 937827.5 | 317.116 |
| PT24 | SH | 668141.1 | 937890.4 | 315.886 |  | PT284 | SH | 668117.8 | 937831.4 | 317.048 |
| PT25 | SH | 668145.9 | 937889.2 | 315.599 |  | PT285 | SH | 668112.9 | 937833.2 | 317.012 |
| PT26 | SH | 668147.8 | 937891.6 | 315.619 |  | PT286 | SH | 668108.8 | 937832.7 | 317.058 |
| PT27 | GT | 668148.7 | 937893.4 | 315.804 |  | PT287 | SH | 668106.3 | 937831.7 | 317.095 |
| PT28 | GT | 668147.9 | 937894 | 315.765 |  | PT288 | SH | 668120.4 | 937780 | 318.343 |
| PT29 | BLD | 668148.9 | 937893.2 | 315.928 |  | PT289 | SH | 668125.9 | 937783 | 318.036 |
| PT30 | SH | 668148.1 | 937886.3 | 315.852 |  | PT290 | SH | 668122.6 | 937785.4 | 318.043 |
| PT31 | SH | 668144.4 | 937882.9 | 315.942 |  | PT291 | SH | 668120.9 | 937789.4 | 318.013 |
| PT32 | SH | 668140.3 | 937879.1 | 316.287 |  | PT292 | BLD | 668120.6 | 937791.6 | 317.908 |
| PT33 | SH | 668144.4 | 937877.2 | 316.236 |  | PT293 | SH | 668125 | 937790.3 | 317.853 |
| PT34 | SH | 668147.4 | 937877.9 | 316.065 |  | PT294 | SH | 668128.5 | 937785.7 | 317.944 |
| PT35 | BLD | 668149.4 | 937879.9 | 315.956 |  | PT295 | SH | 668133 | 937781.5 | 317.952 |
| PT36 | BLD | 668156.1 | 937879.6 | 315.966 |  | PT296 | SH | 668132.9 | 937786.5 | 317.857 |
| PT37 | BLD | 668156 | 937874.1 | 315.966 |  | PT297 | SH | 668132.2 | 937790.7 | 317.807 |
| PT38 | EP | 668150.9 | 937875.2 | 316.093 |  | PT298 | SH | 668137 | 937790.8 | 317.772 |
| PT39 | SH | 668152 | 937878.1 | 315.912 |  | PT299 | SH | 668139.7 | 937786.5 | 317.824 |
| PT40 | BLD | 668146.3 | 937874.1 | 316.297 |  | PT300 | SH | 668140 | 937781.4 | 318.046 |
| PT41 | SH | 668132 | 937896.8 | 316.129 |  | PT301 | SH | 668144.3 | 937781.2 | 318.023 |
| PT42 | BLD | 668128.4 | 937894.7 | 316.378 |  | PT302 | SH | 668145.5 | 937786.4 | 317.727 |
| PT43 | SH | 668124.8 | 937894.3 | 316.451 |  | PT303 | SH | 668146 | 937791.2 | 317.717 |
| PT44 | SH | 668121.3 | 937892 | 316.499 |  | PT304 | SH | 668149.5 | 937790.8 | 317.75 |
| PT45 | SH | 668118.3 | 937894.3 | 316.594 |  | PT305 | SH | 668149.7 | 937786.7 | 317.692 |
| PT46 | SH | 668115.7 | 937888.6 | 316.573 |  | PT306 | SH | 668148.9 | 937781.3 | 317.842 |
| PT47 | BLD | 668113.6 | 937886.9 | 316.529 |  | PT307 | BLD | 668150.4 | 937781.4 | 317.8 |
| PT48 | SH | 668110.4 | 937889.4 | 316.736 |  | PT308 | BLD | 668150 | 937791.5 | 317.803 |
| PT49 | SH | 668108.3 | 937891.8 | 316.596 |  | PT309 | SH | 668113.7 | 937768.9 | 318.789 |
| PT50 | SH | 668104.4 | 937892.2 | 316.593 |  | PT310 | SH | 668113.6 | 937766 | 318.901 |
| PT51 | SH | 668104.2 | 937889.1 | 316.626 |  | PT311 | BLD | 668112.8 | 937763.3 | 319.05 |
| PT52 | SH | 668117.5 | 937886.8 | 316.551 |  | PT312 | SH | 668117.2 | 937761.4 | 318.943 |
| PT53 | SH | 668119.7 | 937883.9 | 316.541 |  | PT313 | SH | 668117.5 | 937764.3 | 318.911 |
| PT54 | SH | 668117.1 | 937883.2 | 316.562 |  | PT314 | SH | 668122.4 | 937765.5 | 318.977 |
| PT55 | SH | 668114.2 | 937882.6 | 316.592 |  | PT315 | SH | 668124.5 | 937762.3 | 318.933 |
| PT56 | BLD | 668113.5 | 937874.4 | 316.659 |  | PT316 | SH | 668122.4 | 937758.6 | 318.969 |
| PT57 | SH | 668115.5 | 937874 | 316.641 |  | PT317 | SH | 668121.7 | 937753.2 | 319.012 |
| PT58 | SH | 668117.6 | 937873.9 | 316.666 |  | PT318 | SH | 668119.4 | 937752.3 | 319.022 |
| PT59 | SH | 668116.9 | 937866.3 | 316.759 |  | PT319 | BLD | 668113 | 937752.1 | 319.283 |
| PT60 | SH | 668114.9 | 937865.7 | 316.81 |  | PT320 | BLD | 668113.1 | 937748.7 | 319.275 |
| PT61 | SH | 668112.2 | 937865.8 | 317.066 |  | PT321 | BLD | 668113.2 | 937730.1 | 319.819 |
| PT62 | BLD | 668111.7 | 937870.2 | 316.664 |  | PT322 | BLD | 668118.1 | 937726.7 | 320.172 |
| PT63 | SH | 668124 | 937868.3 | 316.542 |  | PT323 | BLD | 668118.2 | 937750.2 | 319.117 |
| PT64 | BLD | 668129.2 | 937867.4 | 316.594 |  | PT324 | SH | 668122.5 | 937751.6 | 319.006 |
| PT65 | SH | 668128.1 | 937869.8 | 316.579 |  | PT325 | SH | 668124.5 | 937754.3 | 318.972 |
| PT66 | SH | 668127.9 | 937872.5 | 316.566 |  | PT326 | SH | 668125.4 | 937757 | 318.942 |
| PT67 | SH | 668125 | 937873.8 | 316.569 |  | PT327 | SH | 668126.4 | 937759.8 | 318.902 |
| PT68 | SH | 668123.6 | 937877 | 316.822 |  | PT328 | SH | 668127.7 | 937763.6 | 318.945 |
| PT69 | SH | 668126 | 937879.4 | 316.682 |  | PT329 | SH | 668129.6 | 937766.7 | 319.003 |
| PT70 | SH | 668121.7 | 937869.6 | 316.558 |  | PT330 | SH | 668132.3 | 937766.4 | 319.052 |
| PT71 | SH | 668121.5 | 937862 | 316.817 |  | PT331 | SH | 668132.9 | 937763.9 | 319.098 |
| PT72 | SH | 668123.1 | 937858.4 | 316.857 |  | PT332 | SH | 668133.1 | 937760.8 | 318.995 |
| PT73 | SH | 668118.9 | 937856.2 | 316.91 |  | PT333 | SH | 668133.8 | 937757.1 | 318.991 |
| PT74 | BLD | 668120.4 | 937852 | 316.833 |  | PT334 | SH | 668134 | 937753.4 | 318.981 |
| PT75 | SH | 668116.8 | 937852.9 | 316.833 |  | PT335 | SH | 668134.5 | 937750.7 | 319.07 |
| PT76 | CV | 668119 | 937855.1 | 316.791 |  | PT336 | BLD | 668137.6 | 937750.4 | 319.035 |
| PT77 | CV | 668118.5 | 937855.2 | 316.819 |  | PT337 | SH | 668138.2 | 937752.8 | 319.024 |
| PT78 | SH | 668111.8 | 937851.5 | 317.396 |  | PT338 | BLD | 668140.6 | 937755.3 | 318.91 |
| PT79 | SH | 668115.7 | 937850.6 | 316.866 |  | PT339 | SH | 668137.7 | 937757.7 | 319.018 |
| PT80 | SH | 668118.3 | 937849 | 316.835 |  | PT340 | SH | 668136 | 937762 | 319.013 |
| PT81 | SH | 668118.1 | 937843.1 | 316.939 |  | PT341 | SH | 668138.3 | 937764.7 | 319.044 |
| PT82 | SH | 668113.8 | 937842.7 | 316.91 |  | PT342 | BLD | 668139.4 | 937765.7 | 319.051 |
| PT83 | SH | 668108.1 | 937843.3 | 317.4 |  | PT343 | SH | 668141.4 | 937749.2 | 319.111 |
| PT84 | SH | 668106.1 | 937847.2 | 317.641 |  | PT344 | SH | 668143.4 | 937751 | 319.05 |
| PT85 | SH | 668102.4 | 937849.2 | 317.563 |  | PT345 | SH | 668144.9 | 937753.8 | 318.948 |
| PT86 | SH | 668101.2 | 937843.6 | 317.586 |  | PT346 | SH | 668147.2 | 937751.6 | 318.691 |
| PT87 | SH | 668099.2 | 937840.3 | 317.489 |  | PT347 | SH | 668146.9 | 937747.9 | 319.135 |
| PT88 | SH | 668098.1 | 937836.4 | 317.051 |  | PT348 | SH | 668144.5 | 937745.9 | 319.098 |
| PT89 | SH | 668094.9 | 937829.9 | 317.405 |  | PT349 | SH | 668143.8 | 937741.8 | 319.252 |
| PT90 | SH | 668099 | 937830 | 317.366 |  | PT350 | BLD | 668137.8 | 937739.5 | 319.454 |
| PT91 | SH | 668102.2 | 937835.3 | 317 |  | PT351 | SH | 668140 | 937740.1 | 319.295 |
| PT92 | SH | 668093 | 937842.7 | 317.597 |  | PT352 | SH | 668141.4 | 937742 | 319.263 |
| PT93 | SH | 668090.5 | 937849.3 | 317.02 |  | PT353 | SH | 668145.1 | 937740.6 | 319.475 |
| PT94 | SH | 668088 | 937845.4 | 317.154 |  | PT354 | SH | 668147.6 | 937743.1 | 319.297 |
| PT95 | CV | 668089.1 | 937840.1 | 317.01 |  | PT355 | SH | 668149.8 | 937746.8 | 319.15 |
| PT96 | CV | 668089.5 | 937840.1 | 317.036 |  | PT356 | SH | 668151.4 | 937750.4 | 319.144 |
| PT97 | CV | 668089.7 | 937835.1 | 316.97 |  | PT357 | SH | 668153.9 | 937753.9 | 319.13 |
| PT98 | CV | 668089.1 | 937835.1 | 316.951 |  | PT358 | BLD | 668156.8 | 937753.8 | 319.16 |
| PT99 | SH | 668087.3 | 937837.4 | 317.032 |  | PT359 | BLD | 668155 | 937750.1 | 319.175 |
| PT100 | SH | 668084.9 | 937841.3 | 317.015 |  | PT360 | GT | 668156.7 | 937750.4 | 319.189 |
| PT101 | SH | 668081.1 | 937845.6 | 316.694 |  | PT361 | GT | 668156.4 | 937744 | 319.379 |
| PT102 | BLD | 668082.4 | 937849.5 | 316.774 |  | PT362 | BLD | 668151.3 | 937744.1 | 319.293 |
| PT103 | SH | 668079.2 | 937850.8 | 316.576 |  | PT363 | BLD | 668157.6 | 937720.9 | 319.595 |
| PT104 | SH | 668076.3 | 937851.1 | 316.606 |  | PT364 | BLD | 668075.3 | 937717.9 | 320.555 |
| PT105 | SH | 668076.4 | 937857.2 | 316.522 |  | PT365 | BLD | 668042.2 | 937720.6 | 320.353 |
| PT106 | SH | 668079.6 | 937859 | 316.481 |  | PT366 | BLD | 668036 | 937729.4 | 320.261 |
| PT107 | SH | 668082.3 | 937859.3 | 316.539 |  | PT367 | BLD | 668035.2 | 937744.9 | 319.623 |
| PT108 | SH | 668081.6 | 937865.3 | 316.444 |  | PT368 | BLD | 668011 | 937744.6 | 320.389 |
| PT109 | SH | 668078.7 | 937867 | 316.454 |  | PT369 | BLD | 668000.4 | 937749.6 | 319.55 |
| PT110 | SH | 668075.8 | 937866.3 | 316.351 |  | PT370 | BLD | 667993.6 | 937757.8 | 318.942 |
| PT111 | SH | 668074 | 937869.4 | 316.329 |  | PT371 | BLD | 667980.6 | 937772.8 | 318.59 |
| PT112 | SH | 668078.7 | 937870.4 | 316.547 |  | PT372 | BLD | 667974.3 | 937782.4 | 318.279 |
| PT113 | SH | 668081.1 | 937872.7 | 316.472 |  | PT373 | BLD | 667968.1 | 937789.6 | 318.03 |
| PT114 | SH | 668081.9 | 937877.2 | 316.469 |  | PT374 | RD | 667966.1 | 937793.6 | 317.734 |
| PT115 | SH | 668079 | 937877.3 | 316.472 |  | PT375 | CL | 667964.4 | 937795.8 | 317.839 |
| PT116 | SH | 668076.3 | 937876 | 316.604 |  | PT376 | RD | 667962.9 | 937797.2 | 317.658 |
| PT117 | EP | 668074.3 | 937876.1 | 316.594 |  | PT377 | SH | 667967.3 | 937798.1 | 317.599 |
| PT118 | TM | 668071.8 | 937879 | 316.342 |  | PT378 | SH | 667972.1 | 937795.5 | 317.821 |
| PT119 | TM | 668076.7 | 937878.8 | 316.445 |  | PT379 | SH | 667976.9 | 937798.1 | 317.559 |
| PT120 | TM | 668077.2 | 937885.8 | 316.536 |  | PT380 | SH | 667975.5 | 937802 | 317.229 |
| PT121 | SH | 668079.9 | 937885 | 316.362 |  | PT381 | SH | 667973.8 | 937805.3 | 317.111 |
| PT122 | SH | 668082.8 | 937883.4 | 316.48 |  | PT382 | SH | 667974.4 | 937809.3 | 317.087 |
| PT123 | SH | 668079.8 | 937880.6 | 316.437 |  | PT383 | SH | 667980.3 | 937808.6 | 317.008 |
| PT124 | SH | 668064.8 | 937896.9 | 315.429 |  | PT384 | SH | 667986.3 | 937808.3 | 317.055 |
| PT125 | BLD | 668064.8 | 937885.4 | 315.743 |  | PT385 | SH | 667989 | 937806.7 | 317.471 |
| PT126 | SH | 668069.7 | 937888.7 | 315.93 |  | PT386 | SH | 667991.4 | 937808.2 | 317.321 |
| PT127 | SH | 668069.8 | 937882.8 | 315.817 |  | PT387 | BLD | 667995.8 | 937809.1 | 317.181 |
| PT128 | EP | 668066.9 | 937881.4 | 316.367 |  | PT388 | BLD | 667996.6 | 937819.4 | 317.014 |
| PT129 | BLD | 668073.2 | 937873.9 | 316.28 |  | PT389 | BLD | 667997.1 | 937825.8 | 316.715 |
| PT130 | SH | 668076.5 | 937842.5 | 316.803 |  | PT390 | BLD | 667997.7 | 937836.1 | 316.436 |
| PT131 | SH | 668080.9 | 937837.3 | 316.882 |  | PT391 | SH | 667999.8 | 937837.9 | 316.489 |
| PT132 | SH | 668076.4 | 937833.4 | 317.117 |  | PT392 | SH | 668004.1 | 937837 | 316.697 |
| PT133 | SH | 668079.1 | 937829.3 | 317.088 |  | PT393 | SH | 668006.3 | 937835.7 | 316.836 |
| PT134 | SH | 668084.6 | 937829.3 | 317.095 |  | PT394 | SH | 668012.2 | 937839.5 | 317.101 |
| PT135 | SH | 668084.6 | 937825.2 | 317.166 |  | PT395 | BLD | 668024 | 937834.2 | 316.943 |
| PT136 | SH | 668080.9 | 937824.4 | 317.192 |  | PT396 | EP | 668028.4 | 937833.8 | 316.701 |
| PT137 | EP | 668075.2 | 937830.8 | 317.409 |  | PT397 | SH | 668027.3 | 937830.5 | 316.834 |
| PT138 | SH | 668076.5 | 937824.8 | 317.309 |  | PT398 | SH | 668024.8 | 937827.5 | 316.95 |
| PT139 | SH | 668080.1 | 937820.9 | 317.285 |  | PT399 | SH | 668027.4 | 937825.1 | 316.977 |
| PT140 | SH | 668083.9 | 937818.9 | 317.208 |  | PT400 | SH | 668025.7 | 937818 | 317.088 |
| PT141 | SH | 668084.3 | 937814.6 | 317.38 |  | PT401 | BLD | 668022.6 | 937817.1 | 317.238 |
| PT142 | SH | 668080.3 | 937812.9 | 317.483 |  | PT402 | SH | 668025.6 | 937813.3 | 317.258 |
| PT143 | SH | 668077 | 937811.8 | 317.722 |  | PT403 | SH | 668030.1 | 937809.1 | 317.516 |
| PT144 | EP | 668075.5 | 937812.1 | 317.622 |  | PT404 | BLD | 668031.1 | 937802.8 | 317.66 |
| PT145 | CV | 668075.9 | 937810.5 | 317.869 |  | PT405 | SH | 668027.5 | 937801.2 | 317.556 |
| PT146 | CV | 668076.6 | 937810.4 | 317.877 |  | PT406 | SH | 668031.4 | 937799.3 | 317.671 |
| PT147 | CV | 668076.1 | 937808 | 317.884 |  | PT407 | BLD | 668035 | 937801.7 | 318.075 |
| PT148 | CV | 668076.5 | 937808 | 317.896 |  | PT408 | SH | 668032 | 937800 | 317.586 |
| PT149 | BLD | 668073.4 | 937815.4 | 317.468 |  | PT409 | SH | 668031.4 | 937796 | 317.74 |
| PT150 | BLD | 668073.5 | 937803.4 | 317.824 |  | PT410 | SH | 668033.4 | 937793.9 | 317.725 |
| PT151 | SH | 668077.3 | 937804.7 | 317.737 |  | PT411 | BLD | 668035.2 | 937792.8 | 317.863 |
| PT152 | SH | 668080.4 | 937805.5 | 317.546 |  | PT412 | BLD | 668029.7 | 937792.2 | 318.048 |
| PT153 | SH | 668083.8 | 937807.7 | 317.514 |  | PT413 | BLD | 668029.5 | 937779.7 | 318.482 |
| PT154 | SH | 668084 | 937803.1 | 317.576 |  | PT414 | BLD | 668035.6 | 937839.2 | 316.76 |
| PT155 | SH | 668080.5 | 937801.9 | 317.645 |  | PT415 | BLD | 668030 | 937849.5 | 316.014 |
| PT156 | SH | 668076.8 | 937799.4 | 318.005 |  | PT416 | BLD | 668014.7 | 937849.8 | 315.771 |
| PT157 | SH | 668080.5 | 937797 | 317.729 |  | PT417 | RD | 668001.4 | 937850 | 315.553 |
| PT158 | SH | 668084.2 | 937798.5 | 317.584 |  | PT418 | RD | 667996.5 | 937852.3 | 315.646 |
| PT159 | SH | 668086.6 | 937795.5 | 317.625 |  | PT419 | BLD | 668015.4 | 937870.2 | 315.216 |
| PT160 | SH | 668088.9 | 937794.7 | 317.784 |  | PT420 | RD | 668022.4 | 937889 | 314.617 |
| PT161 | SH | 668087.8 | 937791.7 | 318.04 |  | PT421 | RD | 668020.3 | 937890.7 | 314.467 |
| PT162 | BLD | 668090.2 | 937790.5 | 318.057 |  | PT422 | BLD | 668035.6 | 937896.8 | 314.523 |
| PT163 | SH | 668086 | 937790 | 318.019 |  | PT423 | CV | 668032.5 | 937898.1 | 314.441 |
| PT164 | SH | 668079.9 | 937790.6 | 317.829 |  | PT424 | CV | 668032.1 | 937898.6 | 314.354 |
| PT165 | SH | 668076.8 | 937791.1 | 317.997 |  | PT425 | EP | 668026.8 | 937902.6 | 314.367 |
| PT166 | EP | 668075.9 | 937791.5 | 318.028 |  | PT426 | EP | 668042.4 | 937923.4 | 313.423 |
| PT167 | SH | 668081.3 | 937788.3 | 317.911 |  | PT427 | EP | 668045.7 | 937913.1 | 313.825 |
| PT168 | SH | 668086.2 | 937787 | 318.037 |  | PT428 | CV | 668045.7 | 937915.3 | 314.041 |
| PT169 | SH | 668088.3 | 937782.1 | 318.256 |  | PT429 | CV | 668045.2 | 937915.7 | 314.017 |
| PT170 | SH | 668083.1 | 937782.1 | 318.113 |  | PT430 | RD | 668050.4 | 937924.9 | 313.193 |
| PT171 | SH | 668077.2 | 937783.1 | 318.448 |  | PT431 | RD | 668048 | 937928.1 | 313.163 |
| PT172 | SH | 668077.1 | 937779.4 | 318.482 |  | PT432 | CL | 668049.6 | 937927.2 | 313.142 |
| PT173 | SH | 668080.8 | 937779.2 | 318.382 |  | PT433 | BLD | 668057.8 | 937924.9 | 313.443 |
| PT174 | SH | 668086.1 | 937777 | 318.527 |  | PT434 | BLD | 668139.5 | 937920.4 | 313.76 |
| PT175 | SH | 668088.4 | 937774.7 | 318.351 |  | PT435 | BD | 668156.3 | 937920.7 | 312.957 |
| PT176 | CV | 668076.7 | 937775.7 | 318.733 |  | PT436 | BLD | 668156.4 | 937893.6 | 315.143 |
| PT177 | CV | 668076.4 | 937775.7 | 318.755 |  | PT437 | BLD | 668120.5 | 937803.6 | 317.77 |
| PT178 | SH | 668075.6 | 937773.1 | 318.598 |  | PT438 | SH | 668121.8 | 937806.3 | 317.702 |
| PT179 | BLD | 668073.5 | 937768.6 | 318.708 |  | PT439 | BLD | 668120.3 | 937810 | 317.679 |
| PT180 | SH | 668073.1 | 937767.9 | 318.686 |  | PT440 | SH | 668124.2 | 937808.2 | 317.693 |
| PT181 | SH | 668075.1 | 937765.4 | 318.807 |  | PT441 | SH | 668124.2 | 937805.5 | 317.683 |
| PT182 | BLD | 668076.2 | 937763 | 318.99 |  | PT442 | SH | 668127.7 | 937808.3 | 317.623 |
| PT183 | SH | 668077.8 | 937762.3 | 318.92 |  | PT443 | SH | 668129.9 | 937805.3 | 317.555 |
| PT184 | SH | 668079.4 | 937758.5 | 319.226 |  | PT444 | SH | 668132.3 | 937808.2 | 317.557 |
| PT185 | SH | 668076.9 | 937758.6 | 318.98 |  | PT445 | SH | 668135.2 | 937805 | 317.569 |
| PT186 | SH | 668079.6 | 937754.5 | 319.302 |  | PT446 | SH | 668139.8 | 937808.7 | 317.523 |
| PT187 | SH | 668077.8 | 937753.6 | 319.218 |  | PT447 | SH | 668142.5 | 937806.5 | 317.477 |
| PT188 | SH | 668078.5 | 937746.8 | 319.408 |  | PT448 | SH | 668143.5 | 937805.6 | 317.641 |
| PT189 | SH | 668076.3 | 937744 | 319.51 |  | PT449 | BLD | 668152.8 | 937810.8 | 317.334 |
| PT190 | SH | 668079.7 | 937740.7 | 319.611 |  | PT450 | BLD | 668152.8 | 937803.9 | 317.388 |
| PT191 | SH | 668080.3 | 937743.2 | 319.569 |  | PT451 | BLD | 668120.4 | 937826.9 | 317.189 |
| PT192 | SH | 668078.8 | 937739.9 | 319.631 |  | PT452 | SH | 668123.3 | 937828 | 316.989 |
| PT193 | SH | 668078.8 | 937737.1 | 319.719 |  | PT453 | SH | 668126.8 | 937828.7 | 316.943 |
| PT194 | SH | 668081 | 937736.3 | 319.732 |  | PT454 | SH | 668126.3 | 937832 | 316.931 |
| PT195 | SH | 668078.1 | 937736.2 | 319.726 |  | PT455 | SH | 668123.9 | 937832.8 | 316.937 |
| PT196 | SH | 668074.3 | 937736.5 | 319.73 |  | PT456 | SH | 668121.8 | 937833.5 | 316.965 |
| PT197 | BLD | 668076 | 937739.9 | 319.718 |  | PT457 | SH | 668128 | 937833 | 316.904 |
| PT198 | SH | 668070.5 | 937735 | 319.725 |  | PT458 | SH | 668128.4 | 937836.4 | 316.889 |
| PT199 | SH | 668067.7 | 937732.6 | 319.769 |  | PT459 | SH | 668135.6 | 937832.3 | 316.921 |
| PT200 | SH | 668065.8 | 937735.6 | 319.673 |  | PT460 | SH | 668134.3 | 937834.4 | 316.926 |
| PT201 | SH | 668064.2 | 937739.6 | 319.579 |  | PT461 | SH | 668138.4 | 937829.2 | 316.911 |
| PT202 | SH | 668061 | 937736.3 | 319.632 |  | PT462 | SH | 668141.1 | 937835.5 | 316.628 |
| PT203 | SH | 668060.3 | 937732.6 | 319.705 |  | PT463 | SH | 668120.5 | 937838.7 | 317.002 |
| PT204 | SH | 668057.4 | 937735.8 | 319.652 |  | PT464 | BLD | 668073.3 | 937862.2 | 316.453 |
| PT205 | SH | 668056.3 | 937739.5 | 319.449 |  | PT465 | SH | 668072.4 | 937859 | 316.477 |
| PT206 | SH | 668052.5 | 937736.1 | 319.664 |  | PT466 | SH | 668069.5 | 937857 | 316.409 |
| PT207 | SH | 668050.9 | 937732.6 | 319.757 |  | PT467 | SH | 668065.8 | 937859.2 | 316.319 |
| PT208 | SH | 668048.6 | 937736.2 | 319.724 |  | PT468 | SH | 668064.3 | 937857.5 | 316.229 |
| PT209 | BLD | 668043.3 | 937739.9 | 319.659 |  | PT469 | SH | 668063.9 | 937853.2 | 316.483 |
| PT210 | BLD | 668043.6 | 937731.8 | 319.686 |  | PT470 | SH | 668055.7 | 937859.2 | 316.296 |
| PT211 | BLD | 668075.8 | 937729.8 | 319.966 |  | PT471 | SH | 668050.3 | 937854.7 | 316.365 |
| PT212 | BLD | 668079.9 | 937728.9 | 320.175 |  | PT472 | BLD | 668050.1 | 937852.9 | 316.654 |
| PT213 | BLD | 668079.7 | 937763.4 | 318.77 |  | PT473 | SH | 668056.8 | 937852.8 | 316.47 |
| PT214 | SH | 668080.8 | 937767.6 | 318.711 |  | PT474 | SH | 668063.8 | 937852.2 | 316.438 |
| PT215 | SH | 668083.6 | 937771.3 | 318.618 |  | PT475 | SH | 668067.5 | 937853.8 | 316.473 |
| PT216 | CV | 668088.4 | 937773.9 | 318.388 |  | PT476 | BLD | 668073.3 | 937851.7 | 316.674 |
| PT217 | CV | 668088.4 | 937774.1 | 318.401 |  | PT477 | BLD | 668073.4 | 937838.8 | 317.02 |
| PT218 | CV | 668088.9 | 937774.3 | 318.362 |  | PT478 | SH | 668071.2 | 937834.5 | 316.988 |
| PT219 | CV | 668089.2 | 937774.2 | 318.355 |  | PT479 | SH | 668071.7 | 937829.5 | 317.224 |
| PT220 | CV | 668089.7 | 937774 | 318.348 |  | PT480 | BLD | 668073.4 | 937827.2 | 317.205 |
| PT221 | CV | 668089.7 | 937773.6 | 318.358 |  | PT481 | SH | 668067.6 | 937828.1 | 317.111 |
| PT222 | SH | 668092.5 | 937776.3 | 318.551 |  | PT482 | SH | 668064.6 | 937832.8 | 317.034 |
| PT223 | SH | 668090.5 | 937779.5 | 318.338 |  | PT483 | SH | 668061.8 | 937837.4 | 316.978 |
| PT224 | SH | 668092.9 | 937780.6 | 318.233 |  | PT484 | SH | 668056.2 | 937836.7 | 316.98 |
| PT225 | SH | 668091.7 | 937784.1 | 318.136 |  | PT485 | SH | 668057.4 | 937831 | 317.048 |
| PT226 | SH | 668090.5 | 937787.6 | 318.117 |  | PT486 | SH | 668058.5 | 937827.6 | 317.15 |
| PT227 | SH | 668095.2 | 937788.3 | 318.215 |  | PT487 | SH | 668050.6 | 937828 | 316.962 |
| PT228 | SH | 668098.3 | 937788.8 | 318.206 |  | PT488 | SH | 668049.9 | 937833.6 | 316.97 |
| PT229 | SH | 668098.2 | 937786.1 | 318.204 |  | PT489 | SH | 668049 | 937837 | 316.957 |
| PT230 | SH | 668097.9 | 937782.4 | 318.229 |  | PT490 | BLD | 668043.9 | 937838 | 317.267 |
| PT231 | SH | 668097.4 | 937777.8 | 318.321 |  | PT491 | BLD | 668073.7 | 937792 | 318.073 |
| PT232 | SH | 668097.1 | 937773.1 | 318.529 |  | PT492 | SH | 668073.1 | 937784.7 | 318.318 |
| PT233 | SH | 668100.9 | 937773.3 | 318.58 |  | PT493 | BD | 668073.2 | 937780.3 | 318.439 |
| PT234 | SH | 668102.2 | 937777.1 | 318.319 |  | PT494 | SH | 668069.9 | 937784.4 | 318.355 |
| PT235 | SH | 668101.1 | 937781.3 | 318.285 |  | PT495 | SH | 668064.9 | 937790.3 | 318.076 |
| PT236 | SH | 668100.2 | 937785.2 | 318.239 |  | PT496 | SH | 668062.6 | 937784.4 | 318.262 |
| PT237 | SH | 668102.1 | 937788.4 | 318.071 |  | PT497 | SH | 668061.5 | 937781.1 | 318.253 |
| PT238 | BLD | 668103.3 | 937790.4 | 317.873 |  | PT498 | SH | 668054.5 | 937781.3 | 318.237 |
| PT239 | SH | 668105.1 | 937786.6 | 318.137 |  | PT499 | SH | 668053 | 937785.2 | 318.171 |
| PT240 | SH | 668105.9 | 937783.1 | 318.189 |  | PT500 | SH | 668051.4 | 937789.3 | 318.097 |
| PT241 | SH | 668106.2 | 937778.4 | 318.236 |  | PT501 | SH | 668045 | 937791.3 | 318.148 |
| PT242 | SH | 668106.3 | 937774.5 | 318.368 |  | PT502 | SH | 668044.7 | 937785.9 | 318.238 |
| PT243 | SH | 668106.4 | 937771.7 | 318.607 |  | PT503 | BD | 668043.5 | 937789.4 | 318.204 |
| PT244 | SH | 668109.8 | 937771.4 | 318.611 |  | PT504 | BD | 668046.8 | 937781.7 | 318.191 |
| PT245 | SH | 668114.7 | 937770.8 | 318.727 |  | PT505 | SH | 668050.5 | 937781.3 | 318.192 |
| PT246 | CV | 668118.6 | 937769.6 | 318.768 |  | PT506 | BLD | 668073.6 | 937768.4 | 318.52 |
| PT247 | CV | 668118.6 | 937769.2 | 318.875 |  | PT507 | SH | 668072.8 | 937765.9 | 318.772 |
| PT248 | BLD | 668120.4 | 937768.5 | 318.906 |  | PT508 | SH | 668068.1 | 937767.9 | 318.712 |
| PT249 | SH | 668118 | 937771.4 | 318.745 |  | PT509 | SH | 668064.3 | 937765.9 | 318.846 |
| PT250 | SH | 668114.6 | 937774.6 | 318.45 |  | PT510 | SH | 668061.8 | 937764.8 | 318.757 |
| PT251 | SH | 668112.3 | 937778.2 | 318.273 |  | PT511 | SH | 668059.2 | 937766.6 | 318.779 |
| PT252 | SH | 668116.9 | 937781 | 318.219 |  | PT512 | SH | 668056.8 | 937767.3 | 318.813 |
| PT253 | SH | 668114.7 | 937784 | 318.086 |  | PT513 | SH | 668055.6 | 937765.9 | 317.208 |
| PT254 | SH | 668110.9 | 937786.4 | 318.028 |  | PT514 | SH | 668054.7 | 937765.1 | 317.217 |
| PT255 | SH | 668108.3 | 937788.8 | 318.063 |  | PT515 | SH | 668050.4 | 937765 | 317.161 |
| PT256 | SH | 668106.5 | 937792.3 | 317.922 |  | PT516 | SH | 668048.2 | 937766.1 | 317.281 |
| PT257 | SH | 668104.3 | 937794.3 | 317.593 |  | PT517 | SH | 668047.6 | 937767.1 | 318.799 |
| PT258 | SH | 668106.6 | 937796.8 | 317.885 |  | PT518 | BLD | 668043.7 | 937764.7 | 322.083 |
| PT259 | SH | 668110.6 | 937794.3 | 317.866 |  |  |  |  |  |  |