CHAPTER ONE

1.0 	INTRODUCTION

1.1 	BACKGROUND OF THE STUDY

The term engineering surveying can be described as a large scale survey operation carried out in order to provide special information for construction purpose. It involves measuring height difference and angle on site either for the preparation of large scale plan or that engineering work can be located in their right position on the earth surface. Route surveying which is the important focus work of this project is define as the process that can be applied to established the horizontal and vertical railways, pipeline, funnel, highways, rapid transit guide way and transmission line.

Route survey provides necessary information for horizontal and vertical alignment for construction and design of the route. Route survey supplies the data necessary to determine alignment, grading and earth work quantities for the construction and design of various engineering work or project. This merged required services and product that adequately locate the planned path of a linear projects or right of way which crosses a prescribed area of real state, extending at another known point.

Route survey system is generally associated with highway design and construction. It requires a proper planning before execution. This is so because the data obtained aid in the construction and engineering design of the road the features (both natural and man-made artificial) were also fixed and as well plotted.

1.2	STATEMENT OF PROBLEMS

It was discovered that the route from New IES road to Ara it is not tarred. Soil erosion had greatly affected the untarred part of road and it has rendered a great portion not motor able. This is greatly affecting the socio-economic activities of those living in the area merged. Procession of the road should be embarked upon in order to increase its carrying capacity thereby increase the physical and human development activities along this road with other useful.

1.3	AIM OF THE STUDY

The aim of the project is to carry out route survey for the road and to produce a digital plan showing both horizontal and vertical alignment as well as profile levelling depicting the nature of the project site for future decision making.

1.4	OBJECTIVES OF THE STUDY

- Reconnaissance which shall be both office and field.

- Planning to pick update: when searching for control and as well the selection of traverse station.

- To carry out the cross and longitude section.

- Processing data.

- Location plan.

1.5	PROJECT SPECIFICATION

The project specification carries out on the site are:

- Traverse must commence on three co-ordinate (known) and closed on another set of the three coordinate control which must be confirmed undisturbed by necessary measurement (control checking).

- Third older theodolite traverse must be run along the route on all turning point at one zero observation.

- Linear measurement must be done with the used of steel tape or electronic distance measurement (EDM).

- levelling must be observed at every 30m interval on the center line at 7.5m and 15m interval on both side of the centre line for the cross sectioning edges of drainage at both side should be heightened.

- Fixing of relevant of enhance assessment and necessary composition for good interpretation of plan.

- Three plan I. E location plan, profile and cross section plan of the route survey should be produced.

1.6	SCOPE OF THE STUDY

The following are the scope to engage in

- Reconnaissance: which shall include both office and field.

- Monumentation: creation of bench mark at every 500m.

- Data acquisition: using total station for X, Y, and Z.

- Data downloading and processing using appropriate suffering.

- Information presentation in form of map and plan.

1.7	PERSONNEL

All the under list name is member of group 8 that participated in the successful execution of the project

| NAME | MATRIC NO | ROLE PLAYED |
| --- | --- | --- |
| Nairabakan Khadijat Gbemisola | ND/23/SGI/FT/0012 | Author |
| Adebunmi Yusuf Ayomide | ND/23/SGI/FT/0006 | Member |
| Laode Abdullahi Alabi | ND/23/SGI/FT/0007 | Member |
| Nurudeen Muhammed Jamiu | ND/23/SGI/FT/0010 | Member |
| Onimago Afisat Idowu | ND/23/SGI/FT/0013 | Member |
| Arewa Eniola Oluwaseun | ND/23/SGI/FT/0015 | Member |
| Yusuf Faisat | ND/23/SGI/FT/0016 | Member |
| Kehinde Rofiat Oluwabukola | ND/23/SGI/FT/0018 | Member |
| Ambrose Sunday | ND/22/SGI/FT/0024 | Member |

1.8	STUDY AREA

The location of the project site is the route from New IES road to ARA village. The distance covered was approximately 3km. The site lies between

LATITUDE -	008˚ 25’ 30’’ to 008˚ 57’ 33’’

LONGITUDE -	004˚ 35’ 23’’ to 004˚ 38’ 10’’

CHAPTER TWO

2.0	LITERATURE REVIEW

Barrister and Raymond (1977) define survey as the art of making measurement of relative position of natural and man-made feature on the surface and the plotting of the measurements to some suitable scale to form a map, plan or chart. Thomas and Noriman (1961) define engineering survey as expression for any survey work carried out in connection with the construction of civil engineering and building project. He also stated that engineers and surveyor involved in site surveying are responsible for all aspects of dimensional control on such scheme.

According to Norman the main purpose of engineering surveying are

- To provide large scale topological maps /plan and other measurement at the conceptual and design stage.

- To monitor structural movement on major retaining structure at the post construction stage.

- To provide precise framework at the construction stage.

Also, engineering surveying is divided into design data-survey and construction survey. A design-data-survey is an orderly process of obtaining data that is needed for the planning and design of an engineering report. The activities involved in design-data-surveying very according to the type and existing owner of the engineering or construction project for example, the activities might include simply obtained topographic data for a proposed building site or they made include extensive route surveying and soil investigation for a highway.

A route survey as the name implies is a survey the deal with the route or course that imaginary road or utility Line will follow while the end product of a route survey for a highway certainly differs from that of a utility line, it may never the less be said that the purpose of the route survey are to:

Selection one or more tentative general route for the roadway or utility.

Mark this final location

Regional (1968) defined route survey as the survey necessary for the location and construction of lines of transportation or communication such as highways, canal, transmissions line and pipeline. He stated that the location and construction survey may consist of Establishing the center line by setting at interval and running level to determine profile of the ground along the centre line;

- taking cross section.

- plotting such profile and fixing grade.

- calculating volume of earthwork.

- measurement of drainage area.

- layout structures such as bridges and culvert

The execution of route surveying entailed some basic process as far as data acquisition is concerned.

These are outlined as follows

- Traversing

- Levelling

- Detailing

- Curve design

- profile and alignment

Traverse is a sequence of connected straight line whose direction and distance have been measured. Traverse is also the process of connecting series of line with known bearing and distance. It is subdivided into first order for precise measurement and second order for the establishment of secondary control while the third order is meant for survey of tertiary and topographical features.

Basak, (2000) explained levelling as the act of determine the relative vertical height of different points on the earth surface. Detailing is defined as the process of fixing topographical features to the survey line. Any of these under listed methods could be used.

- Chain survey method (ties line and offset).

- Plane table survey method.

- Compass survey method.

- By the use total station

Robert (2000) started that designing of route without a large scale and up-to-date napping through dense vegetation is a challenge. He further explained the aerial photograph is very unpunctual and may not give the expected result but air borne LIDAR of remote sensing techniques could be more accurate, faster, reduce the required ground survey control. Brinker and Minick, (1987) stated that tunnels, dams, sewer lines, pipeline and transmission line are engineering construction works having linear shapes which are classified as route. Hence, route survey as a unique system for expressing route geometry has developed to broad range used in executing project by all surveyors, designers and contractors. It was narrated by Vincent Tao and El-sheimy, (2000) that the best way of acquired data and information during the survey for the purpose of rehabilitation of a highway route could be done using mobile napping system.

A comprehensive route survey consists of some operation which are:

- Reconnaissance

- Preliminary survey

- Location survey

- Construction survey

RECONNAISSANCE

A reconnaissance is a rapid and rough survey in which a thorough examination of the area through which the area through which is proposed survey line is to run is conducted to ascertain the best routes and the approximate cost of the project, the reconnaissance survey is the key to the project and therefore must be done under the supervision of an experienced engineer who should be unbiased, resourceful and having wide power of observation for the present as well as for the future requirements. The first step in reconnaissance survey is to collect the available map and aerial photography of the area. Various possible route is marked on the map keeping the general topography of the area in mind. The area under consideration is examined in detail in order to assess the feasibility and relative merit of all the possible routes, in the second step of reconnaissance, approximate elevation and distance are measured the direction and angel are also measured by using magnetic compass. PRELIMINARY SURVEY

A preliminary survey is a detailed survey of a narrow strip of the country through which the proposal line is expected to run. The propose of such survey is to prepare an accurate topographical map of the selected belt of the country to arrive at a close estimate of the cost of the line, preliminary survey also help in the preparation of construction plans. The field work in preliminary survey usually consist of traversing by theodolite along the selected route and the distance, height and angle between the traverse station are measured accuracy in order to prepare a topographical map of the selected narrow strip. The preliminary survey should be quite accurate so that the selection of the final alignment can be made.

LOCATION SURVEYS

This is the process of transferring the paper location of the final alignment of a route to the ground I. E it is the setting out of the paper location on the ground. Sometimes minor changes in the alignment are made in the field location if necessary. The centre line of the final alignment of the route is set out on the ground by the following method.

- By intersection between the paper location of the alignment and the traverse.

- By scaling the position of the various points on the map and translating them to the ground.

Whereas, it could also involve marking of the selected final alignment including the horizontal curves at the desired location on the map of the selected strip. This is known as paper location. The final alignment may be anywhere in the strip and in most favorable position. The longitudinal section strip or profile is the final alignment from the container on the map and grade line is marked on the profile in pencil.

The following point should be considered while selecting the final alignment

- minimum gradient

- minimum curvature

- equalization (balancing) of earth work

- heavy earthwork

- minimum number of expensive bridge

- minimum number of retaining and breast walls

- suitable crossing for river

CONTRUCTION SURVEY

The purpose of the construction survey is to re-established point, lines and grade on the ground for construction of the project. It is essential settings out the details of the route on the ground. It also consists in setting out culvert, bridge, aqueduct, siphons etc. and in carrying on such other surveying as may be needed for the purpose of construction.

A project construction engineers is entrusted with the entire job which may consist the following

Visiting the located Line and checking the location stakes

Resetting the missing or disturbed stakes from the field notes

Checking the level of various points and establishing additional benchmark

Setting out additional stakes at the intersection of the tangents.

- setting out the side slope stakes for earth work as well as grade stakes.

- setting out the curve (simple, transition and vertical)

- setting out the stakes for structures such as culvert, bridge abutment e.t.c.

- setting out the side of barrow pits.

- taking final cross section for ascertain the quantity of earthwork

- determining the water way for each bridge.

- measuring the finished work at regular intervals for regular payments of the constructors.

Finally, route survey involve the determination of ground configuration and location of physical features both natural and artificial along the proposed route, establishing the line on the ground and computing volume of earthwork involve where applicable (schofield,2000) this kind of survey operation is very important on all road network and in construction of new road which will increase the durability and carrying of road network and it also improving the condition of the road and hence alleviate the problem of transportation.

CHAPTER THRRE

3.1 	METHODOLOGY

Methodology is a method and principles /way used to perform a particular activity and the activities is said to be successfully executed, proper planning and the development of the work. Plan show how good and objectives are to be accomplished. The proper planning that was taken for the successful execution of this project and this are involved

- the choice of the most appropriate techniques for execution of the project

- selection of equipment used.

- the design of a monitoring scheme that really helped in achieving the require accuracy for the project, starting from reconnaissance to the final product.

3.1 	RECONNAISSANCE

This is a very important and first aspect in any survey project /work carried out to obtain the general view of the study area in term of nature of the terrain and to properly plan the best way to the set aim and objectives of the project. And it can help to minimize any delays or changes in the observing schedule but for this it was carried out in two ways :

- Office reconnaissance

- Field reconnaissance

3.1.1 	OFFICE PLANNING

It is the act of involved the collection of equipment needed for the project, test of the instrument, missing planning assembling and studying of all available data relevant to the project, the costing of the operation was also done in the office. In executing the project, necessary information related to the project give such as coordinate of the control use were obtained from the SIWES and the practical unit of the school. Office planning is also known as office reconnaissance.

Table 3.1:  show the given bench marks Values

| Stn. I.D | Easting (m) | Northing (m) | Height (m) |
| --- | --- | --- | --- |
| BM1 | 682642.92mE | 944970.38mN | 347.21m |
| BM2 | 682626.44mE | 944965.70mN | 347.56m |
| BM3 | 682636.20mE | 944944.22mN | 348.01m |

3.2	EQUIPMENT USED

The following were the equipment used in the execution of the project

The equipment was grouped in to two categories

3.2.1 Hardware Equipment Used.

- Total station (South NTS 352L) and it's tripod

- one steel tape

- one plumb bob

- one cutlass

- two ranging poles

- paint

- field book

3.3.2 SOFTWARE USED

The software components used for this project are

- Sokkia link

- AUTOCAD civil 3D 2009

- Microsoft office 2010

- Notepad

- HP printer drivers

3.3	TEST OF INSTRUMENT

The instrument (total station) contain an inbuilt program that carries out self-test including signal test collimation, test battery, temperature test, then the instrument use set up and all their temporary adjustment were carried out, horizontal and vertical reading were taken and the telescope transmitted and the target bisected the second time and reading obtained were recorded below.

The two total stations used for this project, that is Leica TS02 and Sokkia set 630R total stations were tested to ensure that they were in good working condition and also to determine if the instrument meet up with the required accuracy of the project at hand which is a third order project. The total station equipment was set up over point (A) and then powered on all temporary station adjustment were carried out. The instrument menu was navigated to the instrument constant sub-menu which was selected after which the instrument then prompt to bisect the target. The target already set upon point (B) was bisected and enter key pressed, the instrument then prompted to bisect the same target again using the second face of the telescope which was done. After the whole observations, the instrument displayed the old and new values for the instrument constant error, the new value was set to replace the old value. The result and analysis are shown in the tables below.

Fig. 3.2 show the instrument set up for collimation test.

Table 3.2:	show the Test of Instrument result.

| Stn. | Sight | Face | Horizontal circle reading | Deduced angle | Vertical circle reading | Sum |
| --- | --- | --- | --- | --- | --- | --- |
| X | Y | L | 2250 30’ 20’’ |  | 0890 32’ 12’’ |  |
|  | Y | R | 450 30’ 20’’ | 1800 00’ 10’’ | 2700 28’ 01’’ | 3600 00’ 13’’ |

Difference in horizontal circle reading = 1800  00’ 10’’

Horizontal collimation error = 1800  00’ 10’’ - 1800  00’ 00’’ = 000  00’  10’’

Collimation error = 000  00I  10II

2

Horizontal collimation error = 000  00I  05II

Sum of vertical circle reading = 3600 00’ 46’’

Vertical collimation error = 3600 00’ 46’’ - 3600 00’ 00’’

= 0000 00’ 13’’

2

= 000  00’  6.5’’

The above table shows the collimation errors and the new reading was used because it is similar to the old reading.

3.4	MONUMENTATION

This is the act of identify the selected and marking them with wooden pegs or nail /bottles cover during the execution of the project. This could be temporary or permanent, depending on the nature of the job specifically for the project. The wooden pegs or bottles cover were 3cm in diameter and above 15cm long with nail on top for easy identification of the point. These were driven into the ground leaving about 2cm above the ground.

Ground level

18cm

18cm

Beneath Ground level

75cm long

60cm

80cm Iron rod

Above Ground level

15cm

Figure 3:1: show the diagram T.B.M

3.5	CONTROL CHECK

Control check involves checking the control used for the execution of the project to know if they are still intact or not. The instrument was set up on a known point (TBM1) and all necessary temporary adjustment were made, the reflector was taken to another known point (TBM2), the angle difference between compute angle, measure angle, computed distance and measured distance are within allowable.

| Stn. I.D | Easting (m) | Northing (m) | Height (m) | Remark |
| --- | --- | --- | --- | --- |
| BM1 | 682642.92mE | 944970.38mN | 347.21m | Given |
| BM1 | 682642.82mE | 944970.31mN | 347.19m | observed |
|  | -0.100 | -0.07 | -0.03 | difference |
| BM2 | 682626.44mE | 944965.70mN | 347.56m | Given |
| BM2 | 682626.46mE | 944965.79mN | 347.62m | observed |
|  | +0.02 | +0.09 | +0.06 | difference |
| BM3 | 682636.20mE | 944944.22mN | 348.01m | Given |
| BM3 | 682636.10mE | 944944.12mN | 348.11m | observed |
|  | -0.10 | -0.01 | +0.10 | difference |

3.6	DATA ACQUISITION

The data for this project work (digital route surveying) were acquired using the total station and its accessories, the automated surveying instrument and their hardware components. The information acquired were for the existence of the natural and man-made features.

3.6.1	DATA SOURCE

Data source is also called a data file, it is simply a collection of records that stores data. Digital route surveying requires input of relevant digital data the data consist of two types;

- Attribute data (Descriptive Information)

- Spatial data representing geographical features (culvert, buildings, area, trees etc.)

There are two types of data source used in this project, i.e. primary and secondary data source

Primary data source is the type of data captured on the field with the use of total station (i.e. X, Y, Z) while secondary data source are the type deals with the coordinate of the control point used for the project

3.6.2	INSTRUMENT OPERATION

The procedure for the operation of instrument is this:

- Orientation of the instrument

- Level of the instrument with the three roof screw

- Rotation of both the vertical and horizontal axis of the instrument to 3600

- Press menu

- Press F1, it will immediately display all the files present

- Press F3 to enter new file name.

- After certifying the above, press enter

- It will take you to page 2

F1 -------- occupy print

F2 -------- back sight

F3 -------- forward sight

- Press F1 to input coordinate

N:

E:

Z:

F1 – input, F2 – back, F3 – space, F4 – enter

- After certifying, press enter, it immediately returns to page 2, then enter the back sight coordinate.

Below shows the result of TBM.

| Stn. I.D | Easting (m) | Northing (m) | Height (m) |
| --- | --- | --- | --- |
| BM1 | 682642.92mE | 944970.38mN | H=347.21m |
| BM2 | 682626.44mE | 944965.70mN | H=347.56m |
| BM3 | 682636.20mE | 944944.22mN | H=348.01m |
| BM4 | 682674.19mE | 945459.38mN | H=341.76m |
| BM5 | 682755.17mE | 945457.52mN | H=342.56m |
| BM6 | 682850.45mE | 945949.18mN | H=350.00m |
| BM7 | 682905.59mE | 945928.27mN | H=349.67m |
| BM8 | 682613.33mE | 944316.61mN | H=351.65m |
| BM9 | 682653.25mE | 944357.07mN | H=352.42m |

CHAPTER FOUR

4.0	DATA PROCESSING AND RESULT ANALYSIS

This are the mathematical process, which entails the computational procedure carried out on raw data collected from the field. These were used in producing the finished product known as plan/chart and it is usually done in the office hence it is called office computation various computation procedures carried out as follow. This stage also involved various computational process such as checks on both linear and angular disclosure of the observed data in order to ascertain if the job carried out is within the accuracy of third order job so as to meet the purpose for which the project was carried out.

4.1	DATA DOWNLOADING AND EDITING

Sokkia link software was used for downloading from Sokkia total station equipment following the procedure below;

- The total station was connected to the desktop computer

- The PC was reboot to establish a new COM port cI created a new project by clicking the new project and named it group five

- On the Sokkia link software, the port set up was clicked and the parameters was set to match to the instrument setting

- I then switched on the station and set the parameters boudrate 9600, data bits and stop bit s1, parity none

- I clicked on ok

- I clicked on send data on the instrument

- The data stored on the instrument was located.

- I clicked data on the menu bar of the software and it dropped down, I clicked receive data

- The connection was clicked on the software

- I then pressed the SEND button on the instrument simultaneously.

- The data was then copied from Sokkia link software to notepad for further processing

4.2	DATA PROCESSING

The downloaded data from the total station was further edited using Microsoft excel and notepad, the final edited copy was saved as the text file containing X, Y and Z coordinate of all points observed in the field.

4.3	RESULT ANALYSIS

The results were analyzed so as to check the accuracy of the job by comparing the results obtained with the minimum allowable error acceptable for this order of survey job in accordance with survey rules and departmental instructions.

4.4	DESIGN AND VOLUME COMPUTATION

The longitudinal profile of the route was created before the designing was done. The longitudinal profile was created using AutoCAD civil 3D land desktop companion 2009. The steps followed are as follows

- The AutoCAD civil 3D land desktop companion 2009 suite was a new project was created and named

- The unit icon was clicked to set the unit of the drawing as follows

- Length type decimal

- Length precision – three places (I.e. 0.000)

- Insertion scale

- Unit to scale inserted content- meters

- Angle type- Deg/Min/Sec.

- Angle precision – Od 00’00”

- The clockwise box below was clicked

- The direction icon below was clicked and a direction control dialog box was opened. The North was selected and Ok was clicked to exit the dialog box

- The point menu was then clicked in the menu bar and the point settings was selected. The “coords.” icon was clicked to select the method of data arrangements (i.e. Easting – Northing). The type of marker was also selected here.

- Under the point menu, the import/ export point icon was selected

- The import point was selected after which a dialog box opens the format in which the data was arranged was selected i.e. ENZ (space delimited). Also the source input data files earlier served and ok was clicked to continue

- The points were imported and the centre lines joined with the polyline drawing tool in AutoCAD toolbar, the OSNAP (object snap) was switched on so as to snap the polyline ends to the points to enable precise and easy joining

- Alignment was then selected from the menu bar and define from sub-menu was selected

- The polyline used in joining all the center line was then clicked, where the alignment was to start was also clicked. A dialog box then opens and the alignment was named.

- On the alignment menu on menu bar, the station label settings was clicked, the station label increment, station tick increment and station label offset were edited appropriately. The perpendicular labels and plus sign location was also clicked. Then the station labels were created.

- The terrain was created by clicking on the terrain menu on the menu bar, then “Terrain Model Explorer” was selected. A dialogue box opens up and a new surface was created and named. The points file was added and the arrangement for format selected as well as the file path where it was located. The surface was built by right clicking on it and select and the dialog box was closed after building the surface.

5.4	LINEAR ACCURACY CALCULATION

This was computed using the formulae:

Linear Accuracy =        1_____

√∆E² + ∆N²

total distance

Linear Accuracy = 1 ÷ √ (0.010) ² + (-0.008) ² ÷ (3206.115)

Linear Accuracy = 1 ÷ √0.0004+0.0064 ÷ (3206.115)

Linear Accuracy = 1 ÷ √0.0065 ÷ (3206.115)

Linear Accuracy = 1 ÷ 0.0806225 ÷ (3206.115)

Linear Accuracy = 1 ÷ 0.000025146

Linear Accuracy = 1 ÷ 39766.962

Linear Accuracy = 1: 39,000

Expected Linear Accuracy = 1:3,000

Obtained Linear Accuracy= 1: 39,000

4.5	RESULT DISPLAY

This show the figure of horizontal alignment and the vertical alignment obtain during the processing of the result.

HORIZONTAL ALIGNMENT

VERTICAL ALIGNMENT

4.7	APPLICATION OF THE PRODUCT

- The horizontal alignment is used for correcting horizontal curves along the route during the construction work.

- Horizontal alignment is also used for location and allocation of the route facilities like, pipe lines, electricity line, setting out of drainages some case for plots alignments.

- Vertical alignment is used to correct for the vertical curves along the road during the construction.

CHAPTER FIVE

SUMMARY, PROBLEM ENCOUNTERED, RECOMMENDATION AND CONCLUSION

5.1 	SUMMARY

The reconnaissance, which was the main planning was carefully carried out because of its important to good execution of any survey work /project. The purpose or significance of study, the scope and the personnel involved to the study area was also stated in this project report and appropriate methodology was adopted in other to achieve the aim and objectives.

5.2 	PROBLEM ENCOUNTERED

During execution of this project, the under listed problems were encountered

1. The movement of people and vehicles was a problem which inhibit the rate at which the operation was carried out

2. Another problem is weather condition of the temperature sometimes it become tense and unbearable.

3. Due to the nature of total station being determine type, changing of batteries was another case has light supply was mad quote.

5.3	RECOMMENDATIONS

Having participated fully to the success of this project, I hereby recommend the following to the department and school authority.

- The school should provide sufficient number of instrument so that every project and practical exercise would be carried out very fast.

- There is need to provide walkies for effective communication in the field.

- Practical oriented courses should be in all level, so that every student will be familiar with the instrument.

- The instrument must be in the reach of every student at any given time.

5.4	CONCLUSION

This project covered field observation procedure, data acquisition data downloading and graphical plotting for the production of plan showing the part of Kwara State Polytechnic road network. The acquisition of data was done by the use of total station and its accessories with some supportable equipment which has already been mentioned before, data were able to be downloaded by downloading cable and plotting commenced immediately using AutoCAD and Civil CAD software for plotting route alignment and longitudinal profile.

Despite the problem encountered during the execution of this project with the respect to the instruction, the result obtained were of accepted accuracy, these were achieving due to the precaution taken on the field during the execution.

REFERENCE

Banister and Raymond (1977):	Surveying Great Britain, 2nd Edition, pg. 222, chapter 6.

Thomas and Norman (1961):	Tachometry. A practical Treatise Great Britain, Chapter 1, pg 1.

Reginnal (1968): Engineering Surveying, Butterworth-Heinemann

Basak (2000):	Surveying and levelling, Tata Mc. raw Hill Publishing company Delhi pg. 173, 439,444.

Robert (2000):	Plane and Geodetic Surveying for Engineering 6th Edition, Vol. I. London: Constable and co. Limited pg. 144.

Brinker and Minick (1987):	Elementary surveying 6th Edition, Itp-A, Donnelley New York. Pg.3.

Toa and El-Sheimy (2000): Land surveying

APPENDIX

| 6 | 682025.8 | 945084.8 | 99.413 |
| --- | --- | --- | --- |
| 7 | 682038.6 | 945093.7 | 100.5105 |
| 8 | 682050 | 945106.5 | 100.9379 |
| 9 | 682005.6 | 945096.7 | 99.1825 |
| 10 | 682006.7 | 945112.1 | 99.2829 |
| 11 | 682017.2 | 945093.9 | 99.7548 |
| 12 | 682021.9 | 945093.8 | 100.1744 |
| 13 | 682026.4 | 945093.5 | 100.3013 |
| 14 | 682042.6 | 945073.5 | 99.75 |
| 15 | 682044.2 | 945076.8 | 99.7942 |
| 16 | 682046.7 | 945079.7 | 99.873 |
| 17 | 682068.9 | 945060.8 | 99.905 |
| 18 | 682066 | 945056.8 | 99.7495 |
| 19 | 682071.3 | 945064.3 | 100.0481 |
| 20 | 682090.7 | 945043.6 | 100.3955 |
| 21 | 682092.8 | 945046.5 | 100.4931 |
| 22 | 682088 | 945040.3 | 100.3069 |
| 23 | 682112.8 | 945025.4 | 101.0093 |
| 24 | 682115.7 | 945028.5 | 101.0807 |
| 25 | 682128.3 | 945013.1 | 101.5195 |
| 26 | 682135.6 | 945005 | 101.6583 |
| 41 | 682135.6 | 945005 | 101.658 |
| 27 | 682128.2 | 945013.1 | 101.5286 |
| 28 | 682109.2 | 945022.7 | 100.7413 |
| 29 | 682131.3 | 945003.6 | 101.661 |
| 30 | 682128.6 | 945001.2 | 101.4917 |
| 31 | 682134.5 | 945006.1 | 101.5765 |
| 32 | 682147.2 | 944979.6 | 102.1656 |
| 33 | 682143.5 | 944977.3 | 101.9381 |
| 34 | 682150.4 | 944981.2 | 102.189 |
| 35 | 682163.3 | 944955.1 | 101.2801 |
| 36 | 682160.1 | 944952.7 | 101.0766 |
| 37 | 682166.8 | 944957.3 | 101.3514 |
| 38 | 682177.7 | 944931.9 | 100.9789 |
| 39 | 682174.2 | 944929.6 | 100.9376 |
| 40 | 682181.1 | 944934.2 | 101.1852 |
| 41 | 682192.5 | 944907.2 | 101.108 |
| 42 | 682189.5 | 944905.5 | 100.9958 |
| 43 | 682195.5 | 944909.2 | 101.0546 |
| 44 | 682208 | 944884.2 | 101.3188 |
| 45 | 682204.9 | 944881.9 | 101.278 |
| 46 | 682211.2 | 944886.6 | 101.4122 |
| 47 | 682221.6 | 944858.1 | 101.4747 |
| 48 | 682225 | 944860.4 | 101.5198 |
| 49 | 682228.4 | 944862.6 | 101.72 |
| 50 | 682238.2 | 944834.6 | 101.6794 |
| 51 | 682240.9 | 944836.6 | 101.844 |
| 52 | 682244.2 | 944838.6 | 101.9053 |
| 53 | 682254.6 | 944811.7 | 102.2803 |
| 54 | 682257.5 | 944813.9 | 102.2792 |
| 55 | 682260.7 | 944816.2 | 102.3919 |
| 56 | 682271 | 944788.3 | 102.3721 |
| 57 | 682273.9 | 944790.3 | 102.6997 |
| 58 | 682276.6 | 944792.1 | 102.5937 |
| 59 | 682286.6 | 944766.1 | 102.8219 |
| 60 | 682290 | 944768.2 | 103.0714 |
| 61 | 682304.6 | 944740.8 | 103.2686 |
| 62 | 682310.1 | 944732.5 | 103.3306 |
| 42 | 682304.6 | 944740.8 | 103.269 |
| 63 | 682310.1 | 944732.4 | 103.3666 |
| 64 | 682292.6 | 944772.5 | 103.0856 |
| 65 | 682304.2 | 944746.8 | 103.4408 |
| 66 | 682301.2 | 944744.5 | 103.1813 |
| 67 | 682323.2 | 944726.4 | 103.7364 |
| 68 | 682319.9 | 944724 | 103.6967 |
| 69 | 682316.1 | 944721.7 | 103.6322 |
| 70 | 682335.8 | 944700.8 | 104.2236 |
| 71 | 682338.5 | 944702.9 | 104.2273 |
| 72 | 682332.8 | 944698.3 | 104.3279 |
| 73 | 682354.8 | 944679.3 | 104.6155 |
| 74 | 682351.9 | 944677.2 | 104.6354 |
| 75 | 682348.3 | 944675.1 | 104.6815 |
| 76 | 682365.7 | 944652.9 | 105.0503 |
| 77 | 682362.4 | 944650.6 | 105.0144 |
| 78 | 682368.9 | 944655.1 | 105.0834 |
| 79 | 682384.6 | 944631.2 | 105.4746 |
| 80 | 682381.2 | 944629.1 | 105.561 |
| 81 | 682376.6 | 944629.5 | 105.4252 |
| 82 | 682396.8 | 944608.4 | 105.962 |
| 83 | 682393.7 | 944607.1 | 105.9661 |
| 84 | 682390.5 | 944605.4 | 105.9599 |
| 85 | 682408.2 | 944582.9 | 106.4976 |
| 86 | 682405 | 944581.3 | 106.4912 |
| 87 | 682411.2 | 944584.8 | 106.3901 |
| 88 | 682423.4 | 944559.5 | 106.6502 |
| 89 | 682420 | 944557.5 | 106.7922 |
| 90 | 682418.1 | 944555.9 | 106.9239 |
| 91 | 682422.8 | 944559.9 | 106.6742 |
| 92 | 682425.1 | 944554.6 | 106.769 |
| 43 | 682422.8 | 944559.9 | 106.674 |
| 44 | 682422.8 | 944559.9 | 106.674 |
| 93 | 682425.1 | 944554.6 | 106.77 |
| 94 | 682415.6 | 944557.5 | 106.7882 |
| 95 | 682427 | 944532.2 | 107.0686 |
| 96 | 682429.8 | 944533.7 | 107.1339 |
| 97 | 682433.4 | 944535.4 | 107.0119 |
| 98 | 682439.2 | 944506.3 | 107.4392 |
| 99 | 682443.4 | 944508 | 107.5352 |
| 100 | 682446.7 | 944509.7 | 107.4372 |
| 101 | 682452.5 | 944482.6 | 107.8221 |
| 102 | 682456.2 | 944484.7 | 107.9294 |
| 103 | 682459.4 | 944486.6 | 107.8592 |
| 104 | 682466 | 944457.9 | 108.3992 |
| 105 | 682469.3 | 944459.5 | 108.5506 |
| 106 | 682472 | 944461 | 108.3765 |
| 107 | 682482.1 | 944434.8 | 109.1427 |
| 108 | 682478.5 | 944433.1 | 108.9704 |
| 109 | 682493.7 | 944408.6 | 109.7479 |
| 110 | 682483.6 | 944436.4 | 109.12 |
| 111 | 682491.3 | 944407.3 | 109.6568 |
| 112 | 682496.4 | 944410.1 | 109.813 |
| 113 | 682505.9 | 944382.1 | 110.7567 |
| 114 | 682503.2 | 944380.5 | 110.7638 |
| 115 | 682508.3 | 944383.2 | 110.7571 |
| 116 | 682516.6 | 944355.3 | 111.6726 |
| 117 | 682519.4 | 944356.7 | 111.6934 |
| 118 | 682517 | 944354.3 | 111.5071 |
| 119 | 682519.7 | 944349.9 | 111.8667 |
| 45 | 682517 | 944354.3 | 111.507 |
| 120 | 682519.7 | 944349.9 | 111.6799 |
| 121 | 682522.9 | 944358.4 | 111.3804 |
| 122 | 682533.8 | 944331.7 | 112.5338 |
| 123 | 682536.4 | 944333.7 | 112.4887 |
| 124 | 682531 | 944330 | 112.386 |
| 125 | 682552.5 | 944310.3 | 113.4871 |
| 126 | 682550.3 | 944308.6 | 113.5288 |
| 127 | 682547.6 | 944306.6 | 113.4817 |
| 128 | 682570.5 | 944287.1 | 114.37 |
| 129 | 682568.1 | 944285.2 | 114.4096 |
| 130 | 682565.9 | 944283.4 | 114.3668 |
| 131 | 682585.8 | 944263.5 | 115.29 |
| 132 | 682588.3 | 944265.3 | 115.2021 |
| 133 | 682583.1 | 944261.3 | 115.1886 |
| 134 | 682603.6 | 944241.7 | 115.7531 |
| 135 | 682606.1 | 944243.8 | 115.689 |
| 136 | 682601.1 | 944239.8 | 115.6595 |
| 137 | 682623.9 | 944220.6 | 116.1817 |
| 138 | 682621.7 | 944218.8 | 116.2727 |
| 139 | 682619.1 | 944216.7 | 116.3477 |
| 140 | 682642.1 | 944197.2 | 116.7863 |
| 141 | 682639.2 | 944195.3 | 116.8587 |
| 142 | 682636.6 | 944192.8 | 116.901 |
| 143 | 682660.2 | 944173.6 | 117.3205 |
| 144 | 682656.6 | 944171.6 | 117.3986 |
| 145 | 682653.6 | 944169.5 | 117.3994 |
| 146 | 682676.2 | 944150.6 | 117.783 |
| 147 | 682672.9 | 944148.6 | 117.8805 |
| 148 | 682669.6 | 944146.3 | 117.8648 |
| 149 | 682692 | 944127.1 | 118.2677 |
| 150 | 682688.8 | 944125.1 | 118.3393 |
| 151 | 682685.9 | 944122.9 | 118.3408 |
| 152 | 682696.9 | 944119.8 | 118.3344 |
| 153 | 682694 | 944124.6 | 118.2354 |
| 46 | 682694 | 944124.6 | 118.235 |
| 154 | 682696.9 | 944119.7 | 118.3095 |
| 155 | 682686 | 944123.2 | 118.2222 |
| 156 | 682707.4 | 944102 | 118.6681 |
| 157 | 682704.4 | 944100.4 | 118.7399 |
| 158 | 682700.8 | 944098.3 | 118.6396 |
| 159 | 682722.3 | 944077.9 | 119.0533 |
| 160 | 682719.1 | 944076.2 | 119.1673 |
| 161 | 682715.5 | 944074.2 | 119.1063 |
| 162 | 682738.3 | 944053.4 | 119.2849 |
| 163 | 682734.5 | 944051.7 | 119.382 |
| 164 | 682730.4 | 944049.6 | 119.2995 |
| 165 | 682754.8 | 944030.1 | 119.211 |
| 166 | 682750.9 | 944027.5 | 119.3803 |
| 167 | 682746.9 | 944024.9 | 119.3085 |
| 168 | 682771 | 944006.4 | 119.1626 |
| 169 | 682767.1 | 944004 | 119.3029 |
| 170 | 682763.8 | 944002.1 | 119.2275 |
| 171 | 682788.1 | 943982.4 | 119.2347 |
| 172 | 682783.8 | 943979.7 | 119.4079 |
| 173 | 682779.2 | 943976.5 | 119.3832 |
| 174 | 682804.3 | 943957.2 | 119.419 |
| 175 | 682800.4 | 943954.9 | 119.5152 |
| 176 | 682797 | 943952.7 | 119.4804 |
| 177 | 682817.9 | 943931.1 | 119.7777 |
| 178 | 682821.5 | 943932.8 | 119.7828 |
| 179 | 682815.1 | 943928.8 | 119.8386 |
| 180 | 682836 | 943900.3 | 120.2448 |
| 181 | 682836.4 | 943900.4 | 120.2463 |
| 47 | 682836 | 943900.3 | 120.245 |
| 182 | 682835.2 | 943892.8 | 120.119 |
| 183 | 682808.9 | 943902.9 | 120.6629 |
| 184 | 682809.5 | 943905.2 | 120.6695 |
| 185 | 682810.3 | 943908.1 | 120.6672 |
| 186 | 682782.5 | 943911.9 | 121.2278 |
| 187 | 682783.9 | 943914.4 | 121.2087 |
| 188 | 682784.9 | 943917.2 | 121.1419 |
| 189 | 682756 | 943920.7 | 121.7873 |
| 190 | 682757 | 943923.8 | 121.8478 |
| 191 | 682757.9 | 943926.8 | 121.9166 |
| 192 | 682728.2 | 943930.3 | 122.4308 |
| 193 | 682729.4 | 943933.3 | 122.5375 |
| 194 | 682730.2 | 943935.5 | 122.5611 |
| 195 | 682701.6 | 943940.9 | 122.9612 |
| 196 | 682703.2 | 943944.1 | 123.152 |
| 197 | 682670.9 | 943956.3 | 123.9009 |
| 198 | 682676.8 | 943953 | 123.7493 |
| 48 | 682670.9 | 943956.3 | 123.9 |
| 199 | 682675.1 | 943954 | 123.8231 |
| 200 | 682676.8 | 943953.1 | 123.7431 |
| 201 | 682675.1 | 943954 | 123.8217 |
| 202 | 682676.5 | 943956.6 | 123.747 |
| 203 | 682678.7 | 943960.3 | 123.705 |
| 204 | 682652 | 943966.9 | 124.2516 |
| 205 | 682653.7 | 943970 | 124.3123 |
| 206 | 682655.3 | 943973.6 | 124.2712 |
| 207 | 682625.2 | 943986 | 124.9082 |
| 208 | 682623.3 | 943982.6 | 124.9048 |
| 209 | 682626.8 | 943989.1 | 124.8581 |
| 210 | 682595.6 | 944003.9 | 125.3075 |
| 211 | 682594.2 | 944000.8 | 125.4026 |
| 212 | 682597.1 | 944006.4 | 125.5699 |
| 213 | 682566.3 | 944022.7 | 125.9199 |
| 214 | 682564.3 | 944019 | 125.8905 |
| 215 | 682567.8 | 944025.2 | 125.8537 |
| 216 | 682538.4 | 944040.1 | 126.2323 |
| 217 | 682539.5 | 944042 | 126.1642 |
| 218 | 682536.6 | 944034.8 | 126.1717 |
| 219 | 682507.4 | 944050.1 | 126.4418 |
| 220 | 682508.3 | 944053 | 126.4105 |
| 221 | 682504.6 | 944054.2 | 126.4461 |
| 222 | 682501.7 | 944055 | 126.4166 |
| 49 | 682504.6 | 944054.2 | 126.446 |
| 223 | 682501.5 | 944055 | 126.4538 |
| 224 | 682506 | 944044.4 | 126.3143 |
| 225 | 682477.2 | 944056.5 | 126.7075 |
| 226 | 682478.5 | 944060.3 | 126.537 |
| 227 | 682476.6 | 944051.1 | 126.5581 |
| 228 | 682446.5 | 944062.1 | 126.7904 |
| 229 | 682445.7 | 944058 | 126.7217 |
| 230 | 682447 | 944066.2 | 126.7224 |
| 231 | 682415.8 | 944067.2 | 126.8935 |
| 232 | 682416.5 | 944072 | 126.7752 |
| 233 | 682415.1 | 944062.7 | 126.7175 |
| 234 | 682385.6 | 944073.1 | 126.9591 |
| 235 | 682387 | 944077.7 | 126.7497 |
| 236 | 682384.5 | 944068.3 | 126.7548 |
| 237 | 682354.7 | 944079.4 | 127.0357 |
| 238 | 682353.8 | 944074.8 | 126.9073 |
| 239 | 682323.4 | 944079.6 | 126.9917 |
| 240 | 682324.9 | 944084.8 | 127.165 |
| 241 | 682295.1 | 944095 | 126.8495 |
| 242 | 682294.4 | 944090.9 | 127.0306 |
| 243 | 682293 | 944086.2 | 126.8278 |
| 244 | 682261.1 | 944097.3 | 126.9411 |
| 245 | 682261.6 | 944102 | 126.8073 |
| 246 | 682259.4 | 944091.9 | 126.8211 |
| 247 | 682493.3 | 944056.3 | 126.4576 |