CONSTRUCTION OF A WOODEN SUSPENDED GROUND FLOOR

BY

AHMAD KHALID OLUMOH

ND/23/CEC/FT/0031

A PROJECT SUBMITTED TO THE DEPARTMENT OF CIVIL ENGINEERING TECHNOLOGY, INSTITUTE OF TECHNOLOGY, KWARA STATE POLYTECHNIC ILORIN

IN PARTIAL FULFILLMENT OF THE REQUIREMENT FOR THE AWARD OF NATIONAL DIPLOMA (ND) IN CIVIL ENGINEERING TECHNOLOGY

JULY 2025

DECLARATION

I hereby declare that this project tile: CONSTRUCTION OF A WOODEN SUSPENDED GROUND FLOOR is a collection of my original research work and it has not been presented for any other qualification anywhere.

CERTIFICATION

This is to certify that the research study was conducted by AHMAD KHALID OLUMOH

and approved as meeting the requirement of the Department of Civil Engineering, Institute of Technology, Kwara State Polytechnic Ilorin, to partial Fulfillment of Requirement for the award of National Diploma in Civil Engineering.

..........................................                                                                                      .......................................

ENGR MUJI RAJI.                                                                                                             Date

Project Supervisor

............................................                                                                                 .......................................

ENGR. A. NA'ALLAH.                                                                                                                 Date

Head of Department

..............................................                                                                                  ....................................

External Examiner                                                                                                                 Date

ACKNOWLEDGEMENT

First and foremost, I thank God Almighty for making this vision a reality. The foundation of everything in life is of utmost importance. I thank God Almighty for his endless blessings upon me, and for the opportunity to partake in two years National Diploma (ND) programe, I'm thankful to him for his guidance throughout the two years programe.

I extend my acknowledgement to my beloved supervisor Engr Muji Raji for her knowledge used to lighten up my darkness, may Almighty God continue impact you with more knowledge and bless you and your entire family. My sincere gratitude also goes to Engr Sanni Ridwan who supervised me for the project practical may Almighty God continue to help and strengthen him, my sincere gratitude also to the HOD of Civil Engineering Department, Engr Nallah May Almighty God bless him.

I'm immensely indebted to my  beloved beloved parents Mr & Mrs Ahmad for their parental care over me before and after this program. I thank them for their dearest love and support during the two years programe.

Also I show a heart of gratitude to my friends, colleagues and relatives for their support and words of encouragement during the period of my program. I'm really grateful to you all.

ABSTRACT

This report presents the design, construction, and performance evaluation of a wooden suspended ground floor system, a traditional yet sustainable flooring method commonly used in residential and light commercial buildings. The suspended timber floor system is constructed above ground level, supported by sleeper walls or a perimeter foundation, and is especially suitable for sites with poor soil conditions, high water tables, or the need for underfloor ventilation.

The construction process involves the use of treated timber joists, floorboards or engineered wood panels, and appropriate insulation and damp-proofing layers to ensure structural integrity, thermal efficiency, and protection against moisture. Key considerations include proper ventilation to prevent timber decay, the selection of durable and treated wood materials, and compliance with building regulations.

This method offers several benefits including ease of installation, adaptability to uneven terrain, improved access to services (such as plumbing and wiring), and environmental sustainability when responsibly sourced timber is used. The abstract also highlights challenges such as moisture control, pest prevention, and long-term maintenance.

Through this study, the viability of wooden suspended ground floors is reaffirmed as a cost-effective, environmentally friendly alternative to solid concrete slabs in appropriate conditions.

TABLE OF CONTENT

i. Title page

ii. Declaration

iii. Certification

iv. Acknowledgement

v. Abstract

CHAPTER ONE: INTRODUCTION TO THE CONSTRUCTION OF A WOODEN SUSPENDED GROUND FLOOR

1.1. Background and purpose of the project

1.2. Scope of the project

1.3. Importance of a suspended wooden ground floor

1.4. Aims and Objectives of the study

CHAPTER TWO: LITERATURE REVIEW

2.1. Historical content

2.2. Overview of a suspended wooden ground floor

2.3. Advantages and Disadvantages of a suspended wooden ground floor

2.4. Challenges in suspended wooden floor construction

2.5. Comparison between timber and concrete suspended floors

CHAPTER THREE: METHODOLOGY

3.1. Structural requirements

3.2. Load analysis and calculations

3.3. Moisture and pest protection

3.4.  Design phase

3.5. Material selection

CHAPTER FOUR: CONSTRUCTION PROCESS

4.1. Site preparation

4.2. Laying foundation and supporting walls

4.3. Installation of floor joists

4.4. Fixing floor boards and finishes

4.5. Safety measures

4.6.  Testing and Evaluation

CHAPTER FIVE

5.1. Conclusion

5.2. Recommendation

5.3. Reference

CHAPTER ONE

INTRODUCTION TO THE CONSTRUCTION OF A WOODEN SUSPENDED GROUND FLOOR

A suspended ground floor refers to a ground floor that is elevated above the ground level, usually supported by beams, columns, or other structural elements. This type of construction is common in areas with high water tables, flood-prone zones, or where the ground is uneven. The space underneath the floor may be left open or used for ventilation, storage, or other purposes. The suspension helps prevent issues like water damage or dampness from affecting the building.

A wooden suspended ground floor is a floor made of timber joists and floorboards that are raised above the ground by small supporting walls. This creates a gap between the floor and the ground that allows for air circulation. The gap beneath the floor allows for ventilation and helps prevent moisture buildup, which is especially useful in areas with a high water table or in regions prone to damp conditions. This design also allows easier access to plumbing and wiring that may run beneath the floor. It is often found in older homes and buildings but can still be used in modern construction, particularly in certain climates or regions.

The construction of a wooden suspended ground floor is a specialized building technique that involves creating a floor structure elevated above the ground, typically supported by timber beams or posts. This technique is commonly used in areas where the ground is prone to moisture, flooding or where traditional foundations are not feasible. The suspended wooden ground floor ensures a stable and dry environment for the building's interior, promoting longevity and reducing the risk of damage from environmental factors. Suspended wooden ground floor provide an effective solution for managing these issues while offering several advantages over traditional concrete slab.

1.1. BACKGROUND AND PURPOSE OF THE PROJECT

Suspended floors were historically used in many region as a method to overcome issues with ground-level moisture. In modern construction, they are still used in areas with high groundwater tables, in earthquake-prone zones, or where the ground is uneven. The use of timber in these floors provides both structural support and aesthetic appeal, making it a popular choice for residential and commercial buildings.

It is designated to elevate the floor of a building above the ground level, typically supported by beams and joists made from timber or steel. This type of structure not only helps to mitigate the effects of moisture, rising damp and pests but also allows for better air circulation underneath the building, improving the overall health and logetivity of the property. Additionally, wooden floors are considered an eco-friendly option as they make use of renewable materials, helping reduce the carbon footprint of the construction.

PURPOSE

The purpose of this project is to analyze the processes involved in construction of a wooden suspended ground floor, identify potential design challenges, and provide practical solutions for ensuring the stability of the floor system.

It is also to create a raised, ventilated space above the ground level, offering several practical benefits. Key purposes include:

1. MOISTURE CONTROL: By elevating the floor above the ground, a suspended floor helps prevent moisture from seeping into the building, which can cause dampness, mold, and rot. The gap between the ground and floor allows for airflow, reducing the chances of moisture buildup underneath.

2. VENTILATION: The space beneath the floor acts as a natural ventilation system, allowing air to circulate freely. This helps to keep the building dry, particularly in areas with high humidity or in regions where groundwater levels can fluctuate.

3. THERMAL INSULATION: The air space under the suspended floor can also provide thermal insulation. In colder climates, this air gap can help maintain a more consistent indoor temperature by preventing cold from transferring directly from the ground into the living space.

4. PREVENTION OF GROUND MOVEMENT: A wooden suspended ground floor can accommodate slight movements of the ground beneath the structure, such as shifting or settling. The raised design allows the building to flex without directly impacting the floor structure.

5. PROTECTION FROM FLOODING: In flood-prone areas, a suspended ground floor raises the living space above potential flood levels, providing a layer of protection against water damage.

6. ACCESS TO UTILITIES: The void beneath the suspended floor provides easy access to utilities like plumbing, electrical wiring, and drainage, making maintenance and repairs easier.

7. PEST CONTROL: Elevating the floor can help in reducing pest infestation, as the raised space may deter rodents or insects from entering the building.

Overall, a wooden suspended ground floor is designed to increase durability, comfort, and safety, particularly in regions with unstable or wet soil conditions.

1.2. SCOPE OF THE PROJECT

This project covers the entire process involved in constructing a suspended wooden ground floor, from initial planning and design through to the final installation. It will include detailed information on material selection, structural consideration, and construction techniques, as well as the benefits and potential drawbacks of using timber for such a structure.

1. DESIGN & PLANNING:

Structural Design: The floor design needs to accommodate the load-bearing requirements, including the support structure for the suspended floor.

Building Codes & Permits: Ensuring the design complies with local building codes and obtaining the necessary permits.

Environmental Factors: Considering factors such as moisture, temperature, and air circulation, which are crucial for wooden floors, especially in suspended designs.

2. MATERIALS:

Wood: Select appropriate timber, such as treated softwood, hardwood, or engineered wood, based on the load requirements and environmental conditions.

Foundation Supports: Concrete or steel beams may be required to support the suspended structure, depending on the design.

Floor Decking: Timber boards or panels (e.g., plywood, OSB) are used for the actual flooring.

Subfloor Ventilation: Ensure adequate airflow beneath the suspended floor to avoid moisture buildup and wood degradation.

3. SITE PREPARATION:

Excavation: Prepare the foundation by excavating trenches or placing footings for support.

Subfloor Clearance: Ensure adequate space (typically a few inches to a few feet) between the ground and the wooden floor for ventilation.

4. CONSTRUCTION:

Frame Installation: Construct the timber frame, including joists, beams, and supporting posts, to create the suspended structure.

Decking Installation: Install the floor decking material, ensuring it is properly secured and aligned.

Ventilation and Insulation: Install any necessary ventilation systems (such as air bricks) and consider insulation between the joists if needed for energy efficiency.

Moisture Barriers: Apply moisture barriers or membranes if needed to prevent rot and deterioration.

5. FINISHING:

Flooring Treatment: Apply finishes to the wooden floor (e.g., sealing, staining) to enhance durability and aesthetics.

Final Inspection: Ensure the structure meets safety and regulatory standards, with the floor properly anchored and level.

The construction of a suspended wooden ground floor can be a cost-effective and energy-efficient solution, but it requires careful planning and execution to ensure stability, durability, and moisture control.

1.3. IMPORTANCE OF A SUSPENDED WOODEN GROUND FLOOR

1. EASY ACCESS TO PLUMBING AND ELECTRICAL SYSTEMS

Makes maintenance easier: The flexibility of suspended timber floors makes it easy to access plumbing and electrical systems.

Durability: Can last a long time. With proper installation and regular maintenance, suspended timber floors can be durable

Aesthetic appeal: Can be a timeless addition to a space. Suspended timber floors can add aesthetic appeal to a space

2. STRUCTURAL INTEGRITY: It can protect the building from ground movement or settlement. A well-designed suspended floor can adapt to slight shifts in the ground without compromising the structure.

3. ADAPTABILITY TO GROUND MOVEMENT: Suspended floors are more adaptable to slight movements or settling of the ground, which can help preserve the overall structural integrity of the building.

4. SUSTAINABILITY: In some cases, a suspended floor can be more sustainable by using timber from renewable sources, offering an eco-friendly alternative to concrete or other materials.

5. FLEXIBILITY FOR UTILITIES: The gap under the floor provides space to run plumbing, electrical wiring, and heating systems (like underfloor heating), making repairs or upgrades easier without disrupting the main structure.

In essence, a suspended wooden ground floor offers several benefits in construction. The use of timber allows for quick installation and ease of maintenance. These floors are also high customizable in terms of finish and can be adapted to such a wide range of architectural styles. Furthermore, the space underneath the floor can be utilized for utilities such as plumbing and electrical systems, adding further ability to adapt to the design.

1.4. AIMS AND  OBJECTIVES OF THE STUDY

The primary aim of constructing a suspended ground floor is to create a floor structure that is raised above the ground level, allowing for proper ventilation and preventing moisture build-up by providing air circulation underneath, thus minimizing the risk of rot and dampness while also potentially improving thermal insulation within the building; this is achieved by supporting the floor joists on sleeper walls or other support structures, creating a gap between the ground and the floor itself.

OBJECTIVES

i. To outline the fundamental principles involved in designing and constructing a wooden ground floor.

ii. To assess the materials and techniques most suitable for creating a durable and efficient system.

iii. To highlight the advantages and limitations of this approach, focusing on factors such as cost sustainability and performance.

iv. To provide a practical understanding of the construction process, enabling both professionals and beginners to apply this knowledge to their own projects.

v. To provide space for the installation of insulation materials between the floor joists to improve thermal efficiency of the building.

vi. To create a level floor surface even when the underlying ground is uneven.

vii. To protect the main structure by elevating the wooden floor above the ground, reducing direct contact with soil and moisture, which can lead to decay over time.

viii. To keep the building warmer in the winter and cooler in the summer.

ix. To prevent moisture buildup that could lead to issues like dampness or mold growth in the building

CHAPTER TWO

LITERATURE REVIEW

Suspended floors have been widely used in construction for many years. They consist of a platform raised above the ground and supported by timber beams or other materials like steel post, concrete blocks or stone piers. This type of floor system allows for better air circulation beneath the building and is often used in timber-framed homes.

2.1. HISTORICAL CONTENT

The construction of wooden suspended ground floors has a long history, especially in regions with certain climatic conditions or building traditions. Here’s a historical context on their use and evolution:

1. ORIGINS AND EARLY USE:

* Medieval and Early Modern Europe: The concept of a suspended floor system, often referred to as a "raised floor" or "ventilated floor," can be traced back to medieval times. Early buildings, especially those in damp or marshy areas, used timber beams or stone to elevate floors above the ground, providing ventilation and protecting structures from ground moisture.

* Post-Roman Britain and Scandinavia: After the fall of the Roman Empire, in regions like Britain and Scandinavia, wooden suspended floors were common in domestic and agricultural buildings. The raised floor system helped protect buildings from the cold and wet conditions that were prevalent in these northern climates. Timber was abundant and easy to work with, making it a favored material.

2. INDUSTRIAL REVOLUTION (18TH-19TH CENTURY):

* During the Industrial Revolution, the demand for housing and infrastructure increased significantly. Wooden suspended floors became even more common in urban areas, especially in the UK and the US, as they offered a relatively quick and cost-effective method for constructing homes.

* As cities expanded and more buildings were constructed with limited space and dense populations, wooden suspended floors provided better hygiene and comfort, particularly by promoting airflow and reducing the impact of rising damp in poorly drained areas.

3. 19TH CENTURY - THE RISE OF MODERN TIMBER FLOOR:

* By the mid-19th century, suspended timber ground floors were widely used in the construction of both domestic and commercial buildings in many parts of the world. This was partly due to the increased use of mass-produced timber beams and joists, which made it easier and cheaper to create the floor system.

* The system gained more widespread use in countries like Australia, New Zealand, and parts of the United States, especially in areas where wooden construction was the norm. In Australia, for example, the suspended timber floor became a standard feature of homes due to the dry climate and the need to keep the interiors cooler during hot summers.

4. 20TH CENTURY - CHANGES AND CHALLENGES:

* As new building materials like concrete, steel, and later, reinforced concrete, became more widely available, the use of wooden suspended ground floors started to decline. Concrete floors were seen as more durable, easier to maintain, and more fire-resistant.

* However, wooden suspended floors remained in use in many older homes and buildings, especially in rural areas, where they were often seen as more adaptable and easier to repair than concrete floors.

5. MODERN REVIVAL AND SUSTAINABILITY:

* In recent decades, there has been a resurgence in interest in wooden suspended floors, particularly in the context of sustainable building practices. As concerns about energy efficiency and environmental impact have grown, wood, as a renewable material, is being favored for its low carbon footprint compared to concrete and steel.

* Additionally, in the context of heritage preservation, many historic buildings with suspended timber floors are being renovated and restored, maintaining this traditional construction method.

6. CULTURAL SIGNIFICANCE:

* In many parts of the world, wooden suspended ground floors have cultural significance, often seen in vernacular architecture. They symbolize a connection to traditional building methods and local materials, reflecting the natural environment and resources available in a region.

2.2. OVERVIEW OF THE SUSPENDED GROUND FLOOR

A suspended wooden ground floor is a type of floor structure where the wooden floorboards are raised above the ground, supported by a series of beams, joists, and sometimes brick or concrete supports. This type of construction is often used to create a ventilated space beneath the floor, which helps to protect the structure from moisture and pests. It is also used in areas with poor drainage where ventilation and moisture control are important to prevent dampness from affecting the wooden structure. Here's an overview of its construction:

1. FOUNDATION AND SUPPORT STRUCTURE

* Perimeter Foundation: The perimeter of the building may have a concrete foundation or brickwork, which supports the external walls.

Internal Supports: Internal support may consist of concrete or brick piers or timber bearers. These support the wooden beams (joists) that hold up the floor.

* Bearers: Horizontal wooden beams are positioned on the foundations or piers. These are typically spaced 3 to 4 feet apart, depending on the load and design requirements.

2. JOISTS

* Joists are placed parallel to the bearers, running perpendicular to the direction of the floorboards. These provide the primary support for the floor.

* The spacing between joists is typically 16 to 24 inches, depending on the floor design and load-bearing requirements.

3. SUSPENDED FLOORBOARDS

* Floorboards are laid across the joists and nailed or screwed into place. These are usually tongue-and-groove wooden planks to create a stable, durable surface.

* The gap between the floorboards allows air circulation underneath, which helps to prevent moisture buildup that could damage the wood.

4. VENTILATION AND DAMP-PROOFING

* To ensure proper airflow beneath the floor, ventilation is crucial. Ventilation vents may be placed around the perimeter of the building or in the walls.

* Damp-proofing membranes can also be used to prevent moisture from rising into the timber structure and floorboards.

5. FINISHING

* Once the structural elements are in place, the floor is finished with either a polished wood surface, carpet, or other flooring materials depending on the desired aesthetic.

2.3. ADVANTAGES AND DISADVANTAGES OF A SUSPENDED WOODEN FLOOR

ADVANTAGES

1. FLEXIBILITY AND EASE OF INSTALLATION: Wooden floors are easier and quicker to install compared to concrete or stone alternatives. This makes the construction process more efficient.

2. BETTER AIR CIRCULATION: The suspended design allows air to circulate underneath, reducing the chances of mold buildup and improving overall ventilation.

3. SUSTAINABILITY: Wood is a renewable resource, making it an environmentally friendly choice if sourced responsibly.

4. REDUCED RISK OF SUBSIDENCE: In areas with unstable ground, a suspended wooden floor can provide better stability than direct contact with the earth, reducing the risk of cracking or sinking.

5. VENTILATION: The raised design allows for ventilation, which helps prevent mold and dry rot.

6. HOME VALUE: Timber floors can increase the value of a home, making it more attractive to buyers.

7. DURABILITY: Timber floors are strong and durable, and can last a long time.

8. LOW MAINTENANCE: Timber floors are easy to clean and require low maintenance.

9. VERSATILE: Timber floors can fit into both traditional and modern home decor.

These benefits make wooden suspended ground floors a popular choice, especially in regions with damp conditions or where environmental considerations are a priority.

DISADVANTAGES

A suspended wooden ground floor, while providing certain benefits like ventilation and preventing dampness, also has a few disadvantages:

1. SUSCEPTIBILITY TO ROT AND DECAY: The wooden beams and floorboards can be vulnerable to moisture, especially if there is poor ventilation or water ingress. Over time, wood can rot or become infested with pests like termites.

2. LIMITED INSULATION: Suspended wooden floors can lack the insulation properties that solid floors provide, which can lead to cold floors and higher heating costs.

3. UNEVEN SETTLEMENT: Over time, the structure may settle unevenly, which could cause the floor to sag or become uneven. This could also affect the overall stability of the building.

4. PEST INFESTATION: Insects and rodents may find the voids under suspended floors a suitable habitat, leading to potential infestations if not properly sealed or maintained.

5. DIFFICULT TO MAINTAIN: Regular maintenance of a suspended floor can be challenging, especially if access to the underside is restricted or requires lifting the flooring.

6. STRUCTURAL CONCERNS: If the supporting timbers or joists are not properly maintained, they can weaken, leading to potential structural instability over time.

7. DESIGN AND LAYOUT: Solid lumber floor joists may have restrictions on span distances, which can affect the design and layout of a structure.

8. COST: Wood flooring can be more expensive than other materials.

9. WEIGHT CAPACITY: They can't hold as much weight as concrete floors, so they might not be suitable for all types of builds.

Each of these factors should be carefully considered when deciding whether a suspended wooden ground floor is appropriate for a particular building or location.

2.4. CHALLENGES IN SUSPENDED WOODEN FLOOR CONSTRUCTION

The main challenges with a wooden suspended ground floor include: moisture susceptibility leading to rot or warping, potential for drafts due to gaps between floorboards, noise from creaking joists, inadequate insulation, and the need for proper ventilation to manage moisture levels; all of which can significantly impact the comfort and longevity of the floor structure if not addressed properly.

SPECIFIC CHALLENGES:

1. MOISTURE ISSUES:

Rot and decay: Wood is prone to rotting when exposed to prolonged moisture from ground contact or leaks, especially in poorly ventilated areas.

Warping: Changes in humidity can cause wood floorboards to warp or cup, leading to uneven surfaces.

2. THERMAL PERFORMANCE:

Heat loss: Without proper insulation, suspended wooden floors can significantly contribute to heat loss from a building due to air circulation under the floor.

Drafts: Gaps between floorboards can create drafts, making the room uncomfortable.

3. NOISE CONCERNS:

Creaking: Over time, wooden joists can loosen and creak when walked on, especially if not properly secured.

4. STRUCTURAL CONSIDERATIONS:

Span limitations: Depending on the wood type and size, joists may not be able to span large distances without additional support, impacting room layout.

5. INSTALLATION COMPLEXITY:

Precise leveling: Ensuring a level and stable floor requires careful planning and installation techniques to avoid unevenness.

MITIGATING THESE CHALLENGES:

* PROPER VENTILATION: Ensure sufficient airflow under the floor to prevent moisture buildup.

* MOISTURE BARRIER: Use a damp proof membrane to protect the wood from ground moisture.

* INSULATION: Install appropriate insulation between the joists to improve thermal efficiency and reduce drafts.

* QUALITY MATERIALS: Choose treated wood for increased moisture resistance and select appropriate joist sizes for the span required.

* SECURE FASTENINGS: Properly secure joists and floorboards with nails or screws to minimize creaking.

* PROFESSIONAL INSTALLATION: Consult with a qualified builder to ensure proper design and installation techniques are used.

2.5. COMPARISON BETWEEN TIMBER AND CONCRETE SUSPENDED FLOORS

When considering suspended floors for construction, both timber and concrete offer distinct advantages and challenges. Here’s a comparison between the two, focusing on factors like cost, durability, installation time, and maintenance:

1. Material Properties

Timber Suspended Floor:

Composition: Uses timber beams or joists with timber or plywood flooring boards.

Weight: Lightweight compared to concrete, making it easier to handle and install.

Thermal Insulation: Timber has natural insulating properties, providing better warmth retention.

Sound Insulation: Timber can provide good acoustic insulation but may not block sound as well as concrete.

Concrete Suspended Floor:

Composition: Made of reinforced concrete beams and concrete slabs.

Weight: Much heavier than timber, which may require additional structural support.

Thermal Insulation: Concrete has poor natural thermal insulation and may require additional insulation material.

Sound Insulation: Concrete is denser and offers better soundproofing between floors.

2. Cost

Timber Suspended Floor:

Material Cost: Generally less expensive for both materials and labor.

Labor: Easier and faster to install, reducing construction time and labor costs.

Long-Term Costs: Timber floors may require more maintenance over time, especially for pest control and moisture management.

Concrete Suspended Floor:

Material Cost: Concrete and reinforcement materials are generally more expensive.

Labor: Installation is more labor-intensive and requires specialized labor, making it more costly in terms of both time and money.

Long-Term Costs: Concrete requires less maintenance than timber but may be prone to cracking or settling if not properly installed.

3. Durability and Maintenance

Timber Suspended Floor:

Durability: Timber is susceptible to rot, moisture damage, and pest infestation (termites, etc.). It requires regular treatment and inspections to ensure its longevity.

Maintenance: Requires periodic treatment for moisture protection, pest control, and checking for structural issues like warping or sagging.

Concrete Suspended Floor:

Durability: Concrete is generally more durable and resistant to fire, decay, and pests. However, it can crack over time due to settling or foundation movement.

Maintenance: Requires minimal maintenance, but cracks or damage may need to be addressed to prevent further deterioration.

4. Installation Time

Timber Suspended Floor:

Time to Install: Timber floors are quicker to install, typically taking a few days to a week depending on the size of the project

Weather Dependency: Timber can be installed in various weather conditions, although moisture during installation can be problematic.

Concrete Suspended Floor:

Time to Install: Concrete floors take longer to install, as they often require forming, reinforcement, and curing time. It can take weeks depending on the complexity and curing time required.

Weather Dependency: Concrete installation is highly weather-dependent, as it may need to be protected from rain during curing.

5. Structural Performance

Timber Suspended Floor:

Load-Bearing Capacity: Timber is lighter but may require more frequent support structures to carry loads. It is suitable for residential buildings but may have limits in larger commercial structures.

Flexibility: Timber can be more flexible, but this can lead to some movement or sagging over time, especially under heavy load.

Concrete Suspended Floor:

Load-Bearing Capacity: Concrete has a high load-bearing capacity and is better suited for larger buildings or structures with significant weight requirements.

Flexibility: Concrete is rigid and does not flex, which is advantageous for larger spans and in areas that require heavy load-bearing capacity.

6. Environmental Impact

Timber Suspended Floor:

Sustainability: Timber is renewable and can have a smaller carbon footprint if sourced sustainably. However, deforestation and the use of chemical treatments for pest and moisture resistance can impact its sustainability.

Carbon Sequestration: Timber absorbs and stores carbon, making it more environmentally friendly in terms of carbon footprint.

Concrete Suspended Floor:

Sustainability: Concrete production is energy-intensive and has a high carbon footprint due to the materials used (e.g., cement).

Recyclability: Concrete can be crushed and reused, but the environmental impact of production remains high.

7. Aesthetics and Design Flexibility.

Timber Suspended Floor:

Aesthetic Appeal: Timber floors offer a natural, warm appearance. They can be finished and treated to match different interior design styles.

Design Flexibility: Easier to modify, cut, or replace specific sections if needed.

Concrete Suspended Floor:

Aesthetic Appeal: Concrete tends to be more industrial or utilitarian in appearance unless it is finished or covered with flooring materials.

Design Flexibility: Concrete is harder to modify once installed, and any changes would require significant effort or demolition.

CHAPTER THREE

METHODOLOGY

The construction of a wooden suspended ground floor involves several key steps and methodologies to ensure stability, durability, and safety.

Constructing a wooden suspended ground floor requires careful planning, quality materials, and adherence to safety standards. By following these steps, creation of a sturdy and functional floor that meets needs will be executed. This project will follow a system approach to evaluate the design and construction of a suspended wooden ground floor. The methodology involves multiples stages from design through to execution, with an emphasis practical application and material selection.

3.1. STRUCTURAL REQUIREMENTS

The structural requirements for a wooden suspended ground floor focus on safety, load-bearing capacity, moisture protection, and ventilation. Here's a breakdown of the key elements:

1. Load-Bearing Capacity

Joist sizing must match the span and expected live/dead loads (e.g. furniture, people, finishes).

Follow building codes or standards like Eurocode 5 (EN 1995) or British Standards (BS 5268) for timber.

Joist spacing typically 400mm or 600mm centers, depending on board type and thickness.

2. Timber Specification

Use pressure-treated, kiln-dried timber to resist decay.

Common grades: C16 or C24 structural timber.

Ensure joists are free from warping, splits, or defects.

3. Ventilation

Cross-ventilation below the floor is crucial to avoid rot and damp.

Provide air bricks or ventilation grilles at opposing walls, minimum 1500mm² per meter of wall.

Maintain 150mm clear air gap between ground and underside of joists.

4. DPC and Damp Protection

Damp-proof course (DPC) under walls at least 150mm above ground level.

Damp-proof membrane (DPM) if needed for additional protection in areas with high moisture risk.

5. Support and Bridging

Sleeper/dwarf walls or piers must support long-span joists.

Use herringbone strutting or solid blocking between joists to prevent twisting and increase stability.

6. Floor Decking

Structural floorboards (e.g. 18–22mm T&G chipboard, plywood).

Boards should run perpendicular to joists, joints staggered.

Fix securely with adhesive and nails or screws.

7. Fire and Insulation Requirements

May need fire-resistant board or insulation between joists (depending on building regulations).

Thermal insulation (rigid board or mineral wool) between joists for energy efficiency.

3.2. LOAD ANALYSIS AND AND CALCULATIONS

To perform a load analysis for a wooden suspended ground floor, we must consider both the dead load and live load on the floor structure. The design and calculations for a wooden suspended ground floor will focus on the floor's ability to support the imposed loads while staying within acceptable limits for bending, shear, and deflection.

Here's a step-by-step outline and explanation of the load analysis for a suspended wooden ground floor.

1. Defining Parameters

We need to define a few key parameters:

Span of floor joists (L): The distance between the supports (typically walls or beams).

Spacing of floor joists (s): The distance between adjacent joists (commonly 400 mm or 600 mm apart).

Timber properties: This includes the type of timber, grade, and size of the joists.

Dead Load (DL): The permanent load from the floor construction itself, including the weight of floor finishes, ceiling, insulation, etc.

Live Load (LL): The temporary load from people, furniture, and activities (based on occupancy type).

Deflection limits: These are usually given as a ratio of the span (e.g., span/360 for domestic use) and help ensure the floor doesn't deflect too much under load.

2. Loads on the Floor

Dead Load (DL)

This includes the weight of the materials used to construct the floor, such as:

Floor finish (e.g., timber or concrete overlay)

Insulation

Ceiling materials

Typical values for dead load:

Timber floor finish: ~0.5–1.0 kN/m²

Ceiling finish: ~0.2–0.5 kN/m²

Insulation: ~0.1–0.2 kN/m²

For simplicity, let's assume

Dead load (DL) = 1.0 kN/m² for the floor.

Live Load (LL)

The live load will depend on the use of the building. For residential buildings, the typical live load is:

Domestic use: 1.5 kN/m² (per BS EN 1991-1-1)

Total Load on Floor

The total load (w) per unit area will be the sum of the dead load (DL) and the live load (LL).

This is the uniform load applied to the floor structure.

3. Load Distribution on Joists

If the joists are spaced at 600 mm centers (0.6 m), the load on each joist is calculated by multiplying the total load by the spacing (in meters)

Each joist will carry a uniform load of 1.5 kN per meter of its length.

4. Bending Moment and Shear Calculation

For a simply supported beam (like a floor joist), the maximum bending moment and shear can be calculated using the following formulas:

Maximum Shear (V)

The maximum shear occurs at the supports and is given by

V= wl/2

W = total load per meter of joist (1.5 kN/m)

L = span of the joist (let's assume 4 m)

For a span of 4 m:

V = 1.5×4/2 = 3KN

Maximum Bending Moment (M)

The maximum bending moment occurs at the midpoint of the joist and is calculated as:

M = WL square devided by 8

For a span of 4 m:

M = 1.5×4 square divided by 8 = 3KN

5. Deflection Calculation

The deflection of the joist under the applied load can be calculated using the following formula:

\Delta = {5wL^4} divided by {384EI}

Where:

W = load per meter of joist (1.5 kN/m)

L = span of the joist (4 m)

E = modulus of elasticity of the timber (typically around 10,000 MPa for softwoods)

I = second moment of area (calculated for the joist's cross-section)

6. Timber Selection & Safety Factors

Based on the maximum bending moment (3 kNm) and shear (3 kN), you will select a timber joist size that can safely carry these loads. Ensure that the bending stress and shear stress in the timber do not exceed the allowable values for the selected timber grade.

Timber Design Check

Bending Stress (): Ensure the maximum bending stress in the timber does not exceed the allowable bending stress for the chosen timber species and grade.

Shear Stress (): Similarly, the shear stress in the timber must be below the allowable shear stress.

7. Final Checks

Deflection: The deflection of the joist should not exceed the allowable limit (typically span/360).

Strength: The bending and shear stresses should not exceed the allowable stresses for the selected timber.

3.3. MOISTURE AND PEST PROTECTION

When constructing a wooden suspended ground floor, ensuring moisture and pest protection is crucial for the longevity and stability of the structure. Here are key methods to achieve this:

1. MOISTURE PROTECTION:

Damp-Proof Membrane (DPM): A high-quality damp-proof membrane should be placed under the timber flooring to prevent moisture from rising through the ground. Typically, polyethylene or bitumen-based membranes are used. Ensure that the DPM covers the entire floor area and overlaps at the edges to seal out moisture effectively.

Ventilation: Proper ventilation beneath the suspended floor is crucial. Use ventilation grilles or air bricks at regular intervals to allow airflow and prevent moisture accumulation. This is particularly important in the void space beneath the floor.

Use of Treated Timber: Timber used for the floor joists and bearers should be pressure-treated for moisture resistance. This helps prevent rot or decay that could arise from exposure to moisture.

Sub-floor Drainage: Ensure that the ground beneath the floor is well-drained. A gravel layer or drainage channels can be laid to allow any water to drain away, further preventing moisture buildup.

Flooring Installation: Consider using moisture-resistant flooring materials such as marine plywood or treated timber for the suspended floor itself.

2. PEST PROTECTION:

Termite-Resistant Timber: Use timber that is naturally resistant to pests such as termites. Pressure-treated timber or timber species with natural insect-repelling properties (e.g., cedar) can help reduce the risk of pest infestation.

Pest Barriers: Install physical pest barriers, such as metal mesh or specialized pest control membranes, around the perimeter of the suspended floor to prevent rodents and insects from entering the void space.

Chemical Pest Treatments: For additional protection, consider applying a pesticide or insect repellent treatment to the timber before installation. This can help deter termites, ants, and other pests.

Sealing Gaps: Carefully seal all gaps around the edges of the floor, especially where the joists meet the foundation or walls. This helps prevent entry points for pests like rodents.

Regular Inspections: It's essential to have regular inspections of the space beneath the suspended floor for signs of pest activity or moisture damage. Early detection can help prevent major issues.

By implementing these methods, you can effectively protect a wooden suspended ground floor from moisture and pest-related damage, ensuring its durability and maintaining a healthy living environment.

3.4. DESIGN PHASE

Designing a wooden suspended ground floor is a crucial phase that involves careful consideration of various factors to ensure structural integrity, functionality, and compliance with building codes. The design are based on factors such as location, type of building, load requirements, and environmental considerations. The floor system will be designed to ensure adequate support while allowing sufficient air flow beneath the structure. Here’s a detailed

breakdown of the design phase:

1. ARCHITECTURAL DESIGN

* Create architectural drawings that outline the layout of the floor, including dimensions, room configurations, and any openings (like staircases or vents).

* Consider the aesthetics of the floor, including the type of finish and how it will integrate with the overall design of the building.

2. MATERIAL SPECIFICATIONS

* Select suitable materials for the frame and flooring. For the frame, choose treated timber that meets local building codes. For the flooring, decide on the type of finish (e.g., hardwood, laminate).

* Specify the size and grade of the timber to be used for bearers, joists, and any other structural components.

3. JOIST SPACING AND SIZE

* Determine the spacing and size of the joists based on the load calculations. Common joist spacing is 16 to 24 inches, but this may vary depending on the design load and the type of material used.

* Choose the appropriate depth and width for the joists to ensure they can carry the load without excessive deflection.

4. CONCEPTUAL DESIGN

* Floor Layout: Define the dimensions of the suspended ground floor, considering spaces for living, storage, and utilities. The layout should be suitable for the building's overall design.

* Clearance: Determine the necessary height between the ground and the underside of the

suspended floor for ventilation and accessibility (especially for plumbing and wiring).

* Structural Considerations: Decide on the type of wood and structural elements (joists, beams) that will be used for the suspended floor. Commonly used timber includes treated wood for durability.

5. STRUCTURAL DESIGN

* Suspension Method: A suspended ground floor typically involves joists and beams suspended above the ground, often supported by concrete or masonry pillars or piers. Determine how the floor will be supported and ensure adequate spacing between supports.

* Load-Bearing Calculations: Ensure that the timber can bear the expected loads, both live (people, furniture) and dead (the weight of the structure itself).

* Ventilation: Design for proper airflow underneath the suspended floor to prevent moisture buildup, which could lead to wood rot. Vents or crawl space access points might be necessary.

6. CONNECTION DETAILS

* Design the connections between the bearers, joists, and flooring. Use appropriate fasteners and brackets to ensure strong connections that can handle the loads.

* Include details on how the floor will be anchored to the foundation or footings to prevent movement.

7. COMPLIANCE WITH BUILDING CODES

* Ensure that the design complies with local building codes and regulations, including fire safety, structural integrity, and accessibility requirements.

* Consult with a structural engineer if necessary to verify that the design meets the required

standards.

8. 3D MODELING AND VISUALIZATION

* Consider using 3D modeling software to create a visual representation of the suspended floor. This can help in identifying any potential issues in the design and provide a clearer understanding of the final product.

9. DOCUMENTATION

* Prepare detailed construction documents that include drawings, specifications, and notes for builders. This documentation will serve as a guide during the construction phase.

3.5. MATERIAL SELECTION

When selecting materials for a wooden suspended ground floor, several factors including durability, strength, moisture resistance, thermal insulation, and cost will be considered. Here's a list of material options commonly used for this kind of construction:

1. TIMBER FOR JOISTS AND BEAMS:

* Softwood (e.g., Pine, Spruce, or Douglas Fir): Softwoods are commonly used for joists, beams,

and the floor frame due to their good strength-to-weight ratio and cost-effectiveness. However, they may require treatment for increased durability and resistance to pests.

* Hardwood (e.g., Oak, Beech, or Ash): Hardwood can be more expensive but offers greater durability and strength, making it suitable for heavily loaded areas.

2. FLOORBOARDS:

* Solid Timber Floorboards: Solid timber is aesthetically pleasing, durable, and can be sanded or refinished over time. It is suitable for a suspended floor, though it may need additional treatment for moisture control.

* Engineered Wood: This is an excellent alternative, consisting of layers of wood bonded together, providing enhanced stability and resistance to moisture fluctuations.

3. INSULATION:

* Mineral Wool (Rock Wool or Glass Wool): It provides excellent thermal and sound insulation, is fire-resistant, and is commonly used in suspended floors.

*  Polystyrene Boards: These materials are good for thermal insulation but may not provide the same soundproofing as mineral wool.

* Cellulose Insulation: An eco-friendly option, made from recycled paper, providing good thermal and sound insulation.

4. VAPOR BARRIER:

* Plastic Sheeting (Polyethylene): A vapor barrier is essential in preventing moisture from rising through the floor and damaging the timber. Polyethylene is commonly used for this purpose.

5. SCREED OR UNDERFLOOR MATERIALS:

* Cement or Lime Screed: If you're using a solid floor, a screed layer can help provide a stable surface for finishing materials. Lime screeds can be more flexible and breathable compared to cement.

* Boarding (e.g., Plywood, OSB): For additional structural support, plywood or Oriented Strand Board (OSB) can be used as underflooring before applying the final floor surface.

6. FOUNDATION AND SUPPORT:

*"Concrete Piers or Brick: For the foundations, piers made from concrete or brick are often used to support the suspended floor and ensure stability.

ADDITIONAL CONSIDERATIONS:

* Ensure all timber is treated to resist rot, pests, and moisture, especially in areas with high humidity.

*'Consider local building codes, as some areas may have specific requirements regarding suspended floors, insulation, and moisture control.

The choice of materials will depend on your climate, budget, and the specific load-bearing requirements for your project.

CHAPTER FOUR

CONSTRUCTION PROCESS

4.1. SITE PREPARATION

Site preparation for a wooden suspended ground floor primarily involves clearing the area, excavating to the required depth, leveling the ground, installing a damp-proof membrane, and potentially laying a compacted hardcore base to ensure a stable foundation for the supporting piers or padstones that will hold the wooden joists of the suspended floor.

Key Steps In Site Preparation:

1.CLEAR THE SITE:

* Remove any existing vegetation, debris, or obstructions from the area where the suspended floor will be built.

2. EXCAVATE TO DEPTH:

* Dig down to the desired level for the floor, ensuring a consistent depth across the entire area.

3. LEVEL THE GROUND:

* Use a rake or other leveling tools to create a flat and even surface for the base of the suspended floor.

4. INSTALL A DAMP-PROOF MEMBRANE:

* Lay a damp-proof course (DPC) membrane across the excavated area to prevent moisture from rising from the ground into the wooden structure.

5. COMPACTED HARDCORE BASE (OPTIONAL):

* Depending on soil conditions, you may need to lay a layer of compacted hardcore material to provide additional stability for the floor supports.

Important Consideration

* Drainage: Ensure proper drainage around the building perimeter to prevent water from

accumulating under the suspended floor.

* Soil conditions: Assess the soil type on the site to determine if additional measures like ground stabilization are needed.

* Building regulations: Check local building codes for specific requirements regarding suspended floors, including ventilation and damp proofing.

4.2. LAYING FOUNDATION AND SUPPORTING WALLS

Laying the foundation and supporting walls for a wooden suspended ground floor involves several key steps to ensure stability, ventilation, and protection from moisture. Here’s a general outline of the process:

1. Site Preparation

Clear the area of vegetation, topsoil, and debris.

Excavate trenches to the required depth for footings.

2. Foundation Construction

Trench fill or strip foundations are typically used, depending on soil conditions.

Pour concrete footings (usually 600mm wide x 225mm deep for standard walls).

Let concrete cure for at least 24–48 hours.

3. Build the Substructure Walls

Use concrete blocks or engineering bricks to build supporting walls (also called sleeper or dwarf walls).

These walls are spaced to support floor joists and often include air bricks or vents to allow cross-ventilation and prevent damp.

Damp-proof course (DPC) should be laid above ground level in all supporting walls.

4. Install Ground Beams or Wall Plates

Fix timber wall plates (treated timber) on top of supporting walls using bolts or straps.

These plates serve as anchors for the floor joists.

5. Prepare for Floor Joists

Supporting walls may also include piers or pad stones for intermediate support depending on span. Ensure spacing aligns with floor joist layout.

4.3. INSTALLATION OF FLOOR JOISTS

Installing floor joists for a wooden suspended ground floor requires careful planning and proper execution to ensure structural stability and longevity. Here’s a step-by-step guide on how to install floor joists for a wooden suspended ground floor:

1. Planning and Preparation

Check the Foundation: Ensure the foundation is solid, level, and free of debris. The suspended floor is supported by the foundation walls or piers, so the foundation must be sound.

Joist Spacing: Typically, floor joists are spaced 400mm (16 inches) to 600mm (24 inches) apart, depending on the load and local building codes. For standard residential construction, 600mm is common. Ensure that you know the correct spacing required for your project.

Joist Material: Select appropriately treated timber for the joists (e.g., pressure-treated softwood like spruce or pine) to ensure it’s resistant to moisture and pests.

Joist Size: Joists should be sized according to the span they need to cover, as well as the load they will bear. Common dimensions for floor joists are 47mm x 225mm or 47mm x 150mm. The span calculator or building code will help you choose the correct size.

2. Marking and Positioning the Bearing Walls

Mark the Wall or Piers: Measure and mark the exact location where the joists will sit on the foundation walls or piers. Ensure that the marks are level and spaced according to the planned joist spacing.

Install Joist Hangers (if needed): If you are not resting the joists directly on the wall but using joist hangers, install these hangers onto the foundation wall or wooden beams according to manufacturer instructions. Joist hangers help secure the joists in place and prevent movement.

3. Installing the End Joists (Bearer Joists)

Place the End Joists: Begin by placing the first and last joists, known as the bearing joists or end joists, along the outer edges of the floor area. These will rest on the foundation walls or piers.

Level the Joists: Use a spirit level to make sure these joists are level. If necessary, adjust the height by packing under the joist with timber or shims.

Secure the Joists: Fix the joists to the walls or piers using appropriate fixings, such as screws or bolts. If using joist hangers, the joist should slot into the hanger and be secured using nails or screws.

4. Installing Intermediate Joists

Measure and Cut Intermediate Joists: Measure the distance between the bearing joists and cut the intermediate joists to the correct length. These joists will sit perpendicular to the bearing joists and provide the structure for the floor.

Place the Intermediate Joists: Start placing the intermediate joists at the marked spacing. Ensure that each joist is level with the bearing joists.

Fixing the Joists: If using joist hangers, secure each joist into the hangers. If resting directly on the foundation or supporting beams, fix the joists with screws or nails.

Check Spacing: As you place each joist, double-check the spacing between them to ensure that they are consistent and in line with your planned measurements.

Temporary Bracing: If needed, temporarily brace the joists in place while securing them to ensure they stay aligned.

5. Installing Cross Bracing or Blocking

Cross Bracing/Blocking: Cross bracing or blocking is often placed between the joists to add extra support and prevent twisting. These are short pieces of timber installed perpendicular to the joists at regular intervals.

Install Blocking: Install the blocking between every second or third joist, depending on the load requirements. Blocking should be securely nailed or screwed into place.

6. Checking and Adjusting Alignments

Check All Joists for Level: As you progress with the installation, use a spirit level to make sure all joists are aligned and level. This is crucial for the overall structural integrity and for the even distribution of loads.

Adjustments: If any joist is out of alignment or level, adjust by either shimming or repositioning the joist.

7. Installing Floor Decking

Lay the Floorboards or Decking: Once the joists are fully installed and secured, the next step is to install the floor decking (e.g., plywood or oriented strand board, OSB). This will be placed across the joists to create the surface for the floor.

Fixing the Decking: The decking is nailed or screwed into place along the edges of the joists. Be sure to leave a small gap between each sheet for expansion due to moisture changes.

8. Final Checks and Adjustments

Inspection: Before moving on to the next stage, carefully inspect the entire joist structure. Make sure all joists are properly aligned, level, and securely fastened.

Strength Test: Walk along the joists to ensure they are firmly in place and do not shift or sag under weight. Check that the floor is level and has no noticeable dips.

4.4. FIXING FLOORBOARDS AND FINISHES

Once the floor joists for a wooden suspended ground floor are installed, the next step is to fix the floorboards and apply any finishing touches. Here’s a step-by-step guide for fixing the floorboards and completing the finishes:

1. Preparing the Floorboards

Material Selection: Common materials for floorboards include plywood, chipboard, or solid timber boards. Ensure that the floorboards are made from a durable, moisture-resistant material, especially if the floor is in a damp environment.

Acclimatization: If you're using solid timber boards, it's important to allow them to acclimatize to the room’s temperature and humidity for a few days before installation. This prevents warping or expansion after installation.

Cutting the Boards: Cut the floorboards to size using a circular saw or handsaw. Leave a small expansion gap (about 5-10mm) between the edge of the floorboard and the walls to allow for expansion and contraction of the wood due to changes in temperature and humidity.

2. Laying the Floorboards

Start at One Edge: Begin laying the first row of floorboards along one side of the room, against the perimeter walls. Use a chalk line to ensure the first row is straight and aligned.

Place the First Board: Lay the first board with its edge flush against the wall. Ensure the tongue or groove (depending on the type of flooring) is oriented properly for ease of installation

3. Fixing the Floorboards to the Joists

Screwing or Nailing: The most common methods for securing floorboards to joists are using screws or nails. The choice depends on the type of floorboard:

For Plywood or Chipboard: Use screws or ring-shank nails to secure the boards to the joists.

Screws are preferred for a more secure and lasting hold.

For Timber Boards: If using solid timber boards, nails are commonly used, though screws can also be applied for better hold. Finish nails are used to ensure that the nails sit flush with the surface of the boards.

Fixing Method:

For tongue-and-groove boards, the nails or screws should go into the grooves where they will be hidden once the next board is installed.

For butt-jointed floorboards, nails or screws are placed at regular intervals along the length of the board, making sure to hit the joists beneath.

Spacing the Fixings: Fasten the floorboards every 300mm to 400mm along each joist, making sure to place the nails/screws along the edges of the boards or in the tongue groove (if applicable). This will prevent the boards from shifting and ensure a solid, secure floor.

4. Installing the Perimeter (Skirting or Architrave)

Skirting Boards: To cover the expansion gap around the perimeter of the floor, install skirting boards or baseboards along the walls. These can be nailed or screwed into place, covering the gap and giving a finished look to the floor.

Fixing Skirting: Attach the skirting boards to the wall using finishing nails or screws. Ensure they’re flush with the floor and aligned with the walls.

5. Sanding and Finishing the Floorboards

Sanding: If you are installing timber boards or plywood with a natural finish, sanding is important to smooth out any rough edges or uneven areas. Use a fine-grit sandpaper or an orbital sander to smooth the surface of the floorboards.

Filling Gaps: After sanding, inspect the floor for any gaps or nail holes. Use a wood filler to fill any imperfections. Allow the filler to dry and sand it down to ensure a smooth surface

6. Applying a Finish

Varnish, Lacquer, or Oil: Choose the appropriate finish for your floorboards. A polyurethane varnish will provide a durable, protective coating, while oils such as Danish oil or tung oil will enhance the wood’s natural look and provide some protection.

Varnish or Lacquer: Apply a thin, even coat using a brush or roller, and allow it to dry completely before applying a second coat. Sand lightly between coats with fine-grit sandpaper to remove any imperfections.

Oil: Apply oil with a cloth, working it into the wood’s surface. Wipe off any excess oil and let it cure.

Staining (Optional): If you wish to change the color of the wood, you can apply a wood stain before finishing with varnish or oil. Be sure to test the stain on a small area first.

7. Final Inspection

Inspect the Floor: Once the finish is dry, inspect the entire floor for any imperfections such as uneven boards, gaps, or missed spots.

Final Touches: If necessary, apply touch-ups to any areas that may need additional staining, sanding, or finishing.

8. Cleaning the Floor

Cleaning: After installation and finishing, clean the floor by sweeping or vacuuming to remove dust and debris. If needed, mop with a damp cloth, being careful not to get the floor too wet.

Maintaining the Finish: Follow the manufacturer’s recommendations for maintaining the finish, whether it’s varnish, oil, or lacquer, to ensure the floor remains in good condition over time.

4.5.  SAFETY MEASURES

When constructing a wooden suspended ground floor, key safety measures include: proper scaffolding and support systems for workers, secure anchoring of joists and beams, adequate ventilation to prevent moisture build-up, protection against potential falls from height, use of appropriate personal protective equipment (PPE), and regular inspections of the structure throughout the construction process; ensuring all workers are trained in safe practices for working at heights and with power tools is also crucial.

Specific Safety Measures To Consider:

1. STRUCTURAL STABILITY:

Proper support systems: Utilize adequate scaffolding and shoring to support the structure while work is in progress, especially when installing large joists or beams.

Secure anchoring: Ensure all joists and beams are securely anchored to the perimeter walls and intermediate supports to prevent movement or collapse.

Load calculations: Always design the floor system based on calculated load capacities to avoid overloading.

2. FALL PREVENTION:

Guardrails and safety nets: Install perimeter guardrails and safety nets on elevated platforms where necessary.

Harness systems: Require workers to wear safety harnesses when working at heights where falls could occur.

Access points: Provide safe and stable access points to the working area.

Personal protective equipment (PPE):

Hard hats: Mandatory for all workers on site.

Safety gloves: To protect hands from cuts and splinters.

Safety goggles: To protect eyes from debris.

Hearing protection: Where necessary, due to loud machinery.

3. WORK PRACTICES:

Training: Ensure all workers are properly trained in safe working practices, including the use of power tools and working at heights.

Clear communication: Maintain clear communication between workers and site supervisors to identify potential hazards and coordinate activities.

Regular inspections: Conduct regular inspections of the construction site to identify any safety concerns and address them promptly.

Specific Considerations For Wooden Suspended Floors

Joist spacing: Maintain proper spacing between joists to ensure adequate load distribution.

Treatment of timber: Use pressure-treated timber where necessary to resist moisture damage and insect infestation.

Fire safety: Consider fire-retardant treatments for timber components where required.

4.6. TESTING AND EVALUATION

Testing and evaluation of a wooden suspended ground floor would involve assessing its structural integrity, deflection under load, thermal performance, sound insulation, moisture content, and overall durability by conducting a series of tests including: load tests, deflection measurements, thermal imaging, sound transmission tests, moisture content checks, visual inspections, and potentially destructive testing of individual components to ensure the floor meets design specifications and performs as intended in terms of strength, stability, energy efficiency, and acoustic properties.

Key Aspects To Evaluate

1. STRUCTURAL INTEGRITY:

Static load tests: Applying calculated dead and live loads to the floor to check for excessive deflection and potential failure points.

Point load tests: Applying concentrated loads at specific locations to assess the localized strength of the joists and supports.

Visual inspection: Checking for cracks, splits, or other damage to the timber members.

2. DEFLECTION PERFORMANCE:

Deflection Measurements: Using precise instruments to measure the amount of deflection under load at different points on the floor to ensure it falls within acceptable limits.

3. THERMAL PERFORMANCE:

Thermal Imaging: Using infrared cameras to identify areas of heat loss through the floor, particularly around the perimeter and joist spaces.

Heat flux measurements: Directly measuring the heat transfer through the floor to calculate its U-value.

4. ACOUSTIC PERFORMANCE:

Sound transmission tests: Measuring the sound transmission loss between rooms to assess the floor's ability to dampen noise.

5. MOISTURE CONTENT:

Moisture meter readings: Checking the moisture content of the timber members to ensure they are within acceptable levels to prevent warping and decay.

6. DURABILITY AND LONGEVITY:

Visual inspection of treated timber: Assessing the quality and effectiveness of any preservative treatments applied to the wood.

Destructive testing (if necessary): Taking samples of the wood to analyze its strength and resistance to decay in a laboratory setting.

Factors To Consider During Testing And Evaluation

* Design specifications: Comparing the measured performance of the floor to the original design calculations.

* Building codes and regulations: Ensuring the floor construction complies with relevant building standards for load capacity, deflection, and fire safety.

* Environmental conditions: Taking into account the local climate and potential exposure to moisture when assessing the durability of the wooden floor.

CHAPTER FIVE

5.1. CONCLUSION

In conclusion, the construction of a wooden suspended ground floor is an effective and practical solution for many residential buildings, especially where moisture control, ease of installation, and thermal insulation are priorities. The process involves careful planning and execution, from ensuring a solid foundation to the precise installation of joists, floorboards, and finishes.

Overall, a well-constructed wooden suspended ground floor offers many advantages, including better thermal performance, ease of access for maintenance, and reduced construction costs compared to alternatives like concrete floors. With careful attention to detail and proper materials, a wooden suspended floor can be a durable, cost-effective solution for a wide range of building projects.

5.2. RECOMMENDATION

For a wooden suspended ground floor, the primary recommendation is to prioritize effective ventilation and moisture control to prevent rot and damage to the joists and floorboards. This involves ensuring adequate ventilation under the floor, preventing moisture buildup, and considering insulation to improve thermal efficiency and reduce drafts.

Detailed Recommendations:

1. VENTILATION: Ensure adequate ventilation under the floor, especially if there's a crawl space. Air vents should not be blocked, and sufficient airflow should be maintained to remove moisture.

2. MOISTURE CONTROL: A waterproof membrane (like a roofing membrane or Tyvek) can be used underneath the insulation to help prevent moisture from reaching the joists and floorboards. Consider using moisture-resistant or marine plywood for the subfloor.

3. AIRTIGHTNESS: Sealing gaps and joints with a vapor-open membrane (like a Tyvek membrane) can help reduce air flow and minimize condensation, according to DuPont. Draught-proofing can also be achieved by applying a weak sand and cement parging coat to the masonry between joist ends and the gap between the wall plaster and floor.

4. MAINTENANCE: Regular inspection of the underfloor void and ventilation system is important to identify and address any potential issues early on.

5. ACCESS: Consider providing access covers to the underfloor void for easy inspection and maintenance.

6. MONITOR MOISTURE LEVELS: Using moisture sensors can help detect and address moisture buildup in the underfloor void.

7. CONSIDER ADDING BLOCKING BETWEEN JOISTS: This can help reduce bouncing or squeaking in the floor.

By implementing these recommendations, you can help ensure the long-term durability and comfort of your wooden suspended ground floor.

5.3. REFERENCE

* Book of woodworking by Tom carpenter(1998): challenges in suspended wooden floor construction.

* Essential guide to timber framing by Steve chappell volume 1 (1911): Advantages and Disadvantages of a wooden suspended ground floor & the importance of a suspended wooden ground floor.

* The building of wooden floors by Frank G.J Smith (2000): Historical content & background of the construction of a wooden suspended ground floor

* Designing Floors by British Woodworking Federation (BWF) 1945: Design phase of a wooden suspended ground floor.

* The foundation of modern timber construction by Peter L. Walker volume 1 & 2 (2012): construction process and material selection of a wooden suspended ground floor.

* National Association of Home builders (NAH) 1942: Safety measures in constructing a wooden suspended ground floor.

* The timber framers guilds (1985):  Aims and objective of the construction of a wooden suspended ground floor