KWARA STATE POLYTECHNIC, ILORIN
INSTITUTE OF ENVIRONMENTAL STUDIES
DEPARTMENT OF SURVEYING AND GEO-INFORMATICS

A PROJECT REPORT ON :

UTILITY SURVEY

OF

ROYAL VALLEY ESTATE

AT KUNLENDE/AKEREBIATA ROAD, ILORIN ILORIN EAST LOCAL GOVERNMENT AREA KWARA STATE

SUBMITTED BY:  RAHMAN ZAINAB OLAMIDE               MATRICULATION: ND/23/SGI/FT/064

SUPERVISOR IN CHARGE: SURV. DIRAN FELIX & SURV. ADEBANJI

INSTITUTION: INSTITUTE OF ENVIRONMENTAL STUDIES

DATE: JUNE, 2025

CERTIFICATE

I, RAHMAN ZAINAB OLAMIDE with Matric Number ND/23/SGI/FT/064 hereby certify that the information contained in this project report were obtained as a result of observations and movements taken by me and the utility Survey  was done in accordance to Surveying rules and regulations and Departmental instructions.

Signature of student:  			___________________________

Name of student:				___________________________

Date of completion: 			___________________________

Matric Number: 				ND/23/SGI/FT/064

CERTIFICATION

This is to certify that RAHMAN  ZAINAB OLAMIDE with Matric number ND/23/SGI/FT/064 carried out this project work and has been approved as meeting the requirement for the award of National Diploma (ND) in Surveying and Geo-informatics in the Department of Surveying and Geo-informatics of the Institute of Environmental Studies, Kwara state polytechnic, Ilorin.

Project supervisor							     _____________

SURV. ADEBANJI &SURV. DIRAN FELIX	 			                      Signature and Date

Project Cordinator 							       _____________

SURV. AWOLEYE RAPHAEL. S

Signature and Date

Head of  Department 						     _______________

SURV. AMBIMBOLA ISAU					     Signature and Date

______________

External Moderator 						Signature and Date

SURV. J.O OPALEYE

...............................

DEDICATION

This project is dedicated to Almighty God, the creator of heaven and earth, ancient of days, the I am that I am the beginning and the end whose supremacy in the knowledge of everything is absolute, And to my supervisors Surveyor Diran Felix and Surveyor Banji, and  my noble HOD Surv. AMBIMBOLA ISAU and other lecturers.

ACKNOWLEDGMENT

I will like to express my profound appreciation to almighty God who in his infinite mercy gave me the ability to commence and complete my program in peace and God health, if not for God, where will I be today? All things are possible through him for the knowledge, wisdom and understanding and also his moral, protection and provision on me through my ND programme thank God for the great things he has done, most especially for the success of this project work God be the glory.

My appreciation goes to my lovely parent in person of MR & MRS  RAHMAN for their prayer and support, financially and morally throughout this journey thank for all the beautiful star and end through my ND program. I know I can't repay you back but I have special prayer to offer you both, wishing you both long life and prosperity in good health and wealth, more success, blessing and happiness along with a lot of luck (Amen).

I also give thanks to my project supervisors in the person of Surv.

FELIX DIRAN And Suvr.  ADEBANJI  for their strictly and  through  supervision. I will like to thank all lecturers  of this noble  department starting  from H.O.D Surv. Abinbola isau, Surv. A. Ayuba,  Surv.  Bello Felix Diran,  Surv. Williams Kzeem, Surv. A.O.

Akinyede, and also the Director of  IES

Surv.  A.G. Aremu and other supportive staff of the department of   Surveying and  Geo-informatics, Kwara State Polytechnic, Ilorin.

ABSTRACT

This project is based on utility survey of ROYAL VALLEY ESTATE

at kunlende/akerebiata road, ilorin ilorin east local government area kwara state

This project has been divided into different chapters.Chapter one of this project gives an introduction about the project topic as a whole.This enables better understanding of the project so as to know what perimeter and detailing entails as well as the scope and aim of the project.Chapter two is the literature review i.e. the works of past professionals and projects that had been done in the past as regards this specific topic which were examined in order to help to shed more light on what perimeter and detailing survey is all about. Chapter three of this project is the methodology which describes how the project was carried out using digital instrument e.g. Total Station and its accessories from the first stage to the final stage.Chapter four of this project is Data presentation which consists of all the data acquired from site during the project execution. Chapter five comprises of summary, problems encountered, conclusion and recommendations. Reference comprises of list ofnames and works of prominent authors or professionals whose works on survey as regards this specific topic were used in the course of this project.

TABLE OF CONTENTS

Title page

Certificate

Certification

Dedication

Acknowledgement

Abstract

Table of content

CHAPTER ONE

1.0	INTRODUCTION

1.1	BACKGROUND TO THE STUDY

1.2	STATEMENT OF THE PROBLEMS

1.3	AIM AND OBJECTIVES

1.4	AIM

1.5	OBJECTIVES

1.6	PROJECT SPECIFICATION

1.7	SCOPE OF THE PROJECT

1.8	PERSONAL INVOLVED

1.9	STUDY AREA

CHAPTER TWO

- LITERATURE REVIEW

CHAPTER THREE

- METHODOLOGY

  - RECONNAISSANCE

    - OFFICE PLANNING

    - FIELD RECONNAISSANCE

    - MONUMENTATION

  - EQUIPMENT USED

    - TEST OF INSTRUMENT

    - COLLIMATION TEST FOR TOTAL STATION

    - FORWARD COMPUTATION OF THE CONTROL USED

    - DATA ACQUISATION

    - GEOMETRIC DATA ACQUISATION

3.2.6	PERIMETER TRAVERSING

3.2.7	DETAIL SURVEY

CHAPTER FOUR

4.0 DATA PROCESSING

4.1.1 RESULT ANALYSIS

4.2 BACK COMPUTATION

4.2.1 COMPUTE FOR TOTAL AREA USING DOUBLE LATITUDE AND DEPARTURE

4.2.2 GRAPHICS PLOTTING

CHAPTER FIVE

- SUMMARY, CONCLUSION AND RECOMMENDATIONS

  - SUMARRY

  - PROBLEM ENCOUNTERED

  - CONCLUSION

  - RECOMMENDATION

REFFERENCES

1.0 INTRODUCTION

The continuous expansion of urban centers and the evolution of modern infrastructure have significantly increased the complexity and density of underground utility systems. These systems—including water supply pipelines, sewer networks, electrical conduits, gas mains, fiber-optic cables, and telecommunications lines—form the invisible framework that sustains urban life. As cities grow vertically and horizontally, the ability to accurately detect, map, and assess underground utilities becomes crucial for planning, safety, and sustainability.

A utility survey is a systematic process that identifies and documents the location, depth, type, and condition of subsurface utilities. It employs a combination of traditional and advanced technologies, such as electromagnetic detection, ground-penetrating radar (GPR), hydro-excavation, sonar, LiDAR, and geospatial mapping systems. These tools enable professionals to visualize underground assets without disruptive or invasive digging methods, thereby preserving existing infrastructure and protecting public safety.

Utility surveys serve as a foundational component of modern civil engineering, construction, urban planning, and asset management. Before initiating any major excavation, development, or infrastructure upgrade, it is imperative to have detailed and reliable information about what lies beneath the surface. Accidental damage to utilities during construction not only causes significant project delays and financial losses but also poses severe safety hazards—ranging from electrical shocks and gas leaks to flooding and telecommunications outages.

The role of utility surveys has become increasingly important in today’s data-driven infrastructure environment. The accuracy and comprehensiveness of utility information are key to minimizing risks, ensuring compliance with health and safety regulations, and optimizing construction sequencing. Moreover, with the rise of smart cities and integrated digital systems, utility data is now being used to inform intelligent planning, real-time monitoring, and predictive maintenance. This dynamic integration positions utility surveys not just as supportive tasks but as strategic tools in the broader context of sustainable urban development.

This study aims to provide a comprehensive examination of utility survey practices, focusing on their methodologies, technologies, and applications in urban environments. It also addresses the major challenges—such as data inaccuracy, limited access to records, and legal or institutional constraints—that often hinder the efficiency and reliability of these surveys. In addition, the study explores the current trends shaping the field, including advancements in non-invasive technologies, the use of Geographic Information Systems (GIS), and the adoption of Building Information Modeling (BIM) for utility coordination. By evaluating the present state and future direction of utility surveys, this research underscores their indispensable role in managing urban infrastructure and supporting safe, efficient development.

SIGNIFICANCE OF UTILITY SURVEYS

Utility surveys play a critical role in the development and maintenance of modern infrastructure by offering comprehensive data about underground utility networks. Their significance spans across multiple domains—engineering, construction, public safety, environmental management, emergency response, and urban planning—making them essential components of any development project. The ability to accurately identify and map underground utilities helps mitigate the risk of unintentional utility strikes during excavation, which can result in severe safety incidents, costly repairs, legal consequences, and service disruptions.

In construction projects, utility surveys are crucial for ensuring site readiness and minimizing project risks. Early identification of existing utilities enables project managers and contractors to design construction activities that avoid conflicts with underground systems. This not only enhances safety on site but also streamlines workflows and reduces costly delays. Moreover, adherence to utility survey protocols ensures compliance with local and national regulations, which increasingly mandate non-invasive assessment of underground services before excavation.

In the context of urban planning and development, utility surveys provide essential data for infrastructure layout, zoning, and service delivery. Planners and municipal authorities use utility information to assess the capacity and suitability of existing systems, predict future demand, and plan for infrastructure expansion or upgrades. Accurate utility data also supports sustainability goals by enabling the integration of green infrastructure, energy-efficient networks, and water-sensitive urban design.

For utility service providers, the value of utility surveys lies in effective asset management and maintenance planning. Accurate maps of underground assets reduce operational uncertainty and support proactive maintenance strategies, thereby extending the lifespan of infrastructure and reducing emergency repairs. Additionally, when paired with technologies like GIS and BIM, utility survey data becomes a part of a broader digital ecosystem, enabling real-time monitoring, predictive analytics, and optimized resource management.

The public safety implications of utility surveys cannot be overstated. Excavation without prior utility identification has been linked to accidents involving gas explosions, electrical hazards, and water main bursts. These incidents not only threaten lives and property but also erode public trust in infrastructure safety. Thorough utility surveys minimize such risks, providing assurance to both stakeholders and the public.

From an environmental perspective, utility surveys also play a role in reducing the ecological impact of construction. By preventing unnecessary digging and identifying potential environmental hazards—such as old asbestos pipes or oil tanks—surveys help ensure that development proceeds in an environmentally responsible manner. Furthermore, in areas with archaeological or heritage significance, utility surveys enable sensitive excavation planning that avoids damage to cultural resources.

As cities adopt smart technologies and move toward digital twins, the integration of utility survey data into centralized management platforms becomes increasingly valuable. Such systems allow stakeholders to access real-time information, simulate construction impacts, and coordinate multi-utility projects efficiently. This level of coordination is particularly important in megacities and complex urban environments where the failure of one utility system can have cascading effects on others.

In conclusion, utility surveys are far more than technical assessments; they are strategic tools in urban development, safety management, and infrastructure sustainability. Their ability to provide precise, non-intrusive insights into the underground environment empowers stakeholders to build safer, smarter, and more resilient cities. As technological capabilities expand, the importance of utility surveys will only grow, reaffirming their central role in shaping the future of urban infrastructure.

1.1 BACKGROUND TO THE STUDY

As urbanization continues to accelerate across the globe, cities face increasing demands on their infrastructure systems. Central to this infrastructure are underground utilities—such as water mains, power cables, gas pipelines, and telecommunication lines—that serve as vital lifelines for urban functioning. With the growing density of these networks, construction and development activities are at heightened risk of disrupting existing services, leading to safety hazards, project delays, and financial losses. In this context, utility surveys have become a critical component of urban development projects. They provide essential data on the location and condition of subsurface utilities, enabling planners, engineers, and contractors to make informed decisions. This study focuses on investigating the methods, tools, and emerging technologies used in utility surveying, and evaluates their role in promoting safe, efficient, and sustainable infrastructure development in modern urban settings.

1.2 STATEMENT OF THE PROBLEM

Although utility surveys are vital to the success and safety of urban development projects, their full potential is often undermined by persistent challenges. A key concern is the unreliability or absence of accurate, current data on the location and status of underground utilities. This information gap frequently results in accidental utility strikes during excavation, posing serious safety risks, delaying project timelines, and incurring substantial financial losses. Compounding the issue, many existing utility records are incomplete, poorly maintained, or difficult to access due to legal or institutional restrictions. Moreover, as urban infrastructure becomes more intricate and layered, traditional surveying techniques are increasingly inadequate to meet the demands of modern construction environments. These challenges underscore the urgent need for improved utility survey methodologies, enhanced data-sharing protocols, and the integration of advanced technologies. This study aims to explore and address these critical issues, advocating for more effective and collaborative utility survey practices in urban planning and infrastructure management.

1.4 AIM

The primary aim of a utility survey is to accurately locate, identify, and map underground utilities such as water pipes, electrical cables, gas lines, sewer systems, and communication networks. This process is essential for ensuring safety, minimizing project risks, and supporting efficient planning and execution of construction and infrastructure projects. Utility surveys play a vital role in preventing accidental utility strikes, improving coordination among stakeholders, and enabling sustainable urban development.

1.5 OBJECTIVES

The specific objectives of a utility survey include:

- 1. To detect and map underground utility assets using appropriate technologies such as ground-penetrating radar (GPR), electromagnetic locating, and GPS/GIS integration.

- 2. To provide accurate and up-to-date subsurface utility data that supports planning, design, and construction processes.

- 3. To reduce the risk of service disruption, accidents, and financial loss caused by unintentional utility damage during excavation.

- 4. To support decision-making in urban planning, infrastructure development, and emergency response by offering clear and reliable utility information.

- 5. To identify gaps in existing utility records and address issues of data inaccuracy and inconsistency.

- 6. To enhance communication and coordination among engineers, utility providers, planners, and contractors through the standardized sharing of utility information.

1.6 PROJECT SPECIFICATION

This study will focus on a comprehensive analysis of utility surveying methodologies, including both traditional and advanced technologies, covering:

- Theoretical frameworks that underpin utility surveys.

- Practical case studies illustrating the application of utility surveys in urban development.

- A review of technological advancements that integrate GIS, AI, and drone technology.

- An examination of the regulatory environment affecting utility surveys.

- Recommendations for best practices in utility surveying to ensure safety and efficiency.

1.7 SCOPE OF THE PROJECT

The scope of this project will encompass an extensive review of literature and case studies related to utility surveys in urban contexts. The focus will be on geographically diverse urban areas to understand how local regulations, technological adoption, and infrastructure complexity impact surveying practices. The study will also examine the integration of utility surveys in urban planning and emergency management, alongside an assessment of future trends that may reshape the industry. However, the research will primarily concentrate on the methodologies and technologies relevant to utility surveys, while recognizing that factors such as social, political, and environmental influences also play a significant role in infrastructure development.

1.8	PERSONAL INVOLVED

| S/N | NAMES | MATRIC NUMBER | ROLES |
| --- | --- | --- | --- |
| 1 | RAHMAN ZAINAB OLAMIDE | ND/23/SGI/FT/066 | AUTHOR |
| 2 | OLAFIMIHAN EMMANUEL . O | ND/23/SGI/FT/083 | MEMBER |
| 3 | MAFTAU OMOWUNMI AISHAT | ND/23/SGI/FT/054 | MEMBER |
| 4 | OLAWALE AZEEZ .O | ND/23/SGI/FT/055 | MEMBER |
| 5 | AWOLUSI TAIWO IREYEMI | ND/23/SGI/FT/066 | MEMBER |
| 6 | MUHAMMED AISHAT | ND/23/SGI/FT/058 | MEMBER |
| 7 | BABALOLA OLAMIDE GRACE | ND/23/SGI/FT/062 | MEMBER |

1.9	STUDY AREA

The study area is located at royal Valley estate, At Kunlende/akerebiata road, ilorin ilorin East Local government area Kwara State

Map of Nigeria

Map of Ilorin.                         Map of study area

CHAPTER TWO

LITERATURE REVIEW ON UTILITY SURVEYS

Introduction

The growing complexity of urban infrastructure has made utility surveys an indispensable part of modern engineering and planning. Subsurface utilities, including water mains, sewer lines, electrical conduits, gas pipelines, and telecommunication cables, form the invisible framework that supports city life. Proper identification, documentation, and management of these underground networks are essential for preventing costly damages, ensuring public safety, and promoting sustainable development. This chapter reviews the evolution, methodologies, technologies, applications, challenges, and emerging trends associated with utility surveys, drawing insights from both academic literature and practical implementations.

Utility surveys play a vital role in identifying and mapping underground infrastructure such as water pipes, gas lines, electrical cables, and communication conduits. These surveys are essential in preventing accidental utility strikes, reducing service disruptions, and ensuring safe and efficient construction and urban development activities. As cities continue to expand and urban environments become more complex, the demand for accurate and up-to-date subsurface information has become increasingly critical.

Among the most widely adopted technologies in modern utility surveying is Ground Penetrating Radar (GPR), which utilizes radar pulses to detect underground utilities. GPR is valued for its ability to locate both metallic and non-metallic utilities without excavation, although its effectiveness can be influenced by soil type and moisture. Electromagnetic locators are also commonly used to detect metallic utility lines by tracing electromagnetic signals. These tools are particularly useful for identifying conductive materials like steel and copper. In addition to detection equipment, digital platforms such as Geographic Information Systems (GIS) have transformed the way utility data is visualized and managed. GIS enables spatial analysis and facilitates the overlay of utility data with other infrastructure layers, improving the accuracy and usability of survey information. Building Information Modeling (BIM) is another innovation being integrated into utility surveying, allowing for the creation of detailed 3D models that enhance planning and decision-making in infrastructure projects.

Utility surveys are applied in a wide range of scenarios. In construction, accurate utility maps help teams avoid damaging hidden infrastructure, thereby enhancing safety and reducing delays. In urban development, survey data supports more strategic planning by revealing spatial relationships among utilities and other infrastructure components. Emergency management agencies also benefit from reliable utility data, especially during natural disasters or hazardous events when rapid decision-making is required. Furthermore, utility surveys can be used to assess environmental risks, such as potential leaks or contaminations, aiding in regulatory compliance and ecological protection.

Despite notable progress, the field still faces several persistent challenges. Many utility records remain incomplete or inaccurate, especially for older infrastructure. In some regions, undocumented or abandoned utilities further complicate the surveying process. Additionally, issues such as data privacy and the reluctance of utility companies to share information can limit the scope and accuracy of surveys. The lack of standardization in utility data formats and survey procedures also poses a barrier to creating integrated and comprehensive utility databases. These issues are compounded in dense urban settings, where overlapping infrastructure makes accurate detection and mapping more difficult.

Emerging trends offer promising solutions to these challenges. Artificial Intelligence (AI) is being explored to assist in automating the interpretation of survey data, detecting patterns, and improving predictive modeling. Drones are increasingly used to complement traditional ground-based surveys, offering rapid and cost-effective data collection over large or difficult-to-reach areas. In combination with cloud-based platforms, these tools facilitate real-time data sharing and collaboration among stakeholders. Furthermore, the integration of Internet of Things (IoT) technologies is enabling continuous monitoring of utility infrastructure, which can alert teams to changes or anomalies before they escalate into major problems.

Historical Evolution of Utility Surveys

Utility surveying, as a professional practice, has evolved significantly over the last few decades. In its earliest forms, it relied heavily on paper-based records and rudimentary mapping methods. These early maps often lacked precision and completeness, resulting in frequent conflicts during construction activities (Schmidt & Schütze, 2003). Misidentification or complete absence of utility data led to repeated service interruptions, legal disputes, and increased project costs.

By the late 20th century, increasing urban density and aging infrastructure prompted governments and engineering firms to seek more reliable surveying practices. The limitations of traditional techniques highlighted the urgent need for innovation. Researchers like Morris and Heneghan (2004) advocated for the use of geophysical and digital technologies to modernize utility detection and mapping.

SURVEYING METHODS AND TECHNOLOGICAL DEVELOPMENTS

Over the years, utility surveying has incorporated a range of technological tools and data collection strategies. Prominent among these is Ground Penetrating Radar (GPR), a geophysical technique that employs radar pulses to image the subsurface. GPR is favored for its non-invasiveness and ability to detect both metallic and non-metallic utilities (Baker & Hernandez, 2009). It has been widely used in urban areas where excavation is limited or undesirable.

Electromagnetic Induction (EMI) is another widely employed method, particularly effective in detecting conductive materials like metal pipes and electric cables. EMI systems measure the magnetic fields induced by underground conductors, making it suitable for quick assessments in congested areas (Geyer et al., 2020). The synergy of GPR and EMI in multi-sensor platforms has significantly improved detection accuracy.

Digital innovations such as Geographic Information Systems (GIS) have also transformed utility management. GIS allows for the storage, integration, and analysis of spatial utility data, enabling better visualization and planning (Liu et al., 2018). These systems provide a centralized platform where utility maps can be layered with environmental, zoning, and topographic data for holistic infrastructure planning.

More recently, 3D modeling and Building Information Modeling (BIM) have emerged as powerful tools for visualizing subsurface utilities. These technologies enhance spatial understanding, facilitate interdisciplinary collaboration, and support advanced simulations for utility conflict detection (Sheng & Li, 2019). The combination of BIM with GIS is gaining traction for large-scale infrastructure projects due to its ability to integrate real-time updates and lifecycle data.

Applications in Infrastructure and Urban Planning

Utility surveys serve multiple purposes across a broad spectrum of industries. In construction, pre-construction utility detection is a critical step for minimizing the risk of damaging buried infrastructure. Accidental strikes on utilities can delay projects, increase costs, and endanger workers (Pawlukiewicz et al., 2005). Hence, accurate utility surveys form an essential component of site risk assessments.

Urban planners use utility data to inform land-use decisions, optimize infrastructure layouts, and avoid potential conflicts during redevelopment. For example, accurate knowledge of existing underground assets enables planners to reroute new services or realign project scopes to avoid high-risk zones (Federico et al., 2021). Additionally, local governments use utility maps to coordinate maintenance schedules and infrastructure upgrades across multiple departments.

In the realm of disaster response, utility maps provide emergency teams with critical insights into the locations of pipelines, water mains, and electrical lines. This information aids in preventing secondary hazards, such as gas leaks or flooding, during natural disasters (Levine et al., 2015). Furthermore, utility surveys support environmental monitoring, helping assess the impact of infrastructure projects on surrounding ecosystems by mapping potential contaminant pathways.

Challenges and Limitations in Utility Surveying

Despite technological advancements, several obstacles continue to hinder the effectiveness of utility surveys. Chief among these is the inaccuracy of historical records, which may be outdated, incomplete, or inconsistent with current utility placements. In many developing areas, utility networks have evolved informally without standardized documentation (Santos & Correia, 2016).

Another persistent issue is the lack of data interoperability and standardized formats. Different utility providers often maintain separate databases, which may not be compatible with one another or shared openly due to security, privacy, or competitive concerns (Meyer et al., 2017). This fragmentation creates gaps in knowledge and complicates comprehensive mapping efforts.

Physical constraints also affect survey quality. Urban environments with dense, overlapping infrastructure can cause interference that limits the resolution of GPR and EMI readings. Surface conditions—such as concrete, traffic, or noise—can also restrict equipment use or skew data interpretation.

The field additionally suffers from a skills gap. As tools become more technologically advanced, the demand for highly trained personnel increases. Many surveying professionals lack access to training in using advanced systems such as BIM, GPR analysis, and GIS software, which leads to underutilization of available resources and inconsistent data quality.

RECENT INNOVATIONS AND FUTURE DIRECTIONS

The future of utility surveying is poised for substantial transformation through emerging digital and automation technologies. Artificial Intelligence (AI) and Machine Learning (ML) are being employed to enhance utility detection algorithms, automate feature classification, and predict subsurface layouts from incomplete data (Zhou & Li, 2020). These technologies improve decision-making and reduce the potential for human error in data interpretation.

Unmanned Aerial Vehicles (UAVs), or drones, are gaining popularity for their ability to gather high-resolution topographic and visual data from hard-to-access or hazardous locations. Although currently more suited for surface mapping, UAVs can support ground-based surveys by providing contextual imagery and aiding site logistics (Kumar et al., 2022).

Cloud-based platforms are also transforming the way utility data is stored and shared. These platforms facilitate real-time collaboration, mobile access, and centralized updates, making them ideal for managing large infrastructure projects with multiple stakeholders. Moreover, as cities adopt smart infrastructure, integration with Internet of Things (IoT) sensors may enable continuous monitoring of underground assets, improving responsiveness and maintenance planning.

In line with the global push toward sustainability, utility surveys are increasingly incorporating environmental impact assessments. By identifying and mitigating potential harm to ecosystems during excavation or installation, these surveys support broader environmental goals (Harlow et al., 2019). This approach aligns utility mapping with climate resilience and sustainable development objectives.

CONCLUSION

Utility surveys have become foundational in modern infrastructure development, serving as essential tools for risk management, planning, and sustainability. The transition from manual, paper-based records to digital, sensor-enhanced systems has greatly improved the reliability and accuracy of underground utility detection. Technologies like GPR, EMI, GIS, and BIM have enhanced our ability to visualize, document, and manage complex underground networks.

Nevertheless, the field continues to face several constraints, including data fragmentation, limited standardization, environmental challenges, and skill shortages. Addressing these issues will require coordinated efforts from governments, industry stakeholders, and academic institutions.

Looking ahead, the integration of AI, UAVs, and cloud computing promises to redefine the utility surveying landscape, offering smarter, faster, and more connected solutions. These advancements, coupled with efforts to align utility surveys with sustainability and resilience goals, will ensure that future infrastructure development is both intelligent and inclusive.

CHAPTER THREE

3.0 	METHODOLOGY

This aspect refers to a system of methods and principles, rules used in a field of study to achieve the aim and objectives of this project. The modality to successful execute this project was carried out based on the survey rules and regulation, supervisor advice and departmental instructions.

3.1 	RECONNAISSANCE

Reconnaissance survey is a preliminary stage for exploratory investigations conducted to gather initial data concern the choice of Site area of Subject before more detailed research or filed work commenced. Reasons for reconnaissance survey of the project area are the following;

- To know the location of the site.

- To determine the preliminary area/perimeter of the land.

- To know the survey method suitable for the field data acquisition and the instrument need for the it.

Reconnaissance in this project involved preliminary investigations to gather information about the survey area before conducting a detailed survey, reconnaissance of this project was carried out in two way namely as follows.

- Office planning

- Field Reconnaissance

3.1.1 	OFFICE PLANNING

It involved the process adopted in the collection of relevant data and specification about the as-built Surveying with the used of traversing survey method. During the office planning the given coordinate for control point PBIL 100, PBIL 107, and PBIL 110, was obtained and computed as shown in the table 3.1 below. The choice of equipment to be use for project was obtained from the survey store in the department of surveying and Geoinformatic, Kwara State Polytechnic, Ilorin, and it was handled accordingly to the survey rules and regulations and departmental instruction.

TABLE 3.1 Showing the list of the coordinate of the control pillars used

| Stations | Northing (m) | Easting (m) |
| --- | --- | --- |
| PBIL 100 | 939353.130 | 648821.110 |
| PBIL 107 | 939306.460 | 648932.560 |
| PBIL 110 | 939224.969 | 649120.403 |

3.1.2 	FIELD RECONNAISSANCE

Field recce and site visitation was conducted as preliminary survey for investigation of the basic information about a specified project area, location, or subject in the field. It allows the members of the groups to be familiar with the terrain, boundary lines, selection number of station required, Equipment that will be used for the job, pillars number PBIL 100, PBIL 107, and PBIL 110, used as a control for the project and so on. The stations points selected was Inter-visible to one another and finally a reconnaissance diagram was produced which was attached to this project

RECCE DIAGRAM

3.2	INSTRUMENT SELECTION

The equipment used for this project was grouped into two categories;

- Hardware

- Software

3.2.1	Hardware used

- Total Station with its Accessories (KTS-442L)

- Reflector poles

- Field book

- Writing material (pen)

- Survey beacons

- Steel Tape (5m)

- Tripod Stand

3.2.2	Software used

Below are the list of the software used during the project processing;

- AutoCAD 2007

- Notepad

- Microsoft word

- Microsoft Excel

3.3 	INSTRUMENT TEST

Total station

LINE OF SIGHT

RANGING POLE

20M

STATION  X

TERRAIN

STATION  Y

The instrument test was carried out before it used for angular observation. The purpose of testing the instrument before used is to know the assurance and maybe other instrument is in good Condition the instrument was set up over station A and it was pointed a Sighted to station B whore a pole Is hold, and the angle was recorded for face left the instrument was turn to farce right on the same point and it was sighted to station B, where a pole is held at the same position and the angle was recorded. The distance between station A and B 1s 10m. The steel tape was hold at 60m an point A and it was extend to point B. the results for the instrument test was shown i to figure 3.3 below and table 3:2 below.

Figure 3.1 showing the instrument set up for collimation test

Table 3.2 showing the test of instrument result

| STATION | SIGHT | FACE | HORIZONTAL READING | DEDUCED ANGLE |
| --- | --- | --- | --- | --- |
| A | B | L | 180° 00’ 03” |  |
|  | B | R | 00°00’ 00” | 180° 00’ 03” |

Different in horizontal circle reading: 180° 00’ 03’’

Horizontal collimation error

= 180° 00’ 03’’

= 000° 00 03’’

Collimation error = 00° 00'03"2

Horizontal collimation error = 00° 00’ 01”

Because the collimation  error falls within the allowable 01”. The instrument was in good condition.

3.4 	MONUMENTATION

This is the process in which the beacon is being buried in to ground. On the project site there are same existing  beacons on the boundary lines m, nail with cap (bottle cork) was used as the 	station pegs.

18cm

15cm above the ground

60cm below the ground

18cm

30cm

Nail

Bottle cork

PILLAR							   PEG

FIGURE 3.4	Diagram and Dimension of Pillars buried

3.5 	CONTROL CHECK

The Control check was carried out to know the control point PBIL 100, PBIL 107, and PBIL 110, are still in their positions. The instrument was set on Control FP01107 and the temporary adjustment was carried out i.e centring, levelling, and focusing. Back site was taken to control PBIL 107 and it was recorded.

The fore sight was served to PEG1 and it was recorded Steel tape was used to measure the distance between PBIL 110 and PEGI and it was recorded. The data observed was computed for given coordinate and Observed Coordinate, as shown in the table 3.2 below.

TABLE 3.2	THE GIVEN COORDINATE OF THE CONTROL POINTS

| STATION | NORTHING (m) | EASTING (m) |
| --- | --- | --- |
| PBI 100 | 939353.130 | 648821.110 |
| PBIL 107 | 939306.460 | 648932.560 |
| PBIL 110 | 939224.969 | 649120.403 |

TABLE 3.3	The Backward Computation for the Given Controls pillars

| From station | Bearing | Distance | ∆N | ∆E | Northing (m) | Easting (m) | To station |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | 899353.130 | 688821.110 | PBIL 100 |
| PBIL 100 | 1120 43̍ 17 ̎ | 120.827 | -46.670 | 111.450 | 899306.460 | 688932.560 | PBIL 107 |
| PBIL 107 | 1130 27̍ 09 ̎ | 204.758 | -81.491 | 187.843 | 899224.969 | 689120.403 | PBIL 110 |

PBIL 100 to PBIL 107

BEARING = 1120 43̍ 17 ̎

DISTANCE = 120.827m

PBIL 107 to PBIL 110

BEARING = 1130 27̍ 09 ̎

DISTANCE = 204.758m

TABLE 3.4	Show the Observed Coordinate of the Controls Pillars

| Station | Northing (m) | Easting (m) |
| --- | --- | --- |
| PBIL 100 | 939353.126 | 648821.105 |
| PBIL 107 | 939306.460 | 648932.560 |
| PBIL 110 | 939224.969 | 649120.403 |

TABLE 3.5	Show the Backward Computation for the Observed Control Pillars

| From station | Bearing | Distance | ∆N | ∆E | Northing (m) | Easting (m) | To station |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | 899353.126 | 688821.105 | PBIL 100 |
| PBIL 100 | 1120 43̍ 08 ̎ | 120.830 | -46.666 | 111.455 | 899306.460 | 688932.560 | PBIL 107 |
| PBIL 107 | 1130 27̍ 09 ̎ | 204.758 | -81.491 | 187.843 | 899224.969 | 689120.403 | PBIL 110 |

PBIL 100 to PBIL 107

BEARING = 1120 43̍ 08 ̎

DISTANCE = 120.830m

PBIL 107 to PBIL 110

BEARING = 1130 27̍ 09 ̎

DISTANCE = 204.758m

TABLE 3.6	Show the Difference between the Given Control Points and the Observed Control Pillars

| STATION | NORTHING (m) | EASTING (m) |
| --- | --- | --- |
| PBIL 100 | 0.004 | 0.005 |
| PBIL 107 | 0.000 | 0.000 |
| PBIL 110 | 0.000 | 0.000 |

Therefore the difference between the given and observer bearing was 00° 00’ 09” and 00° 00’ 00” and the difference between the given and the observer linear distance was 0.0003m, 0.000m the results obtained fall within the allowable accuracy for both angular and linear observation.

It was concluded the instrument was in good condition and fit to be used for the project.

3.6 	DATA AQUISITION

This was carried out on the field using Total station with its accessories (KTS-442L) was used for coordinating the entire site for both the perimeter and details of the site. The Total station was set on the 	control Pillar PBIL 107, and the temporary adjustment was carried out centering, leveling and 	focusing. The coordinates of the occupied station was input after that the coordinates of the backsight was record as well and the observation was done to backsight pillar and the coordinates was take and check against the given. Having check and found accurate then the perimeter survey commenced for the entire project site.

3.6.1 	DETAILING SURVEY

The detailing survey was done immediately after completely carried out the perimeter traverse to determine the position of the features on the Site was fixed using radiation survey method. During this several stations was established to pick the coordinates of the details and the coordinates was stored on the total station as its observed. This was later downloaded. The details that was picked during the detailing survey was coded in different ways like Building edges as ‘BLD’, Electric Poles ‘EP’, Drainage ‘DR’, Roads Line ‘RD’, Spot Height ‘SP’ and so on.

CHAPTER FOUR

4.0   	DATA PROCESSING

This involved several step to ensure accurate and reliable result. It explains the method used in capturing; processing, downloading, and analyzing the data acquired Data reduction in surveying involves various methods to simplify and summarize the collected data for analysis. These are some common methods of data reduction that involves

- Combining multiple values into a single value. The average value is calculated by summing up all the values and dividing by the number of values.

- Least Square Adjustment (LSA) is a method of data reduction that involves minimizing the sum of the squared errors between observed and calculated values.

- Interpolation is a method of data reduction that involves estimating values between observed data points.

- Spatial data reduction refers to the process of simplifying and reducing the complexity of spatial data, such as geographic information system (GIS) data, while preserving its essential characteristics.

4.1	DATA EDITING

- Open excel

- Click on the desire file

- Set to delimited

- Click on next

- Mack space and comma

- Click

- On next

- Then click on finish

4.3	DATA PROCESSING USING AUTOCAD

The plan produced digitally with AutoCAD software. The following are the procedure used:

STEP 1: Open Notepad

Open Notepad on your computer. You can find it in the start menu (Window)

STEP 2: Create a New File

Click on "file" "New" to create a new file

STEP3: Enter Coordinates

Enter your coordinates in the following format:

X. Y. Data

STEP 4: Save the Fil

Click on "File" > "Save as" and choose a location to save your file. "ALADA Coordinate" txt, 	and select "All files" as the file type.

Importing the Coordinate File Into 2007 AutoCAD

STEP 1: Double click AutoCAD

Go under format

Click on format after that pick your unit

Drawing unit

Type.... Decimal

Precision... 0.000

Angle type Degree/Min/See

Precision Od00

Clockwise.... 노

Unit to scale Meters

Direction control North 270d0

Press "OK"

STEP 2: Create a new drawing

Click on "File" > "Name" to create a new drawing

STEP 3: Import the coordinate file

Click on "Tools" >"Reference" > "Import" and select "ALADA coordinate (.txt)" as the file 	type. Navigate to the location where you saved your coordinate file ("ALADA coordinate .txt") 	and select it.

STEP 4: Configure Import Setting. In the import select "Point" as the object type and choose 	the correct coordinate format (eg. X and Y) click "OK" to import the coordinates.

4.4	DATA EXTRACTION

The data recorded in the field book was picked to compute for the bearing and distance. Forward computation and backward computation after all necessary correction have been applied. Correction like slope correction, tension correction and standardization correction.

4.3  	BACK COMPUTATION

The result get for forward computation was used for Back computation.

The following program was required for back computation

i. Input N2-N1 to get ∆N

ii. Input E2-E1 to get ∆E

iii.Input (∆N2+∆E2 to get distance

iv.Input shift tan ∆E∆Nto get the corrected bearing when there us more station repeat step ¡¡¡ and ii.

Table 4.1 Showing Boundary Coordinates

| Station | Northing (m) | Easting(m) |
| --- | --- | --- |
| FPO1107 | 939366.406 | 648932.560 |
| PL1 | 939240.312 | 648901.750 |
| PL2 | 939309.331 | 648780.785 |
| PL3 | 939292.197 | 648603.139 |
| PL4 | 939438.718 | 648619.716 |
| PL5 | 939467.847 | 648693.558 |
| PL 6 | 939353.126 | 648821.105 |

Table 4.1.1 Showing back computation for the boundary coordinate

| FROM Station | BEARING | DISTANCE | ΔN | ΔE | N |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | 89 |
| PBIL 107 | 204 58 29 | 72.971 | -66.148 | 30.810 |  |
| PL1 | 299 42 28 | 139.270 | 69.019 | 120.965 |  |
| PL2 | 295 39 02 | 193.739 | 83.866 | 174.646 |  |
| PL3 | 16 40 37 | 47.520 | 45.521 | 13.637 |  |
| PL4 | 112 42 17 | 79.980 | 30.871 | 73.782 |  |
| PL5 | 113 13 14 | 113.790 | 54.721 | 127.547 |  |
| PL6 | 112 43 59 | 120.870 | 48.666 | 455 |  |

TABLE 4.1.2	showing area computation for the boundary coordination using double latitude

| ΔN | ΔE | product |
| --- | --- | --- |
| 66.148 | × - 30.810 | = 2038.020 |
| 66.148 |  |  |
| 69.019 |  |  |
| -63.277 | × - 120.965 | =7654.302 |
| 69.619 |  |  |
| 83.866 |  |  |
| 89.608 | × -174.646 | -=15649.679 |
| 83.866 |  |  |
| 45.521 |  |  |
| 218.995 | × 13.637 | =2986.435 |
| 45.521 |  |  |
| -30.871 |  |  |
| 233.645 | × 73.782 | =17238.792 |
| -30.871 |  |  |
| -54.721 |  |  |
| 148.653 | × 127.547 | =18883.716 |
| -54.721 |  |  |
| -46.666 |  |  |
| 46.666 | × 111.455 | =5201.159 |
| -46.666 |  |  |
| 00.000 |  |  |

Area =

2038.020+7654.302-15649.679+2986.435+17238.795+18883.716+5201.1592

54002.427-18649.6792

38352.7482

=19176.374=> 1.9 hectares

1.9 Hect meter square

Hectare=19176.37410000

= 1.9176374 Hectare

Linear Accuracy = 1(EL)2-((ED)2TOTALLENGHT

EL=EDN=0.066

ED=EDE=0.072

Total length =793.100

∴LinearAccuracy

1(0.66)2+(0.072)2793.100x = 10.09767793.100

=1:8000

CHAPTER FIVE

5.0 	SUMMARY, PROBLEMS ENCOUNTERRED, CONCLUSION, RECOMMENDATION

5.1 	SUMMARY

5.2 	PROBLEM ENCOUNTERED

- Too much of rainfall which does not allowed site

- Unavailable of the group members due to dean's cup football tournament

- While taking observing there are some object obstructing the view when taking the observation e.g electric pole, building, tree etc.

5.3 	RECOMMENDATION

The department should considered the season and time that the students should be given the terminal project.

5.4 	CONCLUSION

At the end the aim and objective of this project was achieved. It was carried out using third order surveying method this project was carried out in accordance with survey rules and regulation and departmental instruction.

| Name | Code | E | N | h |  | PT260 | SH | 668115.4 | 937791.5 | 317.946 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PT1 | ST17 | 668069.6 | 937900.4 | 324.137 |  | PT261 | SH | 668118 | 937790.7 | 318.041 |
| PT2 | CV | 668071.1 | 937900.4 | 324.553 |  | PT262 | SH | 668117.7 | 937795.6 | 317.967 |
| PT3 | CV | 668072 | 937900.3 | 324.541 |  | PT263 | SH | 668113.5 | 937797.2 | 317.656 |
| PT4 | BLD | 668072.6 | 937900.6 | 324.383 |  | PT264 | SH | 668110.1 | 937800.8 | 317.647 |
| PT5 | BLD | 668066.5 | 937905.8 | 323.724 |  | PT265 | SH | 668110.4 | 937806.9 | 317.571 |
| PT6 | SH | 668069.7 | 937897.9 | 324 |  | PT266 | SH | 668114.1 | 937808.4 | 317.326 |
| PT7 | SH | 668072.6 | 937896.1 | 324.671 |  | PT267 | SH | 668117.8 | 937810 | 317.601 |
| PT8 | SH | 668072.2 | 937891.5 | 324.604 |  | PT268 | SH | 668117.6 | 937814.7 | 317.532 |
| PT9 | SH | 668076.5 | 937892.6 | 324.821 |  | PT269 | SH | 668113.8 | 937816 | 317.212 |
| PT10 | SH | 668083.2 | 937892.3 | 324.831 |  | PT270 | SH | 668109 | 937815.1 | 317.485 |
| PT11 | SH | 668085.6 | 937889.4 | 324.87 |  | PT271 | SH | 668104.5 | 937814.4 | 317.27 |
| PT12 | BLD | 668085.4 | 937886.8 | 324.848 |  | PT272 | SH | 668104.8 | 937819.5 | 317.197 |
| PT13 | SH | 668088.7 | 937888.5 | 324.887 |  | PT273 | SH | 668109.7 | 937818.5 | 317.278 |
| PT14 | SH | 668090.4 | 937893.3 | 324.882 |  | PT274 | SH | 668113.6 | 937819 | 317.192 |
| PT15 | SH | 668095.3 | 937895.1 | 324.957 |  | PT275 | SH | 668117.7 | 937821.2 | 317.334 |
| PT16 | SH | 668095.6 | 937891.4 | 324.958 |  | PT276 | SH | 668114.1 | 937823.2 | 317.152 |
| PT17 | BLD | 668139.4 | 937901.3 | 324.586 |  | PT277 | SH | 668109 | 937824.1 | 317.181 |
| PT18 | CV | 668139.1 | 937901.2 | 316.047 |  | PT278 | SH | 668104.9 | 937822.2 | 317.161 |
| PT19 | CV | 668139.5 | 937900.4 | 316.104 |  | PT279 | SH | 668105 | 937825.4 | 317.107 |
| PT20 | CV | 668137.3 | 937899 | 317.048 |  | PT280 | SH | 668103.5 | 937827.1 | 317.276 |
| PT21 | SH | 668133.7 | 937894.7 | 316.106 |  | PT281 | SH | 668108.1 | 937827.2 | 317.179 |
| PT22 | SH | 668136.7 | 937894.8 | 316.021 |  | PT282 | SH | 668112 | 937827.1 | 317.097 |
| PT23 | BLD | 668137 | 937893.5 | 316.015 |  | PT283 | SH | 668116.6 | 937827.5 | 317.116 |
| PT24 | SH | 668141.1 | 937890.4 | 315.886 |  | PT284 | SH | 668117.8 | 937831.4 | 317.048 |
| PT25 | SH | 668145.9 | 937889.2 | 315.599 |  | PT285 | SH | 668112.9 | 937833.2 | 317.012 |
| PT26 | SH | 668147.8 | 937891.6 | 315.619 |  | PT286 | SH | 668108.8 | 937832.7 | 317.058 |
| PT27 | GT | 668148.7 | 937893.4 | 315.804 |  | PT287 | SH | 668106.3 | 937831.7 | 317.095 |
| PT28 | GT | 668147.9 | 937894 | 315.765 |  | PT288 | SH | 668120.4 | 937780 | 318.343 |
| PT29 | BLD | 668148.9 | 937893.2 | 315.928 |  | PT289 | SH | 668125.9 | 937783 | 318.036 |
| PT30 | SH | 668148.1 | 937886.3 | 315.852 |  | PT290 | SH | 668122.6 | 937785.4 | 318.043 |
| PT31 | SH | 668144.4 | 937882.9 | 315.942 |  | PT291 | SH | 668120.9 | 937789.4 | 318.013 |
| PT32 | SH | 668140.3 | 937879.1 | 316.287 |  | PT292 | BLD | 668120.6 | 937791.6 | 317.908 |
| PT33 | SH | 668144.4 | 937877.2 | 316.236 |  | PT293 | SH | 668125 | 937790.3 | 317.853 |
| PT34 | SH | 668147.4 | 937877.9 | 316.065 |  | PT294 | SH | 668128.5 | 937785.7 | 317.944 |
| PT35 | BLD | 668149.4 | 937879.9 | 315.956 |  | PT295 | SH | 668133 | 937781.5 | 317.952 |
| PT36 | BLD | 668156.1 | 937879.6 | 315.966 |  | PT296 | SH | 668132.9 | 937786.5 | 317.857 |
| PT37 | BLD | 668156 | 937874.1 | 315.966 |  | PT297 | SH | 668132.2 | 937790.7 | 317.807 |
| PT38 | EP | 668150.9 | 937875.2 | 316.093 |  | PT298 | SH | 668137 | 937790.8 | 317.772 |
| PT39 | SH | 668152 | 937878.1 | 315.912 |  | PT299 | SH | 668139.7 | 937786.5 | 317.824 |
| PT40 | BLD | 668146.3 | 937874.1 | 316.297 |  | PT300 | SH | 668140 | 937781.4 | 318.046 |
| PT41 | SH | 668132 | 937896.8 | 316.129 |  | PT301 | SH | 668144.3 | 937781.2 | 318.023 |
| PT42 | BLD | 668128.4 | 937894.7 | 316.378 |  | PT302 | SH | 668145.5 | 937786.4 | 317.727 |
| PT43 | SH | 668124.8 | 937894.3 | 316.451 |  | PT303 | SH | 668146 | 937791.2 | 317.717 |
| PT44 | SH | 668121.3 | 937892 | 316.499 |  | PT304 | SH | 668149.5 | 937790.8 | 317.75 |
| PT45 | SH | 668118.3 | 937894.3 | 316.594 |  | PT305 | SH | 668149.7 | 937786.7 | 317.692 |
| PT46 | SH | 668115.7 | 937888.6 | 316.573 |  | PT306 | SH | 668148.9 | 937781.3 | 317.842 |
| PT47 | BLD | 668113.6 | 937886.9 | 316.529 |  | PT307 | BLD | 668150.4 | 937781.4 | 317.8 |
| PT48 | SH | 668110.4 | 937889.4 | 316.736 |  | PT308 | BLD | 668150 | 937791.5 | 317.803 |
| PT49 | SH | 668108.3 | 937891.8 | 316.596 |  | PT309 | SH | 668113.7 | 937768.9 | 318.789 |
| PT50 | SH | 668104.4 | 937892.2 | 316.593 |  | PT310 | SH | 668113.6 | 937766 | 318.901 |
| PT51 | SH | 668104.2 | 937889.1 | 316.626 |  | PT311 | BLD | 668112.8 | 937763.3 | 319.05 |
| PT52 | SH | 668117.5 | 937886.8 | 316.551 |  | PT312 | SH | 668117.2 | 937761.4 | 318.943 |
| PT53 | SH | 668119.7 | 937883.9 | 316.541 |  | PT313 | SH | 668117.5 | 937764.3 | 318.911 |
| PT54 | SH | 668117.1 | 937883.2 | 316.562 |  | PT314 | SH | 668122.4 | 937765.5 | 318.977 |
| PT55 | SH | 668114.2 | 937882.6 | 316.592 |  | PT315 | SH | 668124.5 | 937762.3 | 318.933 |
| PT56 | BLD | 668113.5 | 937874.4 | 316.659 |  | PT316 | SH | 668122.4 | 937758.6 | 318.969 |
| PT57 | SH | 668115.5 | 937874 | 316.641 |  | PT317 | SH | 668121.7 | 937753.2 | 319.012 |
| PT58 | SH | 668117.6 | 937873.9 | 316.666 |  | PT318 | SH | 668119.4 | 937752.3 | 319.022 |
| PT59 | SH | 668116.9 | 937866.3 | 316.759 |  | PT319 | BLD | 668113 | 937752.1 | 319.283 |
| PT60 | SH | 668114.9 | 937865.7 | 316.81 |  | PT320 | BLD | 668113.1 | 937748.7 | 319.275 |
| PT61 | SH | 668112.2 | 937865.8 | 317.066 |  | PT321 | BLD | 668113.2 | 937730.1 | 319.819 |
| PT62 | BLD | 668111.7 | 937870.2 | 316.664 |  | PT322 | BLD | 668118.1 | 937726.7 | 320.172 |
| PT63 | SH | 668124 | 937868.3 | 316.542 |  | PT323 | BLD | 668118.2 | 937750.2 | 319.117 |
| PT64 | BLD | 668129.2 | 937867.4 | 316.594 |  | PT324 | SH | 668122.5 | 937751.6 | 319.006 |
| PT65 | SH | 668128.1 | 937869.8 | 316.579 |  | PT325 | SH | 668124.5 | 937754.3 | 318.972 |
| PT66 | SH | 668127.9 | 937872.5 | 316.566 |  | PT326 | SH | 668125.4 | 937757 | 318.942 |
| PT67 | SH | 668125 | 937873.8 | 316.569 |  | PT327 | SH | 668126.4 | 937759.8 | 318.902 |
| PT68 | SH | 668123.6 | 937877 | 316.822 |  | PT328 | SH | 668127.7 | 937763.6 | 318.945 |
| PT69 | SH | 668126 | 937879.4 | 316.682 |  | PT329 | SH | 668129.6 | 937766.7 | 319.003 |
| PT70 | SH | 668121.7 | 937869.6 | 316.558 |  | PT330 | SH | 668132.3 | 937766.4 | 319.052 |
| PT71 | SH | 668121.5 | 937862 | 316.817 |  | PT331 | SH | 668132.9 | 937763.9 | 319.098 |
| PT72 | SH | 668123.1 | 937858.4 | 316.857 |  | PT332 | SH | 668133.1 | 937760.8 | 318.995 |
| PT73 | SH | 668118.9 | 937856.2 | 316.91 |  | PT333 | SH | 668133.8 | 937757.1 | 318.991 |
| PT74 | BLD | 668120.4 | 937852 | 316.833 |  | PT334 | SH | 668134 | 937753.4 | 318.981 |
| PT75 | SH | 668116.8 | 937852.9 | 316.833 |  | PT335 | SH | 668134.5 | 937750.7 | 319.07 |
| PT76 | CV | 668119 | 937855.1 | 316.791 |  | PT336 | BLD | 668137.6 | 937750.4 | 319.035 |
| PT77 | CV | 668118.5 | 937855.2 | 316.819 |  | PT337 | SH | 668138.2 | 937752.8 | 319.024 |
| PT78 | SH | 668111.8 | 937851.5 | 317.396 |  | PT338 | BLD | 668140.6 | 937755.3 | 318.91 |
| PT79 | SH | 668115.7 | 937850.6 | 316.866 |  | PT339 | SH | 668137.7 | 937757.7 | 319.018 |
| PT80 | SH | 668118.3 | 937849 | 316.835 |  | PT340 | SH | 668136 | 937762 | 319.013 |
| PT81 | SH | 668118.1 | 937843.1 | 316.939 |  | PT341 | SH | 668138.3 | 937764.7 | 319.044 |
| PT82 | SH | 668113.8 | 937842.7 | 316.91 |  | PT342 | BLD | 668139.4 | 937765.7 | 319.051 |
| PT83 | SH | 668108.1 | 937843.3 | 317.4 |  | PT343 | SH | 668141.4 | 937749.2 | 319.111 |
| PT84 | SH | 668106.1 | 937847.2 | 317.641 |  | PT344 | SH | 668143.4 | 937751 | 319.05 |
| PT85 | SH | 668102.4 | 937849.2 | 317.563 |  | PT345 | SH | 668144.9 | 937753.8 | 318.948 |
| PT86 | SH | 668101.2 | 937843.6 | 317.586 |  | PT346 | SH | 668147.2 | 937751.6 | 318.691 |
| PT87 | SH | 668099.2 | 937840.3 | 317.489 |  | PT347 | SH | 668146.9 | 937747.9 | 319.135 |
| PT88 | SH | 668098.1 | 937836.4 | 317.051 |  | PT348 | SH | 668144.5 | 937745.9 | 319.098 |
| PT89 | SH | 668094.9 | 937829.9 | 317.405 |  | PT349 | SH | 668143.8 | 937741.8 | 319.252 |
| PT90 | SH | 668099 | 937830 | 317.366 |  | PT350 | BLD | 668137.8 | 937739.5 | 319.454 |
| PT91 | SH | 668102.2 | 937835.3 | 317 |  | PT351 | SH | 668140 | 937740.1 | 319.295 |
| PT92 | SH | 668093 | 937842.7 | 317.597 |  | PT352 | SH | 668141.4 | 937742 | 319.263 |
| PT93 | SH | 668090.5 | 937849.3 | 317.02 |  | PT353 | SH | 668145.1 | 937740.6 | 319.475 |
| PT94 | SH | 668088 | 937845.4 | 317.154 |  | PT354 | SH | 668147.6 | 937743.1 | 319.297 |
| PT95 | CV | 668089.1 | 937840.1 | 317.01 |  | PT355 | SH | 668149.8 | 937746.8 | 319.15 |
| PT96 | CV | 668089.5 | 937840.1 | 317.036 |  | PT356 | SH | 668151.4 | 937750.4 | 319.144 |
| PT97 | CV | 668089.7 | 937835.1 | 316.97 |  | PT357 | SH | 668153.9 | 937753.9 | 319.13 |
| PT98 | CV | 668089.1 | 937835.1 | 316.951 |  | PT358 | BLD | 668156.8 | 937753.8 | 319.16 |
| PT99 | SH | 668087.3 | 937837.4 | 317.032 |  | PT359 | BLD | 668155 | 937750.1 | 319.175 |
| PT100 | SH | 668084.9 | 937841.3 | 317.015 |  | PT360 | GT | 668156.7 | 937750.4 | 319.189 |
| PT101 | SH | 668081.1 | 937845.6 | 316.694 |  | PT361 | GT | 668156.4 | 937744 | 319.379 |
| PT102 | BLD | 668082.4 | 937849.5 | 316.774 |  | PT362 | BLD | 668151.3 | 937744.1 | 319.293 |
| PT103 | SH | 668079.2 | 937850.8 | 316.576 |  | PT363 | BLD | 668157.6 | 937720.9 | 319.595 |
| PT104 | SH | 668076.3 | 937851.1 | 316.606 |  | PT364 | BLD | 668075.3 | 937717.9 | 320.555 |
| PT105 | SH | 668076.4 | 937857.2 | 316.522 |  | PT365 | BLD | 668042.2 | 937720.6 | 320.353 |
| PT106 | SH | 668079.6 | 937859 | 316.481 |  | PT366 | BLD | 668036 | 937729.4 | 320.261 |
| PT107 | SH | 668082.3 | 937859.3 | 316.539 |  | PT367 | BLD | 668035.2 | 937744.9 | 319.623 |
| PT108 | SH | 668081.6 | 937865.3 | 316.444 |  | PT368 | BLD | 668011 | 937744.6 | 320.389 |
| PT109 | SH | 668078.7 | 937867 | 316.454 |  | PT369 | BLD | 668000.4 | 937749.6 | 319.55 |
| PT110 | SH | 668075.8 | 937866.3 | 316.351 |  | PT370 | BLD | 667993.6 | 937757.8 | 318.942 |
| PT111 | SH | 668074 | 937869.4 | 316.329 |  | PT371 | BLD | 667980.6 | 937772.8 | 318.59 |
| PT112 | SH | 668078.7 | 937870.4 | 316.547 |  | PT372 | BLD | 667974.3 | 937782.4 | 318.279 |
| PT113 | SH | 668081.1 | 937872.7 | 316.472 |  | PT373 | BLD | 667968.1 | 937789.6 | 318.03 |
| PT114 | SH | 668081.9 | 937877.2 | 316.469 |  | PT374 | RD | 667966.1 | 937793.6 | 317.734 |
| PT115 | SH | 668079 | 937877.3 | 316.472 |  | PT375 | CL | 667964.4 | 937795.8 | 317.839 |
| PT116 | SH | 668076.3 | 937876 | 316.604 |  | PT376 | RD | 667962.9 | 937797.2 | 317.658 |
| PT117 | EP | 668074.3 | 937876.1 | 316.594 |  | PT377 | SH | 667967.3 | 937798.1 | 317.599 |
| PT118 | TM | 668071.8 | 937879 | 316.342 |  | PT378 | SH | 667972.1 | 937795.5 | 317.821 |
| PT119 | TM | 668076.7 | 937878.8 | 316.445 |  | PT379 | SH | 667976.9 | 937798.1 | 317.559 |
| PT120 | TM | 668077.2 | 937885.8 | 316.536 |  | PT380 | SH | 667975.5 | 937802 | 317.229 |
| PT121 | SH | 668079.9 | 937885 | 316.362 |  | PT381 | SH | 667973.8 | 937805.3 | 317.111 |
| PT122 | SH | 668082.8 | 937883.4 | 316.48 |  | PT382 | SH | 667974.4 | 937809.3 | 317.087 |
| PT123 | SH | 668079.8 | 937880.6 | 316.437 |  | PT383 | SH | 667980.3 | 937808.6 | 317.008 |
| PT124 | SH | 668064.8 | 937896.9 | 315.429 |  | PT384 | SH | 667986.3 | 937808.3 | 317.055 |
| PT125 | BLD | 668064.8 | 937885.4 | 315.743 |  | PT385 | SH | 667989 | 937806.7 | 317.471 |
| PT126 | SH | 668069.7 | 937888.7 | 315.93 |  | PT386 | SH | 667991.4 | 937808.2 | 317.321 |
| PT127 | SH | 668069.8 | 937882.8 | 315.817 |  | PT387 | BLD | 667995.8 | 937809.1 | 317.181 |
| PT128 | EP | 668066.9 | 937881.4 | 316.367 |  | PT388 | BLD | 667996.6 | 937819.4 | 317.014 |
| PT129 | BLD | 668073.2 | 937873.9 | 316.28 |  | PT389 | BLD | 667997.1 | 937825.8 | 316.715 |
| PT130 | SH | 668076.5 | 937842.5 | 316.803 |  | PT390 | BLD | 667997.7 | 937836.1 | 316.436 |
| PT131 | SH | 668080.9 | 937837.3 | 316.882 |  | PT391 | SH | 667999.8 | 937837.9 | 316.489 |
| PT132 | SH | 668076.4 | 937833.4 | 317.117 |  | PT392 | SH | 668004.1 | 937837 | 316.697 |
| PT133 | SH | 668079.1 | 937829.3 | 317.088 |  | PT393 | SH | 668006.3 | 937835.7 | 316.836 |
| PT134 | SH | 668084.6 | 937829.3 | 317.095 |  | PT394 | SH | 668012.2 | 937839.5 | 317.101 |
| PT135 | SH | 668084.6 | 937825.2 | 317.166 |  | PT395 | BLD | 668024 | 937834.2 | 316.943 |
| PT136 | SH | 668080.9 | 937824.4 | 317.192 |  | PT396 | EP | 668028.4 | 937833.8 | 316.701 |
| PT137 | EP | 668075.2 | 937830.8 | 317.409 |  | PT397 | SH | 668027.3 | 937830.5 | 316.834 |
| PT138 | SH | 668076.5 | 937824.8 | 317.309 |  | PT398 | SH | 668024.8 | 937827.5 | 316.95 |
| PT139 | SH | 668080.1 | 937820.9 | 317.285 |  | PT399 | SH | 668027.4 | 937825.1 | 316.977 |
| PT140 | SH | 668083.9 | 937818.9 | 317.208 |  | PT400 | SH | 668025.7 | 937818 | 317.088 |
| PT141 | SH | 668084.3 | 937814.6 | 317.38 |  | PT401 | BLD | 668022.6 | 937817.1 | 317.238 |
| PT142 | SH | 668080.3 | 937812.9 | 317.483 |  | PT402 | SH | 668025.6 | 937813.3 | 317.258 |
| PT143 | SH | 668077 | 937811.8 | 317.722 |  | PT403 | SH | 668030.1 | 937809.1 | 317.516 |
| PT144 | EP | 668075.5 | 937812.1 | 317.622 |  | PT404 | BLD | 668031.1 | 937802.8 | 317.66 |
| PT145 | CV | 668075.9 | 937810.5 | 317.869 |  | PT405 | SH | 668027.5 | 937801.2 | 317.556 |
| PT146 | CV | 668076.6 | 937810.4 | 317.877 |  | PT406 | SH | 668031.4 | 937799.3 | 317.671 |
| PT147 | CV | 668076.1 | 937808 | 317.884 |  | PT407 | BLD | 668035 | 937801.7 | 318.075 |
| PT148 | CV | 668076.5 | 937808 | 317.896 |  | PT408 | SH | 668032 | 937800 | 317.586 |
| PT149 | BLD | 668073.4 | 937815.4 | 317.468 |  | PT409 | SH | 668031.4 | 937796 | 317.74 |
| PT150 | BLD | 668073.5 | 937803.4 | 317.824 |  | PT410 | SH | 668033.4 | 937793.9 | 317.725 |
| PT151 | SH | 668077.3 | 937804.7 | 317.737 |  | PT411 | BLD | 668035.2 | 937792.8 | 317.863 |
| PT152 | SH | 668080.4 | 937805.5 | 317.546 |  | PT412 | BLD | 668029.7 | 937792.2 | 318.048 |
| PT153 | SH | 668083.8 | 937807.7 | 317.514 |  | PT413 | BLD | 668029.5 | 937779.7 | 318.482 |
| PT154 | SH | 668084 | 937803.1 | 317.576 |  | PT414 | BLD | 668035.6 | 937839.2 | 316.76 |
| PT155 | SH | 668080.5 | 937801.9 | 317.645 |  | PT415 | BLD | 668030 | 937849.5 | 316.014 |
| PT156 | SH | 668076.8 | 937799.4 | 318.005 |  | PT416 | BLD | 668014.7 | 937849.8 | 315.771 |
| PT157 | SH | 668080.5 | 937797 | 317.729 |  | PT417 | RD | 668001.4 | 937850 | 315.553 |
| PT158 | SH | 668084.2 | 937798.5 | 317.584 |  | PT418 | RD | 667996.5 | 937852.3 | 315.646 |
| PT159 | SH | 668086.6 | 937795.5 | 317.625 |  | PT419 | BLD | 668015.4 | 937870.2 | 315.216 |
| PT160 | SH | 668088.9 | 937794.7 | 317.784 |  | PT420 | RD | 668022.4 | 937889 | 314.617 |
| PT161 | SH | 668087.8 | 937791.7 | 318.04 |  | PT421 | RD | 668020.3 | 937890.7 | 314.467 |
| PT162 | BLD | 668090.2 | 937790.5 | 318.057 |  | PT422 | BLD | 668035.6 | 937896.8 | 314.523 |
| PT163 | SH | 668086 | 937790 | 318.019 |  | PT423 | CV | 668032.5 | 937898.1 | 314.441 |
| PT164 | SH | 668079.9 | 937790.6 | 317.829 |  | PT424 | CV | 668032.1 | 937898.6 | 314.354 |
| PT165 | SH | 668076.8 | 937791.1 | 317.997 |  | PT425 | EP | 668026.8 | 937902.6 | 314.367 |
| PT166 | EP | 668075.9 | 937791.5 | 318.028 |  | PT426 | EP | 668042.4 | 937923.4 | 313.423 |
| PT167 | SH | 668081.3 | 937788.3 | 317.911 |  | PT427 | EP | 668045.7 | 937913.1 | 313.825 |
| PT168 | SH | 668086.2 | 937787 | 318.037 |  | PT428 | CV | 668045.7 | 937915.3 | 314.041 |
| PT169 | SH | 668088.3 | 937782.1 | 318.256 |  | PT429 | CV | 668045.2 | 937915.7 | 314.017 |
| PT170 | SH | 668083.1 | 937782.1 | 318.113 |  | PT430 | RD | 668050.4 | 937924.9 | 313.193 |
| PT171 | SH | 668077.2 | 937783.1 | 318.448 |  | PT431 | RD | 668048 | 937928.1 | 313.163 |
| PT172 | SH | 668077.1 | 937779.4 | 318.482 |  | PT432 | CL | 668049.6 | 937927.2 | 313.142 |
| PT173 | SH | 668080.8 | 937779.2 | 318.382 |  | PT433 | BLD | 668057.8 | 937924.9 | 313.443 |
| PT174 | SH | 668086.1 | 937777 | 318.527 |  | PT434 | BLD | 668139.5 | 937920.4 | 313.76 |
| PT175 | SH | 668088.4 | 937774.7 | 318.351 |  | PT435 | BD | 668156.3 | 937920.7 | 312.957 |
| PT176 | CV | 668076.7 | 937775.7 | 318.733 |  | PT436 | BLD | 668156.4 | 937893.6 | 315.143 |
| PT177 | CV | 668076.4 | 937775.7 | 318.755 |  | PT437 | BLD | 668120.5 | 937803.6 | 317.77 |
| PT178 | SH | 668075.6 | 937773.1 | 318.598 |  | PT438 | SH | 668121.8 | 937806.3 | 317.702 |
| PT179 | BLD | 668073.5 | 937768.6 | 318.708 |  | PT439 | BLD | 668120.3 | 937810 | 317.679 |
| PT180 | SH | 668073.1 | 937767.9 | 318.686 |  | PT440 | SH | 668124.2 | 937808.2 | 317.693 |
| PT181 | SH | 668075.1 | 937765.4 | 318.807 |  | PT441 | SH | 668124.2 | 937805.5 | 317.683 |
| PT182 | BLD | 668076.2 | 937763 | 318.99 |  | PT442 | SH | 668127.7 | 937808.3 | 317.623 |
| PT183 | SH | 668077.8 | 937762.3 | 318.92 |  | PT443 | SH | 668129.9 | 937805.3 | 317.555 |
| PT184 | SH | 668079.4 | 937758.5 | 319.226 |  | PT444 | SH | 668132.3 | 937808.2 | 317.557 |
| PT185 | SH | 668076.9 | 937758.6 | 318.98 |  | PT445 | SH | 668135.2 | 937805 | 317.569 |
| PT186 | SH | 668079.6 | 937754.5 | 319.302 |  | PT446 | SH | 668139.8 | 937808.7 | 317.523 |
| PT187 | SH | 668077.8 | 937753.6 | 319.218 |  | PT447 | SH | 668142.5 | 937806.5 | 317.477 |
| PT188 | SH | 668078.5 | 937746.8 | 319.408 |  | PT448 | SH | 668143.5 | 937805.6 | 317.641 |
| PT189 | SH | 668076.3 | 937744 | 319.51 |  | PT449 | BLD | 668152.8 | 937810.8 | 317.334 |
| PT190 | SH | 668079.7 | 937740.7 | 319.611 |  | PT450 | BLD | 668152.8 | 937803.9 | 317.388 |
| PT191 | SH | 668080.3 | 937743.2 | 319.569 |  | PT451 | BLD | 668120.4 | 937826.9 | 317.189 |
| PT192 | SH | 668078.8 | 937739.9 | 319.631 |  | PT452 | SH | 668123.3 | 937828 | 316.989 |
| PT193 | SH | 668078.8 | 937737.1 | 319.719 |  | PT453 | SH | 668126.8 | 937828.7 | 316.943 |
| PT194 | SH | 668081 | 937736.3 | 319.732 |  | PT454 | SH | 668126.3 | 937832 | 316.931 |
| PT195 | SH | 668078.1 | 937736.2 | 319.726 |  | PT455 | SH | 668123.9 | 937832.8 | 316.937 |
| PT196 | SH | 668074.3 | 937736.5 | 319.73 |  | PT456 | SH | 668121.8 | 937833.5 | 316.965 |
| PT197 | BLD | 668076 | 937739.9 | 319.718 |  | PT457 | SH | 668128 | 937833 | 316.904 |
| PT198 | SH | 668070.5 | 937735 | 319.725 |  | PT458 | SH | 668128.4 | 937836.4 | 316.889 |
| PT199 | SH | 668067.7 | 937732.6 | 319.769 |  | PT459 | SH | 668135.6 | 937832.3 | 316.921 |
| PT200 | SH | 668065.8 | 937735.6 | 319.673 |  | PT460 | SH | 668134.3 | 937834.4 | 316.926 |
| PT201 | SH | 668064.2 | 937739.6 | 319.579 |  | PT461 | SH | 668138.4 | 937829.2 | 316.911 |
| PT202 | SH | 668061 | 937736.3 | 319.632 |  | PT462 | SH | 668141.1 | 937835.5 | 316.628 |
| PT203 | SH | 668060.3 | 937732.6 | 319.705 |  | PT463 | SH | 668120.5 | 937838.7 | 317.002 |
| PT204 | SH | 668057.4 | 937735.8 | 319.652 |  | PT464 | BLD | 668073.3 | 937862.2 | 316.453 |
| PT205 | SH | 668056.3 | 937739.5 | 319.449 |  | PT465 | SH | 668072.4 | 937859 | 316.477 |
| PT206 | SH | 668052.5 | 937736.1 | 319.664 |  | PT466 | SH | 668069.5 | 937857 | 316.409 |
| PT207 | SH | 668050.9 | 937732.6 | 319.757 |  | PT467 | SH | 668065.8 | 937859.2 | 316.319 |
| PT208 | SH | 668048.6 | 937736.2 | 319.724 |  | PT468 | SH | 668064.3 | 937857.5 | 316.229 |
| PT209 | BLD | 668043.3 | 937739.9 | 319.659 |  | PT469 | SH | 668063.9 | 937853.2 | 316.483 |
| PT210 | BLD | 668043.6 | 937731.8 | 319.686 |  | PT470 | SH | 668055.7 | 937859.2 | 316.296 |
| PT211 | BLD | 668075.8 | 937729.8 | 319.966 |  | PT471 | SH | 668050.3 | 937854.7 | 316.365 |
| PT212 | BLD | 668079.9 | 937728.9 | 320.175 |  | PT472 | BLD | 668050.1 | 937852.9 | 316.654 |
| PT213 | BLD | 668079.7 | 937763.4 | 318.77 |  | PT473 | SH | 668056.8 | 937852.8 | 316.47 |
| PT214 | SH | 668080.8 | 937767.6 | 318.711 |  | PT474 | SH | 668063.8 | 937852.2 | 316.438 |
| PT215 | SH | 668083.6 | 937771.3 | 318.618 |  | PT475 | SH | 668067.5 | 937853.8 | 316.473 |
| PT216 | CV | 668088.4 | 937773.9 | 318.388 |  | PT476 | BLD | 668073.3 | 937851.7 | 316.674 |
| PT217 | CV | 668088.4 | 937774.1 | 318.401 |  | PT477 | BLD | 668073.4 | 937838.8 | 317.02 |
| PT218 | CV | 668088.9 | 937774.3 | 318.362 |  | PT478 | SH | 668071.2 | 937834.5 | 316.988 |
| PT219 | CV | 668089.2 | 937774.2 | 318.355 |  | PT479 | SH | 668071.7 | 937829.5 | 317.224 |
| PT220 | CV | 668089.7 | 937774 | 318.348 |  | PT480 | BLD | 668073.4 | 937827.2 | 317.205 |
| PT221 | CV | 668089.7 | 937773.6 | 318.358 |  | PT481 | SH | 668067.6 | 937828.1 | 317.111 |
| PT222 | SH | 668092.5 | 937776.3 | 318.551 |  | PT482 | SH | 668064.6 | 937832.8 | 317.034 |
| PT223 | SH | 668090.5 | 937779.5 | 318.338 |  | PT483 | SH | 668061.8 | 937837.4 | 316.978 |
| PT224 | SH | 668092.9 | 937780.6 | 318.233 |  | PT484 | SH | 668056.2 | 937836.7 | 316.98 |
| PT225 | SH | 668091.7 | 937784.1 | 318.136 |  | PT485 | SH | 668057.4 | 937831 | 317.048 |
| PT226 | SH | 668090.5 | 937787.6 | 318.117 |  | PT486 | SH | 668058.5 | 937827.6 | 317.15 |
| PT227 | SH | 668095.2 | 937788.3 | 318.215 |  | PT487 | SH | 668050.6 | 937828 | 316.962 |
| PT228 | SH | 668098.3 | 937788.8 | 318.206 |  | PT488 | SH | 668049.9 | 937833.6 | 316.97 |
| PT229 | SH | 668098.2 | 937786.1 | 318.204 |  | PT489 | SH | 668049 | 937837 | 316.957 |
| PT230 | SH | 668097.9 | 937782.4 | 318.229 |  | PT490 | BLD | 668043.9 | 937838 | 317.267 |
| PT231 | SH | 668097.4 | 937777.8 | 318.321 |  | PT491 | BLD | 668073.7 | 937792 | 318.073 |
| PT232 | SH | 668097.1 | 937773.1 | 318.529 |  | PT492 | SH | 668073.1 | 937784.7 | 318.318 |
| PT233 | SH | 668100.9 | 937773.3 | 318.58 |  | PT493 | BD | 668073.2 | 937780.3 | 318.439 |
| PT234 | SH | 668102.2 | 937777.1 | 318.319 |  | PT494 | SH | 668069.9 | 937784.4 | 318.355 |
| PT235 | SH | 668101.1 | 937781.3 | 318.285 |  | PT495 | SH | 668064.9 | 937790.3 | 318.076 |
| PT236 | SH | 668100.2 | 937785.2 | 318.239 |  | PT496 | SH | 668062.6 | 937784.4 | 318.262 |
| PT237 | SH | 668102.1 | 937788.4 | 318.071 |  | PT497 | SH | 668061.5 | 937781.1 | 318.253 |
| PT238 | BLD | 668103.3 | 937790.4 | 317.873 |  | PT498 | SH | 668054.5 | 937781.3 | 318.237 |
| PT239 | SH | 668105.1 | 937786.6 | 318.137 |  | PT499 | SH | 668053 | 937785.2 | 318.171 |
| PT240 | SH | 668105.9 | 937783.1 | 318.189 |  | PT500 | SH | 668051.4 | 937789.3 | 318.097 |
| PT241 | SH | 668106.2 | 937778.4 | 318.236 |  | PT501 | SH | 668045 | 937791.3 | 318.148 |
| PT242 | SH | 668106.3 | 937774.5 | 318.368 |  | PT502 | SH | 668044.7 | 937785.9 | 318.238 |
| PT243 | SH | 668106.4 | 937771.7 | 318.607 |  | PT503 | BD | 668043.5 | 937789.4 | 318.204 |
| PT244 | SH | 668109.8 | 937771.4 | 318.611 |  | PT504 | BD | 668046.8 | 937781.7 | 318.191 |
| PT245 | SH | 668114.7 | 937770.8 | 318.727 |  | PT505 | SH | 668050.5 | 937781.3 | 318.192 |
| PT246 | CV | 668118.6 | 937769.6 | 318.768 |  | PT506 | BLD | 668073.6 | 937768.4 | 318.52 |
| PT247 | CV | 668118.6 | 937769.2 | 318.875 |  | PT507 | SH | 668072.8 | 937765.9 | 318.772 |
| PT248 | BLD | 668120.4 | 937768.5 | 318.906 |  | PT508 | SH | 668068.1 | 937767.9 | 318.712 |
| PT249 | SH | 668118 | 937771.4 | 318.745 |  | PT509 | SH | 668064.3 | 937765.9 | 318.846 |
| PT250 | SH | 668114.6 | 937774.6 | 318.45 |  | PT510 | SH | 668061.8 | 937764.8 | 318.757 |
| PT251 | SH | 668112.3 | 937778.2 | 318.273 |  | PT511 | SH | 668059.2 | 937766.6 | 318.779 |
| PT252 | SH | 668116.9 | 937781 | 318.219 |  | PT512 | SH | 668056.8 | 937767.3 | 318.813 |
| PT253 | SH | 668114.7 | 937784 | 318.086 |  | PT513 | SH | 668055.6 | 937765.9 | 317.208 |
| PT254 | SH | 668110.9 | 937786.4 | 318.028 |  | PT514 | SH | 668054.7 | 937765.1 | 317.217 |
| PT255 | SH | 668108.3 | 937788.8 | 318.063 |  | PT515 | SH | 668050.4 | 937765 | 317.161 |
| PT256 | SH | 668106.5 | 937792.3 | 317.922 |  | PT516 | SH | 668048.2 | 937766.1 | 317.281 |
| PT257 | SH | 668104.3 | 937794.3 | 317.593 |  | PT517 | SH | 668047.6 | 937767.1 | 318.799 |
| PT258 | SH | 668106.6 | 937796.8 | 317.885 |  | PT518 | BLD | 668043.7 | 937764.7 | 322.083 |
| PT259 | SH | 668110.6 | 937794.3 | 317.866 |  |  |  |  |  |  |