KWARA STATE POLYTECHNIC, ILORIN
INSTITUTE OF ENVIRONMENTAL STUDIES
DEPARTMENT OF SURVEYING AND GEO-INFORMATICS

A PROJECT REPORT ON :

UTILITY SURVEY

OF

ROYAL VALLEY ESTATE

AT KUNLENDE/AKEREBIATA ROAD, ILORIN ILORIN EAST LOCAL GOVERNMENT AREA KWARA STATE

SUBMITTED BY:  MUHAMMED AISHAT

MATRICULATION: ND/23/SGI/FT/058

SUPERVISOR IN CHARGE: SURV. DIRAN FELIX & SURV. ADEBANJI

INSTITUTION: INSTITUTE OF ENVIRONMENTAL STUDIES

JULY, 2025

CERTIFICATE

I, MUHAMMED AISHAT with Matric Number ND/23/SGI/FT/058 hereby certify that the information contained in this project report were obtained as a result of observations and movements taken by me and the utility Survey  was done in accordance to Surveying rules and regulations and Departmental instructions.

Signature of student:  			___________________________

Name of student:				___________________________

Date of completion: 			___________________________

Matric Number: 				ND/23/SGI/FT/058

CERTIFICATION

This is to certify that MUHAMMED AISHAT with Matric number ND/23/SGI/FT/058 carried out this project work and has been approved as meeting the requirement for the award of National Diploma (ND) in Surveying and Geo-informatics in the Department of Surveying and Geo-informatics of the Institute of Environmental Studies, Kwara state polytechnic, Ilorin.

Project supervisor							_____________

SURV. DIRAN FELIX	 AND

SURV. ADEBANJI			                              Signature and Date

Project Cordinator 							_____________

SURV. AWOLEYE RAPHAEL. S

Signature and Date

Head of  Department 						_______________

SURV. AMBIMBOLA ISAU					Signature and Date

______________

External Moderator 						Signature and Date

SURV. J.O OPALEYE

...............................

DEDICATION

This project is dedicated to Almighty God, the creator of heaven and earth, ancient of days, the I am that I am the beginning and the end whose supremacy in the knowledge of everything is absolute, And to my supervisors Surveyor Diran Felix and Surveyor Banji, and  my noble HOD Surv. AMBIMBOLA ISAU and other lecturers.

ACKNOWLEDGMENT

I will like to express my profound appreciation to almighty God who in his infinite mercy gave me the ability to commence and complete my program in peace and God health, if not for God, where will I be today? All things are possible through him for the knowledge, wisdom and understanding and also his moral, protection and provision on me through my ND programme thank God for the great things he has done, most especially for the success of this project work God be the glory.

My appreciation goes to my lovely parent in person of  MR & MRS  MUHAMMED for their prayer and support, financially and morally throughout this journey thank for all the beautiful star and end through my ND program. I know I can't repay you back but I have special prayer to offer you both, wishing you both long life and prosperity in good health and wealth, more success, blessing and happiness along with a lot of luck , to my supervisors in the person of Surv. Below Diran Felix and Surveyor Adebanji thanks for all your support throughout the project.

And to all my lectures and in the department starting from  H.O.D Surv. Abinbola isau, Surv. A. Ayuba, Surv.  Bello Felix Diran,  Surv. Williams Kzeem, Surv. A.O. Akinyede, and also the Director of  special duty

Surv.  A.G. Aremu and other supportive staff of the department of  Surveying and  Geo-informatics, Kwara State Polytechnic, Ilorin.

ABSTRACT

This project is based on utility survey of  ROYAL VALLEY ESTATE

at kunlende/akerebiata road, ilorin ilorin east local government area kwara state

This project has been divided into different chapters.Chapter one of this project gives an introduction about the project topic as a whole.This enables better understanding of the project so as to know what perimeter and detailing entails as well as the scope and aim of the project.Chapter two is the literature review i.e. the works of past professionals and projects that had been done in the past as regards this specific topic which were examined in order to help to shed more light on what perimeter and detailing survey is all about. Chapter three of this project is the methodology which describes how the project was carried out using digital instrument e.g. Total Station and its accessories from the first stage to the final stage.Chapter four of this project is Data presentation which consists of all the data acquired from site during the project execution. Chapter five comprises of summary, problems encountered, conclusion and recommendations. Reference comprises of list ofnames and works of prominent authors or professionals whose works on survey as regards this specific topic were used in the course of this project.

TABLE OF CONTENTS

Title page										i

Certificate										ii

Certification										iii

Dedication										iv

Acknowledgement									v

Abstract										vi

Table of content 									vii

CHAPTER ONE

1.0	INTRODUCTION								1

1.1	BACKGROUND TO THE STUDY						1

1.2	STATEMENT OF THE PROBLEMS					2

1.3	AIM AND OBJECTIVES							3

1.4	AIM										3

  - OBJECTIVES									3

  - PROJECT SPECIFICATION							4

  - SCOPE OF THE PROJECT							4

  - PERSONAL INVOLVED							5

  - STUDY AREA								6

CHAPTER TWO

- LITERATURE REVIEW							6-10

CHAPTER THREE

- METHODOLOGY								11

  - RECONNAISSANCE

    - OFFICE PLANNING								11

    - FIELD RECONNAISSANCE						12

    - MONUMENTATION								13

  - EQUIPMENT USED								13

    - TEST OF INSTRUMENT							14

    - COLLIMATION TEST FOR TOTAL STATION				14

    - FORWARD COMPUTATION OF THE CONTROL USED		15

    - DATA ACQUISATION							16

    - GEOMETRIC DATA ACQUISATION					16

3.2.6	PERIMETER TRAVERSING						20-22

3.2.7	DETAIL SURVEY								23

CHAPTER FOUR

4.0 DATA PROCESSING								24

4.1.1 RESULT ANALYSIS								24

4.2 BACK COMPUTATION								24

4.2.1 COMPUTE FOR TOTAL AREA USING DOUBLE LATITUDE AND DEPARTURE				 						25-26

4.2.2 GRAPHICS PLOTTING							27

CHAPTER FIVE

- SUMMARY, CONCLUSION AND RECOMMENDATIONS 			28

  - SUMARRY 									28

  - PROBLEM ENCOUNTERED 							28

  - CONCLUSION									28

  - RECOMMENDATION								29

REFFERENCES

1.0 INTRODUCTION

A utility survey is a systematic process used to detect, locate, identify, and map underground utility infrastructure, such as water pipelines, gas mains, sewer lines, electrical conduits, fiber-optic cables, and other essential subsurface services. These surveys serve as a foundational component in modern construction, engineering design, and urban planning. The primary objective of a utility survey is to provide accurate, up-to-date information about existing underground utilities to support safe excavation, informed decision-making, and efficient infrastructure development.

As cities continue to grow and infrastructure becomes more complex, the need for precise utility data has become increasingly critical. Inaccurate or missing information about buried services can result in severe consequences, including service interruptions, costly project delays, environmental hazards, and even fatalities due to accidental strikes during excavation. Utility surveys help prevent such occurrences by delivering reliable data that guides planners, engineers, and contractors during construction and maintenance activities.

Traditionally, the documentation of subsurface utilities has been fragmented, often relying on outdated maps, inconsistent record-keeping, and anecdotal knowledge. However, with advancements in technology, modern utility surveys now incorporate high-precision tools such as Ground Penetrating Radar (GPR), electromagnetic locators, GPS/GIS systems, and even drone-assisted mapping to enhance detection accuracy and data visualization.

Moreover, utility surveys play a vital role in promoting sustainable development. By enabling informed planning and reducing the likelihood of infrastructure conflicts, these surveys contribute to the longevity and resilience of urban systems. They also facilitate inter-agency coordination, regulatory compliance, and public safety by ensuring that utility networks are properly documented, maintained, and accessible.

In light of these benefits and challenges, the study of utility surveying is essential to improving urban infrastructure practices. This research aims to explore the methodologies, technologies, and implications of utility surveys, highlighting their significance in shaping safe, efficient, and intelligent urban environments.

The significance of utility surveys extends beyond construction logistics—they are a vital tool in the lifecycle management of urban infrastructure. In the absence of reliable subsurface utility data, projects face heightened uncertainty, often leading to reactive measures that inflate costs and increase the risk of damage. According to the American Society of Civil Engineers (ASCE), inaccurate or incomplete utility information is one of the leading causes of construction delays and workplace accidents in developed and developing nations alike.

Modern urban environments are layered with increasingly dense and intertwined utility systems. As cities retrofit aging infrastructure and expand into previously undeveloped areas, the complexity of managing underground networks intensifies. In this context, utility surveys are not merely a procedural step, but a strategic necessity. They support a proactive approach to infrastructure development, allowing stakeholders to anticipate risks, optimize designs, and allocate resources more effectively.

Technological innovation has revolutionized the practice of utility surveying. Ground Penetrating Radar (GPR), electromagnetic induction, acoustic locating techniques, and GPS/GIS mapping have drastically improved detection precision and data presentation. These tools have enabled engineers and surveyors to produce three-dimensional utility maps that not only depict location and depth but also reveal the condition and material of buried assets. Additionally, the integration of Artificial Intelligence (AI) and cloud-based data platforms is opening new avenues for real-time analysis, predictive maintenance, and cross-organizational data sharing.

Beyond technology, utility surveys hold important implications for sustainable development and resilience. Accurate utility mapping helps reduce unnecessary excavation and material waste, thus lowering a project’s environmental impact. It also enhances a city’s capacity to respond to emergencies—such as utility leaks, electrical faults, or water main bursts—by providing immediate access to critical subsurface information.

Furthermore, utility surveys promote transparency and accountability in urban governance. As municipalities and utility providers strive to modernize infrastructure, the standardization of subsurface data collection and reporting ensures consistency across projects and jurisdictions. It fosters collaboration between public and private stakeholders, including engineers, planners, utility companies, and regulatory agencies.

STATEMENT OF THE PROBLEM

Despite their critical importance to urban infrastructure and development, utility surveys face several persistent challenges that limit their effectiveness. One of the most pressing issues is the lack of reliable, up-to-date data on the location and condition of underground utilities. This information gap often leads to accidental utility strikes during excavation activities, resulting in serious safety hazards, construction delays, and substantial financial losses.

Complicating matters further, existing utility records are frequently incomplete, outdated, or poorly maintained. In many cases, access to these records is restricted due to legal, bureaucratic, or institutional barriers. As urban environments become more complex and utility networks grow more intertwined, conventional surveying techniques are proving insufficient to meet current demands. The limitations of traditional methods are particularly evident in densely populated or rapidly developing areas, where infrastructure systems are layered, congested, and often undocumented.

These challenges highlight the need for more advanced and reliable utility surveying practices. There is a growing urgency to develop improved methodologies, promote better data-sharing protocols, and integrate cutting-edge technologies such as GIS, ground-penetrating radar (GPR), and AI-driven tools. This study seeks to address these critical gaps by exploring innovative solutions that enhance the accuracy, safety, and efficiency of utility surveys in urban infrastructure management.

AIM OF THE STUDY

The overarching aim of this study is to investigate and promote effective utility survey practices by examining modern technologies, methodologies, and collaborative frameworks. Specifically, the research seeks to contribute to safer and more efficient urban development by enhancing the ability to accurately detect, identify, and map subsurface utilities such as water mains, electrical lines, gas pipes, sewage systems, and telecommunications networks. Through this, the study aspires to reduce the incidence of utility strikes, support efficient project planning, and encourage sustainable urban growth.

OBJECTIVES OF THE STUDY

- To investigate and apply advanced utility detection techniques such as ground-penetrating radar (GPR), electromagnetic location tools, and GPS/GIS systems.

- To generate accurate and current data on underground utilities that informs design, planning, and construction processes.

- To minimize the risk of accidents, service disruptions, and financial losses caused by unintentional utility damage during excavation.

- To support informed decision-making in urban planning, emergency response, and infrastructure development through reliable utility data.

- To identify and analyze inconsistencies or gaps in existing utility records and propose methods to improve data quality.

- To foster improved coordination among engineers, contractors, utility providers, and planners by advocating standardized data-sharing frameworks.

PROJECT SPECIFICATION

This study will deliver a comprehensive review and evaluation of utility surveying practices, covering a blend of traditional methods and emerging technologies. The key focus areas include:

Exploration of theoretical frameworks and foundational concepts related to utility surveying.

Analysis of real-world case studies that demonstrate practical applications in various urban settings.

Review of technological innovations, including GIS integration, artificial intelligence, remote sensing, and drone-assisted surveys.

Assessment of the regulatory and institutional environment that governs utility surveying practices.

Formulation of recommendations and best practices aimed at improving the accuracy, safety, and coordination of utility surveys in construction and infrastructure projects.

SCOPE OF THE STUDY

This research will focus on the role of utility surveys within the context of urban infrastructure development. It will draw on literature and case studies from a range of geographic locations to assess how regional differences in regulation, technology, and infrastructure influence survey practices. Particular attention will be given to the integration of utility surveys in urban planning, construction risk management, and emergency response systems.

While the study centers on the technical aspects—such as methodologies, tools, and data systems—it will also acknowledge the broader socio-political and environmental contexts in which utility surveys are conducted. These contextual factors, although not the primary focus, are essential for understanding the challenges and opportunities in implementing effective utility survey strategies.

1.8	PERSONAL INVOLVED

| S/N | NAMES | MATRIC NUMBER | ROLES |
| --- | --- | --- | --- |
| 1 | MUHAMMED AISHAT | ND/23/SGI/FT/058 | AUTHOR |
| 2 | OLAFIMIHAN EMMANUEL . O | ND/23/SGI/FT/083 | MEMBER |
| 3 | MAFTAU OMOWUNMI AISHAT | ND/23/SGI/FT/054 | MEMBER |
| 4 | OLAWALE AZEEZ .O | ND/23/SGI/FT/055 | MEMBER |
| 5 | AWOLUSI TAIWO IREYEMI | ND/23/SGI/FT/066 | MEMBER |
| 6 | RAHMAN ZAINAB OLAMIDE | ND/23/SGI/FT/064 | MEMBER |
| 7 | BABALOLA OLAMIDE GRACE | ND/23/SGI/FT/062 | MEMBER |

1.9	STUDY AREA

The study area is located at royal Valley estate, At Kunlende/akerebiata road, ilorin ilorin East Local government area Kwara State

Map of Nigeria

Map of Ilorin.                           Map of study area

CHAPTER TWO

LITERATURE REVIEW ON UTILITY SURVEYS

Introduction

Utility surveys are essential in infrastructure planning, construction, and maintenance. These surveys help identify and map underground utilities such as water pipes, gas lines, electricity cables, telecommunications conduits, and sewage systems. In today’s complex urban environments, where utilities are densely packed and often poorly documented, accurate surveying is vital to prevent costly errors, delays, and potential hazards. This review explores the historical context, methodologies, technologies, applications, challenges, and future directions of utility surveys.

Methodologies in Utility Surveys

Various methodologies are used to detect, locate, and document underground utilities. These approaches have evolved from traditional visual inspections to sophisticated geophysical technologies.

Desktop Utility Records Review

This method involves reviewing historical utility records such as as-built drawings and GIS databases. While cost-effective, these records are often incomplete or outdated, necessitating on-site validation.

Visual Inspection and Site Reconnaissance

Physical site visits allow surveyors to observe visible infrastructure such as manholes, valves, and utility markers. However, this technique only reveals surface-level data.

Geophysical Survey Techniques

Modern surveys commonly rely on geophysical methods:

Ground Penetrating Radar (GPR): Uses radar pulses to detect changes in subsurface materials. GPR is effective for both metallic and non-metallic utilities but can be limited in high-clay or wet environments.

Electromagnetic Location (EML): Detects metallic pipes or cables by tracking electromagnetic signals. It is reliable for metallic assets but not suitable for plastic or ceramic pipes.

Other Geophysical Methods: Techniques such as magnetometry, seismic methods, and resistivity mapping are occasionally employed for specific applications or geological conditions.

Vacuum Excavation (Potholing)

This technique physically exposes the utility using air or water suction without causing damage. It offers high accuracy and is often used to verify geophysical survey results.

Historical Development of Utility Surveys

The concept of mapping utilities dates back to the early 20th century when urbanization accelerated the demand for coordinated underground infrastructure. Initially, utility placement was documented manually, often with limited precision. Early surveys relied heavily on human memory and handwritten maps, which led to frequent conflicts between different utility service providers.

As infrastructure networks grew in complexity, especially post-World War II, the need for systematic and standardized documentation became evident. The rise of urban planning in the 1960s and 1970s brought about increased awareness of the dangers posed by poorly mapped underground utilities. This period saw the adoption of photogrammetry, aerial mapping, and early geophysical methods.

In the 1990s and early 2000s, the integration of GPS, GIS, and early CAD systems revolutionized utility documentation and spatial analysis. This shift also marked the beginning of a move toward digitization, centralization of records, and the early stages of what would become smart infrastructure systems.

Role of Utility Surveys in Risk Management

One of the most critical aspects of utility surveys is risk mitigation. According to the Common Ground Alliance (CGA), utility strikes remain a significant risk on construction sites, leading to injuries, delays, legal liabilities, and costly repairs. Comprehensive surveys reduce these risks by:

Identifying conflict zones in design and planning stages.

Preventing service outages, especially for essential services like gas, electricity, and water.

Minimizing excavation errors, especially in congested or urbanized areas.

Supporting insurance claims and legal documentation in the event of accidents.

Additionally, survey data contributes to infrastructure asset management, allowing municipalities and private firms to better allocate resources, plan maintenance, and forecast long-term needs.

Ethical and Social Considerations

Utility surveying also encompasses ethical dimensions, particularly in how data is collected, shared, and used. Key considerations include:

Data Privacy: Utility data can reveal sensitive information about infrastructure, access points, and usage patterns. Improper handling or exposure of such data could pose security risks.

Public Safety: Surveyors have a responsibility to ensure their work minimizes risks to the public. This includes clear communication with local authorities and adherence to safety protocols during physical inspections.

Equity and Accessibility: In developing regions, access to modern surveying tools and accurate utility data is limited. This can perpetuate infrastructure inequalities and contribute to service disruptions or environmental hazards.

The ethical management of utility survey data is an emerging topic, with calls for standardized data governance frameworks and improved transparency in data collection practices.

Cross-Disciplinary Integration

Utility surveying increasingly intersects with other fields, making it a multi-disciplinary domain:

Civil Engineering: Uses utility survey data to design foundations, roadways, and public works while avoiding existing utilities.

Environmental Science: Survey data helps identify and mitigate risks associated with leaking pipelines or chemical spills.

Urban Planning: Accurate utility mapping supports sustainable development and future-proofing of cities.

Computer Science and Data Analytics: AI, machine learning, and data visualization tools enhance the interpretation of geospatial data and real-time monitoring.

This integration is leading to the emergence of “Digital Twin” cities — virtual replicas of real urban environments where utility networks are mapped, monitored, and managed digitally.

Global Perspectives and Case Studies

Utility surveying practices vary significantly across countries due to differences in regulations, infrastructure maturity, and technological access.

United Kingdom: The UK is a leader in utility survey standardization, with the PAS 128 standard widely adopted across public and private projects. Programs like the National Underground Asset Register (NUAR) aim to centralize utility data nationally.

United States: The ASCE 38-22 standard defines quality levels of utility data, supporting utility coordination and risk management. However, enforcement and adoption vary by state.

Developing Nations: In many parts of Africa, Asia, and South America, utility surveys are still largely manual, and records are sparse. However, mobile technologies, low-cost drones, and open-source GIS platforms are helping bridge the gap.

Case Study: Crossrail Project, London

The Crossrail railway project is a well-documented example of large-scale utility survey integration. The project faced thousands of utility conflicts, requiring advanced GPR, EML, and vacuum excavation technologies. A centralized GIS-based model helped manage data and reduce risks, saving millions in potential delays.

Educational and Workforce Implications

As utility surveying becomes more technology-driven, there is a growing need for a skilled workforce trained in both field techniques and digital tools. Key areas of focus include:

Technical Training: Surveyors must be proficient in GPR operation, GIS software, and CAD integration.

Interdisciplinary Curriculum: Universities and technical institutions are encouraged to incorporate utility surveying into civil engineering, urban planning, and data science programs.

Certification and Standards Compliance: Professionals should be educated on compliance with national and international standards, such as PAS 128, ASCE 38-22, and ISO guidelines.

The professionalization of utility surveying is critical to ensuring quality and safety across projects.

Recommendations for Future Research

Given the complexity and importance of utility surveys, several research areas warrant further exploration:

Automated Feature Extraction: Developing AI models to automatically interpret GPR or EML signals and identify utilities with minimal human input.

Subsurface 3D Modeling: Improving subsurface modeling techniques using multi-sensor data fusion and real-time visualization.

Utility Data Sharing Platforms: Creating open, interoperable databases for utility data sharing among stakeholders.

Impact of Climate Change: Studying how shifting water tables, soil erosion, and extreme weather affect underground utilities and survey methods.

Technologies and Innovations

Technology has significantly enhanced the accuracy, efficiency, and usability of utility surveys.

Geographic Information Systems (GIS)

GIS enables the integration and visualization of spatial utility data. It is widely used for planning, asset management, and decision-making in urban development projects.

Building Information Modeling (BIM)

BIM combines architectural, engineering, and utility data into a single 3D model. This integration aids in clash detection and planning for infrastructure projects.

Drones and UAVs

While not used directly for underground utility detection, drones help map terrain and identify surface features that correlate with buried infrastructure.

LiDAR and Photogrammetry

These technologies are used to create detailed 3D maps of the surface environment, which can be integrated with subsurface data to improve planning and coordination.

Applications of Utility Surveys

Utility surveys are critical across a variety of sectors and use cases.

Urban Planning and Construction

Proper utility mapping ensures efficient planning and helps avoid construction delays and safety hazards.

Transportation Projects

Highways, railways, and airport expansions require detailed surveys to locate and manage underground utilities before excavation begins.

Environmental and Risk Assessment

Surveying can help identify aging or leaking infrastructure that may pose environmental or health risks.

Emergency Services and Disaster Recovery

Accurate utility maps are essential for emergency responders to avoid electrical or gas hazards during rescue operations or natural disasters.

Challenges in Utility Surveys

Despite advancements, several challenges continue to affect the accuracy and reliability of utility surveys.

Data Inaccuracy and Gaps

Many utilities lack comprehensive or updated records. As-built drawings may be missing, incorrect, or inconsistent.

Technical Limitations

GPR and EML, though powerful, have limitations in specific soils, depths, and environments. Signal overlap and interference can reduce clarity.

Regulatory and Legal Issues

There is often no standardized requirement for utility data accuracy, making it difficult to ensure consistency across jurisdictions.

Cost and Resource Constraints

High-precision surveys, especially those requiring excavation or advanced modeling, can be time-consuming and expensive.

Standards and Best Practices

To improve quality and reliability, various standards and protocols have been developed:

PAS 128 (UK): Specifies standards for detection, verification, and location of underground utilities.

ASCE 38-22 (USA): Classifies utility information by quality levels, from record-based data to physically exposed utilities.

ISO Standards: ISO 19439 and ISO 55000 provide frameworks for asset and utility management across infrastructure sectors.

These guidelines promote better project coordination, data integrity, and safety.

FUTURE DIRECTIONS

The future of utility surveying lies in deeper integration with smart technologies and more efficient data systems.

Artificial Intelligence (AI) and Machine Learning

AI can enhance geophysical data interpretation, helping detect patterns and automate decision-making processes.

Augmented Reality (AR)

AR systems may allow surveyors to visualize underground utilities in real time using smart devices, improving on-site decision-making.

Blockchain for Data Security

Blockchain technology could provide secure, traceable records of utility data, ensuring tamper-proof historical documentation.

Smart Utility Networks and IoT

Embedded sensors in pipelines and cables can transmit real-time location and status data, reducing the need for repeated surveys and improving infrastructure monitoring.

CHAPTER THREE

3.0 	METHODOLOGY

This aspect refers to a system of methods and principles, rules used in a field of study to achieve the aim and objectives of this project. The modality to successful execute this project was carried out based on the survey rules and regulation, supervisor advice and departmental instructions.

3.1 	RECONNAISSANCE

Reconnaissance survey is a preliminary stage for exploratory investigations conducted to gather initial data concern the choice of Site area of Subject before more detailed research or filed work commenced. Reasons for reconnaissance survey of the project area are the following;

- To know the location of the site.

- To determine the preliminary area/perimeter of the land.

- To know the survey method suitable for the field data acquisition and the instrument need for the it.

Reconnaissance in this project involved preliminary investigations to gather information about the survey area before conducting a detailed survey, reconnaissance of this project was carried out in two way namely as follows.

- Office planning

- Field Reconnaissance

3.1.1 	OFFICE PLANNING

It involved the process adopted in the collection of relevant data and specification about the as-built Surveying with the used of traversing survey method. During the office planning the given coordinate for control point PBIL 100, PBIL 107, and PBIL 110, was obtained and computed as shown in the table 3.1 below. The choice of equipment to be use for project was obtained from the survey store in the department of surveying and Geoinformatic, Kwara State Polytechnic, Ilorin, and it was handled accordingly to the survey rules and regulations and departmental instruction.

TABLE 3.1 Showing the list of the coordinate of the control pillars used

| Stations | Northing (m) | Easting (m) |
| --- | --- | --- |
| PBIL 100 | 939353.130 | 648821.110 |
| PBIL 107 | 939306.460 | 648932.560 |
| PBIL 110 | 939224.969 | 649120.403 |

3.1.2 	FIELD RECONNAISSANCE

Field recce and site visitation was conducted as preliminary survey for investigation of the basic information about a specified project area, location, or subject in the field. It allows the members of the groups to be familiar with the terrain, boundary lines, selection number of station required, Equipment that will be used for the job, pillars number PBIL 100, PBIL 107, and PBIL 110, used as a control for the project and so on. The stations points selected was Inter-visible to one another and finally a reconnaissance diagram was produced which was attached to this project

RECCE DIAGRAM

3.2	INSTRUMENT SELECTION

The equipment used for this project was grouped into two categories;

- Hardware

- Software

3.2.1	Hardware used

- Total Station with its Accessories (KTS-442L)

- Reflector poles

- Field book

- Writing material (pen)

- Survey beacons

- Steel Tape (5m)

- Tripod Stand

3.2.2	Software used

Below are the list of the software used during the project processing;

- AutoCAD 2007

- Notepad

- Microsoft word

- Microsoft Excel

3.3 	INSTRUMENT TEST

Total station

LINE OF SIGHT

RANGING POLE

20M

STATION  X

TERRAIN

STATION  Y

The instrument test was carried out before it used for angular observation. The purpose of testing the instrument before used is to know the assurance and maybe other instrument is in good Condition the instrument was set up over station A and it was pointed a Sighted to station B whore a pole Is hold, and the angle was recorded for face left the instrument was turn to farce right on the same point and it was sighted to station B, where a pole is held at the same position and the angle was recorded. The distance between station A and B 1s 10m. The steel tape was hold at 60m an point A and it was extend to point B. the results for the instrument test was shown i to figure 3.3 below and table 3:2 below.

Figure 3.1 showing the instrument set up for collimation test

Table 3.2 showing the test of instrument result

| STATION | SIGHT | FACE | HORIZONTAL READING | DEDUCED ANGLE |
| --- | --- | --- | --- | --- |
| A | B | L | 180° 00’ 03” |  |
|  | B | R | 00°00’ 00” | 180° 00’ 03” |

Different in horizontal circle reading: 180° 00’ 03’’

Horizontal collimation error

= 180° 00’ 03’’

= 000° 00 03’’

Collimation error = 00° 00'03"2

Horizontal collimation error = 00° 00’ 01”

Because the collimation  error falls within the allowable 01”. The instrument was in good condition.

3.4 	MONUMENTATION

This is the process in which the beacon is being buried in to ground. On the project site there are same existing  beacons on the boundary lines m, nail with cap (bottle cork) was used as the 	station pegs.

18cm

15cm above the ground

60cm below the ground

18cm

30cm

Nail

Bottle cork

PILLAR							   PEG

FIGURE 3.4	Diagram and Dimension of Pillars buried

3.5 	CONTROL CHECK

The Control check was carried out to know the control point PBIL 100, PBIL 107, and PBIL 110, are still in their positions. The instrument was set on Control FP01107 and the temporary adjustment was carried out i.e centring, levelling, and focusing. Back site was taken to control PBIL 107 and it was recorded.

The fore sight was served to PEG1 and it was recorded Steel tape was used to measure the distance between PBIL 110 and PEGI and it was recorded. The data observed was computed for given coordinate and Observed Coordinate, as shown in the table 3.2 below.

TABLE 3.2	THE GIVEN COORDINATE OF THE CONTROL POINTS

| STATION | NORTHING (m) | EASTING (m) |
| --- | --- | --- |
| PBI 100 | 939353.130 | 648821.110 |
| PBIL 107 | 939306.460 | 648932.560 |
| PBIL 110 | 939224.969 | 649120.403 |

TABLE 3.3	The Backward Computation for the Given Controls pillars

| From station | Bearing | Distance | ∆N | ∆E | Northing (m) | Easting (m) | To station |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | 899353.130 | 688821.110 | PBIL 100 |
| PBIL 100 | 1120 43̍ 17 ̎ | 120.827 | -46.670 | 111.450 | 899306.460 | 688932.560 | PBIL 107 |
| PBIL 107 | 1130 27̍ 09 ̎ | 204.758 | -81.491 | 187.843 | 899224.969 | 689120.403 | PBIL 110 |

PBIL 100 to PBIL 107

BEARING = 1120 43̍ 17 ̎

DISTANCE = 120.827m

PBIL 107 to PBIL 110

BEARING = 1130 27̍ 09 ̎

DISTANCE = 204.758m

TABLE 3.4	Show the Observed Coordinate of the Controls Pillars

| Station | Northing (m) | Easting (m) |
| --- | --- | --- |
| PBIL 100 | 939353.126 | 648821.105 |
| PBIL 107 | 939306.460 | 648932.560 |
| PBIL 110 | 939224.969 | 649120.403 |

TABLE 3.5	Show the Backward Computation for the Observed Control Pillars

| From station | Bearing | Distance | ∆N | ∆E | Northing (m) | Easting (m) | To station |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | 899353.126 | 688821.105 | PBIL 100 |
| PBIL 100 | 1120 43̍ 08 ̎ | 120.830 | -46.666 | 111.455 | 899306.460 | 688932.560 | PBIL 107 |
| PBIL 107 | 1130 27̍ 09 ̎ | 204.758 | -81.491 | 187.843 | 899224.969 | 689120.403 | PBIL 110 |

PBIL 100 to PBIL 107

BEARING = 1120 43̍ 08 ̎

DISTANCE = 120.830m

PBIL 107 to PBIL 110

BEARING = 1130 27̍ 09 ̎

DISTANCE = 204.758m

TABLE 3.6	Show the Difference between the Given Control Points and the Observed Control Pillars

| STATION | NORTHING (m) | EASTING (m) |
| --- | --- | --- |
| PBIL 100 | 0.004 | 0.005 |
| PBIL 107 | 0.000 | 0.000 |
| PBIL 110 | 0.000 | 0.000 |

Therefore the difference between the given and observer bearing was 00° 00’ 09” and 00° 00’ 00” and the difference between the given and the observer linear distance was 0.0003m, 0.000m the results obtained fall within the allowable accuracy for both angular and linear observation.

It was concluded the instrument was in good condition and fit to be used for the project.

3.6 	DATA AQUISITION

This was carried out on the field using Total station with its accessories (KTS-442L) was used for coordinating the entire site for both the perimeter and details of the site. The Total station was set on the 	control Pillar PBIL 107, and the temporary adjustment was carried out centering, leveling and 	focusing. The coordinates of the occupied station was input after that the coordinates of the backsight was record as well and the observation was done to backsight pillar and the coordinates was take and check against the given. Having check and found accurate then the perimeter survey commenced for the entire project site.

3.6.1 	DETAILING SURVEY

The detailing survey was done immediately after completely carried out the perimeter traverse to determine the position of the features on the Site was fixed using radiation survey method. During this several stations was established to pick the coordinates of the details and the coordinates was stored on the total station as its observed. This was later downloaded. The details that was picked during the detailing survey was coded in different ways like Building edges as ‘BLD’, Electric Poles ‘EP’, Drainage ‘DR’, Roads Line ‘RD’, Spot Height ‘SP’ and so on.

CHAPTER FOUR

4.0   	DATA PROCESSING

This involved several step to ensure accurate and reliable result. It explains the method used in capturing; processing, downloading, and analyzing the data acquired Data reduction in surveying involves various methods to simplify and summarize the collected data for analysis. These are some common methods of data reduction that involves

- Combining multiple values into a single value. The average value is calculated by summing up all the values and dividing by the number of values.

- Least Square Adjustment (LSA) is a method of data reduction that involves minimizing the sum of the squared errors between observed and calculated values.

- Interpolation is a method of data reduction that involves estimating values between observed data points.

- Spatial data reduction refers to the process of simplifying and reducing the complexity of spatial data, such as geographic information system (GIS) data, while preserving its essential characteristics.

4.1	DATA EDITING

- Open excel

- Click on the desire file

- Set to delimited

- Click on next

- Mack space and comma

- Click

- On next

- Then click on finish

4.3	DATA PROCESSING USING AUTOCAD

The plan produced digitally with AutoCAD software. The following are the procedure used:

STEP 1: Open Notepad

Open Notepad on your computer. You can find it in the start menu (Window)

STEP 2: Create a New File

Click on "file" "New" to create a new file

STEP3: Enter Coordinates

Enter your coordinates in the following format:

X. Y. Data

STEP 4: Save the Fil

Click on "File" > "Save as" and choose a location to save your file. "ALADA Coordinate" txt, 	and select "All files" as the file type.

Importing the Coordinate File Into 2007 AutoCAD

STEP 1: Double click AutoCAD

Go under format

Click on format after that pick your unit

Drawing unit

Type.... Decimal

Precision... 0.000

Angle type Degree/Min/See

Precision Od00

Clockwise.... 노

Unit to scale Meters

Direction control North 270d0

Press "OK"

STEP 2: Create a new drawing

Click on "File" > "Name" to create a new drawing

STEP 3: Import the coordinate file

Click on "Tools" >"Reference" > "Import" and select "ALADA coordinate (.txt)" as the file 	type. Navigate to the location where you saved your coordinate file ("ALADA coordinate .txt") 	and select it.

STEP 4: Configure Import Setting. In the import select "Point" as the object type and choose 	the correct coordinate format (eg. X and Y) click "OK" to import the coordinates.

4.4	DATA EXTRACTION

The data recorded in the field book was picked to compute for the bearing and distance. Forward computation and backward computation after all necessary correction have been applied. Correction like slope correction, tension correction and standardization correction.

4.3  	BACK COMPUTATION

The result get for forward computation was used for Back computation.

The following program was required for back computation

i. Input N2-N1 to get ∆N

ii. Input E2-E1 to get ∆E

iii.Input (∆N2+∆E2 to get distance

iv.Input shift tan ∆E∆Nto get the corrected bearing when there us more station repeat step ¡¡¡ and ii.

Table 4.1 Showing Boundary Coordinates

| Station | Northing (m) | Easting(m) |
| --- | --- | --- |
| FPO1107 | 939366.406 | 648932.560 |
| PL1 | 939240.312 | 648901.750 |
| PL2 | 939309.331 | 648780.785 |
| PL3 | 939292.197 | 648603.139 |
| PL4 | 939438.718 | 648619.716 |
| PL5 | 939467.847 | 648693.558 |
| PL 6 | 939353.126 | 648821.105 |

Table 4.1.1 Showing back computation for the boundary coordinate

| FROM Station | BEARING | DISTANCE | ΔN | ΔE | N |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | 89 |
| PBIL 107 | 204 58 29 | 72.971 | -66.148 | 30.810 |  |
| PL1 | 299 42 28 | 139.270 | 69.019 | 120.965 |  |
| PL2 | 295 39 02 | 193.739 | 83.866 | 174.646 |  |
| PL3 | 16 40 37 | 47.520 | 45.521 | 13.637 |  |
| PL4 | 112 42 17 | 79.980 | 30.871 | 73.782 |  |
| PL5 | 113 13 14 | 113.790 | 54.721 | 127.547 |  |
| PL6 | 112 43 59 | 120.870 | 48.666 | 455 |  |

TABLE 4.1.2	showing area computation for the boundary coordination using double latitude

| ΔN | ΔE | product |
| --- | --- | --- |
| 66.148 | × - 30.810 | = 2038.020 |
| 66.148 |  |  |
| 69.019 |  |  |
| -63.277 | × - 120.965 | =7654.302 |
| 69.619 |  |  |
| 83.866 |  |  |
| 89.608 | × -174.646 | -=15649.679 |
| 83.866 |  |  |
| 45.521 |  |  |
| 218.995 | × 13.637 | =2986.435 |
| 45.521 |  |  |
| -30.871 |  |  |
| 233.645 | × 73.782 | =17238.792 |
| -30.871 |  |  |
| -54.721 |  |  |
| 148.653 | × 127.547 | =18883.716 |
| -54.721 |  |  |
| -46.666 |  |  |
| 46.666 | × 111.455 | =5201.159 |
| -46.666 |  |  |
| 00.000 |  |  |

Area =

2038.020+7654.302-15649.679+2986.435+17238.795+18883.716+5201.1592

54002.427-18649.6792

38352.7482

=19176.374=> 1.9 hectares

1.9 Hect meter square

Hectare=19176.37410000

= 1.9176374 Hectare

Linear Accuracy = 1(EL)2-((ED)2TOTALLENGHT

EL=EDN=0.066

ED=EDE=0.072

Total length =793.100

∴LinearAccuracy

1(0.66)2+(0.072)2793.100x = 10.09767793.100

=1:8000

CHAPTER FIVE

5.0 	SUMMARY, PROBLEMS ENCOUNTERRED, CONCLUSION, RECOMMENDATION

5.1 	SUMMARY

5.2 	PROBLEM ENCOUNTERED

- Too much of rainfall which does not allowed site

- Unavailable of the group members due to dean's cup football tournament

- While taking observing there are some object obstructing the view when taking the observation e.g electric pole, building, tree etc.

5.3 	RECOMMENDATION

The department should considered the season and time that the students should be given the terminal project.

5.4 	CONCLUSION

At the end the aim and objective of this project was achieved. It was carried out using third order surveying method this project was carried out in accordance with survey rules and regulation and departmental instruction.

| Name | Code | E | N | h |  | PT260 | SH | 668115.4 | 937791.5 | 317.946 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PT1 | ST17 | 668069.6 | 937900.4 | 324.137 |  | PT261 | SH | 668118 | 937790.7 | 318.041 |
| PT2 | CV | 668071.1 | 937900.4 | 324.553 |  | PT262 | SH | 668117.7 | 937795.6 | 317.967 |
| PT3 | CV | 668072 | 937900.3 | 324.541 |  | PT263 | SH | 668113.5 | 937797.2 | 317.656 |
| PT4 | BLD | 668072.6 | 937900.6 | 324.383 |  | PT264 | SH | 668110.1 | 937800.8 | 317.647 |
| PT5 | BLD | 668066.5 | 937905.8 | 323.724 |  | PT265 | SH | 668110.4 | 937806.9 | 317.571 |
| PT6 | SH | 668069.7 | 937897.9 | 324 |  | PT266 | SH | 668114.1 | 937808.4 | 317.326 |
| PT7 | SH | 668072.6 | 937896.1 | 324.671 |  | PT267 | SH | 668117.8 | 937810 | 317.601 |
| PT8 | SH | 668072.2 | 937891.5 | 324.604 |  | PT268 | SH | 668117.6 | 937814.7 | 317.532 |
| PT9 | SH | 668076.5 | 937892.6 | 324.821 |  | PT269 | SH | 668113.8 | 937816 | 317.212 |
| PT10 | SH | 668083.2 | 937892.3 | 324.831 |  | PT270 | SH | 668109 | 937815.1 | 317.485 |
| PT11 | SH | 668085.6 | 937889.4 | 324.87 |  | PT271 | SH | 668104.5 | 937814.4 | 317.27 |
| PT12 | BLD | 668085.4 | 937886.8 | 324.848 |  | PT272 | SH | 668104.8 | 937819.5 | 317.197 |
| PT13 | SH | 668088.7 | 937888.5 | 324.887 |  | PT273 | SH | 668109.7 | 937818.5 | 317.278 |
| PT14 | SH | 668090.4 | 937893.3 | 324.882 |  | PT274 | SH | 668113.6 | 937819 | 317.192 |
| PT15 | SH | 668095.3 | 937895.1 | 324.957 |  | PT275 | SH | 668117.7 | 937821.2 | 317.334 |
| PT16 | SH | 668095.6 | 937891.4 | 324.958 |  | PT276 | SH | 668114.1 | 937823.2 | 317.152 |
| PT17 | BLD | 668139.4 | 937901.3 | 324.586 |  | PT277 | SH | 668109 | 937824.1 | 317.181 |
| PT18 | CV | 668139.1 | 937901.2 | 316.047 |  | PT278 | SH | 668104.9 | 937822.2 | 317.161 |
| PT19 | CV | 668139.5 | 937900.4 | 316.104 |  | PT279 | SH | 668105 | 937825.4 | 317.107 |
| PT20 | CV | 668137.3 | 937899 | 317.048 |  | PT280 | SH | 668103.5 | 937827.1 | 317.276 |
| PT21 | SH | 668133.7 | 937894.7 | 316.106 |  | PT281 | SH | 668108.1 | 937827.2 | 317.179 |
| PT22 | SH | 668136.7 | 937894.8 | 316.021 |  | PT282 | SH | 668112 | 937827.1 | 317.097 |
| PT23 | BLD | 668137 | 937893.5 | 316.015 |  | PT283 | SH | 668116.6 | 937827.5 | 317.116 |
| PT24 | SH | 668141.1 | 937890.4 | 315.886 |  | PT284 | SH | 668117.8 | 937831.4 | 317.048 |
| PT25 | SH | 668145.9 | 937889.2 | 315.599 |  | PT285 | SH | 668112.9 | 937833.2 | 317.012 |
| PT26 | SH | 668147.8 | 937891.6 | 315.619 |  | PT286 | SH | 668108.8 | 937832.7 | 317.058 |
| PT27 | GT | 668148.7 | 937893.4 | 315.804 |  | PT287 | SH | 668106.3 | 937831.7 | 317.095 |
| PT28 | GT | 668147.9 | 937894 | 315.765 |  | PT288 | SH | 668120.4 | 937780 | 318.343 |
| PT29 | BLD | 668148.9 | 937893.2 | 315.928 |  | PT289 | SH | 668125.9 | 937783 | 318.036 |
| PT30 | SH | 668148.1 | 937886.3 | 315.852 |  | PT290 | SH | 668122.6 | 937785.4 | 318.043 |
| PT31 | SH | 668144.4 | 937882.9 | 315.942 |  | PT291 | SH | 668120.9 | 937789.4 | 318.013 |
| PT32 | SH | 668140.3 | 937879.1 | 316.287 |  | PT292 | BLD | 668120.6 | 937791.6 | 317.908 |
| PT33 | SH | 668144.4 | 937877.2 | 316.236 |  | PT293 | SH | 668125 | 937790.3 | 317.853 |
| PT34 | SH | 668147.4 | 937877.9 | 316.065 |  | PT294 | SH | 668128.5 | 937785.7 | 317.944 |
| PT35 | BLD | 668149.4 | 937879.9 | 315.956 |  | PT295 | SH | 668133 | 937781.5 | 317.952 |
| PT36 | BLD | 668156.1 | 937879.6 | 315.966 |  | PT296 | SH | 668132.9 | 937786.5 | 317.857 |
| PT37 | BLD | 668156 | 937874.1 | 315.966 |  | PT297 | SH | 668132.2 | 937790.7 | 317.807 |
| PT38 | EP | 668150.9 | 937875.2 | 316.093 |  | PT298 | SH | 668137 | 937790.8 | 317.772 |
| PT39 | SH | 668152 | 937878.1 | 315.912 |  | PT299 | SH | 668139.7 | 937786.5 | 317.824 |
| PT40 | BLD | 668146.3 | 937874.1 | 316.297 |  | PT300 | SH | 668140 | 937781.4 | 318.046 |
| PT41 | SH | 668132 | 937896.8 | 316.129 |  | PT301 | SH | 668144.3 | 937781.2 | 318.023 |
| PT42 | BLD | 668128.4 | 937894.7 | 316.378 |  | PT302 | SH | 668145.5 | 937786.4 | 317.727 |
| PT43 | SH | 668124.8 | 937894.3 | 316.451 |  | PT303 | SH | 668146 | 937791.2 | 317.717 |
| PT44 | SH | 668121.3 | 937892 | 316.499 |  | PT304 | SH | 668149.5 | 937790.8 | 317.75 |
| PT45 | SH | 668118.3 | 937894.3 | 316.594 |  | PT305 | SH | 668149.7 | 937786.7 | 317.692 |
| PT46 | SH | 668115.7 | 937888.6 | 316.573 |  | PT306 | SH | 668148.9 | 937781.3 | 317.842 |
| PT47 | BLD | 668113.6 | 937886.9 | 316.529 |  | PT307 | BLD | 668150.4 | 937781.4 | 317.8 |
| PT48 | SH | 668110.4 | 937889.4 | 316.736 |  | PT308 | BLD | 668150 | 937791.5 | 317.803 |
| PT49 | SH | 668108.3 | 937891.8 | 316.596 |  | PT309 | SH | 668113.7 | 937768.9 | 318.789 |
| PT50 | SH | 668104.4 | 937892.2 | 316.593 |  | PT310 | SH | 668113.6 | 937766 | 318.901 |
| PT51 | SH | 668104.2 | 937889.1 | 316.626 |  | PT311 | BLD | 668112.8 | 937763.3 | 319.05 |
| PT52 | SH | 668117.5 | 937886.8 | 316.551 |  | PT312 | SH | 668117.2 | 937761.4 | 318.943 |
| PT53 | SH | 668119.7 | 937883.9 | 316.541 |  | PT313 | SH | 668117.5 | 937764.3 | 318.911 |
| PT54 | SH | 668117.1 | 937883.2 | 316.562 |  | PT314 | SH | 668122.4 | 937765.5 | 318.977 |
| PT55 | SH | 668114.2 | 937882.6 | 316.592 |  | PT315 | SH | 668124.5 | 937762.3 | 318.933 |
| PT56 | BLD | 668113.5 | 937874.4 | 316.659 |  | PT316 | SH | 668122.4 | 937758.6 | 318.969 |
| PT57 | SH | 668115.5 | 937874 | 316.641 |  | PT317 | SH | 668121.7 | 937753.2 | 319.012 |
| PT58 | SH | 668117.6 | 937873.9 | 316.666 |  | PT318 | SH | 668119.4 | 937752.3 | 319.022 |
| PT59 | SH | 668116.9 | 937866.3 | 316.759 |  | PT319 | BLD | 668113 | 937752.1 | 319.283 |
| PT60 | SH | 668114.9 | 937865.7 | 316.81 |  | PT320 | BLD | 668113.1 | 937748.7 | 319.275 |
| PT61 | SH | 668112.2 | 937865.8 | 317.066 |  | PT321 | BLD | 668113.2 | 937730.1 | 319.819 |
| PT62 | BLD | 668111.7 | 937870.2 | 316.664 |  | PT322 | BLD | 668118.1 | 937726.7 | 320.172 |
| PT63 | SH | 668124 | 937868.3 | 316.542 |  | PT323 | BLD | 668118.2 | 937750.2 | 319.117 |
| PT64 | BLD | 668129.2 | 937867.4 | 316.594 |  | PT324 | SH | 668122.5 | 937751.6 | 319.006 |
| PT65 | SH | 668128.1 | 937869.8 | 316.579 |  | PT325 | SH | 668124.5 | 937754.3 | 318.972 |
| PT66 | SH | 668127.9 | 937872.5 | 316.566 |  | PT326 | SH | 668125.4 | 937757 | 318.942 |
| PT67 | SH | 668125 | 937873.8 | 316.569 |  | PT327 | SH | 668126.4 | 937759.8 | 318.902 |
| PT68 | SH | 668123.6 | 937877 | 316.822 |  | PT328 | SH | 668127.7 | 937763.6 | 318.945 |
| PT69 | SH | 668126 | 937879.4 | 316.682 |  | PT329 | SH | 668129.6 | 937766.7 | 319.003 |
| PT70 | SH | 668121.7 | 937869.6 | 316.558 |  | PT330 | SH | 668132.3 | 937766.4 | 319.052 |
| PT71 | SH | 668121.5 | 937862 | 316.817 |  | PT331 | SH | 668132.9 | 937763.9 | 319.098 |
| PT72 | SH | 668123.1 | 937858.4 | 316.857 |  | PT332 | SH | 668133.1 | 937760.8 | 318.995 |
| PT73 | SH | 668118.9 | 937856.2 | 316.91 |  | PT333 | SH | 668133.8 | 937757.1 | 318.991 |
| PT74 | BLD | 668120.4 | 937852 | 316.833 |  | PT334 | SH | 668134 | 937753.4 | 318.981 |
| PT75 | SH | 668116.8 | 937852.9 | 316.833 |  | PT335 | SH | 668134.5 | 937750.7 | 319.07 |
| PT76 | CV | 668119 | 937855.1 | 316.791 |  | PT336 | BLD | 668137.6 | 937750.4 | 319.035 |
| PT77 | CV | 668118.5 | 937855.2 | 316.819 |  | PT337 | SH | 668138.2 | 937752.8 | 319.024 |
| PT78 | SH | 668111.8 | 937851.5 | 317.396 |  | PT338 | BLD | 668140.6 | 937755.3 | 318.91 |
| PT79 | SH | 668115.7 | 937850.6 | 316.866 |  | PT339 | SH | 668137.7 | 937757.7 | 319.018 |
| PT80 | SH | 668118.3 | 937849 | 316.835 |  | PT340 | SH | 668136 | 937762 | 319.013 |
| PT81 | SH | 668118.1 | 937843.1 | 316.939 |  | PT341 | SH | 668138.3 | 937764.7 | 319.044 |
| PT82 | SH | 668113.8 | 937842.7 | 316.91 |  | PT342 | BLD | 668139.4 | 937765.7 | 319.051 |
| PT83 | SH | 668108.1 | 937843.3 | 317.4 |  | PT343 | SH | 668141.4 | 937749.2 | 319.111 |
| PT84 | SH | 668106.1 | 937847.2 | 317.641 |  | PT344 | SH | 668143.4 | 937751 | 319.05 |
| PT85 | SH | 668102.4 | 937849.2 | 317.563 |  | PT345 | SH | 668144.9 | 937753.8 | 318.948 |
| PT86 | SH | 668101.2 | 937843.6 | 317.586 |  | PT346 | SH | 668147.2 | 937751.6 | 318.691 |
| PT87 | SH | 668099.2 | 937840.3 | 317.489 |  | PT347 | SH | 668146.9 | 937747.9 | 319.135 |
| PT88 | SH | 668098.1 | 937836.4 | 317.051 |  | PT348 | SH | 668144.5 | 937745.9 | 319.098 |
| PT89 | SH | 668094.9 | 937829.9 | 317.405 |  | PT349 | SH | 668143.8 | 937741.8 | 319.252 |
| PT90 | SH | 668099 | 937830 | 317.366 |  | PT350 | BLD | 668137.8 | 937739.5 | 319.454 |
| PT91 | SH | 668102.2 | 937835.3 | 317 |  | PT351 | SH | 668140 | 937740.1 | 319.295 |
| PT92 | SH | 668093 | 937842.7 | 317.597 |  | PT352 | SH | 668141.4 | 937742 | 319.263 |
| PT93 | SH | 668090.5 | 937849.3 | 317.02 |  | PT353 | SH | 668145.1 | 937740.6 | 319.475 |
| PT94 | SH | 668088 | 937845.4 | 317.154 |  | PT354 | SH | 668147.6 | 937743.1 | 319.297 |
| PT95 | CV | 668089.1 | 937840.1 | 317.01 |  | PT355 | SH | 668149.8 | 937746.8 | 319.15 |
| PT96 | CV | 668089.5 | 937840.1 | 317.036 |  | PT356 | SH | 668151.4 | 937750.4 | 319.144 |
| PT97 | CV | 668089.7 | 937835.1 | 316.97 |  | PT357 | SH | 668153.9 | 937753.9 | 319.13 |
| PT98 | CV | 668089.1 | 937835.1 | 316.951 |  | PT358 | BLD | 668156.8 | 937753.8 | 319.16 |
| PT99 | SH | 668087.3 | 937837.4 | 317.032 |  | PT359 | BLD | 668155 | 937750.1 | 319.175 |
| PT100 | SH | 668084.9 | 937841.3 | 317.015 |  | PT360 | GT | 668156.7 | 937750.4 | 319.189 |
| PT101 | SH | 668081.1 | 937845.6 | 316.694 |  | PT361 | GT | 668156.4 | 937744 | 319.379 |
| PT102 | BLD | 668082.4 | 937849.5 | 316.774 |  | PT362 | BLD | 668151.3 | 937744.1 | 319.293 |
| PT103 | SH | 668079.2 | 937850.8 | 316.576 |  | PT363 | BLD | 668157.6 | 937720.9 | 319.595 |
| PT104 | SH | 668076.3 | 937851.1 | 316.606 |  | PT364 | BLD | 668075.3 | 937717.9 | 320.555 |
| PT105 | SH | 668076.4 | 937857.2 | 316.522 |  | PT365 | BLD | 668042.2 | 937720.6 | 320.353 |
| PT106 | SH | 668079.6 | 937859 | 316.481 |  | PT366 | BLD | 668036 | 937729.4 | 320.261 |
| PT107 | SH | 668082.3 | 937859.3 | 316.539 |  | PT367 | BLD | 668035.2 | 937744.9 | 319.623 |
| PT108 | SH | 668081.6 | 937865.3 | 316.444 |  | PT368 | BLD | 668011 | 937744.6 | 320.389 |
| PT109 | SH | 668078.7 | 937867 | 316.454 |  | PT369 | BLD | 668000.4 | 937749.6 | 319.55 |
| PT110 | SH | 668075.8 | 937866.3 | 316.351 |  | PT370 | BLD | 667993.6 | 937757.8 | 318.942 |
| PT111 | SH | 668074 | 937869.4 | 316.329 |  | PT371 | BLD | 667980.6 | 937772.8 | 318.59 |
| PT112 | SH | 668078.7 | 937870.4 | 316.547 |  | PT372 | BLD | 667974.3 | 937782.4 | 318.279 |
| PT113 | SH | 668081.1 | 937872.7 | 316.472 |  | PT373 | BLD | 667968.1 | 937789.6 | 318.03 |
| PT114 | SH | 668081.9 | 937877.2 | 316.469 |  | PT374 | RD | 667966.1 | 937793.6 | 317.734 |
| PT115 | SH | 668079 | 937877.3 | 316.472 |  | PT375 | CL | 667964.4 | 937795.8 | 317.839 |
| PT116 | SH | 668076.3 | 937876 | 316.604 |  | PT376 | RD | 667962.9 | 937797.2 | 317.658 |
| PT117 | EP | 668074.3 | 937876.1 | 316.594 |  | PT377 | SH | 667967.3 | 937798.1 | 317.599 |
| PT118 | TM | 668071.8 | 937879 | 316.342 |  | PT378 | SH | 667972.1 | 937795.5 | 317.821 |
| PT119 | TM | 668076.7 | 937878.8 | 316.445 |  | PT379 | SH | 667976.9 | 937798.1 | 317.559 |
| PT120 | TM | 668077.2 | 937885.8 | 316.536 |  | PT380 | SH | 667975.5 | 937802 | 317.229 |
| PT121 | SH | 668079.9 | 937885 | 316.362 |  | PT381 | SH | 667973.8 | 937805.3 | 317.111 |
| PT122 | SH | 668082.8 | 937883.4 | 316.48 |  | PT382 | SH | 667974.4 | 937809.3 | 317.087 |
| PT123 | SH | 668079.8 | 937880.6 | 316.437 |  | PT383 | SH | 667980.3 | 937808.6 | 317.008 |
| PT124 | SH | 668064.8 | 937896.9 | 315.429 |  | PT384 | SH | 667986.3 | 937808.3 | 317.055 |
| PT125 | BLD | 668064.8 | 937885.4 | 315.743 |  | PT385 | SH | 667989 | 937806.7 | 317.471 |
| PT126 | SH | 668069.7 | 937888.7 | 315.93 |  | PT386 | SH | 667991.4 | 937808.2 | 317.321 |
| PT127 | SH | 668069.8 | 937882.8 | 315.817 |  | PT387 | BLD | 667995.8 | 937809.1 | 317.181 |
| PT128 | EP | 668066.9 | 937881.4 | 316.367 |  | PT388 | BLD | 667996.6 | 937819.4 | 317.014 |
| PT129 | BLD | 668073.2 | 937873.9 | 316.28 |  | PT389 | BLD | 667997.1 | 937825.8 | 316.715 |
| PT130 | SH | 668076.5 | 937842.5 | 316.803 |  | PT390 | BLD | 667997.7 | 937836.1 | 316.436 |
| PT131 | SH | 668080.9 | 937837.3 | 316.882 |  | PT391 | SH | 667999.8 | 937837.9 | 316.489 |
| PT132 | SH | 668076.4 | 937833.4 | 317.117 |  | PT392 | SH | 668004.1 | 937837 | 316.697 |
| PT133 | SH | 668079.1 | 937829.3 | 317.088 |  | PT393 | SH | 668006.3 | 937835.7 | 316.836 |
| PT134 | SH | 668084.6 | 937829.3 | 317.095 |  | PT394 | SH | 668012.2 | 937839.5 | 317.101 |
| PT135 | SH | 668084.6 | 937825.2 | 317.166 |  | PT395 | BLD | 668024 | 937834.2 | 316.943 |
| PT136 | SH | 668080.9 | 937824.4 | 317.192 |  | PT396 | EP | 668028.4 | 937833.8 | 316.701 |
| PT137 | EP | 668075.2 | 937830.8 | 317.409 |  | PT397 | SH | 668027.3 | 937830.5 | 316.834 |
| PT138 | SH | 668076.5 | 937824.8 | 317.309 |  | PT398 | SH | 668024.8 | 937827.5 | 316.95 |
| PT139 | SH | 668080.1 | 937820.9 | 317.285 |  | PT399 | SH | 668027.4 | 937825.1 | 316.977 |
| PT140 | SH | 668083.9 | 937818.9 | 317.208 |  | PT400 | SH | 668025.7 | 937818 | 317.088 |
| PT141 | SH | 668084.3 | 937814.6 | 317.38 |  | PT401 | BLD | 668022.6 | 937817.1 | 317.238 |
| PT142 | SH | 668080.3 | 937812.9 | 317.483 |  | PT402 | SH | 668025.6 | 937813.3 | 317.258 |
| PT143 | SH | 668077 | 937811.8 | 317.722 |  | PT403 | SH | 668030.1 | 937809.1 | 317.516 |
| PT144 | EP | 668075.5 | 937812.1 | 317.622 |  | PT404 | BLD | 668031.1 | 937802.8 | 317.66 |
| PT145 | CV | 668075.9 | 937810.5 | 317.869 |  | PT405 | SH | 668027.5 | 937801.2 | 317.556 |
| PT146 | CV | 668076.6 | 937810.4 | 317.877 |  | PT406 | SH | 668031.4 | 937799.3 | 317.671 |
| PT147 | CV | 668076.1 | 937808 | 317.884 |  | PT407 | BLD | 668035 | 937801.7 | 318.075 |
| PT148 | CV | 668076.5 | 937808 | 317.896 |  | PT408 | SH | 668032 | 937800 | 317.586 |
| PT149 | BLD | 668073.4 | 937815.4 | 317.468 |  | PT409 | SH | 668031.4 | 937796 | 317.74 |
| PT150 | BLD | 668073.5 | 937803.4 | 317.824 |  | PT410 | SH | 668033.4 | 937793.9 | 317.725 |
| PT151 | SH | 668077.3 | 937804.7 | 317.737 |  | PT411 | BLD | 668035.2 | 937792.8 | 317.863 |
| PT152 | SH | 668080.4 | 937805.5 | 317.546 |  | PT412 | BLD | 668029.7 | 937792.2 | 318.048 |
| PT153 | SH | 668083.8 | 937807.7 | 317.514 |  | PT413 | BLD | 668029.5 | 937779.7 | 318.482 |
| PT154 | SH | 668084 | 937803.1 | 317.576 |  | PT414 | BLD | 668035.6 | 937839.2 | 316.76 |
| PT155 | SH | 668080.5 | 937801.9 | 317.645 |  | PT415 | BLD | 668030 | 937849.5 | 316.014 |
| PT156 | SH | 668076.8 | 937799.4 | 318.005 |  | PT416 | BLD | 668014.7 | 937849.8 | 315.771 |
| PT157 | SH | 668080.5 | 937797 | 317.729 |  | PT417 | RD | 668001.4 | 937850 | 315.553 |
| PT158 | SH | 668084.2 | 937798.5 | 317.584 |  | PT418 | RD | 667996.5 | 937852.3 | 315.646 |
| PT159 | SH | 668086.6 | 937795.5 | 317.625 |  | PT419 | BLD | 668015.4 | 937870.2 | 315.216 |
| PT160 | SH | 668088.9 | 937794.7 | 317.784 |  | PT420 | RD | 668022.4 | 937889 | 314.617 |
| PT161 | SH | 668087.8 | 937791.7 | 318.04 |  | PT421 | RD | 668020.3 | 937890.7 | 314.467 |
| PT162 | BLD | 668090.2 | 937790.5 | 318.057 |  | PT422 | BLD | 668035.6 | 937896.8 | 314.523 |
| PT163 | SH | 668086 | 937790 | 318.019 |  | PT423 | CV | 668032.5 | 937898.1 | 314.441 |
| PT164 | SH | 668079.9 | 937790.6 | 317.829 |  | PT424 | CV | 668032.1 | 937898.6 | 314.354 |
| PT165 | SH | 668076.8 | 937791.1 | 317.997 |  | PT425 | EP | 668026.8 | 937902.6 | 314.367 |
| PT166 | EP | 668075.9 | 937791.5 | 318.028 |  | PT426 | EP | 668042.4 | 937923.4 | 313.423 |
| PT167 | SH | 668081.3 | 937788.3 | 317.911 |  | PT427 | EP | 668045.7 | 937913.1 | 313.825 |
| PT168 | SH | 668086.2 | 937787 | 318.037 |  | PT428 | CV | 668045.7 | 937915.3 | 314.041 |
| PT169 | SH | 668088.3 | 937782.1 | 318.256 |  | PT429 | CV | 668045.2 | 937915.7 | 314.017 |
| PT170 | SH | 668083.1 | 937782.1 | 318.113 |  | PT430 | RD | 668050.4 | 937924.9 | 313.193 |
| PT171 | SH | 668077.2 | 937783.1 | 318.448 |  | PT431 | RD | 668048 | 937928.1 | 313.163 |
| PT172 | SH | 668077.1 | 937779.4 | 318.482 |  | PT432 | CL | 668049.6 | 937927.2 | 313.142 |
| PT173 | SH | 668080.8 | 937779.2 | 318.382 |  | PT433 | BLD | 668057.8 | 937924.9 | 313.443 |
| PT174 | SH | 668086.1 | 937777 | 318.527 |  | PT434 | BLD | 668139.5 | 937920.4 | 313.76 |
| PT175 | SH | 668088.4 | 937774.7 | 318.351 |  | PT435 | BD | 668156.3 | 937920.7 | 312.957 |
| PT176 | CV | 668076.7 | 937775.7 | 318.733 |  | PT436 | BLD | 668156.4 | 937893.6 | 315.143 |
| PT177 | CV | 668076.4 | 937775.7 | 318.755 |  | PT437 | BLD | 668120.5 | 937803.6 | 317.77 |
| PT178 | SH | 668075.6 | 937773.1 | 318.598 |  | PT438 | SH | 668121.8 | 937806.3 | 317.702 |
| PT179 | BLD | 668073.5 | 937768.6 | 318.708 |  | PT439 | BLD | 668120.3 | 937810 | 317.679 |
| PT180 | SH | 668073.1 | 937767.9 | 318.686 |  | PT440 | SH | 668124.2 | 937808.2 | 317.693 |
| PT181 | SH | 668075.1 | 937765.4 | 318.807 |  | PT441 | SH | 668124.2 | 937805.5 | 317.683 |
| PT182 | BLD | 668076.2 | 937763 | 318.99 |  | PT442 | SH | 668127.7 | 937808.3 | 317.623 |
| PT183 | SH | 668077.8 | 937762.3 | 318.92 |  | PT443 | SH | 668129.9 | 937805.3 | 317.555 |
| PT184 | SH | 668079.4 | 937758.5 | 319.226 |  | PT444 | SH | 668132.3 | 937808.2 | 317.557 |
| PT185 | SH | 668076.9 | 937758.6 | 318.98 |  | PT445 | SH | 668135.2 | 937805 | 317.569 |
| PT186 | SH | 668079.6 | 937754.5 | 319.302 |  | PT446 | SH | 668139.8 | 937808.7 | 317.523 |
| PT187 | SH | 668077.8 | 937753.6 | 319.218 |  | PT447 | SH | 668142.5 | 937806.5 | 317.477 |
| PT188 | SH | 668078.5 | 937746.8 | 319.408 |  | PT448 | SH | 668143.5 | 937805.6 | 317.641 |
| PT189 | SH | 668076.3 | 937744 | 319.51 |  | PT449 | BLD | 668152.8 | 937810.8 | 317.334 |
| PT190 | SH | 668079.7 | 937740.7 | 319.611 |  | PT450 | BLD | 668152.8 | 937803.9 | 317.388 |
| PT191 | SH | 668080.3 | 937743.2 | 319.569 |  | PT451 | BLD | 668120.4 | 937826.9 | 317.189 |
| PT192 | SH | 668078.8 | 937739.9 | 319.631 |  | PT452 | SH | 668123.3 | 937828 | 316.989 |
| PT193 | SH | 668078.8 | 937737.1 | 319.719 |  | PT453 | SH | 668126.8 | 937828.7 | 316.943 |
| PT194 | SH | 668081 | 937736.3 | 319.732 |  | PT454 | SH | 668126.3 | 937832 | 316.931 |
| PT195 | SH | 668078.1 | 937736.2 | 319.726 |  | PT455 | SH | 668123.9 | 937832.8 | 316.937 |
| PT196 | SH | 668074.3 | 937736.5 | 319.73 |  | PT456 | SH | 668121.8 | 937833.5 | 316.965 |
| PT197 | BLD | 668076 | 937739.9 | 319.718 |  | PT457 | SH | 668128 | 937833 | 316.904 |
| PT198 | SH | 668070.5 | 937735 | 319.725 |  | PT458 | SH | 668128.4 | 937836.4 | 316.889 |
| PT199 | SH | 668067.7 | 937732.6 | 319.769 |  | PT459 | SH | 668135.6 | 937832.3 | 316.921 |
| PT200 | SH | 668065.8 | 937735.6 | 319.673 |  | PT460 | SH | 668134.3 | 937834.4 | 316.926 |
| PT201 | SH | 668064.2 | 937739.6 | 319.579 |  | PT461 | SH | 668138.4 | 937829.2 | 316.911 |
| PT202 | SH | 668061 | 937736.3 | 319.632 |  | PT462 | SH | 668141.1 | 937835.5 | 316.628 |
| PT203 | SH | 668060.3 | 937732.6 | 319.705 |  | PT463 | SH | 668120.5 | 937838.7 | 317.002 |
| PT204 | SH | 668057.4 | 937735.8 | 319.652 |  | PT464 | BLD | 668073.3 | 937862.2 | 316.453 |
| PT205 | SH | 668056.3 | 937739.5 | 319.449 |  | PT465 | SH | 668072.4 | 937859 | 316.477 |
| PT206 | SH | 668052.5 | 937736.1 | 319.664 |  | PT466 | SH | 668069.5 | 937857 | 316.409 |
| PT207 | SH | 668050.9 | 937732.6 | 319.757 |  | PT467 | SH | 668065.8 | 937859.2 | 316.319 |
| PT208 | SH | 668048.6 | 937736.2 | 319.724 |  | PT468 | SH | 668064.3 | 937857.5 | 316.229 |
| PT209 | BLD | 668043.3 | 937739.9 | 319.659 |  | PT469 | SH | 668063.9 | 937853.2 | 316.483 |
| PT210 | BLD | 668043.6 | 937731.8 | 319.686 |  | PT470 | SH | 668055.7 | 937859.2 | 316.296 |
| PT211 | BLD | 668075.8 | 937729.8 | 319.966 |  | PT471 | SH | 668050.3 | 937854.7 | 316.365 |
| PT212 | BLD | 668079.9 | 937728.9 | 320.175 |  | PT472 | BLD | 668050.1 | 937852.9 | 316.654 |
| PT213 | BLD | 668079.7 | 937763.4 | 318.77 |  | PT473 | SH | 668056.8 | 937852.8 | 316.47 |
| PT214 | SH | 668080.8 | 937767.6 | 318.711 |  | PT474 | SH | 668063.8 | 937852.2 | 316.438 |
| PT215 | SH | 668083.6 | 937771.3 | 318.618 |  | PT475 | SH | 668067.5 | 937853.8 | 316.473 |
| PT216 | CV | 668088.4 | 937773.9 | 318.388 |  | PT476 | BLD | 668073.3 | 937851.7 | 316.674 |
| PT217 | CV | 668088.4 | 937774.1 | 318.401 |  | PT477 | BLD | 668073.4 | 937838.8 | 317.02 |
| PT218 | CV | 668088.9 | 937774.3 | 318.362 |  | PT478 | SH | 668071.2 | 937834.5 | 316.988 |
| PT219 | CV | 668089.2 | 937774.2 | 318.355 |  | PT479 | SH | 668071.7 | 937829.5 | 317.224 |
| PT220 | CV | 668089.7 | 937774 | 318.348 |  | PT480 | BLD | 668073.4 | 937827.2 | 317.205 |
| PT221 | CV | 668089.7 | 937773.6 | 318.358 |  | PT481 | SH | 668067.6 | 937828.1 | 317.111 |
| PT222 | SH | 668092.5 | 937776.3 | 318.551 |  | PT482 | SH | 668064.6 | 937832.8 | 317.034 |
| PT223 | SH | 668090.5 | 937779.5 | 318.338 |  | PT483 | SH | 668061.8 | 937837.4 | 316.978 |
| PT224 | SH | 668092.9 | 937780.6 | 318.233 |  | PT484 | SH | 668056.2 | 937836.7 | 316.98 |
| PT225 | SH | 668091.7 | 937784.1 | 318.136 |  | PT485 | SH | 668057.4 | 937831 | 317.048 |
| PT226 | SH | 668090.5 | 937787.6 | 318.117 |  | PT486 | SH | 668058.5 | 937827.6 | 317.15 |
| PT227 | SH | 668095.2 | 937788.3 | 318.215 |  | PT487 | SH | 668050.6 | 937828 | 316.962 |
| PT228 | SH | 668098.3 | 937788.8 | 318.206 |  | PT488 | SH | 668049.9 | 937833.6 | 316.97 |
| PT229 | SH | 668098.2 | 937786.1 | 318.204 |  | PT489 | SH | 668049 | 937837 | 316.957 |
| PT230 | SH | 668097.9 | 937782.4 | 318.229 |  | PT490 | BLD | 668043.9 | 937838 | 317.267 |
| PT231 | SH | 668097.4 | 937777.8 | 318.321 |  | PT491 | BLD | 668073.7 | 937792 | 318.073 |
| PT232 | SH | 668097.1 | 937773.1 | 318.529 |  | PT492 | SH | 668073.1 | 937784.7 | 318.318 |
| PT233 | SH | 668100.9 | 937773.3 | 318.58 |  | PT493 | BD | 668073.2 | 937780.3 | 318.439 |
| PT234 | SH | 668102.2 | 937777.1 | 318.319 |  | PT494 | SH | 668069.9 | 937784.4 | 318.355 |
| PT235 | SH | 668101.1 | 937781.3 | 318.285 |  | PT495 | SH | 668064.9 | 937790.3 | 318.076 |
| PT236 | SH | 668100.2 | 937785.2 | 318.239 |  | PT496 | SH | 668062.6 | 937784.4 | 318.262 |
| PT237 | SH | 668102.1 | 937788.4 | 318.071 |  | PT497 | SH | 668061.5 | 937781.1 | 318.253 |
| PT238 | BLD | 668103.3 | 937790.4 | 317.873 |  | PT498 | SH | 668054.5 | 937781.3 | 318.237 |
| PT239 | SH | 668105.1 | 937786.6 | 318.137 |  | PT499 | SH | 668053 | 937785.2 | 318.171 |
| PT240 | SH | 668105.9 | 937783.1 | 318.189 |  | PT500 | SH | 668051.4 | 937789.3 | 318.097 |
| PT241 | SH | 668106.2 | 937778.4 | 318.236 |  | PT501 | SH | 668045 | 937791.3 | 318.148 |
| PT242 | SH | 668106.3 | 937774.5 | 318.368 |  | PT502 | SH | 668044.7 | 937785.9 | 318.238 |
| PT243 | SH | 668106.4 | 937771.7 | 318.607 |  | PT503 | BD | 668043.5 | 937789.4 | 318.204 |
| PT244 | SH | 668109.8 | 937771.4 | 318.611 |  | PT504 | BD | 668046.8 | 937781.7 | 318.191 |
| PT245 | SH | 668114.7 | 937770.8 | 318.727 |  | PT505 | SH | 668050.5 | 937781.3 | 318.192 |
| PT246 | CV | 668118.6 | 937769.6 | 318.768 |  | PT506 | BLD | 668073.6 | 937768.4 | 318.52 |
| PT247 | CV | 668118.6 | 937769.2 | 318.875 |  | PT507 | SH | 668072.8 | 937765.9 | 318.772 |
| PT248 | BLD | 668120.4 | 937768.5 | 318.906 |  | PT508 | SH | 668068.1 | 937767.9 | 318.712 |
| PT249 | SH | 668118 | 937771.4 | 318.745 |  | PT509 | SH | 668064.3 | 937765.9 | 318.846 |
| PT250 | SH | 668114.6 | 937774.6 | 318.45 |  | PT510 | SH | 668061.8 | 937764.8 | 318.757 |
| PT251 | SH | 668112.3 | 937778.2 | 318.273 |  | PT511 | SH | 668059.2 | 937766.6 | 318.779 |
| PT252 | SH | 668116.9 | 937781 | 318.219 |  | PT512 | SH | 668056.8 | 937767.3 | 318.813 |
| PT253 | SH | 668114.7 | 937784 | 318.086 |  | PT513 | SH | 668055.6 | 937765.9 | 317.208 |
| PT254 | SH | 668110.9 | 937786.4 | 318.028 |  | PT514 | SH | 668054.7 | 937765.1 | 317.217 |
| PT255 | SH | 668108.3 | 937788.8 | 318.063 |  | PT515 | SH | 668050.4 | 937765 | 317.161 |
| PT256 | SH | 668106.5 | 937792.3 | 317.922 |  | PT516 | SH | 668048.2 | 937766.1 | 317.281 |
| PT257 | SH | 668104.3 | 937794.3 | 317.593 |  | PT517 | SH | 668047.6 | 937767.1 | 318.799 |
| PT258 | SH | 668106.6 | 937796.8 | 317.885 |  | PT518 | BLD | 668043.7 | 937764.7 | 322.083 |
| PT259 | SH | 668110.6 | 937794.3 | 317.866 |  |  |  |  |  |  |

Reference

American Society of Civil Engineers. (2022). ASCE 38-22: Standard Guideline for Investigating and Documenting Existing Utilities. ASCE Press.

https://doi.org/10.1061/9780784416285

Cardarelli, E., & Di Filippo, G. (2009). Integrated geophysical methods for the characterisation of archaeological sites: A case study from the Sabine necropolis. Journal of Geophysics and Engineering, 6(3), 276–287.

https://doi.org/10.1088/1742-2132/6/3/002

Common Ground Alliance. (2023). Dirt Report: Analysis and Recommendations to Reduce Damages to Underground Infrastructure.

https://commongroundalliance.com

Davis, P. L., & Pacheco, S. (2019). Utility Coordination for Civil Engineering Projects. ASCE Press.

Gill, E., & Finley, A. (2020). Leveraging Geographic Information Systems (GIS) for utility data integration and risk assessment. International Journal of Infrastructure Systems, 14(2), 129–145.

McMullan, J. M., & Parkinson, D. (2017). Managing subsurface utility data for design and construction risk mitigation. Tunnelling and Underground Space Technology, 64, 221–231.

https://doi.org/10.1016/j.tust.2017.01.011

National Underground Asset Register (NUAR). (2022). NUAR Strategy and Implementation Report. UK Geospatial Commission.

https://www.gov.uk/government/publications/national-underground-asset-register-nuar

Osunmakinde, I. O., & Olorunfemi, M. O. (2021). Emerging technologies for underground utility detection in Sub-Saharan Africa: Opportunities and challenges. Journal of African Infrastructure Research, 8(1), 34–51.

Pasquet, A., & Smith, R. (2018). Drone-assisted mapping for infrastructure inspection: An innovative approach to underground utility surveys. Automation in Construction, 96, 421–430.

https://doi.org/10.1016/j.autcon.2018.10.001

Subsurface Utility Engineering Association (SUEA). (2021). Best Practices for Subsurface Utility Engineering Projects.

https://www.sueassociation.org