LAND INFORMATION OF ASA DAM COMMUNITY AREA

ILORIN, ILORIN WEST LOCAL GOVERNMENT AREA

KWARA STATE

BY

GANIYU AYUBA ALANI

HND/23/SGI/FT/0009

SUBMITTED IN PARTIAL FULFILMENT OF THE REQUIREMENTS FOR THE AWARD OF HIGHER NATIONAL DIPLOMA IN SURVEYING AND GEO-INFORMATICS TO THE DEPARTMENT OF SURVEYING AND GEO-INFORMATICS, KWARA STATE POLYTECHNIC ILORIN, KWARA STATE

JUNE, 2025

CERTIFICATE

I hereby certify that the information contained in this project report was obtained as a result of observations and measurements made by me on the field and that the survey was done in accordance with survey rules, regulations, and departmental instructions.

GANIYU AYUBA ALANI					HND/23/SGI/FT/0009         NAME OF STUDENT 				            MATRIC NUMBER

CERTIFICATION

This is to certify that GANIYU AYUBA ALANI with matriculation Number HND/23/SGI/FT/0009 has satisfactorily carried out his project under my instruction and supervision.

I here declare that He has conducted himself with diligence, honestly and sobriety on the project.

SURV. ROTIMI ASONIBARE						DATE

Project supervisor

MR. A I ISSAU							DATE

Head of Department

SURV.R.S AWOLEYE 							DATE

Project coordinator

External supervisor							DATE

DEDICATION

This project is dedicated to Almighty God for his, protection and guidance and also to my caring parent in person MR& MRS GANIYU

AKNOWLEDGEMENTS

The dream of becoming an achiever in life comes true, not only by one’s fortitude and hard work based on his knowledge, but it is also involving the positive influence of various factors and ideas of some people in the community.

First and foremost, all glory, honor and adoration is to Almighty God. I appreciate God for his grace upon my life, for keeping me safe since my advent in this school and most especially, during the execution of this project.

I sincerely wish to express my affectionate gratitude to my lovely and caring parent MR&MRS. GANIYU  whose toils and struggles have helped a lot in transforming my life for better may Almighty God grant them long life to enable her reap the fruits of their labour.

My next gratitude goes to my supervisor, SURV. ROTIMIASONIBARE for his useful guidance, understanding, patience, and professional advice during the execution of this project that led to the successful completion of the exercise. I pray that God will continue to bless you (amen).  More so, my appreciation goes to my able HOD SURV. ISSAU ABINBOLA and other lecturers in the department SURV AG AREMU, SURV R S AWOLEYE, SURV BANJI, SURV AYUBA ABD SALAM,SURV WILLIAMS KAZEEM, SURV BABATUNDE KABIR, SURV BELLO DIRAN  God bless you all.

More so, I seize this opportunity in acknowledging the efforts and contribution of my bossSURV. BASIT OLUOKUN

for his support morally, financially and for the impartation of knowledge in me that pertains to surveying profession.

I will not forget to express my gratitude to my friends and family in the likes of AROWO, ALFA HAMEED,SIS ABIBAT, ABEEBLAHI, RODIATand all my classmatesfor their support, advice and love I really appreciate you all.

ABSTRACT

This project topic present the development of a land information system (LIS) designed to manage and analyze land-related data. The LIS integrates spatial and non-spatial data to provide comprehensive platform for land administration, planning, and decision making. Key feature includes data visualization, and mapping capabilities. The system aims to enhance land governance, reduce land dispute, and promote sustainable land use practices

TABLE OF CONTENTS

CERTIFICATE	ii

CERTIFICATION	iii

DEDICATION	iv

AKNOWLEDGEMENTS	v

ABSTRACT	vi

CHAPTER ONE	1

1.0	 INTRODUCTION	1

1.1 	BACKGROUND TO THE STUDY	1

1.2 	STATEMENT OF THE PROBLEM	3

1.3      AIM OF THE PROJECT	4

1.6    SCOPE OF THE PROJECT	7

1.7     LOCATION OF PROJECT AREA	8

1.7	 PERSONNEL	9

CHAPTER TWO	10

2.0	LITERATURE REVIEW	10

CHAPTER THREE	24

3.0	 METHODOLOGY	24

3.1 	RECONNAISSANCE	24

3.2.1	OFFICE RECONNAISSANCE	24

3.2.2	FIELD RECONNAISSANCE	25

3.3.1	EQUIPMENT USED	25

3.3.2 	HARDWARE	25

3.3.3	SOFTWARE	25

3.4 CONTROL CHECK	26

3.5	Control Check Procedure (Control Comparison Method)	27

3.5.1 DATA SOURCE	27

3.5.2PRIMARY SOURCE	27

3.5.3 SECONDARY SOURCE	28

3.6.1	​GEOMETRIC DATA ACQUISITION	28

3.6.2	​ PERIMETER	28

3.6.3	DETAILING	28

3.6.4​ATTRIBUTE/ SOCIAL SURVEY	28

3.7.1	DATABASE CREATION/IMPLEMENT	29

3.7.2	DATABASE MANAGEMENT SYSTEM (DBMS)	29

3.7.3​	DATA QUALITY	30

3.7.4    ​DATA INTEGRITY	30

3.7.5​	DATA SECURITY	30

3.8.1.	DATA PROCESSING	31

3.8.2.	DRAFTING AND PLOTTING	31

3.8.3​	DATABASE CREATION PROCEDURES	32

CHAPTER FOUR	34

4.0	SPATIAL ANAYSIS, DATA PROCESSIN AND PRESENTATION	34

4.1 	SPATIAL ANALYSIS	34

4.2. 	TESTING OF DATABASE	34

4.3.1 	QUERIES	35

4.3.2	SPATIAL QUERY	35

4.5	MULTIPLE CRITERIA QUERY	40

CHAPTER FIVE	53

5.0 COSTING, PROBLEMS ENCOUNTERED, RECOMMENDATIO AND CONCLUSION	53

5.1	COSTING	53

5.2 	Problem Encountered	56

5.3 	Recommendations	57

5.4	Conclusion	57

REFERENCES	59

LIST OF TABLES

Table 3.4.1: Coordinate of the observed and the original values of PT 31	26

Table 3.4.2: Coordinate of the observed and the original values of PT 32	26

Table 3.4.3: Coordinate of the observed and the original values of PT 33	26

Figure: 4.4.1 Shows a single query made on the database of the project	36

Figure: 4.4.2 showing the question made on the database for built _up _area <196.12msq/m2	37

Figure: 4.4.3 showing the result of the query made on the database for built _up _area <196.12msq/m2 in form of a table	38

Figure:4.5.1 showing the question made on the databasefor built _up_ area > 403.99m2 and their purpose are for commercial use	41

Fig: 4.5.3 showing the result of the query made on the database for built _up_ area > 403.99 m2 and their purpose are for commercial use on the plan	43

Figure: 4.5.6showing the question made on the database for vacant_ area less than or equal to 901.93m2and are not fenced	44

Figure: 4.5.7showing the result of the query made on the database for vacant_ area <= 901.13m2and are not fenced in form of a tabl	45

Figure: 4.5.8 showing the result of the query made on the database for vacant_ area <= 901.93m2and are not fenced on the plan.	46

CHAPTER ONE

1.0	 INTRODUCTION

1.1 	BACKGROUND TO THE STUDY

Land information System (LIS) is defined as the combination of human and technical resources, together with a set of organizing procedures that produce information on land in support of a broad range of managerial requirements.

According to UN-ECE (2005) “Land information System (LIS) is defined as a tool for legal, administration and economic decision making and an aid for planning and development which consist on one hand a database containing spatially reference land related data for a defined area, and on the other hand procedures and techniques for the systematic collection, updating, processing and distribution of data.

The project land information system of Ara village Kwara State polytechnic l, Ilorin Kwara State involved traversing through the corner point of the project Area known as perimeter, spot height, detailing and contouring of the same area were carried out.

A land information system for state administration on land is expected to consist of thefollowing components.

- Geospatial data

- Software and programs

- Hardware system (data server, workstation, computers, scanners, printers, plotters, computer network: LAN, WAN, UPS etc.)

- The operators (surveyors, land officers etc.).

- The integrated approaches and methods

- These are the diverse essence of land information system. Map-base. Land Information Systems deliver real benefits to a range of organizations
1. Land owners-including estate managers, agribusiness cooperatives, and government authorities

- 2. Natural resource use companies-including forestry, extraction and wind energy companies

- 3. Local And central government requiring accurate land and building use inventories
4. Reduction of Risk -converting and holding data on land parcels, Ownership, production,costs, uses, etc.in digital format safe guards to a greater extent from loss or damage.

- Analysis and Reporting Building on the integration of relevant data, and the use of key land map layers such as topography and transport, it is possible to model and analyze scenarios of land usage, including minerals extraction, wind turbine location, forestry wind throw hazard, nutrient run-off, forestry and agricultural planning and harvesting, and building usage and valuation.

- Access to, and Sharing of, Information -Possibilities for sharing data and results of analyses greatly increase with access to integrated data holdings augmented by mapping information systems. Using web-based or mobile platforms,information can be widely shared and accessed while in the office, in the field or on- site. Combining the power of mapping with report creation, raw data becomes valuable information that can be delivered in accessible and interpretable form for multiple uses and users.

Land Information Systems can be designed at a range of scales, from farm or building level to national level, for supporting a single or group of quarries to national inventories of land ownership, forestry, or bioenergy. Stages in the development of a Land Information System include the following: Bioenergy Mapping System, discussion, discovery and data acquisition, processing, analysis and storage.

1.2 	STATEMENT OF THE PROBLEM

As the population of the world increasing, the analogue method of map production and management has been in use over the years with its associated errors and limitation. More organization and institution are now clamoring for modernization in techniques for keeping and updating information relating to land.

This gives room for vital information; about improper handling of data, encroachment, uneven distribution of basic facilities and method of operating such record is expensive, stressful, huge human effort required and time consuming. In order to facilitate its update with modern and necessary information, there is need for Information System that will help in managing andachieving this aim, which is Land Information System (LIS). Land management system and allocation is confronted with a herd of problem with the use of analogue information recording of data. Hence the need to use the automation of spatial information record of the Study area in order to solve such problems such as:

- Ambiguity in land description and allocation.

- Difficulties faced by tax accessories in evaluating real properties since the real sizes and dimensions of properties are unknown.

- Slowness, inefficiency and obsolescence of the analogue (manual) system of record keeping with all other problem associated with it such as exposure of records files to fire, water and even theft.

- Difficulties encountered in retrieving and updating information in an analogue database system.

To meet these challenges, there is need for creating an information system for these geographical entities. In view of the foregoing, there is a pressing need to create an avenue for the automation of spatial information record of Kwara State polytechnic Ilorin Kwara State and Implementation of Land Information System (LIS) which will serve as Spatial Decision Support System(SDSS) for efficient planning, allocations of amenities and proper development generally.

1.3      AIM OF THE PROJECT

The aim of the project is to create land information system (LIS) Asa Dam community Area, Ilorin Kwara State using automation approach.

1.4      OBJECTIVES OF PROJECT

In order to attain to the above mentioned aim, the following objectives were precisely followed:

    - Design and creation of a database by integrating the two data sets (i.e. query by either names in alphabetical order or numerical).

    - To create a database for individual each details that existed between the project site for easy retrieval of information.

    - Information presentation both in alphanumeric and graphic format in softcopy & hardcopy.

    - Comprehensive report of all the phases and facets of the project work.

1.5 Specification of the project

In other to obtain the land information system of Ara village (Land Use), special key factors must be considered. These specifications include;

Data Types:

Spatial data:  Parcel boundaries, property lines, topographic features and infrastructural networks

Attribute Data: Ownership details (owner’s names, addresses), land use classification (residential, commercial, and agricultural), zoning regulations, property values and legal descriptions

Data Collection Methods:

Cadastral surveys (field measurements, aerial photographs)

Existing land records digitization

Remote sensing data integration

Data Quality standards:

Accuracy level for spatial data

Data validation procedures

Database Design:

Entity relationship diagrams defining data relationships

Spatial data storage and indexing mechanisms

Data integrity constraints to maintain data consistency

System Functionalities:

Data capture and update capabilities

Spatial analysis tools (buffering, overlay analysis)

Query and reporting functions for data retrieval

Maps visualization and display options

Reporting and visualization tools(maps, graphs, tables.)

Integration with other systems (e.g tax assessment, property registration

Access Controls And Security:

User authentication and authorization levels

Data encryption for sensitive information

When these specifications are adhered to properly it brings about Improved land title security and clarity, streamlined land registration process, Efficient land managements and planning, Reduced land disputes and fraud, and Facilitate informed decision-making for land development and land use projects.

1.6    SCOPE OF THE PROJECT

The project scope consists of the following:

  - Reconnaissance (office and Field)

  - Station Selection

  - Data acquisition

  - Perimeter Traversing and Coordination

  - Details and Spot Height Observation

  - Data Downloading and processing

  - Data Analysis

  - Presentation (Soft and Hard copy)

  - Report writing and printing.

1.7     LOCATION OF PROJECT AREA

Based on the location, here are the details:

UTM Zone

The location of project area is part of Asa-Dam Community Area, Ilorin, Asa local government area, Kwara State, is located in UTM system between Latitude: 8° 48’ N and Longitude: 4° 55’ E in the north central geopolitical zone.of Nigeria.

1.7	 PERSONNEL

MATRIC NO				 ROLE

HND/23/SGI/FT/0009			Author

HND/23/SGI/FT/0006			Member

HND/23/SGI/FT/0007			Member

HND/23/SGI/FT/0012			Member

HND/23/SGI/FT/0011			Member

HND/23/SGI/FT/0013			Member

HND/23/SGI/FT/0014			Member

CHAPTER TWO

2.0	LITERATURE REVIEW

“Land” according to Lexicon Webster’s dictionary referred to as the solid surface of theearth where it is not covered with water. Land is the foundation of all forms of human activities;from which we obtain the food we eat, the shelter we need, the space we work, the road we driveour cars and the room to relax etc. Land is finite in extent and permanent by nature, and it isman’s most valuable resources. It is the means of life, without which he could never have existedand on which his continued existence and progress depend.

Land is the habitat of man and its wide use is crucial for the economic, social, religious,scientific and environmental advancement of all countries. Indeed, for human survival. Land isthe basic natural resources of any nation on which human and economic activities take place.Hence, the demand for land information has grown in multiple folds over many years forplanning and implementation of land development projects and the awareness of revenue thatcould be obtained from it through effective collection of land related taxes.

The first idea of cadastral records was obtained from the ancient Egypt. The source oflivelihood was from the land around River Nile, when the river was flooded, it inhibitedcultivation. But, when it dried up, the land around it became fertile. Then, land was demarcated,assigned to ownership and gave for taxes to generate revenue for King Pharaoh of Egypt.

Simpson (1976), as society was growing, land becomes an important asset, a man’s wealth is measured in the size of land he had and upon which he raised funds. With the development of writing and measurement technologies, recording started to be made of information relating to land. This collection of land record was referring to as cadastre and was principally for purpose of taxation adjudication.

Dashe (1987), observed that common to all these cadastral practices, was the absence of the idea of map or plan. There were only written records. In fact, the origin of cadastre was attributed to the concept ‘‘capitastrum’’ means a ‘‘Register of capital and all related information’’. Cadastral map are registrable instruments which can be used to bail out a suspect from police or prison custody. Banks honor its applications for overdraft. In fact, land is referred to as responsible human asset.

Form early days of civilization, human activities have always depended in Geographic information. The decision-makers and planner have always needed locations, Qualitative and statistical information regarding various types of socio-economic development projects. Also the studies using information system for Agricultural management in practice in Botswana which covered degradation of grazing land by climatic and man-induced factors, impact of wild life on the natural environment, rural regional planning utility, Land information system and remote sensing are integrated to determine crop suitability and site selection for abattoirs and other public places and or parcels of land.

The need for LIS implementation includes; transfer of legal rights on land, which couldbe attributed to three methods for processing such relevant records as follows:

First method is by private Conveyance; a situation whereby the records and transfer of land are handled by private arrangement. Registration of Deed is another method that has copiesof such records being maintained by the state Government. Registration of Title of Land is the third method; this is a situation where a state organization maintains the records of land ownership. Many of the defects in deeds based systems, currently in operations, can be curve by changing to Registration of Title under which the unit for registration is the land parcel, not the deeds. The register must be kept up to date at all time, so as to be a reflection of the legal position on ground (Dale and McLaughlin, 2008).

Land information, on the other hand refers to the geographic extent of past, current and future right and interest of private individual and corporate bodies in the land of a country. Cadastre can be defined as a methodically arranged public inventory of data of all properties (parcels) within a certain country or district, based on a survey of their boundaries. Such properties are systematically identified by means of some separate designation. The boundaries of the properties (parcels) and the parcel identifier are normally shown on large-scale maps which, together with registers may show for each property, the legal right, the nature use, size and value.

Land administration is the process whereby land and information about its ownership is effectively in the control of a central authority that regulates the structure and patterns of land ownerships, land use, and access of the public to land resources in such a way that it is used in a sustainable manner from both environmental and economic perspectives. An ideal land administration must be able to include the provision of information on land in an effective and efficient way to correctly identify those people who have interest in real estate and providing information about these interests (such as duration of leases). It must also allow easy access to land for development where it is needed, increase efficiency in land use management through good planning, and promote greater social equity. It should also allow for revenue generation to the government (taxation), maintenance of environmental quality, and provide security of tenure. In Nigeria’s cities, land administration is devoid of the efficiency attributes indicated above. A number of problems are encountered in the current system. Prominent among these is the lack of a centralized database that is easily accessible. Also, International and landing agencies worldwide are increasingly recognizing the importance of improving the operation and management of cities in developing countries. A key component to this improvement is the use of Land Information Management (Willie, 1998). The United Nation (UN) sponsored interregional meeting of Cadastral experts in Bagor, Indonesia declared its vision as “to develop modern cadastral infrastructures that facilitate efficient land and properly markets, protect the land rights of all and support long term sustainable development and management”. In Nigeria, Cadastre started in 1883, yet the problem of management has increased over the years. In Northern Nigeria, much land exists that is not indisputably held by anyone. Of the land held under some sort of title, the greater portions are under customary tenure that has never been encoded and is known to vary in significant respect from place to place. Such title is unregistered and reliant upon only the knowledge of the community elders. Yet, as the slogan has it, the land is the nation’s lasting resource and inheritance of future generations. However, its present use is subject only to controls that even urban planning authorities have trouble in enforcing. It is no wonder that it has been observed that the pattern of growth in most cities of Nigeria consist of built up areas intermingling haphazardly with farmland and unused open spaces. More significant and conspicuous ramification of this is the absence of environmental safeguards in the use of land.

Land administration and management in Nigeria is presently bedeviled with poor remuneration, poor conditions of service, and inadequate logistics; lack of transparency in work processes, delays, and cumbersome manual procedures; poor records management; perceived corruption; mistrust on the part of customary land owners in land administration generally; lack of technical expertise in new technology available; and lack of effective collaboration and cooperation between the agencies. Thus, Nigeria’s land management presents formidable challenges to the land administrator, the planner, the surveyor, the lawyer, the sociologist, the environmentalist, and the economist, especially in an environment where baseline data is scant.
The lack of adequate functional and coordinated land information system and networks underpins this weak system, requiring that land related agencies maintain up-to-date scientific data, maps, and plans.

In a broad sense, land administration is defined as an operational syste designed to implement land policies. Many definitions are found in the literature endorsing this meaning. For example, the United Nations (1996) defines land administration as “the process of determining, recording, and disseminating information about ownership, value and use of land when implementing land policies.” Similarly, land administration comprises the sub-systems that actualize strategies to implement land policy, and other related policies, within land management systems." Land administration is sometimes viewed as the management of land. From this perspective, the term ‘land administration’ is used to refer to “the processes of regulating land and property development and the use and conservation of the land, the gathering of revenues from the land through sales, leasing, and taxation, and the resolving of conflicts concerning the ownership and use of land” (Dale and McLaughlin, 2009). For this study, the author adopts Barry’s definition (1999) of land administration as a set of operational sub-systems that puts policies concerning land into action. This definition implies that the administration of land is not in itself a strategic process. In particular, land administration does not involve strategic planning or policy development; rather, it is merely an implementation system that follows regulations and rules stated in the policies.

A computer-based land administration system will force standardization in the collation and processing of land data; decrease the cost and space required for storing records; prevent unnecessary duplication; facilitate access to land related data; improve information distribution, reduce the time and cost involved in transferring property rights, processing mortgages, facilitates the monitoring and analysis of market and rental value of land and property; and provide built in mechanism for quality control. The system will permit the integration of records of land ownership, land values, and land use with sociological, economic,and environmental data in support of physical planning. This will allow spatial analysis of any kind - this is the power of the system as a generic tool. The use of a database system will allow managers to re-examine their current procedures and thereby improve delivery. It will also permit better monitoring of organizational performance, providing information on what is happening on the land in a better way so that problem areas are identified and appropriate remedial measures applied.

A Database Management System was used to accomplish this study. Specifically, a Structured Query Language (SQL) database containing information that describes or defines a plot of land among several others within a layout was designed. The SQL database has a client/server relationship whereby other stakeholders from a distance could access it. Interfaces which are user friendly were used for the purpose of extracting the required information of a particular plot within a particular layout, in such a way that every plot involved would have its information coded, and displayed when needed from the database.

GIS could then be described as an integrated computerized system for the storage, retrieval, manipulation, analysis and display of geographically referenced data (Burrough, 2010; Smith and Johan, 2011). It integrates spatial and other kinds of data or information within single system. It offers the consistent framework for analyzing geographic data by putting maps and other spatial data and information into digital forms. Geographic information allows geoscientist to apply general principles to specific condition of each location on the earth surface, allows for the tracking of what is happening at a place, and helps to explain and provide the understanding on how one place differs from another (Ojigi, 2009).

GIS is relevant and valuable tool in the information harvest and planning of natural physical, bio-medical, socio-cultural, eco-climate economics, engineering sciences and business environments. The global community is now so dependent on computers and computerizedninformation that we scarcely can do without them. Computerization has opened vast new potential in the way we communicate, analyze our surroundings and make decision.

The power of GIS comes from the ability to relate different information in a spatial context and reach a conclusion about this relationship. Most of the information we have about our world contain a location reference, placing that information at some point on the globe. When a parcel of land information is collected, it is important to know where the plot is located and its topology. This is done by using a location reference system, such as longitude and latitude and perhaps elevation. Comparing the parcel information with other information such as the location of roads in the vicinity, may show the strategic location of the parcel vis-à-vis the economic value, this fact may indicate that these roads are likely to assist the easy conveyance of building material to site, and this inference can help us make the most appropriate decision about how busy the road might be during physical development. A GIS therefore, can reveal important new information that leads to better decision making. Different kind of data in map form can be entered into a GIS. A GIS can also convert existing digital information, which may not be in map form into forms it can recognize and use. For example, digital satellite images can be analyzed to produce a map of digital information about land use and land cover.

However, land information system is an important system to support administration of land. Land Information System as defined by FIG is a tool for legal, administrative and economic decision marking and aid for planning and development which consist on one hand a database containing spatially referenced land related data for a defined area, and on the other hand procedures and techniques for the systematic collection, updating processing and distribution of the data. The base of a land information system is a uniform spatial referencing system for the data in the systems, which also facilitate the linking of data within the system with other hand related data. Most countries in the world started with a manual system of recording land information, but from the 1970’s most countries began to initiate some form of computerized land information/automation and mapping system. This was pertinent due to the many problems and constraints that emerge as a result of increasingly complex land ownership, tenure and registration issues. Several important facts associated with the increasingly popular land information system include;

- Necessity of modern computerized land records information services that those involved in the generation and manipulation of the record can in a timely manner, correctly answer fundamental question about public and private rights and responsibilities to land.

- Knowledge of the spatial reliability of all data sets of all times so that land and land ownership rights and responsibilities are not incorrectly determined because of imprecise information. This for instance, results in the quick access and efficient monitoring of rates management, particularly rates collection.

The components of land information system are:

- Geospatial data

- Software and programs

- Hardware system (data server, computer, scanner, computer, network, LAN, WAN)

- The operator (Surveyor, Land Officers, etc.)

There is no consensus in the cadastral literature on one universal definition of the term ‘cadastral system’. Furthermore, Silva and Stubkjaer (2011) note that most of the time, the meaning of the term ‘cadastral system’ has not been made explicit and its use is typically very loose. As its main goal, this discussion does not aim to debate the various definitions of cadastral systems. Instead, it aims to present the definition which clearly explains cadastral systems as being used by the author in this research. In this study, a cadastral system is defined as a collection of integrated sub-systems which perform processes such as adjudication, boundary definition, demarcation, surveying, registration and dispute resolution (Barry, 1999). The cadastral system is constituted primarily of a set of processes together with an information system (IS). These processes are carried out by institutions and are constrained by the technical and human resources available. Adjudication is the process of determining rights in a land unit (i.e. a parcel, or in more general sense, a physical space). The processes of boundary definition, demarcation, and surveying generally involve marking, measuring and mapping the limits of a land unit. Registration is the official recording of recognized interests in land (McLaughlin and Nicholas, 2009) whereas dispute resolution is the process of resolving conflicts that may arise over a unit of land, Barry, (1999).

The institutions are the environment within which cadastral systems operate. These institutions follow their own regulations, and are given a mandate to carry out cadastral processes. Cadastral institutions may be formal, e.g. a government department, or informal, e.g. atribe leader or a village committee.

Human resources comprise the people who are responsible for operating the system. These people are required to possess sufficient knowledge and skills to be able to run and maintain the system. Human resources may include, surveyors, lawyers, mapping and data entry specialists, and information system specialists. Technical resources consist of the devices and technologies used to carry out the system tasks. These include devices that support information management, e.g. computers and digital information storage devices; surveying devices, e.g. GPSs; as well as communication and networking infrastructures, e.g. the Internet. Cadastral systems must match the technical and human resources available in a particular situation in order for it to be appropriate. As stated by Williamson and Parkville (2010), cadastral systems designed for developing countries should be simple, flexible, and attainable at low cost. While cadastral systems found in the developed countries are more complex, rigid, and expensive. Information system (IS) is placed at the center of the cadastral system. It is the core component of the system since it integrates the outputs produced by all the cadastral processes mentioned above. Ideally, the information stored in the IS should be comprehensive in terms of spatial and non-spatial information pertaining to land (Barry 1999). In the literature, this component is found to be termed a ‘cadastre’ (FIG 1995; Hensen 1995). Cadastre can be used to denote the Information System component of a cadastral system. Yet, he uses it with caution. This is because, as Dale (1997) stated, the term cadastre has been used differently by every country. Further, occasionally, the terms cadastre and cadastral system are used as synonyms (Silva and Stubkjær, 2011). Three forms of cadastral systems are distinguished namely fiscal cadastre, juridical cadastral and multipurpose cadastre. Therefore, for the purpose of this project, there is need to distinguish cadastral system from cadastre by defining the latter as merely an information system that manages and stores land information, including spatial and non-spatial information. On the other hand, a cadastral system, as defined earlier, is a combination of a set of processes and an information system (the cadastre).

Cadastre holds a variety of data. According to Navratil and Frank (2004) this data is classified into (1) technical and (2) legal data. Technical data includes positioning, taxation, and planning data. Positioning data provides information about boundaries of areas and creates the reference to the earth’s surface. Taxation data includes data needed for land tax calculation, such as the size of a piece of land, land use type, and location.

Planning data is represented by land use and existing structures which inform future development. Legal data includes legal rights and encumbrances, such as mortgages and servitudes. Based on the information contained in the information system (i.e. the cadastre), the cadastral system can be classified into three categories according to the purpose it serves (Barry,1999);

• Juridical: serves a legally recognized record of tenure.
• Fiscal: serves as a record for taxation and property valuation.
• Multipurpose: serves both purposes of the previous categories as it encompasses both the fiscal and the juridical records with other information, such as planning, environmental, and socioeconomic information.

Land information system (LIS) focuses on land parcel as the primary unit of information. It maintains analyses and disseminates information about land ownerships, value, records and registration, land assessment and allocation. The major data fields of LIS include coordinate of beacons, lengths and bearing of boundary lines, area of a plots, perimeter, parcel owner, ground rent, land market value, land use adjoining plots, reference controls etc. It is mainly applicable inbuilt-up or urban areas in large scale maps (e.g. scales 1:250, 1:5000). Land information includes every record about land (agriculture, commercial land use etc) and their market/economic. Therefore, cadastral information is an aspect of Land Information System, just as LIS is an aspect of geographic information system (GIS).

sGIS help to store and manage large amount of spatially referenced data. It provides analyses tools and therefore helps in a better understanding of the activities on the earth surface. Cadastral information system (CIS) or land information system (LIS) is crucial to any planning and management activity, particularly in our cities. The challenges facing CIS in Nigeria are to:

Determine what data and information is needed at hand in our cities.

Find out if it exists and where;

- Get hold of it, if it exists and how to collect it, if does not exist

- Store this information in easily accessible and referenced form

- Process the data, resolve question of quality, analysis contradiction and incompleteness.
Determine who need the information when and in what format

- Actually up-date and disseminate it as required.

- Relevant government authorities and developers need it for sustainable planning and development.

Basically the components of cadastral information system are not different from the popular five basic components of GIS (data, hardware, software, method of manipulation/analysis and human ware). What differentiate the components of LIS and GIS lies within the purpose and scale of the either of the two. For CIS of district or ward, the use of precise data collection techniques and equipment such as GNSS, total station, digital theodolite and sometimes laser techniques are not to be compromised.

The cost of any land information system is a combination of the data gathering process and data management system. The database management system is just like any GIS system consists of structural spatial data that is based on vector or tessellation with content such as:

- Positional data of object, this can be represented in vector forms as area.

- The attribute data (observed facts about the parcel)

- Spatial relationship between one parcel and another (topology).

The modern cadastral, according to Cichocinsia (1999) can be described as a full and comprehensive inventory of land ownership, structure, incorporating the official survey land and building designed primarily to meet the need, this can be regarded as “multitask cadastral”. The design of geographic database proceeds through several steps. After user’s requirement definition, the database structure is created conceptually, the physical design is documented to meet user’s requirement. The database is finally constructed and implemented to run a computer. In order to meet the future requirement expansion capacities must be taken into account.

Kufoniyi (1998) further defined the process of structuring as the logical arrangement of data used by a system for data management. Dastabase creation comes after the database has been logically designed. It is the imputing of data into the computer environment of the database for further manipulation in DBMS. In order to generate information and to support decision making. For this exercise, queries are performed through spatial search operations in conformity to the format of implementing software.

Using GIS for cadastral management is capable of improving the rate at spatial problems can be solved within a short time. This is because it assists in acquiring storing and retrieving information about land for the purpose of planning and administration. In other words, accurate and exercise is particularly useful in decision making in respect of solving some of the spatial problems.

sIn conclusion, this project presents the procedure involved in using GIS for land management. The procedure begins with a reconnaissance survey; through acquisition of geometric and attribute data; design, creation and management of database; to data analysis and product generation which provides the queries used in displaying the expected results.

CHAPTER THREE

3.0	 METHODOLOGY

​The following are the methods adopted in carrying out land information system analysis of part of Asa-Dam area, Ilorin, Asa local government area,Nigeria.

3.1 	RECONNAISSANCE

​This is the preliminary field inspection of the project site to be surveyed. It involves the overall view of the entire land in question or the project site in order to plan on how to go about the surveying operation to achieve  the aim of the work in accordance with the survey rules and regulation pertaining to the order of job required (i.e. first. Second or third order), the reconnaissance was done by considering the purpose of the survey operation and method of observation. There are the two types of reconnaissance that are carried out are:-

- Field Reconnaissance

- Office Reconnaissance

3.2.1	OFFICE RECONNAISSANCE

​Information about the study area like existing map/plan of the project site and preferably historical data of the site in question, letter of notification, necessary information equipment needed to carry out the survey and coordinates of control point was carried out under this section.

3.2.2	FIELD RECONNAISSANCE

At this stage, we visited the project area and take note of the crucial points, locating of existing control points, drawing of the reconnaissance diagram and also the determination of the best method for the Survey.

3.3.1	EQUIPMENT USED

Total Station and its accessories.

3.3.2 	HARDWARE

1. Laptop Computer

2. Mouse

3. Printer etc.

3.3.3	SOFTWARE

The following Software was used:

1. Microsoft word

2. Microsoft Excel

3. Note Pad

4. Google Earth

5. ArcGis 10.0

3.4 CONTROL CHECK

It is not enough to work from whole to part but it is equally expected that the set of controls to be used are properly checked to ascertain their suitability for the job. The in situ check for both linear and angular measurements as well as the coordinates.3.5 Control Check Procedure (Control Comparison Method)

In order to ensure the reliability and accuracy of the existing control network, a control comparison was conducted as part of the pre-observation checks. This procedure involved reoccupying selected known control points within the project area and measuring their spatial relationships (coordinates, bearings, and distances) to other established points. The results were then compared with the original coordinate records to determine any positional shifts or inconsistencies.

Procedure:

1. The total station was set up over a known control point, leveled, and oriented.

2. Observations (angles and distances) were taken to adjacent known control points.

3. Coordinates of the observed points were computed based on the measurements.

4. The computed coordinates were compared against the recorded coordinates.

5. Differences (∆Easting, ∆Northing, ∆Elevation) were calculated for each control point.

Table 3.4.1: Coordinate of the observed and the original values of PT 31

| PILLAR | NORTHING | EASTING | STATUS | REMARKS |
| --- | --- | --- | --- | --- |
| PT 31 (KW/AW/ 1206) | 934630.687 | 668601.336 |  | ORIGINAL |
| PT 31 ( KW/AW/1206) | 934630.677 | 668601.356 | FIXED | OBSERVED |
| DISCREPANCY | 0.010 | 0.020 |  |  |

Table 3.4.2: Coordinate of the observed and the original values of PT 32

| PILLAR | NORTHING(m) | EASTING(m) | STATUS | REMARKS |
| --- | --- | --- | --- | --- |
| PT32 ( KW/RB/37) | 934634.584 | 668590.327 |  | ORIGINAL |
| PT32 ( KW/RB/37) | 934634.563 | 668590.349 | FIXED | OBSERVED |
| DISCREPANCY | 0.021 | 0.022 |  |  |

Table 3.4.3: Coordinate of the observed and the original values of PT 33

| PILLAR | NORTHING(m) | EASTING(m) | STATUS | REMARKS |
| --- | --- | --- | --- | --- |
| PT33 KW/RS/12210) | 934663.590 | 668506.841 |  | ORIGINAL |
| PT33 KW/RS/12210) | 934663.579 | 668506.861, | FIXED | OBSERVED |
| DISCREPANCY | 0.011 | 0.020 |  |  |

3.5	Control Check Procedure (Control Comparison Method)

In order to ensure the reliability and accuracy of the existing control network, a control comparison was conducted as part of the pre-observation checks. This procedure involved reoccupying selected known control points within the project area and measuring their spatial relationships (coordinates, bearings, and distances) to other established points. The results were then compared with the original coordinate records to determine any positional shifts or inconsistencies.

Procedure:

1. The total station was set up over a known control point, leveled, and oriented.

2. Observations (angles and distances) were taken to adjacent known control points.

3. Coordinates of the observed points were computed based on the measurements.

4. The computed coordinates were compared against the recorded coordinates.

5. Differences (∆Easting, ∆Northing, ∆Elevation) were calculated for each control point.

3.5.1 DATA SOURCE

Control coordinate were given from existing map, which is considered as secondary data. This was plotted using AutoCAD. There are two main source of data used primary and secondary source.

3.5.2PRIMARY SOURCE

This involve the collection of the X, Y and Z coordinate (Spatial data) using land surveying Technique with Total Stationand its accessories.

3.5.3 SECONDARY SOURCE

This type of source that was gotten from the google earth application for land information system of part of Asa DamAgakaarea. Ilorin, Kwara State.

3.6.1	​GEOMETRIC DATA ACQUISITION

This involve the acquisition of both Northing, Easting AndHeight value of features that are present on the project site. In the aspect of this project digital land survey method was adopted.

3.6.2	​ PERIMETER

This coordinate of the boundary pillar of the project site were obtained through perimeter traversing of the area

3.6.3	DETAILING

​Detailing refer to fixing of features that are found in an area where survey operation had been carried out. Building, Roads, Electricity pole and other facilities located within the project area were detailed using land survey method.

3.6.4​ATTRIBUTE/ SOCIAL SURVEY

​This aspect of data acquisition entails the collection of other data which Such data were directly related to the features to which geometric data was acquired. They included building names, the purpose of which the building is used for etc.

​To collect attribute data, questionnaire Survey was employed. This involve oral interviews, reading information from sign posts, wall signs, virtual observation, etc.

3.7.1	DATABASE CREATION/IMPLEMENT

For efficient and effective management of data in the computer environment, data item are usually arranged and stored in a database or databank. The content of this database could be in form of a text, number, polygon or graphics. The creation of this database involved the combination and storage of the acquired graphical and attributes data obtained in former designed GIS database of a generic structure for the purpose in spatial analysis and queries on project site

In the creation of a land information system data mode, a widely used technique called layering was employed. The features that are present within the project site have been classified into different layers in the AutoCAD software independently. The polyline entities were joined using the polyline tool while appropriate symbols were used for the point entities. These layers were then exported to ArcGIS environment where shape files was created using attributes fields as conceptualized in the schema. These attribute table were then populated accordingly with attributes values for each particular entity as observed in the field and from the social survey template (attached as appendix)

The personal Geo database was then created finally in Arc Catalog environment. Where other tables that are non-geometric where created while the already created shape files where imported. Relationship between these tables were also established and the tables were later populated in the Arc Map environment. The following are some of the table created

3.7.2	DATABASE MANAGEMENT SYSTEM (DBMS)

​According to Dale and McLaughlin (1998), database management system was defined as a computer program to control the storage, retrieval and modification of data in the database. DBMS comprises of set of programmers which are used to maintain and manipulate the data orderly and acts as the central control over all the interactions. It is manages that data using alphanumeric data with limited capabilities of performing spatial queries

​A DBMS must allow the definition of data and their attributes and relationships as well as providing security and on interface between the end users and their application and the data themselves it reduces redundancy. Therefore, Arc GIS 10.0version was used to create, manipulate, maintain and access the database easily.

3.7.3​	DATA QUALITY

Some forms of quality control and quality assurance were incorporated in the project at every phase. These include conformity with data templates, data competences and data accuracy. Conformity with data templates in this premises refers to the degree to which the captured data conformed with the designed templates, while data competence was understood as the degree to which the available data in the report and for which there are specific templates have been extracted.

3.7.4    ​DATA INTEGRITY

​The data captured as exactly downloaded into the system then exported to AutoCAD via notepad and eventually into Arc GIS. The process involves ensuring that the data in the database were accurate and setting of certain constraint to prevent inconsistency in the database.

3.7.5​	DATA SECURITY

​Security is of great concern to land administration because of the legal implication of cadastral records. Security of the records is of almost importance to all concerned. This includes:

- Physical and system security

- Physical security: The use of burgling proof, firefighting equipment controlled access, proper records of the moment of personal and our of the office circuit break

- System Security: Uninterrupted power supply (UPS) will be used to control voltage, use of passwords and backups

In view of the foregoing, locking mechanism was adopted to protect the data in the database from unconscious deletion. Password was used to prevent unauthorized user from breaking into the database and a back up was created for the whole project on the rentable DVD.

Having succeeded in analysis the methodology employed in the execution of this project to arrive at the successful completion. It is equally necessary to examine the processes undertaken to ascertain the reliability and effectiveness of the created land information system

3.8.1.	DATA PROCESSING

AutoCAD and ArcGIS were used for plotting and database creation.

3.8.2.	DRAFTING AND PLOTTING

​During plotting of the data, was firstly used to plot out all the points that are picked on the field. The downloaded data of those points were well arranged in the Microsoft excel. The following steps were carried out:

- The data was save on the desktop for easy location

- AutoCAD 2007 was launched

- All the necessary parameters were set ranging from the drawing units, text, point style and others

- The data was then located on the desktop through the Civil CAD and opened,

- The refresh button was clicked and the data was refresh with a successful message displayed

- Then the zoom extend was used to bring all the plotted point into view.

The Civil CAD plotted the coordinate along with their point ID and this make the detailing of the features (such as Roads, electric poles and other) quite easier when transfer into AutoCAD environment.

3.8.3​	DATABASE CREATION PROCEDURES

​The plan in the AutoCAD format was added as data on ArchMap10.0. on Arc Catalog, a file Geo-database was created and subsequently a feature classes such as the line feature for the roads and others and polygon feature for the buildings.

Below are the procedures:

1. A folder was created for the project on the desktop and the plan with Dwgextension was saved in it,

2. ArcMap in ArcGIS10.0 software was launched

3. Then an empty new map which was displayed after loading was clicked on the minimize catalog on the right side was then maximize to view the ready created folder which contain the drawing in the AutoCAD format. The drawing was dragged from the folder and placed on the empty map and then the coordinate system was set.

CHAPTER FOUR

4.0	SPATIAL ANAYSIS, DATA PROCESSIN AND PRESENTATION

4.1 	SPATIAL ANALYSIS

Spatial analysis is a specialized function that distinguishes GIS from other information systems. It entails the examination of spatial and attributes characteristics of geographic features that are within the database to establish relationships from which spatial problems can be tackled. In this project work, spatial analyses were performed to select, combine and intersect existing geospatial data-sets in order to generate new information suitable for answering specific spatially-related questions.

The results from these analyses can be shown in a number of ways depending on the required output format. Where attribute information about map features are required, they can be presented as tables containing such valves as are needed from the query analysis. They can also be presented as maps with legend information showing the queried features and their topological relationships with other features shown on the map.

4.2. 	TESTING OF DATABASE

This is carried out to ascertain the quality of the database created, it was necessary to tryout the database so as to determine if information could be readily retrieved from it as required. Testing the database was important to determine whether the established relationships between features and their characteristics attributes are so represented on the database.

4.3.1 	QUERIES

Queries are specific questions in the form of "what is where", "where is what", "when" or how long has it been there. This provides answer to the needed information through processing or manipulating spatial data.

4.3.2	SPATIAL QUERY

Queries were designed for the purpose of retrieving information from the database. The queries performed in this project gave answers to certain generic questions asked from the database. This was made possible as a result of the implicit link of both the spatial and attributes data. The queries were based on the products from the analysis carried out on the database.

4.4	SINGLE CRITERION QUERY

A single criterion is carried out where one condition is used to design query. This condition is used to retrieve the information from the database. An example of a single criteria query is shown below

Figure: 4.4.1 Shows a single query made on the database of the project

Query 1: The query one is in interested in selecting or showing the built up area that have an area less than 196.12msq/m2

Syntax: built _up _area <196.12msq/m2

Figure: 4.4.2 showing the question made on the database for built _up _area <196.12msq/m2

Figure: 4.4.3 showing the result of the query made on the database for built _up _area <196.12msq/m2 in form of a table

Figure: 4.4.4showing the result of the query made on the database for built _up _area <196.12msq/m2on the plan

Discussion of Result

Figure 4.4.3 and 4.4.4 Shows the built up area that have an area less than 196.12msq/m It  consists of the syntax  model  or  the  query  builder  box,  attribute  table  as  well  as  the  map  of  the selected plot in mint blue color. The result shows that 15 built_up_areaout of the 44built _up _area have an area less than 196.12msq/m2

4.5	MULTIPLE CRITERIA QUERY

The database created is then used for implementing several selection queries in determination of user-defined requirements such as built up area whose purpose are for commercial or private use, year of construction, type of building (bungalow,2 story, duplex etc.) and other security which will be effective in planning in such environment.

Query2: The query 2 is interested in showing the building area that are greater than 403.99m2and their purpose that are used for commercial

Syntax:built_up_ area > 403.99 m2ANDtheir purpose are for commercial use.

Figure:4.5.1 showing the question made on the databasefor built _up_ area > 403.99m2 and their purpose are for commercial use.

Figure: 4.5.2 showing the result of the query made on the databaseforbuilt _up_ area > 403.99 m2 and their purpose are for commercialusein form of a table

Fig: 4.5.3 showing the result of the query made on the database for built _up_ area > 403.99 m2 and their purpose are for commercial use on the plan

Discussion of Result

Figure 4.5.2 and 4.5.3 Shows the built _ up _ area > 403.99㎡and purpose are commercial on the planIt  consists of the syntax  model  or  the  query  builder  box,  attribute  table  as  well  as  the  map  of  the selected plot in mint blue color. The result shows that 2 built _up _area out of the 44 built _up _area have an area greater than 403.99 m2 and their purpose are for commercial use.

Query3: The query 3 is interested in showing the vacant area less than or equal to 901.99m and are not fenced to query about the vacant area of land less than or equal to 901.13m2and are not fenced

Syntax:vacant_ area <= to 901.13m2AND are not fenced

Figure: 4.5.6showing the question made on the database for vacant_ area less than or equal to 901.93m2and are not fenced

Figure: 4.5.7showing the result of the query made on the database for vacant_ area <= 901.13m2and are not fenced in form of a table

Figure: 4.5.8 showing the result of the query made on the database for vacant_ area <= 901.93m2and are not fenced on the plan.

Discussion of Result

Figure 4.5.7 and 4.5.8 Shows the vacant area less than or equal to 901.93m2and are not fenced on the plan It  consists of the syntax  model  or  the  query  builder  box,  attribute  table  as  well  as  the  map  of  the selected plot in mint blue color. The result shows that 5 vacant_areaout of the 12vacant _area have an area less than 901.93m2 and are not fenced.

4.6 RESULTS ANALYSIS

The results analysis gotten from this project can be used by authority in charge to facilitate planning on the land available to also know certain features about the area in question such as the type of building, the purpose of the building etc. The pie chart shows the representation of the features that existing the project site, it also shows the percentage of the each features.

Table 4.1: RESULTS ANALYSIS​

| LOCATION | SIZE | PERCETAGE |
| --- | --- | --- |
| Total built Up Area | 1285.2724 | 42.3% |
| Vacant area | 48165.342 | 52.2% |
| Total Road Distance (meters) | 1685.195 | 5.5% |

TOTAL ROAD DISTANCE

TOTAL BUILT-UP AREA

VACANT AREA

4.7APPLICATION OF PRODUCT

- It facilitates the use of GIS for effective land management and physical planning.

- It can be used for distribution of infrastructural facility in the study area.

- It can be used for utility distribution, planning and operation.

- The product will provide an integrated approach to analysis of land information.

- It can be used for sustainable land development project in the study area

4.8 SPATIAL SEARCH

The database developed/created was queried in order to reduce the spatial variation in the various information categories ranging from built up area, vacant area, electric poles, and roads (as revealed by the database created) were used for the information system.

4.9 SPATIAL DEPENDENCE

Information system is not the same in every place, even within the project area, to talk of the whole institution neither the whole state, therefore, to detect the information increment/decrement within Part of Asa Damarea,Agaka ( which is the study area) the strength of spatial relationships between the attributes must be established, the strength is known as "Spatial Dependence" and is based on Waldo Tabler's "First Law of Geography" whereby everything is related to everything else, but closer things are more related. Spatial dependence must be measured to establish a relationship limit between the environment and how the facilities is used by the people. This dependence willlikely changes over the study area as the environmental factors change (Haining, 2003). This is known as a spatial process, and when it changes across space it will non-stationery. This cannot be measured just by visually determining what is on ground because it is subjective and this must be done with a scientific approach to achieve a more concrete and precise information.

Screen-shot showing the database created for the study area

CHAPTER FIVE

5.0 COSTING, PROBLEMS ENCOUNTERED, RECOMMENDATIO AND CONCLUSION

5.1	COSTING

The costing of this project was done using the Nigeria Institution of Surveyor’s (NIS) professional scale of fees for consultant in the construction industry. This stages how the total cost that was spent on the project from day one to the final stage.

5.1.1	RECCI

| PERSONAL | DAYS | RATE | TOTAL |
| --- | --- | --- | --- |
| SEN SURVEYOUR | 1 | 15,000 | 15,000 |
| ASS SURVEYOUR | 1 | 8,000 | 8,000 |
| TRANSP0RT | 1 | 7,000 | 7,000 |
| BASIC EQUIPMENT | 1 | 8,000 | 8,000 |
|  |  |  | 38,000 |

5.1.2 BEACON=2,100X5=10,500

5.1.3 BEACONING

| PERSONAL | DAYS | RATE | TOTAL |
| --- | --- | --- | --- |
| ASS SURVEYOUR | 1 | 8,000 | 8,000 |
| BASIC EQUIPMEN | 1 | 8,000 | 8,000 |
| TRANSPORT | 1 | 7,000 | 7,000 |
| TOTAL |  |  | 23,000 |

5.1.4 TRAVERSING

| PERSONAL | DAYS | RATE | TOTAL |
| --- | --- | --- | --- |
| ASS SURVEYOUR | 2 | 8,000 | 16,000 |
| BASIC EQUIPMENT | 2 | 8,000 | 16,000 |
| TRANSPORT | 2 | 7,000 | 14,000 |
| TOTAL |  |  | 46,000 |

5.1.5 DATA DOWNLOADING\PL0TING

| PERSONAL | DAYS | RATE | TOTAL AMOUNT |
| --- | --- | --- | --- |
| SEN SURVEYOUR | 2 | 15,000 | 30,000 |
| ASS SURVEYOUR | 2 | 8,000 | 16,000 |
| TRANSPORT | 2 | 7,000 | 14,000 |
| CONSUMABLES | 2 | 7,000 | 14,000 |
| TOTAL |  |  | 74,000 |

5.1.6 INFORMATION PRESANTATION

| PERSONAL | DAYS | RATE | TOTAL AMOUNT |
| --- | --- | --- | --- |
| ASS SURVEYOUR | 1 | 8,000 | 8,000 |
| TRANSPORT | 1 | 7,000 | 8,000 |
| TOTAL |  |  | 15,000 |

SUB TOTAL =206,500

5.57 CONSTIGENCY

206,500×5%÷100=10,325

5.58 VAT=7.5

206.500×7.5%÷100=15,487.5

5.59 ACCOMODATION

206,500×1.5%÷100=3,097.5

5.5.10M0BILIZATION AND DEMOBILIZATION

206,500×10%÷100=20,650

206,500

10,325

15,487.5

3,097.5

20,650

256,059

5.2 	Problem Encountered

During the execution of the project the main problem encountered was in the aspect ofsocial survey. Residents refused to give out information about their land due to the fact that they were not notified before and took serious permission before they could comply,some were not around during our visit to their house

5.3 	Recommendations

The following recommendations are suggested after having executed the project successfully:

i. The application of the product should be used for planning, the integrity of the job must be maintained and provision of back-ups storage and maintenance of the database must be ensured at all times.

ii. There should be reference to any cadastral information system before any physical development on a land.

iii. A copy of produced map and its usage should be presented to the site used for

documentation on their part and usage as at when due.

iv. There must be a routine and systematic documentation of all spatial database for all projects carried out by the students for future reference and usage.

5.4	Conclusion

The project based on incorporating the digital technology into land administration to propagate the switching from convectional or analogue system of storing, assessing, manipulating and retrieving land data. The techniques and general procedures for cadastral information system were fully described in the study. The study also shows how computer technology has come to play a vital role in keeping land records by making possible for information to be kept in different formats, allowing them to be assembled in any desired manner by individuals, ministry or parastatals that are directly involves with land related matters in satisfying their required needs.

REFERENCES

Aronoff, S. (1991): Geographic information system; A management perspective WDI Publication, Ottawa, Canada. P41.

Arslanoglu and Ozcelik (2005): An Integration of GIS and Remote Sensing in Ground Water Investigation: A case study in Burdur, Turkey; Hydrogeology Journal, Vo1 13, Pp. 826-834.

BartelsM.andWeiH.(2006):DTM Generation from LiDAR data using Skewness Balancing. In 18th International Conference on Pattern Recognition.

Derby, R.A,I. TenDam& C. Van Westwrn, (1999): “Principle of Geographical Information Systems”. ITC Core Module Textbook, Section 4.6.1 to 4.6.5

Genovese I. (2005): Definitions of Surveying And Associated Terms, The American Congress on Surveying and Mapping (ACSM), 314pp, ISBN 0-9765991-0-4.

Ghilani, C. D. and Wolf, P.R. (2008):Elementary Surveying, An IntroductiontoGeomatics,Twelfth edition, Pearson Education Inc., Upper Saddle River, New Jersey, P1.Igor Vilus (2010):Croatian Topographic Information System (CROTIS), Using Airborne Light Detection and Ranging (LIDAR) Technology, PE & RS, Vol. 66, No. 8.

Knabenschuh M. and PetzoldB. (1999): Data post-processing of Laser Scan Data for county wide DTM proguction. In D. Fritsch and R. Spiller (eds). Photogrammetric week 99,WichmannVerlag, Heidelberg.

KnabenschuhM.andPetzold.B. (1999):Data post-processing of Laser Scan Data for countywide DTM proguction. In D. Fritsch and R. Spiller (eds). Photogrammetric week 99,WichmannVerlag, Heidelberg.

Kufoniyi and Bouloucus T. (1994):Flexible Integration of Terrain Objects and DTM investorGIS, ITC Journal Vol. 2 1994 p. 38.

Kufoniyi, O. (1998):Database Design and Creation in C.U. Ezeigbo, Principles and Applicationof Geographic Information System. Pp. 46.

Li, (1994); Maume et al., (2001); Li et al., (2005) Jobin, (2010):Digital Terrain Modellingprinciples and methodology. Pp. 7 - 9. CRC Press. Boca Raton.

Maras (1993) &habib (2015): Digital Terrain Model Product; M.sc. Thesis (in Turkish), SelcukTechnical University, Konya.

Mchibbon&Eyton, J.R. (1996): Rate of change maps, Cartography and GeographicInformation System. School of Geographic Science, University of Bristol vol. 18(2), Pp. 87-103.

Microsoft Encarta Dictionary (2009): Redmond, WA: Microsoft Corporation, 2009. (Definitionof Digital, Surface and Modelling).

Salau K. (2010): Digital Terrain Model for Government Technical College, Awe. Anunpublished HND project report submitted to the department of Surveying and Geo-Informatics,Federal School of Surveying Oyo, Pp 21.

Toutin, T. and Gray, L. (2000): "State-of-the-Art of Elevation from Satellite SAR Data", ISPRSJournal of Photogrammetry & Remote Sensing, vol. 55, Pp. 13-33.

Wood (1999): The Geomorphological Characteristics of Digital Elevation Model. Phd LeicesterUniversity, Leicester.

BLD1

668626.797,934915.257

668655.239,934900.925

66 8636.939,934862.120

66 8606.291,934878.436

BLD 2

668582.135,934923.274

668990.002,934936.997

668613.526,934922.098

668606.012,934 909.257

BLDЗ

667581.284,934925.320

668574.434,934928.791

668577. 656,934934.690

668569.118,934939.551

668565.190,934934. 563

668557. 551,934938.348

668568.729,934953.110

668589.761,934939.436

BLD 4

668597.814,934905.349

668554.219,934896.160

668532.488,934905.191

668533.837,934908.841

668537.475,934907.355

668539.454,934912. 843

BLD5

668562.080,934896.001

668572.247,934893.172

668567.945,934877.615

668557.656,934880.406

BLD6

668543.561,934895.656

668544.532,934897.729

668552.168,934894.888

668548.215 ,934885.003

668530. 032,934 892.242

668533.088,934899.629

BLD7

668544.978,934870.182

668521.971,934877.929

668526.061,934889.496

668548.770,934881.029

BLD8

668559.781,934851.894

668544.682,934 886.634

668546.155,934862.120

668542.735,934863.153

668543.623,934866•627

668547.081,934865.723

668551.651,934880.294

668566.475,934875.936

BLD9

668562.491,934 834.992

668964.027,934837.372

668567.796,934837.618

668563.342,934825.139

668536.459,934836.463

668539.937,934843.647

BLD10

668536.937,934822.969

668533.127,934824.557

668529.634,934817.466

668514.029,934824.958

668519.686,934835.669

668538.900,934826.854

BLD11

668551.965,934816.506

668543.781,934819.752

668545.979,934825.341

668554.082,934822.151

BLDI2

668561.631,934811.073

668552.812,934815.377

668555.474,934820.635

668563.942,934816.676

BLD13

668542.087,934815.310

668949.672,93481.767

668546.497,934804.712

668538.560,934808.416

BLD14

668551.100,934810.930

668560.259,934807.005

668557.080,934799.773

668547.908,934803.653

BLD 15

668529. 661,934775.485

668522.839,934769.770

668512.546,934774.975

668513.168,934776.129

668508.446,934778.377

668510.780,934782.925

BLD16

668529.722,934772.970

668540.941,934767.996

668535.449,934753.535

668523.690,934758.048

BLD17

668491.411,934779.638

668521.573,934768.525

668517.552,934757.413

668486.436,934768.314

BLD 18

668478.664,934755.096

668502.586,934747.359

668498.769,934735.803

668475.179,934742.057

BLD19

668487.983,934130.863

668489.730,934736•840

668499.189,934733.242

668494.121,934717.669

668468.507,934725.914

668471.612,934736.6396

BLD20

668472.336,934719.302

668499.935,934711.471

668495.831,934692.394

668467.361,934705.332

BLD21

668467.256,934702.792

668495.090,934695•066

668490.856,934680.779

668463.763,934689.140

BLD22

668525.636,934743.364

668538.477,934739.131

668529.446,934705.264

668515.476,934710.062

BLD 23

BLD24

668559.452,934742.918

668593.208,934710-062

668557.124,934738.618

668563.322,934793.646

BLD25

668564.451,934773.631

668568.684,934772.572

668562.775,934754.757

668558.542,934756.433

BLD26

668570.724,934771.418

668581.413,934767.397

668574.005,934741.785

668563.422,934746.230

BLD27

668583.107,934767.820

668593.161,934764.539

668585.485,934738.500

668573.613,934741.282

BLD 28

668561.760,934729.818

668574.249,934726.854

668568.110,934702.089

668555.182,934705.424

BLD29

668587.315,934693.524

668575.195,934696.281

668576.094,934702.166

668570 476,934704.754

668577.928,934711.276

668578.490,934709.230

668580.112.934713.993

668591.013,934710.747

BLD 30

668607.730,934760.304

668625.546,934755.189

668619.141,934734.899

668601.380,934741•254

BLD31

668610.692,934731.438

668611.256,934733.153

668617.942,934731.041

668613.621,934716.224

668595.629,934721.868

668601.007

934733.501,934734.885

BLD 32

668624.192,934728.371

668645.831,934721.322

668 640.723,934705.011

668618.490,934712.150

BLD33

668568.579,934698.250

668659.944,934702.312

668666.124,934699.505

668660.908,934683.022

668657.963,934681.152

668462.464,934686.534

668648.293,934702.269

BLD 34

668672.141,934850.381

668674.343,934847.763

668678.185,934850.416

668684.180,934847.276

668679.638,934838.563

668668.569,934843.501

BID 35

668674.710,934829.943

668683.830,934827.209

668679.002,934813.262

668670.271,934816.702

BLD36

668674.467,934801.820

668656.126,934810.275

668 659.857 ,934819.077

668 677.052,934810.800

BLD 37

668650.907,934823.276

66865.185 ,934820.211

668654.187 ,934809.391

668646.128,934812.549

BLD38

68642.147, 934819.631

668829.937,934821.071

668631.150,934 823.646

668 634.793,934 822.211

668635.876,934825.218

66864.324,934821-704

BLD 39

668630.751,934795-092

668622.108,934800.031

668630. 574,934817.846

668639.747,934812.907

BLD 40

668643.627,934810.347

668653.152,934 805.940

668645. 038,934788.124

668 635.954,934793.151

BLD 41

668656.768,934783.009

668647.419,934787.419

668655.445,934804.881

668664.706,934800•031

BLD 42

668669.556,934775•248

668660.649,934779.393

668668.939,934797.385

668677.406,934793.592

BLD 43

668682.962,934768.369

668672.202,934773.749

668680.228,934791.140

668690.635,934786.937

BLD44