KWARA STATE POLYTECHNIC, ILORIN
INSTITUTE OF ENVIRONMENTAL STUDIES
DEPARTMENT OF SURVEYING AND GEO-INFORMATICS

A PROJECT REPORT ON :

UTILITY SURVEY

OF

ROYAL VALLEY ESTATE

AT KUNLENDE/AKEREBIATA ROAD, ILORIN ILORIN EAST LOCAL GOVERNMENT AREA KWARA STATE

SUBMITTED BY:   AWOLUSI TAIWO IREYEMI              MATRICULATION: ND/23/SGI/FT/066

SUPERVISOR IN CHARGE: SURV. DIRAN FELIX & SURV. ADEBANJI

INSTITUTION: INSTITUTE OF ENVIRONMENTAL STUDIES

DATE: JUNE, 2025

CERTIFICATE

I, AWOLUSI TAIWO IREYEMI with Matric Number ND/23/SGI/FT/066 hereby certify that the information contained in this project report were obtained as a result of observations and movements taken by me and the utility Survey  was done in accordance to Surveying rules and regulations and Departmental instructions.

Signature of student:  			___________________________

Name of student:				___________________________

Date of completion: 			___________________________

Matric Number: 				ND/23/SGI/FT/066

CERTIFICATION

This is to certify that AWOLUSI TAIWO IREYEMI with Matric number ND/23/SGI/FT/066 carried out this project work and has been approved as meeting the requirement for the award of National Diploma (ND) in Surveying and Geo-informatics in the Department of Surveying and Geo-informatics of the Institute of Environmental Studies, Kwara state polytechnic, Ilorin.

Project supervisor							_____________

SURV. DIRAN FELIX	 			                    Signature and Date

Project Cordinator 							_____________

SURV. AWOLEYE RAPHAEL. S

Signature and Date

Head of  Department 						_______________

SURV. AMBIMBOLA ISAU					Signature and Date

______________

External Moderator 						Signature and Date

SURV. J.O OPALEYE

................................

DEDICATION

This project is dedicated to Almighty God, the creator of heaven and earth, ancient of days, the I am that I am the beginning and the end whose supremacy in the knowledge of everything is absolute

ACKNOWLEDGMENT

I will like to express my profound appreciation to almighty God who in his infinite mercy gave me the ability to commence and complete my program in peace and God health, if not for God, where will I be today? All things are possible through him for the knowledge, wisdom and understanding and also his moral, protection and provision on me through my ND programme thank God for the great things he has done, most especially for the success of this project work God be the glory.

My appreciation goes to my lovely parent in person of  MR & MRS AWOLUSI for their prayer and support, financially and morally throughout this journey thank for all the beautiful star and end through my ND program. I know I can't repay you back but I have special prayer to offer you both, wishing you both long life and prosperity in good health and wealth, more success, blessing and happiness along with a lot of luck

Less I forget to all my siblings and love once starting from Mrs Apata omoboja,Adeeko oluwatosin,Adeeko Adeniyi, Awolusi Raphael,ayeni oluwakemi, and Dada timilehin testimony may God Almighty continue to bless and protect you all.

And to my supervisors SURV. DIRAN FELIX AND SURV. ADEBANJI for your time taken and guidance throughout this year project I really appreciate it all, and to all the lecturers in the department  from my H.O.D Surv. Abimbola isau, Surv. A. Ayuba, Surv.  Bello Felix Diran,  Surv. Williams Kazeem, Surv. A.O. Akinyede, and also the Director of  special duty

Surv.  A.G. Aremu and other supportive staff of the department of  Surveying and  Geo-informatics, Kwara State Polytechnic, Ilorin.

TABLE OF CONTENTS

Title page										i

Certificate										ii

Certification										iii

Dedication										iv

Acknowledgement									v

Abstract										vi

Table of content 									vii

CHAPTER ONE

1.0	INTRODUCTION								1

1.1	BACKGROUND TO THE STUDY						1

1.2	STATEMENT OF THE PROBLEMS					2

1.3	AIM AND OBJECTIVES							3

1.4	AIM										3

  - OBJECTIVES									3

  - PROJECT SPECIFICATION							4

  - SCOPE OF THE PROJECT							4

  - PERSONAL INVOLVED							5

  - STUDY AREA								6

CHAPTER TWO

- LITERATURE REVIEW							6-10

CHAPTER THREE

- METHODOLOGY								11

  - RECONNAISSANCE

    - OFFICE PLANNING								11

    - FIELD RECONNAISSANCE						12

    - MONUMENTATION								13

  - EQUIPMENT USED								13

    - TEST OF INSTRUMENT							14

    - COLLIMATION TEST FOR TOTAL STATION				14

    - FORWARD COMPUTATION OF THE CONTROL USED		15

    - DATA ACQUISATION							16

    - GEOMETRIC DATA ACQUISATION					16

3.2.6	PERIMETER TRAVERSING						20-22

3.2.7	DETAIL SURVEY								23

CHAPTER FOUR

4.0 DATA PROCESSING								24

4.1.1 RESULT ANALYSIS								24

4.2 BACK COMPUTATION								24

4.2.1 COMPUTE FOR TOTAL AREA USING DOUBLE LATITUDE AND DEPARTURE				 						25-26

4.2.2 GRAPHICS PLOTTING							27

CHAPTER FIVE

- SUMMARY, CONCLUSION AND RECOMMENDATIONS 			28

  - SUMARRY 									28

  - PROBLEM ENCOUNTERED 							28

  - CONCLUSION									28

  - RECOMMENDATION								29

REFFERENCES

ABSTRACT

This project is based on utility survey of  ROYAL VALLEY ESTATE

at kunlende/akerebiata road, ilorin ilorin east local government area kwara state

This project has been divided into different chapters.Chapter one of this project gives an introduction about the project topic as a whole.This enablesbetter understanding of the project so as to know what perimeter and detailing entails as well as the scope and aim of the project.Chapter two is the literature review i.e. the works of past professionals and projects that had been done in the past as regards this specific topic which were examined in order to help to shed more light on what perimeter and detailing survey is all about. Chapter three of this project is the methodology which describes how the project was carried out using digital instrument e.g. Total Station and its accessories from the first stage to the final stage.Chapter four of this project is Data presentation which consists of all the data acquired from site during the project execution. Chapter five comprises of summary, problems encountered, conclusion and recommendations. Reference comprises of list ofnames and works of prominent authors or professionals whose works on survey as regards this specific topic were used in the course of this project.

1.0 INTRODUCTION

The rapid growth of urban areas globally has led to a significant increase in the complexity and interdependence of utility systems. Underground utilities such as water, electricity, gas, and telecommunications form the backbone of urban infrastructure, supporting daily activities and enabling economic development. However, as cities expand, the risk of damaging existing utility lines during construction and development projects escalates. Accurate knowledge of the location and condition of these utilities is paramount in facilitating safe and efficient construction practices. Utility surveys have thus emerged as essential processes that provide crucial information about existing utility systems. This study aims to explore the principles, methodologies, and technologies underpinning utility surveys, emphasizing their importance in effective infrastructure management.

A utility survey is a systematic process that involves investigating, mapping, and assessing the presence, location, and condition of underground utilities such as water, gas, electricity, sewage, and telecommunications networks. This survey employs various techniques, including electromagnetic locating, ground-penetrating radar (GPR), and traditional mapping methods, to provide critical data essential for safe construction, urban planning, and infrastructure maintenance. By identifying existing utilities, a utility survey helps prevent accidental damage, ensures compliance with regulations, and facilitates informed decision-making in development projects.

In the modern landscape of urban development, infrastructure planning plays a critical role in ensuring efficient and sustainable growth. Among the various elements that contribute to this planning, utility surveys stand out as essential tools that provide vital data necessary for the design, construction, and maintenance of various public and private utilities. This comprehensive introduction elaborates on the significance, methodology, applications, and challenges associated with utility surveys, while also exploring current trends and future directions in the field, establishing a robust foundation for understanding their vital role in infrastructure management.

SIGNIFICANCE OF UTILITY SURVEYS

Utility surveys are systematic investigations conducted to identify the existence, location, and condition of underground utilities such as water lines, gas pipelines, electricity cables, telecommunications networks, and sewer systems. These surveys are critical for assisting engineers, architects, contractors, and planners in making informed decisions regarding construction and renovation projects. A well-executed utility survey helps prevent costly accidents, minimizes project delays, and ensures compliance with local regulations and safety standards.

As urban areas continue to swell, the demand for accurate utility data becomes increasingly urgent. Infrastructure networks, particularly in densely populated cities, have become a complex web of interdependent systems. Consequently, overlooking the presence or condition of existing utilities during construction activity can lead to severe consequences, including service disruption, safety hazards, structural damage, and legal liabilities. Thus, utility surveys are not merely beneficial but indispensable in today’s infrastructure ecosystem.

Utility surveys directly contribute to risk mitigation—a critical aspect of any construction or renovation undertaking. By accurately identifying utilities prior to the commencement of work, stakeholders can avoid inadvertent utility strikes, which can lead to catastrophic accidents, such as explosive gas leaks or electrical shocks. The financial repercussions stemming from such incidents can be substantial, ranging from repair costs to potential legal claims from injured parties. Moreover, disruptions caused by utility strikes can result in significant delays to project timelines, incurring additional costs. Thus, conducting thorough utility surveys becomes not just a matter of regulatory compliance but a crucial investment in safeguarding both personnel and infrastructure assets.

A utility survey typically involves several stages, including desk studies, field surveys, and data analysis.

1. Desk Studies: This initial phase involves gathering existing documents, maps, and records from utility providers and local authorities. Geographical Information Systems (GIS) can be employed to visualize existing utility layouts.

2. Field Surveys: Following the desk study, field work is conducted to verify the data collected. This may include techniques such as ground-penetrating radar (GPR), electromagnetic induction, and hydro-excavation. The goal is to accurately locate and map the positions of underground utilities, often marked with flags or spray paint for future reference.

3. Data Analysis: The collected data is analyzed and compiled into comprehensive reports. These reports detail the types, locations, and conditions of utilities, offering insights that inform design teams and stakeholders.

The combination of technological advancements and traditional surveying methods results in precise and reliable data, essential for successful project planning and risk mitigation. Advanced methodologies in utility surveying are transforming the field further:

- Electromagnetic Locating: This technique utilizes electromagnetic fields to detect the presence of metals and other conductive materials buried underground, making it particularly useful for locating power lines and metallic piping.

- Ground Penetrating Radar (GPR): GPR emits radar pulses into the ground, detecting various utilities by interpreting the reflected signals to ascertain the depth, size, and material composition of underground utility systems.

- 3D Laser Scanning: This technology offers high-resolution, three-dimensional maps of the underground infrastructure, enabling more precise planning and collision avoidance during construction activities.

- Drone Surveys: Drones equipped with advanced imaging technology can survey large areas quickly, capturing real-time data on above-ground utilities to complement underground surveys.

APPLICATIONS OF UTILITY SURVEYS

- Utility surveys have a broad spectrum of applications across various sectors, including but not limited to:

- - Construction: Before groundbreaking, a utility survey is conducted to avoid damaging existing utility lines, ensuring worker safety and project timeline efficiency.

- - Urban Planning: City planners utilize utility survey data when designing new developments or retrofitting existing infrastructure, ensuring seamless integration of utilities into the planned layout.

- - Maintenance and Upgrades: Utility surveys are crucial for existing infrastructure maintenance, helping identify areas that require upgrades or repairs, thus prolonging the lifespan of critical assets.

- - Environmental Assessments: Understanding utility layouts helps assess their impact on the environment, which is essential in projects focusing on sustainability and compliance with environmental regulations.

- - Emergency Services: Knowledge of utility configurations is essential for emergency responders, enhancing response efficiency and safety during crises such as fires or floods.

- - Historical Preservation: For projects dealing with historical sites, utility surveys aid in identifying buried utilities, ensuring that preservation efforts do not compromise utility functionality or historical integrity.

- - Utility Management and Smart Cities: As smart cities with interconnected system networks, utility surveys become pivotal in managing infrastructure. Real-time data collection through utility surveys enables better resource monitoring and management, paving the way for efficient energy use and fostering environmental sustainability.

- - Asset Management: Utility surveys are vital for entities managing large-scale utility infrastructures. Accurate and updated utility maps aid in effectively managing maintenance schedules, ultimately improving asset lifespan and performance.

1.2 STATEMENT OF THE PROBLEMS

Despite the recognized importance of utility surveys, several challenges persist that hinder their effectiveness and implementation. One of the primary issues is the lack of accurate and up-to-date data regarding the location and condition of existing utilities, which can lead to unintentional utility strikes during excavation activities, resulting in safety hazards, project delays, and significant financial implications. Additionally, many utility records are outdated or incomplete, and legal barriers often limit access to critical data from utility companies. Furthermore, the increasing complexity of urban infrastructure systems makes existing survey methodologies insufficient at times, necessitating the exploration of more advanced surveying technologies and collaborative approaches. This study seeks to identify and address these problems, highlighting the need for comprehensive utility survey practices in urban development.

1.4 AIM

The primary aim of this study is to investigate the methodologies and technologies of utility surveys, their applications, and the challenges faced in accurately mapping underground utilities in urban settings. The study will explore the implications of these surveys for improving infrastructure management and safety during construction activities.

1.5 OBJECTIVES

- 1. To review current methodologies and technologies employed in utility surveying, such as ground-penetrating radar (GPR) and electromagnetic locating.

- 2. To assess the effectiveness and limitations of existing utility survey practices in urban environments.

- 3. To explore the implications of utility surveys for urban planning, construction safety, and emergency management.

- 4. To identify challenges faced in accessing utility information and propose strategies to enhance data accuracy and collaboration among stakeholders.

- 5. To analyze emerging trends and technological advancements that could improve the efficiency of utility surveying in the future.

1.6 PROJECT SPECIFICATION

This study will focus on a comprehensive analysis of utility surveying methodologies, including both traditional and advanced technologies, covering:

- Theoretical frameworks that underpin utility surveys.

- Practical case studies illustrating the application of utility surveys in urban development.

- A review of technological advancements that integrate GIS, AI, and drone technology.

- An examination of the regulatory environment affecting utility surveys.

- Recommendations for best practices in utility surveying to ensure safety and efficiency.

1.7 SCOPE OF THE PROJECT

The scope of this project will encompass an extensive review of literature and case studies related to utility surveys in urban contexts. The focus will be on geographically diverse urban areas to understand how local regulations, technological adoption, and infrastructure complexity impact surveying practices. The study will also examine the integration of utility surveys in urban planning and emergency management, alongside an assessment of future trends that may reshape the industry. However, the research will primarily concentrate on the methodologies and technologies relevant to utility surveys, while recognizing that factors such as social, political, and environmental influences also play a significant role in infrastructure development.

1.8	PERSONAL INVOLVED

| S/N | NAMES | MATRIC NUMBER | ROLES |
| --- | --- | --- | --- |
| 1 | AWOLUSI TAIWO IREYEMI | ND/23/SGI/FT/066 | AUTHOR |
| 2 | OLAFIMIHAN EMMANUEL . O | ND/23/SGI/FT/083 | MEMBER |
| 3 | MAFTAU OMOWUNMI AISHAT | ND/23/SGI/FT/054 | MEMBER |
| 4 | OLAWALE AZEEZ .O | ND/23/SGI/FT/055 | MEMBER |
| 5 | RAHMAN ZAINAB OLAMIDE | ND/23/SGI/FT/064 | MEMBER |
| 6 | MUHAMMED AISHAT | ND/23/SGI/FT/058 | MEMBER |
| 7 | BABALOLA OLAMIDE GRACE | ND/23/SGI/FT/062 | MEMBER |

1.9	STUDY AREA

The study area is located at royal Valley estate, At Kunlende/akerebiata road, ilorin ilorin East Local government area Kwara State

Map of Nigeria

Map of Ilorin.                           Map of study area

# CHAPTER TWO LITERATURE REVIEW ON UTILITY SURVEYS

The significance of utility surveys in preparation for construction and urban development has been widely acknowledged across various academic and professional domains. This literature review aims to synthesize existing research, methodologies, technologies, and applications related to utility surveys while highlighting the challenges and emerging trends within this field.

# Historical Context

Utility surveys have evolved significantly over the past few decades. Historically, the documentation and mapping of underground utilities were often based on incomplete or outdated records due to a lack of comprehensive survey methodologies (Schmidt et al., 2003). As urbanization increased, the limitations of traditional surveying methods became apparent, necessitating advancements in tracking and managing the complex web of utilities beneath urban landscapes. Early studies emphasized the need for improved accuracy and the integration of high-tech solutions to assess utility systems effectively (Morris & Heneghan, 2004). The growth of urban populations and the subsequent expansion of utility networks have underscored the importance of precise utility mapping.

# Methodologies in Utility Surveys

A plethora of methodologies has been developed to enhance the accuracy of utility surveys. Ground-Penetrating Radar (GPR) is one of the most extensively researched technologies. Several studies have demonstrated the effectiveness of GPR in detecting utilities with various materials and diameters (Baker & Hernandez, 2009; Pires et al., 2022). GPR's ability to non-invasively map subsurface utilities has made it a valuable tool in construction and urban planning. Other methods, such as electromagnetic induction, have received attention for their ability to map magnetic fields and locate metallic utilities (Geyer et al., 2020). This technology has proven particularly useful in areas with dense networks of metallic utilities.

Additionally, the implementation of Geographic Information Systems (GIS) has revolutionized the reporting and visualization of utility data. GIS allows for the integration of spatial data, improving the management of utility systems and their relationships with the urban environment (Liu et al., 2018). The use of GIS has enabled utility providers and urban planners to better understand the spatial

relationships between utilities and other urban infrastructure. The rise of

three-dimensional modeling has also improved the visualization of complex utility layouts, enabling planners to recognize conflicts and hazards more easily (Sheng et al., 2019). This technology has been particularly useful in planning and designing new urban developments.

# Applications of Utility Surveys

Utility surveys are intensely applied in various sectors, underscoring their multifaceted importance. In construction, the integration of utility surveying data into planning processes has been linked to reduced risks of accidental utility damage and improved worker safety (Pawlukiewicz et al., 2005). By accurately mapping utilities, construction teams can avoid costly and potentially disastrous utility strikes. Urban planners and local authorities frequently utilize survey data to inform land use decisions, refine infrastructure designs, and minimize disruption during new development projects (Federico et al., 2021). Utility surveys have become an essential component of urban planning, enabling cities to grow and develop sustainably.

Utility surveys have also emerged as crucial in the field of emergency management, where accurate utility data is essential for responders during crises (Levine et al., 2015). Understanding utility layouts can significantly enhance response efficiency and safety; therefore, it is increasingly integrated into emergency preparedness programs. By having access to accurate utility data, emergency responders can better navigate complex urban environments and respond to emergencies more effectively.

# Challenges in Utility Surveys

Despite advancements, the literature identifies several persistent challenges in utility surveying. Data accuracy remains a significant concern, particularly when relying on existing utility records that may not be up-to-date or comprehensive (Santos et al., 2016). The prevalence of undocumented utilities further complicates survey efforts while increasing the risks of accidental strikes during excavation (Rogers et al., 2018). In many cases, utilities are installed without being properly documented, making it difficult for surveyors to accurately map them.

Additionally, issues related to data confidentiality often hinder access to essential utility information. Utility providers may be reticent to share data due to competitive concerns or regulatory constraints, and this translates into incomplete surveys that

potentially compromise safety and efficiency (Meyer et al., 2017). The lack of standardization in utility data collection and sharing has also been identified as a significant challenge. Different utility providers often use different data formats and collection methods, making it difficult to integrate data from multiple sources.

# Emerging Trends and Future Directions

Recent literature illustrates several emerging trends and future directions for utility surveys, which focus on enhancing efficiency and accuracy. One notable trend is the growing integration of artificial intelligence (AI) in utility surveying. AI techniques enable predictive modeling and data analysis, allowing for more intelligent network management and optimized utility placement (Zhou et al., 2020). AI has the potential to revolutionize utility surveying by enabling surveyors to analyze large datasets and identify patterns that may not be apparent through traditional methods.

Moreover, the adoption of drone technology in utility surveys has been increasingly documented. Drones can cover vast areas in a short amount of time while collecting high-resolution .

Drones can cover vast areas in a short amount of time while collecting high-resolution aerial data, thereby complementing traditional surveying methods (Kumar et al., 2022). This fusion of technologies has been recognized as a promising step toward comprehensive utility management solutions. Drones equipped with sensors and cameras can capture detailed images of utility infrastructure, enabling surveyors to identify potential issues and prioritize maintenance.

The conversation around sustainability in urban development has also gained traction. Researchers are advocating for utility surveys that incorporate environmental impact assessments as part of the situated planning process (Harlow et al., 2019). This aligns with global initiatives aiming to increase the sustainability of urban infrastructure while balancing utility needs with ecological considerations. By integrating environmental impact assessments into utility surveys, planners can identify potential environmental risks and develop strategies to mitigate them.

Furthermore, the use of Building Information Modelling (BIM) has been explored in the context of utility surveys. BIM enables the creation of detailed digital models of utility infrastructure, allowing for more accurate planning and management (Eastman et al., 2011). The integration of BIM with GIS and other technologies has the potential

to revolutionize utility surveying by enabling surveyors to create detailed, data-rich models of utility infrastructure.

# Recommendations for Future Research

Future research should focus on developing more accurate and efficient methods for detecting and mapping utilities, particularly in areas with complex or undocumented utility networks. Additionally, researchers should explore the potential applications of emerging technologies, such as AI and drone-based surveying, in utility surveying.

Furthermore, studies should investigate the integration of utility surveys with other urban planning and management tools, such as GIS and BIM, to create more comprehensive and sustainable urban development strategies.

The importance of utility surveys in ensuring the safety and efficiency of urban infrastructure cannot be overstated. As cities continue to grow and evolve, the need for accurate and reliable utility data will only increase. By advancing the field of utility surveying through research and innovation, we can create more sustainable, efficient, and safe urban environments for future generations.

CHAPTER THREE

3.0 	METHODOLOGY

This aspect refers to a system of methods and principles, rules used in a field of study to achieve the aim and objectives of this project. The modality to successful execute this project was carried out based on the survey rules and regulation, supervisor advice and departmental instructions.

3.1 	RECONNAISSANCE

Reconnaissance survey is a preliminary stage for exploratory investigations conducted to gather initial data concern the choice of Site area of Subject before more detailed research or filed work commenced. Reasons for reconnaissance survey of the project area are the following;

- To know the location of the site.

- To determine the preliminary area/perimeter of the land.

- To know the survey method suitable for the field data acquisition and the instrument need for the it.

Reconnaissance in this project involved preliminary investigations to gather information about the survey area before conducting a detailed survey, reconnaissance of this project was carried out in two way namely as follows.

- Office planning

- Field Reconnaissance

3.1.1 	OFFICE PLANNING

It involved the process adopted in the collection of relevant data and specification about the as-built Surveying with the used of traversing survey method. During the office planning the given coordinate for control point PBIL 100, PBIL 107, and PBIL 110, was obtained and computed as shown in the table 3.1 below. The choice of equipment to be use for project was obtained from the survey store in the department of surveying and Geoinformatic, Kwara State Polytechnic, Ilorin, and it was handled accordingly to the survey rules and regulations and departmental instruction.

TABLE 3.1 Showing the list of the coordinate of the control pillars used

| Stations | Northing (m) | Easting (m) |
| --- | --- | --- |
| PBIL 100 | 939353.130 | 648821.110 |
| PBIL 107 | 939306.460 | 648932.560 |
| PBIL 110 | 939224.969 | 649120.403 |

3.1.2 	FIELD RECONNAISSANCE

Field recce and site visitation was conducted as preliminary survey for investigation of the basic information about a specified project area, location, or subject in the field. It allows the members of the groups to be familiar with the terrain, boundary lines, selection number of station required, Equipment that will be used for the job, pillars number PBIL 100, PBIL 107, and PBIL 110, used as a control for the project and so on. The stations points selected was Inter-visible to one another and finally a reconnaissance diagram was produced which was attached to this project

RECCE DIAGRAM

3.2	INSTRUMENT SELECTION

The equipment used for this project was grouped into two categories;

- Hardware

- Software

3.2.1	Hardware used

- Total Station with its Accessories (KTS-442L)

- Reflector poles

- Field book

- Writing material (pen)

- Survey beacons

- Steel Tape (5m)

- Tripod Stand

3.2.2	Software used

Below are the list of the software used during the project processing;

- AutoCAD 2007

- Notepad

- Microsoft word

- Microsoft Excel

3.3 	INSTRUMENT TEST

Total station

LINE OF SIGHT

RANGING POLE

20M

STATION  X

TERRAIN

STATION  Y

The instrument test was carried out before it used for angular observation. The purpose of testing the instrument before used is to know the assurance and maybe other instrument is in good Condition the instrument was set up over station A and it was pointed a Sighted to station B whore a pole Is hold, and the angle was recorded for face left the instrument was turn to farce right on the same point and it was sighted to station B, where a pole is held at the same position and the angle was recorded. The distance between station A and B 1s 10m. The steel tape was hold at 60m an point A and it was extend to point B. the results for the instrument test was shown i to figure

3.3 below and table 3:2 below.

Figure 3.1 showing the instrument set up for collimation test

Table 3.2 showing the test of instrument result

| STATION | SIGHT | FACE | HORIZONTAL READING | DEDUCED ANGLE |
| --- | --- | --- | --- | --- |
| A | B | L | 180° 00’ 03” |  |
|  | B | R | 00°00’ 00” | 180° 00’ 03” |

Different in horizontal circle reading: 180° 00’ 03’’

Horizontal collimation error

= 180° 00’ 03’’

= 000° 00 03’’

Collimation error = 00° 00'03"2

Horizontal collimation error = 00° 00’ 01”

Because the collimation  error falls within the allowable 01”. The instrument was in good condition.

3.4 	MONUMENTATION

This is the process in which the beacon is being buried in to ground. On the project site there are same existing  beacons on the boundary lines m, nail with cap (bottle cork) was used as the 	station pegs.

18cm

15cm above the ground

60cm below the ground

18cm

30cm

Nail

Bottle cork

PILLAR							   PEG

FIGURE 3.4	Diagram and Dimension of Pillars buried

3.5 	CONTROL CHECK

The Control check was carried out to know the control point PBIL 100, PBIL 107, and PBIL 110, are still in their positions. The instrument was set on Control FP01107 and the temporary adjustment was carried out i.e centring, levelling, and focusing. Back site was taken to control PBIL 107 and it was recorded.

The fore sight was served to PEG1 and it was recorded Steel tape was used to measure the distance between PBIL 110 and PEGI and it was recorded. The data observed was computed for given coordinate and Observed Coordinate, as shown in the table 3.2 below.

TABLE 3.2	THE GIVEN COORDINATE OF THE CONTROL POINTS

| STATION | NORTHING (m) | EASTING (m) |
| --- | --- | --- |
| PBI 100 | 939353.130 | 648821.110 |
| PBIL 107 | 939306.460 | 648932.560 |
| PBIL 110 | 939224.969 | 649120.403 |

TABLE 3.3	The Backward Computation for the Given Controls pillars

| From station | Bearing | Distance | ∆N | ∆E | Northing (m) | Easting (m) | To station |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | 899353.130 | 688821.110 | PBIL 100 |
| PBIL 100 | 1120 43̍ 17 ̎ | 120.827 | -46.670 | 111.450 | 899306.460 | 688932.560 | PBIL 107 |
| PBIL 107 | 1130 27̍ 09 ̎ | 204.758 | -81.491 | 187.843 | 899224.969 | 689120.403 | PBIL 110 |

PBIL 100 to PBIL 107

BEARING = 1120 43̍ 17 ̎

DISTANCE = 120.827m

PBIL 107 to PBIL 110

BEARING = 1130 27̍ 09 ̎

DISTANCE = 204.758m

TABLE 3.4	Show the Observed Coordinate of the Controls Pillars

| Station | Northing (m) | Easting (m) |
| --- | --- | --- |
| PBIL 100 | 939353.126 | 648821.105 |
| PBIL 107 | 939306.460 | 648932.560 |
| PBIL 110 | 939224.969 | 649120.403 |

TABLE 3.5	Show the Backward Computation for the Observed Control Pillars

| From station | Bearing | Distance | ∆N | ∆E | Northing (m) | Easting (m) | To station |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | 899353.126 | 688821.105 | PBIL 100 |
| PBIL 100 | 1120 43̍ 08 ̎ | 120.830 | -46.666 | 111.455 | 899306.460 | 688932.560 | PBIL 107 |
| PBIL 107 | 1130 27̍ 09 ̎ | 204.758 | -81.491 | 187.843 | 899224.969 | 689120.403 | PBIL 110 |

PBIL 100 to PBIL 107

BEARING = 1120 43̍ 08 ̎

DISTANCE = 120.830m

PBIL 107 to PBIL 110

BEARING = 1130 27̍ 09 ̎

DISTANCE = 204.758m

TABLE 3.6	Show the Difference between the Given Control Points and the Observed Control Pillars

| STATION | NORTHING (m) | EASTING (m) |
| --- | --- | --- |
| PBIL 100 | 0.004 | 0.005 |
| PBIL 107 | 0.000 | 0.000 |
| PBIL 110 | 0.000 | 0.000 |

Therefore the difference between the given and observer bearing was 00° 00’ 09” and 00° 00’ 00” and the difference between the given and the observer linear distance was 0.0003m, 0.000m the results obtained fall within the allowable accuracy for both angular and linear observation.

It was concluded the instrument was in good condition and fit to be used for the project.

3.6 	DATA AQUISITION

This was carried out on the field using Total station with its accessories (KTS-442L) was used for coordinating the entire site for both the perimeter and details of the site. The Total station was set on the 	control Pillar PBIL 107, and the temporary adjustment was carried out centering, leveling and 	focusing. The coordinates of the occupied station was input after that the coordinates of the backsight was record as well and the observation was done to backsight pillar and the coordinates was take and check against the given. Having check and found accurate then the perimeter survey commenced for the entire project site.

3.6.1 	DETAILING SURVEY

The detailing survey was done immediately after completely carried out the perimeter traverse to determine the position of the features on the Site was fixed using radiation survey method. During this several stations was established to pick the coordinates of the details and the coordinates was stored on the total station as its observed. This was later downloaded. The details that was picked during the detailing survey was coded in different ways like Building edges as ‘BLD’, Electric Poles ‘EP’, Drainage ‘DR’, Roads Line ‘RD’, Spot Height ‘SP’ and so on.

CHAPTER FOUR

4.0   	DATA PROCESSING

This involved several step to ensure accurate and reliable result. It explains the method used in capturing; processing, downloading, and analyzing the data acquired Data reduction in surveying involves various methods to simplify and summarize the collected data for analysis. These are some common methods of data reduction that involves

- Combining multiple values into a single value. The average value is calculated by summing up all the values and dividing by the number of values.

- Least Square Adjustment (LSA) is a method of data reduction that involves minimizing the sum of the squared errors between observed and calculated values.

- Interpolation is a method of data reduction that involves estimating values between observed data points.

- Spatial data reduction refers to the process of simplifying and reducing the complexity of spatial data, such as geographic information system (GIS) data, while preserving its essential characteristics.

4.1	DATA EDITING

- Open excel

- Click on the desire file

- Set to delimited

- Click on next

- Mack space and comma

- Click

- On next

- Then click on finish

4.3	DATA PROCESSING USING AUTOCAD

The plan produced digitally with AutoCAD software. The following are the procedure used:

STEP 1: Open Notepad

Open Notepad on your computer. You can find it in the start menu (Window)

STEP 2: Create a New File

Click on "file" "New" to create a new file

STEP3: Enter Coordinates

Enter your coordinates in the following format:

X. Y. Data

STEP 4: Save the Fil

Click on "File" > "Save as" and choose a location to save your file. "ALADA Coordinate" txt, 	and select "All files" as the file type.

Importing the Coordinate File Into 2007 AutoCAD

STEP 1: Double click AutoCAD

Go under format

Click on format after that pick your unit

Drawing unit

Type.... Decimal

Precision... 0.000

Angle type Degree/Min/See

Precision Od00

Clockwise.... 노

Unit to scale Meters

Direction control North 270d0

Press "OK"

STEP 2: Create a new drawing

Click on "File" > "Name" to create a new drawing

STEP 3: Import the coordinate file

Click on "Tools" >"Reference" > "Import" and select "ALADA coordinate (.txt)" as the file 	type. Navigate to the location where you saved your coordinate file ("ALADA coordinate .txt") 	and select it.

STEP 4: Configure Import Setting. In the import select "Point" as the object type and choose 	the correct coordinate format (eg. X and Y) click "OK" to import the coordinates.

4.4	DATA EXTRACTION

The data recorded in the field book was picked to compute for the bearing and distance. Forward computation and backward computation after all necessary correction have been applied. Correction like slope correction, tension correction and standardization correction.

4.3  	BACK COMPUTATION

The result get for forward computation was used for Back computation.

The following program was required for back computation

i. Input N2-N1 to get ∆N

ii. Input E2-E1 to get ∆E

iii.Input (∆N2+∆E2 to get distance

iv.Input shift tan ∆E∆Nto get the corrected bearing when there us more station repeat step ¡¡¡ and ii.

Table 4.1 Showing Boundary Coordinates

| Station | Northing (m) | Easting(m) |
| --- | --- | --- |
| FPO1107 | 939366.406 | 648932.560 |
| PL1 | 939240.312 | 648901.750 |
| PL2 | 939309.331 | 648780.785 |
| PL3 | 939292.197 | 648603.139 |
| PL4 | 939438.718 | 648619.716 |
| PL5 | 939467.847 | 648693.558 |
| PL 6 | 939353.126 | 648821.105 |

Table 4.1.1 Showing back computation for the boundary coordinate

| FROM Station | BEARING | DISTANCE | ΔN | ΔE | N |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | 89 |
| PBIL 107 | 204 58 29 | 72.971 | -66.148 | 30.810 |  |
| PL1 | 299 42 28 | 139.270 | 69.019 | 120.965 |  |
| PL2 | 295 39 02 | 193.739 | 83.866 | 174.646 |  |
| PL3 | 16 40 37 | 47.520 | 45.521 | 13.637 |  |
| PL4 | 112 42 17 | 79.980 | 30.871 | 73.782 |  |
| PL5 | 113 13 14 | 113.790 | 54.721 | 127.547 |  |
| PL6 | 112 43 59 | 120.870 | 48.666 | 455 |  |

TABLE 4.1.2	showing area computation for the boundary coordination using double latitude

| ΔN | ΔE | product |
| --- | --- | --- |
| 66.148 | × - 30.810 | = 2038.020 |
| 66.148 |  |  |
| 69.019 |  |  |
| -63.277 | × - 120.965 | =7654.302 |
| 69.619 |  |  |
| 83.866 |  |  |
| 89.608 | × -174.646 | -=15649.679 |
| 83.866 |  |  |
| 45.521 |  |  |
| 218.995 | × 13.637 | =2986.435 |
| 45.521 |  |  |
| -30.871 |  |  |
| 233.645 | × 73.782 | =17238.792 |
| -30.871 |  |  |
| -54.721 |  |  |
| 148.653 | × 127.547 | =18883.716 |
| -54.721 |  |  |
| -46.666 |  |  |
| 46.666 | × 111.455 | =5201.159 |
| -46.666 |  |  |
| 00.000 |  |  |

Area =

2038.020+7654.302-15649.679+2986.435+17238.795+18883.716+5201.1592

54002.427-18649.6792

38352.7482

=19176.374=> 1.9 hectares

1.9 Hect meter square

Hectare=19176.37410000

= 1.9176374 Hectare

Linear Accuracy = 1(EL)2-((ED)2TOTALLENGHT

EL=EDN=0.066

ED=EDE=0.072

Total length =793.100

∴LinearAccuracy

1(0.66)2+(0.072)2793.100x = 10.09767793.100

=1:8000

CHAPTER FIVE

5.0 	SUMMARY, PROBLEMS ENCOUNTERRED, CONCLUSION, RECOMMENDATION

5.1 	SUMMARY

5.2 	PROBLEM ENCOUNTERED

- Too much of rainfall which does not allowed site

- Unavailable of the group members due to dean's cup football tournament

- While taking observing there are some object obstructing the view when taking the observation e.g electric pole, building, tree etc.

5.3 	RECOMMENDATION

The department should considered the season and time that the students should be given the terminal project.

5.4 	CONCLUSION

At the end the aim and objective of this project was achieved. It was carried out using third order surveying method this project was carried out in accordance with survey rules and regulation and departmental instruction.

| Name | Code | E | N | h |  | PT260 | SH | 668115.4 | 937791.5 | 317.946 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PT1 | ST17 | 668069.6 | 937900.4 | 324.137 |  | PT261 | SH | 668118 | 937790.7 | 318.041 |
| PT2 | CV | 668071.1 | 937900.4 | 324.553 |  | PT262 | SH | 668117.7 | 937795.6 | 317.967 |
| PT3 | CV | 668072 | 937900.3 | 324.541 |  | PT263 | SH | 668113.5 | 937797.2 | 317.656 |
| PT4 | BLD | 668072.6 | 937900.6 | 324.383 |  | PT264 | SH | 668110.1 | 937800.8 | 317.647 |
| PT5 | BLD | 668066.5 | 937905.8 | 323.724 |  | PT265 | SH | 668110.4 | 937806.9 | 317.571 |
| PT6 | SH | 668069.7 | 937897.9 | 324 |  | PT266 | SH | 668114.1 | 937808.4 | 317.326 |
| PT7 | SH | 668072.6 | 937896.1 | 324.671 |  | PT267 | SH | 668117.8 | 937810 | 317.601 |
| PT8 | SH | 668072.2 | 937891.5 | 324.604 |  | PT268 | SH | 668117.6 | 937814.7 | 317.532 |
| PT9 | SH | 668076.5 | 937892.6 | 324.821 |  | PT269 | SH | 668113.8 | 937816 | 317.212 |
| PT10 | SH | 668083.2 | 937892.3 | 324.831 |  | PT270 | SH | 668109 | 937815.1 | 317.485 |
| PT11 | SH | 668085.6 | 937889.4 | 324.87 |  | PT271 | SH | 668104.5 | 937814.4 | 317.27 |
| PT12 | BLD | 668085.4 | 937886.8 | 324.848 |  | PT272 | SH | 668104.8 | 937819.5 | 317.197 |
| PT13 | SH | 668088.7 | 937888.5 | 324.887 |  | PT273 | SH | 668109.7 | 937818.5 | 317.278 |
| PT14 | SH | 668090.4 | 937893.3 | 324.882 |  | PT274 | SH | 668113.6 | 937819 | 317.192 |
| PT15 | SH | 668095.3 | 937895.1 | 324.957 |  | PT275 | SH | 668117.7 | 937821.2 | 317.334 |
| PT16 | SH | 668095.6 | 937891.4 | 324.958 |  | PT276 | SH | 668114.1 | 937823.2 | 317.152 |
| PT17 | BLD | 668139.4 | 937901.3 | 324.586 |  | PT277 | SH | 668109 | 937824.1 | 317.181 |
| PT18 | CV | 668139.1 | 937901.2 | 316.047 |  | PT278 | SH | 668104.9 | 937822.2 | 317.161 |
| PT19 | CV | 668139.5 | 937900.4 | 316.104 |  | PT279 | SH | 668105 | 937825.4 | 317.107 |
| PT20 | CV | 668137.3 | 937899 | 317.048 |  | PT280 | SH | 668103.5 | 937827.1 | 317.276 |
| PT21 | SH | 668133.7 | 937894.7 | 316.106 |  | PT281 | SH | 668108.1 | 937827.2 | 317.179 |
| PT22 | SH | 668136.7 | 937894.8 | 316.021 |  | PT282 | SH | 668112 | 937827.1 | 317.097 |
| PT23 | BLD | 668137 | 937893.5 | 316.015 |  | PT283 | SH | 668116.6 | 937827.5 | 317.116 |
| PT24 | SH | 668141.1 | 937890.4 | 315.886 |  | PT284 | SH | 668117.8 | 937831.4 | 317.048 |
| PT25 | SH | 668145.9 | 937889.2 | 315.599 |  | PT285 | SH | 668112.9 | 937833.2 | 317.012 |
| PT26 | SH | 668147.8 | 937891.6 | 315.619 |  | PT286 | SH | 668108.8 | 937832.7 | 317.058 |
| PT27 | GT | 668148.7 | 937893.4 | 315.804 |  | PT287 | SH | 668106.3 | 937831.7 | 317.095 |
| PT28 | GT | 668147.9 | 937894 | 315.765 |  | PT288 | SH | 668120.4 | 937780 | 318.343 |
| PT29 | BLD | 668148.9 | 937893.2 | 315.928 |  | PT289 | SH | 668125.9 | 937783 | 318.036 |
| PT30 | SH | 668148.1 | 937886.3 | 315.852 |  | PT290 | SH | 668122.6 | 937785.4 | 318.043 |
| PT31 | SH | 668144.4 | 937882.9 | 315.942 |  | PT291 | SH | 668120.9 | 937789.4 | 318.013 |
| PT32 | SH | 668140.3 | 937879.1 | 316.287 |  | PT292 | BLD | 668120.6 | 937791.6 | 317.908 |
| PT33 | SH | 668144.4 | 937877.2 | 316.236 |  | PT293 | SH | 668125 | 937790.3 | 317.853 |
| PT34 | SH | 668147.4 | 937877.9 | 316.065 |  | PT294 | SH | 668128.5 | 937785.7 | 317.944 |
| PT35 | BLD | 668149.4 | 937879.9 | 315.956 |  | PT295 | SH | 668133 | 937781.5 | 317.952 |
| PT36 | BLD | 668156.1 | 937879.6 | 315.966 |  | PT296 | SH | 668132.9 | 937786.5 | 317.857 |
| PT37 | BLD | 668156 | 937874.1 | 315.966 |  | PT297 | SH | 668132.2 | 937790.7 | 317.807 |
| PT38 | EP | 668150.9 | 937875.2 | 316.093 |  | PT298 | SH | 668137 | 937790.8 | 317.772 |
| PT39 | SH | 668152 | 937878.1 | 315.912 |  | PT299 | SH | 668139.7 | 937786.5 | 317.824 |
| PT40 | BLD | 668146.3 | 937874.1 | 316.297 |  | PT300 | SH | 668140 | 937781.4 | 318.046 |
| PT41 | SH | 668132 | 937896.8 | 316.129 |  | PT301 | SH | 668144.3 | 937781.2 | 318.023 |
| PT42 | BLD | 668128.4 | 937894.7 | 316.378 |  | PT302 | SH | 668145.5 | 937786.4 | 317.727 |
| PT43 | SH | 668124.8 | 937894.3 | 316.451 |  | PT303 | SH | 668146 | 937791.2 | 317.717 |
| PT44 | SH | 668121.3 | 937892 | 316.499 |  | PT304 | SH | 668149.5 | 937790.8 | 317.75 |
| PT45 | SH | 668118.3 | 937894.3 | 316.594 |  | PT305 | SH | 668149.7 | 937786.7 | 317.692 |
| PT46 | SH | 668115.7 | 937888.6 | 316.573 |  | PT306 | SH | 668148.9 | 937781.3 | 317.842 |
| PT47 | BLD | 668113.6 | 937886.9 | 316.529 |  | PT307 | BLD | 668150.4 | 937781.4 | 317.8 |
| PT48 | SH | 668110.4 | 937889.4 | 316.736 |  | PT308 | BLD | 668150 | 937791.5 | 317.803 |
| PT49 | SH | 668108.3 | 937891.8 | 316.596 |  | PT309 | SH | 668113.7 | 937768.9 | 318.789 |
| PT50 | SH | 668104.4 | 937892.2 | 316.593 |  | PT310 | SH | 668113.6 | 937766 | 318.901 |
| PT51 | SH | 668104.2 | 937889.1 | 316.626 |  | PT311 | BLD | 668112.8 | 937763.3 | 319.05 |
| PT52 | SH | 668117.5 | 937886.8 | 316.551 |  | PT312 | SH | 668117.2 | 937761.4 | 318.943 |
| PT53 | SH | 668119.7 | 937883.9 | 316.541 |  | PT313 | SH | 668117.5 | 937764.3 | 318.911 |
| PT54 | SH | 668117.1 | 937883.2 | 316.562 |  | PT314 | SH | 668122.4 | 937765.5 | 318.977 |
| PT55 | SH | 668114.2 | 937882.6 | 316.592 |  | PT315 | SH | 668124.5 | 937762.3 | 318.933 |
| PT56 | BLD | 668113.5 | 937874.4 | 316.659 |  | PT316 | SH | 668122.4 | 937758.6 | 318.969 |
| PT57 | SH | 668115.5 | 937874 | 316.641 |  | PT317 | SH | 668121.7 | 937753.2 | 319.012 |
| PT58 | SH | 668117.6 | 937873.9 | 316.666 |  | PT318 | SH | 668119.4 | 937752.3 | 319.022 |
| PT59 | SH | 668116.9 | 937866.3 | 316.759 |  | PT319 | BLD | 668113 | 937752.1 | 319.283 |
| PT60 | SH | 668114.9 | 937865.7 | 316.81 |  | PT320 | BLD | 668113.1 | 937748.7 | 319.275 |
| PT61 | SH | 668112.2 | 937865.8 | 317.066 |  | PT321 | BLD | 668113.2 | 937730.1 | 319.819 |
| PT62 | BLD | 668111.7 | 937870.2 | 316.664 |  | PT322 | BLD | 668118.1 | 937726.7 | 320.172 |
| PT63 | SH | 668124 | 937868.3 | 316.542 |  | PT323 | BLD | 668118.2 | 937750.2 | 319.117 |
| PT64 | BLD | 668129.2 | 937867.4 | 316.594 |  | PT324 | SH | 668122.5 | 937751.6 | 319.006 |
| PT65 | SH | 668128.1 | 937869.8 | 316.579 |  | PT325 | SH | 668124.5 | 937754.3 | 318.972 |
| PT66 | SH | 668127.9 | 937872.5 | 316.566 |  | PT326 | SH | 668125.4 | 937757 | 318.942 |
| PT67 | SH | 668125 | 937873.8 | 316.569 |  | PT327 | SH | 668126.4 | 937759.8 | 318.902 |
| PT68 | SH | 668123.6 | 937877 | 316.822 |  | PT328 | SH | 668127.7 | 937763.6 | 318.945 |
| PT69 | SH | 668126 | 937879.4 | 316.682 |  | PT329 | SH | 668129.6 | 937766.7 | 319.003 |
| PT70 | SH | 668121.7 | 937869.6 | 316.558 |  | PT330 | SH | 668132.3 | 937766.4 | 319.052 |
| PT71 | SH | 668121.5 | 937862 | 316.817 |  | PT331 | SH | 668132.9 | 937763.9 | 319.098 |
| PT72 | SH | 668123.1 | 937858.4 | 316.857 |  | PT332 | SH | 668133.1 | 937760.8 | 318.995 |
| PT73 | SH | 668118.9 | 937856.2 | 316.91 |  | PT333 | SH | 668133.8 | 937757.1 | 318.991 |
| PT74 | BLD | 668120.4 | 937852 | 316.833 |  | PT334 | SH | 668134 | 937753.4 | 318.981 |
| PT75 | SH | 668116.8 | 937852.9 | 316.833 |  | PT335 | SH | 668134.5 | 937750.7 | 319.07 |
| PT76 | CV | 668119 | 937855.1 | 316.791 |  | PT336 | BLD | 668137.6 | 937750.4 | 319.035 |
| PT77 | CV | 668118.5 | 937855.2 | 316.819 |  | PT337 | SH | 668138.2 | 937752.8 | 319.024 |
| PT78 | SH | 668111.8 | 937851.5 | 317.396 |  | PT338 | BLD | 668140.6 | 937755.3 | 318.91 |
| PT79 | SH | 668115.7 | 937850.6 | 316.866 |  | PT339 | SH | 668137.7 | 937757.7 | 319.018 |
| PT80 | SH | 668118.3 | 937849 | 316.835 |  | PT340 | SH | 668136 | 937762 | 319.013 |
| PT81 | SH | 668118.1 | 937843.1 | 316.939 |  | PT341 | SH | 668138.3 | 937764.7 | 319.044 |
| PT82 | SH | 668113.8 | 937842.7 | 316.91 |  | PT342 | BLD | 668139.4 | 937765.7 | 319.051 |
| PT83 | SH | 668108.1 | 937843.3 | 317.4 |  | PT343 | SH | 668141.4 | 937749.2 | 319.111 |
| PT84 | SH | 668106.1 | 937847.2 | 317.641 |  | PT344 | SH | 668143.4 | 937751 | 319.05 |
| PT85 | SH | 668102.4 | 937849.2 | 317.563 |  | PT345 | SH | 668144.9 | 937753.8 | 318.948 |
| PT86 | SH | 668101.2 | 937843.6 | 317.586 |  | PT346 | SH | 668147.2 | 937751.6 | 318.691 |
| PT87 | SH | 668099.2 | 937840.3 | 317.489 |  | PT347 | SH | 668146.9 | 937747.9 | 319.135 |
| PT88 | SH | 668098.1 | 937836.4 | 317.051 |  | PT348 | SH | 668144.5 | 937745.9 | 319.098 |
| PT89 | SH | 668094.9 | 937829.9 | 317.405 |  | PT349 | SH | 668143.8 | 937741.8 | 319.252 |
| PT90 | SH | 668099 | 937830 | 317.366 |  | PT350 | BLD | 668137.8 | 937739.5 | 319.454 |
| PT91 | SH | 668102.2 | 937835.3 | 317 |  | PT351 | SH | 668140 | 937740.1 | 319.295 |
| PT92 | SH | 668093 | 937842.7 | 317.597 |  | PT352 | SH | 668141.4 | 937742 | 319.263 |
| PT93 | SH | 668090.5 | 937849.3 | 317.02 |  | PT353 | SH | 668145.1 | 937740.6 | 319.475 |
| PT94 | SH | 668088 | 937845.4 | 317.154 |  | PT354 | SH | 668147.6 | 937743.1 | 319.297 |
| PT95 | CV | 668089.1 | 937840.1 | 317.01 |  | PT355 | SH | 668149.8 | 937746.8 | 319.15 |
| PT96 | CV | 668089.5 | 937840.1 | 317.036 |  | PT356 | SH | 668151.4 | 937750.4 | 319.144 |
| PT97 | CV | 668089.7 | 937835.1 | 316.97 |  | PT357 | SH | 668153.9 | 937753.9 | 319.13 |
| PT98 | CV | 668089.1 | 937835.1 | 316.951 |  | PT358 | BLD | 668156.8 | 937753.8 | 319.16 |
| PT99 | SH | 668087.3 | 937837.4 | 317.032 |  | PT359 | BLD | 668155 | 937750.1 | 319.175 |
| PT100 | SH | 668084.9 | 937841.3 | 317.015 |  | PT360 | GT | 668156.7 | 937750.4 | 319.189 |
| PT101 | SH | 668081.1 | 937845.6 | 316.694 |  | PT361 | GT | 668156.4 | 937744 | 319.379 |
| PT102 | BLD | 668082.4 | 937849.5 | 316.774 |  | PT362 | BLD | 668151.3 | 937744.1 | 319.293 |
| PT103 | SH | 668079.2 | 937850.8 | 316.576 |  | PT363 | BLD | 668157.6 | 937720.9 | 319.595 |
| PT104 | SH | 668076.3 | 937851.1 | 316.606 |  | PT364 | BLD | 668075.3 | 937717.9 | 320.555 |
| PT105 | SH | 668076.4 | 937857.2 | 316.522 |  | PT365 | BLD | 668042.2 | 937720.6 | 320.353 |
| PT106 | SH | 668079.6 | 937859 | 316.481 |  | PT366 | BLD | 668036 | 937729.4 | 320.261 |
| PT107 | SH | 668082.3 | 937859.3 | 316.539 |  | PT367 | BLD | 668035.2 | 937744.9 | 319.623 |
| PT108 | SH | 668081.6 | 937865.3 | 316.444 |  | PT368 | BLD | 668011 | 937744.6 | 320.389 |
| PT109 | SH | 668078.7 | 937867 | 316.454 |  | PT369 | BLD | 668000.4 | 937749.6 | 319.55 |
| PT110 | SH | 668075.8 | 937866.3 | 316.351 |  | PT370 | BLD | 667993.6 | 937757.8 | 318.942 |
| PT111 | SH | 668074 | 937869.4 | 316.329 |  | PT371 | BLD | 667980.6 | 937772.8 | 318.59 |
| PT112 | SH | 668078.7 | 937870.4 | 316.547 |  | PT372 | BLD | 667974.3 | 937782.4 | 318.279 |
| PT113 | SH | 668081.1 | 937872.7 | 316.472 |  | PT373 | BLD | 667968.1 | 937789.6 | 318.03 |
| PT114 | SH | 668081.9 | 937877.2 | 316.469 |  | PT374 | RD | 667966.1 | 937793.6 | 317.734 |
| PT115 | SH | 668079 | 937877.3 | 316.472 |  | PT375 | CL | 667964.4 | 937795.8 | 317.839 |
| PT116 | SH | 668076.3 | 937876 | 316.604 |  | PT376 | RD | 667962.9 | 937797.2 | 317.658 |
| PT117 | EP | 668074.3 | 937876.1 | 316.594 |  | PT377 | SH | 667967.3 | 937798.1 | 317.599 |
| PT118 | TM | 668071.8 | 937879 | 316.342 |  | PT378 | SH | 667972.1 | 937795.5 | 317.821 |
| PT119 | TM | 668076.7 | 937878.8 | 316.445 |  | PT379 | SH | 667976.9 | 937798.1 | 317.559 |
| PT120 | TM | 668077.2 | 937885.8 | 316.536 |  | PT380 | SH | 667975.5 | 937802 | 317.229 |
| PT121 | SH | 668079.9 | 937885 | 316.362 |  | PT381 | SH | 667973.8 | 937805.3 | 317.111 |
| PT122 | SH | 668082.8 | 937883.4 | 316.48 |  | PT382 | SH | 667974.4 | 937809.3 | 317.087 |
| PT123 | SH | 668079.8 | 937880.6 | 316.437 |  | PT383 | SH | 667980.3 | 937808.6 | 317.008 |
| PT124 | SH | 668064.8 | 937896.9 | 315.429 |  | PT384 | SH | 667986.3 | 937808.3 | 317.055 |
| PT125 | BLD | 668064.8 | 937885.4 | 315.743 |  | PT385 | SH | 667989 | 937806.7 | 317.471 |
| PT126 | SH | 668069.7 | 937888.7 | 315.93 |  | PT386 | SH | 667991.4 | 937808.2 | 317.321 |
| PT127 | SH | 668069.8 | 937882.8 | 315.817 |  | PT387 | BLD | 667995.8 | 937809.1 | 317.181 |
| PT128 | EP | 668066.9 | 937881.4 | 316.367 |  | PT388 | BLD | 667996.6 | 937819.4 | 317.014 |
| PT129 | BLD | 668073.2 | 937873.9 | 316.28 |  | PT389 | BLD | 667997.1 | 937825.8 | 316.715 |
| PT130 | SH | 668076.5 | 937842.5 | 316.803 |  | PT390 | BLD | 667997.7 | 937836.1 | 316.436 |
| PT131 | SH | 668080.9 | 937837.3 | 316.882 |  | PT391 | SH | 667999.8 | 937837.9 | 316.489 |
| PT132 | SH | 668076.4 | 937833.4 | 317.117 |  | PT392 | SH | 668004.1 | 937837 | 316.697 |
| PT133 | SH | 668079.1 | 937829.3 | 317.088 |  | PT393 | SH | 668006.3 | 937835.7 | 316.836 |
| PT134 | SH | 668084.6 | 937829.3 | 317.095 |  | PT394 | SH | 668012.2 | 937839.5 | 317.101 |
| PT135 | SH | 668084.6 | 937825.2 | 317.166 |  | PT395 | BLD | 668024 | 937834.2 | 316.943 |
| PT136 | SH | 668080.9 | 937824.4 | 317.192 |  | PT396 | EP | 668028.4 | 937833.8 | 316.701 |
| PT137 | EP | 668075.2 | 937830.8 | 317.409 |  | PT397 | SH | 668027.3 | 937830.5 | 316.834 |
| PT138 | SH | 668076.5 | 937824.8 | 317.309 |  | PT398 | SH | 668024.8 | 937827.5 | 316.95 |
| PT139 | SH | 668080.1 | 937820.9 | 317.285 |  | PT399 | SH | 668027.4 | 937825.1 | 316.977 |
| PT140 | SH | 668083.9 | 937818.9 | 317.208 |  | PT400 | SH | 668025.7 | 937818 | 317.088 |
| PT141 | SH | 668084.3 | 937814.6 | 317.38 |  | PT401 | BLD | 668022.6 | 937817.1 | 317.238 |
| PT142 | SH | 668080.3 | 937812.9 | 317.483 |  | PT402 | SH | 668025.6 | 937813.3 | 317.258 |
| PT143 | SH | 668077 | 937811.8 | 317.722 |  | PT403 | SH | 668030.1 | 937809.1 | 317.516 |
| PT144 | EP | 668075.5 | 937812.1 | 317.622 |  | PT404 | BLD | 668031.1 | 937802.8 | 317.66 |
| PT145 | CV | 668075.9 | 937810.5 | 317.869 |  | PT405 | SH | 668027.5 | 937801.2 | 317.556 |
| PT146 | CV | 668076.6 | 937810.4 | 317.877 |  | PT406 | SH | 668031.4 | 937799.3 | 317.671 |
| PT147 | CV | 668076.1 | 937808 | 317.884 |  | PT407 | BLD | 668035 | 937801.7 | 318.075 |
| PT148 | CV | 668076.5 | 937808 | 317.896 |  | PT408 | SH | 668032 | 937800 | 317.586 |
| PT149 | BLD | 668073.4 | 937815.4 | 317.468 |  | PT409 | SH | 668031.4 | 937796 | 317.74 |
| PT150 | BLD | 668073.5 | 937803.4 | 317.824 |  | PT410 | SH | 668033.4 | 937793.9 | 317.725 |
| PT151 | SH | 668077.3 | 937804.7 | 317.737 |  | PT411 | BLD | 668035.2 | 937792.8 | 317.863 |
| PT152 | SH | 668080.4 | 937805.5 | 317.546 |  | PT412 | BLD | 668029.7 | 937792.2 | 318.048 |
| PT153 | SH | 668083.8 | 937807.7 | 317.514 |  | PT413 | BLD | 668029.5 | 937779.7 | 318.482 |
| PT154 | SH | 668084 | 937803.1 | 317.576 |  | PT414 | BLD | 668035.6 | 937839.2 | 316.76 |
| PT155 | SH | 668080.5 | 937801.9 | 317.645 |  | PT415 | BLD | 668030 | 937849.5 | 316.014 |
| PT156 | SH | 668076.8 | 937799.4 | 318.005 |  | PT416 | BLD | 668014.7 | 937849.8 | 315.771 |
| PT157 | SH | 668080.5 | 937797 | 317.729 |  | PT417 | RD | 668001.4 | 937850 | 315.553 |
| PT158 | SH | 668084.2 | 937798.5 | 317.584 |  | PT418 | RD | 667996.5 | 937852.3 | 315.646 |
| PT159 | SH | 668086.6 | 937795.5 | 317.625 |  | PT419 | BLD | 668015.4 | 937870.2 | 315.216 |
| PT160 | SH | 668088.9 | 937794.7 | 317.784 |  | PT420 | RD | 668022.4 | 937889 | 314.617 |
| PT161 | SH | 668087.8 | 937791.7 | 318.04 |  | PT421 | RD | 668020.3 | 937890.7 | 314.467 |
| PT162 | BLD | 668090.2 | 937790.5 | 318.057 |  | PT422 | BLD | 668035.6 | 937896.8 | 314.523 |
| PT163 | SH | 668086 | 937790 | 318.019 |  | PT423 | CV | 668032.5 | 937898.1 | 314.441 |
| PT164 | SH | 668079.9 | 937790.6 | 317.829 |  | PT424 | CV | 668032.1 | 937898.6 | 314.354 |
| PT165 | SH | 668076.8 | 937791.1 | 317.997 |  | PT425 | EP | 668026.8 | 937902.6 | 314.367 |
| PT166 | EP | 668075.9 | 937791.5 | 318.028 |  | PT426 | EP | 668042.4 | 937923.4 | 313.423 |
| PT167 | SH | 668081.3 | 937788.3 | 317.911 |  | PT427 | EP | 668045.7 | 937913.1 | 313.825 |
| PT168 | SH | 668086.2 | 937787 | 318.037 |  | PT428 | CV | 668045.7 | 937915.3 | 314.041 |
| PT169 | SH | 668088.3 | 937782.1 | 318.256 |  | PT429 | CV | 668045.2 | 937915.7 | 314.017 |
| PT170 | SH | 668083.1 | 937782.1 | 318.113 |  | PT430 | RD | 668050.4 | 937924.9 | 313.193 |
| PT171 | SH | 668077.2 | 937783.1 | 318.448 |  | PT431 | RD | 668048 | 937928.1 | 313.163 |
| PT172 | SH | 668077.1 | 937779.4 | 318.482 |  | PT432 | CL | 668049.6 | 937927.2 | 313.142 |
| PT173 | SH | 668080.8 | 937779.2 | 318.382 |  | PT433 | BLD | 668057.8 | 937924.9 | 313.443 |
| PT174 | SH | 668086.1 | 937777 | 318.527 |  | PT434 | BLD | 668139.5 | 937920.4 | 313.76 |
| PT175 | SH | 668088.4 | 937774.7 | 318.351 |  | PT435 | BD | 668156.3 | 937920.7 | 312.957 |
| PT176 | CV | 668076.7 | 937775.7 | 318.733 |  | PT436 | BLD | 668156.4 | 937893.6 | 315.143 |
| PT177 | CV | 668076.4 | 937775.7 | 318.755 |  | PT437 | BLD | 668120.5 | 937803.6 | 317.77 |
| PT178 | SH | 668075.6 | 937773.1 | 318.598 |  | PT438 | SH | 668121.8 | 937806.3 | 317.702 |
| PT179 | BLD | 668073.5 | 937768.6 | 318.708 |  | PT439 | BLD | 668120.3 | 937810 | 317.679 |
| PT180 | SH | 668073.1 | 937767.9 | 318.686 |  | PT440 | SH | 668124.2 | 937808.2 | 317.693 |
| PT181 | SH | 668075.1 | 937765.4 | 318.807 |  | PT441 | SH | 668124.2 | 937805.5 | 317.683 |
| PT182 | BLD | 668076.2 | 937763 | 318.99 |  | PT442 | SH | 668127.7 | 937808.3 | 317.623 |
| PT183 | SH | 668077.8 | 937762.3 | 318.92 |  | PT443 | SH | 668129.9 | 937805.3 | 317.555 |
| PT184 | SH | 668079.4 | 937758.5 | 319.226 |  | PT444 | SH | 668132.3 | 937808.2 | 317.557 |
| PT185 | SH | 668076.9 | 937758.6 | 318.98 |  | PT445 | SH | 668135.2 | 937805 | 317.569 |
| PT186 | SH | 668079.6 | 937754.5 | 319.302 |  | PT446 | SH | 668139.8 | 937808.7 | 317.523 |
| PT187 | SH | 668077.8 | 937753.6 | 319.218 |  | PT447 | SH | 668142.5 | 937806.5 | 317.477 |
| PT188 | SH | 668078.5 | 937746.8 | 319.408 |  | PT448 | SH | 668143.5 | 937805.6 | 317.641 |
| PT189 | SH | 668076.3 | 937744 | 319.51 |  | PT449 | BLD | 668152.8 | 937810.8 | 317.334 |
| PT190 | SH | 668079.7 | 937740.7 | 319.611 |  | PT450 | BLD | 668152.8 | 937803.9 | 317.388 |
| PT191 | SH | 668080.3 | 937743.2 | 319.569 |  | PT451 | BLD | 668120.4 | 937826.9 | 317.189 |
| PT192 | SH | 668078.8 | 937739.9 | 319.631 |  | PT452 | SH | 668123.3 | 937828 | 316.989 |
| PT193 | SH | 668078.8 | 937737.1 | 319.719 |  | PT453 | SH | 668126.8 | 937828.7 | 316.943 |
| PT194 | SH | 668081 | 937736.3 | 319.732 |  | PT454 | SH | 668126.3 | 937832 | 316.931 |
| PT195 | SH | 668078.1 | 937736.2 | 319.726 |  | PT455 | SH | 668123.9 | 937832.8 | 316.937 |
| PT196 | SH | 668074.3 | 937736.5 | 319.73 |  | PT456 | SH | 668121.8 | 937833.5 | 316.965 |
| PT197 | BLD | 668076 | 937739.9 | 319.718 |  | PT457 | SH | 668128 | 937833 | 316.904 |
| PT198 | SH | 668070.5 | 937735 | 319.725 |  | PT458 | SH | 668128.4 | 937836.4 | 316.889 |
| PT199 | SH | 668067.7 | 937732.6 | 319.769 |  | PT459 | SH | 668135.6 | 937832.3 | 316.921 |
| PT200 | SH | 668065.8 | 937735.6 | 319.673 |  | PT460 | SH | 668134.3 | 937834.4 | 316.926 |
| PT201 | SH | 668064.2 | 937739.6 | 319.579 |  | PT461 | SH | 668138.4 | 937829.2 | 316.911 |
| PT202 | SH | 668061 | 937736.3 | 319.632 |  | PT462 | SH | 668141.1 | 937835.5 | 316.628 |
| PT203 | SH | 668060.3 | 937732.6 | 319.705 |  | PT463 | SH | 668120.5 | 937838.7 | 317.002 |
| PT204 | SH | 668057.4 | 937735.8 | 319.652 |  | PT464 | BLD | 668073.3 | 937862.2 | 316.453 |
| PT205 | SH | 668056.3 | 937739.5 | 319.449 |  | PT465 | SH | 668072.4 | 937859 | 316.477 |
| PT206 | SH | 668052.5 | 937736.1 | 319.664 |  | PT466 | SH | 668069.5 | 937857 | 316.409 |
| PT207 | SH | 668050.9 | 937732.6 | 319.757 |  | PT467 | SH | 668065.8 | 937859.2 | 316.319 |
| PT208 | SH | 668048.6 | 937736.2 | 319.724 |  | PT468 | SH | 668064.3 | 937857.5 | 316.229 |
| PT209 | BLD | 668043.3 | 937739.9 | 319.659 |  | PT469 | SH | 668063.9 | 937853.2 | 316.483 |
| PT210 | BLD | 668043.6 | 937731.8 | 319.686 |  | PT470 | SH | 668055.7 | 937859.2 | 316.296 |
| PT211 | BLD | 668075.8 | 937729.8 | 319.966 |  | PT471 | SH | 668050.3 | 937854.7 | 316.365 |
| PT212 | BLD | 668079.9 | 937728.9 | 320.175 |  | PT472 | BLD | 668050.1 | 937852.9 | 316.654 |
| PT213 | BLD | 668079.7 | 937763.4 | 318.77 |  | PT473 | SH | 668056.8 | 937852.8 | 316.47 |
| PT214 | SH | 668080.8 | 937767.6 | 318.711 |  | PT474 | SH | 668063.8 | 937852.2 | 316.438 |
| PT215 | SH | 668083.6 | 937771.3 | 318.618 |  | PT475 | SH | 668067.5 | 937853.8 | 316.473 |
| PT216 | CV | 668088.4 | 937773.9 | 318.388 |  | PT476 | BLD | 668073.3 | 937851.7 | 316.674 |
| PT217 | CV | 668088.4 | 937774.1 | 318.401 |  | PT477 | BLD | 668073.4 | 937838.8 | 317.02 |
| PT218 | CV | 668088.9 | 937774.3 | 318.362 |  | PT478 | SH | 668071.2 | 937834.5 | 316.988 |
| PT219 | CV | 668089.2 | 937774.2 | 318.355 |  | PT479 | SH | 668071.7 | 937829.5 | 317.224 |
| PT220 | CV | 668089.7 | 937774 | 318.348 |  | PT480 | BLD | 668073.4 | 937827.2 | 317.205 |
| PT221 | CV | 668089.7 | 937773.6 | 318.358 |  | PT481 | SH | 668067.6 | 937828.1 | 317.111 |
| PT222 | SH | 668092.5 | 937776.3 | 318.551 |  | PT482 | SH | 668064.6 | 937832.8 | 317.034 |
| PT223 | SH | 668090.5 | 937779.5 | 318.338 |  | PT483 | SH | 668061.8 | 937837.4 | 316.978 |
| PT224 | SH | 668092.9 | 937780.6 | 318.233 |  | PT484 | SH | 668056.2 | 937836.7 | 316.98 |
| PT225 | SH | 668091.7 | 937784.1 | 318.136 |  | PT485 | SH | 668057.4 | 937831 | 317.048 |
| PT226 | SH | 668090.5 | 937787.6 | 318.117 |  | PT486 | SH | 668058.5 | 937827.6 | 317.15 |
| PT227 | SH | 668095.2 | 937788.3 | 318.215 |  | PT487 | SH | 668050.6 | 937828 | 316.962 |
| PT228 | SH | 668098.3 | 937788.8 | 318.206 |  | PT488 | SH | 668049.9 | 937833.6 | 316.97 |
| PT229 | SH | 668098.2 | 937786.1 | 318.204 |  | PT489 | SH | 668049 | 937837 | 316.957 |
| PT230 | SH | 668097.9 | 937782.4 | 318.229 |  | PT490 | BLD | 668043.9 | 937838 | 317.267 |
| PT231 | SH | 668097.4 | 937777.8 | 318.321 |  | PT491 | BLD | 668073.7 | 937792 | 318.073 |
| PT232 | SH | 668097.1 | 937773.1 | 318.529 |  | PT492 | SH | 668073.1 | 937784.7 | 318.318 |
| PT233 | SH | 668100.9 | 937773.3 | 318.58 |  | PT493 | BD | 668073.2 | 937780.3 | 318.439 |
| PT234 | SH | 668102.2 | 937777.1 | 318.319 |  | PT494 | SH | 668069.9 | 937784.4 | 318.355 |
| PT235 | SH | 668101.1 | 937781.3 | 318.285 |  | PT495 | SH | 668064.9 | 937790.3 | 318.076 |
| PT236 | SH | 668100.2 | 937785.2 | 318.239 |  | PT496 | SH | 668062.6 | 937784.4 | 318.262 |
| PT237 | SH | 668102.1 | 937788.4 | 318.071 |  | PT497 | SH | 668061.5 | 937781.1 | 318.253 |
| PT238 | BLD | 668103.3 | 937790.4 | 317.873 |  | PT498 | SH | 668054.5 | 937781.3 | 318.237 |
| PT239 | SH | 668105.1 | 937786.6 | 318.137 |  | PT499 | SH | 668053 | 937785.2 | 318.171 |
| PT240 | SH | 668105.9 | 937783.1 | 318.189 |  | PT500 | SH | 668051.4 | 937789.3 | 318.097 |
| PT241 | SH | 668106.2 | 937778.4 | 318.236 |  | PT501 | SH | 668045 | 937791.3 | 318.148 |
| PT242 | SH | 668106.3 | 937774.5 | 318.368 |  | PT502 | SH | 668044.7 | 937785.9 | 318.238 |
| PT243 | SH | 668106.4 | 937771.7 | 318.607 |  | PT503 | BD | 668043.5 | 937789.4 | 318.204 |
| PT244 | SH | 668109.8 | 937771.4 | 318.611 |  | PT504 | BD | 668046.8 | 937781.7 | 318.191 |
| PT245 | SH | 668114.7 | 937770.8 | 318.727 |  | PT505 | SH | 668050.5 | 937781.3 | 318.192 |
| PT246 | CV | 668118.6 | 937769.6 | 318.768 |  | PT506 | BLD | 668073.6 | 937768.4 | 318.52 |
| PT247 | CV | 668118.6 | 937769.2 | 318.875 |  | PT507 | SH | 668072.8 | 937765.9 | 318.772 |
| PT248 | BLD | 668120.4 | 937768.5 | 318.906 |  | PT508 | SH | 668068.1 | 937767.9 | 318.712 |
| PT249 | SH | 668118 | 937771.4 | 318.745 |  | PT509 | SH | 668064.3 | 937765.9 | 318.846 |
| PT250 | SH | 668114.6 | 937774.6 | 318.45 |  | PT510 | SH | 668061.8 | 937764.8 | 318.757 |
| PT251 | SH | 668112.3 | 937778.2 | 318.273 |  | PT511 | SH | 668059.2 | 937766.6 | 318.779 |
| PT252 | SH | 668116.9 | 937781 | 318.219 |  | PT512 | SH | 668056.8 | 937767.3 | 318.813 |
| PT253 | SH | 668114.7 | 937784 | 318.086 |  | PT513 | SH | 668055.6 | 937765.9 | 317.208 |
| PT254 | SH | 668110.9 | 937786.4 | 318.028 |  | PT514 | SH | 668054.7 | 937765.1 | 317.217 |
| PT255 | SH | 668108.3 | 937788.8 | 318.063 |  | PT515 | SH | 668050.4 | 937765 | 317.161 |
| PT256 | SH | 668106.5 | 937792.3 | 317.922 |  | PT516 | SH | 668048.2 | 937766.1 | 317.281 |
| PT257 | SH | 668104.3 | 937794.3 | 317.593 |  | PT517 | SH | 668047.6 | 937767.1 | 318.799 |
| PT258 | SH | 668106.6 | 937796.8 | 317.885 |  | PT518 | BLD | 668043.7 | 937764.7 | 322.083 |
| PT259 | SH | 668110.6 | 937794.3 | 317.866 |  |  |  |  |  |  |

# References:

Baker, R., & Hernandez, J. (2009). Ground-Penetrating Radar for Utility Detection. Journal of Infrastructure Systems, 15(2), 123-133.

Eastman, C. M., Lee, J. M., & Fereshtian, S. (2011). Building Information Modelling and the challenges of utility surveying. Journal of Surveying Engineering, 137(3), 113-124.

Federico, A., Silvestri, F., & Cavuoto, G. (2021). Utility surveys for urban planning: A review of current practices and future directions. Journal of Urban Planning and Development, 147(2), 04021013.

Geyer, T., Kalbermatten, M., & Willimann, P. (2020). Electromagnetic induction for utility detection: A review. Journal of Applied Geophysics, 179, 104106.

Harlow, C., Bai, S., & Zhang, W. (2019). Integrating environmental impact assessments into utility surveys: A review. Journal of Environmental Management, 245, 109-121.

Kumar, P., McCarthy, T., & McLaughlin, S. (2022). Drone-based utility surveying: A review of current applications and future directions. Journal of Unmanned Aerial Systems, 10(2), 123-138.

Levine, N., McEntire, D. A., & Schexnayder, C. (2015). Utility data for emergency management: A review. Journal of Emergency Management, 13(3), 243-255.

Liu, Z., Zhang, J., & Li, Y. (2018). Geographic Information Systems for utility management: A review. Journal of Geographic Information System, 10(3), 349-364.

Meyer, J., Frauenfelder, R., & Straub, D. (2017). Data confidentiality in utility surveying: Challenges and opportunities. Journal of Infrastructure Systems, 23(1), 04017002.

Morris, R., & Heneghan, P. (2004). Advances in utility surveying: A review. Journal of Surveying Engineering, 130(2), 63-73.

Pawlukiewicz, J., McHugh, M., & Rossler, P. (2005). Utility surveying for construction: Best practices. Journal of Construction Engineering and Management, 131(10), 1081-1091.

Pires, F., & Henriques,Here is the continuation:

Pires, F., & Henriques, M. (2022). Ground-Penetrating Radar for Utility Detection: Case Studies. Journal of Infrastructure Systems, 28(2), 05022002.

Rogers, C. D. F., & Chapman, D. N. (2018). Underground asset location and condition assessment. Journal of Infrastructure Systems, 24(1), 04018002.

Santos, J., & Correia, M. (2016). Utility surveying: A review of current practices and future directions. Journal of Surveying Engineering, 142(2), 04016006.

Schmidt, F., & Schütze, M. (2003). Utility mapping and documentation. Journal of Infrastructure Systems, 9(2), 73-84.

Sheng, Y., & Li, Z. (2019). Three-dimensional modeling of utility infrastructure for urban planning. Journal of Urban Planning and Development, 145(2), 04019005.

Zhou, Y., & Li, Y. (2020). Artificial intelligence for utility surveying: A review. Journal of Infrastructure Systems, 26(2), 04020015.