PERIMETER AND DETAILING SURVEY

OF

PART OF INSTITUTE OF FINANCE AND MANAGEMENT STUDIES (IFMS), KWARA STATE POLYTECHNIC, ILORIN, MORO LOCAL GOVERNMENT, KWARA STATE

BY

AREMU IYANUOLUWA FUNMILAYO

ND/23/SGI/FT/0075

A PROJECT WORK SUBMITTED TO THE DEPARTMENT OF SURVEYING AND GEO-INFORMATICS, INSTITUTE OF ENVIRONMENTAL STUDIES, KWARA STATE POLYTECHNIC, ILORIN

IN  PARTIAL  FULFILLMENT  OF THE  REQUIREMENT  FOR  THE AWARD  OF NATIONAL  DIPLOMA  IN  SURVEYING  AND  GEO-INFORMATICS

JUNE, 2025

CERTIFICATE

I hereby certified that all information given in this project were obtained as a result of observation and measurement made by me that the survey was carried out in accordance with the survey rules, supervisor and departmental instruction.

………………………….				       …………………….

AREMU IYANUOLUWA FUNMILAYO	                    DATE       ND/23/SGI/FT/0075

CERTIFICATION

This is to certify that I AREMU IYANUOLUWA FUNMILAYO with matric no; ND/23/SGI/FT/0075 a student of Surveying & Geo-Informatics Department has actually carried out the field work in connection with this project and it was carried out in accordance with survey rules and regulations and department instructions.

……………………………..    				….………………

SURV R.S. AWOLEYE						DATE

SURV. WILLIAMS KAZEEM.A

PROJECT SUPERVISOR

……………………………....                      		….………………

DATE

PROJECT  CO-ORDINATOR

……………………………….                 		….………………

MR. ABIMBOLA ISAU              	                            DATE

HEAD OF DEPARTMENT

……………………………….                 		….………………

DATE

EXTERNAL  EXAMINER

DEDICATION

This project is dedicated to Almighty God, the magnificent, the over merciful to my lovely parents for their supports towards the successful completion of this project.

ACKNOWLEDGEMENT

All glory, honors, praise and adoration goes to Almighty God, , The creator of the universe, the author and finisher of my faith, under whose protection I have been since I was born even from the beginning of my life.

My profound gratitude goes to my loving, caring, humble and beloved parent in person MR. & MRS. AREMU I pray may Allah spare your life to reap the fruits of your labour.

Also express my sincere appreciation to my supervisor in person of SURV R.S. AWOLEYE & SURV WILLIAMS KAZEEM for their assistance and precise guidance during the course of this project. I say may God bless you abundantly.

My appreciation will be incomplete if I fail to recognize my HOD surveying and Geo-information MR. ABIMBOLA ISAU for what he has done, I pray that Allah continue to guide you in all your Endeavour and all other staff of the Department of Surveying and Geo-informatics like, SURV. ASONIBARE ROTIMI, SURV. AWOLEYE, R.S., SURV. FELIX DIRAN, SURV. AREMU ABUBAKAR, SURV. ADEBANJI AKINYEDE, Thanks and God bless you all.

I wish to appreciate my wonderful group member despite that the misunderstanding we had between ourselves, we later work as a team in order to acquire our aims and objective.

TABLE OF CONTENTS

Title page                                                                             		 i

Certification                                                                         		ii

Dedication                                                                           		iii

Acknowledgement                                                               		iv

Abstract                                                                               		v

Table of content                                                                  		vi

CHAPTER ONE

INTRODUCTION

  - Background of the study                                                		1

  - Statement of the problem                                               		4

  - Aims of the project                                                        		4

  - Objective of the project                                                 	 	5

  - Scope of the project                                                       	 	5

  - Specification of the project                                           	  	6

  - Study Area                                                                      		6

  - Personnel                                                                        		7

CHAPTER TWO                                                                    	 	8

LITERATURE REVIEW                                                           	8

CHAPTER THREE                                                           		14

3.0 Methodology                                                                 		14

3.1 Reconnaissance.                                                            		14

3.2 Instrument  Test                                                             		16

3.3 Monumentation                                                              		21

3.4 Perimeter Traversing                                                      		22

3.5 Detail survey                                                                  	24

CHAPTER FOUR

4.0 Data processing                                                              		26

4.1 Result Analysis                                                               		26

4.2 Back computation                                                           		27

4.3 Compute for the Area using Double latitude and departure     	28

4.4 Graphic plotting                                                           		30

CHAPTER FIVE

5.1 Summary                                                                             		27

5.2 Recommendation                                                                 		27

5.3 Problem Encountered                                                           	28

5.4 Conclusion                                                                           		28

References								29

APPENDIX

CHAPTER ONE

BACKGROUND TO THE STUDY

- INTRODUCTION

Perimeter and detailing survey is a type of land survey that defines the boundaries of a particular parcel of land or real estate property. This requires surveying the entire perimeter of the said real estate to determine the exact average and geometry of the property and identify any easement that may be present within the land. Also, perimeter survey is used to identify the location of old boundaries documentation and survey pins or they used to set up new ones.

Perimeter Surveys are carried out for the purpose of delineating the boundary of a parcel of land, determining its area and preparation of survey plan. The survey plan is usually the end product of a boundary survey. The survey plan shows ownership and describes the land. A detail survey is carried out to locate all features on a piece of land. This includes both natural and man-made structures. Natural features include vegetation of all sorts – rocks, trees, stumps and so on. Man-made structures include anything built above the ground – buildings, walls, driveways, utilities and so on.

Perimeter survey is defined as the type of survey which maps a strip along the boundaries while detailing is an act of fixing details such as building, road, electric pole etc. It can as well be define as the manmade and natural features on the ground within the project site which are determined and obtained by the method of total station and are finally represented with a suitable scale on plan. It can be said to be the process of fixing both naturals and artificial features to the corresponding traverse line or control are fixed and represented to scale on the plan.

A perimeter survey is important to find out the exits location of the landed property and property determine the extent of such land and also the extent of Encroachment can be evaluated in case of the landed property is in dispute. Perimeter and details survey is a survey that requires transferring of the details on the limits of the heights, depicting all details on the limit of the landed property, which consists of both natural and artificial features. It also refers to a cadastral survey because it contains coordinate all corners point of the boundaries and determination of relative position of both natural and artificial features on the Earth’s surface and addicting them by means of conversation  symbols on the plan.

  - STATEMENT OF PROBLEM

The reason for carrying out this project operation is due to the irregular or changing boundary line between the project area and the surrounding institution, in order to help the school authority plan for future expansion and development of the project area, the need for a more current and valid perimeter and detailing is suggested.

  - AIM OF THE PROJECT

The aim of this project is to carry out perimeter and detailing and to produce a current and comprehensive perimeter and detail plan of part of Institute of Finance and Management Studies, Kwara state Polytechnic, Ilorin.

  - OBJECTIVE OF THE PROJECT

The following objectives are perused to achieve the aim of the project.

- Reconnaissance which include office planning and field reconnaissance.

- Identification of existing features

- Pegging out some selected point along the boundary line

- Traverse to determine X and Y coordinate of the selected point

- Fixation of detail around the school using total station method

- Production of plan showing the boundary lines and all detail in the school (institute)

- Project report writing

  - SCOPE OF THE PROJECT

The entire project covered the following

- Reconnaissance

- Boundary demarcation of the project area

- Perimeter traversing

- Detailing by total station method

- Field reduction and computation

- Plan production

1.5	STUDY AREA

The area to be covered by this project is part of Institute of Finance and Management Studies, Kwara state Polytechnic, Ilorin. Moro local government, which falls on the longitude and latitude of 0080 33’ 20” N Latitude. 0040 38’ 14.584”.

MAP OF NIGERIA

MAP OF KWARA STATE

MAP  OF  MORO L.G.A

Google earth imagery of project site

1.6	 PERSONNELS INVOLVED

For easy and proper execution of this project (6) six members were engages in the operations. The roles were interchanged among the members in the group for the benefit for everyone to participate all the operations. The below table shows the name and matric number of the group members.

| S/N | NAME | MATRIC NO | ROLE PLAYED |
| --- | --- | --- | --- |
| 1 | Aremu Iyanuoluwa Funmilayo | ND/23/SGI/FT/0075 | Author |
| 2 | Olanrewaju Misturah Damilola | ND/23/SGI/FT/0073 | Member |
| 3 | Omoniyi Adenike Baraka | ND/23/SGI/FT/0074 | Member |
| 4 | Ayeni Hanifat Morenikeji | ND/23/SGI/FT/0078 | Member |
| 5 | Gabriel Ayomide Deborah | ND/23/SGI/FT/0077 | Member |
| 6 | Olanrewaju Abdulsamod | ND/23/SGI/FT/0081 | Member |

CHAPTER TWO

2.0	LITERATURE REVIEW

Dashes (2017) talk about Perimeter Survey is a type of land survey that defines the perimeter boundaries of a particular parcel of real estate property. This survey maps a strip along the boundaries using a minimum width of 15 feet. This requires surveying the entire perimeter of the said real estate in order to determine the exact acreage and geometry of the property and identify any easements and encroachments that may be present within the land.

Abiodun M. (2020), said there is some evidence that, in addition to a marked cord, wooden rods were used by the Egyptians for distance measurement. There is no record of any angle-measuring instruments of that time, but there was a level consisting of a vertical wooden A-frame with a plumb bob supported at the peak of the A so that its cord hung past an indicator, or index, on the horizontal bar. The index could be properly placed by standing the device on two supports at approximately the same elevation, marking the position of the cord, reversing the A, and making a similar mark. Halfway between the two marks would be the correct place for the index. Thus, with their simple devices, the ancient Egyptians were able to measure land areas, replace property corners lost when the Nile covered the markers with silt during floods, and build the huge pyramids to exact dimensions.

According to Albert Bodge (2019) the described a perimeter survey as a type of land survey that defines the perimeter boundaries of a particular parcel of real estate property. This survey maps a trip along the boundaries using a minimum width of 15 feet. This requires surveying the entire perimeter of the said real estate in order determine the exact acreage and geometry of the  property and identify any easements and enrichment that may be present within the land. This present will require with installation of monument to mark the boundary corners of the land for future references. This process will require the installation of monuments to mark the boundary corners of the land for future references. This requirement can be waived through a written agreement between the surveyor and homeowner.

Perimeter Surveying is a service that can be performed by a professional land surveying firm, such as Godfrey-Hoffman & Hodge in Connecticut, to ensure reliable, accurate, and detailed surveying results. Perimeter Surveys are used to identify the location of old boundary monumentation and survey pins, or they can be used to set up new ones. A Perimeter Survey does not identify the features and improvements that exist within a property, such as barns, garages, sheds, dwellings, surface utilities, roadways, pools and visible bodies of water; only features that fall within the 15 foot width around the boundary perimeter will be depicted. A Perimeter Survey pinpoints any visible evidence of easements and other public facilities within the property. A Perimeter Survey detects any visible ingress and egress on the property. A Perimeter Survey resolves conflicts on maps and deed descriptions. A Perimeter Survey shows structures such as fences, hedges, yards, and walls. A Perimeter Survey may be a cost-effective way to map your boundary without the added cost of making locations of features on your entire parcel.

The Greeks used a form of log line for recording the distances run from point to point along the coast while making their slow voyages from the Indus to the Persian Gulf about 325 BCE. The magnetic compass was brought to the West by Arab traders in the 12th century CE. The astrolabe was introduced by the Greeks in the 2nd century BCE. An instrument for measuring the altitudes of stars, or their angle of elevation above the horizon, took the form of a graduated arc suspended from a hand-held cord. A pivoted pointer that moved over the graduations was pointed at the star. The instrument was not used for nautical surveying for several centuries, remaining a scientific aid only.

The Greeks also possibly originated the use of the groma, a device used to establish right angles, but Roman surveyors made it a standard tool. It was made of a horizontal wooden cross pivoted at the middle and supported from above. From the end of each of the four arms hung a plumb bob. By sighting along each pair of plumb bob cords in turn, the right angle could be established. The device could be adjusted to a precise right angle by observing the same angle after turning the device approximately 90°. By shifting one of the cords to take up half the error, a perfect right angle would result.

About 15 BCE the Roman architect and engineer Vitruvius mounted a large wheel of known circumference in a small frame, in much the same fashion as the wheel is mounted on a wheelbarrow; when it was pushed along the ground by hand it automatically dropped a pebble into a container at each revolution, giving a measure of the distance traveled. It was, in effect, the first odometer.

The water level consisted of either a trough or a tube turned upward at the ends and filled with water. At each end there was a sight made of crossed horizontal and vertical slits. When these were lined up just above the water level, the sights determined a level line accurate enough to establish the grades of the Roman aqueducts. In laying out their great road system, the Romans are said to have used the plane table. It consists of a drawing board mounted on a tripod or other stable support and of a straightedge—usually with sights for accurate aim (the alidade) to the objects to be mapped—along which lines are drawn. It was the first device capable of recording or establishing angles. Later adaptations of the plane table had magnetic compasses attached.

Modern surveying can be said to have begun by the late 18th century. One of the most notable early feats of surveyors was the measurement in the 1790s of the meridian from Barcelona, Spain, to Dunkirk, France, by two French engineers, Jean Delambre and Pierre Méchain, to establish the basic unit for the metric system of measurement.

Total station: - This is the combination of digital theodolite and electro-magnetic distances measurement (E.D.M) which are used to measure both vertical and horizontal angle and their distance electronically in survey operation.

This surveying has two similar bits opposite function:

i.	The determination of existing relative horizontal and vertical position, such as that used for the process of mapping and

ii.	The establishment of marks to control construction or to indicate land boundaries further analyzed that surveying has been on essential element in the development of man’s environment for so many centuries that its importance is often forgotten. It is an imperative requirement in the planning and execution of nearly every form of constructions. Surveying was essential at the door of history and some of the significant scientific discoveries could never have been implemented were it not for the contribution of surveying. So believed that its principle modern uses are in the field of transportation, building appointment of land and communication inside which man can do without development is to be mention in man’s environment.

Survey dashes (2019) saw a perimeter and detail survey produce to provide the following.

1.	To determine the extent of individual holding to avoid to disputed of the land.

2.	To provide vital imprecation which must be preserved for land management and maintaining both of natural and artificial features.

3.	To locate portions of land of the physical surface of earth together with detail and it by means of survey beacons and showing the survey one plan.

4.	To determine the extent, size, value, ownership, of the land.

Many improvements and refinements have been incorporated in all the basic surveying instruments. These have resulted in increased accuracy and speed of operations and opened up possibilities for improved methods in the field. In addition to modification of existing instruments, two revolutionary mapping and surveying changes were introduced: photogrammetry, or mapping from aerial photographs (about 1920), and electronic distance measurement, including the adoption of the laser for this purpose as well as for alignment (in the 1960s). Important technological developments starting in the late 20th century include the use of satellites as reference points for geodetic surveys and electronic computers to speed the processing and recording of survey data.

CHAPTER THREE

3.0	METHODOLOGY

This chapter basically describes the methods and principles adopted in carrying out the project work. The execution of this project was based on the following basic principles of survey.

- The principle of working from whole to part.

- The principle of choosing the method of survey that is most appropriate to get the desired result.

- The principle of provision of adequate checks to meet the required accuracy.

The processes adopted include:

- Reconnaissance

- Test of instrument

- Control check

- Traversing (perimeter)

- Linear measurement & detailing by radiation method)

For this project, the ground survey method was adopted with the use of total station and its accessories for data acquisition in order to identify the physical features and to determine The XY coordinates (i.e Northing, Easting).

- RECONNAISSANCE

Reconnaissance which is the first stage and vital aspect of any survey work Carried out is as well the preliminary stage of this project.

This also is the initial operation or preliminary investigation undertaken by the surveyor in order to have a thorough overview of the site before the commencement of the actual survey ,it can also be abbreviated as “recci’’ as the project was concerned the reconnaissance was carried out in two ways.

- Field reconnaissance

- Office reconnaissance

3.1.0	Field Reconnaissance

This involves the actual [physical] visit to the site and was carried out before the actual operation. This project site was visited by the group in order to have a prior knowledge as well as true picture of the site and to ascertain the information collected during the office planning. The boundaries were marked with wooden peg driven into the ground to avoid disturbance or removal, taking into consideration the following factors.

- Inter-visibility of the selected traverse station.

- Safety of the selected station for future reference.

- Accessibility of the stations.

    - Office Reconnaissance

This involves knowing the type of instrument, purpose and accuracy required for the survey to be carried out. Information related to the given project was collected from various sources such as project supervisor. The specification/ instructions and coordinates of the control stations were collected from the department of surveying and geo-informatics Kwara State Polytechnic.

3.2	INSTRUMENT TEST

HORIZONTAL COLLIMATION TEST

The aim of this test was to be sure that the line of sight is perpendicular to the trunion axis.

Procedure:

The Total Station instrument was set over a point and all necessary temporary adjustments (centering, leveling and focusing) performed. Then the configuration menu of the total station was accessed by pressing down the menu key for about two seconds and the calibration sub-menu and consequently the horizontal collimation test was chosen. This test was done by sighting and bisecting a well-defined vertical target about 100m away and taking the horizontal readings on face left and face right. From the analysis of the results, the total station was in good adjustment.

VERTICAL INDEX ERROR TEST

This adjustment ensures that the vertical circle reading is exactly 90° when the line of sight is horizontal. Any deviation from this figure is termed vertical index error.

Procedure:

The instrument was set over a point and necessary temporary adjustments (centering, leveling and focusing) performed. The vertical index error test was carried out by sighting a target at a distance of about 120m on face left. The vertical circle reading was recorded and on face right the target was sighted and bisected again and the vertical circle reading recorded.

3.2.1	IN-SITU CHECK FOR CONTROL

In-situ checks observations (angular and linear) were executed for the purpose of verifying the integrity of the existing controls. The following observational schedules were executed:

The instrument was set on KWT3001 and angular observations were made to targets on KWT3002 as back station and KWT3003 as forward station.

FC3184S

The results of the observations as shown below confirm that the controls were still in their original positions and therefore suitable for use.

TABLE 2: IN-SITU CHECK DATA ANALYSIS (control pillars).

| STATION | COORDINATE (m) | KNOWN VALUES (m) | MEASURED VALUES (m) | DIFFERENCE (m) |
| --- | --- | --- | --- | --- |
| KWT3001 | NORTHING EASTING | 946478.324 678540.211 | 946478.322 678540.208 | 0.002 0.003 |
| KWT3002 | NORTHING EASTING | 946448.336 678500.342 | 946448.330 678500.346 | +0.006 -0.004 |
| KWT3003 | NORTHING EASTING | 946321.452 678612.394 | 946321.457 678612.397 | -0.005 -0.003 |

.

KWT3002

.

.111

KWT3003

KWT3001

FIGURE 3.2.1 Diagram Showing Control Used

3.2.2	DATA ACQUISITION

This involves the processes in acquiring the data needed for the project. This involves the actual making of measurements and recording of observed data on the field. There are different methods of acquiring data in the site with different instrument such as Total station, Theodolite, Compass, Level Instrument etc.

3.2.3 Geometric Data Acquisition.

These are positional data, that is, they are data having the [X, Y] coordinates which is possible to locate their position on the surface of the earth.

3.2.4 Attribute Data Acquisition.

These data are acquired by social survey, these are data used for defining the purpose of features located on the earth surface.

3.2.5 EQUIPMENT USED/SYSTEM SELECTION AND SOFTWARE

This comprises of two components, namely: the hardware components and software components.

HARDWARE COMPONENT: These are the physical equipment used for the execution of the project and they are:

- Total station and its accessories

- Steel tape

- Nails and bottle corks

- Field book and pen

- Personal computer

- pegs

SOFTWARE USED FOR DATA PROCESSING

- AutoCAD 2017 for plotting the boundary and detailing

- Note Pad, and Microsoft Excel (for Script preparation, editing and restructuring of data and report writing).

3.2.6    SETTING OUT OF PERIMETER BOUNDARY

Based on the office planning and field reconnaissance conducted, the instrument was first set on Control Pillar KWT3001 being the closest control and all temporary adjustment performed. The coordinate of the control point KWT3001 was key into the instrument.KWT3002 was sighted as back sight.

The coordinates of the station KWT3002 was key-in the instrument via the keyboard. The instrument then computed the bearing between the two stations for orientation. Then coordinates of points to be set out were entered into the total station and the setting out program of the instrument was used to get the angle to turn in order to face the direction of the point after orientation, the instrument was rotated until horizontal angle read 0 00’ 01”, reflector was held along the direction and distance between the instrument and the reflector was measured.  The instrument displayed the remaining of the point to be fixed distance as either positive or negative. Positive distance means that the reflector should move away from the instrument by that amount while negative distance means that the reflector should move towards the

instrument. When the horizontal angle read 000’ 01” and measured distance displayed 0.000m this marked the exact position to be set out.

3.3     MONUMENTATION

The beacons measuring 18cm by 18cm by 75 cm were molded in-situ with a mixture of 1:2:3 of cement, sand and gravel respectively.  A 12mm diameter Iron rod defines the center of the beacon was placed. The perimeter boundary line was cleared to ensure inter-visibility between the beacons. The numbering of the beacons was carried out after molding in a clockwise pattern with an arrow pointing to the succeeding station.

KW

T

3001

FIGURE 3.2: PLAN VIEW

18cm

●

15cm Above ground level

12 mm iron rod

75cm

60cm

Beneath ground level

18cm

FIGURE 3.2:	PROPERTY BEACON

3.4  	PERIMETER TRAVERSING

After the demarcation, capping and numbering of the beacons, the actual data acquisition using the total station

commenced. The traverse started from KWT3001 with KWT3002 as reference point. The total station was set up over control KWT3001, centered, leveled and telescope focused to eliminate parallax. The parameters of the instrument station i.e. station name, height of instrument over the station mark, and the XYZ coordinates of the station were keyed in. The reference control point was then bisected and the station name KWT3002, height of target over the station mark, and the XYZ coordinates of the station were key in.  Though the total station was set in coordinate mode it actually measured and recorded horizontal readings, vertical readings and distances automatically into the internal memory of the instrument on both faces which it used to compute and display coordinates. At every set up of the total station, the temporary adjustment was carried out and the following parameters measured:

- Height of instrument

- Height of the back target

- Height of the fore target

- Distance to back and fore station

This is the determination of bearing and distance of series of connected lines from known coordinated point so as to obtain coordinate of the newly established station.

This include the following with formula

- Linear measurement : the difference between the coordinates were first derived using (∆E=E2-E1, E3-E2) etc. and the distance of end traverse leg was obtained  using the formula: distance =√(∆E)^2 +(∆N)^2

- Angular measurement: to calculated the bearing after the difference in coordinates has been derived, the formula is Bearing=Tan-1 ∆E/∆N.

3.4.1.   COORDINATES OF THE BOUNDARY

TABLE 3: The coordinates are as follows

| S/n | Easting | Northing |
| --- | --- | --- |
| 1 | 678567.030 | 946551.942 |
| 2 | 678692.242 | 946546.081 |
| 3 | 678840.570 | 946538.912 |
| 4 | 678837.737 | 946354.577 |
| 5 | 678835.596 | 946251.888 |
| 6 | 678560.478 | 946251.018 |

Source: Author 2025

3.5   DETAIL SURVEY

Detailing of all features (both natural and man-made) within the site was made by shooting ray to fixing with the instrument.

The instrument was set up on station KWT3001,switched on and adjustments were carried out. Then, “Job” and “Station Name” were set in the instrument so as to recall the coordinates of the boundary point from the instrument’s memory. Also, heights of instrument above the instrument station  and heights of reflectors were measured with steel tape and stored in the instrument’s memory. A reflector placed on beacon KWT3002 was bisected for orientation. The Total station was instructed to compute the bearing between the two stations after input of the orientation station name KWT3003. One of the site assistant placed a reflector  at the edge of a building , the reflector’s cross hair was bisected with that of the telescope of the total station and “DIST” key was pressed for measurement, display and recording. Then the width of the scream was measured with a 50meter steel tape. The same procedure was adopted in detailing the express way by setting on a boundary beacon KWT3002and orienting KWT3003.In this case, all the edges of the carriage way and some buildings were picked.

CHAPTER FOUR

4.0	DATA PROCESSING

As the instrument downloading cable is faulty, Microsoft Excel 2007 Software was used to type the final coordinates of all points except the unwanted part of the data like the temporary controls which were later transported to Note pad and AutoCAD software 2017.

4.1	RESULT ANALYSIS

The data was analyzed and found to meet with the departmental standards and this is the main traverse result extracted from field, then it was arranged accordingly as the observation was held in the field. The results are as follows.

4.2	COMPUTATION

Table 4: Back Computation

| FROM PIL | Bearing | Distance | ∆E | ∆N | Coordinates E N |  | TO PIL |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | 678567.030 | 946551.942 | PIL 1 |
| PIL 1 | 920 40’ 48” | 125.349 | 125.212 | -5.861 | 678692.242 | 946546.081 | PIL 2 |
| PIL 2 | 920 40’ 01” | 148.501 | 148.328 | -7.169 | 678840.570 | 946538.912 | PIL 3 |
| PIL 3 | 1800 5’ 30” | 184.357 | -2.833 | -184.335 | 678564.737 | 946354.577 | PIL 4 |
| PIL 4 | 1810 10’ 18” | 104.711 | -2.141 | -104.689 | 678291.596 | 946249.888 | PIL 5 |
| PIL 5 | 2700 14’ 07” | 275.120 | -277.118 | 01.130 | 678573.583 | 946251.018 | PIL 6 |
| PIL 6 | 010 14’ 50” | 300.995 | 06.552 | 300.924 | 678567.030 | 946551.942 | PIL 1 |

DOUBLE LATITUDE METHOD

-5.861 X+125.21.=-733867532

-5.861

-11.722

-7.169

-18.891 x 148.328= - 2802.064248

-7.169

-26.060

-184.335

-210.395 x -2.833= 596.049035

-184.335

-394.730

-104.689

-499.419 x -2.141 = 1069.356079

-104.689

-604.108

+01.130

-602.978 x -277.118= 167096.0574

+01.130

-601.848

+300.924

-300.924 x + 06552 = - 1971.654048

+300.924

+ 000.000

163253.876

2

Area= 81626.938sqm

= 8.1626 Hectares

4.3: COMPUTE FOR TOTAL AREA USING DOUBLE LATITUDE AND DEPARTURE

Table 6

| FROM PIL | ∆E | ∆N | Easting | Northing | TO PIL |
| --- | --- | --- | --- | --- | --- |
|  |  |  | 678567.030 | 946551.942 | PIL 1 |
| PIL 1 | 125.212 | -5.861 | 678692.242 | 946546.081 | PIL 2 |
| PIL 2 | 148.328 | -7.169 | 678840.510 | 946546.912 | PIL 3 |
| PIL 3 | -2.833 | -184.335 | 678837.737 | 946354.577 | PIL 4 |
| PIL 4 | -2.141 | -104.689 | 678835.596 | 946249.888 | PIL 5 |
| PIL 5 | -277.118 | 01.130 | 678560.478 | 946251.018 | PIL 6 |
| PIL 6 | 06.552 | 300.924 | 678567.030 | 946551.942 | PIL 1 |

Source: Author Survey, 2025

4.4	GRAPHIC PLOTTING

This simply refers to the graphically representation i.e. plotting of plan. It was plotted using AutoCAD and other software in a computer system and a suitable scale was used to for the hard copy format. Presented information include boundary details and beg, conventional sign and symbol were also used in the plan.

The digital plan was produced using AutoCAD software and these are procedures followed.

- Switch on the computer and allow it to boost

- Select notepad, from notepad, a script file for the coordinate p-line easting, northing was structured.

- File was saved with extension. SCR

- AutoCAD was launched.

- Format was clicked and set the unit then press “OK”

- Press “Tool” and select Run script to pick your saved file then press escape and press zoom, extent and the image was displayed.

The boundary line was changed to Red and necessary editing was done.

CHAPTER FIVE

SUMMARY, CONCLUSION, PROBLEM ENCOUNTERED AND RECOMMENDATION

5.1	SUMMARY

This project is located at Institute of Finance and Management Studies, Kwara State Polytechnic, Ilorin. Kwara State. The project was carried out in accordance with third order specification.

The reconnaissance survey was properly carried out both in office and field, this is done for proper planning of the operation by locating initial controls that is within the project site for proper orientation, the instrument to be used and selection of traverse station in which the indelibility of the selected were put into consideration and finally, drawing of sketched diagram of the area to be surveyed. The field operation included perimeter traverse and detailing survey. Therefore; data processing was done and the plan was produced finally in digital format title plan showing all the feature of the area.

5.2	CONCLUSION

In conclusion the project task and exercise has been successfully executed since the results of the above operation agreed with the requirements and accuracy of third order and the survey plan of the study area were produced the practical was done in accordance with survey rules and regulation and department instructions in carrying out the project conclusively the report writing was done on how the entire project was executed both field work.

5.3	PROBLEM ENCOUNTERED

The problem encountered on the site was the movement of student in the school premises and some social survey by the student in the school premises which leads to delaying of observation and causing distraction from focusing on the work to make it faster.

Also our practical was done in the raining season we have to be doing it under the rain.

Also, the gradation on the leveling slate it has almost faded away; there by making observed a little but hard.

5.4	RECOMMENDATION

Based on the results and conclusion the following recommendation are being made

- I hereby recommend this particular project practical to be done often and often to update the infrastructure features in the society for the development of the particular area.

- The school management should give proper supervision to the any project and to be monitoring committee that will be looking into the reestablishment of disturb control.

- Also, it should be carried out in school for the next development in the school premises,

- More over it is necessary for every surveying and Geo informatics student to be able to carried out this particular practical.

REFERENCES

Abiodun M. (2020), Introduction to Perimeter and detailing. Said there is some evidence that, in addition to a marked cord, wooden rods were used by the Egyptians for distance

Albert Bodge (2019. Described a perimeter survey as a type of land survey that defines the perimeter boundaries of a particular parcel of real estate property.

Dashes (2017). Perimeter and detailing as a survey research .

Igbokwe (1999) Digital mapping-a big stride vol-12 no 1 Magazine of Nigeria institution of Surveying Students University of Nigeria chapter.

APPENDIX

| ID | EASTINGS | NORTHINGS |
| --- | --- | --- |
| PT1 | 680146.6 | 946572.7 |
| PT2 | 680349.3 | 946569.9 |
| PT3 | 680345.6 | 946440.4 |
| PT4 | 680145.2 | 946447 |
| B1 | 680152.4 | 946552.1 |
| B2 | 680201.3 | 946550.7 |
| B3 | 680200.5 | 946541.6 |
| B4 | 680152.4 | 946544 |
| B5 | 680151 | 946532.8 |
| B6 | 680201.3 | 946531.8 |
| B7 | 680200.5 | 946519.9 |
| B8 | 680151.3 | 946520.7 |
| B9 | 680151 | 946512.4 |
| B10 | 680200.5 | 946512.4 |
| B11 | 680200 | 946500.8 |
| B12 | 680151.3 | 946503.2 |
| B13 | 680150.8 | 946492.9 |
| B14 | 680199.2 | 946492.1 |
| B15 | 680198.9 | 946481 |
| B16 | 680150.8 | 946481.8 |
| B17 | 680238.6 | 946469.5 |
| B18 | 680289.6 | 946468.5 |
| B19 | 680289.6 | 946456.3 |
| B20 | 680238.6 | 946456.9 |
| B21 | 680212.8 | 946460.6 |
| B22 | 680226.1 | 946460.2 |
| B23 | 680227.1 | 946506.5 |
| B24 | 680214.8 | 946505.2 |
| B25 | 680213.2 | 946560.5 |
| B26 | 680308.1 | 946558.5 |
| B27 | 680309.1 | 946485.7 |
| B28 | 680297.5 | 946485.1 |
| B29 | 680299.5 | 946545.6 |
| B30 | 680228 | 946548.9 |
| B31 | 680239.3 | 946517.8 |
| B32 | 680283.9 | 946517.5 |
| B33 | 680284.9 | 946505.2 |
| B34 | 680239 | 946505.2 |
| R1 | 680350.3 | 946629.8 |
| R2 | 680353.2 | 946593.6 |
| R3 | 680354 | 946521.3 |
| R4 | 680351 | 946442.2 |
| R5 | 680348.3 | 946384 |
| R6 | 680358.3 | 946383 |
| R7 | 680360.6 | 946441.2 |
| R8 | 680361.9 | 946565 |
| R9 | 680361.9 | 946594.8 |
| R10 | 680360.2 | 946630.5 |
| TR1 | 680319.7 | 946498.1 |
| TR2 | 680313.1 | 946481.2 |
| TR3 | 680312.8 | 946460.1 |
| TR4 | 680293.9 | 946442.1 |
| TR5 | 680347.6 | 946510 |
| TR6 | 680324 | 946546.4 |
| TR7 | 680286.3 | 946564.2 |
| TR8 | 680267.1 | 946566.2 |
| TR9 | 680267.1 | 946558.6 |
| TR10 | 680257.6 | 946533.2 |
| TR11 | 680287.3 | 946535.1 |
| TR12 | 680241.4 | 946479.9 |
| TR13 | 680231.4 | 946539.4 |
| TR14 | 680167.6 | 946513.3 |
| TR15 | 680182.5 | 946450.8 |
| TR16 | 680211.3 | 946450.8 |