PROJECT REPORT

ON

ROUTE SURVEY

OF

OKE-OSE- SENTU ROAD, OFF OLD JEBBA ROAD, ILORIN EAST LOCAL GOVERNMENT AREA, KWARA STATE.

BY

BADMUS FATHIA ARIKE

MATRIC NO.: ND/23/SGI/FT/070

BEING A PROJECT REPORT SUBMITTED TO THE DEPARTMENT OF SURVEYING AND GEO-INFORMATICS, INSTITUTE OF ENVIRONMENTAL STUDIES.

IN PARTIAL FULFILMENT OF THE REQUIREMENT FOR THE AWARD OF ORDINARY NATIONAL DIPLOMA (OND) IN SURVEYING AND GEO-INFORMATICS, KWARA STATE POLYTECHNIC, ILORIN.

JUNE, 2025

CERTIFICATION

This is to certify that BADMUS FATHIA ARIKE; with matriculation number ND/23/SGI/FT/0070 has satisfactorily carried out the survey duties contained in his project under my instruction and direct supervision.

I hereby declare, that he has conducted himself with due diligence, honesty and sobriety on the project.

____________________					___________________

Surv. Abdulsalam Ayuba						Sign and Date

Project supervisor

____________________					___________________

Surv. S. A. Awoleye							Sign and Date

Project Coordinator

____________________					__________________

Surv. I. I. Abimbola						Sign and DateHead of Department

____________________					__________________

External Examiner							Sign and Date

DEDICATION

This project is dedicated to Almighty God and my lovely parents MR AND MRS BADMUS who saw me through the programme.

ACKNOWLEDGEMENT

All glory, Honor and Adoration to God Almighty, the gracious one who has given me the opportunity to complete my ND programme.

My appreciation goes to my supervisor Surv. Abdulsalam Ayuba, for his encouragement and motivation as regard the writing of this project thank you and God bless you. And to all lecturers in the Department of Surveying and Geoinformatics, Surv. I .I.Abimbola (HOD). Surv. A. G. Aremu, Surv. R. S. Awoleye, Surv. R. O. Asonibare, Surv. Ayuba Abdulsalam, Surv. Oguntayo Benard, Surv. Babatunde Kabir, Surv. Bello F.D, Surv. Sam and others. May God bless you all (Amen).

I also express my sincere gratitude to my wonderful parent Mr. and Mrs. BADMUS for their financial moral support toward the completion of this program. You will reap the fruit of your labor. (Amen).

ABSTRACT

The Route Survey from G.T Junction way to Sentu Village was carried out by carried out recce survey, planning at the office. Differential GPS GNNSS receiver was used for the traversing and detailing while the controls establishment and benchmarks was also done by DGPS all across the route line at every 1km and the data was processed using CivilCAD2014 with the AutoCAD 2007. Series of engineering plans was produces like horizontal alignment plan and vertical alignment plan.

TABLE OF CONTENTS

TITLE PAGE		-	-	-	-	-	-	-	-	i

CERTIFICATION	-	-	-	-	-	-	-	-	ii

DEDICATION	-	-	-	-	-	-	-	-	iii

ACKNOWLEDGEMENT	-	-	-	-	-	-	-	iv

ABSRACT	-	-	-	-	-	-	-	-	-	v

TABLE OF CONTENT	-	-	-	-	-	-	-	vi

CHAPTER ONE:

1.0	INTRODUCTION 	-	-	-	-	-	-	-	1

1.1 	AIM 	-	-	-	-	-	-	-	-	-	2

1.2 	OBJECTIVE 	-	-	-	-	-	-	-	3

1.3 	SCOPE 	-	-	-	-	-	-	-	-	3

1.4 	PERSONNEL 	-	-	-	-	-	-	-	4

1.5 	LOCATION 	-	-	-	-	-	-	-	-	4

1.5.1	MAP OF THE SITE LOCATION	-	-	-	-	-	5

CHAPTER TWO

LITERATURE REVIEW 	-	-	-	-	-	-	-	7

CHAPTER THREE

3.0 	METHODOLOGY 	-	-	-	-	-	-	-	12

3.1 	RECONNAISSANCE 	-	-	-	-	-	-	12

3.1.1 	OFFICE PLANNING 	-	-	-	-	-	-	13

3.1.2	FIELD PLANNING 	-	-	-	-	-	-	13

3.1.3 	FIELD PREPARATION 		-	-	-	-	-	14

3.2 	 MOMENTATION 	-	-	-	-	-	-	-	14

3.3 	EQUIPMENT USED 	-	-	-	-	-	-	15

3.4    METHOD USED 	-	-	-	-	-	-	-	16

CHAPTER FOUR

4.0	DATA PROCESSING AND RESULT ANALYSIS 	-	-	18

4.1  	DATA DOWNLOADING 		-	-	-	-	-	18

4.2	DATA PROCESSING 	-	-	-	-	-	-	19

4.3	RESULT ANALYSIS 	-	-	-	-	-	-	19

4.3.1	LONGITUDINAL /HORIZONTAL ALIGNMENT PROFILE 	19

4.4	INFORMATION PRESENTATION/ PLAN PRODUCTION 		20

CHAPTER FIVE:

5.0	SUMMARY AND CONCLUSION	-	-	-	-	24

5.1 	SUMMARY 		-	-	-	-	-	-	-	24

5.2 	PROBLEMS ENCOUNTERED 	-	-	-	-	-	24

5.3	SOLUTION TO THE PROBLEM 	-	-	-	-	25

5.3 	CONCLUSIONS 	-	-	-	-	-	-	-	25

5.4 	RECOMMENDATION 	-	-	-	-	-	-	25

REFERENCE 	-	-	-	-	-	-	-	26

APPENDIX 	-	-	-	-	-	-	-	-	28

CHAPTER ONE

1.0 	INTRODUCTION

Route surveying is a major type of surveying under the broad engineering surveying which is one of the branches of surveying. Route survey is a large-scale engineering survey carried out for the purpose of obtaining necessary data required by the engineers for planning, and design of any route such as highways, railways, canals and pipelines etc. This kind of information is obtained by the Surveyor for proper assessment of natural and manmade features as well as to provide information for vertical and horizontal alignment along the route for setting out routes and constructions. It requires surveyors with excellent experience in planning, designing and setting out for the construction works.

A route survey is a detailed investigation and analysis of a proposed or existing pathway for transportation or infrastructure development. The route survey of Oke-Ose-Sentu road, in Ilorin, Kwara State, involves the systematic collection of data on the physical, environmental, and socio-economic characteristics of the area to guide planning and construction activities OkeOseSentu road, in Ilorin, is a significant urban area that requires thorough assessment to improve traffic flow, safety, and accessibility. The survey aims to identify the current conditions of the road, existing infrastructure, traffic patterns, and potential challenges such as drainage, land use, and encroachments. This information is crucial for designing road improvements, expansions, or new developments that meet the needs of the community and support sustainable urban growth.

Route surveying is comprised of all survey operations required for design and construction of engineering works such as highways, pipelines, canals, or railroads. At Caltrans a route surveying system is generally associated with highway design and construction.

Engineering surveying may be regarded as a specialty within the broader professional practice of engineering and includes all surveying activities required to support the conception planning, design, construction, maintenance, and operation of engineered projects. Engineering surveying excludes the of real property, for the establishment of land boundaries, right of way, easements, and the dependent or independent surveys or resurveys of the public land survey system. ASCE believes that this definition should be adopted by state engineering licensing boards.

Road failure could be cracks, potholes, bulges, and depression, which makes road networks unsafe and not motor-able to road users. Road failure in Nigeria is a major problem after the problem of power supply, it has not only caused a setback to the Nigerian economy but has led to the loss of lives and properties in millions of Naira annually. Rehabilitation of this road network has become a financial burden to the Federal, State, and Local government. Federal Road Safety Corps (FRSC 2011) reported that Nigeria ranks 191 of 192 countries in the world with unsafe roads, and as a result, 162 deaths per 10,000 populations occur from road crashes. Most accidents recorded occurred within the bad portions that are due to failure. Vehicles in the bid to avoid potholes, cracks, depressions take the wrong direction sometimes in the dark, jump into these bad spots, and results in a head-on collision. Herbert Macaulay Yaba Lagos state road also has been affected; vehicles spend longer hours before getting to their destinations due to road failure and this has negatively affected the socio-economic activities of people living along with the road network.

1.1 	AIM

The aim of the route survey project at OkeOse is to carry out a detailed route survey that includes determining the exact location, alignment, and profile of the road segment for construction or improvement purposes. This involves producing both horizontal and vertical alignment plans, marking chainages, conducting traversing to establish spot heights, and collecting data for design and earthwork volume computations.

1.2 	OBJECTIVE

Objectives of the Project

The following objectives were strictly followed for the actualization of the aims of the project.

- Carryout proper reconnaissance which involves office planning and field reconnaissance for route survey of OkeOseSentu road.

- Carryout the three (3) dimensional observation (X,Y,Z) of the profile and cross section of the route as well as fixing of all the details along the route of the project site.

- Carryout the design of the route by following the survey rules and departmental instructions.

- Present all the information in both soft and hardcopy For route survey of OkeOseSentu road.

1.3 	SCOPE

A route survey project-such as the one proposed for OkeOseSentu road encompasses the systematic collection and analysis of data necessary for the planning, design, and eventual construction or rehabilitation of a roadway. The scope defines the boundaries, tasks, and deliverables required to achieve the project’s objectives.

1.4 	PERSONNEL

The personnel involved in this execution of this project are as follows.

| NAME | MATRIC NUMBER |
| --- | --- |
| ALADE FLORENCE ABOSEDE | ND/23/SGI/FT/0071 |
| AWOSUNLE BOLUWATIFE OLUWAPONMILE | ND/23/SGI/FT/0072 |
| KAZEEM FARIDAH ENIOLA | ND/23/SGI/FT/0069 |
| MURITALA MUJIDAH KEHINDE | ND/23/SGI/FT/0068 |
| BADMUS FATHIA ARIKE ADE | ND/23/SGI/FT/0070 |
| AZEEZ FARUQ AYOMIDE | ND/23/SGI/FT/006 |
| OLAREWAJU BOLUWATIFE ADURAMIGBA | ND/23/SGI/FT/0067 |

1.5 	LOCATION

The study area is at Sentu – Oke-Ose Road, off Old Jebba road, Ilorin East Local Government Area Kwara State. The length of this project is 5km covered the Latitude (8⁰32’54.40”)N Longitude (4⁰39’10.90”)E and Latitude (8⁰30’52.57”)N Longitude (4⁰40’55.30”)E.

1.5.1 MAP OF THE SITE LOCATION

FIG.1.0  Showing Nigeria map, Kwara state map, and topographical map covering the project area .

Fig. 1.1  showing the imagery covering the project area

CHAPTER TWO

2.0	LITERATURE REVIEW

A literature review on route survey covers the methodologies, technologies, and applications involved in establishing horizontal and vertical alignments for transportation routes such as roads, railways, and pipelines. Route surveying involves determining terrain, topography, and ground configuration to plan and construct transportation facilities, focusing on horizontal and vertical control for design and construction.

Various methodologies are used, including traditional surveying, GPS-based techniques, and advanced technologies like LiDAR, each with strengths and limitations. Applications span transportation planning, urban development, environmental monitoring, and infrastructure maintenance, highlighting the importance of accurate route data. Challenges include data accuracy, cost-effectiveness, and processing large datasets; my research aims to improve these aspects and integrate new technologies.

Studies also emphasize the importance of systematic route selection criteria in route surveys and navigation experiments to ensure reliability and reduce bias, considering factors like route length, environment type, and decision points along the route.

Important references for further reading include works by Smith et al. (2018) on route survey methodologies, Johnson (2019) on urban infrastructure, Chen et al. (2020) on LiDAR and GPS integration, and Lee (2021) on comprehensive route survey technologies.

Route Surveying is such a survey exercise that requires all field works and calculations made for the purpose of locating and constructing a cross country social utilities such as highways, railways, canals, transmission lines and pipelines. This involves the determination of the ground configuration and location of physical features (naturals and man-made) along the route, establishing the line on the ground and computing the volume of earthwork. The main purpose of any route survey is to:

- Select one or more tentative general route for the roadway or utility

- Gather enough information about the general route to make it possible for the route designers to select the final location of the route, and

- Mark this final location selected from the available options.

With these purpose, route survey usually entails reconnaissance, preliminary and final-location survey phases that satisfy respectively, each of the purposes given above. Sometimes, the prevailing circumstances may preclude the requirement to perform all three phases; for example if a new road or utility line is to be constructed on a military installation, having a well-marked vertical and horizontal control networks and up-to-date topographic maps and utility maps, then the reconnaissance and preliminary survey phases may not necessarily be required.

Punmia et al (2005) explained route surveying as surveys along a comparatively narrow strip of territory for the location, design and construction of any route of transportation, such as highways and railroads, aqueducts, canals and flumes, pipeline for water, sewage, oil and gas, cableways, belt conveyors and power, telephone and telegraph transmission line is called ‘ROUTE SURVEY’. At this stage, it must be noted that the examples mentioned above cannot design themselves until some methods are used to make them refined.

It is worthy to note the construction of roads in Nigeria dated back to decades ago or during colonial era; roads are commonly found narrow till now, few are found serviceably and their horizontal and vertical alignment are much influenced by their topography. The road construction was then constrained by non-availability of earth moving equipment, limited funds, no skillful personnel and fewer numbers of vehicles on roads.

More so, certain procedures or method are used when carrying out a route survey explained by Basak (1994) that when the question of constructing a new road for public demand or some strategic reasons arises, the procedures to be taken are:

- To find the necessity for the existence of the road

- The marking of the tentative alignment

- The reconnaissance survey

- Preliminary location survey traverse

- Final location survey and report

He further explained that the tentative alignments are marked on the general map and contour map and contour map of the area through which the route is expected to pass.

Meanwhile, in every construction work, the surveyor stands to perform certain roles and also must be familiar with some procedures of the work at hand as at the time of execution. In view of the foregoing, Anderson and Raymond (1985) stressed that “In order to plan and perform, the surveyors needed to acquire the data of a route survey and the surveyor must be familiar with the following:

- The geometry of horizontal and vertical control and how they are used in the route alignment procedure.

- The methods of acquiring terrain data utilized in route design.

- The procedures followed in processing terrain data to obtain earth work volumes

- Establishment on the ground, a system of staves both in plane and elevation, from which measurement of earth work and structures can be taken conveniently by the construction force

- Giving lines and grades as needed either to replace stakes disturbed by construction or to reach additional points on the structure itself.

- Making measurements necessary to verify the location of completed part of the structure as-built survey.

In addition to the above, the surveyor concerned must also learn or be familiar with the reasoning and interpretation of maps, the symbols and signs, scales, directive, locations etc. certain parameters therefore are necessary to explain routes e.g. cross section, longitudinal sections, curves and setting.

Engineering surveying involves determining the position of natural and man-made features on or beneath the Earth’s surface and utilizing these features in the planning, design and construction works. It is a critical part of any engineering project. Without an accurate understanding of the size, shape and nature of the site the project risks expensive and time-consuming errors or even catastrophic failure. Olalekan et al. (2018).

Ajibola et al. (2020). Rural Roads play a vital role in the development of any emergent country. In India, rural roads share more than 85% of the road network of the country. A well-planned road network in rural areas is one of the most important infrastructure elements which improves rural accessibility and contributes to the rural development as a whole. Planning the road network in rural areas is crucial, not only in terms of network efficiency (concerning accessibility), but also regarding construction and operation costs, as limited funding is available.

John, H. and Gorden, H. (1976): Route surveying includes the field and office work required to plan, design, and lay out any “long and narrow transportation facility. Ahmed Abdel-Rahim 2011 Route surveying which is a branch of Surveying has three different components, measuring distance, angles and elevation. Route surveys are essential in the planning, design, and execution of infrastructure projects. In the case of Kwara State, Nigeria, route surveys provide valuable data for infrastructure planning, urban development, and transportation networks. The focus of this literature review is to examine previous studies, reports, and relevant academic literature related to route surveys, particularly in the context of the administrative block of Kwara State. This review aims to highlight the importance of route surveys, methodologies used, challenges faced, and their relevance to urban planning and transportation development in Kwara State

CHAPTER THREE

3.0 	METHODOLOGY

This can be termed as a set of methods and principles used to perform a particular activity. For the activities to be successfully performed, proper planning is very important. This involves development of a work plan showing how goals and objectives are to be accomplished. Hence, planning is one of the essential factors for the effective project execution and management. Proper planning was taken for the execution of this project and this involved;

- The choice of the most appropriate techniques for carry out of the project

- Selection of equipment used

- The design of a monitoring scheme that really helped in achieving the required accuracy for the project, starting from reconnaissance to the final product of the project.

3.1	RECONNAISSANCE

This is an important and first aspect in any survey project carried out to obtain the general view of the study area in terms of the nature of the terrain and to adequately plan the best ways to the set aim and objectives of the project. The importance of reconnaissance to any survey work of any size and nature cannot be over-emphasized. Experience has proved that time spent in carrying out a good reconnaissance is not a wasted time since it contributes to the quick execution of any survey exercise and promotes easy survey work. Reconnaissance simply connotes the summation of all activities preceding the actual execution of a survey job. It involves taking a general study or view of an area of operation with a view of knowing how best the operation is to be carried out in terms of energy and time. As this project was concerned, the reconnaissance was carried out in two stages.

The two stages of reconnaissance are;

- Office planning

- Field planning

3.1.1       OFFICE PLANNING

Office planning is also known as office reconnaissance. It is a vital component of route surveying, enabling surveyors to gather existing data and information before conducting fieldwork. This process involves a thorough review of available resources, including maps, aerial imagery, and existing reports.

3.1.2.      FIELD PLANNING

The field reconnaissance was first carried out before the actual operation. This aspect involved site visitation to the project site by all the group members to have a pre-requisite knowledge of how it looks and how the field operation will be carried out.

During the visit, the control points planned to be used were marked, the reconnaissance facilitated the planning and carrying out of the actual survey as it was taking into consideration, the possible problem that are likely to be encountered, how such problems can be overcome or reduced to the barest minimum.

3.2	FIELD PREPARATION

This involved the operation carried out before the actual observation. The operation involved marking of chainages which is done at 25cm interval.

3.3. MOMENTATION

Temporary bench mark (TBM) were established and coordinated along the entire route of the project, which were meant to serve as controls for establishing Centerline chainage, setting out of curve and other road features during the actual construction.

The position of these beacons were selected in such a way that they are intervisible to each other, not too far from the proposed road and considerable number of Centerline can be set out from them.

The property beacon used were precast with dimensions 18cm square by 75cm in length. 65cm of the precast beacon was buried beneath the surface and 10cm above. This was done in compliance with the specification of cadastral survey regulations as specified in CAP 425 law of the federation of Nigeria. The property beacon was made of concrete mixture of ratio 3:2:1 of sand, granite and cement. The iron rod protruding at the center of the beacon formed the station mark.

The numbering was done serially from the beginning to the end of the proposed road and were prefixed with the identification number KP ND11 001

Fig. 3.0 showing typical survey beacon

3.4.   EQUIPMENT USED

3.4.1   HARDWARE USED

- Differential GPS

- Handheld GPS

- Linen tape

- Power supplies

- Nails and bottles cork

- Hammer

- Cabling and connectors

3.4. 11. 	SOFTWARE USED

- AutoCAD/ CivilCAD 2014

- Microsoft office (word and excel)

- Notepad

3.5.    METHOD USED

CORS TECHNOLOGY

CORS stands for (Continuously Operating Reference Station). It is a type of GPS or GNSS (Global Navigation Satellite System) station that:

  - Collects and transmits GPS/GNSS data continuously.

  - Provides real-time corrections to improve the accuracy of GPS signals.

CORS are used to enhance the precision and reliability of GPS positioning for various applications, including surveying, mapping, navigation, and more.

How you use a CORS with a data logger when collecting survey data:

How to Use a CORS with a Data Logger

1. 	Set Up Your GNSS Receiver & Data Logger

- Mount the GNSS antenna securely on your survey pole or tripod.

- Connect your data logger/controller to the GNSS receiver. The data logger is usually a handheld device or tablet used to configure settings and record data.

2. 	Configure the CORS Connection

- On the data logger, enter the CORS network settings:

- Enter the username

- IP address & port of the CORS provider.

- You’ll need mobile internet (, hotspot) on your data logger or receiver to access the CORS network in real time.

3. Select Correction Service Type

- Choose RTK corrections (Real-Time Kinematic) if you want live centimeter-level accuracy.

- Some systems also allow post-processing (PPK), where you log raw data and apply CORS corrections later.

4. Start Receiving Corrections

- Once connected, the GNSS receiver will start applying correction data from the CORS.

- The data logger will show “Fixed RTK” or “Float RTK” status, indicating correction quality.

5. 	Begin Surveying & Logging Points

- Move to the points you want to survey.

- Use the data logger to record positions, adding descriptions, codes, or attributes as needed.

- Each recorded point will have high-precision coordinates

6. Save & Export Data

- After collecting your points, you can export the data (CSV, DXF, shapefiles, etc.) from the data logger for further use in GIS, CAD, or mapping software.

In Simple Terms:

The CORS sends corrections to your rover via the internet. Your data logger controls the receiver and records corrected point data.

CHAPTER FOUR

4.0	DATA PROCESSING AND RESULT ANALYSIS

This stage involves downloading of the acquired data on field from the digital equipment to the personal computer for further processing. The data obtained were downloaded using a data transfer cable. After successfully downloaded of those data, they were edited using Microsoft Excel and Notepad Software which made it possible to easily import the edited copy into AutoCAD for drafting and designing. The coordinate obtained were in X, Y, Z format which were used for plotting the route’s longitudinal profile

4.1.  DATA DOWNLO1ADING

- The instrument was connected to the personal computer via downloading cable, the corresponding software was launched and the instrument port was selected.

- All the folders on the instrument were displayed. The folder containing the data for the group was then copied and pasted on another folder already created on the local drive of the personal computer.

- The folder was launched and the file containing the data was opened with notepad application.

- The results were in the format; Point ID, Easting, Northings and Height. The downloaded data were edited in Notepad, Microsoft Excel and a script were prepared in Notepad in order to be plotted in AutoCAD.

4.2	DATA PROCESSING

Data processing is a critical component of route surveys, enabling the transformation of raw data into usable information for design, analysis, and decision-making. Route surveys involve collecting vast amounts of data, including topographic information, environmental factors, and infrastructure details.

The data processing stage involves several key steps, including data cleaning, transformation, analysis, and visualization. Data cleaning removes errors, inconsistencies, and outliers, ensuring the accuracy and reliability of the data. Data transformation converts the data into suitable formats, while data analysis applies algorithm

The downloaded data from the equipment was further edited using Microsoft Excel and Notepad, the final edited copy was saved as text file containing X, Y, Z coordinates of all points observed in the field.

4.3	RESULT ANALYSIS

The results were analyzed so as to check the accuracy of the job by comparing the result obtained with the minimum allowable error acceptable for this order of survey job in accordance with survey rules and departmental instructions.

4.3.1	LONGITUDINAL /HORIZONTAL ALIGNMENT PROFILE

- In CivilCAD environment, Road menu was clicked and HORIZONTAL ALIGNMENT chosen

- Options button was clicked in the appeared dialogue box and Define section was then clicked to choose the section format and the distance between the sections. Format 2 was chosen and the distance between sections was taken to be 25m.

- Having chosen these options, OK was clicked twice.

- Pick tool was selected from the right pane dialogue box to pick the intersection points (IP), and appropriate radius values of curve was given to each IP as specified by the client.

- Apply button was then clicked to effect all the given parameters on the drawing. On each IP position, information about the IP is been displayed. Such information are; IP number, X coordinate, Y coordinate, Radius of Curve, Length of curve, Deflection angle and so on.

4.4	INFORMATION PRESENTATION/ PLAN PRODUCTION

The data acquired were processed into plan for visual presentation with AutoCAD/CivilCAD. The visual display graphical information in AutoCAD was printed as hardcopy of the plan. The longitudinal section, and profile were plotted.

Fig. 4.0 showing the Profile and Longitudinal Section from Chainage 0+000 – 1+000

Fig. 4.1 showing the Profile and Longitudinal Section from Chainage 1+000 – 2+000

Fig. 4.2 showing the Profile and Longitudinal Section from Chainage 2+000 – 3+000

Fig. 4.3 showing the Profile and Longitudinal Section from Chainage 3+000 – 4+000

Fig. 4.4 showing the Profile and Longitudinal Section from Chainage 4+000 – 5+025

CHAPTER 5

5.0	SUMMARY, PROBLEM ENCOUNTERED, RECOMMENDATION AND CONCLUSION.

5.1	SUMMARY

The project covered a total length of 5km. The field work however involved the following processes; Recce, monumentation traversing and detailing. CORS was used for data acquisition and its software for downloading and transforming the acquired data respectively. The adjusted coordinates were used for the production of the final plans.

5.2.	PROBLEMS ENCOUNTERED

- Accuracy concerns: Accuracy decrease distance from the CORS station. Interference from vegetation or buildings which also affect signal quality.

- Station Maintenance: CORS require regular maintenance and monitoring to ensure their reliability and accuracy. Unmaintained stations can introduce errors into the positioning data.

- Inadequate CORS Coverage: In areas with sparse CORS networks, the ability to provide accurate positioning across the entire route can be limited.

5.3	SOLUTION TO THE PROBLEMS

- Optimize station placement: Choose CORS that are strategically located for optimal coverage and accuracy in your survey area.

- Use higher-quality receivers: Employ survey-grade GPS receivers that are more robust to interference and capable of maintaining high accuracy.

- Implement error detection and correction techniques: Utilize post-processing techniques or real-time kinematic (RTK) methods to minimize errors.

5.4	RECOMMENDATION

Using CORS stations in route surveys offers significant advantages in terms of accuracy, efficiency, and cost-effectiveness. Choosing the appropriate CORS network, utilizing compatible equipment, and implementing effective field procedures are crucial for achieving high-quality survey results.  By leveraging this technology, surveyors can achieve higher quality results while reducing project timelines and expenses.

5.5	CONCLUSION

CORS systems are a valuable tool for route surveys, providing accurate, reliable, and efficient positioning solutions. With their ability to deliver centimeter-level accuracy and streamline survey workflows, CORS systems are becoming increasingly important for a wide range of surveying and mapping applications.

REFERENCE

Ajibola et al. (2020).Rural Roads, Standard Publishers Distributors. 4th Edition.

Abynayaka (1977), Paul and Radnor (1976), UNESCO (1991), World Bank (1991), Jain and Kumar (1998), FMWH (1995); Poor Maintenance according to John and Gordon (1976), Oglesby and Garry (1978), TRRL (1991); and Traffic Effects and Human Impacts on the Roads according to AASHTO (1976), FMWH (1995) ANSMWH (1998), and Ibrahim (2011).

Dr. I.P. Ifabiyi (2013): “Geotechnical Investigation of Road Failure along Ilorin-Ajase – Ipo Road Kwara State, Nigeria”.

Gupta, B. L. and Gupta, A. (2003): “Roads, Railways, Bridges, Tunnel and Harbor Dock Engineering”; Standard Publishers Distributors, NaiSarak, Delhi. 5th Edition.

John, H. and Gorden, H. (1976): “A practical Guide to Earth Road Construction and Maintenance”, Engineering Manual Vol. 2, No. 1. Pp. 43-45.

Momoh, L.O., Akintorinwa, O. and Olorunfemi, M.O. (2008): “Geophysical Investigation of Highway Failure- A Case Study from the Basement Complex Terrain of Southwestern Nigeria”. Journal of Applied Sciences Research. 4(6): 637-648.

Oglesby, C. H. and Garry, H. R. (1978): “Highway Engineering”, fourth edition, John Willey and sons, New York, p.691.

Ogunola, 1999, Geographic Information System

Olalekan et al. (2018). Defined Engineering Surveying

Priscilla. A. Glasgow Fundamentals of Survey Research methodology April 2005

Pinsonneault, A., & Kraemer, K. L. (1993). Survey research methodology in management information systems: An assessment. Journal of Management Information Systems, 10, 75-105.

Salant, P., &Dillman, D. A. (1994). How to conduct your own survey. New York.

APPENDIX

| Name | N | E | H |
| --- | --- | --- | --- |
| Base_0 | 937113.1 | 671374.3 | 290.1564 |
| PT1 | 945199.3 | 682040.6 | 376.7882 |
| PT2 | 945213.6 | 682040.1 | 376.9413 |
| PT3 | 945211.5 | 682003.7 | 375.523 |
| PT4 | 945195.8 | 682006.7 | 375.3559 |
| PT5 | 945196.7 | 682015.1 | 376.0015 |
| PT6 | 945195.3 | 682015.3 | 375.6575 |
| PT7 | 945198.1 | 682029.9 | 376.6706 |
| PT8 | 945197 | 682030 | 376.6791 |
| TBM | 945198.3 | 682030.3 | 376.5831 |
| PT9 | 945196.8 | 682023.3 | 376.4395 |
| TBM2 | 945195.5 | 682016 | 376.149 |
| PT10 | 945180.9 | 682042.2 | 375.4776 |
| PT11 | 945189.1 | 682045.9 | 376.1932 |
| PT12 | 945185.8 | 682044.1 | 375.9832 |
| PT13 | 945169.6 | 682068.4 | 375.8128 |
| PT14 | 945173.3 | 682070.4 | 376.1018 |
| PT15 | 945177.4 | 682070.2 | 376.1243 |
| PT16 | 945164.7 | 682102.3 | 377.128 |
| PT17 | 945158.5 | 682099.6 | 376.68 |
| PT18 | 945154.6 | 682097.6 | 376.4599 |
| PT19 | 945140.1 | 682122.3 | 376.9666 |
| PT20 | 945144.6 | 682125.7 | 377.2596 |
| PT21 | 945148.2 | 682127.9 | 377.3442 |
| PT22 | 945120.7 | 682147.4 | 377.4701 |
| PT23 | 945124.2 | 682150.8 | 378.0427 |
| PT24 | 945125.9 | 682152.4 | 378.0537 |
| PT25 | 945098.5 | 682168.5 | 378.0107 |
| PT26 | 945101.5 | 682173.1 | 378.1797 |
| PT27 | 945105.2 | 682176.5 | 378.1743 |
| PT28 | 945081 | 682188.1 | 377.3532 |
| PT29 | 945075.5 | 682192.4 | 377.1792 |
| PT30 | 945070.3 | 682187.8 | 376.6787 |
| PT31 | 945073.3 | 682183.8 | 376.4857 |
| PT32 | 945080.9 | 682194.6 | 377.4948 |
| PT33 | 945085.9 | 682197.4 | 377.5604 |
| PT34 | 945063.1 | 682219.9 | 377.4999 |
| PT35 | 945058.1 | 682215.5 | 377.3388 |
| PT36 | 945052.9 | 682210.7 | 377.3088 |
| PT37 | 945031.9 | 682232.7 | 377.3118 |
| PT38 | 945036 | 682236.9 | 377.4859 |
| PT39 | 945040.9 | 682242.2 | 377.962 |
| PT40 | 945019.5 | 682262.2 | 377.834 |
| PT41 | 945015.2 | 682258.3 | 377.7304 |
| PT42 | 945011 | 682255.5 | 377.7909 |
| PT43 | 944991.5 | 682273.8 | 377.8099 |
| PT44 | 944994.4 | 682278.7 | 377.9765 |
| PT45 | 944998.6 | 682283.3 | 378.3025 |
| PT46 | 944979.2 | 682305.5 | 378.9486 |
| PT47 | 944974.5 | 682300.9 | 378.4325 |
| PT48 | 944970.2 | 682296.8 | 378.4865 |
| PT49 | 944949.1 | 682317.9 | 378.8185 |
| PT50 | 944953.3 | 682323.2 | 378.8761 |
| PT51 | 944958.3 | 682328.4 | 379.2777 |
| PT52 | 944942.5 | 682346.5 | 379.5567 |
| PT53 | 944939.2 | 682351.6 | 379.7302 |
| PT54 | 944949 | 682356.3 | 379.9093 |
| PT55 | 944952.4 | 682352.9 | 379.9308 |
| PT56 | 944934.6 | 682344.2 | 379.2306 |
| PT57 | 944928.6 | 682339.6 | 379.1981 |
| PT58 | 944909.6 | 682361.3 | 379.4121 |
| PT59 | 944915.3 | 682366.2 | 379.7087 |
| PT60 | 944919 | 682370.1 | 380.1443 |
| PT61 | 944900 | 682390.6 | 380.2548 |
| PT62 | 944895.8 | 682386.2 | 379.8353 |
| PT63 | 944891.8 | 682382.8 | 379.7662 |
| PT64 | 944872.1 | 682400.3 | 380.2427 |
| PT65 | 944876.4 | 682405.1 | 380.2813 |
| PT66 | 944879.6 | 682408.2 | 380.6578 |
| PT67 | 944862.6 | 682424.9 | 380.9054 |
| PT68 | 944858.9 | 682421.8 | 380.8158 |
| PT69 | 944855.1 | 682417.9 | 380.8773 |
| PT70 | 944851.4 | 682421.7 | 381.0128 |
| PT71 | 944843.4 | 682416.4 | 380.5042 |
| PT72 | 944848.8 | 682411.8 | 380.4407 |
| PT73 | 944836.2 | 682437.8 | 381.1378 |
| PT74 | 944835.4 | 682436.9 | 381.2068 |
| PT75 | 944839.1 | 682440.6 | 381.1278 |
| PT76 | 944843.1 | 682444.7 | 381.3639 |
| PT77 | 944817.3 | 682455.6 | 381.5858 |
| PT78 | 944816.1 | 682454.6 | 381.7118 |
| PT79 | 944820.7 | 682459.4 | 381.6384 |
| PT80 | 944824.3 | 682463.1 | 381.8139 |
| PT81 | 944806 | 682480.9 | 382.633 |
| PT82 | 944797.9 | 682472.8 | 382.1354 |
| PT83 | 944801.9 | 682477 | 382.1574 |
| PT84 | 944785.9 | 682496.9 | 382.6775 |
| PT85 | 944778.8 | 682489.9 | 382.4644 |
| PT86 | 944782.6 | 682493.5 | 382.4369 |
| PT87 | 944764.9 | 682513 | 382.8735 |
| PT88 | 944762.1 | 682509.8 | 382.9524 |
| PT89 | 944758.6 | 682506.3 | 382.7369 |
| PT90 | 944737.4 | 682521 | 383.1014 |
| PT91 | 944740.6 | 682526.5 | 383.1414 |
| PT92 | 944744.8 | 682531 | 383.4055 |
| PT93 | 944721.6 | 682544.4 | 383.583 |
| PT94 | 944718.3 | 682540.9 | 383.5089 |
| PT95 | 944715.2 | 682536.7 | 383.4319 |
| PT96 | 944694.3 | 682552.6 | 383.8999 |
| PT97 | 944687.7 | 682545.1 | 383.3217 |
| PT98 | 944678.5 | 682553.9 | 383.4058 |
| PT99 | 944684.7 | 682563.7 | 384.1829 |
| PT100 | 944691.4 | 682573.8 | 384.36 |
| PT101 | 944702.8 | 682568.4 | 384.2641 |
| PT102 | 944705.4 | 682578.7 | 384.5537 |
| PT103 | 944698.9 | 682584.3 | 384.4892 |
| PL | 944699.7 | 682541.9 | 383.8583 |
| PL1 | 944671.7 | 682562.9 | 384.1068 |
| PT104 | 944679 | 682577.9 | 384.475 |
| PT105 | 944676.2 | 682575.2 | 384.6084 |
| PT106 | 944671.7 | 682571.4 | 384.1429 |
| PT107 | 944654.1 | 682586.9 | 384.8209 |
| PT108 | 944657.7 | 682590.8 | 385.0929 |
| PT109 | 944661 | 682594.8 | 384.9365 |
| PT110 | 944640.3 | 682610.2 | 385.5405 |
| PT111 | 944637.8 | 682607 | 385.5779 |
| PT112 | 944633.8 | 682603.1 | 385.6409 |
| PT113 | 944614 | 682619.2 | 386.2424 |
| PT114 | 944612.7 | 682618.6 | 386.0484 |
| PT115 | 944616.5 | 682623.3 | 386.1459 |
| PT116 | 944619.5 | 682626.9 | 386.2865 |
| PT117 | 944598.9 | 682640.3 | 386.9915 |
| PT118 | 944592.9 | 682633.9 | 387.0579 |
| PT119 | 944598.5 | 682628.4 | 386.9764 |
| PT120 | 944586.2 | 682627.1 | 387.1628 |
| PT121 | 944584.8 | 682632 | 387.3333 |
| PT122 | 944575 | 682630.7 | 387.2937 |
| PT123 | 944575.1 | 682626.5 | 387.2332 |
| PT124 | 944573.1 | 682649.1 | 387.7229 |
| PT125 | 944575.7 | 682653.2 | 387.7684 |
| PT126 | 944578.9 | 682658.2 | 387.681 |
| PT127 | 944558.3 | 682672.7 | 388.857 |
| PT128 | 944554.5 | 682669.7 | 388.8159 |
| PT129 | 944550.8 | 682667.6 | 388.7539 |
| PT130 | 944535.3 | 682684.2 | 389.5004 |
| PT131 | 944537.9 | 682688 | 389.5585 |
| PT132 | 944540.6 | 682690.9 | 389.5255 |
| AkehindeApekun Community | 944577.7 | 682638.6 | 387.7788 |
| AkehindeApekun Community1 | 944518.9 | 682702.6 | 390.1885 |
| PT133 | 944522.5 | 682706 | 390.2825 |
| PT134 | 944525.4 | 682708.1 | 390.3206 |
| PT135 | 944509.9 | 682727.9 | 391.0931 |
| PT136 | 944505.8 | 682725.4 | 391.1421 |
| PT137 | 944501.5 | 682723 | 391.112 |
| PT138 | 944486.2 | 682741.6 | 391.9906 |
| PT139 | 944488.9 | 682744.5 | 391.8936 |
| PT140 | 944492 | 682747.1 | 391.7972 |
| PT141 | 944476.5 | 682767.2 | 392.2418 |
| PT142 | 944472.7 | 682765 | 392.3517 |
| PT143 | 944468.2 | 682762 | 392.3047 |
| PT144 | 944451.4 | 682782.2 | 392.9462 |
| PT145 | 944455 | 682785.1 | 392.8338 |
| PT146 | 944458.7 | 682787.8 | 393.0683 |
| PT147 | 944441.5 | 682807.9 | 393.1609 |
| PT148 | 944438.6 | 682804.9 | 393.3268 |
| PT149 | 944433.6 | 682801 | 393.3697 |
| TBM4 | 944424.8 | 682795.9 | 393.9601 |
| TBM5 | 944435.8 | 682821.7 | 393.8164 |
| PT150 | 944424.5 | 682828.5 | 393.5674 |
| PT151 | 944419.9 | 682824.7 | 393.9648 |
| PT152 | 944415.5 | 682820.9 | 393.9723 |
| PT153 | 944399.9 | 682839 | 394.2888 |
| PT154 | 944403.6 | 682842.6 | 394.3559 |
| PT155 | 944409.9 | 682847.6 | 394.176 |
| PT156 | 944390 | 682864.5 | 394.4079 |
| PT157 | 944385.6 | 682860.7 | 394.7839 |
| PT158 | 944380.6 | 682856.8 | 394.8008 |
| PT159 | 944364.9 | 682872.9 | 395.0643 |
| PT160 | 944368.9 | 682876.6 | 395.0744 |
| PT161 | 944373.9 | 682881.5 | 394.702 |
| PT162 | 944354.8 | 682897.6 | 395.1435 |
| PT163 | 944350.9 | 682892.9 | 395.2114 |
| PT164 | 944347.4 | 682889.1 | 395.5428 |
| PT165 | 944330.1 | 682906.4 | 395.8108 |
| PT166 | 944334.3 | 682912.1 | 395.6854 |
| PT167 | 944338.7 | 682916.6 | 395.3885 |
| PT168 | 944320.8 | 682935 | 395.728 |
| PT169 | 944315.9 | 682930.3 | 395.9204 |
| PT170 | 944311.3 | 682926.1 | 395.8679 |
| PT171 | 944296.1 | 682942.1 | 395.8444 |
| PT172 | 944299.5 | 682946.2 | 395.8974 |
| PT173 | 944304.1 | 682951.5 | 395.673 |
| PT174 | 944289 | 682967.9 | 395.7765 |
| PT175 | 944283.3 | 682964.5 | 395.8435 |
| PT176 | 944277 | 682958.6 | 395.9109 |
| PT177 | 944262.5 | 682977.1 | 396.1149 |
| PT178 | 944267.7 | 682981.6 | 396.042 |
| PT179 | 944273.4 | 682986.3 | 395.8726 |
| PT180 | 944255.4 | 683004.7 | 396.0681 |
| PT181 | 944250.1 | 683000.1 | 396.048 |
| PT182 | 944245.1 | 682995.7 | 396.0924 |
| PT183 | 944230.6 | 683014.2 | 396.131 |
| PT184 | 944234.6 | 683018.3 | 396.144 |
| PT185 | 944237.9 | 683022.1 | 396.0736 |
| PT186 | 944223.4 | 683039 | 396.3541 |
| PT187 | 944219 | 683035.5 | 396.4091 |
| PT188 | 944214.8 | 683032.7 | 396.3505 |
| PT189 | 944199.5 | 683049.1 | 396.6295 |
| PT190 | 944203.9 | 683052.7 | 396.5721 |
| PT191 | 944207.9 | 683056.3 | 396.4497 |
| PT192 | 944191.8 | 683073.5 | 396.8382 |
| PT193 | 944186.1 | 683070.4 | 396.9311 |
| PT194 | 944182.9 | 683068 | 396.9721 |
| PT195 | 944169.1 | 683086.8 | 397.4131 |
| PT196 | 944171.7 | 683089.9 | 397.2822 |
| PT197 | 944174.7 | 683092.7 | 397.2282 |
| PT198 | 944160.1 | 683110.5 | 397.8203 |
| PT199 | 944157.2 | 683108.5 | 397.7892 |
| PT200 | 944153.1 | 683104.5 | 397.7321 |
| PT201 | 944137.1 | 683123.9 | 398.4182 |
| PT202 | 944140.5 | 683127.2 | 398.4042 |
| PT203 | 944143.3 | 683129.7 | 398.3403 |
| PT204 | 944127.5 | 683147.2 | 398.7463 |
| PT205 | 944124.6 | 683144.5 | 398.9343 |
| PT206 | 944121.7 | 683142 | 399.0392 |
| PT207 | 944105.7 | 683158.6 | 400.0922 |
| PT208 | 944110.8 | 683159.7 | 399.4933 |
| PT209 | 944106.7 | 683155.6 | 399.5002 |
| PT210 | 944113.4 | 683164.4 | 399.2974 |
| PT211 | 944098.7 | 683178.1 | 399.5654 |
| PT212 | 944095 | 683174.7 | 399.7783 |
| PT213 | 944091.5 | 683171.6 | 399.8762 |
| PT214 | 944074.9 | 683183.7 | 400.4902 |
| PT215 | 944077.8 | 683189.3 | 400.3653 |
| PT216 | 944080.9 | 683193.5 | 400.1593 |
| PT217 | 944063.6 | 683204 | 400.6243 |
| PT218 | 944061.2 | 683200.2 | 400.8602 |
| PT219 | 944058.2 | 683196.6 | 400.8792 |
| PT220 | 944040.4 | 683210.2 | 401.2952 |
| PT221 | 944043 | 683214.5 | 401.1792 |
| PT222 | 944045.5 | 683218.2 | 401.4213 |
| PT223 | 944028.7 | 683229.1 | 401.7632 |
| PT224 | 944025.9 | 683225.4 | 401.5782 |
| PT225 | 944023.6 | 683223 | 401.6441 |
| PT226 | 944005.5 | 683234.9 | 402.0471 |
| PT227 | 944007.6 | 683238.4 | 401.8831 |
| PT228 | 944009.8 | 683241.8 | 402.0652 |
| PT229 | 943990.8 | 683255.3 | 402.3592 |
| PT230 | 943987.5 | 683250.9 | 402.3871 |
| PT231 | 943985.1 | 683247.3 | 402.353 |
| PT232 | 943965.5 | 683261.2 | 402.636 |
| PT233 | 943968.2 | 683265.1 | 402.6051 |
| PT234 | 943971.1 | 683267.8 | 402.6341 |
| PT235 | 943953.9 | 683283.3 | 402.9241 |
| PT236 | 943950.3 | 683280.3 | 402.867 |
| PT237 | 943946.2 | 683276.6 | 402.855 |
| PT238 | 943930 | 683293.6 | 403.034 |
| PT239 | 943937.8 | 683302.3 | 402.9241 |
| PT240 | 943934.5 | 683298.7 | 402.9391 |
| PT241 | 943922.9 | 683319.2 | 403.0002 |
| PT242 | 943918.8 | 683316.4 | 403.1341 |
| PT243 | 943914.3 | 683313.5 | 403.171 |
| PT244 | 943900.2 | 683331 | 403.3321 |
| PT245 | 943905.3 | 683335.2 | 403.3572 |
| PT246 | 943909.6 | 683338.2 | 403.0542 |
| PT247 | 943896.5 | 683357.2 | 403.2603 |
| PT248 | 943892 | 683355 | 403.4442 |
| PT249 | 943886.5 | 683352.5 | 403.4612 |
| PT250 | 943874.9 | 683368.9 | 403.4962 |
| PT251 | 943880.5 | 683372.6 | 403.4413 |
| PT252 | 943885.1 | 683375.7 | 403.2514 |
| PT253 | 943875.4 | 683394.6 | 403.4564 |
| PT254 | 943869 | 683391.3 | 403.5534 |
| PT255 | 943861.9 | 683387.7 | 403.7203 |
| PT256 | 943850.9 | 683406.5 | 403.4833 |
| PT257 | 943855.2 | 683410.6 | 403.6394 |
| PT258 | 943859.4 | 683413.5 | 403.4705 |
| PT259 | 943845 | 683434.6 | 403.5495 |
| PT260 | 943841.1 | 683433.2 | 403.7225 |
| PT261 | 943834.4 | 683432.3 | 403.7144 |
| PT262 | 943823.2 | 683448.8 | 403.5955 |
| PT263 | 943827.3 | 683452.4 | 403.7766 |
| PT264 | 943831.5 | 683456.2 | 403.7476 |
| PT265 | 943818 | 683475.5 | 403.5397 |
| PT266 | 943814.3 | 683472.6 | 403.7506 |
| PT267 | 943809.1 | 683469.6 | 403.6386 |
| SUHALLAH COMMUNITY | 943829.5 | 683463.2 | 404.2717 |
| PT268 | 943804.4 | 683495.6 | 403.6028 |
| PT269 | 943799.5 | 683493 | 403.7517 |
| PT270 | 943794.4 | 683489.8 | 403.5156 |
| TBM6 | 943829.4 | 683463.1 | 404.2497 |
| PT271 | 943791.1 | 683515.3 | 403.5258 |
| PT272 | 943786.5 | 683512 | 403.6738 |
| PT273 | 943781.5 | 683509 | 403.4507 |
| PT274 | 943777.3 | 683536 | 403.4439 |
| PT275 | 943772.1 | 683532.8 | 403.5268 |
| PT276 | 943768.3 | 683529.1 | 403.3418 |
| PT277 | 943755.1 | 683548.2 | 403.3119 |
| PT278 | 943759.4 | 683551.8 | 403.3889 |
| PT279 | 943763.8 | 683554.9 | 403.425 |
| PT280 | 943749.8 | 683573 | 403.3001 |
| PT281 | 943745.3 | 683570.5 | 403.414 |
| PT282 | 943740.2 | 683568.9 | 403.22 |
| PT283 | 943727.7 | 683587.6 | 402.9841 |
| PT284 | 943731.9 | 683591.5 | 403.2912 |
| PT285 | 943736.2 | 683594.5 | 403.1083 |
| PT286 | 943723.5 | 683614.3 | 402.8764 |
| PT287 | 943718.5 | 683611.9 | 403.0073 |
| PT288 | 943713.2 | 683608.8 | 402.8282 |
| PT289 | 943701.6 | 683627 | 402.5364 |
| PT290 | 943706.2 | 683630.6 | 402.6514 |
| PT291 | 943709.9 | 683633.3 | 402.3815 |
| PT292 | 943698.1 | 683652.8 | 402.3056 |
| PT293 | 943693.6 | 683650.3 | 402.3576 |
| PT294 | 943688.5 | 683647.2 | 401.9605 |
| PT295 | 943675 | 683666.9 | 402.0186 |
| PT296 | 943680.3 | 683670.3 | 402.1567 |
| PT297 | 943684.1 | 683672.8 | 402.0927 |
| PT298 | 943661.1 | 683686.9 | 401.5427 |
| PT299 | 943666 | 683689.7 | 402.0488 |
| PT300 | 943669.9 | 683693 | 401.6279 |
| PT301 | 943656.8 | 683711.7 | 401.458 |
| PT302 | 943652.5 | 683710.4 | 401.4469 |
| PT303 | 943645.9 | 683707.4 | 401.3538 |
| PT304 | 943635.7 | 683726.8 | 401.412 |
| PT305 | 943638.6 | 683728.8 | 401.502 |
| PT306 | 943642.6 | 683732.5 | 401.4641 |
| PT307 | 943628.5 | 683753.3 | 401.6352 |
| PT308 | 943626.2 | 683750.7 | 401.4542 |
| PT309 | 943622.4 | 683748.9 | 401.4881 |
| PT310 | 943610.3 | 683766.1 | 401.4962 |
| PT311 | 943612 | 683767.3 | 401.5643 |
| PT312 | 943617.2 | 683773.3 | 401.5304 |
| PT313 | 943604.4 | 683790 | 401.5655 |
| PT314 | 943601.1 | 683787.6 | 401.6204 |
| PT315 | 943598.2 | 683786.3 | 401.6554 |
| PT316 | 943586 | 683803.2 | 401.7885 |
| PT317 | 943588.5 | 683805.9 | 401.8065 |
| PT318 | 943591.7 | 683808.3 | 401.8456 |
| PT319 | 943579.1 | 683826.5 | 401.9187 |
| PT320 | 943575.4 | 683824.7 | 401.8486 |
| PT321 | 943571.4 | 683822.3 | 401.9046 |
| PT322 | 943561.4 | 683841 | 401.9397 |
| PT323 | 943563.7 | 683842.6 | 401.8987 |
| PT324 | 943567 | 683844.8 | 401.9258 |
| PT325 | 943554.4 | 683864.9 | 402.0649 |
| PT326 | 943551.4 | 683862.7 | 402.0289 |
| PT327 | 943548.9 | 683861.3 | 402.0618 |
| PT328 | 943535.8 | 683879.3 | 402.1429 |
| PT329 | 943538.9 | 683881.2 | 402.114 |
| PT330 | 943542.1 | 683883.1 | 402.204 |
| PT331 | 943530.9 | 683903.2 | 402.2822 |
| PT332 | 943526.9 | 683900.9 | 402.2491 |
| PT333 | 943523.2 | 683899 | 402.4031 |
| PT334 | 943511.5 | 683917.6 | 402.2792 |
| PT335 | 943514.7 | 683919.6 | 402.2862 |
| PT336 | 943518 | 683921.5 | 402.3533 |
| PT337 | 943505.8 | 683941.2 | 402.4984 |
| PT338 | 943501.2 | 683938.9 | 402.5274 |
| PT339 | 943497.3 | 683936.9 | 402.5353 |
| ALASE VILLAGE | 943494.8 | 683935.2 | 402.6553 |
| PT340 | 943487.3 | 683956.8 | 402.7385 |
| PT341 | 943489.8 | 683959.7 | 402.7575 |
| PT342 | 943493.1 | 683961.9 | 402.6666 |
| PT343 | 943480.1 | 683981.2 | 402.8557 |
| PT344 | 943477.5 | 683979.1 | 402.7316 |
| PT345 | 943473.3 | 683976.6 | 402.8186 |
| PT346 | 943460.8 | 683994.8 | 403.0757 |
| PT347 | 943467 | 683999.7 | 403.3348 |
| PT348 | 943464.3 | 683997.8 | 403.0817 |
| PT349 | 943448.8 | 684014.6 | 403.3758 |
| PT350 | 943451.9 | 684017.1 | 403.2569 |
| PT351 | 943454.9 | 684019.1 | 403.3959 |
| PT352 | 943443.2 | 684038.5 | 403.588 |
| PT353 | 943439.8 | 684037 | 403.432 |
| PT354 | 943435.2 | 684034.5 | 403.5069 |
| PT355 | 943423 | 684052.1 | 403.583 |
| PT356 | 943427.3 | 684055.4 | 403.6141 |
| PT357 | 943430.5 | 684057.4 | 403.6992 |
| PT358 | 943414.9 | 684079.3 | 403.8163 |
| PT359 | 943410.1 | 684075.5 | 403.6772 |
| PT360 | 943407.4 | 684073.9 | 403.5122 |
| PT361 | 943388.6 | 684091.5 | 403.7762 |
| PT362 | 943391.8 | 684094.3 | 403.7163 |
| PT363 | 943395.6 | 684098.3 | 403.7253 |
| PT364 | 943379.3 | 684114.2 | 403.6714 |
| PT365 | 943375.8 | 684111.6 | 403.6683 |
| PT366 | 943372.6 | 684109.1 | 403.6363 |
| PT367 | 943355.7 | 684124.8 | 403.5133 |
| PT368 | 943358.2 | 684127.4 | 403.6634 |
| PT369 | 943361.6 | 684130.6 | 403.6694 |
| PT370 | 943344.2 | 684147 | 403.5245 |
| PT371 | 943340.8 | 684143.2 | 403.5764 |
| PT372 | 943338.1 | 684139.7 | 403.5203 |
| PT373 | 943320.7 | 684154.6 | 403.5264 |
| PT374 | 943323.6 | 684158.2 | 403.5614 |
| PT375 | 943327.6 | 684162.3 | 403.4665 |
| PT376 | 943309.8 | 684176.9 | 403.4535 |
| PT377 | 943306 | 684172.2 | 403.3285 |
| PT378 | 943301.7 | 684169.1 | 403.5694 |
| ALENIBORO COMMUNITY | 943295.2 | 684165.9 | 403.0973 |
| PT379 | 943284.4 | 684187.7 | 403.1025 |
| PT380 | 943287.9 | 684190.6 | 403.1965 |
| PT381 | 943291.5 | 684193.6 | 403.4116 |
| PT382 | 943277.6 | 684210.4 | 403.1217 |
| PT383 | 943273.9 | 684207.8 | 402.9646 |
| PT384 | 943270.9 | 684205.8 | 402.9056 |
| PT385 | 943256.6 | 684222.1 | 402.6606 |
| PT386 | 943261.4 | 684225.9 | 402.7717 |
| PT387 | 943264.2 | 684227.8 | 402.9428 |
| PT388 | 943242.7 | 684239.7 | 402.5087 |
| PT389 | 943246.5 | 684242.8 | 402.5398 |
| PT390 | 943250.5 | 684245.9 | 402.5219 |
| PT391 | 943235.6 | 684264.6 | 402.2889 |
| PT392 | 943231.3 | 684261.2 | 402.3969 |
| PT393 | 943226.9 | 684258.4 | 402.4078 |
| PT394 | 943213.6 | 684275.3 | 402.0999 |
| PT395 | 943217.4 | 684278.3 | 402.158 |
| PT396 | 943221.3 | 684281.9 | 402.013 |
| PT397 | 943205.2 | 684300.2 | 401.8681 |
| PT398 | 943201.3 | 684296.6 | 401.715 |
| PT399 | 943198.1 | 684293.6 | 401.552 |
| PT400 | 943184.5 | 684309 | 401.327 |
| PT401 | 943187 | 684311.4 | 401.3761 |
| PT402 | 943190.6 | 684314.5 | 401.3481 |
| PT403 | 943176.7 | 684332.8 | 401.1502 |
| PT404 | 943173.7 | 684330.5 | 400.8842 |
| PT405 | 943170.3 | 684328.3 | 401.0601 |
| PT406 | 943155.2 | 684345.6 | 400.8052 |
| PT407 | 943157.8 | 684348.9 | 400.6273 |
| PT408 | 943161.3 | 684352 | 400.7473 |
| PT409 | 943145.2 | 684369.7 | 400.2214 |
| PT410 | 943142.3 | 684367.6 | 400.2464 |
| PT411 | 943138.9 | 684364.7 | 400.4823 |
| PT412 | 943123.2 | 684381.9 | 400.2554 |
| PT413 | 943126.2 | 684384.6 | 399.9804 |
| PT414 | 943129.1 | 684387.6 | 400.0895 |
| PT415 | 943114.8 | 684405.8 | 399.7496 |
| PT416 | 943111.1 | 684403.8 | 399.6165 |
| PT417 | 943107.5 | 684401 | 399.6505 |
| PT418 | 943091.8 | 684419 | 399.2945 |
| PT419 | 943095.4 | 684421.5 | 399.1506 |
| PT420 | 943099 | 684423.8 | 399.3997 |
| PT421 | 943085.3 | 684442.7 | 399.0988 |
| PT422 | 943081.9 | 684440.7 | 399.0557 |
| PT423 | 943078.5 | 684438.6 | 398.9587 |
| IDI IGBA COMMUNITY | 943075.7 | 684436.2 | 399.0676 |
| PT424 | 943066 | 684459 | 398.5508 |
| PT425 | 943068.5 | 684460.3 | 398.3778 |
| PT426 | 943071.9 | 684463.4 | 398.8219 |
| PT427 | 943056.6 | 684483 | 398.321 |
| PT428 | 943054.5 | 684481.1 | 398.1469 |
| PT429 | 943052.2 | 684479.7 | 398.1649 |
| PT430 | 943038.9 | 684498.5 | 397.99 |
| PT431 | 943041.6 | 684500.8 | 397.8441 |
| PT432 | 943044.2 | 684502.6 | 397.9951 |
| PT433 | 943030.7 | 684521.9 | 397.7282 |
| PT434 | 943027.2 | 684519.9 | 397.5842 |
| PT435 | 943024.1 | 684518 | 397.6521 |
| PT436 | 943011.3 | 684536.3 | 397.4482 |
| PT437 | 943014 | 684538.6 | 397.3783 |
| PT438 | 943017.8 | 684541.3 | 397.3963 |
| PT439 | 943003.6 | 684559.8 | 397.0914 |
| PT440 | 943000.7 | 684557.8 | 397.0834 |
| PT441 | 942996.4 | 684555 | 397.1223 |
| PT442 | 942983.3 | 684573.3 | 396.8934 |
| PT443 | 942987.1 | 684576.8 | 396.7625 |
| PT444 | 942990.2 | 684578.7 | 396.8495 |
| PT445 | 942977.3 | 684597.1 | 396.7316 |
| PT446 | 942974.4 | 684595.6 | 396.6306 |
| PT447 | 942970.1 | 684593.1 | 396.6015 |
| PT448 | 942957.6 | 684611.6 | 396.4636 |
| PT449 | 942961.3 | 684614.5 | 396.4547 |
| PT450 | 942964.4 | 684616.6 | 396.4018 |
| PT451 | 942948.4 | 684638.3 | 396.2329 |
| PT452 | 942945.3 | 684636.3 | 396.1168 |
| PT453 | 942942 | 684634.4 | 396.2358 |
| PT454 | 942928.3 | 684652.7 | 396.0799 |
| PT455 | 942931.6 | 684655 | 396.0239 |
| PT456 | 942934.4 | 684657.2 | 396.01 |
| PT457 | 942919.5 | 684675.9 | 395.8801 |
| PT458 | 942916.5 | 684673.4 | 395.897 |
| PT459 | 942913.4 | 684670.9 | 396.0229 |
| PT460 | 942898.1 | 684689.1 | 395.992 |
| PT461 | 942901.2 | 684692 | 395.8401 |
| PT462 | 942903.6 | 684694.2 | 395.6701 |
| PT463 | 942881.3 | 684705.4 | 395.6951 |
| PT464 | 942883.5 | 684707.9 | 395.6491 |
| PT465 | 942885.9 | 684710.8 | 395.6662 |
| PT466 | 942863.4 | 684720.8 | 395.5871 |
| PT467 | 942865.9 | 684724 | 395.6482 |
| PT468 | 942867.8 | 684726.7 | 395.4832 |
| PT469 | 942849.4 | 684738.7 | 395.5092 |
| PT470 | 942847.1 | 684735.9 | 395.6041 |
| PT471 | 942845.2 | 684732.9 | 395.5361 |
| PT472 | 942823.2 | 684742.6 | 395.53 |
| PT473 | 942828 | 684748.8 | 395.4821 |
| PT474 | 942831.8 | 684754.3 | 395.5552 |
| PT475 | 942814.7 | 684767 | 395.5912 |
| PT476 | 942808.3 | 684778.1 | 395.5143 |
| PT477 | 942802.6 | 684776.2 | 395.4882 |
| PT478 | 942804.9 | 684770.5 | 395.5472 |
| PT479 | 942800.7 | 684764.9 | 395.5571 |
| PT480 | 942798.3 | 684761.2 | 395.465 |
| PT481 | 942784.9 | 684770.4 | 395.723 |
| PT482 | 942787.1 | 684773.9 | 395.6591 |
| PT483 | 942789.1 | 684777.2 | 395.4491 |
| PT484 | 942771.9 | 684790.5 | 395.7091 |
| PT485 | 942769.6 | 684787.7 | 395.6871 |
| PT486 | 942767.3 | 684783.1 | 395.841 |
| PT487 | 942746.2 | 684794.2 | 395.841 |
| PT488 | 942747.2 | 684798.3 | 395.719 |
| PT489 | 942750.1 | 684802.4 | 395.9241 |
| PT490 | 942728.6 | 684812.4 | 395.961 |
| PT491 | 942726.5 | 684808.8 | 396.019 |
| PT492 | 942724 | 684805.7 | 396.0389 |
| PT493 | 942703.8 | 684815.8 | 396.1288 |
| PT494 | 942705.5 | 684820 | 396.0899 |
| PT495 | 942706.9 | 684823.5 | 396.205 |
| PT496 | 942685.2 | 684832 | 395.9899 |
| PT497 | 942683.3 | 684828.8 | 396.1598 |
| PT498 | 942681.9 | 684824.9 | 396.3408 |
| SABO COMMUNITY | 942700.7 | 684830.8 | 396.12 |
| PT499 | 942662.3 | 684838.5 | 396.1607 |
| PT500 | 942660.6 | 684835.9 | 396.1627 |
| PT501 | 942659.4 | 684832.4 | 396.1977 |
| PT502 | 942637.5 | 684838.7 | 396.8165 |
| PT503 | 942638.6 | 684844.3 | 395.8056 |
| PT504 | 942640.3 | 684848.8 | 395.7327 |
| PT505 | 942618.5 | 684855.9 | 396.0546 |
| PT506 | 942617 | 684852 | 396.0585 |
| PT507 | 942615.5 | 684848.3 | 395.6225 |
| PT508 | 942594.7 | 684855 | 395.6774 |
| PT509 | 942594.9 | 684860.6 | 395.6794 |
| PT510 | 942596.2 | 684864.5 | 395.5445 |
| PT511 | 942572.8 | 684870.5 | 395.4184 |
| PT512 | 942571.9 | 684865.3 | 395.5373 |
| PT513 | 942570.1 | 684861.2 | 395.5612 |
| PT514 | 942547.4 | 684867.6 | 395.5091 |
| PT515 | 942548.3 | 684871.6 | 395.4762 |
| PT516 | 942549.5 | 684876.4 | 395.4712 |
| PT517 | 942527.1 | 684883.1 | 395.1201 |
| PT518 | 942525.3 | 684878.2 | 395.104 |
| PT519 | 942523.8 | 684874.8 | 395.165 |
| PT520 | 942504.5 | 684881.6 | 395.2779 |
| PT521 | 942504.7 | 684886.6 | 394.918 |
| PT522 | 942505.6 | 684890.5 | 394.842 |
| PT523 | 942483.6 | 684899 | 394.4579 |
| PT524 | 942481 | 684894.2 | 394.3448 |
| PT525 | 942479.3 | 684890.8 | 394.3748 |
| PT526 | 942459.5 | 684899 | 394.0797 |
| PT527 | 942461.3 | 684903.1 | 393.9368 |
| PT528 | 942462.9 | 684907.1 | 393.9938 |
| PT529 | 942443.1 | 684917.1 | 393.9038 |
| PT530 | 942440.8 | 684913.1 | 393.9077 |
| PT531 | 942438.5 | 684909.2 | 394.0016 |
| PT532 | 942417.9 | 684919.9 | 393.6826 |
| PT533 | 942418.8 | 684923.7 | 393.6616 |
| PT534 | 942420.7 | 684927.4 | 393.8017 |
| PT535 | 942399.4 | 684937.8 | 393.8146 |
| PT536 | 942397.4 | 684934.3 | 393.7496 |
| PT537 | 942395.4 | 684931.4 | 393.8265 |
| PT538 | 942376.3 | 684940.9 | 393.1745 |
| PT539 | 942377.8 | 684945.1 | 393.1815 |
| PT540 | 942379.9 | 684949.1 | 393.3016 |
| PT541 | 942357.8 | 684960.9 | 392.8755 |
| PT542 | 942355.1 | 684957 | 393.0325 |
| PT543 | 942352.5 | 684953 | 392.9864 |
| PT544 | 942333.7 | 684964.1 | 393.1104 |
| PT545 | 942335.1 | 684966.8 | 393.0774 |
| PT546 | 942336.9 | 684970.1 | 393.0615 |
| PT547 | 942316.8 | 684980.7 | 393.4094 |
| PT548 | 942315 | 684977.4 | 393.2634 |
| PT549 | 942312.8 | 684974 | 393.4113 |
| PT550 | 942292.9 | 684984.6 | 393.6503 |
| PT551 | 942294.5 | 684988.8 | 393.5963 |
| PT552 | 942295.8 | 684992.1 | 393.8044 |
| PT553 | 942274.7 | 685002.9 | 394.0113 |
| PT554 | 942272.4 | 684998.6 | 393.9422 |
| PT555 | 942270.3 | 684995.2 | 394.0232 |
| PT556 | 942250.1 | 685002.7 | 394.3881 |
| PT557 | 942251.4 | 685006.3 | 394.2462 |
| PT558 | 942252.9 | 685010.2 | 394.5312 |
| PT559 | 942230.9 | 685016.9 | 394.6821 |
| PT560 | 942229.3 | 685012.6 | 394.62 |
| PT561 | 942228.5 | 685008.6 | 394.753 |
| PT562 | 942205.9 | 685014 | 395.0919 |
| PT563 | 942206.3 | 685017.4 | 394.9939 |
| PT564 | 942207.4 | 685022.4 | 395.108 |
| PT565 | 942184.5 | 685026.3 | 395.5768 |
| PT566 | 942183.8 | 685022.3 | 395.4278 |
| PT567 | 942182.4 | 685018.3 | 395.5327 |
| PT568 | 942160.9 | 685023.1 | 395.6606 |
| PT569 | 942161.6 | 685026.9 | 395.7736 |
| PT570 | 942162.3 | 685031 | 395.7387 |
| PT571 | 942138.4 | 685037.3 | 395.8365 |
| PT572 | 942137.3 | 685032 | 395.9045 |
| PT573 | 942136.1 | 685027.6 | 395.9264 |
| PT574 | 942114.1 | 685033.3 | 395.6873 |
| PT575 | 942114.6 | 685036.9 | 395.6823 |
| PT576 | 942115.6 | 685041.7 | 395.5394 |
| PT577 | 942091.7 | 685045.3 | 395.4452 |
| PT578 | 942090.9 | 685041.8 | 395.3602 |
| PT579 | 942089.5 | 685038 | 395.4101 |
| PT580 | 942067.4 | 685042.8 | 395.185 |
| PT581 | 942067.8 | 685046.3 | 395.104 |
| PT582 | 942068.7 | 685050.1 | 395.1771 |
| PT583 | 942045.6 | 685056 | 394.803 |
| PT584 | 942044.4 | 685052.1 | 394.8289 |
| PT585 | 942043.3 | 685047.6 | 395.2068 |
| PT586 | 942022.1 | 685052.6 | 394.8137 |
| PT587 | 942023 | 685057.5 | 394.6988 |
| PT588 | 942024 | 685060.7 | 394.4838 |
| PT589 | 942001.2 | 685064.5 | 394.5327 |
| PT590 | 942000.6 | 685061.4 | 394.4856 |
| PT591 | 941999.9 | 685057.7 | 394.6626 |
| PT592 | 941977.9 | 685061.9 | 394.4264 |
| PT593 | 941978.2 | 685065.2 | 394.2385 |
| PT594 | 941978.6 | 685068.7 | 394.2355 |
| PT595 | 941956.1 | 685074 | 394.0724 |
| PT596 | 941955.4 | 685070.6 | 393.9873 |
| PT597 | 941954.9 | 685067.3 | 394.1103 |
| PT598 | 941931.1 | 685071.5 | 393.9181 |
| PT599 | 941931.8 | 685075.7 | 393.5872 |
| PT600 | 941932.6 | 685079 | 393.5632 |
| PT601 | 941910.1 | 685084.5 | 393.4451 |
| PT602 | 941908.9 | 685081.2 | 393.3271 |
| PT603 | 941907.5 | 685077.6 | 393.454 |
| PT604 | 941884.6 | 685083.9 | 393.3579 |
| PT605 | 941886.8 | 685091.8 | 393.294 |
| PT606 | 941885.6 | 685088.1 | 393.2559 |
| PT607 | 941864.5 | 685098.7 | 393.3139 |
| PT608 | 941862.4 | 685093.9 | 393.4578 |
| PT609 | 941861.7 | 685089.2 | 393.2957 |
| PT610 | 941840.8 | 685096.2 | 393.3476 |
| PT611 | 941841.2 | 685100.2 | 393.3317 |
| PT612 | 941842.5 | 685104.1 | 393.2928 |
| PT613 | 941820.2 | 685110.7 | 393.4426 |
| PT614 | 941818.7 | 685106.7 | 393.3076 |
| PT615 | 941816.1 | 685101.4 | 393.4565 |
| PT616 | 941795.8 | 685109.6 | 393.6834 |
| PT617 | 941798.4 | 685117.8 | 393.4445 |
| PT618 | 941797.1 | 685114.5 | 393.4265 |
| PT619 | 941773.6 | 685116.6 | 393.4543 |
| PT620 | 941774.7 | 685120.4 | 393.3884 |
| PT621 | 941776.1 | 685125.5 | 393.5564 |
| PT622 | 941752.8 | 685132.1 | 393.1173 |
| PT623 | 941751 | 685126.7 | 393.2072 |
| PT624 | 941749.3 | 685122.2 | 393.2872 |
| PT625 | 941729 | 685128.8 | 392.9981 |
| PT626 | 941729.5 | 685133.2 | 392.8461 |
| PT627 | 941730.9 | 685137.2 | 392.7742 |
| PT628 | 941709.3 | 685144.1 | 392.3991 |
| PT629 | 941707.5 | 685139.3 | 392.423 |
| PT630 | 941705.3 | 685135.4 | 392.5969 |
| PT631 | 941685.8 | 685151.3 | 391.8119 |
| PT632 | 941684.1 | 685145.7 | 391.8919 |
| PT633 | 941682.6 | 685141.8 | 391.8788 |
| PT634 | 941659.5 | 685148.9 | 391.2957 |
| PT635 | 941660.7 | 685153.8 | 390.9597 |
| PT636 | 941662.3 | 685158.4 | 391.1748 |
| PT637 | 941642.6 | 685165.6 | 390.7387 |
| PT638 | 941640.6 | 685161.2 | 390.6247 |
| PT639 | 941638.7 | 685156.8 | 390.7506 |
| PT640 | 941617.1 | 685164.8 | 390.1645 |
| PT641 | 941618.4 | 685169.5 | 390.0186 |
| PT642 | 941620.9 | 685174.6 | 390.0676 |
| PT643 | 941599.5 | 685181.6 | 389.3895 |
| PT644 | 941597.4 | 685176.9 | 389.4675 |
| PT645 | 941595.2 | 685171.8 | 389.3384 |
| PT646 | 941573.5 | 685179 | 388.6103 |
| PT647 | 941574.9 | 685183.3 | 388.5753 |
| PT648 | 941576.8 | 685187.9 | 388.2504 |
| PT649 | 941555.5 | 685194.7 | 387.9843 |
| PT650 | 941553.8 | 685190.8 | 387.9912 |
| PT651 | 941552 | 685188.1 | 387.8992 |
| PT652 | 941534.2 | 685203.7 | 388.6022 |
| PT653 | 941510.7 | 685205.6 | 388.307 |