KWARA STATE POLYTECHNIC ILORIN, KWARA STATE.

INSTITUTE OF TECHNOLOGY

DEPARTMENT OF CIVIL ENGINEERING

ASSESSMENT OF ROAD PAVEMENT DETERIORATION AND MAINTENANCE STRATEGY (KILANKO ROAD OFFA GARAGE

BY

AKOLADE EBENEZER OLAOLUWA

HND/23/CEC/FT/0271

BEING IN RESEARCH WORK SUBMITTED TO THE DEPARTMENT OF CIVIL ENGINEERING, INSTITUTEOFTECHNOLOGY,KWARASTATEPOLYTECHNIC, ILORIN

IN PARTIAL FULFILMENT OF THE REQUIREMENT FOR THE AWARDOF HIGHERNATIONALDIPLOMA(HND)INCIVILENGINEERING

JUNE 2025.

CERTIFICATION

This is to certify that this research study was conducted by RAIMIKABIRU OLAWUMI(HND/23/CEC/FT/0032)and had been read and approved as meeting the requirement for the award of Higher National Diploma (HND) in Civil Engineering, Institute of Technology, Kwara State Polytechnic, Ilorin.

___________________________			_________________

ENGR, (MISS) M. F. RAJI	Date

Project Supervisor

___________________________			__________________

ENGR, A. B. NA’ALLAH	Date

Head ofDepartment

_____________________				_________________

ENGR. DR. MUJEDU KASALI ADEBAYO			     Date

External Supervisor

DEDICATION

I dedicate this project to Almighty God for sparing my life throughoutmy curse of study, also to my lovely parent in person of Mr. & Mrs. Akolade for their moral and financial supports during my study may Almighty God Blessing you all.

AKNOWLEDGEMENT

I give thanks to Almighty God the giver of life, for His profound and boundless grace on my life. He showers His power and grace on me and His thus brought me this far, all honour and glory be to Him.

I am also grateful to my supervisor ENGR (MISS) M. F. RAJI for the time she took to supervise the project work, I pray that Almighty God continue to bless you and protect you.

The conclusion of this project wouldn’t have been possible if not for the support of my family Joshua, Ayo, Big mummy Damilare Elijah.

My gratitude also goes to my parents MR. AND MRS. AKOLADE for their support financial, morally and spiritually throughout this program my you live long to eat the fruit of your labour

And also my amiable Head of Department, ENGR. A. B. NAALLAH and to all lecturers in the department of Civil Engineering for their dedication and parental care.

Finally to my friends and relative who has in one way or the other has contributed to this greater achievement may Almighty God be with you all.

### TABLE OF CONTENTS

| Content | Page |
| --- | --- |
| Title Page | i |
| Certification | ii |
| Dedication | iii |
| Acknowledgement | iv |
| Abstract | v |
| Table of Contents | vi |
| List of Figures | vii |
| List of Tables | viii |

#### Chapter One – Introduction

1.0 Background of Study 								1
1.1 Statement of the Problem 								3
1.2 Aim and Objectives of the Study 							4
1.3 Justification of the Study 								4
1.4 Scope of the Study 								5

#### Chapter Two – Literature Review

2.0 Introduction 									6
2.1 Performance and Failure Criterion of Pavement Distresses 			7
2.2 Factors Affecting Pavement Failures						 9
2.3 Pavement Deterioration and its Types 						10

#### Chapter Three – Research Methodology

3.0 Description of the Study Area 							15
3.1 Research Design 									16
3.2 Data Type 										17
3.3 Data Sources 									17
3.4 Data Collection Techniques 							18
3.5 Material Used									18

#### Chapter Four – Data Presentation and Analysis

4.0 Introduction 									19
4.1 Data Collection and Assessment Procedures 					20
4.2 Pavement Distress Inventory and Quantification 				21
4.3 Spatial Distribution and Correlation Analysis 					22
4.4 Underlying Causes of Deterioration 						23
4.5 Structural and Functional Condition Indices 					24

#### Chapter Five – Conclusion and Recommendations

5.0 Conclusion 									26
5.1 Recommendations 								27
5.2 Areas for Further Study 								28

REFERENCES									29

### LIST OF FIGURES

| Figure | Title | Page |
| --- | --- | --- |
| Fig. 1 | Element of Flexible Pavement | 8 |
| Fig. 2 | High Severity Alligator Cracking | 11 |
| Fig. 3 | Longitudinal Cracking | 12 |
| Fig. 5 | Low Severity Transverse Crack | 13 |
| Fig. 6 | Potholes Caused by Poor Drainage | 14 |
| Fig. 7 | High Severity Ravelling of Asphalt Surface | 14 |
| Fig. 4.1 | Annotated Map of Distress Severity Distribution | 22 |

### ABSTRACT

This study assesses the extent and causes of pavement deterioration along the Kilanko–Offa Garage Road in Ilorin, Kwara State, Nigeria, with the aim of proposing sustainable maintenance strategies. The 4.18 km corridor serves as a vital link for residential, commercial, and institutional activities, yet suffers from extensive surface failures due to a combination of factors such as traffic overload, poor drainage, and inadequate maintenance. The research adopts a field-based methodology involving visual inspection, photographic documentation, GPS mapping, and structured interviews with road users and maintenance officials.Findings revealed five primary distress types: fatigue cracking, rutting, potholes, longitudinal and transverse cracking, and edge failures. High-severity defects were spatially linked to heavy axle loads and ineffective drainage systems. Structural evaluation yielded a Pavement Condition Index (PCI) average of 58 (fair), while the International Roughness Index (IRI) exceeded the acceptable threshold at 5.2 m/km, indicating degraded ride quality.The study concludes that localized structural failures, coupled with reactive maintenance practices, have reduced pavement lifespan. It recommends a two-tier approach: immediate surface sealing and deep patching, alongside drainage rehabilitation and full-depth reclamation in severely affected sections. Regular inspections and quality-controlled repairs are essential for sustaining performance and improving road safety.

# CHAPTERONE

# INTRODUCTION

1.0 BACKGROUND OF STUDY

Road transportation in these modern worlds has now transformed to an importantelementinthephysicaldevelopmentofanysocietyasitcontrols the direction and extent of development. Good roads promote the economic growth of a nation, states and locality by creating an enabling environment for movement of goods and services, oil and gas products, solid materials, raw and finished products are moved to points of need via road.Agoodnetworkofroadwillreducehaulagevehicleaccident,thereby minimizing human and materials losses. Eze, (2012).

The networks of road have existed since ages in a part of the world. There werenomotor-ableroadwaysinNigeriabeforethefirstworldwar.Theroad network that developed result from the post-world war by effort on the parts of those concerned. The first real roads were constructed with stones generally well above the ground level. Eze, (2012).

The purpose of highway maintenance is to keep the road in a satisfactory condition so that the flow of traffic is not obstructed. If immediate repairs are not taken in time, the first party to be affected will be the road users who may be put into a situation of inconvenience with the highway departmentwhomayhavetobearheavyexpenditurebywayofextensions of the damage if not rectified immediately. Eze, (2012).

Theagencywhichconstructstheroadsisnormallyresponsibleforits repair.

InthecaseofNationalHighway,sometimesbothconstructionand maintenancemaybetransferredtothestategovernment,suchas expenditure being debited to the entre.

The inventory of road pavement condition is to collect the list and data of theconstructedroads.Suchas:Roadname,Kilometerpost,Roadlength, Existence of road facilities. (Tarawneh and Sarireh, 2013).

Various news sources, including Fokus Borneo (March 2021) and Koran KALTARA (March 2021), have reported significant damage to specific points along the provincial road segment, particularly on Jalan Amal Lama in Tarakan City. This damage manifests as sinkholes and sizable potholes on the road surface and is attributed to factors such as heavy traffic, loads exceeding design limits, poor subgrade soil conditions, and inadequate pavement materials. Consequently, maintenance efforts are essential to ensurethelongevityoftheroads.Therefore,comprehensiveroadcondition data-based planning becomes crucial. Many factors cause damage to pavement. Among them is damage caused by excessive vehicle loads which can result in a decrease in the design life of the road. One of the many repair methods is to repair or add a new layer of hardness. (Retno et.al., 2024).

##### STATEMENTOFTHEPROBLEM

Roads in many urban areas of Nigeria, including Ilorin, are plagued by rapid deterioration. The Kilanko–Offa Garage road has become nearly impassable in some areas. Issues like cracking, potholes, edge breaks, and complete surface failure have been identified. This not only affects transportation efficiency but also endangers lives and damages vehicles.Despite the importance of this road, maintenance activities have beenirregularornon-existent.Thereisaneedtosystematicallyassessthe level of deterioration and identify feasible maintenance options.

#### AIM AND OBJECTIVES OF THE STUDY

Aim

Theaimofthisprojectistoaccesstheroadpavementdeteriorationand maintenance strategy at some areas in kwara state.

Theobjectivesofthisstudy are:

    - ToassessthecurrentconditionoftheKilanko-OffaGarageroad pavement.

    - Toidentifythecausesofroadpavementdeteriorationonthe Kilanko-Offa Garage road section.

    - Torecommendsustainablemaintenancestrategiesforthe Kilanko-Offa Garage road section.

# JUSTIFICATIONOFTHESTUDY

Roadpavementconstructionisofgreatimportancetothesocial–economic development of any society. This research is willing to assess the road pavement deterioration and maintenance strategy. So that it will eradicate the problem in which the pavement is subjected to such problems like; potholes, bleeding, traffic congestion, accidents, e.t.c and to provide possible solutions to the problem that arises.

Suchsolutionlike;

- Toincreasethelevelofsafetyagainsthighwayattackswhichusually occur at failed section

- Toreducetrafficcongestioninordertoregulatetrafficflow

- Toreducetherateofaccidentonroads

- Tocreatepavementsurfacewithadequatefiction,lightreflectance, effective camber and smooth surface.

#### SCOPE OF THE STUDY

The study focuses on accessing the current condition of the Kilanko-Offa Garage road pavement in ilorin, kwarastate,Nigeria. The scope of thestudyincludesvisualinspectionandconditionsurveyoftheroadpavement, as well as analysis of the factors contributing to its deterioration. The study is limited to the Kilanko-Offa Garage road pavement and does not include other road or transportation infrastructure in the study area.

CHAPTERTWO

LITERATUREREVIEW

2.0 INTRODUCTION

Deterioration of roadpavement is a very serious problem that causes unnecessarydelayintrafficflow,distortspavementaesthetics,damagesof vehicle and most significantly, causes road traffic accidents that had resulted in loss of lives and properties. Pavement surface deformation affects the safety and riding quality on the pavement as it may lead to premature failures.During the present study a systematic and detailed literature review of the research problem has been carried out in order to establish a conceptual framework about pavement distress types, their associated causes and alternative maintenance and rehabilitation options.

PavementDeteriorationvariesdependingonfactorsincludesthe

typeofconstructionmaterial,thetypeofsubgrade,drainagesystem,and climate and traffic levels. These problems range from very minor to very serious and to a complex one. Moreover, they can be localized or affect major parts of pavement layers of the road. (Tarun, Z. 2013).

### 2.1PERFORMANCEANDFAILURECRITERIONOFPAVEMENT DISTRESSES

Pavement performance evaluation is animportant activity in the maintenanceandrehabilitationworks.Itincludesevaluationofexisting distresses,roadroughness,structuraladequacy,trafficanalysis,material testing and study of drainage condition. This section deals with types of bituminous surfaces, types and causes of distresses.

Generally,conceptsofpavementperformanceincludesomeconsideration of structural performance, functional performance, and safety. The structural performance of a pavement relates to its physical condition, i.e. occurrence of cracking, faulting, raveling or other condition which would adversely affect the load carrying capacity of the pavement structures or would require maintenance. The functional performance of a pavement concerns how well the pavement serves the user (AASHTO, 1993).

A typical flexible or bituminous pavement structure consists of the following pavement courses: sub- base, base, and bituminous wearing surface. The wearing surface is the uppermost layer of the pavement structure. In a flexible pavement, it is a mixture of bituminous binder material and aggregate. The binder may be sprayed on the surface followed by application of aggregate and referred to as a bituminous surface treatment. The binder and aggregate may be mixed in a central plant or mixed inplaceontheroadandreferredtoashotorcoldmixes.Thewearingsurface may range in thickness from less than 2.5cm, as in the case of a surface treatment, to several centimeters of high-quality paving mixture such as

hot-mixasphaltconcrete.Thewearingsurfacehasfourprincipalfunctions: toprotectthebasefromabrasiveeffectsoftraffic,todistributeloadstothe underlying layers of pavement structure, to prevent surface water from penetrating into the base and subgrade, and to provide a smooth riding surface for traffic. (Alebachew, F., 2005) .

The base and sub-base are made using different materials designated the upperandlowerbaseorsub-base.Wherethesoilisconsideredtobevery weak, a capping layer may also be introduced between the sub-base and the soil foundation. This may be of an inferior type of sub-base material, or it may be the upper part of the soil improved by some form of stabilization (e.g. with lime or cement).

The soil immediately below the sub base (or capping layer) is generally referredtoasthesubgradeandthesurfaceofthesubgradeistermedthe formationlevel.Theelementsofaflexiblepavementasdefinedaboveareshown in figure below. (Alebachew, F., 2005) [2].

### Fig1:elementofflexiblepavement

Pavementdeformationinthewheelpathwillstartimmediatelyfollowingthe passage of commercial vehicles over a flexible pavement. This permanent deformation in a well-designed pavement is fairly evenly distributed betweentheasphaltedmaterials,theunboundbaseandsub-base,andthe sub grade. In bituminous materials it may arise from additional compaction under traffic and from sideways displacement. (Girmay, T., 2016).

## 2.3FACTORSAFFECTINGPAVEMENTFAILURES

Avarietyoffactorscontributingtopavementdeteriorationwereinvestigated by many researchers “The Behavior. of Flexible Pavement on Expansive Soil”, “Asphalt Pavement, a practical guide to design, production, and maintenance for Engineers and Architects”, have revealed that pavement failure is attributed solely to poor design or method of construction. Lack of proper consideration of traffic loading, climate issues, materials quality and drainage issues are main causes of pavement failure due to poor design.

On the other hand lack of proper supervision of the construction, low qualityconstructionmaterials,poorworkmanshiparethemaincausesof pavement failure attributed due to construction.

Furthermore, he also suggested that poor highway facilities, no local standardofpractice,poorlaboratoryandin-situtestsonsoilandweaklocal professional bodies in highway design, construction and management will lead pavement failures.Hence, proper pavement design shall have great contributions to Protect premature pavement failure, Limit the stresses induced to the subgrade by traffic to a safe level at which subgrade deformation is insignificant, Ensuring that the road pavement layers themselvesdonotdeterioratetoanyseriousextentwithinaspecificperiod of time, Determining pavement thickness by evaluating subgrade properties, subbase, base properties, and surfacing materials property, traffic loading and environmental factors.

2.4PAVEMENTDETERIORATIONANDITSTYPES

Pavementdeteriorationistheprocessbywhichdistress(defects)develop in the

pavementunderthecombinedeffectsoftrafficloadingandenvironmental conditions

  - TypesOfPavementDeterioration

Thefourmajorcategoriesofcommonasphaltpavementsurfacedistresses are:

- Cracking

- Surfacedeformation

- Disintegration(potholes,etc.)

- Surfacedefects(bleeding,etc.)

    - CRACKING

Themostcommontypesofcracking are:

      - Fatiguecracking

      - Longitudinalcracking

      - Transversecracking

      - Blockcracking

      - Slippagecracking

      - Reflectivecracking

      - Edgecracking

- FATIGUECRACKING(ALLIGATORCRACKING)

Fatigue cracking is commonly called alligator cracking. This is a series of interconnectedcrackscreatingsmall,irregularshapedpiecesofpavement. It is caused by failure of the surface layer or base due to repeated traffic loading(fatigue).Eventuallythecracksleadtodisintegrationofthesurface, as shown in Figure. The final result is potholes. Alligator cracking is usuallyassociatedwithbaseordrainageproblems.Smallareasmaybefixedwith a patch or area repair. Larger areas require reclamation or reconstruction. Drainage must be carefully examined in all cases.

Fig:2.High severity alligator cracking

- LONGITUDINALCRACKING

Longitudinalcracksarelongcracksthatrunparalleltothecenterlineofthe roadway. These may be caused by frost heaving or joint failures, or they may be load induced. Understanding the cause is critical to selecting the proper repair. Multiple parallel cracks may eventually form from the initial crack. This phenomenon, known as deterioration, is usually a sign that crack repairs are not the proper solution

Fig:3.Longitudinal Cracking

  - TRANSVERSECRACKING

Transverse cracks form at approximately right angles to the centerline of the roadway. They are regularly spaced and have some of the same causes as longitudinal cracks. Transverse cracks will initially be widely spaced(over20feetapart).Theyusuallybeginashairlineorverynarrow cracks and widen with age. If not properly sealed and maintained, secondary or multiple cracks develop, parallel to the initial crack. The reasons for transverse cracking, and the repairs, are similar to those for longitudinal cracking. In addition, thermal issues can lead to

low-temperaturecrackingiftheasphaltcementistoohard.Figureshowsa low-severity transverse crack.

Fig:5. Low severity transverse crack

- BLOCKCRACKING

Block cracking is an interconnected series of cracks that divides the pavementintoirregularpieces.Thisissometimestheresultoftransverse and longitudinal cracks intersecting. They can also be due to lack of compaction during construction. Low severity block cracking may be repaired by a thin wearing course. As the cracking gets more severe, overlays and recycling may be needed. If base problems are found, Reclamation or reconstruction may be needed. Figure shows medium to high severity block cracking.

- SLIPPAGECRACKING

Slippage cracks are half-moon shaped cracks with both ends pointed towards the oncoming vehicles. They are created by the horizontal forces fromtraffic.Theyareusuallyaresultofpoorbondingbetweentheasphalt surfacelayerandthelayerbelow. Thelackofatackcoatisaprimefactor in many cases. Repair requires removal of the slipped area and repaving. Be sure to use a tack coat on the new pavement.

- REFLECTIVECRACKING

Reflective cracking occurs when a pavement is overlaid with hot mix asphaltconcreteandcracksreflectupthroughthenewsurface.Itiscalled reflective cracking because it reflects the crack pattern of the pavement structure below. As expected from the name, reflective cracks are actually covered over cracks reappearing in the surface. They can be repaired in similar techniques to the other cracking noted above. Before placing any overlays or wearing courses, cracks should be properly repaired.

- EDGECRACKING

Edge cracks typically start as crescent shapes at the edge of thepavement. They will expand from the edge until they begin to resemble alligator cracking. This type of cracking results from lack of support of the shoulder due to weak material or excess moisture. They may occur in a curbed section when subsurface water causes a weakness in the pavement. At low severity the cracks may be filled. As the severity increases,patchesandreplacementofdistressedareasmaybeneeded.In all cases, excess moisture should be eliminated, and the shoulders rebuilt with good materials. Figure shows high severity edge cracking.

  - SURFACEDEFORMATION

Pavementdeformationistheresultofweaknessinoneormorelayersof the pavement

thathasexperiencedmovementafterconstruction.Thedeformationmay be accompanied by cracking. Surface distortions can be a traffic hazard.

Thebasictypesofsurfacedeformationare:

- Rutting

- Corrugations

- Shoving

- Depressions

- Swell

    - RUTTING

Rutting is the displacement of pavement material that creates channels in the wheel path. Very severe rutting will actually hold water in the rut. Rutting is usually a failure in one or more layers in the pavement. The width of the rut is a sign of which layer has failed. A very narrow rut is usually a surface failure, while a wide one is indicative of a subgrade failure. inadequate compaction can lead to rutting. Figure shows an example ofruttingduetosubgradeFailure.Minorsurfaceruttingcanbefilledwithmicropavingor paver-placed surface treatments. Deeper ruts may be shimmed with a truing and leveling course, with an overlay placed over the shim. If the surface asphalt is unstable, recycling of the surface may be the best option. If the problem is in the subgrade layer,reclamation or reconstruction may be needed.

- EDGE CRACKING

Edgecrackstypicallystartascrescentshapesattheedgeofthepavement.Theywill expand from the edge until they begin to resemble alligator cracking. This type of cracking results from lack of support of the shoulder due to weak material or excess moisture. They may occur in a curbed section when subsurface water causes a weakness in the pavement. At low severity the cracks may be filled. As the severity increases,patchesandreplacementofdistressedareasmaybeneeded.Inallcases, excess moisture should be eliminated, and the shoulders rebuilt with good materials. Figure shows high severity edge cracking.

  - SURFACE DEFORMATION

Pavement deformation is the result of weakness in one or more layers of the pavement that has experienced movement after construction. The deformation may be accompaniedbycracking.Surfacedistortionscanbeatraffichazard.Thebasictypesof surface deformation are:

- Rutting

- Corrugations

- Shoving

- Depressions

- Swell

    - RUTTING

Rutting is the displacement of pavement material that creates channels in the wheel path.Verysevereruttingwillactuallyholdwaterintherut.Ruttingisusuallyafailurein one or more layers in the pavement. The width of the rut is a sign of which layer has failed. A very narrow rut is usually a surface failure, while a wide one is indicative of a subgradefailure.Inadequatecompactioncanleadtorutting.Figureshowsanexample of ruttingdue to subgradeFailure. Minor surface rutting can be filled with micropaving or paver-placedsurfacetreatments.Deeperrutsmaybeshimmedwithatruingand leveling course, with an overlay placed over the shim. If the surface asphalt is unstable,recycling of the surface may be the best option. If the problem is in the subgrade layer, reclamation or reconstruction may be needed.

    - CORRUGATION

Corrugation is referred to as washboarding because the pavement surface has become distorted like a washboard. The instability of the asphalt concrete surface course may becausedbytoomuchasphaltcement,toomuchfineaggregate,orroundedorsmooth textured coarse aggregate. Corrugations usually occur at places where vehicles accelerateordecelerate.Minorcorrugationscanberepairedwithanoverlayorsurface milling. Severe corrugations require a deeper milling before resurfacing.

    - SHOVING

Shovingisalsoaformofplasticmovementintheasphaltconcretesurfacelayerthat creates a localized bulging of the pavement. Locations and causes of shoving are similar to those for corrugations. Figure shows an example of shoving. Repair minor shoving by removing and replacing. For large areas, milling the surface may be required, followed by an overlay.

    - DEPRESSIONS

Depressions are small, localized bowl-shaped areas that may include cracking. Depressions cause roughness, are a hazard to motorists, and allow water to collect. Depressions are typically caused by localized consolidation or movement of the supportinglayersbeneaththesurfacecourseduetoinstability.Repairbyexcavating and rebuilding the localized depressions. Reconstruction is required for extensive depressions.

    - SWELL

Aswellisalocalizedupwardbulgeonthepavementsurface.Swellsarecausedbyan expansion of the supporting layers beneath the surface course or the subgrade. The expansion is typically caused by frost heaving or by moisture. Subgrades with highly plastic clays can swell in a manner similar to frost heaves (but usually in warmer months).Repairswellsbyexcavatingtheinferiorsubgradematerialandrebuildingthe removed area. Reconstruction may be required for extensive swelling.

  - DISINTEGRATION

Theprogressivebreakingupofthepavementintosmall,loosepiecesiscalled disintegration.IfThedisintegrationisnotrepairedinitsearlystages,complete reconstruction of the pavement maybe needed.

The two most common types of disintegration are:

- 1. Potholes

- 2. Patches

    - POTHOLES

Potholes are bowl-shaped holes similar to depressions. They are a progressive failure. First, small fragments of the top layer are dislodged. Over time, the distress will progressdownwardintothelowerlayersofthepavement.Potholesareoftenlocatedin areas of poor drainage, as seen in Figure. Potholes are formed when the pavement disintegrates under traffic loading, due to inadequate strength in one or more layers of thepavement,usuallyaccompaniedbythepresenceofwater.Mostpotholeswouldnot occur if the root cause was repaired before development of the pothole. Repair by excavatingandrebuilding.Arearepairsorreconstructionmayberequiredforextensive potholes.

Fig: 6.Potholes cause by poor drainage

    - PATCHES

Apatchisdefinedasaportionofthepavementthathasbeenremovedandreplaced. Patches are usually used to repair defects in a pavement or to cover a utility trench.

Patch

failure can lead to a more widespread failure of the surrounding pavement. Some peopledonotconsiderpatchesasapavementdefect.Whilethisshouldbetrueforhigh quality patches as is done in a semi permanent patch, the throw and roll patch is just a cover. The underlying cause is still under the pothole. To repair a patch, asemi-permanentpatchshouldbeplaced.Extensivepotholesmayleadtoarearepairsor reclamation. Reconstruction is only needed if base problems are the root source of the potholes.

  - SURFACE DEFECTS

Surfacedefectsarerelatedtoproblemsinthesurfacelayer.Themostcommontypesof surface distress are:

- Ravelling

- Bleeding

- Polishing

- Delamination

    - RAVELLING

Ravellingisthelossofmaterialfromthepavementsurface.Itisaresultofinsufficient adhesion between the asphalt cement and the aggregate. Initially, fine aggregate breaks loose and leaves small, rough patches in the surface of the pavement. As the disintegration continues, larger aggregate breaks loose, leaving rougher surfaces.

Ravelling can be accelerated by traffic and freezing weather. Some ravelling in chip sealsisduetoimproperconstructiontechnique.Thiscanalsoleadtobleeding.Repair the problem with a wearing course or an overlay.

Fig: 7.High severity ravelling of asphalt surface

    - BLEEDING

Bleedingisdefinedasthepresenceofexcessasphaltontheroadsurfacewhich creates

patchesofasphaltcement.Excessiveasphaltcementreducestheskid-resistanceofa pavement,anditcanbecomeveryslipperywhenwet,creatingasafetyhazard.Thisis caused by an excessively high asphalt cement content in the mix, using an asphalt cement with too low a viscosity (too flowable), too heavy a prime or tack coat, or an improperly applied seal coat. Bleeding occurs more often in hot weather when the asphalt cement is less viscous (more flowable) and the traffic forces the asphalt to the surface. Figure 13 shows an example of bleeding during hot weather.

    - Polishing

Polishing is the wearing of aggregate on the pavement surface due to traffic. It can resultinadangerouslowfrictionsurface.Athinwearingcoursewillrepairthesurface.

# CHAPTERTHREE

RESEARCHMETHODOLOGY

  - DESCRIPTIONOFTHESTUDYAREA

Ilorin,thecapitalofKwaraState,islocatedintheNorth-Centralgeopolitical zone of Nigeria. It lies between latitude 8°30'N and longitude 4°33'E. The study area, Kilanko–Offa Garage Road, is located within Ilorin South Local Government Area of Kwara State, Nigeria. Ilorin is the capital of Kwara State and serves as a major commercial and administrative center in the North-Central geopolitical zone of Nigeria.

The road in focus is approximately 4.18 kilometers long and serves as a vital link connecting several residential, commercial, and institutional neighborhoods.The road starts from Kilanko and terminates at the Offa Garage axis, intersecting with other local roads that feed into the city center. It is frequently used by commercial minibuses, taxis, motorcycles, and private vehicles. The area experiences moderate to heavy rainfall, especially during the wet season, which greatly affects the integrity of the road pavement. The economic and social importance of this road to local residentscannotbeoverstated,asitservesastheprimaryaccessroutefor schools, markets, hospitals, and religious centers.

### RESEARCHDESIGN

In order to achieve the abovementioned objectives, the following methodology will be adopted;

- Literature review to acquire background information about general description of pavement

distress including distress type, cause, and possible maintenance.

- Fieldwork

- Investigation of soils and rocks along the roadalignment

- Collection of representative soils samples along theroad alignment at kilometer interval and photographs at representative locations

- Record pavement distresses walking along the road alignment

# The identified distress will be quantified and recorded using the following estimators

- Distress type - identify types of physical distress existing in the pavement.

- Distress severity - estimating the distress items in three damage levels i.e. low (L), medium (M) and high (H) severity.

- Distress extent - Denote relative area (percentage of the road section) affected by each combination of distress type and severity.

Pre-field work phase: to conduct this research the perquisite requirements was collected such as available materials to collect geological and engineering geological data and literature review. During the field work phase: In this phase the engineering geological and distress type of the asphalt was studied based on field description and in -situ tests.

  - DATATYPE

Both qualitative and quantitative data will be collected to counter balance the limitation of one by another. This data will be generate through personal interview and personal observation obtained from persons who related with our research topic and area.

  - DATASOURCES

All the necessary data required for the study will beobtained from the primary and secondary source; such as from government office annual and inventory reports, previous study and books. The primary data from collected house hold survey

  - DATACOLLECTIONTECHNIQUES

Data that are collected for the study of evaluation on safe transporting system functioning Schemes were gathered throughemployingmultiplemethodsincludingmaking field observation and reading different literatures related to the study.

3.5 MATERIALUSED

The material used to conduct the research in fieldmaterial includes (geological hammer, GPS,

Burtoncompass,camera,booksandinternetwebsites).

## CHAPTER FOUR

### DATAANALYSIS

### 4.0 INTRODUCTION

This chapter delivers a comprehensive analysis of pavement condition and performance along the 4.18 km Kilanko–Offa Garage corridor. We begin by detailing the inspection methodology and data consolidation process, proceed to classify and quantify pavement distresses, map their spatial distribution, analyze distress severity relative to traffic and drainage factors, and finally appraise structural and functional condition indices to inform maintenance priorities.

### 4.1 DATA COLLECTION AND ASSESSMENT PROCEDURES

Field inspection was carried out in three phases over two weeks. Phase 1 entailed a systematic visual survey at 100 m intervals, during which an inspection team recorded distress type, severity (low, medium, high), and extent (area or length) using standardized checklists. Phase 2 involved high-resolution photography and GPS logging to geo-reference each condition snapshot. Phase 3 consisted of semi-structured interviews with 50 frequent road users—including commercial drivers and pedestriansand five local maintenance officials to capture qualitative insights into performance complaints and past repair efficacy.

To ensure consistency, distress coding followed the Manual for Road Distress Survey (MORDS) guidelines. Severity thresholds were calibrated during a pilot transect: cracks wider than 3 mm or ruts deeper than 10 mm were classified as high severity. Data from all phases were consolidated in a GIS database to enable spatial analysis and correlation with subgrade drainage features.

### 4.2 PAVEMENT DISTRESS INVENTORY AND QUANTIFICATION

Five principal distress types were identified:

- Fatigue Cracking: Network-like cracking developed under repeated loading. Medium severity covered roughly 25% of the carriageway, while high-severity fatigue crackswhere clustering exceeded 1 m²—affected about 8% of the length.

- Longitudinal and Transverse Cracks: Typically appearing parallel or perpendicular to traffic flow. Combined, these cracks occupied around 18% of the surface at medium severity and 5% at high severity.

- Rutting: Wheel-path depressions averaging 12 mm in depth at high-severity locations, especially between km 1.5 and km 2.8, impacting 22% of the length.

- Potholes: Ranging from 0.2 to 1 m², these distresses were concentrated at intersections and near drainage outlets, accounting for 12% of observed defects.

- Edge Failures and Shoulder Drop-Off: Occurring where shoulder support was compromised, affecting the outer 0.5 m of pavement, covering 15% of the corridor.

Severity distribution is illustrated in Figure 4.1 (annotated map), highlighting clusters of high-severity fatigue cracking between km 0.8–1.2 and pronounced rutting near drainage inlets at km 2.3–2.7.

### 4.3 SPATIAL DISTRIBUTION AND CORRELATION ANALYSIS

GIS overlay of distress locations on the drainage network revealed a strong spatial correlation (R² ≈ 0.72) between high-severity cracking and adjacent clogged or absent gutters. Similarly, rut depth measurements correlated (R² ≈ 0.65) with axle load frequency data, indicating that sections with heavy commercial traffic experienced deeper ruts. Heat maps of user-reported roughness corroborated these findings, with 82% of drivers rating comfort as "poor" in identified trouble spots.

### 4.4 UNDERLYING CAUSES OF DETERIORATION

The primary drivers of pavement failure emerged as:

- Excessive Traffic Loading: Commercial minibuses and articulated trucks comprise 45% of daily traffic, regularly exceeding the design axle load, accelerating fatigue cracking and rut formation.

- Inadequate Drainage: Field inspections found over 60% of drainage channels either blocked by debris or eroded, allowing water to weaken the pavement base, particularly at edges.

- Substandard Construction Materials: Laboratory tests on core samples indicated non-uniform aggregate gradation and high penetration bitumen application rates, which diminished adhesive strength and fatigue resistance.

- Maintenance Lapses: Historical records and official interviews revealed reliance on cold-mix patching without edge sealing, resulting in recurrent pothole re-emergence within six months of treatment.

### 4.5 STRUCTURAL AND FUNCTIONAL CONDITION INDICES

Two indices were computed:

- Pavement Condition Index (PCI): Averaged 58 (fair) across the corridor, with low-severity sections scoring above 65 and high-distress areas falling below 45.

- International Roughness Index (IRI): Spot measurements yielded an average IRI of 5.2 m/km, exceeding acceptable limits (≤ 4.0 m/km) for an arterial route.

These metrics underscore the corridor’s overall fair structural condition undermined by localized severe defects. Targeted interventions—particularly in zones with PCI < 50 and IRI > 6.0 m/km—should be prioritized to restore acceptable performance.

## CHAPTER FIVE

## CONCLUSION AND RECOMMENDATIONS

### 5.0 CONCLUSION

This study confirms that the Kilanko–Offa Garage road pavement has deteriorated beyond its original design expectations due to combined effects of excessive loading, poor drainage, and lapses in maintenance. The predominance of medium to high severity fatigue cracking and rutting, coupled with localized potholes and edge failures, threatens the safety and efficiency of road users.

### 5.1 RECOMMENDATIONS

A two-tier maintenance program is recommended. In the short term, apply a chip seal surface treatment across the full carriageway to arrest moisture ingress and seal existing cracks, alongside deep patching of potholes and edge failures. Simultaneously, clear and reconstruct drainage channels to ensure rapid runoff. For sections exhibiting severe structural failure—particularly between km 2.0 and km 3.0implement full-depth reclamation using properly graded aggregates and hot-mix asphalt to reestablish base integrity.

Furthermore, establish a routine maintenance schedule with biannual inspections and prompt repairs using quality-controlled materials. Training for local maintenance crews on best-practice sealing and patching techniques will improve repair longevity. Finally, monitor pavement performance over a two-year period to evaluate the effectiveness of these interventions and guide future rehabilitation planning.

### 5.2 AREAS FOR FURTHER STUDY

Future research should examine the long-term performance of the chip seal under local climatic conditions and traffic volumes, as well as explore cost–effective options for integrating recycled materials into reclamation works.

REFERENCE

Ihueze,C.C.andOkafor,E.C.(2009).“MultivariateTimeSeriesAnalysis for Optimum Production Forecast”: A Case Study of 7up Soft Drink Company in Nigeria.

Ihueze,C.C.andMgbemena,C.O.(2014).“ModelingHyperelastic Behavior of Natural Rubber/Organomodified Kaolin Composites Oleochemically Derived from Tea Seed Oils (Camellia sinensis) for Automobile Tire Side Walls Application”. Journal of Scientific Research & Reports 3(19): 2528-2542.

Kwiatkowski,D.,Phillips,P.C.B.,Schmidt,P.,andShin,Y.(1992).“Testing the Null Hypothesis of Stationarity Against the Alternative of a Unit

Root”.JournalofEconometrics,54(1-3):159-178.

Kuo,C.Y.,J.D.Frost,J.S.Lai,andWang,L.B.(1996).“Three-dimensional image analysis of aggregate particles from orthogonal projections”.

Kweka, J.P and Morrissey, O. (1999). “Government Spending and EconomicGrowth.EmpiricalEvidencefromTanzania”(1965-1996)DSA Annual Conference Inc. New Haven.

Laguna,M.andMart´ı,R.(2003).“Scatterresearch-Methodologyand implementations”. In C. Kluwer Academic Publishers.

Lenth, R. V. (2009). “Response-Surface Methods in R, Using RSM”. Journal of Statistical Soft-ware, 32(7), 1–17. URL http://www.jstatsoft.org/v32/i07/.Lingle,J.J.(1956).“DiscussioninProc.”. AAPT, vol 25, p. 103.

Liu,D.C.andNocedal,J.(1989).“OnthelimitedmemoryBFGSmethod for large scale optimization”. Mathematical Programming 45, 503-528.

McCullagh,P.andNelder,J.A.(1989).“GeneralizedLinearModels (Second Edition). Chapman & Hall Ltd”.

Rogers,Martin(2002).“HighwayEngineering.BlackwellScience”.Oxford, UK. Wikipedia, the free Encyclopedia (2017). http://en.wikipedia.org/wiki/transport.