CHAPTER TWO

2.0			ANALYSIS OF CREEP MEASUREMENT APPARATUS

Creep is the continuous deformation with the passage of time when material is subjected to persistent stress. This deformation is plastic and occur if the applied stress is below the yield strength of the material. One of the approaches of interpretation of creep test data is to determine an empirical equation to fit a particular creep time curve at a given stress for a given temperate and material. A relationship between the creep strain, time and stress is required to select on allowable creep stress deformation. Microscopically material react to an applied stress. When the material strain is not permanent and return to zero after the applied stress is removed, it is elastic behavior. The stress at which the material behavior change from elastic to plastic is known as yield strength. In plastic deformation, strain is due to strain hardening. The process by which the material grows stronger as it deformed. There is another stress called ultimate tensile strength needed for further strain decreases result in necking of the material that is, decrease in the local area of the samples due to localization stress. If the material is plastically deformed too much, voids start to form which further weaken the material and eventually tears the material apart due to the ductile fracture.

Strain

Plastic Strain

Fracture

Stress

0.2 offset yield point Elastic Strain

Figure 2.1 Stress-strain Curve with distinct yield point.

For metals at higher values of stress or temperature, four distinct stages of creep are noticed. These stages are

    - Initial Stage:- in which the total deformation is partially elastic and partially plastic.

following an instantaneous elastic strain region, the curve generally has three

parts. These are:-

    - Second Stage:-A primary creep where the rate of creep deformation decreases with time, indicating the influence of strain- hardening

    - Third stage:- A secondary creep or steady state creep where the strain Increases steadily with time. The effect of strain counteracted by an annealing influence, which result in a stage of constant minimum creep

rate.

    - Final stage:-Tertiary creep where the rate of creep deformation increases and eventually causes failure. During the tertiary creep period, there is reduction in the cross sectional area of the specimen leads to higher stress.

Stage 1= Initial elastic Strain

Strain

2

3

4

1

Time

Stage 2= Primary Creep

Stage 3= Secondary creep

Stage 4= Tertiary Creep

Figure 2.2 The General Creep Curve.

As shown above creep is the plastic deformation of a material at a high temperature when subjected to a stress below the yield strength over a period of time. The effect of increase the stress and temperature is shown below.

𝛿

𝛿

𝛿

Time

Increasing Stress

Strain

𝛿

𝛿

𝛿

Figure 2.3 Effect of applied Stresses on the creep Curve

𝑇

𝑇

𝑇

𝑇

𝑇

𝑇

Increasing Temperature

Strain

Time

Figure 2.4 Effect of increase temperature on the creep-time curve.

2.1 	RELATED CREEP MEASUREMENT APPARATUS.

There are various types of creep measurement apparatus. hey different size their sizes, capacities, mode of operation and the types of material they measured. These includes series 2100 direct load creep frames, series 2300/2500 lever arm creep frames and series 2600 pneumatic activated creep frames.

SERIES 2100 DIRECT LOAD CREEP FRAMES.

This is a type of creep measurement apparatus with a loading capacity up to about 44400N and used for stress rapture and creep. It is a low cost system and has a multi-station units that offer separate control for each individual component for maximum system flexibility. It is incorporated with electric weight elevator and automatically maintain a small clearance between the stack and injury upon specimen breakage.

SERIES 2300 LEVER ARM CREEP FRAMES

This is another creep measurement apparatus with a loading capacity up to 22240N. It is long life test machine to meet virtually any requirement. It comprises models available to perform stress relaxation and constant stress testing. It has manual or automatic leveling draw head as well as electric or hydraulic weight elevators. All the model offer computer control with the versatile win CCSII soft ward from American Teaching System (ATS). This may involves flexibility test, preparation of specimen and generation of genuine report and analysis of data.

SERIES 2600 PNEUMATIC-ACTUATED CREEP FRAMES

This apparatus is efficient and accurate for creep testing. It is designed especially for pneumatic actuator with frictionless rolling diaphragm construction and nonrotating shaft with capacity up to 22200N. It as manual computer control with data acquisition, analysis and cyclic load capacity and more.

2.2 	THE DESIGNED CREEP MEASUREMENT APPARATUS

The creep measurement apparatus constructed which is manually operation is a tension creep test. Specimen are usually subject to constant loads by lever system of loading. The creep information are usually measure by dial indicator with accuracy of 0.01mm and rang of 001-10mm which served as the

Extensometer to measure the extension for room temperature and the creep test at elevated temperature. The designed creep measurement apparatus with heating chamber consist of an electric furnace which is maintained at constant temperature with thermostatically controlled heater that are used to apply elevated temperature to the specimen.

2.3  	CHOICE OF HEATING CHAMBER

Heat furnace are usually classified accordingly to the purpose for which the material is heated, the nature of the heat transfer to the material, the method of firing the furnace and method of handling the material through the furnace. The maximum temperatures are limited only by the nature of the material of the charge. The heating chamber are designed to operate at higher temperatures range.

The temperature of the inner face of the heating chamber determines the characteristics of material required for the insulation. Fiber grass is used for insulation of the heating chamber, an insulator of this kind has less heat storage capacity and it use accordingly decreases the time period of heating and cooling the chamber and also decreases the stored heat loss for a given cycles of operation other advantages are its high heat insulating value and light weight.

CHAPTER THREE

CONSTRUCTION OF THE CREEP MEASUREMENT APPARATUS

3.1  	GENERAL DESCRIPTION AND ITS WORKING PRINCIPLE

The creep measurement apparatus as a whole is rectangular in shape and it comprises of the two main components.

These are:-

- The rigid base plate

- The top mechanism

The rigid base plate consists of the bottom base plate and the cover base plate. The bottom base plate was made and bent to shape to give provision for accommodation of the cover base plate. It also forms the handle through which the apparatus can be carried with ease. The cover base plate serves as the rigid base on which the mechanism rest. On it, there is stand for holding the weights to be used when performing the experiment.

The top mechanism consist of the bottom beam, lever arm, dial indicator, grip or stirrups, heating chamber, front bar, rear bar and other bars.

The creep measurement apparatus with heating chamber employs lever action to apply load to the specimen. The lever beam is notched to receive the edge of the fulcrum which is at the top column rigidly attached to the base.

Typically, specimen is loaded in tension usually at constant load inside the furnace (Heating chamber) attached to the front bar and is maintained at constant temperature with thermostatically controlled heater with the specimen bolted to two stirrup for self-alignment. The lower stirrup is attached to the bottom beam by means of bolt and nut to allow height adjustment during set-up. The upper grip pivot on the lever arm. The weight hanger is bolt to the opposite end of the lever arm.

The tip the dial indicator attached to the front bar is aligned on the top flat edge of the upper stirrups. The dial indicator is set to zero.

The weight is suspended from the weight hanger and supported so that the specimen in the chamber is not loaded. After heating the specimen to the desire temperature, the hanger is lowered slowly to apply the load at equal interval of time.

3.2  	COMPONENTS OF THE APPARATUS AND MATERIAL CONSIDERATION

The material used for the construction of the creep measurement apparatus are low carbon steel sheet metals, mild steel bar, aluminums foil, zinc metal and fiber grass.

The low carbon steel is ductile and can be easily bend to the required shape of part. The mild steel has relatively good tensile strength and also compressive strength. It can be worked upon easily. It has a high melting point and thus withstand high working temperate. However, it has poor corrosion resistance.

Therefore, the bar were painted to prevent corrosion. The aluminium foil is very malleable and ductile; can be rolled into cylindrical shape easily and the zinc metal has a high tensile strength, malleable and ductile between 100°-150°C, but brittle at room temperature and at a temperatures above 200°C with relatively high thermal conductivity.

THE BOTTOM BASE PLATE: This is made of low carbon steel sheet metal. It is made and bend to give provision for the accommodation of the cover base plate. It also form the handle through which the apparatus can be carried with ease.

THE COVER BASE PLATE: This is also made of low carbon steel sheet metal. It is serve as the rigid base on which the mechanism may rest. On it, there is a stand for holding the weight to be used.

THE MECHANISM: This consist of the following components.

- The Heating Chamber: Is a fixed hollow cylinder having an outer casing of aluminum foil and inner casing of zinc metal. The heating element is a sinuous winding round the zinc metal mounted on the inner surface of the heating chamber with a thermostatically control devices. The heat insulation fiber grass are coated in the annular space between the outer and the inner casing of the chamber.

- The Lever Arm: This is made of mild steel bar with thickness 6mm. It provides the region for weight hanger for loading It is then inserted into a stirrup, connected to the specimen and having its flat top touching the tip of dial indicator.

- The Bottom Beam: This hold four mild steel bars supporting the lever arm. Tight in position to resist unpermitted relative movement of the bars. It is also made of mild steel bar.

- The Stirrups: This is made of low carbon steel sheet. It is made and bent to accuracy so as to accommodate the specimen. Therefore, it hold the specimen in position inside the heating chamber with the aid of bolts and nuts.

- The Dial Indicator: This is a sensitive measuring instrument that perform the function of an extensometer used to measures the extension of the specimen at an equal internal of time. It is an attached mechanism but not among the constructed components.

- The Stopwatch: This is also an attached mechanism of the creep measurement apparatus but not among the constructed component. It is

Used to regulate the time. It works along with the dial indicator when the specimen is elongating.

  - DIMENSION OF THE COMPONENT PARTS

| Components |  | Dimension |  |  | No. of Pieces |
| --- | --- | --- | --- | --- | --- |
|  |  | Length | Breadth | Thickness |  |
| The bottom base plate |  | 1000mm | 300mm | 1.5mm | 1 |
| The cover base plate |  | 650mm | 580mm | 1.5mm | 1 |
| The lever arm |  | 550mm | 30mm | 6mm | 1 |
| The bottom beam |  | 350mm | 30mm | 6mm | 1 |
| The front bar |  | 300mm | 30mm | 6mm | 2 |
| The rear bar |  | 250mm | 30mm | 6mm | 2 |
| The stirrups or grips |  | 100mm | 35mm | 1.5mm | 2 |
| The bar holding the indictor | dial | 100mm | 20mm | 2.0mm | 1 |
| The stand for the Weight |  | 90mm | 60mm | 2.0mm | 2 |
| Outer casing for the heating chamber |  | 250mm | 80mm | 1.5mm | 1 |
| Inner casing of the heating chamber |  | 125mm | 75mm | 1.5mm | 1 |

Table: 3:1 Dimension of the component part of the Apparatus.

  - CONSTRUCTION PROCESSES

Here the work includes metal cutting, marking out, screwing, and material measurement and so on. These are carried out to cut out and shape the component and give it the desire finished, correct fit with any mating part. These might be merely operation of assembly part together. During the process of cutting, fabrication and integration of the component parts to construct the apparatus, a clearance of + 2.0mm was taken into consideration so as to take care of the folding, welding, grinding effects and others in order to maintain their sizes.

The sequences of operation are as follow:

MEASUREMENT AND MARKING OUT PROCESS

Measuring and marking out of the sheet metals and steel bars according to the dimension. These are carried out with measuring tape, try square, vernier caliper, scriber and punch.

CUTTING PROCESS

This involved the cutting out of the part marked out on the sheet metal and the steel bars according to the specification with the aid of hacksaw, chisel and so on.

BENDING AND FOLDING PROCESSES

The marked out region of the sheet metal and the bars which required folding or bending were made to give the required shape.

This is done to bend and fold the metal by means of special mechanism into section such as angles bars, L-section e.t.c.

DRILLING OPERATION

This operation involved the originating of a round hole on the component at appropriate position after cutting, bending and folding of the sheet metal and the steel bars to the required shape to give provision for bolts and nuts. It is performed using hand and electrical drilling machine.

WELDING OPERATION

Some parts were joined together to arrive at the required shape of the creep measurement apparatus. Electric arc welding machine was used to carry out this operation.

GRINDING AND FILLING OPERATIONS

The bars and the sheet metals that have been cut, welded are filed and grinded so that any hardening, distortion and scaling that may have occurred to the components is detrimental to its shape and for accurate and better surface finish.

Grinding discard file of difference grades were used for the finishing operation.

PAINTING

The components parts of the apparatus were painted by spraying paint to every part. After painting, the components were hanged and allowed to dry for two days before assembly.

3.5 	ASSEMBLING THE COMPONENT PARTS OF THE APPARATUS

The components of the creep measurement apparatus were assembled with bolts and nuts through the already drilled holes.

The procedures are as follows:

- The bottom base plate is placed on a flat workshop bench.

- The cover base plate is gently put inside the space provided by the bottom base plate and the bolts are tightened.

- The front bars and the rears bars are welded to the bottom beam.

- The top holes of the rear bars and the front bars are connected to the lever arm through the corresponding holes on the lever arm and the bolts are also tightened.

- The front and the rear bars are placed on the rigid base plate at their bent positions which have been drilled at each bent, four bolts are tightened and the mechanism sits on the rigid base plate.

- The upper stirrup is connected to the front edge of the lever arm and the

Bolt is inserted.

- The lower stirrup is connected to the front edge of the bottom bean and the bolt is also inserted.

- The heating chamber is placed between the upper and the lower stirrups and connected to the front bars by means of two bolts and nuts.

- The specimen is inserted between the stirrups through the holes at the top and bottom of the heating chamber.

- The dial indictors is attached to the bar that has been welded to the top edge of the front bar at right angle and adjusted until its tip is allowed to touch the flat top of the upper stirrup with aligned vertically to the specimen.

- The weight hanger is placed at the rear end of the lever arm and the bolt inserted.

- The timer is inserted into the rigid base plate and connected through an external circuit to the heating chamber and other electrical components inside the bottom base plate.

- The stop watch is placed behind the set up.

With the above assembling procedures, the setup of the apparatus can be achieved completely within some minutes and then ready for test.

3.6  	DESIGN CALCULATIONS FOR THE APPARATUS

Calculating the bending moment of the lever arm with a known weight of.

50

N

.

A

330

mm

B

C

220

mm

RA       	        RR 	 	     50N

Fig 3.1: Free body Diagram

Let upward force = 1 + 𝑣𝑒

Let Downward force = − 𝑣𝑒

Taking moment about point B ∑ 𝑀 = 0

Let the clockwise moment = + 𝑣𝑒

Let the Anticlockwise moment = − 𝑣𝑒

330 𝑥 𝑅 + 50 𝑥 220 = 0

330 𝑥 𝑅 + 11000 = 0

330 𝑅	= − 11000

𝑅 = − 11000/330

𝑅 = − 33.3𝑁

𝑅	= 33.3𝑁

The direction of the reaction at A is downward since the reaction is negative. The free body diagram is shown below:

A

330

mm

B

C

220

mm

33.3

N

R

R

50N

Resolving the vertical component force  ∑ 𝐹 = 0

− + 𝑅 − 50 = 0

𝑤ℎ𝑒𝑟𝑒:

𝑅  = 𝑅𝑒𝑎𝑐𝑡𝑖𝑜𝑛 𝑎𝑡 𝑝𝑜𝑖𝑛𝑡 𝐴

𝑅	  = 𝑅𝑒𝑎𝑐𝑡𝑖𝑜𝑛 𝑎𝑡 𝑝𝑜𝑖𝑛𝑡 𝐵,

− 33.3 + 𝑅 − 50 = 0

𝑅 = 50 + 33.3

𝑅 = + 83.3𝑁

= 83.3𝑁

The direction of the reaction at B remain upward since the reaction is

Positive. The free body diagram is shown below.

330

mm

220

mm

33.3

83.3N

50N

B

C

A

m

330

m

220

83.3

N

33.3

Fig 3.2 Shear force Diagram

𝐹𝑜𝑟 𝑏𝑒𝑛𝑑𝑖𝑛𝑔 𝑚𝑜𝑚𝑒𝑛𝑡

𝐴𝑡 𝑃𝑜𝑖𝑛𝑡 𝐴

𝐵𝑒𝑛𝑑𝑖𝑛𝑔 𝑚𝑜𝑚𝑒𝑛𝑡 𝑎𝑡 𝑝𝑜𝑖𝑛𝑡 𝐴

33.3𝑥 0 = 0𝑁𝑚𝑚

𝐴𝑡 𝑝𝑜𝑖𝑛𝑡 𝐵

𝐵𝑒𝑛𝑑𝑖𝑛𝑔 𝑚𝑜𝑚𝑒𝑛𝑡 𝑎𝑡 𝑝𝑜𝑖𝑛𝑡 𝐵

83.2 𝑥 330 = 10989 𝑁𝑚𝑚

At point C

Bending moment at point C.

(− 33.3 𝑥 550) + (83.3 𝑥 220) = 𝑂𝑁𝑚𝑚

The Resultant Bending moment =10989 𝑁𝑚𝑚.

10989

𝑁𝑚𝑚

.

O

O

A B C

Fig: 3.3 bending moment diagram

CHAPTER FOUR

TESTING OF THE CREEP MEASUREMENT APPARATUS

The aim for constructing the creep measurement apparatus is to determine and show the creep time curve of an aluminum specimen for the purpose of knowing it mechanical property and how it will operate to an agreement level of performance under the condition specified for it use.

4.1  	PROCEDURES FOR OPERATING THE APPARATUS

- The specimen is placed between the stirrups through a hole at the top and bottom end of the heating chamber

- The dial indicator is set to zero

- The thermostat is set to the elevated temperature required.

- The stop watch is also set to zero.

- The apparatus is plug to the power supply with the switch off.

- The power supply is switch on and the test specimen loaded in tension by suspending the load from the weight hanger.

- The lever arm is gently lowered until the specimen takes up the full load.

- The stop watch is start.

- The extension of the specimen during heating is recoded at suitable interval of time say 30 seconds.

- The time for heating is taken and recorded using the timer.

- The steps are repeated for another temperature value.

- The extension of the specimen during heating at the same interval of time are also recorded.

4.2  	RESULTS AND DISCUSSION

The apparatus tested and the reading is tabulated as shown below.

The graph of strain against time at the two temperature was plotted and shown below.

Table 4.1 Table of extension and time at a temperature of 200°C and constant of

20N

Extension during loading = 0.30mm

| Load (N) | Extension (mm) | Time (Secs) |
| --- | --- | --- |
| 20 | 0.30 | 0.0 |
| 20 | 0.35 | 30 |
| 20 | 0.37 | 60 |
| 20 | 0.39 | 90 |
| 20 | 0.42 | 120 |
| 20 | 0.44 | 150 |
| 20 | 0.47 | 180 |

Table 4.2 Table of extension and time at a temperature of 250°C and constant load of 20N.

Extension during loading = 0.35mm

| Load (N) | Extension (mm) | Time (Secs) |
| --- | --- | --- |
| 20 | 0.35 | 0.0 |
| 20 | 0.42 | 30 |
| 20 | 0.45 | 60 |
| 20 | 0.48 | 90 |
| 20 | 0.52 | 120 |
| 20 | 0.54 | 150 |
| 20 | 0.56 | 180 |

To determine the creep rate

𝑐ℎ𝑎𝑛𝑔𝑒 𝑖𝑛 𝑠𝑡𝑟𝑎𝑖𝑛

𝐶𝑟𝑒𝑒𝑝 𝑟𝑎𝑡𝑒 =

𝐶ℎ𝑎𝑛𝑔𝑒 𝑖𝑛 𝑡𝑖𝑚𝑒

𝐸𝑥𝑡𝑒𝑛𝑠𝑖𝑜𝑛

𝑆𝑡𝑟𝑎𝑖𝑛 =

𝑂𝑖𝑔𝑖𝑛𝑎𝑙 𝑙𝑒𝑛𝑔𝑡ℎ

𝐴𝑛𝑑 𝑡ℎ𝑒 𝑜𝑟𝑖𝑔𝑖𝑛𝑎𝑙 𝑙𝑒𝑛𝑔𝑡ℎ 𝑜𝑓 𝑡ℎ𝑒 𝑠𝑝𝑒𝑐𝑖𝑚𝑒𝑛 = 110𝑚𝑚.

𝑆𝑡𝑟𝑎𝑖𝑛 𝑎𝑛𝑑 𝑐𝑟𝑒𝑒𝑝 𝑎𝑡 200°𝐶

𝑨𝒕 𝟎 𝒔𝒆𝒄

𝑆𝑡𝑎𝑖𝑛 = 0.30/110         = 0.0027

𝐶𝑟𝑒𝑒𝑝 = 0.0027/0        = 0/ 𝑠𝑒𝑐

𝑨𝒕 𝟔𝟎 𝒔𝒆𝒄.

𝑆𝑡𝑟𝑎𝑖𝑛 = 0.37/110        = 0.0033

𝐶𝑟𝑒𝑒𝑝 = 0.0033/60       = 0.00005/ 𝑠𝑒𝑐.

𝑨𝒕 𝟗𝟎 𝒔𝒆𝒄

𝑆𝑡𝑎𝑖𝑛 = 0.39/110        = 0.0035          𝐶𝑟𝑒𝑒𝑝 = 0.0035/90    = 0.000038/𝑠𝑒𝑐

𝑨𝒕 𝟏𝟐𝟎 𝒔𝒆𝒄.

𝑆𝑡𝑟𝑎𝑖𝑛 = 0.42/110      = 0.0038  𝑒𝑝  = 0.0038/120 = 0.000032 / 𝑠𝑒𝑐.

𝑨𝒕 𝟏𝟓𝟎 𝒔𝒆𝒄.

𝑆𝑡𝑟𝑎𝑖𝑛 = 0.44/110       = 0.0040 𝐶𝑟𝑒𝑒= 0.0040/130 = 0.000027 / 𝑠𝑒𝑐.

𝑨𝒕 𝟏𝟖𝟎 𝒔𝒆𝒄.

𝑆𝑡𝑟𝑎𝑖𝑛 = 0.47 / 110   = 0.0043 𝐶𝑟𝑒𝑒𝑝 = 0.0043/180 = 0.000023/ 𝑠𝑒𝑐

𝑆𝑡𝑟𝑎𝑖𝑛 𝑎𝑛𝑑 𝑐𝑟𝑒𝑒𝑝 𝑎𝑡 250°𝐶.

𝑨𝒕 𝟎 𝒔𝒆𝒄.

𝑆𝑡𝑟𝑎𝑖𝑛 = 𝑒𝑥𝑡𝑒𝑛𝑠𝑖𝑜𝑛/ 𝑜𝑟𝑖𝑔𝑖𝑛𝑎𝑙 𝑙𝑒𝑛𝑔𝑡ℎ

𝑆𝑡𝑟𝑎𝑖𝑛 = 0.35/110  = 0.0032 𝐶𝑟𝑒𝑒𝑝 = 𝑆𝑡𝑟𝑎𝑖𝑛/𝑡𝑖𝑚𝑒 0.0032/0  = 0. 𝑠𝑒𝑐

𝑨𝒕 𝟑𝟎 𝒔𝒆𝒄.

𝑆𝑡𝑟𝑎𝑖𝑛 = 𝑒𝑥𝑡𝑒𝑛𝑠𝑖𝑜𝑛/ 𝑜𝑟𝑖𝑔𝑖𝑛𝑎𝑙 𝑙𝑒𝑛𝑔𝑡ℎ

𝑆𝑡𝑟𝑎𝑖𝑛 = 0.42/110  = 0.0038

𝐶𝑟𝑒𝑒𝑝 = 0.0038/30 = 0.00013/ 𝑠𝑒𝑐

𝑨𝒕 𝟔𝟎 𝒔𝒆𝒄.

𝑆𝑡𝑟𝑎𝑖𝑛 = 0.45/110  = 0.0041 𝐶𝑟𝑒𝑒𝑝 = 0.0041/60 = 0.000068/ 𝑠𝑒𝑐

𝑨𝒕 𝟗𝟎 𝒔𝒆𝒄.

𝑆𝑡𝑟𝑎𝑖𝑛 = 0.48/110  = 0.0044 𝐶𝑟𝑒𝑒𝑝 = 0.0044/90 = 0.000049/ 𝑠𝑒𝑐.

𝑨𝒕 𝟏𝟐𝟎 𝒔𝒆𝒄.

𝑆𝑡𝑟𝑎𝑖𝑛 = 0.52/110 = 0.0047 𝐶𝑟𝑒𝑒𝑝 = 0.0047/120 = 0.000039/ 𝑠𝑒𝑐

𝑨𝒕 𝟏𝟓𝟎 𝒔𝒆𝒄.

𝑆𝑡𝑟𝑎𝑖𝑛 = 0.54/110      = 0.0049 𝐶𝑟𝑒𝑒= 0.0049/150 = 0.000033/ 𝑠𝑒𝑐

𝑨𝒕 𝟏𝟖𝟎 𝒔𝒆𝒄.

𝑆𝑡𝑟𝑎𝑖𝑛 = 0.56/110      = 0.0051

𝐶𝑟𝑒𝑒= 0.0051/180 = 0.000028/ 𝑠𝑒𝑐

𝑆𝑡𝑟𝑒𝑠𝑠 = 𝐹𝑜𝑟𝑐𝑒/𝐴𝑟𝑒𝑎. 𝐹𝑜𝑟𝑐𝑒 = 20𝑁, 𝐴𝑟𝑒𝑎 = 𝐴𝑟𝑒𝑎 𝑜𝑓 𝑆𝑝𝑒𝑐𝑖𝑚𝑒𝑛

=   110 𝑥 10 = 1100𝑚𝑚2,𝑒𝑠𝑠 = 20/1100

= 1.8 𝑀𝑁/𝑚2

Table 4.3 Table of Strain and Time at 2000c

| TIME (SECS) | STRAIN |
| --- | --- |
| 0.0 | 0.0027 |
| 30 | 0.0031 |
| 60 | 0.0033 |
| 90 | 0.0035 |
| 120 | 0.0038 |
| 150 | 0.0040 |
| 180 | 0.0043 |

Table 4.4 Table Strain and Time at 2500c

| TIME (SECS) | STRAIN |
| --- | --- |
| 0.0 | 0.0032 |
| 30 | 0.0038 |
| 60 | 0.0041 |
| 90 | 0.0044 |
| 120 | 0.0047 |
| 150 | 0.0049 |
| 180 | 0.0051 |

CHAPTER FIVE

CONCLUSION AND RECOMMENDATIONS

5.1  	CONCLUSION

In order to determine the effect of temperature on creep and arrive at the required objectives of this report, the result and data analysis shows that the need for incorporating the heating chamber with the creep measurement apparatus is good and meets the t is intended for since the specimen show the creep curve of material at elevated temperature arrive at the required objectives of this report, the result and data analysis shows that the need for incorporating the heating chamber with the creep measurement apparatus is good and meets the t is intended for since the specimen show the creep curve of material at elevated temperature.

5.2  	RECOMMENDATIONS

Having constructed the creep measurement Apparatus with heating

Chamber, it is tested and found to be effective.

For further construction of the apparatus with heating chamber, stainless steel should be used for the construction of the lever arm, bottom been and the bars to enhances preservation of the material. More practical work should be encourage on the designed and construction of the creep measurement Apparatus with heating chamber in order to determine the required heat input of the Heating device and thus there must be a standard calibration of the thermostat.

REFERENCES

Hertzberg, R.W. (1998). "Deformation and Fracture Machines of Engineering Materials" 3rd edition, Wiley and Sons, New York

Michael, F. Ashy and David, R.H Joens, (1999). "Engineering Materials," 2nd editions, Cambridge University, England.

Vermon John, (1992). "Introduction to Engineering Materials" 3rd edition, Macmillan education Ltd, London. C.

William, D.C, J.R (1996). "Introduction to materials science and Engineering" 4th

edition, John Wiley and Sons, Inc Canada.