ROUTE SURVEY FROM NEW IES TO ARA VILLAGE

BY

AREWA ENIOLA OLUWASEUN

ND/23/SGI/FT/0015

A PROJECT SUBMITTED TO

THE DEPARTMENT OF SURVEYING AND GEOINFORMATICS

INSTITUTE OF ENVIRONMENTAL STUDIES

KWARA STATE POLYTECHNIC, ILORIN. KWARA STATE

IN PARTIAL FULFILMENT OF THE REQUIREMENT FOR THE AWARD OF NATIONAL DIPLOMA IN (ND) SURVEYING AND GEOINFORMATICS

JULY, 2025

CERTIFICATE

I hereby certify that the field work and information given in this project were obtained as a result of my observation and measurement and were carried out in accordance with survey laws and departmental instruction

_______________  					_______________

AREWA ENIOLA OLUWASEUN				Date

Student Name

CERTIFICATION

This is to certify that AREWA ENIOLA OLUWASEUN with matric number ND/23/SGI/FT/0015 from Department of Surveying and Geoinformatics, Institute of Environmental Studies carried out a practical field work which formed basic of the project in accordance with survey rules and regulation and departmental instruction

_______________  					_______________

SURV. AKINYEDE .A.O						DATE

SURV. BELLO .F.D

Project Supervisor

_______________  					_______________

DATE

Project Co-Ordinator

_______________  					_______________

MR ISAU A.I							DATE

Head Of Department

_______________  					_______________

External Examiner							DATE

DEDICATION

This project is dedicated to Almighty God and to my beloved parent MR and MRS AREWA

ACKNOWLEDGEMENT

I give glory and adoration to Almighty God the creator of heaven and the earth, the first and the last who give me the opportunity to complete this program. I also give thanks to my lovely parent MR & MRS AREWA for their maximum support towards achieving all these.

I had acquired in my life and the one am still looking up for, may Almighty God let them reap the fruit or their labour (amen).

I also acknowledge the support of my project supervisors in person of my project supervisor in person of SURV AKINYEDE & SURV BELLO DIRAN for their strictly and through supervision of this project. Also 	the HOD of my department MR ISAU A.I and my lectures SURV KABIR, SURV KAZEEM, MR AYUBA and those that that could not mention their name may Almighty God guide and bless your families (Amen).

My appreciation also goes to my lovely brother and sister’s for their absolute co-operation and wonderful support towards my career. Because without their reasonable influence and advise they do rendered tome. May almighty god bless them together (amen).

My piece of appreciation cannot be completed without the appreciating the support of all my villa mates and my friends both in school, at home and other different institutions for their maximum co-operation throughout my life. I thank you all, may almighty god be with you.

Finally my profound gratitude goes to my friends who gave me full support throughout my national diploma (ND) days. I say a big thanks to you all and wishing you all the best in our entire endeavour.

ABSTRACT

This project considered the route survey from New IES to Ara village. It involve planning and designing of survey field work like determination of center line of the road coordinators of the point and sectioning such that observation were made in the field which met the condition and specification of the third order survey using total survey (south) instrument Data obtained from the field were download into a computer system using south NTS software version for the data processing. The X, Y, Z coordinates of all point where obtain of which was used to produce a digital map of the showing the height differences between point along the Centre line of the road. Recommendations were also made based in the result obtained and the experience gathered in the excursion of the work.

TABLE CONTENT

TITLE PAGE								i

CERTIFICATION								ii

DEDICATION								iii

ACKNOWLEDGEMENT						iv

ABSTRACT								v

TABLE OF CONTENT							vi

LIST OF FIQURES							vii

LIST OF TABLES								viii

CHAPTER ONE

1.0	Introduction								1

1.1	Background of study						2

1.2	Statement of problem						4

1.3	Aim of study							5

1.4	Objective of study							5

1.5	Scope of study							6

1.6	Project specification						6

1.7	Personnel								7

1.8	Study Area								7

CHAPTER TWO

2.0	Literature review							8

CHAPTER THREE

3.0	Methodology							13

3.1	Reconnaissance							14

3.1.1	Data Acquisition (Data acquisition method)			15

3.1.1.1	Office Planning						16

3.1.1.1.1	Field Reconnaissance					16

3.2	Equipment used & system selection				16

3.3	Instrument test & configuration					17

3.4	Control check							18

3.5	Geometric data acquisition					18

3.5.2	Control check/Establishment					19

3.5.3	Traversing								19

3.5.4	Spot Heighting							19

3.6	Detailing								20

3.7	Attribute data							20

3.8	Data downloading and Editing					20

3.8.1	Data processing and Report analysis				21

3.8.2	Data bank								22

3.8.3	Data security							23

3.8.4	Data integrity							24

3.8.5	Data maintenance							24

CHAPTER FOUR

4.0	Analysis & Information presentation				25

4.1	Longitudinal section						27

4.2	Vertical section							29

4.3	Cross section							31

4.4	Application								38

CHAPTER FIVE

5.1	Summary, Conclusion and Recommendation	s		39

5.2	Summary								40

5.3	Conclusion								41

5.4	Recommendations							41

Reference

LIST OF FIGURES

Figure 1:	Recci diagram

Figure 2:	Map of the study area

LIST OF TABLES

Table 3.1:	Co-ordinate of given existing control point

Table 3.2:	Established Co-ordinate of control point

Table 3.3:	Showing new co-ordinate of control pillars

CHAPTER ONE

BACKGROUND TO THE STUDY

1.1	INTRODUCTION

Route survey is a survey for the design and construction of linear works, such as roads and pipelines, is the way of collecting data about a proposed new route for a road utility pipe,railway, rapid transits guide ways, canal, meanwhile surveying comprises of all surveying / survey operation required for the design and construction of engineering works such as traversing of the road, profit, leveling and cross sectional leveling. (Raymond 1992)

Wilkinson .D. (1997) In construction of highways, route survey works are required for the development of the project estimation of cost. Route surveying collect data about proposed new route for road, utility pipe and railway transmit guide, canal and transmission line, route surveying pertains to the laying out of the proposed corridor for transportation system. A survey of the earth’s surface along a particular route in the compilation and updating of topographical, geological, soil and other maps and the correlation of selected contours and objects with geodetic reference points or landmarks during linear surveys, and also in the study of the dynamics of natural and socioeconomically phenomena in a narrow strip of terrain. In route survey, representation of the plane horizontal features (including the terrain if necessary) on both sides of it within the limits of directs visibility are plotted on a map board using method of instrument surveying.

To carry out a good economic and easy maintenance of construction.

- Investigation and planning

- Designing

- Construction

In order to carryout route survey successfully the survey engineer must be familiar with

    - The geometry of horizontal and vertical curve how they are used in the route.

    - Making measurement necessary to verify the location of the structure.

    - How to determine the volume of work actually performed up to a given level.

Route survey it involve in measuring and computing horizontal and vertical angles, elevation and horizontal distance the results of these surveys are used prepare detailed plan and profile base maps of proposed road ways. It the elevations determined in the survey serve as the basic for calculation of construction cut and fill quantities and in determining roadway banking. This section presents a review of basic terminology. Concepts and standard procedures used in high way surveys. The principle of mobility is of immense global concern to human and plant in such a way that the objective of mobility is achieved in most conducive manner. It is an idea that has a natural influence on activity of both plants and man that its effect has direct impact on life plant extend their root in search of nutrients and support, this is a form of mobility. Any obstruction in the course may result to life termination. This may invariably have an adverse effect on the environment. This project is basically on route surveying. Route survey is a process in surveying that can be applied to establishment of horizontal and vertical alignment for transportation facilities these include: high ways, canals, pipelines, transmission lines and rapid transit.

A Route Survey is defined as being the required service and product that adequately locates the planned path of a linear project or right of way which crosses a prescribed area of real estate, extending from at least one known point and turning or terminating at another known point. Adequate location shall mean substantial compliance with the conditions and tolerances expressed in this standard. A route survey which defines new or proposed boundaries shall be conducted as a boundary survey and must adhere to the rules and regulations of the Texas Board of Professional Land Surveying (TBPLS).

A Route Survey is usually required for the planning of a right of way, the acquisition of fee or easement property and for eventual construction layout work. The location of the facilities within the right of way are often held in respect to the center line or a right of way line. A Route Survey is made on the ground to provide for the location of right of way lines, a centerline, or reference lines in relation to property lines and terrain features. Route Surveys shall include but are not limited to the proper location, monumentation, description or platting of the following routes:

- Transmission lines for communications, fuel, chemical, water and electrical needs.

- Canals, waterways, drainage ditches and sewers.

- View easements, air space easements, ingress and egress easements such as approach routes.

1.2 	STATEMENT OF PROBLEM

The route from new IES to Ara village, Moro Local Government of Kwara State has been discovered for long, for having bad outlook, narrow, strip, poor drainage facility and not good for transportation of goods, and not form main road of Polytechnic opposite side therefore for all this problem should be a need for design of the road.

1.3	AIM OF THE PROJECT

The main aim of this project is to carry out route survey from new IES to Ara village in Moro Local Government Area of Kwara State

1.4	 OBJECTIVES OF THE PROJECT

- Reconnaissance survey which include determination of centre line of the road and marked at an interval of 25m along the road, determine the proposed width of the road, approximate total chainage of the road from starting point to the end point

- Running of accurate traversing

- Detail survey of all existing features along the chosen road

Leveling to determine the height difference along selected point along the road (both longitudinal, profile and cross section)

(iv)	Good report writing

1.5	SCOPE OF THE PROJECT

  - Reconnaissance

  - Office planning

  - Field recci

  - Chainage marking at 25m & bench mark establishment at every 500

  - Test of instrument

  - Geometric data acquisition

  - Attribute data acquisition

  - Data downloading editing

  - Data processing and analysis

  - Information presentation both hard/soft copies

1.6	SPECIFICATION OF THE PROJECT

i. The equipment and method adopted in carrying out the project ensured that the specification of third order were followed.

ii. The road alignment was obtained at every chainage of direction profile at 25m interval along the Centre lines and 4m to both right and left of the Centre line.

iii. Fixing of relevant details along the road for good interpretation of plan.

1.7	PERSONNEL INVOLVED

All the under list name is member of group 8 that participated in the successful execution of the project

| NAME | MATRIC NO | ROLE PLAYED |
| --- | --- | --- |
| Arewa Eniola Oluwaseun | ND/23/SGI/FT/0015 | Author |
| Adebunmi Yusuf Ayomide | ND/23/SGI/FT/0006 | Member |
| Laode Abdullahi Alabi | ND/23/SGI/FT/0007 | Member |
| Nurudeen Muhammed Jamiu | ND/23/SGI/FT/0010 | Member |
| Onimago Afisat Idowu | ND/23/SGI/FT/0013 | Member |
| Ambrose Sunday | ND/23/SGI/FT/0024 | Member |
| Yusuf Faisat | ND/23/SGI/FT/0016 | Member |
| Nairabakan Khadijat Gbemisola | ND/23/SGI/FT/0012 | Member |
| Kehinde Rofiat Oluwabukola | ND/22/SGI/FT/0018 | Member |

1.8	SITE LOCATION

The location of the project site is the route from New IES road to ARA village. The distance covered was approximately 3km. The site lies between

LATITUDE -	008˚ 25’ 30’’ to 008˚ 57’ 33’’

LONGITUDE -	004˚ 35’ 23’’ to 004˚ 38’ 10’’

Fig 1.0: showing the study location

Source: Google earth 2025

CHAPTER TWO

2.1	LITERATURE REVIEW

Route surveying is an important tool in the planning and construction of road. There has been a gradual improvement in the technique used in road construction in the world. Surveying is mostly embraced in every operation relating to the surface of the and consist geographical features surveying could therefore be referents to as the plant form upon which all construction work depend assuming the position of the bedrock of all physical development. (Anderson. 1985)

Raymond Metal surveying, (2009), said that a right of way survey is a boundary survey depending the limits of private ownership and that of the entity legacy that has the interest or jurisdiction of the subject property. Depending on the location of the site, the entity/agency may be the entity, unlike an easement which is the right of use over the property of others. Title to the right of way may exist by different sources that include dedication, eminent domain, and maintenance. Many times it is a combination of these that make the total parcel. The surveyor will make a determination of the position of the deeded right of way. If there appears to be additional right of way by maintenance that has been interrupted for more than four years, an updated maintained map may be field. The right of ways survey does not include above or below ground improvements, features or encroachments.

Engineering survey which is focus of this project could be define as the surveying carried out to provide special information’s for construction project which were usually drawn to large scale topographical surveying ranging in scale from 1:500 to 1:2500 supplying details for particular engineering schemes (Ramsey 1976).

Such scheme include hydroelectric, indignation project alignment of road, tunnel and transmission lines, the location and construction of drawn, the development and Maintenance of harbor, with respect his project the principles in surveying operation were copied to route alignment which conversed planning designing constructing development and rehabilitation of road depending on the survey data provided by the surveying.

The essence of route surveying was to establish the horizontal and vertical alignment for transportation which was assumed to comprise a network that include by way highway rapid transit guide ways, canals, pipeline and transmission line (Anderson, 2007) also defined surveying as the survey necessary for the location and construction of line of transportation or communication, such as highways, rail road’s, canal, transmission line and pipeline.

Raymond at all stated further that the location and construction survey further consist of:-

- Establishing the centerline by setting at interval

- Running level to determine the profit of the ground along the centerline at 25 meter interval.

- Plotting such profile and fixing grades

- Taking cross section

- Calculating volumes of earth work

- Measuring drainage areas

- Laying out structures such as culverts and bridges

- Location right of way boundaries.

Surveying operation involve in route surveying include the following to determine the horizontal and vertical alignments and leveling and for heighten purpose.

Raymond (1966) said that route surveying of some type are require in practically all highway construction works. For small project involving widening or minor improvement of existing highway, the survey may be relatively simple and may include sufficient information for the design engineer to prepare plans and specification defining the work to be done for the more complex construction of project involving multi-lane highway in a new location, prepare plans specification and estimate for construction, and to prepare deed descriptions and maps for appraisal and acquisition of necessary right of ways. In general, it is the function of the surveyor or location engineer all survey data which may be needed to make a necessary decision concerning the particular project. The process will include obtaining the necessary information regarding terrain and land use, making surveys to determine detailed topography and the location of plan metric features and finally the stationing of Centre line of the proposed highway on the ground. The survey engineer may also be given the responsibility for obtaining and utilizing data gather by specialist in related fields such as traffics right of way, geology, soil and foundation and drainage prior to survey for a particular project, the beginning and ending point are visually been defined in general authorization planning and so on. According to him, route survey procedure generally carried out in three stages: reconnaissance, preliminary survey and location survey.

Reconnaissance: include a general study of the entire area, the development of one or more alternatives routes or corridors and the study of each corridor in sufficient details to enable the proper official to decide which will provide the optimum location. The preliminary survey is a survey of the selected corridors in sufficient details to permit staking of the final Centre line can be complete and staked in the field without variation. In other instance minor adjustments may be made during the location survey, the location involve staking the final line and obtaining all additional information necessary to enable the engineer to prepare complete plan, specification and estimates including accurate estimate on earth work quantities and to prepare deed description covering the right of way to be acquired.

Route surveying is the engine and cradle of development useful information for the development of an appropriate course for transporting energy, good and services from one place to another. In order to carryout route survey successfully the survey engineer must be familiar with the following. Barrister A. and Raymond (1977).

Thomas & Noriman (1961) defined engineering surveying as expression for any survey work carried out in connection with the construction of civil engineering and building project. He also stated that engineers and surveyor involved in site surveying are responsible for all aspect of dimensional control on such schemes. According to Norman the main purpose of engineering surveying are: To provide large scale topographical maps/plans and other measurements at the conceptual and design stage, to provide precise framework at the construction stage, to monitor structural movement on major retaining structure at the post construction stage,

Wilson (1977) believed that when there is need for design of route to be carried out, different type of plans must be acquired to enable such design to be approximately done; such plan may be cadastral and topographical in nature. Also, engineering surveying is divided into design-data-survey and construction survey. A design- data –survey is an orderly process of obtaining data that is needed for the planning and design of an engineering report. The activities involved in design-data surveying vary according to the type and existing owner of the engineering or construction project. For example, the activities might include simply    obtaining topographic data for a proposed building site or they may include extensive route surveying and soil investigation for a highway.

A route survey as the name implies is a survey that deals with the route or course that imaginary road or utility line will follow while the end product of a route survey for a highway certainly differs from that of a utility line, it may nevertheless be said that the purpose of the route survey are to: Selection one or more tentative general route for the roadway or utility, gather enough information about the general route to make it possible for designers to select the final location of the route, mark this final location.

Reginnal (1968) defined route survey as the survey necessary for the location and construction of lines of transportation or communication such as highways, canals, transmission lines and pipelines. He stated that the location and construction survey may consist of Establishing the centre line by setting at intervals and running level to determine profile of the ground along the centre line, taking cross section, Plotting such profile and fixing grades, Laying out structures such as bridges and culverts.

CHAPTER THRRE

3.1 	METHODOLOGY

Methodology is a method and principles /way used to perform a particular activity and the activities is said to be successfully executed, proper planning and the development of the work. Plan show how good and objectives are to be accomplished. The proper planning that was taken for the successful execution of this project and this are involved

- the choice of the most appropriate techniques for execution of the project

- selection of equipment used.

- the design of a monitoring scheme that really helped in achieving the require accuracy for the project, starting from reconnaissance to the final product.

3.1 	RECONNAISSANCE

This is a very important and first aspect in any survey project /work carried out to obtain the general view of the study area in term of nature of the terrain and to properly plan the best way to the set aim and objectives of the project. And it can help to minimize any delays or changes in the observing schedule but for this it was carried out in two ways :

- Office reconnaissance

- Field reconnaissance

3.1.1 	OFFICE PLANNING

It is the act of involved the collection of equipment needed for the project, test of the instrument, missing planning assembling and studying of all available data relevant to the project, the costing of the operation was also done in the office. In executing the project, necessary information related to the project give such as coordinate of the control use were obtained from the SIWES and the practical unit of the school. Office planning is also known as office reconnaissance.

Table 3.1:  show the given bench marks Values

| Stn. I.D | Easting (m) | Northing (m) | Height (m) |
| --- | --- | --- | --- |
| BM1 | 682642.92mE | 944970.38mN | 347.21m |
| BM2 | 682626.44mE | 944965.70mN | 347.56m |
| BM3 | 682636.20mE | 944944.22mN | 348.01m |

3.2	EQUIPMENT USED

The following were the equipment used in the execution of the project

The equipment was grouped in to two categories

3.2.1 Hardware Equipment Used.

- Total station (South NTS 352L) and it's tripod

- one steel tape

- one plumb bob

- one cutlass

- two ranging poles

- paint

- field book

3.3.2 SOFTWARE USED

The software components used for this project are

- Sokkia link

- AUTOCAD civil 3D 2009

- Microsoft office 2010

- Notepad

- HP printer drivers

3.3	TEST OF INSTRUMENT

The instrument (total station) contain an inbuilt program that carries out self-test including signal test collimation, test battery, temperature test, then the instrument use set up and all their temporary adjustment were carried out, horizontal and vertical reading were taken and the telescope transmitted and the target bisected the second time and reading obtained were recorded below.

The two total stations used for this project, that is Leica TS02 and Sokkia set 630R total stations were tested to ensure that they were in good working condition and also to determine if the instrument meet up with the required accuracy of the project at hand which is a third order project. The total station equipment was set up over point (A) and then powered on all temporary station adjustment were carried out. The instrument menu was navigated to the instrument constant sub-menu which was selected after which the instrument then prompt to bisect the target. The target already set upon point (B) was bisected and enter key pressed, the instrument then prompted to bisect the same target again using the second face of the telescope which was done. After the whole observations, the instrument displayed the old and new values for the instrument constant error, the new value was set to replace the old value. The result and analysis are shown in the tables below.

Fig. 3.2 show the instrument set up for collimation test.

Table 3.2:	show the Test of Instrument result.

| Stn. | Sight | Face | Horizontal circle reading | Deduced angle | Vertical circle reading | Sum |
| --- | --- | --- | --- | --- | --- | --- |
| X | Y | L | 2250 30’ 20’’ |  | 0890 32’ 12’’ |  |
|  | Y | R | 450 30’ 20’’ | 1800 00’ 10’’ | 2700 28’ 01’’ | 3600 00’ 13’’ |

Difference in horizontal circle reading = 1800  00’ 10’’

Horizontal collimation error = 1800  00’ 10’’ - 1800  00’ 00’’ = 000  00’  10’’

Collimation error = 000  00I  10II

2

Horizontal collimation error = 000  00I  05II

Sum of vertical circle reading = 3600 00’ 46’’

Vertical collimation error = 3600 00’ 46’’ - 3600 00’ 00’’

= 0000 00’ 13’’

2

= 000  00’  6.5’’

The above table shows the collimation errors and the new reading was used because it is similar to the old reading.

3.4	MONUMENTATION

This is the act of identify the selected and marking them with wooden pegs or nail /bottles cover during the execution of the project. This could be temporary or permanent, depending on the nature of the job specifically for the project. The wooden pegs or bottles cover were 3cm in diameter and above 15cm long with nail on top for easy identification of the point. These were driven into the ground leaving about 2cm above the ground.

Ground level

18cm

18cm

Beneath Ground level

75cm long

60cm

80cm Iron rod

Above Ground level

15cm

Figure 3:1: show the diagram T.B.M

3.5	CONTROL CHECK

Control check involves checking the control used for the execution of the project to know if they are still intact or not. The instrument was set up on a known point (TBM1) and all necessary temporary adjustment were made, the reflector was taken to another known point (TBM2), the angle difference between compute angle, measure angle, computed distance and measured distance are within allowable.

| Stn. I.D | Easting (m) | Northing (m) | Height (m) | Remark |
| --- | --- | --- | --- | --- |
| BM1 | 682642.92mE | 944970.38mN | 347.21m | Given |
| BM1 | 682642.82mE | 944970.31mN | 347.19m | observed |
|  | -0.100 | -0.07 | -0.03 | difference |
| BM2 | 682626.44mE | 944965.70mN | 347.56m | Given |
| BM2 | 682626.46mE | 944965.79mN | 347.62m | observed |
|  | +0.02 | +0.09 | +0.06 | difference |
| BM3 | 682636.20mE | 944944.22mN | 348.01m | Given |
| BM3 | 682636.10mE | 944944.12mN | 348.11m | observed |
|  | -0.10 | -0.01 | +0.10 | difference |

3.6	DATA ACQUISITION

The data for this project work (digital route surveying) were acquired using the total station and its accessories, the automated surveying instrument and their hardware components. The information acquired were for the existence of the natural and man-made features.

3.6.1	DATA SOURCE

Data source is also called a data file, it is simply a collection of records that stores data. Digital route surveying requires input of relevant digital data the data consist of two types;

- Attribute data (Descriptive Information)

- Spatial data representing geographical features (culvert, buildings, area, trees etc.)

There are two types of data source used in this project, i.e. primary and secondary data source

Primary data source is the type of data captured on the field with the use of total station (i.e. X, Y, Z) while secondary data source are the type deals with the coordinate of the control point used for the project

3.6.2	INSTRUMENT OPERATION

The procedure for the operation of instrument is this:

- Orientation of the instrument

- Level of the instrument with the three roof screw

- Rotation of both the vertical and horizontal axis of the instrument to 3600

- Press menu

- Press F1, it will immediately display all the files present

- Press F3 to enter new file name.

- After certifying the above, press enter

- It will take you to page 2

F1 -------- occupy print

F2 -------- back sight

F3 -------- forward sight

- Press F1 to input coordinate

N:

E:

Z:

F1 – input, F2 – back, F3 – space, F4 – enter

- After certifying, press enter, it immediately returns to page 2, then enter the back sight coordinate.

Below shows the result of TBM.

| Stn. I.D | Easting (m) | Northing (m) | Height (m) |
| --- | --- | --- | --- |
| BM1 | 682642.92mE | 944970.38mN | H=347.21m |
| BM2 | 682626.44mE | 944965.70mN | H=347.56m |
| BM3 | 682636.20mE | 944944.22mN | H=348.01m |
| BM4 | 682674.19mE | 945459.38mN | H=341.76m |
| BM5 | 682755.17mE | 945457.52mN | H=342.56m |
| BM6 | 682850.45mE | 945949.18mN | H=350.00m |
| BM7 | 682905.59mE | 945928.27mN | H=349.67m |
| BM8 | 682613.33mE | 944316.61mN | H=351.65m |
| BM9 | 682653.25mE | 944357.07mN | H=352.42m |

CHAPTER FOUR

4.0	DATA PROCESSING AND RESULT ANALYSIS

This are the mathematical process, which entails the computational procedure carried out on raw data collected from the field. These were used in producing the finished product known as plan/chart and it is usually done in the office hence it is called office computation various computation procedures carried out as follow. This stage also involved various computational process such as checks on both linear and angular disclosure of the observed data in order to ascertain if the job carried out is within the accuracy of third order job so as to meet the purpose for which the project was carried out.

4.1	DATA DOWNLOADING AND EDITING

Sokkia link software was used for downloading from Sokkia total station equipment following the procedure below;

- The total station was connected to the desktop computer

- The PC was reboot to establish a new COM port cI created a new project by clicking the new project and named it group five

- On the Sokkia link software, the port set up was clicked and the parameters was set to match to the instrument setting

- I then switched on the station and set the parameters boudrate 9600, data bits and stop bit s1, parity none

- I clicked on ok

- I clicked on send data on the instrument

- The data stored on the instrument was located.

- I clicked data on the menu bar of the software and it dropped down, I clicked receive data

- The connection was clicked on the software

- I then pressed the SEND button on the instrument simultaneously.

- The data was then copied from Sokkia link software to notepad for further processing

4.2	DATA PROCESSING

The downloaded data from the total station was further edited using Microsoft excel and notepad, the final edited copy was saved as the text file containing X, Y and Z coordinate of all points observed in the field.

4.3	RESULT ANALYSIS

The results were analyzed so as to check the accuracy of the job by comparing the results obtained with the minimum allowable error acceptable for this order of survey job in accordance with survey rules and departmental instructions.

4.4	DESIGN AND VOLUME COMPUTATION

The longitudinal profile of the route was created before the designing was done. The longitudinal profile was created using AutoCAD civil 3D land desktop companion 2009. The steps followed are as follows

- The AutoCAD civil 3D land desktop companion 2009 suite was a new project was created and named

- The unit icon was clicked to set the unit of the drawing as follows

- Length type decimal

- Length precision – three places (I.e. 0.000)

- Insertion scale

- Unit to scale inserted content- meters

- Angle type- Deg/Min/Sec.

- Angle precision – Od 00’00”

- The clockwise box below was clicked

- The direction icon below was clicked and a direction control dialog box was opened. The North was selected and Ok was clicked to exit the dialog box

- The point menu was then clicked in the menu bar and the point settings was selected. The “coords.” icon was clicked to select the method of data arrangements (i.e. Easting – Northing). The type of marker was also selected here.

- Under the point menu, the import/ export point icon was selected

- The import point was selected after which a dialog box opens the format in which the data was arranged was selected i.e. ENZ (space delimited). Also the source input data files earlier served and ok was clicked to continue

- The points were imported and the centre lines joined with the polyline drawing tool in AutoCAD toolbar, the OSNAP (object snap) was switched on so as to snap the polyline ends to the points to enable precise and easy joining

- Alignment was then selected from the menu bar and define from sub-menu was selected

- The polyline used in joining all the center line was then clicked, where the alignment was to start was also clicked. A dialog box then opens and the alignment was named.

- On the alignment menu on menu bar, the station label settings was clicked, the station label increment, station tick increment and station label offset were edited appropriately. The perpendicular labels and plus sign location was also clicked. Then the station labels were created.

- The terrain was created by clicking on the terrain menu on the menu bar, then “Terrain Model Explorer” was selected. A dialogue box opens up and a new surface was created and named. The points file was added and the arrangement for format selected as well as the file path where it was located. The surface was built by right clicking on it and select and the dialog box was closed after building the surface.

5.4	LINEAR ACCURACY CALCULATION

This was computed using the formulae:

Linear Accuracy =        1_____

√∆E² + ∆N²

total distance

Linear Accuracy = 1 ÷ √ (0.010) ² + (-0.008) ² ÷ (3206.115)

Linear Accuracy = 1 ÷ √0.0004+0.0064 ÷ (3206.115)

Linear Accuracy = 1 ÷ √0.0065 ÷ (3206.115)

Linear Accuracy = 1 ÷ 0.0806225 ÷ (3206.115)

Linear Accuracy = 1 ÷ 0.000025146

Linear Accuracy = 1 ÷ 39766.962

Linear Accuracy = 1: 39,000

Expected Linear Accuracy = 1:3,000

Obtained Linear Accuracy= 1: 39,000

4.5	RESULT DISPLAY

This show the figure of horizontal alignment and the vertical alignment obtain during the processing of the result.

HORIZONTAL ALIGNMENT

VERTICAL ALIGNMENT

4.7	APPLICATION OF THE PRODUCT

- The horizontal alignment is used for correcting horizontal curves along the route during the construction work.

- Horizontal alignment is also used for location and allocation of the route facilities like, pipe lines, electricity line, setting out of drainages some case for plots alignments.

- Vertical alignment is used to correct for the vertical curves along the road during the construction.

CHAPTER FIVE

5.0	SUMMARY, PROBLEM ENCOUNTERED, CONCLUSION AND RECOMMENDATIONS

5.1	SUMMARY

The aim and purpose of this project was to carry out route survey from New IES to Ara village, Kwara State. We started with reconnaissance which involved office planning and reconnaissance. Reconnaissance entails total planning and usually visitation to study area. The instrument used was examined to make sure that it is in good working condition. The field operation includes carrying out a third order traverse, leveling and detailing. The raw data obtained from the field work were downloaded into a computer system using downloading cable and later process using software like ( AutoCAD CivilCAD). The process data were used for the production of digital plan using a suitable sale in order to appreciate the work done so far and to make the work understandable to others.

5.2	PROBLEM ENCOUNTERED

The following problem were encountered when project is going on

- The movement of the masses and vehicle from one place to another along the selected road for the project was a great problem which had an effect on the rate at which the operation was carried out.

- The batteries of the instrument (total station) easily got exhausted irrespective of the bar shown

- The villagers disturbing when work is going on that we have no access to set our instrument in front of their house

- The weather condition at times also had an effect in the rate at which the work was to be executed.

5.3	CONCLUSION

The goal of this project was successfully achieves in which all operation was carried out in accordance to survey rules and regulation and high data integrity was ensured. Moreover this project have broaden my knowledge in carrying out route survey within an area and a good background in surveying operation. The survey plan of selected location (New IES to Ara village) was finally produced showing the alignments and levels.

5.4	RECOMMENDATIONS

Below are my recommendations to the department of surveying and geo informatics and the school authority

- Survey instrument available in the school should be shown to the student, including the working principle to broaden their knowledge

- School authority should provide more updated surveying instrument in order to enable student to carry out their project accurately and on time.

- The instrument with accessories should be provided for the students when the time of need arise, this will be of great help to the students.

- The school authority should try to repair the unfunctioning instrument in the department to reduce the overused of the functioning ones.

REFERENCES

Agro R. (1984):	Surveying and leveling 4th Edition, Vol 1 London Pitman. Publisher limited.

Anderson (1985) Surveying basic principle pg. 2 ch 1

Barrister A and Raymond S. (1972) surveying great Britian

Opaleye .J. oluwafemi (2006):	Basic principle of geoinformatics press, 1990, vol. 8 page 341-348.

Ojelade kabir .A. (2010):	Final year, kwara state polytechnic Ilorin, Department of Surveying and Geoinformatics.

W.G. critain and R.F Lane (1960) classification of route surveying.

APPENDIX

| 6 | 682025.8 | 945084.8 | 99.413 |
| --- | --- | --- | --- |
| 7 | 682038.6 | 945093.7 | 100.5105 |
| 8 | 682050 | 945106.5 | 100.9379 |
| 9 | 682005.6 | 945096.7 | 99.1825 |
| 10 | 682006.7 | 945112.1 | 99.2829 |
| 11 | 682017.2 | 945093.9 | 99.7548 |
| 12 | 682021.9 | 945093.8 | 100.1744 |
| 13 | 682026.4 | 945093.5 | 100.3013 |
| 14 | 682042.6 | 945073.5 | 99.75 |
| 15 | 682044.2 | 945076.8 | 99.7942 |
| 16 | 682046.7 | 945079.7 | 99.873 |
| 17 | 682068.9 | 945060.8 | 99.905 |
| 18 | 682066 | 945056.8 | 99.7495 |
| 19 | 682071.3 | 945064.3 | 100.0481 |
| 20 | 682090.7 | 945043.6 | 100.3955 |
| 21 | 682092.8 | 945046.5 | 100.4931 |
| 22 | 682088 | 945040.3 | 100.3069 |
| 23 | 682112.8 | 945025.4 | 101.0093 |
| 24 | 682115.7 | 945028.5 | 101.0807 |
| 25 | 682128.3 | 945013.1 | 101.5195 |
| 26 | 682135.6 | 945005 | 101.6583 |
| 41 | 682135.6 | 945005 | 101.658 |
| 27 | 682128.2 | 945013.1 | 101.5286 |
| 28 | 682109.2 | 945022.7 | 100.7413 |
| 29 | 682131.3 | 945003.6 | 101.661 |
| 30 | 682128.6 | 945001.2 | 101.4917 |
| 31 | 682134.5 | 945006.1 | 101.5765 |
| 32 | 682147.2 | 944979.6 | 102.1656 |
| 33 | 682143.5 | 944977.3 | 101.9381 |
| 34 | 682150.4 | 944981.2 | 102.189 |
| 35 | 682163.3 | 944955.1 | 101.2801 |
| 36 | 682160.1 | 944952.7 | 101.0766 |
| 37 | 682166.8 | 944957.3 | 101.3514 |
| 38 | 682177.7 | 944931.9 | 100.9789 |
| 39 | 682174.2 | 944929.6 | 100.9376 |
| 40 | 682181.1 | 944934.2 | 101.1852 |
| 41 | 682192.5 | 944907.2 | 101.108 |
| 42 | 682189.5 | 944905.5 | 100.9958 |
| 43 | 682195.5 | 944909.2 | 101.0546 |
| 44 | 682208 | 944884.2 | 101.3188 |
| 45 | 682204.9 | 944881.9 | 101.278 |
| 46 | 682211.2 | 944886.6 | 101.4122 |
| 47 | 682221.6 | 944858.1 | 101.4747 |
| 48 | 682225 | 944860.4 | 101.5198 |
| 49 | 682228.4 | 944862.6 | 101.72 |
| 50 | 682238.2 | 944834.6 | 101.6794 |
| 51 | 682240.9 | 944836.6 | 101.844 |
| 52 | 682244.2 | 944838.6 | 101.9053 |
| 53 | 682254.6 | 944811.7 | 102.2803 |
| 54 | 682257.5 | 944813.9 | 102.2792 |
| 55 | 682260.7 | 944816.2 | 102.3919 |
| 56 | 682271 | 944788.3 | 102.3721 |
| 57 | 682273.9 | 944790.3 | 102.6997 |
| 58 | 682276.6 | 944792.1 | 102.5937 |
| 59 | 682286.6 | 944766.1 | 102.8219 |
| 60 | 682290 | 944768.2 | 103.0714 |
| 61 | 682304.6 | 944740.8 | 103.2686 |
| 62 | 682310.1 | 944732.5 | 103.3306 |
| 42 | 682304.6 | 944740.8 | 103.269 |
| 63 | 682310.1 | 944732.4 | 103.3666 |
| 64 | 682292.6 | 944772.5 | 103.0856 |
| 65 | 682304.2 | 944746.8 | 103.4408 |
| 66 | 682301.2 | 944744.5 | 103.1813 |
| 67 | 682323.2 | 944726.4 | 103.7364 |
| 68 | 682319.9 | 944724 | 103.6967 |
| 69 | 682316.1 | 944721.7 | 103.6322 |
| 70 | 682335.8 | 944700.8 | 104.2236 |
| 71 | 682338.5 | 944702.9 | 104.2273 |
| 72 | 682332.8 | 944698.3 | 104.3279 |
| 73 | 682354.8 | 944679.3 | 104.6155 |
| 74 | 682351.9 | 944677.2 | 104.6354 |
| 75 | 682348.3 | 944675.1 | 104.6815 |
| 76 | 682365.7 | 944652.9 | 105.0503 |
| 77 | 682362.4 | 944650.6 | 105.0144 |
| 78 | 682368.9 | 944655.1 | 105.0834 |
| 79 | 682384.6 | 944631.2 | 105.4746 |
| 80 | 682381.2 | 944629.1 | 105.561 |
| 81 | 682376.6 | 944629.5 | 105.4252 |
| 82 | 682396.8 | 944608.4 | 105.962 |
| 83 | 682393.7 | 944607.1 | 105.9661 |
| 84 | 682390.5 | 944605.4 | 105.9599 |
| 85 | 682408.2 | 944582.9 | 106.4976 |
| 86 | 682405 | 944581.3 | 106.4912 |
| 87 | 682411.2 | 944584.8 | 106.3901 |
| 88 | 682423.4 | 944559.5 | 106.6502 |
| 89 | 682420 | 944557.5 | 106.7922 |
| 90 | 682418.1 | 944555.9 | 106.9239 |
| 91 | 682422.8 | 944559.9 | 106.6742 |
| 92 | 682425.1 | 944554.6 | 106.769 |
| 43 | 682422.8 | 944559.9 | 106.674 |
| 44 | 682422.8 | 944559.9 | 106.674 |
| 93 | 682425.1 | 944554.6 | 106.77 |
| 94 | 682415.6 | 944557.5 | 106.7882 |
| 95 | 682427 | 944532.2 | 107.0686 |
| 96 | 682429.8 | 944533.7 | 107.1339 |
| 97 | 682433.4 | 944535.4 | 107.0119 |
| 98 | 682439.2 | 944506.3 | 107.4392 |
| 99 | 682443.4 | 944508 | 107.5352 |
| 100 | 682446.7 | 944509.7 | 107.4372 |
| 101 | 682452.5 | 944482.6 | 107.8221 |
| 102 | 682456.2 | 944484.7 | 107.9294 |
| 103 | 682459.4 | 944486.6 | 107.8592 |
| 104 | 682466 | 944457.9 | 108.3992 |
| 105 | 682469.3 | 944459.5 | 108.5506 |
| 106 | 682472 | 944461 | 108.3765 |
| 107 | 682482.1 | 944434.8 | 109.1427 |
| 108 | 682478.5 | 944433.1 | 108.9704 |
| 109 | 682493.7 | 944408.6 | 109.7479 |
| 110 | 682483.6 | 944436.4 | 109.12 |
| 111 | 682491.3 | 944407.3 | 109.6568 |
| 112 | 682496.4 | 944410.1 | 109.813 |
| 113 | 682505.9 | 944382.1 | 110.7567 |
| 114 | 682503.2 | 944380.5 | 110.7638 |
| 115 | 682508.3 | 944383.2 | 110.7571 |
| 116 | 682516.6 | 944355.3 | 111.6726 |
| 117 | 682519.4 | 944356.7 | 111.6934 |
| 118 | 682517 | 944354.3 | 111.5071 |
| 119 | 682519.7 | 944349.9 | 111.8667 |
| 45 | 682517 | 944354.3 | 111.507 |
| 120 | 682519.7 | 944349.9 | 111.6799 |
| 121 | 682522.9 | 944358.4 | 111.3804 |
| 122 | 682533.8 | 944331.7 | 112.5338 |
| 123 | 682536.4 | 944333.7 | 112.4887 |
| 124 | 682531 | 944330 | 112.386 |
| 125 | 682552.5 | 944310.3 | 113.4871 |
| 126 | 682550.3 | 944308.6 | 113.5288 |
| 127 | 682547.6 | 944306.6 | 113.4817 |
| 128 | 682570.5 | 944287.1 | 114.37 |
| 129 | 682568.1 | 944285.2 | 114.4096 |
| 130 | 682565.9 | 944283.4 | 114.3668 |
| 131 | 682585.8 | 944263.5 | 115.29 |
| 132 | 682588.3 | 944265.3 | 115.2021 |
| 133 | 682583.1 | 944261.3 | 115.1886 |
| 134 | 682603.6 | 944241.7 | 115.7531 |
| 135 | 682606.1 | 944243.8 | 115.689 |
| 136 | 682601.1 | 944239.8 | 115.6595 |
| 137 | 682623.9 | 944220.6 | 116.1817 |
| 138 | 682621.7 | 944218.8 | 116.2727 |
| 139 | 682619.1 | 944216.7 | 116.3477 |
| 140 | 682642.1 | 944197.2 | 116.7863 |
| 141 | 682639.2 | 944195.3 | 116.8587 |
| 142 | 682636.6 | 944192.8 | 116.901 |
| 143 | 682660.2 | 944173.6 | 117.3205 |
| 144 | 682656.6 | 944171.6 | 117.3986 |
| 145 | 682653.6 | 944169.5 | 117.3994 |
| 146 | 682676.2 | 944150.6 | 117.783 |
| 147 | 682672.9 | 944148.6 | 117.8805 |
| 148 | 682669.6 | 944146.3 | 117.8648 |
| 149 | 682692 | 944127.1 | 118.2677 |
| 150 | 682688.8 | 944125.1 | 118.3393 |
| 151 | 682685.9 | 944122.9 | 118.3408 |
| 152 | 682696.9 | 944119.8 | 118.3344 |
| 153 | 682694 | 944124.6 | 118.2354 |
| 46 | 682694 | 944124.6 | 118.235 |
| 154 | 682696.9 | 944119.7 | 118.3095 |
| 155 | 682686 | 944123.2 | 118.2222 |
| 156 | 682707.4 | 944102 | 118.6681 |
| 157 | 682704.4 | 944100.4 | 118.7399 |
| 158 | 682700.8 | 944098.3 | 118.6396 |
| 159 | 682722.3 | 944077.9 | 119.0533 |
| 160 | 682719.1 | 944076.2 | 119.1673 |
| 161 | 682715.5 | 944074.2 | 119.1063 |
| 162 | 682738.3 | 944053.4 | 119.2849 |
| 163 | 682734.5 | 944051.7 | 119.382 |
| 164 | 682730.4 | 944049.6 | 119.2995 |
| 165 | 682754.8 | 944030.1 | 119.211 |
| 166 | 682750.9 | 944027.5 | 119.3803 |
| 167 | 682746.9 | 944024.9 | 119.3085 |
| 168 | 682771 | 944006.4 | 119.1626 |
| 169 | 682767.1 | 944004 | 119.3029 |
| 170 | 682763.8 | 944002.1 | 119.2275 |
| 171 | 682788.1 | 943982.4 | 119.2347 |
| 172 | 682783.8 | 943979.7 | 119.4079 |
| 173 | 682779.2 | 943976.5 | 119.3832 |
| 174 | 682804.3 | 943957.2 | 119.419 |
| 175 | 682800.4 | 943954.9 | 119.5152 |
| 176 | 682797 | 943952.7 | 119.4804 |
| 177 | 682817.9 | 943931.1 | 119.7777 |
| 178 | 682821.5 | 943932.8 | 119.7828 |
| 179 | 682815.1 | 943928.8 | 119.8386 |
| 180 | 682836 | 943900.3 | 120.2448 |
| 181 | 682836.4 | 943900.4 | 120.2463 |
| 47 | 682836 | 943900.3 | 120.245 |
| 182 | 682835.2 | 943892.8 | 120.119 |
| 183 | 682808.9 | 943902.9 | 120.6629 |
| 184 | 682809.5 | 943905.2 | 120.6695 |
| 185 | 682810.3 | 943908.1 | 120.6672 |
| 186 | 682782.5 | 943911.9 | 121.2278 |
| 187 | 682783.9 | 943914.4 | 121.2087 |
| 188 | 682784.9 | 943917.2 | 121.1419 |
| 189 | 682756 | 943920.7 | 121.7873 |
| 190 | 682757 | 943923.8 | 121.8478 |
| 191 | 682757.9 | 943926.8 | 121.9166 |
| 192 | 682728.2 | 943930.3 | 122.4308 |
| 193 | 682729.4 | 943933.3 | 122.5375 |
| 194 | 682730.2 | 943935.5 | 122.5611 |
| 195 | 682701.6 | 943940.9 | 122.9612 |
| 196 | 682703.2 | 943944.1 | 123.152 |
| 197 | 682670.9 | 943956.3 | 123.9009 |
| 198 | 682676.8 | 943953 | 123.7493 |
| 48 | 682670.9 | 943956.3 | 123.9 |
| 199 | 682675.1 | 943954 | 123.8231 |
| 200 | 682676.8 | 943953.1 | 123.7431 |
| 201 | 682675.1 | 943954 | 123.8217 |
| 202 | 682676.5 | 943956.6 | 123.747 |
| 203 | 682678.7 | 943960.3 | 123.705 |
| 204 | 682652 | 943966.9 | 124.2516 |
| 205 | 682653.7 | 943970 | 124.3123 |
| 206 | 682655.3 | 943973.6 | 124.2712 |
| 207 | 682625.2 | 943986 | 124.9082 |
| 208 | 682623.3 | 943982.6 | 124.9048 |
| 209 | 682626.8 | 943989.1 | 124.8581 |
| 210 | 682595.6 | 944003.9 | 125.3075 |
| 211 | 682594.2 | 944000.8 | 125.4026 |
| 212 | 682597.1 | 944006.4 | 125.5699 |
| 213 | 682566.3 | 944022.7 | 125.9199 |
| 214 | 682564.3 | 944019 | 125.8905 |
| 215 | 682567.8 | 944025.2 | 125.8537 |
| 216 | 682538.4 | 944040.1 | 126.2323 |
| 217 | 682539.5 | 944042 | 126.1642 |
| 218 | 682536.6 | 944034.8 | 126.1717 |
| 219 | 682507.4 | 944050.1 | 126.4418 |
| 220 | 682508.3 | 944053 | 126.4105 |
| 221 | 682504.6 | 944054.2 | 126.4461 |
| 222 | 682501.7 | 944055 | 126.4166 |
| 49 | 682504.6 | 944054.2 | 126.446 |
| 223 | 682501.5 | 944055 | 126.4538 |
| 224 | 682506 | 944044.4 | 126.3143 |
| 225 | 682477.2 | 944056.5 | 126.7075 |
| 226 | 682478.5 | 944060.3 | 126.537 |
| 227 | 682476.6 | 944051.1 | 126.5581 |
| 228 | 682446.5 | 944062.1 | 126.7904 |
| 229 | 682445.7 | 944058 | 126.7217 |
| 230 | 682447 | 944066.2 | 126.7224 |
| 231 | 682415.8 | 944067.2 | 126.8935 |
| 232 | 682416.5 | 944072 | 126.7752 |
| 233 | 682415.1 | 944062.7 | 126.7175 |
| 234 | 682385.6 | 944073.1 | 126.9591 |
| 235 | 682387 | 944077.7 | 126.7497 |
| 236 | 682384.5 | 944068.3 | 126.7548 |
| 237 | 682354.7 | 944079.4 | 127.0357 |
| 238 | 682353.8 | 944074.8 | 126.9073 |
| 239 | 682323.4 | 944079.6 | 126.9917 |
| 240 | 682324.9 | 944084.8 | 127.165 |
| 241 | 682295.1 | 944095 | 126.8495 |
| 242 | 682294.4 | 944090.9 | 127.0306 |
| 243 | 682293 | 944086.2 | 126.8278 |
| 244 | 682261.1 | 944097.3 | 126.9411 |
| 245 | 682261.6 | 944102 | 126.8073 |