ROUTE SURVEY FROM NEW IES TO ARA VILLAGE

BY

YUSUF FAISAT

ND/23/SGI/FT/0016

A PROJECT SUBMITTED TO

THE DEPARTMENT OF SURVEYING AND GEOINFORMATICS

INSTITUTE OF ENVIRONMENTAL STUDIES

KWARA STATE POLYTECHNIC, ILORIN. KWARA STATE

IN PARTIAL FULFILMENT OF THE REQUIREMENT FOR THE AWARD OF NATIONAL DIPLOMA IN (ND) SURVEYING AND GEOINFORMATICS

JULY, 2025

CERTIFICATE

I hereby certify that the field work and information given in this project were obtained as a result of my observation and measurement and were carried out in accordance with survey laws and departmental instruction

_______________  				_______________

YUSUF FAISAT						Date

Student Name

CERTIFICATION

This is to certify that YUSUF FAISAT with matric number ND/23/SGI/FT/0016 from Department of Surveying and Geoinformatics, Institute of Environmental Studies carried out a practical field work which formed basic of the project in accordance with survey rules and regulation and departmental instruction

_______________  				_______________

SURV. AKINYEDE .A.O					DATE

SURV. BELLO .F.D

Project Supervisor

_______________  				_______________

DATE

Project Co-Ordinator

_______________  				_______________

MR ISAU A.I						DATE

Head Of Department

_______________  				_______________

External Examiner						DATE

DEDICATION

This project is dedicated to Almighty God and to my beloved parent MR and MRS YUSUF

ACKNOWLEDGEMENT

I give glory and adoration to Almighty God the creator of heaven and the earth, the first and the last who give me the opportunity to complete this program. I also give thanks to my lovely parent MR & MRS YUSUF for their maximum support towards achieving all these.

I had acquired in my life and the one am still looking up for, may Almighty God let them reap the fruit or their labour (amen).

I also acknowledge the support of my project supervisors in person of my project supervisor in person of SURV AKINYEDE & SURV BELLO DIRAN for their strictly and through supervision of this project. Also 	the HOD of my department MR ISAU A.I and my lectures SURV KABIR, SURV KAZEEM, MR AYUBA and those that that could not mention their name may Almighty God guide and bless your families (Amen).

My appreciation also goes to my lovely brother and sister’s for their absolute co-operation and wonderful support towards my career. Because without their reasonable influence and advise they do rendered tome. May almighty god bless them together (amen).

My piece of appreciation cannot be completed without the appreciating the support of all my villa mates and my friends both in school, at home and other different institutions for their maximum co-operation throughout my life. I thank you all, may almighty god be with you.

Finally my profound gratitude goes to my friends who gave me full support throughout my national diploma (ND) days. I say a big thanks to you all and wishing you all the best in our entire endeavour.

ABSTRACT

This project considered the route survey from New IES to Ara village. It involve planning and designing of survey field work like determination of center line of the road coordinators of the point and sectioning such that observation were made in the field which met the condition and specification of the third order survey using total survey (south) instrument Data obtained from the field were download into a computer system using south NTS software version for the data processing. The X, Y, Z coordinates of all point where obtain of which was used to produce a digital map of the showing the height differences between point along the Centre line of the road. Recommendations were also made based in the result obtained and the experience gathered in the excursion of the work.

TABLE CONTENT

TITLE PAGE								i

CERTIFICATION								ii

DEDICATION								iii

ACKNOWLEDGEMENT						iv

ABSTRACT								v

TABLE OF CONTENT							vi

LIST OF FIQURES							vii

LIST OF TABLES								viii

CHAPTER ONE

1.0	Introduction								1

1.1	Background of study						2

1.2	Statement of problem						4

1.3	Aim of study							5

1.4	Objective of study							5

1.5	Scope of study							6

1.6	Project specification						6

1.7	Personnel								7

1.8	Study Area								7

CHAPTER TWO

2.0	Literature review							8

CHAPTER THREE

3.0	Methodology							13

3.1	Reconnaissance							14

3.1.1	Data Acquisition (Data acquisition method)			15

3.1.1.1	Office Planning						16

3.1.1.1.1	Field Reconnaissance					16

3.2	Equipment used & system selection				16

3.3	Instrument test & configuration					17

3.4	Control check							18

3.5	Geometric data acquisition					18

3.5.2	Control check/Establishment					19

3.5.3	Traversing								19

3.5.4	Spot Heighting							19

3.6	Detailing								20

3.7	Attribute data							20

3.8	Data downloading and Editing					20

3.8.1	Data processing and Report analysis				21

3.8.2	Data bank								22

3.8.3	Data security							23

3.8.4	Data integrity							24

3.8.5	Data maintenance							24

CHAPTER FOUR

4.0	Analysis & Information presentation				25

4.1	Longitudinal section						27

4.2	Vertical section							29

4.3	Cross section							31

4.4	Application								38

CHAPTER FIVE

5.1	Summary, Conclusion and Recommendation	s		39

5.2	Summary								40

5.3	Conclusion								41

5.4	Recommendations							41

Reference

LIST OF FIGURES

Figure 1:	Recci diagram

Figure 2:	Map of the study area

LIST OF TABLES

Table 3.1:	Co-ordinate of given existing control point

Table 3.2:	Established Co-ordinate of control point

Table 3.3:	Showing new co-ordinate of control pillars

CHAPTER ONE

1.0	INTRODUCTION

1.1	BACKGROUND OF THE PROJECT

Road play major role in the development of a nation or a state, as it is popularly known as the most common means of communication. Roads are also the common way of transportation of goods and services and people from village, town and cities, road services as links between railways stations, air ports and seaports they are used for the spreading of social amenities to villages and towns, hence their location design and construction are of prime importance.

Route surveying is the determination of the part of a propose road in other to portray the relief of the path for office planning and design of such road by road engineers. The field work of this project was also proceeded by reconnaissance i.e a visit to the site in order to become familiarized with the site. Route surveying is one of the major aspects of engineering the surveyor is exposed to in this day-to-day activities. It is a vital tool which must not be neglected when an existing road is to be reconstructed or when a new road is required in an area eyibbin (2005)

Davils et al 1981 in their attempt to define route surveying stated that the expression “route surveying” used in a very general sense can be applied to the surveys required to establish horizontal and vertical alignment for transportation facilities which include the transport of people or goods an or by way of highways, railways, rapid transit guide ways canals pipelines and transmission lines. Every survey operations carried out for designing, redesigning and construction of engineering works, such as high ways roads, pipelines, canal and transmission lines e.t.c could in other words be called preliminary investigation survey hence the project involves the determination of the centre line of the proposed road beginning at chain age or 000km and terminates at chainage 1.5km

The route survey was carried out to provide preliminary spatial data for the design of the horizontal and vertical alignment, calculation of volume of cut and fill quantities of the road. Before any geographical location or transport networks come into existence, the rout survey must have been carried out. Route survey is an aspect of surveying made along a narrow belt or strip of territory for location, designing and eventual construction of any route of transportation such as highways, railways, canal, pipeline for water, sewage and transmission lines for power and telephone.

In the same vein, the preliminary survey of this kind of a project takes the form of a topographic survey. But it involves traversing and leveling along a centre-line with a view to acquiring 3D co-ordinates (x,y,z) not only for engineers to be able to perfectly plan for designing and construction of route but also for the client to appreciate visual interpretation of a proposed project. The importance of road to life and activities cannot be over emphasized roads have important role to play in the transportation system of the country.

Good transportation system through the pavement design has in the recent years provide to be of much significance commercially and other wise history testifies that the provision of roadways is necessary to draw a country out of a state of uncivilization, but that civilization is not attained until good road network between neigbours is made so easy. That local difference which branch narrowness is minimized. However this project has been designed strictly to modern design standards e.g. ASSHO method and it centers on the route survey and pavement design of the road to meet with the present day effective services. Moreover, this project work could really sever as a future guide in making a new rural road, down to industrial layout to connect mass communication. Therefore being in a school environment a good transportation system will improve the healthy studying condition of the new road pavement and maintenance of the existing one.

Route survey it involve in measuring and computing horizontal and vertical angles, elevation and horizontal distance the results of these surveys are used prepare detailed plan and profile base maps of proposed road ways. It the elevations determined in the survey serve as the basic for calculation of construction cut and fill quantities and in determining roadway banking. This section presents a review of basic terminology. Concepts and standard procedures used in high way surveys. The principle of mobility is of immense global concern to human and plant in such a way that the objective of mobility is achieved in most conducive manner. It is an idea that has a natural influence on activity of both plants and man that its effect has direct impact on life plant extend their root in search of nutrients and support, this is a form of mobility. Any obstruction in the course may result to life termination. This may invariably have an adverse effect on the environment.

This project is basically on route surveying. Route survey is a process in surveying that can be applied to establishment of horizontal and vertical alignment for transportation facilities these include: high ways, canals, pipelines, transmission lines and rapid transit. Means while to every movement, there must be means either by road, sea, air, tunnel or defined track or in each case any choice adopted is a function of resources at hand as well as the urgency of the desiring purpose. Movements were on legs in the olden days and this made the scope of movement very limited and hazardous.

1.2	STATEMENT OF PROBLEM

The route survey from New IES to Ara village at Moro local Government area, Kwara State has been discovered for long for having a bad outlook, narrow strip, poor drainage facility around poor design and not easy for transportation of goods from farm to market therefore for all this problem they should be a need for re-design of the project site.

1.3	AIM OF THE PROJECT

The aim of this project is to produce a standard design plan for their construction of the project site the design plan will involved the horizontal alignment, vertical alignment and longitudinal section of the project area.

1.4	OBJECTIVE OF THE PROJECT

The objectives of the project are summarized as the following

- Data acquisition

- Data processing

- Information presentation

- Comprehensive report writing

1.5	SCOPE OF THE PROJECT

- Recconaisances

- Office planning

- Field Recci

- Chainage marking at 25m & bench mark establishment at every 500m

- Test of instrument

- Geometric data acquisition

- Attribute data acquisition

- Data downloading editing

- Data processing analysis

- Information presentation both hard/soft copies

- Appendix

1.6	PROJECT SPECIFICATION

- A third order traverse must be carried out

- The control checks must be perfectly done

- The length of each traverse line must not be greater than 2.50 meters

- Horizontal length were approximated to the nearest three decimal places

- Determine of a new better horizontal alignment

- Chainage at every 25m

- Design vertical alignment

- Determine cross station area of the road

1.7	PERSONNEL

All the under list name is member of group 8 that participated in the successful execution of the project

| NAME | MATRIC NO | ROLE PLAYED |
| --- | --- | --- |
| Yusuf Faisat | ND/23/SGI/FT/0016 | Author |
| Adebunmi Yusuf Ayomide | ND/23/SGI/FT/0006 | Member |
| Laode Abdullahi Alabi | ND/23/SGI/FT/0007 | Member |
| Nurudeen Muhammed Jamiu | ND/23/SGI/FT/0010 | Member |
| Onimago Afisat Idowu | ND/23/SGI/FT/0013 | Member |
| Arewa Eniola Oluwaseun | ND/23/SGI/FT/0015 | Member |
| Nairabakan Khadijat Gbemisola | ND/23/SGI/FT/0012 | Member |
| Kehinde Rofiat Oluwabukola | ND/23/SGI/FT/0018 | Member |
| Ambrose Sunday | ND/22/SGI/FT/0024 | Member |

.

1.8	SITE LOCATION

The location of the project site is the route from New IES road to ARA village. The distance covered was approximately 3km. The site lies between

LATITUDE - 	008˚ 25’ 30’’ to 008˚ 57’ 33’’

LONGITUDE -	004˚ 35’ 23’’ to 004˚ 38’ 10’’

Fig 1.0: showing the study location

Source: Google earth 2025

CHAPTER TWO

2.0	LITERATURE REVIEW

The essence of route surveying was to establish the horizontal and vertical alignment for transportation which were assumed to comprises a network that include by way highways vapid transit guide way, canal, pipelines and transmission lines (Anderson. 1985)

Raymond et’al (1968) also defined route surveying construction of line and transportation or communication such as highways, railroads, canal, transmission lines, and pipelines, Raymond at all stated further that the location and construction survey may further consist of

- Establishing the centre line by setting interval

- Running level to determine the profile of the ground along the centre line at 25 meters internal

- Plotting such profile and fixing grades

- Taking cross sections

- Calculating volumes of earth work measuring drainage areas

- Lagging out structure, such as culverts and bridge

- Locating right of way boundaries

Surveying operations involves in route surveying include the following to determine the horizontal alignment and leveling for heighten purpose.

Generally, route survey consist of

- Establishing the centre line by setting at the interval

- Running level to determine the profile of the ground along the centre line at 25m interval.

American congress on surveying and mapping (ACSM) defined surveying as the science and art of making all essential measurement to determine the relative position of points or physical and cultural detail above, on or beneath cultural details above, on or beneath the surface of the earth and to depict them in a usable form or to establish the position of points or details

Anderson (1985), defined surveying as the art of measuring horizontal and vertical distance between lines and establishing points by predetermined angular and linear measurement.

These project is focus on engineering surveying which is defined as the surveying which is defined as the surveying which involved determining the position of natural and man made feature in the planning, design and construction or works.

Ramsey (1976) defined engineering surveying as the surveying carried out to provide special information for construction purpose which are usually drawn on a large scale, topographical surveys ranging in scale from 1:500 to 1:2,500. Such schemes include tunnel, transmission line by hydroelectric and alignment of road the location and construction of dams the development and maintenance of harbor with respect to this project, the principle in surveying operation were adopted for route alignment  which covered paining. Designing construction, developing and rehabilitation of road depending on the survey data acquired.

Banister (1977), defined leveling as the operation carried out for determination of height of point on the surface of the earth it is indispensable since the height of point can only be given relative to another point on the surface of the earth. Detailing is the act of fixing relevant feature on the field of observation such as electric poles, street light, and culvert and buildingsetc. Data presentation is the graphical representation of the field work at a suitable scale.

Route surveying, bearing and aspect of engineering work is a survey that start from design and ends after the construction stage and it could be explained as the survey carried out in establishing the horizontal and vertical alignment of a particular area needed for social utilities like highways, railways, transmission lines, pipelines and canal (Mikhail 1976). Hence, the principle and scope of surveying in general and route surveying in particularly is the comprehensive aim of the project.

MC MAURRY (2007),defined traversing as “the process of connecting the series of line with known bearing and distance” subdividing it into first order precise measurement and second order control for the establishment of secondary control while the third order for survey of tertiary control and topographical features.

Allan et al (2008) defined traverse as an orderly sequences of measurement of length and directions of line between points on the earth surface to determine the position of points. There fore, in leveling the measurement are taken only as the vertical plane.

Clark (2003), explained profile leveling as term used to described the field work involved in surveying a longitudinal section of a road to be surveyed. The profile leveling may be of any regular interval such as 25m which would be a continuous process until the closing station is reached.

CHAPTER THRRE

3.1 	METHODOLOGY

Methodology is a method and principles /way used to perform a particular activity and the activities is said to be successfully executed, proper planning and the development of the work. Plan show how good and objectives are to be accomplished. The proper planning that was taken for the successful execution of this project and this are involved

- the choice of the most appropriate techniques for execution of the project

- selection of equipment used.

- the design of a monitoring scheme that really helped in achieving the require accuracy for the project, starting from reconnaissance to the final product.

3.1 	RECONNAISSANCE

This is a very important and first aspect in any survey project /work carried out to obtain the general view of the study area in term of nature of the terrain and to properly plan the best way to the set aim and objectives of the project. And it can help to minimize any delays or changes in the observing schedule but for this it was carried out in two ways :

- Office reconnaissance

- Field reconnaissance

3.1.1 	OFFICE PLANNING

It is the act of involved the collection of equipment needed for the project, test of the instrument, missing planning assembling and studying of all available data relevant to the project, the costing of the operation was also done in the office. In executing the project, necessary information related to the project give such as coordinate of the control use were obtained from the SIWES and the practical unit of the school. Office planning is also known as office reconnaissance.

Table 3.1:  show the given bench marks Values

| Stn. I.D | Easting (m) | Northing (m) | Height (m) |
| --- | --- | --- | --- |
| BM1 | 682642.92mE | 944970.38mN | 347.21m |
| BM2 | 682626.44mE | 944965.70mN | 347.56m |
| BM3 | 682636.20mE | 944944.22mN | 348.01m |

3.2	EQUIPMENT USED

The following were the equipment used in the execution of the project

The equipment was grouped in to two categories

3.2.1 Hardware Equipment Used.

- Total station (South NTS 352L) and it's tripod

- one steel tape

- one plumb bob

- one cutlass

- two ranging poles

- paint

- field book

3.3.2 SOFTWARE USED

The software components used for this project are

- Sokkia link

- AUTOCAD civil 3D 2009

- Microsoft office 2010

- Notepad

- HP printer drivers

3.3	TEST OF INSTRUMENT

The instrument (total station) contain an inbuilt program that carries out self-test including signal test collimation, test battery, temperature test, then the instrument use set up and all their temporary adjustment were carried out, horizontal and vertical reading were taken and the telescope transmitted and the target bisected the second time and reading obtained were recorded below.

The two total stations used for this project, that is Leica TS02 and Sokkia set 630R total stations were tested to ensure that they were in good working condition and also to determine if the instrument meet up with the required accuracy of the project at hand which is a third order project. The total station equipment was set up over point (A) and then powered on all temporary station adjustment were carried out. The instrument menu was navigated to the instrument constant sub-menu which was selected after which the instrument then prompt to bisect the target. The target already set upon point (B) was bisected and enter key pressed, the instrument then prompted to bisect the same target again using the second face of the telescope which was done. After the whole observations, the instrument displayed the old and new values for the instrument constant error, the new value was set to replace the old value. The result and analysis are shown in the tables below.

Fig. 3.2 show the instrument set up for collimation test.

Table 3.2:	show the Test of Instrument result.

| Stn. | Sight | FFace | Horizontal circle reading | Deduced angle | Vertical circle reading | Sum |
| --- | --- | --- | --- | --- | --- | --- |
| X | Y | L | 2250 30’ 20’’ |  | 0890 32’ 12’’ |  |
|  | Y | R | 450 30’ 20’’ | 1800 00’ 10’’ | 2700 28’ 01’’ | 3600 00’ 13’’ |

Difference in horizontal circle reading = 1800  00’ 10’’

Horizontal collimation error = 1800  00’ 10’’ - 1800  00’ 00’’ = 000  00’  10’’

Collimation error = 000  00I  10II

2

Horizontal collimation error = 000  00I  05II

Sum of vertical circle reading = 3600 00’ 46’’

Vertical collimation error = 3600 00’ 46’’ - 3600 00’ 00’’

= 0000 00’ 13’’

2

= 000  00’  6.5’’

The above table shows the collimation errors and the new reading was used because it is similar to the old reading.

3.4	MONUMENTATION

This is the act of identify the selected and marking them with wooden pegs or nail /bottles cover during the execution of the project. This could be temporary or permanent, depending on the nature of the job specifically for the project. The wooden pegs or bottles cover were 3cm in diameter and above 15cm long with nail on top for easy identification of the point. These were driven into the ground leaving about 2cm above the ground.

Ground level

18cm

18cm

Beneath Ground level

75cm long

60cm

80cm Iron rod

Above Ground level

15cm

Figure 3:1: show the diagram T.B.M

3.5	CONTROL CHECK

Control check involves checking the control used for the execution of the project to know if they are still intact or not. The instrument was set up on a known point (TBM1) and all necessary temporary adjustment were made, the reflector was taken to another known point (TBM2), the angle difference between compute angle, measure angle, computed distance and measured distance are within allowable.

| Stn. I.D | Easting (m) | Northing (m) | Height (m) | Remark |
| --- | --- | --- | --- | --- |
| BM1 | 682642.92mE | 944970.38mN | 347.21m | Given |
| BM1 | 682642.82mE | 944970.31mN | 347.19m | observed |
|  | -0.100 | -0.07 | -0.03 | difference |
| BM2 | 682626.44mE | 944965.70mN | 347.56m | Given |
| BM2 | 682626.46mE | 944965.79mN | 347.62m | observed |
|  | +0.02 | +0.09 | +0.06 | difference |
| BM3 | 682636.20mE | 944944.22mN | 348.01m | Given |
| BM3 | 682636.10mE | 944944.12mN | 348.11m | observed |
|  | -0.10 | -0.01 | +0.10 | difference |

3.6	DATA ACQUISITION

The data for this project work (digital route surveying) were acquired using the total station and its accessories, the automated surveying instrument and their hardware components. The information acquired were for the existence of the natural and man-made features.

3.6.1	DATA SOURCE

Data source is also called a data file, it is simply a collection of records that stores data. Digital route surveying requires input of relevant digital data the data consist of two types;

- Attribute data (Descriptive Information)

- Spatial data representing geographical features (culvert, buildings, area, trees etc.)

There are two types of data source used in this project, i.e. primary and secondary data source

Primary data source is the type of data captured on the field with the use of total station (i.e. X, Y, Z) while secondary data source are the type deals with the coordinate of the control point used for the project

3.6.2	INSTRUMENT OPERATION

The procedure for the operation of instrument is this:

- Orientation of the instrument

- Level of the instrument with the three roof screw

- Rotation of both the vertical and horizontal axis of the instrument to 3600

- Press menu

- Press F1, it will immediately display all the files present

- Press F3 to enter new file name.

- After certifying the above, press enter

- It will take you to page 2

F1 -------- occupy print

F2 -------- back sight

F3 -------- forward sight

- Press F1 to input coordinate

N:

E:

Z:

F1 – input, F2 – back, F3 – space, F4 – enter

- After certifying, press enter, it immediately returns to page 2, then enter the back sight coordinate.

Below shows the result of TBM.

| Stn. I.D | Easting (m) | Northing (m) | Height (m) |
| --- | --- | --- | --- |
| BM1 | 682642.92mE | 944970.38mN | H=347.21m |
| BM2 | 682626.44mE | 944965.70mN | H=347.56m |
| BM3 | 682636.20mE | 944944.22mN | H=348.01m |
| BM4 | 682674.19mE | 945459.38mN | H=341.76m |
| BM5 | 682755.17mE | 945457.52mN | H=342.56m |
| BM6 | 682850.45mE | 945949.18mN | H=350.00m |
| BM7 | 682905.59mE | 945928.27mN | H=349.67m |
| BM8 | 682613.33mE | 944316.61mN | H=351.65m |
| BM9 | 682653.25mE | 944357.07mN | H=352.42m |

CHAPTER FOUR

4.0	DATA PROCESSING AND RESULT ANALYSIS

This are the mathematical process, which entails the computational procedure carried out on raw data collected from the field. These were used in producing the finished product known as plan/chart and it is usually done in the office hence it is called office computation various computation procedures carried out as follow. This stage also involved various computational process such as checks on both linear and angular disclosure of the observed data in order to ascertain if the job carried out is within the accuracy of third order job so as to meet the purpose for which the project was carried out.

4.1	DATA DOWNLOADING AND EDITING

Sokkia link software was used for downloading from Sokkia total station equipment following the procedure below;

- The total station was connected to the desktop computer

- The PC was reboot to establish a new COM port cI created a new project by clicking the new project and named it group five

- On the Sokkia link software, the port set up was clicked and the parameters was set to match to the instrument setting

- I then switched on the station and set the parameters boudrate 9600, data bits and stop bit s1, parity none

- I clicked on ok

- I clicked on send data on the instrument

- The data stored on the instrument was located.

- I clicked data on the menu bar of the software and it dropped down, I clicked receive data

- The connection was clicked on the software

- I then pressed the SEND button on the instrument simultaneously.

- The data was then copied from Sokkia link software to notepad for further processing

4.2	DATA PROCESSING

The downloaded data from the total station was further edited using Microsoft excel and notepad, the final edited copy was saved as the text file containing X, Y and Z coordinate of all points observed in the field.

4.3	RESULT ANALYSIS

The results were analyzed so as to check the accuracy of the job by comparing the results obtained with the minimum allowable error acceptable for this order of survey job in accordance with survey rules and departmental instructions.

4.4	DESIGN AND VOLUME COMPUTATION

The longitudinal profile of the route was created before the designing was done. The longitudinal profile was created using AutoCAD civil 3D land desktop companion 2009. The steps followed are as follows

- The AutoCAD civil 3D land desktop companion 2009 suite was a new project was created and named

- The unit icon was clicked to set the unit of the drawing as follows

- Length type decimal

- Length precision – three places (I.e. 0.000)

- Insertion scale

- Unit to scale inserted content- meters

- Angle type- Deg/Min/Sec.

- Angle precision – Od 00’00”

- The clockwise box below was clicked

- The direction icon below was clicked and a direction control dialog box was opened. The North was selected and Ok was clicked to exit the dialog box

- The point menu was then clicked in the menu bar and the point settings was selected. The “coords.” icon was clicked to select the method of data arrangements (i.e. Easting – Northing). The type of marker was also selected here.

- Under the point menu, the import/ export point icon was selected

- The import point was selected after which a dialog box opens the format in which the data was arranged was selected i.e. ENZ (space delimited). Also the source input data files earlier served and ok was clicked to continue

- The points were imported and the centre lines joined with the polyline drawing tool in AutoCAD toolbar, the OSNAP (object snap) was switched on so as to snap the polyline ends to the points to enable precise and easy joining

- Alignment was then selected from the menu bar and define from sub-menu was selected

- The polyline used in joining all the center line was then clicked, where the alignment was to start was also clicked. A dialog box then opens and the alignment was named.

- On the alignment menu on menu bar, the station label settings was clicked, the station label increment, station tick increment and station label offset were edited appropriately. The perpendicular labels and plus sign location was also clicked. Then the station labels were created.

- The terrain was created by clicking on the terrain menu on the menu bar, then “Terrain Model Explorer” was selected. A dialogue box opens up and a new surface was created and named. The points file was added and the arrangement for format selected as well as the file path where it was located. The surface was built by right clicking on it and select and the dialog box was closed after building the surface.

5.4	LINEAR ACCURACY CALCULATION

This was computed using the formulae:

Linear Accuracy =        1_____

√∆E² + ∆N²

total distance

Linear Accuracy = 1 ÷ √ (0.010) ² + (-0.008) ² ÷ (3206.115)

Linear Accuracy = 1 ÷ √0.0004+0.0064 ÷ (3206.115)

Linear Accuracy = 1 ÷ √0.0065 ÷ (3206.115)

Linear Accuracy = 1 ÷ 0.0806225 ÷ (3206.115)

Linear Accuracy = 1 ÷ 0.000025146

Linear Accuracy = 1 ÷ 39766.962

Linear Accuracy = 1: 39,000

Expected Linear Accuracy = 1:3,000

Obtained Linear Accuracy= 1: 39,000

4.5	RESULT DISPLAY

This show the figure of horizontal alignment and the vertical alignment obtain during the processing of the result.

HORIZONTAL ALIGNMENT

VERTICAL ALIGNMENT

4.7	APPLICATION OF THE PRODUCT

- The horizontal alignment is used for correcting horizontal curves along the route during the construction work.

- Horizontal alignment is also used for location and allocation of the route facilities like, pipe lines, electricity line, setting out of drainages some case for plots alignments.

- Vertical alignment is used to correct for the vertical curves along the road during the construction.

CHAPTER FIVE

SUMMARY, CONCLUSION AND RECOMMENDATIONS

5.0	SUMMARY

The aim of this project is to carryout the Route survey from New IES to Ara village, Ilorin it was done in accordance with the survey rules and regulation. A digital ground survey method was basically use for data acquisition. Reconnaissance was first stage (both office and field) and control was established around the study area and data acquisition was carried out both (Geometric and Attribute data) of the field, AutoCAD, suffer, Microsoft word, Microsoft Excel and notepad were combined carryout data processing. Manipulation, analysis and retriever. The final presentation of information was produced inform of digital map.

5.1	PROBLEM ENCOUNTER

i.	Pedestrian movement: The study area been a busy area, at every point in time people are moving around it obstruct/delay the measurement process of the total station. This problem was resolves by waiting for more time in such occasion

ii.	Intervisibility problem: Due to big structure of the buildings in the project area, it is not possible to pick most of  the details from one station point, hence we have to set in numerous points in order to details the structure effectively.

5.2	CONCLUSION

It is an indisputable fact, that to carry out project practical is to gain more knowledge after the execution of the project. I can therefore conclude that is was really interesting and also afforded me the opportunity of putting into practice various survey method and techniques that I have learnt in the course of my study. It was enhanced my confidence and ability to handle survey work in relation to any engineering work/project without any difficulty. The project was a success the accuracy of each task carried out was written the required limit

In conclusion despite all odds problems encountered aims of this project was achieved and was carried out in accordance with survey rules and specification and department instruction.

5.3	RECOMMENDATIONS

Most of the work carried out in this project are beneficial to the student. So I will recommend that this type of route project should always to given at ND level.

I would recommend that the school authority should educate the people living in the vanity of the institution and cabs/taxi driver on the reason why student were required to carry out such project to avoid embarrassment.

REFERENCE

Adeniyi, Gbenga (2007) Quick Access to AutoCAD plotting and surfer 7 for surveyor and engineers P.C2 17 and 18 (second Edition) department of surveying and Geo-informatics. Federal University of Technology Minna Nigeria.

Banister A Raymond “Surveying great British” chapter 7 PG 223 Vol2 No 4- New EditionStanley throns (publishers)Ltd – London (1972)

Engineering Surveying, Sixth edition byWilfred Sheffield mark breach

Gilani Charles D Wolf Paul R. (2008)elementary surveying and introduction to geometrics prentics page 226

Johnson Anthony, solvingStonehenge, the new key to an ancient enigma, (Thames and Hudson 2008)

Penn state surveying Program (2004) chapter 7 surveying engineering.

APPENDIX

006           682025.7819     945084.7846     99.4130

007           682038.6168     945093.6937     100.5105

008           682050.0134     945106.5497     100.9379

009           682005.6020     945096.7273     99.1825

010           682006.6927     945112.0727     99.2829

011           682017.1741     945093.9299     99.7548

012           682021.8908     945093.8083     100.1744

013           682026.3856     945093.4900     100.3013

014           682042.6028     945073.5034     99.7500

015           682044.2000     945076.7797     99.7942

016           682046.7200     945079.6649     99.8730

017           682068.9018     945060.7950     99.9050

018           682065.9868     945056.8481     99.7495

019           682071.3204     945064.2724     100.0481

020           682090.6688     945043.5891     100.3955

021           682092.7647     945046.5016     100.4931

022           682087.9850     945040.3302     100.3069

023           682112.7715     945025.3632     101.0093

024           682115.6615     945028.4587     101.0807

025           682128.2932     945013.0509     101.5195

026           682135.5597     945005.0302     101.6583

041           682135.5590     945005.0300     101.6580

027           682128.2465     945013.1023     101.5286

028           682109.1708     945022.7343     100.7413

029           682131.2552     945003.6180     101.6610

030           682128.6494     945001.2214     101.4917

031           682134.4793     945006.0582     101.5765

032           682147.1771     944979.5850     102.1656

033           682143.5417     944977.3202     101.9381

034           682150.3719     944981.1613     102.1890

035           682163.3264     944955.1111     101.2801

036           682160.0652     944952.7020     101.0766

037           682166.8240     944957.3415     101.3514

038           682177.6536     944931.8729     100.9789

039           682174.1631     944929.5568     100.9376

040           682181.1446     944934.1733     101.1852

041           682192.4558     944907.1810     101.1080

042           682189.4715     944905.5021     100.9958

043           682195.5252     944909.1830     101.0546

044           682207.9993     944884.1549     101.3188

045           682204.8675     944881.8900     101.2780

046           682211.1964     944886.6037     101.4122

047           682221.6193     944858.0633     101.4747

048           682225.0046     944860.4031     101.5198

049           682228.4063     944862.6411     101.7200

050           682238.2441     944834.5708     101.6794

051           682240.8990     944836.5946     101.8440

052           682244.1653     944838.6128     101.9053

053           682254.5578     944811.7444     102.2803

054           682257.5481     944813.9446     102.2792

055           682260.6788     944816.2335     102.3919

056           682270.9590     944788.3016     102.3721

057           682273.9218     944790.3492     102.6997

058           682276.6255     944792.1309     102.5937

059           682286.5631     944766.0962     102.8219

060           682290.0121     944768.1675     103.0714

061           682304.5657     944740.8423     103.2686

062           682310.0684     944732.5142     103.3306

042           682304.5660     944740.8420     103.2690

063           682310.0659     944732.4355     103.3666

064           682292.6382     944772.5410     103.0856

065           682304.1791     944746.8399     103.4408

066           682301.1611     944744.4851     103.1813

067           682323.2204     944726.4327     103.7364

068           682319.9030     944724.0170     103.6967

069           682316.1366     944721.6976     103.6322

070           682335.7669     944700.8243     104.2236

071           682338.4743     944702.8973     104.2273

072           682332.8044     944698.3401     104.3279

073           682354.7769     944679.3323     104.6155

074           682351.9003     944677.2367     104.6354

075           682348.3444     944675.0681     104.6815

076           682365.6697     944652.9487     105.0503

077           682362.4340     944650.6473     105.0144

078           682368.9170     944655.1217     105.0834

079           682384.5766     944631.1868     105.4746

080           682381.2040     944629.0851     105.5610

081           682376.5872     944629.4522     105.4252

082           682396.7736     944608.4408     105.9620

083           682393.6922     944607.1091     105.9661

084           682390.4633     944605.4499     105.9599

085           682408.1616     944582.9005     106.4976

086           682405.0317     944581.2927     106.4912

087           682411.2402     944584.7736     106.3901

088           682423.3518     944559.5384     106.6502

089           682420.0470     944557.5139     106.7922

090           682418.1394     944555.8639     106.9239

091           682422.8478     944559.8735     106.6742

092           682425.0908     944554.6325     106.7690

093           682425.1240     944554.5520     106.7700

094           682415.6326     944557.5332     106.7882

095           682426.9558     944532.2070     107.0686

096           682429.8125     944533.6945     107.1339

097           682433.4405     944535.4283     107.0119

098           682439.2492     944506.3200     107.4392

099           682443.3772     944507.9657     107.5352

100           682446.6725     944509.7071     107.4372

101           682452.5124     944482.5882     107.8221

102           682456.1946     944484.7199     107.9294

103           682459.4442     944486.5926     107.8592

104           682465.9673     944457.9182     108.3992

105           682469.2804     944459.5007     108.5506

106           682471.9873     944460.9821     108.3765

107           682482.1362     944434.8035     109.1427

108           682478.5189     944433.1004     108.9704

109           682493.6660     944408.6229     109.7479

110           682483.5502     944436.4111     109.1200

111           682491.2674     944407.2945     109.6568

112           682496.3899     944410.0650     109.8130

113           682505.9488     944382.1408     110.7567

114           682503.2152     944380.5186     110.7638

115           682508.2727     944383.2047     110.7571

116           682516.5588     944355.2714     111.6726

117           682519.4182     944356.6538     111.6934

118           682517.0492     944354.3153     111.5071

119           682519.6556     944349.9367     111.8667

120           682519.6555     944349.9380     111.6799

121           682522.9013     944358.3719     111.3804

122           682533.8035     944331.7136     112.5338

123           682536.4090     944333.6625     112.4887

124           682531.0073     944329.9981     112.3860

125           682552.4804     944310.3039     113.4871

126           682550.2624     944308.5650     113.5288

127           682547.5682     944306.6014     113.4817

128           682570.5465     944287.1465     114.3700

129           682568.1234     944285.1926     114.4096

130           682565.9303     944283.4345     114.3668

131           682585.8098     944263.4886     115.2900

132           682588.2739     944265.2842     115.2021

133           682583.1048     944261.2549     115.1886

134           682603.6117     944241.6747     115.7531

135           682606.1033     944243.8273     115.6890

136           682601.1192     944239.8216     115.6595

137           682623.9388     944220.6381     116.1817

138           682621.7366     944218.8165     116.2727

139           682619.1200     944216.6697     116.3477

140           682642.1326     944197.2154     116.7863

141           682639.2304     944195.2850     116.8587

142           682636.5766     944192.8337     116.9010

143           682660.2199     944173.5508     117.3205

144           682656.6375     944171.6155     117.3986

145           682653.6274     944169.5483     117.3994

146           682676.1782     944150.6322     117.7830

147           682672.9240     944148.5706     117.8805

148           682669.5635     944146.2802     117.8648

149           682692.0128     944127.1386     118.2677

150           682688.8167     944125.0635     118.3393

151           682685.8613     944122.9068     118.3408

152           682696.9263     944119.7571     118.3344

153           682693.9851     944124.6214     118.2354

154           682696.9401     944119.7337     118.3095

155           682685.9921     944123.1819     118.2222

156           682707.4102     944101.9903     118.6681

157           682704.4347     944100.3925     118.7399

158           682700.7536     944098.3053     118.6396

159           682722.3370     944077.8542     119.0533

160           682719.0745     944076.2476     119.1673

161           682715.4769     944074.1788     119.1063

162           682738.3179     944053.3904     119.2849

163           682734.5168     944051.6684     119.3820

164           682730.4204     944049.5512     119.2995

165           682754.8448     944030.0811     119.2110

166           682750.8643     944027.4873     119.3803

167           682746.9357     944024.9369     119.3085

168           682771.0366     944006.4412     119.1626

169           682767.0911     944003.9959     119.3029

170           682763.7668     944002.0564     119.2275

171           682788.0679     943982.3794     119.2347

172           682783.7592     943979.7363     119.4079

173           682779.1809     943976.4727     119.3832

174           682804.3369     943957.1898     119.4190

175           682800.3988     943954.8836     119.5152

176           682796.9980     943952.7007     119.4804

177           682817.8778     943931.1158     119.7777

178           682821.5153     943932.8212     119.7828

179           682815.0867     943928.7769     119.8386

180           682835.9649     943900.3001     120.2448

181           682836.4164     943900.3887     120.2463

182           682835.2443     943892.7511     120.1190

183           682808.8890     943902.8814     120.6629

184           682809.4877     943905.1762     120.6695

185           682810.3023     943908.1409     120.6672

186           682782.5130     943911.9335     121.2278

187           682783.9383     943914.3699     121.2087

188           682784.8617     943917.2296     121.1419

189           682756.0305     943920.6863     121.7873

190           682756.9581     943923.8256     121.8478

191           682757.9242     943926.7805     121.9166

192           682728.1606     943930.2789     122.4308

193           682729.3821     943933.2985     122.5375

194           682730.2432     943935.5292     122.5611

195           682701.6303     943940.8568     122.9612

196           682703.1730     943944.1081     123.1520

197           682670.9021     943956.3387     123.9009

198           682676.7594     943953.0441     123.7493

199           682675.0689     943953.9856     123.8231

200           682676.8112     943953.0878     123.7431

201           682675.1112     943953.9930     123.8217

202           682676.4723     943956.6324     123.7470

203           682678.6616     943960.3239     123.7050

204           682651.9756     943966.9381     124.2516

205           682653.6562     943970.0133     124.3123

206           682655.3298     943973.5623     124.2712

207           682625.1587     943985.9593     124.9082

208           682623.3161     943982.6262     124.9048

209           682626.8486     943989.0757     124.8581

210           682595.6088     944003.9467     125.3075

211           682594.1928     944000.8001     125.4026

212           682597.0679     944006.4356     125.5699

213           682566.3222     944022.6662     125.9199

218           682536.6247     944034.8150     126.1717

219           682507.3593     944050.1443     126.4418

220           682508.3018     944052.9566     126.4105

221           682504.6395     944054.2118     126.4461

222           682501.6857     944054.9793     126.4166

223           682501.5021     944055.0074     126.4538

224           682505.9551     944044.4381     126.3143

225           682477.2103     944056.4675     126.7075

226           682478.4825     944060.2897     126.5370

227           682476.6441     944051.0866     126.5581

228           682446.5247     944062.1256     126.7904

229           682445.7294     944057.9570     126.7217

230           682447.0142     944066.2102     126.7224

231           682415.7865     944067.1982     126.8935

232           682416.5107     944072.0172     126.7752

233           682415.1001     944062.6879     126.7175

234           682385.5776     944073.0709     126.9591

235           682386.9866     944077.7065     126.7497

236           682384.4882     944068.2747     126.7548

237           682354.7374     944079.3675     127.0357

238           682353.8090     944074.8395     126.9073

239           682323.3671     944079.6329     126.9917

240           682324.8908     944084.7548     127.1650

241           682295.1233     944094.9624     126.8495

242           682294.3700     944090.9218     127.0306

243           682292.9678     944086.1947     126.8278

244           682261.0979     944097.2825     126.9411

245           682261.5711     944101.9959     126.8073