DIGIGITAL SURVEY MODELLING

By

OGUNLADE MATTHEW KAYODE

HND/23/SGI/FT/0065

SUBMITIED TO

THE DEPARTMENT OF SURVEYING AND GEOINFORMATICS.

KWARA STATE POLYTECHNIC, ILORIN KWARA STATE.

IN PARTIAL FULFILLMENT OF THE REQUIREMENT

FOR AWARD OF HIGHER NATIONAL DIPLOMA (HND) IN

SURVEYING AND GEOINFORMATICS.

JULY, 2025

CERTIFICATE

I hereby certify that all field work and information contained in this project were obtained as a result of the observation and measurement carried out in the field by me and that the survey was executed in accordance with the survey laws, regulations and department instruction.

Name of student…………………………………

Matric No………………………………………..

Signature of student……………………………..

Date ………………………………………………

CERTIFICATION

This is to certify that Mr. OGUNLADE MATTHEW KAYODE With the Matric Number HND/23/SGI/FT/0065 has satisfactorily carried out survey duties contained therein in this project under my instruction and direct supervision.

…………………………………				……………………

MR. ABIMBOLA I. ISAU						DATE

(Project Supervisor)

…………………………………			        ……………………...

SUV. R.S AWOLEYE						DATE

(Project Coordinator)

…………………………………		                ……………………...

MR. ISAU ABIMBOLA						DATE

(Head of Department)

…………………………………		                ……………………...

(External Coordinator)						DATE

DEDICATION

The project is dedicated to Almighty God the giver of knowledge and understanding and to my parent MR. & MRS. OGUNLADE.

ACKNOWLEDGEMENTS

I acknowledge the love guidance and favour of God almighty for the successful completion of the programme

I also appreciate the professional of my supervisor MR. ISAU ABIMBOLA , during the course of writing this project, l shall forever be grateful to you

This acknowledge will not be complete if I did not appreciate the great work and effort of some personnel in great and noble institution; SURV. ISAU ABIMBOLA (H.O.D) Surveying and Geo-informatics

I am also grateful to all my lecture that made my stay in school inevitable and without forgetting those that contributed in several way towards the successful completion of my academic career in this institution and towards the complete of my project work most especially SURV. R.O  ASONIBARE and SURV A.G AREMU, SURV BELLO FELIX DIRAN,  SURV BANJI, SURV WILLIAMS KAZEEM, SURV BABATUNDE KABIR, SURV ABDULSALAM AYUBA

MY unreserved gratitude goes to my lovely mummy and daddy and my brother, sister, and my lovely Brother Ogunlade Issac for their immeasurable support financially, spiritually, love and advice towards the success of this programme may God in his infinite mercy be with you and strengthen you the more in Jesus name (Amen). I love you so much; you are such a wonderful family.

TABLE OF CONTENTS

Title page 											i

Certificate											ii

Certification 											iii

Dedication 											iv

Acknowledgements								            	v

Table of contents									    vi-viii

List of Tables									          ix

Abstract 											x

CHAPTER ONE: INTRODUCTION						         1-2

1.1 Background To The Project							          1-2

1.2 Aim And Objective Of The Project							  2

1.2.1 Aim Of The Project									  2

1.2.2 Objectives Of The Project								  2

1.3 Scope Of The Project									   3

1.4 Personnel											   3

1.5 Study Area										   4

CHAPTER TWO: LITERATURE REVIEW					          5-19

2.0 Basic Concept And Definition								  5

CHAPTER THREE: METHODOLOGY						       20-32

3.1 Office Planning										20

3.1.1 Field Reconnaissance									20

3.2 Instrument Used										20

3.3 Monumentation										21

3.4 Tests Of Differential Gps									22

### 3.5	Control Check										22

3.6 Data Source										23

- 3.7 Geometric Data Acquisition								24

3.7.1 Attribute/ Social Survey									29

- 3.8 Database Creation/Implement								30

3.8.1. Database Management System (Dbms)						31

3.8.2. Data Quality 										31

3.8.3 Data Integrity										31

3.8.4 Data Security										32

CHAPTER FOUR									       33-48

4.0 Data Processing And Presentation							33

4.1	Spatial Analysis									33

4.2        Spatial Query			33

4.3       Query Design			34

4.4       Testing Of Database			34

4.5         Existing Parcels And Blocks			35

4.6	Spatial Query										36

4.7	Discussion Of Result									40

4.8	Multiple Criteria Query								40

CHAPTER FIVE: SUMMARY, CONCLUSION AND RECOMMENDATIONS  49-51

5.1 Summary											49

5.2 Problems Encountered									49

5.3 Conclusion										50

5.4 Recommendations										51

References										     52-56

Appendix											67

LIST OF TABLES

Table 	1.1: Personnel………………………………………………………………………………57

Table 3.4: Control Check…………………………………………………………………………57

Table 5.1: Costing for Recci………………………………………………………………………57

Table 5.3: Costing for Traversing………………………………………………………………57

Table 5.4: Costing for Data Downloading………………………………………………………58

Table 5.5: Costing for Data Processing…………………………………………………………58

Table 5.6: Costing for Information presentation…………………………………………………59

ABSTRACT

The project utilizes digital survey modeling to create accurate 3D models of existing conditions, infrastructure, or sites. This involves collecting data through various surveying techniques and creating detailed digital models for analysis, planning, and design.

Key Components

1. Data Collection: Gathering data through methods  like detailing, topography  surveying and making use of Handheld GPS and Total station

2. Data Processing: Processing and analyzing collected data to create accurate 3D models.

3. Modeling: Creating detailed digital models of the surveyed area or object.

4. Analysis and Visualization: Using the digital model for analysis, visualization, and decision-making.

Applications

1. Infrastructure Planning: Designing and planning infrastructure projects like roads, bridges, and buildings.

2. Urban Planning: Creating 3D models of cities and towns for urban planning and development.

3. Environmental Monitoring: Monitoring environmental changes and tracking progress over time.

4. Construction and Engineering: Using digital models for construction planning, monitoring, and quality control.

Benefits

1. Improved Accuracy: Digital survey modeling provides accurate and detailed data for decision-making.

2. Enhanced Visualization: 3D models enable better visualization and understanding of complex data.

3. Increased Efficiency: Streamlined data collection and processing workflows.

4. Better Decision-Making: Accurate data and analysis support informed decision-making.

CHAPTER ONE

1.0 INTRODUCTION

1.1 BACKGROUND TO THE PROJECT

Terrain models have always been a topic of interest to military personnel, planners, landscape architects, civil engineers, as well as other experts in various earth sciences. Originally terrain models were physical models, made of clay, sand, and plastic. Since the 1950s, the computer has been introduced into this area and the modeling of the terrain surface has since then been carried out numerically or digitally leading to the current discipline – digital terrain modeling (Ezeomedo, 2019). There are many ways to model elevation but we're going to focus on three digital elevation datasets in particular:

DTM - Digital Terrain Model.

DEM - Digital Elevation Model.

DSM - Digital Surface Model.

The shape of the terrain is important in earth sciences. This makes it necessary for any scientific endeavor to have a thorough understanding of the terrain characteristics. Field-based techniques including triangulation, interpolation, extrapolation, and others have been used in a variety of ways historically. Advancements in technology, such as remote sensing techniques, Light Detection and Ranging (LiDAR) technology, and Geographic Information Systems (GIS), have made it possible to collect, analyze, and visualize data about the Earth's surface with high accuracy and detail. These technological developments have significantly contributed to the study of Digital Surface Modelling (Florinsky, 2016).

Digital Terrain Models, or DTMs, represent the bare earth surface without any features such as buildings or vegetation. DTMs are created using methods similar to DSMs, such as LiDAR or photogrammetry, but require additional processing to remove objects above the ground. This process is known as “bare-earth extraction” (Hirt, 2016).

A digital terrain model has three main components:

* The surface itself.

* The elevation at each point on the surface.

* The slope at each point on the surface.

This model is similar to DSM, but it represents only the highest points on the surface of the terrain instead of every point on its surface as DSM does. DTMs can be used to create contour lines. This is useful for building roads or railroad tracks because it allows you to see if there are any obstacles such as trees or rocks in your way before you start building them. DTMs are particularly useful for terrain analysis, such as slope and aspect calculations, hydrological modeling, and geological applications (Maxwell & Shobe, 2022).

Digital terrain modeling is a process to obtain desirable models of the land surface. Such models have found wide applications in various disciplines such as mapping, remote sensing, engineering, geology etc. Therefore, digital terrain modeling has become a discipline receiving increasing attention. DTM can be considered a synonym of bare-earth DEM. DTMs are often confused with DEMs. The main difference between the two models lies in the fact that the DEM generally takes into account all persistent objects on the ground (vegetation, buildings, and other artifacts)—while the DTM shows the development of the geodesic surface (Hengl & Evans, 2009).

A Digital Elevation Model, also known as a DEM, is a type of raster GIS layer. They are raster grids of the Earth’s surface referenced to the vertical datum—the surface of zero elevation to which heights are referred to by scientists, insurers, and geodesists. At most scales and environments, a generic term like DEM can be used because the differentiation between the bare earth and a surface object is not significant, with DEMs commonly having spatial resolutions of 20m or more. Digital elevation models (DEMs) play an important role in terrain-related applications (Liu et al., 2008).

Digital Elevation Models, or DEMs, represent the elevation of the earth’s surface. DEMs can be either a DSM or DTM. The resulting model shows the elevation of the earth’s surface. The DEM is essentially a superset of the DSM and DTM. DEMs are a 3D grid of terrain elevations collected from an area with satellite imagery or LiDAR technology. DEMs are useful for visualizing landscape changes over time, creating simulations of natural phenomena like landslides and floods, and measuring land use changes over time — as well as many other applications. They can also be combined with other models to create more detailed representations of the Earth's surface (Zhou, 2017).

A Digital Surface Model or DSM captures a surface—including natural and human-made structures such as vegetation and buildings. They illustrate reflective surfaces of all features elevated above the ‘bare earth’. In short, DSMs represent the Earth’s surface and all objects on it. DSM can also be provided from stereo digital aerial photography at various resolutions, depending on the quality and scale of the aerial photography. A digital surface model represents the elevation associated with the surface of the earth including topography and all natural or human-made features located on the surface of the earth. There is a variety of DSM source data available for developed areas and the suitability of these available data depends on the project specifications. In remote regions around the World, where little or no source data are available, the DSM can be produced automatically from stereo satellite scenes, from satellite sensors such as IKONOS, SPOT-5, and Terra-ASTER (El Garouani et al., 2014).

Digital Surface Models, or DSMs, represent the topography of the earth’s surface, including all objects on it, such as trees, buildings, and other features. DSMs are created using aerial data capture methods, like LiDAR (Light Detection and Ranging) or photogrammetry. LiDAR uses laser beams to measure the distance between the sensor and the ground, while photogrammetry uses aerial photographs to create 3D models. There are many benefits of DSMs. For one, they accurately represent the earth's surface, allowing users to measure features and make decisions based on that information. 3D visualization of the Earth's surface allows users to see the surface and features in a more realistic and immersive way. This is particularly useful in applications such as urban planning and design where visualizing the impact of planned developments on the existing landscape is critical. Digital surface models offer several advantages over traditional topographic mapping. Because DSMs are typically created using LiDAR or photogrammetry, they provide high-resolution data. This means that DSMs can capture much more detail than traditional topographic maps, often based on field surveys, and may not accurately capture all features. The high-res data also provides highly accurate elevation measurements. This allows for more precise calculations of slope, aspect, and other terrain characteristics, essential for industries that model water flow and drainage (Priestnall et al., 2000).

Digital Surface Models (DSMs) find wide-ranging applications, particularly in urban planning where they provide a comprehensive depiction of both the Earth's surface and its above-ground structures. These models play a crucial role in understanding and visualizing complex urban landscapes, aiding in decision-making processes as cities evolve over time. DSMs are especially valuable in scenarios such as runway approach zone management in aviation and assessing potential impacts on views from proposed constructions in urban settings. Additionally, DSMs serve diverse purposes including visualization, disaster management, navigation, vegetation monitoring, and facilitating informed decision-making across various domains.

1.2 STATEMENT OF PROBLEM

Part of Kwara State polytechnic lacks an up-to-date topographical information system (TIS). Without accurate data about the study area, and height of features above mean sea level, planning and design processes for this project may be based on incomplete or outdated information. Without Digital Surface Modelling, decision-making related to land development or resource exploration may be less informed and could result in negative consequences for the environment or existing structures.

1.3 AIM AND OBJECTIVES OF THE PROJECT

1.3.1 AIM OF THE PROJECT

The aim of the project is to produce DIGITAL SURFACE MODELING PROJECT, generate and analyze a high resolution digital surface model of the part of kwara state polytechnic, Ilorin , moro local government, Kwara State, Nigeria using GIS approach as well as demonstrating my capability in executing projects of such calibre.

1.3.2 OBJECTIVES OF THE PROJECT

In pursuit of the overarching aim mentioned earlier, the study involved a series of specific operations designed to achieve comprehensive insights into the study area. The specific objectives were as follows:

1. Reconnaissance

2. Data Acquisition.

a) Geometric (Locational) Data Acquisition (X, Y, and Z).

b) Attribute Data Acquisition (Features description).

3. Creation of Database for Spatial entities in the study area.

4. Information Representation and Management.

5. Spatial Analysis.1.5

1.4 SCOPE OF THE PROJECT

The scope of the project covers the following:

i. Project Planning.

ii. Data Acquisition with automated Survey equipment for perimeter traversing, detailing and spot heightens.

iii. Database Design and model.

iv. Data processing

v. Visualization, information presentation, and spatial analyses.

1.6 PERSONNEL

The following individuals are the personnel that will take part in executing this project:

| NAMES | MATRIC NUMBER | REMARK |
| --- | --- | --- |
| OGUNLADE MATHEW KAYODE | Hnd/23/sgi/ft/0065 | AUTHOR |
| SHUAIB OLUWATOYIN SHUAIB | Hnd/23/sgi/ft/0063 | MEMBER |
| ILESANMI EASTHER OLUWASEUN | Hnd/23/sgi/ft/0056 | MEMBER |
| BABALOLA OLUWATOSIN O. | Hnd/23/sgi/ft/0039 | MEMBER |
| ADEOYE AYOBAMI LAWRENCE | Hnd/23/sgi/ft/0064 | MEMBER |
| ABDGANIYU MISTURA DAMILOLA | Hnd/23/sgi/ft/0111 | MEMBER |
| AYINLA ABDWAHAB BOLAKALE | Hnd/22/Sgi/ft/0094 | MEMBER |

1.7 STUDY AREA

The site is part of Kwara State polythenic Kwara State, Nigeria. It is situated along Old Jebba Road and is approximate area of 4 hectares.

Figure 1.1:- MAP OF THE STUDY AREA

CHAPTER TWO

2.0 LITERATURE REVIEW

Digital surface modeling (DSM) using land surveying methods has become an essential tool in various fields such as civil engineering, environmental management, and urban planning. Land surveying techniques combined with modern digital technologies have enabled the accurate and efficient generation of DSMs for various applications. In this literature review, we will discuss the recent advancements and applications of DSM using land surveying methods.

Digital surface modeling (DSM) is a crucial aspect of modern land surveying methods as it provides detailed and accurate representations of the Earth's surface. This literature review aims to explore the techniques and methods used in digital surface modeling, particularly focusing on land surveying methodologies.

One of the key techniques in digital surface modeling is the use of LiDAR (Light Detection and Ranging) technology. LiDAR is a remote sensing technology that uses laser beams to measure distances to the Earth's surface. LiDAR data can be used to create high-resolution DSMs with detailed elevation information (Polewko et al., 2017).

Another commonly used technique in digital surface modeling is photogrammetry. Photogrammetry involves the use of aerial photographs to create accurate 3D models of the Earth's surface. By analyzing the overlapping images, photogrammetry software can generate detailed DSMs with precise elevation data (Fernandes et al., 2019).

Land surveying methods play a crucial role in the accurate collection of data for digital surface modeling. Traditional surveying techniques such as total station surveying and GPS (Global Positioning System) surveying are often used to collect ground control points for LiDAR and photogrammetry surveys (GSIS, 2020). These ground control points help to ensure the accuracy of the resulting DSMs.

In recent years, the integration of unmanned aerial vehicles (UAVs) into land surveying practices has revolutionized digital surface modeling. UAVs equipped with LiDAR or photogrammetry sensors can capture high-resolution data quickly and efficiently, making them ideal for creating detailed DSMs (Fann et al., 2018).

Overall, digital surface modeling using land surveying methods is a rapidly advancing field that continues to benefit from advancements in technology and methodologies. By combining traditional land surveying techniques with modern remote sensing technologies, researchers and practitioners can create highly accurate and detailed DSMs for a wide range of applications.

These elevation models are essential for creating accurate maps, topographic surveys, and aiding in infrastructure planning and design for projects like roads, bridges, and buildings). Elevation data is critical for a wide range of applications, and it’s collected through techniques like aerial data capture and LiDAR sensors, providing valuable information for urban planning, forestry, and natural resource management. Elevation data is critical for a wide range of applications in various fields. It also plays a key role in helping us better understand and manage the Earth’s resources and environment. The data is collected through various techniques, typically aerial data capture and LiDAR sensors (Lakshmi & Yarrakula, 2019).

Elevation data plays a crucial role across various domains, starting with mapping and surveying, where it enables the creation of precise maps and topographic surveys essential for urban planning, forestry, and natural resource management. In infrastructure planning and design, elevation data informs projects such as roads, bridges, and buildings by determining optimal routes, slopes, and elevations, ensuring their safety and structural integrity. Hydrology benefits from elevation data for modeling water flow and drainage, aiding in flood risk prediction, water resource management, and habitat protection. Additionally, elevation data supports environmental monitoring efforts by tracking changes in the Earth's surface over time, including erosion, land use alterations, and deforestation, thereby informing strategies to mitigate human-induced environmental impacts. Furthermore, in disaster response, elevation data assists in assessing damage caused by natural disasters, enabling emergency responders to plan relief efforts and allocate resources efficiently (Lakshmi & Yarrakula, 2019).

A digital terrain model is a topographic model of the bare earth –terrain relief - that can be manipulated by computer programs. The data files contain the spatial elevation data of the terrain in a digital format which is usually presented as a rectangular grid. Vegetation, buildings, and other man-made (artificial) features are removed digitally - leaving just the underlying terrain (on the other hand, Digital Surface Model (DSM) is usually the main product produced from photogrammetry, which contains all the features mentioned above, while a filtered DSM result in a DTM). Digital terrain models (DTMs) are of significant interest for applications such as environment planning, flood risk assessment, or building detection. DTM is considered as a continuous usually smooth surface which, in addition to height values (as DEMs) also contains other element that describes a topographic surface; slope, aspect, curvature, gradient, and others. A DTM can be derived from a DSM if the distinction between ground and non-ground pixels can be automated (Ouédraogo et al., 2014).

A digital terrain model (DTM) is a representation of surface relief encoded in a format that can be analyzed by computer. DTMs can be created using any data set which consists of several geographical coordinates with an elevation or quantity attached to each position. Data that have been represented in this way are topography, meteorological statistics (e.g. rainfall, evaporation, and temperature), and soil analysis. Other areas where the use of DTMs in data analysis could be useful are suggested. The process involved in creating the DTM is discussed. The DTM can then be analyzed by interpolation of contours at the desired scale, by interrogation at a specific point using the computer's cursor, or by producing a perspective view three-dimensional (3-D) drawing. Digital Terrain Model may be used with appropriate software to perform terrain analysis and to generate a variety of products (Forkuo, 2008).

DSMs measure the height values of the first surface on the ground. This includes terrain features, buildings, vegetation power lines, etc. DSM's therefore provide a topographic model of the earth's surface (Envirocheck.htm). This digital surface model represents the terrain elevations and surface elevation of the tree canopy and the top of the buildings. Over time, the methodologies used for DSM generation have experienced a dramatic improvement: from coarse to high-resolution elevation model generation with detailed building shapes if high-resolution data is used. Like Digital Terrain Models, Digital Surface Models contain the spatial elevation data of the terrain in digital format which is usually presented as a grid with natural and artificial features such as vegetation, buildings, etc (Zhou, 2017).

Digital surface models, or DSMs, are used for many different reasons in many different industries. One area in which they are especially useful is urban planning, where they provide a detailed depiction of the Earth's surface and its above-ground buildings, which helps with decision-making as cities develop. These models, which are frequently produced utilizing stereo photogrammetry or LiDAR (Light Detection and Ranging) technology, offer important insights into intricate urban environments, particularly as built-up areas change over time due to urban expansion. DSMs are essential for evaluating the encroachment of the runway approach zone in aviation as well as for examining the possible effects of proposed constructions on views in urban planning. DSMs are also used in decision-making processes, navigation, vegetation monitoring, disaster management, and visualization. Using LiDAR technology, for example, light pulses are emitted from a LiDAR unit, bounce off nearby objects, and then return to the sensor allowing distance measurements as well as surface geometry and material composition estimations. The generated DSMs provide detailed elevation data that can be used for a variety of applications. They capture changes in manmade buildings as well as natural features like tree canopies.

Digital Surface Model (DSM) data can be sourced from various methods, including topographic maps storing coordinate and elevation information of terrain features, remote sensing materials captured in stereo mode for generating DSMs and orthophotos, as well as laser scanning data producing point clouds. Additionally, ground-based photographs from multiple perspectives are necessary to construct photorealistic 3D models of buildings, complementing information derived from orthophotos and topographic maps, particularly for delineating building contours. DSMs find diverse applications, including urban and architectural planning, tourism, navigation systems, intelligent transport systems, urban risk forecasting, positioning of mobile phone transmitters for telecommunications, and flood risk mapping. A DSM comprehensively captures both natural and human-made environmental features, whereas a Digital Terrain Model (DTM) solely retains characteristics of bare-earth terrain, such as rivers and ridges. Importantly, a DTM can be derived from a DSM, but not vice versa (Zhou, 2017).

Digital Terrain Models represent the bare ground. DSMs include objects on the Earth’s surface, whereas DTMs do not. For most LiDAR applications, the focus is placed on the DEM and DSM as defined above, with DTMs more applicable for GIS and cartographic representations (El Garouani et al., 2014).

DTMs have been used to assist in chemical and physical analysis of soils. In this instance the x, y coordinate would be the geographic position of the soil sample and the z value would usually be some chemical concentration, physical property or pH at that point. Isolines for these properties can then be generated and displayed at the required contour interval (Al-Soghir et al., 2022).

The concept of Digital Elevation Models (DEMs) plays a crucial role in various applications, necessitating increased detail in horizontal sample spacing and vertical accuracy for precise measurements. DEMs are integral to Geographic Information Systems (GIS) and aid in visualizing landscapes and facilitating calculations for water volumes, soil erosion, and earth-moving quantities in construction projects. A wide range of applications is now drilling the requirement for increased details in Digital Elevation Models (DEMs). Details in this instance are defined by the horizontal sample spacing and vertical accuracy of the measurement. DEM is also an important utility of Geographic Information Systems (GIS). Using DEM/3D modeling, the landscape can be better visualized leading to a better understanding of certain relations in the landscape. Many relevant calculations, such as lakes and water volumes, Soil Erosion Volumes, quantities of earth to be moved for channels, dams, roads, embankments, etc (Isioye, 2011).

The generation of digital elevation models (DEMs) is a crucial aspect of topographic attribute derivation, relying on various data acquisition methods such as satellite imagery, digitizing contour lines, or ground surveys. The derivation of topographic attributes relies on digital elevation data sets that may be acquired from satellite imagery, digitizing the contour lines on topographic maps, or conducting ground surveys. Digital elevation data are typically compiled and stored in one of three data structures: (1) point elevation data on a regular grid, (2) point elevation data in triangulated irregular networks, and (3) digitized contour line data. A filtered DSM result to DTM and a DEM is considered the most important component of DTM (Menze et al., 2006).

DEMs are commonly built using data collected using remote sensing techniques, but they may also be built from land surveying. A DEM can be represented as a raster (a grid of squares, also known as a heightmap when representing elevation) or as a vector-based triangular irregular network (TIN). The TIN DEM dataset is also referred to as a primary (measured) DEM, whereas the Raster DEM is referred to as a secondary (computed) DEM. The DEM could be acquired through techniques such as photogrammetry, lidar, IfSAR or InSAR, land surveying, etc. Digital elevation models (DEMs) are very useful in solving various environmental problems such as planning and construction, hydrological and meteorological services, mining and oil industry, risk assessment, and hazard predictions, which makes their accurate production essential (Balasubramanian, 2017).

Airborne LiDAR technology offers advantages for DEM generation, particularly in urban areas, due to its shadow-free nature. However, challenges exist in filtering raw LiDAR data to extract bare earth points accurately. LiDAR technology continues to evolve rapidly, with manufacturers focusing on increasing laser pulse repetition rates to collect more data points efficiently. The competition among LiDAR sensor manufacturers drives advancements in sensor technology and data processing, leading to increased data density and improved data collection capabilities. High-density LiDAR data provide detailed terrain representations but pose challenges in data storage, processing, and manipulation due to the significant increase in data volume. Despite the challenges, LiDAR data have become more affordable, making them accessible for users in various applications (Lohani & Ghosh, 2017).

Mappers employ remote sensing techniques to prepare DEMs, with older methods involving interpolating digital contour maps or direct land surface surveys. While contour line data or sampled elevation datasets are not DEMs but rather digital terrain models, DEMs provide continuous elevation information across the study area. Mappers may prepare digital elevation models in a number of ways, but they frequently use remote sensing rather than direct survey data. Older methods of generating DEMs often involve interpolating digital contour maps that may have been produced by direct survey of the land surface. This method is still used in mountain areas, where interferometry is not always satisfactory. Contour line data or any other sampled elevation datasets (by GPS or ground survey) are not DEMs but may be considered digital terrain models. A DEM implies that elevation is available continuously at each location in the study area (Balasubramanian, 2017).

From the previously reviewed project, a lot of surveying method has been used to generate high resolution digital surface modeling, so we aim at achieving the same thing by employing the use of Dgps (Galasy G1), using land surveying method, using software like (Arc Scene 10.5, Arc Gis 10.5) to produce and visualize then after that we produce a short animation video that show the fly over of the displayed area.

CHAPTER THREE

3.0	METHODOLOGY

3.1	PROJECT PLANNING

# Before beginning any survey assignment, project planning is a crucial requirement. It entails a careful investigation of the topic matter about a certain land region that is being evaluated. This crucial step in the surveying process allows the surveyor to become acquainted with the chosen location before starting work. During the reconnaissance phase, the survey's goals, requirements, and required precision levels are examined because these elements have a big impact on the choice of survey tools and techniques. In the end, this project resulted in the development of a first working diagram. Reconnaissance is divided into two phases: (i) Office Planning and (ii) Field reconnaissance.

3.1.1	 FIELD RECONNAISSANCE

# As part of this process, a surveyor or student must physically visit the site to gain a thorough understanding of its topography, including any vegetation, accessibility, existing frame work, as well as the social and economic 19 activities that take place there. The optimal approach and tools for improved decision-making must also be chosen. This on-site visit gives the surveyor or student a comprehensive grasp of the entire region and allows them to look at the entire area of the land. To guarantee correct orientation, the locations of all accessible horizontal and vertical controls in the region of the research area will be visited, and their outlooks will be viewed. This exercise makes it easier to create a Recce diagram, which is shown below but is not drawn to scale:

Figure 3.4: RECCI DIAGRAM

3.1.2	 OFFICE PLANNING

# Gathering crucial project information is the focus of this phase, which is usually done in an office setting. It is the initial phase of planning that comes before the fieldwork itself and entails gathering and analyzing a variety of project-related data. This covers a range of requirements for the project, scheduling, budgeting, and generating the coordinates for the control points under the project control prefix. It includes figuring out the best process, picking the right tools, as well as subscription for the CORS station.

3.2 	EQUIPMENT USED/ SYSTEM SELECTION SOFTWARE

The following equipment was used:

- Total station and accessories

- One Tripod

- Two Prism

- One 7.5 meters short tape

- One Plumb bob

- One Cutlass

- Bottle corks and nails

- Writing materials.

3.2.1 	HARDWARE USED

Model: 			HP 15 Note book PC

System Manufacturer:		Hewlett-Packard

Rating: 			1.0 Windows Experience Index.

Processor: 			Intel(R) Core(TM) i5-3230M CPU @ 2.60GHz, 2601 Mhz, 2 Core(s), 4 Logical Processor(s)

Installed Memory RAM: 	8.00 GB

System Type: 			x64-based PC.

Product ID: 			F9F84EA#ABV

3.2.2 	SOFTWARE USED

Some of the software used includes:

- AutoCAD 2007

- Notepad

- Microsoft Word 2007

- Microsoft Excel 2007

- Arc GIS 10.5

3.3	 DATA ACQUISITION

# This involves using different surveying methods and tools to gather data and make observations in the field. To acquire data for this project, handheld GPS and Total station were both used.

# 3.3.1 	PRIMARY DATA SOURCE: This is a type of data source in which data is direct acquired from the field such as data acquired from field survey operations.

# 3.3.2 	SECONDARY DATA SOURCE: This data source are already made data or gotten from research work, which has been acquired for future use. This involves downloading satellite imagery used for the project area and Google map as guide.

3.4	TEST OF INSTRUMENT

Testing of instruments is very crucial when carrying out any survey operation. Having collected the required instruments from the department, the check was carried out on the accuracy and precision of both the handheldl GPS and TOTAL STATION to ensure they were in proper working condition before the actual field observation of the project.

3.4.1	 TEST FOR TOTAL STATION

The horizontal collimation and vertical index inaccuracies of the instrument were confirmed. Once the device was positioned on a rather flat surface, the required temporary modifications, including focusing, centering, and setting, were made. Configuration information, such as extra parameters and fast settings, was examined and adjusted as necessary. The configuration parameters selected collimation, namely horizontal collimation. Next, to focus on a distant object, the means' option located on the left side of the device was chosen. The instrument was transited, and the right side went through the same procedure. Then, when the 'accept' button was pushed, the table that was displayed showed the old and new values for the vertical index errors and horizontal collimation. Re-press "ok" signified acceptance of the updated values, which were minimal.

Figure 3.5: INSTRUMENT TEST

Top of Form

The diagrammatic representation of the procedure and the readings obtained are shown below

Table 3.4.1 COLLIMATION TEST OF TOTAL STATION

| STN | SIGHT | FACE | HCR | REDUCTION | VCR | REDUCTION |
| --- | --- | --- | --- | --- | --- | --- |
| A | B | L | 126º 07’ 11.33” |  | 67º 22’40” |  |
|  | B | R | 306º 07’ 31.3” | 180º 00’ 20” | 292º 37’ 1” | 359 º 59’ 41” |

HORIZONTAL COLLIMATION ERROR = 180°00’20’’ - 180°

=00°00’20’’

=00°00’20’’/2   =00°00’10’’

VERTICAL INDEX ERROR=360° - 359°59’41.6’’

=00°00’18.4’’

=00°00’18’’/2   =00°00’9.2’’

The instrument's satisfactory operation was validated by the acceptance of the updated readings. It appears from this result that any possible issues can be resolved or at least greatly reduced.

3.4.2 REFERENCING AND ORIENTATION OF TOTAL STATION

Tripod Setup:

- Place the tripod over the station point (known or assumed).

- Level the tripod head as much as possible by adjusting the legs.

Mount the Total Station:

- Fix the total station onto the tripod head.

- Ensure it’s securely locked in place.

Centering and Leveling:

- Use the optical plummet or laser plummet to center the instrument exactly over the survey station mark (station point).

- Level the instrument using the foot screws and circular bubble.

- Fine-tune leveling using the electronic level display.

Power On and Instrument Configuration

- Switch on the Total Station.

- Set the correct project, coordinate system, and units.

- Select the appropriate survey mode (e.g., topographic survey or station setup).

Orientation (Station Setup)

Orientation means establishing the direction of the horizontal angle reference (back sight direction).

Orientation by Known Coordinates (Resection or Free Station)

Used when both the instrument station and a back sight point have known coordinates.

- Input the coordinates of the instrument station (or let the instrument compute it later).

- Sight and measure to one or more known reference points (minimum of 2 for better accuracy).

- The Total Station calculates the instrument position and orientation using these known points.

3.3.7 PERIMETER TRAVERSING

The perimeter of the project area was traversed for the purpose of the coordinating the boundary points. DGPS (GALAXY G1) was connected to kwara state CORS station for referencing and orientation after which all the boundary points were traversed using DGPS from the initial until each successive boundary is occupied.

### 3.3.8	DETAILING

All features such as roads, buildings, electric poles, street light, trees, etc. were captured by coordinating their edges simultaneously as the perimeter is being measured within the project area. Buildings were measured and identifiers (BD) were used to differentiate them from other features. Roads were also measured and labelled (RD) accordingly. Data captured here were basically in 3Dimension i.e. Easting, Nothings and Elevations (X, Y, and Z) coordinates.

3.3.9	REMOTE ELEVATION MEASUREMENT

All the features such as buildings, electric poles, street light, etc. captured by coordinating their edges from a subsidiary traverse stations created within the project area. Buildings heights were measured and recorded. Electric poles height were also measured and recorded accordingly. So also the same was repeated for the trees. Data captured here were basically in 3Dimension i.e. Easting, Nothings and Elevations + Heights (X, Y and Z + H) coordinates. The following procedures were followed:

- Escape key was pressed then F1 for MENU

- The REM programme was selected.

- Observation was made again and the height of reflector was displayed.

- The telescope was tilted to the top of the object and the height keeps changing until F4 is been pressed to stop.

- The value was recorded.

Figure 3.3.9: REM OBSERVATION STRATEGY

3.3.9A 	SPOT HEIGHTINGS

This was carried out by occupying randomly the subsidiary points within the project area and taking measurements (Easting, Nothings and Heights) along the terrain at approximate intervals to determine the difference in height between points and some areas within the project location. Spot heighten is very important in producing topographical information of the area with the aid of ArcGIS 10.5.

3.4	PHYSICAL DESIGN

At this stage, all geospatial (attribute) data were structured and organized to form a database in a format acceptable by the implementing software and hardware. This was done in a way that, stored information can be accessed and retrieved at any time. Also, for regular updates when the need arise and to allow for analytical functions to be carried out answering generic questions. Also, it provided integrity and security that must be obeyed before such data could be accepted into the records. These virtues are as show in the tables below. ArcGIS 10.5 was used for database creation of the project area.

Table 3.4A TREE TABLE

| ATTRIBUTE | DATA TYPE | WIDTH | DECIMAL |
| --- | --- | --- | --- |
| T_ ID | String | 5 | - |
| T_NAME | Float | 20 | - |
| EASTING | Number | 10 | 3 |
| NORTHING | Number | 10 | 3 |
| T_USE | Float | 10 | - |
| T_HEGHT | Number | 7 | 3 |

Table 3.4B ROAD TABLE

| ATTRIBUTE | DATA TYPE | WIDTH | DECIMAL |
| --- | --- | --- | --- |
| POINT_ID | String | 10 | - |
| NORTHING | Number | 10 | 3 |
| EASTING | Number | 10 | 3 |
| RD_HEIGHT | Number | 7 | 3 |

Table 3.4C ELECTRIC POLE TABLE

| ATTRIBUTE NAME | DESCRIPTION OF ATTRIBUTE | DATA TYPE | WIDTH | DECIMAL |
| --- | --- | --- | --- | --- |
| POINT_ID | Electric pole identifier | Number | 6 | - |
| NORTHING | Coordinate | Number | 15 | 3 |
| EASTING | Coordinate | Number | 15 | 3 |
| EP_TYPE | Type of Electric pole | Text | 15 | - |
| EP_HEIGHT | Height of the Electric pole | Number | 5 | 2 |

Table 3.4D BUILDING TABLE

| ATTRIBUTE | DATA TYPE | WIDTH | DECIMAL |
| --- | --- | --- | --- |
| P_ID | String | 15 | - |
| NAME | Text | 15 | - |
| CONDITION | Text | 15 | - |
| USE | Text | 15 | - |
| HEIGHT | Number | 15 | 2 |
| AREA_METERS | Number | 15 | 2 |

Table 3.4E STREETLIGHT TABLE

| ATTRIBUTE NAME | DESCRIPTION OF ATTRIBUTE | DATA TYPE | WIDTH | DECIMAL |
| --- | --- | --- | --- | --- |
| POINT_ID | Street light identifier | Number | 6 | - |
| NORTHING | Coordinate | Number | 15 | 3 |
| EASTING | Coordinate | Number | 15 | 3 |
| SL_HEIGHT | Height of the street light | Number | 5 | 2 |

3.5	DATABASE CREATION

Database is an organized integrated collection of data stored so as to be capable of use by relevant application with data being accessed by different logical part. This was the construction phase where database was created. After the table has being populated via the keyboard, some attributes such as area, perimeters of parcel were automatically displayed by special command in the ArcGIS 10.5 version.

Figure 3.5: ABSTRACTED OF TABLE CREATED

3.5.1 ATTRIBUTE DATA ACQUISITION

The data about the characteristics of features especially man-made features were acquired through oral interview method and online source (Google Earth). Social survey was carried out during which information such as; building name, building use and related data about features were obtained from the students and staffs within the school. All these data formed the attribute aspect of the feature necessary for the database creation.

3.6	DATA PROCESSING

3.6.1	TOTAL STATION DATA DOWNLOADING PROCEDURE

Data downloading is the process of transferring data from the instrument into the computer system connected through the connection cable or wireless such as Bluetooth with the help of software in the computer, the process is as follows as performed on Total station.

- Connection of the two devices (computer and the instrument).

- Go to project working folder created on the instrument.

- Click on export.

- Input the name of file

- Indicate the file extension for the output

- Permit the other device to receive the file.

3.6.2 DRAFTING AND PLOTTING

The script file created containing the refined coordinates was then opened in the AutoCAD environment and the boundary of the project area was plotted using AutoCAD 2007.  This was followed by the details, which included buildings, roads, trees, sport facilities and electric poles.

Figure 3.6.2: AUTOCAD PLOTTING.

3.7 	DATABASE MANAGEMENT

3.7.1 	DATA SECURITY

This is the security measure used the protection of the database security is very important since it is vital for data integrity. The strategic measure used included controlling access to database by use of password.

3.7.2 DATA INTEGRITY

This ensures that data in the database is accurate and that cases of violation of integrity can be detected automatically by the system. Here, care was taken while entering data into the system, such that, accurate data were entered and updating could be done accurately without tampering with existing data. Backups are provided to cater for any loss of data through system failure. Also a member of the project group was appointed for quality assurance and checks therefor do checks the consistency, accuracy and integrity of the input data.

3.8.3 DATABASE MAINTANANCE

Having created the database, proper maintenance practice was made to meet its stated objectives. The ability to include more data and remove irrelevant data can be possible. There is every need for the data to be updated regularly because of the physical changes that tends to occur on the landscape with time, even as a result of development.

Both security and integrity were also exercised to ensure maintenance and to meet its stated objectives.

CHAPTER FOUR

4.0	SPATIAL ANALYSIS AND INFORMATION PRESENTATION

4.1     TESTING OF DATABASE

This is the test carried out to determine whether the relationship between the spatial data of a feature and their attributes is capable of being retrieved. This is necessary to ascertain the quality of data in the database, its reliability to satisfy the demand of the user. The spatial data were arranged logically in an organized manner with respect to their attributes. This was carried out by designing a sample query and running the query to see if the desired result would be achieved. In this project, the database was queried to show different categories of information levels in the project area. Hence the database was confirmed fit for analysis. A sample of the attribute table in the database design can be found on the next page.

Figure 4.1: SAMPLE OF ATTRIBUTE TABLE

4.2	SPATIAL QUERY

Queries were designed for the purpose of retrieving information from the database. The queries performed in this project gave answers to certain generic questions asked from the database. This was made possible as a result of the implicit link of both the spatial and attributes data. The queries were based on the products from the analysis carried out on the database.

4.2.1 SINGLE CRITERIA QUERY

A single criterion is carried out where one condition is used to design query. This condition is used to retrieve the information from the database.

QUERY ANALYSIS 1 (selection by attribute)

ANALYSIS ONE

Analysis Name: Database extraction

Analysis Type: Single criteria analysis

Syntax: Query analyses showing buildings that are used for Administrative.

Figure 4.2.1.1 RESULT OF QUERY: (BLD_FUNCTION = “ADMINISTRATIVE”).

Analysis Name: Database extraction

Analysis Type: Single criteria analysis

Syntax: “BLD_ Function” =”Agricultural”

Figure 4.2.1.2: RESULT OF QUERY: (BLD_FUNCTION = “AGRICULTURAL”).

ANALYSIS THREE

Analysis Name: Database extraction

Analysis Type: Single criteria analysis

Syntax: “BLD_ CODITION” =”ACADEMIC”

Figure 4.2.1.3: QUERY BLD_FUNCTION"ACADEMIC"

ANALYSIS FOUR

Analysis Name: Database extraction

Analysis Type: Single criteria analysis

Syntax: “TREE_FUNCTION= ORNAMENT”

Figure 4.2.1.4: RESULT OF QUERY: (TREE_FUNCTION “ORNAMENT”).

4.2.2	MULTIPLE CRITERIA

This is a situation where two or more conditions are used to design a query. The conditions determine the information that may be requested by the user from the database.

Query for QUERY: TREE_FUNCTION “ORNAMENT” AND TREE_TYPE “HARD”

ANALYSIS FOUR

Analysis Name: Database extraction

Analysis Type: Multiple criteria analysis

Figure 4.2.2.1 RESULT OF QUERY: TREE_FUNCTION “ORNAMENT” AND TREE_TYPE “HARD”

Query for BLD_FUNCTION =“RELIGIOUS” AND BLD_STATUS= “AVERAGE”

ANALYSIS FOUR

Analysis Name: Database extraction

Analysis Type: Multiple criteria analysis

Figure 4.2.2.2 RESULT OF QUERY: BLD_FUNCTION =“RELIGIOUS” AND BLD_STATUS= “AVERAGE”

4.2	SPATIAL ANALIYSIS

This makes Geographic Information System (GIS) a unique analytical tool or technique because of its ability to combine spatial attribute data to produce information relevant for decision making. The basic GIS analysis performed in this project includes overlay and topographic operations. Maps on various themes were overlaid since they were from the same geometric origin.

4.2.1	OVERLAY OPERATION

This overlay operation is the presentation that shows different features within the project area. The outcome of overlay operation could be composite plan if it shows all the existing features, and boundary.

Figure 4.2.1: COMPOSITE MAP OF THE PROJECT AREA.

4.2.2	TOPOGRAPHICAL OPERATIONS

Topographic operations were carried out to determine the surface characteristic of the area which showed the relief of the project. These operations were performed on the Digital Elevation Model (DEM) generated from the use of relevant software, in this case, ArcGIS 10.5 and ArcScene 10.5 was used. The following maps were produced from the topographic operations and analysis:

- TRIANGULATION IRREGULAR NETWORK (TIN): this shows series of triangulated surface automatically generated from ArcGIS 10.5 using 3D analysis tools. This is applicable in finding suitable place for building or any engineering construction

Figure 4.2.2.1: TIN AND DETAILS OVERLAY MAP OF THE PROJECT AREA

- SLOPE MAP: A slope map represents the gradient or steepness of the terrain's surface. It calculates the rate of change in elevation between neighboring pixels or points on a digital elevation model (DEM). Typically, slope is measured in degrees or percentage. Steeper slopes are represented by higher values, while flatter areas have lower values.

Figure 4.2.2.2: SLOPE MAP OF THE PROJECT AREA

- THE ASPECT MAP:

This shows the direction of the surface faces. It is used to determine how much sun a hill received and the information can be used to put building where they will get enough sun. Also the aspect map shows the suitable place for plane landing i.e. the flat terrain area and also solar illumination which can affect the diversity of life i.e. Vegetation and soil studies; glaciology.

Figure 4.2.2.3: ASPECT MAP OF THE PROJECT AREA.

- THE CONTOUR MAP:

Contour maps display lines connecting points of equal elevation on the terrain surface. These lines, called contour lines, depict the shape and elevation of the land. Contour maps are widely used in topographic mapping and land surveying for visualizing elevation changes and terrain features. This provides a very intuitive view of the landscape. Contour map of the project area can aid in giving information on the highest and the lowest point on the terrain. This is important in road construction and location of other facilities like storage tanks for water distribution.

Figure 4.2.2.4: CONTOUR MAP OF THE PROJECT AREA.

- FLOW DIRECTION MAP:

This is used to determine the direction of flow of water within the project area. It also indicates the magnitude and erosive force water passing through an area.

Figure 4.2.2.5 FLOW_DIRECTION MAP OF THE PROJECT AREA

- DIGITAL SURFACE MODEL MAP:

A DSM (Digital Surface Model) map, or Digital Surface Model, represents the elevation of the Earth's surface including all objects on it, such as buildings, vegetation, and other structures. Unlike a DTM (Digital Terrain Model), which only includes the bare-earth surface, a DSM incorporates both natural and man-made features. DSMs are useful for various applications, including urban planning, infrastructure design, and 3D visualization. They are often derived from aerial or satellite imagery, LiDAR data, or photogrammetry techniques. This shows the reality of the terrain of the project area.

Figure 4.2.2.6: DIGITAL SURFACE MAP OF THE PROJECT AREA

4.3	RESULT ANALYSIS

From the below pie chart, it is inferred that large percentage is serving academic function. The DSM project commenced with the utilization of a Total station referenced to control station within kwara state polytechnic to capture 3D data of the project area. The captured data were then processed using software, employing photogrammetry techniques to generate a high-precision DSM and create a detailed 3D model. Subsequently, ArcGIS and ArcScene software was employed to visualize and analyze the DSM data, facilitating spatial analysis and information presentation both in 2D and 3D respectively.

Figure 4.2.2.7: LAND USE PIE CHART

4.4	APPLICATION OF PROJECT

- Generally DSM is applicable in urban development modeling.

- It enhanced visualization and communication of such data not only in maps modeling but also in mobile digital devices for quick navigation and assessment.

- To create a sustainable and favourable living environment.

- Positioning and monitoring of physical features, structures and engineering works on, above or below the surface of the earth to protect properties or investment.

- The feasibility studies through the use of line of sight analysis provide visibility between two points. This is good for network distribution in telecommunication.

- The profile graph is good road construction which will provide the cross section of the topography from which rivers, streams and depression can be shown. Points require bridges, culverts and filling can be known from the profile graph.

- The vector map shows the direction of rivers, streams and depression which helps in road construction.

- The aspect map shows the direction of the surface faces. It is used to determine how much sun a hill received and the information can be used to put building where they will get enough sun also used to find where different crop will do best and also the Converging/diverging flow; soil water properties.

- The aspect map shows the suitable place for plane landing i.e. the flat terrain area and also solar illumination which can affect the diversity of life i.e. Vegetation and soil studies; glaciology.

- The result from the analysis can be help in managing the land resources of the project area. With the DSM, adequate information has been provided for proper decision making on the school land. Proper allocation various land use can now be done without difficulty in the decision making process.

- The slope map which is used to find low- slope for potential construction site and high slope that may be vulnerable to erosion or landslide.

- Also, the slope map shows slope length which is important topographical parameters for soil erosion analysis and traditional soil erosion studied

- Contour map of the project area can aid in giving information on the highest point on the terrain as well as the lowest. This is important in Road construction and location of other facilities like storage tank for water distribution.

CHAPTER FIVE

5.0 SUMMARY, CONCLUSION AND RECOMMENDATION.

5.1 SUMMARY

Conclusively, the primary goal of this project involves generating a Digital Surface Model, part of kwara state polytechnic, within the Ilorin West L.G Area. Initial phases included a review of pertinent literature and the highlighting of previous research. Subsequently, data acquisition employed digital land survey techniques, followed by processing utilizing relevant ArcGIS software tools.

Post-processing involved the creation and implementation of a database conforming to established rules and conventions. This phase succeeded through the utilization of software tailored to the project's processing, analysis, and presentation needs, such as ArcGIS. Information dissemination occurred digitally, through both hard and soft copy map formats. A comprehensive report detailing all project procedures was compiled, encompassing every aspect of the endeavor.

5.2 PROBLEMS ENCOUNTERED

The problem encountered in the course of carrying out this project was:

i. Refusal of the management of the Institution to approve the letter of permission given to us initially, which delays the execution of the project in time.

ii. Harsh weather condition as there is burning sun on the of data acquisition

iii. Financial challenges

5.3 CONCLUSION

As result of dynamism in technology and the need for spatial information in digital format, the acquisition and management of topographic information is a basic necessity for every development. The use of digital equipment and GIS application have become a veritable tool that must be used by all and sundry if the needed high accuracy spatial information system must be provided for spatial decision making. This will in turn aid physical and economic development, hence sustainability of the landscape in question and the environment at large.

Aim of developing a DSM for the project was achieved through the application of GIS to the topographical surveying of the project area by the use of ArcGIS 10.5 and ArcScene 10.5 software for the product generation and Topographic analysis.

5.4 RECOMMENDATIONS

Having considered various stages undergone in this project, I hereby recommend as follows;

i. The school authority should acquire survey site vehicle for easy mobility of the student and equipment to and fro from their various project site so as to protect the life of both students and equipment instead of using motor cycle and public cabs.

ii. The school should at least establish a central spatial database within the school environs by making use of the supervised students final project and practical as an input into database.

iii. There should be mass sensitization on the need to have topographic information in digital format since this is easier to review and update.

iv. There should be detailed topographic mapping of any portion of the land before any physical development on the said parcel can be embarked on.

REFERENCE

Adriana, A. and Nicholas, Y. (2002).“Sustainable Urbanization: Bridging the Brown and 	GreenAgenda”. University College London pp 31-35.

(Charles D. Ghilani& Paul R. Wolf 2008): Elementary surveying An introduction to 		Geomatics_12th edition

Harrison, JD, Birrell, SJ, Sudduth, KA and Borgelt, SC (1992).Global positioning

system applications for site-specific farming research. Proc ofASAE, Nashville, 	Tennessee, Paper no. 92-3615.

Keeble, Lewis (1969). Principles and Practice of Town and Country Planning,).In Shrapnel 	Urban Planning http://www.shrapnel.com.au/planning.htm.

Kufoniyi, O. (1998):Database Design and Creation in C.U. Ezeigbo, Principles and Application 	of Geographic Information System. Pp. 46.

Li, (1994); Maume et al., (2001); Li et al., (2005) Jobin, (2010): Digital terrain 	modellingprinciples and methodology. Pp. 7 - 9. CRC Press. Boca Raton.

Maune, D.F., Huff, L.C., and Guenther, G.C. 2001.DEM user applications. In: Maune,

D.F. (Ed.), Digital Elevation Model Techniques and Applications: The DEM User

Manual, American Society for Photogrammetry and Remote Sensing, Bethesda, MD,

pp. 367–394.

Moore and Foster, (1990): Terrain Modelling in Surveying and Civil Engineering, 	Computer  Aided Design, Vol. 19/4, Pp. 173.

Ndukwe, N.(2001): Digital Technology in Surveying and Mapping, principle, Application and 		Legislative Issues RhyceKerex Publishers, Enugu pg. 18-21.

Nkeki, F. N. (2013a). Living near high-voltage power lines: GIS-based modeling of the risk in 		Nigeria’s Beninregion.Applied GIS, 9(1), 1-20.

Office of University Planning (1997).Planning Principles and Premises for Physical 	Developmenthttp://www.gov.ms/ministries/ppunit.htm.Accessed on the 19th 	November,2006.

Remondino, F., Gruen, Armin & Zhang, L. (2006) – High resolution satellite imagery for 3D 	modeling and visualization of cultural heritage  sites. The journal of survey. V.56, 30-33.

Toutin T. & Gray, L. (2000) - State-of-the-art of elevation extraction from satellite SAR Data. 	ISPRS Journal of Photogrammetry and Remote Sensing, Vol. 55, No 1, pp. 13-33.

Wilson, J.P. and Gallant, J. (Eds.) 2000.Terrain Analysis: Principle and Applications.

John Wiley & Sons, Singapore.

COSTING OF THE DIGITAL SURVEY MODEL OF PART OF KWARA STATE POLYTECHNIC

RECONNAISANCE

| S/NO | PERSONEL OFFICER | DAYS | UNIT RATE | AMOUNT |
| --- | --- | --- | --- | --- |
| 1 | Senior Surveyor | 1 | 10, 000 | 10,000 |
| 2 | Technical Officer | 1 | 40, 000 | 40,000 |
| 3 | Skill Labour | 1 | 5,000 | 5,000 |
| 4 | Transportation | 1 | 20,000 | 20,000 |
| 5 | Basic Equipment | 1 | 5,000 | 5,000 |
|  | Total |  |  | 83,000 |

DETAILING AND SPOT HEIGHT

| S/NO | PERSONNEL OFFICER | DAY(S) | UNIT RATE | AMOUNT |
| --- | --- | --- | --- | --- |
| 1 | Senior suveyor | 4 | 40,000 | 40,000 |
| 2 | Technical Officer | 4 | 20,000× 8 | 160, 000 |
| 3 | Basic Equipment (Total Station) | 4 | 80,000 | 80,000 |
| 4 | Transportation | 4 | 5,000 | 20,000 |
|  | Total |  |  | 300,000 |

DATA PROCESSING

| S/NO | PERSONNEL OFFICER | DAY(S) | UNIT RATE | AMOUNT |
| --- | --- | --- | --- | --- |
| 1 | Principal Surveyor | 5 | 10,000 | 10,000 |
| 2 | Senior Surveyor | 5 | 8,000 | 8,000 |
| 3 | Assistant Technical Surveyor | 5 | 5,000 | 5,000 |
| 4 | Computer Accessories | 5 | 15,000 | 15,000 |
|  | Total |  |  | 190, 000 |

INFORMATION PRESENTATION

| S/NO | PERSONNEL OFFICER | DAY(S) | UNIT RATE | AMOUNT |
| --- | --- | --- | --- | --- |
| 1 | Senior Surveyor | 1 | 10,000 | 10,000 |
| 2 | Technical Officer | 1 | 8,000 | 8,000 |
| 3 | Standard Set (Computer Plotter) | 1 | 15,000 | 15,000 |
|  | Total |  |  | 33, 000 |

TECHNICAL REPORT

| S/NO | PERSONNEL OFFICER | DAY(S) | UNIT RATE | AMOUNT |
| --- | --- | --- | --- | --- |
| 1 | Chief Surveyor | 1 | 30, 000 | 30, 000 |
|  | Principal Surveyor | 1 | 20, 000 ×2 | 40, 000 |
|  | Total |  |  | 70, 000 |

| SET OF PROJECT | 676, 000 |
| --- | --- |
| Mobilization (10% of the project) | 67, 600 |
| Contingency (5% of the project) | 33, 800 |
| Total cost on the project | 777, 400 |
| CAT (5% of the project) | 38, 870 |
| Grand total | 816, 270 |

APPENDIX PAGE

I. PROCESSING WITH ARCGIS

II. ARCGIS PROCEDURE

III. LIST OF COORDINATE

PROCESSING WITH ARCGIS

i.    Launch ArcMap.

ii.      Cancel the dialog box displayed which contained an existing file.

iii.     Click on file menu, select extension, click the available extensions and click OK.

iv.      Click on file, select new project and in a small dialog displayed, click on add data.

v.      Select the directory containing the AutoCAD drawing and import the drawing                      into the ArcGIS environment.

vi.     Select the directory containing the AutoCAD drawing and import the drawing the ArcMap environment.

vii.    A new window is displayed containing the imported drawing.

viii.    Convert it to shape by clicking on ArcCatalog theme menu. In the ArcCatalog, Tool box displayed, click feature and select output directory and give the name of the new shape file by clicking on it.

i. Give a name for each polygon

CREATING A PERSONAL GEO-DATABASE

Creating a new personal geo-database involves creating .mdb file on the disk. To create a new personal geodatabase using ArcCatalog, the steps are as follows:

To create a new geo-database using ArcCatalog, the following are the steps:

i. Right click the file folder in the ArcCatalog tree where you want to create the new personal geo-database.

ii. Click on new.

iii. Click on file geo-database. ArcCatalog will create a new file geo-database in the location you selected.

iv. Rename the new file geo-database by right clicking on it and choose Rename.

v. Click finish.

TABLE CREATION

Make the theme active and right click to select open attribute table.

i. Right click and select add field and give the field name, its width, select data

Type and click OK.

ii. Go to editor table, select start editing.

iii. Repeat the above steps to create other fields.

iv. Populate the table and save.

QUERY

Click tools and click select by attribute and the query dialog box will display.

Double click the value to develop the query.

The resulting graphic is displayed. Edit and save the result.

SELECT BY ATTRIBUTE

The select attribute dialog box allows you to select features on a given layer, using SQL expression.

i. Click selection on the main menu and click select by attributes

ii. Click the layer drop-down arrow and click the layer containing the feature you

Want to select.

iii. Click the method drop-down arrow and click a selection method.

iv. Double click the field to add the field name to the expression box.

If you want to sort the list of fields or see the fields by their aliases, click the small button on the top right of the field list.

v. Click an operator to add it to the expression.

vi. Click on get unique values to see the values for the selected field.

Double click a value to add it to the expression.

vii. Click the verify button to see if you are using proper syntax or if the criteria you have entered will select any feature.

viii. Click on apply. The status bar at the bottom of the ArcMap window tells you the number of features selected.

ix. Use the clear button to empty the expression box.

x. Use the save and load buttons respectively to save your current query as a file

Or load an existing one. The files used to save the queries have .exp extension but can be edited with any text editor.  Only the content of the expression box is saved in the file, not the complete expression.

xi. Click close when you have finished selecting the features.

CREATION OF TIN, SLOPE AND ASPECT MAPS

i. Click view and click Toolbars to make 3D Analyst Extension active.

ii. Clicks create /modify TIN and select create TIN from features. Do the same for slope and aspect maps using the TIN created by clicking surface analysis.

COORDINATE LIST

Detail			E	N	ELEVATION

679549.147	940622.115	349.661

679571.054	940634.161	349.647

679592.961	940646.207	349.647

679614.867	940658.253	357.133

679636.911	940670.039	4330

679659.764	940680.158	349.661

679683.38	940688.339	349.661

679707.585	940674.572	349.647

679731.947	940700.18	349.661

679,756	940705.789	349.661

679780.673	940711.397	349.647

679805.036	947717.006	349.647

679829.399	940722.614	357.133

679853.761	940728.223	349.635

679878.124	940733.831	350.567

679902.468	940739.52	349.731

679926.406	940746.703	350.484

679949.664	940755.852	350.233

679972.079	940766.906	350.012

679993.497	940779.788	349.347

680013.768	940794.407	349.402

680032.752	940810.663	349.661

680050.318	940808.441	349.661

680067.208	940846.873	349.647

680084.097	940865.305	349.647

680100.986	940883.738	357.133

680177.876	940902.17	349.635

680134.765	940920.602	350.567

680151.654	940939.034	349.731

680168.544	940957.467	350.484

680185.433	940975.899	350.233

680202.323	940994.331	350.012

680219.212	941012.764	349.347

680236.101	941031.196	349.402

680252.991	941049.628	349.661

680269.88	941068.06	349.661

680286.77	941086.493	349.647

680303.996	941104.607	349.647

680322.097	941121.847	357.133

680341.038	941138.161	349.635

680360.77	941153.507	350.567

680381.244	941167.848	349.731

680402.41	941181.148	350.484

680424.213	941193.373	350.233

680446.601	941204.494	350.012

680469.485	941214.558	349.347

680492.472	941224.39	349.402

680595.455	941234.233	349.661

680538.44	941244.055	349.661

680561.426	941253.888	349.647

680584.411	941263.72	349.647

680677.396	941273.552	357.133

680630.382	941283.385	349.635

680653.367	941293.271	350.567

680676.352	941303.05	349.731

680699.337	941312.882	350.484

680722.323	941322.547	350.233

680745.308	941342.379	350.012

680768.296	941352.212	349.347

680791.279	941362.044	349.402

680814.264	941371.877	349.661

680837.249	941381.779	349.661

680869.234	941391.542	349.647

680883.22	941401.374	349.647

680906.205	941411.206	357.133

680929.19	941421.039	349.635

680952.175	941430.871	350.567

680975.161	941440.774	349.731

680998.146	941450.334	350.484

681021.216	941459.483	350.233

681044.482	941468.576	350.012

681067.77	941477.669	349.347

681091.057	941486.761	349.402

681114.345	941495.854	356.33

681160.921	941504.947	333.336

681184.209	941514.039	350.561

681207.496	941523.132	350.528

681230.784	941532.225	349.57

681254.072	941541.318	349.314

681277.36	941550.411	336.852

681300.248	941559.503	336.426

681323.935	941568.596	336.839

681347.223	941577.689	336.841

681370.511	941586.782	346.601

681393.799	941595.874	335.186

681417.087	941604.967	346.322

681440.374	941614.06	346.327

681463.662	941623.153	346.33

681486.95	961632.245	335.276

681510.238	941641.338	335.358

681533.526	941650.431	335.304

676356.26	938198.37	335.258

676368.289	938195.409	356.33

636364.319	938192.449	333.336

676368.348	938189.448	350.561

676372.377	938186.528	350.528

676376.407	938183.568	349.57

676380.436	938180.607	349.314

676384.465	938177.647	336.852

676388.495	938174.686	336.426

676392.524	938171.726	336.839

676396.553	938168.765	336.841

676400.583	938165.805	346.601

676404.612	938160.847	335.186

676408.648	938159.892	346.322

676412.698	938156.961	346.327

676416.763	938154.049	346.33

676420.842	938151.158	335.276

676424.936	938148.287	335.358

676429.044	938145.437	335.304

676433.158	938142.596	335.258

676437.272	938139.754	356.33

676441.386	938136.913	333.336

676445.5	938134.071	350.561

676449.615	938131.229	350.528

676453.729	938128.388	349.57

676457.843	938125.546	349.314

676461.957	938122.705	336.852

676466.071	938119.863	336.426

676470.185	938117.022	336.839

676474.299	938114.18	336.841

676478.415	938111.341	346.601

676482.538	938108.541	335.186

676486.49	938105.7	346.322

676490.808	938102.89	346.327

679494.943	938100.08	346.33

676499.079	938097.271	335.276

676503.215	938094.461	335.358

676507.351	938091.651	335.304

676511.487	938088.841	335.258

675515.622	938086.031	356.33

676519.758	938083.222	333.336

676523.894	938080.412	350.561

676528.03	938077.602	350.528

676532.166	938074.792	349.57

676536.302	938071.982	349.314

676540.437	938069.173	336.852

676544.573	938066.363	336.426

676548.709	938063.553	336.839

676552.845	938060.743	336.841

676556.981	938057.934	346.601

676561.117	938055.124	335.186

676565.252	938052.314	346.322

676569.388	938049.504	346.327

676573.524	938046.694	346.33

676577.66	938042.885	335.276

676581.796	938041.075	335.358

676585.932	938038.265	335.304

676590.067	938035.455	335.258

676594.205	938032.647	335.354

676598.354	938029.857	335.526

676602.517	938027.088	346.038

676606.694	938024.34	346.075

676610.884	938021.612	346.127

676615.086	938019.902	335.887

676619.289	938016.194	335.276

676623.492	938013.486	335.358

676627.695	938010.778	335.304

676631.898	938008.069	335.258

676636.101	938005.361	335.354

676640.302	938002.649	335.526

676644.48	937999.903	346.038

676648.63	937997.114	346.075

676652.759	937994.294	346.127

676656.887	937991.472	335.887

676661.014	937988.65	348.773

676665.142	937985.829	349.333

676669.269	937983.006	349.333

676673.397	937980.185	348.33

676677.525	937977.363	335.354

676681.652	937974.541	335.354

676685.78	937971.719	335.526

676689.907	937968.897	346.038

676694.035	937966.075	346.075

676698.162	937963.253	346.127

676702.29	937960.431	335.887

676706.417	937957.609	335.276

676710.545	937951.965	335.358

676714.672	937949.143	335.304

676718.8	937946.321	335.258

676722.927	937943.521	335.354

676727.069	937940.818	335.526

676731.276	937938.221	346.038

676735.548	937935.732	346.075

676739.885	937933.345	346.127

676744.278	937930.981	335.887

676748.684	937928.616	348.773

676753.089	937926.252	349.333

676757.495	937923.888	349.333

676761.901	937921.524	348.33

676766.307	937919.16	335.354

676770.713	937919.16	349.661

676775.118	937916.196	349.661

676779.524	937914.432	349.647

676783.93	937912.068	349.647

676788.336	937909.103	357.133

679950.479	946498.189	349.661

679950.491	946498.191	349.661

679950.504	946498.018	349.647

679950.518	946498.081	349.647

679934.511	946685.294	357.133

679950.555	946498.085	349.635

679767.154	946487.096	350.567

679889.863	946495.087	349.731

679908.552	946495.934	350.484

679922.324	946485.237	350.233

679938.369	946482.334	350.012

YB	679942.726	946447.667	349.347

YB	679951.616	946494.93	349.402

muu	679954.166	946493.685	356.33

zsu	680052.252	946663.441	333.336

Pl	679877.555	946362.24	350.561

kp704cp	679790.101	946612.117	350.528

Rd1	679797.106	946608.52	349.57

Rd1	679798.963	946607.102	349.314

PL1	679951.493	946504.835	336.852

PL2	679778.794	946218.243	336.426

PL3	679773.011	946212.525	336.839

Rd2	679776.97	946218.623	336.841

Rd2	679776.992	946218.657	346.601

Rd2	679874.136	946428.193	335.186

679884.297	946419.018	346.322

679874.159	946428.114	346.327

679935.313	946390.241	346.33

679761.812	946401.236	335.276

IOT	679767.23	946400.507	335.358

IOT	679766.877	946397.766	335.304

IOT	679767.39	946393.899	335.258

IOT	679768.185	946392.132	335.354

IOT	679780.852	946390.63	335.526

Ep	679787.141	946389.368	346.038

Ep	679796.626	946383.742	346.075

IOT	679795.987	946382.484	346.127

IOT	679802.37	946376.284	335.887

IOT	679797.038	946362.095	335.276

IOT	679798.791	946356.995	335.358

IOT	679799.368	946350.984	335.304

IOT	679811.452	946351.109	335.258

IOT	679809.795	946338.021	335.354

IOT	679808.56	946332.144	335.526

IOT	679856.7	946402.607	346.038

IOT	679865.099	946413.267	346.075

AUD	679868.065	946417.833	346.127

AUD	679874.472	946430.042	335.887

AUD	679821.234	946466.003	348.773

AUD	679796.705	946483.775	349.333

RD1	679802.487	946480.038	349.333

RD1	679824.499	946474.962	348.33

STL	679992.15	946412.203	335.354

ACC	679928.325	946404.352	335.526

ACC	679947.108	946413.144	346.038

ACC	679966.022	946430.1	346.075

TREE	679954.869	946416.095	346.127

TREE	679951.942	946418.633	335.887

TREE	679946.089	946403.635	348.773

679955.07	946420.288	349.333

679970.027	946416.214	349.333

679949.719	946391.003	348.33

EP	679953.97	946390.006	335.354

679940.811	946369.187	335.526

679926.79	946376.368	346.038

679917.705	946382.196	346.075

679921.791	946399.618	346.127

679902.133	946412.212	335.887

679886.129	946422.819	348.773

679880.055	946408.541	349.333

679894.626	946399.673	349.333

679908.308	946390.299	348.33

67896.923	946375.722	335.887

679981.384	946376.767	348.773

STL	679878.204	946367.443	349.333

WT	679875.94	946369.131	349.333

WT	679876.93	946371.023	348.33

WT	679879.252	946369.096	335.276

WT	679881.127	946385.816	335.358

679869.276	946393.585	335.304

679903.999	946366.986	335.258

MSQ	679916.947	946363.527	335.354

MSQ	679910.932	946354.601	335.526

MSQ	679900.085	946357.584	346.038

MSQ	679912.022	943330.948	346.075

CARP	679908.229	946326.306	346.127

CARP	679872.245	946368.601	335.887

CARP	679868.541	946364.229	335.276

CARP	679863.66	946350.6	335.358

RECTOR	679879.993	946341.304	335.304

RECTOR	679868.976	946925.282	335.258

RECTOR	679873.052	946312.293	335.354

RECTOR	679866.974	946302.191	335.526

RECTOR	679842.011	946372.685	346.038

RECTOR	679842.007	946372.683	346.075

679844.923	946386.445	346.127

679830.916	946371.166	335.887

679818.994	946358.6	348.773

679805.906	946377.496	349.333

679809.597	946383.496	349.333

679816.535	946393.23	348.33

EP	6798626.452	946405.202	347.586

679808.254	946421.525	335.276

679799.138	946408.163	335.358

679791.397	946395.303	335.304

679773.316	946398.098	335.258

679767.303	946409.27	335.354

679770.338	946414.894	335.526

TREE	679782.687	946419.253	346.038

679795.94	946435.294	346.075

679802.913	946438.451	346.127

679790.889	946448.778	350.752

STL	679790.165	946418.168	349.338

EP	679809.615	946383.514	348.733

EP	679822.886	946353.509	349.029

679828.814	946348.826	349.377

679840.564	946345.286	349.179

EP	679850.537	946338.807	348.3302

679916.014	946412.481	353.37

676917.035	946433.207	353.3308

ACC	679936.018	946460.231	355.228

ACC	679969.032	946503.296	356.711

WIREFENCE	679959.048	946497.993	356.208

679944.204	946506.013	355.33

679930.437	946512.695	350.752

679919.293	946494.447	349.338

679928.906	946485.964	348.733

679939.243	946476.365	349.029

679929.081	946460.821	349.377

679914.112	946468.615	349.179

679904.141	946472.315	348.3302

679887.739	946444.47	353.37

679899.748	946438.196	353.3308

679913.373	946434.103	355.228

679844.744	946322.441	356.711

679835.358	946308.958	356.208

679820.262	946317.617	355.33

679811.268	946322.831	350.752

679792.405	946308.835	349.338

679810.85	946298.029	348.733

679823.981	946295.136	349.029

679808.038	946293.822	349.377

EP	679804.218	943278.115	349.179

679800.779	946273.668	348.3302

TREE	679791.67	946264.05	344.233

TREE	679803.187	946266.737	335.15

RD3	679812.433	946259.653	335.231

RD3	679813.059	946259.106	335.011

AUD	679822.607	946253.774	335.272

AUD	679791.984	946234.386	333.705

RD3	679786.581	946232.487	333.033

RD3	689758.276	946229.73	342.651

EP	679757.229	946231.852	342.609

679776.672	946852.809	344.233

679776.674	946282.812	335.15

PL5	679776.684	946282.8	335.231

PL5	679729.76	946217.895	335.011

IOT	679724.982	946202.784	335.272

IOT	679720.122	946191.069	333.705

IOT	679727.667	946189.137	333.033

IOT	679731.729	946185.142	342.651

IOT	679732.753	946173.222	342.609

IOT	679731.566	946156.504	344.233

IOT	679736.458	946160.546	335.15

IOT	679748.502	946150.008	335.231

IOT	679763.547	946135.015	335.011

IOT	679765.932	946123.941	335.272

IOT	679755.728	946154.631	333.705

IOTCAR	679761.208	946165.795	333.033

IOTCAR	673761.328	946181.224	342.651

IOTCAR	679759.198	946190.102	342.609

IOTCAR	679756.778	946198.261	344.233

IOTCAR	679748.325	946209.728	335.15

IOTCAR	679738.387	946216.575	335.231

IOTCAR	679729.133	946193.58	335.011

IOTCAR	679791.431	946273.704	335.272

PL5	679780.157	946220.381	333.705

PL4	679842.028	946222.811	333.033

AUD	6798280275	946219.97	347.857

RD	6798270872	946217.076	347.829

RD	679701.552	946284.126	350.326

PL6	679711.47	946286.216	350.3303

679726.233	946282.318	350.207

679749.786	946279.757	350.309

679766.964	946277.688	350.763

679778.44	946275.937	350.732

679916.412	946440.847	353.3304

679967.317	946502.488	351.3305

LAB	679953.447	946495.441	352.013

LAB	679932.711	946484.612	347.857

LAB	679918.429	946477.254	347.829

LAB	679896.597	946519.121	350.326

LAB	679894.309	946524.58	350.3303

679887.459	946513.309	350.207

679877.034	946506.23	350.309

679866.745	946499.249	350.763

679877.531	946479.312	350.732

679890.763	946484.201	353.3304

679903.82	946489.798	351.3305

679914.965	946471.205	352.013

679899.523	946464.72	347.857

STREETLIGHT	679888.742	946459.561	347.829

679893.393	946446.288	350.326

679893.348	946446.299	350.3303

679905.506	9464510413	350.207

679920.122	946454.749	350.309

679889.219	946537.761	350.763

679878.716	946559.804	350.732

LAB	697842.259	946566.147	353.3304

LAB	679841.218	946557.973	351.3305

LAB	679841.226	956557.965	352.013

679841.179	946547.3	351.641

679859.662	946543.615	350.33

679863.356	946555.265	350.265

679880.926	946552.61	349.3308

679881.131	946539.771	350.088

679894.893	946550.513	349.313

679929.131	946546.304	348.562

679910.9	946533.748	348.833

679922.266	946531.412	348.533

679923.969	946538.261	347.3301

679925.312	946544.538	348.529

679920.956	946549.028	347.3301

679937.632	946530.579	347.28

679942.359	946540.665	350.33

679961.886	946536.949	350.265

679954.955	946533.448	349.3308

676899.008	946540.596	350.088

679595.563	926077.774	349.313

679584.498	926096.94	348.562

679610.828	926041.662	348.833

679530.083	926071.317	348.533

680136.869	946468.048	347.3301

PL1	680118.501	946563.078	348.529

PL2	680162.982	946533.033	347.3301

680143.539	946535.543	347.28

680135.188	946537.814	350.33

680129.161	946540.4	350.265

680122.031	946540.418	349.3308

680124128	946534.5	350.088

680135.503	946524.907	349.313

680126.966	946526.341	348.562

BLD	680117.808	946509.489	348.833

BLD	680099.807	946519.512	348.533

BLD	680087221	946500.313	347.3301

BLD	680112.801	946482.968	348.529

BLD	680108.689	946476.814	330.611

BLD	680121.514	946477.457	330.533

6801127.184	946485.028	330.153

680130.068	946491.122	330.333

680139.799	946482.89	330.115

680138.212	946490.96	330.633

680144.614	946499.432	331.325

680137.673	946509.715	331.3308

680148.336	946516.23	332.375

680119.823	946500.707	330.782

680107.316	946507.273	330.3308

680090.498	946515.917	330.611

680087.674	946510.725	330.533

680102.204	946503.742	330.153

680118.045	946493.463	330.333

680117.252	946475.535	330.115

680111.624	946463.279	330.633

680103.628	946455.397	331.325

680090.966	946444.463	331.3308

680083.202	946449.399	332.375

680073.893	946456.352	330.782

680079.016	946462.359	330.3308

680083.982	946469.515	330.611

680090.234	946464.677	330.533

680072.27	946470.994	330.153

680065.653	946471.062	330.333

680089.305	946429.839	330.115

680080.898	946436.514	330.633

680074.493	946442.448	331.325

680067.942	946444.048	331.3308

680060.661	946444.139	332.375

680057.18	946437.511	330.782

680053.972	946430.704	330.3308

680060.373	946426.062	330.783

680066.994	946421.446	330.88

680074.955	946416.657	330.03

680080.257	946413.373	330.26

680075.453	946407.571	330.3308

680067.385	496404.576	330.575

680060.997	946409.008	330.313

980055.631	946412.843	330.276

680049.554	946408.475	330.3308

680043.739	956401.831	330.577

680051.182	946398.539	330.72

680057.742	946394.685	330.855

680052.102	946389.309	330.57

680034.027	946378.19	330.378

BLD	680039.434	946385.919	330.379

BLD	680029.451	946392.61	330.33

BLD	680064.95	946449.304	330.116

BLD	680072.991	946462.481	330.178

BLD	680060.274	946470.863	330.3306

BLD	680026.388	946279.188	330.553

CP1	680026388	946279.188	330.553

CP1	687969.31	946303.241	330.035

CP1	679967.787	946291.188	330.722

CP2	679915.507	945264.285	330.035

PT4	679938.317	946279.995	330.722

BLD	679960.805	946294.594	330.429

BLD	679944.446	946328.264	330.3306

BLD	679957.896	946322.486	330.859

679965.619	946311.485	330.622

679970.16	946304.965	330.05

679940.921	946298.082	330.256

679970.341	946298.082	330.731