PROJECT REPORT

ON

PERIMETER AND DETAILS SURVEY

OF

FEDERAL STAFF SCHOOL ADEWOLE KANO ROAD, ILORIN KWARA STATE

BY

OSUNLEKE HABEEB OLAIDE

HND/22/SGI/FT/0052

SUBMITTED TO:

THE DEPARTMENT OF SURVEYING AND GEO-INFORMATICS

INSTITUTE OF ENVIRONMENTAL STUDIES (IES)

KWARA STATE POLYTECHNIC, ILORIN

IN PARTIAL FULFILMENT OF THERE QUIREMENT FOR THE AWARD OF HIGHER NATIONAL DIPLOMA (HND) IN SURVEYING AND GEO-INFORMATICS.

JULY 2025

# CERTIFICATE

I hereby certify that the field work and information given in this project were obtained as a result of my observation and measurement and were carried out in accordance with survey laws and departmental instruction.

_______________________					_____________________

OSUNLEKE HABEEB OLAIDE	            			Date

Student Name

# CERTIFICATION

This is to certify that OSUNLEKE HABEEB OLAIDE with matriculation number HND/22/SGI/FT/0052 from the department of Surveying and Geo-informatics, Institute of Environmental Studies carried out a practical field work which formed basic of the project in accordance with survey rules and regulation and departmental instruction.

_____________________				_______________________

Surv. A.G. AREMU						Date

Project Supervisor

_____________________				________________________

Surv. R.S AWOLEYE					Date

Project Co-Ordinator

_____________________				________________________

Surv. I.I. ABIMBOLA					Date

Head of Department

_____________________				________________________

External Examiner						Date

# DEDICATION

This is project work is dedicated to God Almighty.

# ACKNOWLEDGEMENT

All glory and adoration goes to almighty God for his countless blessings and mercy and forgiving me wisdom, aspiration, strength and opportunity to embark on this project report and complete it successfully.

My appreciation also goes to my supervisor SURVR.A.G AREMU for all his fatherly advise and encouragement and also to the HOD of my department SURV.I.I ABIMBOLA and project coordinator SURV. R.S AWOLEYE, SURV.ASONIBARE, SURV.FELIX DIRAN, SURV KABIR and all my lecturers, I pray that the lord bless you and your family.

I also extend my sincere gratitude to my amiable caring parents Mr and Mrs OSUNLEKE for their continuous support both morally and financially, also to my sibling you have always been my back bone and I so much cherish the fact that I have you all my life.

My appreciation also goes to all my friends for your support in my journey at kwara state polytechnic: such as ADEBISI, ELIZABETH, EZEKIEL, JOSHUA, BLESSING to all my group member and to those I didn't mention I so much cherish those moment we shared together. I pray that mercy of God will never seize in your various houses.

# ABSTRACT

This project base on digital mapping in surveying of Federal staff's school Adewole, ilorin kwara states. The project state a simple explanation of the through the introduction with background of project area, statement of problem, aims, objective, scope, specification, personnel and the project location. The literature review which involves discussion previous executed of digital mapping with reference, The actual field measurement, plotting were done with the use of Autocad and micro soft word software for the process and analysis of the result was printed in hard copy. Finally summary, problem encountered. conclusion and recommendation were thoroughly stated at the later part of the report.

TABLE OF CONTENTS

TITLE 										        PAGE

Certificate 											ii

Certification											iii

Dedication											iv

Acknowledgment										v

Abstract											vi

Table of contents										vii

CHAPTER ONE

1.0 introduction										1

1.1 Importance of Perimeter and Detail Survey						5

1.2 Aim of the project										5

1.3 objective of the project									5

1.4 Project specification									5

1.5 scope of the project									6

1.6 Participants involved									7

1.7 project location										8

CHAPTER TWO

2.1 Literature review										9

CHAPTER THREE

3.0 Methodology										16

3.1 Planning											16

3.1.1 office planning										16

3.1.2 field Reconnaissance									17

3.2 Instrument used										18

(1) Hardware used										18

(2) Software used										18

3.3 Checking of Instrument Error								19

3.4 Monumentation										20

3.5 Data Acquisition										21

3.5.1 Instrument check									22

3.5.2 Geometric data acquisition								22

3.5.3 Perimeter Traverse and Geometry Points Generation					22

3.6 Data Processing										23

CHAPTER FOUR

4.0 Data processing										25

4.1 Data Download and editing								25

4.2 Data processing in AutoCAD								26

4.3 Area computation										27

4.4 Back computation										29

4.5 Product Application									29

4.6 Costing											30

CHAPTER FIVE

5.0 Summary, conclusion, problem Encountered, References				33

5.1 Summary											33

5.2 Conclusion										34

5.3 Recommendations										34

5.4 Challenges Encountered									35

Reference											37

Appendices											38

LIST OF TABLES

1.6 Participants involved									7

3.1 Show the values of control								17

3.2 Showing the coordinate of pillar								20

3.3 The observed coordination of SC/KWI.334R from Bm500				23

3.4 The observed coordination of SC/KWI.33.3 from SC/KWBM500			23

3.5 The results of the computed control check from the given coordinate			24

3.6 The results of the computed control check from the observed coordinate		25

4.4.1 Showing the back computation								27

LIST OF THE FIGURES

Fig 1.1 show the project location								8

Fig 3.1.1 Show the typical beacons used							17

CHAPTER ONE

1.0 INTRODUCTION

1.1 BACKGROUND TO THE STUDY

Perimeter survey is a specific type of survey that measures the distance along the boundary lines of a given land. A perimeter survey is important to find out the exact location of the landed property and probably determine the extent of such land also the extent of encroachments can be evaluated in case of the landed property dispute. Surveying began in Babylon and Egypt in the form of field measurements. The great pyramids which are mystery hitherto could not have been built without the knowledge of surveying principal in spite of the conflicting views about its physic construction. The Egyptians used stones to mark boundaries of their farmland along Nile valley as temporary beacons even though the beacons used to be washed away later. This led to proper and solid demarcation of boundaries by permanent beacons.

Detailing is defined as the process of fixing topographical features to the survey line, Any of these under listed methods could be used,

  - Chain survey method (tie line and offset)

  - Plane table survey method

A perimeter and detail survey is a comprehensive method used in land surveying to determine the boundaries and features of a specific parcel of land. This type of survey is essential for property disputes, construction projects, and land management.

STAGES OF OPERATION IN PERIMETER AND DETAIL SURVEY

Perimeter and details survey involves the following operation:

- Reconnaissance both in the office and field

- Total station for detail fixing/contouring

- Computation

- Plan production

Land is considered to be one of the best long-term assets that anyone can have. It is of high value, is always in demand, and is easy to liquidate. On the other hand, owning a piece of land is not that easy. It includes certain duties and responsibilities that the landowner is required to perform in order to maintain its market value.  Perhaps one of the most important duties is performing an updated Perimeter and detail Survey to determine the land’s boundaries and size. A Perimeter and detail Survey is a type of land survey that defines the perimeter boundaries of a particular parcel of real estate property. This survey maps a strip along the boundaries using a minimum width of 15 feet. This requires surveying the entire perimeter of the said real estate in order to determine the exact acreage and geometry of the property and identify any easements and encroachments that may be present within the land.

This process will require the installation of monuments to mark the boundary corners of the land for future references.  This requirement can be waived through a written agreement between the surveyor and homeowner. Perimeter and detail Surveying is a service that can be performed by a professional land surveying firm, such as Godfrey-Hoffman & Hodge in Connecticut, to ensure reliable, accurate, and detailed surveying results.

A Perimeter and detail Survey pinpoints any visible evidence of easements and other public facilities within the property.

• A Perimeter and detail Survey detects any visible ingress and egress on the property.

• A Perimeter and detail Survey resolves conflicts on maps and deed descriptions.

• Perimeter and detail Surveys are used to identify the location of old boundary monumentation and survey pins, or they can be used to set up new ones.

• A Perimeter and detail Survey does not identify the features and improvements that exist within a property, such as barns, garages, sheds, dwellings, surface utilities, roadways, pools and visible bodies of water; only features that fall within the 15 foot width around the boundary perimeter will be depicted.

A detail survey is used to determine and locate the features and improvement on a parcel of lane. The word ‘’FEATUES’’ here means both natural and man-made structures on a pieces of land such as vegetation, types of soil, buildings, and utilities fences and boundaries roads, and marks and so on. Professional land surveyors use different tools & techniques in Perimeter and detail Survey that includes both conventional and modern technologies. However, most of the surveyors prefer to use the conventional surveying tools & techniques that include bearing and distance or compass and tape. Although they are considered primitive tools, they still remain an important asset to a land surveyor. Although there are some minor adjustments and improvements in the compass and tape, they still follow the same age-old concept and technique. In addition to them, surveyors will use GPS technology in a Perimeter and detail Survey which is one of the greatest advancements in the surveying industry. Both of these two techniques, if carried out by a professional, can result in a detailed analysis of your property with a small margin of error. Typically, a surveyor would prefer to use the bearing and distance technique when working over a smaller distance and use GPS technology on larger distances.

A Perimeter and detail Survey is a specific type of property survey that maps and measures a distance along the property boundary. Professional land surveyors use either Bearing and Distance (compass and tape) or GPS technology for a Perimeter and detail Survey. They are generally out using survey equipment such as total station and theodolites. The data is then carried to the office for analysis and preparations of detail plan are. These plans are usually useful for engineers and architects who use them in their design and plain. The survey should be carried out by a qualified land surveyor who may be assisted by a chairman. With this kind of survey one can obtain a property which would consist of the inventory of all the property lines together with a list of the reputed owners and the type and use of building thereon. These data will be plotted on maps as accurately as possible considering the best sources of information.

A Perimeter and detail Survey can determine the measurement around your property and is conducted by a professional surveyor to ensure that an accurate report is conducted and recorded.  A Perimeter and detail Survey is a specific type of survey that measures the distance along the boundary lines of your land. A Perimeter and detail Survey is important to find out the exact location of your property lines and you can also discover if there is any encroachment on your property. Although it might sound like a simple survey where you just need to measure the length of your boundary, in reality it includes many other assumptions and calculations. While the surveyors use this approach, they have to analyze, calculate and measure a wide range of factors which includes but not limits to:

- Boundary monumentation

- Boundary encroachments

- Unresolved conflicts with existing property deeds & maps

- Lines of occupation: fences, walls, hedges, yards, and natural elements

1.1 IMPORTANCE OF PERIMETER AND DETAIL SURVEY

A perimeter and detailed survey is foundational for any land development or construction project. It provides the necessary data to ensure that projects are planned and executed within legal boundaries, with accurate placement of structures and infrastructure. This survey not only protects the interests of property owners but also ensures compliance with legal and regulatory requirements.

AIM AND OBJECTIVES

1.2 AIM

The aim of this project is to carryout a perimeter/detail survey of Federal Staff School Adewole, Kano Road, Ilorin, Kwara State

1.3 OBJECTIVES

The following objectives are considered in order to accomplish the above aim:

- Reconnaissance operation i.e office and field.

- Data acquisition

- Data processing and computations

- Data presentation i.e plan production.

- Report writing

1.4 PROJECT SPECIFICATIONS

The following are the specification to be ascertained in the project.

- A third order total station traverse must be carried out.

- The control checks must be thoroughly done.

- The length of each traverse line must not be greater than 250metres.

- The liners accuracy should not be less than 1:5000

- Angular misclosure should not exceed ±30;; √n where n is the number of station traversed.

- the carved out area must cover the entire land of the study area

- The horizontal distance should be given in 3 decimal places.

- The consecutive station must be inter visible and should be established where it would not be disturbed.

1.5 SCOPE OF THE PROJECT

The scope of this project covers

- Perimeter traversing

- Detailing

- Data downloading, editing and processing

- Computation

- Area computation

- Information presentation

- Report writing

1.6 PERSONNEL INVOLVED

For easy and proper execution of this project (7) seven members were engages in the operations. The roles were interchanged among the members in the group for the benefit for everyone to participate all the operations. The below table shows the name and matric number of the group members.

| S/N | NAME | MATRIC NO | ROLE PLAYED |
| --- | --- | --- | --- |
| 1 | OSUNLEKE HABEEB OLAIDE | HND/22/SGI/FT/0052 | AUTHOR |
| 2 | OJO ADEBISI MOTUNRAYO | HND/23/SGI/FT/0041 | Member |
| 3 | ADEBAYO BLESSING MARY | HND/23/SGI/FT/0042 | Member |
| 4 | DADA ELIZABETH OLUWAPELUMI | HND/23/SGI/FT/0043 | Member |
| 5 | OLADIMEJI RAPHAEL OLUWASEUN | HND/23/SGI/FT/0045 | Member |
| 6 | IBITOYE JOSHUA AYOKUNLE | HND/23/SGI/FT/0047 | Member |
| 7 | AKINYEJO EZEKIEL DAMILOLA | HND/23/SGI/FT/0051 | Member |

1.7	PROJECT LOCATION

The project area of Federal Staff School Adewole, Kano Road, Ilorin, Kwara State which covered the primary and secondary school on the geographical co-ordinator of Easting and Northing

LAT: 40 513’E			LONG: 80 4777’N

LAT: 40 30’ 47”		LONG: 80 28’ 40’’

CHAPTER TWO

LITERATURE REVIEW

Introduction

Surveying, as a scientific and practical discipline, plays a central role in the acquisition, analysis, and interpretation of geospatial data necessary for land development, planning, and administration. With the rapid pace of urbanization, the accurate definition of land boundaries and detailed mapping of surface features have become indispensable to modern society. In particular, perimeter and detail surveys provide the foundation upon which infrastructure, legal boundaries, land titles, and resource management are built.

In many parts of the world, particularly in developing countries such as Nigeria, disputes over land ownership and encroachment have highlighted the urgent need for accurate boundary surveys. These disputes are often exacerbated by population growth, urban expansion, and poor land governance. As noted by Ogunleye and Ojo (2018), the proper delineation of property lines through perimeter surveying serves not only legal and cadastral purposes but also helps to foster societal peace and ensure sustainable land use. The importance of detail surveys is equally profound. According to Abdulkadir et al. (2019), detail surveys provide the spatial and positional data necessary for engineering design, environmental assessment, and infrastructure development.

The historical roots of surveying”ate back thousands of years, yet the field has continued to evolve alongside technological advancement. Ancient Egyptians, for example, are known to have used rope stretchers and primitive tools for land measurement as far back as 1400 BC. The Greeks and Romans advanced the practice with instruments like the dioptra and groma. The modern era has witnessed a remarkable transformation in surveying practices with the integration of satellite systems, total stations, GIS, UAVs (drones), and GNSS technologies. As pointed out by Uren and Price (2010), the synergy of traditional field techniques and digital innovations has revolutionized land measurement and mapping.

This literature review explores in detail the concepts of perimeter and detail surveys, their significance, methodologies, instruments, historical development, regulatory context, challenges, and the future of surveying technologies. It synthesizes scholarly contributions and professional practices to create a robust framework upon which this project is anchored.

According to Yusuf Y.D (2009), surveying is the technique, profession, and science of determining the terrestrial or three-dimensional positions of points and the distances and angles between them. It is primarily used to establish land maps and boundaries for ownership or governmental purposes.

Maguire et al. (1991) explain that surveying is fundamental to geographic data acquisition and provides the spatial basis upon which planning and development decisions are made. Without accurate survey data, land administration becomes prone to disputes, overlapping claims, and poor spatial planning.

PERIMETER SURVEYING

A perimeter survey involves the measurement and demarcation of the boundaries of a specific land parcel. It is used to establish the limits of ownership or control over a piece of land. This form of survey is essential in property registration, cadastral mapping, land titling, and legal documentation.

Karikari (2006) emphasizes that perimeter surveys are essential in ensuring land tenure security, especially in institutional and governmental land holdings. A proper perimeter survey provides the foundation for fencing, legal defense against encroachment, and development planning.

Enemark (2009) further asserts that accurate boundary delineation is fundamental for the efficient functioning of a land administration system. When boundaries are uncertain or poorly documented, it becomes difficult to manage land sustainably or resolve land-related disputes.

DETAIL SURVEYING

A detail survey, also known as a topographic or feature survey, involves capturing all visible and permanent features on a given parcel of land. This includes buildings, roads, fences, vegetation, drainage systems, utility lines, and other landmarks. The objective is to produce a comprehensive map showing the relative positions and attributes of all natural and man-made features.

Agunbiade and Kolawole (2016) define detail surveying as the process of collecting data on all existing features of interest within a specific area for the purpose of mapping, planning, or analysis. This type of survey is critical for design, engineering works, and layout planning.

Meijer (2009) explains that detail surveys are essential for civil works, especially during the initial planning and feasibility studies. Accurate feature location allows for optimal infrastructure layout and environmental management.

Historical Development of Surveying

Surveying is one of the oldest technical professions in human history. The earliest known surveyors emerged in ancient Egypt, where land had to be redistributed after the annual flooding of the Nile. These early practices involved the use of ropes and measuring rods to define parcels of land (Brinker & Minnick, 1995). Ancient Greek mathematicians such as Pythagoras and Euclid formalized the geometric principles that underpin modern surveying.

In the Roman Empire, the groma was widely used to lay out roads, towns, and aqueducts. According to Schofield and Breach (2007), the Romans developed a relatively advanced cadastral system, some of which influenced European land registration centuries later.

The Renaissance and Enlightenment periods brought significant technological improvements. The invention of the theodolite, plane table, and leveling instruments allowed for greater accuracy in land measurement. During the industrial revolution, as cities grew and infrastructure expanded, surveying became an essential profession. The 20th century saw the introduction of EDM (Electronic Distance Measurement) devices, total stations, and later GNSS, which have greatly enhanced both the speed and precision of survey work.

Perimeter Survey: Concept and Relevance

A perimeter survey, also known as a boundary survey, refers to the precise demarcation and documentation of the physical limits of a parcel of land. This type of survey is often the foundation for any legal or cadastral representation of land ownership. According to Ige and Alabi (2017), perimeter surveys are crucial in preparing documents such as Certificates of Occupancy, lease agreements, and deed plans.

Perimeter surveys help prevent and resolve land disputes by clearly defining ownership extents. They are also critical for land subdivision, title registration, compensation evaluation, and land acquisition for public use. Enemark (2009) emphasized that accurate boundary surveys contribute significantly to the development of robust land administration systems (LAS), which are necessary for economic growth and urban sustainability.

Detail Survey: Concept and Application

Detail surveying involves the measurement and recording of the positions of both natural and artificial features within a specific area. These features include buildings, roads, fences, trees, utility poles, drainage channels, and topographic contours. As Adebayo and Salawu (2016) noted, detail surveys are instrumental in producing topographic maps, engineering site plans, and land development models.

Detail surveys are conducted using a combination of horizontal and vertical observations, often supplemented with sketches and photographs. In recent times, the integration of UAVs (drones) has enabled the creation of highly accurate orthophotos and 3D surface models, thereby enhancing the level of detail captured during fieldwork.

Surveying Equipment and Technological Evolution

Surveying instruments can be categorized into traditional, optical, and digital systems. Traditional tools include chains, tapes, and compasses, while optical systems include theodolites and dumpy levels. According to El-Rabbany (2006), electronic instruments like total stations and GNSS receivers now dominate the field due to their speed and precision.

Modern surveying often employs RTK-GPS, laser scanners, and drone-based photogrammetry, making it possible to achieve millimeter-level precision in both horizontal and vertical measurements. The use of AutoCAD, ArcGIS, and Surfer software has also streamlined data processing and presentation.

Legal and Institutional Framework

Surveying in Nigeria is governed by several legal instruments and institutions, including:

Survey Coordination Act of 1962

SURCON Act of 1989

Land Use Act of 1978

According to Onwuka (2014), compliance with these regulations is essential to ensure that survey plans are admissible in court and recognized by government agencies. SURCON (Surveyors Council of Nigeria) regulates the licensure, ethics, and professional conduct of surveyors in Nigeria.

Role of GIS and Spatial Data in Surveying

Geographic Information Systems (GIS) enable the capture, storage, analysis, and visualization of spatial data. Integrating GIS with surveying enhances data utility for decision-making, especially in urban planning, land allocation, and resource management.

Longley et al. (2005) explained that GIS allows surveyors to analyze topological relationships, overlay thematic maps, and manage large datasets efficiently. In perimeter and detail surveys, GIS aids in updating cadastral maps and integrating survey plans with planning databases.

Challenges in Perimeter and Detail Surveying

Despite technological advances, the practice of surveying in Nigeria faces several challenges:

- Lack of Modern Instruments

- Inconsistent Geodetic Control Network

- Bureaucracy in Survey Plan Approval

- Land Encroachment and Informal Settlements

- Inadequate Training and Manpower

As Karikari (2006) observed, the absence of a reliable and unified geodetic network in many parts of Africa makes georeferencing survey data difficult. Additionally, poor power supply and low investment in geospatial infrastructure hamper efficient survey operations.

2.9 Future of Surveying and Emerging Technologies

The future of surveying is being shaped by innovations such as:

Drone-based photogrammetry

3D Laser Scanning

LiDAR (Light Detection and Ranging)

Mobile Mapping Systems

Cloud GIS and Smart Surveying Apps

These technologies promise faster, more accurate, and more cost-effective surveys. According to Meijer (2009), the convergence of AI, IoT, and geospatial technologies will enable real-time decision-making in land and infrastructure management.

CHAPTER THREE

3.0	METHODOLOGY

This is the process where we determine the method to use, instrument to use when to go and how to carry out the project to have a successful work done at the end.

It explain the steps by step procedure equipment and technique in executing the digital Mapping of federal staff school, Adewole Kano road Ilorin. (Data processing, accuracy check and final map production.

- Working from whole to the part.

- The principle of choosing the method of survey most appropriate to meet the desired result.

- The principle of provision for adequate checks to meet the required accuracy

- Planning is divided into two stages:

Office planning

Field planning

3.1	OFFICE PLANNING: Office planning which could be termed as preparation, analyze and organized in the office. office reconnaissance involved knowing the type of instruments, purpose, specification and accuracy require of the survey to be carried out. These led to the choosing of appropriate equipment and method to be employed, also costing of the survey operation was done in the office. Information related to the give project was collected from various sources the coordinate (x, y, and z) of the initial and that of the three choosing controls used for orientation.

Tab. 3.1 shows the value of Controls

| Station | Northing | Easting |
| --- | --- | --- |
| SC/KWI.334R | 938052.240 | 675605.928 |
| SC/KWI.333R | 9377797.689 | 675548.031 |
| SC/KWI.332R | 937809.422 | 675500.648 |

3.1.1	FIELD RECONNAISSANCE

The project site was visited by all the group members to have the true picture of the site for the better planning. The recce diagram was drawn alongside the carrying out and the reasonable artificial features were fixed along and within the traverse lines, the traverse was fixed to maintain perfect indivisibility.

3.2	INSTRUMENT USED

Selection of instrument to be used is:

- Total station

- Tripod

- Linear tape

- Steal tape

- Field book

- Pencil

- Targets and their tripod

- Reflectors stand and target

- Nails

- Pegs

HARDWARE USED

- Total station

- Computer system

SOFTWARE USED

- ArcGIS

- Ms Excel

- Google earth

- Ms word

INSTRUMENT TEST

All instrument used for the executive of the project were tested before the commencement of the field observation inorder to ascertain the efficiency and reliability of the instrument.

3.3	CHECKING OF INSTRUMENT ERROR (TOTAL STATION)

The total station was tested for horizontal and vertical collimination error and the instrument was mounted on a good condition before being used at the point mark (A) with the necessary adjustment. A target was set up on another point and bisect with the cross hair if the total station telescope recording the angles (ie horizontal and vertical).

This is where we exercise for any controls to be used for orientation; the control was checked by observation on the control pillars as to  ascertained stability and reliability both linear and angular.

The check was carried out as follows:-

The total station was set on a pillar, temporary adjustment include centering, leveling, and focusing.

Fig. 3.1: show the instrument test.

Tab. 3.2: Show the result of instrument test

| Inst Stn. | Sight (Reflector) | Face | Hor.Circle Reading | Ver. Circle Reading |
| --- | --- | --- | --- | --- |
|  | B | L | 87º 35’ 10’’ | 88º 26’ 15’’ |
| A | B | R | 267º 35’ 12’’ | 271º 33’ 46’’ |
|  |  |  | Diff= 180º 00’ 02’’ | Sum= 360º 00’ 01’ |

Horizontal Collimation = [(FR – FL) - 180º]/2

= [(180º 00’ 02’ - 180º 00’ 00’’) /2]

= 00º 00’ 02’’/2

= 00° 00’ 01’’

Vertical Collimation      = [(FR – FL) - 360º]/2

= [(360º 00’ 01’’ - 360º 00’ 00’’)/2]

= 00º 00’ 01’’/2

= 00° 00’ 0.5"

3.4 	MONUMENTATION

This is the process where we established and do physical marking of control point on the ground to serve as reference position. These control point are for ensuring accurate and consistency throughout digital Mapping project. The digital Mapping of federal staff school Adewole Kano road Ilorin monumentation formed the foundational frame works of the entire survey

PURPOSE OF MONUMENTATION.

- It  serves as the origin point for horizontal and vertical measurements

- It is a reference point for instrument set up

- It provide permanent marker for future survey

Ground level

18cm

18cm

Beneath Ground level

75cm long

60cm

80cm Iron rod

Above Ground level

15cm

To facilitate proper geo reference and coordinate.

Fig.3.2 typical third order survey beacon

3.5	DATA ACQUISITION

Data acquisition is the next stage after we ave done reconnaissance, this was done on the field and it includes the determination of point’s geometry and attribute value i.e. linear measurement and the coordinating of each station using total station.

3.5.1 INSTRUMENT CHECK

Before carrying out the survey operation, the working condition of the instrument was checked to see if the instrument was tested. This was done by setting the instrument such as centering and parallax elimination were applied measurements was carried out by sighting a target on another station to determine both collimination and vertical error.

During the data acquisition we carry total station instrument to mount it on a tripod and level it by operating the levelling screw within the range so dat we can do temporary adjustment on the level position. We bisect the horizontal, vertical, slope and height of the land. The processor target point and compute the data of the point and display it on screen,It is stored in the electronic book.  During the process of data acquisition we carry out perimeter traverse.

3.5.2 GEOMETRIC DATA ACQUISITION

Geometric data acquisition were obtained using total station ie combination of electromagnetic theodolite as and the electronic distance measurement (EDM). Geometric data are positional data ie (X,Y,Z) coordinate which make it easy to locate their actual position of features on the earth surface.

Detail is a referred to as man-made and natural features on the ground with in the project site which are determined and obtained by using total station and are finally represented with a suitable scale on a plan..

3.5.3 	PERIMETER TRAVERSE AND GEOMETRY POINTS GENERATION

Traverse may be defined as sequence of connected straight lines whose direction and distance has been measured, that is, it involved the determination of the bearings and distance of series of connected straight line from known coordinated point so as to obtain coordinates of the newly established station, this include the following: -

- linear measurement

- angular measurement

| STATION | EASTING (m) | NORTHING (m) | REMARKS |
| --- | --- | --- | --- |
| PT1 | 675945.300 | 940823.730 | Established |
| PT2 | 676048.460 | 940788.200 | Established |
| PT3 | 675981.000 | 940628.000 | Established |
| PT4 | 676020.620 | 940593.770 | Established |

3.6	DATA PROCESSING

This is the method in retrieving, downloading, sorting, and analysis of the acquired data (field data), the data is being downloaded from the total station to a computer system and processed into information using the appropriate method and software.

This simply refers to the graphical representation i.e. plotting of plan. it was plotted using AutoCAD and Ms-word software in a computer system and a suitable scale was used to have the hard copy format. presented information includes; boundary, details and pegs. conventional signs and symbols were also used to represent features of the plan accordingly. The digital map was produced using AutoCAD software and following the under listed procedures;

- switch on the computer and it was allowed to boot

- start menu was clicked

- select programs was clicked

- from the notepad, a script files for the coordinate as p-line easting, northing, was structured

- file was saved with the extension. scr.

- AutoCAD was launched

- file menu was clicked

- sub menu [news] was clicked and the name was saved

- format was clicked and all necessary settings were carried out [i.e. units, direction etc.]

- then ‘ok’ was clicked to aspect the parameters settings

- tools were selected

- run script was clicked on

- escape key was clicked, z enter and e enter were pressed one after the other in order to zoom the extent of the plan being drawn and the plotted plan was displayed

- text was clicked

- escape key was pressed, Z then E enter key

- text writing and other necessary editing were done

- coordinates of the details were all typed

- coordinates were pasted and then the points were all displayed

- With polyline the points were joined as they were sketched.

CHAPTER FOUR

DATA PROCESSING AND ANALYSIS OF RESULT

Data processing is also referred to as the computation stage. It is the intermediary between the field observation and data presentation stage. At this stage, all the data acquired from the field were processed and analyzed in order to proceed to the final stage.

Data processing and analysis comprises of the following

- Traverse field book reduction

- Traverse computation

- Computation of leveling

- Detailing computation

- Traverse field book reduction – Angular

STEPS IN DATA PROCESSING

- Data Collection (Field Work)

- Perimeter Survey: Involves identifying and measuring the boundary lines of the school property using surveying instruments like a Total Station, GPS, or Theodolite.

- Detail Survey: Captures topographic features within the boundary (e.g, buildings, road, trees, drainage, fences, utility poles.)

4.1 DATA DOWNLOAD AND EDITING

This is the transfer of data from the memory unit of digital instrument into the computer system for the processing and storage stage for easy retrieval. The total station was connected to the computer through a data transfer cable using a data processing software for the downloading.

DATA TRANSFER

Data collected during the fieldwork were downloaded from the Total Station to a computer system through a USB connection using the instrument’s proprietary software. The exported data included;

- Horizontal and Vertical angles

- Distances between survey points

- Coded representing different features (building, trees and roads etc)

- Control and detail points coordinates.

DATA EDITING

- Error Checking: Field notes were cross checked with downloaded data to detect and correct any inconsistencies

- Traverse Adjustment: The survey was adjusted to minimize closure error

- Reclassification: features codes were verified and edited to correspond with standard plotting symbol

4.2 DATA PROCESSING IN AUTOCAD

To open AUTOCAD software and following the underlisted procedure:

* Switch on the computer and allowed it to boot

* Start menu was clicked

* Select program was clicked

* From the Notepad, a script files for the coordinates, line, text and other was structured.

* Files was saved with the extension Script.

* AUTOCAD was launched

* File menu was clicked

* Sub menu (New) was clicked and the name was saved.

* Format was clicked and all necessary settings were carried out (i.e units, dimension etc.)

* Then 'OK' was clicked to the aspect of parameter setting.

* Tools were selected

* Run script was clicked on.

* Escape key was clicked, Z enter and E enter were pressed three after the other.

* Text was clicked

* Escape key was pressed, Z then E enter.

* Text writing and other necessary editing were done.

* Coordinates of the details were all typed.

* Coordinates were pasted and then the point were all displayed.

* With polyline the point were joined as they were sketched.

4.3 AREA COMPUTATION

|  | ΔE | ΔN |
| --- | --- | --- |
| 2 – 1 | +103.16 | -35.53 |
| 3 – 2 | -67.46 | -16.02 |
| 4 – 3 | +39.62 | -34.23 |
| 1 – 4 | -75.32 | +229.96 |

Using Double Latitude and Departure

+103.16   x   - 35.53		= - 3665.275

+103.16

+206.32

- 67.46   x   - 106.20		= + 10807.092

+138.86

- 67.46

+71.400

+  39.62   x   - 34.23		= - 1356.193

+111.02

+  39.62

+150.64

- 75.32   x   + 229.96		= + 34641.174

+   75.32

- 75.32

00.00

Sum of + - Sum of –

2

= (10807.092 + 34641.174) – (-3665.275 + (-1356.193)

2

Sum of Positive 	= 45448.266

Sum of Negative 	= 5021.468

A = 45448.266 – 5021.468

2

A = 40426.798	= 20213.399 square

2

= 20213.399		= 2.021 Hectares

10,000

=4.99 Acres

| Stn. | Bearing | Distance | Δ East | Δ North | Easting | Northing | Stn. |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | 675605.928 | 938052.240 | A |
| A | 073˚ 10’ 02’’ | 110.05 | -57.891 | -254.551 | 675548.031 | 937793.689 | B |
| B | 185˚ 19’ 46’’ | 120.54 | -150.574 | 34.798 | 675397.457 | 937832.487 | C |
| C | 254˚ 09’ 58’’ | 110.78 | 103.191 | -23.065 | 675500.648 | 937809.422 | D |
| D | 345˚ 16’ 25’’ | 121.66 | 105.280 | 242.819 | 675605.928 | 938052.240 | A |

4.4 BACK COMPUTATION

4.5 PRODUCT APPLICATION

Uses of Perimeter Plan

- Perimeter Plan is used for building of site or plan for construction

- Perimeter Plan is used to manage and maintain facilities including defining boundaries and access point.

Uses of Detail Plan

- Detail plan is regularly used when designing for roads, buildings, extension and other new infrastructure.

- It is used to show the location and height of any number of varieties of features of an area.

4.6 COSTING

RECCI

| S/N | Personnel | Quantity | Daily Rate | Days | Remark |
| --- | --- | --- | --- | --- | --- |
|  | Principal Surv. | 1 | 40,000 | 1 | 40,000 |
|  | Sen Surv. | 1 | 30,000 | 1 | 30,000 |
|  | Asst. Surv. | 1 | 18,000 | 1 | 18,000 |
|  | Basic Equipment | 1 | 18,000 | 1 | 18,000 |
|  | Transportation | 1 | 18,000 | 1 | 18,000 |
| TOTAL |  |  |  |  | N124,000 |

BEACON = 5,000 x 4 = 20,000

BEACONING/EMPLACEMENT OF PROPERTY BEACON

| S/N | Personnel | Quantity | Daily Rate | Days | Remark |
| --- | --- | --- | --- | --- | --- |
|  | Asst. Surv | 1 | 18,000 | 1 | 18,000 |
|  | Skilled Labour | 4 | 10,000 | 1 | 40,000 |
|  | Unskilled Labour | 3 | 8,000 | 1 | 24,000 |
|  | Transportation | 2 | 18,000 | 1 | 36,000 |
|  | Basis Equipment | 1 | 18,000 | 1 | 18,000 |
| TOTAL |  |  |  |  | N136,000 |

TRAVERSING & CORRECTION TO CONTROL

| S/N | Personnel | Quantity | Daily Rate | Days | Remark |
| --- | --- | --- | --- | --- | --- |
|  | Sen. Surv | 1 | 30,000 | 1 | 30,000 |
|  | Asst. Surv. | 1 | 18,000 | 1 | 18,000 |
|  | Skilled Labour | 4 | 10,000 | 1 | 40,000 |
|  | Unskilled Labour | 3 | 8,000 | 1 | 24,000 |
|  | Transportation | 2 | 18,000 | 1 | 36,000 |
|  | Basis Equipment | 2 | 18,000 | 1 | 36,000 |
| TOTAL |  |  |  |  | N184,000 |

PLOTTING & DRAFTING (TOPOGRAPHY)

| S/N | Personnel | Quantity | Daily Rate | Days | Remark |
| --- | --- | --- | --- | --- | --- |
|  | Principal Surv. | 1 | 40,000 | 2 | 80,000 |
|  | Senior Surv. | 1 | 30,000 | 2 | 60,000 |
|  | Asst. Surv. | 1 | 18,000 | 2 | 36,000 |
|  | System | 1 | 46,000 | 2 | 92,000 |
|  | Consumable (Paper) | 1 | 15,000 | 2 | 30,000 |
| TOTAL |  |  |  |  | N298,000 |

TECHNICAL REPORT

| S/N | Personnel | Quantity | Daily Rate | Days | Remark |
| --- | --- | --- | --- | --- | --- |
|  | Principal Surv. | 1 | 40,000 | 1 | 40,000 |
|  | Senior Surv. | 1 | 30,000 | 1 | 30,000 |
|  | Asst. Surv. | 1 | 18,000 | 1 | 18,000 |
|  | System | 1 | 46,000 | 1 | 46,000 |
|  | Consumable (Paper) | 1 | 15,000 | 1 | 15,000 |
|  | Secretary | 1 | 18,000 | 1 | 18,000 |
| TOTAL |  |  |  |  | N167,000 |

Cumulative Total = N929,000

ESTIMATE

- Accommodation 1.5% = 1.5 x 929,000 = 13,935

100

- Mobilization/D. Mob = 10% =  10  x 929,000 = 92,900

100

- Contingencies = 5% =  5  x 929,000 = 46,450

100

- V.A.T = 7.5% = 75  x 929,000 = 69,675

100

Then Assumed Total + Acct + Mob/D mob + Contingencies + VAT = Amount of Charge (For the Project)

929,000 + 13,935+ 92,900+ 46,450 + 69,675 =

= N1,151,960

CHAPTER FIVE

SUMMARY, CONCLUSION AND RECOMMENDATIONS

5.1. SUMMARY

This project was centered on the Perimeter and Detail Survey of the Federal Staff School, Adewole, Ilorin, with the objective of producing an accurate map showing the boundaries and physical features of the school premises. The project followed a professional approach that included reconnaissance, control establishment, field data acquisition, data processing, and map production.

A comprehensive literature review was carried out In Chapter Two, which discussed the fundamental concepts, methodologies, equipment, technological innovations, and regulatory frameworks that underpin perimeter and detail surveys. Chapter Three focused on the methodology, detailing the step-by-step procedures adopted for the survey. It included the reconnaissance visit, selection and establishment of control points, traversing, detailing, booking observations, and applying relevant checks. Chapter Four presented the results and analysis, including maps, coordinates, and feature interpretations.

The fieldwork was executed using modern surveying equipment such as the Total Station and GPS devices. Data were processed using software including AutoCAD, ArcGIS, and Microsoft Excel to compute coordinates and generate the final plan.

The results of this project demonstrate that accurate field observation and data processing can yield reliable plans that can serve multiple purposes such as planning, development control, and documentation of school infrastructure.

This perimeter and detail survey provided an accurate spatial representation of the Federal Staff School, Adewole, Ilorin. The survey map generated captures the school boundaries as well as various internal features such as buildings, fences, roads, drainages, trees, electric poles, and open spaces. The project underscores the importance of perimeter and detail surveying in urban development, land management, and public facility planning. With the continuous growth of the educational sector in Nigeria, especially in urban areas like Ilorin, such surveys are essential for expansion, redevelopment, and infrastructural planning.

The project also highlights the relevance of modern surveying techniques and tools in producing fast, accurate, and comprehensive survey outputs. Integration with geospatial software further enhances the usefulness of the data collected.

5.2 CONCLUSIONS

To conclude, the project was effectively carried out, achieving results that aligned with the accuracy and criteria of third-order survey standards.

Adequate and precise data was gathered, analyzed, and presented in the completed drawings. All essential calculations and procedures were accurately performed and documented …were conducted in accordance with established protocols, strictly following recognized surveying procedures and departmental standards.

5.3 RECOMMENDATIONS

Based on the experience gathered from this project, the following recommendations are offered:

- Periodic Surveying of Public Properties: Institutions like schools should be surveyed periodically to monitor encroachment, plan expansion, and update records.

- Investment in Modern Survey Tools: Organizations and government agencies should equip survey departments with up-to-date equipment such as GNSS, total stations, and drones for efficient and accurate surveys.

- Capacity Building: Continuous training and capacity development should be encouraged among survey professionals and students to stay updated with evolving technologies.

- Incorporate GIS in School Management: School administrators can benefit from integrating GIS in managing their facilities, utilities, and land.

- Digitization and Archiving: All survey records should be digitized and stored in retrievable formats to ensure longevity and easy access to spatial information.

- Proper Planning Before Fieldwork: Adequate reconnaissance and planning help reduce unexpected issues during data collection.

5.4 CHALLENGES ENCOUNTERED

Several challenges were encountered during the course of this project. These include:

Obstructions on Site: Vegetation and human activity in certain areas hindered direct line-of-sight measurements.

Instrument Calibration: Time was spent ensuring all instruments were properly calibrated before data collection.

Access Restrictions: Gaining full access to certain areas of the school required administrative permissions and delayed part of the fieldwork.

Power Supply: Limited power supply for charging electronic instruments and running computer systems occasionally disrupted processing.

Despite these obstacles, all necessary procedures were eventually completed successfully.

SUGGESTIONS FOR FURTHER STUDY

Future research and projects can explore:

The use of Unmanned Aerial Vehicles (UAVs) for detailed topographic mapping of school and campus environments.

Incorporation of 3D laser scanning and BIM (Building Information Modelling) in the mapping of educational institutions.

Establishing a School GIS Database System for asset and infrastructure successful

This project contributes to knowledge by:

- Demonstrating the practical application of surveying theories in a real-world environment.

- Producing a reliable survey plan for future planning and development of the Federal Staff School, Adewole.

- Encouraging the use of digital tools in land survey projects, enhancing efficiency and productivity in the profession.

REFERENCE

Allan, A. L. Hollwey, J.R. Marynes, J.H.B. (1968): Practical Field Surveying and     Computation, Heinemann, London. Pg.201.

Banister A. and Raymond S. (1986): Surveying 4th Edition London, pitman publishing limited.

Basak, N.N (2005): Surveying and levelling, 4th Edition. Tata mac graw publisher. Great Britain

Dashe J. P. (1987): cadastral surveying and practice in Nigeria.

Duggal, S.K (2006): Surveying 3rd Edition, Addision Wesley longman (united Edinburgh gate, halow England, pp 101 Ghana C.D and wolf P.R (2008). Elementary surveying, pearson Edition.

Demers, M.N (1997):	Fundamental of Geographical information system join willey and sons inc. New York pg 20

Encarta encyclopedia (2009): www.google.com

Eruteya E. O. (2009): basic principles in surveying handout

Fajemirokun, F.A (1996): The profession of surveying in Nigeria and its contribution to National development in graduate surveyor. 1996, 8th Edition. Pg 8.

H.H.P (2005):	125 history of mapping, date accessed, October 13th 2013.

Kufoniyi (1999): Paper presentation at Nigeria Institution of Surveyors AGM at Akwa, 28th-30th April, 1999. Pg 6

Oxford advance leaner’s dictionary (2001): 6th edition oxford university press Walton Street.

Punmia, B.C (2005): Surveying vol 1 Lakimi publisher, New Delhi page 56.

Web Knowledge Online (2003): Assorted that topographical surveying are three dimensional that employed the techniques of plane surveying and other special techniques to establish both horizontal (X,Y) and vertical (Z) control.

Appendix

| ID | EASTINGS | NORTHINGS |  |
| --- | --- | --- | --- |
| B1 | 666472 | 937402.5 |  |
| B2 | 666484.9 | 937400.9 |  |
| B3 | 666483.2 | 937390.2 |  |
| B4 | 666470 | 937391.8 |  |
| B5 | 666488.5 | 937412 |  |
| B6 | 666516 | 937406.7 |  |
| B7 | 666514.7 | 937395 |  |
| B8 | 666486.5 | 937399.9 |  |
| B9 | 666493.4 | 937431.3 |  |
| B10 | 666505.4 | 937428.8 |  |
| B11 | 666501.4 | 937411.7 |  |
| B12 | 666489 | 937414.4 |  |
| B13 | 666502.4 | 937464.7 |  |
| B14 | 666512.1 | 937462.9 |  |
| B15 | 666505.4 | 937428.8 |  |
| B16 | 666495.4 | 937431.5 |  |
| B17 | 666496.1 | 937473.4 |  |
| B18 | 666501.2 | 937472.2 |  |
| B19 | 666499.9 | 937465.1 |  |
| B20 | 666494.2 | 937467 |  |
| B21 | 666502.1 | 937480.3 |  |
| B22 | 666555 | 937468.8 |  |
| B23 | 666553.3 | 937459.1 |  |
| B24 | 666501 | 937470.5 |  |
| B25 | 666556 | 937468.2 |  |
| B26 | 666563.7 | 937466.8 |  |
| B27 | 666560.9 | 937455.8 |  |
| B28 | 666553.1 | 937457.4 |  |
| B29 | 666536.1 | 937394 |  |
| B30 | 666550.7 | 937457.5 |  |
| B31 | 666563.2 | 937454.9 |  |
| B32 | 666548.6 | 937390.6 |  |
| B33 | 666520.1 | 937405.8 |  |
| B34 | 666536.3 | 937403.6 |  |
| B35 | 666534.7 | 937393.3 |  |
| B36 | 666516.3 | 937395.6 |  |
| B37 | 666516.6 | 937401.6 |  |
| B38 | 666519.4 | 937401.4 |  |
| B39 | 666512.5 | 937495.1 |  |
| B40 | 666559 | 937484.2 |  |
| B41 | 666556.4 | 937472 |  |
| B42 | 666509.4 | 937484.4 |  |
| B43 | 666508.3 | 937512.3 |  |
| B44 | 666546 | 937503.8 |  |
| B45 | 666541.7 | 937489.7 |  |
| B46 | 666505.1 | 937499.1 |  |
| B47 | 666512.5 | 937530.7 |  |
| B48 | 666550.1 | 937522.3 |  |
| B49 | 666546 | 937506.4 |  |
| B50 | 666508.8 | 937514.8 |  |
| B51 | 666552.2 | 937521.6 |  |
| B52 | 666564 | 937518.3 |  |
| B53 | 666556.3 | 937489.5 |  |
| B54 | 666545.2 | 937493.2 |  |
| B55 | 666566.8 | 937517.6 |  |
| B56 | 666577.2 | 937515 |  |
| B57 | 666569.7 | 937485.5 |  |
| B58 | 666559.1 | 937489.6 |  |
| B59 | 666574.3 | 937526 |  |
| B60 | 666604 | 937520 |  |
| B61 | 666601.9 | 937508.7 |  |
| B62 | 666572.4 | 937516.2 |  |
| B63 | 666612.9 | 937520.2 |  |
| B64 | 666617.9 | 937519.7 |  |
| B65 | 666617.2 | 937510.3 |  |
| B66 | 666611.8 | 937511.1 |  |
| B67 | 666612.3 | 937503 |  |
| B68 | 666617 | 937502.6 |  |
| B69 | 666615.9 | 937491.5 |  |
| B70 | 666611.5 | 937492 |  |
| B71 | 666470.8 | 937486.2 |  |
| B72 | 666474.4 | 937486.1 |  |
| B73 | 666474.1 | 937481.5 |  |
| B74 | 666470.7 | 937481.7 |  |
| B75 | 666473.3 | 937500.4 |  |
| B76 | 666476.9 | 937500.3 |  |
| B77 | 666476.9 | 937497.6 |  |
| B78 | 666473 | 937497.8 |  |
| B79 | 666472.1 | 937517.6 |  |
| B80 | 666476.4 | 937517.2 |  |
| B81 | 666475.8 | 937510.8 |  |
| B82 | 666471.9 | 937511.4 |  |
| B83 | 666472.7 | 937535 |  |
| B84 | 666478.9 | 937534.7 |  |
| B85 | 666478.1 | 937526.2 |  |
| B86 | 666472.2 | 937526.3 |  |
| B87 | 666479.4 | 937539.6 |  |
| B88 | 666485.9 | 937539.1 |  |
| B89 | 666485.9 | 937535.6 |  |
| B90 | 666479.2 | 937536.3 |  |
| B91 | 666495 | 937536.2 |  |
| B92 | 666508 | 937534 |  |
| B93 | 666505.6 | 937519.6 |  |
| B94 | 666492.6 | 937522.1 |  |
| PL1 | 666472.2 | 937540.7 |  |
| PL2 | 666618.3 | 937521.3 |  |
| PL3 | 666607.4 | 937371.4 |  |
| PL4 | 666468.4 | 937392 |  |
| RD 1 | 666651 | 937536.1 |  |
| RD2 | 666470.5 | 937557.3 |  |
| RD3 | 666469.8 | 937584.6 |  |
| RD 4 | 666651.8 | 937521.6 |  |
| RD 5 | 666469.2 | 937545.7 |  |
| RD 6 | 666463.1 | 937332.5 |  |
| RD 7 | 666445.1 | 937341.2 |  |
| RD 8 | 666452.1 | 937584.6 |  |
| RD 9 | 666464.5 | 937381.6 |  |
| RD 10 | 666609.9 | 937359.6 |  |
| RD 11 | 666464.5 | 937381.6 |  |
| RD 12 | 666609.9 | 937359.6 |  |
| RD 13 | 666624.2 | 937525.6 |  |
| RD 14 | 666608.8 | 937346.4 |  |
| RD 15 | 666637.5 | 937523.5 |  |
| RD 16 | 666619.4 | 937345.5 |  |