Office Security Alarm using SMS Alert and Motion Detection

BY:

ABIOYE SAMUEL SUNDAY

ND/23/COM/FT/0128

A Project Submitted to the Department of Computer Science, Institute of Information and Communication Technology, Kwara State Polytechnic, Ilorin

In Partial Fulfillment of the Requirements for the Award of National Diploma (ND) in Computer Science

May, 2025

CERTIFICATION

This is to certify that this project was carried out by ABIOYE SAMUEL SUNDAY with matriculation number ND/23/COM/FT/0128 in the Department of Computer Science, Institute of Information and Communication Technology, Kwara State Polytechnic, Ilorin.

_______________________					___________________

Mr. Bolaji Adetoro D.F			            	  	   	Date

(Project Supervisor)

_______________________				         ___________________

Mr. Oyedepo F. S.				  	 	            Date

(Head of Department)

_________________________				      ____________________

External Examiner				        			Date

DEDICATION

This project is dedicated to the creator of the earth and universe, the Almighty God. It is also dedicated to my parents for their moral and financial support.

ACKNOWLEDGEMENT

All praises unto Almighty God the Lord of the Universe. I praise Him and thank Him for giving me the strength and knowledge to complete my ND programme and also for my continue existence on the earth.

I appreciate the utmost effort of my supervisor, Dr. Funsho whose patience support and encourages have been the driving force behind the success of this research work. She gave useful corrections, constructive criticisms, comments, recommendations, advice and always ensures that an excellent research is done. My sincere gratitude goes to Head of the Department and other members of staff of the Department of Computer Science, Kwara State Polytechnic, Ilorin, for their constant cooperation, constructive criticisms and encouragements throughout the programme.

Special gratitude goes to my parents who exhibited immeasurable financial, patience, support, prayers and understanding during the periods in which I was busy tirelessly in my studies. Special thanks go to all my lovely siblings.

My sincere appreciation goes to my friends and classmates.

TABLE OF CONTENTS

Title Page 										i

Certification 									ii

Dedication 										iii

Acknowledgements								iv

Abstract 										v

Table of Content									vi

CHAPTER ONE: GENERAL INTRODUCTION

  - Background to  the Study							1

  - Statement of the Problem							3

  - Aim and Objectives of the Study					3

  - Significance of the Study							4

  - Scope of the Study								4

  - Organization of the Report						5

CHAPTER TWO: LITERATURE REVIEW

2.1	Review of Related Works						7

2.3 	Review of General Text							10

CHAPTER THREE: RESEARCH METHODOLOGY AND ANALYSIS

  - Research Methodology							14

  - Analysis of the Existing System 					16

  - Problems  of the Existing System					16

  - Description of the Proposed System					17

  - Advantages of the proposed system					17

CHAPTER FOUR: DESIGN, IMPLEMENTATION AND DOCUMENTATION OF THE SYSTEM

  - Design of the System 							18

    - Output Design 							18

    - Input Design 							19

    - Procedure Design 							21

  - Implementation of the System 						21

    - Choice of Programming Language				21

    - Hardware Support 							22

  - Documentation of the System 						22

    - Maintaining the System						22

CHAPTER FIVE: SUMMARY, CONCLUSION AND RECOMMENDATION

  - Summary 									23

  - Conclusion 									23

  - Recommendation								24

References 									25

Appendices

Appendix 1		Object Detection Flowchart

Appendix 2		SMS Alert Flowchart

ABSTRACT

The comfort of being able to secure your office through a sensory system is imperative as it saves a lot of time and efforts. Ordinary human body motion detective security system without GSM Module has drawbacks of limited working alert range; however, the incorporation of GSM module overcomes this limitation. It provides comfort of being able to secure your house through a sensory, control over certain things that required constant attention. Instead of using a watchman that might eventually sleep off on post. Remotely the system allows the user to effectively monitor the house even when the house owner is at work; he can easily put his mind at rest. This project presents dependable security system that is able to recognize human body (intruder) motion and send a notification text message to inform the owner(at any location in the world where there is GSM mobile network coverage) The system design is in three main phases: generally, the sensory, processing and the task. The sensory is the perception section that is done through PIR sensor mounted at watch-area, the processing is done by a programmed microcontroller, and the action (task) is done through an interaction of an attached GSM module to the processor (the microcontroller) that then send an SMS alert to the user.

CHAPTER ONE

GENERAL INTRODUCTION

  - BACKGROUND TO THE STUDY

Home security has been of worldwide concern. As the technology is emerging every second, abundant home based security systems have been developed and implemented to keep their welfare safe. Home security system is an essential mean of protecting our home from illegal invasion. A conventional home security system consists of a Closed Circuit Television, CCTV and burglar alarm. CCTV captures video in 24 hour to identify what goes on around the house and in the house as well as get a hold of the evidence if there is a house breaking around the captured areas. Burglar alarm acts as the tool to alert the house owners and their neighbours. In additional, it may also chase away the burglar as the system may emit a high frequency sound wave. Nevertheless, the memory consumption is considerably large as the camera keeps recording non-stop. The power consumption is considered as a concern of installing a security system (Nicholas, et al., 2020).

Bangali and Shaligram claimed that the comfort of being able to secure your house through a sensory system is imperative as it saves a lot of time and effort. Ordinary, human body motion detective security system without GSM Module has drawbacks of limited working alert range; however, the incorporation of GSM module overcomes this limitation. It provides comfort of being able to secure your house through a sensory, control over certain things that required constant attention. Instead of using a watchman that might eventually sleep off on post. Remotely the system allows the user to effectively monitor the house even when the house owner is at work; he can easily put his mind at rest.

Advances in electronic and informatics technology have also helped in the development of reliable security systems. Many electronic devices are used for shop security systems. Therefore a security system is needed to maintain security at all times in order to protect the asset owned. By utilizing this system, it is expected to protect from theft or intruder, which lately are rife, causing security every time. This system is used to detect security conditions in a store using an alarm as a warning sign and notification via short message to the shop owner itself. This security system is made to know the motion of a human object when the store is out of operating hours. This system has the advantage of using Passive Infra Red (PIR) sensor that can detect infrared radiation emitted by the human body so that it can detect even dark conditions (Nwalozie et al., 2019).

It must be taking into account the actions of people attempting to trespass or cause destruction. Security is the degree of protection against danger, loss and criminals. Throughout history, human have sought to protect their life, property and professions .The busy lifestyle of people is leading to the necessity of controlling the devices at home remotely and increasing the necessity of keeping surveillance over their home. Now when everything is available to us we can now design something that provides us complete security. The very basic of this began with the simple alarm systems which include notifying suspicious activities at a very low cost. The need for security system have rapidly grown from being specialized for high risk areas such as banks, companies, governmental institutions), to be available and demanded rapidly by the average public. Some of the common characteristics of motion detection alarm and security system as follow 24 hours monitoring, difficult to hack, Ability to control doors and motion sensor. The motion sensor used in this work is the electric device that detects motion by measuring changes in the infrared levels emitted by surrounding objects. This motion can be detected by checking for a high signal on a single I/O pin (Iyapo, 2018).

  - STATEMENT OF THE PROBLEM

The security devices is to prevent an intruder from gaining access to a location. Most of these gadgets however have loop holes which give an unauthorized individuals access to where they are excluded; that is, the CCTV camera will only record an intruder and the owner may not be available to view it on screen. If an intruder get access to an office then, all the resources in that office is at risk. This project aimed to solve these problems by installing a device that will detect motion and send short message service to the office owner. When an intruder moves near the device, it identifies the movement and sends a message to the office owner mobile number.

  - AIM AND OBJECTIVES

The aim of this project is to develop an office security alarm using SMS alert and motion sensors. The objectives are to:

- implement a system that is capable of identifying an intruder and send information to user regarding a break- in at their office/resident;

- install a security device which have low cost, easy installation, fast response, and low power consumption and a better way of security; and

- implement a system that will eliminate the need of watchman to guard an office.

  - SIGNIFICANCE OF THE STUDY

The importance of this project is to safeguard an office by using an advance technology. This project “office security alarm with SMS alert and human body motion detective system” will help to prevent an intruder from entering into the important office/chamber using GSM networks, an alert system that will be embedded into the system which can be used to alert the owner through SMS anytime an unauthorized movement is detected. Remotely, the system allows the user to effectively monitor the house/office via the GSM module by sending commands in the form of SMS messages.

  - SCOPE OF THE STUDY

The office alarm security using SMS notification and motion detection system is a real-time security system for the office that is capable of securing the house/office through a sensory system which is imperative as it saves a lot of time and effort. Ordinary, human body motion detective security system without GSM Module has drawbacks of limited working alert range; however, the incorporation of GSM module overcomes this limitation. It provides comfort of being able to secure your house through a sensory, control over certain things that required constant attention. Instead of using a watchman that might eventually sleep off on post. The goal is to reduce the stress of the security guard and the office owner by detecting an authorized body from intruding the office.

1.6 ORGANIZATION OF THE REPORT

The project write-up is organized into five distinct chapters. Chapter one covers general introduction, which contains introduction to the research topic, statement of the problem, aim and objectives, significance of the study, scope and limitation and organization of the report. Chapter two focuses on literature review, which contains review of related past works, overview of office security alarm using SMS alert and motion detection and review of related concept. Chapter three explains the project methodology which includes analysis of existing system, problems of the existing system, and the description of the proposed system, advantages of proposed system and design and implementation techniques used. Chapter four centres on the design, implementation and documentation of the system which contain system design output design, input design, database design and procedure design, implementation of the system hardware and software support and documentation of the new system installation procedure, operating the system and system maintenance. Lastly, chapter five contains the summary of the research, recommendations and conclusion.

CHAPTER TWO

LITERATURE REVIEW

2.1	REVIEW OF RELATED LITERATURE

Designed and implemented an intruder detector system with SMS alert. The project was designed and constructed using an ATmega8 microcontroller, system connected to a GSM module with SIM (subscriber identification module) from a network provider. Light dependent resistor (LDR) sensor which is used in this project is connected to a microcontroller unit, GSM module and other electronic devices which detects any intruder in a classified area and then activates the alarm system and sends out text message notifications to the house owner specifying the location and time of intrusion to the programmed phone number of the house owner. The initial testing of the design shows that it worked in real-time in area with good network coverage as expected. It has wide range of application in surveillance especially in industrial and home security, especially when internet connectivity is not available. (Nicholas et al., 2020)

A home security system with text alert and motion detective. The paper presented dependable security system that was able to recognize human body (intruder) motion and send a notification text message to inform the owner (at any location in the world where there is GSM mobile network coverage). The system design is in three main phases: generally, the sensory, processing and the task. The sensory is the perception section that is done through PIR sensor mounted at watch-area, the processing was done by a programmed microcontroller, and the action (task) is done through an interaction of an attached GSM module to the processor (the microcontroller) that then send an sms alert to the user. (Olalekan et al,. 2019)

Work on enhancing home security using SMS-based intruder detection system. The research project aimed at the design and construction of a Short Message Service (SMS)-based intruder detection system, the system consisted of controller, receiver and sensor circuit. This system unlike the traditional magnetic switch alarms equipped on doors and windows has incorporated motion sensors so that a short message service, SMS is sent to the house owner on any attempt of a break. The project was built using a programmed microcontroller interfaced with SIM 900 module, motion detectors and switches. The Passive Infrared (PIR) sensor which is the motion detector used in this project is placed at the roof of the prototype and a switch close to the door, so that when an intruder pass through the PIR or press the door switch a message is displayed on the liquid crystal display and SMS is sent to the phone numbers embedded in the c language program use to program the microcontroller. (Nwalozie et al., 2017)

Designed and implemented of motion detection alarm and security system. The project was built using an embedded microcontroller system capable of detecting motion of an intruder in a restricted area and then triggering an alarm system, motion detector system, however passive infrared sensor detected the motion of the person using the person body heat. The passive infrared (PIR) sensor which is the motion detector used in this project is attached to a microcontroller which activates the alarm system and any other attached output device to notify the house owner. The initial testing of the design shows that it worked as expected. This was achieved by mechanical devices that physically interact with the field or by electronic device that quantifies and measures changes in the given environment. The motion detector is not only used as intruder alarm but also used in many applications like home automation system, energy efficiency system. (Iyapo et al., 2018)

Proposed motion detector security system for indoor geo-location. The system focused on the development of a microcontroller based security system for indoor geo-location using motion detector. The system carried out intruder detection that reinforces surveillance technology to provide essential security with associated control and alert operations. The security pivots on the integration of cameras and motion detectors into web application. The Raspberry Pi, a smart surveillance system, takes input from the motion detector and controls (actuates) the pi camera for remote sensing and surveillance, sends the video to a web server which allows the user or homeowner to access the videos using a web application. The system, on intrusion alerts the owner by SMS, buzzes the alarm located at a convenient distant. The designed security system is characterized with efficient video camera for remote sensing and surveillance, featured with stream live video and records for subsequent replay and offers a cost effective ubiquitous surveillance solution, efficient and easy to implement. (Nosiri et al., 2018)

Proposed a space security system using motion sensor and notification of short message service with arduino-based fuzzy logic algorithm. There is need for security system that can provide up-to-date information in real time to detect human movements within the store when it is outside its operating hours. This system was built using Passive Infra-Red Sensors (PIR) which are connected to the microcontroller as the central control system to detect human movements outside the store's operating hours based on the results of design and testing, the system was used properly and provides real time notifications to users. When the sensor detects human movement outside the store's operating hours, the system will send an SMS notification to the shop owner's mobile number. (Darmawan and Taufan 2019).

Work on design and implementation of security systems for smart home based on GSM technology. This paper suggested two methods for home security system. The first system uses web camera. Whenever there is a motion in front of the camera, it gives security alert in terms of sound and a mail is delivered to the owner. The second method sends SMS which uses GSMGPS Module (sim548c) and Atmega644p microcontroller, sensors, relays and buzzers. (Bangali and Shaligram, 2016)

2.2	REVIEW OF RELATED CONCEPTS

2.2.1	Security Alarm

A security alarm or detection system is precisely a system by which one or something is sheltered through a system of interworking devices and components called the internet of things (IoT). When a user activates the system, the Arduino microcontroller will read the data when the PIR sensor detects an intruder. The Arduino Uno Microcontroller activates the buzzer and sends a signal to the LCD Showing the intruding. The GSM modem gets the information from the Microcontroller it will send a short message service to the designated Mobile number registered in the system. To communicate between the Mobile phone and GSM modem, AT command is applied to this research. This is because the GSM modem can only understand AT command declaration. From this, it can communicate with Mobile phones, computers, and Arduino microcontrollers. To accomplish this project, the whole component must work effectively. The GSM modem acts as a medium to receive the instructions from the microcontroller and Sends a message to the designated mobile number. Some security has changed a lot from the last century and will be changing in coming years. Security is an important aspect or feature in the smart home applications. The new and emerging concept of smart homes offers a comfortable, convenient, and safe environment for occupants. Conventional security systems keep homeowners, and their property, safe from intruders by giving the indication in terms of alarm. However, a smart home security system offers many more benefits (Kartik, Kumar, & Srimadhavan.

2.2.2 GSM

GSM is a specialized type of modem which accepts SIM card, and operates over a subscription to mobile operators. When the GSM modem and computer are interconnected, there is communication over the mobile network. Though these GSM modems are most used to provide mobile internet connectivity, most of them can also be used for sending and receiving SMS and MMS messages. This device can also receive and process GSM signals from virtually all GSM bands. The GSM module will help to send Message to the user and also receive instruction from the user cell phone for reprogramming the phone number

2.2.3	Motion Detection

Motion detectors are mainly used in security systems. It is typically positioned near exterior doorways or windows of a building to monitor the area around it. Since motion detectors are so flexible and have so many uses, it offers feelings of protection and security for the average homeowner as well as commercial organizations. An electronic motion detector is a device used to detect any physical movement in a given area and transforms motion into an electric signal. It consist of sensor that electrically connected to other devices such as security system, lighting, audio alarms, and other applications. Motion sensors are used in a wide variety of applications and as a result, many different types of motion sensors are available including the infrared sensor. Infrared sensors are widely known in the arts of intrusion detection and in fire or smoke detection. It is a device that often used in automatic light switches and security systems to turn on a light or to activate some other form of alarm or warning indicator when a person enters a monitored area. The infrared sensors have basically two forms: active and passive. An active infrared detector includes a radiation source and an infrared sensor which is sensitive to interruptions in the radiation sensed from the source. These detectors are used as intrusion detectors by providing a path of radiation from the source to the sensor in a place where the path is likely to be interrupted by an intruder.

2.2.4	Motion Sensor/Detector

A motion detector is a device that detects moving objects, particularly people. A motion detector is often integrated as a component of a system that automatically performs a task or alerts a user of motion in an area. Motion detectors form a vital component of security, automated lighting control, home control, energy efficiency, and other useful systems. An electronic motion detector contains an optical, microwave, or acoustic sensor, and in many cases a transmitter for illumination. However, a passive sensor only senses a signal emitted by the moving object itself. Changes in the optical, microwave, or acoustic field in the device's proximity are interpreted by the electronics based on one of the technologies listed below. Most inexpensive motion detectors can detect up to distances of at least 15 feet (5 meters). Specialized systems are more expensive but have much longer ranges. Tomographic motion detection systems can cover much larger areas because the radio waves are at frequencies which penetrate most walls and obstructions, and are detected in multiple locations, not just at the location of the transmitter.

Motion detectors have found wide use in domestic and commercial applications. One common application is activation of automatic door openers in businesses and public buildings. Motion sensors are also widely used in lieu of a true occupancy sensor in activating street lights or indoor lights in walkways (such as lobbies and staircases). In such "Smart Lighting" systems, energy is conserved by only powering the lights for the duration of a timer, after which the person has presumably left the area. A motion detector may be among the sensors of a burglar alarm that is used to alert the home owner or security service when it detects the motion of a possible intruder. Such a detector may also trigger a security camera in order to record the possible intrusion.

Sensor Technologies

There are several motion detection technologies in wide use:

- Passive infrared (PIR)

Passive infrared sensors are sensitive to a person's skin temperature through emitted black body radiation at mid-infrared wavelengths, in contrast to background objects at room temperature. No energy is emitted from the sensor, thus the name "passive infrared" (PIR). This distinguishes it from the electric eye for instance (not usually considered a "motion detector"), in which the crossing of a person or vehicle interrupts a visible or infrared beam.

- Microwave

These detect motion through the principle of Doppler radar, and are similar to a radar speed gun. A continuous wave of microwave radiation is emitted, and phase shifts in the reflected microwaves due to motion of an object toward (or away from) the receiver result in a heterodyne signal at low audio frequencies.

- Ultrasonic

An ultrasonic wave (sound at a frequency higher than a human can hear) is emitted and reflections from nearby objects are received.[1]Exactly as in Doppler radar, heterodyne detection of the received field indicates motion. The detected doppler shift is also at low audio frequencies (for walking speeds) since the ultrasonic wavelength of around a centimetre is similar to the wavelengths used in microwave motion detectors. One potential drawback of ultrasonic sensors is that the sensor can be sensitive to motion in areas where coverage isn't desired, for instance, due to reflections of sound waves around corners. Such extended coverage may be desirable for lighting control, where the point is detection of any occupancy in an area. But for opening an automatic door, for example, one would prefer a sensor selective to traffic in the path toward the door.

- Tomographic motion detector

Tomographic motion detection systems sense disturbances to radio waves as they pass from node to node of a mesh network. They have the ability to detect over complete areas because they can sense through walls and obstructions.

- Video camera software

With the proliferation of inexpensive digital cameras capable of shooting video, it is possible to use the output of such a camera to detect motion in its field of view using software. This solution is particularly attractive when the intention was to record video triggered by motion detection, as no hardware beyond the camera and computer is required. Since the observed field may normally be illuminated, this may be considered another passive technology. However it can also be used in conjunction with near-infrared illumination to detect motion in the "dark" (that is, with the illumination at a wavelength not detected by the human eye).

- Dual Technology motion Detection

Many modern motion detectors use combinations of different technologies. While combining multiple sensing technologies into one detector can help reduce false triggering, it does so at the expense of reduced detection probabilities and increased vulnerability. For example, many dual-tech sensors combine both a PIR sensor and a microwave sensor into one unit. In order for motion to be detected, both sensors must trip together. This lowers the probability of a false alarm since heat and light changes may trip the PIR but not the microwave, or trees may trigger the microwave but not the PIR. If an intruder is able to fool either the PIR or microwave, however, the sensor will not detect.

CHAPTER THREE

METHODOLOGY AND ANALYSIS OF THE SYSTEM AND ANALYSIS OF THE EXISTING SYSTEM

3.1	RESEARCH METHODOLOGY

The methodology that will be used in the proposed model is to install a motion detection device with GSM module for SMS alert notification, in the system mercury switches, motion detectors, and GSM module is being developed with a microcontroller. The communication between the microcontroller and other components of the system takes place serially. The microcontroller continuously receives data from the mercury switches and the motion detectors, takes decision on the basis of the readings collected onto the microcontroller. On intrusion the microcontroller will generate a message to the owner or to a set of predefined numbers stating the sort of an intrusion that has been made.

Figure 3.1: The Proposed System Architecture of the GSM-based Alert system Source: (Nosiri, Akwiwu-Uzoma, Nmaju & Elumeziem, 2018)

3.2	ANALYSIS OF THE EXISTING SYSTEM

A security detection system basically includes CCTV/webcam, alarms, SMS and various sensors. Different technologies have advanced over the years in the monitoring, detection and control of security systems. These ranges from systems that allow compactable products to communicate with each other through wired connections to wireless connections. Wireless intrusion security systems today can easily control the home or office mechanical systems and applications over cellular phone or internet. GSM technology nowadays provides a viable ubiquitous access to most systems security and control. The existing system is the use of Close Circuit Television (CCTV) for monitoring and security purpose in which most of this CCTV uses digital mode of data transmission. The surveillance industry has grown in the last few years. This growth though affected by the economic downturn in Europe is on the increase especially as the year 2013 is expected to be a landmark year in the industry from various market forecasts. The use of CCTV only records movements and sometimes sound but doesn’t notify the user of any intruder.

3.3	PROBLEMS OF THE EXISTING PROBLEM

34.3% of the world’s population had access to the internet a great rise when compared to March 2008 when 20.3% of the world’s population had access to the Internet. It is significant however to note that both Africa and the Mideast have experienced the most significant increase in access in recent years. A glaring challenge to the wide deployment of security systems are as follows:

- Low internet penetration and poor internet speed in most countries of the world.

- A region specific issue like poor power supply in developing nations is another major barrier to widespread deployment of CCTV surveillance systems.

- Another issue of concern in the use of CCTV surveillance cameras is security flaws. It has been reported that the inherent design of web interface of the IP surveillance cameras is not robust and is therefore prone to security flaws.

- Network attacks such as the injection of video streams through ARP spoofing, Cross Site Scripting (XSS), Cross Site Request Forging (CSRF) and Cross Domain Image Streaming (CDIS) based on the concept of Remote File Inclusion (RFI) have been experienced. This is reported to be due to insecure Common Gateway Interface (CGI), factory defaults and undocumented credentials, insecure front end design in the web interfaces of IP surveillance cameras, invalidated entry points and unencrypted cameras.

3.4	DESCRIPTION OF THE PROPOSED SYSTEM

This system consists of an infrared sensor, laser and motion sensor. Also a wireless alarm is installed which will ring if the motion sensor detect a person coming in front of it. Sensor will be directly connected to GUI interface. The proposed system is controlled by an Atmeg328 microcontroller. There are several steps to be applied in designing an active infrared motion detector for house security system. The relevant information is gathered through literature review from previous chapter. Data on motion detection and security system projects has been collected where the theoretical design is studied based on the motion detector for security concept. The understanding on the electrical structure for the hardware development is needed for the design circuit process of the motion detector and the basic security circuit. The next is the hardware development according to the circuit designed. This process is just only being preceded if each part of the circuit being improved is valid, else, it will be repeated until it is valid as the theoretical. Once the hardware development circuits have the output as the expected, then, the comparison for both hardware and theoretical analysis will be done.

Next is the step where software structure is developed for the security system to be interface with the hardware development. While the final step of this research is on applying the whole project to the real office entrance like doors and windows.

3.5	ADVANTAGES OF THE PROPOSED SYSTEM

IP based security surveillance system is the modern way of securing lives and properties, the implemented system has various advantages and some of its advantages are:

i.	It increases the security and compliance to lives and properties.

ii.	IP based  security surveillance has a low hacking investment.

iii.	It helps to find and report as accurately as possible the previous security incident conducted by someone.

CHAPTER FOUR

DESIGN, IMPLEMENTATION AND DOCUMENTATION OF THE SYSTEM

4.1	DESIGN OF THE SYSTEM

This is the computation of the particulars of a new system and the determination of what the new system would be and the function it is to perform. This may involve changing from one system to another or modifying the existing system operation.

The most challenging phase of the system life cycle is the change from manual operation to a faster and more accurate one; system design stage covers the technical specifications that will be employed in the implementation of the new system in order to modify the previous system. Some factors are put in consideration. These factors include input design, output design, the database and procedure designs and other documentation.

4.1.1	OUTPUT DESIGN

This incorporates the objectives of solving the existing system problems and challenges. The outputs extracted from the system are displayed in the screenshot images below:

Figure 4.1: The Motion Detection and SMS Alert System

The figure 4.1 above is the Installed motion detection system. The system will recognize every individual entering into the a restricted area (i.e office) and alert the owner of the office through SMS about an intruder.

Figure 4.2: The Alarm System

This image above depict the alarm system, anytime the motion detector detect n intruder this device above will continue ringing until the intruder moved away from the area.

4.1.2	INPUT DESIGN

The input to implement this hardware is obtained from the office alarm and motion detection system materials purchased from the market. The researchers are expected to install the system. He can achieve this by coupling these materials together. These materials are shown below:

Figure 4.3: Andruino UNO

This image above is the Andruino UNO used in the implementation of the system. Andruino is an open source electronic platform based on easy to use hardware and software. Andruino boards are able to read input light on a sensor, finger on a button or a twitter message and turn it into an output.

Figure 4.4: A Microcontroller

This is the Microcontroller that controls the entire system; it is coupled together with the andruino for the purpose on motion detection. Microcontroller are used in multiple industries and applications, including the home and enterprises, building automation, robotics and internet of things.

Figure 4.5: Infrared Sensors (Transmitter and Receiver)

The image above is the infrared sensors; it is capable of sensing every in an office area. This sensor detect if the object is moving in or out of the area. An infrared sensor is an electronic device that measures and detect infrared radiation in it surrounding environment.

4.1.4	PROCEDURE DESIGN

These are the steps involved in unifying the whole process to produce the desired output. It involves implementation of the system which starts from the original input lessons to the output that is the implemented system. This allows the processing of user information and result to be possible. Screen is provided to help user with some information.

4.2	IMPLEMENTATION OF THE SYSTEM

This entails the choice of the programming language employed to implement the system which should-be suitable for student counter system. The system is designed for the use of individuals which should serve as an assistant. It is also expected to be used in conjunction with the user.

4.2.1	HARDWARE REQUIREMENTS

- The Microcontroller

- Andruino Uno

- Power Box

- Sensor

- Alarm Device

4.3	MAINTAINING THE SYSTEM

The system maintenance refers to making modification to an already existing system without necessarily changing from the beginning. System maintenance includes modification of the system to meet-up with certain requirements of the users. In this course, additional features can be added, errors corrected, ambiguous materials replaced to eliminate confusions and unnecessary features removed.

Maintaining this system can be by a technician. Any future modification can be by replacing some parts that has fault and re-install the system.

CHAPTER FIVE

SUMMARY, CONCLUSION AND RECOMMENDATION

5.1	SUMMARY

The office security alarm using SMS and motion detector has been successfully implemented. The microcontroller that has been assembled was inputted in such a way that it has a function to detect human movement from the Passive Infrared (PIR) sensor. The device was run with a predetermined time according to the operational condition of the office that is the location of the study. The space condition in question is the process of reading human movements by sensors, while the condition of time is a predetermined real time clock state. This process starts when the device operates, a device that is active will detect human movement in the around the door. The process of sending notifications, if the sensor detects a movement around the door using motion detector. The process of sending notifications is the result of reading the sensor and the time specified. If there is movement at the door, the system will emit a sound through the buzzer alarm and send an SMS notification to the office owner's number.

With the use of GSM technology, owner of the office or house gets remote indication through SMS. So, even if the user is away from home or office, he/she will be informed of the presence of anyone within the office. The system is fully automated. So once this system is installed inside an office, then it does not require any human interaction to operate.

5.2	CONCLUSION

The SMS based home security system has been designed and tested with the mobile network. The user can get alerts anywhere through the GSM technology thus making the system location independent. A flexible way to control and explore the services of the mobile, Microcontroller was used in the system. The communication of office is only through the SMS which has been tested with the mobile networks and is working on any mobile network. The implementation of GSM technology in this work has three major advantages. First, an alert message can be received by user in time of intrusion. Secondly, ease with less wire or cable applied and lastly, its low power consumption. Since a microcontroller is used for this research, the flexibility of the algorithm of the system can be increased by introducing more sensors instead of using separate circuit board with components to consume more power. This means small installation space is required and less power consumption. Further improvements can be made to the system to enable it differentiate between a human intruder and a household pet by computer vision technology to minimize false positives and false negatives to the system.

5.3	RECOMMENDATIONS

Due to time constraints, this installation was not done early enough to be used for the past security. A measure of the alarm effectiveness will further be established if data of real-time supervision are used. We recommend the use of the system for the invigilation of security in the upcoming academic session to further authenticate its effectiveness. Then, every higher institution and industries should adopt the system as this will make their property more secured.

REFERENCES

Adebisi, J., A., Abdulsalam, K., A. & Adams, I., O. (2021). Multi-Power Source and Cloud-Backup Enabled Security Framework for Surveillance in Nigeria, IOP Conference Series: Earth and Environmental Science. Pp. 7-9. doi:10.1088/1755-1315/730/1/012005

Al-Rawi, M, Abdulhamid, M. & Murungi, M. (2019). System Design of Video Surveillance System, Research and Information Technologies. Vol. 1. Issue: 2. Pp. 4-5. DOI: 10.20535/SRIT.2308-8893.2019.4.05

Baballe, A., M., Bello, M., I., Bello Imam, A., Sule, A., T. & Muhammad, A., S. (2021). Implementation of Security Alarm using Arduino with P.I.R Motion Sensor and GSM Module, Artificial & Computational Intelligence. Vol 1, Issue: 1. Pp. 12-14.

Bangali, J. & Shaligram, A. (2016). Design and Implementation of Security Systems for Smart Home based on GSM technology, International Journal of Smart Home. Vol. 7, Issue: 6. Pp.201-208.

Darmawan, E. & Taufan, R. (2018). Space Security System using Motion Sensor and Notification of Short Message Service with Arduino-Based Fuzzy Logic Algorithm. IOP Conf. Series: Journal of Physics: Conf. Series, IOP Publishing. Pp. 22-23. doi:10.1088/1742-6596/1179/1/012034 1

Efe, O., E. Chukwudebe, G. Ekwonwune, E. & Biodun, O. S. (2019). Web Based Surveillance System in Nigeria, International Journal of Advanced Research in Science, Engineering and Technology. Vol. 6, Issue: 4. Pp. 89-90.

Harish, K., S., Swati, G., R., Sonali, K., B. & Sangmesh, P. (2015). Smart Video Surveillance, International Journal of Emerging Engineering Research and Technology. Vol. 3, Issue: 1. Pp. 109-112.

Iyapo, K., O., Fasunla, O., M., Egbuwalo, S., A., Akinbobola, A., J. & Oni, O., T. (2018). Design and Implementation of Motion Detection Alarm and Security System, International Journal of Engineering and Advanced Technology Studies. Vol.6, Issue: 1 Pp.26-38.

Kartik, K., J. Kumar, R., K. & Srimadhavan, V., S.  (2016). Security System With Face Recognition, Sms Alert And Embedded Network Video Monitoring Terminal, International Journal of Security, Privacy and Trust Management (IJSPTM). Vol. 2, Issue: 5. Pp. 23-24.

Matthews, V., O., (Ph.D), Olowononi, F., O. & Ike, D. (2013). On the Implementation of IP Video Surveillance Systems, Journal Of Information and Communication Technologies. Vol. 3, Issue: 5. Pp. 56-57.

Malmenbratt, J. (2015). The differing aims and functions of CCTV aims and functions of CCTV within the corporate stratum, Australian Security and Intelligence Conference Conferences, Symposia and Campus Events 2015 CCTV surveillance. Pp. 34-35.

Nicholas, N., T., Risi, I. & Robert, J., A (2020). Design and Implementation of Intruder Detector System with SMS Alert, American Journal of Engineering Research (AJER). Vol. 9, Issue: 11. pp-128-132.

Nosiri O.C., Akwiwu-Uzoma C.C., Nmaju U.A., Elumeziem C.H. (2018). Motion Detector Security System for Indoor Geo-location, International Journal of Engineering and Applied Sciences (IJEAS). Vol. 5, Issue: 11. Pp. 67-68.

Nwalozie, G., C., Aniedu, A., N., Nwokoye, C., S. &  Abazuonu, I. E. (2015). Enhancing Home Security Using SMS-based Intruder Detection System, International Journal of Computer Science and Mobile Computing A Monthly Journal of Computer Science and Information Technology (IJCSMC). Vol. 4, Issue. 6. Pp. 1177 – 1184.

Ohaneme, C. O., Eke, J. Augustine, C., O., Emmanuel, A., N. & Ohaneme, L., C. (2012). Design and Implementation of an IP-Based Security Surveillance System, IJCSI International Journal of Computer Science. Vol. 9, Issue: 5. Pp. 1002-1004.

Olalekan, O., B. (2015). Home Security System with Text Alert and Motion Detective, International Journal of Research (IJR). Vol. 2, Issue: 6, Pp.  42-43.

Sadiqzada, H., Panda, P., K., Mowlawizadah, W. & Ayobi, B., A. (2019). Securing IP Surveillance Cameras using Adaptive Security Appliance (ASA), International Research Journal of Engineering and Technology (IRJET). Vol. 6, Issue: 12. Pp. 1394-1394.

Appendices: System Flowchart

Appendix 1: Intruder Detection Flowchart

Start

Object Moved Closer to the System

No

Is Object Detected?

Yes

Send SMS

More Intruder?

Yes

No

Close

Return

Appendix 2: SMS Alert Flowchart

Start

Open

Detect an Object

No Object Detected

Is Object Detected?

No

Yes

Send SMS

Modify Another Record?

Yes

Close

No

Return