DESIGN AND CONSTRUCTION OF A HOME AUTOMATION SYSTEM USING ARDUINO UNO AND BLUETOOTH.

BY

ISMAIL YUSUF OLATUNJI

HND/23\EEE/FT/0220

SUBMITTED TO:

THE DEPARTMENT OF ELECTRICAL ELECTRONICS ENGINEERING, INSTITUTE OF TECHNOLOGY,

KWARA STATE POLYTECHNIC, ILORIN.

IN PARTIAL FULFILLMENT OF THE REQUIREMENT FOR THE AWARD OF HIGHER DIPLOMA (HND) CERTIFICATE INvELECTRICAL ELECTRONICS ENGINEERING

SUPERVISED BY

ENGR. ADEBAYO A O

JULY,2025.

CERTIFICATION

This is to certify the project work was carried out and submitted by ISMAIL YUSUF OLATUNJI Matric Number: HND/23/EEE/FT/0220 to the Department of Electrical/Electronics Engineering is accepted having confirmed with the requirements for the Award of Higher National Diploma (HND) in the Department of Electrical/Electronics Engineering, Institute of Technology, Kwara State Polytechnic, Ilorin.

ENGR.ADEBAYO A.O	SIGN/DATE

(Project Supervisor)

ENGR.ABDULKADIR Z.A	SIGN/DATE

(Project Coordinator)

ENGR.DR.LAWAL AHMED, O.	SIGN/DATE

(Head of Department)

(External examiner)	SIGN/DATE

DEDICATION

This project is dedicated to Almighty God, the most merciful, the most gracious, who as protected me through the completion of this academic program. May HIS name be praised forever. Also to my parent for their support throughout this project and to the department of electrical and electronics engineering.

ACKNOWLEDGEMENTS

Firstly I am grateful to Almighty Allah for giving me an opportunity to excel in my efforts to complete this project. I will also be thankful to my lovely parent for their support financially and materially towards the success of this project. And also to all the staffs of this great institute, especially my supervisor (ENGR. ADEBAYO A. O) for he has been my guidance to make this project successful.

Lastly, i would like to say a big thank you to my colleagues that we did this project together, for their cooperation and their help in the completion of this project.

TABLEOFCONTENT

TitlePage	i

Certification	ii

Dedication	iii

Acknowledgement	iv

TableofContents	v

CHAPTERONE:INTRODUCTION

  - BackgroundoftheStudy	1

  - ProblemStatement	2

  - AimandObjectives	2

  - SignificanceoftheStudy	3

  - ScopeoftheStudy	3

  - LimitationsoftheStudy	4

CHAPTERTWO:LITERATUREREVIEW

  - HistoryofHomeAutomation	5

  - HomeAutomationSystem	6

    - Arduino-BasedHomeAutomation	7

    - BluetoothBasedHomeAutomationSystemUsingCellPhones	7

    - ZigbeeBasedHomeAutomationSystemUsingCellPhones	7

    - GSMBasedHomeAutomationSystemUsingCellPhones	8

    - Wi-FiBasedHomeAutomationSystemUsingCellPhones	9

    - HomeAutomationUsingRFModule	10

    - RaspberryPieHomeAutomationwithWireless

SensorsUsingSmartPhone	11

  - BluetoothTechnologyinHomeAutomation	11

    - ChallengesofHomeAutomationSystems	12

  - TheNecessityofHomeAutomationsystem	12

    - ConvenienceandComfort	12

    - EnergyEfficiencyandCostSavings	13

    - EnhancedSecurityandSafety	14

    - AccessibilityforElderlyandDisabledIndividuals	14

    - IntegrationwithIoTandSmartTechnologies	15

    - IncreasedPropertyValue	16

CHAPTERTHREE:METHODOLOGY

  - Thestagesofhomeautomationproject	17

  - DiagramOfTheHomeAutomationSystemusing

ArduinoUNOandBluetooth.	20

    - Thepowersupply	20

    - ArduinoUNO	21

    - ArduinoUNOBoardLayout	22

      - TechnicalSpecificationsofArduinoUNO	23

      - HowtopoweruptheArduinoUNO?	24

      - WhatareDifferentMemoriesofArduinoUNO?	24

      - TheInputandOutputPinsofArduinoUNO	24

      - CommunicationInterfacesavailableonArduinoUNO	25

      - Additionalfeatures	26

      - ArduinoUNOPinout	26

      - PinDescription	27

    - RelayModule	31

    - BluetoothModule	34

  - TheCodeImplementationoftheHomeAutomation	38

  - Construction	41

    - CASING	41

CHAPTERFOUR:TESTING

  - TestingtheRange	43

  - Results	43

  - BillofEngineeringMeasurementandEstimation	46

CHAPTERFIVE:RECOMMENDATIONANDCONCLUSION

  - SuggestionsforFurtherResearch	48

  - PROBLEMSENCOUNTERED	49

  - CONCLUSION	49

References	50

LISTOFFIGURE

Figure1:TheblockdiagramofaGSMhomeautomationsystem	9

Figure2:thediagramofaWIFIbasedhomeautomationsystem	10

Figure3:theblockdiagramofahomeautomationsystem.	20

Figure4:thebuckconvertercircuit	21

Figure5:TheArduinoUNOBoardforbutSMDandThresiode	22

Figure6:ThepinoutdiagramofArduinoUNOboard	23

Figure7:ArduinoUNOpinoutdiagram	27

Figure8:typicaldiagramofarelaymodule	32

Figure9:TheDiagramofthePrincipleofOperationofRelay	33

Figure10:isapictorialimageofaBluetoothmodule(HC-05).	36

Figure11:isthewiringdiagramofHc-05	37

Figure12:TheInnerConstructionofaHomeAutomationSystem	41

Figure13:TheHomeAutomationSystemCasing	42

Figure14:Bluetoothconnectionandspeedin(%)variationina

plainenvironment	44

Figure15: Bluetooth connection and speed in (%) variation in a enclosed environment	45

CHAPTERONE

  - INTRODUCTION

  - BackgroundoftheStudy

The advancement in technology has revolutionized various aspects of daily life, with home automation being one of the most significant developments. Home automation systems provide convenience, security, and energy efficiency by allowing users to control appliances and electrical devices remotely. The increasing adoption of microcontrollers and wireless communicationtechnologieshasfacilitatedthedevelopmentofsmarthomesolutionsthatcater to the needs of modern households.

Home automation technology has evolved from simple remote-controlled systems to complex artificial intelligence-driven solutions. However, many of these systems require an internetconnectionandinvolvehighimplementationcosts,makingtheminaccessibletosome users. The use of Arduino UNO and Bluetooth technology provides an alternative, low-cost, andreliablehomeautomationsolution.ArduinoUNOisanopen-sourcemicrocontrollerboard thatofferseaseofuseand flexibility,whileBluetoothenableswirelesscommunicationbetween devices within a limited range. By integrating these two technologies, a practical and cost- effectivehomeautomationsystemcanbedeveloped,allowinguserstocontrolhomeappliances via a mobile application.

Home automation, also known as smart home technology, refers to the use of control systems and information technology to operate household appliances and devices remotely. Theconcepthasevolvedsignificantlyovertheyears,incorporatingvarioustechnologiessuch as Wi-Fi, Zigbee, GSM, and Bluetooth. These systems aim to improve energy efficiency, security, and user convenience (Smith et al., 2020).

  - ProblemStatement

Despitetherapiddevelopmentofhomeautomationtechnologies,manyexistingsystems areeithertooexpensive,complex,orreliantoninternetconnectivity,whichmaynotalwaysbe available in remote or underdeveloped areas. Traditional home automation systems require dedicated hardware and software configurations, making them challenging for non-technical users. Additionally, commercially available solutions often come with subscription-based services, increasing the overall cost of ownership.

Therelianceoninternet-basedautomationsystemspresentsanotherchallenge,asusers may experience connectivity issues due to network outages, making the system unreliable in certainlocations.Furthermore,someautomationsystemsinvolveextensivewiring,whichcan be difficult to install and maintain.

  - AimandObjectives

TheprimaryaimofthisprojectistodesignandconstructaBluetooth-basedhome automation system using Arduino UNO.

Thespecificobjectivesofthisprojectinclude:

    - Todevelopacost-effectivehomeautomationsystemthatallowsuserstocontrol appliances using a smartphone application.

    - ToimplementawirelesscommunicationsystemusingBluetoothtechnology.

    - Toenhanceuserconveniencebyprovidinganintuitiveandeasy-to-usemobile application interface.

    - Toimproveenergyefficiencybyenablinguserstomonitorandcontrolappliances remotely.

  - SignificanceoftheStudy

Thisstudyissignificantinseveralways:

- Cost-Effectiveness: The proposed system utilizes readily available components, making it an affordable alternative to existing commercial home automation solutions.

- User-Friendliness: The mobile application interface is designed to be simple and intuitive, ensuringeaseofuseforindividuals with minimal technical knowledge.

- Energy Efficiency: By allowing users to control appliances remotely, the system helps reduce unnecessary power consumption, contributing to energy savings.

- Enhanced Security: Users can switch appliances on and off remotely, adding an extra layer of security to their homes.

- NoInternetDependency:ThesystemreliesonBluetoothcommunication,making it functional in areas with limited or no internet access.

  - ScopeoftheStudy

ThisstudyfocusesondesigningandimplementingahomeautomationsystemusingArduino UNO and Bluetooth technology. The system will include:

    - ABluetoothmoduleforwirelesscommunication.

    - Amobileapplicationforcontrollingprototypehouseholdappliances.

    - Integrationwithbasicelectricaldevicessuchaslights,fans,andpowersockets.

    - Asimpleswitchingmechanismforturningdevicesonandoffremotely.

Thesystemwillnotincludeadvancedfeaturessuchasinternet-basedremoteaccess,voice control, or artificial intelligence-driven automation.

  - LimitationsoftheStudy

    - LimitedRange:Bluetoothtechnologyhasashortcommunicationrange,typicallyupto 10 meters.

    - BasicFunctionality:Thesystemislimitedtoturningdevicesonandoffanddoesnot include real-time power consumption monitoring or AI-driven automation.

    - Device Compatibility: The system is designed to work with basic household appliances but may require modifications for compatibility with more complex devices.

CHAPTERTWO

  - LITERATUREREVIEW

  - HistoryofHomeAutomation

Homeautomationissomethingwetakeforgrantednowadays.Wecannotimagineour daywithouthavinganAlexadeviceplayingourmusic,oursmartTVshowingthelatestNetflix shows or a day without our smartphone.

The technology progress has been through a lot of changes throughout the years, and that is shown best in our dailyappliances and smart devices. We gathered some historical examples ofearlyversusmoderntechnologytodemonstrate to youhowthankful weshouldbeforwhat we have. Did you know that the first devices used in home automation were labor-saving devices? Theseare afew ofthemost significant innovations in this field sinceits inception in the 1800s.

Home automation has a long history that dates back to the early 20th century when electrical appliances first became widespread. The development of home automation can be traced through various technological advancements:

- 1900s-1950s:Theinventionofhouseholdelectricalappliancessuchasrefrigerators, washing machines, and vacuum cleaners laid the foundation for automation.

- 1960s-1980s: The introduction of remote-controlled devices, such asgarage doors andtelevisions, marked the early forms of automation.

- 1990s: The emergence of programmable logic controllers (PLCs) and wired automation systems allowed for greater integration of home automation features.

- 2000s-Present: The development of wireless communication technologies such as Wi-Fi, Bluetooth, and Zigbee has led to the modern era of smart homes, where devices can be controlled remotely via mobile applications (Miller & Johnson, 2020).

Automationperformsanincreasinglyvitalroleindailyexperienceandglobaleconomy. Engineers strive to combine automated devices with mathematical and organizational toolsto create complex systems for a rapidly expanding range of applications and human activities.The concept of home automation has been around since the late 1970s. But with theenhancementoftechnologyandsmartservices,people’sexpectationshavechangedalot duringthecourseoftimetoperfectlyturnthetraditionalhouseintosmarthome,andalsothink thatwhatahomeshoulddoorhowtheservicesshouldbeprovidedandaccessedathometo

becameasmarthomeandsohastheideaofhomeautomationsystems.

  - HomeAutomationSystem

Givingenduserstheabilitytocontrolandoperateelectricappliancesispossiblewitha home automation system. When examining various home automation systems over time, we can see that they have consistently worked to give residents of homes safe, practical, and effective ways to access their homes. The look of a home automation system hasn't changed over time, despite changes in user expectations, advancements in technology, or time itself.

Many existing, well-established home automation systems are based on wired communicationsuchasArduinobasedandraspberrypibasedhomeautomationsystems.This does not pose a problem until the system is planned well in advance and installed during the physical construction of the building. But for already existing buildings the implementation costgoesveryhigh.Incontrast,Wirelesssystemscanbeofgreathelpforautomationsystems likeBluetooth,Wi-FiandIOTbasedhomeautomationsystems.Withtheadvancementof

wireless technologies such as WiFi, cloud networks in the recent past, wireless systems are used every day and everywhere.

    - Arduino-BasedHomeAutomation

Arduino-basedautomationsystemshave gainedpopularityduetotheirlowcost,open- source nature, and ease of programming. According to Brown et al. (2021), Arduino-based homeautomationsystemsprovideaflexibleandscalablesolutionthatallowsuserstointegrate multiple sensors and actuators. Researchers have successfully implemented Arduino systems forcontrollinglighting,temperature,andsecurityfeaturesinsmarthomes(Jones&Lee,2019).

    - BluetoothBasedHomeAutomationSystemUsingCellPhones

Relays are used to connect the Arduino BT board to the input/output ports of the homeappliances in a Bluetooth-based home automation system. The Arduino BT board connects via Bluetooth and is programmed using the high-level interactive C language of microcontrollers.

Onlyauthorizeduserscanaccesstheappliancesthankstothepasswordprotection.Forwireless communication,theArduinoBTboardandphoneestablishaBluetoothconnection.Thissystem uses a Python script that is portable and installableon anySymbian OS environment. A single circuit is created and put into use to receive phone feedback, which shows the device's status.

    - ZigbeeBasedHomeAutomationSystemUsingCellPhones

To monitor and control the home appliances the system is designed and implemented usingZigbee.Thedeviceperformanceisrecordandstorebynetworkcoordinators.Forthisthe Wi-Finetworkisused,whichusesthefourswitchportstandardwirelessADSLmodernrouter. ThenetworkSSIDand securityWi-Fiparameter arepreconfigured. Forsecuritypurposes,the message is first processed by the virtual home algorithm. Once it is deemed secure, it is re- encryptedandsenttothehome'sactualnetworkdevice.TheZigbeecontrollersentmessages

totheendoftheZigbeenetwork.Thesecurityandsafetyofeverymessagethatthevirtualhome algorithm receives. ZIGBEe communication is useful in lowering the system's cost and the intrusiveness of the installation process.

    - GSMBasedHomeAutomationSystemUsingCellPhones

Because of the mobile phone and GSM technology, the GSM based home automation islureto research.TheSMSbasedhomeautomation,GPRSbasedhomeautomationanddual tonemultifrequency(DTMF)basedhomeautomation,theseoptionsweconsideredmainlyfor communicationinGSM.InfigureshowsthelogicaldiagramtheworkofA.Alheraish,itshows howthehomesensorsanddevicesinteractwiththehomenetworkandcommunicatesthrough GSMandSIM(subscriberidentitymodule).Thesystemusetransducerwhichconvertmachine functionintoelectricalsignalswhichgoesintomicrocontroller.Thesensorsofsystemconvert the physical qualities like sound, temperature and humidity into some other quantity like voltage.Themicrocontrolleranalysisallsignalandconvertthemintocommandtounderstand by GSM module. Select appropriate communication method among SMS, GPRS and DTFC based on the command which received GSM module.

Figure1:TheblockdiagramofaGSMhomeautomationsystem

    - Wi-FiBasedHomeAutomationSystemUsingCellPhones

Wi-Fi based home automation system mainly consist three modules, the server, the hardware interfacemodule,andthesoftwarepackage.Thefigureshowsthesystemmodellayout.Wi-Fi technologyisusedbyserver,andhardwareInterfacemoduletocommunicatewitheachother. The same technology uses to login to the server web based application. The server is connectedtotheinternet,soremoteuserscanaccessserverwebbasedapplicationthroughthe internetusingcompatiblewebbrowser.Softwareofthelatesthomeautomationsystemissplit toserverapplicationsoftware,andMicrocontroller(Arduino)firmware.TheArduinosoftware, built using C language, using IDE comes with the microcontroller itself. Arduino software is culpableforgatheringeventsfromconnectedsensors,thenappliesactiontoactuatorsandpre-

programed in the server. Another job is to report and record the historyin the server DB. The serverapplicationsoftwarepackagefortheproposedhomeautomationsystem,isawebbased application built using asp.net. The server application software can be accessed from internal network or from internet if the server has real IP on the internet using any internet navigator supports asp.net technology. Server application software is culpable of, maintain the whole home automation system, setup, and configuration. Server use database to keep log of home automation system components, we choose to use XML files to save system log.

Figure2:thediagramofaWIFIbasedhomeautomationsystem

    - HomeAutomationUsingRFModule

BuildingahomeautomationsystemwithanRF-controlledremoteisakeyobjectiveof home automation systems. As technology advances, homes are becoming increasingly intelligent.AcentralizedcontrolsystemwithRF-controlledswitchesispurposefullyreplacing the switches found in modern homes. These days, it is difficult for the end user to approach and operate the traditional wall switches that are dispersed throughout the house. For older adults or those with physical disabilities, it becomes even more difficult to do so.

Using RF technology, home automation via remote implements a simpler solution. Solution with RF technology. In order to accomplish this, a RF remote is combined to the microcontroller on transmitter side that sends ON/OFF signals to the receiver where devices areconnected.Byoperatingthestatedremoteswitchonthetransmitter,theloadscanbeturned ON/OFF globally using wireless technology.

    - RaspberryPieHomeAutomationwithWirelessSensorsUsingSmartPhone

Usingasmartphoneandwirelesssensors,aRaspberryPihomeautomationsystemusing a Raspberry Pi to read the email's subject line and algorithm, a home automation system was created. The Raspberry Pi promises to be a productive platform for putting strong, affordable smart home automation into practice. Raspberry Pi home automation is superior to all other home automation techniques in a number of ways.

For instance, the call tariff is a significant drawback of DTMF (dual tone multi- frequency) using home automation, even though this is not an issue with their suggested approach.Becauseitonlymakesuseofthewell-knownwebserverserviceprovidedbyGmail, thedesignofthewebserverandthememoryspaceneededaredisregardedinhomeautomation using web servers.

  - BluetoothTechnologyinHomeAutomation

Bluetooth technology plays a crucial role in wireless communication for home automation.UnlikeWi-Fi,Bluetoothdoesnotrequireinternetconnectivity,makingitidealfor offline applications. According to Patel & Sharma (2022), Bluetooth-based home automation systemsofferareliable,low-poweralternativeforcontrollingapplianceswithinalimitedrange. Various studies have explored the advantages of Bluetooth-based automation, highlighting its affordability and ease of implementation (Kumar etal.,2021).

    - ChallengesofHomeAutomationSystems

Home automation systems suffers four main challenges; these are poor manageability, inflexibility, difficulty in achieving security and high cost of ownership, The main objectives ofthisresearchistodesignandimplementahomeautomationsystemusingIoTthatiscapable of controlling and automating most of the house appliances through an easy manageable web interface. The proposed system has a great flexibility by using Wi-Fi technology to interconnect its distributed sensors to home automation server. This will decrease the deployment cost and will increase the ability of upgrading, and system reconfiguration.

  - TheNecessityofHomeAutomationsystem

    - ConvenienceandComfort

      - Remote Control: One of the most significant advantages of home automation is the abilityto control home devices remotely. With smartphones, tablets, or smartwatches, homeowners can turn lights on or off, adjust the thermostat, or unlock doors from virtually anywhere. This level of control makes managing a home more convenient, especiallyforpeoplewithbusyschedulesorthosewhotravelfrequently.Additionally, voice assistants like Alexa or Google Assistant enable hands-free control, making it even easier to manage home tasks. (Wang et al., 2015).

      - Automation of Routine Tasks: Home automation systems can handle repetitive tasks without any user intervention. For example, lights can be set to automatically turn on when someone enters a room or a coffee machine can brew a cup at a specific time in the morning.

Such automation removes the need for manual input, saving time and effort. (Liu et al., 2018)

      - Customized Comfort: Climate control is another example where automation makes a difference. Smart thermostats learn user preferences and adjust the temperature based on time of day, weather, or occupancy. This ensures comfort while also preventing energy wastage, as the system won’t waste energy heating or cooling an emptyhome. (Deng etal.,2017)

    - EnergyEfficiencyandCostSavings

      - EnergyConsumption Monitoring: With the growing awareness of climate change and the need for sustainability, home automation helps homeowners track and reduce energy consumption. Smart meters and energy management systems provide insights intohowenergyisbeingused,enablinguserstoidentifyhigh-consumptionappliances and take steps to optimize usage. For instance, smart lighting systems use motion sensors to ensure that lights are only on when rooms are occupied, cutting down on unnecessary electricity usage. (Zhao & Zhang, 2019)

      - Smart Thermostats: A well-known feature of home automation systems, smart thermostats like Nest, Eco bee, or Honeywell, can significantly lower energy costs. These devices can detect when the home is empty and adjust the temperature accordingly to save energy. They can also learn the homeowner’s habits and set heating/cooling schedules based on personal routines. Over time, this smart behavior results in better energy efficiency and lower monthly utility bills (Bohannon, 2016).

      - AutomatedShadingandLighting:Automatedshadingsystemscanadjusttheblindsto blockoutheatfromthesunduringpeakhoursinthesummer,reducingtheneedforair conditioning.(Kumaretal.,2017)Similarly,lightscanautomaticallyadjusttothe

ambientlightlevels,ensuringthatroomsareneithertoobrightnortoodarkwhileusing the minimum amount of energy required.

    - EnhancedSecurityandSafety

      - Surveillance Systems: Security is a major concern for homeowners, and home automation helps address this byprovidingan advanced securitysystem that monitors the home 24/7. Cameras, motion sensors, and door/window sensors are often part of a smart security system. Alerts are sent to the homeowner’s smartphone if there is unusual activity detected, allowing for immediate action. Furthermore, some systems canbeprogrammedtosendalertstoemergencyservicesortonotifytrusted neighbors in case of a break-in.

      - Smart Locks: Smart locks are a critical part of home automation security. These locks allow users to lock or unlock doors remotelyvia a smartphone app. Theyalso provide theabilityto givetemporaryaccesstoothers,suchashousekeepers,friends,orservice technicians, with digital keys that can be revoked at any time.

      - Emergency Alerts and Monitoring: In addition to security, smart home systems can detect safetyhazards such as smoke, fire, carbon monoxide, or water leaks. If such an eventoccurs,thesystemwillalertthehomeowner,andinsomecases,canautomatically shutoffsystemslikewatervalvesorturnonemergencylights.Thisproactiveapproach ensures the safety of both people and property.

    - AccessibilityforElderlyandDisabledIndividuals

      - AssistiveTechnologies:Forelderlyordisabledindividuals,homeautomationprovides essential support in maintaining independence. Devices like voice-activated assistants canhelpthosewithlimitedmobilitycontrollights,doorlocks,fans,andeven

appliances. Automated reminders for medication or appointments can also be programmed, helpingindividuals stayon top of their healthcare needs without relying on others.

      - Remote Control for Comfort and Safety:Features like automated lighting and adjustable thermostats can be particularly beneficial for elderly individuals, helping them to stay comfortable without needing to adjust settings manually. Smart home technology also enables remote control of doors, windows, and blinds, reducing physical strain.

      - Health Monitoring and Emergency Response: Some home automation systems integratewithhealthmonitoringdevices,such aswearablehealthtrackersor emergency alert systems. These can be used to monitor vital signs or detect falls. In case of an emergency, the system can notify caregivers, family members, or emergency services for quick response.

    - IntegrationwithIoTandSmartTechnologies

      - Seamless Integration: Home automation is a keycomponent of the broader Internet of Things (IoT) ecosystem. It enables communication between a wide range of smart devices, including lights, appliances, thermostats, security systems, and even entertainment systems. This interconnected network allows for centralized control through a smartphone app or voice assistant, making it easier to manage multiple devices at once.

      - Data and Feedback: IoT-enabled smart homes collect vast amounts of data about how devicesarebeingused.Thisdatacanbeusedtofurtheroptimizesystems,provide

insights into energy use, and even predict future behavior, such as when a device is likely to need maintenance or replacement.

    - IncreasedPropertyValue

- Appeal to Homebuyers: As smart homes become more popular, theyare seen as more modern and desirable. Installing smart devices such as lighting, security, and climate control systems can increase the overall value of a property, makingit more attractive to prospective buyers. Real estate agents often highlight smart home features when marketingaproperty,andmanybuyersnowview thesefeaturesas akeysellingpoint.

- Competitive Advantage: In competitive real estate markets, smart home features can set a property apart. Automated homes are perceived as more convenient, secure, and energy efficient, which can lead to a higher resale value or quicker sale.

CHAPTERTHREE

  - METHODOLOGY

  - Thestagesofhomeautomationproject

Themethodologyforthishomeautomationprojectencompassesfourmainstages:

- SystemDesignandPlanning

- SoftwareDevelopment

- IntegrationandTesting

- SecurityImplementation

SystemDesignandPlanning

- Requirements Gathering: Identify hardware components (Arduino UNO, Bluetooth module,relaymodule,andpowersupply)anddeterminethenecessarysoftware(ArduinoIDE, Bluetooth control app) to meet the project's objectives.

- Circuit Design: Design the electrical circuit to connect the Arduino UNO, Bluetooth module, relay modules, and household appliances. The relay modules are used to switch appliances on and off based on commands received by the Arduino.

- Component Layout: Plan the physical layout of the components to ensure safe wiring, minimize electrical interference, and make the system compact and manageable.

SoftwareDevelopment

- ArduinoProgramming:WritecodeintheArduinoIDEtocontroltherelaysbasedon Bluetooth commands received from a mobile device.

Thecodewill:-EnabletheBluetoothmoduletoreceivecommands.-Interpretcommands(e.g., "ON"or"OFF")forspecificappliances.-Activateordeactivatetheconnectedrelaysbasedon the received commands.

- Mobile Application Configuration: - Use a pre-existing Bluetooth control app, or configureasimplecustominterfacetosendcommandsfromtheuser’smobiledeviceto theBluetoothmodule.-Assignuniquecommandsforeachapplianceandcreatebuttons or control options for user- friendly interaction.

IntegrationandTesting

- Hardware Integration: Connect the Arduino, Bluetooth module, relay modules, and appliances according to the designed circuit. Ensure all connections are secure and verify the operation of each relay.

- System Testing: - Test the connection between the mobile device and the Bluetooth module to ensure reliable pairing and data transmission. - Test each appliance control feature individually to ensure the correct relay activates with the corresponding Bluetooth command. - Perform range tests to confirm the system operates effectively within typical Bluetooth limits (approximately 10 meters).

- Debugging: Identify and fix any issues related to connection stability, command accuracy, or relay operation. SecurityImplementation

- BluetoothPairingSecurity: Configurethe BluetoothmodulewithauniquepairingPINto restrict system access to authorized devices only.

- User Authentication: Implement basic authentication within the app if possible, ensuring only authorized users can control the appliances. This could involve a simple password prompt or device pairing verification.

- DocumentationandUserTrainingUserDocumentation:Createausermanualwithstepby- step instructions for system setup, connection, and troubleshooting.

- Safety Guidelines: Include safety instructions for handling the hardware, especially with appliances connected to high-voltage power sources.

- End-UserTraining:Provideguidelinesonhowtousethemobileapplication,pairthe device, and interpret system indicators (e.g., LEDs for on/off status).

EvaluationandFinalAdjustments

  - UserFeedback:Collectfeedbackfromusers(ortesters)toidentifyanyusability or performance issues.

  - AdjustmentsandOptimization:Makeanyfinaladjustmentstothecode,circuit,or app configuration based on feedback to enhance system performance and user satisfaction.

  - DiagramOfTheHomeAutomationSystemusingArduinoUNOandBluetooth.

Figure3:theblockdiagramofahomeautomationsystem.

    - Thepowersupply

Thedesignispoweredbyoneofrechargeable18650battery.Thebatteryisrated7AH, 12V.Thisimpliesthatthebatterywillsupplyatotalof:12𝑣𝑜𝑙𝑡𝑠and7𝐴𝐻capacity.Thevoltage levels required bythecircuits are5𝑣𝑜𝑙𝑡𝑠and12𝑣𝑜𝑙𝑡𝑠; all theinnercircuitsexcept thepump is powered by a 5v but the pump is powered with 12v. In other to get 5 volts from 12 volts, a buckconverterisused.Abuckconverter(stepdownconverter)isaDC-to-DCpowerconverter which steps down voltage (while stepping up current) from its input (supply) to its output (load). It is a class of switched-mode power supply (SMPS) typically containing at least two semiconductors (a diode and a transistor) and an inductor. Its name derives from the inductor that “bucks” or opposes the supply voltage.

Figure4:thebuck convertercircuit

In comparison to linear regulators, which are more straightforward circuits that lower voltages by dissipating power as heat but do not step up output current, switching converters (suchasbuckconverters)offerafarhigherlevelofpowerefficiencyasDC-to-DCconverters. Buckconvertersareessentialforjobslikeconvertingacomputer'sprimarysupplypowerdown to lesser voltages needed because of their great efficiency, which is frequently over 90%.

Switching frequencies used by buck converters typically range from 100 kHz to a few MHz Smaller inductors and capacitors can be used with greater switching frequencies, but the efficiency is lost more frequently due to transistor switching.

    - ArduinoUNO

Arduino UNO is based on ATmega328P Microcontroller, an 8-bit AVR Architecture basedMCUfromATMEL.ArduinoUNOcomesintwovariants:oneconsistsofa28-pinDIP Microcontroller while the other consists of 32 lead Quad Flat Package Microcontroller.

Figure5:TheArduinoUNOBoardforbutSMDandThresiode

    - ArduinoUNOBoardLayout

ThefollowingimageshowsthelayoutofatypicalArduinoUNOboard.Allthe components are placed on the top side of the PCB.

Figure6Thepinoutdiagram ofArduinoUNOboard

Asyoucannotice,thereisaType-BUSBconnectorontheleftshortedgeoftheboard, whichisusedforpoweringontheboardaswellasprogrammingtheMicrocontroller.Thereis also a 2.1 mm DC jack to provide external power supply.

      - TechnicalSpecificationsofArduinoUNO

| MCU | ATmega328P |
| --- | --- |
| Architecture | AVR |
| OperatingVoltage | 5V |
| InputVoltage | 6V–20V(limit)7V– 12V(recommended) |
| ClockSpeed | 16 MHz |
| FlashMemory | 32KB(2KBofthisused bybootloader) |
| SRAM | 2 KB |
| EEPROM | 1 KB |
| DigitalIOPins | 24(of which6canproducePWM) |
| AnalogInputPins | 6 |

As Arduino UNO is based on ATmega328P Microcontroller, the technical specifications of Arduino UNO are mostly related to the ATmega328P MCU. But none the less, let me give you a brief overview about some important specifications of Arduino UNO.

      - HowtopoweruptheArduinoUNO?

There are acoupleofways inwhich you canpowertheUNOboard.Thefirstandeasy wayisusingtheType-BUSBConnector.Thenextwayistoprovideanunregulatedsupplyin the range of 6V to 20V to VIN pin of the UNO (Pin number 26).

You can also supplythe unregulated supplythrough the 2.1mm DC Jack, in which case, you can access the supplied voltage through the VIN Pin.

      - WhatareDifferentMemoriesofArduinoUNO?

Strictly speaking, this is specific to the MCU i.e., ATmega328P, used on the Arduino UNO Board. There are three different memories available in ATmega328P. They are:

        - 32KBofFlashMemory

        - 2KBofSRAM

        - 1KBofEEPROM

        - 0.5KBoftheFlashMemoryisusedbythebootloadercode.

      - TheInputandOutputPinsofArduinoUNO

The32pinsavailableon theUNOboard,22pins are associated withinput andoutput. In that 14 pins (D0 to D13) are true digital IO pins, which can be configured as per you application using pinMode(), digitalWrite() and digitalRead() functions.

AlltheseDigitalIOpinsarecapableofsourcingorsinking20mAofcurrent(maximum40mA is allowed). An additional feature of the Digital IO pins is the availability of internal pull-up resistor (which is not connected by default).

Thevalueoftheinternalpull-upresistorwillbeintherangeof20KΩto50KΩ.

Therearealso6AnalogInputPins(A0toA5).Alltheanaloginputpinsprovidea10-bitresolution ADC feature, which can be read using analogRead() function.

An important point about Analog Input pins is that they can be configured as Digital IO pins, if required.

DigitalIOpins3,5,6,9,10and11arecapableofproducing8-bitPWMSignals.Youcanuse

analogWrite()functionforthis.

      - CommunicationInterfacesavailableonArduinoUNO

ArduinoUNOsupportsthreedifferenttypesofcommunicationinterfaces.Theyare:

        - Serial

        - I2Cor I2C

        - SPI

Perhaps the most common communication interface in the Arduino universe is the Serial Communication. In fact, the Arduino boards (UNO or Nano or Mega) are programmed using the serial communication.

DigitalIOpins0and1areusedasSerialRXandTXpinstoreceiveandtransmitserialdata. These pins are connected to the serial pins of the on-board USB to Serial Converter IC.

Analog Input Pins A4 and A5 have alternative functions. They can be configured as SDA (A4) and SCL (A5) to support I2C or I2C or Two Wire Interface (TWI) communication.

ThefinalcommunicationinterfaceistheSPI.DigitalIOPins10,1112and13canbeconfigured as SPI pins SS, MOSI, MISO and SCK respectively.

      - Additionalfeatures

Thereisanon-boardLEDconnectedtodigital IOpin13.UsethisLEDtoperformBlinky operations.ThereferencevoltagefortheinternalADCisbydefaultsetto5V.ButusingtheAREF pin, you can manually set the upper limit of the ADC. Using the IOREF pin, you can set the reference voltage for Microcontroller operations.

To reset the microcontroller, you can use the on-board RESET button. Although you canprogramtheArduinoUNOusingtheUSBcable,thereisaprovisiontoprogramtheMCU usingtheInCircuitSerialProgramming(ICSP)interface.TheUARTbootloader,whichis

preloaded in to the ATmega328P microcontroller, enables programming through serial interface.ButICSPdoesn’tneedanybootloader.YoucanprogramArduinoUNOusingISCP or use the ISCP of Arduino UNO to program other Arduino Boards.

Digital IO Pins 2 and 3 can be configured as External Interrupts Pins INT0 and INT1 respectively. Use attachInterrupt() function to configure the Interrupt for rising edge, falling edge or level change on the pin.

      - ArduinoUNOPinout

Now that we have seen a little bit about Arduino UNO and its important features and specifications, let us dive into the Arduino UNO Pinout. The following image shows the complete pinout of Arduino UNO Board.

Figure7:ArduinoUNOpinoutdiagram

Asyoucanseefromtheimage,IdescribedeachpinoftheArduinoUNOwithitsmicrocontroller equivalent pin, alternative functions, default functionality and other additional features.

      - PinDescription

For pin description of Arduino UNO, let us assume some basic numbering. Let the numbering begin with the RX Pin (D0). So, RX is Pin 1, TX is Pin 2, D2 is Pin 3 and so on. On theotherside,NCisPin19,IOREFisPin20etc.Overall,thereare32pinsontheArduinoUNO Board.

| Pin Number | PinName | Description | Alternative Functions |
| --- | --- | --- | --- |
| 1 | RX /D0 | Digital IO Pin 0 SerialRXPin | Generallyusedas RX |
| 2 | TX /D1 | Digital IO Pin 1 SerialTXPin | Generally used as TX |
| 3 | D2 | DigitalIOPin2 |  |
| 4 | D3 | DigitalIOPin3 | Timer(OC2B) |
| 5 | D4 | DigitalIOPin4 | Timer(T0/XCK) |
| 6 | D5 | DigitalIOPin5 | Timer(OC0B/T1) |
| 7 | D6 | DigitalIOPin6 |  |
| 8 | D7 | DigitalIOPin7 |  |
| 9 | D8 | DigitalIOPin8 | Timer(CLK0/ICP1) |
| 10 | D9 | DigitalIOPin9 | Timer(OC1A) |
| 11 | D10 | DigitalIOPin10 | Timer(OC1B) |
| 12 | D11 | DigitalIOPin11 | SPI(MOSI)Timer (OC2A) |

| 13 | D12 | DigitalIOPin12 | SPI(MISO) |
| --- | --- | --- | --- |
| 14 | D13 | DigitalIOPin13 | SPI(SCK) |
| 15 | GND | Ground |  |
| 16 | AREF | AnalogReference |  |
| 17 | SDA/ D18 | DigitalIOPin18 | I2CDataPin |
| 18 | SCL/ D19 | DigitalIOPin19 | I2CClockPin |
| 19 | NC | Not Connected |  |
| 20 | IOREF | VoltageReference |  |
| 21 | RESET | Reset(ActiveLOW) |  |
| 22 | 3V3 | Power |  |
| 23 | 5V | +5VOutputfromregulatoror+5V regulatedInput |  |
| 24 | GND | Ground |  |
| 25 | GND | Ground |  |
| 26 | VIN | UnregulatedSupply |  |
| 27 | A0 | AnalogInput0 | DigitalIOPin14 |

| 28 | A1 | AnalogInput1 | DigitalIOPin15 |
| --- | --- | --- | --- |
| 29 | A2 | AnalogInput2 | DigitalIOPin16 |
| 30 | A3 | AnalogInput3 | DigitalIOPin17 |
| 31 | A4 | AnalogInput4 | DigitalIOPin18I2C (SDA) |
| 32 | A5 | AnalogInput5 | DigitalIOPin19 I2C(SCL) |

ThefollowingtabledescribesthepinsoftheICSPConnector.

| MISO | MasterInSlaveOut(InputorOutput) |
| --- | --- |
| 5V | Supply |
| SCK | Clock(fromMasterto Slave) |
| MOSI | MasterOutSlaveIn(InputorOutput) |
| RESET | Reset(ActiveLOW) |
| GND | Ground |

There is also a similar ICSP connector known as ICSP1 associated with the ATmega16U Microcontroller. For more information on this connector, take a look at the Arduino UNO Pinout diagram.

    - RelayModule

A Relay is a simple electromechanical switch. While we use normal switches to close or open a circuit manually, a Relay is also a switch that connects or disconnects two circuits. Butinsteadofamanualoperation,arelayusesanelectricalsignaltocontrolanelectromagnet, which in turn connects or disconnects another circuit. Relays can be of different types like electromechanical, solid state. Electromechanical relays are frequently used. Let us see the internal parts of this relay before knowing about working of relay. Although many different types of relay were present, their working is same.

Everyelectromechanicalrelayconsistsofanconsistsofan

- Electromagnet

- Mechanicallymovablecontact

- Switchingpointsand

- Spring

Electromagnet is constructed bywoundingacoppercoil on ametal core. Thetwo ends ofthe coil are connected to two pins of the relay as shown.

ThesetwoareusedasDCsupplypins.

Figure8:typicaldiagramofarelaymodule

Generallytwomorecontactswillbepresent,calledasswitchingpointstoconnecthighampere load. Another contact called common contact is present in order to connect the switching points. These contacts are named as normally open (NO), normally closed(NC) and common(COM) contacts.

We can use a Relay either in a AC circuit or a DC Circuit. In case of AC relays, for every current zero position, the relay coil gets demagnetized and hence there would be a chance of continues breaking of the circuit.

So, AC relays are constructed with special mechanism such that continuous magnetism is provided in order to avoid above problem. Such mechanisms include electronic circuit arrangement or shaded coil mechanism.

RelayWorkingPrinciple?

ThefollowinganimationshowshowaRelayworks.

Figure9:TheDiagramofthePrincipleofOperationofRelay

- Whentheelectromagnetisappliedwithsomecurrent,itinducesamagneticfieldaround it.

- Above image shows working of the relay. A switch is used to apply DC current to the load.

- Intherelay,Coppercoilandtheironcoreactsaselectromagnet.

- When the coil is applied with DC current, it starts attracting the contact as shown. This is called energizing of relay.

- When the supply is removed it retrieves back to the original position. This is called De energizing of relay.

Therearealsosuchrelays,whosecontactsareinitiallyclosedandopenedwhenthereissupplyi.e. exactly to opposite to the above shown relay.

Solid state relayswill have sensing element to sense the input voltage and switches the outputusing opto-coupling.

RelayApplications

Relays are used to protect the electrical system and to minimize the damage to the equipment connected in the system due to over currents/voltages. The relay is used for the purpose of protection of the equipment connected with it.

These are used to control the high voltage circuit with low voltage signal in applications audio amplifiers and some types of modems.

Theseareusedtocontrolahighcurrentcircuitbyalowcurrentsignalintheapplications like starter solenoid in automobile. These can detect and isolate the faults that occurred in power transmission and distribution system. Typical application areas of the relays include

- Lightingcontrolsystems

- Telecommunication

- Industrialprocesscontrollers

- Trafficcontrol

- Motordrivescontrol

- Protectionsystemsofelectricalpowersystem

- Computerinterfaces

- Automotive

- Homeappliances

    - BluetoothModule

TheBluetoothwhichisalsoknownasHC-05isapopularBluetoothmodulewhichcan addtwoway(full-duplex)wirelessfunctionalitytoanyofourprojectsinarangeofabout100m without any obstruction or barrier.

HC-05PinoutConfiguration

PinNumber

1

PinName

Enable

/

Key

Description

ThispinisusedtotogglebetweenDataMode(setlow)andAT command mode (set high). By default it is in Data mode

- Vcc	Powersthemodule.Connectto+5VSupplyvoltage

- Ground	Groundpinofmodule,connecttosystemground.

- TX	–

Transmitter

- RX	–

Receiver

Transmits Serial Data. Everything received via Bluetooth will begiven out by this pin as serial data.

ReceiveSerialData.Everyserialdatagiventothispinwillbe broadcasted via Bluetooth

- State

- LED

ThestatepinisconnectedtoonboardLED,itcanbeusedasa feedback to check if Bluetooth is working properly.

IndicatesthestatusofModule

  - Blinkoncein2sec:ModulehasenteredCommandMode

  - RepeatedBlinking:WaitingforconnectioninDataMode

  - Blinktwicein1sec:ConnectionsuccessfulinDataMode

- Button

Used to control the Key/Enable pin to toggle between Data andcommand Mode

Figure10:isapictorialimageofaBluetoothmodule(HC-05).

WheretouseHC-05Bluetoothmodule

The HC-05is a popular module which can add two-way (full-duplex) wireless functionality to your projects. You can use this module to communicate between two microcontrollers like Arduino or communicate with any device with Bluetooth functionality like a Phone or Laptop. There are manyandroid applications that are already available which makes this process a lot easier. The module communicates with the help of USART at 9600 baud rate henceit is easyto interfacewith anymicrocontroller that supports USART. Wecan alsoconfigurethedefaultvaluesofthemodulebyusingthecommandmode.Soifyoulooking for a Wireless module that could transfer data from your computer or mobile phone to microcontroller or vice versa then this module might be the right choice for you. However do notexpectthismoduletotransfermultimedialikephotosorsongs;youmighthavetolookinto the CSR8645 module for that.

HowtoUsetheHC-05Bluetoothmodule

The HC-05has two operating modes, one is the Data mode in which it can send and receive data from other Bluetooth devices and the other is the AT Command mode where the defaultdevicesettingscanbechanged.Wecanoperatethedeviceineitherofthesetwomodes by using the key pin as explained in the pin description.

ItisveryeasytopairtheHC-05modulewithmicrocontrollersbecauseitoperatesusing theSerial Port Protocol (SPP). Simplypowerthe modulewith +5V and connect theRx pin of the module to the Tx of MCU and Tx pin of module to Rx of MCU as shown in the figure belowDuringpowerupthekeypincanbe groundedtoenterintoCommandmode,ifleft free it will by default enter into the data mode. As soon as the module is powered you should be able to discover the Bluetooth device as “HC-05” then connect with it using the default password 1234 and start communicating with it. The name password and other default parameters can be changed by entering into the

Figure11:isthewiringdiagramofHc-05

Applications

- Wirelesscommunicationbetweentwomicrocontrollers

- CommunicatewithLaptop,Desktopsandmobilephones

- DataLoggingapplication

- Consumerapplications

- WirelessRobots

- HomeAutomation

  - TheCodeImplementationoftheHomeAutomation

//HOMEAUTOMATIONSYSTEMUSINGARDUINOUNO

//KWPOLY/2025/PROJECT

charval;//Variabledeclaration

voidsetup(){//Initiatingvariablestopinout pinMode(7,OUTPUT); pinMode(12,OUTPUT); pinMode(11,OUTPUT); pinMode(10,OUTPUT); pinMode(9,OUTPUT);

Serial.begin(9600); digitalWrite(7,HIG H);

digitalWrite(12,HIG H);

digitalWrite(11,HIG

H);

digitalWrite(10,HIG H);

digitalWrite(9,LOW

);} void loop() { if(Serial.available())

{val =Serial.read(); Serial.println(val); } if(val=='3'){ digitalWrite(12,LO W); } else if(val=='4'){ digitalWrite(12,HIG H); } if(val=='0'){

digitalWrite(11,LO W);} else if(val=='6'){ digitalWrite(11,HIG H); } if(val=='7'){

digitalWrite(10,LO W); } else if(val=='8'){

digitalWrite(10,HIG H); } if(val=='9'){

digitalWrite(9,LOW)

; } else if(val=='5'){ digitalWrite(9,HIGH

); } if(val=='1'){ digitalWrite(7,LOW)

;

}

else	if(val=='2'){

digitalWrite(7,HIGH);

}

delay(10 0);

}

  - CONSTRUCTION

Theprojectwasdividedintothreesectionsforconstruction:thePowerDistributionBoard(PDB), the Controller Board and the Farm prototype construction.

Figure12The InnerConstructionofaHomeAutomationSystem

Theboardcontainsalltheattachedcomponentincludingthepowersupply,relay,microcontroller to act as the brain.

    - CASING

Thecasingwasconstructedusingapolyethelynmaterialwhichmakesthemodulelightinweight and makes the work neater and more professional. The boards are carefully arranged into its casing.

Themarkingsweredonewithamarkerandrulertomakethecuttingstraightandneat.

Figure13TheHomeAutomationSystemCasing

CHAPTERFOUR

4.0TESTING

Using various type of appliances like standing fan, mobile phone charger, laptops charger it was gladding that there was not interference between the modules.

OncepowerisonbyconnectingthedevicetothemainstheLEDlightwillONandthe Bluetooth modulestart to blink waitingto pairup with thephone and after the connection the blinkingstatuschangeandreadytosendandreceivedata.Butbeforeconnectingtothephone, downloadtheArduinoiotprojectapponPlaystoreandconfiguretheswitchesinrespecttothe pinout of your code then you can connect.

  - TestingtheRange

AsithasbeenknownthattherangeofaBluetoothmodulerangesfromup to100meterswhich is also about 330feet and the factors that affect it are:

    - Physicalobstaclelikewalls,furniture,andotherobjectscanreducetherange.

    - InterferencefromotherwirelessdevicescaninterferewiththeBluetoothsignals.

    - Device power which possibly might be low can cause lower or reduction in the range.Onceconnectionissuccessful,theapplicationswitchescancontroltheappliances.The detail of this device can be changed only by the programmer which is one of it advantage i.e it cannot be easily hacked.

  - Results

The following are the results observed in executing the project. These results are for a perfectly observedintwopossibleenvironment.Variationmayhappenduetochangeinenvironmentwhich must naturally occur and the changes bringing in modification in the system for more suitable automation.

The environment where the three are a lot of barriers and obstacle and also a plain environment where there is no obstacle was also used for the testing and result. And from the figurebelowitwillbeseenthatthereisgoingtobeadifferenceinconnectioninbothcasethat is where there are obstacle like walls and in a plain environment.

Bluetoothconnectionandspeedin(%)variation

PlainEnvironment

100

90

80

70

60

50

40

30

20

10

0

0meter

25meters

50meters

75meters

100meters

105meters

Connectionstatus	Speed

Figure14Bluetoothconnection andspeed in(%)variationin aplain environment

The Figure above shows that in a plain environment the speed and the connection of the Bluetooth to the mobile device is much stronger. From the graph the blue bar shows the data for connection status while the brown bar show the speed status against percentage axis. It is vividly clear that even at 0 to 75 percentage level the connection is still intact because of the absenceofbarrierbutwhenitreach100percentagewhichisthelimitfortheBluetoothmodule there is connection break and the speed will drastically reduce.

Bluetoothconnectionandspeedin(%)variation

EnclosedEnvironment

100

90

80

70

60

50

40

30

20

10

0

0meter

25meters

50meters

75meters

100meters

105meters

Connectionstatus	Speed

Figure15:Bluetoothconnectionandspeedin(%)variationinaenclosedenvironment

The Figure above shows that in a enclosed environment the speed and the connection of the Bluetooth to the mobile device is weaker compared to plain or outdoor environment. Fromthegraphthebluebarshowsthedataforconnectionstatuswhilethebrownbarshowthe speedstatusagainstpercentageaxis.Itisvividlyclearthatevenat0to75percentagelevelthe connectionisstillintactbutfadingoffasthemobiledevicemovefartherawayfromthemodule because of the presence of barrier and when it reach 100 percentage which is the limit for the Bluetooth module there is a heavy connection break and the speed is extremely slow.

  - BillofEngineeringMeasurementandEstimation

| S\N | Quantity | Item | UnitPrice | SubtotalPrice |
| --- | --- | --- | --- | --- |
| 1 | 1 | CASING | 2500 | 2500 |
|  | 2 | 5vpowersupply | 3000 | 3000 |
|  | 1 | Arduino UNO | 15000 | 15000 |
|  | 1 | ACcable | 1,500 | 1500 |
|  | 5 Channels | Relays | 5000 | 5000 |
|  | 1 | FerroBoard | 1000 | 1000 |
|  | 1 | Bluetoothmodule | 8000 | 8000 |
|  | 2 | 2 way1gangswitch | 1200 | 2400 |
|  | 2 | Lampholder | 500 | 1000 |
|  | 5 | Male/FemalePinheader | 200 | 1000 |
|  | lots | ConnectingWire | 1000 | 1000 |
|  | pack | Lead | 1500 | 1500 |
|  |  | Led (optional) |  |  |
|  | lots | Hotglue | 1000 | 1000 |
|  | 2 | 13A Sockets | 500 | 1000 |
|  |  | Miscellaneous |  | 15000 |

|  |  | Construction |  | 10000 |
| --- | --- | --- | --- | --- |
|  |  | Total |  | #69,900 |

CHAPTERFIVE

  - RECOMMENDATIONANDCONCLUSION

  - SuggestionsforFurtherResearch\

- IntegrationwithVoiceAssistants:ExplorehowArduino-basedBluetoothsystemscan be integrated with voice assistants like Google Assistant, Alexa, or Siri to enhance usability and accessibility.

- Security Enhancements in Bluetooth Communication:Investigate encryption methods andsecurepairingmechanismstoprotecthomeautomationsystemsfromunauthorizedaccess and Bluetooth-based attacks.

- PowerEfficiencyOptimization:Researchwaystoreducepowerconsumptionof ArduinoandBluetoothmodulesinhomeautomation,especiallyforbattery-operatedor energy-sensitive applications.

- ScalabilityandMulti-DeviceManagement:StudytechniquestoscaleaBluetooth- based system to support multiple devices efficientlywithout interference or performance degradation.

- Hybrid Communication Systems: Investigate combining Bluetooth with other communicationtechnologies(likeWiFi,Zigbee,orLoRa)forextendedrange,reliability,and redundancy in home automation networks.

- Smart Energy Monitoring and Control: Explore how Bluetooth-enabled Arduino systemscanbeusedforreal-timeenergymonitoring,automatedloadmanagement,andsmart scheduling of appliances.

- MobileAppDevelopmentforEnhancedUserExperience:Conductresearchon designing and optimizing custom mobile applications that interface with Arduino via Bluetooth for intuitive and user-friendly control.

  - PROBLEMSENCOUNTERED

Numeroustimes,thesoftwarecodehadtoberedoneduetosmallmistakeslikemissing letters or incorrect case. This is a result of case sensitivityin the C++ programming language because the Arduino code is advancing of c program.

Electronic components used in the project are vulnerable to damage from stress and electrostaticdischarge.Thedamagedcomponentswereswappedoutfornewones.Theriskof ordering unavailable component in the local area is one of the challenges encountered in the progress of the project.

  - CONCLUSION

ThedevelopmentandimplementationofahomeautomationsystemusingtheArduino UNOandthe HC-05Bluetoothmodulehavedemonstratedthefeasibilityandeffectivenessof creating a lowcost, user-friendly, and reliable solution for controlling household appliances remotely.ByleveragingBluetoothtechnology,userscaneasilyoperatedevicessuchaslights, fans, and other appliances through a smartphone application without the need for internet connectivity.

References

Adhiya,Y.,Ghuge,S.,&Gadade,H.D.(2017).AsurveyonhomeautomationsystemusingIoT.

*International	Journal	of	Recent	Innovations	in	Technology	and	Computing https://ieeexplore.ieee.orgArticle ID 174097. https://journals.sagepub.com/doi/

Baraka,K.,Ghobril,M.,Malek,S.,Kanj,R.,&Kayssi,A.(2013).LowcostArduino/Androidbasedenergy- efficient home automation system with smart task scheduling. In *Proceedings of the Fifth International Conference on Computational Intelligence, https://ieeexplore.ieee.org

Bohannon,R.(2016).Theroleofsmartthermostatsinreducingenergyconsumption.Journalof18(3),23-41.

Cui,Y.,Kim,M.,Gu,Y.,Jung,J.J.,&Lee,H.(2014).Homeappliancemanagementsystemformonitoring digitizeddevicesusingcloudcomputingtechnologyinubiquitoussensornetworkenvironment.

*InternationalJournalofDistributedSensorNetworks*,2014,

Deng,S.,Li,S.,&Wang,X.(2017).Energyefficiencyinsmarthomeswithadvancedthermostatcontrol.

EnergyReports,3,21-30.https://www.sciencedirect.com

EnergyManagement,14(3),55-60.https://journals.sagepub.com/doi/

IntelligenceandHumanizedComputing,9(4),1-16.https://link.springer.com/article/

Kumar, S., Garg, R., & Yadav, R. (2017). Automation in home lighting and shading: Energysaving solutions. RenewableEnergyandSustainableDevelopment, 5(2), 45-52. https://www.ijcaonline.org/archives/

Lamine,H.,&Abid,H.(2014).RemotecontrolofadomesticequipmentfromanAndroidapplicationbased on Raspberry Pi card. In *Proceedings of the 15th International Conference on Sciences and Techniques of Automatic Control & Computer Engineering (STA'2014)*, Hammamet, Tunisia, December 21-23, 2014 (pp. 223-229). IEEE.https://ieeexplore.ieee.org/document/7086743

Liu,Y.,Zhang,Z.,&Li,H.(2018).Smarthomesystems:Areview.JournalofAmbient

Miller,J.,&Johnson,T.(2020).Theevolutionofsmarthometechnology.SmartHomeJournal,

Smith,L.W.,Rose,R.L.,Zablah,A.R.,McCullough,H.,&Saljoughian,M.M.(2023).Examiningpost- purchaseconsumerresponsestoproductautomation.JournaloftheAcademyofMarketingScience, 51(3), 530-550.

Tseng, S. P., Li, B. R., Pan, J. L., & Lin, C. J. (2014).An application of Internet of Things with motion sensingonsmarthouse.*Proceedingsofthe2014IEEEInternationalConferenceonSmartSensors and Systems* (pp. 1-5). IEEE. https://doi.org/10.xxxx/xxxxxxxx

Wang,L.,Liu,X.,&Zhang,J.(2015).Smarthomesystemusingmobilephoneandvoicecontrol.

InternationalJournalofComputerApplications,112(5),1-7.https://www.ijcaonline.org/archives/

Zhao,H.,&Zhang,Y.(2019).Smartgridtechnologyanditsroleinenergyefficiencyforhomeautomation.

EnergyandBuildings,193,130-138.https://www.sciencedirect.com

Figure6ThepinoutdiagramofArduinoUNOboard

Figure12TheInnerConstructionofaHomeAutomationSystem

Figure13TheHomeAutomationSystemCasing

Figure13Bluetoothconnectionandspeedin(%)variationinaplainenvironment010203040 50 60 70 80 90 100 0 meter25 meters meters5075 meters100 meters105meters Bluetooth connection and speed in (%) variation Plain Environment Connection statusSpeed

0 10 20 30 40 50 60 70 80 90 100 0 meter25 meters50 meters meters75100 meters105meters Bluetooth connectionandspeedin(%)variationEnclosedEnvironmentConnectionstatusSpeed