DIGITAL SURFACE MODEL

OF PART OF KWARA STATE POLYTECHNIC, ILORIN

BY

GANIYU MISTURA DAMILOLA

MATRIC NO: HND/22/SGI/FT/111

SUBMITTED IN PARTIAL FULFILMENT OF THE REQUIREMENTS FOR THE

AWARD OF HIGHER NATIONAL DIPLOMA (HND) IN SURVEYING AND

GEO-INFORMATICS TO THE DEPARTMENT OF SURVEYING AND

GEOINFORMATICS, KWARA STATE POLYTECHNIC, ILORIN.

JULY, 2025

# CERTIFICATE

This is to certify that all the field work and information given in this project were obtained as a result of the observations and measurements carried out by me and that the survey was carried out in accordance with survey rules and regulations as well as departmental instructions.

Name of the Student:	GANIYU MISTURA DAMILOLA

Signature of the Student:	______________________

Date of Completion:

CERTIFICATION

This is to certify that this project report was prepared and submitted by GANIYU MISTURA DAMILOLA  with Matric No:- HND / 22 / SGI / FT / 111  to the department of Surveying and Geo-informatics, Institute of Environmental Studies, Kwara State Polytechnic, Ilorin, as a result of field observation and measurement using appropriate instruments and equipment to carry out the task.

------------------------------------				   	---------------------------

MR ISAU. A . I.							DATE

Project Supervisor

------------------------------------					   -------------------------

DATE

Project Coordinator

------------------------------------					  -------------------------

MR ISAU A. I.							DATE

Head of Department

……………………………………………….					………………………………..

# (EXTERNAL EXAMINER)					DATE

# DEDICATION

This project is dedicated to the author of life and finisher of our faith. And also to my lovely parent MR and MRS GANIYU for their unceasing and parental cares of all through, you people are the best I can ever have in my life time.

ACKNOWLEDGEMENT

All glory and adoration goes to Almighty God for his protection over my life throughout the programe, the journey was rough and tough but he saw me through.

My gratitude goes to my supervisor in person of MR ISAU A. I. for his guidance and support during the project, and for making this project a successful one. Also my appreciation goes to the Head of Department (H O D) of Surveying and Geoinformatics, Kwara State Polytechnic, Ilorin, Mr. Isau Ibraheem Abimbola and all the staffs of the department, may God be with you and grant all your heart desires, amen.

I will also like to show my heart of gratitude to my parents Mr. & Mrs. Ganiyu for their parental care, love, encouragement and words of advice day and night, I pray that God almighty should grant you long life to harvest the fruit u labor for .Amen.

I will never seize my breath without appreciating all my lecturers for their fatherly advice, love, care, and the word of motivations and encouragement before and during my period of study, I say a big thank you for standing by me and for keeping my company day and night, may God reward you here on Earth and in Heaven, I really appreciate you both.

ABSTRACT

The project was carried out to generate a Digital Surface Model of part of Kwara State Polytechnic, Ilorin. The reconnaissance was first carried out, spatial data were acquired directly from the field using HandheldGPS, while non-spatial data were obtained through social survey by means of oral interviews and observations. The data were processed and analyzed using ArcGIS 10.1 software for database design and geospatial analysis, AutoCAD2007 for graphical representation, surfer 10 for generating the contour as well as Digital Terrain Modelling (DTM) and other relevant software. Some queries were generated to enable spatial search on the database created and transform such dataset into information system which will support decision making process, as a result of this we produced Topographic data in softcopy to be accessible for various application . Finally, a comprehensive technical report was written.

TABLE OF CONTENTS

CERTIFICATE………………………………………………………………………………………….ii

CERTIFICATION………………………………………………………………………………………iii

DEDICATION…………………………………………………………………....................................iv

ACKNOWLEDGEMENTS……………………………………………………………………………..v

ABSTRACT……………………………………………………………………………………………vi

TABLE OF CONTENTS……………………………………………………………………………....vii

LSIT OF FIGURES………………………………………………………………………………….viii

LIST OF TABLES…………………………………………………………………………………...ix

CHAPTER ONE……………………………………………………………………………………1

1.0 INTRODUCTION ……………………………………………………………………………...1

1.1 BACKGROUND TO THE PROJECT……………………………………………………………1

1.2 STATEMENT OF PROBLEM ……………………………………………………………………3

1.3 RESEARCH QUESTIONS…………………………………………………………………….4

1.4 AIM OF THE PROJECT…………………….…………………………………………………4

1.5 OBJECTIVES OF THE PROJECT…………...………………………………………………...5

1.6 SCOPE OF THE PROJECT…………………………………………………………………….6

1.5 PERSONNEL INVOLVED IN THE PROJECT………………………………………………6

1.6 PROJECT AREA………………………………………………………………………………7

CHAPTER TWO…………………………………………………………………………………..8

2.0 LITERATURE REVIEW……………….………………………………………………………8

CHAPTER THREE………………………………………………………………………………16

- METHODOLOGY……………………………………………………………………………16

3.1.1 VIEW OF REALITY……………………………………….………………………………..17

3.1.2 CONCEPTUAL DESIGN…………………………………………………………………….18

3.2 RECONNAISSACE...………………..………………………………………………………………19

3.2.1 OFFICE PLANNING………………………………………………………………………….19

3.2.1.2 FIELD RECONNAISSANCE………………………………………………………………..20

3.2.3 EQUIPMENT USED/SYSTEM SELECTION AND SOFTWARE…………………………….21

3.2.7 GEOMETRIC DATA ACQUISITION…………………………………………………………..23

3.3 DATA PROCESSING……………………………………………………………………………..23

3.4 PHYSICAL DESIGN………………………………………………………………………………25

3.5 DATABASE IMPLEMENTATION………………………………………………………………26

3.6.1 DATA SECURITY……………………………………………………………………………..26

3.6.2 DATA INTEGRITY……………………………………………………………………………27

3.6.3 DATABASE MANITENANCE………………………………………………………………..27

CHAPTER FOUR…………………………………………………………………………………..28
4.0  SPATIAL ANALYSES AND INFORMATION PRESENTATION……………………………28

4.1 TESTING OF DATABASE……………………….……………………………………………..32

CHAPTER FIVE…………………………………………………………………………………….40

5.0 PROJECT COSTING, SUMMARY, CONCLUSION AND RECOMMENDATIONS………….40

5.1 PROJECT COSTING…………………………………………………………………………… 40

5.2  SUMMARY……………………………………………………………………………………...44

5.3 CONCLUSION…………………………………………………………………………………..45

5.4RECOMMENDATION…………………………………………………………………………..46

REFERENCE………………………………………………………………………………………...47

LIST OF FIGURES

FIG 1.6 STUDY AREA………………………………………………………………………………..6

FIG 3.1 DESIGN AND CONSTRUCTION PHASE OF A SPATIAL……………………….……….21

FIG 3.3 POINT ENTITY AND ITS ATTRIBUTES………………….…….………….……………..22

FIG 3.4  LINE ENTITY AND ITS ATTRIBUTES……………………………………..……………22

FIG 3.5 POLYGON ENTITY AND ITS ATTRIBUTES……………………………….…………….22

FIG 3.7 RECCE DIAGRAM OF THE PROJECT AREA…………………………………………….26

LIST OF TABLE

TABLE 3.1: NODE ENTITY AND ITS ATTRIBUTES……………………………………………23

TABLE 3.2: ARC ENTITY AND ITS ATTRIBUTES………………………………………………24

TABLE 3.3: POLYGON ENTITY AND ITS ATTRIBUTES………………………………………24

TABLE 3.11 NODE ENTITY DATA STRUCTURE……………………………………………….36

TABLE 3.1 ARC ENTITY AND ITS ATTRIBUTES STRUCTURE………………………………36

TABLE 3.13: POLYGON DATA STRUCURE ……………………………………………………36

# CHAPTER ONE

# 1.0	INTRODUCTION

# 1.1	BACKGROUND TO THE STUDY

The surface of the land is an important resource which human activities are continually modifying. Land is the earth's surface, land and water, plus natural resources in their original state, such as: mineral deposit, wildlife, fish, timber, etc. Particular areas of land can be utilized by humans in diverse ways. These can include residential, institutional, business, industrial, agricultural, forestry, park, and other relatively natural land uses. In order to create a sustainable and favourable living, it is essential to protect our property or investment, in which land surveying helps in the positioning of objects in space and time as well as the positioning and monitoring of physical features, structures and engineering works on, above or below the surface of the earth and the analysis, interpretation and integration of spatial objects and phenomena in GIS, including the visualisation and communication of such data in maps, models and mobile digital devices. (http://en.wikipedia.org/)

A Digital Elevation Model (DEM) is a digital representation of ground surface, topography or terrain. A DEM can be represented as a raster (a grid of squares) or as a triangular irregular network. DEMs are commonly built using remote sensing techniques; however, they may also be built from land surveying. DEMs are used often in geographic information systems, and are the most common basis for digitally-produced relief maps.

A Digital Surface Model (DSM) is a representation of any surface by using three dimensional (3D) coordinates, normally X, Y, Z Cartesian coordinates. The surface might be part of a small object, for example a vase, or it may be a very large object such as the surface of the Earth. When related to the Earth.s surface, these coordinates are often converted into Easting, Northing and Height (E, N and H). A Digital Elevation Model (DEM) specifically relates to elevation and therefore height, and so will be defined as a DSM of the Earth’s surface. It is used generically to define the ground surface, also called a Digital Terrain Model (DTM), and the ground surface plus the tops of features above the ground surface such as artificial structures and vegetation. The DSM is therefore the first surface that many airborne and spaceborne sensors will interact with. A DTM is normally created by stripping off all above ground surface features from the DSM to reveal a bald-earth Model.

A digital surface model (DSM) is an important geospatial infrastructure used in various fields, such as ortho image generation, 3D analysis, and city modelling. It is usually represented as grid data composed of regularly spaced cells with elevation values. For automatic generation of DSMs, light detection and ranging (LiDAR) survey and stereo image matching are mainly used. While high-quality point clouds can be extracted from LiDAR, it may be expensive and might lack texture information. On the other hand, stereo image matching has become a more attractive solution for DSM generation with the recent spread of high-resolution satellite and unmanned aerial vehicle (UAV) images. In this paper, we focus on DSM generation from stereo image matching.

Digital Elevation Model (DEM) represents a very important geospatial data type in the analysis and modelling of different hydrological and ecological phenomenon which are required in preserving our immediate environment. DEMs are typically used to represent terrain relief. DEMs are particularly relevant for many applications such as lake and water volumes estimation, soil erosion volumes calculations, flood estimate, quantification of earth materials to be moved for channels, roads, dams, embankment etc.

There are known methods by which DSMs can be constructed. Such methods include but not limited to digitising existing topographic maps or by using stereoscopic aerial photographs, with the advance of digital photogrammetry.

Maune (2001) stated that Surface modelling is essential for various applications; orthophoto productions, engineering design, floodplain mapping, telecommunication, etc. all require surface data with different level of detail and accuracy.

# 1.2	STATEMENT OF PROBLEM

Topographic map of part of Kwara State Polytechnic, Ilorin is not up-to-date and heights of features were not represented. The production of Digital Surface Model will solve the following problems.

- Lack of current maps for the school’s management use.

- Lack of information about the height of details within the School.

# 1.3	AIM AND OBJECTIVE OF THE STUDY

## 1.3.1	AIM

The aim of the project is to produce a digital surface model of part of Kwara State Polytechnic, Ilorin and to demonstrate my capability in executing projects of such calibre.

# 1.3.2	OBJECTIVES OF THE STUDY

In achieving the above stated aim, the following operations were carried out:

- Database Design for the study area

- Data Acquisition.

- Creation of Database for Spatial entities in the study area.

- Spatial Analysis.

- Information Presentation and Management.

# 1.4	SIGNIFICANCE OF THE STUDY

The significance of this project is to, as stated below;

- Easy planning of any meaningful development.

- Provide up-to-date Digital Elevation Model for the School.

- Provision of useful spatial analysis of information about any object for viable decision making and deformation studies.

- For map revision purposes i.e. easy retrieval and updating of the database, and

- To get student acquainted with the knowledge needed for future environmental occurrences.

# 1.5	SCOPE OF THE PROJECT

The scope of the project covers the following:

- Review of previous projects, literatures, journals, reports and text books.

- Project Planning.

- Database Design techniques.

- Data Acquisition with automated Survey equipment for perimeter traversing, detailing and spot heighting.

- Data processing, spatial analyses and information presentation.

# 1.6	PERSONNEL

The following are the personnel involved in executing the project:

| NAMES | POSITION |
| --- | --- |
| GANIYU MISTURA DAMILOLA | AUTHOR |
| ILESANMI ESTHER | MEMBER |
| TIAMIYU ZAINAB | MEMBER |
| BABALOLA OLUWATOSIN | MEMBER |
| ADEOYE AYOBAMI | MEMBER |
| OGUNLADE MATTHEW | MEMBER |

SOURCE: AUTHOR 2025

# 1.7	STUDY AREA

The site is part of Kwara State Polytechnic, Ilorin, Nigeria. It is situated along old Jebba Road and have geographic locations between latitude 04o 49’ 25”N and 4o 49’17” longitude 8o 55’ 12”E and 8o 55’ 10”E. The project area is developed with existing structures such as, road, electricity line, buildings etc

# Source: Author 2025

# CHAPTER TWO

# 2.0	LITERATURE REVIEW

Digital Surface Models, or DSMs, represent the topography of the earth’s surface, including all objects on it, such as trees, buildings, and other features. DSMs are created using aerial data capture methods, like LiDAR (Light Detection and Ranging) or photogrammetry. LiDAR uses laser beams to measure the distance between the sensor and the ground, while photogrammetry uses aerial photographs to create 3D models. There are many benefits of DSMs. For one, they accurately represent the earth’s surface, allowing users to accurately measure features and make decisions based on that information. 3D visualization of the Earth’s surface allows users to see the surface and features in a more realistic and immersive way. This is particularly useful in applications such as urban planning and design where visualizing the impact of planned developments on the existing landscape is critical. Digital surface models offer several advantages over traditional topographic mapping. Because DSMs are typically created using LiDAR or photogrammetry, they provide high-resolution data. This means that DSMs can capture much more detail than traditional topographic maps, often based on field surveys, and may not accurately capture all features. The high-res data also provides highly accurate elevation measurements. This allows for more precise calculations of slope, aspect and other terrain characteristics, essential for industries that model water flow and drainage.

Digital Terrain Models, or DTMs, represent the bare earth surface without any features such as buildings or vegetation. DTMs are created using methods similar to DSMs, such as LiDAR or photogrammetry, but require additional processing to remove objects above the ground. This process is known as “bare-earth extraction.”

A digital terrain model has three main components:

- The surface itself

- The elevation at each point on the surface

- The slope at each point on the surface

This model is similar to DSM, but it represents only the highest points on the surface of the terrain instead of every point on its surface as DSM does. DTMs can be used to create contour lines. This is useful for building roads or railroad tracks because they allow you to see if there are any obstacles such as trees or rocks in your way before you start building them.

DTMs are particularly useful for terrain analysis, such as slope and aspect calculations, hydrological modeling and geological applications. Digital Elevation Models, or DEMs, represent the elevation of the earth’s surface. DEMs can be either a DSM or DTM. The resulting model shows the elevation of the earth’s surface. The DEM is essentially a superset of the DSM and DTM. DEMs are a 3D grid of terrain elevations collected from an area with satellite imagery or LiDAR technology. DEMs are useful for visualizing landscape changes over time, creating simulations of natural phenomena like landslides and floods, and measuring land use changes over time — as well as many other applications. They can also be combined with other models to create more detailed representations of the earth’s surface.

Digital models offer numerous benefits over traditional methods of mapping and surveying. They accurately represent the earth’s surface, allowing for better decision-making and planning. They can also be created quickly and efficiently using aerial data capture methods such as LiDAR, which reduces the need for ground-based surveys and minimizes environmental impact. In addition, digital models can be easily shared and used in many applications, making them a valuable resource for many industries. They can be used in GIS software for spatial analysis, integrated into 3D modeling software, or applied in conjunction with other data sources such as satellite imagery.

However, because of the peculiar nature and importance of the built environment, planners around the world have emphasized the need to cater for physical development within the cities and other human settlement using the planning principles and concepts. The essence of this is to achieve a livable settlement described as ‘a welcoming, organized and comprehensible environment, where physical elements is unifying, accessibility within and without facilitate communication and promote interactions and flexibility, involving the design and development of buildings, circulation and service/utility system adapt to the needs of an evolving environment’ (Office of University Planning,1997). Physical development planning which is concerned with the process of ordering the use of land and siting of buildings structures and communications to secure the maximum degree of economy, functionality, convenience, and beauty (Keeble, 1969) has, therefore, come to be accepted as a major area of planning because it encompasses all other facets of human interaction with land, including building engineering, mining or other operations in, on, over or under any land.

Aerial data capture methods such as LiDAR have revolutionized how we map and analyze the earth’s surface. LiDAR is a highly accurate and efficient method of collecting data and can be used to create highly detailed digital models of the earth’s surface.

LiDAR emits laser beams from a sensor mounted on an aircraft, which bounce off the earth’s surface and return to the sensor. The time it takes for the laser beam to return to the sensor is used to calculate the distance between the sensor and the ground.

LiDAR can be used to collect data over large areas quickly and efficiently, making it ideal for creating digital models of the earth’s surface. It is particularly useful in complex topography areas where ground-based surveys would be difficult or impossible. DSM Includes all objects on the earth’s surface such as trees, buildings, and other features. In other words, it represents the topography of the earth’s surface including all objects on it. Also it is Useful in applications such as land-use planning, urban modeling, and forestry. DTM Represents the bare earth’s surface elevation with all features removed, including buildings, trees, and other objects. Therefore, additional processing is required to remove any objects above the ground. DTMs are created by removing objects from a digital surface model (DSM) to create a more accurate representation of the terrain. Particularly useful for terrain analysis, such as slope and aspect calculations, hydrological modeling, and geological applications. DEM Represents the earth’s elevation. It is Useful for applications such as urban planning, forestry and land-use planning.  It provides a more complete understanding of the topography of a given area.

As technology advances, digital models will become more prevalent and valuable in many areas. By understanding the difference between these models and their benefits, we can continue to make informed decisions and utilize these powerful tools to their fullest potential so we can understand the world around us.

GIS technology has made topographical mapping and geo-visualization more accurate, appealing and a large area can be mapped within a far lesser time frame. It has the power to visualize geographic data by allowing the user to dynamically integrate multiple data sources, unconstrained by format, scale and coordinate system. Also, the capability of GIS technology to visualize and analyze spatial and non-spatial information from diverse sources makes it a powerful platform for multilevel decision making, and so greater credit should be given to its geo-visualization capabilities. They enable a large volume of geographic data to be summarized into a map which, without doubt, facilitates fast visual interpretation (Nkeki, 2013a). In this paper, GIS technology is viewed from two dimension–the spatial data gathered with sensor-based space satellite devices used for producing topographic maps such as DEM and those used for manipulating and geo-visualizing the DEM data such as GIS softwares and algorithms. Geovisualization (geographic visualization) refers to the technologies and procedures of displaying geographic data for quick visual interpretation. Presently, this method of visualizing geographic dataset is fast replacing the cumbersome paper topographical map. It is capable of visualizing multiple spatial data and presents a friendly interface between the map and the user.

DSM (digital surface model) is a topographic digital model, which describes the relief and the situation on the surface. It consists of digital terrain model and represents the earth's surface, including all objects on it. GEO Innoter provides the creation of digital surface models (DSM) based on satellite or aerial survey materials, followed using digital surface models for 3D modeling of cities and other terrain objects. A digital surface model (digitized map) is a three-dimensional representation of the heights of the Earth's surface, including natural or man-made objects located on it. The main data sources for creating digital surface models, digitized map and 3D modeling are:

- topographic map-provides a geographic reference that stores coordinate and elevation information about terrain features such as terrain, buildings and structures, roads, rivers, land-use contours, administrative boundaries, and so on.

- Remote sensing materials obtained in stereo mode are used to create digital surface models and orthophotos.

- a cloud of points obtained by laser scanning

The orthophoto and topographic map are used to obtain the contour of the buildings. Ground-based photos of each individual building from multiple points of view are necessary to build a photorealistic 3D model of the buildings.

The digital surface model is used in a variety of planning, analysis, visualization, and modeling applications, including:

- localities for various purposes (urban planning, architectural planning, tourism, virtual tours, etc.),

- navigation systems,

- intelligent transport systems,

- forecasting of urban risks;

- telecommunications-positioning of mobile phone transmitters;

- in the mapping of flood risk.

Podobnikar and Vrec ˇko proposed a methodology for extraction of terrain (bare

earth) points from point clouds captured with airborne laser scanning. It is focused on

the areas that are covered with dense vegetation – the places that create extra difﬁculties  for DEM generation. They tested a number of different ﬁltering techniques accessible by freeware. The resulting methodology was fused using three ﬁlters: multiscale curvature classiﬁcation, hierarchical robust interpolation and LAStools ﬁltering. The article by Zawadzki, Nikie, and Warszawski addresses the problem of modeling virtual caves and tunnels, and presents an alternative solution in the form of a hybrid system. The innovative approach combines two independent methods well known in computer graphics: shape grammars and shape morphing for modelling three-dimensional geometry. Bühler, Marty, and Ginzler examine the quality of digital surface models derived from opto-electronic scanner data using photogrammetric image correlation techniques based on the multispectral nadir and backward looking sensor data. Tested datasets came from high alpine terrain and the results showed that the deviations are surprisingly low. The work presented by Caha, Tuc ˇek, Vondráková, and Paclíková looks at uncertainty processing. They work with fuzzy surface and utilized the Neighbourhood method to calculate slope. Georelief transﬁguration in areas affected by open-cast mining is the focus of the article of Pacina, Novák, and Popelka. They reconstructed the original georelief in different time periods together with analysis showing the process of landscape devastation. The workﬂow and analysis is based on precise elevation data obtained from aerial photographs and old maps. The use of a digital surface model in solar radiation modelling is presented by Hoﬁerka and Zlocha. They developed new open-source 3-D solar radiation tools for GRASS GIS. Their module is based on the existing solar radiation methodology with a new capability to process 3-D vector data frequently used in the 3-D city models representing complex urban environments. The calculation procedure is based on the effective volume tessallation using a voxel data structure. The shadowing algorithm uses a combined vector-voxel approach. Another application of using a digital surface model is presented by Zieher, Formanek, Bremer, Meissl, and Rutzinger. Their research was focused on estimating the extent of rockfall area, especially the sensitivity of this estimation to the resolution of an input surface model Persson, Hasan, Tang, and Pilesjo targeted permafrost landscapes and the water table depth and water depths above the frozen layer for different peat land types. They examined modeled ﬂow routing patterns with a topographical wetness index in these areas and then evaluated the correlations between the topographical wetness index and site-septic wetness.

Finally, Kubiak and Dzieszko present an unconventional use of thermo vision for topoclimatic delimitation. The method is based on the spatial distribution of land surface temperature, which indicates the amount of solar energy reaching the Earth’s surface, and depends primarily on a terrain’s shape and land cover types. Land surface temperature based on topoclimatic classiﬁcation was compared with a classical, theoretical topoclimatic classiﬁcation. New classes of topoclimates were delimited and some existing types were diversiﬁed.

# CHAPTER THREE

## 3.0	METHODOLOGY

## 3.1	PROJECT PLANNING

# Before beginning any survey assignment, project planning is a crucial requirement. It entails a careful investigation of the topic matter about a certain land region that is being evaluated. This crucial step in the surveying process allows the surveyor to become acquainted with the chosen location before starting work. During the reconnaissance phase, the survey's goals, requirements, and required precision levels are examined because these elements have a big impact on the choice of survey tools and techniques. In the end, this project resulted in the development of a first working diagram. Reconnaissance is divided into two phases: (i) Office Planning and (ii) Field reconnaissance.

### 3.1.1	 FIELD RECONNAISSANCE

# As part of this process, a surveyor or student must physically visit the site to gain a thorough understanding of its topography, including any vegetation, accessibility, existing frame work, as well as the social and economic 19 activities that take place there. The optimal approach and tools for improved decision-making must also be chosen. This on-site visit gives the surveyor or student a comprehensive grasp of the entire region and allows them to look at the entire area of the land. To guarantee correct orientation, the locations of all accessible horizontal and vertical controls in the region of the research area will be visited, and their outlooks will be viewed. This exercise makes it easier to create a Recce diagram, which is shown below but is not drawn to scale:

Figure 1.4: RECCI DIAGRAM

### 3.1.2	 OFFICE PLANNING

# Gathering crucial project information is the focus of this phase, which is usually done in an office setting. It is the initial phase of planning that comes before the fieldwork itself and entails gathering and analyzing a variety of project-related data. This covers a range of requirements for the project, scheduling, budgeting, and generating the coordinates for the control points under the project control prefix. It includes figuring out the best process, picking the right tools, as well as subscription for the CORS station.

## 3.2 	EQUIPMENT USED/ SYSTEM SELECTION SOFTWARE

The following equipment was used:

- Total station and accessories

- One Tripod

- Two Prism

- One 7.5 meters short tape

- One Plumb bob

- One Cutlass

- Bottle corks and nails

- Writing materials.

### 3.2.1 	HARDWARE USED

Model: 			HP 15 Note book PC

System Manufacturer:		Hewlett-Packard

Rating: 			1.0 Windows Experience Index.

Processor: 			Intel(R) Core(TM) i5-3230M CPU @ 2.60GHz, 2601 Mhz, 2 Core(s), 4 Logical Processor(s)

Installed Memory RAM: 	8.00 GB

System Type: 			x64-based PC.

Product ID: 			F9F84EA#ABV

### 3.2.2 	SOFTWARE USED

Some of the software used includes:

- AutoCAD 2007

- Notepad

- Microsoft Word 2007

- Microsoft Excel 2007

- Arc GIS 10.5

## 3.3	 DATA ACQUISITION

# This involves using different surveying methods and tools to gather data and make observations in the field. To acquire data for this project, HANDHELD GPS and Total station were both used.

# 3.3.1 	PRIMARY DATA SOURCE: This is a type of data source in which data is direct acquired from the field such as data acquired from field survey operations.

# 3.3.2 	SECONDARY DATA SOURCE: This data source are already made data or gotten from research work, which has been acquired for future use. This involves downloading satellite imagery used for the project area and Google map as guide.

## 3.4	TEST OF INSTRUMENT

Testing of instruments is very crucial when carrying out any survey operation. Having collected the required instruments from the department, the check was carried out on the accuracy and precision of both the Handheld GPS and TOTAL STATION to ensure they were in proper working condition before the actual field observation of the project.

### 3.4.1	 TEST FOR TOTAL STATION

The horizontal collimation and vertical index inaccuracies of the instrument were confirmed. Once the device was positioned on a rather flat surface, the required temporary modifications, including focusing, centering, and setting, were made. Configuration information, such as extra parameters and fast settings, was examined and adjusted as necessary. The configuration parameters selected collimation, namely horizontal collimation. Next, to focus on a distant object, the means' option located on the left side of the device was chosen. The instrument was transited, and the right side went through the same procedure. Then, when the 'accept' button was pushed, the table that was displayed showed the old and new values for the vertical index errors and horizontal collimation. Re-press "ok" signified acceptance of the updated values, which were minimal.

Figure 3.5: INSTRUMENT TEST

Top of Form

The diagrammatic representation of the procedure and the readings obtained are shown below

Table 3.4.1 COLLIMATION TEST OF TOTAL STATION

| STN | SIGHT | FACE | HCR | REDUCTION | VCR | REDUCTION |
| --- | --- | --- | --- | --- | --- | --- |
| A | B | L | 126º 07’ 11.33” |  | 67º 22’40” |  |
|  | B | R | 306º 07’ 31.3” | 180º 00’ 20” | 292º 37’ 1” | 359 º 59’ 41” |

HORIZONTAL COLLIMATION ERROR = 180°00’20’’ - 180°

=00°00’20’’

=00°00’20’’/2   =00°00’10’’

VERTICAL INDEX ERROR=360° - 359°59’41.6’’

=00°00’18.4’’

=00°00’18’’/2   =00°00’9.2’’

The instrument's satisfactory operation was validated by the acceptance of the updated readings. It appears from this result that any possible issues can be resolved or at least greatly reduced.

### 3.4.2 REFERENCING AND ORIENTATION OF TOTAL STATION

Tripod Setup:

- Place the tripod over the station point (known or assumed).

- Level the tripod head as much as possible by adjusting the legs.

Mount the Total Station:

- Fix the total station onto the tripod head.

- Ensure it’s securely locked in place.

Centering and Leveling:

- Use the optical plummet or laser plummet to center the instrument exactly over the survey station mark (station point).

- Level the instrument using the foot screws and circular bubble.

- Fine-tune leveling using the electronic level display.

Power On and Instrument Configuration

- Switch on the Total Station.

- Set the correct project, coordinate system, and units.

- Select the appropriate survey mode (e.g., topographic survey or station setup).

Orientation (Station Setup)

Orientation means establishing the direction of the horizontal angle reference (back sight direction).

Orientation by Known Coordinates (Resection or Free Station)

Used when both the instrument station and a back sight point have known coordinates.

- Input the coordinates of the instrument station (or let the instrument compute it later).

- Sight and measure to one or more known reference points (minimum of 2 for better accuracy).

- The Total Station calculates the instrument position and orientation using these known points.

### 3.3.7 PERIMETER TRAVERSING

The perimeter of the project area was traversed for the purpose of the coordinating the boundary points. HANDHELD GPS (GALAXY G1) was connected to Kwara State CORS station for referencing and orientation after which all the boundary points were traversed using HANDHELD GPS from the initial until each successive boundary is occupied.

### 3.3.8	DETAILING

All features such as roads, buildings, electric poles, street light, trees, etc. were captured by coordinating their edges simultaneously as the perimeter is being measured within the project area. Buildings were measured and identifiers (BD) were used to differentiate them from other features. Roads were also measured and labelled (RD) accordingly. Data captured here were basically in 3Dimension i.e. Easting, Nothings and Elevations (X, Y, and Z) coordinates.

### 3.3.9	REMOTE ELEVATION MEASUREMENT

All the features such as buildings, electric poles, street light, etc. captured by coordinating their edges from a subsidiary traverse stations created within the project area. Buildings heights were measured and recorded. Electric poles height were also measured and recorded accordingly. So also the same was repeated for the trees. Data captured here were basically in 3Dimension i.e. Easting, Nothings and Elevations + Heights (X, Y and Z + H) coordinates. The following procedures were followed:

- Escape key was pressed then F1 for MENU

- The REM programme was selected.

- Observation was made again and the height of reflector was displayed.

- The telescope was tilted to the top of the object and the height keeps changing until F4 is been pressed to stop.

- The value was recorded.

Figure 3.3.9: REM OBSERVATION STRATEGY

### 3.3.9 	SPOT HEIGHTINGS

This was carried out by occupying randomly the subsidiary points within the project area and taking measurements (Easting, Nothings and Heights) along the terrain at approximate intervals to determine the difference in height between points and some areas within the project location. Spot heighten is very important in producing topographical information of the area with the aid of ArcGIS 10.5.

## 3.4	PHYSICAL DESIGN

At this stage, all geospatial (attribute) data were structured and organized to form a database in a format acceptable by the implementing software and hardware. This was done in a way that, stored information can be accessed and retrieved at any time. Also, for regular updates when the need arise and to allow for analytical functions to be carried out answering generic questions. Also, it provided integrity and security that must be obeyed before such data could be accepted into the records. These virtues are as show in the tables below. ArcGIS 10.5 was used for database creation of the project area.

Table 3.4A TREE TABLE

| ATTRIBUTE | DATA TYPE | WIDTH | DECIMAL |
| --- | --- | --- | --- |
| T_ ID | String | 5 | - |
| T_NAME | Float | 20 | - |
| EASTING | Number | 10 | 3 |
| NORTHING | Number | 10 | 3 |
| T_USE | Float | 10 | - |
| T_HEGHT | Number | 7 | 3 |

Table 3.4B ROAD TABLE

| ATTRIBUTE | DATA TYPE | WIDTH | DECIMAL |
| --- | --- | --- | --- |
| POINT_ID | String | 10 | - |
| NORTHING | Number | 10 | 3 |
| EASTING | Number | 10 | 3 |
| RD_HEIGHT | Number | 7 | 3 |

Table 3.4C ELECTRIC POLE TABLE

| ATTRIBUTE NAME | DESCRIPTION OF ATTRIBUTE | DATA TYPE | WIDTH | DECIMAL |
| --- | --- | --- | --- | --- |
| POINT_ID | Electric pole identifier | Number | 6 | - |
| NORTHING | Coordinate | Number | 15 | 3 |
| EASTING | Coordinate | Number | 15 | 3 |
| EP_TYPE | Type of Electric pole | Text | 15 | - |
| EP_HEIGHT | Height of the Electric pole | Number | 5 | 2 |

Table 3.4D BUILDING TABLE

| ATTRIBUTE | DATA TYPE | WIDTH | DECIMAL |
| --- | --- | --- | --- |
| P_ID | String | 15 | - |
| NAME | Text | 15 | - |
| CONDITION | Text | 15 | - |
| USE | Text | 15 | - |
| HEIGHT | Number | 15 | 2 |
| AREA_METERS | Number | 15 | 2 |

Table 3.4E STREETLIGHT TABLE

| ATTRIBUTE NAME | DESCRIPTION OF ATTRIBUTE | DATA TYPE | WIDTH | DECIMAL |
| --- | --- | --- | --- | --- |
| POINT_ID | Street light identifier | Number | 6 | - |
| NORTHING | Coordinate | Number | 15 | 3 |
| EASTING | Coordinate | Number | 15 | 3 |
| SL_HEIGHT | Height of the street light | Number | 5 | 2 |

## 3.5	DATABASE CREATION

Database is an organized integrated collection of data stored so as to be capable of use by relevant application with data being accessed by different logical part. This was the construction phase where database was created. After the table has being populated via the keyboard, some attributes such as area, perimeters of parcel were automatically displayed by special command in the ArcGIS 10.5 version.

Figure 3.5: ABSTRACT OF TABLE CREATED

### 3.5.1 ATTRIBUTE DATA ACQUISITION

The data about the characteristics of features especially man-made features were acquired through oral interview method and online source (Google Earth). Social survey was carried out during which information such as; building name, building use and related data about features were obtained from the students and staffs within the school. All these data formed the attribute aspect of the feature necessary for the database creation.

## 3.6	DATA PROCESSING

### 3.6.1	TOTAL STATION DATA DOWNLOADING PROCEDURE

Data downloading is the process of transferring data from the instrument into the computer system connected through the connection cable or wireless such as Bluetooth with the help of software in the computer, the process is as follows as performed on Total station.

- Connection of the two devices (computer and the instrument).

- Go to project working folder created on the instrument.

- Click on export.

- Input the name of file

- Indicate the file extension for the output

- Permit the other device to receive the file.

### 3.6.2 DRAFTING AND PLOTTING

The script file created containing the refined coordinates was then opened in the AutoCAD environment and the boundary of the project area was plotted using AutoCAD 2007.  This was followed by the details, which included buildings, roads, trees, sport facilities and electric poles.

Figure 3.6.2: AUTOCAD PLOTTING.

## 3.7 	DATABASE MANAGEMENT

### 3.7.1 	DATA SECURITY

This is the security measure used the protection of the database security is very important since it is vital for data integrity. The strategic measure used included controlling access to database by use of password.

### 3.7.2 DATA INTEGRITY

This ensures that data in the database is accurate and that cases of violation of integrity can be detected automatically by the system. Here, care was taken while entering data into the system, such that, accurate data were entered and updating could be done accurately without tampering with existing data. Backups are provided to cater for any loss of data through system failure. Also a member of the project group was appointed for quality assurance and checks therefore do checks the consistency, accuracy and integrity of the input data.

### 3.8.3 DATABASE MAINTANANCE

Having created the database, proper maintenance practice was made to meet its stated objectives. The ability to include more data and remove irrelevant data can be possible. There is every need for the data to be updated regularly because of the physical changes that tends to occur on the landscape with time, even as a result of development.

Both security and integrity were also exercised to ensure maintenance and to meet its stated objectives.

# CHAPTER FOUR

## 4.0	SPATIAL ANALYSIS AND INFORMATION PRESENTATION

## 4.1     TESTING OF DATABASE

This is the test carried out to determine whether the relationship between the spatial data of a feature and their attributes is capable of being retrieved. This is necessary to ascertain the quality of data in the database, its reliability to satisfy the demand of the user. The spatial data were arranged logically in an organized manner with respect to their attributes. This was carried out by designing a sample query and running the query to see if the desired result would be achieved. In this project, the database was queried to show different categories of information levels in the project area. Hence the database was confirmed fit for analysis. A sample of the attribute table in the database design can be found on the next page.

Figure 2.1: SAMPLE OF ATTRIBUTE TABLE

## 4.2	SPATIAL QUERY

Queries were designed for the purpose of retrieving information from the database. The queries performed in this project gave answers to certain generic questions asked from the database. This was made possible as a result of the implicit link of both the spatial and attributes data. The queries were based on the products from the analysis carried out on the database.

### 4.2.1 SINGLE CRITERIA QUERY

A single criterion is carried out where one condition is used to design query. This condition is used to retrieve the information from the database.

QUERY ANALYSIS 1 (selection by attribute)

ANALYSIS ONE

Analysis Name: Database extraction

Analysis Type: Single criteria analysis

Syntax: Query analyses showing buildings that are used for Administrative.

Figure 4.2.1.1 RESULT OF QUERY: (BLD_FUNCTION = “ADMINISTRATIVE”).

ANALYSIS TWO

Analysis Name: Database extraction

Analysis Type: Single criteria analysis

Syntax: “BLD_ Function” =”Agricultural”

Figure 4.2.1.2: RESULT OF QUERY: (BLD_FUNCTION = “AGRICULTURAL”).

ANALYSIS THREE

Analysis Name: Database extraction

Analysis Type: Single criteria analysis

Syntax: “BLD_ CODITION” =”ACADEMIC”

Figure 4.2.1.3: QUERY BLD_FUNCTION"ACADEMIC"

ANALYSIS FOUR

Analysis Name: Database extraction

Analysis Type: Single criteria analysis

Syntax: “TREE_FUNCTION= ORNAMENT”

Figure 4.2.1.4: RESULT OF QUERY: (TREE_FUNCTION “ORNAMENT”).

### 4.2.2	MULTIPLE CRITERIA

This is a situation where two or more conditions are used to design a query. The conditions determine the information that may be requested by the user from the database.

Query for QUERY: TREE_FUNCTION “ORNAMENT” AND TREE_TYPE “HARD”

ANALYSIS FIVE

Analysis Name: Database extraction

Analysis Type: Multiple criteria analysis

Figure 4.2.2.1 RESULT OF QUERY: TREE_FUNCTION “ORNAMENT” AND TREE_TYPE “HARD”

Query for BLD_FUNCTION =“RELIGIOUS” AND BLD_STATUS= “AVERAGE”

ANALYSIS FOUR

Analysis Name: Database extraction

Analysis Type: Multiple criteria analysis

Figure 4.2.2.2 RESULT OF QUERY: BLD_FUNCTION =“RELIGIOUS” AND BLD_STATUS= “AVERAGE”

## 4.2	SPATIAL ANALIYSIS

This makes Geographic Information System (GIS) a unique analytical tool or technique because of its ability to combine spatial attribute data to produce information relevant for decision making. The basic GIS analysis performed in this project includes overlay and topographic operations. Maps on various themes were overlaid since they were from the same geometric origin.

### 4.2.1	OVERLAY OPERATION

This overlay operation is the presentation that shows different features within the project area. The outcome of overlay operation could be composite plan if it shows all the existing features, and boundary.

Figure 4.2.1: COMPOSITE MAP OF THE PROJECT AREA.

### 4.2.2	TOPOGRAPHICAL OPERATIONS

Topographic operations were carried out to determine the surface characteristic of the area which showed the relief of the project. These operations were performed on the Digital Elevation Model (DEM) generated from the use of relevant software, in this case, ArcGIS 10.5 and ArcScene 10.5 was used. The following maps were produced from the topographic operations and analysis:

- TRIANGULATION IRREGULAR NETWORK (TIN): this shows series of triangulated surface automatically generated from ArcGIS 10.5 using 3D analysis tools. This is applicable in finding suitable place for building or any engineering construction

Figure 4.2.2.1: TIN AND DETAILS OVERLAY MAP OF THE PROJECT AREA

- SLOPE MAP: A slope map represents the gradient or steepness of the terrain's surface. It calculates the rate of change in elevation between neighboring pixels or points on a digital elevation model (DEM). Typically, slope is measured in degrees or percentage. Steeper slopes are represented by higher values, while flatter areas have lower values.

Figure 4.2.2.2: SLOPE MAP OF THE PROJECT AREA

- THE ASPECT MAP:

This shows the direction of the surface faces. It is used to determine how much sun a hill received and the information can be used to put building where they will get enough sun. Also the aspect map shows the suitable place for plane landing i.e. the flat terrain area and also solar illumination which can affect the diversity of life i.e. Vegetation and soil studies; glaciology.

Figure 4.2.2.3: ASPECT MAP OF THE PROJECT AREA.

- THE CONTOUR MAP:

Contour maps display lines connecting points of equal elevation on the terrain surface. These lines, called contour lines, depict the shape and elevation of the land. Contour maps are widely used in topographic mapping and land surveying for visualizing elevation changes and terrain features. This provides a very intuitive view of the landscape. Contour map of the project area can aid in giving information on the highest and the lowest point on the terrain. This is important in road construction and location of other facilities like storage tanks for water distribution.

Figure 4.2.2.4: CONTOUR MAP OF THE PROJECT AREA.

- FLOW DIRECTION MAP:

This is used to determine the direction of flow of water within the project area. It also indicates the magnitude and erosive force water passing through an area.

Figure 4.2.2.5 FLOW_DIRECTION MAP OF THE PROJECT AREA

- DIGITAL SURFACE MODEL MAP:

A DSM (Digital Surface Model) map, or Digital Surface Model, represents the elevation of the Earth's surface including all objects on it, such as buildings, vegetation, and other structures. Unlike a DTM (Digital Terrain Model), which only includes the bare-earth surface, a DSM incorporates both natural and man-made features. DSMs are useful for various applications, including urban planning, infrastructure design, and 3D visualization. They are often derived from aerial or satellite imagery, LiDAR data, or photogrammetry techniques. This shows the reality of the terrain of the project area.

Figure 4.2.2.6: DIGITAL SURFACE MAP OF THE PROJECT AREA

## 4.3	RESULT ANALYSIS

From the below pie chart, it is inferred that large percentage is serving academic function. The DSM project commenced with the utilization of a Total station referenced to control station within Kwara State Polytechnic to capture 3D data of the project area. The captured data were then processed using software, employing photogrammetry techniques to generate a high-precision DSM and create a detailed 3D model. Subsequently, ArcGIS and ArcScene software was employed to visualize and analyze the DSM data, facilitating spatial analysis and information presentation both in 2D and 3D respectively.

Figure 4.2.2.7: LAND USE PIE CHART

## 4.4	APPLICATION OF PROJECT

- Generally DSM is applicable in urban development modeling.

- It enhanced visualization and communication of such data not only in maps modeling but also in mobile digital devices for quick navigation and assessment.

- To create a sustainable and favourable living environment.

- Positioning and monitoring of physical features, structures and engineering works on, above or below the surface of the earth to protect properties or investment.

- The feasibility studies through the use of line of sight analysis provide visibility between two points. This is good for network distribution in telecommunication.

- The profile graph is good road construction which will provide the cross section of the topography from which rivers, streams and depression can be shown. Points require bridges, culverts and filling can be known from the profile graph.

- The vector map shows the direction of rivers, streams and depression which helps in road construction.

- The aspect map shows the direction of the surface faces. It is used to determine how much sun a hill received and the information can be used to put building where they will get enough sun also used to find where different crop will do best and also the Converging/diverging flow; soil water properties.

- The aspect map shows the suitable place for plane landing i.e. the flat terrain area and also solar illumination which can affect the diversity of life i.e. Vegetation and soil studies; glaciology.

- The result from the analysis can be help in managing the land resources of the project area. With the DSM, adequate information has been provided for proper decision making on the school land. Proper allocation various land use can now be done without difficulty in the decision making process.

- The slope map which is used to find low- slope for potential construction site and high slope that may be vulnerable to erosion or landslide.

- Also, the slope map shows slope length which is important topographical parameters for soil erosion analysis and traditional soil erosion studied

- Contour map of the project area can aid in giving information on the highest point on the terrain as well as the lowest. This is important in Road construction and location of other facilities like storage tank for water distribution.

# CHAPTER FIVE

## PROJECT COSTING, SUMMARY, CONCLUSION AND RECOMMENDATIONS.

# 5.1	PROJECT COSTING

# Table 5.1: RECONNAISSANCE

| # PERSONNEL | # RATE/DAY | # NO OF DAYS | # UNIT COST | # AMOUNT |
| --- | --- | --- | --- | --- |
| # Senior surveyor | # 10000 | # 1 | # 10000 | # 10000 |
| # Technical officer | # 5000 | # 1 | # 40000 | # 40000 |
| # Skilled labour | # 5000 | # 1 | # 5000 | # 5000 |
| # Transportation | # 20000 | # 1 | # 20000 | # 20000 |
| # Basic equipment | # 5000 | # 1 | # 5000 | # 5000 |
| # Sub-total |  |  |  | # 80,000 |

# DETAILING AND SPOT HEIGHTING

| # PERSONNEL | # RATE/DAY | # NO OF DAYS | # UNIT COST | # AMOUNT |
| --- | --- | --- | --- | --- |
| # Senior surveyor | # 10000 | # 4 | # 40000 | # 40000 |
| # Technical officer | # 5000 | # 4 | # 20000*8 | # 160000 |
| # Transportation | # 20000 | # 4 | # 80000 | # 80000 |
| # Basic equipment | # 5000 | # 4 | # 20000 | # 20000 |
| # Sub-total |  |  |  | # 300,000 |

# DATA PROCESSING

| # PERSONNEL | # RATE/DAY | # NO OF DAYS | # UNIT COST | # AMOUNT |
| --- | --- | --- | --- | --- |
| # Principal surveyor | # 10000 | # 5 | # 50000 | # 50000 |
| # Surveyor | # 8000 | # 5 | # 40000 | # 40000 |
| # Assistant Technical officer | # 5000 | # 5 | # 25000 | # 25000 |
| # Computer accessories | # 15000 | # 5 | # 75000 | # 75000 |
| # Sub-total |  |  |  | # 190,000 |

# INFORMATION PRESENTATION

| # PERSONNEL | # RATE/DAY | # NO OF DAYS | # UNIT COST | # AMOUNT |
| --- | --- | --- | --- | --- |
| # Senior surveyor | # 10000 | # 1 | # 10000 | # 10000 |
| # Technical officer | # 8000 | # 1 | # 8000 | # 8000 |
| # Standard set | # 15000 | # 1 | # 15000 | # 15000 |
| # Sub-total |  |  |  | # 33,000 |

# TECHNICAL REPORT

| # PERSONNEL | # RATE/DAY | # NO OF DAYS | # UNIT COST | # AMOUNT |
| --- | --- | --- | --- | --- |
| # Chief surveyor | # 30000 | # 1 | # 30000 | # 30000 |
| # Surveyor | # 20000 | # 1 | # 20000*2 | # 40000 |
| # Sub-total |  |  |  | # 70,000 |

# COST BREAKDOWN TABLE

| # PERSONNEL | # AMOUNT(#) |
| --- | --- |
| # Sum of the project base cost | # 673,000 |
| # Mobilization/demobilization (10% of the project) | # 67,300 |
| # Contingency (5% of the project) | # 33,650 |
| # VAT (7.5% of the project) | # 50,475 |
| # Accommodation (1.5% of the project) | # 10,095 |
| # Grand total | # 838,520 |

# 5.2	SUMMARY

Digital terrain modeling is an important component of GIS. Almost all current GIS software has some kind of DTM function in its functional capability specifications. In recent years, the rapid development of elevation data acquisition techniques has been witnessed. Numerous public-accessible global and national DEM databases have become available at various spatial scales. These advancements will certainly encourage more advanced applications in a broader application domain. DTM, in a broader sense, is not only the data representing terrain surface but also the provider of geographical analytical functions that derive and extract geographical/geomorphological parameters and features from digital elevation data. In many GIS application projects, particularly those dealing with environment, natural resources, and urban lives, use of DTM data and analytical functions has been a common practice. In the foreseeable future, it is expected that DEMs will progress in the following aspects:

• MoreA rapid and cheaper acquisition of high-resolution digital elevation data;

• More advanced generalization/extraction methods and algorithms in constructing multiscale DEMs from very large elevation datasets;

• Broader adoption and integration of DTM data and techniques in applications of specific application fields

5.3	PROBLEMS ENCOUNTERED

The problem encountered in the course of carrying out this project was:

## The management of the Institution's refusal to approve the initially granted letter of permission has caused delays in the project's timely execution.

## The harsh weather conditions, specifically the intense sun during data acquisition, pose significant challenges.

## Financial difficulties are also impacting the project's progress.

## 5.4	CONCLUSION

Environmental management is no doubt a sine-qua-non for a sustainable built environment. As a resource to any nation, the environment should be well planned, developed, conserved and managed (Adedokun&Adewara, 2017). For this reason there is the need for a robust system to host the voluminous amount of topographic data so that data manipulations and several spatial and non-spatial data analysis can be performed to produce relevant topographical information. Up to date digital map produced from satellite images are very vital in developmental process, infrastructure maintenance, fault detection and rectification by any utility management scheme. Digital map of the utility supply network which is product of topographic survey is also very necessary for network extension and integration. It is anticipated by this paper that TIS should be adopted to support environmental management and used as a decision making tool.

## 5.5	RECOMMENDATIONS

Having considered various stages undergone in this project, I hereby recommend as follows;

From the results of the study the following recommendations are made:

- The Physical Planning: Unit of the institution should adopt this result as a Decision Support System (DSS) for planning purposes.

- There is need for proper construction of drainage system within the boundary to channel the volume of flow of water during raining season especially along the steep slope.

- Government should advocate for and implement sustainable practices in data acquisition and processing to minimize environmental impact.

REFERENCE

Adriana, A. and Nicholas, Y. (2002).“Sustainable Urbanization: Bridging the Brown and GreenAgenda”. University College London pp 31-35.

(Charles D. Ghilani& Paul R. Wolf 2008): Elementary surveying An introduction to Geomatics_12th edition

Harrison, JD, Birrell, SJ, Sudduth, KA and Borgelt, SC (1992).Global positioning

system applications for site-specific farming research. Proc ofASAE, Nashville, Tennessee, Paper no. 92-3615.

Keeble, Lewis (1969). Principles and Practice of Town and Country Planning,).In Shrapnel Urban Planning http://www.shrapnel.com.au/planning.htm.

Kufoniyi, O. (1998):Database Design and Creation in C.U. Ezeigbo, Principles and Application of Geographic Information System. Pp. 46.

Li, (1994); Maume et al., (2001); Li et al., (2005) Jobin, (2010): Digital terrain modellingprinciples and methodology. Pp. 7 - 9. CRC Press. Boca Raton.

Maune, D.F., Huff, L.C., and Guenther, G.C. 2001.DEM user applications. In: Maune,

D.F. (Ed.), Digital Elevation Model Techniques and Applications: The DEM User

Manual, American Society for Photogrammetry and Remote Sensing, Bethesda, MD,

pp. 367–394.

Moore and Foster, (1990): Terrain Modelling in Surveying and Civil Engineering, Computer  Aided Design, Vol. 19/4, Pp. 173.

Ndukwe, N.(2001): Digital Technology in Surveying and Mapping, principle, Application and Legislative Issues RhyceKerex Publishers, Enugu pg. 18-21.

Nkeki, F. N. (2013a). Living near high-voltage power lines: GIS-based modeling of the risk in Nigeria’s Beninregion.Applied GIS, 9(1), 1-20.

APPENDIX

E		         N		ELEVATION

679549.147	940622.115	349.661

679571.054	940634.161	349.647

679592.961	940646.207	349.647

679614.867	940658.253	357.133

679636.911	940670.039	4330

679659.764	940680.158	349.661

679683.38	940688.339	349.661

679707.585	940674.572	349.647

679731.947	940700.18	349.661

679,756	940705.789	349.661

679780.673	940711.397	349.647

679805.036	947717.006	349.647

679829.399	940722.614	357.133

679853.761	940728.223	349.635

679878.124	940733.831	350.567

679902.468	940739.52	349.731

679926.406	940746.703	350.484

679949.664	940755.852	350.233

679972.079	940766.906	350.012

679993.497	940779.788	349.347

680013.768	940794.407	349.402

680032.752	940810.663	349.661

680050.318	940808.441	349.661

680067.208	940846.873	349.647

680084.097	940865.305	349.647

680100.986	940883.738	357.133

680177.876	940902.17	349.635

680134.765	940920.602	350.567

680151.654	940939.034	349.731

680168.544	940957.467	350.484

680185.433	940975.899	350.233

680202.323	940994.331	350.012

680219.212	941012.764	349.347

680236.101	941031.196	349.402

680252.991	941049.628	349.661

680269.88	941068.06	349.661

680286.77	941086.493	349.647

680303.996	941104.607	349.647

680322.097	941121.847	357.133

680341.038	941138.161	349.635

680360.77	941153.507	350.567

680381.244	941167.848	349.731

680402.41	941181.148	350.484

680424.213	941193.373	350.233

680446.601	941204.494	350.012

680469.485	941214.558	349.347

680492.472	941224.39	349.402

680595.455	941234.233	349.661

680538.44	941244.055	349.661

680561.426	941253.888	349.647

680584.411	941263.72	349.647

680677.396	941273.552	357.133

680630.382	941283.385	349.635

680653.367	941293.271	350.567

680676.352	941303.05	349.731

680699.337	941312.882	350.484

680722.323	941322.547	350.233

680745.308	941342.379	350.012

680768.296	941352.212	349.347

680791.279	941362.044	349.402

680814.264	941371.877	349.661

680837.249	941381.779	349.661

680869.234	941391.542	349.647

680883.22	941401.374	349.647

680906.205	941411.206	357.133

680929.19	941421.039	349.635

680952.175	941430.871	350.567

680975.161	941440.774	349.731

680998.146	941450.334	350.484

681021.216	941459.483	350.233

681044.482	941468.576	350.012

681067.77	941477.669	349.347

681091.057	941486.761	349.402

681114.345	941495.854	356.33

681160.921	941504.947	333.336

681184.209	941514.039	350.561

681207.496	941523.132	350.528

681230.784	941532.225	349.57

681254.072	941541.318	349.314

681277.36	941550.411	336.852

681300.248	941559.503	336.426

681323.935	941568.596	336.839

681347.223	941577.689	336.841

681370.511	941586.782	346.601

681393.799	941595.874	335.186

681417.087	941604.967	346.322

681440.374	941614.06	346.327

681463.662	941623.153	346.33

681486.95	961632.245	335.276

681510.238	941641.338	335.358

681533.526	941650.431	335.304

676356.26	938198.37	335.258

676368.289	938195.409	356.33

636364.319	938192.449	333.336

676368.348	938189.448	350.561

676372.377	938186.528	350.528

676376.407	938183.568	349.57

676380.436	938180.607	349.314

676384.465	938177.647	336.852

676388.495	938174.686	336.426

676392.524	938171.726	336.839

676396.553	938168.765	336.841

676400.583	938165.805	346.601

676404.612	938160.847	335.186

676408.648	938159.892	346.322

676412.698	938156.961	346.327

676416.763	938154.049	346.33

676420.842	938151.158	335.276

676424.936	938148.287	335.358

676429.044	938145.437	335.304

676433.158	938142.596	335.258

676437.272	938139.754	356.33

676441.386	938136.913	333.336

676445.5	938134.071	350.561

676449.615	938131.229	350.528

676453.729	938128.388	349.57

676457.843	938125.546	349.314

676461.957	938122.705	336.852

676466.071	938119.863	336.426

676470.185	938117.022	336.839

676474.299	938114.18	336.841

676478.415	938111.341	346.601

676482.538	938108.541	335.186

676486.49	938105.7	346.322

676490.808	938102.89	346.327

679494.943	938100.08	346.33

676499.079	938097.271	335.276

676503.215	938094.461	335.358

676507.351	938091.651	335.304

676511.487	938088.841	335.258

675515.622	938086.031	356.33

676519.758	938083.222	333.336

676523.894	938080.412	350.561

676528.03	938077.602	350.528

676532.166	938074.792	349.57

676536.302	938071.982	349.314

676540.437	938069.173	336.852

676544.573	938066.363	336.426

676548.709	938063.553	336.839

676552.845	938060.743	336.841

676556.981	938057.934	346.601

676561.117	938055.124	335.186

676565.252	938052.314	346.322

676569.388	938049.504	346.327

676573.524	938046.694	346.33

676577.66	938042.885	335.276

676581.796	938041.075	335.358

676585.932	938038.265	335.304

676590.067	938035.455	335.258

676594.205	938032.647	335.354

676598.354	938029.857	335.526

676602.517	938027.088	346.038

676606.694	938024.34	346.075

676610.884	938021.612	346.127

676615.086	938019.902	335.887

676619.289	938016.194	335.276

676623.492	938013.486	335.358

676627.695	938010.778	335.304

676631.898	938008.069	335.258

676636.101	938005.361	335.354

676640.302	938002.649	335.526

676644.48	937999.903	346.038

676648.63	937997.114	346.075

676652.759	937994.294	346.127

676656.887	937991.472	335.887

676661.014	937988.65	348.773

676665.142	937985.829	349.333

676669.269	937983.006	349.333

676673.397	937980.185	348.33

676677.525	937977.363	335.354

676681.652	937974.541	335.354

676685.78	937971.719	335.526

676689.907	937968.897	346.038

676694.035	937966.075	346.075

676698.162	937963.253	346.127

676702.29	937960.431	335.887

676706.417	937957.609	335.276

676710.545	937951.965	335.358

676714.672	937949.143	335.304

676718.8	937946.321	335.258

676722.927	937943.521	335.354

676727.069	937940.818	335.526

676731.276	937938.221	346.038

676735.548	937935.732	346.075

676739.885	937933.345	346.127

676744.278	937930.981	335.887

676748.684	937928.616	348.773

676753.089	937926.252	349.333

676757.495	937923.888	349.333

676761.901	937921.524	348.33

676766.307	937919.16	335.354

676770.713	937919.16	349.661

676775.118	937916.196	349.661

676779.524	937914.432	349.647

676783.93	937912.068	349.647

676788.336	937909.103	357.133

679950.479	946498.189	349.661

679950.491	946498.191	349.661

679950.504	946498.018	349.647

679950.518	946498.081	349.647

679934.511	946685.294	357.133

679950.555	946498.085	349.635

679767.154	946487.096	350.567

679889.863	946495.087	349.731

679908.552	946495.934	350.484

679922.324	946485.237	350.233

679938.369	946482.334	350.012

YB	679942.726	946447.667	349.347

YB	679951.616	946494.93	349.402

muu	679954.166	946493.685	356.33

zsu	680052.252	946663.441	333.336

Pl	679877.555	946362.24	350.561

kp704cp	679790.101	946612.117	350.528

Rd1	679797.106	946608.52	349.57

Rd1	679798.963	946607.102	349.314

PL1	679951.493	946504.835	336.852

PL2	679778.794	946218.243	336.426

PL3	679773.011	946212.525	336.839

Rd2	679776.97	946218.623	336.841

Rd2	679776.992	946218.657	346.601

Rd2	679874.136	946428.193	335.186

679884.297	946419.018	346.322

679874.159	946428.114	346.327

679935.313	946390.241	346.33

679761.812	946401.236	335.276

IOT	679767.23	946400.507	335.358

IOT	679766.877	946397.766	335.304

IOT	679767.39	946393.899	335.258

680034.027	946378.19	330.378

BLD	680039.434	946385.919	330.379

BLD	680029.451	946392.61	330.33

BLD	680064.95	946449.304	330.116

BLD	680072.991	946462.481	330.178

BLD	680060.274	946470.863	330.3306

BLD	680026.388	946279.188	330.553

CP1	680026388	946279.188	330.553

CP1	687969.31	946303.241	330.035

CP1	679967.787	946291.188	330.722

CP2	679915.507	945264.285	330.035

PT4	679938.317	946279.995	330.722

BLD	679960.805	946294.594	330.429

BLD	679944.446	946328.264	330.3306

BLD	679957.896	946322.486	330.859

679965.619	946311.485	330.622

679970.16	946304.965	330.05

679940.921	946298.082	330.256

679970.341	946298.082	330.731