PROJECT REPORT ON

PERIMETER AND DETAILING SURVEY

OF

PART OF INSTITUTE OF FINANCE AND MANAGEMENT STUDIES (IFMS), KWARA STATE POLYTECHNIC, ILORIN,

By

OLANREWAJU MISTURAH DAMILOLA

ND/23/SG/FT/0073

SUBMITTED TO THE DEPARTMENT OF SURVEYING AND GEOINFORMATICS, INSTITUTE OF ENVIRONMENTAL STUDIES KWARA SATE POLYTECHNIC, ILORIN

IMPARTIAL FULFILMENT FOR THE AWARD OF ORDINARY NATIONAL DIPLOMA (OND) IN SURVEYING AND GEOINFORMATICS

JUNE, 2025

CERTIFICATION

I hereby certify that all field work and information contained in this project report were obtained as a result of the observation and measurement carried by me in the field  and that the survey was executed in accordance with the survey laws and departmental instructions

_____________________________			___________________

OLANREWAJU MISTURAH DAMILOLA		DATE

ND/23/SG/FT/0073

CERTIFICATION

This is to certify that OLANREWAJU MISTURAH DAMILOLA with Matric No:-ND/23/SG/FT/0073 has satisfactorily carried out her project under my instructions and direct supervision. I hereby declare that she has conducted herself with diligence, honesty and sobriety on the project.

SURV R.S. AWOLEYE      	      	                          ……………………….

PROJECT SUPERVISOR                                            DATE AND SIGN

SURV. WILLIAMS KAZEEM.A                            ………………………

PROJECT SUPERVISOR                                             DATE AND SIGN

SURV R.S. AWOLEYE                  		   ….……………………

PROJECT CO-ORDINATOR                                   DATE AND SIGN

MR. ABIMBOLA ISAU                             		  ….……………………

HEAD OF DEPARTMENT                                        DATE AND SIGN

EXTERNAL EXAMINER                                       ………………………..

DATE AND SIGN

DEDICATION

This project is dedicated to Almighty GOD who saw me through the programme.

ACKNOWLEDGEMENT

My profound gratitude goes to the Almighty GOD for his unfeigned admiration towards me, for the grace, guidance and opportunity given me to move forward against all odds. Also for his support that led to successful completion of this project.

Also, I express my sincere appreciation to my supervisors, SURV. R.S. AWOLEYE and SURV. WILLIAMS KAZEEM for their assistance and precise guidance given during the course of this project despite of their tight schedule.

My special ovation goes to my dearest, energetic, hardworking, caring and loving parent MR & MRS OLANREWAJU and my brothers and sister for their unflinching support, moral advice and their unrelenting effort toward my education.

My immense appreciation goes to head of department in person of MR ISAU ABIMBOLA and departmental staff for the support and advice during the project period.	I can’t forget all my colleagues for their cooperation, believe and understanding towards the successful completion of this project. I wish all of us success.

ABSTRACT

This project report focused on various methods used in exclusion of perimeter and detail survey part of Institute of Finance and Management Studies, Kwara state polytechnic Ilorin, locate at Ilorin, Moro Local Government Area Kwara State. The project was carried out using the basic survey operation include reconnaissance which involves filed and office reconnaissance survey, followed by data acquisition which involves third order traversing, total station for detailing. All the data acquired from the field were deduced, computed and adjusted according to specification and result were analysed and found to be within the expected accuracy. Finally computed data were presented in graphical form in digital using AUTOCAD software and a comprehensive report on how the whole operation was carried out was finally written.

TABLE OF CONTENTS

Title page                                                                                                       	 i

Certification                                                                                                   	ii

Dedication                                                                           		            iii

Acknowledgement                                                                               	iv

Abstract                                                                               		              v

Table of content                                                                  		              vi

CHAPTER ONE

INTRODUCTION

  - Background of the study                                                	                      1

  - Statement of the problem                                               	                      4

  - Aims of the project                                                        	                      4

  - Objective of the project                                                  	                      5

  - Scope of the project                                                        	                      5

  - Specification of the project                                             	                      6

  - Study Area                                                                      	                      6

  - Personnel                                                                        	                      7

CHAPTER TWO                                                                    	                      8

LITERATURE REVIEW                                                           	                      8

CHAPTER THREE                                                           	                                14

3.0 Methodology                                                                 		                    14

3.1 Reconnaissance.                                                            		                    14

3.2 Instrument Test                                                             	                                16

3.3 Monumentation                                                                                        	21

3.4 Perimeter Traversing                                                      	                                22

3.5 Detail survey                                                                  	        24

CHAPTER FOUR

4.0 Data processing                                                              	                                26

4.1 Result Analysis                                                               	                                26

4.2 Back computation                                                                                    	27

4.3 Compute for the Area using Double latitude and departure                            28

4.4 Graphic plotting                                                           		                    30

CHAPTER FIVE

5.1 Summary                                                                             	                    31

5.2 Recommendation                                                                 	                    31

5.3 Problem Encountered                                                           	                    32

5.4 Conclusion                                                                           	                    33

References								        34

APPENDIX								        35

CHAPTER ONE

1.0 INTRODUCTION

1.1 BACKGROUND OF THE STUDY

The great pyramids which are mystery hitherto could not have been built without the knowledge of surveying principal in spite of the conflicting views about its physic construction. The Egyptians used stones to mark boundaries of their farmland along Nile valley as temporary beacons even though the beacons used to be washed away later. This led to proper and solid demarcation of boundaries by permanent beacons. Detailing is defined as the process of fixing topographical features to the survey line, any of these under listed methods could be used,

  - Chain survey method (tie line and offset)

  - Plane table survey method

A detail survey is used to determine and locate the features and improvement on a parcel of lane. The word ‘’FEATUES’’ here means both natural and man-made structures on a pieces of land such as vegetation, types of soil, buildings, and utilities fences and boundaries roads, and marks and so on. Professional land surveyors use different tools & techniques in Perimeter and detail Survey that include both conventional and modern technologies. However, most of the surveyors prefer to use the conventional surveying tools & techniques that include bearing and distance or compass and tape. Although they are considered primitive tools, they still remain an important asset to a land surveyor. Although there are some minor adjustments and improvements in the compass and tape, they still follow the same age-old concept and technique. In addition to them, surveyors will use GPS technology in a Perimeter and detail Survey which is one of the greatest advancements in the surveying industry. Both of these two techniques, if carried out by a professional, can result in a detailed analysis of your property with a small margin of error. Typically, a surveyor would prefer to use the bearing and distance technique when working over a smaller distance and use GPS technology on larger distances.

Surveying generally is considered to be the bedrock of any environment development. It plays a very important role in every aspect of human because various activities of other professional are based usually on the laid down foundation by the surveyor. In all aspect of environmental development, surveying is usually the first to provide the basic information for better decision making and probably the last for integrity checking of the executed environment project. Encarta encyclopaedia 2009) also defined surveying as mathematical science used to determine and delineate the firms, extent and position of the feature on or beneath the surface of the earth for control purposes. That is provision of checks for construction boundaries. Surveying is the technique, profession and science of accurately determining the terrestrial or three dimension positions of points and the distance and angles between them. These points are usually on the surface of the earth and they are often used to establish land maps and boundaries for ownership or governmental purpose (oxford advance dictionary).

Surveying consist of many different operation and techniques but underlying them all are some basic principles which provide unity and discipline to subject. These principles are few and they provide the economic and effective basis for conducting surveys. In this particular project, perimeter and detail survey is the main aspect of survey that would be considered. In new of this perimeter survey is a specific type of survey that measures the distance along the boundary lines of a given land. A perimeter survey is important to find out the exact location of the landed property and probably determine the extent of such land also the extent of encroachments can be evaluated in case of the landed property dispute.

PERIMETER AND DETAIL SURVEYS: is a survey that requires traversing of the detail on the limit of the heights, depicting all detail on the limit of the landed property, which consist of both natural and artificial features. It also refers to as cadastral survey because it contain coordinate all corners point of the boundaries and determination of relative position of point of both natural and artificial features on the earth’s surface and addicting them by means of conventional symbol on the plan.

A detail survey is used to determine and locate the features and improvement on a parcel of lane. The word ‘’FEATUES’’ here means both natural and man-made structures on a pieces of land such as vegetation, types of soil, buildings, and utilities fences and boundaries roads, and marks and so on. They are generally out using survey equipment such as total station and theodolites. The data is then carried to the office for analysis and preparations of detail plan are. These plans are usually useful for engineers and architects who use them in their design and plain. The survey should be carried out by a qualified land surveyor who may be assisted by a chairman. Perimeter and detail survey is a type of survey that involve delineation of boundary points on the boundary lines of a property, measurement and location of features in their relative position, so as to come out with a survey plan showing the detail and boundary points properly demarcated.

With this kind of survey one can obtain a property which would consist of the inventory of all the property lines together with a list of the reputed owners and the type and use of building thereon. These data will be plotted on maps as accurately as possible considering the best sources of information. This process will require the installation of monuments to mark the boundary corners of the land for future references.  This requirement can be waived through a written agreement between the surveyor and homeowner. Perimeter and detail Surveying is a service that can be performed by a professional land surveying firm, such as Godfrey-Hoffman & Hodge in Connecticut, to ensure reliable, accurate, and detailed surveying results. Perimeter and detail Surveys are used to identify the location of old boundary monumentation and survey pins, or they can be used to set up new ones. A Perimeter and detail Survey does not identify the features and improvements that exist within a property, such as barns, garages, sheds, dwellings, surface utilities, roadways, pools and visible bodies of water; only features that fall within the 15 foot width around the boundary perimeter will be depicted.

1.2 STATEMENT OF THE PROBLEM

Due to the fact that the perimeter surveying operation took place in part of Institute of Finance and Management studies, Kwara State Polytechnic, when measuring the horizontal distance by steel tape. Cars are parked between the station and the detail being measure. Also these cars also serve as obstruction foe the instrument when sighting to this detail.

1.2 AIM AND OBJECTIVE

1.2.1 AIM

The aim of this project is to produce a perimeter/detail plan of part of Institute of Finance and Management Studies, Kwara State Polytechnic, Ilorin, Moro Local Government Area, Kwara State.

1.2.2. OBJECTIVE

`The following objectives are considered in order to accomplish the above aim:

- Reconnaissance operation i.e office and field.

- Data acquisition

- Data processing and computations

- Data presentation i.e plan production.

- Report writing

1.4 PROJECT SPECIFICATIONS

The following are the specification to be ascertained in the project.

- A third order total station traverse must be carried out.

- The control checks must be thoroughly done.

- The length of each traverse line must not be greater than 250metres.

- The liners accuracy should not be less than 1:5000

- Angular misclosure should not exceed ±30;; √n where n is the number of station traversed.

- the carved out area must cover the entire land of the study area

- The horizontal distance should be given in 3 decimal places.

- The consecutive station must be inter visible and should be established where it would not be disturbed.

1.5. SCOPE OF THE PROJECT

The scope of this project covers

- Perimeter traversing

- Detailing

- Data downloading, editing and processing

- Computation

- Area computation

- Information presentation

- Report writing

1.6 PERSONNEL INVOLVED

For easy and proper execution of this project (6) six members were engages in the operations. The roles were interchanged among the members in the group for the benefit for everyone to participate all the operations. The below table shows the name and matric number of the group members.

| S/N | NAME | MATRIC NO | ROLE PLAYED |
| --- | --- | --- | --- |
| 1 | Olanrewaju Misturah Damilola | ND/23/SGI/FT/0073 | AUTHOR |
| 2 | Ayeni Hanifat Morenikeji | ND/23/SGI/FT/0078 | Member |
| 3 | Omoniyi Adenike Baraka | ND/23/SGI/FT/0074 | Member |
| 4 | Aremu Iyanuoluwa Funmilayo | ND/23/SGI/FT/0075 | Member |
| 5 | Gabriel Ayomide Deborah | ND/23/SGI/FT/0077 | Member |
| 6 | Olanrewaju Abdulsamod | ND/23/SGI/FT/0081 | Member |

1.7	STUDY AREA

The area to be covered by this project is part of Institute of Finance and Management Studies, Kwara state Polytechnic, Ilorin. Moro local government, which falls on the longitude and latitude of 0080 33’ 20” N Latitude. 0040 38’ 14.584”.

MAP OF NIGERIA

MAP OF KWARA STATE

MAP  OF  MORO L.G.A

Google earth imagery of project site

CHAPTER TWO

2.0 LITERATURE REVIEW

Perimeter / Boundary Surveys are carried out for the purpose of delineating the boundary of a parcel of land, determining its area and preparation of survey plan. The survey plan is usually the end product of a boundary survey. The survey plan shows ownership and describes the land. It is a document required by law during processing of any land title. We will help you prepare your survey plan and lodge the record (red) copy in the Office of the Surveyor General of the state. It is very important to request for a Perimeter and detail Survey before acquisition of land in Lagos. The word ‘features’ here means both natural and man-made structures on a piece of land – such as vegetation, types of soil, buildings, land utilities, fences and boundaries, roads, land marks and so on. This kind of survey is usually confined to the boundaries of the parcel of land. The survey will often include data such as the elevation of the land, that is, how high the land is above an arbitrary datum (level). A commonly used arbitrary level is the Mean Sea Level which is taken as zero metres high. The Easting and Northing coordinates of the land (exact position in relation to the earth’s surface) may have to be taken too. They are generally carried out using survey equipment such as total stations and theodolites. The data is then carried to the office for analysis and preparation of detail maps known as Digital Terrain Models, which provide the details that have been collected in the form of a map. These maps are useful for engineers and architects who use them in their designs and plans. The survey should be carried out by a qualified land surveyor who may be assisted by a chainman.

Surveying consist of many different operation and techniques but underlying them all are some basic principles which provide unity and discipline to subject. These principles are few and they provide the economic and effective basis for conducting surveys. Perimeter surveying may be described as the type of surveys made purposely for producing plans showing property boundaries or plan on which area necessary for the assessment of property or land taxes may be computed, it is also referred to as legal surveying because such survey forms the basis of a statutory registration of ownership and other rights land (dashe, 1987. Surveying began in Babylon and Egypt in the form of field measurements. The great pyramids which are mystery hitherto could not have been built without the knowledge of surveying principal in spite of the conflicting views about its physic construction. The Egyptians used stones to mark boundaries of their farmland along Nile valley as temporary beacons even though the beacons used to be washed away later. This led to proper and solid demarcation of boundaries by permanent beacons.

Perimeter surveying may be described as the type of surveys made purposely for producing plans showing property boundaries or plan on which area necessary for the assessment of property or land taxes may be computed, it is also referred to as legal surveying because such survey forms the basis of a statutory registration of ownership and other rights land (dashe, 1987).

Perimeter can be define as the boarder or outer boundary of a two dimensional figure. I can also refer to a strip or boundary usually protecting a military position.  A perimeter is also a path that surround a two dimensional shape. The word comes from a geek word “peri” mean around and “meter” mean measure. The term may be used either for the path or is length. Perimeter survey is a type of survey that measured the distance along the boundary line of an area of land. A perimeter survey is important to find out the exact location of your property line and you can also discover if there is any encroachment on your property. Although it can serve like a simple survey where you just need to measured the length of your boundary.

Perimeter survey in reality includes mean other assumptions and calculation in the use of this approach, they have to analyse, calculate and measured a wide range of factors which includes but not limits to:

  - Boundary monumentation

  - Boundary encroachments

  - Unresolved conflicts with exiting property deeds and maps

  - Lines occupation, fences, walls, hedges, yards, and natural elements.

Boundary of perimeter survey defines the perimeter or whole property (rather than say a subdivision which might subdivide out one or two house plots from a 50 to 10 acre property. This clearly involves surveying the entire perimeter of the property to determine the exact geometry and acreage where the legal access to the property is from any easements that might be registered over the property benefiting an adjoining property.

Some techniques and tools are used in perimeter survey that includes both conventional and modern technologies. However, most of the surveyors prefer to use conventional surveying tools and techniques that include bearing and distance or compass and tape. Although they are considered primitive tools, they still remain an important asset to a land surveyor.

Although there are some minor adjustments and improvements in the compass and tape, they still follow the same age-old concept and technique. In addition to them, surveyors will use GPS technology in a perimeter survey which is one of the greatest advancements in the surveying industry.

Both of these two techniques, if carried out by professional, can result to detail analysis of your property with a small margin or error. Typically one will prefer to use bearing and distance technique when working over a small distance and use GPS technology on larger distances.

A detail survey is about defining the location and height of variety of feature on the property in question, A contour plan along with the physical feature located, are show on a plan. This plan can be viewed electronically or on a large format plot. This type of survey is regularly used when designing for roads, buildings, extensions and infrastructures.

  - Traversing

  - Detailing

Traversing is the process of connecting the series of line with known bearing and distance. Subdividing into first order of precise measurement and second order control for the establishment of secondary controls while the third order for survey of tertiary controls and topographical features.

Detailing is defined as the process of fixing topographical features to the survey line, Any of these under listed methods could be used,

  - Chain survey method (tie line and offset)

  - Plane table survey method

Perimeter and details survey involves the following operation:

- Reconnaissance both in the office and field

- Total station for detail fixing/contouring

- Computation

- Plan production

Encyclopaedia Britannica (1973-1974) commented that bench mark or marked points on the surface of the earth, connected to the elevation of bench marks are given in terms of their height above a selected level surface called datum which is limited to horizontal and vertical control only because it is the only two types. Noting that these controls used are in a particular order depending on the specification given for their establishment ranging from first order and second order and third order respectively. The network of first and second order triangulations and traversing can be used to establish more controls when the need arises to provide enough or sufficient controls.

Dashe (1987) saw a perimeter and detail survey produced to provide the following:-

- To determine the extent of an individual holding to avoid conflict over the land.

- To provide vital information, which must be preserved for future management, maintenance, modification and monitoring of both natural and artificial features.

- To locate portions of land on the physical surface of earth together with detail on it by means of survey beacons and showing that survey on a plan.

- To determine the extent, size, value, ownership and transfer of land.

- To define the boundaries of a particular area of land showing them on a plan for further development of the area.

Perimeter and detail survey are applicable in the following ways:-

- The survey plan can be used to supply information for the assessing and developing of construction works in the survey area.

- They can be used for updating existing plan of an area.

- Perimeter and detail survey plan aids cadastral surveyors in relocating existing property by relating them to existing details on the ground.

Also, a perimeter and detail survey is useful in the field of agriculture for the study of soil types and aids in soil conservation.

With today’s technology is possible to generate personal map and plan on a own computer. Digital technique will continue to influence map making, enable more rapid production of accurate and current map. Indeed, perimeter and detail survey is greatly hampered by non-availability of good horizontal controls. In view of this, perimeter and detail survey can be improved upon by establishing basic horizontal control network by first order geodetic triangulation, precise traverse or Global Positioning System (GPS).

CHAPTER THREE

3.0	METHODOLOGY

This chapter basically describes the methods and principles adopted in carrying out the project work. The execution of this project was based on the following basic principles of survey.

- The principle of working from whole to part.

- The principle of choosing the method of survey that is most appropriate to get the desired result.

- The principle of provision of adequate checks to meet the required accuracy.

The processes adopted include:

- Reconnaissance

- Test of instrument

- Control check

- Traversing (perimeter)

- Linear measurement & detailing by radiation method)

For this project, the ground survey method was adopted with the use of total station and its accessories for data acquisition in order to identify the physical features and to determine The XY coordinates (i.e Northing, Easting).

- RECONNAISSANCE

Reconnaissance which is the first stage and vital aspect of any survey work Carried out is as well the preliminary stage of this project.

This also is the initial operation or preliminary investigation undertaken by the surveyor in order to have a thorough overview of the site before the commencement of the actual survey ,it can also be abbreviated as “recci’’ as the project was concerned the reconnaissance was carried out in two ways.

- Field reconnaissance

- Office reconnaissance

3.1.0	Field Reconnaissance

This involves the actual [physical] visit to the site and was carried out before the actual operation. This project site was visited by the group in order to have a prior knowledge as well as true picture of the site and to ascertain the information collected during the office planning. The boundaries were marked with wooden peg driven into the ground to avoid disturbance or removal, taking into consideration the following factors.

- Inter-visibility of the selected traverse station.

- Safety of the selected station for future reference.

- Accessibility of the stations.

    - Office Reconnaissance

This involves knowing the type of instrument, purpose and accuracy required for the survey to be carried out. Information related to the given project was collected from various sources such as project supervisor. The specification/ instructions and coordinates of the control stations were collected from the department of surveying and geo-informatics Kwara State Polytechnic.

3.2	INSTRUMENT TEST

HORIZONTAL COLLIMATION TEST

The aim of this test was to be sure that the line of sight is perpendicular to the trunion axis.

Procedure:

The Total Station instrument was set over a point and all necessary temporary adjustments (centering, leveling and focusing) performed. Then the configuration menu of the total station was accessed by pressing down the menu key for about two seconds and the calibration sub-menu and consequently the horizontal collimation test was chosen. This test was done by sighting and bisecting a well-defined vertical target about 100m away and taking the horizontal readings on face left and face right. From the analysis of the results, the total station was in good adjustment.

VERTICAL INDEX ERROR TEST

This adjustment ensures that the vertical circle reading is exactly 90° when the line of sight is horizontal. Any deviation from this figure is termed vertical index error.

Procedure:

The instrument was set over a point and necessary temporary adjustments (centering, leveling and focusing) performed. The vertical index error test was carried out by sighting a target at a distance of about 120m on face left. The vertical circle reading was recorded and on face right the target was sighted and bisected again and the vertical circle reading recorded.

3.2.1	IN-SITU CHECK FOR CONTROL

In-situ checks observations (angular and linear) were executed for the purpose of verifying the integrity of the existing controls. The following observational schedules were executed:

The instrument was set on KWT3001 and angular observations were made to targets on KWT3002 as back station and KWT3003 as forward station.

FC3184S

The results of the observations as shown below confirm that the controls were still in their original positions and therefore suitable for use.

TABLE 2	IN-SITU CHECK DATA ANALYSIS (control pillars).

| STATION | COORDINATE (m) | KNOWN VALUES (m) | MEASURED VALUES (m) | DIFFERENCE (m) |
| --- | --- | --- | --- | --- |
| KWT3001 | NORTHING EASTING | 946478.324 678540.211 | 946478.322 678540.208 | 0.002 0.003 |
| KWT3002 | NORTHING EASTING | 946448.336 678500.342 | 946448.330 678500.346 | +0.006 -0.004 |
| KWT3003 | NORTHING EASTING | 946321.452 678612.394 | 946321.457 678612.397 | -0.005 -0.003 |

.

KWT3002

.

.111

KWT3003

KWT3001

FIGURE 3.2.1 Diagram Showing Control Used

3.2.2	DATA ACQUISITION

This involves the processes in acquiring the data needed for the project. This involves the actual making of measurements and recording of observed data on the field. There are different methods of acquiring data in the site with different instrument such as Total station, Theodolite, Compass, Level Instrument etc.

3.2.3 Geometric Data Acquisition.

These are positional data, that is, they are data having the [X, Y] coordinates which is possible to locate their position on the surface of the earth.

3.2.4 Attribute Data Acquisition.

These data are acquired by social survey, these are data used for defining the purpose of features located on the earth surface.

3.2.5 EQUIPMENT USED/SYSTEM SELECTION AND SOFTWARE

This comprises of two components, namely: the hardware components and software components.

HARDWARE COMPONENT: These are the physical equipment used for the execution of the project and they are:

- Total station and its accessories

- Steel tape

- Nails and bottle corks

- Field book and pen

- Personal computer

- pegs

SOFTWARE USED FOR DATA PROCESSING

- AutoCAD 2017 for plotting the boundary and detailing

- NotePad, and Microsoft Excel (for Script preparation, editing and restructuring of data and report writing).

3.2.6    SETTING OUT OF PERIMETER BOUNDARY

Based on the office planning and field reconnaissance conducted, the instrument was first set on Control Pillar KWT3001 being the closest control and all temporary adjustment performed. The coordinate of the control point KWT3001 was key into the instrument.KWT3002 was sighted as back sight.

The coordinates of the station KWT3002 was key-in the instrument via the keyboard. The instrument then computed the bearing between the two stations for orientation. Then coordinates of points to be set out were entered into the total station and the setting out program of the instrument was used to get the angle to turn in order to face the direction of the point after orientation, the instrument was rotated until horizontal angle read 0 00’ 01”, reflector was held along the direction and distance between the instrument and the reflector was measured.  The instrument displayed the remaining of the point to be fixed distance as either positive or negative. Positive distance means that the reflector should move away from the instrument by that amount while negative distance means that the reflector should move towards the

instrument. When the horizontal angle read 000’ 01” and measured distance displayed 0.000m this marked the exact position to be set out.

3.3     MONUMENTATION

The beacons measuring 18cm by 18cm by 75 cm were molded in-situ with a mixture of 1:2:3 of cement, sand and gravel respectively.  A 12mm diameter Iron rod defines the center of the beacon was placed. The perimeter boundary line was cleared to ensure inter-visibility between the beacons. The numbering of the beacons was carried out after molding in a clockwise pattern with an arrow pointing to the succeeding station.

KW

T

3001

FIGURE 3.2: PLAN VIEW

18cm

●

15cm Above ground level

12 mm iron rod

75cm

60cm

Beneath ground level

18cm

FIGURE 3.2:	PROPERTY BEACON

3.4  	PERIMETER TRAVERSING

After the demarcation, capping and numbering of the beacons, the actual data acquisition using the total station  commenced. The traverse started from KWT3001 with KWT3002 as reference point. The total station was set up over control KWT3001, centered, leveled and telescope focused to eliminate parallax. The parameters of the instrument station i.e. station name, height of instrument over the station mark, and the XYZ coordinates of the station were keyed in. The reference control point was then bisected and the station name KWT3002, height of target over the station mark, and the XYZ coordinates of the station were key in.  Though the total station was set in coordinate mode it actually measured and recorded horizontal readings, vertical readings and distances automatically into the internal memory of the instrument on both faces which it used to compute and display coordinates. At every set up of the total station, the temporary adjustment was carried out and the following parameters measured:

- Height of instrument

- Height of the back target

- Height of the fore target

- Distance to back and fore station

This is the determination of bearing and distance of series of connected lines from known coordinated point so as to obtain coordinate of the newly established station.

This include the following with formula

- Linear measurement : the difference between the coordinates were first derived using (∆E=E2-E1, E3-E2) etc. and the distance of end traverse leg was obtained  using the formula: distance =√(∆E)^2 +(∆N)^2

- Angular measurement: to calculated the bearing after the difference in coordinates has been derived, the formula is Bearing=Tan-1 ∆E/∆N.

3.4.1.   COORDINATES OF THE BOUNDARY

TABLE 3: The coordinates are as follows

| S/n | Easting | Northing |
| --- | --- | --- |
| 1 | 678567.030 | 946551.942 |
| 2 | 678692.242 | 946546.081 |
| 3 | 678840.570 | 946538.912 |
| 4 | 678837.737 | 946354.577 |
| 5 | 678835.596 | 946251.888 |
| 6 | 678560.478 | 946251.018 |

Source: Author 2025

3.5   DETAIL SURVEY

Detailing of all features (both natural and man-made) within the site was made by shooting ray to fixing with the instrument.

The instrument was set up on station KWT3001,switched on and adjustments were carried out. Then, “Job” and “Station Name” were set in the instrument so as to recall the coordinates of the boundary point from the instrument’s memory. Also, heights of instrument above the instrument station  and heights of reflectors were measured with steel tape and stored in the instrument’s memory. A reflector placed on beacon KWT3002 was bisected for orientation. The Total station was instructed to compute the bearing between the two stations after input of the orientation station name KWT3003. One of the site assistant placed a reflector  at the edge of a building , the reflector’s cross hair was bisected with that of the telescope of the total station and “DIST” key was pressed for measurement, display and recording. Then the width of the scream was measured with a 50meter steel tape. The same procedure was adopted in detailing the express way by setting on a boundary beacon KWT3002and orienting KWT3003.In this case, all the edges of the carriage way and some buildings were picked.

CHAPTER FOUR

4.0	DATA PROCESSING

As the instrument downloading cable is faulty, Microsoft Excel 2007 Software was used to type the final coordinates of all points except the unwanted part of the data like the temporary controls which were later transported to Note pad and AutoCAD software 2017.

4.1	RESULT ANALYSIS

The data was analyzed and found to meet with the departmental standards and this is the main traverse result extracted from field, then it was arranged accordingly as the observation was held in the field. The results are as follows.

4.2	COMPUTATION

Table 4: Back Computation

| FROM PIL | Bearing | Distance | ∆E | ∆N | Coordinates E N |  | TO PIL |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | 678567.030 | 946551.942 | PIL 1 |
| PIL 1 | 920 40’ 48” | 125.349 | 125.212 | -5.861 | 678692.242 | 946546.081 | PIL 2 |
| PIL 2 | 920 40’ 01” | 148.501 | 148.328 | -7.169 | 678840.570 | 946538.912 | PIL 3 |
| PIL 3 | 1800 5’ 30” | 184.357 | -2.833 | -184.335 | 678564.737 | 946354.577 | PIL 4 |
| PIL 4 | 1810 10’ 18” | 104.711 | -2.141 | -104.689 | 678291.596 | 946249.888 | PIL 5 |
| PIL 5 | 2700 14’ 07” | 275.120 | -277.118 | 01.130 | 678573.583 | 946251.018 | PIL 6 |
| PIL 6 | 010 14’ 50” | 300.995 | 06.552 | 300.924 | 678567.030 | 946551.942 | PIL 1 |

DOUBLE LATITUDE METHOD

-5.861 X+125.21.=-733867532

-5.861

-11.722

-7.169

-18.891 x 148.328= - 2802.064248

-7.169

-26.060

-184.335

-210.395 x -2.833= 596.049035

-184.335

-394.730

-104.689

-499.419 x -2.141 = 1069.356079

-104.689

-604.108

+01.130

-602.978 x -277.118= 167096.0574

+01.130

-601.848

+300.924

-300.924 x + 06552 = - 1971.654048

+300.924

+ 000.000

163253.876

2

Area= 81626.938sqm

= 8.1626 Hectares

4.3

Table 6:	COMPUTE FOR TOTAL AREA USING DOUBLE LATITUDE AND DEPARTURE

| FROM PIL | ∆E | ∆N | Easting | Northing | TO PIL |
| --- | --- | --- | --- | --- | --- |
|  |  |  | 678567.030 | 946551.942 | PIL 1 |
| PIL 1 | 125.212 | -5.861 | 678692.242 | 946546.081 | PIL 2 |
| PIL 2 | 148.328 | -7.169 | 678840.510 | 946546.912 | PIL 3 |
| PIL 3 | -2.833 | -184.335 | 678837.737 | 946354.577 | PIL 4 |
| PIL 4 | -2.141 | -104.689 | 678835.596 | 946249.888 | PIL 5 |
| PIL 5 | -277.118 | 01.130 | 678560.478 | 946251.018 | PIL 6 |
| PIL 6 | 06.552 | 300.924 | 678567.030 | 946551.942 | PIL 1 |

Source: Author Survey, 2025

4.4	GRAPHIC PLOTTING

This simply refers to the graphically representation i.e. plotting of plan. It was plotted using AutoCAD and other software in a computer system and a suitable scale was used to for the hard copy format. Presented information include boundary details and beg, conventional sign and symbol were also used in the plan.

The digital plan was produced using AutoCAD software and these are procedures followed.

- Switch on the computer and allow it to boost

- Select notepad, from notepad, a script file for the coordinate p-line easting, northing was structured.

- File was saved with extension. SCR

- AutoCAD was launched.

- Format was clicked and set the unit then press “OK”

- Press “Tool” and select Run script to pick your saved file then press escape and press zoom, extent and the image was displayed.

The boundary line was changed to Red and necessary editing was done.

CHAPTER FIVE

5.0 SUMMARY CONCLUSION AND ECOMMENDATIONS

5.1 SUMMARY

The project site is part of Institute of Finance and Management Studies, Kwara State Polytechnic, Ilorin located in Moro Local Government area Ilorin, in Kwara State Nigeria. The project is carried out in accordance with third order specifications. The reconnaissance survey was properly carried out, office and field, was done for proper planning of the operation by locating initial controls that is within the project site for proper orientation, the instrument to be used, and selection of traverse station in which the indivisibility of the selected stations were put into consideration and finally, drawing of sketched diagram of the area to be surveyed. The field operation included (Traversing and detailing).  Therefore, data processing was done and the plan was produced in digital format title plan showing perimeter and details of all project area. Finally, report was written on how the entire project was executed.

5.2 	PROBLEMS ENCOUNTERED

The problem encountered on the site were unavoidable especially the movement of vehicles and students which disturbs the work since there is no free flow of traffic.

5.3 CONCLUSION

Having gone through all stages of this project, it is right to say that the task had much interesting, particularly at the planning and execution stage, though the field procedure was very tedious and time consuming. From all indications, the project as being successfully executed and adequate data acquired, processed and represented in plans, all necessary computation were carried out to meet specification given.

5.4	RECOMMENDATIONS

Having participated in this practical work and due to the experience I had acquired during the course of this project, I hereby recommend the following;

1.	The school management should provide better instrument and give proper supervision to project like this and also should set up monitoring committee that will be looking into re-establishment of disturbed control points.

2.	The school should give out the project at right time and also under minimum pressure so that the student can achieve the aim of this project in order for them to be able to implement it.

3	Application of computer programming should be fully implemented so as to make the students carry out the data processing exercise more efficiently and faster.

4.	Courses like survey computation should be introduced again so that students will get acquainted to every computational aspect of surveying.

5.	Softwares like AUTOCAD, Surfer, QGIS and ILWIS should be introduced so as to ease the problem of data processing analysis and presentation.

6.	Digital instruments such as Total Station, G.P.S, Digital Level, and E.D.M should be given to students for practical and project execution instead of the analogue ones.

7.	Owing to the project executed, property inventory could be created which could be referenced to by any individual or the government for planning.

REFERENCES

Allan, A.L. Hollwey, J.R. Marynes, J.H.B.(1968): Practical field surveying and computation, Heinemann, London. P 201

Banister, A and Raymond, S (1986): Surveying 4th Edition London, Pitman publishing limited.

Dashe J.P. (1987): Cadastral surveying and practice in Nigeria.

Encarta Encyclopedia (2009): WWW.google.com

Eruteya E.O (2009): Basic principles in surveying handout

Oxford Advance Leaner’s Dictionary(2001): 6th Edition Oxford University  Press Walton Street.

APPENDIX

| ID | EASTINGS | NORTHINGS |
| --- | --- | --- |
| PT1 | 680146.6 | 946572.7 |
| PT2 | 680349.3 | 946569.9 |
| PT3 | 680345.6 | 946440.4 |
| PT4 | 680145.2 | 946447 |
| B1 | 680152.4 | 946552.1 |
| B2 | 680201.3 | 946550.7 |
| B3 | 680200.5 | 946541.6 |
| B4 | 680152.4 | 946544 |
| B5 | 680151 | 946532.8 |
| B6 | 680201.3 | 946531.8 |
| B7 | 680200.5 | 946519.9 |
| B8 | 680151.3 | 946520.7 |
| B9 | 680151 | 946512.4 |
| B10 | 680200.5 | 946512.4 |
| B11 | 680200 | 946500.8 |
| B12 | 680151.3 | 946503.2 |
| B13 | 680150.8 | 946492.9 |
| B14 | 680199.2 | 946492.1 |
| B15 | 680198.9 | 946481 |
| B16 | 680150.8 | 946481.8 |
| B17 | 680238.6 | 946469.5 |
| B18 | 680289.6 | 946468.5 |
| B19 | 680289.6 | 946456.3 |
| B20 | 680238.6 | 946456.9 |
| B21 | 680212.8 | 946460.6 |
| B22 | 680226.1 | 946460.2 |
| B23 | 680227.1 | 946506.5 |
| B24 | 680214.8 | 946505.2 |
| B25 | 680213.2 | 946560.5 |
| B26 | 680308.1 | 946558.5 |
| B27 | 680309.1 | 946485.7 |
| B28 | 680297.5 | 946485.1 |
| B29 | 680299.5 | 946545.6 |
| B30 | 680228 | 946548.9 |
| B31 | 680239.3 | 946517.8 |
| B32 | 680283.9 | 946517.5 |
| B33 | 680284.9 | 946505.2 |
| B34 | 680239 | 946505.2 |
| R1 | 680350.3 | 946629.8 |
| R2 | 680353.2 | 946593.6 |
| R3 | 680354 | 946521.3 |
| R4 | 680351 | 946442.2 |
| R5 | 680348.3 | 946384 |
| R6 | 680358.3 | 946383 |
| R7 | 680360.6 | 946441.2 |
| R8 | 680361.9 | 946565 |
| R9 | 680361.9 | 946594.8 |
| R10 | 680360.2 | 946630.5 |
| TR1 | 680319.7 | 946498.1 |
| TR2 | 680313.1 | 946481.2 |
| TR3 | 680312.8 | 946460.1 |
| TR4 | 680293.9 | 946442.1 |
| TR5 | 680347.6 | 946510 |
| TR6 | 680324 | 946546.4 |
| TR7 | 680286.3 | 946564.2 |
| TR8 | 680267.1 | 946566.2 |
| TR9 | 680267.1 | 946558.6 |
| TR10 | 680257.6 | 946533.2 |
| TR11 | 680287.3 | 946535.1 |
| TR12 | 680241.4 | 946479.9 |
| TR13 | 680231.4 | 946539.4 |
| TR14 | 680167.6 | 946513.3 |
| TR15 | 680182.5 | 946450.8 |
| TR16 | 680211.3 | 946450.8 |