UTILITY SURVEY OF ROYAL VALLEY ESTATE

BY

MAFTAU AISHAT OMOWUNMI

ND/23/SGI/FT/0054

A PROJECT SUBMITTED TO

THE DEPARTMENT OF SURVEYING AND GEOINFORMATICS

INSTITUTE OF ENVIRONMENTAL STUDIES

KWARA STATE POLYTECHNIC, ILORIN. KWARA STATE

IN PARTIAL FULFILMENT OF THE REQUIREMENT FOR THE AWARD OF NATIONAL DIPLOMA IN (ND) SURVEYING AND GEOINFORMATICS

JULY, 2025

CERTIFICATE

I hereby certify that the field work and information given in this project were obtained as a result of my observation and measurement and were carried out in accordance with survey laws and departmental instruction

_______________  					_______________

MAFTAU AISHAT OMOWUNMI				Date

Student Name

CERTIFICATION

This is to certify that MAFTAU AISHAT OMOWUNMI with matric number ND/23/SGI/FT/0054 from Department of Surveying and Geoinformatics, Institute of Environmental Studies carried out a practical field work which formed basic of the project in accordance with survey rules and regulation and departmental instruction

_______________  					_______________

SURV. AKINYEDE .A.O						DATE

SURV. BELLO .F.D

Project Supervisor

_______________  					_______________

DATE

Project Co-Ordinator

_______________  					_______________

MR ISAU A.I							DATE

Head Of Department

_______________  					_______________

External Examiner							DATE

DEDICATION

This project is dedicated to Almighty God and to my beloved parent MR and MRS MAFTAU

ACKNOWLEDGEMENT

I give glory and adoration to Almighty God the creator of heaven and the earth, the first and the last who give me the opportunity to complete this program. I also give thanks to my lovely parent MR & MRS MUFTAU for their maximum support towards achieving all these.

I had acquired in my life and the one am still looking up for, may Almighty God let them reap the fruit or their labour (amen).

I also acknowledge the support of my project supervisors in person of my project supervisor in person of SURV AKINYEDE & SURV BELLO DIRAN for their strictly and through supervision of this project. Also 	the HOD of my department MR ISAU A.I and my lectures SURV KABIR, SURV KAZEEM, MR AYUBA and those that that could not mention their name may Almighty God guide and bless your families (Amen).

My appreciation also goes to my lovely brother and sister’s for their absolute co-operation and wonderful support towards my career. Because without their reasonable influence and advise they do rendered tome. May almighty god bless them together (amen).

My piece of appreciation cannot be completed without the appreciating the support of all my villa mates and my friends both in school, at home and other different institutions for their maximum co-operation throughout my life. I thank you all, may almighty god be with you.

Finally my profound gratitude goes to my friends who gave me full support throughout my national diploma (ND) days. I say big thanks to you all and wishing you all the best in our entire endeavour.

ABSTRACT

This project is termed' utility survey of Royal Valley Estate, Akerebiata road, Sango area, Ilorin. Nigeria '. The accurate location of utilities at a construction site can prevent any damage to the utilities during works. The means of fixing the damage utilities can be very costly and will reduced the expected profit of the construction works. Therefore utility survey using total station is important in providing accurate information of the utilities within the project site. The detected utilities are then shown in the 3-D drawing for better visualization and identification. The project entails reconnaissance, data acquisition, data processing and data. The plan was produced using ArcGIS 10.5 and Auto-Cad.

TABLE CONTENT

TITLE PAGE								i

CERTIFICATION								ii

DEDICATION								iii

ACKNOWLEDGEMENT						iv

ABSTRACT								v

TABLE OF CONTENT							vi

LIST OF FIQURES							vii

LIST OF TABLES								viii

CHAPTER ONE

1.0	Introduction								1

1.1	Background of study						2

1.2	Statement of problem						4

1.3	Aim of study							5

1.4	Objective of study							5

1.5	Scope of study							6

1.6	Personnel								7

1.7	Study Area								7

CHAPTER TWO

2.0	Literature review							8

CHAPTER THREE

3.0	Methodology							13

3.1	Reconnaissance							14

3.1.1	Data Acquisition (Data acquisition method)			15

3.1.1.1	Office Planning						16

3.1.1.1.1	Field Reconnaissance					16

3.2	Equipment used & system selection				16

3.3	Instrument test & configuration					17

3.4	Control check							18

3.5	Geometric data acquisition					18

3.5.2	Control check/Establishment					19

3.5.3	Traversing								19

3.6	Detailing								20

3.7	Attribute data							20

3.8	Data downloading and Editing					20

3.8.1	Data processing and Report analysis				21

CHAPTER FOUR

4.0	Analysis & Information presentation				25

4.4	Application								38

CHAPTER FIVE

5.1	Summary, Conclusion and Recommendation	s		39

5.2	Summary								40

5.3	Conclusion								41

5.4	Recommendations							41

Reference

LIST OF FIGURES

Figure 1:	Recci diagram

Figure 2:	Map of the study area

LIST OF TABLES

Table 3.1:	Co-ordinate of given existing control point

Table 3.2:	Established Co-ordinate of control point

Table 3.3:	Showing new co-ordinate of control pillar

CHAPTER ONE

INTRODUCTION

1.1	BACKGROUND TO THE STUDY

A utility survey, also known as an underground survey or subsurface utility engineering (SUE) survey, is a process of locating and mapping underground utilities like water, gas, power, telecommunications, and drainage. These surveys are crucial for construction and infrastructure projects to avoid damage to underground services and ensure project safety. Utility surveys are critical components of construction projects. They provide valuable insights into the underground environment and help mitigate risks. When they leverage advanced technologies and methodologies, surveyors can accurately map underground utilities, ensuring worker safety and the integrity of existing infrastructure. Conducting a utility survey ideally during the earliest phases of project planning, when it's most cost-effective and can deliver the highest value is essential to mitigating risks on a construction site. When teams know the location of existing underground infrastructure, they can avoid accidental damage during excavation and drilling and the liability associated with those risks. Creating a utility survey before breaking ground on a construction project is not optional, but you can use a combination of approaches and methods to achieve this.

This growing demand for utility services has resulted in construction of new utility installations, maintenance and repair systems (Lester and Leonard, 2009). Various geophysical tools including electromagnetic induction techniques and ground penetrating radar technology can be utilized to map the position and depth of buried utilities. Each of these techniques has their benefits and limitations but through a careful and diligent application, a comprehensive understanding of subsurface utility network can be Since the advent of civilization, mankind has interacted more with the near-surface part of the Earth's crust than the other deeper layers (Akindulureni et., al,. Therefore, an understanding of the dynamics of this part of the Earth is paramount; industrialization has caused rapid growth, leading to the expansion of man's habitat through the construction of roads, bridges, high-rise buildings, layouts and so on. One of the priority considerations is the design of the foundation for civil engineering structures; therefore, a preconstruction investigation of the proposed site is required in order to ascertain the fitness of the host earth material. This therefore calls for a better understanding of the subsurface geometry and the structural setting in situ. With the rapid urbanization, industries have started relying on more complex and larger utility networks underground. You'll be surprised to know that utility surveying has been with us since the beginning of the 20th century. From underground utility locating and mapping, utility surveying has become an essential step to identify, and map buried infrastructure.(Hussin, Ahamad, & Alhasanat , 2011).

In the early years, people used electromagnetic locator, the very first tool to detect utility maze of pipes and cables underneath the ground. Since its first investigation, the EM field detection theory hasn't been changed. Found in the 1970s, electromagnetic (EM) locator has evolved drastically. Earlier, it used to be a needle-based analogue display whereas, with the advancement in technologies, modern EM locators come with digital display and are more efficient, compact and user-friendly. It now also provides important data like direction, current and strength of the signal. In the early days of utility location services, surveying was done with the help of the only EM locator available in the market and few handy tools. Fiber optics weren't so much popular and thus locators were only locating metallic utilities. Similarly, GPS technology wasn't used widely as it was expensive. Earlier, utility location services provider has simple rules, lift and trace every possible thing to get the required data and leave no room for errors. These still remain the key points of performing any underground utility locating survey. Over the years, the locator has changed while the theory and the methodology remain the same. Ground Penetrating Radar (GPR) is a real-time NDT technique that uses high frequency radio waves, yielding data with very high resolution in a short amount of time. This technique uses electromagnetic waves that travel at a specific velocity determined by the permittivity of the material. GPR can be used in a variety of different media including rock, soil, ice, fresh water, pavements and structures Adepelumi & Fayemi. (2012). It incorporates the geophysical method that uses radar pulses to image the subsurface. This non-destructive method uses electromagnetic radiation within the frequencies of the radio spectrum, and detects the reflected signals from subsurface structures. It can detect objects, changes in material, voids and cracks. GPR Radar measures the depth in terms of the time it takes for a signal to return after emission. The depth in cm or m, depends on soil conditions and how fast the electromagnetic waves can travel through the ground Jeong & Abraham (2004).

Utility survey is a vital process for identifying and mapping utilities accurately. By utilizing advanced technology and digital mapping techniques, utility survey helps ensure the safety of construction projects, reduce service disruptions, and enhance overall urban planning and infrastructure management. These mains may include Schools, boreholes, public toilets/latrines. Major investments need to be made in society’s utility supply detail information, such as Schools, boreholes, public toilets/latrines and electricity distribution, etc. Often the geographic location is recorded on inferior quality maps and in some cases it is not documented at all. Certainty of supply, environmental protection and efficiency of operations, however, require good basic data. Excavation damage can be largely avoided when reliable information regarding location and description of utility lines are available. Repair and replacement of utility lines can be carried out in the optimal manner when based on an efficient GIS/utility mapping system (GIS Landmark,2014). The urban surface utilities of the most densely populated modern cities (United States, United Kingdom, China, Japan and etc.) are characterized by many different types of utility networks including electricity cables, gas pipelines, water and sanitary sewer pipelines and street lighting circuits.

GPR which is widely used to detect underground infrastructures, such as underground utilities, consists of an electromagnetic transmitter and receiver. This technology has been used primarily for detecting structural defects, such as voids and cavities in pavement. The principle of detection is the electromagnetic wave which is radiated from a transmitting antenna, and travels through the material at a velocity which is determined primarily by the permittivity of the material. The wave spreads out and travels downwards until it hits an object which has different electrical properties from the surrounding medium, is scattered from the object, and is detected by a receiving antenna. A straight line drawn from the transmitter to the edge of the wave front is called a ray. Rays are used to show the direction of travel of the wave front in any direction away from the transmitting antenna. If the wave hits a buried object, then part of the wave energy is reflected back to the surface, while part of its energy continues to travel downward. The wave that is reflected back to the surface is captured by a receiving antenna, and recorded in a digital storage device for interpretation. GPR waves can reach depths up to 30 meters in low conductivity materials such as dry sand or granite. The depth of the penetration is also determined by the GPR Antenna used. Antennas with low frequencies obtain subsurface reflections from deeper depths. These low frequency antennas are used to investigate the geology of a site such as for locating sinkholes or fractures and to locate large and deep buried objects. Antennas with higher frequencies obtain reflections from shallow depths and have high resolution. These high frequency antennas are used to investigate surface soils and to locate small, large or shallow buried objects such as utilities and also rebars in concrete.

Utility mapping is done to portray the location of the underground utilities obtained from the GPR survey. In order to place such information in the right context and to understand their mutual relationship over a larger area, it is necessary to map their distribution in the terrain (Annan, 2001). These systems become problematic as soils become saturated with pollutants and effluents with very high suspended and dissolved solids causing environmental pollution within the study and even groundwater pollution in some cases. This gave rise to plan to construct a central sewage system within the study area to mitigate this problem and to construct this system successfully an underground utility survey is needed within the study area as presence of other utilities such as gas line, power line etc.. This information will aid the construction engineers for the design and construction of this system.

1.2	STATEMENT OF PROBLEM

Due to the challenge faced by governments, urban planners, and utility providers worldwide. Traditional methods of infrastructure mapping and monitoring, which rely on manual inspections, paper-based records, and static maps, have proven to be inefficient, time-consuming, and prone to inaccuracies. These outdated approaches often lead to poor decision-making, delays in maintenance, and increased operational costs, ultimately affecting service delivery and public safety. This study seeks to address these issues by exploring the role of utility survey in infrastructure management, identifying its benefits and challenges, and providing recommendations for its effective implementation.

1.3	AIM AND OBJECTIVES

1.3.1	AIM

The aim of the project is to carryout utility survey of Royal valley Estate in order to have a good design and planning of the Estate which will not to affect all facilities in the study area.

1.3.2	OBJECTIVES

The following objectives were considered in order to accomplish the above aim:

1. Project planning

2. Monumentation

3 Data acquisition

4 Data processing

5 Information presentations

1.4	SCOPE OF THE PROJECT

Based on the purpose and significance of this project, the scope includes;

1 Project planning

2 Station selection and monumentation

3 Data Acquisition

4 Data downloading and Data processing

5 Data Analysis and information presentation

1.5	PERSONNEL

The underlisted students of ND II 2025/2025 set are the personnel that participated in the execution of this project. They are: -

| S/N | NAME | MATRIC NUMBER | ROLE |
| --- | --- | --- | --- |
| 1 | Muftau Aishat Omowunmi | ND/23/SGI/FT/0054 | Author |
| 2 | Olawale Azeez Olamilekan | ND/23/SGI/FT/0055 | Member |
| 3 | Olafimihan Emmanuel .O | ND/23/SGI/FT/0053 | Member |
| 4 | Rahman Zainab Olamide | ND/23/SGI/FT/0064 | Member |
| 5 | Awolusi Taiwo Ireyemi | ND/23/SGI/FT/0066 | Member |
| 6 | Muhammed Aishat | ND/23/SGI/FT/0058 | Member |
| 7 | Babalola Olamide Grace | ND/23/SGI/FT/0062 | Member |

1.6	STUDY AREA

The study area is Royal valley estate Sango Ilorin, Kwara State, Ilorin East Local Government Area which lies within longitude 08°29’21”E latitude 04°30’50’’N and longitude 08°29’43”E latitude 04°31’01”N.

Fig 1.1:  Map of the Study Area

CHAPTER TWO

LITERATURE REVIEW

Many have not realized the significance of the underground utility detection which is vital before starting any excavation works. A misfortune could take place if the work is ongoing before the underground utilities are located. Basically, underground utility detection is the procedure of recognizing, isolating and labeling the public and private subsurface utility lines which are hidden beneath the ground surface. These lines comprise of electricity distribution cable, telephones, fibre optics and communication lines, water and waste water conduits and major oil and gas pipeline, mass transit, road tunnels and rail etc. (Dharam, Ranjit, Mohamed, March 2020).

An electromagnetic technology is used in GPR system to detect lines among subsurface mediums with differing dielectric constants. GPR system comprise of antenna that houses the receiver and transmitter with a recorder that reproduce the received signal data in form of a graphic display. The instrument employs two antennas offering different detection frequencies. The 700 MHz antenna detects objects with very high resolution but with a smaller depth range. On the other hand, the antenna with a lower frequency (250 MHz) has a lower resolution, but at the same time, it offers a more extensive depth range. Therefore, one can read the resolution and depth information from a given frequency. The resolution of GPR measurements is called the instrumental ability to determine a minimum distance at which two identical targets can be recognized as separate objects. One can distinguish vertical and horizontal resolutions in the increment values of changing antenna frequency and the dielectric constant of the medium. Therefore reliable and accurate information on underground assets is of vital importance for the planning and management of land and the space below its surface. Underground utilities typically reside in the shallow part of such space. They are just one of its many competing uses, which include building basements and interlinking pedestrian corridors, piles, transportation infrastructure, and more (Chandran, 2019).

The rapid development of electromagnetic technology in the last few years has achieved a new method for exploration geophysics science. In this technology, the electromagnetic pulse is transmitted into the ground and recorded by antenna in the surface. The GPR method is intensively used on the land in mapping of underground structure, and searching of archaeological objects, pipes, cable, mineral substance, either underground or below sea level (Witten Technologies Inc. (2001). Beneath the surface of urban region and big cities all over the world lies a wide complex network of cables and pipelines which provide the vital utility services that reinforce modern civilized life. An extraordinary call for utility services within the urban region leads to bury an accumulative digit of utility pipelines such as; telecommunication lines, fibre optics, water and gas pipelines electrical cables. Today many of underground utilities have reached its ending practical life which makes it necessary to be replaced or repaired. While new utilities are awaiting installation due to urban development and the advancement of human life style such as; advanced communication technologies. Then an accurate data and info of these utilities is needed for engineers, surveyors, utility owners, or contractors, generally as reference for the excavation work.

The presence, layout, and organization of underground utilities directly affect the value that the land can continuously deliver. It may limit the potential for future development and use or the capacity to host new infrastructure and may present significant obstacles, risks, and nuisances for owners, developers, engineers, and users of the land. It is reasonable to expect that the need for reliable information on underground utilities will become increasingly relevant as cities over the world continue to grow, densify, and change. Vertical and in particular underground development is a way to deal with the limited availability of land above ground. Furthermore, ongoing developments such as the introduction of new services such as 5G (Mims, 2019) and district cooling or heating systems (Bradley, 2020; Wee, 2017), the transition to non-fossil energy sources, and the need to replace existing ageing utility infrastructure will only increase the pressure on an often already congested under- ground space. Some cities have responded to these challenges. The City of Helsinki released its first Underground Master Plan in 2011 (City of Helsinki, 2011). The Municipality of Rotterdam has adopted an asset management strategy for the management of underground space and infrastructure. It has a specialized unit coordinating utility developments and the collection of survey data (Rotterdam, 2019).

In Japan, the Road Administration Information Centre coordinates all utility works that occur on roads in metropolitan areas and, to support that, maintains a centralized dataset of all road and utility information (Zeiss, 2019). For the city-state of Singapore, going underground‖ is one of its strategies to deal with limited land availability. The Master Plan 2019 (Urban Redevelopment Authority of Singapore, 2019a) highlighted underground space use as a means to create space for growing needs. The Urban Redevelopment Authority of Singapore (URA) furthermore mentions that planning the underground space upfront in 3D based on BIM and GIS data and the mapping and dissemination of data on shallow utility lines are required to safeguard underground space for future uses and to eliminate uncertainty and risk during the planning process (Urban Redevelopment Authority of Singapore, 2019b). Besides master planning, reliable information on underground utilities is expected to continuously provide value throughout the life cycle of land assets and management.

Currently in Nigeria, the management and maintenance of underground utilities are non-existence. As more currently there is a water project going on within the city of Port Harcourt without any information taken about the buried utilities, such as the location and size of the facility. This will affect future construction because of the absence of the underground -utility data.

In Nigeria the lack of pre-construction under-ground utility investigations, which assist engineers in obtaining as-built subsurface information has led to building collapse and structural failures. A number of factors such as subsurface geological material, inadequate information about the soil, poor foundation design and poor building materials have also attributed to this failure (Fatoba et al., 2013). This has led to the loss of life and lots of goods and properties worth millions aside from the cost of rehabilitation or complete redesign and reconstruction at much higher cost. Recently, the collapse of civil engineering structures has increased greatly for reasons associated with lack of information about subsurface geological and utility data. As a result, the need for subsurface investigation has become very crucial so as to prevent loss of lives and valuable properties that consistently occur with such a failure. Foundation assessment of a new site is required so as to provide subsurface and aerial information that normally assist civil engineers, builders and town planners in the design and citing of foundations of civil engineering structures (Omoyoloye et. al, 2008).

CHAPTER THREE

3.0 	METHODOLOGY

This aspect refers to a system of methods and principles, rules used in a field of study to achieve the aim and objectives of this project. The modality to successful execute this project was carried out based on the survey rules and regulation, supervisor advice and departmental instructions.

3.1 	RECONNAISSANCE

Reconnaissance survey is a preliminary stage for exploratory investigations conducted to gather initial data concern the choice of Site area of Subject before more detailed research or filed work commenced. Reasons for reconnaissance survey of the project area are the following;

- To know the location of the site.

- To determine the preliminary area/perimeter of the land.

- To know the survey method suitable for the field data acquisition and the instrument need for it.

Reconnaissance in this project involved preliminary investigations to gather information about the survey area before conducting a detailed survey, reconnaissance of this project was carried out in two way namely as follows.

- Office planning

- Field Reconnaissance

3.1.1 	OFFICE PLANNING

It involved the process adopted in the collection of relevant data and specification about the as-built Surveying with the used of traversing survey method. During the office planning the given coordinate for control point PBIL 100, PBIL 107, and PBIL 110, was obtained and computed as shown in the table 3.1 below. The choice of equipment to be use for project was obtained from the survey store in the department of surveying and Geoinformatic, Kwara State Polytechnic, Ilorin, and it was handled accordingly to the survey rules and regulations and departmental instruction.

TABLE 3.1 Showing the list of the coordinate of the control pillars used

| Stations | Northing (m) | Easting (m) |
| --- | --- | --- |
| PBIL 100 | 939353.130 | 648821.110 |
| PBIL 107 | 939306.460 | 648932.560 |
| PBIL 110 | 939224.969 | 649120.403 |

3.1.2 	FIELD RECONNAISSANCE

Field recce and site visitation was conducted as preliminary survey for investigation of the basic information about a specified project area, location, or subject in the field. It allows the members of the groups to be familiar with the terrain, boundary lines, selection number of station required, Equipment that will be used for the job, pillars number PBIL 100, PBIL 107, and PBIL 110, used as a control for the project and so on. The stations points selected was Inter-visible to one another and finally a reconnaissance diagram was produced which was attached to this project

3.2	INSTRUMENT SELECTION

The equipment used for this project was grouped into two categories;

- Hardware

- Software

3.2.1	Hardware used

- Total Station with its Accessories (KTS-442L)

- Reflector poles

- Field book

- Writing material (pen)

- Survey beacons

- Steel Tape (5m)

- Tripod Stand

3.2.2	Software used

Below are the lists of the software used during the project processing;

- AutoCAD 2007

- Notepad

- Microsoft word

- Microsoft Excel

3.3 	INSTRUMENT TEST

The instrument test was carried out before it used for angular observation. The purpose of testing the instrument before used is to know the assurance and maybe other instrument is in good Condition the instrument was set up over station A and it was pointed a Sighted to station B whore a pole Is hold, and the angle was recorded for face left the instrument was turn to farce right on the same point and it was sighted to station B, where a pole is held at the same position and the angle was recorded. The distance between station A and B 1s 10m. The steel tape was hold at 60m an point A and it was extend to point B. the results for the instrument test was shown i to figure 3.3 below and table 3:2 below.

Total station

LINE OF SIGHT

RANGING POLE

20M

STATION  X

TERRAIN

STATION  Y

Figure 3.1 showing the instrument set up for collimation test

Table 3.2 showing the test of instrument result

| STATION | SIGHT | FACE | HORIZONTAL READING | DEDUCED ANGLE |
| --- | --- | --- | --- | --- |
| A | B | L | 180° 00’ 03” |  |
|  | B | R | 00°00’ 00” | 180° 00’ 03” |

Different in horizontal circle reading: 180° 00’ 03’’

Horizontal collimation error

= 180° 00’ 03’’

= 000° 00 03’’

Collimation error = 00°00'03"2

Horizontal collimation error = 00° 00’ 01”

Because the collimation error falls within the allowable 01”. The instrument was in good condition.

3.4 	MONUMENTATION

18cm

15cm above the ground

60cm below the ground

18cm

This is the process in which the beacon is being buried in to ground. On the project site there are same existing beacons on the boundary lines m, nail with cap (bottle cork) was used as the station pegs.

30cm

Nail

Bottle cork

PILLAR							   PEG

FIGURE 3.4	Diagram and Dimension of Pillars buried

3.5 	CONTROL CHECK

The Control check was carried out to know the control point PBIL 100, PBIL 107, and PBIL 110, are still in their positions. The instrument was set on Control FP01107 and the temporary adjustment was carried out i.e centring, levelling, and focusing. Back site was taken to control PBIL 107 and it was recorded.

The fore sight was served to PEG1 and it was recorded Steel tape was used to measure the distance between PBIL 110 and PEGI and it was recorded. The data observed was computed for given coordinate and Observed Coordinate, as shown in the table 3.2 below.

TABLE 3.2	THE GIVEN COORDINATE OF THE CONTROL POINTS

| STATION | NORTHING (m) | EASTING (m) |
| --- | --- | --- |
| PBI 100 | 939353.130 | 648821.110 |
| PBIL 107 | 939306.460 | 648932.560 |
| PBIL 110 | 939224.969 | 649120.403 |

TABLE 3.3	The Backward Computation for the Given Controls pillars

| From station | Bearing | Distance | ∆N | ∆E | Northing (m) | Easting (m) | To station |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | 899353.130 | 688821.110 | PBIL 100 |
| PBIL 100 | 1120 43̍ 17 ̎ | 120.827 | -46.670 | 111.450 | 899306.460 | 688932.560 | PBIL 107 |
| PBIL 107 | 1130 27̍ 09 ̎ | 204.758 | -81.491 | 187.843 | 899224.969 | 689120.403 | PBIL 110 |

PBIL 100 to PBIL 107

BEARING = 1120 43̍ 17 ̎

DISTANCE = 120.827m

PBIL 107 to PBIL 110

BEARING = 1130 27̍ 09 ̎

DISTANCE = 204.758m

TABLE 3.4	Show the Observed Coordinate of the Controls Pillars

| Station | Northing (m) | Easting (m) |
| --- | --- | --- |
| PBIL 100 | 939353.126 | 648821.105 |
| PBIL 107 | 939306.460 | 648932.560 |
| PBIL 110 | 939224.969 | 649120.403 |

TABLE 3.5	Show the Backward Computation for the Observed Control Pillars

| From station | Bearing | Distance | ∆N | ∆E | Northing (m) | Easting (m) | To station |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | 899353.126 | 688821.105 | PBIL 100 |
| PBIL 100 | 1120 43̍ 08 ̎ | 120.830 | -46.666 | 111.455 | 899306.460 | 688932.560 | PBIL 107 |
| PBIL 107 | 1130 27̍ 09 ̎ | 204.758 | -81.491 | 187.843 | 899224.969 | 689120.403 | PBIL 110 |

PBIL 100 to PBIL 107

BEARING = 1120 43̍ 08 ̎

DISTANCE = 120.830m

PBIL 107 to PBIL 110

BEARING = 1130 27̍ 09 ̎

DISTANCE = 204.758m

TABLE 3.6	Show the Difference between the Given Control Points and the Observed Control Pillars

| STATION | NORTHING (m) | EASTING (m) |
| --- | --- | --- |
| PBIL 100 | 0.004 | 0.005 |
| PBIL 107 | 0.000 | 0.000 |
| PBIL 110 | 0.000 | 0.000 |

Therefore the difference between the given and observer bearing was 00° 00’ 09” and 00° 00’ 00” and the difference between the given and the observer linear distance was 0.0003m, 0.000m the results obtained fall within the allowable accuracy for both angular and linear observation.

It was concluded the instrument was in good condition and fit to be used for the project.

3.6 	DATA AQUISITION

This was carried out on the field using Total station with its accessories (KTS-442L) was used for coordinating the entire site for both the perimeter and details of the site. The Total station was set on the 	control Pillar PBIL 107, and the temporary adjustment was carried out centering, leveling and 	focusing. The coordinates of the occupied station was input after that the coordinates of the backsight was record as well and the observation was done to backsight pillar and the coordinates was take and check against the given. Having check and found accurate then the perimeter survey commenced for the entire project site.

3.6.1	DETAILING SURVEY

The detailing survey was done immediately after completely carried out the perimeter traverse to determine the position of the features on the Site was fixed using radiation survey method. During this several stations was established to pick the coordinates of the details and the coordinates was stored on the total station as its observed. This was later downloaded. The details that was picked during the detailing survey was coded in different ways like Building edges as ‘BLD’, Electric Poles ‘EP’, Drainage ‘DR’, Roads Line ‘RD’, Spot Height ‘SP’ and so on.

CHAPTER FOUR

4.0   	DATA PROCESSING

This involved several step to ensure accurate and reliable result. It explains the method used in capturing, processing, downloading, and analyzing the data acquired Data reduction in surveying involves various methods to simplify and summarize the collected data for analysis. These are some common methods of data reduction that involves

- Combining multiple values into a single value. The average value is calculated by summing up all the values and dividing by the number of values.

- Least Square Adjustment (LSA) is a method of data reduction that involves minimizing the sum of the squared errors between observed and calculated values.

- Interpolation is a method of data reduction that involves estimating values between observed data points.

- Spatial data reduction refers to the process of simplifying and reducing the complexity of spatial data, such as geographic information system (GIS) data, while preserving its essential characteristics.

4.1	DATA EDITING

- Open excel

- Click on the desire file

- Set to delimited

- Click on next

- Mack space and comma

- Click

- On next

- Then click on finish

4.3	DATA PROCESSING USING AUTOCAD

The plan produced digitally with AutoCAD software. The following are the procedure used:

STEP 1: Open Notepad

Open Notepad on your computer. You can find it in the start menu (Window)

STEP 2: Create a New File

Click on "file" "New" to create a new file

STEP3: Enter Coordinates

Enter your coordinates in the following format:

X. Y. Data

STEP 4: Save the Fil

Click on "File" > "Save as" and choose a location to save your file. "ALADA Coordinate" txt, 	and select "All files" as the file type.

Importing the Coordinate File Into 2007 AutoCAD

STEP 1: Double click AutoCAD

Go under format

Click on format after that pick your unit

Drawing unit

Type.... Decimal

Precision... 0.000

Angle type Degree/Min/See

Precision Od00

Clockwise.... 노

Unit to scale Meters

Direction control North 270d0

Press "OK"

STEP 2: Create a new drawing

Click on "File" > "Name" to create a new drawing

STEP 3: Import the coordinate file

Click on "Tools" >"Reference" > "Import" and select "ALADA coordinate (.txt)" as the file 	type. Navigate to the location where you saved your coordinate file ("ALADA coordinate .txt") 	and select it.

STEP 4: Configure Import Setting. In the import select "Point" as the object type and choose 	the correct coordinate format (eg. X and Y) click "OK" to import the coordinates.

4.4	DATA EXTRACTION

The data recorded in the field book was picked to compute for the bearing and distance. Forward computation and backward computation after all necessary correction have been applied. Correction like slope correction, tension correction and standardization correction.

4.3  	BACK COMPUTATION

The result get for forward computation was used for Back computation.

The following program was required for back computation

i. Input N2-N1 to get ∆N

ii. Input E2-E1 to get ∆E

iii.Input(∆N2+∆E2 to get distance

iv.Input shift tan ∆E∆Nto get the corrected bearing when there us more station repeat step ¡¡¡ and ii.

Table 4.1 Showing Boundary Coordinates

| Station | Northing (m) | Easting(m) |
| --- | --- | --- |
| PBIL107 | 939366.406 | 648932.560 |
| PL1 | 939240.312 | 648901.750 |
| PL2 | 939309.331 | 648780.785 |
| PL3 | 939292.197 | 648603.139 |
| PL4 | 939438.718 | 648619.716 |
| PL5 | 939467.847 | 648693.558 |
| PL 6 | 939353.126 | 648821.105 |

Table 4.1.1 Showing back computation for the boundary coordinate

| FROM Station | BEARING | DISTANCE | ΔN | ΔE | Easting | Northing | TO STATION |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | 648932.560 | 939366.406 | PBIL 107 |
| PBIL 107 | 204 58 29 | 72.971 | -66.148 | 30.810 | 648901.750 | 939240.312 | PL1 |
| PL1 | 299 42 28 | 139.270 | 69.019 | 120.965 | 648780.785 | 939309.331 | PL2 |
| PL2 | 295 39 02 | 193.739 | 83.866 | 174.646 | 648603.139 | 939292.197 | PL3 |
| PL3 | 16 40 37 | 47.520 | 45.521 | 13.637 | 648619.716 | 939438.718 | PL4 |
| PL4 | 112 42 17 | 79.980 | 30.871 | 73.782 | 648693.558 | 939467.847 | PL5 |
| PL5 | 113 13 14 | 113.790 | 54.721 | 127.547 | 648821.105 | 939353.126 | PL6 |
| PL6 | 112 43 59 | 120.870 | 48.666 | 45.500 | 648932.560 | 939366.406 | PBIL 107 |

TABLE 4.1.2: showing area computation for the boundary coordination using double latitude

| ΔN | ΔE | product |
| --- | --- | --- |
| 66.148 | × - 30.810 | = 2038.020 |
| 66.148 |  |  |
| 69.019 |  |  |
| -63.277 | × - 120.965 | =7654.302 |
| 69.619 |  |  |
| 83.866 |  |  |
| 89.608 | × -174.646 | -=15649.679 |
| 83.866 |  |  |
| 45.521 |  |  |
| 218.995 | × 13.637 | =2986.435 |
| 45.521 |  |  |
| -30.871 |  |  |
| 233.645 | × 73.782 | =17238.792 |
| -30.871 |  |  |
| -54.721 |  |  |
| 148.653 | × 127.547 | =18883.716 |
| -54.721 |  |  |
| -46.666 |  |  |
| 46.666 | × 111.455 | =5201.159 |
| -46.666 |  |  |
| 00.000 |  |  |

Area =

2038.020+7654.302-15649.679+2986.435+17238.795+18883.716+5201.1592

54002.427-18649.6792

38352.7482

=19176.374=> 1.9 hectares

1.9 Hectares

Hectare=19176.37410000

= 1.9176374 Hectare

Linear Accuracy = 1(EL)2-((ED)2TOTALLENGHT

EL=EDN=0.066

ED=EDE=0.072

Total length =793.100

∴LinearAccuracy

1(0.66)2+(0.072)2793.100
 = 10.09767793.100

=1:8000

CHAPTER FIVE

SUMMARY, PROBLEMS ENCOUNTERRED, CONCLUSION, RECOMMENDATIONS

5.1 	SUMMARY

This study was carried out to determine the utilities within the study area and produced a map depicting the position, depth and size of the underground facilities in focus. The plan showing the utilities and facilities was produced with appropriate colour code. It can be concluded that total station equipment was used to acquire the data of the utilities. The technique used is efficient and non-destructive for locating utilities in the study area. It is advisable to perform utility survey before carrying out any works in order to avoid damage to the utilities which can be quite expensive to fix.

5.2 	PROBLEM ENCOUNTERED

- Too much of rainfall which does not allowed site

- Unavailable of the group members due to rector’s cup football tournament

- While taking observing there are some object obstructing the view when taking the observation e.g electric pole, building, tree etc.

5.3 	RECOMMENDATIONS

As a matter of fact, utility survey should be encouraged within the city as they form the foundation within which modern cities are built. Also, there should be awareness on the importance of the utility survey, especially during road construction going within our city.

5.4 	CONCLUSION

At the end the aim and objective of this project was achieved. It was carried out using third order surveying method this project was carried out in accordance with survey rules and regulation and departmental instruction.

REFERENCES

Adepelumi. A. A. and Fayemi. O. (2012):	Joint application of ground penetrating radar and electrical resistivity measurements for characterization of subsurface stratigraphy south western Nigeria. Journal of Geophysics and Engineering. Vol. 9. P. 397 – 412, 2012.

Akindulureni J. O, Adepelumi A., A., and Ebiko O., (2015):	Integrated Geophysical Mapping of Near-surface Structures and Utilities in Ile-Ife, Nigeria. Universal Journal of Geoscience 3(4): 135-145, 2015 http://www.hrpub.org DOI: 10.13189/ujg.2015.030403.

Al-Nuaimy,W., HuangY, Nakhkash, M (2023):	Automatic Detection of Buried Utilities and Solid Objects with GPR Using Neural Network and Pattern Recognition. Journal of Applied Geophysical, vol. 43, issue 2-4 2000 .pp 157-165.

American Society of Civil Engineering (ASCE), (2002):	Standard Guideline for the Collection and Depiction of Existing Subsurface Utility Data. ASCE Code and Standards Activity Committee (CSAC), New York, US, pp. 4–6.

Ariaratnam, Samuel. (2010):	Survey questionnaire results of the current level of knowledge on trench less technologies in China. Tunnelling and Underground Space Technology. 25. 802-810. 10.1016/j.

Aydin, Cevdet. (2008):	Usage of Underground Space for 3D Cadastre Purposes and Related Problems in Turkey. Sensors. 8. 10.3390/s8116972.

Barde Jatau, Rui M.S, Fernandes, (2010):	The New Permanent GNSS Network of Nigeria

B.Hofman-Wellenhof, H.Lichtenneger, and J.Collins,(2001):	GPS Theory and practice.‖ Fifth revised edition .

APPENDIX

| Name | Code | E | N | H |  | PT260 | SH | 668115.4 | 937791.5 | 317.946 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PT1 | ST17 | 668069.6 | 937900.4 | 324.137 |  | PT261 | SH | 668118 | 937790.7 | 318.041 |
| PT2 | CV | 668071.1 | 937900.4 | 324.553 |  | PT262 | SH | 668117.7 | 937795.6 | 317.967 |
| PT3 | CV | 668072 | 937900.3 | 324.541 |  | PT263 | SH | 668113.5 | 937797.2 | 317.656 |
| PT4 | BLD | 668072.6 | 937900.6 | 324.383 |  | PT264 | SH | 668110.1 | 937800.8 | 317.647 |
| PT5 | BLD | 668066.5 | 937905.8 | 323.724 |  | PT265 | SH | 668110.4 | 937806.9 | 317.571 |
| PT6 | SH | 668069.7 | 937897.9 | 324 |  | PT266 | SH | 668114.1 | 937808.4 | 317.326 |
| PT7 | SH | 668072.6 | 937896.1 | 324.671 |  | PT267 | SH | 668117.8 | 937810 | 317.601 |
| PT8 | SH | 668072.2 | 937891.5 | 324.604 |  | PT268 | SH | 668117.6 | 937814.7 | 317.532 |
| PT9 | SH | 668076.5 | 937892.6 | 324.821 |  | PT269 | SH | 668113.8 | 937816 | 317.212 |
| PT10 | SH | 668083.2 | 937892.3 | 324.831 |  | PT270 | SH | 668109 | 937815.1 | 317.485 |
| PT11 | SH | 668085.6 | 937889.4 | 324.87 |  | PT271 | SH | 668104.5 | 937814.4 | 317.27 |
| PT12 | BLD | 668085.4 | 937886.8 | 324.848 |  | PT272 | SH | 668104.8 | 937819.5 | 317.197 |
| PT13 | SH | 668088.7 | 937888.5 | 324.887 |  | PT273 | SH | 668109.7 | 937818.5 | 317.278 |
| PT14 | SH | 668090.4 | 937893.3 | 324.882 |  | PT274 | SH | 668113.6 | 937819 | 317.192 |
| PT15 | SH | 668095.3 | 937895.1 | 324.957 |  | PT275 | SH | 668117.7 | 937821.2 | 317.334 |
| PT16 | SH | 668095.6 | 937891.4 | 324.958 |  | PT276 | SH | 668114.1 | 937823.2 | 317.152 |
| PT17 | BLD | 668139.4 | 937901.3 | 324.586 |  | PT277 | SH | 668109 | 937824.1 | 317.181 |
| PT18 | CV | 668139.1 | 937901.2 | 316.047 |  | PT278 | SH | 668104.9 | 937822.2 | 317.161 |
| PT19 | CV | 668139.5 | 937900.4 | 316.104 |  | PT279 | SH | 668105 | 937825.4 | 317.107 |
| PT20 | CV | 668137.3 | 937899 | 317.048 |  | PT280 | SH | 668103.5 | 937827.1 | 317.276 |
| PT21 | SH | 668133.7 | 937894.7 | 316.106 |  | PT281 | SH | 668108.1 | 937827.2 | 317.179 |
| PT22 | SH | 668136.7 | 937894.8 | 316.021 |  | PT282 | SH | 668112 | 937827.1 | 317.097 |
| PT23 | BLD | 668137 | 937893.5 | 316.015 |  | PT283 | SH | 668116.6 | 937827.5 | 317.116 |
| PT24 | SH | 668141.1 | 937890.4 | 315.886 |  | PT284 | SH | 668117.8 | 937831.4 | 317.048 |
| PT25 | SH | 668145.9 | 937889.2 | 315.599 |  | PT285 | SH | 668112.9 | 937833.2 | 317.012 |
| PT26 | SH | 668147.8 | 937891.6 | 315.619 |  | PT286 | SH | 668108.8 | 937832.7 | 317.058 |
| PT27 | GT | 668148.7 | 937893.4 | 315.804 |  | PT287 | SH | 668106.3 | 937831.7 | 317.095 |
| PT28 | GT | 668147.9 | 937894 | 315.765 |  | PT288 | SH | 668120.4 | 937780 | 318.343 |
| PT29 | BLD | 668148.9 | 937893.2 | 315.928 |  | PT289 | SH | 668125.9 | 937783 | 318.036 |
| PT30 | SH | 668148.1 | 937886.3 | 315.852 |  | PT290 | SH | 668122.6 | 937785.4 | 318.043 |
| PT31 | SH | 668144.4 | 937882.9 | 315.942 |  | PT291 | SH | 668120.9 | 937789.4 | 318.013 |
| PT32 | SH | 668140.3 | 937879.1 | 316.287 |  | PT292 | BLD | 668120.6 | 937791.6 | 317.908 |
| PT33 | SH | 668144.4 | 937877.2 | 316.236 |  | PT293 | SH | 668125 | 937790.3 | 317.853 |
| PT34 | SH | 668147.4 | 937877.9 | 316.065 |  | PT294 | SH | 668128.5 | 937785.7 | 317.944 |
| PT35 | BLD | 668149.4 | 937879.9 | 315.956 |  | PT295 | SH | 668133 | 937781.5 | 317.952 |
| PT36 | BLD | 668156.1 | 937879.6 | 315.966 |  | PT296 | SH | 668132.9 | 937786.5 | 317.857 |
| PT37 | BLD | 668156 | 937874.1 | 315.966 |  | PT297 | SH | 668132.2 | 937790.7 | 317.807 |
| PT38 | EP | 668150.9 | 937875.2 | 316.093 |  | PT298 | SH | 668137 | 937790.8 | 317.772 |
| PT39 | SH | 668152 | 937878.1 | 315.912 |  | PT299 | SH | 668139.7 | 937786.5 | 317.824 |
| PT40 | BLD | 668146.3 | 937874.1 | 316.297 |  | PT300 | SH | 668140 | 937781.4 | 318.046 |
| PT41 | SH | 668132 | 937896.8 | 316.129 |  | PT301 | SH | 668144.3 | 937781.2 | 318.023 |
| PT42 | BLD | 668128.4 | 937894.7 | 316.378 |  | PT302 | SH | 668145.5 | 937786.4 | 317.727 |
| PT43 | SH | 668124.8 | 937894.3 | 316.451 |  | PT303 | SH | 668146 | 937791.2 | 317.717 |
| PT44 | SH | 668121.3 | 937892 | 316.499 |  | PT304 | SH | 668149.5 | 937790.8 | 317.75 |
| PT45 | SH | 668118.3 | 937894.3 | 316.594 |  | PT305 | SH | 668149.7 | 937786.7 | 317.692 |
| PT46 | SH | 668115.7 | 937888.6 | 316.573 |  | PT306 | SH | 668148.9 | 937781.3 | 317.842 |
| PT47 | BLD | 668113.6 | 937886.9 | 316.529 |  | PT307 | BLD | 668150.4 | 937781.4 | 317.8 |
| PT48 | SH | 668110.4 | 937889.4 | 316.736 |  | PT308 | BLD | 668150 | 937791.5 | 317.803 |
| PT49 | SH | 668108.3 | 937891.8 | 316.596 |  | PT309 | SH | 668113.7 | 937768.9 | 318.789 |
| PT50 | SH | 668104.4 | 937892.2 | 316.593 |  | PT310 | SH | 668113.6 | 937766 | 318.901 |
| PT51 | SH | 668104.2 | 937889.1 | 316.626 |  | PT311 | BLD | 668112.8 | 937763.3 | 319.05 |
| PT52 | SH | 668117.5 | 937886.8 | 316.551 |  | PT312 | SH | 668117.2 | 937761.4 | 318.943 |
| PT53 | SH | 668119.7 | 937883.9 | 316.541 |  | PT313 | SH | 668117.5 | 937764.3 | 318.911 |
| PT54 | SH | 668117.1 | 937883.2 | 316.562 |  | PT314 | SH | 668122.4 | 937765.5 | 318.977 |
| PT55 | SH | 668114.2 | 937882.6 | 316.592 |  | PT315 | SH | 668124.5 | 937762.3 | 318.933 |
| PT56 | BLD | 668113.5 | 937874.4 | 316.659 |  | PT316 | SH | 668122.4 | 937758.6 | 318.969 |
| PT57 | SH | 668115.5 | 937874 | 316.641 |  | PT317 | SH | 668121.7 | 937753.2 | 319.012 |
| PT58 | SH | 668117.6 | 937873.9 | 316.666 |  | PT318 | SH | 668119.4 | 937752.3 | 319.022 |
| PT59 | SH | 668116.9 | 937866.3 | 316.759 |  | PT319 | BLD | 668113 | 937752.1 | 319.283 |
| PT60 | SH | 668114.9 | 937865.7 | 316.81 |  | PT320 | BLD | 668113.1 | 937748.7 | 319.275 |
| PT61 | SH | 668112.2 | 937865.8 | 317.066 |  | PT321 | BLD | 668113.2 | 937730.1 | 319.819 |
| PT62 | BLD | 668111.7 | 937870.2 | 316.664 |  | PT322 | BLD | 668118.1 | 937726.7 | 320.172 |
| PT63 | SH | 668124 | 937868.3 | 316.542 |  | PT323 | BLD | 668118.2 | 937750.2 | 319.117 |
| PT64 | BLD | 668129.2 | 937867.4 | 316.594 |  | PT324 | SH | 668122.5 | 937751.6 | 319.006 |
| PT65 | SH | 668128.1 | 937869.8 | 316.579 |  | PT325 | SH | 668124.5 | 937754.3 | 318.972 |
| PT66 | SH | 668127.9 | 937872.5 | 316.566 |  | PT326 | SH | 668125.4 | 937757 | 318.942 |
| PT67 | SH | 668125 | 937873.8 | 316.569 |  | PT327 | SH | 668126.4 | 937759.8 | 318.902 |
| PT68 | SH | 668123.6 | 937877 | 316.822 |  | PT328 | SH | 668127.7 | 937763.6 | 318.945 |
| PT69 | SH | 668126 | 937879.4 | 316.682 |  | PT329 | SH | 668129.6 | 937766.7 | 319.003 |
| PT70 | SH | 668121.7 | 937869.6 | 316.558 |  | PT330 | SH | 668132.3 | 937766.4 | 319.052 |
| PT71 | SH | 668121.5 | 937862 | 316.817 |  | PT331 | SH | 668132.9 | 937763.9 | 319.098 |
| PT72 | SH | 668123.1 | 937858.4 | 316.857 |  | PT332 | SH | 668133.1 | 937760.8 | 318.995 |
| PT73 | SH | 668118.9 | 937856.2 | 316.91 |  | PT333 | SH | 668133.8 | 937757.1 | 318.991 |
| PT74 | BLD | 668120.4 | 937852 | 316.833 |  | PT334 | SH | 668134 | 937753.4 | 318.981 |
| PT75 | SH | 668116.8 | 937852.9 | 316.833 |  | PT335 | SH | 668134.5 | 937750.7 | 319.07 |
| PT76 | CV | 668119 | 937855.1 | 316.791 |  | PT336 | BLD | 668137.6 | 937750.4 | 319.035 |
| PT77 | CV | 668118.5 | 937855.2 | 316.819 |  | PT337 | SH | 668138.2 | 937752.8 | 319.024 |
| PT78 | SH | 668111.8 | 937851.5 | 317.396 |  | PT338 | BLD | 668140.6 | 937755.3 | 318.91 |
| PT79 | SH | 668115.7 | 937850.6 | 316.866 |  | PT339 | SH | 668137.7 | 937757.7 | 319.018 |
| PT80 | SH | 668118.3 | 937849 | 316.835 |  | PT340 | SH | 668136 | 937762 | 319.013 |
| PT81 | SH | 668118.1 | 937843.1 | 316.939 |  | PT341 | SH | 668138.3 | 937764.7 | 319.044 |
| PT82 | SH | 668113.8 | 937842.7 | 316.91 |  | PT342 | BLD | 668139.4 | 937765.7 | 319.051 |
| PT83 | SH | 668108.1 | 937843.3 | 317.4 |  | PT343 | SH | 668141.4 | 937749.2 | 319.111 |
| PT84 | SH | 668106.1 | 937847.2 | 317.641 |  | PT344 | SH | 668143.4 | 937751 | 319.05 |
| PT85 | SH | 668102.4 | 937849.2 | 317.563 |  | PT345 | SH | 668144.9 | 937753.8 | 318.948 |
| PT86 | SH | 668101.2 | 937843.6 | 317.586 |  | PT346 | SH | 668147.2 | 937751.6 | 318.691 |
| PT87 | SH | 668099.2 | 937840.3 | 317.489 |  | PT347 | SH | 668146.9 | 937747.9 | 319.135 |
| PT88 | SH | 668098.1 | 937836.4 | 317.051 |  | PT348 | SH | 668144.5 | 937745.9 | 319.098 |
| PT89 | SH | 668094.9 | 937829.9 | 317.405 |  | PT349 | SH | 668143.8 | 937741.8 | 319.252 |
| PT90 | SH | 668099 | 937830 | 317.366 |  | PT350 | BLD | 668137.8 | 937739.5 | 319.454 |
| PT91 | SH | 668102.2 | 937835.3 | 317 |  | PT351 | SH | 668140 | 937740.1 | 319.295 |
| PT92 | SH | 668093 | 937842.7 | 317.597 |  | PT352 | SH | 668141.4 | 937742 | 319.263 |
| PT93 | SH | 668090.5 | 937849.3 | 317.02 |  | PT353 | SH | 668145.1 | 937740.6 | 319.475 |
| PT94 | SH | 668088 | 937845.4 | 317.154 |  | PT354 | SH | 668147.6 | 937743.1 | 319.297 |
| PT95 | CV | 668089.1 | 937840.1 | 317.01 |  | PT355 | SH | 668149.8 | 937746.8 | 319.15 |
| PT96 | CV | 668089.5 | 937840.1 | 317.036 |  | PT356 | SH | 668151.4 | 937750.4 | 319.144 |
| PT97 | CV | 668089.7 | 937835.1 | 316.97 |  | PT357 | SH | 668153.9 | 937753.9 | 319.13 |
| PT98 | CV | 668089.1 | 937835.1 | 316.951 |  | PT358 | BLD | 668156.8 | 937753.8 | 319.16 |
| PT99 | SH | 668087.3 | 937837.4 | 317.032 |  | PT359 | BLD | 668155 | 937750.1 | 319.175 |
| PT100 | SH | 668084.9 | 937841.3 | 317.015 |  | PT360 | GT | 668156.7 | 937750.4 | 319.189 |
| PT101 | SH | 668081.1 | 937845.6 | 316.694 |  | PT361 | GT | 668156.4 | 937744 | 319.379 |
| PT102 | BLD | 668082.4 | 937849.5 | 316.774 |  | PT362 | BLD | 668151.3 | 937744.1 | 319.293 |
| PT103 | SH | 668079.2 | 937850.8 | 316.576 |  | PT363 | BLD | 668157.6 | 937720.9 | 319.595 |
| PT104 | SH | 668076.3 | 937851.1 | 316.606 |  | PT364 | BLD | 668075.3 | 937717.9 | 320.555 |
| PT105 | SH | 668076.4 | 937857.2 | 316.522 |  | PT365 | BLD | 668042.2 | 937720.6 | 320.353 |
| PT106 | SH | 668079.6 | 937859 | 316.481 |  | PT366 | BLD | 668036 | 937729.4 | 320.261 |
| PT107 | SH | 668082.3 | 937859.3 | 316.539 |  | PT367 | BLD | 668035.2 | 937744.9 | 319.623 |
| PT108 | SH | 668081.6 | 937865.3 | 316.444 |  | PT368 | BLD | 668011 | 937744.6 | 320.389 |
| PT109 | SH | 668078.7 | 937867 | 316.454 |  | PT369 | BLD | 668000.4 | 937749.6 | 319.55 |
| PT110 | SH | 668075.8 | 937866.3 | 316.351 |  | PT370 | BLD | 667993.6 | 937757.8 | 318.942 |
| PT111 | SH | 668074 | 937869.4 | 316.329 |  | PT371 | BLD | 667980.6 | 937772.8 | 318.59 |
| PT112 | SH | 668078.7 | 937870.4 | 316.547 |  | PT372 | BLD | 667974.3 | 937782.4 | 318.279 |
| PT113 | SH | 668081.1 | 937872.7 | 316.472 |  | PT373 | BLD | 667968.1 | 937789.6 | 318.03 |
| PT114 | SH | 668081.9 | 937877.2 | 316.469 |  | PT374 | RD | 667966.1 | 937793.6 | 317.734 |
| PT115 | SH | 668079 | 937877.3 | 316.472 |  | PT375 | CL | 667964.4 | 937795.8 | 317.839 |
| PT116 | SH | 668076.3 | 937876 | 316.604 |  | PT376 | RD | 667962.9 | 937797.2 | 317.658 |
| PT117 | EP | 668074.3 | 937876.1 | 316.594 |  | PT377 | SH | 667967.3 | 937798.1 | 317.599 |
| PT118 | TM | 668071.8 | 937879 | 316.342 |  | PT378 | SH | 667972.1 | 937795.5 | 317.821 |
| PT119 | TM | 668076.7 | 937878.8 | 316.445 |  | PT379 | SH | 667976.9 | 937798.1 | 317.559 |
| PT120 | TM | 668077.2 | 937885.8 | 316.536 |  | PT380 | SH | 667975.5 | 937802 | 317.229 |
| PT121 | SH | 668079.9 | 937885 | 316.362 |  | PT381 | SH | 667973.8 | 937805.3 | 317.111 |
| PT122 | SH | 668082.8 | 937883.4 | 316.48 |  | PT382 | SH | 667974.4 | 937809.3 | 317.087 |
| PT123 | SH | 668079.8 | 937880.6 | 316.437 |  | PT383 | SH | 667980.3 | 937808.6 | 317.008 |
| PT124 | SH | 668064.8 | 937896.9 | 315.429 |  | PT384 | SH | 667986.3 | 937808.3 | 317.055 |
| PT125 | BLD | 668064.8 | 937885.4 | 315.743 |  | PT385 | SH | 667989 | 937806.7 | 317.471 |
| PT126 | SH | 668069.7 | 937888.7 | 315.93 |  | PT386 | SH | 667991.4 | 937808.2 | 317.321 |
| PT127 | SH | 668069.8 | 937882.8 | 315.817 |  | PT387 | BLD | 667995.8 | 937809.1 | 317.181 |
| PT128 | EP | 668066.9 | 937881.4 | 316.367 |  | PT388 | BLD | 667996.6 | 937819.4 | 317.014 |
| PT129 | BLD | 668073.2 | 937873.9 | 316.28 |  | PT389 | BLD | 667997.1 | 937825.8 | 316.715 |
| PT130 | SH | 668076.5 | 937842.5 | 316.803 |  | PT390 | BLD | 667997.7 | 937836.1 | 316.436 |
| PT131 | SH | 668080.9 | 937837.3 | 316.882 |  | PT391 | SH | 667999.8 | 937837.9 | 316.489 |
| PT132 | SH | 668076.4 | 937833.4 | 317.117 |  | PT392 | SH | 668004.1 | 937837 | 316.697 |
| PT133 | SH | 668079.1 | 937829.3 | 317.088 |  | PT393 | SH | 668006.3 | 937835.7 | 316.836 |
| PT134 | SH | 668084.6 | 937829.3 | 317.095 |  | PT394 | SH | 668012.2 | 937839.5 | 317.101 |
| PT135 | SH | 668084.6 | 937825.2 | 317.166 |  | PT395 | BLD | 668024 | 937834.2 | 316.943 |
| PT136 | SH | 668080.9 | 937824.4 | 317.192 |  | PT396 | EP | 668028.4 | 937833.8 | 316.701 |
| PT137 | EP | 668075.2 | 937830.8 | 317.409 |  | PT397 | SH | 668027.3 | 937830.5 | 316.834 |
| PT138 | SH | 668076.5 | 937824.8 | 317.309 |  | PT398 | SH | 668024.8 | 937827.5 | 316.95 |
| PT139 | SH | 668080.1 | 937820.9 | 317.285 |  | PT399 | SH | 668027.4 | 937825.1 | 316.977 |
| PT140 | SH | 668083.9 | 937818.9 | 317.208 |  | PT400 | SH | 668025.7 | 937818 | 317.088 |
| PT141 | SH | 668084.3 | 937814.6 | 317.38 |  | PT401 | BLD | 668022.6 | 937817.1 | 317.238 |
| PT142 | SH | 668080.3 | 937812.9 | 317.483 |  | PT402 | SH | 668025.6 | 937813.3 | 317.258 |
| PT143 | SH | 668077 | 937811.8 | 317.722 |  | PT403 | SH | 668030.1 | 937809.1 | 317.516 |
| PT144 | EP | 668075.5 | 937812.1 | 317.622 |  | PT404 | BLD | 668031.1 | 937802.8 | 317.66 |
| PT145 | CV | 668075.9 | 937810.5 | 317.869 |  | PT405 | SH | 668027.5 | 937801.2 | 317.556 |
| PT146 | CV | 668076.6 | 937810.4 | 317.877 |  | PT406 | SH | 668031.4 | 937799.3 | 317.671 |
| PT147 | CV | 668076.1 | 937808 | 317.884 |  | PT407 | BLD | 668035 | 937801.7 | 318.075 |
| PT148 | CV | 668076.5 | 937808 | 317.896 |  | PT408 | SH | 668032 | 937800 | 317.586 |
| PT149 | BLD | 668073.4 | 937815.4 | 317.468 |  | PT409 | SH | 668031.4 | 937796 | 317.74 |
| PT150 | BLD | 668073.5 | 937803.4 | 317.824 |  | PT410 | SH | 668033.4 | 937793.9 | 317.725 |
| PT151 | SH | 668077.3 | 937804.7 | 317.737 |  | PT411 | BLD | 668035.2 | 937792.8 | 317.863 |
| PT152 | SH | 668080.4 | 937805.5 | 317.546 |  | PT412 | BLD | 668029.7 | 937792.2 | 318.048 |
| PT153 | SH | 668083.8 | 937807.7 | 317.514 |  | PT413 | BLD | 668029.5 | 937779.7 | 318.482 |
| PT154 | SH | 668084 | 937803.1 | 317.576 |  | PT414 | BLD | 668035.6 | 937839.2 | 316.76 |
| PT155 | SH | 668080.5 | 937801.9 | 317.645 |  | PT415 | BLD | 668030 | 937849.5 | 316.014 |
| PT156 | SH | 668076.8 | 937799.4 | 318.005 |  | PT416 | BLD | 668014.7 | 937849.8 | 315.771 |
| PT157 | SH | 668080.5 | 937797 | 317.729 |  | PT417 | RD | 668001.4 | 937850 | 315.553 |
| PT158 | SH | 668084.2 | 937798.5 | 317.584 |  | PT418 | RD | 667996.5 | 937852.3 | 315.646 |
| PT159 | SH | 668086.6 | 937795.5 | 317.625 |  | PT419 | BLD | 668015.4 | 937870.2 | 315.216 |
| PT160 | SH | 668088.9 | 937794.7 | 317.784 |  | PT420 | RD | 668022.4 | 937889 | 314.617 |
| PT161 | SH | 668087.8 | 937791.7 | 318.04 |  | PT421 | RD | 668020.3 | 937890.7 | 314.467 |
| PT162 | BLD | 668090.2 | 937790.5 | 318.057 |  | PT422 | BLD | 668035.6 | 937896.8 | 314.523 |
| PT163 | SH | 668086 | 937790 | 318.019 |  | PT423 | CV | 668032.5 | 937898.1 | 314.441 |
| PT164 | SH | 668079.9 | 937790.6 | 317.829 |  | PT424 | CV | 668032.1 | 937898.6 | 314.354 |
| PT165 | SH | 668076.8 | 937791.1 | 317.997 |  | PT425 | EP | 668026.8 | 937902.6 | 314.367 |
| PT166 | EP | 668075.9 | 937791.5 | 318.028 |  | PT426 | EP | 668042.4 | 937923.4 | 313.423 |
| PT167 | SH | 668081.3 | 937788.3 | 317.911 |  | PT427 | EP | 668045.7 | 937913.1 | 313.825 |
| PT168 | SH | 668086.2 | 937787 | 318.037 |  | PT428 | CV | 668045.7 | 937915.3 | 314.041 |
| PT169 | SH | 668088.3 | 937782.1 | 318.256 |  | PT429 | CV | 668045.2 | 937915.7 | 314.017 |
| PT170 | SH | 668083.1 | 937782.1 | 318.113 |  | PT430 | RD | 668050.4 | 937924.9 | 313.193 |
| PT171 | SH | 668077.2 | 937783.1 | 318.448 |  | PT431 | RD | 668048 | 937928.1 | 313.163 |
| PT172 | SH | 668077.1 | 937779.4 | 318.482 |  | PT432 | CL | 668049.6 | 937927.2 | 313.142 |
| PT173 | SH | 668080.8 | 937779.2 | 318.382 |  | PT433 | BLD | 668057.8 | 937924.9 | 313.443 |
| PT174 | SH | 668086.1 | 937777 | 318.527 |  | PT434 | BLD | 668139.5 | 937920.4 | 313.76 |
| PT175 | SH | 668088.4 | 937774.7 | 318.351 |  | PT435 | BD | 668156.3 | 937920.7 | 312.957 |
| PT176 | CV | 668076.7 | 937775.7 | 318.733 |  | PT436 | BLD | 668156.4 | 937893.6 | 315.143 |
| PT177 | CV | 668076.4 | 937775.7 | 318.755 |  | PT437 | BLD | 668120.5 | 937803.6 | 317.77 |
| PT178 | SH | 668075.6 | 937773.1 | 318.598 |  | PT438 | SH | 668121.8 | 937806.3 | 317.702 |
| PT179 | BLD | 668073.5 | 937768.6 | 318.708 |  | PT439 | BLD | 668120.3 | 937810 | 317.679 |
| PT180 | SH | 668073.1 | 937767.9 | 318.686 |  | PT440 | SH | 668124.2 | 937808.2 | 317.693 |
| PT181 | SH | 668075.1 | 937765.4 | 318.807 |  | PT441 | SH | 668124.2 | 937805.5 | 317.683 |
| PT182 | BLD | 668076.2 | 937763 | 318.99 |  | PT442 | SH | 668127.7 | 937808.3 | 317.623 |
| PT183 | SH | 668077.8 | 937762.3 | 318.92 |  | PT443 | SH | 668129.9 | 937805.3 | 317.555 |
| PT184 | SH | 668079.4 | 937758.5 | 319.226 |  | PT444 | SH | 668132.3 | 937808.2 | 317.557 |
| PT185 | SH | 668076.9 | 937758.6 | 318.98 |  | PT445 | SH | 668135.2 | 937805 | 317.569 |
| PT186 | SH | 668079.6 | 937754.5 | 319.302 |  | PT446 | SH | 668139.8 | 937808.7 | 317.523 |
| PT187 | SH | 668077.8 | 937753.6 | 319.218 |  | PT447 | SH | 668142.5 | 937806.5 | 317.477 |
| PT188 | SH | 668078.5 | 937746.8 | 319.408 |  | PT448 | SH | 668143.5 | 937805.6 | 317.641 |
| PT189 | SH | 668076.3 | 937744 | 319.51 |  | PT449 | BLD | 668152.8 | 937810.8 | 317.334 |
| PT190 | SH | 668079.7 | 937740.7 | 319.611 |  | PT450 | BLD | 668152.8 | 937803.9 | 317.388 |
| PT191 | SH | 668080.3 | 937743.2 | 319.569 |  | PT451 | BLD | 668120.4 | 937826.9 | 317.189 |
| PT192 | SH | 668078.8 | 937739.9 | 319.631 |  | PT452 | SH | 668123.3 | 937828 | 316.989 |
| PT193 | SH | 668078.8 | 937737.1 | 319.719 |  | PT453 | SH | 668126.8 | 937828.7 | 316.943 |
| PT194 | SH | 668081 | 937736.3 | 319.732 |  | PT454 | SH | 668126.3 | 937832 | 316.931 |
| PT195 | SH | 668078.1 | 937736.2 | 319.726 |  | PT455 | SH | 668123.9 | 937832.8 | 316.937 |
| PT196 | SH | 668074.3 | 937736.5 | 319.73 |  | PT456 | SH | 668121.8 | 937833.5 | 316.965 |
| PT197 | BLD | 668076 | 937739.9 | 319.718 |  | PT457 | SH | 668128 | 937833 | 316.904 |
| PT198 | SH | 668070.5 | 937735 | 319.725 |  | PT458 | SH | 668128.4 | 937836.4 | 316.889 |
| PT199 | SH | 668067.7 | 937732.6 | 319.769 |  | PT459 | SH | 668135.6 | 937832.3 | 316.921 |
| PT200 | SH | 668065.8 | 937735.6 | 319.673 |  | PT460 | SH | 668134.3 | 937834.4 | 316.926 |
| PT201 | SH | 668064.2 | 937739.6 | 319.579 |  | PT461 | SH | 668138.4 | 937829.2 | 316.911 |
| PT202 | SH | 668061 | 937736.3 | 319.632 |  | PT462 | SH | 668141.1 | 937835.5 | 316.628 |
| PT203 | SH | 668060.3 | 937732.6 | 319.705 |  | PT463 | SH | 668120.5 | 937838.7 | 317.002 |
| PT204 | SH | 668057.4 | 937735.8 | 319.652 |  | PT464 | BLD | 668073.3 | 937862.2 | 316.453 |
| PT205 | SH | 668056.3 | 937739.5 | 319.449 |  | PT465 | SH | 668072.4 | 937859 | 316.477 |
| PT206 | SH | 668052.5 | 937736.1 | 319.664 |  | PT466 | SH | 668069.5 | 937857 | 316.409 |
| PT207 | SH | 668050.9 | 937732.6 | 319.757 |  | PT467 | SH | 668065.8 | 937859.2 | 316.319 |
| PT208 | SH | 668048.6 | 937736.2 | 319.724 |  | PT468 | SH | 668064.3 | 937857.5 | 316.229 |
| PT209 | BLD | 668043.3 | 937739.9 | 319.659 |  | PT469 | SH | 668063.9 | 937853.2 | 316.483 |
| PT210 | BLD | 668043.6 | 937731.8 | 319.686 |  | PT470 | SH | 668055.7 | 937859.2 | 316.296 |
| PT211 | BLD | 668075.8 | 937729.8 | 319.966 |  | PT471 | SH | 668050.3 | 937854.7 | 316.365 |
| PT212 | BLD | 668079.9 | 937728.9 | 320.175 |  | PT472 | BLD | 668050.1 | 937852.9 | 316.654 |
| PT213 | BLD | 668079.7 | 937763.4 | 318.77 |  | PT473 | SH | 668056.8 | 937852.8 | 316.47 |
| PT214 | SH | 668080.8 | 937767.6 | 318.711 |  | PT474 | SH | 668063.8 | 937852.2 | 316.438 |
| PT215 | SH | 668083.6 | 937771.3 | 318.618 |  | PT475 | SH | 668067.5 | 937853.8 | 316.473 |
| PT216 | CV | 668088.4 | 937773.9 | 318.388 |  | PT476 | BLD | 668073.3 | 937851.7 | 316.674 |
| PT217 | CV | 668088.4 | 937774.1 | 318.401 |  | PT477 | BLD | 668073.4 | 937838.8 | 317.02 |
| PT218 | CV | 668088.9 | 937774.3 | 318.362 |  | PT478 | SH | 668071.2 | 937834.5 | 316.988 |
| PT219 | CV | 668089.2 | 937774.2 | 318.355 |  | PT479 | SH | 668071.7 | 937829.5 | 317.224 |
| PT220 | CV | 668089.7 | 937774 | 318.348 |  | PT480 | BLD | 668073.4 | 937827.2 | 317.205 |
| PT221 | CV | 668089.7 | 937773.6 | 318.358 |  | PT481 | SH | 668067.6 | 937828.1 | 317.111 |
| PT222 | SH | 668092.5 | 937776.3 | 318.551 |  | PT482 | SH | 668064.6 | 937832.8 | 317.034 |
| PT223 | SH | 668090.5 | 937779.5 | 318.338 |  | PT483 | SH | 668061.8 | 937837.4 | 316.978 |
| PT224 | SH | 668092.9 | 937780.6 | 318.233 |  | PT484 | SH | 668056.2 | 937836.7 | 316.98 |
| PT225 | SH | 668091.7 | 937784.1 | 318.136 |  | PT485 | SH | 668057.4 | 937831 | 317.048 |
| PT226 | SH | 668090.5 | 937787.6 | 318.117 |  | PT486 | SH | 668058.5 | 937827.6 | 317.15 |
| PT227 | SH | 668095.2 | 937788.3 | 318.215 |  | PT487 | SH | 668050.6 | 937828 | 316.962 |
| PT228 | SH | 668098.3 | 937788.8 | 318.206 |  | PT488 | SH | 668049.9 | 937833.6 | 316.97 |
| PT229 | SH | 668098.2 | 937786.1 | 318.204 |  | PT489 | SH | 668049 | 937837 | 316.957 |
| PT230 | SH | 668097.9 | 937782.4 | 318.229 |  | PT490 | BLD | 668043.9 | 937838 | 317.267 |
| PT231 | SH | 668097.4 | 937777.8 | 318.321 |  | PT491 | BLD | 668073.7 | 937792 | 318.073 |
| PT232 | SH | 668097.1 | 937773.1 | 318.529 |  | PT492 | SH | 668073.1 | 937784.7 | 318.318 |
| PT233 | SH | 668100.9 | 937773.3 | 318.58 |  | PT493 | BD | 668073.2 | 937780.3 | 318.439 |
| PT234 | SH | 668102.2 | 937777.1 | 318.319 |  | PT494 | SH | 668069.9 | 937784.4 | 318.355 |
| PT235 | SH | 668101.1 | 937781.3 | 318.285 |  | PT495 | SH | 668064.9 | 937790.3 | 318.076 |
| PT236 | SH | 668100.2 | 937785.2 | 318.239 |  | PT496 | SH | 668062.6 | 937784.4 | 318.262 |
| PT237 | SH | 668102.1 | 937788.4 | 318.071 |  | PT497 | SH | 668061.5 | 937781.1 | 318.253 |
| PT238 | BLD | 668103.3 | 937790.4 | 317.873 |  | PT498 | SH | 668054.5 | 937781.3 | 318.237 |
| PT239 | SH | 668105.1 | 937786.6 | 318.137 |  | PT499 | SH | 668053 | 937785.2 | 318.171 |
| PT240 | SH | 668105.9 | 937783.1 | 318.189 |  | PT500 | SH | 668051.4 | 937789.3 | 318.097 |
| PT241 | SH | 668106.2 | 937778.4 | 318.236 |  | PT501 | SH | 668045 | 937791.3 | 318.148 |
| PT242 | SH | 668106.3 | 937774.5 | 318.368 |  | PT502 | SH | 668044.7 | 937785.9 | 318.238 |
| PT243 | SH | 668106.4 | 937771.7 | 318.607 |  | PT503 | BD | 668043.5 | 937789.4 | 318.204 |
| PT244 | SH | 668109.8 | 937771.4 | 318.611 |  | PT504 | BD | 668046.8 | 937781.7 | 318.191 |
| PT245 | SH | 668114.7 | 937770.8 | 318.727 |  | PT505 | SH | 668050.5 | 937781.3 | 318.192 |
| PT246 | CV | 668118.6 | 937769.6 | 318.768 |  | PT506 | BLD | 668073.6 | 937768.4 | 318.52 |
| PT247 | CV | 668118.6 | 937769.2 | 318.875 |  | PT507 | SH | 668072.8 | 937765.9 | 318.772 |
| PT248 | BLD | 668120.4 | 937768.5 | 318.906 |  | PT508 | SH | 668068.1 | 937767.9 | 318.712 |
| PT249 | SH | 668118 | 937771.4 | 318.745 |  | PT509 | SH | 668064.3 | 937765.9 | 318.846 |
| PT250 | SH | 668114.6 | 937774.6 | 318.45 |  | PT510 | SH | 668061.8 | 937764.8 | 318.757 |
| PT251 | SH | 668112.3 | 937778.2 | 318.273 |  | PT511 | SH | 668059.2 | 937766.6 | 318.779 |
| PT252 | SH | 668116.9 | 937781 | 318.219 |  | PT512 | SH | 668056.8 | 937767.3 | 318.813 |
| PT253 | SH | 668114.7 | 937784 | 318.086 |  | PT513 | SH | 668055.6 | 937765.9 | 317.208 |
| PT254 | SH | 668110.9 | 937786.4 | 318.028 |  | PT514 | SH | 668054.7 | 937765.1 | 317.217 |
| PT255 | SH | 668108.3 | 937788.8 | 318.063 |  | PT515 | SH | 668050.4 | 937765 | 317.161 |
| PT256 | SH | 668106.5 | 937792.3 | 317.922 |  | PT516 | SH | 668048.2 | 937766.1 | 317.281 |
| PT257 | SH | 668104.3 | 937794.3 | 317.593 |  | PT517 | SH | 668047.6 | 937767.1 | 318.799 |
| PT258 | SH | 668106.6 | 937796.8 | 317.885 |  | PT518 | BLD | 668043.7 | 937764.7 | 322.083 |