A PROJECT

ON

DIGITAL SURFACE MODELING

(A CASE STUDY OF PART OF KWARA STATE POLYTECHNIC)

BY

TIAMIYU ZAINAB OLAITAN

HND/23/SGI/FT/0556

SUBMMITTED IN PARTIAL FUFILMENT OF THE REQUIREMENTS OF THE AWARD OF HIGHER NATIONAL DIPLOMAL (HND) IN SURVEYING AND GEO-INFORMATICS TO THE DEPARTMENT OF SURVEYING AND GEO-INFOMATICS,

KWARA STATE POLYTECHNIC, ILORIN KWARA STATE.

JUNE, 2025.

CERTIFICATE

I hereby certified that all the information given in this project were obtained as a result of observations and measurements made by me and that the survey was carried out in accordance with Survey Rules, Regulations and Departmental instructions.

NAME: TIAMIYU ZAINAB OLAITAN

MATRICNO.:HND/23/SGI/FT/056

SIGNATURE: ………………………………….

DATE OF COMPLETION: ……………………………………………….

CERTIFICATION

This is to certify that TIAMIYU ZAINAB OLAITAN with matriculation number HND/23/SGI/FT/056 has satisfactorily carried out this project under our instructions and direct supervision.

_______________________				_______________________

MR. ABIMBOLA I. ISAU				SIGNATURE AND DATE

(PROJECT SUPERVISOR)

_______________________				_______________________

SURV. AWOLEYE R. S  				SIGNATURE AND DATE

(PROJECT COODINATOR)

_______________________				________________________

MR. ABIMBOLA I. ISAU				SIGNATURE AND DATE

(HEAD OF DEPARTMENT)

_______________________      	 		_______________________

EXTERNAL MODERATOR  	     			SIGNATURE AND DATE

DEDICATION

I dedicate this project to Almighty God, whose guidance, strength, and blessings have been my constant source of inspiration and perseverance.
ACKNOWLEDGEMENTS

All adoration be to God Almighty, the gracious one who has given me the opportunity to complete my HND program in Surveying and Geo Informatics. My appreciation goes to my supervisor MR.ABIMBOLA.I.ISAU for his encouragement and motivation as regards the project thank you and God bless you sir.

To all my lecturers in the department of Surveying and Geo Informatics, Surv.R.O ASONIBARE, SURV.A.GAREMU, SURV.R.S AWOLEYE, SURV.A.O AKINYEDE, SURV.AYUBA ABDULSALAM, SURV.F.D DIRAN AND SURV. BABATUNDE KABIR, may God bless you all (Amen). I also express my sincere gratitude to my wonderful parents MRS. ADEBISI OLUWATOYIN their parental care and supports once the inception of my educational career, they stood by me when things were tough and rough for me, they gave me hope, I will forever be grateful for the motivation you always gave to me at all time, if I would have to have parents in my life again, I will pray to have people like you around me again and to all my brothers and sisters JOSHUA TIMOTHY, MARVELOUS.

To all my understanding friends and lovers, I so much appreciate your support and advice thanks so much, May almighty God bless you all (Amen).

ABSTRACT

The project was carried out to generate a Digital Surface Model for part of Kwara state polytechnic, Moro Local Government Area, Kwara State, Nigeria. The reconnaissance was first carried out, spatial data were acquired directly from the field using Total Station, while non-spatial data were obtained through social survey by means of oral interviews and observations. The data were processed and analyzed using Arc GIS 10.1 software for database design and geospatial analysis, AutoCAD2007 for graphical representation, surfer 10 for generating the contour as well as Digital Terrain Modelling (DTM) and other relevant software. Some queries were generated to enable spatial search on the database created and transform such dataset into information system which will support decision making process, as a result of this we produced Topographic data in softcopy to be accessible for various application. Finally, a comprehensive technical report was written.

TABLE OF CONTENTS

Cover page……………………………………………………………………….i

Certificate………………………………………………………………………..ii

Certification……………………………………………………………………..iii

Dedication……………………………………………………………………….iv

Acknowledgement…………………………………………………………….....v

Abstract………………………………………………………………………….vi

Table of contents ….…………………………………………………………..vii-ix

CHAPTER ONE

- Introduction……………………………………………………. ..1

1..1	Background to the study………………………………..……..….1

1.2	Statement of the problem……………………………...…...……..3

1.3	Aim……………………………………………………………….3

1.4	Objectives ………………………………………………………..4

1.5	Scope of the Project……………………………………………….4

1.6	Pesonnel……………………………………….……………..…....4

1.7	Study Area…………………………………………………………6

CHAPTER TWO

- Literature Review…………………………………………….…..7

CHAPTER THREE

## 3.0	Methodology……………………………..………………………….17

## 3.1	Project Planning …………………………..………………………...17

## 3.2	Equipment Used/ System Selection Software………………………19

## 3.3	Data Acquisition….…....………………………………………...…22

## 3.4	Physical Design ......………………………………………………....25

## 3.5	Database Creation………..…..……………………………………...27

## 3.6	Data Processing.............…………………………………..………...28

3.7	Database Management…………………..……………………….30

CHAPTER FOUR

4.0	Spatial Analysis And Information Presentation ……………..………31

4.1	Testing of Database…………………………………………………31

## 4.2	Spatial Query……………………………………………………..…32

## 4.3	Result Analysis……………………...………………..……………..46

4.4	Application of Project…………………………………….…………47

CHAPTER FIVE

5.0	Costing, Summary, Problem Encountered, Possible Solution,

Recommendation And Conclusion ………………………...……...…49

5.1	Costing……………………………………………..………….……49

5.2	Summary……………………………………………..………….……49

5.3	Problem Encountered and Possible Solution..……………………..….49

5.4	Recommendation……………………………………………………...49

5.5	Conclusion…………………………………………………...………..50

REFERENCES…….……………………………………………….….....…51

APPENDIX DATA……………………………..……….……………….….52

CHAPTER ONE

- INTRODUCTION

1.1	Background to Study

The Earth's surface exhibits a variety of forms, including mountains, foothills, basins, terraces, ditches, cuttings, and embankments, and its elevation varies from place to place. Despite these natural variations, humans have learned to adapt to living on Earth. Different professionals, such as civil engineers, geologists, geomorphologists, and geomatic engineers, interact with the Earth's surface in various ways. While their areas of focus and interests may differ, they share a common goal of accurately describing and representing the Earth's surface.

This desire to represent the Earth's surface has led to the development of digital surface modeling, digital elevation modeling, and digital terrain modeling. Initially, terrain models were physical representations made from materials like clay, sand, and plastic. However, since the 1950s, computers have been introduced into this field, enabling the numerical or digital modeling of the terrain surface, which has evolved into the discipline of digital terrain modeling. Digital terrain modeling is a process used to create accurate models of the land surface. These models have found extensive applications in various disciplines, including mapping, remote sensing, civil engineering, mining engineering, geology, hydrology, land planning, and communications. Consequently, digital terrain modeling has garnered increasing attention as a discipline.

A Digital Terrain Model (DTM) is a description of the relief of a terrain, enabling computer processing and facilitating more efficient and accurate solutions to problems related to the Earth's surface geometry. For example, when designing a road, the computer can determine the optimal road alignment if the terrain's digital representation is available. A DTM can be described as a three-dimensional representation of a terrain surface, consisting of X, Y, Z coordinates stored in digital form. It includes not only heights and elevations but also other geographical elements and natural features, such as rivers and ridge lines.

In practice, the terms DTM, DEM, and DSM are often used interchangeably, although they can sometimes refer to different products. Li (1990) conducted a comparative analysis highlighting some differences between these terms. Elevation refers to the height above a given level, such as sea level or the horizon. Terrain is a broader concept, encompassing a tract of country considered in terms of its natural features, among other aspects. As a result, the term DTM tends to have a wider meaning than DEM and may incorporate specific terrain features, such as rivers, ridge lines, and break lines, into the model. On the other hand, Digital Surface Models (DSMs) measure the height values of the first surface on the ground, including terrain features, buildings, vegetation, power lines, etc., providing a topographic model of the Earth's surface. A digital surface model represents the Earth's surface and includes all objects on it, making it a superset of both DTM and DEM. Therefore, DSMs cannot be discussed without mentioning DEM and DTM, as they are subsets of the broader DSM concept.

Digital Surface Models (DSMs) are often used to model the first reflective or visible surface, while DTMs typically refer to the bare earth without trees, buildings, and other natural or manmade features. DSMs and DTMs are common products derived from various sources, such as (digital) photogrammetric systems, optical satellite imagery through block adjustment, real-time kinematic GPS, topographic maps, total station surveys, and LIDAR mapping, among others. In modern map production, DSMs and DTMs serve as valuable information sources for scene analysis, change detection, GIS database updating, 3D feature extraction and reconstruction, inter-visibility calculations for optimizing telecommunication antenna locations, risk mapping, generating true ortho-images, mission planning and rehearsal, virtual and augmented reality applications, and urban planning. Consequently, research in this field contributes to a better1.2.

1.2	Statement of Problem

Kwara state polytechnic Ilorin is an institution which has been established and functioning, for years, the population is increasing every year and also increasing in the artificial features like buildings, roads, electric poles, trees and many more. It was discovered that there is no up to date digital surface plan/map of the study area, so there is need for digital surface map for future several purpose of the study area which may be used for polytechnic modeling, landscaping and also erosion control in some area.

The demand for Digital Surface Model for various needs and application is on the increase owing to the rapid physical changes and development taking place on different part of the earth surface, hence, there is need for the creation of 3D digital database for the topography. The major problem here is the lack of up-to-date topographic database, therefore demands for topographic information in the area are not met and this occurs due to the following problems.

- The information contained in the available maps are not enough and current for the school’s management use, meaning there is need for an up-to-date Digital Surface Model.

- Due to the new development on the proposed site, the current school map does not contain an up to date details, there by calling for an update.

- Lack of information about the height of the features in the school which is poor deformation studies of the features within the school premises and the study area.

1.3	Aim of the Project

This project is aimed to create an up to date Digital Surface Model of the part of Kwara state polytechnic Ilorin.

1.4	Objective of the Project

- Design and develop a comprehensive database for the study area, integrating spatial and attribute data.

- Conduct a ground survey using Total Station to acquire accurate geometric and attribute data.

- Create a spatial database that captures the study area's features and characteristics.

- Perform spatial analysis to extract insights and patterns from the collected data.

- Develop an effective information presentation and management system to facilitate data-driven decision-making.

1.5	Scope of the Project

- Field reconnaissance

- Office planning

- Boundary establishment

- Data acquisition (Both geometric and attribute)

- Data processing

- Data presentation

1.6	List of the Personnel

The following are the personnel involved in executing the project:

| S/N | NAMES | MATRIC/NO | POSITION |
| --- | --- | --- | --- |
| 1 | TIAMIYU ZAINAB OLAITAN | HND/23/SGI/FT/056 | Writer |
| 2 | ILESANMI ESTHER O. | HND/23/SGI/FT/055 | Member |
| 3 | BABALOLA OLUWATOSIN O. | HND/23/SGI/FT/059 | Member |
| 4 | ADEOYE AYOBAMI L. | HND/23/SGI/FT/063 | Member |
| 5 | SHUAIB OLOHUNTOYIN | HND/23/SGI/FT/064 | Member |
| 6 | OGUNLADE MATHEW K. | HND/23/SGI/FT/065 | Member |
| 7 | AYINLA ABDULWAHAB B. | HND/22/SGI/FT/094 | Member |
| 8 | GANIYU MISTURA D. | HND/22/SGI/FT/111 | Member |

1.7	Project Location

The project site is part of Kwara state polytechnic Ilorin fall on geographic coordinate of Latitude: 008 33 25.96; Longitude : 004 37 54.39 to Latitude: 008 33 24.78 ; Longitude: 004 38 05.73, covered total area of 5 hects.

Map of Kwara State

Map of Nigeria

Google Imagery of the Study Area

Figure 1.10: Study Area Ma

Imagery of Project Site

Showing the study area

FIG 1.2: Diagram Showing Location of Study Area.

p

CHARPTER TWO

2.1	LITERATURE REVIEW

Digital Surface Models (DSMs) offer the possibility of extracting the elevations of surface features to leave the ground surface DEM. For many applications in the urban environment this separation of above-surface feature information from ground information canoffer a useful combination of data sets. For example, detailed knowledge of the elevation of the ground surface is essential for predicting good inundation and the potential effects of sea level rise, whilst a detailed model of the man-made objects that would be affected is essential for property owners, planning authorities and insurance companies.

This project discusses several such applications and focuses on how Airborne Laser Scanning devices can provide digital surface models which can be used to separate surface features from the ground for modeling flood inundation from rivers in urban and semi-urban environments. DEMs can be constructed by digitizing existing topographic maps (Gao, 1995, 1997) or by using stereoscopic aerial photographs. With the advance of digital photogrammetry, DSMs can be created using stereo image matching techniques (Smith & Smith, 1996). Many authors (Abanmy, Khamees, Scarpace & Vonderohe, 1995; Jaafar & Priestnall, 1998) have considered the potential for these DSMs to provide the heights of surface features such as buildings. Recently LiDAR (Light Detection And Ranging) has become an established technique for deriving geometric information in three dimensions. The system is seen to offer a relatively quick technique for extracting accurate surface models and thus offers the potential for the creation of DEMs and other mapping products (Kost, Loddenkemper& Petring, 1996; Lohr, 1998). LiDAR is considered to offer several potential benefits for the creation of DSMs:

1.	Offers Cost

Effective way of producing DSMs with an accuracy in the order of decimeters  (Lohr, 1998), and is less prone than other remote measuring systems to difficulties in measurement due to variations in weather. 2. Offers precise definition of surface features through a very high density of recorded points, allowing the creation of gridded DSMs with cell resolutions of 1±3 m (Lohr, 1998). Automatic DSM generation using digital photogrammetry has problems with feature definition because of surface smoothing and the difficulty of controlling the image-matching algorithms (Smith, 1997; Smith & Waldram, 1996; Smith, Smith, Tragheim& Holt, 1997).

Reliable and accurate three-dimensional models of the urban environment are required by many applications and constructing these models has beenan activeresearch involving a diversity of approaches using remotely sensed data (Gruen, Baltsavias&Henricsson, 1997). A comparison of photogrammetric and Lidar (Baltsavias, 1999) considers these technologies not merely competitive but complementary and suggests that their close integration should be encouraged. Nevertheless, the production of DSM products using LiDAR is a quicker, more automated process and coupled with the high density of point measurements can offer greater definition of urban features, these factors encouraging research into the automated extraction and characterization of surface features.

The degree to which precisely defined surface features need to be extracted and categorized depends upon the application. The reconstruction and visualization of three-dimensional urban models, for example, requires precise definitions of building walls and roofs and much research effort has focused on their automated extraction (Gruen, Kubler &Aqouris, 1995). Typically, utilizing raw laser data or 1-m DSMs, buildings have been identified, removed and replaced with Computer Aided Design (CAD)-generated building objects, often with the aid of existing building outlines or through the use of additional optical information (Haala & Brenner, 1997, 1999). With CAD objects placed on the DEM surface, textures can be mapped onto individual faces of these objects giving the high levels of photorealism necessary for many urban planning visualization exercises. For land use change detection, the incorporation of building heights offers important extra information over and above that offered by optical sensors alone (Preinstall & Glover, 1998). Change detection between map vector outlines and building edges from newer imagery may be possible under certain conditions but processing can be complex. Due to the nature of gradient maps derived from the Lidar data, the identification of building edges becomes possible. Although not sensitive to lighting conditions, as are edges extracted from optical sensors, edge extraction from laser data is complicated by the presence of objects in close proximity to buildings such as parked vehicles and trees. Land use-change detection need not, however, rely upon the recognition of individual surface features. As the capture of laser data becomes more routine and time-series data sets become available, then differencemaps produced by subtracting successive DSMs should prove a successful means of urban change detection without the requirement to recognize features at every stage (Murakami, Nakagawa, Hasegawa, Shibata & Iwanami, 1999).

For certain applications the extraction of discrete geometrically precise objects is less important than the separation of the surface feature layer from the underlying ground elevations. Laser scanning can offer a fast, cost-effective alternative to conventional methods of producing DEMs from digitized contours or stereo plotters. Forflood hazard mapping over large areas ground DEMs derived from contour data are often used, but for more detailed modelling of flood inundation the use of LiDAR derived surface models can be considered. DEMs created from contour data are reliant upon a suitably small contour interval and a large number of surveyed spot heights in order to represent the ground surface faithfully. For floodplain mapping and modelling, laser scanning offers the chance to represent the ground surface at finer resolutions and, therefore, to predict the extent and depth of flood inundation more precisely (Marks & Bates, 2000). Gomes Pereira and Wicherson (1999) consider the potential of LiDAR for supplying topographic data for the management of alluvial zones. A simple simulation of river flooding using a LiDAR DSM where objects impacted by the flooding are clearly seen is shown in Fig. 2. Models of flood inundation can be one-dimensional where cross-sections extend across the floodplain from points along the river channel, each section being assigned a roughness coefficient. Where a one-dimensional model predicts significant floodplain show a two-dimensional model can be used to give more reliable estimates of flood depth and extent. The floodplain is represented as a triangulated surface where each triangle is assigned a roughness coefficient (such as Mannings n) which is typically estimated manually using field or photographic observations (Chow, 1973).

The representation of a more complete and precise coverage of surface roughness coefficients over the flood plain, coupled with a high resolution elevation surface, would offer improved data provision for this modelling process. LiDAR could provide such data if the surface features are removed to leave the ground elevation surface, and in so doing the spatial distribution of surface roughness could be estimated by summarising the texture of the features removed.

2.	Rationale

Attempts to separate surface features from the ground elevations work on the assumption that these features are higher than the surrounding surface (Jaafar &Preinstall, 1999; Weidner & Forstner, 1995). If the above-surface features can be identified and removed from the DSM, then the ground elevations for the gaps left by the removed features can be replaced by interpolating between DSM elevation values around the edge of each gap. To identify the above surface features a `reference surface' which is lower than the LiDAR DSM can be subtracted from the DSM, leaving a `residual surface representing the locations of all above-surface features. The areas defined in this residual surface can be used to mask out the equivalent areas in the DSM.

The final stage involves interpolation to fill in the gaps masked out of the DSM, resulting in the ground DEM. This study investigates the above procedure with particular emphasis on the creation of a reference surface using a filtering process to smooth the DSM surface. The sensitivity of the results to varying the filter size used was an important aspect of the study. Further discussion of this technique will be outlined in the Methodology section. Although the isolation of the above-surface features from the ground surface is paramount in this study, the classification of these features based on topographic and spectral character is discussed. The use of an Artificial Neural Network (ANN) to extract features based upon their topographic maps produced by subtracting successive DSMs should prove a successful means of urban change detection without the requirement to recognize features at every stage (Murakami, Nakagawa, Hasegawa, Shibata & Iwanami, 1999). For certain applications the extraction of discrete geometrically precise objects is less important than the separation of the surface feature layer from the underlying ground elevations. Laser scanning can offer a fast, cost-effective alternative to conventional methods of producing DEMs from digitized contours or stereo plotters. One application for which an accurate DEM is important is the modeling of flood inundation from rivers. For flood hazard mapping over large areas ground DEMs derived from contour data are often used, but for more detailed modeling of flood inundation the use of LiDAR derived surface models can be considered. DEMs created from contour data are reliant upon a suitably small contour interval and a large number of surveyed spot heights in order to represent the ground surface faithfully. For floodplain mapping and modeling, laser scanning offers the chance to represent the ground surface at finer resolutions and, therefore, to predict the extent and depth of flood inundation more precisely (Marks & Bates, 2000). Gomes Pereira and Wicherson (1999) consider the potential of LiDAR for supplying topographic data for the management of  alluvial zones.

A simple simulation of river flooding using a LiDAR DSM where objects impacted by the are clearly seen is shown in Fig. 2. Models of flood inundation can be one-dimensional where cross-sections extend across the floodplain from points along the river channel, each section being assigned a roughness coefficient. Where a one-dimensional model predicts significant floodplain show a two-dimensional model can be used to give more reliable estimates of flood depth and extent. The floodplain is represented as a triangulated surface where each triangle is assigned a roughness coefficient (such as Mannings n) which is typically estimated manually using field or photographic observations (Chow, 1973). The representation of a more complete and precise coverage of surface roughness coefficients over the flood plain, coupled with a high-resolution elevation surface, would offer improved data provision for this modeling process. LiDAR could provide such data if the surface features are removed to leave the ground elevation surface, and in so doing the spatial distribution of surface roughness could be estimated by summarizing the texture of the features removed.

3.	The data

LiDAR is based on sequential laser range measurements from an airborne sensor to points on the ground surface. Knowing the precise position and orientation of the airborne platform, from differential Global Positioning Systems and Inertial Navigation Systems, the laser beam is reflected o the ground surface to enable the three-dimensional positions of the surface points to be determined with decimeter accuracy (Hug, 1997; SoÈhne, Heinze, Hug &KaÈlberer, 1993). It is reported that an accuracy of the order of 0.2 m horizontally (x,y) and 0.1 m vertically (z) can be achieved in the production of DSMs (Lohr, 1998) from LiDAR depending on the system used. The nature of the surface models produced allows surface features to be distinctly defined and, consequently, the identification and extraction of these features both as discrete objects and in terms of roughness coefficients becomes a reality.

In this study, the LiDAR data used are those produced by the Environment Agency and the study area is an area of the Trent floodplain at Newark-on-Trent, Nottinghamshire, UK. The study area consists of an undulating topography that includes man-made objects and natural features such as trees. The spacing of the surveyed points is approximately 2.5 m and the final gridded elevation matrices used for analysis have a spatial resolution of 2 m. The DSMs are referenced to the Ordnance Survey of Great Britain (OSGB36) and the heights are transformed to Ordnance Survey Datum Newlyn (OSDN). In the data-acquisition phase, the swath angle used is ‹19 and the flying height approximately 700 m. The precise large-scale geometry of building objects as represented in the 2-m DSM (as illustrated in Fig. 1) cannot be extracted reliably but the surface texture is sufficiently well defined to allow the filtering procedure described in Section 2 to be implemented. Although the present study uses the surface topography alone, the availability of multi-spectral remotely sensed imagery offers additional potential for discriminating trees from buildings (Haala & Brenner, 1999) and is the focus of ongoing investigations.

4.	Methodology

The task is to separate surface features from ground elevations but the specific techniques very much depending upon the requirements of the application. For example, in the case of three-dimensional urban model reconstruction and mapping it may be appropriate to extract and generalize edges from the gradient image in a similar way to image edges in previous studies (Preinstall & Glover, 1998). Generalized edge features can be grouped to form a candidate building object, supported by height information `internal to', and `external to' the building boundary. Techniques such as this rely on high-resolution laser scanning data using either raw point measurements or gridded DSMs with a resolution of around 1 m. For example, Maas and Vosselman (1999) present algorithms to segment individual roof faces of buildings based upon clusters of common aspect values present in the triangulated data set.

The approach taken by the current study does not attempt to extract building geometry but focuses upon extracting the whole surface feature layer with a view to characterizing the features present. Several approaches to `stripping' this surface feature layer have been investigated. The Environment Agency, Bath, UK, have explored the potential of using 33 variance filters to identify areas above a certain threshold limit equivalent to a variance of 2 m (or 66.gradient). These areas are buffered and the resulting mask is used to remove areas of the DSM which are replaced by re-interpolating elevations across the gaps. This study presents a modified approach for isolating surface features. Working on the assumption that man-made objects and natural features stand above the surrounding surface a lower reference surface can be subtracted from the DSM, leaving a residual surface representing all surface features (Jaafar & Priestnall, 1998, 1999; Weidner & Forstner, 1995).

An important issue at this point is the method used to create the reference surface. In the first part of the investigation, the effects of filtering on the LiDAR DSM using a mean filter of varying size are investigated with a view to deriving the most appropriate smoothed `reference' surface. The standard deviations (s) for the derived surfaces are then computed for comparative purposes. The main aim of this filter exercise is to understand the smoothing effects of the DSM in the creation of the reference surface. As would be expected the value for s decreases with respect to the field size used, Choosing an appropriate filter size based upon this effect alone would be difficult. In order to assist the choice of filter size the effect must be understood with respect to the features on the DSM being smoothed. To achieve this an unsupervised classification of the DSM elevation data is carried out. This process of thresholding results in a pre-defined number of clusters, in this case five clusters being assigned to ensure the separation of the three main entities (the terrain surface, buildings and trees). The choice of the number of clusters varies according to the range of elevation values in the DSM and the number of entities to be differentiated. In order to understand the effect of various sizes of mean filters on the smoothing of features on the LiDAR DSM, well-defined regions generated by the unsupervised classification that represent trees, buildings and bare land are randomly identified.

The reference surface' which is lower than the LiDAR DSM could be created using any mean filter with window sizes between 1111 and 2525. In this study the 1111 mean filter is applied to the LiDAR DSM to generate the `reference sur- face'. The `reference surface' is then subtracted from the LiDAR DSM to isolate the above-surface features. As a result of the subtraction, the above-surface features are revealed as scattered regions (Jaafar & Priestnall, 1999) with positive elevation values. However, it should be pointed out that the whole area corresponding to a particular above-surface feature is not necessarily revealed after the subtraction.

Remotely sensed data obtained from satellites and airborne platforms are useful in providing an understanding of the distribution of damage due to natural disasters (Yamazaki 2001). The platforms and sensors of remote sensing should be selected with regards to the coverage and resolution required and the urgency, weather and time conditions. Recently, the digital aerial imaging system has become widely used (Hinz 1999; Leberl and Gruber 2005). The imaging system saves time associated with analog imagery processes and provides more precise and better-quality aerial images. In addition, the exterior orientations of airborne image can be determined by integrating GPS (Global Positioning System) and IMU (Inertial Measurement Unit) observations made during the mapping flight. Hence, it is expected that the digital surface model (DSM) can be constructed with good accuracy using the images captured by the imaging system. Aerial and satellite images are employed to detect collapsed buildings due to an earthquake.

Mitomi et al. (2001) tried to reveal building debris automatically using images captured by a high-definition television camera. Saito et al. (2004) performed visual damage inspection using the post-event IKONOS image, in which individual buildings can be identified, and pre-event other satellite images for the 2001 Gujarat, India earthquake. QuickBird images with the maximum spatial resolution of 0.6 m were also employed to detect collapsed buildings after the 2003 Bam, Iran, earthquake (Yamazaki et al. 2005). The object-based classification technique using digital aerial images was applied to access damaged buildings in the 2007 Niigata Chuetsu-oki earthquake (Suzuki et al. 2010). So far, remotely sensed images might be effective to detect earthquake induced damages. Since we can obtain only the top view of the affected area, some of the damaged buildings are difficult to reveal. Figure 1 shows an example of the aerial image captured after the 2007 Niigata Chuetsu-oki earthquake. A two-story building with the collapsed ground floor was found from the field photo. The building could not be pointed out from the aerial image because no damage was observed on its roof.

The Niigata Chuetsu-oki earthquake occurred on July 16, 2007 with a JMA (Japan Meteorological Agency) magnitude of 6.8 (JMA, 2007). Severe ground shaking was observed near the epicenter and the JMA seismic intensity of 6+ was recorded in Kashiwazaki city, Niigata Prefecture, Japan. The number of collapsed buildings is 1,121 and that of casualties is 14 in the affected area. The digital aerial image was captured for Kashiwazaki city on April 27, 2007, about three months before the earthquake. The digital aerial camera, UltraCamD, was employed to obtain the aerial images. The operated digital airborne imaging system was integrated with in-flight control systems consisting of GPS and inertial measurement units (IMU). The altitude of aircraft during capturing was about 1970 m and the spatial resolution of the aerial images is about 0.17 m. Performing aerial triangulation using the GPS/IMU dataset for the pre-event image with ground control points, the exterior orientations of airborne images can be determined accurately (Smith et al. 2005). The exterior orientations mean the three dimensional position and the three rotational angles of digital camera. The relative orientation of a stereo-pair of digital aerial images was conducted by introducing pass points, which are used as control points to match the two images, to reveal the accuracy of the exterior orientations.

Due to the limitation of the automated method, the shape of buildings could not be extracted so accurately. Hence, the breaklines (Briese 2004), which describe the discontinuities of the elevations in 3D model, were developed from the pre-event DSM. After the elevations were assigned to the both ends of breaklines, the 3D model for buildings were constructed based on the pre-event image.

# CHAPTER THREE

## 3.0	METHODOLOGY

## 3.1	PROJECT PLANNING

# Before beginning any survey assignment, project planning is a crucial requirement. It entails a careful investigation of the topic matter about a certain land region that is being evaluated. This crucial step in the surveying process allows the surveyor to become acquainted with the chosen location before starting work. During the reconnaissance phase, the survey's goals, requirements, and required precision levels are examined because these elements have a big impact on the choice of survey tools and techniques. In the end, this project resulted in the development of a first working diagram. Reconnaissance is divided into two phases: (i) Office Planning and (ii) Field reconnaissance.

### 3.1.1	 FIELD RECONNAISSANCE

# As part of this process, a surveyor or student must physically visit the site to gain a thorough understanding of its topography, including any vegetation, accessibility, existing frame work, as well as the social and economic 19 activities that take place there. The optimal approach and tools for improved decision-making must also be chosen. This on-site visit gives the surveyor or student a comprehensive grasp of the entire region and allows them to look at the entire area of the land. To guarantee correct orientation, the locations of all accessible horizontal and vertical controls in the region of the research area will be visited, and their outlooks will be viewed. This exercise makes it easier to create a Recce diagram, which is shown below but is not drawn to scale:

Figure 3.4: RECCI DIAGRAM

### 3.1.2	 OFFICE PLANNING

# Gathering crucial project information is the focus of this phase, which is usually done in an office setting. It is the initial phase of planning that comes before the fieldwork itself and entails gathering and analyzing a variety of project-related data. This covers a range of requirements for the project, scheduling, budgeting, and generating the coordinates for the control points under the project control prefix. It includes figuring out the best process, picking the right tools, as well as subscription for the CORS station.

## 3.2 	EQUIPMENT USED/ SYSTEM SELECTION SOFTWARE

The following equipment was used:

- Total station and accessories

- One Tripod

- Two Prism

- One 7.5 meters short tape

- One Plumb bob

- One Cutlass

- Bottle corks and nails

- Writing materials.

### 3.2.1 	HARDWARE USED

Model: 			HP 15 Note book PC

System Manufacturer:		Hewlett-Packard

Rating: 			1.0 Windows Experience Index.

Processor: 			Intel(R) Core(TM) i5-3230M CPU @ 2.60GHz, 2601 Mhz, 2 Core(s), 4 Logical Processor(s)

Installed Memory RAM: 	8.00 GB

System Type: 			x64-based PC.

Product ID: 			F9F84EA#ABV

### 3.2.2 	SOFTWARE USED

Some of the software used includes:

- AutoCAD 2007

- Notepad

- Microsoft Word 2007

- Microsoft Excel 2007

- Arc GIS 10.5

## 3.3	 DATA ACQUISITION

# This involves using different surveying methods and tools to gather data and make observations in the field. To acquire data for this project, Handed GPS and Total station were both used.

# 3.3.1 	PRIMARY DATA SOURCE: This is a type of data source in which data is direct acquired from the field such as data acquired from field survey operations.

# 3.3.2 	SECONDARY DATA SOURCE: This data source are already made data or gotten from research work, which has been acquired for future use. This involves downloading satellite imagery used for the project area and Google map as guide.

## 3.4	TEST OF INSTRUMENT

Testing of instruments is very crucial when carrying out any survey operation. Having collected the required instruments from the department, the check was carried out on the accuracy and precision of both the Handed GPS and TOTAL STATION to ensure they were in proper working condition before the actual field observation of the project.

### 3.4.1	 TEST FOR TOTAL STATION

The horizontal collimation and vertical index inaccuracies of the instrument were confirmed. Once the device was positioned on a rather flat surface, the required temporary modifications, including focusing, centering, and setting, were made. Configuration information, such as extra parameters and fast settings, was examined and adjusted as necessary. The configuration parameters selected collimation, namely horizontal collimation. Next, to focus on a distant object, the means' option located on the left side of the device was chosen. The instrument was transited, and the right side went through the same procedure. Then, when the 'accept' button was pushed, the table that was displayed showed the old and new values for the vertical index errors and horizontal collimation. Re-press "ok" signified acceptance of the updated values, which were minimal.

Figure 3.5: INSTRUMENT TEST

Top of Form

The diagrammatic representation of the procedure and the readings obtained are shown below

Table 3.4.1 COLLIMATION TEST OF TOTAL STATION

| STN | SIGHT | FACE | HCR | REDUCTION | VCR | REDUCTION |
| --- | --- | --- | --- | --- | --- | --- |
| A | B | L | 126º 07’ 11.33” |  | 67º 22’40” |  |
|  | B | R | 306º 07’ 31.3” | 180º 00’ 20” | 292º 37’ 1” | 359 º 59’ 41” |

HORIZONTAL COLLIMATION ERROR = 180°00’20’’ - 180°

=00°00’20’’

=00°00’20’’/2   =00°00’10’’

VERTICAL INDEX ERROR=360° - 359°59’41.6’’

=00°00’18.4’’

=00°00’18’’/2   =00°00’9.2’’

The instrument's satisfactory operation was validated by the acceptance of the updated readings. It appears from this result that any possible issues can be resolved or at least greatly reduced.

### 3.4.2 REFERENCING AND ORIENTATION OF TOTAL STATION

Tripod Setup:

- Place the tripod over the station point (known or assumed).

- Level the tripod head as much as possible by adjusting the legs.

Mount the Total Station:

- Fix the total station onto the tripod head.

- Ensure it’s securely locked in place.

Centering and Leveling:

- Use the optical plummet or laser plummet to center the instrument exactly over the survey station mark (station point).

- Level the instrument using the foot screws and circular bubble.

- Fine-tune leveling using the electronic level display.

Power On and Instrument Configuration

- Switch on the Total Station.

- Set the correct project, coordinate system, and units.

- Select the appropriate survey mode (e.g., topographic survey or station setup).

Orientation (Station Setup)

Orientation means establishing the direction of the horizontal angle reference (back sight direction).

Orientation by Known Coordinates (Resection or Free Station)

Used when both the instrument station and a back sight point have known coordinates.

- Input the coordinates of the instrument station (or let the instrument compute it later).

- Sight and measure to one or more known reference points (minimum of 2 for better accuracy).

- The Total Station calculates the instrument position and orientation using these known points.

### 3.3.7 PERIMETER TRAVERSING

The perimeter of the project area was traversed for the purpose of the coordinating the boundary points. HandedGPS (GALAXY G1) was connected to kwara state CORS station for referencing and orientation after which all the boundary points were traversed using HandedGPS from the initial until each successive boundary is occupied.

### 3.3.8	DETAILING

All features such as roads, buildings, electric poles, street light, trees, etc. were captured by coordinating their edges simultaneously as the perimeter is being measured within the project area. Buildings were measured and identifiers (BD) were used to differentiate them from other features. Roads were also measured and labelled (RD) accordingly. Data captured here were basically in 3Dimension i.e. Easting, Nothings and Elevations (X, Y, and Z) coordinates.

### 3.3.9	REMOTE ELEVATION MEASUREMENT

All the features such as buildings, electric poles, street light, etc. captured by coordinating their edges from a subsidiary traverse stations created within the project area. Buildings heights were measured and recorded. Electric poles height were also measured and recorded accordingly. So also the same was repeated for the trees. Data captured here were basically in 3Dimension i.e. Easting, Nothings and Elevations + Heights (X, Y and Z + H) coordinates. The following procedures were followed:

- Escape key was pressed then F1 for MENU

- The REM programme was selected.

- Observation was made again and the height of reflector was displayed.

- The telescope was tilted to the top of the object and the height keeps changing until F4 is been pressed to stop.

- The value was recorded.

Figure 3.3.9: REM OBSERVATION STRATEGY

### 3.3.9A 	SPOT HEIGHTINGS

This was carried out by occupying randomly the subsidiary points within the project area and taking measurements (Easting, Nothings and Heights) along the terrain at approximate intervals to determine the difference in height between points and some areas within the project location. Spot heighten is very important in producing topographical information of the area with the aid of ArcGIS 10.5.

## 3.4	PHYSICAL DESIGN

At this stage, all geospatial (attribute) data were structured and organized to form a database in a format acceptable by the implementing software and hardware. This was done in a way that, stored information can be accessed and retrieved at any time. Also, for regular updates when the need arise and to allow for analytical functions to be carried out answering generic questions. Also, it provided integrity and security that must be obeyed before such data could be accepted into the records. These virtues are as show in the tables below. ArcGIS 10.5 was used for database creation of the project area.

Table 3.4A TREE TABLE

| ATTRIBUTE | DATA TYPE | WIDTH | DECIMAL |
| --- | --- | --- | --- |
| T_ ID | String | 5 | - |
| T_NAME | Float | 20 | - |
| EASTING | Number | 10 | 3 |
| NORTHING | Number | 10 | 3 |
| T_USE | Float | 10 | - |
| T_HEGHT | Number | 7 | 3 |

Table 3.4B ROAD TABLE

| ATTRIBUTE | DATA TYPE | WIDTH | DECIMAL |
| --- | --- | --- | --- |
| POINT_ID | String | 10 | - |
| NORTHING | Number | 10 | 3 |
| EASTING | Number | 10 | 3 |
| RD_HEIGHT | Number | 7 | 3 |

Table 3.4C ELECTRIC POLE TABLE

| ATTRIBUTE NAME | DESCRIPTION OF ATTRIBUTE | DATA TYPE | WIDTH | DECIMAL |
| --- | --- | --- | --- | --- |
| POINT_ID | Electric pole identifier | Number | 6 | - |
| NORTHING | Coordinate | Number | 15 | 3 |
| EASTING | Coordinate | Number | 15 | 3 |
| EP_TYPE | Type of Electric pole | Text | 15 | - |
| EP_HEIGHT | Height of the Electric pole | Number | 5 | 2 |

Table 3.4D BUILDING TABLE

| ATTRIBUTE | DATA TYPE | WIDTH | DECIMAL |
| --- | --- | --- | --- |
| P_ID | String | 15 | - |
| NAME | Text | 15 | - |
| CONDITION | Text | 15 | - |
| USE | Text | 15 | - |
| HEIGHT | Number | 15 | 2 |
| AREA_METERS | Number | 15 | 2 |

Table 3.4E STREETLIGHT TABLE

| ATTRIBUTE NAME | DESCRIPTION OF ATTRIBUTE | DATA TYPE | WIDTH | DECIMAL |
| --- | --- | --- | --- | --- |
| POINT_ID | Street light identifier | Number | 6 | - |
| NORTHING | Coordinate | Number | 15 | 3 |
| EASTING | Coordinate | Number | 15 | 3 |
| SL_HEIGHT | Height of the street light | Number | 5 | 2 |

## 3.5	DATABASE CREATION

Database is an organized integrated collection of data stored so as to be capable of use by relevant application with data being accessed by different logical part. This was the construction phase where database was created. After the table has being populated via the keyboard, some attributes such as area, perimeters of parcel were automatically displayed by special command in the ArcGIS 10.5 version.

Figure 3.5: ABSTRACTED OF TABLE CREATED

### 3.5.1 ATTRIBUTE DATA ACQUISITION

The data about the characteristics of features especially man-made features were acquired through oral interview method and online source (Google Earth). Social survey was carried out during which information such as; building name, building use and related data about features were obtained from the students and staffs within the school. All these data formed the attribute aspect of the feature necessary for the database creation.

## 3.6	DATA PROCESSING

### 3.6.1	TOTAL STATION DATA DOWNLOADING PROCEDURE

Data downloading is the process of transferring data from the instrument into the computer system connected through the connection cable or wireless such as Bluetooth with the help of software in the computer, the process is as follows as performed on Total station.

- Connection of the two devices (computer and the instrument).

- Go to project working folder created on the instrument.

- Click on export.

- Input the name of file

- Indicate the file extension for the output

- Permit the other device to receive the file.

### 3.6.2 DRAFTING AND PLOTTING

The script file created containing the refined coordinates was then opened in the AutoCAD environment and the boundary of the project area was plotted using AutoCAD 2007.  This was followed by the details, which included buildings, roads, trees, sport facilities and electric poles.

Figure 3.6.2: AUTOCAD PLOTTING.

## 3.7 	DATABASE MANAGEMENT

### 3.7.1 	DATA SECURITY

This is the security measure used the protection of the database security is very important since it is vital for data integrity. The strategic measure used included controlling access to database by use of password.

### 3.7.2 DATA INTEGRITY

This ensures that data in the database is accurate and that cases of violation of integrity can be detected automatically by the system. Here, care was taken while entering data into the system, such that, accurate data were entered and updating could be done accurately without tampering with existing data. Backups are provided to cater for any loss of data through system failure. Also a member of the project group was appointed for quality assurance and checks therefor do checks the consistency, accuracy and integrity of the input data.

### 3.8.3 DATABASE MAINTANANCE

Having created the database, proper maintenance practice was made to meet its stated objectives. The ability to include more data and remove irrelevant data can be possible. There is every need for the data to be updated regularly because of the physical changes that tends to occur on the landscape with time, even as a result of development.

Both security and integrity were also exercised to ensure maintenance and to meet its stated objectives.

# CHAPTER FOUR

## 4.0	SPATIAL ANALYSIS AND INFORMATION PRESENTATION

## 4.1     TESTING OF DATABASE

This is the test carried out to determine whether the relationship between the spatial data of a feature and their attributes is capable of being retrieved. This is necessary to ascertain the quality of data in the database, its reliability to satisfy the demand of the user. The spatial data were arranged logically in an organized manner with respect to their attributes. This was carried out by designing a sample query and running the query to see if the desired result would be achieved. In this project, the database was queried to show different categories of information levels in the project area. Hence the database was confirmed fit for analysis. A sample of the attribute table in the database design can be found on the next page.

Figure 4.1: SAMPLE OF ATTRIBUTE TABLE

## 4.2	SPATIAL QUERY

Queries were designed for the purpose of retrieving information from the database. The queries performed in this project gave answers to certain generic questions asked from the database. This was made possible as a result of the implicit link of both the spatial and attributes data. The queries were based on the products from the analysis carried out on the database.

### 4.2.1 SINGLE CRITERIA QUERY

A single criterion is carried out where one condition is used to design query. This condition is used to retrieve the information from the database.

QUERY ANALYSIS 1 (selection by attribute)

ANALYSIS ONE

Analysis Name: Database extraction

Analysis Type: Single criteria analysis

Syntax: Query analyses showing buildings that are used for Administrative.

Figure 4.2.1.1 RESULT OF QUERY: (BLD_FUNCTION = “ADMINISTRATIVE”).

Analysis Name: Database extraction

Analysis Type: Single criteria analysis

Syntax: “BLD_ Function” =”Agricultural”

Figure 4.2.1.2: RESULT OF QUERY: (BLD_FUNCTION = “AGRICULTURAL”).

ANALYSIS THREE

Analysis Name: Database extraction

Analysis Type: Single criteria analysis

Syntax: “BLD_ CODITION” =”ACADEMIC”

Figure 4.2.1.3: QUERY BLD_FUNCTION"ACADEMIC"

ANALYSIS FOUR

Analysis Name: Database extraction

Analysis Type: Single criteria analysis

Syntax: “TREE_FUNCTION= ORNAMENT”

Figure 4.2.1.4: RESULT OF QUERY: (TREE_FUNCTION “ORNAMENT”).

### 4.2.2	MULTIPLE CRITERIA

This is a situation where two or more conditions are used to design a query. The conditions determine the information that may be requested by the user from the database.

Query for QUERY: TREE_FUNCTION “ORNAMENT” AND TREE_TYPE “HARD”

ANALYSIS FOUR

Analysis Name: Database extraction

Analysis Type: Multiple criteria analysis

Figure 4.2.2.1 RESULT OF QUERY: TREE_FUNCTION “ORNAMENT” AND TREE_TYPE “HARD”

Query for BLD_FUNCTION =“RELIGIOUS” AND BLD_STATUS= “AVERAGE”

ANALYSIS FOUR

Analysis Name: Database extraction

Analysis Type: Multiple criteria analysis

Figure 4.2.2.2 RESULT OF QUERY: BLD_FUNCTION =“RELIGIOUS” AND BLD_STATUS= “AVERAGE”

## 4.2	SPATIAL ANALIYSIS

This makes Geographic Information System (GIS) a unique analytical tool or technique because of its ability to combine spatial attribute data to produce information relevant for decision making. The basic GIS analysis performed in this project includes overlay and topographic operations. Maps on various themes were overlaid since they were from the same geometric origin.

### 4.2.1	OVERLAY OPERATION

This overlay operation is the presentation that shows different features within the project area. The outcome of overlay operation could be composite plan if it shows all the existing features, and boundary.

Figure 4.2.1: COMPOSITE MAP OF THE PROJECT AREA.

### 4.2.2	TOPOGRAPHICAL OPERATIONS

Topographic operations were carried out to determine the surface characteristic of the area which showed the relief of the project. These operations were performed on the Digital Elevation Model (DEM) generated from the use of relevant software, in this case, ArcGIS 10.5 and ArcScene 10.5 was used. The following maps were produced from the topographic operations and analysis:

- TRIANGULATION IRREGULAR NETWORK (TIN): this shows series of triangulated surface automatically generated from ArcGIS 10.5 using 3D analysis tools. This is applicable in finding suitable place for building or any engineering construction

Figure 4.2.2.1: TIN AND DETAILS OVERLAY MAP OF THE PROJECT AREA

- SLOPE MAP: A slope map represents the gradient or steepness of the terrain's surface. It calculates the rate of change in elevation between neighboring pixels or points on a digital elevation model (DEM). Typically, slope is measured in degrees or percentage. Steeper slopes are represented by higher values, while flatter areas have lower values.

Figure 4.2.2.2: SLOPE MAP OF THE PROJECT AREA

- THE ASPECT MAP:

This shows the direction of the surface faces. It is used to determine how much sun a hill received and the information can be used to put building where they will get enough sun. Also the aspect map shows the suitable place for plane landing i.e. the flat terrain area and also solar illumination which can affect the diversity of life i.e. Vegetation and soil studies; glaciology.

Figure 4.2.2.3: ASPECT MAP OF THE PROJECT AREA.

- THE CONTOUR MAP:

Contour maps display lines connecting points of equal elevation on the terrain surface. These lines, called contour lines, depict the shape and elevation of the land. Contour maps are widely used in topographic mapping and land surveying for visualizing elevation changes and terrain features. This provides a very intuitive view of the landscape. Contour map of the project area can aid in giving information on the highest and the lowest point on the terrain. This is important in road construction and location of other facilities like storage tanks for water distribution.

Figure 4.2.2.4: CONTOUR MAP OF THE PROJECT AREA.

- FLOW DIRECTION MAP:

This is used to determine the direction of flow of water within the project area. It also indicates the magnitude and erosive force water passing through an area.

Figure 4.2.2.5 FLOW_DIRECTION MAP OF THE PROJECT AREA

- DIGITAL SURFACE MODEL MAP:

A DSM (Digital Surface Model) map, or Digital Surface Model, represents the elevation of the Earth's surface including all objects on it, such as buildings, vegetation, and other structures. Unlike a DTM (Digital Terrain Model), which only includes the bare-earth surface, a DSM incorporates both natural and man-made features. DSMs are useful for various applications, including urban planning, infrastructure design, and 3D visualization. They are often derived from aerial or satellite imagery, LiDAR data, or photogrammetry techniques. This shows the reality of the terrain of the project area.

Figure 4.2.2.6: DIGITAL SURFACE MAP OF THE PROJECT AREA

## 4.3	RESULT ANALYSIS

From the below pie chart, it is inferred that large percentage is serving academic function.The DSM project commenced with the utilization of a Total station referenced to control station within kwara state polytechnic to capture 3D data of the project area. The captured data were then processed using software, employing photogrammetry techniques to generate a high-precision DSM and create a detailed 3D model. Subsequently, ArcGIS and ArcScene software was employed to visualize and analyze the DSM data, facilitating spatial analysis and information presentation both in 2D and 3D respectively.

Figure 4.2.2.7: LAND USE PIE CHART

## 4.4	APPLICATION OF PROJECT

- Generally DSM is applicable in urban development modeling.

- It enhanced visualization and communication of such data not only in maps modeling but also in mobile digital devices for quick navigation and assessment.

- To create a sustainable and favourable living environment.

- Positioning and monitoring of physical features, structures and engineering works on, above or below the surface of the earth to protect properties or investment.

- The feasibility studies through the use of line of sight analysis provide visibility between two points. This is good for network distribution in telecommunication.

- The profile graph is good road construction which will provide the cross section of the topography from which rivers, streams and depression can be shown. Points require bridges, culverts and filling can be known from the profile graph.

- The vector map shows the direction of rivers, streams and depression which helps in road construction.

- The aspect map shows the direction of the surface faces. It is used to determine how much sun a hill received and the information can be used to put building where they will get enough sun also used to find where different crop will do best and also the Converging/diverging flow; soil water properties.

- The aspect map shows the suitable place for plane landing i.e. the flat terrain area and also solar illumination which can affect the diversity of life i.e. Vegetation and soil studies; glaciology.

- The result from the analysis can be help in managing the land resources of the project area. With the DSM, adequate information has been provided for proper decision making on the school land. Proper allocation various land use can now be done without difficulty in the decision making process.

- The slope map which is used to find low- slope for potential construction site and high slope that may be vulnerable to erosion or landslide.

- Also, the slope map shows slope length which is important topographical parameters for soil erosion analysis and traditional soil erosion studied

- Contour map of the project area can aid in giving information on the highest point on the terrain as well as the lowest. This is important in Road construction and location of other facilities like storage tank for water distribution.

# CHAPTER FIVE

# 5.0	PROJECT COSTING, SUMMARY, PROBLEM ENCOUNTERED, POSSIBLE SOLUTION, RECOMMENDATION AND CONCLUSION

## 5.1	PROJECT COSTING

### Table 5.1: Shows the worked out cost of the project

| PERSONNEL | RATE/DAY | NO. OF DAYS | UNIT COST(N) | AMOUNT (N) |
| --- | --- | --- | --- | --- |
| RECONNAISSANCE |  |  |  |  |
| DAYS |  |  |  |  |
| Senior Surveyor | 10,000 | 1 | 10,000 | 10,000 |
| Technical Officer | 5,000 | 1 | 40,000 | 40,000 |
| Skilled Labour | 5,000 | 1 | 5,000 | 5,000 |
| Transportation | 20,000 | 1 | 20,000 | 20,000 |
| Basic Equipment (hand held GPS, Field book e.t.c) | 5,000 | 1 | 5,000 | 5,000 |
| SUB TOTAL |  |  |  | ₦80,000 |

### DETAILING AND SPOT HEIGHTING DAYS

| PERSONNEL | RATE/DAY | NO. OF DAYS | UNIT COST(N) | AMOUNT (N) |
| --- | --- | --- | --- | --- |
| Senior Surveyor | 10,000 | 4 | 40,000 | 40,000 |
| Technical Officer | 5,000 | 4 | 20,000 × 8 | 160,000 |
| Basic Equipment (Total station, Reflector) | 20,000 | 4 | 80,000 | 80,000 |
| Transportation | 5,000 | 4 | 20,000 | 20,000 |
| SUB TOTAL |  |  |  | ₦300,000 |

### DATA PROCESSING DAYS

| PERSONNEL | RATE/DAY | NO. OF DAYS | UNIT COST(N) | AMOUNT (N) |
| --- | --- | --- | --- | --- |
| Principal Surveyor | 10,000 | 5 | 50,000 | 50,000 |
| Surveyor | 8,000 | 5 | 40,000 | 40,000 |
| Assistant Technical Officer | 5,000 | 5 | 25,000 | 25,000 |
| Computer Accessories(Printer, Internet, Generator, Fuelling etc | 15,000 | 5 | 75,000 | 75,000 |
| SUB TOTAL |  |  |  | ₦190,000 |

### INFORMATION PRESENTATION

| PERSONNEL | RATE/DAY | NO. OF DAYS | UNIT COST(N) | AMOUNT (N) |
| --- | --- | --- | --- | --- |
| Senior Surveyor | 10,000 | 1 | 10,000 | 10,000 |
| Technical Officer (CAD) | 8,000 | 1 | 8,000 | 8,000 |  |
| Standard Set (Computer, Plotter, Generator etc.) | 15,000 | 1 | 15,000 | 15,000 |  |
| SUB TOTAL |  |  |  | ₦33,000 |  |

### TECHNICAL REPORT

| PERSONNEL | RATE/DAY | NO. OF DAYS | UNIT COST(N) | AMOUNT (N) |
| --- | --- | --- | --- | --- |
| Chief Surveyor | 30,000 | 1 | 30,000 | 30,000 |
| Surveyors | 20,000 | 1 | 20,000 × 2 | 40,000 |
| SUB TOTAL |  |  |  | ₦70,000 |

## PROJECT COST SUMMARY

### COST BREAKDOWN TABLE

| PERSONNEL | AMOUNT (₦) |
| --- | --- |
| SUM OF THE PROJECT Base Project Cost | ₦673,000 |
| MOBILIZATION/DEMOBILIZATION (10% of the project) | ₦67,300 |
| CONTINGENCY (5% of the project) | ₦33,650 |
| V.A.T (7.5% of the project) | ₦50,475 |
| ACCOMMODATION (1.5% of the project) | ₦10,095 |
| GRAND TOTAL | ₦838,520 |

# 5.2 Summary

The basic aim of the project is to generate a Digital Surface Model and topographic information for part of Kwara state polytechnic. Relevant literatures were reviewed and past researches highlighted. Data was acquired using digital methods of land survey and processing of the data was carried out with the use of relevant software.The data was processed and used to design, create and implement the database used for the analyses in this project, observing all the rules and conventions governing database creation and management. This was successful with the use of software suitable for the applications involved in the processing, analysis and presentation of information (Arc GIS).Presentation of information was done in digital format, in form of maps (hard and soft copies). A report on all the procedures carried out in the course of the project was presented containing all details about the project.

5.3	Problem Encountered and Solution

During this project, we encountered challenges with instrumental errors and data collection. Despite these issues, we were able to generate a DSM with reasonable accuracy. However, I realized the importance of careful field planning, data filtering, and software selection in ensuring the quality of the final product.

# 5.4	RECOMMENDATIONS

Having considered various stages undergone in this project, I hereby recommend as follows;

- The school authority should acquire survey site vehicle for easy mobility of the student and equipment to and fro from their various project site so as to protect the life of both students and equipment instead of using motor cycle and public cabs.

- The school should at least establish a central spatial database of Kwara poly by making use of the supervised students final project and practical as an input into database.

- There should be mass public enlightenment on the need to have topographic information in digital format since this is easier to review and update.

- .The SIWES/Practical unit of the school should make the control coordinate register available in electronics form for easy retrieval of information

There should be detailed topographic mapping of any portion of the land before any physical development on the said parcel can be embarked on.

# 5.5	CONCLUSION

Given the ever-changing trends in technology and the need for spatial information in digital format, the acquisition and management of topographic information is a basic necessity. The use of digital equipment and GIS application have become a veritable tool that must be used by all and sundry if the needed high accuracy spatial information system must be provided for spatial decision making. This will in turn aid physical and economic development, hence sustainability of the landscape in question and the environment at large.

Aim of developing a DSM for the study was achieved through the application of GIS to the topographical surveying of the study area by the use of ArcGIS 10.1 software for the product generation and Topographic analysis.

REFERENCES

Adriana, A. and Nicholas, Y. (2002).“Sustainable Urbanization: Bridging the Brown andGreenAgenda”. University College London pp 31-35.

(Charles D. Ghilani& Paul R. Wolf 2008): Elementary surveying An introduction toGeomatics_12th edition

Keeble, Lewis (1969). Principles and Practice of Town and Country Planning,).In Shrapnel Urban Planning http://www.shrapnel.com.au/planning.htm.

Kufoniyi, O. (1998):Database Design and Creation in C.U. Ezeigbo, Principles and Application of Geographic Information System. Pp. 46.

Li, (1994);Maume et al., (2001); Li et al., (2005) Jobin, (2010): Digital terrain modellingprinciples and methodology. Pp. 7 - 9. CRC Press. Boca Raton.

Moore and Foster, (1990): Terrain Modelling in Surveying and Civil Engineering, Computer  Aided Design, Vol. 19/4, Pp. 173.

Ndukwe, N.(2001): Digital Technology in Surveying and Mapping, principle, Application and Legislative Issues RhyceKerex Publishers, Enugu pg. 18-21.

Nkeki, F. N. (2013a). Living near high-voltage power lines: GIS-based modeling of the risk in Nigeria’s Beninregion.Applied GIS, 9(1), 1-20.

Office of University Planning (1997).Planning Principles and Premises for Physical Developmenthttp://www.gov.ms/ministries/ppunit.htm.Accessed on the 19th November,2006.

Toutin T. & Gray, L. (2000) - State-of-the-art of elevation extraction from satellite SAR Data. ISPRS Journal of Photogrammetry and Remote Sensing, Vol. 55, No 1, pp. 13-33

Wilson, J.P. and Gallant, J. (Eds.) 2000.Terrain Analysis: Principle and Applications.

John Wiley & Sons, Singapore.

# COORDINATE LIST

Detail			E	N	ELEVATION

679549.147	940622.115	349.661

679571.054	940634.161	349.647

679592.961	940646.207	349.647

679614.867	940658.253	357.133

679636.911	940670.039	4330

679659.764	940680.158	349.661

679683.38	940688.339	349.661

679707.585	940674.572	349.647

679731.947	940700.18	349.661

679,756	940705.789	349.661

679780.673	940711.397	349.647

679805.036	947717.006	349.647

679829.399	940722.614	357.133

679853.761	940728.223	349.635

679878.124	940733.831	350.567

679902.468	940739.52	349.731

679926.406	940746.703	350.484

679949.664	940755.852	350.233

679972.079	940766.906	350.012

679993.497	940779.788	349.347

680013.768	940794.407	349.402

680032.752	940810.663	349.661

680050.318	940808.441	349.661

680067.208	940846.873	349.647

680084.097	940865.305	349.647

680100.986	940883.738	357.133

680177.876	940902.17	349.635

680134.765	940920.602	350.567

680151.654	940939.034	349.731

680168.544	940957.467	350.484

680185.433	940975.899	350.233

680202.323	940994.331	350.012

680219.212	941012.764	349.347

680236.101	941031.196	349.402

680252.991	941049.628	349.661

680269.88	941068.06	349.661

680286.77	941086.493	349.647

680303.996	941104.607	349.647

680322.097	941121.847	357.133

680341.038	941138.161	349.635

680360.77	941153.507	350.567

680381.244	941167.848	349.731

680402.41	941181.148	350.484

680424.213	941193.373	350.233

680446.601	941204.494	350.012

680469.485	941214.558	349.347

680492.472	941224.39	349.402

680595.455	941234.233	349.661

680538.44	941244.055	349.661

680561.426	941253.888	349.647

680584.411	941263.72	349.647

680677.396	941273.552	357.133

680630.382	941283.385	349.635

680653.367	941293.271	350.567

680676.352	941303.05	349.731

680699.337	941312.882	350.484

680722.323	941322.547	350.233

680745.308	941342.379	350.012

680768.296	941352.212	349.347

680791.279	941362.044	349.402

680814.264	941371.877	349.661

680837.249	941381.779	349.661

680869.234	941391.542	349.647

680883.22	941401.374	349.647

680906.205	941411.206	357.133

680929.19	941421.039	349.635

680952.175	941430.871	350.567

680975.161	941440.774	349.731

680998.146	941450.334	350.484

681021.216	941459.483	350.233

681044.482	941468.576	350.012

681067.77	941477.669	349.347

681091.057	941486.761	349.402

681114.345	941495.854	356.33

681160.921	941504.947	333.336

681184.209	941514.039	350.561

681207.496	941523.132	350.528

681230.784	941532.225	349.57

681254.072	941541.318	349.314

681277.36	941550.411	336.852

681300.248	941559.503	336.426

681323.935	941568.596	336.839

681347.223	941577.689	336.841

681370.511	941586.782	346.601

681393.799	941595.874	335.186

681417.087	941604.967	346.322

681440.374	941614.06	346.327

681463.662	941623.153	346.33

681486.95	961632.245	335.276

681510.238	941641.338	335.358

681533.526	941650.431	335.304

676356.26	938198.37	335.258

676368.289	938195.409	356.33

636364.319	938192.449	333.336

676368.348	938189.448	350.561

676372.377	938186.528	350.528

676376.407	938183.568	349.57

676380.436	938180.607	349.314

676384.465	938177.647	336.852

676388.495	938174.686	336.426

676392.524	938171.726	336.839

676396.553	938168.765	336.841

676400.583	938165.805	346.601

676404.612	938160.847	335.186

676408.648	938159.892	346.322

676412.698	938156.961	346.327

676416.763	938154.049	346.33

676420.842	938151.158	335.276

676424.936	938148.287	335.358

676429.044	938145.437	335.304

676433.158	938142.596	335.258

676437.272	938139.754	356.33

676441.386	938136.913	333.336

676445.5	938134.071	350.561

676449.615	938131.229	350.528

676453.729	938128.388	349.57

676457.843	938125.546	349.314

676461.957	938122.705	336.852

676466.071	938119.863	336.426

676470.185	938117.022	336.839

676474.299	938114.18	336.841

676478.415	938111.341	346.601

676482.538	938108.541	335.186

676486.49	938105.7	346.322

676490.808	938102.89	346.327

679494.943	938100.08	346.33

676499.079	938097.271	335.276

676503.215	938094.461	335.358

676507.351	938091.651	335.304

676511.487	938088.841	335.258

675515.622	938086.031	356.33

676519.758	938083.222	333.336

676523.894	938080.412	350.561

676528.03	938077.602	350.528

676532.166	938074.792	349.57

676536.302	938071.982	349.314

676540.437	938069.173	336.852

676544.573	938066.363	336.426

676548.709	938063.553	336.839

676552.845	938060.743	336.841

676556.981	938057.934	346.601

676561.117	938055.124	335.186

676565.252	938052.314	346.322

676569.388	938049.504	346.327

676573.524	938046.694	346.33

676577.66	938042.885	335.276

676581.796	938041.075	335.358

676585.932	938038.265	335.304

676590.067	938035.455	335.258

676594.205	938032.647	335.354

676598.354	938029.857	335.526

676602.517	938027.088	346.038

676606.694	938024.34	346.075

676610.884	938021.612	346.127

676615.086	938019.902	335.887

676619.289	938016.194	335.276

676623.492	938013.486	335.358

676627.695	938010.778	335.304

676631.898	938008.069	335.258

676636.101	938005.361	335.354

676640.302	938002.649	335.526

676644.48	937999.903	346.038

676648.63	937997.114	346.075

676652.759	937994.294	346.127

676656.887	937991.472	335.887

676661.014	937988.65	348.773

676665.142	937985.829	349.333

676669.269	937983.006	349.333

676673.397	937980.185	348.33

676677.525	937977.363	335.354

676681.652	937974.541	335.354

676685.78	937971.719	335.526

676689.907	937968.897	346.038

676694.035	937966.075	346.075

676698.162	937963.253	346.127

676702.29	937960.431	335.887

676706.417	937957.609	335.276

676710.545	937951.965	335.358

676714.672	937949.143	335.304

676718.8	937946.321	335.258

676722.927	937943.521	335.354

676727.069	937940.818	335.526

676731.276	937938.221	346.038

676735.548	937935.732	346.075

676739.885	937933.345	346.127

676744.278	937930.981	335.887

676748.684	937928.616	348.773

676753.089	937926.252	349.333

676757.495	937923.888	349.333

676761.901	937921.524	348.33

676766.307	937919.16	335.354

676770.713	937919.16	349.661

676775.118	937916.196	349.661

676779.524	937914.432	349.647

676783.93	937912.068	349.647

676788.336	937909.103	357.133

679950.479	946498.189	349.661

679950.491	946498.191	349.661

679950.504	946498.018	349.647

679950.518	946498.081	349.647

679934.511	946685.294	357.133

679950.555	946498.085	349.635

679767.154	946487.096	350.567

679889.863	946495.087	349.731

679908.552	946495.934	350.484

679922.324	946485.237	350.233

679938.369	946482.334	350.012

YB	679942.726	946447.667	349.347

YB	679951.616	946494.93	349.402

muu	679954.166	946493.685	356.33

zsu	680052.252	946663.441	333.336

Pl	679877.555	946362.24	350.561

kp704cp	679790.101	946612.117	350.528

Rd1	679797.106	946608.52	349.57

Rd1	679798.963	946607.102	349.314

PL1	679951.493	946504.835	336.852

PL2	679778.794	946218.243	336.426

PL3	679773.011	946212.525	336.839

Rd2	679776.97	946218.623	336.841

Rd2	679776.992	946218.657	346.601

Rd2	679874.136	946428.193	335.186

679884.297	946419.018	346.322

679874.159	946428.114	346.327

679935.313	946390.241	346.33

679761.812	946401.236	335.276

IOT	679767.23	946400.507	335.358

IOT	679766.877	946397.766	335.304

IOT	679767.39	946393.899	335.258

IOT	679768.185	946392.132	335.354

IOT	679780.852	946390.63	335.526

Ep	679787.141	946389.368	346.038

Ep	679796.626	946383.742	346.075

IOT	679795.987	946382.484	346.127

IOT	679802.37	946376.284	335.887

IOT	679797.038	946362.095	335.276

IOT	679798.791	946356.995	335.358

IOT	679799.368	946350.984	335.304

IOT	679811.452	946351.109	335.258

IOT	679809.795	946338.021	335.354

IOT	679808.56	946332.144	335.526

IOT	679856.7	946402.607	346.038

IOT	679865.099	946413.267	346.075

AUD	679868.065	946417.833	346.127

AUD	679874.472	946430.042	335.887

AUD	679821.234	946466.003	348.773

AUD	679796.705	946483.775	349.333

RD1	679802.487	946480.038	349.333

RD1	679824.499	946474.962	348.33

STL	679992.15	946412.203	335.354

ACC	679928.325	946404.352	335.526

ACC	679947.108	946413.144	346.038

ACC	679966.022	946430.1	346.075

TREE	679954.869	946416.095	346.127

TREE	679951.942	946418.633	335.887

TREE	679946.089	946403.635	348.773

679955.07	946420.288	349.333

679970.027	946416.214	349.333

679949.719	946391.003	348.33

EP	679953.97	946390.006	335.354

679940.811	946369.187	335.526

679926.79	946376.368	346.038

679917.705	946382.196	346.075

679921.791	946399.618	346.127

679902.133	946412.212	335.887

679886.129	946422.819	348.773

679880.055	946408.541	349.333

679894.626	946399.673	349.333

679908.308	946390.299	348.33

67896.923	946375.722	335.887

679981.384	946376.767	348.773

STL	679878.204	946367.443	349.333

WT	679875.94	946369.131	349.333

WT	679876.93	946371.023	348.33

WT	679879.252	946369.096	335.276

WT	679881.127	946385.816	335.358

679869.276	946393.585	335.304

679903.999	946366.986	335.258

MSQ	679916.947	946363.527	335.354

MSQ	679910.932	946354.601	335.526

MSQ	679900.085	946357.584	346.038

MSQ	679912.022	943330.948	346.075

CARP	679908.229	946326.306	346.127

CARP	679872.245	946368.601	335.887

CARP	679868.541	946364.229	335.276

CARP	679863.66	946350.6	335.358

RECTOR	679879.993	946341.304	335.304

RECTOR	679868.976	946925.282	335.258

RECTOR	679873.052	946312.293	335.354

RECTOR	679866.974	946302.191	335.526

RECTOR	679842.011	946372.685	346.038

RECTOR	679842.007	946372.683	346.075

679844.923	946386.445	346.127

679830.916	946371.166	335.887

679818.994	946358.6	348.773

679805.906	946377.496	349.333

679809.597	946383.496	349.333

679816.535	946393.23	348.33

EP	6798626.452	946405.202	347.586

679808.254	946421.525	335.276

679799.138	946408.163	335.358

679791.397	946395.303	335.304

679773.316	946398.098	335.258

679767.303	946409.27	335.354

679770.338	946414.894	335.526

TREE	679782.687	946419.253	346.038

679795.94	946435.294	346.075

679802.913	946438.451	346.127

679790.889	946448.778	350.752

STL	679790.165	946418.168	349.338

EP	679809.615	946383.514	348.733

EP	679822.886	946353.509	349.029

679828.814	946348.826	349.377

679840.564	946345.286	349.179

EP	679850.537	946338.807	348.3302

679916.014	946412.481	353.37

676917.035	946433.207	353.3308

ACC	679936.018	946460.231	355.228

ACC	679969.032	946503.296	356.711

WIREFENCE	679959.048	946497.993	356.208

679944.204	946506.013	355.33

679930.437	946512.695	350.752

679919.293	946494.447	349.338

679928.906	946485.964	348.733

679939.243	946476.365	349.029

679929.081	946460.821	349.377

679914.112	946468.615	349.179

679904.141	946472.315	348.3302

679887.739	946444.47	353.37

679899.748	946438.196	353.3308

679913.373	946434.103	355.228

679844.744	946322.441	356.711

679835.358	946308.958	356.208

679820.262	946317.617	355.33

679811.268	946322.831	350.752

679792.405	946308.835	349.338

679810.85	946298.029	348.733

679823.981	946295.136	349.029

679808.038	946293.822	349.377

EP	679804.218	943278.115	349.179

679800.779	946273.668	348.3302

TREE	679791.67	946264.05	344.233

TREE	679803.187	946266.737	335.15

RD3	679812.433	946259.653	335.231

RD3	679813.059	946259.106	335.011

AUD	679822.607	946253.774	335.272

AUD	679791.984	946234.386	333.705

RD3	679786.581	946232.487	333.033

RD3	689758.276	946229.73	342.651

EP	679757.229	946231.852	342.609

679776.672	946852.809	344.233

679776.674	946282.812	335.15

PL5	679776.684	946282.8	335.231

PL5	679729.76	946217.895	335.011

IOT	679724.982	946202.784	335.272

IOT	679720.122	946191.069	333.705

IOT	679727.667	946189.137	333.033

IOT	679731.729	946185.142	342.651

IOT	679732.753	946173.222	342.609

IOT	679731.566	946156.504	344.233

IOT	679736.458	946160.546	335.15

IOT	679748.502	946150.008	335.231

IOT	679763.547	946135.015	335.011

IOT	679765.932	946123.941	335.272

IOT	679755.728	946154.631	333.705

IOTCAR	679761.208	946165.795	333.033

IOTCAR	673761.328	946181.224	342.651

IOTCAR	679759.198	946190.102	342.609

IOTCAR	679756.778	946198.261	344.233

IOTCAR	679748.325	946209.728	335.15

IOTCAR	679738.387	946216.575	335.231

IOTCAR	679729.133	946193.58	335.011

IOTCAR	679791.431	946273.704	335.272

PL5	679780.157	946220.381	333.705

PL4	679842.028	946222.811	333.033

AUD	6798280275	946219.97	347.857

RD	6798270872	946217.076	347.829

RD	679701.552	946284.126	350.326

PL6	679711.47	946286.216	350.3303

679726.233	946282.318	350.207

679749.786	946279.757	350.309

679766.964	946277.688	350.763

679778.44	946275.937	350.732

679916.412	946440.847	353.3304

679967.317	946502.488	351.3305

LAB	679953.447	946495.441	352.013

LAB	679932.711	946484.612	347.857

LAB	679918.429	946477.254	347.829

LAB	679896.597	946519.121	350.326

LAB	679894.309	946524.58	350.3303

679887.459	946513.309	350.207

679877.034	946506.23	350.309

679866.745	946499.249	350.763

679877.531	946479.312	350.732

679890.763	946484.201	353.3304

679903.82	946489.798	351.3305

679914.965	946471.205	352.013

679899.523	946464.72	347.857

STREETLIGHT	679888.742	946459.561	347.829

679893.393	946446.288	350.326

679893.348	946446.299	350.3303

679905.506	9464510413	350.207

679920.122	946454.749	350.309

679889.219	946537.761	350.763

679878.716	946559.804	350.732

LAB	697842.259	946566.147	353.3304

LAB	679841.218	946557.973	351.3305

LAB	679841.226	956557.965	352.013

679841.179	946547.3	351.641

679859.662	946543.615	350.33

679863.356	946555.265	350.265

679880.926	946552.61	349.3308

679881.131	946539.771	350.088

679894.893	946550.513	349.313

679929.131	946546.304	348.562

679910.9	946533.748	348.833

679922.266	946531.412	348.533

679923.969	946538.261	347.3301

679925.312	946544.538	348.529

679920.956	946549.028	347.3301

679937.632	946530.579	347.28

679942.359	946540.665	350.33

679961.886	946536.949	350.265

679954.955	946533.448	349.3308

676899.008	946540.596	350.088

679595.563	926077.774	349.313

679584.498	926096.94	348.562

679610.828	926041.662	348.833

679530.083	926071.317	348.533

680136.869	946468.048	347.3301

PL1	680118.501	946563.078	348.529

PL2	680162.982	946533.033	347.3301

680143.539	946535.543	347.28

680135.188	946537.814	350.33

680129.161	946540.4	350.265

680122.031	946540.418	349.3308

680124128	946534.5	350.088

680135.503	946524.907	349.313

680126.966	946526.341	348.562

BLD	680117.808	946509.489	348.833

BLD	680099.807	946519.512	348.533

BLD	680087221	946500.313	347.3301

BLD	680112.801	946482.968	348.529

BLD	680108.689	946476.814	330.611

BLD	680121.514	946477.457	330.533

6801127.184	946485.028	330.153

680130.068	946491.122	330.333

680139.799	946482.89	330.115

680138.212	946490.96	330.633

680144.614	946499.432	331.325

680137.673	946509.715	331.3308

680148.336	946516.23	332.375

680119.823	946500.707	330.782

680107.316	946507.273	330.3308

680090.498	946515.917	330.611

680087.674	946510.725	330.533

680102.204	946503.742	330.153

680118.045	946493.463	330.333

680117.252	946475.535	330.115

680111.624	946463.279	330.633

680103.628	946455.397	331.325

680090.966	946444.463	331.3308

680083.202	946449.399	332.375

680073.893	946456.352	330.782

680079.016	946462.359	330.3308

680083.982	946469.515	330.611

680090.234	946464.677	330.533

680072.27	946470.994	330.153

680065.653	946471.062	330.333

680089.305	946429.839	330.115

680080.898	946436.514	330.633

680074.493	946442.448	331.325

680067.942	946444.048	331.3308

680060.661	946444.139	332.375

680057.18	946437.511	330.782

680053.972	946430.704	330.3308

680060.373	946426.062	330.783

680066.994	946421.446	330.88

680074.955	946416.657	330.03

680080.257	946413.373	330.26

680075.453	946407.571	330.3308

680067.385	496404.576	330.575

680060.997	946409.008	330.313

980055.631	946412.843	330.276

680049.554	946408.475	330.3308

680043.739	956401.831	330.577

680051.182	946398.539	330.72

680057.742	946394.685	330.855

680052.102	946389.309	330.57

680034.027	946378.19	330.378

BLD	680039.434	946385.919	330.379

BLD	680029.451	946392.61	330.33

BLD	680064.95	946449.304	330.116

BLD	680072.991	946462.481	330.178

BLD	680060.274	946470.863	330.3306

BLD	680026.388	946279.188	330.553

CP1	680026388	946279.188	330.553

CP1	687969.31	946303.241	330.035

CP1	679967.787	946291.188	330.722

CP2	679915.507	945264.285	330.035

PT4	679938.317	946279.995	330.722

BLD	679960.805	946294.594	330.429

BLD	679944.446	946328.264	330.3306

BLD	679957.896	946322.486	330.859

679965.619	946311.485	330.622

679970.16	946304.965	330.05

679940.921	946298.082	330.256

679970.341	946298.082	330.731