A PROJECT REPORT

ON

TOPOGRAPHICAL SURVEY OF GOVERNMENT GIRLS DAY SECONDARY SCHOOL, PAKATA, ILORIN WEST LGA, KWARA STATE.

BY

ABDULWAHEED LAWAL OLAREWAJU

HND/23/SGI/FT/0039

SUBMMITED TO:

THE DEPARTMENT OF SURVEYING AND GEO-INFORMATICS, INSTITUTE OF ENVIRONMENTAL STUDIES, KWARA STATE POLYTECHNIC ILORIN.

IN PARTIAL FULFILLMENT OF THE REQUIREMENT FOR THE AWARD OF HIGHER NATIONAL DIPLOMA IN SURVEYING AND GEO-INFORMATICS.

JUNE, 2025

CERTIFICATE

I hereby certified that the information given in this project was obtained as a result of the observation and measurement made by me and that the survey was carried out in accordance with survey laws, regulations and departmental instructions

_________________________________

ABDULWAHEED LAWAL OLAREWAJU

Matric. No: HND/23/SGI/FT/0039

CERTIFICATION

This is to certify that this project was carried out by ABDULWAHEED LAWAL OLAREWAJU with Matric No.: HND/23/SGI/FT/0039 under my instruction and supervision for the award of Higher National Diploma in Surveying and Geo-informatics, Kwara State Polytechnic, Ilorin, Kwara State Nigeria. I hereby declared that he has conducted himself with due diligence and honesty on the said duties.

…………………………………				………………………………

SURV.A. AYUBA							DATE

(Project Supervisor)

……………………………….				…………………………

SURV. R.S. AWOLEYE						DATE

(Project Coordinator)

………………………………				………………………………

A.I ISAU								DATE

Head of Department (H.O.D) 
Surveying & Geo-informatics

………………………………..				……………………………..

EXTERNAL EXAMINER						DATE

DEDICATION

This project is dedicated to Almighty ALLAH, the author and the finisher of everything who gave me the grace and privilege to attain this stage in my education pursuit.

ACKWOLEDGEMENT

I give thanks and glory to my Creator for the privilege bestowed on me that resulted in the accomplishment of this project work and also a big thanks to the management of Kwara State Polytechnic for the privilege and opportunity given to me to study in their great institution.

My sincere gratitude goes to my Head of Department (HOD); A.I ISAU and my Supervisor; SURV.A. AYUBA for their support towards the success of this project, I really appreciate you sir.

To all the lecturers in the department of Surveying and Geo-informatics; Abimbola I. Isau, Surv. A. Ayuba, Surv. R.S. Awoleye, Surv. A.G. Aremu, Surv. B.F Diran, Surv. Wlliams Kazeem, Surv. Kabir Babatunde, Surv. R.O. Asonibare who has imparted their knowledge in me in one area or the other during my year of study, I am grateful for the effort and support. May Almighty ALLAH bless you all (Amiin).

I'm deeply grateful to my parents; MR & MRS ABDULWAHEED for their unconditional love, support, and wisdom. Their encouragement and sacrifices have been the driving force behind my endeavors. Thank you, Mom and Dad, for being my pillars of strength and inspiration. I dedicate this project to you, with love and appreciation.

Special thanks to my brother and sister for your unwavering love, care, and dedication that has shaped me into the person I am today. Your selfless love and your influence and values have guided me throughout this project, and I'm forever grateful for your presence in my life.

I would like to extend my heartfelt thanks to my beloved family members and friends who have supported me throughout this project.

May Almighty ALLAH bless you all (Amiin).

TABLE OF CONTENTS

Pages

Title Page										i

Certification										ii

Dedication 										iii

Acknowledgment 									iv

Table of contents									v

List of Figures										ix

Abstract 										x

CHAPTER ONE

General Introduction

1.1 	Background to the Study 							1

1.2 	Statement of the Problem 							3

1.3	Aim and Objectives of the Project						4

1.3.1	Aim										4

1.3.2	Objectives									4

1.4	Scope of the Project								5

1.5	Personnel									5

1.6	Significance of the Study							5

1.7	Study Area									7

CHAPTER TWO

Literature review

2.1	Historical Development of Topographic Mapping				8

2.2	Conceptual Framework							10

2.2.1	Role of GIS in Topographic Data Management				12

2.2.2	Remote Sensing Technologies in Topography 				13

2.2.3	Digital Elevation Models (DEMs) and Terrain Analysis			15

2.3	Applications of Topographical Surveys					17

2.4	Accuracy and Data Quality Issues						19

2.5	Advancements in 3D Topographic Visualization 				21

CHAPTER THREE

Research Methodology

3.1	Methodology									23

3.2	Equipment and Materials							23

3.3	Survey Procedure								24

3.3.1	Reconnaissance Survey							24

3.3.2	Establishment of Control Points						26

3.3.3	Perimeter Surveying (Boundary Measurement)				30

3.3.4	Fixing of Details								31

3.3.5	Spot Height Measurement							33

3.4	Deliverables and Output							34

CHAPTER FOUR

Result and Analysis

4.1	Perimeter Survey Result							35

4.2	Detailed (Planimetric) Survey Result						36

4.3	Spot Height Measurement and Elevation Analysis				37

4.4	Topographic Map Result							39

4.5	Digital Terrain Model (DTM) Output 					40

4.6	Integrated Spatial Interpretation and Site Assessment 			41

CHAPTER FIVE

Costing, Summary, Problem Encountered, Conclusion and Recommendations

5.1	Cost Estimation 								43

5.2	Summary									45

5.3 	Problem Encountered 								47

5.4	Conclusion									47

5.5	Recommendations								48

References

Appendix

LIST OF TABLES

Pages

Table 1.1: Table showing the personnel 						5

Table 3.1: Table showing the new established control points			27

Table 3.2: Table showing the coordinate of the control point after the orientation 	29

Table 5.1: Table showing the cost estimation 					43

LIST OF FIGURES

Pages

Fig. 1.1: Maps Showing the Study Area						6

Fig. 3.1: Diagram showing the control station connection				28

Fig. 4.1: Detailed plan of the study area						37

Fig. 4.2: Spot Height Plan of the study area						38

Fig. 4.3: Contour Map of the study area						40

Fig. 4.4: Digital Terrain Model Map of the study area				41

ABSTRACT

This project developed a detailed Topographical Survey (TIS) for a section of Government Girls’ Day Secondary School, Pakata, Ilorin West Local Government Area, Kwara State, Nigeria. Recognizing the critical need for accurate spatial data to support effective land use planning, infrastructure development, and environmental management in this rapidly urbanizing area, the study employed field surveys using GPS technology and Total Station (TS). Key topographic features, including elevation points, contours, slopes, drainage patterns, and significant man-made structures, were meticulously captured and integrated. The processed data was used to generate essential digital products, including a high-resolution Digital Elevation Model (DEM), detailed contour maps, slope classification maps, and a comprehensive topographic map of the study area. These outputs were systematically organized and managed within a Geographic Information System (GIS) environment, creating a robust spatial database. The resulting Topographical Survey provides a vital geospatial framework. It offers planners, engineers, environmental scientists, and local government authorities precise and readily accessible information on the terrain characteristics of Government Girl’s Day Secondary School. This system is fundamental for informed decision-making in site suitability analysis, flood risk assessment, road network planning, construction projects, and sustainable land management within the studied locality.

CHAPTER ONE

INTRODUCTION

1.1		BACKGROUND OF THE STUDY

A Topographical Survey (TIS) is a specialized form of Geographic Information System (GIS) that focuses on the detailed representation of the Earth's surface, including its natural and artificial features. The study of TIS is rooted in the need to accurately capture, store, analyze, and visualize spatial data related to the terrain, which is essential for a wide range of applications in fields such as urban planning, environmental management, disaster response, military operations, and infrastructure development.

The concept of topographic mapping dates back to ancient civilizations, where rudimentary maps were created to represent land features for navigation, agriculture, and territorial expansion. However, the systematic study and development of Topographical Surveys began in the 19th and 20th centuries with the advent of advanced surveying techniques, aerial photography, and cartography. The introduction of computers in the mid-20th century revolutionized the field, enabling the digitization of maps and the development of GIS, which laid the foundation for modern TIS.

The evolution of TIS has been closely tied to advancements in technology. The integration of remote sensing, Global Positioning Systems (GPS), and Light Detection and Ranging (LiDAR) has significantly enhanced the accuracy and resolution of topographic data. These technologies allow for the collection of high-resolution elevation data, which is crucial for creating detailed topographic maps. Additionally, the development of sophisticated software tools has enabled the efficient processing, analysis, and visualization of large datasets, making TIS an indispensable tool in various sectors.

Despite its numerous applications, the implementation of TIS faces several challenges. These include the high cost of data acquisition, the complexity of data processing, and the need for specialized skills to interpret and analyze topographic data. Additionally, the rapid pace of technological change requires continuous updates and upgrades to TIS infrastructure and software.

Looking ahead, the future of TIS is likely to be shaped by advancements in artificial intelligence (AI), machine learning, and big data analytics. These technologies have the potential to automate data processing, enhance predictive modeling, and improve the accuracy of topographic maps. Furthermore, the integration of TIS with other emerging technologies such as the Internet of Things (IoT) and augmented reality (AR) could open up new possibilities for real-time terrain analysis and immersive visualization.

Topographic information plays a crucial role in urban planning, land management, and infrastructure development. A Topographical Survey (TIS) integrates geospatial data to provide detailed insights into the physical features of a given area, including elevation, drainage patterns, land use, and built structures. The development of a TIS for a section of Government Girls Day Secondary School, Pakata, Ilorin West LGA, Kwara State is essential for effective planning and sustainable development.

Government Girls Day Secondary School, Pakata, Ilorin, is experiencing rapid urbanization, necessitating accurate topographic data for informed decision-making. Traditional land survey methods, combined with modern geospatial technologies such as Geographic Information Systems (GIS) and Global Positioning Systems (GPS), enable efficient mapping and analysis of the school terrain. This study aims to generate a Topographical Survey that provides a comprehensive understanding of the area's landform, aiding in flood control, infrastructure planning, and environmental management.

The research involves data collection through field surveys, GPS mapping, and GIS applications to produce a digital topographic map of the selected area. The findings will serve as a valuable resource for urban planners, engineers, and policymakers in making strategic land-use decisions. By implementing a TIS, stakeholders can ensure sustainable development while mitigating environmental risks such as erosion and poor drainage.

This study highlights the significance of Topographical Surveys in modern urban development, demonstrating their role in optimizing land use and enhancing infrastructural planning.

In conclusion, the study of Topographical Surveys is a multidisciplinary field that combines elements of geography, computer science, engineering, and environmental science. It plays a crucial role in understanding and managing the Earth's surface, and its importance is expected to grow as we face increasingly complex challenges related to urbanization, climate change, and resource management.

1.2	STATEMENT OF THE PROBLEM

The rapid urbanization of Government Girls Day Secondary School, Pakata, Ilorin, has led to increasing challenges in land use planning, infrastructure development, and environmental management. However, the absence of an accurate and comprehensive Topographical Survey (TIS) for the area has made it difficult for urban planners, engineers, and policymakers to make informed decisions.

One of the major issues in the school is the lack of precise elevation and drainage data, which contributes to poor water management, frequent flooding, and soil erosion. Additionally, inadequate mapping of land features and infrastructure has resulted in inefficient land allocation, haphazard construction, and improper road network planning. These challenges hinder sustainable urban development and may lead to long-term environmental and infrastructural problems.

Moreover, the reliance on outdated or non-digital topographic data limits the effectiveness of planning efforts. Without a well-structured TIS, authorities and developers struggle to assess land suitability, optimize resource utilization, and mitigate environmental risks.

This study seeks to address these issues by developing a reliable Topographical Survey for a section of Government Girls Day Secondary School, Pakata, Ilorin. By integrating field survey data, GPS mapping, and GIS applications, the research aims to provide accurate and up-to-date topographic information to support better decision-making in urban planning and land management.

1.3	AIM AND OBJECTIVES OF THE PROJECT

1.3.1	AIM OF THE PROJECT

The aim of the project is to carry out the Topographical Survey of GOVERNMENT GIRLS DAY SECONDARY SCHOOL PAKATA, ILORIN and presenting the information in a digital cartographic format.

1.3.2	OBJECTIVES OF THE PROJECT

The objectives of this study is to:

- conduct a detailed topographic survey of the study area using geospatial techniques such as GPS mapping and GIS applications.

- generate an accurate topographic map that represents the terrain characteristics, elevation levels, drainage patterns, and land use distribution within the estate.

- analyze the impact of topography on land development, infrastructure planning, and environmental risks such as flooding and erosion.

- create a digital database for storing and managing topographic information, which can be accessed for future planning and decision-making.

- provide recommendations on the application of topographic data in sustainable urban development, land administration, and disaster mitigation strategies.

1.4	SCOPES OF THE PROJECT

The project scopes are the stages that will be involve to achieve our aims.

- Project planning

- Recce

- Data acquisition

- Data processing

- Data analysis

- Data presentation

- Technical report writing

1.5	PERSONNEL

The students listed below were the members of this group who participated in the execution of the project.

Table 1.1: Table showing the personnel

| NAME | MATRIC NO. | ROLE |
| --- | --- | --- |
| AJIBADE TIMILEHIN PRAISE | HND/23/SGI/FT/0036 | AUTHOR |
| AGBENI OLUWATOYIN AMEENAT | HND/23/SGI/FT/0008 | MEMBER |
| HAMMED QUDUS OPEYEMI | HND/23/SGI/FT/0040 | MEMBER |
| ABDULWAHEED LAWAL OLAREWAJU | HND/23/SGI/FT/0039 | MEMBER |
| FANIWA ELIZABETH BRIGHT | HND/23/SGI/FT/0035 | MEMBER |
| SMART OPEYEMI EWAOLA | HND/23/SGI/FT/0037 | MEMBER |

1.6	SIGNIFICANCE OF THE PROJECT

The significance of studying a Topographical Survey (TIS) lies in its critical role in enhancing spatial decision-making and resource management across diverse sectors. By integrating detailed elevation data, landform features, and human-made structures, a TIS provides a comprehensive framework for analyzing terrain characteristics, which is vital for sustainable urban planning, environmental conservation, and disaster risk mitigation. In urban development, it aids in optimizing infrastructure design, zoning, and floodplain management, while in environmental sectors, it supports watershed analysis, habitat preservation, and climate resilience strategies. Additionally, the system’s ability to synthesize real-time geospatial data with advanced technologies like GIS and remote sensing enables governments, planners, and researchers to address challenges such as land degradation, deforestation, and emergency response with greater precision. By democratizing access to accurate topographic insights, a TIS fosters interdisciplinary collaboration, empowers evidence-based policymaking, and contributes to long-term ecological and socio-economic sustainability, ultimately bridging the gap between geospatial innovation and practical applications for community and environmental well-being.

1.7 	STUDY AREA

Government Girls Day Secondary School Pataka, Ilorin is along Oloje in Ilorin West Local Government Area of Kwara State, the premises is on Latitude 0080 30’ 05"N and Longitude 0040 30' 58” E on the geographical location of the State.

Fig. 1.1: Maps showing the study area

CHAPTER TWO

LITERATURE REVIEW

2.1	HISTORICAL DEVELOPMENT OF TOPOGRAPHIC MAPPING

The historical development of topographic mapping is a testament to humanity’s enduring quest to represent the Earth’s surface with precision and utility. Its origins trace back to ancient civilizations, where rudimentary tools like the groma and chorobates were used by Egyptians, Greeks, and Romans for land division, irrigation, and infrastructure projects such as road construction. These early efforts laid the groundwork for systematic cartography, which saw significant strides during the Renaissance. Innovations like Gerardus Mercator’s 1569 cylindrical projection revolutionized navigation by preserving directional accuracy, while Willebrord Snell’s formalization of triangulation in 1615 enabled large-scale terrestrial surveys through geometric principles. By the 18th and 19th centuries, topographic mapping became institutionalized with national initiatives such as France’s Cassini surveys—Europe’s first comprehensive topographic series—and Britain’s Ordnance Survey, established in 1791 to support military and administrative needs. In the United States, the founding of the U.S. Geological Survey (USGS) in 1879 marked a pivotal shift toward standardized mapping, driven by post-Civil War expansion and resource management demands.

Traditional methods of ground surveying dominated this era, relying on tools like theodolites, chains, and levels to measure angles, distances, and elevations manually. Techniques such as triangulation and traversing established control points across landscapes, though these approaches were labor-intensive and limited to accessible terrain. Concurrently, plane table surveying emerged as a hybrid method, combining field observations with real-time map drafting on portable tables, which remained popular for local-scale mapping into the early 20th century. The late 19th century introduced photogrammetry, pioneered by figures like Nadar, who captured aerial photographs from balloons in 1858. This method evolved dramatically during the World Wars, as aerial reconnaissance photos and stereoscopic analysis allowed cartographers to reconstruct 3D terrain models from ovszerlapping images. Analog tools like stereoplotters, such as the Wild A7, mechanized the extraction of contour lines and features, enabling rapid mapping of vast, remote regions post-1945.

The 20th century ushered in transformative technological shifts. Electronic Distance Measurement (EDM) devices replaced manual chains with laser and radio wave technology, accelerating data collection. Satellite remote sensing, exemplified by NASA’s Landsat program (1972) and France’s SPOT satellites (1986), provided global, multispectral imagery that enriched topographic datasets. The integration of GPS in the 1980s–90s further revolutionized surveying by offering real-time georeferencing, reducing dependence on ground control points. By the late 20th century, digital photogrammetry replaced analog systems, allowing for computerized image processing and GIS (Geographic Information Systems) platforms that layered topographic data into dynamic digital maps. Airborne Lidar (Light Detection and Ranging), emerging in the 1990s–2000s, achieved centimeter-level elevation accuracy, while open-source tools like Google Earth democratized access to topographic information.

These historical advancements underscore a transition from labor-intensive, localized practices to automated, global-scale systems. Traditional methods like ground surveying and analog photogrammetry remain foundational, often integrated with modern technologies such as drone-based mapping and AI-driven analysis. Today, topographic mapping underpins critical applications like climate modeling, urban planning, and autonomous navigation, reflecting centuries of innovation shaped by exploration, conflict, and the relentless pursuit of spatial precision.

2.2	CONCEPTUAL FRAMEWORK

The concept of a Topographical Survey (TIS) is rooted in the integration of geospatial data, cartographic principles, and computational tools to model, analyze, and visualize terrain features. Early foundational work by Tomlinson (1967) in "An Introduction to the Geographic Information System of the Canada Land Inventory" highlighted the importance of systematic data layers (e.g., elevation, hydrology) for land management, laying the groundwork for modern TIS. Burrough and McDonnell (1998) expanded this in "Principles of Geographical Information Systems," emphasizing that topographic systems rely on Digital Elevation Models (DEMs) and vector datasets to represent terrain morphology, slope, and aspect, which are critical for applications like flood modeling and infrastructure planning.

The evolution of TIS has been shaped by advancements in data acquisition technologies. Li et al. (2005), in "Digital Terrain Modeling: Principles and Methodology," underscored the role of photogrammetry and LiDAR (Light Detection and Ranging) in generating high-resolution topographic data, replacing labor-intensive field surveys. Similarly, Petrie and Kennie (1990) in "Terrain Modelling in Surveying and Civil Engineering" argued that TIS transformed engineering practices by enabling 3D visualization of terrain, facilitating slope stability analysis and route optimization. The integration of TIS with Geographic Information Systems (GIS), as discussed by Longley et al. (2015) in "Geographic Information Science & Systems," has further enhanced its utility, allowing dynamic overlays of socio-economic and environmental data onto topographic basemaps.

Scholars like Clarke (2003) in "Getting Started with Geographic Information Systems" noted that TIS democratized access to topographic data through platforms like USGS’s National Map and OpenTopography, fostering interdisciplinary research in climatology, ecology, and urban planning. Meanwhile, Hengl and Reuter (2009) in "Geomorphometry: Concepts, Software, Applications" highlighted the rise of machine learning algorithms in TIS, enabling predictive modeling of erosion patterns and landslide risks by analyzing topographic parameters. Critically, Goodchild (2007) in "Citizens as Sensors: The World of Volunteered Geography" emphasized the role of crowdsourced data (e.g., OpenStreetMap) in enriching TIS, though he cautioned about variable data quality and standardization challenges.

Recent innovations, such as cloud-based TIS (e.g., Google Earth Engine) and real-time topographic monitoring using IoT sensors, reflect the field’s ongoing transformation. Gesch et al. (2014) in "The National Elevation Dataset" demonstrated how TIS supports global initiatives like climate adaptation planning, while Tateosian (2019) in "Python For ArcGIS" illustrated the growing reliance on programming tools to automate topographic analysis. Despite these advances, authors like Fisher and Tate (2006) in "Causes and Consequences of Error in Digital Elevation Models" warned that inaccuracies in source data or resolution limitations can propagate errors in TIS outputs, necessitating rigorous validation protocols.

In summary, Topographical Surveys represent a confluence of cartographic tradition and digital innovation, driven by contributions from geographers, engineers, and computer scientists. Their evolution—from manual contour mapping to AI-driven terrain analysis—reflects broader trends in geospatial technology, as noted by Rhind (1999) in "Framework for the World," who described TIS as a cornerstone of the "digital earth" paradigm, essential for addressing 21st-century environmental and infrastructural challenges.

2.2.1	ROLE OF GIS IN TOPOGRAPHIC DATA MANAGEMENT

The role of Geographic Information Systems (GIS) in topographic data management is transformative, revolutionizing how spatial information is stored, analyzed, and visualized. GIS serves as a centralized platform for integrating diverse topographic datasets—such as elevation models (e.g., DEMs, LiDAR), contour lines, hydrologic networks, and land cover—into a unified georeferenced framework. This integration enables efficient storage through structured spatial databases (e.g., geodatabases) that support large-scale data organization, versioning, and metadata management. For instance, Burrough and McDonnell (1998) emphasize that GIS’s relational database architecture allows seamless layering of raster (elevation grids) and vector (streams, roads) data, ensuring interoperability and reducing redundancy.

In terms of analysis, GIS provides advanced tools to derive actionable insights from topographic data. Slope, aspect, and curvature calculations enable terrain characterization, while hydrological modeling tools simulate water flow and watershed delineation (Li et al., 2005). Spatial overlays of topography with socio-economic or environmental datasets (e.g., flood risk zones, infrastructure layouts) support decision-making in urban planning and disaster management. Longley et al. (2015) highlight how GIS facilitates 3D terrain analysis, such as line-of-sight visibility for telecommunications or military applications, and cut-and-fill calculations for engineering projects. Machine learning integration, as noted by Hengl and Reuter (2009), further enhances predictive modeling of erosion or landslide susceptibility by correlating topographic indices with historical data.

For visualization, GIS transforms raw topographic data into intuitive maps and models. Tools like hillshading, 3D rendering, and contour interpolation create dynamic visual representations of terrain, aiding in education, public engagement, and policy formulation. Platforms such as ArcGIS Pro or QGIS enable interactive web maps, allowing stakeholders to explore elevation profiles or simulate landscape changes (e.g., sea-level rise). Clarke (2003) underscores how GIS democratizes access to topographic information through user-friendly interfaces and open-source platforms like Google Earth, which combine satellite imagery with elevation data for global-scale visualization.

Moreover, GIS enhances collaboration by enabling real-time data sharing across disciplines. Cloud-based systems (e.g., ArcGIS Online) allow multiple users to update and analyze topographic datasets simultaneously, fostering transparency in projects like transportation planning or environmental conservation. However, challenges such as data resolution limitations and computational demands persist, as Fisher and Tate (2006) caution, necessitating rigorous error-checking and scalable infrastructure.

In summary, GIS bridges the gap between raw topographic data and actionable knowledge, empowering industries—from agriculture to defense—to leverage terrain intelligence with unprecedented precision. By harmonizing storage, analysis, and visualization, GIS has become indispensable in managing the Earth’s dynamic surface, as Rhind (1999) aptly describes in the context of the "digital earth" paradigm.

2.2.2	REMOTE SENSING TECHNOLOGIES IN TOPOGRAPHY

Remote sensing technologies have revolutionized topographic mapping by enabling the acquisition of high-resolution, large-scale geospatial data through satellite imagery, aerial photography, and advanced sensors. These tools overcome the limitations of traditional ground surveys, offering unparalleled efficiency, coverage, and precision in capturing terrain features. Satellite-based systems, such as NASA’s Landsat and the European Space Agency’s Sentinel missions, provide multispectral and radar data that underpin global elevation models like the Shuttle Radar Topography Mission (SRTM) and TanDEM-X. These datasets, as noted by Farr et al. (2007) in "The Shuttle Radar Topography Mission," achieve near-global coverage with resolutions as fine as 12 meters, enabling detailed slope and relief analysis even in remote or inaccessible regions.

Aerial remote sensing, including LiDAR (Light Detection and Ranging) and photogrammetry, complements satellites by delivering ultra-high-resolution topographic data. Airborne LiDAR systems, as highlighted by Axelsson (2000) in "DEM Generation from Laser Scanner Data," emit laser pulses to measure ground elevation with centimeter-level accuracy, penetrating vegetation to map bare-earth terrain—critical for floodplain mapping and forestry. Similarly, unmanned aerial vehicles (UAVs/drones) equipped with photogrammetric cameras generate 3D point clouds and orthomosaics through Structure-from-Motion (SfM) algorithms, democratizing access to cost-effective, localized mapping (Westoby et al., 2012).

The integration of synthetic aperture radar (SAR) and interferometric SAR (InSAR) further enhances topographic accuracy. SAR satellites, such as ALOS-PALSAR, detect subtle elevation changes over time, useful for monitoring subsidence or volcanic activity (Rosen et al., 2000). InSAR’s phase difference measurements between radar images, as discussed by Massonnet and Feigl (1998), create deformation maps with millimeter precision, bridging gaps in dynamic terrain analysis.

Remote sensing also streamlines data processing and integration. Platforms like Google Earth Engine and ESA’s Copernicus leverage cloud computing to process petabytes of satellite data, automating the extraction of contour lines, drainage networks, and landforms. Machine learning algorithms, as explored by Zhu et al. (2017) in "Deep Learning in Remote Sensing," classify topographic features (e.g., cliffs, valleys) from imagery, reducing manual interpretation errors.

However, challenges persist, including cloud cover obscuring optical satellite data, the high cost of LiDAR surveys, and the need for ground control points (GCPs) to calibrate aerial photogrammetry. Despite these limitations, remote sensing remains indispensable for applications like disaster response, urban planning, and climate modeling. As Petrie and Kennie (1990) assert in "Terrain Modelling in Surveying and Civil Engineering," the synergy of satellite and aerial technologies has transformed topography from a static discipline into a dynamic, real-time science, underpinning sustainable development and environmental stewardship in the 21st century.

2.2.3	DIGITAL ELEVATION MODELS (DEMS) AND TERRAIN ANALYSIS

Digital Elevation Models (DEMs) are foundational tools in modern terrain analysis, providing a gridded or vector-based representation of the Earth’s surface elevation. Derived from remote sensing technologies such as LiDAR, photogrammetry, and radar (e.g., SRTM, ASTER), DEMs serve as the backbone for quantifying and visualizing topographic attributes, enabling precise analysis of slope, aspect, and watershed dynamics.

Slope Analysis:

DEMs calculate slope gradients by determining the rate of elevation change between adjacent pixels. This is critical for assessing terrain steepness, which informs landslide risk mapping, agricultural suitability, and infrastructure planning. For example, in urban development, slope maps derived from high-resolution LiDAR DEMs identify buildable land while avoiding unstable slopes. Algorithms like the Horn method or D8 flow direction compute slope values in degrees or percentages, enabling engineers to design road alignments or drainage systems that adapt to natural topography.

Aspect Analysis:

Aspect, defined as the compass direction a slope faces, is calculated by analyzing the orientation of the steepest descent from each DEM pixel. This parameter influences microclimates, vegetation patterns, and solar exposure. In renewable energy planning, aspect maps identify optimal locations for solar panels by highlighting south-facing slopes (in the Northern Hemisphere) that receive maximum sunlight. Similarly, ecologists use aspect data to study species distribution, as north-facing slopes in temperate regions tend to be cooler and more humid, supporting distinct ecosystems.

Watershed Delineation and Hydrological Modeling:

DEMs are indispensable for watershed analysis, which involves defining drainage basins, stream networks, and flow accumulation. Using hydrological correction techniques (e.g., filling sinks), DEMs simulate water movement across landscapes. Tools like the D8 algorithm or TauDEM derive flow direction and accumulation grids, enabling the delineation of watershed boundaries. This is vital for flood prediction, reservoir management, and soil conservation. For instance, the USDA employs DEM-based watershed models to design contour farming systems that reduce erosion in agricultural regions.

Integration with Advanced Technologies:

Modern DEMs, particularly those from airborne LiDAR (1–2 m resolution) or drone photogrammetry, enhance the accuracy of terrain analysis. Machine learning algorithms now automate feature extraction, such as identifying gully erosion or landslide scars from DEM derivatives. Open-source platforms like QGIS and GRASS GIS democratize access to these tools, allowing researchers to compute slope, aspect, and curvature indices at scale.

Challenges and Considerations:

Despite their utility, DEMs have limitations. Low-resolution datasets (e.g., 30 m SRTM) may overlook fine-scale terrain features, while errors in elevation data can propagate into flawed slope or flow models. Additionally, vegetated or urban areas require ground-filtered LiDAR DEMs to represent bare-earth topography accurately.

In summary, DEMs have transformed terrain analysis into a dynamic, data-driven discipline. By enabling precise slope, aspect, and watershed studies, they underpin sustainable land management, disaster resilience, and environmental science, illustrating the synergy between geospatial technology and Earth’s physical processes.

2.3	APPLICATIONS OF TOPOGRAPHICAL SURVEYS

Topographical Surveys (TIS) are indispensable tools across diverse sectors, leveraging geospatial data to address complex challenges in urban planning, agriculture, infrastructure development, and disaster management. By integrating elevation models, terrain attributes, and spatial analytics, TIS enables data-driven decision-making, optimizing efficiency and sustainability.

### 1. Urban Planning

TIS supports smart city development by analyzing terrain for optimal land use, zoning, and infrastructure placement. Slope and aspect maps identify flood-prone areas or unstable slopes, guiding safe residential and commercial development (Burrough & McDonnell, 1998). For instance, 3D terrain models derived from LiDAR data help planners visualize urban sprawl, design drainage systems to mitigate flooding, and preserve green spaces. In cities like Singapore, TIS integrates with GIS to simulate shadow effects of skyscrapers or solar potential on rooftops, enhancing energy efficiency. Additionally, TIS aids in heritage conservation by modeling historic landscapes and assessing erosion risks to cultural sites.

### 2. Agriculture

In agriculture, TIS enhances precision farming and soil conservation. Slope analysis determines erosion-prone areas, enabling farmers to implement contour plowing or terracing, as practiced in the vineyards of Tuscany or rice paddies in Southeast Asia. DEMs combined with soil moisture data optimize irrigation systems, ensuring water reaches crops efficiently in regions like California’s Central Valley. Aspect maps identify microclimates for crop rotation, while elevation data assists in frost risk assessment for orchards. The USDA’s National Resources Inventory uses TIS to monitor land degradation and promote sustainable practices (Hengl & Reuter, 2009).

### 3. Infrastructure Development

Engineers rely on TIS for route optimization and slope stability analysis in road, rail, and pipeline projects. DEMs calculate cut-and-fill volumes to minimize earthmoving costs during highway construction, such as in the mountainous terrain of the Swiss Alps. Hydrological models predict water flow to design resilient bridges and culverts, while LiDAR-derived elevation data ensures precise alignment of tunnels and dams. For example, China’s Three Gorges Dam project utilized TIS to assess reservoir capacity and landslide risks. TIS also aids in renewable energy projects, such as identifying optimal slopes for wind farms or solar arrays.

### 4. Disaster Management

TIS is critical for risk assessment and emergency response. Flood models using DEMs and watershed delineation predict inundation zones, as seen in the Netherlands’ flood defense systems. Landslide susceptibility maps, generated from slope, curvature, and soil data, guide evacuations in regions like Nepal’s Himalayas. Post-disaster, aerial LiDAR and satellite imagery rapidly assess damage, as demonstrated after Hurricane Katrina or the 2011 Tōhoku tsunami. Real-time TIS platforms, such as FEMA’s Flood Map Service Center, provide dynamic updates during crises, while InSAR monitors ground subsidence in earthquake-prone areas like Mexico City.

### Cross-Sector Impact and Challenges

TIS fosters interdisciplinary collaboration—e.g., combining agricultural soil data with urban expansion models to address food security. However, challenges include data resolution gaps (e.g., coarse DEMs in remote regions), computational costs for large datasets, and the need for localized calibration. Advances in AI-driven terrain analysis and crowdsourced geospatial data (e.g., OpenStreetMap) are addressing these barriers, democratizing access to TIS tools.

In conclusion, Topographical Surveys are transformative across sectors, bridging the gap between terrain data and actionable insights. From designing climate-resilient cities to safeguarding food production and mitigating disasters, TIS exemplifies the convergence of geospatial innovation and real-world problem-solving, as emphasized by Longley et al. (2015): “Terrain intelligence is no longer a luxury—it is a necessity for sustainable development.”

2.4	ACCURACY AND DATA QUALITY ISSUES

The reliability of Topographical Surveys (TIS) hinges on the accuracy and quality of underlying geospatial data, yet challenges related to resolution, consistency, and validation persistently complicate their application. Data resolution—the granularity at which terrain features are captured—directly impacts the precision of outputs. For instance, low-resolution DEMs (e.g., 30–90 m SRTM data) may fail to detect microtopographic features like gullies or small ridges, leading to flawed flood models or infrastructure designs in complex landscapes. Conversely, high-resolution LiDAR (1–2 m) or drone-derived DEMs improve detail but demand significant storage and computational resources, creating trade-offs between accuracy and practicality (Fisher & Tate, 2006). In urban planning, coarse resolution datasets can misrepresent building footprints or drainage networks, risking flawed zoning decisions.

Consistency issues arise when integrating multi-source or multi-temporal datasets. Disparities in coordinate systems, elevation benchmarks, or data collection methods (e.g., combining legacy contour maps with modern LiDAR) introduce errors during fusion. For example, satellite-derived DEMs may misalign with ground-surveyed control points due to georeferencing inaccuracies, skewing watershed delineation (Li et al., 2005). Temporal inconsistencies also pose challenges: vegetation growth or urban expansion can render older DEMs obsolete, as seen in rapidly developing regions like Mumbai or Lagos, where outdated topographic maps fail to account for new infrastructure.

Validation remains a critical yet underappreciated hurdle. Elevation data errors—stemming from sensor noise, interpolation artifacts, or incomplete ground truthing—propagate into slope, aspect, and hydrological models. Fisher and Tate (2006) emphasize that even minor DEM errors (e.g., 1–2 m vertical inaccuracies) can distort floodplain mapping, with cascading impacts on disaster preparedness. Validation often relies on ground control points (GCPs) or independent surveys, but these are resource-intensive and impractical in remote or hazardous areas like the Amazon rainforest or Himalayan glaciers. Automated validation tools, such as cross-comparison with ICESat satellite altimetry, offer partial solutions but struggle with scale-dependent biases.

Emerging technologies like AI-based error correction and crowdsourced validation (e.g., OpenStreetMap edits) are mitigating these challenges. However, issues like subjective data interpretation and algorithmic biases in machine learning models introduce new uncertainties. For instance, automated feature extraction from DEMs may misclassify natural vs. artificial slopes without human oversight, risking engineering failures.

Ultimately, addressing accuracy and data quality issues demands a holistic approach: adopting standardized metadata protocols, prioritizing high-resolution datasets for critical applications, and investing in hybrid validation frameworks. As Burrough and McDonnell (1998) assert, “The value of a topographic system lies not in its data, but in the trustworthiness of its outputs.” Without resolving these foundational challenges, even the most advanced TIS risks generating misleading insights, undermining their role in sustainable development and resilience planning.

2.5	ADVANCEMENTS IN 3D TOPOGRAPHIC VISUALIZATION

The advent of 3D topographic visualization has redefined spatial interpretation, merging cutting-edge technologies like LiDAR (Light Detection and Ranging) and advanced 3D modeling to create immersive, high-fidelity representations of terrain. These innovations address the limitations of traditional 2D maps by providing dynamic, interactive views of landscapes, enhancing decision-making in fields ranging from urban planning to environmental conservation.

LiDAR technology lies at the core of this revolution, capturing sub-centimeter accuracy elevation data through pulsed laser measurements. Unlike traditional photogrammetry, LiDAR penetrates dense vegetation to map bare-earth topography, as demonstrated in Axelsson’s (2000) seminal work on DEM generation from laser scanner data. Airborne and terrestrial LiDAR systems generate dense point clouds, which are processed into Digital Surface Models (DSMs) and Digital Terrain Models (DTMs). These datasets form the foundation for 3D modeling, enabling the reconstruction of complex features like cliffs, river networks, and urban infrastructure with unprecedented precision. For example, the U.S. Geological Survey’s 3D Elevation Program (3DEP) leverages LiDAR to map floodplains and assess landslide risks, providing actionable insights for disaster management.

3D modeling software, such as Esri’s ArcGIS Pro, Blender, and CloudCompare, transforms raw LiDAR data into interactive visualizations. Techniques like texture mapping overlay satellite imagery onto 3D meshes, while voxel-based modeling adds volumetric depth to subsurface features like caves or aquifers. Structure-from-Motion (SfM) photogrammetry, as explored by Westoby et al. (2012), further democratizes 3D visualization by using drone-captured images to build cost-effective, high-resolution terrain models. These tools empower planners to simulate urban growth in 3D, as seen in Singapore’s Virtual Singapore project, which integrates building models with terrain data to optimize energy use and emergency response.

The integration of Virtual Reality (VR) and Augmented Reality (AR) has elevated 3D visualization from static models to immersive experiences. Platforms like Unreal Engine and Unity enable users to “walk through” virtual landscapes, assessing slope stability or sightlines for infrastructure projects. Archaeologists, for instance, use AR to overlay historical terrain changes onto modern landscapes, revealing ancient settlement patterns obscured by urbanization. Similarly, Google Earth Engine combines global DEMs with real-time data streams, allowing scientists to visualize glacier retreat or coastal erosion in 3D, fostering public engagement with climate science.

Despite these advancements, challenges persist. The sheer volume of LiDAR data strains storage and processing systems, necessitating cloud-based solutions like AWS LiDAR Serverless. Additionally, disparities in data resolution—such as mixing low-resolution satellite DEMs with high-resolution LiDAR—can create visual inconsistencies. Hugenholtz et al. (2013) caution that UAV-based SfM models may suffer from “doming” distortions without proper calibration, underscoring the need for rigorous validation.

Looking ahead, machine learning algorithms are automating feature extraction from 3D models, identifying landslide scars or archaeological sites with minimal human intervention (Zhu et al., 2017). Meanwhile, 4D visualization (3D + time) is emerging, enabling dynamic tracking of terrain changes, such as post-volcanic deformation or urban subsidence. As Longley et al. (2015) note, these tools are transforming topography from a static discipline into a living, interactive science.

In essence, 3D topographic visualization bridges the gap between raw geospatial data and human intuition, offering a lens to explore Earth’s complexity with unmatched clarity. By merging LiDAR’s precision with the creative potential of 3D modeling, it empowers industries to navigate the planet’s challenges—one pixel, and one polygon, at a time.

# CHAPTER THREE

3.1 	METHODOLOGY

The methodology adopted for this topographic survey involved the use of a Total Station to establish the perimeter, fix details, and capture spot heights for terrain representation. The science behind the process relies on geodetic principles, trigonometry, and electronic distance measurement (EDM) to determine precise locations, elevations, and feature positions. The Total Station, an advanced surveying instrument, integrates an electronic theodolite, EDM, and a data processor to measure angles and distances with high accuracy. The data collected was later processed using specialized software to produce topographic maps and elevation models.

3.2 	EQUIPMENT AND MATERIALS

The equipment used to carried out the Topographical Survey of Government Day Secondary School, Pakata are;

- Differential Global Positioning System (DGPS)

- Total Station (South Total Station)

- Prism

- Cutlass

- Survey Beacon

- Field Book

- Computer System

The primary instrument used was a Total Station, which operates on the principle of trigonometric distance measurement. It emits an infrared or laser beam that reflects off a prism placed at the target point. By analyzing the time taken for the signal to return, the Total Station calculates distances and, together with angular measurements, determines the precise position of each point. A tripod provided stability for the Total Station, ensuring measurement accuracy. A prism and prism pole were used as target reflectors, allowing the Total Station to lock onto and measure the position of survey points.

Benchmarks and control points provided a reference framework, ensuring all measurements were relative to a fixed coordinate system. A field book was used to manually record observations, sketches, and any necessary adjustments during the survey. Finally, data processing software such as AutoCAD Civil 3D and ArcGIS was employed to analyze and visualize the collected data, transforming it into usable topographic outputs.

3.3 	SURVEY PROCEDURE

The survey was conducted in a systematic manner to ensure precision. It involved pre-survey planning, control point establishment, perimeter measurement, detail fixing, spot height recording, and data processing.

3.3.1 	RECONNAISSANCE SURVEY

A reconnaissance survey is the preliminary stage of a topographic survey that involves gathering essential information about the site both in the office and in the field. The goal is to assess the terrain, identify potential challenges, plan the survey workflow, and ensure all necessary resources are available before the detailed survey begins.

Office Planning

The office reconnaissance involves researching existing data, reviewing maps, and planning the fieldwork to improve efficiency and accuracy. This step ensures that surveyors are well-prepared before stepping into the field.

1. Gathering Existing Data

- Available maps, and previous survey records of the area are collected and reviewed.

2. Identifying Control Points

- Existing geodetic control points are identified from national or local survey databases.

- Since control points are unavailable, plans are made to establish new ones within the survey area.

3. Planning Survey Workflow

- The survey approach, including the use of Total Stations, GPS, and leveling instruments, is determined.

- Team roles and responsibilities are assigned, and required equipment is listed.

- Potential challenges such as dense vegetation, water bodies, or built-up areas are noted for field adaptation.

4. Preparation of Field Materials

- Field sketches, coordinate lists, and necessary software for data collection are prepared.

- Batteries, tripods, and measuring tools are checked and calibrated.

Field Reconnaissance

The field reconnaissance involves physically visiting the site to verify office findings and assess real-time conditions that may impact the survey.

1. Site Visit and Visual Assessment

- The team walks through the survey area, confirming boundaries, terrain conditions, and accessibility.

- The presence of obstacles like water bodies, dense forests, or buildings is noted.

2. Verification of Control Points

- New control point locations are selected for proper coverage of the survey area.

3. Identifying Suitable Instrument Stations

- Locations for Total Station setups are chosen based on line-of-sight and coverage.

- Safe and stable positions for survey equipment are determined to minimize measurement errors.

3.3.2 	ESTABLISHMENT OF CONTROL POINTS

Control points are the foundation of any survey, providing fixed reference positions that all subsequent measurements rely upon. These points ensure that the survey maintains consistency and accuracy across the entire area. The process of establishing control points involves setting up known reference stations from which measurements of boundaries, details, and elevations can be made.

Scientific and Technological Basis

Control points are based on geodetic and trigonometric principles. The positions of these points are determined using total stations, ensuring their precise locations in relation to a coordinate system. The Total Station works by measuring the horizontal and vertical angles, along with the slope distances to reference targets, and calculating the X, Y, and Z coordinates using trigonometry and EDM (Electronic Distance Measurement).

Step-by-Step Procedure

1. Selection of Control Point Locations

- The first step is to identify existing geodetic control points near the site.

- Since, no existing control points are available, new ones are established in stable, accessible locations.

- The locations are selected based on visibility, accessibility, and security, ensuring that the points can be used throughout the survey.

- The equipment used for the establishment of the control point is Differential Global Positioning System (DGPS)

Table 3.1: Table showing the new established control points

| Control Station | Northing | Easting |
| --- | --- | --- |
| KP 001 | 938 774.336 | 666 705.731 |
| KP 002 | 938 668.328 | 666 518.319 |
| KP 003 | 938 665.326 | 666 559.394 |

2. Setting up the Total Station

- A tripod is placed over the control point and carefully leveled to ensure stability.

- The Total Station is mounted on the tripod and centered over the control point using the built-in optical plummet or laser plummet.

- Fine adjustments are made using the leveling screws and circular bubble to ensure accuracy.

3. Backsighting to a Known Point

- The instrument is aligned to a known reference point (geodetic benchmark or previously established control point).

- A prism is placed at the reference point, and a backsight measurement is taken to establish orientation.

KP

001

KP

002

KP

003

Backsight

Total Station

- This ensures that all subsequent measurements are tied to the correct coordinate system.

Fig. 3.1: Diagram showing the control station connection

4. Measurement of New Control Points

- The Total Station emits a laser or infrared beam toward the prism held at the target control point location.

- The instrument measures the angle and distance and automatically calculates the X, Y, and Z coordinates of the new control point.

- Multiple readings are taken for accuracy.

5. Checking and Adjustment of Control Points

- A closed traverse is conducted, ensuring that the last control point connects back to the first one, forming a loop.

- Any misclosure errors are adjusted using the Bowditch method or Least Squares Adjustment, ensuring consistency and reliability of control points.

These control points now serve as the reference framework for all subsequent survey operations.

Table 3.2: Table showing the coordinate of the control point after the orientation

| Control Station | Northing | Easting |
| --- | --- | --- |
| KP 001 | 938 774.214 | 666 705.643 |
| KP 002 | 938 668.328 | 666 518.319 |
| KP 003 | 938 665.200 | 666 559.254 |

3.3.3 	PERIMETER SURVEYING (BOUNDARY MEASUREMENT)

Once control points have been established, the next step is to determine the exact perimeter of the survey area. This involves measuring all boundary points to create an accurate map of the land extent.

Scientific and Technological Basis

Perimeter surveying relies on traverse computations and geospatial mapping. Using the Total Station, precise angular and distance measurements are taken from control points to define the boundary. The EDM system in the Total Station measures distances by calculating the travel time of an emitted laser beam, while the internal processor applies trigonometric calculations to determine the boundary points’ exact coordinates.

Step-by-Step Procedure

1. Setting Up the Total Station

- The Total Station is positioned at a control point and carefully leveled.

- A backsight measurement is taken to orient the instrument correctly to the established control network.

2. Marking Boundary Points

- The boundary points are identified in the field using physical markers (wooden pegs, iron rods, or concrete beacons).

- Each boundary corner is assigned a unique point number.

3. Measuring Boundary Points

- The prism reflector is placed at each boundary corner.

- The Total Station records horizontal angles, vertical angles, and slope distances to compute the exact location of each boundary point.

- If the distance is too far, the instrument is repositioned at a new control point to continue measurements.

4. Closing the Perimeter Traverse

- To ensure accuracy, the last boundary point is connected back to the first boundary point, creating a closed traverse.

- If errors exist in the traverse, adjustments are made using a traverse adjustment technique.

The perimeter survey ensures that the land boundaries are accurately mapped and legally verifiable.

3.3.4 	FIXING OF DETAILS

After defining the perimeter, the next step is to fix natural and man-made details within the site, such as buildings, roads, trees, drainage channels, power poles, fences, and water bodies.

Scientific and Technological Basis

Fixing details involves topographic feature mapping, using the Total Station to determine the precise locations of objects within the survey area. The coordinate geometry (COGO) principles and EDM technology allow for highly accurate spatial positioning of objects.

Step-by-Step Procedure

1. Total Station Setup at a Control Point

- The Total Station is placed at a known control point, and backsighting is performed for orientation.

2. Detail Point Identification

- A prism pole is placed at specific feature locations, such as corners of buildings, road edges, electric poles, and trees.

- For linear features such as roads and drainage channels, multiple points are measured along their alignment.

3. Measurement of Features

- The Total Station records angles, distances, and elevations, calculating the precise X, Y, and Z coordinates of each detail point.

- Each recorded feature is assigned a category, ensuring organized mapping.

4. Verification and Cross-Checking

- The recorded points are plotted on a real-time sketch or digital map to verify their correctness.

Fixing details ensures that all site features are mapped accurately for design, analysis, or documentation.

3.3.5 	SPOT HEIGHT MEASUREMENT

Spot height measurement is essential for understanding the elevation variations across the survey area. It involves recording the altitude (Z-coordinates) of multiple locations, which is crucial for terrain modelling, contour mapping, and engineering design.

Scientific and Technological Basis

Spot height measurement uses trigonometric leveling and terrain modeling principles. The Total Station calculates elevations by measuring vertical angles and slope distances to the prism, computing the precise height difference between the instrument and each point.

Step-by-Step Procedure

1. Setting Up the Total Station and Backsighting

- The instrument is placed on a control point with a known elevation (benchmark).

- A backsight is taken on another reference point to ensure correct height reference.

2. Grid-Based Spot Height Measurement

- A grid system is established, where spot heights are measured at uniform intervals across the site.

- The prism pole is placed at each grid point, and the Total Station records the elevation.

3. Recording Breakline Heights

- Additional spot heights are taken at breaklines (significant slope changes) such as road edges, embankments, and depressions.

4. Processing and Contour Mapping

- The collected elevations are used to generate contour lines and Digital Elevation Models (DEM), which visually represent the terrain.

Spot height measurements are essential for engineering design, flood analysis, and landscape assessment.

3.4 	DELIVERABLES AND OUTPUT

The final deliverables of the topographic survey included:

1. Topographic Map – A detailed representation of the survey area, including contour lines, elevation points, and fixed features.

2. Digital Elevation Model (DEM) – A 3D visualization of the terrain, useful for hydrological and engineering analysis.

3. GIS and CAD Files – Digital data compatible with GIS and CAD software for further analysis and integration into project designs.

CHAPTER FOUR

RESULTS AND ANALYSIS

This chapter presents the results derived from the topographic survey carried out using Total Station equipment. The survey generated key outputs that include the defined perimeter of the site, fixed planimetric features, measured spot heights, a topographic map, and a digital terrain model (DTM). These outcomes were analyzed to evaluate the spatial characteristics, elevation dynamics, and surface morphology of the project site. The interpretations presented in this chapter are intended to aid physical planning, civil engineering design, land assessment, and environmental appraisal.

4.1 	PERIMETER SURVEY RESULT

The first deliverable from the survey was the precise demarcation of the boundary enclosing the project area. Using Total Station equipment, angular and linear measurements were recorded along traverse lines connecting perimeter points. A closed traverse was executed, and adjustments were applied to eliminate angular and linear misclosures using Bowditch’s method. After corrections, coordinate computation yielded a total enclosed area of 5.746 hectares.

Each traverse station was referenced to a consistent datum, and coordinates were computed in a Universal Traverse Mercator (UTM) coordinate system. Bearings between stations and line distances were recorded with high accuracy, and redundant checks (backsight and foresight readings) confirmed data integrity.

Interpretation
The perimeter defines the legal footprint of the land parcel and forms the spatial foundation for all subsequent surveying and design activities. Its accuracy is critical for preventing encroachment and for ensuring compliance with planning laws. The 5.746-hectare area size is moderately expansive and suitable for mixed land uses, such as institutional development, housing schemes, or infrastructural expansion. The successful closure of the traverse with minimal misclosure values indicates a high level of precision in the control framework.

4.2 	DETAILED (PLANIMETRIC) SURVEY RESULT

Following perimeter control, planimetric detail features were fixed within the site. This involved the identification and coordinate measurement of all physical features, both natural and man-made. The detail survey recorded 21 existing buildings, all of which were measured using reflector-based Total Station shots to capture at least three to four defining corners per building. This allowed for accurate plotting of their geometries.

Additionally, two mature trees were observed within the site. These were recorded with their coordinates and plotted for spatial awareness and environmental relevance. The survey also documented the adjoining road infrastructure along the stretch between PL11 and PL1, providing vital context for access and transport planning.

Interpretation:
The detailed feature inventory is essential for base mapping, land-use classification, and infrastructure alignment. The presence of 21 structures indicates active or past land usage, which must be considered in any redevelopment or land allocation strategy. Their layout can also inform site zoning, access corridors, and utility planning. The inclusion of tree positions aids in environmental impact assessments and landscaping design. Moreover, the bordering road increases the site's connectivity and suitability for development by ensuring access and linking the site to external infrastructure networks.

Fig 4.1: Detailed Plan of the Study area

4.3 	SPOT HEIGHT MEASUREMENT AND ELEVATION ANALYSIS

Spot height measurements were obtained at regularly spaced intervals across the terrain to capture the vertical variation of the site. The spot heights ranged from 302.00 meters to 321.00 meters above mean sea level, indicating an elevation variation of 19 meters. Readings were taken at terrain breaks, slope transitions, and grid intersections to provide a full surface elevation dataset.

From the spot heights, contour lines were interpolated at 1.00-meter intervals, generating a continuous representation of elevation change across the site. The contours illustrate gradual and moderate slope patterns, with the steepest gradients located toward the northeastern section and relatively flatter terrain to the southwest.

Interpretation:
The elevation range reflects a moderate topographic undulation, which is advantageous for natural surface drainage, minimizing the risk of waterlogging. Slopes are not extreme, suggesting that grading requirements for construction will be minimal and cost-effective. The elevation profile also guides where best to site sensitive infrastructure such as roads, buildings, and stormwater channels. High points (up to 321m) are ideal for positioning critical structures, while lower points (around 302m) should be evaluated for potential runoff collection or water retention.

PLAN SHEWING THE SPOT HEIGHTS

OF

GOVERNMENT GIRLS DAY SECONDARY SCHOOL,

ALFA YAHAYA ROAD, PAKATA,

ILORIN WEST LOCAL GOVERNMENT AREA,

KWARA STATE.

ORIGIN: UTM ZONE 32N

AREA: - 5.764 HECTARES

SCALE: 1: 1,500

Fig 4.2: Spot Height Plan of the Study area

4.4 	TOPOGRAPHIC MAP RESULT

The topographic map produced combines all collected spatial data — perimeter, details, and elevation — into a georeferenced and scaled map. The map visually represents both the horizontal arrangement of features and the vertical variations in elevation. Buildings are symbolized with clearly marked outlines, trees are shown with natural feature symbols, and roads are represented with line features matching their real-world orientation.

Contour lines dominate the elevation representation, showing continuous terrain levels at 1.00-meter intervals. The map layout includes northing, easting, scale bar, legend, and title block to comply with cartographic standards.

Interpretation:
The topographic map provides a single-source spatial document that aids decision-making across various disciplines. For architects, it allows the preparation of conceptual site layouts; for engineers, it supports terrain grading and slope analysis; and for urban planners, it informs land zoning and development control. Its clarity and completeness make it useful as both a presentation and analytical tool. It also enables integration into GIS environments for further spatial modeling or overlay with satellite imagery and planning datasets.

CONTOUR PLAN

OF

GOVERNMENT GIRLS DAY SECONDARY SCHOOL,

ALFA YAHAYA ROAD, PAKATA,

ILORIN WEST LOCAL GOVERNMENT AREA,

KWARA STATE.

ORIGIN: UTM ZONE 32N

AREA: - 5.764 HECTARES

SCALE: 1: 1,500

Fig 4.3: Contour Map of the Study area

4.5 	DIGITAL TERRAIN MODEL (DTM) OUTPUT

Using the spot height dataset, a Digital Terrain Model (DTM) was generated to present the terrain as a continuous, 3D surface. The DTM was developed using interpolation techniques (e.g., Triangulated Irregular Network or Inverse Distance Weighting), and surface rendering was applied to visualize elevation differences, slope directions, and terrain curvature.

The DTM reveals subtle terrain changes not always visible on contour maps, such as micro-depressions or ridgelines. It is capable of supporting 3D visualization, fly-throughs, and terrain profiling, thereby enabling more advanced engineering applications.

Interpretation:
The DTM is a powerful tool for simulating hydrological flow, estimating earthwork quantities, and visualizing site development impact. It is especially useful for road design, cut-and-fill balancing, and modeling surface runoff. The 19-meter elevation range, when analyzed in the DTM, supports the identification of natural drainage paths and areas of potential erosion. The use of the DTM improves efficiency and reduces risks during planning and construction.

Fig 4.4: Digital Terrain Model Map of the Study area

4.6 	INTEGRATED SPATIAL INTERPRETATION AND SITE ASSESSMENT

From the compiled results, it is evident that the site exhibits a balanced terrain with mild slopes, active land use, and strong access potential due to the bordering road. The presence of several buildings suggests partial occupation or previous use, while the terrain characteristics support new development with minimal site modification.

The combination of perimeter survey, detailed feature fixing, spot height data, contour mapping, and digital modeling ensures a comprehensive understanding of the site. The overall dataset can be used for:

- Land-use planning and zoning (based on building distribution and access routes)

- Engineering design (based on elevation and slope analysis)

- Drainage and stormwater design (from DTM slope flow directions)

- Construction layout and supervision

Integration into a GIS-based land information system

CHAPTER FIVE

## COSTING, SUMMARY, PROBLEM ENCOUNTERED CONCLUSION AND RECOMMENDATIONS.

5.1	COST ESTIMATION

This stage the total cost spent on the project from day one to the final stage.

Table 5.1.1: Direct cost parameters

| S/N | DIRECT COST |  |  |
| --- | --- | --- | --- |
| 1 | Reconnaissance (1 day) |  |  |
|  | PERSONNEL | Rate | Total Amount (N:K) |
|  | 1 x surveyor | 10500 | 10500 |
|  | 1 x assistant surveyor | 7500 | 7500 |
|  | 1 x technical officer | 6500 | 6500 |
|  | 3 x skilled labour | 4000 | 12000 |
|  | basic equipment | 5000 | 5000:00 |
|  | transportation (vehicle, Driver and fuel) | 10000 | 10000:00 |
|  |  | sub-total | 51,500 |
| 2 | Monumentation (1 day) |  |  |
|  | PERSONNEL |  |  |
|  | 1 x assistant surveyor | 7500 | 7500 |
|  | 1 skilled labour | 4000 | 4000 |
|  | transportation (vehicle, Driver and fuel) | 10000 | 10000 |
|  | basic equipment | 5000 | 5000:00 |

|  | Beacon (6) | 1000 | 6000 |
| --- | --- | --- | --- |
|  |  | sub-total | 32,500 |
| 3 | Data acquisition (2 days) |  |  |
|  | PERSONNEL |  |  |
|  | 1 x assistant surveyor | 7500 | 7500 |
|  | 1 x technical officer | 6500 | 6500 |
|  | 3 x skilled labour | 4000 | 12000 |
|  | Total Station and accessories | 15000 | 15000 |
|  | transportation (vehicle, Driver and fuel) | 10000 | 10000 |
|  |  | sub-total | 51,000 |
| 5 | Data Processing (2 days) |  |  |
|  | PERSONNEL |  |  |
|  | 1 x surveyor | 10500 | 10500 |
|  | 1 x assistant surveyor | 7500 | 7500 |
|  | 1 x technical officer | 6500 | 6500 |
|  | computer / accessories | 15000 | 15000 |
|  |  | sub-total | 39,500 |
| 7 | Technical Report (1 day) |  |  |
|  | PERSONNEL |  |  |
|  | 1 x surveyor | 10500 | 10500 |
|  | 1 x assistant surveyor | 7500 | 7500 |

|  | 1 x secretary | 5000 | 5000:00 |
| --- | --- | --- | --- |
|  | computer / accessories | 15000 | 15000:00 |
|  |  | sub-total | 38,000 |

TOTAL (DIRECT COST) 212,500 MOBILIZATION/DEMOBILIZATION = 5% of Direct Cost

= 0.05 × 212,500

# = N10,625

CONTINGENCY ALLOWANCE

= 5% of Direct Cost

= 0.05 × 212,500

# = N10,225

# GRAND TOTAL     = N212,500 + N10,625 + N10,625

# = N233,750

VAT (@5% GRAND TOTAL)   = 0.05 × 233750

# = N11,687.5

TOTAL COST PAYABLE         = GRAND TOTAL + VAT

# = N233,750+ N11,687.5  	       = N245,437.5

## 5.2	SUMMARY

The application of Topographical Surveys (TIS) to the study of part of the Government Girls Day Secondary School, Pakata, Ilorin West (LGA), Kwara State, represents an important step towards modernizing urban planning, infrastructure development, and environmental management in the region. TIS, through the integration of advanced tools such as Digital Elevation Models (DEMs), satellite imagery, and geospatial data analysis, provides a comprehensive understanding of the terrain characteristics of the area, including elevation, slope, and landforms. This information is critical for planning and development in urban zones like the Government Girls Day Secondary School, where the population density is increasing, and there is a need for sustainable infrastructure development.

The topographic data derived from TIS can assist in a variety of applications, including flood modeling, land use planning, drainage system design, and risk assessment for potential natural hazards, particularly flooding due to the region’s topographic features and seasonal rainfall patterns. The detailed mapping and analysis of the area’s topography also provide insights into potential sites for future urban expansion and infrastructure projects while ensuring that the built environment is adapted to the natural terrain. Moreover, TIS can aid local authorities and stakeholders in creating policies that prioritize environmental sustainability, resilient urban infrastructure, and public safety, particularly in the face of climate change and population growth.

As the use of TIS in Government Girls Day Secondary School continues to grow, it will contribute to the integration of smart city technologies, including real-time data integration and IoT-driven solutions for more effective urban management. This could include the continuous monitoring of infrastructure health, soil conditions, and environmental changes. Additionally, the accurate terrain data generated by TIS will be instrumental in addressing challenges related to land tenure, property disputes, and infrastructure maintenance.

5.3      PROBLEM ENCOUNTERED

The following were the problems encountered during the execution of this project.

- The project was financially expensive, energetic and time consuming.

- On the day of carrying out the practical, there was a delay in getting the instrument that resulted in getting to the site lately.

- Throughout the practical, we worked under the sun reason that we got to the site lately.

5.4	CONCLUSION

In conclusion, the integration of Topographical Surveys (TIS) into the planning and development of Government Girls Day Secondary School, Pakata, Ilorin West (LGA), Kwara State, offers a transformative approach to understanding and managing the region’s complex topographic and environmental characteristics. By leveraging advanced geospatial tools such as Digital Elevation Models (DEMs), satellite imagery, and real-time data collection technologies, TIS provides crucial insights into the terrain, which are invaluable for urban planning, disaster management, and infrastructure development. The use of TIS in this area enables stakeholders to make informed decisions about land use, flood risk assessment, and the design of resilient infrastructure that is aligned with the natural landscape, helping to mitigate the risks associated with topographic challenges.

Furthermore, the potential of TIS to integrate with emerging technologies like AI, IoT, and virtual/augmented reality promises to enhance the system's capability for real-time monitoring and dynamic decision-making. This integration will be pivotal in addressing future challenges as the region continues to urbanize, ensuring that growth is sustainable and that urban systems can effectively adapt to environmental changes.

However, for TIS to reach its full potential in Government Girls Day Secondary School, it is essential that data accuracy, accessibility, and local expertise are prioritized. Moreover, addressing the ethical concerns related to data privacy and ensuring equitable access to TIS benefits are key considerations for the long-term success of this technology in the region. With proper implementation and continuous improvements in data integration, TIS will play a critical role in shaping a more sustainable, resilient, and efficient urban future for Ilorin West LGA, enhancing the quality of life for its residents and fostering more effective governance.

5.5	RECOMMENDATION

Here is the recommendation section for the topic "Topographical Surveys (TIS) of Government Girls Day Secondary School, Pakata, Ilorin West (LGA), Kwara State, in a listed format:

  - Enhance Data Accuracy and Resolution: It is recommended to improve the resolution and accuracy of topographic data by using advanced technologies such as LiDAR and UAV-based photogrammetry for more detailed mapping of terrain features in Government Girls Day Secondary School. This will provide more precise information for urban planning and flood risk assessments.

  - Integrate Real-Time Monitoring Systems: The integration of Internet of Things (IoT) sensors and other real-time monitoring systems within the TIS framework will enable continuous updates on environmental changes, infrastructure health, and disaster management. This will facilitate quicker decision-making and responses to dynamic urban challenges.

  - Adopt Advanced Visualization Technologies: Utilize Virtual Reality (VR) and Augmented Reality (AR) technologies to create immersive 3D models of the topography for urban planners, engineers, and the local community. This will improve stakeholder engagement, enhance understanding of the terrain, and aid in effective spatial decision-making.

  - Strengthen Capacity Building and Training: Provide training programs for local government officials, urban planners, and other stakeholders to build capacity in using TIS tools and interpreting topographic data. This will ensure better utilization of the system and foster local expertise in managing urban development challenges.

  - Promote Public-Private Partnerships: Foster collaborations between government agencies, private sector firms, and academic institutions to improve data collection, system development, and the long-term sustainability of TIS in the region. These partnerships can support funding, research, and the sharing of best practices.

REFERENCES

Amirebrahimi, S., Rajabifard, A., Mendis, P., & Ngo, T. (2016). A data model for integrating GIS and BIM for assessment and 3D visualization of flood damage to building infrastructure. International Journal of Digital Earth, 9(4), 363–386.

Billinghurst, M., Clark, A., & Lee, G. (2015). A Survey of Augmented Reality. Foundations and Trends in Human–Computer Interaction, 8(2-3), 73–272.

Burrough, P. A., & McDonnell, R. A. (1998). Principles of Geographical Information Systems. Oxford University Press.

Campbell, J. B., & Wynne, R. H. (2011). Introduction to Remote Sensing (5th ed.). Guilford Press.

Chen, Y., Yu, J., & Khan, S. (2009). Spatial sensitivity analysis of multi-criteria weights in GIS-based land suitability evaluation. Environmental Modelling & Software, 24(12), 1508–1515.

Colomina, I., & Molina, P. (2014). Unmanned aerial systems for photogrammetry and remote sensing: A review. ISPRS Journal of Photogrammetry and Remote Sensing, 92, 79–97.

Crampton, J. W. (2010). Mapping: A Critical Introduction to Cartography and GIS. Wiley-Blackwell.

Goodchild, M. F. (2018). Reimagining the History of GIS. Annals of GIS, 24(1), 1–8.

ISO (2003). ISO 19115: Geographic Information—Metadata. International Organization for Standardization

Kukutai, T., & Taylor, J. (Eds.). (2016). Indigenous Data Sovereignty: Toward an Agenda. ANU Press.

Li, S., Da Xu, L., & Zhao, S. (2020). 5G Internet of Things: A survey. Journal of Industrial Information Integration, 10, 1–9.

Longley, P. A., Goodchild, M. F., Maguire, D. J., & Rhind, D. W. (2015). Geographic Information Science and Systems. Wiley.

Nebert, D. D. (2004). Developing Spatial Data Infrastructures: The SDI Cookbook. GSDI Association.

Percivall, G. (2010). The role of open standards in geospatial interoperability. In Handbook of Research on Geoinformatics, IGI Global, 92–103.

Shekhar, S., & Xiong, H. (2008). Encyclopedia of GIS. Springer.

Turner, M. G., Gardner, R. H., & O'Neill, R. V. (2001). Landscape Ecology in Theory and Practice. Springer.

Wilson, J. P., & Gallant, J. C. (2000). Terrain Analysis: Principles and Applications. Wiley.

Zhu, X. X., Tuia, D., Mou, L., Xia, G. S., Zhang, L., Xu, F., & Fraundorfer, F. (2017). Deep Learning in Remote Sensing: A Comprehensive Review and List of Resources. IEEE Geoscience and Remote Sensing Magazine, 5(4), 8–36.

APPENDIX

GROUP 6A TOPO RAW DATA

| P.I.D EASTING NORTHING ELEVATION | P.I.D EASTING NORTHING ELEVATION |
| --- | --- |
| 1,PT,666559.393,938655.226,324.403 2,BLD,666564.888,938655.589,324.609 3,BLD,666572.856,938656.418,324.845 4,BLD,666579.384,938656.990,325.081 5,BLD,666587.417,938657.779,325.249 6,FF,666584.030,938674.217,324.847 7,FF,666578.461,938674.102,324.602 8,SH,666562.850,938672.506,324.086 9,SH,666548.780,938675.012,323.749 10,SH,666529.505,938681.031,323.445 11,WF,666509.740,938692.262,322.828 12,GT,666501.195,938679.925,323.593 13,GT,666500.602,938679.080,323.629 14,WF,666494.023,938669.408,323.585 15,PIL,666489.939,938663.250,323.784 16,SH,666504.839,938663.969,323.640 17,SH,666518.319,938668.328,323.512 18,SH,666534.636,938665.045,323.618 19,SH,666537.246,938651.342,324.351 20,SH,666537.116,938640.217,324.791 21,PIL,666549.802,938626.341,325.710 22,BLD,666566.949,938635.507,325.534 23,SH,666560.052,938640.072,325.173 24,SH,666551.407,938652.598,324.272 25,SH,666550.568,938689.169,323.494 26,SH,666538.375,938698.658,323.125 27,SH,666518.282,938705.519,322.835 28,TR,666530.522,938714.804,323.065 29,BLD,666532.985,938723.275,322.886 30,BLD,666535.020,938722.153,323.014 31,BLD,666536.550,938724.511,323.043 32,BLD,666537.444,938726.038,323.111 33,BLD,666538.628,938728.344,323.073 34,TR,666547.428,938716.448,323.439 35,SH,666561.960,938714.995,323.735 36,SH,666577.398,938714.165,323.962 37,SH,666588.013,938713.553,324.333 38,SH,666594.902,938733.181,324.533 39,FF,666585.859,938745.789,324.080 40,FF,666580.138,938746.859,324.004 41,SH,666567.418,938742.689,323.674 42,BLD,666562.092,938744.078,323.581 43,BLD,666553.297,938748.849,323.116 44,WF,666548.166,938751.757,323.022 45,BLD,666573.843,938764.473,323.318 46,SH,666582.419,938761.025,323.802 47,SH,666592.201,938757.277,323.907 48,SH,666599.902,938752.070,324.093 49,SH,666602.626,938762.939,324.184 50,SH,666596.583,938765.303,324.118 51,SH,666588.891,938769.861,323.589 52,SH,666576.869,938779.026,323.581 53,SH,666579.252,938790.592,323.338 54,PIL,666578.121,938796.557,323.355 55,WF,666591.126,938795.641,323.822 56,SH,666599.788,938791.439,324.047 57,SH,666596.692,938780.969,323.967 58,SH,666606.748,938775.310,324.247 59,BLD,666616.773,938784.907,324.987 60,BLD,666617.378,938787.730,325.240 61,WF,666617.119,938792.478,324.683 62,BLD,666619.559,938784.367,325.202 63,BLD,666622.361,938785.821,325.247 64,BLD,666621.226,938776.323,324.742 65,SH,666620.229,938765.813,325.022 66,SH,666612.733,938767.946,324.700 67,SH,666611.600,938753.333,324.728 68,SH,666612.441,938739.964,324.961 69,SH,666615.825,938724.599,325.333 70,TR,666621.296,938715.292,325.998 71,SH,666610.096,938713.356,325.410 72,SH,666609.137,938700.165,325.546 73,SH,666611.831,938687.882,325.848 74,SH,666615.842,938679.399,326.144 75,SH,666616.033,938674.189,326.252 76,BLD,666615.837,938662.628,326.442 77,SH,666605.260,938663.643,326.041 78,BLD,666593.748,938659.167,325.634 79,SH,666592.892,938667.768,325.400 80,BLD,666619.597,938663.260,326.505 81,BLD,666639.099,938665.984,327.375 82,BLD,666643.840,938666.654,327.485 83,BLD,666665.520,938669.765,328.330 84,BLD,666668.596,938670.268,328.448 85,BLD,666690.662,938673.491,329.178 86,BLD,666693.533,938675.060,329.155 87,BLD,666691.985,938685.179,328.896 88,SH,666692.573,938693.992,328.445 89,SH,666695.917,938702.946,328.240 90,BLD,666700.844,938707.759,328.343 91,BLD,666709.733,938708.123,328.623 92,SH,666710.532,938699.058,328.854 93,SH,666712.030,938688.499,329.226 94,SH,666693.790,938710.254,327.952 95,SH,666686.278,938708.401,327.550 96,SH,666671.562,938706.433,327.388 97,SH,666672.225,938694.999,327.675 98,SH,666676.052,938681.708,328.054 99,SH,666662.792,938677.496,327.654 100,SH,666652.241,938687.813,327.079 101,SH,666644.262,938681.957,327.057 102,SH,666638.083,938670.981,327.071 103,SH,666628.032,938668.980,326.716 104,SH,666630.313,938687.812,326.474 105,SH,666631.197,938704.013,326.052 106,SH,666642.761,938708.216,326.404 107,EP,666656.192,938706.350,326.926 108,SH,666661.644,938716.765,326.692 109,FF,666667.418,938725.667,326.569 110,FF,666667.138,938728.920,326.538 111,SH,666685.351,938726.847,327.195 112,BLD,666699.212,938737.115,327.714 113,BLD,666698.524,938740.612,327.598 114,SH,666693.983,938749.480,327.262 115,BLD,666697.372,938760.585,327.093 116,BLD,666700.240,938766.370,327.154 117,SH,666688.669,938758.316,326.945 118,SH,666677.388,938760.922,326.541 119,BLD,666679.895,938768.787,326.580 120,SH,666667.611,938770.000,326.252 121,BLD,666655.808,938771.721,325.814 122,BLD,666648.235,938772.767,325.569 123,BLD,666643.650,938773.528,325.455 124,WT,666640.280,938769.175,325.522 | 125,WT,666639.790,938767.687,325.569 126,WT,666641.358,938766.991,325.575 127,SH,666634.854,938764.789,325.446 128,SH,666637.255,938755.218,325.526 129,SH,666648.479,938750.478,326.051 130,SH,666651.677,938748.545,326.060 131,SH,666651.176,938743.616,325.941 132,SH,666653.487,938743.635,326.074 133,EP,666664.619,938749.247,326.356 134,SH,666661.779,938738.151,326.352 135,SH,666650.737,938734.922,326.119 136,SH,666638.825,938734.261,325.675 137,FF,666627.213,938729.227,325.561 138,FF,666627.154,938726.027,325.519 139,FF,666652.971,938722.396,326.084 140,FF,668126.774,939305.647,371.880 141,CP,666731.883,938714.114,329.118 142,CP,666722.880,938709.571,329.095 143,CP,666731.861,938714.103,329.090 144,BLD,666719.330,938776.283,328.847 145,BLD,666724.640,938775.642,329.538 146,BLD,666724.811,938777.345,329.013 147,WF,666727.860,938779.930,328.852 148,SH,666729.117,938765.766,328.551 149,SH,666713.970,938764.828,328.315 150,SH,666705.731,938774.336,328.143 151,WF,666703.715,938782.214,328.655 152,BLD,666707.042,938760.928,328.164 153,SH,666714.641,938751.563,328.524 154,SH,666711.044,938743.638,328.569 155,BLD,666708.008,938741.096,329.147 156,BLD,666708.301,938737.492,329.393 157,SH,666719.039,938737.120,328.664 158,SH,666728.749,938734.462,328.506 159,SH,666741.792,938741.734,328.911 160,SH,666747.719,938751.513,328.909 161,BLD,666740.177,938763.414,328.789 162,SH,666749.275,938757.111,328.961 163,BLD,666759.775,938762.004,329.385 164,SH,666759.454,938748.967,329.185 165,SH,666751.653,938739.045,329.162 166,SH,666753.335,938727.717,329.548 167,SH,666755.412,938716.761,329.795 168,SH,666756.582,938710.874,329.914 169,BLD,666769.016,938706.899,330.306 170,BLD,666749.991,938703.089,329.885 171,BLD,666746.416,938702.440,329.791 172,BLD,666727.171,938698.381,329.546 173,BLD,666729.017,938689.493,329.729 174,PIL,666727.095,938688.956,330.002 175,PIL,666728.798,938676.309,330.773 176,PIL,666714.192,938674.035,330.810 177,BLD,666713.774,938676.953,330.512 178,BLD,666713.612,938677.901,330.458 179,BLD,666712.298,938688.217,329.882 180,SH,666721.299,938676.028,330.531 181,SH,666721.212,938690.196,329.749 182,SH,666717.543,938705.002,329.084 183,SH,666719.324,938719.819,328.830 184,BLD,666772.395,938707.875,330.401 185,BLD,666791.143,938711.646,330.850 186,BLD,666796.208,938712.817,331.437 187,BLD,666815.736,938717.126,331.687 188,SH,666813.009,938726.677,331.091 189,SH,666800.600,938724.265,330.640 190,SH,666785.915,938721.751,330.486 191,SH,666772.111,938719.587,330.221 192,SH,666757.289,938716.674,329.897 193,SH,666753.103,938726.211,329.572 194,SH,666764.504,938730.925,329.584 195,SH,666776.799,938733.664,329.930 196,SH,666787.651,938734.839,330.088 197,SH,666800.791,938739.366,330.340 198,SH,666809.360,938743.631,330.602 199,SH,666824.472,938747.742,330.967 200,SH,666814.001,938751.846,330.764 201,SH,666797.995,938745.083,330.206 202,SH,666785.156,938745.079,329.911 203,SH,666777.323,938745.625,329.701 204,SH,666762.019,938747.240,329.348 205,SH,666753.730,938750.513,329.096 206,BLD,666764.031,938761.616,329.347 207,BLD,666796.632,938759.832,330.269 208,BLD,666806.074,938758.626,330.484 209,BLD,666810.530,938757.750,330.603 210,CP,666837.386,938731.730,332.055 211,CP,666829.131,938729.840,331.835 212,CP,666837.322,938731.715,332.032 213,GT,666843.144,938741.280,331.875 214,GT,666842.294,938746.971,331.905 215,GH,666839.370,938746.798,331.704 216,GH,666838.857,938750.142,331.727 217,PIL,666839.251,938771.826,331.955 218,BLD,666831.234,938766.354,331.569 219,BLD,666830.371,938756.749,331.569 220,TR,666832.903,938751.431,331.596 221,TR,666805.617,938750.605,331.361 222,TR,666790.209,938742.444,331.555 223,BLD,666786.825,938760.003,331.241 224,BLD,666783.240,938760.270,331.293 225,EP,666810.029,938738.888,331.489 226,BLD,666817.703,938707.898,332.618 227,BLD,666818.554,938700.375,332.887 228,BLD,666839.493,938700.429,333.542 229,WF,666813.858,938682.646,333.753 230,SH,666812.465,938693.685,333.032 231,SH,666817.384,938702.759,332.763 232,SH,666823.585,938706.568,332.769 233,SH,666827.141,938719.144,332.147 234,SH,666837.287,938714.607,332.603 235,SH,666847.470,938710.193,332.886 236,SH,666849.643,938698.050,333.158 237,SH,666851.768,938687.648,333.427 238,SH,666845.065,938725.354,332.337 239,SH,666843.736,938733.935,332.005 240,SH,666834.883,938741.786,331.707 241,SH,666825.509,938741.317,331.440 242,SH,666816.523,938749.139,331.327 243,SH,666815.465,938738.232,331.351 244,SH,666808.713,938719.009,331.950 245,SH,666797.606,938718.224,331.998 246,SH,666803.858,938715.008,332.217 247,SH,666790.493,938713.012,332.421 248,SH,666775.471,938711.294,332.557 |