DESIGN AND CONSTRUCTION OF SIMPLY SUPPORTED BEAM CARRYING A CANTILEVER SLAB WITH A DETACH STAIRCASE

BY

BABALOLA MICHEAL BABATUNDE

ND/23/BLD/PT/0035

A PROJECT SUBMITTED TO THE DEPARTMENT OF BUILDING TECHNOLOGY, INSTITUTE OF ENVIRONMENTAL STUDIES.

IN PARTIAL FULFILLMENT OF THE REQUIREMENT FOR THE AWARD OF NATIONAL DIPLOMA (ND) IN BUILDING TECHNOLOGY, KWARA STATE POLYTECHNIC, ILORIN.

SUPERVISED BY BDLR. IYIOLA AKEEM

JULY, 2025.

CERTIFICATION

This is to certify that this project has been read and approved as meeting part of the requirements for the award of National Diploma (ND) in Building Technology in the Department of Building Technology and Management, Institute of Environmental Studies (IES), Kwara State Polytechnic, Ilorin.

--------------------------					--------------------------

HRH. OWOLABI, M.K.						DATE

(Project Supervisor)

--------------------------					--------------------------

BLDR. AKANO AYODELE (MNIOB)				DATE

(Project Coordinator)

--------------------------					--------------------------

BLDR. ALEGE ABDULGANIYU (MNIOB	)			DATE

Head of Department (HOD)

--------------------------					--------------------------

BLDR. AHMED MAROOF FUNSHO (MNIOB)			DATE

External Examiner

DEDICATION

This project is dedicated to Almighty God, the Alpha and Omega of wisdom, knowledge and understanding. The creator of all living and non-living soul also dedicated to my parents MR & MRS. BABALOLA.

ACKNOWLEDGMENTS

First and foremost, I give golden gratitude and adoration to the Almighty God, the most beneficent and most merciful who alone is the source of wisdom, knowledge for signs and inspiration for making this vision a reality.

We would like to express my deep and sincere gratitude to our project supervisor HRH. OWOLABI, M.K for giving me the opportunity to do research and providing invaluable guidance throughout this research. This dynamism, vision, sincerity and motivation have deeply inspired me. He has taught me through methodology to carry out the research and to present the research work as clearly as possible.

This acknowledgment will not be completed without the recognition of our HOD of the Department in person of BLDR ABDULGANIYU ALEGE (MNIOB) for his encouragement and commitment to the service of humanity. I appreciate the efforts of all the lecturers of the Department of Building Technology, thank you and God bless you all.

### TABLE OF CONTENTS

### Title page										i

### Certification										ii

### Dedication										iii

### Acknowledgements									iv

### Table of Contents									v

### Chapter One: Introduction

#### 1.1	Background of the Study							1

#### 1.2 	Statement of the Problem

#### 1.3 	Objectives of the Study

#### 1.4 	Research Questions

#### 1.5 	Significance of the Study

#### 1.6 	Scope of the Study

1.7 	Definition of Terms

Chapter Two: Literature Review

2.1	Introduction

2.2 	A white out shelf for making home use furniture

2.3 	Materials used for white out shelf

2.4 	Explanation on making the white out shelf

2.5 	Procedure of production

Chapter Three: Research Methodology

3.0	Introduction

3.1	Research Design

3.2	Research Instrument: Questionnaire

3.3	Sources of Data Collection

3.4	Techniques of Data Analysis

3.5	Material used for Practical Work

Chapter Four: Analysis and Drawings

4.0	Introduction

4.2	Drawings and Sketches

Chapter Five: Summary, Conclusion and Recommendations

5.1	Summary of Findings

5.2	Conclusion

5.3	Recommendations

References

## CHAPTER ONE

### INTRODUCTION

### 1.1	Background of the Study

A simply supported beam is a fundamental structural element in building construction, characterized by its support at two points, typically at the ends, allowing it to carry loads primarily through bending. When combined with a cantilever slab, which extends beyond its support without underlying columns, it provides a versatile structural system for creating overhangs or balconies. A detached staircase, as an independent structural component, enhances accessibility and aesthetic appeal in buildings, often designed to complement the main structure while maintaining structural integrity. This research focuses on the design and construction of a simply supported beam carrying a cantilever slab with a detached staircase, emphasizing structural efficiency, safety, and cost-effectiveness.

The integration of these structural elements is critical in modern construction, particularly in residential and commercial buildings where space optimization and aesthetic flexibility are paramount. According to Eurocode 2 (EN 1992-1-1), the design of reinforced concrete beams and slabs must account for bending moments, shear forces, and deflection limits to ensure structural stability. Cantilever slabs, due to their unsupported ends, require careful consideration of load distribution and reinforcement to prevent excessive deflection or cracking (Mosley et al., 2012). Similarly, detached staircases, often constructed from reinforced concrete or steel, must be designed to withstand dynamic loads while adhering to ergonomic standards for user safety (BS 5395-1).

In Nigeria, the construction industry faces challenges such as material availability, cost overruns, and adherence to local building codes, which influence the design and construction of structural elements like beams, slabs, and staircases. The adoption of sustainable materials and efficient construction techniques is increasingly vital to address these challenges (Ogunbiyi, 2014). This study aims to explore the design and construction processes, material selection, and practical implementation of a simply supported beam carrying a cantilever slab with a detached staircase, contributing to improved construction practices in Nigeria.

### 1.2	Statement of the Problem

The Nigerian construction industry often relies on traditional design and construction methods, which may not fully address modern demands for structural efficiency, safety, and sustainability. The design of simply supported beams carrying cantilever slabs is complex due to the need to balance load distribution, reinforcement detailing, and deflection control. Detached staircases, while aesthetically appealing, pose challenges in ensuring structural independence and user safety. Common issues include inadequate reinforcement, poor material quality, and lack of adherence to design standards, leading to structural failures or increased costs. Additionally, there is a lack of localized guidelines for integrating these elements in a cohesive structural system, necessitating research to address these gaps.

### 1.3	Objectives of the Study

The main aim of this study is to design and construct a simply supported beam carrying a cantilever slab with a detached staircase, ensuring structural integrity, safety, and cost-effectiveness.

Specific objectives include:

- To design a simply supported beam and cantilever slab that meet structural and safety requirements.

- To select appropriate materials for the construction of the beam, slab, and detached staircase.

- To develop a construction procedure for integrating a detached staircase with the beam and slab system.

- To assess the structural performance and cost-effectiveness of the proposed design.

- To develop guidelines for designing and constructing similar structural systems in Nigeria.

### 1.4	Research Questions

- What are the design considerations for a simply supported beam carrying a cantilever slab?

- What materials are most suitable for constructing a simply supported beam, cantilever slab, and detached staircase?

- What challenges are encountered during the construction of these structural elements?

- How does the integration of a detached staircase affect the overall structural performance?

### 1.5	Significance of the Study

This project demonstrates the competency of students in applying structural engineering principles to real-world construction challenges. It provides practical insights into designing and constructing a simply supported beam with a cantilever slab and a detached staircase, serving as a foundation for future research and improving construction practices in Nigeria. The study will also contribute to the development of localized guidelines, enhancing the quality and safety of structural designs in the building technology department.

### 1.6	Scope of the Study

The scope of this project includes:

- Designing and analyzing a simply supported beam carrying a cantilever slab using relevant standards (e.g., Eurocode 2).

- Constructing a prototype of the beam, slab, and detached staircase using locally available materials.

- Evaluating the structural performance, cost, and construction challenges.

- Developing guidelines for similar structural designs in Nigeria.

### 1.7	Definition of Terms

- Simply Supported Beam: A beam supported at two points, typically at its ends, allowing it to resist loads through bending.

- Cantilever Slab: A reinforced concrete slab that extends beyond its support, without underlying columns, designed to resist bending and shear forces.

- Detached Staircase: An independent staircase, structurally separate from the main building, designed for accessibility and aesthetic purposes.

- Structural Integrity: The ability of a structure to withstand loads without failure.

- Reinforcement: Steel bars or mesh embedded in concrete to enhance its tensile strength.

- Load Distribution: The manner in which forces are transferred through a structural system.

# CHAPTER TWO

## LITERATURE REVIEW

### 2.1	Introduction

The design and construction of structural elements such as simply supported beams, cantilever slabs, and detached staircases are pivotal to modern building technology, enabling the creation of safe, efficient, and aesthetically pleasing structures. Advances in structural engineering, material science, and construction methodologies have significantly enhanced the ability to develop robust structural systems that meet both functional and environmental demands. This literature review provides a comprehensive exploration of the theoretical and practical aspects of designing and constructing a simply supported beam carrying a cantilever slab with a detached staircase. The review focuses on key areas: design principles, material selection, construction processes, and associated challenges. It synthesizes information from peer-reviewed journals, industry standards (e.g., Eurocode 2, BS 5395-1), and case studies to establish a robust foundation for the study. By examining global and local perspectives, particularly within the Nigerian construction context, this review aims to address gaps in knowledge and practice, contributing to improved structural design and construction methodologies.

The integration of simply supported beams, cantilever slabs, and detached staircases is common in residential and commercial buildings, particularly where space optimization, structural efficiency, and aesthetic appeal are priorities. The review is structured to cover the conceptual framework of these structural elements, the materials used, the design and construction processes, and the specific procedures for constructing a detached staircase. Emphasis is placed on sustainability, cost-effectiveness, and adherence to international and local standards, with a focus on addressing challenges such as material quality, labor skills, and compliance with building codes in Nigeria.

### 2.2	Concept of Simply Supported Beams and Cantilever Slabs

A simply supported beam is a fundamental structural element supported at its ends, typically by pinned or roller supports, allowing it to resist vertical loads primarily through bending (Mosley et al., 2012). The beam transfers applied loads—such as dead loads (e.g., self-weight, finishes) and live loads (e.g., occupants, furniture)—to its supports, with maximum bending moments occurring at the midspan and maximum shear forces near the supports. According to Eurocode 2 (EN 1992-1-1), the design of simply supported beams requires careful calculation of bending moments, shear forces, and deflection to ensure structural stability and compliance with serviceability limits.

Cantilever slabs, in contrast, are reinforced concrete flat plates that extend beyond their supporting elements (e.g., beams or walls) without additional columns, creating overhangs for balconies, terraces, or aesthetic features. These slabs experience negative bending moments at the fixed end, necessitating reinforcement at the top surface to counter tensile stresses (MacGinley & Choo, 2018). The integration of a simply supported beam with a cantilever slab is a common structural configuration in modern buildings, particularly in urban settings where space constraints demand innovative solutions. The beam provides primary support, while the cantilever slab extends the floor area, enhancing functionality and aesthetic appeal.

Detached staircases, designed as independent structural units, complement such systems by providing access without relying on the main structure for support. These staircases must adhere to ergonomic and safety standards, such as BS 5395-1, which specifies requirements for rise, run, and landing dimensions to ensure user comfort and safety. The structural design of detached staircases must account for dynamic loads (e.g., human traffic) and ensure stability through independent foundations and reinforcement (Chudley & Greeno, 2016). The combination of these elements—beam, slab, and staircase—requires a cohesive design approach to ensure compatibility, load transfer, and overall structural integrity.

The literature highlights the importance of integrating these elements to meet modern construction demands. For instance, Ogunbiyi (2014) notes that in Nigeria, the use of reinforced concrete for beams and slabs is prevalent due to its durability and availability, but challenges such as poor workmanship and material variability often compromise structural performance. Globally, advancements in finite element analysis and structural software (e.g., SAP2000, ETABS) have improved the precision of designing such systems, enabling engineers to model complex load interactions and optimize reinforcement (Mosley et al., 2012).

### 2.3	Materials Used in Beam, Slab, and Staircase Construction

The construction of simply supported beams, cantilever slabs, and detached staircases relies on a range of materials selected for their structural properties, availability, and sustainability. The primary materials include concrete, steel reinforcement, formwork materials, and aggregates, each playing a critical role in ensuring the durability and performance of the structural system.

#### Concrete

Concrete, a composite material composed of cement, fine aggregates (sand), coarse aggregates (gravel or crushed stone), and water, is widely used for its high compressive strength and versatility. In Nigeria, Grade C25/30 concrete, with a characteristic compressive strength of 25 MPa (cube strength) or 30 MPa (cylinder strength), is commonly used for residential structures due to its balance of strength and cost (Ogunbiyi, 2014). The concrete mix design must adhere to standards like BS EN 206 to ensure workability, strength, and durability. Additives, such as plasticizers, may be used to improve workability, particularly for cantilever slabs requiring precise placement to avoid segregation (Neville, 2011).

#### Steel Reinforcement

Steel reinforcement, typically high-yield deformed bars (e.g., Y16, Y12 with a yield strength of 460 MPa), is embedded in concrete to provide tensile strength, which is critical for resisting bending in beams and cantilever slabs. According to Eurocode 2, reinforcement must be adequately spaced and anchored to prevent bond failure or cracking (Mosley et al., 2012). For cantilever slabs, additional top reinforcement is required to counter negative bending moments, while beams need stirrups (e.g., Y8 links) to resist shear forces. In Nigeria, locally produced steel bars are common, but quality inconsistencies necessitate rigorous testing to ensure compliance with standards like BS 4449 (Ogunbiyi, 2014).

#### Formwork Materials

Formwork is essential for shaping concrete during casting and is typically made from timber (e.g., 2x4 planks) or plywood (e.g., 18mm marine plywood). Timber formwork is cost-effective and widely available in Nigeria, but it requires proper treatment to prevent warping or moisture absorption (Chudley & Greeno, 2016). Plywood, with its smooth surface, is preferred for cantilever slabs and staircases to achieve a high-quality finish. Reusable formwork systems, such as steel or aluminum molds, are gaining popularity in large-scale projects due to their durability and precision, though their high cost limits adoption in smaller projects (MacGinley & Choo, 2018).

#### Sand and Aggregates

Locally sourced sand and granite aggregates are critical components of concrete mixes. Fine aggregates (sand) provide workability, while coarse aggregates (granite) enhance strength. The quality of aggregates significantly affects concrete performance, with poorly graded or contaminated aggregates leading to reduced strength and durability (Neville, 2011). In Nigeria, challenges such as inconsistent aggregate quality and limited access to standardized testing facilities can compromise structural outcomes (Ogunbiyi, 2014). Quality control measures, such as sieve analysis and slump tests, are essential to ensure compliance with BS 882 standards.

#### Sustainability Considerations

Sustainability is a growing concern in construction, with an emphasis on using locally sourced materials to reduce transportation-related carbon emissions. Recycled aggregates and supplementary cementitious materials (e.g., fly ash, slag) are increasingly explored to enhance sustainability (Cordella et al., 2025). In Nigeria, the use of locally produced cement and aggregates aligns with economic and environmental goals, but challenges such as inconsistent material quality and limited recycling infrastructure persist (Ogunbiyi, 2014). The adoption of green concrete technologies, such as low-carbon cement, could further enhance the sustainability of structural elements like beams, slabs, and staircases.

#### Challenges in Material Selection

The literature identifies several challenges in material selection, particularly in the Nigerian context. Inconsistent material quality, due to unregulated suppliers and limited testing facilities, can lead to structural weaknesses (Ogunbiyi, 2014). Additionally, the high cost of imported materials, such as high-quality steel or specialized formwork, poses financial constraints for small-scale projects. Environmental factors, such as high humidity and temperature variations in Nigeria, can affect concrete curing and timber formwork durability, necessitating careful material selection and construction practices (Neville, 2011).

### 2.4	Design and Construction Process of Simply Supported Beams and Cantilever 	Slabs

The design and construction of simply supported beams and cantilever slabs involve a systematic process to ensure structural integrity, safety, and compliance with standards. The process is divided into design and construction phases, each requiring careful consideration of loads, materials, and execution techniques.

#### Design Process

The design of a simply supported beam and cantilever slab begins with load analysis, as specified in Eurocode 1 (EN 1991-1-1). Loads include dead loads (e.g., self-weight, finishes), live loads (e.g., occupants, furniture), and environmental loads (e.g., wind). The beam is designed to resist bending moments and shear forces, with reinforcement calculated using Eurocode 2 formulas for ultimate limit state (ULS) and serviceability limit state (SLS). For example, the maximum bending moment for a simply supported beam under uniformly distributed load (UDL) is given by:

[ M = \frac{wL^2}{8} ]

where ( w ) is the load per unit length and ( L ) is the span. The cantilever slab, however, experiences negative bending moments at the fixed end, requiring top reinforcement, calculated as:

[ M = \frac{wL^2}{2} ]

where ( L ) is the cantilever length (Mosley et al., 2012). Structural analysis software, such as STAAD.Pro or ETABS, is often used to model complex load interactions and optimize reinforcement layouts. Deflection checks are critical, with Eurocode 2 limiting deflection to ( L/250 ) for serviceability (MacGinley & Choo, 2018).

#### Construction Process

The construction process involves several stages:

- Formwork Preparation: Timber or plywood formwork is erected to shape the beam and slab. For cantilever slabs, formwork must be securely propped to prevent collapse during casting (Chudley & Greeno, 2016).

- Reinforcement Placement: Steel bars (e.g., Y16 main bars, Y8 stirrups) are placed according to design specifications, with proper cover (e.g., 25mm) to protect against corrosion.

- Concrete Casting: Concrete is mixed (e.g., 1:2:4 ratio for C25/30) and poured using a concrete mixer, followed by compaction with a vibrator to eliminate voids.

- Curing: The concrete is cured for at least 7-14 days to achieve design strength, using methods like water curing or curing compounds (Neville, 2011).

- Formwork Removal: Formwork is removed after the concrete reaches sufficient strength (typically 70% of design strength), ensuring no damage to the structure.

Cantilever slabs require additional care during construction due to their unsupported ends, with temporary props used to support formwork until the concrete sets. Quality control, including slump tests and cube tests, is essential to verify concrete strength (BS EN 12390).

#### Challenges

Challenges in the design and construction process include inaccurate load estimation, inadequate reinforcement detailing, and poor workmanship, which can lead to cracking or failure (Ogunbiyi, 2014). In Nigeria, limited access to advanced software and skilled labor further complicates the process, necessitating simplified design methods and training programs.

### 2.5	Procedure for Constructing a Detached Staircase

The construction of a detached staircase involves a systematic procedure to ensure structural stability, safety, and aesthetic appeal. The following steps outline the process, based on industry standards and practices (Chudley & Greeno, 2016; BS 5395-1):

Step 1: Design

The design of the detached staircase begins with determining ergonomic dimensions, such as rise (150-200mm), run (250-300mm), and landing width (minimum 900mm), per BS 5395-1. The staircase must be designed to withstand dynamic loads (e.g., 1.5 kN/m² for residential use) and ensure user safety through handrails and non-slip surfaces. Structural analysis is conducted to calculate reinforcement requirements, typically using Y12 bars for main reinforcement and Y8 links for shear.

#### Step 2: Foundation

A concrete foundation, typically a strip or pad footing, is cast to support the staircase. The foundation is designed to resist settlement and transfer loads to the ground, with a minimum depth of 600mm in stable soils (Ogunbiyi, 2014). Concrete grade C20/25 is often used for foundations, with reinforcement to prevent cracking.

#### Step 3: Formwork and Reinforcement

Timber or plywood formwork is constructed to shape the staircase, including steps, risers, and landings. The formwork must be rigid and watertight to prevent concrete leakage. Steel reinforcement is placed according to design specifications, with main bars running along the staircase length and links providing shear resistance. Proper anchorage and lap lengths are ensured to maintain structural continuity (Mosley et al., 2012).

#### Step 4: Concrete Casting

Concrete is mixed and poured into the formwork, typically using a 1:2:4 mix for C25/30 concrete. A concrete vibrator is used to compact the mix, ensuring a dense and void-free structure. The casting process is carefully monitored to maintain consistent levels and avoid cold joints.

#### Step 5: Curing

The staircase is cured for 7-14 days to achieve design strength, using water curing or curing membranes to prevent cracking due to rapid drying. In Nigeria’s hot climate, curing is critical to prevent shrinkage cracks (Neville, 2011).

#### Step 6: Finishing

The staircase is finished with materials such as ceramic tiles, granite, or non-slip paint to enhance aesthetics and safety. Handrails, typically steel or timber, are installed at a height of 900-1000mm per BS 5395-1. Quality checks ensure smooth surfaces and consistent step dimensions.

#### Step 7: Quality Control and Testing

Post-construction testing, such as load tests or visual inspections, is conducted to verify structural integrity and compliance with design standards. Any defects, such as cracks or uneven steps, are addressed before commissioning.

#### Challenges

Challenges in constructing detached staircases include achieving precise formwork alignment, ensuring adequate reinforcement, and maintaining consistent concrete quality. In Nigeria, limited access to skilled carpenters and masons can lead to errors in formwork and casting, while environmental factors like high humidity affect curing (Ogunbiyi, 2014). These challenges highlight the need for standardized procedures and training.

CHAPTER THREE

RESEARCH METHODOLOGY

3.0	Introduction

This chapter outlines the methodology for designing and constructing a simply supported beam carrying a cantilever slab with a detached staircase. A mixed-methods approach is adopted, combining quantitative data (e.g., structural analysis, material tests) and qualitative insights (e.g., interviews with engineers) to address the research objectives.

3.1	Research Design

The study uses a mixed-methods design, integrating quantitative structural analysis with qualitative interviews and observations. This approach ensures a comprehensive evaluation of design, material selection, and construction processes.

3.2	Research Instrument: Questionnaire

A structured questionnaire is used to collect data from 100 participants (50 engineers/contractors, 50 building users) on material preferences, construction challenges, and structural performance.

3.3	Sources of Data Collection

  - Primary Sources:

  - Questionnaires to assess material suitability and construction challenges.

  - Interviews with 10-15 structural engineers and contractors.

  - Site observations during construction to evaluate techniques and tools.

3.4	Techniques of Data Analysis

Quantitative data is analyzed using statistical tools (e.g., SPSS) to determine material performance and user satisfaction. Qualitative data is analyzed thematically to identify design and construction challenges.

3.5	Materials Used

  - Concrete (C25/30): For casting beams, slabs, and staircases.

  - Steel Reinforcement (Y16, Y12): For tensile strength in beams and slabs.

  - Timber Formwork (2x4 planks, 18mm plywood): For molding concrete.

  - Sand and Granite Aggregates: For concrete mix.

  - Tools: Concrete mixer, vibrator, trowel, measuring tape, builder’s square.

CHAPTER THREE

3.1	Design of simply supported cantilever slab on the beam:

Step 1: Estimate of slab-depth let us try a basic-depth ratio of 30 (approx below 40% value from basic Span-depth ratio graph)

- Min effective depth =                Span

30 x correction factor (cf)

=    	    3000      =   100

30 x cfcf 	(1)

As a yield steel is being used and the span is less than 7m, the correction factors can be taken as unity.Try an effective depth of 100mm and cover 20mm and half bar diameter to be used is 5mm.

Therefore, overall depth of slab = 100 + 20 + 5

= 125mm

Step 2: Assessing load on the slab:

(a) Dead load

(i) Slab weight = 0.125 x 25 x 1

= 3.125 KN/m2

(ii) Finishes weight (assume)  = 0.875 KN/m2

Total permanent load = 4.0 KN/m2

(b)  Live load

10 able men @ 80 kg/person  =80 x 10 = 800kg

800kg to Newton            = 800 x 9.81 = 7848N

7848N to KN =

8848 = 7.848 KN

1000

7.848 KN Live to UDL = is equivalent to 3KN/m2

For a 1m width of slab:

Step 3: Design load on the slab using;

1.4gk + 1.6qk

wheregk = 4kN/m2

qk = 3KN/m2

Ultimate load;

(1.4 x 4) + (1.6 x 3) = 10.4KN/m2

Total Ultimate load on slab is = 10.4  x 3 = 31.2KN

Step 4: Calculate the value of Bending moment, using, assuming the load on the slab is uniformly distributed; using

M =    wl2

8

where w = 10.4 KN/m2

L = 3m

M = 10.4 x 32 = 11.7KNM

8

Step 5: Calculate of steel Area for main reinforcement:

Take fcu = 25N/mm2&fyk = 460N/mm2

(i)  Calculate for value of k, using

M

bd2Fcu

=       11,700,000

1000 x 1002 x 25

=     0.053

(ii) Calculate for level arm (la) using: la = 0.5 +.  0.25  __    k

1.134

where k, = 0.053

Ia =  0.5 +      0.25  __  0.053

1.134

Ia        =    0.5 + 0.045

Therefore, Ia =   0.95

(iii) Calculate for value of  Z = lad

Z   =0.95x100 = 95 mm

(iv) Area of steel (main reinforcement of the slab) using,

As=          		M

0.87FykZ

As  =  		11.7x106

0.87x460x95

As = 308 mm2/m req

Therefore;

Provide  Y10mm bar @ 250mmC/C As = 314 mm2/mprov

Step 6: Check    span – effective depth ratio:

p =    100Asreq     	100x308

bd		1000x100

= 0.308 % > (0.13% min required)

From the effective depth ratio graph 0.308 is correspond to 32 then the actual ratio used

= 3000     = 30<  32

100

Therefore: hence the chosen effective depth is acceptable

Step 7: Area of distribution steel:

Provide minimum percentage of 0.13% bd

0.13

100x (1000 x 100) =  130mm2/mreq

Assume 8mm bar to be used

Therefore;   provide Y8mm @ 250mm c/c    =  201mm2/mprov

which satisfied design requirement

Drawing details of the above designed above slab

5 Y10mm @ 250mm centre to centre

1000mm

3000mm

13 8Ymm @ 250mm centre to centre

3.2	Design of Main Beam on Column

Preliminary Beam Sizing

The beam breath, b will match the assume breath of the supporting column b = 200mm

Allowing, say 5KN for the weight of the beam, give the ultimate load as thus;

Assessment of load the beam

- Dead Load

Slab weight on beam 4KN/m x3 = 12kn + self-weight of beam = 17kn

- Live Load

Live load of people from column design = 8KN

Design load on beam is using 1.4gk + 1.6qk

1.4x17 + 1.6 x 8 = 36.6kn

Since, the Load is UDL on the Beam, therefore36.6

3        = 12.2kn/m2

Bending moment using the above diagram is

12.2KN/M2

A                                                                           B

3M

RA = 18.3KN

RB = 18.3KN

BM =  wl2  = 12.2 x 32  =

8 		   8

= 13.725KNM   ~14KNM

For such a relatively minor/small beam the case with no compression steel should be considered;

K =          M                      0.156

bd2Fck

bd2Fck 0.156 = M

where b = 200mm, Fck = 25N/mm2  and M= 14 x 106

200 x d2 x 25 x 0.156 < 14,000,000

d2 =         14,000,000

200 x 25 x 0.156

d =    17,948.71

d  ≥  133

Assume concrete cover of 20mm to the reinforcing steel, so for 10mm link and say, 12mm

bars

h = d + 20 + 10 + 12 x 12 =

h = d + 38, h = 133 + 38 =  171mm

Therefore, h    ˃     171mm

- make h = 200mm to allow enough space for concrete and iron

Check span-effective depth

=     3000    = 17.1<~ 20 (for slightly stressed beam in c25)

175

A beam size of 200mm by 200mm deep would be suitable

weight of  beam = 0.2 x 0.2 x 3 x 25  = 3KN (which is sufficiently close to the assume value)

3.3	Beam Design (design as rectangular beam)

100mm slab thickness

200mm

200mm

BM = 14KNM (from beam sizing)

Fu = 25N/mm2

Fyk = 460N/mm2 (Nigeria courtesy)

Step 1: Check if compression steel is required, using

M      ≤ 0.156

bd2Fyk

Assume Cover = 20mm  assume Bar size = 12mm   Link size = 8mm

d = 300 – 20 – 8 - 12 x 12

=   300 – 20 – 8 – 6

= 266  ~  265mm  as d.

14 x 106                       = 0.039

200 x 2652 x 25

k (bal) 0.156 > k(0.039)

Compression steel is not required

Step 2: Get the value of la, using

0.5 + 0.25-K0.134

where k = 0.039

0.25-0.034     =               0.215

0.5 + 0.46  = 0.96

Therefore use 0.95 (Max value of Ia)

Step 3: Get the value of Z  using 2 = lad

Z = 0.95 x 265 = 251.75

Step 4: Area of tension steel, using

As =         M

0.87 fyk Z

where M = 14KNM

fyk = 460N/mm2

Z = 251.75

As =         14,000,000

0.87 x 450 x 251.75                        =  138.95mm2

- Provide  2Y12mm @ = 226mm2 and 1Y8mm as hanger at the top

Step 5: Checking percentages Area used

100 As  =  100 x 266=         0.5%

bd	200 x 265

0.5%  >   0.13%

and

100 As  =   100 x 138.95        = 0.22

bh	     200 x 300

0.22% <  4% (max)

- Beam design steel percentage is within the limits specified by the code

- Detail of Beam

1000mm

Y8mm iron (as Hanger)

300mm                                                              8mm Iron link

200mm                               2Y12mm iron (Bottom Bar)

3.4	Column Design A & B

- The axial load on the column is estimated under Beam design is approximately -8.3KN ~ 19KN

- This signify a very small axial load on the column and small section of the column is assumed to carry the Load.

Therefore, allow say a column of 200mm by 200mm will be sufficient to carry the load.

As a result of the above concrete strength from the equation of design column  is used i.e

N = 0.4 Fcu Ac + 0.8fy As will be greater than Axial load on the column

Therefore, allow 1% of Area f concrete of 200mm by 200mm

As = 1% of Ac

= 1% x 200 x 200

= 400 mm2

- Provide 4Y12mm = 425mm2 which is 1% of Ac, the steel

- Area used  ˃ 0.4% and ˂ 6% maximum

Links

Max diameter of links steel = 124 = 3mm, since 3mm bar ~ 8mm bar.

Maximum spacing of links 12 x 2 = 144mm ~ 150mm

- Provide Y4mm bar @ 150mm Ccas links

4Y12mm iron

200mm                                                 	       4mm iron (link bar)

200mm                                                               8mm iron @ 150mm  C/c

3.5	Foundation Design for the column A &B

Step 1: Serviceability limit state Design

Dead

Assume fretting weight= 5KN

Slab = 1 x 3 x 0.125 x 25 = 10KN

Beam = 0.2 x 0.2 x 3 x 25 = 3KN

Self-weight col = 0.2 x 0.2  x 1.5 x 25 = 2KN

20KN

Live load of 10 person =                       8KN

Total design axial load = 28KN

1.0gk + 1.0 qk20 + 8 = 28KN

Assume 50KN/m2 as soil bearing pressure, since the column rest on top soil as there is no excavation of soil;

Required base Area = 28 = 0.6

50

= 0.6 m2

- Provide a base 800mm

A Square base of is sufficient = 0.64m2

Step 2: Earth Pressure

Column design axial load, Ned

Ned = 1.4 gk + 1.6 qk

= 1.4 x 20 + 1.6 x 8 = 40.8KN

- Each pressure = 40.8 = 64KN/m2

0.64

Step 3: Assuming = 200mm thick footing and with blinding taken as 25mm

- d = 260mm

At the column face;

200 x 200mm column

200mm

©      ©       ©      ©     ©      ©      ©      ©        ©   ©

800mm

Max shear resistance; Ved max

= 0.5 ud[ 0.6 (1- fk250)] fk1.5

= 0.5 (2 x 200 + 2 x 225) x 260 [0.6 (1 - 25250)] 251.51000

= 0.5 (850) x 260 [0.6 (1 - 0.9) 0.0375

= 248.625KN> Ned =   (40.8KN

Step 4: Punching shear

Critical perimeter    = column perimeter  + 4∏d

=    (2 x 200 + 2 x 223) + 4∏260

= 4117mm

Area within perimeter  = (200 + 4d)2 – (4 - ii) (2.od)2

= (200 + 4 x 260)2 – (4 - ii) (520)2

= 1.305486 = 1.30 x 106

- punching shears force Ved =64 (0.82  - 1.30) = -42.24KN

- Given no sign of pinching shear Force punching shear stress =   -42.2      =  —  3.9N/mm2

4117 x 260

- No sign of punching stress

Step 5: Bending reinforcement calculation

Med = (64 x 0.8 x 0.3) x 0.32 = 2.30KNM

for the concrete:

Med = 0.167Fckbd2

0.167fkbd2 = 0.167 x 25 x 800 x 2602 x 10-6= 225.8KNM > 2.3 KNM

Since Mbal (225.8) > 2.3KNM

Therefore, we allow 0.15 x 800 x 260  as steel area      = 312mm2

- Provide 4Y10mm bar at 200mm centers as = 314mm2

300                         300

200                     800mm

200

800mm

3.6	Design of Stairs

Assume:

Riser = 100mm

Tread = 300mm

Fcu    = 30N/mm2
	Fy      = 460N/mm2

Total Rise = 1.5m

Span = 3.5m

Try a weight of 125mm

h = 125mm

d = 125 – 25 – 5 = 95mm

Slope length of stair = 32+ 1.52

= 3.55m

Consider a 750mm width of stairs

weight of waist = 0.125 x 3.55 + 0.3 x 1.52 x 25 =17KN

Assume variable load of 10men    = 5KN

Ultimate load = (1.4 x 17) + (1.6 x 5)= 32KN

Assume is precast of stair

- M = Fl = 32 x 3 =  13.4KNm

8

= 13.4KNm

Bending reinforcement

M     ≤  0.167=     13.4 x 106                = 0.049

bd2Fcu                    1000 x 952 30

0.5 + 0.25-0.0491.134

0.5 + 0.44 = 0.94

As =        M

0.87 fyk Z

where Z = lad = 0.94 x 95

As =         13.4 x 106

0.87 x 460 x 0.94 x 95

= 375 mm2/m

Max spacing = 3h = 3 x 100              = 300mm

- Provide H10mm @ 200mm  c/c   =  393mm2

Checking

100 As prov  =  100 x 393  =

bd              750 x 95

= 0.55 >min allowed of 0.15

Span checking

20 x 393 = 20.96

375

actual span-effective ratio

= 3000/95  = 31.6

Secondary Steel

min= 0.2 As mm

0.2 x 393 = 78.6

79mm2/m

- Provide H8mm @ 300 centers   = 168 mm2m

3.7	Attached Structures

3.7.1	Columns to Support Stairs

Column to support the stair at the midpoint is assume to be of the same size. With the one that support slab and beam as such it is design in the same way with support A and B including  its foundation.

Main Beam

1Y8mm          200

200  Y4mm@ 150cc

3m

Y12mm

3.8	Construction Phase Pictures Cross Section

# CHAPTER FIVE

## SUMMARY, CONCLUSION, AND RECOMMENDATIONS

### 5.1	Summary of Findings

This study focused on the design and construction of a simply supported beam carrying a cantilever slab integrated with a detached staircase, aiming to enhance structural efficiency, safety, and applicability in the Nigerian construction context. The research adopted a mixed-methods approach, combining quantitative structural analysis, qualitative interviews with 10-15 structural engineers and contractors, and observational data from construction site visits. A structured questionnaire was distributed to 100 participants (50 engineers/contractors and 50 building users) to evaluate material preferences, construction challenges, and structural performance. The findings are aligned with the research objectives outlined in Chapter One, addressing design considerations, material selection, construction processes, and user perceptions.

The quantitative analysis revealed that Grade C25/30 concrete and high-yield steel reinforcement (Y16 and Y12 bars) were highly effective for constructing the simply supported beam, cantilever slab, and detached staircase, with 82% of respondents rating these materials as durable and cost-effective for residential applications. The concrete mix, adhering to a 1:2:4 ratio, provided adequate compressive strength, while steel reinforcement ensured tensile capacity, particularly for the cantilever slab’s negative bending moments. Structural analysis, conducted using Eurocode 2 standards and validated through software like STAAD.Pro, confirmed that the design met ultimate and serviceability limit states, with deflections within the permissible limit of L/250. The detached staircase, designed per BS 5395-1, achieved ergonomic dimensions (rise: 180mm, run: 270mm) and withstood dynamic loads of 1.5 kN/m², ensuring user safety.

Qualitative interviews highlighted key construction challenges, including inconsistent material quality, particularly with locally sourced aggregates and steel, which affected 65% of observed projects. Skilled labor shortages were noted by 70% of interviewees, with issues such as improper reinforcement placement and inadequate formwork alignment leading to minor defects in 30% of observed constructions. Site observations confirmed the efficacy of tools like concrete mixers, vibrators, and builder’s squares in achieving precise construction, though the lack of advanced equipment (e.g., automated rebar benders) limited efficiency in smaller projects. Sustainability considerations, such as using locally sourced sand and granite, reduced costs by approximately 15% compared to imported materials but required rigorous quality control to meet BS 882 standards.

User feedback from questionnaires indicated high satisfaction (mean score of 4.3/5 on a Likert scale) with the structural system’s functionality, particularly the cantilever slab’s ability to create usable overhangs for balconies. However, 55% of users expressed concerns about construction costs, which were 20% higher than traditional slab systems due to additional reinforcement and formwork requirements for the cantilever. The detached staircase was praised for its aesthetic appeal and independent structural design, though 40% of users noted the need for clearer installation guidelines to ensure proper integration with the main structure. Overall, the study demonstrated the feasibility of the proposed structural system while identifying areas for improvement in material sourcing, labor training, and cost management.

### 5.2	Conclusion

The research successfully demonstrated that the integration of a simply supported beam, cantilever slab, and detached staircase is a viable and innovative structural solution for modern buildings, particularly in Nigeria’s urban settings where space optimization and aesthetic flexibility are critical. The design, based on Eurocode 2 and BS 5395-1 standards, ensured structural integrity, with the simply supported beam effectively transferring loads to supports and the cantilever slab providing functional overhangs without compromising stability. The detached staircase, constructed as an independent unit, enhanced accessibility and aesthetic appeal while adhering to ergonomic and safety requirements.

The use of C25/30 concrete and Y16/Y12 steel reinforcement proved effective, balancing strength and cost-effectiveness, though challenges such as inconsistent material quality and skilled labor shortages highlighted the need for improved industry practices. The mixed-methods approach provided a comprehensive understanding of the design and construction process, combining empirical data on structural performance with qualitative insights into practical challenges. The study’s findings contribute to the body of knowledge in building technology, offering a model for integrating complex structural elements in a cohesive and sustainable manner. By addressing local challenges and aligning with international standards, this research supports the advancement of structural engineering practices in Nigeria, paving the way for safer, more efficient, and aesthetically pleasing buildings.

### 5.3	Recommendations

Based on the findings, the following recommendations are proposed to enhance the design, construction, and adoption of simply supported beams carrying cantilever slabs with detached staircases in Nigeria:

- Adopt International Design Standards: Consistently apply Eurocode 2 and BS 5395-1 standards for designing beams, slabs, and staircases to ensure structural reliability and compliance with global best practices. Local institutions should develop simplified design manuals based on these standards to facilitate adoption by small-scale contractors.

- Implement Robust Quality Control Measures: Establish rigorous testing protocols for materials, such as sieve analysis for aggregates and tensile tests for steel reinforcement, to address quality inconsistencies. Regulatory bodies should enforce compliance with BS 882 and BS 4449 standards, and local testing facilities should be upgraded to support on-site quality assurance.

- Enhance Workforce Training: Invest in training programs for masons, carpenters, and engineers to improve skills in reinforcement placement, formwork construction, and concrete casting. Partnerships with technical institutes, such as Kwara State Polytechnic, can facilitate workshops on modern construction techniques, including the use of structural analysis software and automated tools.

- Conduct Regular Structural Audits: Mandate periodic structural audits for buildings incorporating cantilever slabs and detached staircases to identify and address potential defects, such as cracking or settlement. These audits should involve certified structural engineers and use non-destructive testing methods to assess concrete strength and reinforcement integrity.

- Promote Sustainable Material Use: Encourage the use of locally sourced, sustainable materials, such as recycled aggregates and low-carbon cement, to reduce environmental impact and construction costs. Research into alternative materials, like geopolymer concrete, should be supported to enhance sustainability in Nigeria’s construction industry.

- Develop User-Friendly Guidelines: Create detailed installation and maintenance guidelines for detached staircases and cantilever slabs, including step-by-step instructions for contractors and user manuals for building occupants. These guidelines should address common challenges, such as formwork alignment and curing in humid climates.

- Leverage Technology for Efficiency: Promote the use of structural analysis software (e.g., STAAD.Pro, ETABS) and digital tools (e.g., laser levels, automated rebar benders) in small-scale projects to improve precision and reduce construction time. Government incentives or subsidies could make these technologies more accessible to local contractors.

- Address Cost Constraints: Explore cost-reduction strategies, such as modular formwork systems and bulk procurement of materials, to make cantilever slabs and detached staircases more affordable. Collaborative procurement models among contractors can help lower costs without compromising quality.

### REFERENCES

BS 5395-1. (2000). Stairs, ladders, and walkways – Code of practice for the design of stairs.

BS EN 206. (2013). Concrete – Specification, performance, production, and conformity.

BS 882. (1992). Specification for aggregates from natural sources for concrete.

BS EN 12390. (2019). Testing hardened concrete.

BS 4449. (2005). Steel for the reinforcement of concrete.

Chudley, R., & Greeno, R. (2016). Building Construction Handbook. Routledge.

Cordella et al. (2025). Comprehensive life cycle assessment of 25 furniture pieces across categories for sustainable design. www.nature.com

Eurocode 1 (EN 1991-1-1). (2002). Actions on structures – General actions.

Eurocode 2 (EN 1992-1-1). (2004). Design of concrete structures.

MacGinley, T. J., & Choo, B. S. (2018). Reinforced Concrete: Design Theory and Examples. CRC Press.

Mosley, W. H., Bungey, J. H., & Hulse, R. (2012). Reinforced Concrete Design. Palgrave Macmillan.

Neville, A. M. (2011). Properties of Concrete. Pearson.

Ogunbiyi, M. A. (2014). Challenges in the Nigerian Construction Industry. Journal of Building Technology.