ESTIMATING AND BUDGETING A MEDIUM SCALE BUILDING PROJECT

(A CASE STUDY OF A PROPOSED RESIDENTIAL DEVELOPMENT FOR MR ISMAILA ADETUNJI ADEWOLA ALONG EIYENKORIN ASA LGA, KWARA STATE)

WRITTEN BY

ND/23/QTS/PT/0004

IRANOLA ESTHER DAMILOLA

BEING A RESARCH PROJECT SUBMITTED TO THE

DEPARTMENT OF QUANTITY SURVEYING,

INSTITUTE OF ENVIRONMENTAL STUDIES,

KWARA STATE POLYTECHNIC, ILORIN KWARA STATE

IN PARTIAL FULFILLMENT OF THE REQUIREMENTS FOR THE    AWARD OF NATIONAL DIPLOMA (ND) IN QUANTITY SURVEYING

JULY, 2025

CERTIFICATION

This project has been read and approved by the undersigned on behalf of the Department of Quantity Surveying, Institute of Environmental Studies (IES), Kwara States Polytechnic, Ilorin as meeting the requirement for the award of National Diploma in Quantity Surveying.

______________________________________________ 		     ______________

QS. YEKINI ABDULGANIYU (MNIQS)                                                  DATE

(Project Supervisor)

____________________________________ 			_______________

QS AISHAT UMAR (MNIQS)						  DATE

(Project Coordinator)

______________________________________ 			_______________

QS. SIDIQ LATEEF (MNIQS, RQS) 				              DATE

(Head of Department)

_____________________						 _______________

DATE

External Examiner

DEDICATION

This project is dedicated to God Almighty

ACKNOWLEDGEMENT

My appreciation and special thanks goes to Almighty God for his protection  throughout my National Diploma (ND) in the Kwara State Polytechnic, Ilorin.

ABSTRACT

This project focus on preliminary activities that lead to draft bill of quantities for the proposed residential development along Eiyenkorin Asa LGA Kwara State.

Its centralized on preparation of bill of quantities and taking off for project. Meanwhile this is to prepare the construction legation and to understand the construction arrangement between the client and the construction industry.

TABLE OF CONTENT

Title page		       i

Certification		      ii

Dedication 		   Acknowledgement		     iv

Abstract		      v

Table of contents		  v-vi
CHAPTER ONE: BACKGROUND OF THE STUDY

- Description of work	        	   2

- Aim and objectives		   3

1.2.1 Aim		   3

1.2.2 Objectives		   3

- Scope and Limitation of the Study                                                                      4

- Definition of teams

1.4.1 Prime cost sum                                                                                                      5

1.4.2 Contingency sum

1.6.3 Provisional sum

CHAPTER TWO:  TITLE

2.1 Site description                	 	  5

2.2 Location                                                                 	    6

2.2.1 Accessibility                        		7             2.2.2 Nature of the land

2.3 Availability and proximity of resources                                              		8

CHAPTER THREE: PROJECT METHODOLOGY

  - Method of Take Off	15

3.1.1Abstracting	    15

3.1.2 Bill of quantities

CHAPATER FOUR: MEASUREMENTS OF BUILDING WORKS

4.1 Sub Structure 		   				                                        17

4.2 Super Structure 								                18

4.3 Abstracting							                            19

4.4 Bill of Quantities 								                23

4.8 Discussion of Findings

CHAPTER FIVE: CONCLUSION AND RECOMMENDATION

5.1 Conclusion                                                                                                              25

5.2 Recommendation									   25

CHAPTER ONE

INTRODUCTION

This project is based on a proposed Residential

Development for Mr Ismaila Adetunji Adewola along Eiyenkorin Asa LGA Kwara State.

In construction industry, cost is considered as one of the important element for constructing a project. Cost in construction includes the initial capital cost, and also the subsequent operation cost and maintenance cost.

Generally, running a construction project will involve a lot of budget. Furthermore, the increasing rate of inflation by years, will affect the price of raw material, transportation, fuel, machinery, manpower and others will be a bother for the clients such as developer to invest money for a project. Hence, to overcome this problem and alternative material or a new system will be the best option.Construction Industry Development Board Malaysia (CIDBM) proposes cost saving method which are the Industrialized Building System (IBS) where the building component or the building system are per-fabricated and then install on site.Moreover, it is proven not only reducing the cost but also reducing the construction time.

There are five types of Industrialized Building System (IBS) that were introduced for the construction industry to practice which are pre-cast concrete framing; Panel and board system, steel formwork system, steel framing system, perfabricated timber framing system and finally the block work system.

This report will focus on all sections of building works i.e Block work, roof etc block made from cement laterized.The usage of interlocking blocks as the construction materials has been proven to provide a lot of benefits. By implementing the cement laterized interlocking block for buildings will greatly reduce the wastage, volume of building materials, reducing unskilled workers, provide better quality control, promote a safer and more organized site; and last but not least will greatly reduce the construction cost and time.

1.1 DESCRIPTION OF WORK

The building project comprises of the construction of a proposed residential development for Mr Ismaila Adetunji Adewola along Eiyenkorin Asa LGA Kwara State,building has a rectangular shape with the land area of about 10198mm in width and 16276mm length.

The construction is found on the plain in- situ concrete foundation with over site concrete floor which is a reinforced concrete slab (150mm thick).

The walls are 225mm thick hollow sandcrete block.

The roofing used is long span aluminum on timber roof structure, windows are of aluminum frame sliding with two (2) side and of different sizes, doors are: double leaf double swing natural an odixed aluminum special made electronic controlled door, pretty glazed aluminum frame door, slide core flush panel doors with season wood of different sizes.

The walls are rendered and painted with emulsion internally and textured paint externally.

Floor finishing is tiles finishing on 150mm

concrete floor.

Service conducted are mechanical and electrical installation as well as fire fighting and alarm system.

1.2 AIM AND OBJECTIVES

1.2.1 AIM:

The aim of the project is to estimate and budget for a proposed residential development for Mr Ismaila Adetunji Adewola along Eiyenkorin Asa LGA Kwara State.

1.2.2 Objectives:

1.To understand the bill operation of taking off process abstract, bill drafting and pricing.

2. To understand the process of site feasibility report

3. To understand the process of given pre-construction cost of a proposed  project.

1.3 SCOPE AND |LIMITATIONS OF THE PROJECT

The scope of this project is to prepare a bill of Quantities (BOQ) for the proposed residential development for Mr Ismaila Adetunji Adewola along Eiyenkorin Asa LGA Kwara State.

while Limitation are Inability to conduct proper site investigation; insufficient design information and Pricing (market survey) was not carried out to determine actual price of measured items.

1.4 DEFINITION OF TERMS

Prime cost sum: This is a sum provided for work or service to be executed by a nominated subcontractor or for materials or goods to be obtained from a nominated supplier. The term includes specialist work carried out by person apart from the main contractor (Ivor Seelay).

Contingency Sum: It is the sum allocated to unforeseen expenses it is expended only with the consent of the architect or supervisory engineer.

Provisional cost sum: This is the sum allocated to work that is subjected to change or that can't be accurately measured.

The sum will later be adjusted according to the extent and true value of the work.

Estimating: This can be defined as the technical process of predicting cost of construction. It can also be further explained as a process by which cost of item is carried out

Preliminaries item: These are item meant for the smooth execute for smooth execution of project. But which are not related to a particular trade section in BOQ.

Tender: It is the offer to execute a given project at a certain amount of money and specify period of

time.

Plant: This is a process of work organized carefully and designed e.g building or civil engineering work.

CHAPTER TWO

TITLE

A proposed Residential Development For Mr Ismaila Adetunji Adewola along Eiyenkorin Asa LGA Kwara State.

2.1 SITE DESCRIPTION

The proposed Residential Development For Mr Ismaila Adetunji Adewola along Eiyenkorin Asa LGA Kwara State. This has a total length of 20176mm and 10198mm width which appear to be in a rectangular shape.

The site is located along Eiyenkorin Asa LGA Kwara State.

The site has small and big tress and bushes which needs to be cleared away from the site.

2.2 LOCATION

The site is located along Eiyenkorin Asa LGA Kwara State.

2.2.1 ACCESSIBILITY

There are untarred roads leading to the site from the main road in the school. So there is no problem delivery of materials to site and the labour are expected to resume work as early as possible.

2.2.2 NATURE OF THE LAND

The proposed site of the building is a virgin land with a even/flat surface.

This will minimize the construction cost of the foundation because of the balanced surface when compare to the study area which increases the constructional cost.

2.3 AVAILABILITY AND PROXIMITY OF RESOURCES

Availability of construction resources simply means the extent of convenience at which the resources can be found in the market and the selling price.

While proximity means the closeness of the materials resources of the proposed site.

The construction resources include the following:

Plant

All necessary plant needed for the proposed projects are readily available in Abuja, Such plant include: concrete mixer, Bulldozer, Electric generator, Compacting machine etc all mentioned plant can be hired in area's not far to the site.

Materials

The whole material necessary for this project all are available at Niger state, but costly to some materials price in some area of the country.

However, all the material needed for smooth execution of this proposed project is remote to the site.

To an extent preliminaries items have a great effect on construction project. Such preliminaries items are explained below.

Scaffolding: There should be a provision for scaffolding because the project requires roofing works, there will be the need for rendering and painting at the higher level of the building.

Setting out: This is the transfer of information on the building drawing to the ground with high degree of accuracy. In setting out site clearance is necessary by cutting down of trees and edges & top soil excavation before making the provisions of setting out instruments such as profile pegs, measuring tapes, builder square e.t.c before the commencement of site operation.

Temporary Road: This is done temporarily to provide easy access to the site during work.

Temporary fence: Temporary fence should be done to cover and protect the materials on site from vandalism.

First aid box: This is very important in any construction due to some injuries that may occur on site which might be minor or major injuries before being taken to the hospital.

Site supervision: This is the contractor’s agent on the site who supervises the whole of the project. He also coordinates the works of sub-contractors and receives instructions from architect on behalf of the main contractor.

Setting out of the works: This contractor will set out the position of the building on the site before commencement of any actual work to the satction of the Architect.

Temporary latrine: Welfare facility of this type must be provided on the site bycontractor to satisfaction of the architect.

CHAPTER THREE

PROJECT METHODOLOGY

TAKING OFF PROCESS

Taking off process: This implies reading of dimension from drawings setting them down in a specific order on specific paper know as dimension paper and inserting an appropriate description. It also refers to the process of identifying elements of construction works that can be measured and priced.

This is necessary to produce bill of quantities and requires that the design is complete and a specification has been prepared.

Analysis of drawing and specification allow the cost consultant (usually a Quantity Surveyor) to prepare a taking off list, which lists all of the individual elements that comprises the works. These elements can be measured in number, length, area, volume, item then collected and structured to produce an un-price bill of quantity (BOQ). This process is sometimes referred to as “working up”

The traditional dimension sheet is divided vertically in to two identical haves each comprising a set of four columns in effort, the ruling on the right half of the sheet may be considered to be a continuation of these on the left side of the sheets.

|  | A | B | C |  | A | B | C | D |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

Column A: are the timesing columns These column are used to enter multipliers when there is more than one of the particular items being measured.

3

1.2

0

1.2

0

Linear

Timesing column

Column B: Are the dimension columns, these columns are used to record the dimensions in meters. Each linear dimensions is underlined.

7.59

Length

Linear

20.18

10.19

Length

Width

Area

20.18

10.19

0.65

Length

Width

Depth

Volume

Number

NOTE: The order of entering dimension is strictly

Length

Width

Depth/Height

|  |  | Items |  | Allow for bringing to site and removing from site all plant required for this work section. |
| --- | --- | --- | --- | --- |

Column C: Are the squaring column: the resulting area and volumes inserted in timesing and dimension columns are subsequently calculated and entered into the squaring column opposite their respective dimension.

2

7.55

0.90

0.10

1.36

Squaring

Column D: The description column. There are many ways of inserting description on the dimension sheet but the most acceptable however, is description which commence in the same level with it’s first dimension.

The example below show the dimension and description should be arranged in dimension sheet.

4

3.45

0.68

0.45

7.90

Hardcore

filling

ton

make

up

level

under

floor

average

thickness

not

exceeding

250

mm.

Waste calculation

This is an imaginary column occupying the right hand side of the description columns in which the taker-off inserts any preliminary calculation (waste) which may be necessary In order to arrive at the dimensions.

All preliminary calculation must be shown in waste, they need to be set down accurately and carefully so that they can be checked.

They should be written either above or below the description to avoid confusion.

Waste: calculate is necessary for each dimension except in the following situations:

A. When a scaled or figured dimensions, can be transferred direct from the drawing to the dimension column.

When the dimension has been derived from a previous waste calculations.

When a dimension has been inserted from a previous dimension.

D. In all other instance the preliminary calculation, should be shown on waste.

Waste calculation should be done in millimeters (mm) or in three places of decimal. Final figures are then rounded off to the nearest 10mm before bring transferred to the dimension column.

3.1 METHOD OF TAKING OFF

Nill: is a method of cancelling wrong enteries during taking-off process.

Nill may be done in three (3) ways i.e on waste calculation, on framed description and on figured dimensions.

Bracketing: it is used to determine one item of measurement from another

Adding on: This is the method of connecting two or more description related to a common dimension.

3.1.1 ABSTRACTING PROCESS

After the squaring process, the next process is abstracting. This is in a spectral rule sheet The sheet are ruled with series of vertical line that are spaced about 250mm apart and usually on A4 in width, each abstract sheet is headed with the job reference, sheet number and work section and possibly the sub-structure of the work to which the abstract dimension refer, the sheet may be divided into work section or element.

Abstracting id process where various items are stored out into their respective trades as they are entered on the abstract paper.

Description are usually spaced over two columns. The dimension are entered in the left hand column while any deductions are entered on the right hand column.

The order of abstracting is to commence with cubic items, followed by superficial (square), linear and finally enumerated items. Also labour items should proceed labour and materials. Smaller items proceeding larger ones and cheaper items proceeding more expensive items.

3.1.2 BILL OF QUANTITIES (BOQ)

Bill of quantities is a documents which set out the quantities and description of the items of labour, materials and plant require to erect and complete a building or other works in a systematic manner.

The preliminary sections of the bill sets out the practical and contractor conditions of the project, particular of the type of contract to be used and also the details of general matters, which may affect the price of the work together with project overheads. Project overhead are items that cannot

be attributed to any particular measured work section,

The measured items are grouped in appropriate sections and each section starts with a number of preamble clauses.

CONTENTS OF THE BILL OF QUANTITIES (BOQ)

The principal use of the bill of quantities is to obtain competitive tenders and for this purpose B.O.Q should be prepared to recognized method of measurement to standardize the bills Bill of quantities prepared in accordance with a standard method of measurement, fully described and accurate respect the work to be executed and this provides a uniform basis for competitive tendering.

During the construction of the project, the client and the contractor use the bill as the basis for financial management.

Bill of Quantity may consist some all of the following:

Bill Nr.1. Preliminaries

Bill Nr.2. Preamble

Bill Nr.3. Measured works

Foundation (Substructure)

Frame (column & beam)

External & internal walls

Partitions

Floors

Roofs

FINISHING

External & internal finishing (walls, floor & ceiling)

Windows

Doors including opening without joinery

Fixtures & Sundries

Stair & lifts

Plumbing & Engineering service

EXTERNAL WORKS

Drainage

Roads, paths, site layout, out buildings e.t.c

Sport items: These are items that are alteration and repair items of a building:

Bill Nr.4. Provisional quantities

Bill Nr.S. Prime cost & Provisional sum

Bill Nr.6. General summary

PURPOSE OF A BILL OF QUANTITIES

It serves as a schedule on which all variation in the work may be valued.

To provide detailed list of every service to be performed.

To supply each tenders with information which will enable them to tender on a uniform basis.

To describe in addition to any description in the specification of the quantity of the work and the method of carrying it out.

It can be use for further cost investigation and cost planning information.

prevent disputes as to what is and what is not included in the contract prices and the value of any work or labour which has been omitted.

PRODUCTION OF BILL OF QUANTITIES

These are several techniques of preparing bill of quantities (BOQ)

TRADITIONAL BILL METHOD: It is set down in the sequence following the out of the building and engineering standard method of measurement [BESMM]. This system provides the taking off” with a logical system of measurement designed to prevent accurate and efficient taking off.

The work is measured in series of groups, each representing a particular section of building without regard to the order in which items will finally appearing the bill As the sequence of measurement is not related to the order of the bill the “working up” process must involved the preparation of an abstract to collect and arrange the items in the order necessary for whiting the defect bill. This system is mostly used in the examination.

It comprises of the following:

Taking — off usually group system.

Working — up: Squaring, abstracting, totaling,

rounding off & billing.

OPERATIONAL BILL METHOD: It is tendering document for estimating costs prepared by architects that describes a construction project in terms of the operations (which include labour and plant) needed to build it. This form of document contracts with that bills of quantities in which such tendering and estimation is limited to the materials in the completed work operation bills have the advantages of enhancing communication between design and production, enabling realistic tender pricing and making the preparation of critical - path analysis easy for the contractor.

ACTIVITIES BILL METHOD: The activity bill was a development of operational form but without the separation of labour and materials. It was subdivided into section based on activities or operation derived from a network analysis. The work was measured in accordance with the standard method of measurement although on site and off site activities were usually separated and specialist equipment and component of and the work of nominated specialist could be grouped in spate bills (Seelay and winifield 2005).

ELEMENT BILL METHOD: Here the work is billed in trade order not in sequence of the building and engineering standard method of measurement [BESMM] but under heading that from a breakdown of the building into the main element of the structural wall, floor and roof. Although these provide a more useful control document during the construction operation, it makes the tendering procedure more complicated (T.C onouah)

ANNOTATED BILL: It is possible for the bill of quantities to give the contract fill details of the quantities, types and quantity of materials and labour and for an accurate and complete set drawing to show him precisely where and how the work is to be executed (Seelay and winifield 2005)

CUT AND SHUFFLE BILL METHOD: This is a faster method than the traditional method. The civil engineer or surveyor take off quantities into specially designed sheet in duplicate where the original sheet is perforated into strips which can be separated from the book and collected together to form group of items.

DIVISION OF BILL PREPARATION: The traditional method of preparation of bill of quantities can be broken down into two main processes.

Taking off: Seelay 1988 defined “takingoff’ as the process in which dimension are scaled read from drawings and entered in a recognized form in a special ruled paper called dimension paper.

Working up: This process can be divided into two stages as follow:

Squaring: This comprises squaring the dimension and transferring the   resultant lengths, area and volumes to the abstract.

Abstracting: The squared dimension are arranged in convenient order for billing and reduced to the recognized units of measure.

3.3 REQUIREMENT FOR ACCURACY:

Whatever the method adopted for bill preparation, the requirement for accuracy is the same for example;

Work shall be measured not as fixed in position overlaps. (General Rule 4.1) 1) Dimension used in calculating quantities shall be taken to the nearest 10mm (i.e 5mm and less than 5mm shall be disregarded). (General Rule 4.2). All dimension shall be record to two decimal places and waste calculation are to three decimal places. 2) Quantities measured in tones shall be given to two decimal places. (General Rule 4.3). 3) Other quantities shall be given in the nearest whole except that any quantity less than ten shall be given as to the nearest first decimal. (General Rule 4.4).

4) Void or openings, which are wholly within the boundaries of measured area and voids or openings wholly at the boundaries of measured piers, column and similar member, shall always be the subject of deduction, irrespective of size (General Rule 4.5) 5) Items can be measured within a range of stated limits.

6) Average or an interpolation of level may be required for the measurement of execution. Due to problem with reprographics and the inherent inaccuracy of the process, scaling drawing should be avoided whenever possible. However, where dimension are stated on drawing there is no excuse for measurement bring anything other than correct

| A B C D E F G | ELEMENT NR 1 BILL OF QUANTITES FOR CONSTRUCTION OF A PROPOSED RESIDENTIAL DEVELOPMENT FOR MR ISMAILA ADETUNJI ADEWOLA ALONG EIYENKORIN ASA LGA KWARA STATE. SUBSTRUCTURES SITE PREPARATION Clear site of all shrubs, bushes, undergrowth etc. and remove from site Excavate oversite average 300mm deep to remove topsoil and vegetable matter Deposit excavated materials in temporary spoil heaps not exceeding 100 metres away from site Excavation Earthwork Excavate foundation starting trench not exceeding 1.5m deep starting from formation level Excavate pits for column bases not exceeding 1,5m deep starting from formation level (13NR) Return, fill and well ram excavated material around foundation Remove surplus excavated materials from site TO COLLECTION BLD/01 | QTY 234 234 35 63 17 80 14 | UNIT m2 m2 m3 m3 m3 m3 m3 | RATE N | N | K |
| --- | --- | --- | --- | --- | --- | --- |

| A B C D E F | Laterite filling Approved laterite filling, spread and well consolidated in 150mm thick layer Hardcore filling 300mm thick rock hardcore filling on laterite filling laid on laterite filling and blinded with sand Level and compact bottom of excavation to receive concrete foundation Prepare and apply ``Dieldrex 20 anti –termite to surface of trench excavation and laterite filling before laying rock hardcore Concrete work plain in-situ concrete Plain in –situ concrete (1:3: -6- 38mm aggregate) in:- Blinding 50mm thick Plain in –situ concrete (1:3: -6- 38mm aggregate) in:- Foundation CARRIED TO COLLECTION BLD/02 | QTY 21 48 96 345 4 17 | UNIT m2 m3 m2 m2 M3 m3 | RATE | N | K |
| --- | --- | --- | --- | --- | --- | --- |

| A B C D E F | Reinforced in situ concrete (1:2;4-19mm Aggregate filled into formwork and well packed around reinforcement (reinforcement and formwork measured separately) Column bases Columns Bed, 150mm thick Reinforcement High yield tensile reinforcement bars, including hooks, bends, distance blocks, tying wire, etc (Provisional) 10-16mm Fabirc Reinforcement B.R.C Mesh reinforcement to B. S4483 Ref: A252 Weight 3.95kg\m2 laid in bed and step with 300mm minimum lap (measured net) Formwork Sawn Formwork to:- Side of column bases CARRIED TO COLLECTION BLD/03 | QTY 4 1 35 366 244 67 | UNIT m3 m3 m3 kg m2 m2 | RATE N | N | K |
| --- | --- | --- | --- | --- | --- | --- |

| A B C D | FORMWORK Sawn formwork to:- Vertical sides of columns Ditto column base Edges of Bed, 150mm wide Hollow Sandcrete Blockwork laid and bedded in cement and sand (1:5) filled solid with concrete( 1:4:8-40mm aggregate) as work proceeds 230mm thick wall Damp proof membrance 1200 guage polythence damp proof membrane lapped 450mm at all welted joints and laid on rock hardcore filling CARRIED TO COLLECTION BLD/04 | QTY 93 17 73 139 244 | UNIT m2 m2 m m2 m2 | RATE N | N | K |
| --- | --- | --- | --- | --- | --- | --- |

|  | COLLECTION Page BLD\01 Page BLD\02 Page BLD\03 Page BLD\04 SUBSTRUTURE CARRIED TO SUMMARY BLD/05 | QTY | UNIT | RATE N | N | K |
| --- | --- | --- | --- | --- | --- | --- |

| A B C D E F G | ELEMENT NR 2 FRAMES CONCRETE WORK Reinforced in situ concrete (1:2;4-19mm Aggregate filled into formwork and well packed around reinforcement (reinforcement and formwork measured separately) Column Roof beam Lintel Reinforcement High yield tensile reinforcement bars, including hooks, bends, distance blocks, tying wire, etc (Provisional) 10-16mm Sawn from work to:- Vertical side of columns Roof beam Lintel FRAMES & CONCRETE WORK CARRIED TO SUMMARY BLD/06 | QTY 2 5 4 1370 39 58 48 | UNIT m3 m3 m3 kg m2 m2 m2 | RATE N | N | K |
| --- | --- | --- | --- | --- | --- | --- |

| A B C D E F G H J K L M | ELEMENT NR 3 ROOF COVERING AND STRUCTURES 0.55mm thick aluminium longspan roofing sheets fixed with double corrugation side laps to hardwood purlin at an average of 450mm center (measured separately) and including all fixing accessories measured nett no allowance for laps. Roofing laid to slope Ridge capping 600mm girth Valley 300mm Protect all works included in this section Sundries 12mm Mild steel rag bolt 300mm long with nut and washers, one end fish tailed and built into concrete WOODWORK 50 x 75mm Purlins 50 x 150mm Rafters 50 x 150mm struts 50 x 150mm tie beam 50 x 150mm kingpost 75 x 100mm wall plate 30 x 300mm fascia board ROOFING COVERING AND STRUCTURES CARRIED TO SUMMARY BLD/07 | QTY 276 35 - 200 345 210 135 155 55 62 67 | UNIT m2 m m item Nr m m m m m m m | RATE N | N | K |
| --- | --- | --- | --- | --- | --- | --- |

| A B C | ELEMENT NR 4 WINDOWS Supply and fox the following Green colour Aluminium sliding windows complete with 5mm tinted glass fixed into Aluminium frame assembled according to manufacturer’s to specification and as designed by the Architect with sliding mosquito net (Provisional) Ditto sliding window size 1500 x 1200mm high Ditto size 900 x 1200mm high Ditto size 600 x 600mm high WINDOWS CARRIED TO SUMMARY BLD/08 | QTY 11 3 5 | UNIT Nr Nr Nr | RATE N | N | K |
| --- | --- | --- | --- | --- | --- | --- |

| A B C | ELEMENT NR 5 BUGLARY PROOF TO WINDOWS 25 X 25mm black thick hollow pipe burglary proof to all windows welded to approved design patterns, with and including building in cutting and pinning lughs and make good all works disturbed (price to include painting same with gloss paint) Ditto size 1500 x 1200mm high Ditto size 900 x 1200mm high Ditto size 600 x 600mm high BURGLARY PROOF CARRIED TO SUMMARY BLD/09 | QTY 11 3 5 | UNIT Nr Nr Nr | RATE N | N | K |
| --- | --- | --- | --- | --- | --- | --- |

| A B C | ELEMENT NR 5 DOORS Metals works Supply and fix the following (high quality) fabricated from thick Rectangular frame including all necessary ironmongery Complete finished 50mm thick factory steel high quality door frame combination including lock imported (security sure door) 1200 x 2100mm high D1 Ditto size 900x 2100mm high D2 Ditto size 750 x 2100mm high D3 DOORS CARRIED TO SUMMARY BLD/10 | QTY 1 8 4 | UNIT Nr Nr Nr | RATE N | N | K |
| --- | --- | --- | --- | --- | --- | --- |

| A B C D E F | ELEMENT NR 6 FLOOR FINISHERS 300 X 300 X 10mm thick vitrified non-slip ceramic tiles and jointed in cement and sand (1:5) on screeded bed (measured separately) Tilings 600 x 600 x 10mm thick glazed vitrified ceramic tiles laid and jointed in cement and sand (1:5) on screeded bed (measured separately) Tilings Ditto skirting BEDS AND BACKING 40mm thick screeded beds to receive ceramic tiling Ditto skirting BLOCK WALL Hollow sandcrete block wall in cement & sand (1:5) morter on: 230mm thick wall FLOOR FINISHES CARRIED TO SUMMARY BLD/11 | QTY 13 117 345 130 345 375 | UNIT m2 m2 m m2 m m2 |  | RATE N | N | K |
| --- | --- | --- | --- | --- | --- | --- | --- |
| A B C D E F G H J | ELEMENT NR 7 WALL FINISHES INTERNALLY 15mm thick cement and sand (1:5) Rendering to wall Rendering to column Ditto 200 – 300mm grith Painting Prepare and apply one undercoat and two finishing coats of ``Portland`` emulsion paint on: External Rendered wall Ditto internally wall EXTERNALLY 15mm thick cement and sand (1:5) Rendering to wall Rendering to column Ditto 200 – 300mm grith 300 x 300 x 10mm thick vitrified non-slip ceramic tile & jointed in cement & sand 1:5 on sreeeded bed measured separately on: Wall WALL FINISHES CARRIED TO SUMMARY BLD/12 | QTY 415 30 - 156 445 146 10 - 65 |  | UNIT m2 m2 m m2 m2 m2 m2 m m2 | RATE N | N | K |
| A B | ELEMENT NR 8 CEILING FINISHES PVC ceiling PVC plastic ceiling tiles to noggins (m/s) ceiling boards covered with decorative measured separately according to Manufacturer specification. 50mm x 50mm noggins nailed to plastic ceiling CEILING FINISHES CARRIED TO SUMMARY BLD/13 | QTY 170 700 | UNIT m2 m |  | RATE N | N | K |

| A B C | ELEMENT NR 9 ELECTRICAL INSTALLATION Allow for provisional sum of N( ) for electrical installation including, fittings and other accessories to be expended as directed by Electrical Engineer ELECTRICAL INSTALLATION CARRIED TO SUMMARY MECHANICAL INSTALLATION Allow for provisional sum of N( ) for all the plumbing works including piping. MECHANICAL INSTALLATION CARRIED TO SUMMARY FOUL DRAINAGE Allow for provisional sum of N( ) for foul drainage soakaway and septic tank and inspection chambers as directed by supervisor FOUL DRAINAGE CARRIED TO SUMMARY BLD/14 | QTY | UNIT Sum Sum Sum | RATE N N N | N | K |
| --- | --- | --- | --- | --- | --- | --- |

| A B C D E F G H J K L M | SUMMARY SUBSTRUCTURE FRAMES AND CONCRETE WORK ROOF COVERING & STRUCTURES WINDOWS BURGLARY PROOF DOORS FLOOR FINISHES & BLOCK WALL WALL FINISHES CEILING FINISHES ELECTRICAL INSTALLATION PLUMBING INSTALLATION FOUL DRAINAGE SOAKAWAY AND SEPTIC TANK AND INSPECTION CHAMBERS TOTAL COST BLD/15 |  |  | N | N | K |
| --- | --- | --- | --- | --- | --- | --- |

| A B C D E | GENERAL SUMMARY PRELIMINARIES CONSTRUCTION OF A PROPOSED RESIDENTIAL DEVELOPMENT ALONG EIYENKORIN ASA LGA KWARA STATE. Include the sum of N for external works as directed SUB TOTAL Add CONTINGENCIES Provide the provisional sum of N( )for Contingencies to be expended as directed Add Consultancy for Architect and Engineer TOTAL COST GS/01 |  |  | N | N | K |
| --- | --- | --- | --- | --- | --- | --- |

CHAPTER FIVE

CONCLUSION AND RECOMMENDATIONS

5.1 CONCLUSION

In conclusion, we observe that estimating and budgeting for medium scale building project is very important,

It enable all contractors tendering for a contract to price on exactly the same information of variation which often.

5.2 RECOMMENDATION

It is submitted that adequate planning comprehensive pre-project planning, comprehensive brief from the client guided by feasibility and viability studies should be embarked upon before the commencement of any project and to cap it all, a proper and carefully calculated estimate must be made to adequately budget a substantial amount before the start of any project.

In view of the above, it is recommended that:

Before the commencement of any project, a site visit and investigation should be carried out i.e the site should be subjected to scientific analysis like the 500l bearing capacity test, ground water table test and topographic survey.

REFERENCE

Building and Engineering Standard of Measurement (BESMM) Christopher J.Willis (2020) Element of Surveying 7th Edition

Ivor H.Seeley (2018), Building Quantities 3rd Edition MacMillan Press Ltd

Onoavah, T.C (2015): Building Measurement and Quantities