ESTIMATION AND INVESTIGATION OF VARIABILITY OF TROPOSHERIC RADIO REFRACTIVITY AND RADIO FIELD STRENGTH OVER ZARIA, NIGERIA

BY:

MUHAMMED SHAKIRAT OYINDAMOLA		HND/23/SLT/FT/0445

BEING A PROJECT REPORT SUBMITTED TO THE DEPARTMENT OF SCIENCE LABORATORY TECHNOLOGY (PHYSICS/ELECTRONIC UNIT), INSTITUTE OF APPLIED SCIENCES, KWARA STATE POLYTECHNIC ILORIN

IN PARTIAL FULFILMENT OF THE REQUIREMENTS FOR THE AWARD OF HIGHER NATIONAL DIPLOMA (HND) IN SCIENCE LABORATORY TECHNOLOGY (SLT), KWARA STATE POLYTECHNC, ILORIN, KWARA STATE

AUGUST, 2025

i

CERTIFICATION

This is to certify that this project was carried out by group A with the above names and matriculation numbers, submitted to the Department of Science Laboratory Technology, Physics/Electronics unit, Institute of Applied Science (IAS), Kwara State Polytechnic, Ilorin, in partial fulfilment for the requirement of the award of Higher National Diploma (HND) in Science Laboratory Technology (SLT).

___________________						_______________

Dr. IBRAHIM B.B								Date

Project Supervisor

___________________						_______________

Mr. Salahu Bashir								Date

HOU, Physic/Electronics

___________________						_______________

Dr. Abdulkareem Usman							 Date

Head of Department (SLT)

___________________						_______________

External Examiner								Date

ii

DEDICATION

This work is dedicated to Almighty God, for his infinite mercy and Grace. Our utmost appreciation goes to our parents for their parental support all through the course of our academic pursuit. Allah bless you all

iii

ACKNOWLEDGEMENT

First and foremost, I give all thanks to Almighty God for the strength, wisdom, and grace to complete this project successfully.

My sincere appreciation goes to my supervisor, Dr. Ibrahim Bolaji Balogun, for his guidance, support, and encouragement throughout this work. Your patience and dedication truly made a difference.

I am deeply grateful to my wonderful parents, Mr. and Mrs. Muhammed, for their endless love, prayers, and support. Thank you for always believing in me.

To my family and friends, thank you for standing by me and cheering me on, even during the most challenging times.

A heartfelt thanks also goes to my course mates in SLT (Physics and Electronics Unit), Kwara State Polytechnic, for the moments we shared and the teamwork that made the journey easier. This project wouldn’t have been possible without all of you. Thank you.

iv

TABLE OF CONTENTS

Title page

Certification

Dedication

Acknowledgment

Table of content

Abstract

CHAPTER ONE: INTRODUCTION

1.1	Introduction

1.2	Statement of Problem

1.3       Significance of Study

1.4	Aim and Objectives

CHAPTER TWO: LITERATURE REVIEW

2.0	Literature Review

2.1 	Temperature

2.2 	Relative Humidity

2.3 	Water Vapor
2.4       Saturated Water Vapor

2.5      Atmospheric Pressure

CHAPTER THREE: METHODOLOGY

3.1	Study Area

3.2	Data Acquisition

3.3	Theoretical Background

CHAPTER FOUR: RESULTS, DISCUSSION AND CONCLUSSION

4.1	Results

Conclusion

References

V

ABSTRACT

In this study, the yearly and monthly variation of tropospheric radio refractivity and field strength variability was estimated using the monthly average of daily temperature, relative humidity and atmospheric pressure meteorological parameters during the period of eleven years (2013 – 2024). The results shows that higher values of monthly tropospheric radio refractivity were recorded during the rainy and dry seasons, respectively, with highest and lowest value in the month of the months of September and February with 340.0675 N-units and 270.0318 N-units, respectively. The highest and lowest annual average value of tropospheric radio refractivity were found in the year 2022 and 2015 with 318.93 N-units and 304.00 N-units respectively. The study area under investigation yield average gradient of  -39.2031 N-units/km. Additionally, the average effective earth radius K was found to be 1.332. These values align with the condition of Sub-refraction propagation.

vi

CHAPTER ONE

1.1     INTRODUCTION

Refractivity is defined as the physical property of a medium as determined by its index of refraction and it is responsible for the different phenomena in radio wave propagation which comprises of refraction and fading, ducting and scintillation, range and elevation errors in radar acquisition (D. . Akpootu & Rabiu, 2019). The radio refractivity is a physical phenomenon that affects the propagation of radio signals in the Atmosphere. This process takes place in the layer of the atmosphere called the troposphere, which extends from the earth surface to a height of about 10 km along the poles of the earth and 17 km as we move towards the equator. The refractivity itself is being affected by the tropospheric parameters (Temperature, Relative Humidity and Atmospheric Pressure) (Daniel Effiong Oku, Amajama Joseph, 2015). Changes in the Radio refractivity parameters often leads to an alteration of the refractivity pattern. Well documented is the fact that for a tropical climate characterized by two seasons, which are the dry and rainy seasons, Radio refractivity increases in the rainy season and drops in the dry season, as a result of the presence of more water vapor in the atmosphere (Oku, 2015). The behavior of radio waves in the tropospheric layer of the Earth’s atmosphere is very important in this modern age that is highly influenced by radio communications ranging from mobile telephoning through terrestrial digital broadcasting to the propagation of satellite radio signal through the  troposphere (Abimbola, 2021). Radio wave is a very important component of our civilization it is used for point to point communication as well as for data exchange (Falaiye et al., 2016). The structure of the radio refractive index, n, at the lower part of the atmosphere is a very important parameter in planning of the communication links. It is defined as a ratio of the radio wave propagation velocity in free space to its velocity in a specified medium (Valma, 2011). It is a well-established fact that radio wave in the Very High Frequency (V.H.F)., Ultra High Frequency (U.H.F)., and Super High Frequency (S.H.F) bands propagating through the troposphere can be greatly influenced by the variations in the tropospheric weather condition (Onujagbe, 2021).  The composition of the atmosphere has a significant impact on radio wave signal transmission in the lower atmosphere, or troposphere  (Bello et al., 2024). Increased dependence on radio communication in Nigeria through the use of communication technologies such as laptops, smartphones and computers  1in our day to day life ease the way of doing businesses and social activities such as mobile banking, e-businesses, access to news and usage of social media (Sa’adu et al., 2020).

1

1.2    STATEMENT OF PROBLEM

The troposphere plays a critical role in the propagation of radio waves, particularly in the Very High Frequency (VHF) and Ultra High Frequency (UHF) bands, which are widely used for communication and broadcasting. Radio refractivity, a key parameter in tropospheric radio wave propagation, is influenced by meteorological factors such as temperature, pressure and humidity. Variability in these meteorological parameters can lead to significant fluctuations in radio refractivity, which in turn affects radio field strength and signal quality. In regions like Zaria, Nigeria, where meteorological conditions exhibit seasonal and diurnal variations, understanding the behavior of tropospheric radio refractivity and its impact on radio field strength is crucial for optimizing communication systems.

Despite the importance of this phenomenon, there is limited research on the estimation and investigation of both daily and monthly tropospheric radio refractivity and its variability over Zaria, Nigeria at the same time. Existing studies in other regions may not accurately reflect the local conditions due to differences in climate, topography, and atmospheric dynamics. This gap in knowledge poses challenges for the design and optimization of reliable communication networks in the area. Furthermore, the lack of localized data on radio field strength variability hinders the ability to predict signal degradation and implement effective mitigation strategies.

1.3   SIGNIFICANCE OF STUDY

Estimation and Investigation of Variability of Tropospheric Radio Refractivity and Radio Field Strength over Zaria, Nigeria" holds significant importance for both scientific and practical applications, particularly in the fields of meteorology, telecommunications, and radio wave propagation. This study will provide a comprehensive understanding of tropospheric radio refractivity and its variability, as well as its implications for radio field strength over Zaria, Nigeria. The results will contribute to the development of more reliable and efficient communication systems in the region, particularly in the face of changing climatic conditions. It will also help in determining the performance of radio communication systems. Variability in radio refractivity can lead to signal fading, interference, and loss. This will provides data and models that can help engineers and researchers develop strategies to mitigate these issues, ensuring more reliable communication networks. The result findings can inform policymakers and regulatory bodies in Nigeria about the challenges and opportunities related to radio communication in the region, which can lead to better planning and allocation of resources for communication infrastructure development.

2

1.4    AIM AND OBJECTIVES

Aim:

To estimate and investigate the variability of tropospheric radio refractivity and radio field strength over Zaria, Nigeria, and assess their implications for radio communication systems while the specific objectives are to:

OBJECTIVES:

1. Collect and analyze meteorological data (temperature, pressure, and humidity) required for the computation of tropospheric radio refractivity over Zaria,

2. Estimate the tropospheric radio refractivity and its seasonal variations in the study area,

3.  Investigate the relationship between meteorological parameters and radio refractivity,

4. Evaluate the variability of radio field strength and its dependence on tropospheric radio refractivity,

5.  Provide recommendations for optimizing radio communication systems in the region based on the findings and

6.  Compare the results with existing models and studies in other regions to identify unique characteristics of Zaria's tropospheric conditions.

3

CHAPTER TWO

2.0   LITERATURE REVIEW

(D. Akpootu & Iliyasu, 2017) estimated tropospheric radio refractivity under varying meteorological conditions in Ikeja, Nigeria. The results indicated that an average value of 385.99 N-units and an average value of 382.94 N-units were observed during the rainy and dry seasons respectively. (D. . Akpootu & Rabiu, 2019) developed empirical models for estimating tropospheric radio refractivity for Zaria, Nigeria, using meteorological parameter. The study showed that radio refractivity with relative humidity was found more suitable for the one variable correlation.

(Academy et al., 2024) provided valuable insights into the behavior of vertical radio refractivity gradients in the lower atmospheric layer across different cities in Nigeria. The findings contribute to a deeper comprehension of how atmospheric factors influence radio signals behavior and propagation paths, thus aiding in the optimization of communication networks and signal reliability under varying meteorological conditions.

(Onujagbe et al., 2021) observed that surface radio refractivity over Zaria mean monthly values of the surface refractivity, Ns for the Zaria station were observed to be generally high. The implication of this is that radio wave propagating through the atmosphere will bend closer towards the earth surface.

(X. Zou, Y H. Kuo, 1995) monthly average radio refractivity of Zaria, latitude 13.030 N, Longitude 5.20 E and elevation 270.0 m, Ikeja, latitude 6.580

N, Longitude3.330E and elevation 4.0 m, and Niger, latitude 7.80 N, Longitude 6.730E and elevation 128.0m have been determined using the data of temperature, pressure and relative humidity for ten years (1971 to 1980) of NIMET. The result showed that there was variation in the radio refractivity of the study areas but more noticed in Niger compared to

Ikeja and Lokoja

4

2.1   TEMPERATURE

Temperature is a measure of the average kinetic energy within a body. It describes the potential for heat energy to move from one body to another down a gradient from an area of

high temperature to an area of lower temperature. It is measured using a temperature scale which is defined against fixed physical events such as absolute zero or the triple point of water (Gardner, 2020). A thermometer is a device that has some property that changes with temperature so that, when properly calibrated, it can be used to measure temperature. One such device could be a mercury-in-glass thermometer. As the temperature of the mercury increases the mercury expands and this expansion in the glass is a measure of the temperature (Halliday et al., 2005).

2.2 RELATIVE HUMIDITY

Relative humidity (RH) indicates the amount of water vapor (percent) that’s actually in the air compared to the maximum amount that the air could hold under the same conditions. The warmer the air, the more moisture it can hold (Aprilaire, 2013).

2.3   WATER VAPOR

Water vapor is one of the gases in air. Unlike nitrogen and oxygen which are constant in the bottom 100 km of the atmosphere, water-vapor concentration can vary widely in time and space. Most people are familiar with relative humidity as a measure of water-vapor concentration because it affects our body’s moisture and heat regulation. But other humidity variables are much more useful in other contexts. Storms get much of their energy from water vapor when water vapor condenses or freezes it releases latent heat. For this reason we carefully track water vapor as it rises in buoyant thermals or is carried by horizontal winds. The amount of moisture available to a storm also regulates the amount of rain or snow precipitating out. What allows air to hold water as vapor in one case, but forces the vapor to condense in another? This depends on a concept called “saturation” (Wang, 2017).

5

2.4   SATURATED WATER VAPOR

A saturated vapor is a vapor that is in contact with its own liquid within a confined space. When the enclosed space above a liquid is saturated with vapor molecules and can hold no more molecules, the pressure exerted by this saturated vapor is said to be the saturated vapor pressure (s. v. p) of the liquid. The vapor is said to be saturated when the number of molecules escaping from the liquid per unit is equal to the number returning to the liquid per unit time. The saturated vapor is thus said to be in a state of dynamic equilibrium with its own liquid. Saturated vapor pressure increases with temperature (Ummah, 2019)

2.5   ATMOSPHERIC PRESSURE

The air around you has weight, and it presses against everything it touches. That pressure is called atmospheric pressure, or air pressure. It is the force exerted on a surface by the air above it as gravity pulls it to Earth. Atmospheric pressure is commonly measured with a barometer. In a barometer, a column of mercury in a glass tube rises or falls as the weight of the atmosphere changes. An atmosphere is a unit of measurement equal to the average air pressure at sea level at a temperature of 15 degrees Celsius. One atmosphere is 760 millimeters of mercury (Atkinson & Aschmann, 1985)

6

CHAPTER THREE

3.1   STUDY AREA

The study area is Zaria, Kaduna State, Nigeria. It is one of the aviation campus representing the North-West aviation college . It is located between Latitude 11°0'N and 11°10'N   and between Longitudes 7º34'E and 7º48'E. It comprises two Local Government Areas (LGAs) of Sabon-Gari and Zaria. Bounded to the west and North- west by Giwa LGA, to the South by Igabi, to the North- east by Kudan, and to the South-east by Soba L.G.As of the State. The study area is characterized by tropical continental climate (the Koppen’s Aw climate), with two well pronounced seasons (i.e. wet and dry) (Hazo et al., 2020).

7

3.2   DATA ACQUISATION

The data used for this study were 11-year period (2013 to 2024) of daily temperature, atmospheric pressure and relative humidity and 9- years (2013-2022) of the Monthly average temperature, atmospheric pressure and relative humidity of the study area. The data were retrieved from the archive of the Nigeria Meteorological Agency (NIMET) Oshodi, Lagos, Nigeria.

3.3   THEORETICAL BACKGROUND

N = Refractivity, measured in N-unit

N= 77.6TP+4810eT=Ndry+Nwet  …………………….. (1)

Expand to get Ndry and Nwet

With the “dry term” of radio refractivity given by:

Ndry=77.6pT   …………………………………………………… (2)

With the “wet term” of radio refractivity given by:

Nwet=3.73×105eT2 …………………………………………. (3)

Where P= Atmospheric Pressure (hPa)

T= Absolute Temperature (K)

e= Water Vapour (hPa)

8

The relationship between water vapour pressure (e) and relative humidity (RH) is given by the expression:

e= Hes100   ………………………………………………… (4)

Where H = Relative Humidity

es = Saturated Water Vapor

es=a exp (btt+c)  ………………………………… (5)

Where, a = 6.1121

b = 17.502

c = 24097

t = Temperature (℃)

Refractivity Gradient:

The radio refractivity N, also decrease exponentially in the troposphere with height:

No = Nsexp (hoH)  OR  Ns = 315exp-0.136h

Where, Ns = Refractivity at surface of the earth

No = Average values of atmospheric extrapolated to sea level

9

ho = Height of the earth’s surface above sea level

Ho = Scale height = 7.35km

dNodh  = -NsHoexp(-hoHo) refractivity gradient

Thus, k may be expressed in terms of refractivity gradient, dN/dh as:

K = [1+(dNdh)157]-1 …………………………………………. (5)

FSV – Field Strength Variation =

(Ns(max)- Ns(max)) × 0.2db ……………………… (6)

db= decibel

10

CHAPTER FOUR

4.1   RESULTS AND DISCUSIONS

Radio refractivity and its variation with other meteorological parameters

Figure 1 shows the yearly variation of radio refractivity for Zaria, Nigeria during the period under investigation. The figure shows fluctuations in the pattern of variation which is attributed to the yearly variation of atmospheric weather parameters. The highest yearly average value of 16.05 N – units was observed in the year 2014 and the lowest yearly average value of 11.09N – units in 2013.

FIG 1

FIG 1: Yearly variation of radio refractivity over Zaria, Nigeria

11

FIG 2

Fig. 2: Variation of Mean monthly surface refractivity value for Zaria

The mean monthly refractivity value at the Zaria station was observed to increase slowly from its minimum value of 270.0318 N-units in the month of February through March and thereafter rose sharply through April to its maximum value of 340.0675 N-units in September; it however decreases slowly through the month of October till decreases sharply through December.

12

FIG 3

Fig. 3: Variation of Mean yearly surface refractivity value for Zaria

The mean yearly refractivity value at the Zaria station was observed to increase slowly from its minimum value of 304.0778N-units in the year 2015 and thereafter rose sharply to its maximum value of 318.9371 N-units in the 2022.

13

CONCLUSION

The international telecommunication union (ITU) method has been employed to estimate the yearly and monthly tropospheric radio refractivity over Zaria, Nigeria using meteorological parameters of temperature, relative humidity and atmospheric pressure during the period of eleven years (2013 – 2024). The yearly and monthly variations of radio refractivity was also studied. The refractivity gradient, and the field strength variability were also reported. The results in this study revealed that slightly higher values of monthly tropospheric radio refractivity were recorded in the rainy season than in the dry season with the highest and lowest values in the month of February and September with 340.0675 N-units and 270.0318 N-units respectively. The highest and lowest annual average values of tropospheric radio refractivity were found in the year 2022 and 2015 with 318.93  and 304.00 N – units respectively.. The refractivity gradient and effective earth radius found for Zaria, Nigeria during the period under investigation were -39.2031 N-units/km indicating that the propagation in this region is mostly Sub-refraction. This is also supported by the value of effective earth radius, which is less than 4/3 The highest and lowest values of field strength variability (FSV) was estimated in the year 2014 and 2014 as 16.37 dB and 11.88 dB respectively.

14

REFERENCES

Abimbola, O. J., Bada, S. O., Falaiye, A. O., Sukam, Y. M., Otto, M. S., & Muhammad, S. (2021). Estimation of radio refractivity from satellite-derived meteorological data over a decade for West Africa. Scientific African, 14, 1–8. https://doi.org/10.1016/j.sciaf.2021.e01054

Academy, N. D., Osuagwu, F. N., State, K., & Akinbolati, A. (2024). EXPLORING VERTICAL GRADIENTS OF RADIO REFRACTIVITY AND THEIR SIGNIFICANCE FOR RADIO WAVE PROPA ABUJA , JOS AND MAKURDI OF NORTH-CENTRAL NIGERIA. Science World Journal, 19(1), 58–63. https://doi.org/10.4314/swj.v19i1.8

Akpootu, D. ., & Rabiu, A. . (2019). Empirical Models for Estimating Tropospheric Radio Refractivity Over Osogbo , Abstract : THE OPEN ATMOSPHERIC SCIENCE JOURNAL, 13, 43–55. https://doi.org/10.2174/1874282301913010043

Akpootu, D., & Iliyasu, M. (2017). Estimation of Tropospheric Radio Refractivity and Its Variation with Meteorological Parameters over Ikeja, Nigeria. Journal of Geography, Environment and Earth Science International, 10(1), 1–12. https://doi.org/10.9734/jgeesi/2017/32534

Aprilaire. (2013). Aprilaire explains how relative humidity affects indoor comfort.

Atkinson, R., & Aschmann, S. M. (1985). and Atmospheric Pressure. 17, 33–41.

Bello, G., Akpootu, D. O., & Sharafa, S. B. (2024). Assessment of Tropospheric Radio Refractivity and its Variation with Climatic Variables in the Guinea Savannah Region of Nigeria. European Journal of Theoretical and Applied Sciences, 2(3), 88–104. https://doi.org/10.59324/ejtas.2024.2(3).10

Daniel Effiong Oku, Amajama Joseph, E. P. O. (2015). Seasonal Tropospheric Radio Refractivity Variation in Calabar , Cross River State , Nigeria. June, 15–18.

15

Falaiye, O. A., Abimbola, O. J., Omojola, J., & Akinyanju, D. S. (2016). Spacio-Temporal Variation of Radio Refractivity in Lafia, Nasarawa State Using Cm Saf Atovs Satellite Data. FULafia Journal of Science and Technology, 2(1), 111–116. https://doi.org/10.62050/fjst2016.v2n1.40

Gardner, B. (2020). The physics of temperature. 26(6), 55–58.

Halliday, D., Resnick, R., & Walker, J. (2005). Chapter 18: Temperature, Heat and the First Law of Thermodynamics. Fundamentals of Physics Extended Edition, 477–500.

Hazo, A. I., Alemaka, J., Atiku, K., & Musa, A. B. (2020). Analysis of Trends and Variability in Air Temperature as Evidence of Climate Change in Zaria , Kaduna State , Nigeria. 6(1), 1–11.

M Idris, I Nouhou, MI Iliyasu, AO Aina, MJ Abdulsalami Ohaji, D. A. and M. (2024). Estimation and Investigation of the Variability of Tropospheric Radio Refractivity and Radio Field Strength over Accra ,. HSOA Journal Of Atmosopheric And Earth Science, 5(1), 1–9. https://doi.org/10.24966/AES-8780/100026

Oku, D. E., Joseph, A., & Charles, M. O. (2015). Observing the Radio-Refractivity Pattern Variation during Dry Season in. 4(11), 1179–1181.

Onujagbe, I., Ahmed-Ade, F., Mopah, M., & David, A. (2021). Surface Radio Refractivity of Akure and Ondo town, Southwest Nigeria. Nigerian Annals of Pure and Applied Sciences, 4(1), 212–219. https://doi.org/10.46912/napas.173

16

Sa’adu, L., Bawa, M., & Bello, M. (2020). A Review on the Effects of Radio Refractivity and Atmospheric Parameters on Signal Quality at Ultra High Frequency (UHF). International Journal of Science for Global Sustainability, 6(2), 91–99. https://doi.org/10.57233/ijsgs.v6i2.112

Ummah, M. S. (2019). VAPOUR PRESSURE. Sustainability (Switzerland), 11(1), 1–14.

Valma, E., Tamosiunaite, M., Tamosiunas, S., Tamosiuniene, M., & Zilinskas, M. (2011). Variation of radio refractivity with height above ground. Elektronika Ir Elektrotechnika, 5(111), 23–26. https://doi.org/10.5755/j01.eee.111.5.349

Wang, P. K. (2017). Water vapor Water vapor Liquid water (. 86–106.

X. Zou, Y H. Kuo, A. Y. R. G. (1995). Assimilation of Atmospheric Radio Refractivity Using a Nonhydrostatic Adjoint Model. American Metrological Society, 123, 2229–2249.

17