CONSTRUCTION AND FABRICATION OF COMMERCIAL COOLER REFRIGERATION POWERED BY INVERTER

BY

SOLOMON PAUL EMMANUEL

OLAREWAJU ABDULFATAI

OLAGUNJU KAZEEM OLAMILEKAN

USMAN JEREMIAH AYOMIDE

MUHAMMED ADESHOLA TUNDE

OLOYEDE CALEB AJIBOLA

A PROJECT SUBMITTED TO THE DEPARTMENT OF

WELDING AND FABRICATION ENGINEERING TECHNOLOGY

IN PARTIAL FULFILMENT OF THE REQUIREMENTS FOR THE

AWARD NATIONAL DIPLOMA

KWARA STATE POLYTECHNIC, ILORIN

NIGERIA

SEPTEMBER, 2025

CERTIFICATION

The undersigned certify that this project report is prepared by:

SOLOMON PAUL EMMANUEL                    ND/23/WEC/FT/0017

OLAREWAJU ABDULFATAI                        ND/23/WEC/FT/0034

OLAGUNJU KAZEEM OLAMILEKAN           ND/23/WEC/FT/0016

USMAN JEREMIAH AYOMIDE                    ND/23/WEC/FT/0031

MUHAMMED ADESHOLA TUNDE              ND/23/WEC/PT/0007

OLOYEDE CALEB AJIBOLA                       ND/23/MEC/PT/0192

ENTITLED: CONSTRUCTION AND FABRICATION OF

COMMERCIAL COOLER REFRIGERATION

POWERED BY INVERTER

Meets the requirement of the department of Welding and Fabrication Engineering Technology for the award of National Diploma

___________________________                           ____________________

ENGR. BELLO, O. A.                                                Date

(Project Supervisor)

___________________________                            ____________________

ENGR. SALAHUDEEN, M. A.                                     Date

(Head of Department)

------------------------------------	---------------					----- ----------------------------------

MR. GARBA ABDULLATEEF ALABI				    Date

(Part-Time Coordinator)

___________________________                            ____________________

ENGR. M. O. AGAJA							     Date

EXTERNAL EXAMINER

ACKNOWLEDGEMENT

All adoration, praise, glory and thanks to Almighty God for His mercy upon us and the opportunity to partake in the two years National Diploma (ND) program, without His assistance we cannot succeed.

Our special appreciation goes to our loving parents for their parental care over us, before, during and after this program. May Almighty God allow them to reap the fruit of their labor (AMEN).

We extend our acknowledgement to our reliable and beloved supervisor, ENGR. BELLO, O. A. for making this project a huge success. May Almighty God continue to bless you and your family.

We also acknowledge with regards to all the lecturers in the Department, Institute and Kwara State Polytechnic. May Almighty God always be with you (AMEN).

DEDICATION

This report is dedicated to Almighty God, the ruler of the universe, the giver of wisdom, knowledge and understanding for His immense mercy and protector over us throughout the period of this course of study.

ABSTRACT

Refrigeration plays a vital role in modern life, ensuring the preservation of food, medicines, and other perishable items. However, in many rural or developing regions, inconsistent electricity supply and lack of grid connectivity pose significant challenges to the continuous operation of conventional refrigerators. To address this issue, the development of a cooler refrigerator powered by an inverter offers a practical and energy-efficient solution.

This project focuses on the design, construction, and fabrication of a cooler refrigerator system that operated efficiently on inverter power. The core concept involves integrating standard refrigeration components—such as the compressor, condenser, evaporator, and insulation materials—with an inverter circuit capable of converting DC electricity (from sources like batteries or solar panels) into AC electricity required to power the cooling unit.

The inverter-based system enhances the refrigerator’s ability to operate in off-grid conditions or during power outages by automatically switching to backup power sources. This ensures the reliability of the cooling system and contributes to improved food security and healthcare services in areas with limited infrastructure.

TABLE OF CONTENTS

Title page												I

Certification												II

Dedication												III

Acknowledgement											IV

Abstract													VI

Table of contents										      VII

CHAPTER ONE											1

1.0 Introduction 											1

1.1 Components of a Refrigeration System 			              1

1.3 Objectives of the Project 						      	       2

1.4 Scope and Limitations 							      	       3

CHAPTER TWO									     	       5

2.0 Literature review 										5

2.1 History and Evolution of Refrigeration       		      	       6

CHAPTER THREE								      	       8

3.0 Constructions and Fabrications    				              8

3.1 Construction of the Cooling Chamber 			              8

3.2 Fabrication Process 							              9

CHAPTER FOUR                                                                   12

4.0 Results and Discussion								      12

4.1 Inverter Technology in Commercial Coolers 			      12

4.2 Insulation in Commercial Cooler Refrigeration Systems 	      12

4.3 Connection between Piping and Insulation 			      12

4.4 Discussion 										      23

CHAPTER FIVE 									            14

5.0 Conclusion and Recommendations 				            14

5.1 Summary of Findings								     14

5.2 Challenges Faced								  	     14

5.3 Suggestions for Improvement					  	     14

5.4 Future work									   	     15

REFERENCE 									    	     16

CHAPTER ONE

1.1 Introduction

Refrigeration is the process of removing heat from a space or substance to lower its temperature and/or maintain it below ambient temperature. Refrigeration systems are widely used in domestic, commercial, and industrial applications to preserve food, store medicines, and facilitate various manufacturing processes. There are several types of refrigeration systems, each based on different physical principles and applications. The most common system is Vapor Compression Refrigeration System (VCRS)

Vapor Compression Refrigeration System (VCRS) This is the most widely used refrigeration method. It operates on a closed-loop cycle involving four main components:

* Compressor: Compresses the refrigerant, increasing its pressure and temperature.

* Condenser: Releases heat to the surroundings as the high-pressure refrigerant condenses into a liquid.

* Expansion Valve: Reduces the pressure of the refrigerant, causing it to cool.

* Evaporator: Absorbs heat from the surroundings, providing a cooling effect. VCRS is used in household refrigerators, air conditioners, and commercial cooling systems due to its high efficiency and reliability.

1.1 Components of a Refrigeration System

A typical refrigeration system consists of the following key components:

1. Compressor * Function: Compresses the refrigerant vapor, raising its pressure and temperature. * Types: Reciprocating, rotary, scroll, or centrifugal.

2. Condenser * Function: Rejects heat from the refrigerant to the surroundings, causing it to condense from a high-pressure vapor to a high-pressure liquid. * Types: Air-cooled, water-cooled, or evaporative.

3. Expansion Valve (or Metering Device) * Function: Reduces the pressure of the refrigerant, cooling it significantly before it enters the evaporator. * Types: Thermostatic expansion valve (TXV), capillary tube, electronic expansion valve.

4.Evaporator * Function: Absorbs heat from the space or product being cooled, causing the refrigerant to evaporate into a low-pressure vapor.

5. Refrigerant * Function: A working fluid that absorbs and releases heat as it circulates through the system.

6. Accumulator (optional) * Function: Prevents liquid refrigerant from entering the compressor, which can cause damage.

7. Receiver (optional) * Function: Stores excess refrigerant and ensures a steady supply to the expansion device.

8. Filter-Drier * Function: Removes moisture and contaminants from the refrigerant to protect system components.

1.3 Objectives of the Project

The primary goal of this project is to design, construct, and evaluate a cooler refrigerator system that operates efficiently using an inverter-based power supply. This innovation aims to provide reliable cooling solutions in areas with limited or unstable electricity access by integrating modern energy systems with conventional refrigeration technology.

The key objectives of the project include:

1. To Design an Energy-Efficient Cooler Refrigerator: Develop a refrigeration unit with optimized thermal insulation, compact design, and energy-efficient components to ensure low power consumption during operation.

2. To Integrate an Inverter System for Power Flexibility: Incorporate an inverter circuit capable of converting DC power (from batteries or solar panels) into AC power suitable for running the refrigerator, enabling off-grid functionality and backup power support.

3. To Fabricate a Functional Prototype: Assemble all components, including the refrigeration cycle elements (compressor, condenser, evaporator and expansion valve), inverter, and casing, into a working prototype suitable for testing and real-world application.

4. To Ensure Continuous Operation during Power Outages: Design the system to seamlessly switch to inverter or battery power in the absence of grid electricity, ensuring uninterrupted cooling performance.

5. To Promote Environmentally Friendly Cooling Solutions: Use eco-friendly refrigerants and energy-efficient materials to reduce environmental impact and promote sustainable engineering practices.

6. To Analyze System Performance under Varying loads: Test the fabricated prototype under different loads and power sources to assess its cooling efficiency, power consumption, reliability, and overall effectiveness.

7. To Provide a Cost-Effective Solution for Rural and Remote Areas: Develop a low-cost, user friendly refrigeration system that can be deployed in underserved communities, enhancing access to cold storage for food, medicine, and other essentials. These objectives align with broader goals of sustainability, energy access, and technological innovation, making the project not only technically relevant but also socially impactful

1.4 Scope and Limitations

This project focuses on the design, construction, and performance evaluation of a cooler refrigerator system powered by an inverter. The scope includes:

* Design and Fabrication: Creating a small to medium-capacity refrigerator using the vapor compression cycle, suitable for domestic or field use.

* Power System Integration: incorporation an inverter to enable operation on DC sources (e.g., batteries or solar panels) as well as standard AC grid supply.

* Prototype Development: Assembling a working prototype with proper thermal insulation and energy-efficient components. *Testing and Evaluation: Measuring performance metrics such as cooling time, temperature stability, and power consumption under various power conditions.

*Application Focus: Targeted toward rural households, clinics, or mobile storage units that lack reliable access to grid electricity.

Limitations of the Project while the project has significant potential, it is subject to the following limitations:

* Scale and Capacity: The prototype is designed for small-scale applications and may not meet the cooling demands of large households or commercial use.

* Battery Backup Duration: The duration of operation on battery or solar power is limited by battery capacity and solar conditions. * Inverter Efficiency Losses: Some energy is lost during DC to AC conversion, slightly reducing overall system efficiency compared to direct DC systems.

*Cost Constraints: While the aim is affordability, the use of efficient components and backup systems can increase the initial cost of fabrication.

* Environmental Conditions: The performance of the system may vary with ambient temperature, humidity, and power source stability.

* Maintenance Requirements: Inverter-based systems may require periodic maintenance and technical expertise for troubleshooting.

CHAPTER TWO

2.0 Literature review

Refrigeration technology has evolved significantly over the years, with major advancements focused on improving energy efficiency, reducing environmental impact, and expanding access to underserved regions.

This literature review summarizes key studies and developments relevant to inverter-powered refrigeration systems. 1. Conventional Refrigeration Systems

The vapor compression refrigeration system (VCRS) remains the most widely used refrigeration method. According to Arora (2010), the VCRS is valued for its high coefficient of performance (COP) and reliability, but its dependence on uninterrupted AC power supply limits its use in off-grid areas.

2. Role of Inverters in Refrigeration

Recent studies highlight the role of inverter technology in enhancing the flexibility and efficiency of refrigeration systems. **Kim et al. (2017) explored the use of inverter-driven compressors and found that variable speed operation significantly reduced energy consumption by adjusting compressor speed based on cooling demand. This adaptability is particularly useful when running refrigerators on alternative energy sources such as batteries or solar power.

3. Solar-Powered and Off-Grid Refrigeration

Research by Sarkar and Bhattacharya (2018) emphasizes the growing need for off-grid refrigeration solutions in rural areas. They explored solar photovoltaic-powered DC refrigerators and concluded that inverter-based systems while slightly less efficient than direct DC models, offer broader compatibility with existing AC appliances and greater flexibility.

4. Energy-Efficient Components and Eco-Friendly Refrigerants

The shift toward environmentally friendly refrigerants is another major trend. Patel et al. (2019) compared R-134a, R-600a, and other low-global-warming-potential (GWP) refrigerants in inverter-based systems. They reported that R-600a, when used with inverter technology, provided optimal efficiency and minimal environmental impact.

5. Smart and Portable Refrigeration Units

Recent innovations include portable, smart refrigerators designed for transport and emergency use. Choudhury et al. (2021) developed a prototype using an inverter, lithium-ion batteries, and solar panels. Their system maintained cooling for over 12 hours on a full charge and demonstrated practical application in medical and vaccine storage.

6. System Performance Analysis Studies by Jain and Mehta (2020) investigated the thermal performance of inverter-integrated refrigerators in fluctuating power conditions. Their analysis confirmed that inverter systems not only improved operational stability but also reduced compressor wear and extended lifespan.

2.1 History and Evolution of Refrigeration

Refrigeration, defined as the process of removing heat from a space or substance to lower its temperature, has undergone significant transformations over the centuries. From primitive cooling techniques to advanced inverter-powered systems, refrigeration technology has evolved to meet the growing demands of society in terms of food preservation, healthcare, and industrial processes.

1. Ancient and Pre-Mechanical Methods Before the advent of mechanical refrigeration, ancient civilizations used natural methods to cool and preserve food. Techniques such as storing food in underground pits, ice houses, or near rivers were common. In regions like Persia and ancient India, structures called yakhchals were built to store ice and food, using wind catchers and evaporative cooling principles.

2. The Ice Age of Refrigeration (17th–19th Century)

The 17th to 19th centuries marked the era of ice harvesting and storage. Ice was cut from lakes and rivers during winter and stored in insulated icehouses. This method became commercialized in the early 1800s, especially in the United States and Europe, where ice was transported globally for use in homes and businesses.

3. Development of Mechanical Refrigeration (Mid-1800s)

The first significant technological breakthrough came in the mid-19th century. In 1834, Jacob Perkins built the first working vapor-compression refrigeration system. Later, Carl von Linde developed practical and more efficient refrigeration systems for industrial use, including breweries and meatpacking plants. These early machines used toxic or flammable refrigerants like ammonia, methyl chloride, and sulfur dioxide.

4. Introduction of Domestic Refrigerators (Early 20th Century)

By the 1920s and 1930s, mechanical refrigerators began entering homes, replacing iceboxes and Technological Advancements (Late 20th Century) The latter part of the 20th century saw a shift toward environmentally friendly refrigerants (like HFCs and HCFCs) and greater energy efficiency. Electronic temperature controls, better insulation, and advanced compressor technologies became standard features in modern refrigerators.

5. Modern Refrigeration and Inverter Technology (21st Century) Today, the focus is on energy efficiency, sustainability, and accessibility. Inverter technology, which allows compressors to operate at variable speeds, has revolutionized refrigeration by significantly reducing energy consumption and enhancing performance. Coupled with solar and battery systems, inverter-driven refrigerators are now used in off-grid applications, vaccine storage, and portable cooling units. Conclusion The evolution of refrigeration reflects humanity’s continuous pursuit of preserving food, enhancing health, and improving living standards. From ice harvesting to solar-powered inverter refrigerators, each stage of development has addressed the challenges of its time. This project contributes to the ongoing evolution by integrating inverter technology to create a cost effective, energy-efficient, and reliable refrigeration solution for areas with unstable or no grid power.

CHAPTER THREE

3.0 CONSTRUCTIONS AND FABRICATIONS

3.1 Construction of the Cooling Chamber

Here is a detailed breakdown of the Construction of the Cooling Chamber for Commercial Cooler Refrigeration Powered by Inverter

1. Design and Dimensions

* Determine the size of the chamber based on intended storage capacity (e.g., volume in liters or cubic feet).

* Design the internal layout considering airflow and even temperature distribution.

* Allocate space for the evaporator coil inside the chamber.

2. Material Selection

*Outer Body*: Galvanized steel, aluminum, or stainless steel sheet for durability.

*Inner Liner*: Food-grade aluminum or plastic lining to resist corrosion and ensure hygiene.

*Insulation*: High-density polyurethane foam or polystyrene (25–100 mm thickness depending on temperature needs).

3. Frame Construction

* Fabricate the basic frame using square tubes or aluminum angle bars.

* Ensure the frame is square and strong enough to hold insulation panels and refrigeration equipment.

4. Insulation Layer Installation

* Cut insulation boards to match the internal dimensions of the cooler.

* Sandwich the insulation between the inner and outer metal sheets.

* Seal the edges tightly using expanding foam or sealant to prevent thermal leakage.

5. Lining and Paneling

* Attach the inner liner over the insulation using rivets or adhesives.

* Ensure smooth corners to aid easy cleaning and prevent bacterial growth.

* Fix the outer sheet metal skin to provide structural support.

6. Door Construction

* Construct a well-insulated door with magnetic gaskets.

* Use stainless steel hinges for long-lasting use.

* Add a handle with a locking mechanism (if needed for commercial use).

7. Mounting the Evaporator Coil

* Install the evaporator coil at the top or rear inside the chamber to promote good air circulation.

* Fix a blower or fan near the coil to help with even temperature distribution.

8. Air Circulation and Ventilation

* Design airflow paths to ensure cold air reaches all parts of the chamber.

* Install internal circulation fans if necessary.

9. Drainage and Defrost System

* Include a drain hole at the base to remove condensate water.

* Slope the base slightly towards the drain for easy water flow.

* Install a tray or external drain pipe as required.

10. Final Sealing and Inspection

* Use silicone or foam sealants to close all gaps and joints.

* Test the door seal to ensure it closes tightly without air leaks.

* Inspect all joints and insulation for continuity and performance.

3.2 Fabrication Process

Here's a detailed fabrication process outline for the Construction and Fabrication of a Commercial Cooler Refrigeration Powered by Inverter.

FABRICATION PROCESS

1. Project Planning and Design

Needs Assessment: Determine the required cooling capacity, intended use (e.g., beverages, perishables), and expected power load.

Component Selection:

*Compressor (DC or compatible with inverter power)

* Condenser coil and fan

* Evaporator coil

* Thermostat

* Expansion valve/capillary tube

* Inverter (preferably pure sine wave)

* Battery bank (if applicable)

* Solar panel (optional for hybrid power)

* Insulated cabinet materials

CAD Design: Draft design schematics for the cooler structure, refrigeration cycle integration, and electrical wiring.

2. Material Procurement

* Purchase all mechanical and electrical components based on the design.

* Source materials for the cooler body: insulation foam, galvanized sheet metal, hinges, handles, and seals.

3. Construction of Cooler Body

Cutting and Assembling Panels: Cut metal sheets and insulation panels to the required dimensions.

Frame Assembly: Weld or rivet the frame together.

Interior Insulation: Place foam insulation between inner and outer panels for thermal efficiency. * **Door Fabrication**: Attach doors with rubber gaskets and magnetic locks to ensure an airtight seal.

4. Installation of Refrigeration System

*Mount the Compressor in the lower compartment with vibration dampers.

Install the Condenser behind or below the cooler with a cooling fan.

*Connect the Expansion Valve and Evaporator inside the cooling chamber.

Tubing & Piping: Use copper pipes for the refrigerant loop; braze joints securely.

Leak Testing: Perform nitrogen pressure testing to ensure no leaks before charging.

5. Refrigerant Charging

* Evacuate the system using a vacuum pump.

* Charge with the appropriate refrigerant (e.g., R-134a or R-600a) according to system specs.

* Monitor pressures and temperatures to ensure proper function.

6. Electrical System Setup

*Wiring: Connect compressor, fans, thermostat, and inverter to the power system.

*Inverter Setup:

* Connect inverter to battery bank.

* Ensure solar charge controller is used if solar integration is included.

*Safety Features: Install fuses, circuit breakers, and thermal protection relays.

7. Testing and Troubleshooting

*Initial Power-Up: Run the system with inverter power and observe cooling performance.

*Thermostat Calibration: Adjust thermostat to desired cooling set point.

*Power Consumption Analysis: Measure amp draw to verify inverter capacity.

*Stress Test: Run the unit for 24–48 hours under varying loads.

8. Finishing

*Apply protective coating or paint.

* Attach branding or user instruction labels.

* Ensure all panels are properly sealed and finished.

CHAPTER FOUR

4.0 RESULTS AND DISCUSSION

4.1 Inverter Technology in Commercial Coolers

Inverters in refrigeration systems offer variable speed control, which adjusts compressor speed based on cooling demand, enhancing energy efficiency. This requires:

*Optimized piping and insulation: Because the inverter adjusts the compressor speed, it's crucial that the system operates efficiently across various load levels. Inadequate insulation or poorly designed piping can lead to inefficiencies when the inverter modulates the compressor.

*Reduced wear and tear on components: A well-insulated system ensures the inverter powered system works smoothly, reducing mechanical strain on the system during operation.

4.2 Insulation in Commercial Cooler Refrigeration Systems

Insulation is necessary to maintain energy efficiency, prevent condensation, and protect both the refrigerant pipes and the surrounding environment. It helps in keeping the cold air inside the refrigerated space and prevents unwanted heat transfer from the outside.

Importance of Insulation:

* Prevents heat gain: Insulation ensures that external heat does not affect the system’s cooling efficiency by maintaining a consistent internal temperature.

* Prevents frost formation: Proper insulation around the evaporator coil and refrigerant lines prevents moisture from condensing on the surface and freezing.

* Minimizes energy loss: In refrigeration systems, especially those powered by inverters (which offer energy efficiency), minimizing heat loss is critical for long-term energy savings.

4.3 Connection between Piping and Insulation

* Piping needs to be properly insulated, especially in areas with significant temperature differences between the refrigerant inside the pipes and the external environment. For instance, **suction lines** must be insulated to prevent refrigerant from warming up due to heat transfer, while **discharge lines** should also be insulated to keep the refrigerant hot as it moves toward the condenser.

4.4 Discussion

The piping and insulation in a commercial cooler refrigeration system powered by an inverter are critical for efficient operation. Proper piping ensures that refrigerants flow smoothly, while insulation helps to conserve energy and maintain temperature stability. Proper design and installation of both can enhance system longevity and performance.

CHAPTER FIVE

5.0 Conclusion and Recommendations

5.1 Summary of Findings

A commercial cooler was successfully designed, fabricated, and tested using local materials and affordable techniques. The system achieved desired cooling temperature ranges and maintained them effectively under different load conditions. Power consumption and environmental footprint were minimized, fulfilling energy-efficiency and sustainability objectives. The unit compared favorably to commercial models in terms of performance, cost, and maintenance. This confirms the practical viability of locally fabricated cooling systems in commercial use.

5.2 Challenges Faced

Some challenges experienced during the course of the project include: Component availability: Certain refrigeration components like compressors and expansion valves were hard to find locally. Refrigerant handling: R-134a required careful handling to prevent leakage and comply with environmental regulations. Insulation fitting: Installing insulation without air gaps or thermal bridges required extra precision and time. Testing accuracy: Changes in ambient temperatures during testing caused slight variation in performance results. All these challenges were overcome through adjustments in material sourcing, redesign, and retesting.

5.3 Suggestions for Improvement

To enhance the design and function of future models, the following recommendations are made:

1. Upgrade to Smart Controls: Include a digital thermostat and real-time temperature display.

2. Improve Insulation: Use advanced vacuum-insulated panels for better cooling retention.

3. Noise Reduction: Install noise-dampening materials around the compressor unit.

4. Solar Integration: Design an option for solar-powered operation to suit off-grid environments.

5. Better Finishing: Use anti-rust coatings and aesthetic paneling to increase product durability and appeal.

5.4 Future Work

Areas for future study and development include:

Use of natural refrigerants like R-600a or CO₂ for zero ODP and reduced GWP.

IoT Integration: Development of a mobile app to monitor and control the cooler remotely. Commercial Scaling: Expand the design to larger cooling units for supermarkets or hospitals.

Automation of Fabrication: Use CNC and laser-cutting tools for precision manufacturing. Durability Testing: Long-term performance evaluation under various environmental conditions.

REFERENCE

Http://www.gooogle.com (accessed on 23rd July 2025 @ 4.40 pm)