# MODIFICATION AND PERFORMANCE EVALUATION OF AN HYBRID EGG INCUBATOR

# BY

# AMOO BARAKAT OLASILE

# HND/23/ABE/FT/0122

BEING A RESEARCH PROJECT SUBMITTED TO THE

DEPARTMENT OF AGRICULTURAL AND BIO-ENVIRONMENTAL ENGINEERING TECHNOLOGY,

INSTITUTE OF TECHNOLOGY,

KWARA STATE POLYTECHNIC, ILORIN

IN PARTIAL FULFILMENT OF THE REQUIREMENT FOR THE AWARD OF HIGHER NATIONAL DIPLOMA (HND) IN

AGRICULTURAL AND BIO-ENVIRONMENTAL ENGINEERING TECHNOLOGY.

AUGUST, 2025

# DEDICATION

# This project is dedicated to Almighty ALLAH, who begin this project with me and also see me through the completion of the project.

# I also dedicate this project to my loving Parent, Mr. & Mrs. Amoo.

# ACKNOWLEDGMENT

All praise is due to Allah the most Gracious, the Most Merciful, for His endless mercy, protection, and strength throughout the journey of my academic pursuit. His grace has been my constant source of resilience and hope, even in the face of challenges.

I extend my sincere gratitude to my supervisor, Engr. (Mrs) Onipede, E.A for her invaluable guidance, corrections, encouragement, and commitment toward their success of this research. May Allah continue to bless you with wisdom and excellence.

To my wonderful parents, Mr. and Mrs. Amoo, thank you for your endless love, prayers, prayers and support. Your sacrifices have shaped who I am today. I am also deeply grateful to my uncle, Mr. Amoo Mukaila Ayogbe and my lovely brother, Mr. Shittu Babatunde and my siblings, for their unwavering financial and emotional support throughout my academic journey. I’m grateful to my friends and course mates for their collaboration, companionship, and emotional support throughout the years. You all make learning engaging and fulfilling.

TABLE OF CONTENTS

Title page

Certification

Dedication

Acknowledgement

Table of Contents

Abstract

Chapter One: Introduction

1.1 Background of the Study

1.2 Problem Statement of the Study

1.3 Aim and Objectives

1.4 Justification

1.5 Scope of the Study

Chapter Two: Literature Review

2.1 Poultry Farming in Nigeria

2.2 Poultry Incubation Technology

2.3 Natural Incubation

2.4 Artificial Incubation

2.5 Incubation Parameters

2.6 Reviews on Artificial Incubation

Chapter Three: Materials and Methods

3.1 Area of Modification

3.2 Materials

3.3 Description of the Hybrid Egg Incubator

3.3.1 The Main Cabinet

3.3.2 The Auxiliary Unit

3.4 Design Consideration

3.5 Design Calculation and Analysis

3.5.1 Determination of Incubation Chamber

3.5.2 Determination of Heat Required for Incubation

3.5.3 Heat Loss by Incubator

3.5.5 Determination of Humidity

3.5.6 Total Power Consumption

3.5.7 Power Requirement and Selection of Battery, Solar Panel, and Inverter

3.5.8 Size of Solar Charger

3.6 Material Selection

3.7 Construction and Assembly Procedure of Hybrid Egg Incubator

3.8 The Operation of Hybrid Egg Incubator

3.9 Bill of Engineering Measurement and Evaluation

3.10 Performance Evaluation of Hybrid Egg Incubator

3.11 Experimental Procedure

Chapter Four: Result and Discussion

4.1 Result

4.2 Discussion

Chapter Five: Conclusion and Recommendation

5.1 Conclusion

5.2 Recommendation

References

ABSTRACT

A hybrid egg incubator was modified and constructed to solve the problem of power outages that militate against poultry egg incubation. The hybrid egg incubator was constructed from medium density fibreboard (MDF) plywood and lagged with fibre glass to prevent heat loss. The hybrid egg incubator was made of two units; the main cabinet and auxiliary unit. The main cabinet consists of egg trays, bulbs, DC fans, water tray and turning mechanism. The auxiliary unit consists of solar PV panel, battery, inverter and charge control. The incubation parameters were measured by control board. The temperature and humidity range within the incubator were 36.1 - 37.5 °C and 50.1 % to 60% respectively, the angle of turn was 45°. The result of performance evaluation showed 91% hatchability, 66% fertility and mortality rate 9%. The hybrid egg incubator is affordable because it was made from locally available materials and it is therefore recommended for poultry farmers both at rural and urban centers.

CHAPTER ONE

INTRODUCTION

1.1 Background of the Study

Poultry farming is a significant part of Nigeria's agricultural sector, playing a key role in both the country's food security and economic growth. Egg production, in particular, is one of the most valuable aspects of poultry farming, providing affordable protein for many Nigerians (Adebanjo et al., 2020). However, the success of egg production relies heavily on the efficiency of incubation processes. Traditional poultry egg incubation methods often face several challenges, such as inconsistent temperature regulation, high energy consumption, and dependency on the national power grid, which is unreliable in many parts of Nigeria (Adebayo et al., 2021). These issues result in reduced hatchability rates and increased operational costs, which undermine the productivity and sustainability of poultry businesses.

To mitigate these challenges, modern incubators have been developed that utilize renewable energy sources, such as solar power, to supplement or replace conventional electricity. Hybrid poultry egg incubators, which combine solar and electrical power sources, offer promising solutions to enhance the efficiency and sustainability of poultry incubation processes. Previous studies have shown that solar-powered incubators are effective in maintaining optimal incubation conditions without depending solely on the power grid, thereby reducing energy costs and improving hatchability rates (Ogunjimi et al., 2019). Furthermore, hybrid incubators with advanced control systems ensure precise regulation of temperature and humidity, thereby enhancing the chances of successful hatching (Ajayi et al., 2022). Despite these advancements, there is still a need for further modification and performance evaluation of hybrid poultry egg incubators to fully optimize their use in Nigeria's poultry industry.

1.2 Problem Statement of the Study

The world’s growing population and increasing health awareness have led to a surge in demand for poultry product, which are perceived as a heathier alternative to red meat due to their lower cholesterol and trans fat content. this trend is expected to continue as driving growth in the poultry industry. The process of hatching egg by natural method is limited and the artificial incubators that can hatch more eggs at a time are faced with erratic power supply for an electrical incubator, adverse weather condition for solar incubator, lack of adequate control system of incubation system and besides an inadequate hatcheries contributing to high cost of day old chicks

1.3 Aim and Objectives

The aim of this study is to design, modify, and evaluate the performance of a hybrid poultry egg incubator that combines solar and electrical power systems to enhance hatchability rates and reduce energy consumption in Nigeria.

- To design a hybrid poultry egg incubator that integrates both solar and electrical power sources.

- To fabricate the incubator using locally available materials.

- To assess the temperature and humidity control capabilities of the incubator under different environmental conditions.

- To evaluate the hatchability rates of eggs incubated in the hybrid incubator compared to those incubated using conventional methods.

- To analyze the energy consumption patterns of the hybrid incubator and evaluate its cost-effectiveness in terms of both initial investment and operational expenses.

1.4 Justification

The development and optimization of hybrid poultry egg incubators are essential for improving the productivity of Nigeria's poultry sector. These incubators offer an affordable and sustainable alternative to traditional incubators by integrating renewable energy sources, such as solar power. This not only reduces the dependence on unreliable electricity supply but also lowers energy costs, making poultry farming more profitable. Additionally, the use of advanced temperature and humidity control systems in hybrid incubators enhances hatchability rates, thus ensuring a consistent and reliable supply of poultry stock. By addressing these critical issues, this study aims to contribute to the technological advancement of Nigeria's poultry industry, supporting the nation's agricultural development goals (Adebayo et al., 2021).

1.5 Scope of the Study

This study focuses on the design, modification, and performance evaluation of a hybrid poultry egg incubator for use in Nigeria. The study will cover the development of the incubator prototype, the laboratory-based evaluation of its temperature and humidity control systems, and field tests to assess the hatchability performance in various poultry species. The energy consumption and cost-effectiveness of the hybrid incubator will also be evaluated. The study will primarily focus on the climatic conditions in southwestern Nigeria, with potential implications for other regions in the country.

CHAPTER TWO

LITERATURE REVIEW

2.1 Poultry Farming in Nigeria

Poultry farming is a significant segment of Nigeria's agricultural sector, providing essential protein sources through meat and egg production. The country's poultry industry has experienced substantial growth, driven by a large population and increasing urbanization. Nigeria possesses the second-largest chicken population in Africa, estimated at approximately 180 million birds, with broilers constituting 70% and layers 30% of this population (Babban Gona, 2022). Despite this substantial production, local poultry farming meets only about 30% of the nation's demand for chicken meat and eggs, indicating a significant supply gap and potential for industry expansion (Agrocademy Blog, 2024).

2.2 Poultry Incubation Technology

Incubation is a critical process in poultry farming, influencing hatchability rates and the overall productivity of poultry operations. Advancements in incubation technology have led to the development of artificial incubation systems, which offer controlled environments for egg development. These systems regulate temperature, humidity, and ventilation, optimizing conditions for embryo growth and increasing hatch success rates. In Nigeria, the adoption of such technologies varies, with some farmers relying on traditional methods due to cost constraints and limited technical knowledge.

2.3 Natural Incubation

Natural incubation involves hens incubating their eggs, providing warmth and turning them to ensure even development. This traditional method has been practiced for centuries and is still prevalent in many Nigerian households. However, natural incubation is subject to variables such as the hen's experience, environmental conditions, and potential predation, which can affect hatchability rates. Additionally, the time commitment required from farmers and the limited number of eggs a single hen can incubate at a time pose challenges to scaling production.

2.4 Artificial Incubation

Artificial incubation utilizes mechanical devices to simulate the natural incubation environment, offering precise control over temperature, humidity, and turning frequency. This method allows for the simultaneous incubation of a large number of eggs, significantly enhancing hatchery efficiency and productivity. In Nigeria, the adoption of artificial incubation has been increasing, particularly among commercial poultry farmers seeking to improve hatchability rates and meet the growing demand for poultry products. However, challenges such as high initial investment costs, technical expertise requirements, and reliable power supply persist, limiting widespread adoption.

2.5 Incubation Parameters

Successful artificial incubation depends on the careful control of certain parameters that influence embryo development. These parameters include temperature, humidity, ventilation, and egg turning.

i. Temperature: The optimal temperature for hatching chicken eggs is usually between 37.5°C - 38°C (Meijerhof, 2019). Temperature fluctuations can lead to poor hatchability or embryo mortality.

ii. Humidity: Humidity levels should be maintained between 50-55% during the incubation period and increased to 65-70% during the last three days to prevent excessive moisture loss from the eggs (French, 2020).

iii. Ventilation: Adequate oxygen supply and removal of carbon dioxide are crucial for the embryo's survival. Insufficient ventilation can lead to weak chicks or embryo death (Davis & Zhang, 2021).

iv. Egg Turning: Regular turning of eggs is essential to prevent the embryos from sticking to the eggshell and to ensure proper development. Eggs are typically turned 4-6 times per day during incubation (Tullett, 2019).

v. Egg Fertility: Egg fertility is also an important parameter that enhance hatchability hence there is a need for egg candling to ascertain the fertility of eggs incubated.

TABLE 2.1: Condition for various egg incubation

Species     Incubation	   Temp (f)    Humidity       Do not 	  Humidity	Open vent
	    period (day)                         (f)	     turn after       last 3 days	more

Chicken 	21	      100        85- 87	   18 th day 	       90 		18 th day

Turkey		28	        99         84-86	  25 th day               90		25 th day

Duck		28	        99        85- 86	   25 th day 	       90 		25 th day

Muscovy	35-47	        100      84- 85	   31 th day              90		31 th day

Goose 		28-34	        99         86- 88	   25 th day 	       90 		25 th day

Guinea fowl	28	        100       85-87	  25 th day              90		25 th day

Pea fowl 	23-30	         99        84- 86	   25 th day 	      90 		25 th day

Pheasant          23- 28	         100      86-88	  21 th day              90		25 th day

Pigion		17       	         100      85- 87	   15 th day 	      90 		14 th day

Bob white	23-24	         100      84-87	  20 th day             90		20 th day

Cotunmix         17	         100      85-86          15 th day             90                14 th day

Chukar             23-24	         100      81-83          20 th day             90                 20 th day

2.6 Reviews on Artificial Incubation

Oniya et al., develop a 160 a capacity low cost semi-in automated incubator powered by a backup inverter. The inverter could either be powered with an AC or DC electric system. The major components of the system were a heat source(100 w bulb), a thermistor, 12 PV battery, 100 w inverter and 0.14 w electric fan. A replicated analysis on the system showed a temperature variation between 36 to 39° c with hatchability 91%.

Hassan et al., (2021) in characterizing and electric incubator recorded a hatchability percentage of 80.9% and concluded that the major challenge of incubators that can work with electricity are excessive billing rate, erratic power supply as well as complete absence of electricity in remote regions. Equally most of the incubator is developing countries like Nigeria operate at 60% below design capacity grid electric supply.

Olaoye et al., (2013) developed a 120 egg a capacity electric incubator. The system which was coupled with three 100 w electric bulb produced 75.2% fertility in addition to the hatchability rate of 64.8%. Soeb et al., (2021) developed a low-cost incubator that is made up of the embedded controller Incorporated to an egg turning trays with hatchery apartment of 116 egg capacity for Bangladesh market. The manual and automatically evaluations of the developed hatchery showed a 79.3% and 87.1% hatchability rate respectively. The eggs were turned both manually and automatically as the case may be at intervals of 6hours

Mansaray and Yansaneh (2015) developed a solar egg incubator that achieved temperature range of 36.8-37.9° c. The evaluation of the system yielded a percentage fertility and hatchability rate of 43.3% and 23.1% respectively. The study concluded that the low level of hatchability was as a result of several external influences such as an overcast weather, improper egg storage, time and energy  loss, turning of eggs, faulty eggs etc.

Divagar et al., (2022) evaluated and automated solar-driven hybrid egg incubator. The system achieved a hatchability percentage of 81.72%.

Osanyinpeju et al.,(2016) walked on the development of solar powered poultry egg incubator. The heat produced by metabolic activities of 150 eggs in the system was 21.9 w. The percentage fertility of 64% in addition to 44% hatchability rate was observed with the system. The low hatchability was attributed to faulty hygrometer used which led to increase in the number of opening made at the last stage.

Bukola (2008) modified and carry out performance and evaluation of hybrid poultry solar incubator and stated the environmental factors of major importance for artificial incubator include temperature, relative humidity, air supply and egg turning. overheating speed up the rate of development and called abnormal embryo in the early hatches and consfjf lower tratehr percentage hatchability like wise high rate to prevent normal evapouration and results in low hatachability. turing of egg sholud be done 3- times daily from 2nd to the 18th

okonkwo et al (2024) researched on energy sources for poultry egg incubator efficiency and hatachability. the result showed that sources of energy for incubation are categorized into grid supply electricity solar system, fossil fuel, biogas and thermochemical materials, from reviewed articles 50% identified grid supplied electricity as energy source for poultry egg incubation and hatachabiliy range from 80.9%-98.4% however, due to erratic nature of electricity supply and non availability in rural areas especially in developing nations, the study recommends increase research intereston other every source to improve their hatachability.

faki etal (2015) reported on incubation condition such as temperature, humidity, ventilation and turning the most important factor significantly affective the hatchability of poutry egg and chick quality.

Hassan et al., (2021) studied the use of thermal chemical storage media setup as a thermal source of hatching of poultry eggs. In the study, three forms of activated TCMs were utilized as energy containing medium. The media are silica gel, as well as natural zeolites. The TCMs were activated using electricity. Equally, the incubator was partly powered by electricity and partly by the TCMs. The its recapture operation helps to release the stored heat in TCMs. The system's hatchability percentage was 71.4% compared to the solely electric powered conventional incubator with 80.9% hatchability. However, the TCM incubator gave energy savings of 41.8% when compared with the conventional electric type.

Niranjan et al., (2021) developed a chicken egg incubator using (IOT) integrated with smartphone effective monitoring and control. The system temperature wear set at the different temperatures of into (H1) 36.5° c, (H2) 37.5° c and (H3) 38° c. The hatchability percentage of the three temperatures were(H1) 81%, (H2) 92% and (H3) 86% respectively.

Ogunwande et al., (2015) developed a biogas powered poultry egg incubator that utilizes biogas as a fuel to provide thermal energy via a heater situated at the base. The system produced temperature range of 36-39.4° c within the incubating compartment. At the end of an evaluation test, a hatchability percent of 59.7% was realized.

Muhammad et al,. (2022) Studies a thermoelectric egg incubator coupled to heat storage system. Did Samuel electric component is utilized in the daytime process while a heat storage system made up of a phase change material (PCM) which provides derma energy during offsunshine (especially nights) periods is complementary heat source. The electric energy that is changed to heat for starting the operation is provided with the arrays of the solar PV. The result of the 300 egg capacity system showed that temperature inside incubator chamber was between 36 iPhone 39° c with 73.3% hatchability.

Ogunwande et al., (2015) studied a still-air hatchery that has a non-thermostatic control. The system uses electric bulbs as the its source with a resistance varying components as temperature control. With three times daily of the eggs, the system achieved 62% hatchability rate.

Soeb et al., (2021) developed a low-cost incubator that is made up of the embedded controller Incorporated to an egg turning trays with hatchery apartment of 116 egg capacity for Bangladesh market. The manual and automatically evaluations of the developed hatchery showed a 79.3% and 87.1% hatchability rate respectively. The eggs were turned both manually and automatically as the case may be at intervals of 6hours.

Mansaray and Yansaneh (2015) developed a solar egg incubator that achieved temperature range of 36.8-37.9° c. The evaluation of the system yielded a percentage fertility and hatchability rate of 43.3% and 23.1% respectively. The study concluded that the low level of hatchability was as a result of several external influences such as an overcast weather, improper egg storage, time and energy loss, turning of eggs, faulty eggs etc.

Divagar et al., (2022) evaluated and automated solar-driven hybrid egg incubator. The system achieved a hatchability percentage of 81.72%.

CHAPTER THREE

- MATERIALS & METHODS

3.1 Area of Modification

The result from the test of previous machine showed that the machine was able to hatch 20 egg out of 45 eggs after operating the machine this gives 44% hatchabililty rate. The

recommendation suggested that the turning mechanism, the temperature and humidity control should be automated for better performance hence the turning mechanism, temperature and humidity control system will be automated and some technical issues like egg fertility will be addressed

3.2 Materials

The following materials were used in the construction of hybrid egg incubator;

i. MDF Plywood (Medium Density Fibre Body)

ii. Marine Board

iii. Bolt and Nut.

iv. Screw

v. Ardonic Cabinet Hinge

vi. Edges Tape

vii. Door Bolt

viii. Nails

ix. Screw Cover

x. Plank for the Egg Tray Seat

xi. Egg

xii. Hammer

xiii. Glass Fibre

3.3 Description of the Hybrid Egg Incubator

The hybrid eggs incubator is as shown in figure 1 and is made up of two components, the main cabinets and the auxiliary unit.

3.3.1  The Main Cabinet

The main cabinet is the incubating box which consists of the egg trays, fan, bulbs, egg turner, humidity tray, controller and savor motor.

- The incubating box is made from MDF plywood due to it, insulation properties, ease of construction, flexibility and durability. The dimension of the incubating box is 920 mm x 950 mm x 640mm.

- Egg turning trays: The Egg turning trays, is constructed from wood with dimension of 635mm x 335mm x 60 mm to accommodate the egg plastic crates. There are six egg creates, each containing 30 eggs, they were positioned inside incubator and turn 6 hourly.

- Humidity tray Is made up of plastic and placed at the bottom of the incubating box to increase and recover humidity in the incubator during experimental period.

- The fan is incorporated to facilitate the temperature and humidity in the incubator in order to keep them within the requirement of the present values.

- The controller was incorporated to sense the temperature, humidity and the turning mechanism of the incubator for effective incubation.

- The electric bulb 100 w cells as the source of heat to warm the inner unit to temperature range of 36 - 37.5° c. vii. The savor motor

viii. The savor motor is used to drive the egg turning tray and tilt it to an angle of 45° either side after a period of 6 hours.

3.3.2  The Auxiliary Unit

The auxiliary unit consists of solar PV panel, solar charge control, battery and inverter. The solar PV panel: Absorb solar energy to charge the battery, the solar charger controls the charging of the battery to prevent overcharging, the inverter converts the direct current from the battery to alternating current to be used by the incubator and at the same time charged the battery when there is electricity.

3.4 Design Consideration

The following factors were considered when designing hybrid eggs incubation

(a) Environmental Control

- Temperature range of 36.5°C-37-5°C within the incubator

- humidity range of 50-60% to prevent water loss and promote healthy development

iii. ventilation within the incubator is necessary for adequate airflow to maintain oxygen levels and remove carbon dioxide.

(b) Egg Handling

- regular turning (every 1-4 hours) to prevent embryo adhesion to the shell

- egg orientation to ensure eggs are positioned correctly (large end up) to facilitate proper development.

(c) Energy Deficiency and Sustainability.

i. incorporating solar energy source to reduce reliance on electrical energy source

ii. automatic egg turner to reduce labor and ensure consistent turning

iii. temperature and humidity monitoring to ensure optimal condition

3.5 Design Calculation and Analysis

The design criteria are based on making the equipment more affordable, efficient and durable. the incubator capacity, heating system, turning mechanism, size of battery, power from battery size of solar panel were all designed during the development process.

3.5.1 Determination of Incubation Chamber

The capacity of the incubator is directly proportional to the number of eggs and the number of egg trays. The incubator was design to hatch 180eggs per hatch. The design of incubation chamber was done using equation 3.1and 3.2

c =  (3.1)

Where,

C = number of trays

n = number of eggs to be hatched c = c = 3egg trays

the incubation chamber was designed to contain the egg trays; the incubation chamber was rectangular box like structure.

Volume of incubation chamber

V = l × b × h  (3.2)

Where:

V = volume of incubator (m)  (Abubakar et al 2023)

L = length of incubator (m)

B = breadth of incubator (m)

H = height of incubator (m)

3.5.2 Determination of Heat Required for Incubation

Heat required in the incubator was determined from equation 4

Q = MCΔT (Benjamin and Oye ,2012)  (3.3)

Where;

Q = total heat required

C = specific heat capacity

ΔT = change in temperature

M = molar mass

QT = (MaCaΔT) + (MeCeΔT) × Ne + (MpCpΔT) + (MwCwΔT) (3.4) (Abubakar et al 2023)

Where;

MaCaΔT = heat required by air

MeCeΔT = heat required by eggs

Ne = number of eggs

MpCpΔT = heat required by plywood MwCwΔT = heat required by water.

3.5.3  Heat Loss by Incubator

The heat loss due to conduction can be calculated from the equation 3.5 - 3.9

Q = −KA() (3.5)

The flow of air is steady inside the incubator therefore it does not vary with time.

QSx = −KAST (3.6)

Q = − KA) (3.7)

Where;

Q = heat

K = thermal conductivity coefficient of plywood (0.65) in the design

X = is the thickness of the plywood (15mm)

Heat loss during incubation is given as top and bottom of plywood.

Qptpbpspfpbt =−KA) × 2 (3.8)  (Bala et al, 2020)

Where;

Pt = is the plywood top

Pb = plywood backside

Ps = plywood side Pf = plywood front Pbt = plywood bottom.

A = area of incubator chamber

K = thermal conductivity coefficient (0.65 forMDF) plywood (Ajani et al 2020).

The quantity of heat loss is calculated using the following equation

QL = Qptm + Qpss + Qpfb ( 3.9)

The quantity of heat required for the incubation

Q = Ql + Q1 (3.10)

Where:

Q = quantity of heat required for incubation QL = heat loss by the incubator Q1 = heat required of the incubator.

3.5.5 Determination of Humidity

The humidity keeps the eggs from losing two much moistures in the process of incubation it is calculated from equation 3.11 – 3.13

H = (3.11) ρ =MvRvT  (3.12) H =  (3.13)

Where: H = humidity

ρ = energy density/heat density of water

T = temperature

Mv = molar mall of water

Rv = gas content

V = volume of the incubator’s chamber

3.5.6  Total Power Consumption

The total power consumption of the entire incubator system can be calculated from the following equation.

Total power consumption (PT) =P1+Pfh+Pfv+Pc (3.14) (Abba- ayi et al 2024) Where;

P1 = power of the incubator chamber

Pfh = power of humidity fan Pfv = power of ventilation Pc = power of control board.

3.4.7  Power Requirement and Selection of Battery Solar Panel and Inverter

Total power consumption is added up to 130.435W base on the total power requirement of the incubator. A 200AH lithium phosphate acid battery 12v, a 1KVA inverter and a 300W solar panel will be used to power the entire incubator.

3.5.8  Size of Solar Charger

Size of solar charger was determine using equation 3.15.

SSc =

Where;

SSc = size of solar charger (AH)

Wp = wattage of solar panel (W)

V = voltage of the battery (V)

Pf = power factor (1.25)

Size of solar charger =21AH but use 25AH

3.5.9  Battery Running Time

The battery running time was calculated using equation 3.16

BR = Pf  (3.16)

Where;

BR = battery running time (H)

Bc = capacity of battery (AH)

L = load of appliance (w)

Pf = power factor (10)

Battery running time is 16 hours but charging should start after 12 hours.

3.6 Material Selection

The following materials are put into consideration for the selection of materials used for hybrid eggs incubator

- Conservation of heat: MDF plywood was used for construction to conserve heat.

- Availability of material: The material used is easily available for repair and replacement.

- The cost of material: The cost of materials used is not expensive and will eventually leads to low cost of the incubator.

- The durability of the material: The material for construction was the type that will last long and have resistant to insect infestation.

- The easy of construction: The material used for construction is easy to work upon for mass production.

- The weight of material: The material used for construction was light weight for easy transportation.

- Lagging material: Fibre glass was used as a lagging material to further conserve and prevent loss.

3.7 Construction and Assembly Procedure of Hybrid eggs incubator

The tool used for the construction include; the hammer, hacksaw, paper tape, nail, bolt and nut. The medium density fibreboard (MDF)plywood of 50mm thickness was cut according to the design specification and joined together by nails and screw to form the incubating box. The three egg trays were in incorporated into the incubating box using bearing at  ---- interval between one another to serve as egg crates seat. The savor motor was incorporated to the egg tray to facilitate the turning of the egg trays. The control board that houses temperature, humidity and time of turning was also incorporated.

3.8 The operation of hybrid egg incubator

The hybrid egg incubator is operated using two sources of energy (electrical and solar energy) that complement one another to prevent power interruption and for efficient incubation. When electricity is available, the incubator is powered directly from the main but in case of power outage the incubator is powered by solar power system. The solar power system consists of solar panel, solar charge control, batteryandinverter.

The solar PV absorbs solar energy from the sun and charge the battery via the solar charge control, the DC from the battery is converted to AC by inverter for the use of the incubator thereby preventing power interruption that may be caused by power outage. The switching of power from electricity to solar or vice-versa is made possible through automatic incubator control system.

3.9 Bill of Engineering Measurement and Evaluation

Bill of engineering measurement and evaluation is presented in table

Table :- Bill Of Engineering And Evaluation

S/N	     Material			Rate		   Quantity	         Amount

- MDF Plywood		35,000		         1		         35,000

- 15mm                                45,000                      1                      45,000

- Marine board                     20,000                      3                      60,000

- 15mm                                800                            6                        4,800

- Egg tray 			50                               -                       50,000

- Egg create                         30                               1                      30,000

- Turning                             8,000                          2                      16,000

- Mechanism                        8,000                          6                       4,800

- Incubator box                     45,000                       1                       40,000

- Glass                               95,000                      1                          95,000

- DC fan                            7,500                         6                          45,000

- Bearing                            15,000                      -                           15,000

Electronic motor

(savour)

Controller

Eggs

Miscellonous

TOTAL

  - Performance Evaluation of Hybrid Eggs

3.10.1 Source of Material Source: The fertile eggs for evaluation were purchase from a poultry farm in Oke Ose, Ilorin East Local Government of Kwara State.

3.10.2 Preparation of Sample: The eggs were cleaned using sand paper, the eggs were of good size and shape. The incubator was fumigated for a day to prevent any contamination in readiness for incubation.

3.10.3 Output Parameter: The output parameters to be measured are; Percentage Hatchability:

i. Percentage hatchability is the number of hatched eggs to the number of fertile eggs loaded into the incubator in percentage.  The equation is stated in 3.17

Ph =  ……………. 3.17

Where;

Ph = Percentage hatchability (%)

Nh= Number of hatched eggs

Nfe= Number of fertile eggs loaded ii. Percentage Fertility:

Percentage fertility is the number of fertile eggs to the number of eggs loaded into the incubator in percentage.

The equation is stated in table 3.18  Pf =  ……………. 3.18

Where;

Pf = Percentage fertility (%)

Nfe = Number of fertile egg

Nf = Number of eggs loaded into the incubator iii. Mortality Rate

Mortality rate is the number of death to the number of fertile eggs loaded into the incubator

The equation is stated in 3. 19

MR =

Where;

MR = Mortailty rate

Nd = Number of death

Nfe = Number of eggs loaded

3.11 Experimental Procedure

The performance evaluation of hybrid eggs incubator was carried out at Kwara State Polytechnic Institute of Skills Acquisition, Mini Campus, Ilorin. The hybrid eggs incubator was fumigated and run empty for one day to ascertain the satisfactory performance of the incubator. One hundred fowl eggs were purchased from a poultry farm in Oke Ose, Ilorin East Local Government of Kwara State and loaded into the hybrid egg incubator.

The temperature, humidity and turning of egg tray were monitored. The temperature and humidity were taken three times a day and the mean value for each day was recorded while the turning of egg trays was on 6 hourly.

The candling of eggs was done on the 7th day of loading and the second candling was done on the 14th day of loading.

The turning of egg tray was stopped on the 18th day of loading. The unfertilized and bad eggs removed from the incubator was 34 eggs while the remaining 66 eggs were incubated for the period of 21 days. The hatching of eggs started on the 20th day of incubation. The number of hatched eggs were 58 while the remaining 8 were dead in the shell.

CHAPTER FOUR

4.0 RESULT AND DISCUSSION

4.1 Result

The result of performance evaluation of hybrid egg incubator is as shown in table 4.1

Table 4.1 average records of temperature and humidity for 21days of incubation

Table 3.1 Candling test on 7th day on incubation

S/N      No of eggs       Development                 Observation                       Remark

- 10               Not visible                       Clear                                Infertile

- 11               Not visible                      Larger air space                Infertile

- 40               Visible at one end            Liner running                  Fertile

- 39               Visible at the central        Across visible                   Fertile

Patches of red strain

Table 3.2 Candling test on the 14th day of incubation

S/N      No of eggs       Development                 Observation                       Remark

- 13                    Not visible                        Clear                        No development

- 61                    dark appearance              clear air space    development in progress

- 6               the whole egg is dark             no air space           progress development

- 5                     dark                                  beak is visible             ceased

7                  dark appearance, the           visible patch of         almost fertile

whole egg is dark                red stain                 development case

Table 4.3 average records of temperature and humidity for 21 days of incubation

S/N                      Temperature (°c)                       Humidity (%)

1			37.2                                     50.1

2                                   37.4                                    52.1

3                                   37.3                                     53.1

4                                     37.2                                     52.6

5                                     37.3                                     57.7

6                                     37.4                                     54.9

7                                     37.4                                     58.2

8                                     37.5                                     58.3

9                                     37.4                                      59.8

10                                   37.4                                      56.0

11                                   37.4                                      56.3

12                                   36.1                                      50.4

13                                   37.5                                      56.2

14                                   37.5                                      56.2

15                                   37.5                                      56.1

16                                   37.3                                      52.4

17                                   37.3                                      55.9

18                                   37.5                                      53.4

19                                   37.4                                      57.9

20                                   37.3                                      60.0

21                                   37.4                                      54.1

- Percentage hatachability =

- Percentage fertility =

- Mortality rate = 9%

4.2  Discussion

The result of incubation from 4.1 showed the temperature range of (36.1° – 37.5°) during the performance evaluation which corroborate the findings of Hard et al (2010), Gbabo et al (2014), Nithin et al (2014), Oniya et al (2010). This showed that the equipment was suitable for incubating fertile eggs. The relative humidity range (50.1 -60%) during performance evaluation was within the range obtained by Oniya et al(2013). The minimum temperature and humidity from the first day to 21st day were 36.1c° and 50% respectively while the maximum temperature was 37.5°c and maximum humidity was 60% for the period of incubation.

CHAPTER FIVE

5.0 CONCLUSION AND RECOMMENDATION

5.1  Conclusion

From the study carried out, the following are;

i. A hybrid egg incubator that combines electricity with solar energy to complement one another to prevent power interruption of the machine during incubation was modified and constructed.

iii. The temperature and humidity range within the incubator during performance evaluation were 36.1 – 37.5°c and 50.1 – 60% respectively.

iii. The percentage hatachability of the incubation is 91%. The incubator is suitable for the hatching of poultry eggs and is therefore recommended for poultry farmers.

5.2  Recommendation

The following reconmendations are madel;

i. Intensive performance evaluation should be carryout using other species of poultry birds like turkey, duck, guinea fowl

.ii. Sponsorship should be provided by relevant agencies in order to improve the machine and stop importation of incubators.

REFERENCES

Ajani,A.S, Olusoji, A.O, Egbeyele G.B and Adeniyi M.O (2020): Design and Construction of automated Eggs incubator for small scale poultry farmer. Internal Journal of technical Research and Science. Volume 5 issue 8 P 1-9.

Adebayo, A. A., Olaniyan, O. A., & Ajayi, O. O. (2021). Review on the efficiency of hybrid incubators in Nigeria's poultry industry

Abba – Ayi, M.A, Usman, A.R and Shuwa, M(20240: Design of a Smart Solar Hybrid Egg Incubator for of Engineering, technology and environment Vol 20(2) pg 375 – 384.

Abubakar,M.A, Daheer, A.A Adache,A.L Ferdinad, I.O (2023): Development and Testing of an Automated Egg Incubator. International Journal of Advanced in Engineering and Management (IJAEM) 5(6). 191 – 202.

African Journal of Agricultural Technology, 19(1), 77-8 Ajayi, S. O., Ogunjimi, L. O., & Akinwunmi, A. T. (2022). Optimization of solar-assisted hybrid incubators for poultry egg production in Nigeria. Journal of Renewable Energy Technologies, 8(1), 45-58.

Benjanim,N and Oye, N.D(2012): Modification of the Design of Poultry Incubator. International Journal of Application or Innovation in Engineering 1: 90 – 102.

Gbabo, A, Liberty, J.T,Gune, O.N and Owa, G.J Bala, A.M,Abubakar, M.A, Bello M.S and Sakako, T(2020): Design ,Construction of a Fully Automated Egg Incubator using Electric Battery. Egg Incubator using Electric battery. African Scholar journal of African Sustainance Development 18(2): 319 – 332.

Characterization of a photovoltaic powered poultry egg incubator", in 4th International Conference on Agriculture and Animal Science, Singapore, 2012, pp.1-6. Available: https:// doi.org/10.7763/IPCBEE

Design, Construction and Performance Evaluation of an Electric power Egg incubator. International Journal of Research in engineering and Technology 3 : 521 – 526.

Development of an automated solar-driven hybrid egg incubator", 2022. Available: https:// nastec.gov.lk

Divagar, V., Kannan, N., and Thiruvaran, T. Development of an automated solar-driven hybrid egg incubator", 2022. Available: https:// nastec.gov.lk

Gbabo, A, Liberty, J.T,Gune, O.N and Owa, G.J Bala, A.M,Abubakar, M.A, Bello M.S and Sakako, T(2020): Design ,Construction of a Fully Automated Egg Incubator using Electric Battery. Egg Incubator using Electric battery. African Scholar journal of African Sustainance Development 18(2): 319 – 332.

Harb,S.K, Habib, Y.A, Kasfem, A.M and poultry egg incubator to suit small farmer. Egypt Journal Agricultural Research 88 (1) 193 – 210.

Mansaray, K. G., and Yansaneh, O. "Fabrication and performance evaluation of a solar powered chicken egg incubator", Int. J. Emerg. Technol. Adv. Eng., vol. 5, issue 6, pp. 31-36, 2015.

Nithin,T.A,Shamea, L.M and Pradeep, K(2014): Design and Implementation of Solar PV Poultry

Incubator. International Journal of Engineering and Advance Technology (IJEAT) Vol 3 (3) pg 289 – 291

Oniya, O.O, Akande, F.B, Lawal, A.O and Adeyemo, A.S(2013): Development of a low – cost semi – automated Incubator. Journal of Agricultural Engineering and Technology Vol. 21 Issued P 27 -

Olaoye, I. O, Lawal, B. M., Ibrahim, S. O., and Sanusi, B. A. "An Electrically Operated Incubator for Household", Greener Journal of Science and Technological Research, vol. 3, issue 5, pp.160-165, 2013. Available: https://doi.org/10.15580/GJSETR.2013.5.1127 12300

Ogunwande, G. A., Ajayi, O. A., Ogunjimi, L. A. O. and Osunade, J. A. "Development of a stillair incubator with nonthermostatic control", Ife Journal of Technology, vol. 19, issue 2, pp. 59-62, 2010. A

Niranjan, L., Venkatesan, C., Suhas, A. R., Satheeskumaran, S., and Nawaz, S. A. "Design and implementation of chicken egg incubator for hatching using IoT", Int. J. Computational Science and Engineering, vol. 24, issue 4, pp. 363-372, 2021. Available: https ://doi.org/10.1504/ IJCSE.2021.117018

Osanyinpeju, K., Aderinlewo, A., Adetunji, O., and Ajisegiri, E. "Development of Solar powered poultry egg incubator", in College of Engineering International . Conference, Federal University of Agriculture, Abeokuta, 2016, p. 11.

Uzodinma, E. O., Ojike, O., Etoamaihe, U, J., and Okonkwo, W. I. "Performance study, of a solar poultry egg incubator with phase change heat storage subsystem", Case Studies Thermal Engineering, vol. 18, issuę 100593, 2020. Available: https://doi.org/10.1016/j.csite 2020.100593 Divagar, V., Kannan, N., and Thiruvaran, T.

Yadav, B. K., Pokhrel, N., Khatiwada, D., Khanal, M., Bajracharya, T., and Dhakal, R. "Design, Fabrication, and Performance Analysis of an Automatic Horizontal Egg, Incubator", Journal of the Institute of 1. Engineering, vot. 16, issue 1, pp. 78-85, 2021, Available: https://doi.org/10.3126/ jie.v1611.36 557