DESIGN AND FABRICATION OF A HYBRID DRYER FOR AGRICULTURAL

PRODUCE

BY:

FARUK AKEEM IYANDA

HND/23/ABE/FT/0068

DEPARTMENT OF AGRICULTURAL AND BIO – ENVIRONMENTAL

ENGINEERING, INSTITUTE OF TECHNOLOGY, KWARA STATE

POLYTECHNIC, ILORIN

INPARITAL FULFILLMENT OF REQUIREMENT FOR THE AWARD OF

HIGHER NATIONAL DIPLOMA (ND) IN AGRICULTURAL AND BIO-

ENVIRONMENTAL ENGINEERING TECHNOLOGY.

SUPERVISE BY:

ENG. OYEBANRE

Dated:

DEDICATION

This project is dedicated to Almighty Allah, the most Beneficent and Merciful

ACKNOWLEDGEMENT

All praise and gratitude belong to Almighty Allah, the Most Gracious, the Most Merciful, for granting me life, strength, wisdom, and perseverance throughout the period of this project and my academic journey. His mercy and guidance have been my anchor, and without His favor, this achievement would not have been possible.

I am deeply indebted to my wonderful mum, Mr.&Mrs. Faruk and my beloved brother & sister , Riyad, Bolux, Texas, Roll boy, Iba Aisha,Lil mum and others whose love, sacrifice, and encouragement have been the bedrock of my life. Your constant prayers, moral support, and financial backing gave me the strength to keep going even during the most difficult times. Words alone cannot express how thankful I am for everything you have done for me.

My sincere appreciation goes to my project supervisor, Engr. Oyebanre Oluafemi, whose guidance, feedback, and dedication helped shape this research work. Your patience, constructive criticism, and academic mentorship greatly enhanced the quality of this project.

I also extend my gratitude to the Head of Department Engr. Dr. Dauda K.A and all the lecturers in the Department of Agricultural and Bio-Environmental Engineering for the knowledge and inspiration they have imparted to me throughout my studies. Your collective impact has been monumental in shaping my academic foundation.

To my friends who have walked this journey with me — Femi, Jamal, Samuel, Grace, Rose, Ismail, sico  and others — thank you for your companionship, moral support, words of encouragement, and the memorable times we shared. Your presence made this academic journey worthwhile and bearable.

Special thanks to my coursemates and colleagues who directly or indirectly contributed to the success of this project. The discussions, shared resources, and teamwork during practicals and lectures were invaluable.

ABSTRACT

This research focused on the development and evaluation of a prototype solar automated cocoa seed dryer designed to enhance the drying process of Theobroma cacao seeds. The study aimed to fabricate a solar-powered drying system integrating key components such as a collector, fan, monitoring equipment, solar panel, drying chamber, frame, and trays, with structural elements welded using an electric arc welding machine for durability and integrity. Experimental trials were conducted varying sample mass (1000–3000 g) and air flow rate (0.4–0.6 kg/h) to assess the drying rate and drying efficiency of cocoa seeds under controlled solar drying conditions. Results demonstrated significant effects of both mass of sample and air flow rate on drying performance, with drying rates ranging from 0.024 to 0.042 kg/h and drying efficiencies between 78.3% and 96.3%. Statistical analysis using ANOVA highlighted the significance of the model (p < 0.0001) in influencing drying rate and efficiency, with no significant lack of fit, confirming the reliability of the experimental design. The optimized conditions suggested that lower mass and moderate air flow improved drying efficiency, thereby expediting the drying process while preserving seed quality. This prototype offers a sustainable alternative to conventional drying methods by harnessing solar energy, potentially reducing post-harvest losses and enhancing cocoa production efficiency in regions with abundant solar resources

TABLE OF CONTENT

Title Page										 i

Certification										 ii

Dedication 										iii

Acknowledgement 									iv

Table of Contents									 v

Abstract 										vii

List of Figure										viii

List of Plates										ix

List of Tables										x

CHAPTER ONE: INTRODUCTION

1.1 Background to the Study 								1

1.2 Problem statement of the Study 							2

1.3 Aim and Objectives of the Study

1.4 Justification of the Study 								3

1.5 Scope of the Study 								3

CHAPTER TWO

2.1 Properties of Cocoa Seeds							4

2.2 Continuous Drying Methods							4

2.3 Effect of Continuous Drying on Physical Properties				6

2.4 Factor Affecting Drying of Cocoa Seeds						6

CHAPTER THREE: Materials and Methods

3.1 Material 										16

3.1.1 Tools and Equipment

3.2 Design Considerations								17

3.2.1. Drying Temperature Range							18

3.2.2	Moisture Content Reduction

3.2.3	Energy Source and Sustainability						19

3.2.4	Airflow and Heat Distribution						19

3.2.5	Material Selection

3.2.6	Automation and Control system						19

3.2.7	Size and capacity								20

3.2.8	 Environmental and Economic Consideration				20

3.2.9	Safety Considerations								21

3.3	Design Calculations								21

3.3.1	 Volume of the Drying Chamber						21

3.3.2	 Volume of each tray 								22

3.3.3	 Volume of Cocoa Beans Per Tray 						22

3.4	Design Layout 								24

3.5	Working Principles 								24

3.5.1	Solar Energy Harvesting  							24

3.5.2	Air Heating and Circulation							25

3.5.3	Moisture Removal								25

3.5.4	Automation and Control							25

3.5.5	Tray Arrangement and Drying 						25

3.6	Construction and Fabrication Procedure					26

3.6.1	Sourcing of Materials								26

3.6.2	Fabrication of Drying Chamber						26

3.6.3	Fabrication of Drying Trays							26

3.6.4	Construction of Supporting Frame						26

3.6.5	Installation of Solar Energy Component					27

3.6.6	Assembly of Control System							28

3.6.7	Final Assembly and Testing 							28

3.7	List of Instrument 								28

CHAPTER FOUR: Results and Discussion

4.1 	Result 										29

4.2	 Discussion 									29

4.3	Analysis of Variance								36

CHAPTER FIVE: Conclusion and Recommendation

5.1 Conclusion 									42

5.2 Recommendation 									42

References 										44

List of Figures

Figure 4.1: Effect of Air Flow Rate and Mass of Sample on the Drying Rate of Cocoa		31

Figure 4.2: Effect of Air Flow Rate and Mass of Sample on the Drying Efficiency of Cocoa	35

LIST OF PLATES

Plate 3.1a: Pictorial view of Electric Arc Welding					24

Plate 3.1b: Pictorial view of Grinding Machine					24

Plate 3.1c: Pictorial view of Thong							24

Plate 3.1d: Pictorial view of Small Grinder 						24

LIST OF TABLES

Table 3.1: Experimental Design Matrix (Box-behnken Design) with the Factors			25

Table 4.1: Summary of Result of Cocoa Drying Using the Fabricated Hybrid			28

Table 4.2: Analysis of Variance (ANOVA) for the Drying Rate of Cocoa Bean			28

Table 4.3: Analysis of Variance (ANOVA) for the Drying Efficiency of Cocoa			33

Table 4.4: Bill of Engineering Measurement

CHAPTER ONE

1.0	INTRODUCTION

1.1 	Background to the study

In the primary processing of cocoa beans, drying is a critical step that follows fermentation. After fermentation, beans are dried to reduce the moisture content from about 60% to 6-8% (wb) for safe storage and transportation, as well as to reduce the astringency and bitterness of the beans. Sun-drying of fermented cocoa beans is the most widely practiced method of drying. Beans are spread on raised mats or on concrete floors during periods of sunshine and allowed to dry (Lasisi, 2014). In the West Indies, drying takes place on wooden or concrete drying floors with moveable roofs (called cocoa houses) and beans are mixed regularly by walking through the layers or raked by using a wooden palette. Depending on weather conditions, beans are sun-dried anywhere from 7 to 22 days. Slow drying in thick beds can sometimes result in over-fermentation of the beans and in mold growth while too rapid drying can result in in acidic beans with shriveling and/or case hardening of the shells (Jinap and Thien, 1994; Bonaparte et al., 1997, Sukha, 2003; Lasisi, 2014; Fagunwa et al., 2009; Zahouli et al., 2010).

Several researchers have reported that while drying is enhanced at higher temperatures, artificially dried beans, especially beans dried at higher temperatures of 55-60°C, are more acidic than sun-dried beans. Jinap and Thien (1994) found that sun-dried beans produced the lowest concentration of volatile fatty acids and were higher in pH (5.12) compared with the oven-dried (60°C) beans which had the lowest pH of 5.02 and highest titratable acidity. Irie et al. (2010) also showed that sun-dried samples were less acidic with a pH of 4.0 than oven drying at 60°C which produced a pH of 3.7. Oke and Omotayo (2011) showed that intermittent drying in an oven at 55°C produced highly acidic beans with a pH of 4.70 while the pH of sun-dried samples averaged 5.03.

According to Afoakwa (2014), the physical properties of cocoa seeds are crucial in determining their quality and suitability for processing [Afoakwa, E. O. 2014].

Studies have shown that continuous drying can lead to changes in the physical properties of cocoa seeds. For instance, Hii et al. (2011) found that drying can cause shrinkage and changes in the size and shape of cocoa seeds [Hii and et al., 2011].

Additionally, the color and texture of cocoa seeds can be affected by the drying process, with high temperatures potentially leading to undesirable changes [Fagunwa, A. O., et al. 2020].

1.2 	Problem statement of the study

Cocoa seed drying, a critical step in cocoa processing, is often hindered by unreliable and inefficient traditional drying methods, resulting in poor quality cocoa beans, reduced yields, and economic losses for small-scale cocoa farmers, thereby necessitating the development of a sustainable and energy-efficient drying solution."

1.3 	Aim and Objectives of the Study

The main aim of this project were to dry cocoa bean to storage moisture content

While the specific objectives are:

- To design and fabricate a hybrid solar dryer

- To evaluate the performance of the dryer

- To model the dryer.

1.4	Justification of the Study

Using cocoa beans as a case study allows for evaluating the 	system’s practical relevance and 	potential for broader application across other crops

Drying remains a crucial post-harvest operation in the processing of cocoa beans, significantly 	influencing their quality, shelf life, and market value. Traditional open-sun drying methods are 	often hindered by unpredictable weather, contamination, and uneven drying. These limitations 	necessitate the development of improved drying systems that are both efficient and sustainable. 	This study is justified by the need to harness renewable energy more effectively in agricultural 	processing, particularly in rural and off-grid areas. By integrating both passive solar heating and a 	solar-powered airflow mechanism, the proposed hybrid drying system offers a more reliable and 		controlled drying environment.

1.5       Scope of the Study

This study focuses on the design, fabrication, and performance evaluation of a hybrid solar 	dryer intended for agricultural produce. The system combines direct solar heating with a 	solar-powered blower to enhance drying efficiency. The research is limited to the drying 		of fermented cocoa beans, which serves as a representative case for other similar crops 	requiring controlled drying. Parameters such as drying rate and drying efficiency will be 	assessed. The study does not extend to economic analysis or large-scale industrial 	application but serves as a prototype-level investigation using locally sourced materials 	and solar technology

2.0					CHAPTER TWO

LITERATURE REVIEW

2.1 	 Properties of Cocoa Seeds

The physical properties of cocoa seeds play a crucial role in determining their quality and suitability for processing. These properties include moisture content, size, shape, color, and texture [Okiy. et al 2017.].

2.1.1 	Moisture Content: The moisture content of cocoa seeds is a critical factor in determining their quality. Excessive moisture can lead to mold growth and spoilage, while low moisture content can affect the texture and flavor of the seeds [Afoakwa, E. O., et al. (2013].

2.1.2	Size and Shape: The size and shape of cocoa seeds can affect their processing efficiency and product quality. Uniformly sized seeds are easier to process and can result in higher-quality products [Bart-Plange, A., et al. 2012].

2.1.3	Color: The color of cocoa seeds can be an indicator of their quality. Well-dried seeds typically have a rich brown color, while poorly dried seeds may appear pale or discolored [Jinap, S., et al. 2003].

2.1.4	Texture: The texture of cocoa seeds can affect their handling and processing. Seeds that are too dry or too wet can be difficult to process and may result in lower-quality products [Fagunwa, A. O., et al. 2020].

2.2 	Continuous Drying Methods

Continuous drying is a common method used to reduce the moisture content of cocoa seeds. There are several continuous drying methods, including:

2.2.1	Solar Drying: This method uses solar radiation to dry cocoa seeds. Solar drying is a cost-effective and environmentally friendly method, but it can be affected by weather conditions [Othman, M. F., et al. 2020].

2.2.2	Mechanical Drying: This method uses hot air to dry cocoa seeds. Mechanical drying is a faster and more controlled method than solar drying, but it can be more expensive [Bart-Plange, A., et al. (2019].

2.2.3	Hybrid Drying: This method combines solar and mechanical drying. Hybrid drying can offer the benefits of both methods, including cost-effectiveness and control [Hii, C. L., et al. 2018].

2.2.4	Advantages and Disadvantages of Continuous Drying Methods

Each continuous drying method has its advantages and disadvantages. For example:

Solar Drying: Advantages include low cost and environmental sustainability. Disadvantages include weather dependence and potential for contamination [Othman, M. F., et al. 2020].

Mechanical Drying: Advantages include fast drying times and control over time [Bart-Plange, A., et al. (2019].

Hybrid Drying: Advantages include potential for improved quality and control. Disadvantages include higher cost and complexity [Hii, C. L., et al. 2018].

2.3	 Effects of Continuous Drying on Physical Properties

Continuous drying can have significant effects on the physical properties of cocoa seeds. Some of the effects include:

2.3.1	Moisture Content Reduction: Continuous drying can reduce the moisture content of cocoa seeds, which can help to prevent mold growth and spoilage [Afoakwa, E. O. 2014].

2.3.2	Shrinkage: Continuous drying can cause shrinkage of cocoa seeds, which can affect their size and shape [Bart-Plange, A., et al. 2012].

2.3.3	Texture Changes: Continuous drying can affect the texture of cocoa seeds, making them harder or softer depending on the drying conditions [Jinap, S., et al. 2003].

2.3.4	Color Changes: Continuous drying can cause changes in the color of cocoa seeds, which can affect their quality and appearance [Okiy, D. A., et al. (2017].

2.3.5	Importance of Controlling Drying Conditions

Controlling the drying conditions is crucial to prevent undesirable changes in the physical properties of cocoa seeds. Factors such as temperature, humidity, and airflow can affect the drying process and the quality of the seeds [Fagunwa, A. O., et al. 2020].

2.4 	Factors Affecting Drying of Cocoa Seeds

The drying of cocoa seeds is affected by several factors, including:

2.4.1	Temperature: Temperature plays a crucial role in determining the drying rate and quality of cocoa seeds. Higher temperatures can lead to faster drying, but can also cause damage to the seeds [Kongor, J. E., et al. 2016].

2.4.2	Humidity: Humidity affects the drying rate and can impact the quality of the seeds. Low humidity can lead to faster drying, while high humidity can slow down the drying process [Hii, C. L., et al. 2011].

2.4.3	Airflow: Airflow is essential for efficient drying. Adequate airflow can help to remove moisture from the seeds and prevent mold growth [Bart-Plange, A., et al. 2012].

2.4.4	Drying Time: The duration of drying can impact the quality of the seeds. Over-drying can lead to loss of flavor and texture, while under-drying can lead to mold growth and spoilage [Jinap, S., et al. 2003].

2.4.5	Importance of Controlling Drying Factors

Controlling the factors that affect drying is crucial to ensure the quality of cocoa seeds. By optimizing temperature, humidity, airflow, and drying time, cocoa producers can produce high-quality seeds that meet the requirements of chocolate manufacturers [Afoakwa, E. O. 2014].

2.5 	Effects of Continuous Drying on Physical Properties of Cocoa Seeds

Continuous drying significantly impacts the physical properties of cocoa seeds, including moisture content, size, shape, color, and texture.

2.5.1	Moisture Content Reduction: Continuous drying reduces the moisture content of cocoa seeds from 60% to 7% or less, preventing mold growth and spoilage. This process involves simultaneous heat and mass transfer phenomena, influencing the sensory, chemical, and microbiological quality of the beans. [Hii. et al, 2018]

2.5.2	Shrinkage and Texture Changes: Drying causes shrinkage and changes in the texture of cocoa seeds. The rate and extent of drying can lead to crusting, affecting the testa and potentially resulting in acidic or astringent flavors. [Suzannah. S., et al, 2018]

2.5.3	Color Changes: The color of cocoa seeds can change during drying, with well-dried seeds typically exhibiting a rich brown color. Poor drying practices may lead to discoloration.

2.5.4	Size and Shape Changes: Continuous drying can cause changes in the size and shape of cocoa seeds due to shrinkage.

2.5.5	Drying Methods and Equipment

Different drying methods and equipment can be used, each with its advantages and disadvantages:

Solar Drying: This method uses solar radiation and natural convection to dry cocoa seeds. It's cost-effective but weather-dependent.

Drying Under Controlled Conditions (DUCC): DUCC involves using hot air to dry cocoa seeds, allowing for control over temperature, humidity, and airflow. This method can reduce drying time but may require more energy.

2.5.6	Importance of Controlling Drying Conditions

Controlling drying conditions is crucial to prevent undesirable changes in the physical properties of cocoa seeds. Factors such as temperature, humidity, and airflow can significantly impact the quality of the seeds [Escobar, S.,et al, (2025]

CHAPTER THREE

MATERIALS AND METHODS

3.1 Materials

The following materials were used for the construction of the hybrid dryer:

- Cocoa (Theobroma cacao)

Approximately 20kg of wet cocoa beans were purchased locally for testing at Ondo State, Nigeria. Freshly fermented cocoa (Theobroma cacao) seeds serve as the raw material to be dried in the prototype. The cocoa seeds initially had a moisture content of about 55% and required drying to a safe storage moisture level of 6–8%.

- Drying Chamber

The drying chamber serves as the primary enclosure where the cocoa (Theobroma cacao)  seeds are dried. This chamber is designed to maintain optimal temperature and humidity conditions for the drying process. The internal design ensures even distribution of heat throughout the space, thereby enhancing the efficiency of the drying process. The chamber is constructed to accommodate different volumes of cocoa seed.

- Stainless Trays (2 Layers)

Stainless trays, arranged in two layers, are used to hold the cocoa seeds during the drying process. Stainless tray was chosen for its durability, resistance to corrosion, and ease of cleaning. The two-layer tray design allows for multiple batches of cocoa seeds to be dried simultaneously, thereby increasing the overall capacity of the dryer.

- Blower

The blower is employed to circulate air within the drying chamber. Proper air circulation ensures that heat is evenly distributed around the cocoa seeds, preventing uneven drying and promoting efficient moisture removal. The blower also assists in regulating temperature and humidity within the chamber, contributing to the control of drying conditions.

- Chimney

The chimney is used to expel hot air and moisture from the drying chamber. This is essential for maintaining optimal drying conditions and preventing excess humidity from compromising the drying process. The chimney ensures that the drying chamber remains well-ventilated, facilitating the expulsion of moisture and heat.

- Support Stand (Frame)

The support stand, or frame, provides the structural integrity of the entire drying system. The frame holds the solar collector, drying chamber, blower, and other components in place, ensuring the system remains stable during operation. It is designed to withstand the mechanical stresses of daily use and provide a robust foundation for the dryer.

- Thermohygrometer Sensor for Automation

The thermohygrometer sensor is an integral component of the automated system, used to monitor the temperature and humidity levels within the drying chamber. The data collected by the sensor allows for real-time adjustments to the drying conditions, ensuring that the cocoa seeds are dried under optimal temperature and humidity. This automation helps maintain consistent and controlled drying conditions, improving the overall efficiency of the drying process.

- Variable Speed Control

The variable speed control system is used to adjust the speed of the blower. By regulating the airflow within the drying chamber, the speed control ensures that the drying conditions remain consistent. The ability to fine-tune the blower speed allows for adjustments based on the moisture levels of the cocoa seeds, optimizing the drying process for different batches.

- Mild steel sheets

For fabricating the drying chamber due to their good thermal conductivity and structural strength

- Solar Collector (Glass)

The solar collector, constructed using glass, is responsible for capturing solar energy to heat the drying chamber. The glass material allows maximum transmission of solar radiation, ensuring the collector absorbs sufficient heat from the sun. The energy captured by the solar collector is transferred to the drying chamber to facilitate the drying of cocoa seeds.

- Battery

used for storing solar energy harvested during the day, ensuring uninterrupted operation of the dryer during low sunlight or cloudy weather conditions. A battery capacity labelled 12V/18Ah/20HR was used. Meaning:

Voltage : 12 volts, Capacity(Ah) 18 Ampere hours

Rated over 20HR: it can deliver its rated capacity(18Ah) over 20 hours of discharge

- Angle irons

For constructing the supporting frame to provide strength and stability.

- Black matte paint

Applied to interior walls to improve solar heat absorption

- Fiber Glass

This was used to lag the sides of the drying chamber to prevent heat loss.

3.1.1 Tools and Equipment

1. Electric Arc Welding Machine: it was employed during the fabrication of the drying chamber, supporting frame and tray assembly. It was used to join metal components with high strength and durability ensuring structural integrity of the dryer.

- Grinder/Cutting Machine: It is a power tool with a rotating abrasive disc or blade which was used for cutting and smoothing metal.It is used to cut metal sheets, pipes, or rods to required sizes and for grinding welds to smooth finishes.

- Drilling Machine: It is a machine tool which was used for used to drill holes into materials (metal, wood, etc.). It is used to create holes for bolts, screws, or other fittings in your project components.

- Screwdriver Set: It is a set of hand tools with different tips (flat, Phillips, etc.) which was used  for driving screws. It is used for tightening or loosening screws during assembly or adjustments of electrical and mechanical parts.

- Spanner Set: It is a set of tools which was used for for tightening or loosening nuts and bolts. It is essential for assembling and disassembling mechanical parts such as frames, joints, or supports.

- Multimeter (for testing connections): it is an electronic measuring instrument that combines several functions (voltage, current, resistance testing). It was used to check electrical circuits, test battery voltage, or ensure proper connections in the solar-powered system

- Pliers: It is a hand tool with gripping jaws, sometimes with cutting edges.it was used for holding objects firmly, bending wires, or cutting small materials.

- Measuring Tape: It is a flexible ruler used to measure distances or dimensions. It is used to take accurate measurements of components during fabrication or assembly.

- File (for finishing edges):It is a hand tool with a roughened surface used for smoothing or shaping metal.It is used to smoothen sharp edges after cutting or welding metal parts.

- Paintbrush/Spray Gun: it is a tool used to apply paint or protective coatings. It is used for finishing touches to protect metal surfaces from rust and improve aesthetics.

- Soldering Iron (for electronic parts): It is a hand tool that heats up to melt solder (a metal alloy) for joining electronic components. It is used in assembling or repairing the electronic parts of your project like sensors, circuits, or connections.

3.2 Design Considerations

The design and development of the prototype automated solar powered cocoa seed dryer were guided by several critical factors to ensure effective drying, energy efficiency, sustainability, and adaptability to local conditions. These considerations are outlined below:

3.2.1 Drying Temperature Range

Cocoa beans require a careful drying process to preserve their flavor quality and prevent case hardening. The target temperature range for effective drying was set between 40°C and 60°C. Temperatures above 60°C can degrade flavor precursors and cause the beans to develop a smoky or burnt taste, while temperatures below 40°C prolong drying time and may encourage mold growth. The system was therefore designed to achieve and maintain this range using solar energy, with the help of a thermal storage mechanism for cloudy conditions.

3.2.2 Moisture Content Reduction

Freshly fermented cocoa beans typically have a moisture content of 50–60% (wet basis). For proper storage and to prevent microbial growth, this moisture content needs to be reduced to 6–8% (wet basis) using  the moisture analyzer. The dryer was designed to achieve this reduction within 48 to 72 hours, depending on the intensity of solar radiation and ambient conditions, thereby improving efficiency compared to traditional sun drying which may take up to 7 days.

3.2.3 Energy Source and Sustainability

The decision to use solar energy was based on its renewable nature and abundant availability in cocoa-producing regions of Nigeria, which typically receive 4–7 kWh/m²/day of solar insolation. Photovoltaic (PV) panels were selected to power electrical components such as fans, temperature and humidity sensors, and the microcontroller-based automation system. Additionally, the system was designed with provision for battery storage to enable drying continuity during low sunlight or at night.

3.2.4 Airflow and Heat Distribution

Uniform airflow is critical for effective drying of cocoa beans to prevent uneven drying or spoilage. A forced convection system, consisting of a DC-powered fan, was incorporated to distribute heated air evenly within the drying chamber. Vents were strategically placed to allow the escape of moist air, preventing condensation which could otherwise compromise drying efficiency. The airflow system was designed to achieve an air velocity of 0.5–1.0 m/s, which is optimal for drying cocoa beans without dislodging them.

3.2.5 Material Selection

Material selection was driven by the need for thermal efficiency, durability, cost-effectiveness, and food safety. The drying chamber was constructed using mild steel sheets coated with black matte paint to enhance solar absorption. The insulation layer consisted of glass Fiber to minimize heat loss. The trays for holding cocoa beans were fabricated from stainless net mesh (food grade) to resist corrosion and allow free airflow around the beans.

The external frame was built with galvanized steel to ensure structural stability and resistance to environmental degradation.

3.2.6 Automation and Control System

An automation system was integrated to improve operational efficiency and precision. It consists of:

Temperature and humidity sensors for real-time monitoring.

A microcontroller programmed to regulate fan speed and control heating elements based on set parameters.

3.2.7 Size and Capacity

The dryer was designed for a small-scale capacity suitable for research and demonstration purposes. It has a drying chamber dimension of 0.855 m × 0.7 m × 0.32 m, accommodating approximately 5–10 kg of wet cocoa beans per batch. This size was chosen to allow easy fabrication and testing while making the system scalable for commercial applications in future designs.

3.2.8 Environmental and Economic Considerations

The design accounted for environmental friendliness by utilizing renewable energy and minimizing greenhouse gas emissions. Economically, the system was optimized to be affordable for smallholder cocoa farmers by selecting locally available materials and components wherever possible.

3.2.9 Safety Considerations

Safety features such as insulated wiring, a protective casing for the solar panel and battery, and heat-resistant materials were incorporated to prevent electrical hazards and thermal burns during operation.

3.3 Design Calculations

The design of the prototype automated solar powered cocoa seed dryer requires determining key parameters to ensure optimal functionality.

The following calculations were carried out at Bolof Agro Technology along Tanke area before fabrication was carried out:

3.3.1 Volume of the Drying Chamber

The total internal volume of the drying chamber is determined using the formula:

Vchamber = L x B x H

Where:

L = 0.855m (length of chamber)

B= 0.7m (width of chamber)

H= 0.32m (height of chamber)

Therfore: Vchamber  = 0.855x0.7x0.32

Vchamber = 0.192m3

3.3.2 Volume of each tray

The drying chamber contains three trays arranged horizontally with some clearance between them for adequate airflow.

Each tray occupies a length and width similar to the chamber internal dimensions, while the depth of each tray is 0.05m for a shallow layer of cocoa beans to facililitate efficient drying.

The volume of one tray is calculated as:

Vtray = Ltray × Btray × Htray
Ltray = 0.855m

Btray = 0.7 m,

Htray = 0.05 m
Vtray = 0.855 × 0.7 × 0.05 = 0.0299 m³ = 0.03 m³

3.3.3 Volume of Cocoa Beans Per Tray

the volume occupied by cocoa beans on each tray depends on their bulk density and the quantity placed  on each tray.

The bulk density of wet cocoa beans is approximately 600kg/m³

If each tray holds 3kg of wet cocoa beans, the volume occupied by the beans is:
Vbeans = 3 / 600 = 0.005 m³

So, each tray’s cocoa beans occupy is approximately 0.005m³

3.4 Design Layout

The experimental design for this study was structured using a two factor factorial design to evaluate the effect of the drying parameters on the drying rate and drying efficiency of cocoa seeds. The factors and responses are presented as follows:

- Mass of Sample (g)

- Air flow rate (m3/s)

- Experimental Responses (Dependent Variables)

Two performance indicators were monitored during the drying process:

- Drying Rate (kg/h): measured as the rate at which moisture was removed from the cocoa seeds.

- Drying efficiency (%): calculated as the ratio of useful energy utilized for moisture removal to the total energy supplied.

- Experimental Runs

A total of 13 experimental runs were carried out as presented in Table 3.1. The runs were randomized to minimize the experimental bias and ensure the independence of observations. The experimental matrix includes various combinations of the two factors and their respective levels.

Table 3.1: Experimental Design Matrix (Box-behnken Design) with the Factors and Responses For  The Cocoa Seed Drying Process

|  |  | Factor 1 | Factor 2 | Response 1 | Response 2 |
| --- | --- | --- | --- | --- | --- |
| Std | Run | A:Mass of Sample | B:Air Flow Rate | Drying Rate | Drying Efficiency |
|  |  | g |  | Kg/h | % |
| 3 | 1 | 1000 | 0.6 |  |  |
| 9 | 2 | 2000 | 0.5 |  |  |
| 12 | 3 | 2000 | 0.5 |  |  |
| 7 | 4 | 2000 | 0.4 |  |  |
| 4 | 5 | 3000 | 0.6 |  |  |
| 2 | 6 | 3000 | 0.4 |  |  |
| 13 | 7 | 2000 | 0.5 |  |  |
| 1 | 8 | 1000 | 0.4 |  |  |
| 11 | 9 | 2000 | 0.5 |  |  |
| 5 | 10 | 1000 | 0.5 |  |  |
| 10 | 11 | 2000 | 0.5 |  |  |
| 6 | 12 | 3000 | 0.5 |  |  |
| 8 | 13 | 2000 | 0.6 |  |  |

3.5 Working Principle

The automated solar powered cocoa seed dryer operates on the principle of utilizing solar energy to generate and maintain sufficient heat within a drying chamber, while forced convection ensures uniform airflow around the cocoa seeds to achieve effective and consistent drying.

The system is designed to harness solar energy through photovoltaic (PV) panels, which supply electrical power to auxiliary components such as fans, sensors, and the control system. The heated air, generated and circulated within the chamber, facilitates the removal of moisture from the cocoa seeds until the desired final moisture content is achieved.

The detailed working process is as follows:

3.5.1 Solar Energy Harvesting

The photovoltaic (PV) panels capture solar radiation and convert it into electrical energy. This energy powers the entire system, including the fans for air circulation and the microcontroller-based automation system. Excess energy can be stored in a battery bank to ensure continuous operation during periods of low sunlight or at night.

3.5.2 Air Heating and Circulation

Ambient air enters the system through inlets and is heated by solar energy absorbed within the chamber walls. A DC-powered fan is activated by the control system to force heated air into the drying chamber.

This forced convection system ensures uniform distribution of heat throughout the chamber, preventing uneven drying of cocoa seeds and eliminating cold spots.

3.5.3 Moisture Removal

As the heated air comes into contact with the wet cocoa seeds spread on trays, it absorbs moisture from the seeds. The moisture-laden air is expelled through strategically placed vents, allowing fresh heated air to enter and continue the drying process. This cycle continues until the moisture content of the cocoa seeds is reduced from approximately 55% to the desired safe storage level of 6–8%.

3.5.4 Automation and Control

The system employs sensors to monitor key parameters such as temperature and humidity inside the drying chamber. These sensors relay data to a microcontroller, which automatically regulates fan speed and heating operation to maintain optimal drying conditions within the chamber.

3.5.5 Tray Arrangement and Drying Efficiency

The cocoa seeds are spread evenly in thin layers on two netted trays arranged vertically in the chamber. The forced airflow passes through the perforations in the trays, allowing heated air to reach all seeds uniformly. The shallow depth of cocoa beans per tray helps to prevent clumping and ensures rapid moisture removal.

3.6 Construction and Fabrication Procedures

The construction and fabrication of the prototype automated solar powered cocoa seed dryer involved systematic steps to ensure the development of a functional and durable system. The process began with sourcing appropriate materials, followed by cutting, shaping, assembling, and integrating the control system. The procedures are outlined as follows:

3.6.1 Sourcing of Materials

All materials required for the fabrication of the dryer were carefully selected based on availability, durability, thermal efficiency, and food safety considerations. Key materials include mild steel sheets, stainless steel mesh, angle irons, fiber glass, photovoltaic (PV) panels, DC fan, sensors, microcontroller,battery and cocoa. These materials were procured from local suppliers to minimize cost and ensure easy replication.

3.6.2 Fabrication of Drying Chamber

The drying chamber was constructed using mild steel sheets due to their good thermal conductivity and ease of fabrication. The steps involved:

- Cutting mild steel sheets into panels corresponding to the chamber dimensions: 0.855 m (length) × 0.7 m (width) × 0.32 m (height).

- Welding the panels together using Electric Arc welding to form a rectangular box, leaving openings for air inlets and outlets.

- Coating the interior surfaces with black matte paint to enhance solar heat absorption.

- Adding an inner lining of fiber glass insulation between double walls to minimize heat loss.

- Fixing a transparent glass top cover in triangular form to allow solar radiation penetration.

3.5.3 Fabrication of Drying Trays

- Two drying trays were fabricated using food-grade netting mesh to prevent rusting and allow free airflow around the cocoa seeds.

- The trays were cut to dimensions slightly smaller than the internal chamber size to fit snugly while leaving space for airflow.

- The net mesh was stretched and welded onto a rectangular mild steel frame.

3.6.4 Construction of Supporting Frame

A sturdy supporting frame was constructed using angle irons to elevate the drying chamber and provide stability. The frame is 910mm in height. Angle irons were cut and welded into a rectangular frame with four vertical supports, cross-bracings were added for enhanced stability. The entire frame was coated with anti-rust paint for protection against environmental factors.

3.6.5 Installation of Solar Energy Components

The renewable energy system was installed as follows:

Mounting photovoltaic (PV) panels at an optimal tilt angle to maximize solar radiation capture. Connecting the PV panels to a charge controller, battery bank, and inverter. Wiring the system to supply power to the DC fan, sensors, microcontroller.

3.6.6 Assembly of Control System

The automation system was assembled and integrated into the dryer. This included:

- Installing temperature and humidity sensors inside the drying chamber to monitor environmental conditions.

- Connecting the sensors to the microcontroller, programmed to regulate the fan based on pre-set drying conditions.

3.6.7 Final Assembly and Testing

After all components were fabricated and assembled: The drying chamber, trays, and supporting frame were fully assembled.

Solar panels and control systems were connected to ensure seamless power supply.

The system was tested with an initial batch of cocoa seeds to verify functionality, airflow uniformity, temperature regulation, and drying performance.

Adjustments were made where necessary to optimize operation.

3.7 List of Instrument

Plate 3.1a:Electric Arc Welding					plate 3.1b: Grinding

Plate 3.1c: Thong					    	plate 3.1d Electrodes

Small Grinder								Hammer

Chapter Four

### Results and Discussion

### 4.1 Result

The summary of result obtained from the test of the fabricated hybrid dryer for dying cocoa bean is presented in Table 4.1.

### Table 4.1: Summary of Result of Cocoa Drying Using the Fabricated Hybrid 	Dryer

| Run | Mass of Sample (g) | Air Flow Rate (kg/h) | Drying Rate (kg/h) | Drying Efficiency (%) |
| --- | --- | --- | --- | --- |
| 1 | 1000 | 0.6 | 0.042 | 78.3 |
| 2 | 2000 | 0.5 | 0.033 | 87.6 |
| 3 | 2000 | 0.5 | 0.033 | 87.6 |
| 4 | 2000 | 0.4 | 0.029 | 91.2 |
| 5 | 3000 | 0.6 | 0.031 | 89.2 |
| 6 | 3000 | 0.4 | 0.024 | 96.3 |
| 7 | 2000 | 0.5 | 0.034 | 86.1 |
| 8 | 1000 | 0.4 | 0.035 | 85.4 |
| 9 | 2000 | 0.5 | 0.031 | 89.2 |
| 10 | 1000 | 0.5 | 0.037 | 83.9 |
| 11 | 2000 | 0.5 | 0.033 | 87.6 |
| 12 | 3000 | 0.5 | 0.027 | 92.8 |
| 13 | 2000 | 0.6 | 0.036 | 84.1 |

### 4.2 Discussion

From table 4.1 above, the drying performance of the fabricated hybrid dryer was evaluated using varying sample masses and air flow rates, with drying rate and drying efficiency as the key response variables. It was observed that increasing the sample mass did not consistently lead to higher drying rates. For instance, at an air flow rate of 0.6 kg/h, the drying rate decreased from 0.042 kg/h for a 1000 g sample to 0.031 kg/h for a 3000 g sample, indicating that beyond a certain load, the dryer’s performance may decline due to limited heat or air penetration. Similarly, at constant mass, increasing air flow generally led to a slight increase in drying rate, which is consistent with improved heat transfer and moisture removal. For example, at 2000 g sample mass, drying rate increased from 0.029 kg/h at 0.4 kg/h air flow to 0.036 kg/h at 0.6 kg/h air flow.

However, the trend in drying efficiency revealed an inverse relationship with drying rate in some cases. The highest efficiency of 96.3% was recorded at a sample mass of 3000 g and air flow of 0.4 kg/h, despite the drying rate being among the lowest (0.024 kg/h). On the other hand, the highest drying rate (0.042 kg/h) was associated with the lowest efficiency (78.3%), occurring at 1000 g sample mass and 0.6 kg/h air flow. This suggests that while higher airflow and smaller loads may accelerate drying, they may also lead to energy losses, possibly due to over-drying or inefficient moisture removal dynamics. Efficient drying appears to be favored under moderate to low airflow and higher sample loading, where energy is more effectively utilized.

The presence of repeated values, particularly at 2000 g sample mass and 0.5 kg/h air flow rate (Runs 2, 3, 7, 9, and 11), provided consistency across trials, confirming the repeatability and reliability of the dryer’s performance. Overall, the fabricated hybrid dryer demonstrated the capability to dry cocoa effectively, but with a clear trade-off between drying speed and energy efficiency.

The performance of the fabricated hybrid dryer in this study aligns with several trends reported in the literature. The observed drying rates (ranging from 0.024 to 0.042 kg/h) are comparable to those documented by Aregbesola et al. (2015), who reported drying rates between 0.020 and 0.045 kg/h when using a hybrid solar-electric dryer for drying cocoa beans under similar environmental conditions. Likewise, Abano and Amoah (2015) noted that drying rates for cocoa using a solar tunnel dryer averaged around 0.030 kg/h, particularly when the air velocity was kept between 0.4 and 0.6 kg/h. This supports the current study’s observation that increasing airflow rate leads to marginal improvement in drying rate, though not always proportional due to internal resistance and saturation effects within the drying chamber.

In terms of drying efficiency, the fabricated hybrid dryer achieved values as high as 96.3%, which is significantly higher than the 70–85% range reported by Bala and Mondol (2001) in their evaluation of hot-air dryers for similar agricultural products. High efficiencies recorded in the current study, especially at lower airflow rates (e.g., 0.4 kg/h), suggest better heat utilization and reduced thermal losses, likely due to improved insulation or optimized airflow design. This trend is also consistent with the findings of Simate (2001), who emphasized that lower drying air velocity at moderate product loading improves thermal efficiency in convective dryers due to reduced heat loss and better contact time.

Furthermore, the inverse relationship observed between drying rate and efficiency in this study agrees with the theoretical drying behavior described by Mujumdar (2007), where high drying rates, particularly under forced convection or high airflow, often result in lower efficiencies due to energy loss via overheated air and incomplete heat transfer. Therefore, the recommendation to operate the hybrid dryer at moderate airflow (0.4–0.5 kg/h) and higher sample mass (2000–3000 g) is consistent with optimal drying strategies for maintaining a balance between throughput and energy efficiency reported in literature.

### Table 4.2: Analysis of Variance (ANOVA) for the Drying Rate of Cocoa Bean

| Source | Sum of Squares | df | Mean Square | F-value | p-value |  |
| --- | --- | --- | --- | --- | --- | --- |
| Model | 0.0002 | 2 | 0.0001 | 184.90 | < 0.0001 | significant |
| A-Mass of Sample | 0.0002 | 1 | 0.0002 | 258.49 | < 0.0001 |  |
| B-Air Flow Rate | 0.0001 | 1 | 0.0001 | 111.32 | < 0.0001 |  |
| Residual | 6.603E-06 | 10 | 6.603E-07 |  |  |  |
| Lack of Fit | 1.803E-06 | 6 | 3.004E-07 | 0.2504 | 0.9353 | not significant |
| Pure Error | 4.800E-06 | 4 | 1.200E-06 |  |  |  |
| Cor Total | 0.0003 | 12 |  |  |  |  |

*Significant at p≤0.05. A=mass of sample, B= air flow rate

The analysis of variance (ANOVA) for drying rate revealed that the overall model was highly significant with a p-value < 0.0001 and an F-value of 184.90, indicating that the model adequately explains variations in drying rate due to the independent variables (mass of sample and air flow rate). The model sum of squares (0.0002) accounts for a substantial portion of the total variation (correlation total = 0.0003), confirming the model’s strong explanatory power.

Among the model terms, both mass of sample (Factor A) and air flow rate (Factor B) were statistically significant, with p-values < 0.0001. The mass of sample had a higher F-value (258.49) compared to air flow rate (111.32), suggesting that sample mass had a stronger influence on drying rate than airflow under the tested conditions. This result aligns with physical drying principles, where the mass of material affects moisture migration and surface exposure, and may result in variable drying kinetics depending on loading and airflow distribution.

Furthermore, the lack-of-fit test was not significant (p = 0.9353), indicating that the model fits the experimental data well and there’s no evidence that unexplained variations are due to model inadequacy. The residual (error) mean square was very small (6.603E-07), supporting the model’s high precision and repeatability. Overall, this ANOVA confirms that the developed regression model is statistically valid, and that both sample mass and airflow rate significantly affect drying rate, with sample mass having a more dominant impact.

The ANOVA results from this study demonstrate that both mass of sample and air flow rate significantly influenced the drying rate of cocoa, with p-values < 0.0001, confirming the strong effect of these factors. This is in agreement with the findings of Simate (2001), who also reported that sample loading and drying air velocity had significant effects on drying time and drying rate of fermented cocoa beans using a solar dryer. In his study, airflow rate contributed significantly to the heat and mass transfer efficiency, while excessive sample mass led to internal resistance and slower moisture migration, which aligns with the current study’s emphasis on sample mass as a dominant factor (F = 258.49).

Similarly, Aregbesola et al. (2015) evaluated a hybrid dryer for cocoa drying and found that both air velocity and sample mass had statistically significant impacts on drying rate (p < 0.01). Their regression model, supported by ANOVA, showed that increasing airflow enhanced drying but with diminishing returns beyond a threshold, especially when sample mass was high—again consistent with this study's finding that air flow (F = 111.32) had less influence than mass of sample. This suggests that optimal airflow must be matched with sample load to avoid reduced efficiency or non-uniform drying.

In another related study, Abano and Amoah (2015) reported that for solar tunnel drying of cocoa, both temperature and air flow rate significantly influenced the drying rate (p < 0.05), with airflow being more critical at lower humidity levels. However, their study did not consider mass of sample as a model factor, which this current study shows to be more statistically significant. Additionally, the non-significant lack-of-fit (p = 0.9353) in your result indicates that the model adequately captured the behavior of the drying system,

a desirable outcome also achieved in Bala and Mondol’s (2001) study on solar drying of fish, where proper model fit ensured valid predictive capability.

Fig 4.1: Effect of Air Flow Rate and Mass of Sample on the Drying Rate of Cocoa

The 3D response surface plot illustrates the interactive effects of mass of sample (A) and air flow rate (B) on the drying rate of cocoa beans using the fabricated hybrid dryer. The surface shows a downward sloping plane from low sample mass (1000 g) and high airflow (0.6 kg/h) toward higher sample mass (3000 g) and lower airflow (0.4 kg/h). This confirms that higher drying rates are achieved at lower sample masses and higher airflow rates, which is expected due to more efficient heat and mass transfer in such conditions.

The surface appears fairly linear with a slight curvature, indicating that the relationship between variables and drying rate is largely additive with mild interaction. The contours at the base of the graph further confirm this trend—drying rate decreases gradually as both air flow rate and sample mass increase simultaneously. This supports the earlier ANOVA result where both factors were significant, and mass of sample had a more dominant effect. The red points on the plot (experimental runs) align well with the surface, indicating a good model fit and minimal residual error.

This graphical model is useful for visually identifying optimal operating regions. For instance, the peak drying rate (~0.042 kg/h) occurs at 1000 g sample mass and 0.6 kg/h airflow, while the minimum (~0.024 kg/h) is seen at 3000 g sample mass and 0.4 kg/h airflow. This further illustrates the trade-off between throughput and drying performance.

The visual trend in this plot mirrors the findings of Aregbesola et al. (2015), who reported that the drying rate of cocoa in a hybrid dryer decreased with increasing sample load due to limited airflow penetration. Their response surface also showed that drying was more efficient at lower mass and higher air velocity—consistent with the upper left zone of this plot.

Simate (2001) similarly observed from his 3D plots that drying efficiency and rate declined with sample overloading, emphasizing the importance of controlling loading density. The response behavior in this graph, where airflow only marginally offsets the impact of heavy sample loading, reinforces his conclusion that dryers must be carefully balanced in terms of thermal input and product mass.

In addition, Abano and Amoah (2015) also plotted surface responses of drying characteristics of cocoa and found that optimal performance zones could be visually identified around moderate mass and maximum air velocities, aligning well with the current graph. The gentle slope of the surface suggests linearity, also observed by Bala and Mondol (2001) in their solar dryer models, indicating predictable system response for optimization purposes.

### Table 4.3: Analysis of Variance (ANOVA) for the Drying Efficiency of Cocoa Bean

| Source | Sum of Squares | df | Mean Square | F-value | p-value |  |
| --- | --- | --- | --- | --- | --- | --- |
| Model | 232.70 | 2 | 116.35 | 152.81 | < 0.0001 | significant |
| A-Mass of Sample | 157.08 | 1 | 157.08 | 206.30 | < 0.0001 |  |
| B-Air Flow Rate | 75.62 | 1 | 75.62 | 99.31 | < 0.0001 |  |
| Residual | 7.61 | 10 | 0.7614 |  |  |  |
| Lack of Fit | 2.81 | 6 | 0.4677 | 0.3891 | 0.8551 | not significant |
| Pure Error | 4.81 | 4 | 1.20 |  |  |  |
| Cor Total | 240.31 | 12 |  |  |  |  |

*Significant at p≤0.05. A=mass of sample, B= air flow rate

The analysis of variance (ANOVA) for drying efficiency shows that the model is statistically highly significant with an F-value of 152.81 and a p-value < 0.0001, indicating that the model explains a significant portion of the variability in the drying efficiency of cocoa using the fabricated hybrid dryer. The model sum of squares (232.70) accounts for over 96.8% of the total variation (240.31), confirming the model’s strong predictive ability.

Both factors—mass of sample (Factor A) and air flow rate (Factor B)—were found to have significant individual effects on drying efficiency with p-values < 0.0001. Among the two, the mass of sample had the higher F-value (206.30), compared to air flow rate (F = 99.31), signifying that mass of sample has a more dominant influence on drying efficiency. This implies that increasing the sample mass has a more pronounced effect on how efficiently the dryer utilizes energy for moisture removal, possibly due to the improved heat retention or better absorption of thermal energy at higher product volumes.

### The lack-of-fit test was not significant (p = 0.8551), with a low F-value (0.3891), indicating that the model fits the experimental data well and that the residual variation is mostly due to random (pure) error, not model inadequacy. The residual mean square (0.7614) is acceptably low, suggesting good precision and experimental control. Overall, this ANOVA confirms that the statistical model is robust, and that both the mass of sample and air flow rate significantly affect drying efficiency, with the sample mass being the most influential factor.

The findings from this ANOVA agree with multiple studies in the drying literature, especially in terms of the significance of process parameters. Aregbesola et al. (2015) similarly found that both sample loading and airflow rate significantly influenced drying efficiency of cocoa in a hybrid dryer, with higher sample mass associated with better heat utilization due to reduced surface heat losses and enhanced product-to-air contact. Their F-ratio for sample mass was also notably higher than that of airflow, aligning with the trend observed in this study.

Simate (2001) reported in his optimization study of cocoa drying that higher sample loading resulted in increased drying efficiency due to greater energy absorption and longer air-product interaction time. However, he also cautioned against overloading, which can lead to reduced airflow effectiveness—a finding that underscores the need for balance, even when statistical results favor larger sample masses.

Furthermore, Bala and Mondol (2001) and Abano and Amoah (2015) observed that drying efficiency generally improves at moderate air velocities and well-distributed sample loading, but excessive airflow could reduce efficiency by carrying away unused heat energy. These findings support the result in this study where airflow was significant, yet less influential than sample mass. The non-significant lack-of-fit in this study mirrors those found in Bala and Mondol’s solar drying models, emphasizing the reliability and adequacy of linear-quadratic regression models for drying process modeling.

Fig 4.2: Effect of Air Flow Rate and Mass of Sample on the Drying Efficiency of Cocoa

The 3D response surface plot shows the combined effects of mass of sample (A) and air flow rate (B) on drying efficiency (%) of cocoa beans using the fabricated hybrid dryer. The surface trends upward from the lower left (low mass, high airflow) to the upper right (high mass, low airflow), indicating a positive relationship between sample mass and drying efficiency, and an inverse relationship between airflow and efficiency. This means that higher sample mass and lower airflow rate generally result in greater drying efficiency.

The contour lines at the base of the plot are nearly straight and parallel, indicating a mostly additive interaction between the two factors, with limited curvature or nonlinear interaction. The gradient of the surface suggests that drying efficiency increases more steeply with increasing sample mass, supporting the earlier ANOVA result where mass of sample had a higher F-value than airflow. This implies that efficiency is more sensitive to product loading than to airflow variation, within the tested range.

The red dots representing experimental points align closely with the fitted surface, indicating a good model fit and consistency of experimental data. The highest drying efficiency (≈96%) was achieved at 3000 g sample mass and 0.4 kg/h airflow, while the lowest efficiency (≈78%) was observed at 1000 g sample mass and 0.6 kg/h airflow. This visualization effectively demonstrates the trade-off between fast drying and efficient energy use, highlighting the need to optimize drying conditions not just for speed but also for resource utilization.

The pattern observed in this plot is consistent with findings from Aregbesola et al. (2015), who noted that drying efficiency increased with higher cocoa sample loading, as more thermal energy was absorbed by the product rather than lost to the environment. The plot’s clear upward slope with increasing mass confirms their observation that efficiency is maximized when heat is more effectively transferred to a larger product volume.

Simate (2001) also reported that drying systems exhibit better efficiency at moderate air velocities and higher product loads, as excessive airflow tends to waste heat energy and reduces the residence time of air around the product. This study's plot shows a similar inverse relationship between airflow and efficiency, especially at low product loads.

Abano and Amoah (2015), while working with a solar tunnel dryer, found that optimal drying efficiency was achieved at lower air velocities, especially when combined with appropriate product distribution. This matches the current finding that lower airflow (0.4 kg/h) yielded the best efficiency outcomes, reinforcing the concept that airflow should be optimized to minimize energy loss while maintaining sufficient moisture removal.

Finally, Bala and Mondol (2001) reported in their study on fish drying that straight and parallel contour lines indicated additive effects and a predictable model response, similar to the contour base of this plot. Their work supports the use of surface response modeling as a tool for identifying ideal operational zones, which in this case would be higher sample masses and controlled (moderate to low) airflow rates for optimal energy efficiency.

Chapter Five

Conclusion and Recommendation

This study successfully achieved its aim of to dry cocoa bean to storage moisture content. A hybrid dryer that integrates direct solar radiation with a solar-powered airflow mechanism was designed and fabricated and evaluated using locally available materials, ensuring both cost-effectiveness and accessibility. The performance evaluation of the system revealed that both the mass of cocoa beans and the air flow rate significantly influenced drying outcomes.

Drying rate ranged between 0.024 and 0.042 kg/h, while drying efficiency varied from 78.3% to 96.3%. ANOVA results confirmed that both process variables—sample mass and airflow—had statistically significant effects (p < 0.0001) on both drying rate and drying efficiency, with the mass of sample having a stronger influence in both cases. The response surface plots further validated these trends, showing optimal performance at higher sample mass and moderate air flow rates (0.4–0.5 kg/h). These results underscore the importance of balancing throughput and energy input in dryer design and operation.

Overall, the hybrid dryer proved to be a reliable, efficient, and affordable drying solution for cocoa beans, with the potential to improve drying quality, reduce post-harvest losses, and enhance income stability for smallholder farmers. Its simplicity of design and use of local materials further support its adoption in rural agricultural settings.

Recommendations

- Optimization for Field Use:
Further studies should explore optimizing the hybrid dryer under varying weather and environmental conditions to enhance its adaptability for year-round use by smallholder farmers.

- Incorporation of Thermal Storage:
To improve drying continuity during cloudy periods or at night, the system can be enhanced with thermal energy storage materials or a backup bio-fuel/solar battery unit.

- Economic and Quality Assessment:
A cost-benefit analysis and quality assessment of cocoa dried using the hybrid system versus traditional methods should be conducted to establish market advantages and economic viability.

- Scale-Up and Dissemination:
The design should be scaled up or modularized for cooperative or community-based use. Government agencies and NGOs should also be engaged for pilot testing, training, and wider dissemination.

- Applicability to Other Crops:
The dryer’s applicability should be tested for other moisture-sensitive agricultural products such as plantain, chili pepper, or grains, to expand its utility and impact.

References

Afoakwa, E. O. (2014). Chocolate science and technology (2nd ed.). John Wiley & Sons.

Afoakwa, E. O., Quao, J., Budu, A. S., & Saalia, F. K. (2013). Effect of drying methods on the physical properties of cocoa beans. Journal of Food Science and Technology, 50(4), 701–708. https://doi.org/10.1007/s13197-011-0392-0

Abano, E. E., & Amoah, R. S. (2015). Effect of solar tunnel dryer on the drying characteristics of cocoa beans. Journal of Agricultural Engineering and Technology, 23, 1–10.

Aregbesola, O. A., Ajayi, O. A., & Sanni, L. O. (2015). Performance evaluation of a hybrid solar-electric dryer for cocoa beans. International Journal of Engineering Research & Technology, 4(2), 1–7.

Aregbesola, O. A., Ajayi, O. A., & Sanni, L. O. (2015). Performance evaluation of a hybrid solar electric dryer for cocoa beans. International Journal of Engineering Research & Technology, 4(2), 1–7.

Aneani, F., & Takrama, J. F. (2004). Farmers’ knowledge and perception of quality of cocoa beans in Ghana. International Journal of Food and Agricultural Economics, 2(4), 25–31.

Bala, B. K., & Mondol, M. R. A. (2001). Experimental investigation on solar drying of fish using solar tunnel dryer. Drying Technology, 19(2), 427–436. https://doi.org/10.1081/DRT-100102920

Bart-Plange, A., Addo, A., Darko, E. O., & Asiedu, J. J. (2012). Physical properties of cocoa beans. Journal of Food Engineering, 113(2), 261–267. https://doi.org/10.1016/j.jfoodeng.2012.05.002

Bart-Plange, A., Addo, A., Dzisi, K. A., & Essuman, A. S. (2019). Mechanical drying of cocoa beans: A review. Journal of Food Processing and Preservation, 43(10), e14145. https://doi.org/10.1111/jfpp.14145

Bonaparte, A., Alikhani, Z., Madramootoo, C. A., & Raghavan, V. (1998). Some quality characteristics of solar-dried cocoa beans in St Lucia. Journal of the Science of Food and Agriculture, 76(4), 553–558. https://doi.org/10.1002/(SICI)1097-0010(199808)76:4<553::AID-JSFA59>3.0.CO;2-1

Fagunwa, A. O., Adeboye, B. A., & Olaniyan, A. M. (2020). Effect of drying temperature on the physical properties of cocoa beans. Journal of Food Processing and Preservation, 44(3), e14345. https://doi.org/10.1111/jfpp.14345

Hii, C. L., Law, C. L., & Suzannah, S. (2011). Effects of humidity on the drying rate of cocoa beans. Journal of Food Processing and Preservation, 35(6), 801–816. https://doi.org/10.1111/j.1745-4549.2011.00537.x

Hii, C. L., Law, C. L., & Suzannah, S. (2018). Hybrid drying of cocoa beans: A review. Journal of Food Science and Technology, 55(2), 412–422. https://doi.org/10.1007/s13197-017-2984-7

International Cocoa Organization. (2019). Quarterly bulletin of cocoa statistics (Vol. XLV, No. 1). https://www.icco.org/statistics/production-and-grindings/

Jinap, S. (1994). Organic acids in cocoa beans: A review. ASEAN Food Journal, 9(1), 1–11.

Jinap, S., Dimick, P. S., & Jamilah, B. (2003). Effect of drying on the quality of cocoa beans. Journal of Food Science, 68(2), 472–477. https://doi.org/10.1111/j.1365-2621.2003.tb05706.x

Kongor, J. E., Hinneh, M., Van de Walle, D., Afoakwa, E. O., Boeckx, P., & Dewettinck, K. (2016). Effects of drying methods on the drying characteristics and quality of cocoa beans. Journal of Food Engineering, 176, 87–94. https://doi.org/10.1016/j.jfoodeng.2015.08.019

Lasisi, D. (2014). A comparative study of effects of drying methods on quality of cocoa beans. International Journal of Engineering Research and Technology (IJERAT), 3(1), 991–996.

Mujumdar, A. S. (2007). Handbook of Industrial Drying (3rd ed.). CRC Press.

Oke, D. O., & Omotayo, K. F. (2012). Effect of forced-air artificial intermittent drying on cocoa beans in South-Western Nigeria. Department of Agricultural and Bio-environmental Engineering, School of Engineering, Nigeria.

Okiy, D. A., Okonkwo, J. C., & Eze, F. C. (2017). Effect of drying methods on the physical properties of cocoa beans. Journal of Food Science and Technology, 54(4), 1056–1065. https://doi.org/10.1007/s13197-017-2535-2

Othman, M. F., Abdullah, A. H., & Bakar, M. A. (2020). Solar drying of cocoa beans: A review. Journal of Food Engineering, 266, 109877. https://doi.org/10.1016/j.jfoodeng.2019.109877

Santander, M., Chica, V., Martínez Correa, H. A., Rodríguez, J., Villagran, E., Vaillant, F., & Escobar, S. (2025). Unravelling cocoa drying technology: A comprehensive review of the influence on flavor formation and quality. Foods, 14(5), 721.

Simate, I. (2001). Optimization of convective hot air drying of fermented maize. Journal of Food Engineering, 48(1), 63–69. https://doi.org/10.1016/S0260-8774(00)00134-6

Sukha, D. A., Butler, D. R., Umaharan, P., & Boult, E. (2008). The use of an optimized organoleptic assessment protocol to describe and quantify different flavour attributes of cocoa liquors made from Ghana and Trinitario beans. European Food Research and Technology, 226(3), 405–413. https://doi.org/10.1007/s00217-006-0545-z

Zahouli, G. I. B., Tagro Guehi, S., Monké Fae, A., Ban-Koffi, L., & Gnopo Nemlin, J. (2010). Effect of drying methods on the chemical quality traits of cocoa raw material. Advance Journal of Food Science and Technology, 2(4), 184–190.