MODELLING OF AUTOMATED RAILWAY

LEVEL CROSSING SYSTEM

BY

YUSUF ABDULLATEEF ORIYOMI

ND/23/RWE/PT/0003

BEING A RESEARCH PROJECT SUBMITTED TO

RAILWAY ENGINEERING DEPARTMENT,

INSTITUTE OF TECHNOLOGY (IOT)

KWARA STATE POLYTECHNIC ILORIN.

IN PARTIAL FULFILMENT OF THE REQUIREMENT FOR THE

# AWARD OF NATIONAL DIPLOMA (ND) IN RAILWAY ENGINEERING DEPARTMENT.

August,2025

CERTIFICATION

This is to certify that this project work is original research work of YUSUF ABDULLATEEF ORIYOMI matric number ND/23/RWE/PT/0003 and has been examined and approved as meeting the requirements of Department of Railway Engineering Institute Of Technology, Kwara State Polytechnic, Ilorin, Kwara State . In Partial Fulfilment of the Requirement for the Award of National Diploma (ND) in Railway Engineering Department.

# ENGR. NUHU A.O.  	DATE

Project Supervisor

# ENGR. YUSUF K.O.  	DATE

Project Coordinator

# ENGR. LASIS K.A.  	DATE

Head of Department

# ENGR. MAJUS TUKUR LAMURDE 	DATE

External Examiner

DEDICATION

This project is dedicated to Almighty God who throughout the duration of my studies has sustained my life and has given me the needed wisdom and understanding to put this project together.

ACKNOWLEDGEMENT

All glory, appreciation, and adoration go to Almighty Allah, the master of the universe, the most beneficent, the omnipotent, the magnificent, the most high and merciful, for his mercy on the success of this project.

Special thanks go to my supervisor, ENGR. LASISI K.A. He is not just a supervisor but one of the best lecturers in my department who has gone through this project and made meaningful corrections with a tender attribute that led to the successful completion of this project. May Almighty Allah bless him and his family. My appreciation also goes to ENGR. NURU A.O, ENGR. YUSUF K.O and other lecturers. I say a very big thanks to you all for the guidance given to me as your student. May Almighty Allah reward every one of you (Ameen).

I gratefully appreciate my wonderful parents, MR & MRS YUSUF, for their love, care, prayers, moral, and financial support. May Almighty Allah grant you both long life and prosperity to reap the fruits of your labor (Ameen).

TABLE OF CONTENTS

Title Page 	i

Certification 	ii

Dedication 	iii

Acknowledgement 	iv

Table of Contents 	v

Abstract 	viii

# CHAPTER ONE 	1

# INTRODUCTION 	1

1.1 Background of the Study 	1

1.2 Aim and Objectives 	2

1.3 Problem Statement 	2

1.4 Justification 	3

1.5 Scope of Work 	3

# CHAPTER TWO 	5

# LITERATURE REVIEW 	5

2.1 Overview of railway level crossing system 5

2.1.1 Types of Railway Level Crossings: 5

2.1.2 Function of a Railway Level Crossing System: 6

2.1.3 Manual Systems: 	6

2.1.4 Emergence of Automation: 	7

2.2 Automation and sensor-based system 8

2.2.1 Types of Sensors Used In Automated Systems 8

2.2.2 Working Principle of Sensor-Based Railway Gate Automation 9

2.2.3 Advantages of Sensor-Based Automation Systems 10

2.3 Micro controller-based control system 11

2.3.1 Working Principle of Microcontroller-Based Crossing Systems 12

2.3.2 Commonly Used Microcontrollers in Level Crossing Projects 13

2.3.3 Key Components in a Microcontroller-based Railway System 14

2.4 Integration of traffic signals and safety features 15

2.4.1 Components of an Integrated Signaling and Safety System 15

2.4.2 Integration Architecture 	16

2.4.3 Benefits of Integration 	17

2.4.4 Safety Features in Modern Railway Crossing Systems 17

2.4.5 Technological Enhancements 	19

2.5 Gaps in existing research 	20

2.6 Summary of literature insights 	21

# CHAPTER THREE 	23 METHODOLOGY 	23

3.1 Preamble 	23

3.2 Materials and Their Sources 	23

3.3 Research Procedure 	24 CHAPTER FOUR 	34

# DATA PRESENTATION, ANALYSIS AND DISCUSSION OF FINDINGS 34

4.1 Summary of Key Functional Results 35

4.2 System Operation Breakdown 	36

4.3 Achievement of Objectives 	38

4.4 Observations and Limitations 	38

4.5 Recommendations for Enhancement 39

# CHAPTER FIVE 	40

# CONCLUSION AND RECOMMENDATIONS 40

5.1 Conclusion 	40

5.2 Recommendations 	40

# REFERENCES 	42

# ABSTRACT

This research project focuses on the development and implementation of an Automated Railway Level Crossing System using microcontroller technology. The primary goal is to enhance safety at railway crossings by automating the detection of approaching and departing trains, thus ensuring timely closure and opening of barrier gates. The system employs ultrasonic sensors for accurate train detection and utilizes Arduino-based microcontrollers for controlling the gates and activating audio-visual alerts. This study outlines the current challenges faced by traditional manual systems, including human error and inefficiencies, and provides a justification for the need for automated solutions.

The methodology involves sourcing affordable components from local vendors, assembling them into a functional prototype, and programming the system using Arduino IDE. Various testing scenarios were conducted to evaluate the system's performance, which demonstrated successful detection of trains, prompt barrier operations, and effective alert systems. Limitations such as sensor range and environmental factors were identified, along with recommendations for enhancements, including the integration of advanced sensors and renewable energy solutions. This research contributes to the field of railway engineering by providing a scalable and cost-effective approach to improving safety at level crossings.

CHAPTER FIVE

CONCLUSION AND RECOMMENDATIONS

## 5.1 	Conclusion

The implementation of an automated railway level crossing system as demonstrated in this research has proven to be a viable solution to the persistent problem of railway accidents due to human error and delayed manual operations. The system successfully integrated sensor-based detection with microcontrollerbased control logic, enabling accurate, timely, and efficient gate operations.

This project addressed key safety concerns at level crossings by providing a system that is not only functional but also cost-effective and easy to implement, especially in developing regions with limited infrastructure. The system can significantly improve traffic management, reduce human-related errors, and save lives when deployed on a larger scale.

Although this prototype was tested on a small scale, the successful outcomes indicate great potential for real-world application with some enhancements. As urbanization and rail transportation continue to expand, such automated systems will become indispensable in modern transportation networks.

## 5.2 	Recommendations

Based on the findings and limitations of this study, the following recommendations are proposed:

- Adoption of Long-Range Sensors: To improve detection for high-speed trains and accommodate wider crossings, it is recommended to adopt LIDAR

or long-range infrared sensors with improved accuracy under varying environmental conditions.

- Power Backup and Solar Integration: To enhance system reliability, especially in rural or remote areas with unstable power supply, the system should incorporate solar power and battery backups to maintain continuous operation.

- Advanced Object Detection: Integration of AI-enabled cameras and Raspberry Pi modules can be used for obstacle detection (e.g., humans, animals, or vehicles) on the tracks, allowing for more intelligent gate control decisions.

- Remote Monitoring and GSM Integration: To enable remote fault detection and real-time reporting, GSM or IoT modules should be added to notify control centers in case of system failure or unusual activity at the crossing.

- Prototype Scaling and Field Testing: It is essential to move beyond modelscale to full-scale field testing. This will validate performance under realworld dynamics like train speed, environmental factors, and larger infrastructure.

- Government Support and Policy Integration: For widespread adoption, the government and relevant transport agencies should support automation policies, offer subsidies or grant for implementation, and mandate safety standards at level crossings.

REFERENCES

African Railway Safety Review (2022). Railway Crossing Automation in SubSaharan Africa. Safety Monitoring Division Report.

Banzi and Shiloh (2014). Getting Started with Arduino. Maker Media Inc. Covers the use of Arduino microcontrollers in embedded system applications, including transportation

Bhowmik, Sarkar and Roy. (2018). Microcontroller-Based Smart Railway Crossing System. IEEE International Conference on Smart Systems and Inventive Technology (ICSSIT).

Jain, Chauhan, and Gupta (2020), “Sensor-Based Automatic Railway Gate Control”, International Journal of Innovative Research in Electrical, Electronics, Instrumentation and Control Engineering, Vol. 8, Issue 1.

Kumar and Singh, (2022). Smart Railway Level Crossing with Traffic Signal Coordination. International Journal of Scientific & Engineering Research (IJSER), Vol. 13, Issue 3.

Mallick and Rout (2019), “An IoT Based Smart Railway Level Crossing System”, Procedia Computer Science, Vol. 152, pp. 431–438.

Ministry of Railways, India (2005). Annual Safety Review Report. Government of India, New Delhi. Notes that 30% of railway accidents were due to level crossing issues.

Oke and Ayeni (2020), “Evaluation of Railway Level Crossing Accidents in Nigeria”, Nigerian Journal of Transportation Technology.

Rahman, (2020). Arduino-Based Automatic Railway Gate Control Using GSM and IR Sensors. Dhaka University Journal of Engineering and Technology.

Rashid, Abdullah and Rahman (2012). Low-Cost Intelligent Railway Crossing System. IEEE International Conference on Smart Instrumentation,

Measurement 	and 	Applications 	(ICSIMA). 	[DOI:

10.1109/ICSIMA.2012.6501402]

Sharma and Choubey(2017). Integration of Traffic Lights and Railway Signals

Using Embedded Systems. Journal of Engineering Research and Application.

Singh and Sharma (2012). Railway Level Crossing Safety in India: Issues and Remedies. Journal of Traffic and Transportation Engineering, Vol. 1, No. 4. UIC (2022). Level Crossing Safety Report. International Union of Railways. www.uic.org

World Bank Report (2021), “Railway Safety Assessment in Sub-Saharan Africa”.