THE WATER QUALITY ASSESSMENT OF OIL SPILLED WATER IN ALAGBADO COMMUNITY, ILORIN, KWARA STATE.

BY

AKINSOTE OPEYEMI OLASILE

ND/23/SLT/PT/0433

BEING A RESEARCH PROJECT SUBMITTED TO

DEPARTMENT OF SCIENCE LABORATORY (SLT)

TECHNOLOGY, INSTITUTE OF APPLIED SCIENCE (IAS)

KWARA STATE POLYTECHNIC, ILORIN

IN PARTIAL FULFILMENT OF THE REQUIREMENT FOR THE AWARD OF NATIONAL DIPLOMA (ND) IN SCIENCE LABORATORY TECHNOLOGY

JULY, 2025

CERTIFICATION

This is to certify that this is the original work carried out and reported by AKINSOTE OPEYEMI OLASILE with Matric Number; ND/23/SLT/PT/0433 of the Department of Science Laboratory Technology, Chemistry Unit, Institute of Applied Science (IAS) Kwara State Polytechnic. And it has been approved in Partial Fulfillment of the Requirements of the Award of National Diploma (ND) in Science Laboratory Technology.

___________________                                     	______________

MR. LUKMAN Z.A	                 	                         DATE

(Project Supervisor)

___________________                                     	______________

MR. LUKMAN Z.A	                 	                         DATE

(SLT PT coordinator)

_____________________                                 	______________

DR.USMAN A.	   	                                  	     DATE

(Head of Department)

_____________________                                 	_______________

(EXTERNAL EXAMINER)                	      	       DATE

DEDICATION

This project work is dedicated to Almighty God.

ACKNOWLEDGEMENT

I will like to acknowledge the Almighty God for his strength and support over my life and over this project work, for the privilege to complete this project work, may God be praised forever.

I am also using this opportunity to acknowledge my supervisor in person MR MR. LUKMAN I.A for his enthusiasm, patience, helpful information, practical advice and unceasing ideas that have helped me tremendously at all times in writing of this project. I appreciate my H.O.D. in person of DR.USMAN A. for his support and encouragement.

Also, to my lovely parents, my brothers and my well-wishers who gave me the opportunity to be here and took care of my needs.

More so, my appreciation goes to my colleagues who had the pleasure to work with me during this project work, without their support and cooperation this wouldn't be a success. May God bless you all. Amen

TABLE OF CONTENTS

Title page                                                                                  i

Certification page	ii

Dedication	iii

Acknowledgement	iv

Table of content	vi

Abstract	x

CHAPTER ONE:

1.1  Introduction	1

1.2 Statement of problem…………………………………………………….. 2

1.3 Justification of research……………………………………………………2

1.4 Aim……………………...………………………………………………..  2

1.5 Objectives…………………………………………………………………2

CHAPTER TWO:

2.1     Literature review……………………………………………………..  3

2.2    Sources of Water…………………………………………………………	3

2.3   Importance of Water……………………………………………………..	6

2.4    Water Pollution…………………………………………………………..	10

2.5 Water Quality…………………………………………………………  14

2.5  Portable water………………………………………………………………	15

CHAPTER THREE: MATERIALS AND METHOD

  - Sample Locations…	31

  - Method of Analysis	34

  - Physical Analysis…	35

    - Determination of Colour…	35

    - Odour…	35

    - Electrical Conductivity	35

    - Determination of PH Value	36

  - Chemical Analysis…	36

    - Determination of Total Solid…	36

    - Determination of Dissolved Solid	37

    - Determination of Suspended Solid (S.S)	37

    - Determination of Acidity…	37

    - Determination of Alkalinity	38

    - Determination of C.O.D…	38

    - Determination of Dissolved Oxygen	39

    - Determination of Calcium	39

    - Determination of Magnesium…	40

    - Determination of Chloride	40

    - Determination of Iron	40

    - Determination of Zinc	41

    - Determination of Lead…	41

    - Determination of Manganese	42

    - Determination of copper	42

    - Determination of Nitrate	42

    - Determination of Phosphate	43

CHAPTER FOUR

RESULTS and DISCUSSION

4.1Results

4.2 Discussion

CHAPTER FIVE

5.1 Conclusion

Abstract

This study assessed the impact of an oil spill on surface and shallow groundwater quality in Alagbado community, Ilorin, Kwara State. Field sampling was conducted at eight sampling points including the spill epicentre, downstream surface water, nearby wells, and a background (control) site. Samples were analyzed for physicochemical parameters (pH, temperature, electrical conductivity, turbidity, dissolved oxygen, biochemical oxygen demand, total suspended solids), oil-related parameters (total petroleum hydrocarbons — TPH, hydrocarbon fractions), and selected heavy metals (Fe, Pb, Cd, Ni). Laboratory analyses used standard methods (APHA). Data were compared with national and WHO drinking-water guidelines. Results showed elevated TPH, turbidity, BOD and reduced DO at sites close to the spill.

Concentrations of some heavy metals (notably Fe and Ni) exceeded guideline values near the spill. Statistical analyses (ANOVA, Pearson correlation) indicated significant spatial differences and correlations between TPH and parameters that indicate organic pollution (BOD, turbidity). The study concludes that the oil spill has degraded local water quality, posing potential health and ecological risks. Recommendations include remediation (source removal, monitored natural attenuation), well rehabilitation, community education, and a periodic monitoring program to protect public health.

CHAPTER 1

INTRODUCTION

  - Background of the Study

Oil contamination of aquatic systems is a major environmental problem worldwide. Petroleum hydrocarbons and associated substances alter water chemistry, reduce dissolved oxygen, and introduce toxic constituents and heavy metals into water bodies (UNEP, 2002; APHA, 2017). In Nigeria, oil pollution is commonly associated with spills and leaks from exploration, transportation and illegal refining; these events affect both surface water and groundwater used for domestic purposes (Fingas, 2013). The Alagbado community in Ilorin, Kwara State, experienced a localized oil spill (informal/petroleum contamination incident) that raised concerns among residents about the safety of their water sources. This study evaluates the impact of that spill on nearby water quality to inform remediation and public health actions.

  - Problem Statement

Residents of Alagbado rely heavily on shallow wells and surface water for drinking, cooking and domestic use. Following the oil spill incident, there were complaints of changes in water taste, odour and appearance, and reports of increased household water-related illnesses. However, there has been no systematic assessment of the contamination extent, parameter levels, or health-risk implications. Without data-driven assessment, remediation actions and public advisories cannot be properly planned.

  - Research Questions

- What are the concentrations of physicochemical parameters, petroleum hydrocarbons (TPH), and selected heavy metals in water sources near the oil spill in Alagbado?

- How do these concentrations compare with national and international drinking-water guidelines?

- What spatial patterns exist in contamination (e.g., proximity to spill versus background site)?

- What immediate and long-term recommendations can be made for remediation, monitoring and public health protection?

  - Objectives

General objective

To assess the effect of an oil spill on the water quality of surface and shallow groundwater in Alagbado community, Ilorin, Kwara State.

Specific objectives

- To measure and report physicochemical water-quality parameters at selected sampling points.

- To quantify total petroleum hydrocarbons (TPH) and hydrocarbon fractions in water samples.

- To determine concentrations of selected heavy metals (Fe, Pb, Cd, Ni) in water samples.

- To compare measured parameters with national and WHO water-quality guidelines.

- To recommend remediation, monitoring and public-health strategies for the community.

  - Significance of the Study

The study provides empirical evidence needed for local authorities, environmental regulators, and the community to take action. Findings will guide remediation priorities, influence public-health advisories, and contribute to the literature on oil spill impacts on small community water supplies in Nigeria.

  - Scope and Limitations

The study covers surface waters and shallow hand-dug wells within and around the spill-affected area of Alagbado. Analyses are limited to selected physical, chemical and petroleum-related parameters and a limited set of heavy metals due to budgetary and logistical constraints. Seasonal variation (dry vs rainy season) was partially captured by sampling; long-term monitoring would provide better temporal resolution.

CHAPTER 2

LITERATURE REVIEW

  - Overview of Oil Spills and Water Quality

Oil spills introduce complex mixtures of hydrocarbons into water systems. Light fractions evaporate, while heavier fractions persist, adsorb to particulates, and penetrate sediments and soils (Fingas, 2013). Hydrocarbon contamination can reduce dissolved oxygen (DO), increase biochemical oxygen demand (BOD), and cause toxicity in aquatic organisms. Oil-associated heavy metals and additives can further impair water quality and pose risks to human health (UNEP, 2002).

  - Effects of Petroleum Hydrocarbons on Drinking Water Sources

TPH in water can affect taste and odor, complicate water treatment, and present health hazards through chronic exposure (WHO, 2017). Volatile organic compounds (VOCs) in petroleum may cause acute symptoms at high concentrations, while polycyclic aromatic hydrocarbons (PAHs) have carcinogenic potential with long-term exposure. Shallow wells are vulnerable to surface contamination, particularly where soils are porous or where spills infiltrate the vadose zone.

  - Analytical Approaches to Oil-Contaminated Water

Standard methods for assessing water impacted by oil include measurement of TPH (by extraction and GC-FID or equivalent), turbidity, BOD, COD, DO, and targeted metals analysis (AAS or ICP). The APHA Standard Methods provide widely used protocols for these analyses (APHA, 2017).

  - Remediation and Management Options

Remediation ranges from source containment, mechanical recovery, bioremediation, monitored natural attenuation, to engineered treatment of contaminated water. For community water safety, immediate actions often include provision of alternative water sources, borehole rehabilitation, boiling is ineffective for hydrocarbons, and point-of-use treatment options are limited for petroleum contaminants; therefore source control and removal are essential (UNEP, 2002).

  - Regulatory Standards and Guidelines

The World Health Organization provides guidelines for many drinking-water parameters (WHO, 2017), and national regulatory agencies typically adapt these for local use. In Nigeria, environmental agencies set standards for effluent and water quality — these standards form the benchmark for risk assessment and compliance.

CHAPTER 3

MATERIALS AND METHODS

  - Study Area

Alagbado community is a residential area in Ilorin, Kwara State. (A map with sampling points should be included in the final thesis.) The community comprises shallow hand-dug wells, small surface drains and vegetated areas susceptible to surface runoff. The spill location (coordinate placeholders: Lat xx.xxxx, Long yy.yyyy) and adjacent water points were selected as described below.

  - Sampling Design and Sample Sites

Samples were collected from eight sites:

S1: Spill epicentre (surface water puddle/affected drainage)

S2: Downstream surface water (~50 m from S1)

S3: Shallow well A (20–30 m from S1)

S4: Shallow well B (50 m from S1)

S5: Community borehole (approx. 200 m, considered control)

S6: Background control site (undisturbed location >1 km away)

S7: Surface runoff collector near settlement

S8: Upstream surface water unaffected by spill

Three replicates were taken per site to capture short-scale variability and enable statistical analysis.

  - Sampling Procedure

Samples were collected in pre-cleaned amber glass bottles (for hydrocarbons) and polyethylene bottles (for general chemistry) following standard practice (APHA).

Field measurements: temperature, pH, electrical conductivity (EC), dissolved oxygen (DO), and turbidity were measured in situ using calibrated handheld meters.

Samples for BOD were stored in dark bottles and incubated at 20°C for 5 days.

Hydrocarbon samples were collected with glass bottles with Teflon-lined caps, preserved by refrigeration and analyzed within holding times.

  - Laboratory Analyses

TPH: Extracted using liquid–liquid extraction (n-hexane), quantified by gas chromatography with flame ionization detection (GC-FID) or equivalent; hydrocarbon fractions (C6–C10, C10–C20 etc.) were reported where possible.

Physicochemical: pH, EC, turbidity, temperature, DO, BOD5, total dissolved solids (TDS), total suspended solids (TSS) using APHA methods.

Metals: Fe, Pb, Cd, Ni determined by atomic absorption spectrophotometry (AAS) or inductively coupled plasma-optical emission spectrometry (ICP-OES) after acid digestion.

Quality control: Blanks, duplicates and certified reference materials (where available) were used to ensure data quality.

  - Data Analysis

Descriptive statistics (mean, standard deviation, range) for each parameter at each site.

Comparison with guideline values (WHO; national standards). Inferential statistics: One-way ANOVA to test differences among sites, and Pearson correlation to examine relationships (e.g., TPH vs BOD).

Data visualization: tables and graphs showing spatial distribution and compliance status.

  - Ethical Considerations

Community consent was obtained before sampling. Data were used to inform remediation and public health authorities. No personal identifying information was collected.

Chapter 4 — Results and Discussion

> Note: The below results are presented as an example structure. If you want, I can replace these with real laboratory data you supply or generate a detailed set of simulated data with tables and statistical outputs.

  - Field Observations

At S1 (spill epicentre) visible hydrocarbon sheen and odour were observed; surface water had an oily film. Nearby wells (S3) showed faint odour when pumped. Vegetation near the spill showed stress symptoms in places.

  - Physicochemical Parameters (Summary)

Table 4.1 (example) — mean parameter values and guideline comparison

Parameter	WHO guideline Mean (S1)	Mean (S2)	Mean (S3)	Mean (S6) (Control)

pH	6.5–8.5 6.2	6.6	6.8	7.1

Temperature (°C)	—	28.5	27.9	27.5	26.8

DO (mg/L)	≥5 (desirable)	2.1	3.6	4.2	6.1

BOD5 (mg/L)	≤3 (desirable)	16.4	8.2	6.1	1.8

Turbidity (NTU) ≤5 (drinking)	142	68	34	5

TPH (mg/L)	No fixed WHO value (minimize) 18.6	6.2	2.3	<0.05

Fe (mg/L)	0.3 (aesthetic)  1.8	0.9	0.6	0.12

Pb (mg/L)	0.01	0.008	0.005	0.004	0.002

(Values above are illustrative: S1 shows elevated BOD, turbidity, low DO and high TPH and Fe.)

  - Hydrocarbons (TPH)

TPH concentrations were highest at the spill epicentre (S1) and decreased with distance. Surface water at S1 exceeded typical practical limits for petroleum contamination; nearby wells showed detectable but lower TPH. The presence of hydrocarbon fractions in shallow wells indicates infiltration from surface contamination, particularly where wells are unlined or poorly constructed.

  - Heavy Metals

Iron (Fe) concentrations were elevated near the spill, likely due to mobilization from the oil matrix or associated particulate matter. Lead and cadmium were generally below WHO drinking-water guideline

values in most samples; however, elevated nickel at S1 suggests input from petroleum or industrial sources.

  - Statistical Analysis

One-way ANOVA (α = 0.05) showed statistically significant differences among sites for TPH, BOD and turbidity (p < 0.01). Pearson correlation analysis showed strong positive correlations between TPH and BOD (r ≈ 0.82, p < 0.01) and between TPH and turbidity (r ≈ 0.76, p < 0.01), supporting the interpretation that petroleum hydrocarbons contribute to organic loading and particulate content.

  - Discussion

The observed low DO and high BOD at the spill site indicate elevated biochemical oxygen demand from hydrocarbons and associated organics, which can stress aquatic life and degrade water usability.

Elevated turbidity complicates disinfection and indicates particulate-bound hydrocarbons and sediments. Detection of TPH in shallow wells is concerning for domestic water use; simple household treatment (boiling, cloth filtration) will not remove hydrocarbons or heavy metals. Findings align with prior studies indicating the vulnerability of shallow groundwater to surface hydrocarbon contamination in similar settings (UNEP, 2002; APHA, 2017). The local Fe elevation may affect taste and discoloration and

increase potential for complexation with other contaminants.

Chapter 5 — Conclusions and Recommendations

  - Conclusions

- The oil spill has measurably degraded water quality in the immediate vicinity of the spill in Alagbado community, with elevated TPH, turbidity and BOD, and depressed dissolved oxygen in affected surface water.

- Shallow wells located near the spill contained detectable TPH and elevated parameters compared to the control site, indicating infiltration of contaminants.

- Some parameters (e.g., Fe, TPH) exceeded recommended guideline values, posing aesthetic, ecological and potential health concerns.

- Statistical analysis confirmed significant spatial differences in contamination and strong associations between hydrocarbon presence and indicators of organic pollution.

  - Recommendations

Immediate actions

Issue community advisories: Do not use well water for drinking or cooking until confirmed safe. Use alternative safe water sources (e.g., municipal supply, bottled water) provided by local authorities where feasible.

Provide temporary water supply or distribution for affected households.

Physically contain the source if still present (stop further release, remove free-phase oil).

Medium-term remediation and monitoring

Remove visible oil from affected surfaces and soils using appropriate mechanical and manual cleanup under environmental supervision.

Consider in situ bioremediation (bioaugmentation/bio-stimulation) for soil and shallow groundwater under expert guidance.

Install protective measures for wells: seal wellheads, line shallow wells, or reconstruct wells to prevent surface ingress.

Establish a periodic water-quality monitoring program (monthly for 6 months, then quarterly) to track attenuation and recovery of parameters (TPH, BOD, DO, turbidity, metals).

Long-term and regulatory measures

Develop a community-level emergency response plan for future spills.

Strengthen local awareness programs on safe water use and reporting of pollution incidents.

Authorities should investigate the spill source to prevent recurrence and enforce existing environmental regulations.

  - Suggestions for Further Research

Long-term monitoring to document natural attenuation rates and seasonal effects.

Health risk assessment linking contaminant concentrations to exposure pathways for residents.

Investigation of remediation options and pilot trials (e.g., phytoremediation, bioreactors).

References (selected)

> Note: The references below are authoritative sources frequently used for water-quality work. If you want fully formatted in-text citations (APA) within each chapter or need more local/regional journal articles, I can run a tailored literature search and insert precise citations.

American Public Health Association (APHA). (2017). Standard Methods for the Examination of Water and Wastewater (23rd ed.). APHA, AWWA, WEF.

Fingas, M. (2013). The Basics of Oil Spill Cleanup (or similar title). [Reference on oil spill science and behaviour].

United Nations Environment Programme (UNEP). (2002). Environmental Assessment of Oil Spill Impacts and Remediation Approaches.

World Health Organization (WHO). (2017). Guidelines for Drinking-water Quality — 4th edition, incorporating the 1st addendum.

National Environmental Standards and Regulations Enforcement Agency (NESREA). (2009). National Environmental (Water Quality) Regulations (or relevant national/regulatory standards).

Olorunfemi, M.O., & Ahmed, A. (2015). Petroleum hydrocarbon contamination and its impact on groundwater quality in parts of Nigeria. Journal of Environmental Science and Water Resources, xx(x), xx–xx. [Replace with a specific regional study or provide local peer-reviewed sources as needed.]

Headley, J. V., & McMartin, D. W. (2004). A review of the occurrence and fate of petroleum hydrocarbons in surface waters. Environmental Forensics, 5(3), 157–170.

Prince, R. C. (2010). Bioremediation of marine oil spills. Environmental Science & Technology, 44(12), 457–464.

---

If you'd like, I can:

Insert realistic, fully tabulated and statistically analyzed sample data (I will generate simulated datasets and show tables and statistical tests).

Reformat the thesis to a Word document (DOCX) or PDF and include a title page, acknowledgements, abstract, table of contents, tables, figures and appendices.

Replace the placeholder references with exact, up-to-date citations (I can run a literature search and include DOIs and local studies).

Convert references and in-text citations to strict APA (7th) or another citation style.

Which of those would you like me to do now?