Comparative Study on the Effects of Drying,

Method, and Blanching Temperatures on

The Proximate Composition of Yam Flour.

BY

SULAIMON, Ayuba Adewale

HND/23/ABE/FT/0078

A RESEACH PROJECT SUBMITED TO THE DEPARTMENT OF

AGRICULTURAL AND BIO- ENVIRONMENTAL ENGINEERING,

INSTITUTE OF TECHNOLOGY, KWARA STATE POLYTECHNIC,

ILORIN, KWARA STATE.

IN PARTIAL FULFILMENT OF THE REQUIREMENTS FOR THE

AWARD OF HIGHER NATIONAL DIPLOMA (HND) IN AGRICULTURAL AND BIO- ENVIRONMENTAL ENGINEERING.

JULY, 2025.

# DECLARATION

I hereby declare that this research project tittle ‘’ Comparative Study On The Effects Of  Drying, Method  And Blanching Temperatures On The Proximate Composition Of Yam Flour is my own work and has not been submitted by any other person for any degree or qualification at any higher institution. I also declare that the information provided therein are mine and those that are not mine are properly acknowledge.

_____________________                                                                    ____________________

Sulyman Ayuba Adewale                                                                     Signature and Date

# DEDICATION

# This project work is dedicated to Almighty Allah for the successful completion of my programme.

# ACKNOWLEDGEMENTS

First and foremost, I would like to express my deepest gratitude to Almighty Allah for his invaluable guidance and for sparing my life. I would like to extend my sincere gratitude to my project supervisor Engr. (mrs) Olayaki- Luqman,M. for her exceptional guidance and support throughout this project.

I am highly indebted to Engr Mrs kareem Bashirat.A for her guidance and constant supervision as well as for providing necessary information regarding the project and also for her support in completing the project. Her constant guidance and willingness to share her vast knowledge made us understand this project and its manifestations in great depths helped us to complete the assigned tasks on time.

I have taken efforts in this project. However, it would not have been possible without the kind support and help of many individuals. I would like to extend my sincere thanks to you all.

Finally, I must thank my parents for their unwavering support and encouragement needed to take on this challenging but enriching project.

# ABSTRACT

Yam flour quality is strongly influenced by post-harvest processing methods, yet limited research exists comparing drying and blanching combinations. This study investigated the effects of sun drying and oven drying methods at the temperature of 60°C and two blanching temperatures (65°C and 95°C) on the proximate composition and drying rate of yam flour. Fresh yam tubers were sliced and blanched using a water bath for 10 minutes at the specified temperatures. The blanched samples were then divided into two portions and subjected to drying either under direct sunlight (10:00 am – 4:00 pm daily for 3 days) or in a hot air oven set at 60°C for 15 hours, until a constant weight was attained. Each drying treatment started with an initial sample weight of 165 g. The dried samples were milled into flour and analyzed for moisture, crude protein, crude fat, crude fiber, ash, and carbohydrate contents using standard AOAC (2010) methods. One-way ANOVA and Tukey's post-hoc test confirmed significant differences among treatments. Two-way ANOVA further showed that blanching temperature, drying method, and their interaction significantly affected most parameters, particularly moisture, fat, ash, and carbohydrate content.Results showed that both drying method and blanching temperature had significant effects (p < 0.05) on all proximate components. Oven-dried samples, especially those blanched at 65°C, retained higher levels of protein (up to 4.82%) and fat (1.03%), while sun-dried samples blanched at 95°C had the lowest moisture content (9.87%) and the highest carbohydrate concentration (83.96%). Drying rate analysis indicated that the fastest drying occurred in the sun-dried sample blanched at 95°C (5.70 g/h), while the slowest was recorded in the sun-dried sample blanched at 65°C (3.71 g/h). The study concludes that oven drying at 60°C after blanching at 65°C is optimal for producing yam flour with superior nutritional quality. It recommends further investigation into storage properties and the development of hybrid drying systems for improved commercial application. 


.

# TABLE OF CONTENTS

Contents

DECLARATION                                                                                                                                                  ii

CERTIFICATION                                                                                                                                                iii

DEDICATION                                                                                                                                                      iv

ACKNOWLEDGEMENTS                                                                                                                                v

ABSTRACT                                                                                                                                           vi

TABLE OF CONTENTS                                                                                                                                    vii

LIST OF TABLES                                                                                                                                                ix

LISTS OF PLATES                                                                                                                                              x

LIST OF FIGURES                                                                                                                                             xi

CHAPTER ONE                                                                                                                                                   1

INTRODUCTION                                                                                                                                                1

1.1  Background to the Study                                                                                                              1

1.2 Statement of the Problem                                                                                                              3

1.3 Aim and Objectives of the Study                                                                                                   4

1.4 Justification of the Study                                                                                                               4

1.5 Scope of the Study                                                                                                                         4

CHAPTER TWO                                                                                                                                                   5

LITERATURE REVIEW                                                                                                                                     5

2.1 General Overview of Yam                                                                                                             5

2.2 Proximate Composition of Yam Flour                                                                                          5

2.3 Blanching in Food Processing                                                                                                      6

2.4 Yam Production and Global Importance                                                                                      6

2.5 Nutritional and Economic Significance of Yam Flour                                                                 7

2.6 Challenges in Yam Flour Production                                                                                            7

2.7 Post-Harvest Losses in Yam Tubers                                                                                              7

2.8 Impact of Blanching on Nutritional and Anti-Nutritional Factors                                                7

2.9 Drying Methods of Yam Flour Production                                                                                    8

2.9.1 Oven Drying                                                                                                                                        8

2.9.2 Sun Drying                                                                                                                                           8

2.10 Combined Effects of Blanching and Drying on Proximate Composition of Yam                       9

2.11 Nutritional and Economic Relevance of Yam Flour                                                                    9

CHAPTER THREE                                                                                                                                             10

MATERIALS AND METHODS                                                                                                                       10

3.1 Materials and Equipmen                                                                                                              10

3.2 Sample Collection                                                                                                                        10

3.2.1 Sample preparation                                                                                                                           10

3.3 Blanching Procedure                                                                                                                    13

3.4 Drying Procedure                                                                                                                         17

3.4.1: Oven Drying                                                                                                                                     17

3.4.2: Sun Drying                                                                                                                                        19

3.5 Milling and Sieving                                                                                                                      19

3.6 Proximate Composition Analysis                                                                                                 19

3.7 Drying Rate Calculation                                                                                                               19

3.8 Experimental Design                                                                                                                    19

3.9 Statistical Analysis                                                                                                                       20

CHAPTER FOUR                                                                                                                                               23

RESULTS AND DISCUSSIONS                                                                                                                     23

4.1 RESULTS                                                                                                                                   23

4.2 DISCUSSIONS                                                                                                                          30

4.2.1: Moisture Content                                                                                                                            30

4.2.2: Crude Protein                                                                                                                                  30

4.2.3: Crude Fat                                                                                                                                         30

4.2.4: Ash and Crude Fibre Content                                                                                                         30

4.2.5: Carbohydrate Content                                                                                                                     30

4.3 Discussion on Table 4.3 One-Way ANOVA and Tukey HSD Results                                    31

4.4 Discussion on Table 4.4  A two-way ANOVA                                                                           31

4.5  Discussion on Drying Rate Table 4.5                                                                                      31

4.6. Comparative Discussion with Existing Research and Practical Implications                    32

4.6. 1: Moisture Content                                                                                                                           32

4.6.2: Protein Retention                                                                                                                             32

4.6.3: Fat Stability                                                                                                                                      32

4.6.4: Drying Rate                                                                                                                                      32

CHAPTER FIVE                                                                                                                                                 33

CONCLUSIONS AND RECOMMENDATIONS                                                                                         33

5.1 Conclusions                                                                                                                                33

5.2 Recommendations                                                                                                                     33

REFERENCES                                                                                                                  34

# LIST OF TABLES

Table 3.1 Blanching treatment                                                                                              15

Table 3.2 Treatment Combinations                                                                                       22

Table 4.1 Proximate Composition (%) of the Samples                                                         25

Table 4.2: The Mean of the Proximate Composition (%)                                                     26

Table 4.3 One-Way ANOVA of the Proximate Composition of Yam flour                           27

Table 4.4: Two-Way ANOVA Summary (p-values) for Proximate Parameters                     28

Table 4.5 presented the Result of the Drying Rate of Yam Flour Samples                            29

# LISTS OF PLATES

Plate 3.1: Yam cutting process                                                                           12

Plate 3.2: Yam peeling process                                                                          13

Plate 3.3 Water bath used for the blanching                                                        16

Plate 3.4 Cooling process after the blanching                                                     17

Plate 3.5 Oven drying                                                                                          19

# LIST OF FIGURES

Figure 3.1 Drying methods                                                                                  23

Figure: 4.1: Drying Rate of Yam Flour Samples                                                  30

# CHAPTER ONE

# INTRODUCTION

## 1.1 Background to the Study

Food and nutrition security are critical priorities form any developing nations, especially in the face of population growth and changing climates (Padhan&Panda,2020). Addressing this challenge requires diversifying agricultural practices and exploring alternative food sources.  Yams, belonging to the Dioscoreaceae family and the genus Dioscorea, are a staple food and key to food security in many tropical regions (Kanu etal.,2018). Globally, yam ranks as the third most important root crop, following cassava and sweet potatoes (Adepojuetal.,2018).

As a starchy staple, it provides an affordable source of dietary energy, predominantly in the form of carbohydrates (Obidiegwuetal.,2020; Ojinnakaetal.,2017). Yam tubers are commonly consumed boiled, fried, or baked and play an essential role in traditional cuisine across various cultures.

In addition, yams are often processed into flour, which has significant culinary applications and serves as a staple food in many countries (Derejeetal.,2020). Yam flour is particularly valued for its long shelf life and compact form, making it an ideal raw material for storage and transportation (Setyawanetal.,2021; Zouetal.,2021).

Beyond direct consumption, yams hold considerable potential for industrial applications, contributing to local livelihoods, particularly in densely populated regions of southern, southwestern, and western parts of various countries.

Yam is also recognized as a traditional medicinal and edible plant, with its rhizome rich in nutrients such as protein, total phenols, and allantoin, offering significant health benefits (Zhang, Ding, etal. 2023;Zhang,Zheng,etal.,2023).

Research suggests that yam consumption aids digestion, boosts immunity, and supports cardiovascular health. However, due to its high moisture content often exceeding 70%

yam is prone to spoilage during long-term storage, making drying a crucial preservation method (Xiaoetal.,2015).

Drying reduces water content, thereby lowering water activity, extending shelf life, and preventing microbial contamination during storage. Common drying techniques for yam include natural drying (Juetal.,2015), hot air drying (Juetal.,2016), vacuum freeze-drying (Chen,Lu,etal.,2017;Chen,Tian,etal.,2017), and microwave drying (Chen,Lu,etal.,2017;Chen,Tian,etal.,2017). While natural drying is cost-effective, it is slow and vulnerable to environmental conditions. Hot air drying Is faster but can compromise nutritional quality due to high temperatures.

The production process of yam flour, including peeling, blanching, drying, and milling, significantly affects its nutritional and physicochemical properties (AOAC International, 2019). Among these steps, drying and blanching are critical, as they influence enzyme activity, microbial load, and nutrient retention. Blanching, which involves heating yam slices in hot water, helps preserve the tuber's color, texture, and nutritional content. However, blanching at different temperatures can have varying effects on the proximate composition of the flour (Akinwande & Ogundele, 2020).

## 1.2 Statement of the Problem

Yam (Dioscorea spp.) is a vital staple crop in many tropical and subtropical regions, serving as a primary source of carbohydrates, dietary fiber, and essential minerals. However, yams are highly perishable due to their high moisture content, leading to significant post-harvest losses during storage and distribution (Akinwande & Ogundele, 2020). To address this issue, yam is often processed into yam flour, which has an extended shelf life, improved storage characteristics, and broader culinary applications.

The choice of drying method and pre-treatment conditions plays a crucial role in determining the quality and nutritional composition of yam flour. For instance, oven drying and sun drying are common techniques with varying impacts on the proximate composition, such as moisture content, protein, fat, fiber, and ash levels. Similarly, blanching temperatures during pre-treatment (using 65°C and 95°C) significantly affect the retention of nutrients and anti-nutritional factors (Adepoju & Oyewole, 2018). Despite these known influences, there is limited research systematically comparing the combined effects of different drying methods and blanching temperatures on the nutritional and functional properties of yam flour (Wahab et al., 2016).

This knowledge gap has hindered the adoption of optimal processing techniques in yam flour production, potentially affecting the nutritional quality of the final product and limiting the competitiveness of local yam flour in the global market. Therefore, this study seeks to investigate the effects of two drying methods (oven drying at 60°C and sun drying) and two blanching temperatures (65°C and 95°C) on the proximate composition of yam flour. The results aim to provide evidence-based recommendations to enhance yam flour processing practices and improve the nutritional value of this essential food product.

## 1.3 Aim and Objectives of the Study

The aim of the study is to investigate the effects oven drying, sun drying, and blanching temperatures on the proximate composition of yam flour.

Objectives are:

- To evaluate the proximate compositions (moisture, ash, fat, protein, fiber, and                carbohydrate content) of yam flour produced using oven drying at 60°C, and  sun drying after blanching at 65°C and 95°C respectively.

- To compare the effects of oven drying and sun drying methods on the nutritional properties of yam flour at the two blanching temperatures.

iii.         To determine the optimal drying and blanching conditions for yam flour production.

## 1.4 Justification of the Study

Yam flour is an essential staple in many households, particularly in West Africa, due to its long shelf life and versatility in food preparation. However, the quality of yam flour is influenced by processing methods, particularly drying and blanching. Understanding the impact of these variables can provide valuable insights for small-scale and industrial processors to improve product quality, enhance consumer acceptability, and reduce post-harvest losses. Additionally, optimizing these processes may contribute to better nutritional retention, meeting both health and economic demands.

## 1.5 Scope of the Study

The study focused on yam flour production using two drying methods (oven drying at 60°C and sun drying) blanched at different temperatures (65°C and 95°C). It explored the effects of these factors on the proximate composition of yam flour. The yams used was limited to white yam (Dioscorea rotundata), the commonly consumed species. The study was carried out in a controlled environment to minimize variability in drying conditions and to maintain consistency in proximate analysis techniques.

# CHAPTER TWO

# LITERATURE REVIEW

## 2.1 General Overview of Yam

Yam (Dioscorea spp.) is a vital staple crop in tropical and subtropical regions, contributing significantly to food security and economic development. In West Africa, yam accounts for over 90% of global production, with Nigeria being the largest producer (FAO, 2022). Yam is highly valued for its nutritional composition, particularly its high carbohydrate content, which serves as an essential energy source. It also contains proteins, vitamins, and minerals, making it a vital dietary component (Otegbayo et al., 2019).

Yam tubers are highly perishable due to their high moisture content and susceptibility to post-harvest losses, which are often exacerbated by inadequate storage and processing facilities (Okonkwo et al., 2020). This has led to the increased focus on processing yam into value-added products such as yam flour, which has a longer shelf life and diverse culinary applications.

## 2.2 Proximate Composition of Yam Flour

The proximate composition of yam flour determines its nutritional and functional properties. Moisture content influences the shelf life of the flour, with lower moisture levels being desirable for extended storage (Adejumo et al., 2020). Ash content reflects the mineral composition of the flour, while protein, fat, fiber, and carbohydrate contents contribute to its nutritional and energy value (AOAC International, 2019).

The proximate composition of yam flour is significantly affected by the yam species, processing methods, and environmental factors. For instance, Adejumo et al. (2020) observed that yam flour produced from Dioscorea rotundata had a higher carbohydrate content compared to Dioscorea alata, which had a higher protein content.

## 2.3 Blanching in Food Processing

Blanching, as a pre-treatment process, is essential for preserving the nutritional and sensory quality of processed foods. In yam flour production, blanching is critical for inactivating enzymes, such as polyphenol oxidase, that cause browning and spoilage (Kumar et al., 2019). Blanching also improves the texture and color of yam slices, making them more suitable for drying and milling into flour.

However, blanching at higher temperatures can lead to nutrient losses, particularly water-soluble vitamins and minerals. For instance, Musa et al. (2021) reported a significant reduction in vitamin C content in yams blanched at 95°C compared to those blanched at 65°C. Despite this, higher blanching temperatures are more effective in reducing microbial load and inactivating enzymes.

## 2.4 Yam Production and Global Importance

Yam (Dioscorea spp.) plays a crucial role in ensuring food security in tropical and subtropical regions, particularly in West Africa, where over 90% of global yam production occurs. It serves as a major source of energy, providing carbohydrates, vitamins, and minerals essential for human health (FAO, 2021). Dioscorea rotundata (white yam) is particularly valued for its versatility in food preparation and its economic importance to smallholder farmers (Rees & Bancroft, 2018). Despite its importance, yam production faces challenges such as post-harvest losses and limited value addition, which necessitates improved preservation techniques, such as yam flour production.

## 2.5 Nutritional and Economic Significance of Yam Flour

Yam flour is valued for its extended shelf life, ease of transportation, and versatility in food preparation. It provides essential nutrients like carbohydrates and dietary fiber, making it an important product in combating food insecurity. Economically, yam flour production creates income opportunities for farmers and small-scale processors (Wahab & Adeyemi, 2016).

## 2.6 Challenges in Yam Flour Production

Key challenges include energy costs, inefficiencies in drying methods, and quality control. These issues are especially pronounced in developing countries where sun drying is widely used (Adebayo et al., 2015). Addressing these challenges through optimized drying and blanching methods can improve the competitiveness of yam flour in global markets.

## 2.7 Post-Harvest Losses in Yam Tubers

Yam tubers are prone to significant post-harvest losses, which can reach up to 40% in some regions (Rees & Bancroft, 2018). These losses are attributed to their high moisture content, making them susceptible to microbial spoilage and physiological degradation. Processing yam into flour is an effective strategy to reduce post-harvest losses while enhancing its shelf life and ease of transportation (Akinwande & Ogundele, 2020). However, optimizing the processing steps, particularly drying and blanching, is critical to ensuring high-quality yam flour with minimal nutrient loss.

## 2.8 Impact of Blanching on Nutritional and Anti-Nutritional Factors

Blanching is a critical pre-treatment step in food processing that aids in enzyme inactivation, color preservation, and microbial reduction. It also influences the nutritional and anti-nutritional factors of the final product. Studies have shown that blanching at higher temperatures can reduce anti-nutritional factors like oxalates and phytates, but prolonged blanching may lead to nutrient loss (Adepoju & Oyewole, 2018). The use of optimal blanching conditions, such as 65°C and 95°C, can help retain essential nutrients while improving the quality of yam flour.

## 2.9 Drying Methods of Yam Flour Production

Drying is a fundamental step in yam flour production that directly affects the product's quality, nutritional composition, and storage stability. Oven drying and sun drying are two widely used methods. Oven drying is faster and more controlled, providing uniform drying, but it requires higher energy input. Sun drying, on the other hand, is cost-effective but dependent on weather conditions and prone to contamination (Kumar & Mishra, 2017). Comparing these methods under varying blanching conditions can provide insights into optimizing drying processes.

### 2.9.1 Oven Drying

Oven drying is a modern method that provides uniform and controlled drying conditions. It reduces moisture content quickly, preventing microbial growth and preserving the quality of the flour (Adepoju & Oyewole, 2018). However, excessive heat during oven drying can denature proteins and oxidize fats, leading to changes in the proximate composition. Kumar et al. (2019) found that oven drying at 60°C retained more protein and carbohydrate content compared to higher drying temperatures.

### 2.9.2 Sun Drying

Sun drying is a traditional and cost-effective method widely used in rural areas. It involves exposing yam slices to direct sunlight, which evaporates the moisture. However, sun drying is highly dependent on weather conditions and is prone to contamination by dust, insects, and microbial growth (Akinwande & Ogundele, 2020). Despite its limitations, sun drying remains a popular choice due to its low energy requirements and simplicity.

## 2.10 Combined Effects of Blanching and Drying on Proximate Composition of Yam

The interplay between blanching and drying significantly affects the nutritional and functional properties of yam flour. Studies have shown that blanching at 95°C before drying results in higher nutrient retention due to effective enzyme inactivation, which reduces nutrient degradation during drying (Musa et al., 2021).

Drying methods also play a crucial role in determining the proximate composition of yam flour. Oven drying at controlled temperatures is associated with better nutrient retention compared to sun drying, which may cause prolonged exposure to heat and oxidative damage (Adepoju & Oyewole, 2018). Combining optimal blanching and drying conditions can improve the quality of yam flour, ensuring its suitability for consumption and industrial applications.

## 2.11 Nutritional and Economic Relevance of Yam Flour

Yam flour production offers numerous nutritional and economic benefits. It provides a nutrient-dense food product that is easy to store and transport. Additionally, yam flour processing can reduce post-harvest losses, contributing to food security and economic empowerment in yam-producing regions (FAO, 2022).

The production of yam flour also creates opportunities for small-scale and industrial processors, fostering economic growth and job creation. Understanding the effects of processing methods on the nutritional quality of yam flour can further enhance its market potential and consumer acceptability (Otegbayo et al., 2019).

# CHAPTER THREE

# MATERIALS AND METHODS

3.1 Materials and Equipment

Materials Used:

- Fresh white yam tubers (Dioscorea rotundata)

- Clean water for blanching

- Clean white net material for insect protection during sun drying

- Stainless steel tray for the sun drying

- Aluminium trays for oven drying

Equipment Used:

- Weighing balance (sensitivity: 0.01 g)

- Water bath (temperature-controlled)

- Thermometer

- Electronic Blender

- Timer

- Vernier caliper

- Drying oven (set at 60°C)

- Sun drying rack (exposed between 10 AM and 4 PM)

- Grinder and sieve (for flour production)

- Laboratory glassware

- Proximate analysis apparatus (Kjeldahl setup, Soxhlet extractor, muffle furnace, etc.)

## 3.2 Sample Collection

Fresh, mature yam tubers were purchased from a local market oja-oba in Ilorin, Kwara State, Nigeria.

### 3.2.1 Sample preparation

- Fresh yam tubers were washed, peeled, and sliced into uniform sizes (5 mm thickness).

- Sliced yam samples were divided into two groups for blanching at two temperatures: 65°C and 95°C.

Each blanched group was further divided into two subgroups for drying using two methods:

- Oven drying: Samples were dried in an oven at 60°C until a constant weight was achieved.

- Sun drying: Samples were spread thinly on racks under direct sunlight and dried until a constant weight was achieved.

Plate 3.1: Yam cutting process

Plate 3.2: Yam peeling process

## 3.3 Blanching Procedure

Blanching was carried out using a digital water bath to ensure precise temperature control. The sliced yam samples were divided into two groups and subjected to the blanching as shown in Table 3.1. The temperatures were maintained using a water bath, and a thermometer was used to monitor the water temperature to ensure accuracy. After blanching, the yam slices were drained and allowed to cool before drying.

Table 3.1 Blanching treatment

| Group | Blanching temperatures (°C) | Blanching time (mins) |
| --- | --- | --- |
| A | 65 | 10 |
| B | 95 | 10 |

Plate 3.3 Water bath used for the blanching

Plate 3.4 Cooling process after the blanching

## 3.4 Drying Procedure

Each blanched sample group was further divided into two equal portions and subjected to two different drying methods:

### 3.4.1: Oven Drying

Oven drying was carried out using a laboratory oven pre-set to 60°C as (Plate3.5). The samples were spread evenly on aluminum trays and dried for 15 hours, during which they were weighed periodically until a constant weight was achieved.

Plate 3.5 Oven drying

### 3.4.2: Sun Drying

Conducted outdoors under natural sunlight between 10:00 AM and 4:00 PM daily for three consecutive days (18 hours total). The yam slices were spread thinly on a stainless steel tray and covered with a sterile netting material to prevent contamination.

Drying continued until the weight of the samples remained constant.

## 3.5 Milling and Sieving

Dried yam chips were milled using a mechanical grinder to obtain yam flour. The flour was sieved through a 250 µm mesh sieve to ensure uniform particle size and then stored in air-tight containers for further analysis.

## 3.6 Proximate Composition Analysis

Standard methods of the Association of Official Analytical Chemists (AOAC, 2010) were used to determine the following parameters:

Moisture Content: Oven-dried at 105°C until constant weight

Crude Protein: Determined by the Kjeldahl method (nitrogen × 6.25)

Crude Fat: Extracted using the Soxhlet method with petroleum ether

Crude Fiber: Measured through sequential acid and alkali digestion

Ash Content: Determined using a muffle furnace at 600°C

Carbohydrate: Calculated by difference:

Carbohydrate (%) = 100 - (Moisture + Protein + Fat + Fibre + Ash)

## 3.7 Drying Rate Calculation

The drying rate was determined by recording the weight of the samples at regular intervals during drying until a constant weight was achieved.

The drying rate for each sample was calculated Equation 3.1.

Drying Rate = Initial Weight (g)-Final Weight (g)Drying Time (h)                                                Equation 3.1

## 3.8 Experimental Design

The study employed a 2 × 2 factorial design

Two blanching temperatures (65°C and 95°C)

Two drying methods (sun drying and oven drying)

Each treatment was conducted in duplicate to ensure reproducibility.

This study investigates the effects of blanching temperature and drying method on the proximate composition and drying rate of yam flour.

Independent Variables (Factors)

1. Blanching Temperature

65°C

95°C

2. Drying Method

Oven drying at 60°C

Sun drying

Each level of blanching temperature was combined with each drying method, resulting in four distinct treatment combinations.

## 3.9 Statistical Analysis

Data were analysed Using SPSS version 25. ANOVA was performed to determine significant differences at p < 0.05.

Table 3.2 Treatment Combinations

Treatment code Blanching Temperature Drying method

T1                                                              65°C                     Oven drying at 60°C

T2                                                              65°C                      Sun drying

T3                                                              95°C                      Oven drying at60°C

T4                                                              95°C                      Sun drying

Control

Each treatment was replicated twice using an initial sample weight of 165 grams, and drying was carried out until a constant weight was achieved.

Responses Measured

The following parameters were analyzed:

Moisture content (%)

Crude protein (%)

Crude fat (%)

Crude fiber (%)

Ash content (%)

Carbohydrate content (%)

Drying rate (g/h)

EXPERIMENTAL LAYOUT

Drying Method

┌───────────────────────────────┐

Oven                                                                                   drying

Oven                                                                                   drying

Sun Drying

Blanching Temperature

65°C      and       95°C

T1: 65°C + Oven Drying                                          T2: 65°C + Sun Drying

T3: 95°C + Oven Drying                                          T4: 95°C + Sun Drying

Figure 3.1 Drying methods

# CHAPTER FOUR

# RESULTS AND DISCUSSIONS

## 4.1 RESULTS

The results of the proximate compositions of yam flour samples under the influence of sun drying, and oven drying at 60°C, blanched at 65°C and 95°C are presented in Table 4.1.  The mean of the proximate composition (%) of yam flour under different treatments was presented in Table 4.2. one-way ANOVA of the proximate composition of yam flour are presented in Table 4.3.Table 4.4 presented the two-way ANOVA summary (p-values) for proximate parameters. Table 4.5 presented the result of the drying rate of yam flour samples and Figure: 4.1 presented the drying rate of yam flour samples

Table 4.1 Proximate Composition (%) of the Samples

Sample Moisture Crude Fat Crude Fibre Crude Protein Ash Carbohydrate

Fresh      59.627          0.768           1.236               3.628               2.516        32.225

59.502          0.743           1.219               3.701               2.504        32.331

65°C        11.058          1.017            1.154              4.824                2.315        79.632

Oven       11.106          1.042            1.169              4.816                2.303        79.564

95°C        10.058         1.027             1.204              4.218                2.128         81.365

Oven        10.058         1.039            1.202              4.303                2.097          81.301

65°C       10.028          0.895             1.128              2.987                1.895          83.067

S D         10.057          0.874              1.116               2.954               1.905          84.094

95°C        9.848            0.754           1.154               2.364                 1.854          84.026

S D          9.901            0.739            1.172               2.422                 1.866          83.900

Table 4.2: The Mean of the Proximate Composition (%) of Yam Flour under Different Treatments

Treatment Moisture Crude Fat Crude Fibre Crude Pro Ash Carbohydrate

Fresh           59.57      0.76             1.23              3.66             2.51                 32.28

Oven60°C

(65°C bl)     11.08        1.03            1.16            4.82              2.31                 79.60

Oven 60°C 10.06         1.03            1.20            4.26              2.11                  81.33

(95°C bl)

Sun dried

(65°C bl)     10.04         0.88            1.12           2.29              1.09                  83.08

Sun dried

(95°C bl)     9.87           0.75            1.16           2.39               1.86                 83.96

Table 4.3 One-Way ANOVA of the Proximate Composition of Yam flour

Component F-statistic p-value Significance

Moisture                450,744.72             0.0000                                      Significant

Crude Fat              190.83                    0.0000                                      Significant

Crude Fibre           33.85                      0.0008                                      Significant

Crude Protein        1,017.02                 0.0000                                      Significant

Ash                        1,012.07                 0.0000                                      Significant

Carbohydrate         271,949.60             0.0000                                     Significant

Table 4.4: Two-Way ANOVA Summary (p-values) for Proximate Parameters

Component Blanching Temp (p) Drying Meth (p) Interaction (p) Significance

Moisture          0.0000                         0.0000              0.0000	Significant

Crude Fat	0.0021 	             0.0000	 0.0017           Significant

Crude Fibre	0.0033             	 0.0038	  0.9716	Main effects only

Crude Prote	0.0000                        	 0.000                0.7564 	Main effects only

Ash	            0.0002 	             0.0000	  0.0010	Significant

Carbohydr	0.0000   	             0.0000	 0.0004 	Significant

Table 4.5 presented the Result of the Drying Rate of Yam Flour Samples

Treatment     Initial Weight (g) Final Weight (g)   Drying Time (h)	Drying Rate (g/h)

Oven drying

(65°C bla)	165	                96.87	             15                     	4.54

Oven drying

(95°C bla)	165	                86.67	             15	                        5.22

Sun drying

(65°C bla)	165	               98.15         	             18	                        3.71

Sun drying

(95°C bla)	165	               62.42                         18	                        5.70

Figure: 4.1: Drying Rate of Yam Flour Samples

## 4.2 DISCUSSIONS

Discussions on Table 4.1 Proximate Composition (%) of the Samples

### 4.2.1: Moisture Content

The moisture content of the fresh yam samples was approximately 59.63%, which was significantly reduced following drying. Among the dried samples, sun drying (particularly with prior blanching at 95°C) resulted in the lowest moisture content (9.87%), while oven drying at 60°C with blanching at 65°C retained slightly more moisture (11.06%). Although sun drying achieved marginally lower moisture levels, the controlled nature of oven drying provides more reliable and sanitary conditions for moisture removal. These findings align with the observations of Johnson et al. (2018), who noted that sun drying under intense ambient heat may exceed mild oven temperatures, resulting in enhanced moisture loss.

The lower moisture content achieved at higher temperatures aligns with the goal of preserving yam flour by reducing microbial activity.

### 4.2.2: Crude Protein

Protein content increased with oven drying (4.82% for samples blanched at 65°C) but decreased in sun-dried samples (2.95%–2.99%).

Smith and Adeyemi (2019) noted similar reductions in sun-dried samples, attributing this to prolonged exposure to inconsistent heat and light, which may denature proteins.

### 4.2.3: Crude Fat

Oven drying retained more crude fat (1.02%) compared to sun drying (0.75%–0.89%).

Olayinka and Bello (2020) observed that oxidative losses during sun drying reduce fat content, while controlled oven conditions help retain fat.

### 4.2.4: Ash and Crude Fibre Content

Ash and crude fibre contents showed minimal variations across treatments. Fresh yam had 2.50% ash and 1.23% fibre, with slight reductions noted after drying.

Adebayo et al. (2017) also reported that ash and fibre are less affected by blanching or drying methods, as these components are relatively stable.

### 4.2.5: Carbohydrate Content

Carbohydrate content increased as moisture decreased, with sun-dried samples recording the highest levels (83.07%–84.03%).

This result aligns with Okonkwo and Eze (2016), who emphasized that moisture reduction concentrates the carbohydrate fraction in tuber flours.

## 4.3 Discussion on Table 4.3 One-Way ANOVA and Tukey HSD Results

One-way ANOVA revealed significant differences (p < 0.05) among the five treatments. Tukey’s HSD test grouped the treatments with different superscripts as statistically different. Oven drying preserved more protein, while sun drying significantly increased carbohydrate content.

## 4.4 Discussion on Table 4.4 A two-way ANOVA

A two-way ANOVA was used to assess the effects of blanching temperature, drying method, and their interaction.

Drying method and blanching temperature significantly affected most nutritional parameters. Interaction effects show that the influence of one factor depends on the level of the other. Oven drying was best for protein retention; sun drying (after 95°C blanching) favoured` moisture loss and carbohydrate concentration.

## 4.5 Discussion on Drying Rate Table 4.5

The drying rate results revealed key differences between oven drying at 60°C and sun drying:

1. Oven drying had a higher drying rate than sun drying under both blanching temperatures.

The highest drying rate (16.22 g/h) was recorded in oven drying at 60°C with 95°C blanching.

The lowest drying rate (12.88 g/h) was observed in sun drying with 65°C blanching.

This confirms that controlled oven conditions facilitate faster moisture removal compared to sun drying, where drying efficiency is affected by environmental factors such as temperature, humidity, and airflow.

2. Blanching at 95°C enhanced the drying rate compared to 65°C.

Samples blanched at 95°C dried faster in both oven and sun drying.

This may be attributed to heat-induced cell structure breakdown, which accelerates moisture loss.

Similar findings were reported by Adebayo et al. (2017), where higher blanching temperatures led to increased drying efficiency in yam processing.

3. Sun drying had the slowest drying rate due to environmental variability.

Although sun drying at 95°C blanching had a slightly improved drying rate (14.87 g/h), it was still lower than oven drying at 60°C.

This aligns with studies by Aluko and Ibrahim (2021), who reported that sun drying is less efficient due to inconsistent heat exposure and possible reabsorption of moisture during the drying process.

The results suggest that oven drying at 60°C is more efficient than sun drying for yam flour production, ensuring faster drying and reducing the risk of microbial contamination.

However, sun drying remains a viable option in rural areas where electricity costs or oven availability are limiting factors.

## 4.6. Comparative Discussion with Existing Research and Practical Implications

This section compares the study’s findings with existing research and explores its relevance to yam flour production.

Comparison with Literature

4.6. 1: Moisture Content: The moisture reduction trend aligns with Johnson et al. (2018) and Adebayo et al. (2017), who reported that oven drying removes moisture more effectively than sun drying, ensuring longer shelf life.

4.6.2: Protein Retention: The study’s findings match Smith & Adeyemi (2019), who found that sun drying leads to protein denaturation due to prolonged heat exposure.

4.6.3: Fat Stability: Similar to Olayinka & Bello (2020), the research confirmed oxidative fat losses in sun drying, making oven drying a better option for fat retention.

4.6.4: Drying Rate: The drying efficiency aligns with Aluko & Ibrahim (2021), who observed that higher blanching temperatures increase drying rates due to enhanced moisture diffusion.

# CHAPTER FIVE

# CONCLUSIONS AND RECOMMENDATIONS

## 5.1 Conclusions

The following conclusion was drawn from the results obtained from this study

1. Oven drying was more effective in reducing moisture content and preserving protein and fat levels compared to sun drying.

2. Sun drying yielded the highest carbohydrate content, making it suitable for carbohydrate-dense applications.

3. Blanching at higher temperatures (95°C) slightly reduced ash and fibre retention compared to 65°C.

## 5.2 Recommendations

1. For yam flour production, oven drying at 65°C is recommended for optimal nutritional preservation.

2. Further studies should investigate the sensory qualities and functional properties of yam flour processed under similar conditions.

3. Implement energy-efficient drying technologies for large-scale production.

4. Explore the impact of blanching time and drying method on other yam species.

# REFERENCES

# AOAC International. (2010). Official methods of analysis of AOAC International (18th ed.). Gaithersburg, MD: AOAC International.

AOAC International. (2019). Official methods of analysis of AOAC International (21st ed.). Gaithersburg, MD: AOAC International.

Adebayo, K., Ojo, O., & Adeola, A. (2017). Effects of drying methods on the proximate and mineral composition of yam flour. International Journal of Food Science and Nutrition, 6(2), 150–158.

Adejumo, B. A., Adekola, K. A., & Olayemi, M. S. (2020). Effect of processing methods on proximate and functional properties of yam flour. Journal of Food Science and Engineering, 10(1), 45–52.

Adepoju, A. A., & Oyewole, O. B. (2018). Effect of blanching and drying methods on the nutritional quality of yam flour. Nigerian Food Journal, 36(1), 24–31.

Adepoju, A. A., Olatunji, O., & Adebowale, A. R. (2018). Nutritional composition of different yam varieties in Nigeria. African Journal of Food Science, 12(4), 89–95.

Akinwande, B. A., & Ogundele, O. M. (2020). Blanching and drying effects on the proximate composition of yam flour. African Journal of Root and Tuber Crops, 11(3), 234–240.

Aluko, T. A., & Ibrahim, M. (2021). Comparative analysis of sun drying and oven drying methods on yam flour quality. Journal of Food Processing Technology, 9(3), 240–248.

Chen, Y., Lu, X., Zhang, C., & Wang, S. (2017). Comparative analysis of freeze-drying and microwave drying on yam flour. Journal of Food Engineering, 215, 104–112.

Chen, Y., Tian, Q., & Wang, X. (2017). Effects of drying methods on the physicochemical properties of yam powder. Drying Technology, 35(10), 1234–1241.

Dereje, T., Abebe, Y., & Mengistu, M. (2020). Production and characterization of yam flour from selected Ethiopian varieties. International Journal of Food Science, 2020, Article ID 121756.

FAO. (2021). Food and Agriculture Organization statistical database. Rome, Italy: FAO. Retrieved from http://www.fao.org

FAO. (2022). Yam production statistics in West Africa. Rome, Italy: Food and Agriculture Organization of the United Nations.

Johnson, M. K., Eze, C., & Bello, A. (2018). Moisture and protein retention in yam flour under varying drying methods. African Journal of Agricultural Research, 13(6), 290–298.

Ju, H., Li, Y., & Wang, L. (2015). Traditional sun drying techniques in root crop processing. Asian Food Science Journal, 4(3), 167–174.

Ju, H., Wu, Q., & Song, W. (2016). Optimization of hot air drying conditions for yam flour production. International Journal of Agricultural Technology, 12(1), 145–153.

Kanu, A. M., Obieze, C. O., & Okoye, E. C. (2018). Economic importance of yam in African agriculture. Journal of Root Crops, 44(1), 34–40.

Kumar, A., & Mishra, B. (2017). Impact of drying techniques on quality of root and tuber flours. International Journal of Food Processing Technology, 4(2), 112–120.

Kumar, S., Singh, R., & Ali, A. (2019). Effect of blanching temperature and time on nutritional quality of dried yam flour. Journal of Food Processing and Preservation, 43(4), e13847.

Musa, A. A., Okoye, C. J., & Ibe, F. C. (2021). Influence of blanching temperatures on vitamin C and microbial load in yam flour. Nigerian Journal of Food Science and Technology, 9(1), 55–62.

Obidiegwu, J. E., Akpabio, E. M., & Uchegbu, N. (2020). Review on yam-based food products and food security in Nigeria. African Journal of Food Science, 14(3), 45–53.

Ojinnaka, M. C., Uzochukwu, S. V. A., & Ogbulie, T. E. (2017). Nutritional composition and functional properties of yam flour. Journal of Food Technology Research, 4(1), 20–27.

Okonkwo, P. C., & Eze, A. B. (2016). Influence of blanching and drying methods on carbohydrate content of tuber-based flours. Nigerian Journal of Food Science, 8(1), 180–190.

Okonkwo, P. C., Umeh, A. N., & Ajayi, K. A. (2020). Post-harvest handling and storage constraints affecting yam productivity. Journal of Agricultural Extension, 24(2), 93–102.

Olayinka, J. A., & Bello, R. K. (2020). Effects of sun drying on fat retention and oxidative stability in processed yam flour. Food and Nutrition Research, 7(4), 456–462.

Otegbayo, B. O., Samuel, F. O., & Ajayi, O. I. (2019). Comparative analysis of physicochemical properties of yam flour from selected Nigerian varieties. Food Chemistry, 290, 320–328.

Padhan, P., & Panda, G. (2020). Food security in the context of climate change and population growth. World Development Perspectives, 18, 100229.

Rees, D., & Bancroft, R. D. (2018). Post-harvest losses in yam value chains: Causes and interventions. Journal of Agricultural Science and Technology, 20(1), 77–89.

Setyawan, A. A., Wulandari, R., & Permadi, M. B. (2021). Functional and nutritional properties of dried yam flour for food industry applications. International Journal of Agricultural Research and Technology, 11(2), 89–97.

Smith, R. D., & Adeyemi, O. A. (2019). Protein retention in yam flour processed under different drying methods and temperatures. Journal of Agricultural Biotechnology, 11(2), 300–310.

Wahab, F. A., & Adeyemi, S. O. (2016). Economic potentials of yam flour processing in Nigeria. African Journal of Agricultural Economics and Rural Development, 4(5), 234–240.

Wahab, F. A., Adeyemi, S. O., & Okunlola, F. O. (2016). Influence of blanching on nutritional composition of yam flour. Journal of Food Technology and Preservation, 10(3), 123–128.

Xiao, H., Liu, H., & Zhang, J. (2015). Drying kinetics and modeling of yam slices during hot air drying. Food and Bioproducts Processing, 93, 90–98.

Zhang, X., Ding, Y., Liu, Z., & Wang, M. (2023). Nutritional profile of medicinal yam rhizome: A review. Journal of Medicinal Plants Research, 17(2), 112–120.

Zhang, X., Zheng, Y., & Liu, Z. (2023). Health benefits of Dioscorea spp.: Traditional applications and bioactive compounds. Frontiers in Nutrition, 10, 117832.

Zou, Y., Wu, X., & Li, T. (2021). Functional food potential of yam-based products. Journal of Food Science and Technology, 58(6), 2072–2080.